[image: image1.png]



2014
Railway Traffic Safety Report 
State Railway Inspectorate under the Ministry of Transport and Communications 
30 September 2015

Table of Contents 
A.     Introduction ............................................................................................................................................ 1
A.1.      Objectives and Scope of the Railway Traffic Safety Report ................................................ 1
B.     Overall Safety Level in 2014 ............................................................................................................. .2
B.1.      General Conclusions on 2014 …….................................................................................................... 2
B.2.      National Safety Strategies, Programmes and Initiatives  ................................................................ 2
B.3.      General Information on 2014........................................................................................................ 4
B.4.      Priorities for 2015 ……….……............................................................................................................. 4
C.     Safety Development………………….............................................................................................\................. 5
C.1.      Analysis of Trends...................................................................................................................... 5
C.2.      Results of Traffic Safety Measures (Recommendations)..................................................... 6
C.3.      Measures Unrelated to Traffic Safety Recommendations …........................................................... 6
D.    Supervision of Railway Undertakings (Carriers) and Railway Infrastructure Managers .......................... 7
D.1.      Strategy and Plans ........................................................................................................................... 7
D.2.      Human Resources ……...................................................................................................................... 7
D.3.      Competences …................................................................................................................................ 7
D.4.      Decision-making ………….................................................................................................................. 8
D.5.      Cooperation …………......................................................................................................................... 8
D.6.      Implementation of Measures ......................................................................................................... 8
E.     Issuing of Safety Certificates and Safety Authorisations………………………………………………......................... 9
E.1.      Information about the Procedures for the Issuing of Safety Certificates and Safety Authorisations ……………………………….................................................................................................................................... .9
E.2.      Use of Uncertified Interoperability Constituents ............................................................................. 9
E.3.      Communication with National Safety Authorities of Other States on Issues Related to the Issuing of Safety Certificates ……….......................................................................................................................... 9
E.4.      Issues Related to the Procedural Aspects of the Issuing of Safety Certificates and Safety Authorisations …………………………………………………………………………………………………………………..……............... 9
E.5.      Feedback  ………................................................................................................................................ 9
F.     Changes in Legislation on Traffic Safety in 2013 ……………………….......................................................... 10
F.1.      Railway Safety Directive ………........................................................................................................ 10
F.2.      Changes in Legislation in 2013 ……….............................................................................................. 10
.
G.    Application of the Common Safety Method (CSM) on Risk Evaluation and Assessment ..................... ..11
G.1.      NSA Experience.............................................................................................................................. 11
G.2.     Communication and Feedback …................................................................................................... 12
G.3.     Review of National Regulations ..................................................................................................... 12
H.    Alternative Measures for the Certification of Entities in Charge of Maintenance ................................. 13

Annex A. General Safety Indicators …………................................................................................................... 15
Annex B. Amendments to Legal Acts............................................................................................................. 22
Annex C. Significant Traffic Accidents in Lithuania in 2014 ........................................................................... 31

A. Introduction 
A.1.  Purposes and Scope of the Railway Traffic Safety Report 
In accordance with Article 23 of Directive of the European Parliament and of the Council No 2004/49/EC on safety on the Community’s railways1, national safety authorities of the European Union must prepare, by 30 September every year, an annual railway traffic safety report (‘The Traffic Safety Report’). 
The objectives of the Traffic Safety Report are as follows:
1. To provide summarised information on the safety condition of the railway system of the Republic of Lithuania in 2014 and to compare it with the data provided in 2007–2013;
2. To present the trends in the improvement of railway transport traffic safety in 2014;
3. To provide an overview of the problems encountered by the State Railway Inspectorate under the Ministry of Transport and Communications (‘Inspectorate’) in ensuring railway transport traffic safety.
The Traffic Safety Report provides the following information:
1. Improvement of traffic safety and the Common Safety Indicators set by the Minister of Transport and Communications of the Republic of Lithuania;
2. Amendments to legal acts governing traffic safety;
3. Improvement of the procedures for the issuing of safety certificates to railway undertakings (carriers) and granting of safety authorisations to railway infrastructure managers;
4. Results of traffic safety audits of railway infrastructure managers and railway undertakings (carriers) and experience in the area of traffic safety;
[image: image2.png]




5. Maintenance of freight wagons used in the 1520 mm gauge railway network.
In accordance with Article 6(18) of the Republic of Lithuania Law on Railway Traffic Safety, the Inspectorate publishes the Traffic Safety Report on its website2 and submits it to the European Railway Agency3 (ERA).
This Report covers information on traffic safety throughout the railway network of the Republic of Lithuania.
The Traffic Safety Report is being submitted for the ninth time. It is prepared in accordance with the ERA guide for the structure and use of tables in the Annual Traffic Safety Reports by national safety authorities.
The Traffic Safety Report has been prepared on the basis of the statistical data collected and systemised by the Inspectorate and the traffic safety reports of railway infrastructure managers and railway undertakings (carriers) for 2014.

1 Directive 2004/49/EC of the European Parliament and of the Council on safety on the Community’s railways (OJ L 220, 2004 6 21, p. 16)

2 http://www.vgi.lt/lt/veikla/saugos-ataskaita
[image: image3.png]


3 Regulation (EC) No 881/2004 of the European Parliament and of the Council of 29 April 2004 establishing the European Railway Agency (Agency Regulation)

B. Overall Safety Level in 2014
B.1.  General Conclusions on 2014  
Based on an analysis of the safety data on the Lithuanian railway transport system for 2014, one may conclude that the overall safety level remained high in 2014 compared with previous years. Furthermore, both the numbers of significant traffic accidents and of deaths on the railways have decreased:
· The total number of traffic accidents in 2014 compared with 2013 decreased from 61 to 59;
· The number of significant traffic accidents in 2014 compared with 2013 decreased 33.3%;
· The number of deaths in 2014 compared with 2013 decreased 35.3%;
Notably, this positive trend is also confirmed by the National Reference Values (NRV) as of the end of 2014:
· The NRV of risk to passengers is 100% lower than the permissible NRV established by the European Commission (EC) by Commission Decision 2012/226/EU;
· The NRV of risk to employees is 66% lower than the permissible NRV established by Commission Decision 2012/226/EU;
· The NRV of risk to users of level crossings is 52% lower than the permissible NRV established by Commission Decision 2012/226/EU;
· The NRV of risk to unauthorised persons in the railway zone is 63% lower than the permissible NRV established by Commission Decision 2012/226/EU;
· The NRV of risk to persons classified as ‘Other’ is 100% lower than the permissible NRV established by Commission Decision 2012/226/EU.
This data confirms that the Lithuanian railway transport system complies with the European Union’s standards and long-term objectives.


Table 1. NRV values for 2014 after Quarter IV
[image: image4.png]


In 2014, 16 significant railway traffic accidents and 43 minor railway traffic accidents occurred in Lithuania. A detailed list of significant accidents is provided in Annex C.  
B.2. National Safety Strategies, Programmes and Initiatives 
In 2014, Lithuania’s transport policy was developed in line with the National Transport Development Programme for 2014–2022 approved by Resolution of the Government on 18 December 2013. The strategic objective of the programme is to develop a sustainable, environmentally-friendly, competitive transport system that creates a high added value.
Other objectives of the programme:
1. Increase freight and passenger mobility by improving EU Trans-European Network Corridors and their connections with the transport network of national and local significance and by increasing the efficiency of interoperability between different modes of transport;
2. Improve the competitiveness of the transport sector and the quality of transportation and logistics services by pursuing active transport policies;

3. Promote sustainability of local transport systems (urban and suburban);
4. Increase energy efficiency in transport and reduce the impact of transport on the environment;
5. Improve safety and interoperability of traffic including railway traffic.
In 2014 the Inspectorate continued the development of the safety strategy, which was approved in 2015. Based on Sweden’s best practices, the ‘zero’ vision has been formulated: ‘Safe society and safe railway transport with no traffic accidents, deaths and injuries’.  The Inspectorate’s safety strategy based on this ‘zero’ vision was developed using the X-Matrix measure of the LEAN Government Methods Guide. This means that, in developing the said document, individual objectives of the Inspectorate’s employees have been linked to the long-term institutional objective – to ensure zero numbers of deaths, injuries and accidents on the Lithuanian railways. Accordingly, the strategy consists of three parts comprising measures to reduce the numbers of deaths and injuries and to prevent traffic accidents.
In 2014 the Inspectorate continued the implementation of a project ‘Creation of Public E-Services for Railway Transport Risk Management and Supervision’. Under this project, the service of monitoring of the public railway infrastructure of the Republic of Lithuania and the service of managing the hazards related to changes in the railway system have been transferred to the electronic environment.  
For 2015–2020, the Inspectorate has planned a number of projects aimed at increasing safety and interoperability of the Lithuanian railway transport system. The projects include:
1. Increasing safety at railway crossings. An intellectual transport system (ITS) will be developed in order to reduce the number of traffic accidents and the number of violations at railway crossings. The ITS will consist of the information system and the acquired equipment for real-time monitoring of the crossing status, provide information on traffic conditions to drivers and pedestrians, and record violations committed at the crossings.
2. Transferring the SRI’s services, risk management and monitoring public electronic services to the electronic environment and updating thereof. The project will be aimed at updating and supplementing the following services: traffic risk management, noise control, technical control of structures, and railway infrastructure monitoring modules. In addition, examination of individuals electronically, registration of railway rolling stock, investigation of railway accidents and other electronic services will be implemented. 
3. Most efficient public awareness and education measures. The project will be aimed at identifying those public awareness and information measures that are most efficient in reducing the risk of deaths and injuries in railway protection zones by social groups. Based on the results of the study the Inspectorate will implement a programme on public awareness of safe behaviour in the railway protection zone.  
In 2014, the Lithuanian railway infrastructure manager (Lietuvos Geležinkeliai AB) carried out internal audits of the traffic safety management system and, based on their findings, prepared plans of measures to eliminate deficiencies. In addition, periodic certification and employee training on traffic safety were held.
B.3.     General Information on 2014
In 2014, the Inspectorate issued two Parts A of the Safety Certificate, seven Parts B of the Safety Certificate, 20 Parts A of the Safety Authorisations and 20 Parts B of the Safety Authorisations.
As part of its supervisory activities, in 2014 the Inspectorate carried out 65 inspections of railway undertakings (carriers) and railway infrastructure managers. Violations of legal acts governing the railway traffic safety were found during inspections of 27 of railway undertakings (carriers) and railway infrastructure managers.
Internal audits of the Inspectorate were carried out in 2014 according to methodologies of the European Railway Agency (ERA): a cross-assessment and a Matrix assessment. The internal audits covered the examination of maturity of various railway risk management aspects and management systems, for which a 5-point scale was used. The internal audits were completed in 2015, recommendations for the improvement of legal and organisational processes of the Inspectorate were made. 
A client satisfaction survey was conducted by the Inspectorate at the end of 2014. The summarised data showed that the services provided by the SRI receive good evaluation by the clients. The satisfaction index was 83%.   The following criteria were rated as highest on a 10-point scale:  ‘SRI premises are clean and orderly’ (9.21), ‘SRI specialists wear neat and clean clothes appropriate for work’ (9.12), ‘SRI specialists are polite and communication with the clients is pleasant’ (8.91).  The lowest score was recorded for the following: ‘Parking at the SRI is convenient’ (6.92), ’Information provided by the SRI is easily accessible, detailed and understandable’ (7.68), ’SRI specialists are competent and knowledgeable in their area’ (7.79).
B.4.     Priorities for 2015
The Inspectorate has set the following tasks for 2015:
· 
Ensure that the reference values for the Republic of Lithuania for 2014 established by Commission Decision No 2012/226/EU4 are not exceeded;
· Exercise control over the placement on the market of the interoperability constituents of the railway system and ensure that only those constituents that are compliant with the essential requirements are used in the implementation of the Rail Baltica project;
· Complete the implementation of additional e-services under the EU-funded project ‘Creation of Public E-Services for Railway Transport Risk Management and Supervision’:
o  E-service for the rail system change risk management;
o Service for the monitoring of the infrastructure of the railway system of the Republic of Lithuania electronically.
· Implement standard safety management systems in collaboration between the SRI and infrastructure managers to improve the safety of railway undertakings and reduce the number of traffic accidents in the area of railway transport;
· Take part in the programme for priority countries implemented by the ERA, specifically, in its sub-elements: Cross assessment and Matrix assessment. Assist the ERA auditors by providing the required information and documents, implement the recommendations made on completion of audits; 
· Prepare a project application to the Ministry of the Interior for support for the implementation of the LEAN Government Quality Management methods in the activities of the Inspectorate;
  Prepare a project application to the Ministry of the Interior for support for the development of an Information Security Management System according to LST ISO/IEC 27001:2013 or equivalent standard at the Inspectorate in order to ensure security of both incoming and outgoing information.
4 Commission Decision 2012/226/EU on the second set of common safety targets as regards the rail system (OJ L 115, 2012 p. 27)

C. Safety Development
C.1.     Analysis of Trends 
59 traffic accidents occurred in 2014 including 16 significant traffic accidents: train collisions – 0; derailments – 0; traffic accidents at level crossings – 5; traffic accidents due to the fault of unauthorised persons – 11; rolling stock fires – 0; incidents – 11.

Since 2009, there has also been a decreasing trend in the number of traffic accidents at level crossings. It increased slightly in 2014 (+1) but the decrease compared with the average of the past three years is 12.4%.
The number of deaths at level crossings has been varying recently. Despite the fact that in 2014 the number of such deaths increased by 2 persons,

Table 2.  Accident statistics for 2013 and losses including working time losses due to accidents 
	Serious accidents 
	Significant  accidents 
	Other traffic accidents
	Incidents
	Injuries 
	Deaths  
	Losses including working time losses (EUR/hr)

	
	Total
	Per train-km
	
	
	Total
	Per train- km
	Total
	Per train- km
	

	0
	16
	1.14×10-6
	43
	11
	8
	0.57×10-6
	11
	0.57×10-6
	10.724,564
EUR/  79 hr
53 min


The number of significant traffic accidents was decreasing in Lithuania in 2009–2014 inclusively. Compared with previous years, the number of traffic accidents decreased slightly in Lithuania in 2014 (-2). For five years already there have been no train collisions that resulted in considerable losses; the number of derailments has been zero for four years in a row. During the past two years there have been no fires that tend to cause lots of damage and loss of life. 
Since 2009 the number of deaths in railway traffic accidents has been decreasing. The year 2014 was not an exception: there number of deaths decreased by 6 compared with 2013.
The number of deaths of unauthorised persons has been decreasing, as the total number of deaths, since the start of the accident recording. Compared with 2013 a reduction of over 53% has been achieved (-8).
The trends toward the decreasing number of traffic accidents caused by unauthorised persons have been decreasing since 2010. The decrease is obvious when compared with 2013: the number of traffic accidents caused by unauthorised persons has decreased by almost 50% (-9).

a general decreasing trend is observed: based on the indicators for the past three years, one may conclude that in 2014 this indicator was 20.3 % lower than that in 2013.
While in 2009–2013 the number of deaths on level crossings remained stable, it more than doubled in 2014 (from 2 to 5).
The charts of these and other relevant indicators are provided in Annex A. 
In 2014, as in previous years, unlawful actions by level crossing users and unauthorised persons remain the main cause of traffic accidents.  The most frequent violations include driving or walking across railway tracks at places not designated for this purpose; overtaking of other vehicles waiting at the crossing for the train to pass; driving onto the level crossing when the prohibiting traffic light is on and ignoring the staff on duty, with the barrier lowered or being lowered; unauthorised raising of the barrier or passing it by; driving on the crossing and having to stop because a vehicle in front has not fully cleared the crossing.  
Other causes of accidents include agricultural, road building, construction and other vehicles that were not prepared specifically for this purpose; disregarding the ‘Stop’ sign before the crossing; 
and unauthorised presence of persons within the railway and its equipment protection zones.
Other traffic accidents occur due to the condition of the railway infrastructure or rolling stock or deficiencies of traffic organisation and management. 
C.2.     Results of Traffic Safety Measures (Recommendations) 
In 2014 the Inspectorate has not received any safety recommendations from the Serious Accident Investigations Manager. In the said period the Inspectorate presented four main safety measures (recommendations). 

C.3.     Measures Unrelated to Traffic Safety Recommendations 
In 2014 the Inspectorate did not submit any measures not related to the traffic safety recommendations.
A detailed list of significant traffic accidents in Lithuania in 2014 is presented, together with conclusions, in Annex C.

Table 3. Main safety measures implemented by the Inspectorate in order to improve safety indicators 
	Description of the traffic safety recommendation
	Description of the traffic safety measure
	Implementation status

	Carry out checks in order to determine:   1. Whether materials necessary for the thermit welding are properly stored; 2. Whether training of thermit welders is appropriate; 3. Whether certification of non-destructive testing operators is appropriate; 4. What are the controls implemented by Lietuvos Geležinkeliai AB to ensure that thermit welding and non-destructive testing is performed by certified staff; 5. Whether thermit welders comply with the thermit welding technology; 6. Whether performance and control of non-destructive testing is appropriate.
	Carry out checks of thermit welding at Lietuvos Geležinkeliai AB
	In progress

	Develop the railway traffic safety management system in accordance with Article 7(1) of the Republic of Lithuania Law on Railway Traffic Safety. 
	Develop the SMS of Malkų Įlankos Terminalas UAB
	Completed 

	To ensure that Lietuvos  Geležinkeliai AB properly manages risks related to contractors activities, obligate the company to supplement the SMS by including:
1.  Procedures aimed at ensuring that contractors discharge their contractual obligations (organise and perform construction works according to an approved construction project and legal acts, the staff hold the requisite qualifications certificates etc.); 2. Procedures aimed at transmitting information (related to emergency plans, speed and other limitations, equipment failures etc.) between contractors and the infrastructure manager during works, under both normal and poor operating conditions.
	Supplement the SMS of Lietuvos Geležinkeliai AB
	Completed

	Include a requirement in the books of Lietuvos  Geležinkeliai AB’s stations to the effect that, upon preparation of a manoeuvring route by the manoeuvring manager, the permitting signal and the permission manoeuvre would be given only upon satisfying oneself that the train driver has understood the driving route and the manoeuvring conditions correctly.
	Lietuvos Geležinkeliai AB: to supplement the railway station books requirements related to manoeuvring
	In progress


D. Supervision of Railway Undertakings (Carriers) and Railway Infrastructure Managers 
D.1.     Strategy and Plans
Priorities and target activities of the Inspectorate in the area of supervision of railway undertakings (carriers) and railway infrastructure managers are established in: 
1.  Legal acts of the Republic of Lithuania. Types of scheduled inspections to be performed by the Inspectorate as specified in Order No V-604 of 27 December 2010 of the Head of the State Railway Inspectorate under the Ministry of Transport and Communications ‘On the approval of rules for carrying out routine and non-routine inspections by the State Railway Inspectorate under the Ministry of Transport and Communications’. Inspection intervals are specified in the legal acts governing the activities of business entities.
2.  The available information on activities carried out by business entities that might violate the legal provisions and therefore pose risk to the public interest.
The entities’ supervision strategies and the inspection plans are formulated:
· In accordance with the Lithuanian legal acts stating the intervals for the inspection of entities;
· Acting upon an entity’s request to carry out an inspection/provide consulting;
· With the purpose of determining whether the entity complies with the requirements set during previous inspections by the Inspectorate;
· Acting upon persons’ notifications or complaints about violations committed by business entities;
· Based on data on traffic accidents (conclusions drawn upon investigations, recommendations, results presented in reports);
· Based on to information collected during the evaluation of railway transport traffic safety management systems operated by railway undertakings (carriers) and railway infrastructure managers;
· Based on reports submitted to the Inspectorate by entities.

Inspection schedules may be changed in the following cases:
· Changes in the legal status of a business entity or area(s) of its operations;
· Amendments to provisions of legal acts governing the carrying out of inspections.
Based on experience gained in supervision activities, the Inspectorate carries out monthly reviews of the inspection plan to satisfy itself that the target activities, use of data and/or information from various sources, supervision results and resource allocation have remained relevant, and modifies priorities as required.
D.2.     Human Resources
Five types of inspections were carried out in 2014:
1.  Inspections of dangerous facilities;
2.  Inspections of maintenance of structures;
3.  Inspections of certified operations;

4.  Inspections of licenced operations;
5.  Inspections of other railway operations.
Specifically, an inspection of a hazardous facility (including the drafting of an inspection report) takes about 5 hours. In 2014, approx. 70 hours were spent on inspections of hazardous facilities and approx. 160 hours were spent on inspections of maintenance of structures 2014. In all, approx. 230 hours were spent on the inspections.
5 employees of the Inspectorate carried out the inspections in 2014, approx. 46 hours of inspections per employee.  
There were 251 working days (2008 working hours) in Lithuania in 2014. Each employee spent on average 2.3% of their working time on inspections/audits.
D.3.     Competences
Each employee of the Inspectorate that carries out the activities of supervision of entities must possess requisite competences.   The competences are set in the employee’s job description and are tested at hiring and during annual evaluations of employees. 

Each inspection is carried out by two employees. The more experienced employee is assigned as the manager of the inspection. The less experienced employee learns from the manager of the inspection by performing the tasks assigned to them and observing the work of the manager.
Prior to starting the inspection, the employees must get acquainted with the Rules for Inspections, legal acts governing the activities of the business entity being inspected, and be aware of the rights and duties of business entities.
Prior to starting the inspection, the employee identifies its purpose and determines which documents are to be inspected.
D.4.     Decision-making
Legal compliance is monitored and assured by inspecting business entities and providing consulting to them. If a violation is identified the Inspectorate evaluates its scale, potential effects for the public and other circumstances of the violation and makes a decision on sanctions to be applied.
The Inspectorate aims to use the consulting practices as widely as possible in order to assist the entities to understand and comply with legal provisions. The Inspectorate seeks benevolent cooperation with entities; therefore, it rarely imposes administrative penalties and attempts to use other incentive measures.
Notably, in 2014 the Inspectorate received no comments or complaints from railway undertakings (carriers) and railway infrastructure managers regarding the inspections performed and decisions made by the Inspectorate.

D.5.     Cooperation
Implementing the provisions of Article 8 of Commission Regulation (EU) 1077/2012 on a common safety method for supervision by national safety authorities after issuing a safety certificate or safety authorisation, on 10 December 2014 the Inspectorate concluded a cooperation agreement with the Latvian NSA and on 21 July 2015 with the Polish NSA.  
Both agreements have been drafted according to the provisions of the ERA guide on the application of Article 8 of Commission Regulation (EU) 1077/2012 on a common safety method for supervision by national safety authorities after issuing a safety certificate or safety authorisation5.
D.6.     Implementation of Measures 
The Inspectorate supervises the implementation of all traffic safety measures. All the traffic safety measures identified for 2014 have been implemented or their implementation is underway.
5  A guide  for  national  safety  authorities in  applying Article 8  of  Commission  Regulation  on  a  common safety method for supervision by national safety authorities after issuing a safety certificate or safety authorisation.

E.  Issuing of Safety Certificates and Safety Authorisations 
E.1.
Information about the Procedures for the Issuing of Safety Certificates and Safety Authorisations 
Safety certificates and safety authorisations in the area of railway transport traffic safety (here3nafter referred to as the ‘authorisations’) are issued according to a procedure prescribed by the Rules for the Safety Certification of Railway Undertakings (Carriers) and Railway Infrastructure Managers approved by Order No 3-37 of 23 January 2003 of the Minister of Transport and Communications of the Republic of Lithuania. Interested parties can find the legal acts governing the issuing of safety certificates and authorisations in the database of the Seimas (Parliament) of the Republic of Lithuania6 and on the website of the Inspectorate7. The latter also publishes information on the issuing of safety certificates and safety authorisation in the area of railway transport traffic safety (legal grounds, guides, application forms etc.)8 as well as a list of all undertakings issued with safety certificates and/or safety authorisations.
E.2.
Use of Uncertified Interoperability Constituents
In 2014, only a certified railway section of 1435 mm gauge Lithuanian-Polish Border–Mockava was used. No uncertified interoperability constituents were used.
Rail Baltica, an international 1435 mm gauge railway project is being successfully implemented in Lithuania. On completion of the project works the Inspectorate will determine whether any uncertified interoperability constituents were used
in the project.

E.3.
Communication with National Safety Authorities of Other States on Issues Related to the Issuing of Safety Certificates 
In 2014, there were no queries from foreign NSAs regarding information on Safety Certificates - Part A issued in Lithuania when a railway undertaking (carrier) applies for the issuing of a Safety Certificate - Part B in another country
In 2013, the Inspectorate submitted no queries to foreign NSAs regarding information on a Safety Certificate - Part A issued in another country when a railway undertaking (carrier) applies for the issuing of a Safety Certificate - Part B in Lithuania.
E.4.
Issues Related to the Procedural Aspects of Issuing of Safety Certificates and Safety Authorisations 
There were no issues related to the issuing of safety certificates or safety authorisations in 2014.
E.5.     Feedback
At the end of 2014, the Inspectorate’s client satisfaction survey was conducted, during which railway undertakings (carriers) were afforded an opportunity to present their comments and complaints regarding the services provided by the Inspectorate.
This survey has shown that 83% of respondents were satisfied with the services provided by the Inspectorate and no complaints regarding administrative services were received. 
Railway undertakings (carriers) may make complaints concerning the Inspectorate’s activities at any time, either by visiting the Inspectorate in person or electronically, by evaluating administrative services provided9.
In 2014, the Inspectorate received no complaints concerning improperly rendered services.

6  http://www3.lrs.lt/dokpaieska/forma_l.htm
7 http://www.vgi.lt/lt/teisine-informacija/teises-aktai
8 http://www.vgi.lt/lt/paslaugos/sertifikavimas


9 http://www.vgi.lt/lt/paslaugos/administracins- paslaugos
F.  Changes in Legislation on Traffic Safety in 2013 
F.1.     Railway Safety Directive
In 2014, there were 33 legal acts in force in Lithuania transposing the provisions of Directive 2004/49/EC of the European Parliament and of the Council on safety on the Community’s railways (Railway Safety Directive) into the national legal system.
See Appendix B, Table 1 for a complete list of these legal acts.
Information about the status of transposition of amendments to the Railway Safety Directive into the national legal system as of the end of 2014 is provided in Annex B, Table 2.
F.2.     Changes in Legislation in 2013 
Essential changes in legal acts and details thereof (title of a legal act, adoption date, contents of the adoption or amendment, and reasons for the adoption or amendment) are set forth in Table 2 of Annex B.

G. Application of the Common Safety Method (CSM) on Risk Evaluation and Assessment 
G.1.     NSA Experience 
The common safety method on risk evaluation and assessment is applied in accordance with the requirements established in Commission Regulation (EC) No 352/2009 of 24 April 2009 on the adoption of a common safety method on risk evaluation and assessment as referred to in Article 6(3)(a) of Directive 2004/49/EC of the European Parliament and of the Council (‘RegulationNo 352/2009’).      Regulation No 352/2009 in respect of Article 10(2) has been applied in full since 1 July 2012; however, by that date, it had been applied to:
1. All important technical changes affecting vehicles as defined in Article 2(c) of Directive 2008/57/EC of the European Parliament and of the Council of 17 June 2008 on the interoperability of the rail system within the Community (the ‘Directive’);
2. All important changes of structural subsystems required in accordance with Article 15(1) of the Directive or the Technical Specifications for Interoperability (TSI).
In 2014, acting in accordance with Article 6(3) and (4) of Regulation No 352/2009, the Inspectorate participated as an independent assessment body in the risk management processes related to changes in the railway system and implemented  by railway undertakings (carriers) and railway infrastructure managers. By analysing the results of the processes the Inspectorate determines the processes’ compliance with the requirements set out in Regulation No 352/2009 and issues a Safety Evaluation Report.  The report contains information on the relevant change to the railway system, its implementer, implementation of phases of the risk management process,   risk acceptability level, related evidence, and a conclusion on whether the risk management process has been implemented in accordance with Regulation No 352/2009.


In 2014, the Inspectorate as an independent evaluating organisation participated in the risk management processes related to the technical and operation changes in the railway system. The railway system’s changes were related to:
1. 
Construction of new railway infrastructure;
2. Reconstruction of the existing railway infrastructure;
3. Implementation of specific engineering solutions.
Standard CSM application phases are as follows:
1. Railway undertakings (carriers) and infrastructure managers (‘proposers’) make an initial evaluation, according to Article 4(1) of Regulation No 352/2009, to determine whether a change will affect the railway traffic safety. Upon determination that the railway traffic safety will be affected, the proposer has to substantiate such conclusion.
2. Based on the criteria set out in Article 4(2) of Regulation No 352/2009, the proposers determine whether the changes that will affect the railway traffic safety are significant. The relevant criteria include:
· System’s change failure consequence;
· Novelty used in implementing the change;
· Complexity of the change.
3.   Based on decisions made by the proposers, the Inspectorate:
a) Decides that the change concerned is not significant - the SRI assesses the proposer’s decision and the documents substantiating it; or 
b) Decides that the change concerned is significant;

a risk management process is initiated according to Annex I to Regulation No 352/2009. Acting in accordance with Article 6 of the Regulation, the Inspectorate carries out an independent assessment and provides the proposer with a safety assessment report. 
In 2014, there was no separate independent assessment body in Lithuania which would comply with the requirements of Appendix II to Regulation No 352/2009 and issue safety assessment reports.
G.2.     Communication and Feedback 
As part of its efforts to facilitate the implementation of Regulation No 352/2009 in Lithuania, the Inspectorate organised meetings with the proposers in 2014. The meetings involved questions and answers sessions, explanation of the phases of the risk management process related to changes in the railway system, presentation of the e-service of the railway transport risk management and its functions related to the CSM.   Training for the proposers was also organised, at which the user interface of the GERVIS railway transport risk management e-service was presented.
The e-service publicity campaign was carried out; information about the new service was published on news portals.
At joint working meetings with representatives of the Inspectorate, the proposers presented their experiences and results of the application of the risk management process.
G.3.   Review of National Regulations
In 2014, the Inspectorate completed the formulation of the Procedure for an Independent Assessment of the Risk Management Process.  The document describes and standardises the entire process of independent assessment carried out by the Inspectorate. No amendments have been made to the national safety regulations due to the implementation of Regulation No 352/2009.

H. Alternative Measures for the Certification of Entities in Charge of Maintenance 
As 1520 mm gauge accounts for the majority of the railway infrastructure in the Republic of Lithuania, the country relies on derogations provided for in Article 14a(8)(b) of Directive 2008/110/EC.
Pursuant to Paragraph 2 of the Rules for the Certification of Entities in Charge of Maintenance approved by Order No 3-431 of 11 July 2011 of the Minister of Transport and Communications of the Republic of Lithuania, maintenance entities in charge of maintenance of freight wagons used within the 1435 mm gauge railway network are to be certified in accordance with Commission Regulation (EU) No 445/2011 of 10 May 2011 on a system of certification of entities in charge of maintenance for freight wagons and amending Regulation (EU) No 653/2007 (OJ 2011 L 122, p. 22).Entities in charge of maintenance (ECMs) of freight wagons used within the 1520 mm gauge railway network are certified in accordance with the said rules.
Maintenance entities applying for an ECM certificate must have a maintenance system in place, including the organisational measures, assurance measures, control measures and maintenance performance measures. The certification system ensures that the entity in charge of maintenance has developed its own maintenance system and is capable of meeting the legal requirements. This is aimed at ensuring the safe operating condition of freight wagons.
As of 31 December 2014, there were two certified 1520 mm gauge freight wagon ECMs in Lithuania: Lietuvos Geležinkeliai AB (ECM certificate issued on 16 November 2011) and Skinest Baltija UAB (ECM certificate issued on 7 March 2013).

Where an ECM is an infrastructure manager or a railway undertaking (carrier), the compliance of freight wagons is checked by the national safety authority, which issues safety authorisations to 

infrastructure managers or safety certificates to railway undertakings. 

Railway rolling stock managers may obtain an ECM certificate while registering their rolling stock in the Railway Rolling Stock Register of the Republic of Lithuania.


Annexes

Annex A.  Common Safety Indicators

80
72.33


68..67

60                                                              57
55
48
40
33


45.33


35..67


27.67


Graphs
3 previous years’ average
Individual year’s average

26                24
20
16
0
2009                2010                2011                2012                2013                2014
Year
Figure 1. Numbers of significant traffic accidents in 2009–2014 
4              4                 3.67
3           2..67
2                                                             1.67


1.33

Graphs
3 previous years’ average 
Individual year’s average


1
0                   0
0                                       0                   0                   0                   0                   0
2009                 2010                 2011                 2012                 2013                 2014
Year
Figure 2.  Numbers of train collisions in 2009–2014 

11.67
12
9..67
8
Graphs
3 previous years’ average 
  Individual year’s average
4
1                0.67


0.33                 0

0                                                               0                  0                  0                  0
2009                2010                2011                2012                2013                2014
Years
Figure 3. Numbers of derailments in 2009–2014 

17.67
15..33
15
14                                      13
10                                                                                      8.67
6                  6
5


5.33


4.67
5

Graphs
3 previous years’ average 
Individual year’s average

4                  4
0
2009                2010                2011                2012                2013                2014
Year
Figure 4. Numbers of significant traffic accidents at level crossings in 2009–2014 


40          38.33
33
30
20


3343333717

38..67
27


33.67
30
22
20


23
Graphs
3 previous years’ average 
Individual year’s average

10                                                                                                                                        11
0
2009                2010                2011                2012                2013                2014
Year
Figure 5. Numbers of significant accidents due to the fault of unauthorised persons in 2009–2014

3
3                                      2.67
2
2
1
1
0                   0
0                                       0                   0                   0                   0                   0


Graphs
3 previous years’ average 
Individual year’s average

2009                 2010                 2011                 2012                 2013                 2014
Year
Figure 6. Numbers of major fires in rolling stock in 2009–2014

40          36.67               36


34..33

30            32                31
20

29.67
26
19


25..33
17


20.67


Graphs
3 previous years’ average 
Individual year’s average

10                                                                                                                                        11
0
2009                2010                2011                2012                2013                2014
Year
Figure 7. Numbers of deaths in 2009–2014

8              8
6..67              6.67
6
5
4


6.33              6.33
6


4.67


3.67


Graphs
3 previous years’ average 

4                  Individual year’s average
3
2                                                                                                                  2
0
2009                 2010                 2011                 2012                 2013                 2014
Year
Figure 8. Numbers of deaths on level crossings in 2009–2014


30               29..33
30                                                              28
26
24
20                                                              20

23.33


20..33


16.67


Graphs
3 previous years’ average 

15                15

Individual year’s average

10
7
0
2009                2010                2011                2012                2013                2014
Year
Figure 9. Numbers of deaths – unauthorised persons in 2009–2014 
17
15                                     15
12..67
12
10


13..33
11


12.67


11..33


9             Graphs 
3 previous years’ average 

9               

8                  8
5
0
2009                2010                2011                2012                2013                2014
Year
Figure 10. Numbers of injuries in 2009–2014 


6
6
4.33                                                                                5
4
3
3                                                           2.33


Graphs 
3 previous years’ average 
Individual year’s average
2                   2

2                                       2                   2                   2                   2
0
2009                 2010                 2011                 2012                 2013                 2014
Year
Figure 11. Numbers of injuries on level crossings in 2009–2014

13
11
10
8..33
9
5


10..33           10.33
9
9
6
5


6.67


Graphs
3 previous years’ average 
Individual year’s average

3
0
2009                2010                2011                2012                2013                2014
Year
Figure 12. Numbers of injuries – unauthorised persons in 2009–2014


8
6
Precursors
                                                                                                                                                                    signal passed at danger 
                                                                                                                                                                               signalling failure
4                                                                                                                                                                                            broken rails
broken axles

broken wheels
track widening
2
0            2   0

4   0   0   1

0   1   3   0   0   1

4   1   2   0   0   0

3   1   0   0   1   0

1   2   8   0   0   0

2010                        2011                        2012                        2013                        2014
Year
Figure 13. Other types of accidents in 2012–2014 
Annex B. Changes in Legislation
	Table 1. Status of transposition of amendments to the Railway Safety Directive into the national legal system

	Amendments to Railway Safety Directive
	Transposed (Y/N)
	Reference to a legal act
	Effective date 

	Directive 2008/57/EC
	Yes
	Law No XI-642 amending the Republic of Lithuania Law on Railway Traffic Safety 
	30-01-2010

	
	
	Law amending and supplementing Articles 2, 5, 6, 9, 12 and 22 and Annex of the Republic of Lithuania Law on Railway Traffic Safety 
	03-05-2011

	
	
	Law amending and supplementing Articles 3, 7, 10, 11, 13, 16, 21, 22, 23, 24, 25, 26, 28 and 31, repealing Articles 6, 30 and 301 and including Articles 101 and 231 of the Railway Transport Code of the Republic of Lithuania
	01-07-2013

	
	
	Republic of Lithuania Law on Administrative Proceedings 
	14-01-1999

	
	
	Republic of Lithuania Law on Standardisation 
	28-04-2000

	
	
	Resolution No 1468 of 22 November 2004 of the Government of the Republic of Lithuania ‘On the establishment of a Register of Rolling Stock and Containers of the Republic of Lithuania, approval of its regulations and launching of its operations’
	26-11-2004

	
	
	Resolution No 743 of 16 June 2010 of the Government of the Republic of Lithuania ‘On amendments to Resolution No 1468 of 22 November 2004 of the Government of the Republic of Lithuania ‘On the establishment of a Register of Rolling Stock and Containers of the Republic of Lithuania, approval of its regulations and launching of its operations’ 
	23-06-2010

	
	
	Resolution No 674 of 4 July 2006 of the Government of the Republic of Lithuania ‘On the approval of rules for designation and notification of testing laboratories, certification and inspection bodies’
	15-07-2006

	
	
	Order No 3-382 of 4 October 2006 of the Minister of Transport and Communications of the Republic of Lithuania ‘On amendments to Regulations of the Register of Railway Infrastructure of the Republic of Lithuania’
	13-10-2006

	
	
	Order No 3-437 of 13 July 2010 of the Minister of Transport and Communications of the Republic of Lithuania ‘On the approval of the procedure for drafting and approving of technical rules for railway subsystems’
	18-07-2010

	
	
	Order No 3-467 of 3 September 2013 of the Minister of Transport and Communications of the Republic of Lithuania ‘On the amendment of Order No 3-509 of 27 December 2006 of the Minister of Transport and Communications ‘On the approval of the Regulations
	08-09-2013

	
	
	Order No 3-659 of 31 December 2013 of the Minister of Transport and Communications ‘On amendments to Order No 3-586 of 23 December 2004 of the Minister of Transport and Communications ‘On the approval of rules for the establishment and application of requirements for the interoperability of the trans-European rail system’  
	01-01-2014

	
	
	Order No 3-658 of 31 December 2013 of the Minister of Transport and Communications ‘On amendments to Order No 3-507 of 22 December 2006 of the Minister of Transport and Communications ‘On the approval of rules for the issuing of authorisations for using the structural subsystems and rolling stock of the trans-European rail system in Lithuania’ 
	01-01-2014


	
	
	
	

	
	
	Order No 3-660 of 31 December 2013 of the Minister of Transport and Communications ‘On the approval of rules for supplying the market with the interoperability constituents of the railway system’ ( ‘Order No 3-660’)
	01-01-2014

	Directive 
2008/110/EC
	Yes
	Republic of Lithuania Law No XI-642 amending the Law on Railway Traffic Safety 
	30-01-2010

	
	
	Republic of Lithuania Law No XI-1807 on Management of State Information Resources
	01-01-2012 

	
	
	Regulations for the Register of Rolling Stock and Containers of the Republic of Lithuania, approved by Resolution No 1468 of 22 November 2004 of the Government of the Republic of Lithuania
	26-11-2004

	
	
	Resolution No 743 of 16 June 2010 of the Government of the Republic of Lithuania ‘On amendments to Resolution No 1468 of 22 November 2004 of the Government of the Republic of Lithuania ‘On the establishment of the Register of Rolling Stock and Containers of the Republic of Lithuania, approval of its regulations and launching its operation’
	23-06-2010

	
	
	Law No XI-1334 amending and supplementing Articles 2, 5, 6, 9, 12 and 22 and Annex of the Republic of Lithuania Law on Railway Traffic Safety
	01-05-2011

	
	
	Law No X-653 amending and supplementing Article 4 of the Republic of Lithuania Law on the Approval, Entry Into Force and Application of the Railway Transport Code of the Republic of Lithuania, amending and supplementing Articles 3, 4, 6, 7, 10, 11, 12, 13, 16, 19, 28, 30 and 48 and the Annex of and Adding Article 30(1)  to the Railway Transport Code 
	28-06-2006

	
	
	Law No XI-1595 amending and supplementing Articles 1, 3, 4, 7, 11, 12, 13, 16, 23, 24, 25, 29 and 33 and the Annex, Adding  Articles 41, 251 and 331, and Repealing Articles 8 and 27 of the Railway Transport Code of the Republic of Lithuania  
	08-10-2011

	
	
	Law No XII-235 amending and supplementing Articles 3, 7, 10, 11, 13, 22, 23, 24, 25, 26, 28 and 31, repealing Articles 6, 30 and 301 and Adding Articles 101 and 231 of the Code of Railway Transport of the Republic of Lithuania  
	01-07-2013

	
	
	Order No 3-444 of 22 July 2011 of the Minister of Transport and Communications of the Republic of Lithuania ‘On amendments to Order No 3-37 of 23 January 2003 of the Minister of Transport and Communications ‘On the approval of rules for safety certification of railway undertakings (carriers) and railway infrastructure managers’
	29-07-2011

	
	
	Order No 3-421 of 11 June 2012 of the Minister of Transport and Communications of the Republic of Lithuania ‘On amendments to Order 3-37 of 23 January 2003 of the Minister of Transport and Communications ‘On the approval of rules for safety certification of railway undertakings (carriers) and railway infrastructure managers’
	20-06-2012

	
	
	Order No 3-514 of 1 October 2013 of the Minister of Transport and Communications of the Republic of Lithuania ‘On amendments to Order No 3-37 of 23 January 2003 of the Minister of Transport and Communications  ‘On the approval of rules for the safety certification of railway undertakings (carriers) and railway infrastructure managers’
	11-10-2013

	
	
	Order No 3-431 of 19 July 2011 of the Minister of Transport and Communications of the Republic of Lithuania ‘On the approval of safety certification rules of maintenance entities’ 
	21-12-2013

	
	
	Order No 3-634 of 13 December 2013 of the Minister of Transport and Communications of the Republic of Lithuania ‘On the amendment of Order No 3-431 of 19 July 2011 of the Minister of Transport and Communications ‘On the approval of safety certification rules of maintenance entities’
	21-12-2013

	
	
	Order No 3-359 of 13 June 2011 of the Minister of Transport and Communications of the Republic of Lithuania ‘On the amendment of Order No 3-238 of 12 June 2006 of the Minister of Transport and Communications ‘On the approval of the Procedure for Setting the Common Traffic Safety Indicators’
	19-06-2011

	
	
	Order No V-441 of 20 July 2011 of the Head of the State Railway Inspectorate under the Ministry of Transport and 
	24-07-2011


	
	
	Communications ‘On the approval of the procedure for the evaluation of applications of maintenance entities seeking to obtain a certificate of a maintenance entity responsible for freight wagons used in the 1520 mm gauge railway network’   “
	

	
	
	Order No 3-472 of 9 August 2011 of the Minister of Transport and Communications of the Republic of Lithuania ‘On amendments to of Order No 3-398 of 29 July 2004 of the Minister of Transport and Communications ‘On the approval of rules for certification of undertakings engaged in the maintenance and repairs of rolling stock’  
	13-08-2011

	
	
	Order No 3-508 of 30 September 2013 of the Minister of Transport and Communications of the Republic of Lithuania ‘On amendments to Order No 3-398 of 29 July 2004 of the Minister of Transport and Communications ‘On the approval of the rules for the certification of persons engaged in the maintenance of rolling stock’   
	11-10-2013

	Commiss

ion Directive 
2009/149/EC
	Yes 
	  Republic of Lithuania Law No XI-642 amending the Law on Railway Traffic Safety 
	03-05-2011

	
	
	Order No 3-424 of 7 July 2010 of the Minister of Transport and Communications of the Republic of Lithuania ‘On  amendments to of Order No 3-238 of 12 June 2006 of the Minister of Transport and Communications ‘On the setting of common traffic safety indicators’ “
	15-07-2010

	
	
	Order No 3-483 of 9 August 2010 of the Minister of Transport and Communications of the Republic of Lithuania ‘On amendments to Order No 3-238 of 12 June 2006 of the Minister of Transport and Communications of the Republic of Lithuania ‘On the setting of common traffic safety indicators’ 
	13-08-2010

	
	
	Order No 3-359 of 13 June 2011 of the Minister of Transport and Communications of the Republic of Lithuania ‘On amendments to of Order No 3-238 of 12 June 2006 of the Minister of Transport and Communications of the Republic of Lithuania ‘On the setting of common traffic safety indicators’
	19-06-2011

	
	
	Order No 3-685 of 23 October 2012 of the Minister of Transport and Communications of the Republic of Lithuania ‘On  amendments to Order No 3-238 of 12 June 2006 of the Minister of Transport and Communications of the Republic of Lithuania ‘On the setting of common traffic safety indicators’   
	28-10-2012


Note: Information about the implementation of Commission Directive 2014/88/EU and Directive 2012/34/EU will be presented in Lithuania’s Railway Traffic Safety Report for 2015.
	Table 2.  Information on changes in legislation governing railway traffic safety in 2014 

	
	Reference to a legal acts 
	Effective date 
	Changes 
	Reason for the change 

	Implementation of other EU requirements 
	Resolution of the Government of the Republic of Lithuania No 655 of 9 July 2014 ‘On amendments to Resolution No 1468 of 22 November 2004 of the Government of the Republic of Lithuania ‘On the establishment of a Register of Rolling Stock and Containers of the Republic of Lithuania, approval of its regulations and launching of its operations’
	12-07-2014 (new version)
	The NSA stated using the Railway Transport State Supervision IS for the operation of this register, therefore, provisions governing the operation of the register had to be modified. Objects of the register were updated and the name of the register was changed. 
	Implementing Directive of 17 June 2008 of the European Parliament and of the Commission No 2008/57/EC, Commission Decision of 9 November 2007 No 2007/756/EC, and Commission Decision of 10 February 2011 No
2011/107/ES.
Reducing administrative burden for business entities 


	Regarding designated bodies, notified bodies, assessment bodies and third parties 

Implementation of other EU requirements 
	Order of the Minister of Transport and Communications of the Republic of Lithuania No 3-91-(E) of 27 February 2014 ‘On amendments to Order No 3-507 of 22 December 2006 of the Minister of Transport and Communications ‘On the approval of rules for the issuing of authorisations for using the structural subsystems and rolling stock of the trans-European rail system in the Republic of Lithuania’
	28-02-2014
  (amendment)
	Titles of authorisations were updated; it has been stated that NSA is responsible for the checks in Lithuania (as there are no designated bodies); procedures for the checks in Lithuania have been updated. 
	Implementing Directive of 17 June 2008 of the European 
Parliament and of the Council No 2008/57/EC on the interoperability of the rail system within the Community 

	Regarding railway undertakings, railway infrastructure managers, and certified entities in charge of maintenance
	Order of the Minister of Transport and Communications of the Republic of Lithuania No 3-465-(E) of 1 December 2014 ‘On amendments to Order No 3-37 of 23 January 2003 of the Minister of Transport and Communications  ‘On the approval of rules for the safety certification of railway undertakings (carriers) and railway infrastructure managers’ 
	02-12-2014
 (amendment)
	 Updated procedures for the issuing 

of safety certificates and authorisations having regard to commends made by the Ministry of the Economy of the Republic of Lithuania.
	Reducing administrative burden for business entities 
Updating legal regulation 

	Implementation of other EU requirements
	Order of the Minister of Transport and Communications of the Republic of Lithuania No 3-90-(E) of 27 February 2014 ‘On amendments to Order No 3-586 of 23 December 2004 of the Minister of Transport and Communications ‘On the approval of rules for the establishment and application of requirements for the interoperability of the trans-European rail system’  
	28-02-2014
(amendment)
	The term ‘railway system’ was updated in the title and in the text. 
	Updating legal regulation

	Implementation of other EU requirements
	Order of the Minister of Transport and Communications of the Republic of Lithuania No 3-320-(E) of 31 July 2014 ‘On amendments to Order No 3-586 of 23 December 2004 of the Minister of Transport and Communications ‘On the approval of rules for the establishment and application of requirements for the interoperability of the trans-European rail system’  
	01-08-2014

 (amendment)
	It has been established that the railway system must be designed and used in such a way that the permissible noise levels are not exceeded at the railway infrastructure and in the train driver’s cab.
	Implementing Commission Directive of 10 March 2014 No 2014/38/EU, amending Annex III to Directive of the European Parliament and of the Council 2008/57/EC as far as noise  pollution is concerned

	Related to NSA
	Order of the Minister of Transport and Communications of the Republic of Lithuania No 3-522- (E) of 17 December 2014 ‘On amendments to Order No 3-79 of 20 February 2003 of the Minister of Transport and Communications ‘On the approval of the Regulations for Investigation of and Response to Railway Traffic Accidents, Serious Accidents and Incidents’
	01-01-2015
 (amendment)
	Updated provisions on who must report a railway traffic accident, serious accident or incident to the relevant authorities by submitting an initial report. Updated forms of reports on railway traffic accidents, serious accidents or incidents.
	Updating legal regulation


	
	
	
	
	

	Related to NSA
	Order of the Minister of Transport and Communications of the Republic of Lithuania No 3-226-(E) of 10 June 2014 ‘On amendments to Order No 3-274 of 4 June 2002 of the Minister of Transport and Communications ‘On the approval of the Procedure for Training and Examination of Dangerous Goods Carriage Specialists and Issue, Revocation and Renewal of Certificates and Their Duplicates’
	11-06-2014
 (amendment)
	Updated procedure for the issuing of certificates and their duplicates; it has been established that applicants can apply electronically.
	Reducing administrative burden for business entities 

Updating legal regulation

	Related to NSA
	Order of the Minister of Transport and Communications of the Republic of Lithuania No 3-225-(E) of 10 June 2014 ‘On amendments to Order No 3-32 of 21 January 2003 of the Minister of Transport and Communications ‘On the approval of the Procedure for the Certification of Instructors Training Persons Involved in the Dangerous Goods Carriage and Persons Seeking to Become Instructors and the Granting and Revocation of the Right to Train Such Persons’
	11-06-2014

 (amendment)
	Updated procedure for the issuing of certificates and their duplicates; it has been established that applicants can apply electronically.
	Reducing administrative burden for business entities 

Updating legal regulation

	Related to NSA
	Order of the Minister of Transport and Communications of the Republic of Lithuania No 3-224-(E) of 10 June 2014 ‘On amendments to Order No 3-343 of 4 July 2002 of the Minister of Transport and Communications ‘On the approval of the Procedure for the Appointment of Dangerous Goods Carriage Safety Specialists in All Entities Engaged in the Carriage of Dangerous Goods’
	11-06-2014
 (new version)
	Updated procedure for the appointment of the Dangerous Goods Carriage Safety Specialist; it has been established that notifications can be submitted electronically.
	Reducing administrative burden for business entities 

Updating legal regulation

	Related to NSA
	Order of the Minister of Transport and Communications of the Republic of Lithuania No 3-268-(E) of 1 July 2014 ‘On amendments to Order No 3-423 of 7 July 2010 of the Minister of Transport and Communications ‘On the establishment of the Register of Train Drivers, approval of its regulations and launching its operation’
	02-07-2014
(amendment)
	Updated structure and content of the regulations having regard to Lithuanian legal acts governing the establishment, development, reorganisation and liquidation of public registers. 
	Updating legal regulation

	Implementation of other EU requirements
	Order of the Minister of Transport and Communications of the Republic of Lithuania No 3-104(E) of 6 March 2014 ‘On designation of a national contact point responsible for monitoring progress of implementation of Regulation (EU) No 1273/2013’
	07-03-2014 (new)
	NSA designated as the national contact point. 
	Implementing Commission Regulation of 6 December 2013 No 1273/2013


	
	
	
	
	

	Implementation of other EU requirements
	Order of the Minister of Transport and Communications of the Republic of Lithuania No 3-125-(E) of 19 March 2014 ‘On amendments to Order No 3-382 of 4 October 2006 of the Minister of Transport and Communications of the Republic of Lithuania ‘On approval of the Regulations of the Register of Railway Infrastructure of the Republic of Lithuania’
	20-03-2014
(amended preamble, new wording of the legal act)
	
	Implementing Directive of 17 June 2008 of the European 
Parliament and of the Council No
2008/57/EC on the interoperability of the rail system within the Community 

	Implementation of other EU requirements
	Order of the Minister of Transport and Communications of the Republic of Lithuania No 3-317-(E) of 30 July 2014 ‘On amendments to Order No 3-71 of 2 March 2005 of the Minister of Transport and Communications of the Republic of Lithuania ‘On the approval of the Railway Rolling Stock Registration Rules’ 
	31-07-2014 (new
version)
	Updated objects of the register, changed name of the register, procedure for electronic submission of documents established, detailed registration procedure, defined procedure for spot checks of railway rolling stock, detailed procedure for assignment of IDs to rolling stock 
	Implementing Commission Decision of 9 November 2007 No 2007/756/EC
Reducing administrative burden for business entities 



	Implementation of other EU requirements
	Order of the Minister of Transport and Communications of the Republic of Lithuania No 3-64(E) of 12 February 2014 ‘On amendments to Order No 66 of 23 February 1998 of the Minister of Transport and Communications of the Republic of Lithuania ‘On the approval of the Rules for the Construction, Reconstruction, Connection and Commissioning of Sidings’
	13-02-2014 (new
version)
	It has been established that a risk analysis and assessment must be made according to Regulation (EC) No 352/2009 for the purposes of connection of sidings to the public railway infrastructure. 


	Implementing Regulation (EC) No 352/2009


	Related to NSA
	Order of the Minister of Transport and Communications of the Republic of Lithuania No V-207 of 16 December  2014 ‘On amendments to Order of the State Railway Inspectorate under the Ministry of Transport and Communications No V-196 of 29 June 2010 ‘On the approval of the Procedure for the Recognition of Examiners of Train Drivers and the Requirements Set for the Examiners, Examinations and Examination Procedures’
	18-12-2014

(amendment)
	It has been established that, in case of a minor violation, certificates will not be suspended or revoked but an oral or written reprimand issued; it has been established that applicants can apply electronically.
	Reducing administrative burden for business entities 



	Related to NSA
	Order of the State Railway Inspectorate under the Ministry of Transport and Communications No 
V-208 of 16 December 2014 ‘On amendments to Order of the State Railway Inspectorate under the Ministry of Transport and Communications No V-153 of 17 May 2010 ‘On the approval of the Procedure for the Recognition 
	18-12-2014

 (amendment)
	It has been established that, in case of a minor violation, certificates will not be suspended or revoked but an oral or written reprimand issued; it has been established that applicants can apply electronically. Detailed procedure for the 
	Reducing administrative burden for business entities 

Updating legal regulation


	
	Recognition of Train Drivers Training Centres’
	
	consideration of complaints. It has been established that applicants can apply electronically.
	

	Related to NSA
	Order of the State Railway Inspectorate under the Ministry of Transport and Communications No 
V-210 of 18 December 2014 ‘On amendments to Order of the State Railway Inspectorate under the Ministry of Transport and Communications No V-96 of 14 February 2013 ‘On the approval of the Procedure for the Certification of Instructors Training Persons Involved in the Carriage of Dangerous Goods and Persons Applying for the Position of an Instructor’
	19-12-2014
(amendment)
	Updated examination procedures; examination by means of information systems is allowed; established application form for taking examinations. 
	Reducing administrative burden for business entities 

Updating legal regulation

	Related to NSA
	Order of the State Railway Inspectorate under the Ministry of Transport and Communications No 
V-206 of 16 December 2014 ‘On amendments to Order No V-449 of 29 July 2013 of the State Railway Inspectorate under the Ministry of Transport and Communications ‘On the approval of the Procedure for the Issuing of Authorisations to Examination Centres’
	18-12-2014

 (amendment)
	It has been established that, in case of a minor violation, certificates will not be suspended or revoked but an oral or written reprimand issued; it has been established that applicants can apply electronically.
	Reducing administrative burden for business entities 



	Related to NSA
	Order of the State Railway Inspectorate under the Ministry of Transport and Communications No 
V-42 of 5 March 2014 ‘On amendments to Order of the State Railway Inspectorate under the Ministry of Transport and Communications No V-29 of 14 October 2004 ‘On the approval of the Procedure for the Checking of Knowledge of Employees Whose Jobs Are Related to Railway Traffic’
	05-03-2014 (new
version)
	It has been established that the examinations will be held by the examination centres 
(previously – NSA); examination by means of information systems is allowed; updated categories of persons whose work is related to railway traffic; detailed examination procedures.  Mandatory periodic training for certificate holders introduced (every three years).
	Reducing administrative burden for business entities 



	Related to NSA
	Order of the State Railway Inspectorate under the Ministry of Transport and Communications No 
V-41 of 5 March 2014 ‘On amendments to Order of the State Railway Inspectorate under the Ministry of Transport and Communications No V-30 of 14 October 2004 ’On the approval of the Procedure for the Checking of Knowledge of Persons Responsible for Cargo Loading and Securing on Wagons (Works Managers)’
	07-03-2014 (new
version)
	It has been established that the examinations will be held by the examination centres 
(previously – NSA); examination by means of information systems is allowed; detailed examination procedures.  Mandatory periodic training for certificate holders introduced (every two years).
	Reducing administrative burden for business entities 

Updating legal regulation


	Related to NSA
	Order of the State Railway Inspectorate under the Ministry of Transport and Communications No 
V-205 of 16 December 2014 ‘On amendments to Order of the State Railway Inspectorate under the Ministry of Transport and Communications No V-30 of 14 October 2004 ’On the approval of the Procedure for the Checking of Knowledge of Persons Responsible for Cargo Loading and Securing on Wagons (Works Managers)’

	18-12-2014
	Updated list of legal acts included in the examination. 
	Updating legal regulation

	Related to NSA
	Order of the State Railway Inspectorate under the Ministry of Transport and Communications No 
V-56 of 28 March 2014 ‘On approval of the Procedure for the Submission of Applications of Training Institutions for the Right to Train Persons Involved in the Carriage of Dangerous Goods and for the Evaluation of Compliance’
	01-04-2014 (new)
	Detailed procedure for the submission of applications by entities wishing to acquire the right to train persons involved in the carriage of dangerous goods and for the assessment of compliance. 
	Reducing administrative burden for business entities 

Updating legal regulation

	Related to NSA
	Order of the State Railway Inspectorate under the Ministry of Transport and Communications No V-54 of 26 March 2014 ‘On the approval of the Application Form’
	28-03-2014 (new)
	Established forms of applications for the start of the use of rolling stock and/or structural subsystems in the Republic of Lithuania.
	Reducing administrative burden for business entities 

Updating legal regulation

	Regarding railway undertakings, railway infrastructure managers, and certified entities in charge of maintenance
	Order of the State Railway Inspectorate under the Ministry of Transport and Communications No 
V-140 of 23 September 2014 ‘On the approval of a Model Description of a Railway Traffic Safety Management System of a Railway Infrastructure Manager’ 
	24-09-2014 (new)
	Approved model description of the railway infrastructure managers’ SMS; smaller managers assisted in the implementation of the SMS and simplified procedures applied to them.
	Reducing administrative burden for business entities 

Updating legal regulation

	Regarding railway    undertakings, railway infrastructure managers, and certified entities in charge of maintenance 
Implementation of other EU requirements
	Order of the State Railway Inspectorate under the Ministry of Transport and Communications No 
V- 176 of 11 November 2014 ‘On amendments to Order of the State Railway Inspectorate under the Ministry of Transport and Communications No V-604 of 27 December 2010 ‘On the approval of the Rules for Carrying out Scheduled and Non-Scheduled Inspections by the State Railway Inspectorate under the Ministry of Transport and Communications 
	12-11-2014 (new
version)
	Updated rules taking account of issues arising in practice; detailed procedures for the compilation of the inspection plan; formulated inspection checklists by inspection types.
	Implementing Commission Regulation (EU) 1077/2012 on a common safety method for supervision by national safety authorities after issuing a safety certificate or safety authorisation
Reducing administrative burden for business entities 

Updating legal regulation 
Correcting errors that are most frequent in practice 


	Related to NSA
	Order of the State Railway Inspectorate under the Ministry of Transport and Communications No 
V- 122 of 13 August 2014  ‘On amendments to Order of the State Railway Inspectorate under the Ministry of Transport and Communications No V-727 of 9 October 2012 ‘On the approval of the form of a report on an administrative offence’
	01-01-2014 (amendment, addition)
	It has been established that documents can be submitted and received electronically.
	Reducing administrative burden for business entities 



	Related to NSA
	Order of the State Railway Inspectorate under the Ministry of Transport and Communications No 
V-98 of 23 June 2014  ‘On the approval of recommended tables for the inspection of construction projects’
	24-06-2014 (new)
	It has been established that NSA’s representatives must inspect construction projects by completing the recommended tables for their inspection.
	Reducing administrative burden for business entities 

Updating legal regulation

	Related to NSA
	Order of the State Railway Inspectorate under the Ministry of Transport and Communications No 
V-117 of 11 August 2014 ‘On the approval of a model form of notification of appointment of a Dangerous Goods Carriage Specialist by an entity and the setting of the procedure’
	12-08-2014 (new)
	Established form of notification of the appointment of a Dangerous Goods Carriage Specialist.
	Reducing administrative burden for business entities 

Updating legal regulation

	Related to NSA
	Order of the State Railway Inspectorate under the Ministry of Transport and Communications No 
V-173 of 10 November 2014 ‘On the approval of recommended forms for documents of inspection of structures on sidings’
	11-11-2014 (new)
	Due to large number of railway traffic accidents, forms of the documents of inspection of structures on sidings have been set. The forms may be filled by ECMs of sidings where the rolling stock speed is max 60 km/h, during regular monitoring of the condition of railway structures and during completion of the railway structures’ maintenance logs. 
	Ensuring railway traffic safety
Reducing administrative burden for business entities 

Updating legal regulation

	Regarding railway undertakings, railway infrastructure managers, and certified entities in charge of maintenance
	Order of the State Railway Inspectorate under the Ministry of Transport and Communications No 
V-5 of 17 January 2015 ‘On the approval of the Procedure for the Evaluation of Applications Submitted by Railway Undertakings (Carriers) and Other Entities Applying for Part A and/or Part B of the Safety Certificate and by Railway Infrastructure Managers Applying for Part A and/or Part B of the Safety Authorisations and Accompanying Documents’
	18-01-2014 (new)
	Detailed procedure for the evaluation of applications submitted by railway undertakings and other entities applying for Part A and/or Part B of the Safety Certificate and by railway infrastructure managers applying for Part A and/or Part B of the Safety Authorisations and accompanying documents. Established composition of the commission, its working principles, forms of evaluation tables. 
	Updating legal regulation


Annex C.   Significant traffic accidents in Lithuania in 2014 
	Item No
	Date/ time
	Circumstances 
	Conclusions 

	1.
	2014-02-01/
09:05
	A person was seriously injured by a train at 12 km, 4 Pkt between Kyviškės - Naujoji Vilnia 
	The injured person failed to comply with the requirements of safe behaviour on the railways and violated Article 10(3) of the Law on Railway Traffic Safety.

	2.
	2014-02-21/
21:20
	A person was fatally injured by a train at Mažeikiai railway station, 63  km  6 Pkt
	The traffic accident occurred due to carelessness of the person who was fatally injured: he was walking on the closed pedestrian passage in front of approaching train and did not react to the train driver’s audible signal.

	3.
	2014-04-25/
11:15
	A person was slightly injured by a train at 218 km 8 Pkt between Kužiai – Šiauliai 
	The injured person was carelessly walking across the pedestrian passage, ignoring the warning signs and the train driver’s signals.

	4.
	2014-04-25/
17:19
	At a level crossing at 43 km 8 Pkt, between Viekšniai – Akmenė, a train collided with a car, the driver of the car received a fatal injury
	Violations of Clauses 173 through 175 of the Road Traffic Regulations.

	5.
	2014.05.18/
3:55
	A person was fatally injured by a train at 73 km, 8 Pkt between Šilutė  – Stoniškiai 
	The injured person failed to comply with the requirements of safe behaviour on the railways and violated Article 10(3) of the Law on Railway Traffic Safety.

	6.
	2014-07-26/
7:20
	A rail car collided with a car on a level crossing at 145 km Pkt 8 between  Kirtimai  –  Valčiūnai  
	The car driver ignored the light signal and drove on the crossing without satisfying himself that no rail vehicle is approaching the crossing. Violations of Clauses 173 and 174 of the Road Traffic Regulations.

	7.
	2014-07-11/
11:01
	A person was seriously injured by a train at 33 km 4 Pkt between Šeduva – Gustonys 

	The pedestrian was walking across the crossing under a red light signal. Violation of Clause 50 of the Road Traffic Regulations.

	8.
	2014-08-17/
14:37
	A train collided with a bicycle, the cyclist received a non-fatal injury, at a level crossing 129+683 between Kėdainiai – Dotnuva
	The injured person drove on the crossing under a prohibitive light signal and failed to give way to the approaching train. Violation of Clauses 70, 173, 174, 175 of the Road Traffic Regulations..

	9.
	2014-08-30/
12:43
	A person was fatally injured by a freight train at 13 km 4 Pkt between Paneriai – Lentvaris 
	The person was crossing the tracks at a place not designated for this purpose. The injured person failed to comply with the requirements of safe behaviour on the railways and violated Article 10(3) of the Law on Railway Traffic Safety.

	10.
	2014-07-21/
08:40
	An auxiliary worker received a fatal injury during connection of wagon 11 to a train formed of 4 wagons on a siding controlled by Malkų įlankos terminalas UAB
	The auxiliary worker started the wagon cleaning works without obtaining a permission of the shift manager.

	11.
	2014-10-21/
7:37
	A person received a slight injury during a railway traffic accident at Švenčionėliai railway station
	An unauthorised person was walking between the tracks at a place not designated for this purpose. The injured person failed to comply with the requirements of safe behaviour on the railways and violated Article 10(3) of the Law on Railway Traffic Safety.

	12.
	2014-11-11/
11:36
	The train driver had to slam on the brakes at 21 km 2 Pkt between Vievis   -   Lentvaris   
	An incident occurred due to carelessness of an unauthorised person but, due to slamming on the brakes, running over a person was avoided. Violation of Article 10(3) of the Law on Railway Traffic Safety.

	13.
	2014-11-18/
03:50
	A person was fatally injured during a railway accident at 263 km 2 Pkt between Dūseikiai  –  Tryškiai  
	The person was lying on the tracks in the dark, did not react to the audible signal of the approaching train. Violation of Article 10(3) of the Law on Railway Traffic Safety.

	14.
	2014-11-20/
16:12
	A person was fatally injured on a pedestrian crossing by a train at 12 km 2 Pkt between Kyviškės  –  Naujoji  Vilnia  
	The pedestrian saw the approaching train upon walking onto a track at a place designated for this purpose and tried to cross as soon as possible but tripped on a rail and was hit by the train.

	15.
	2014-12-10/
17:10
	A staff member received an injury during manoeuvring works on Anglinė tracks in Klaipėda railway station
	The train adjuster acted carelessly during manoeuvring and failed to maintain a safe distance to the manoeuvring train. Violation of Article 112 (1)2 of the Administrative Code.

	16.
	2014-12-31/
13:33
	A person was fatally injured by a train on track 7 of Kaišiadorys railway station, 68  km  1 Pkt.
	The person was within the railway protection zone, where presence of unauthorised persons is prohibited. Violation of Article 10(3) of the Law on Railway Traffic Safety.





National reference values �



2012/226/EU permissible  NRV X 109�
Projected annual  NRV X 109�



Change�
�
Passengers�
97.20�
0.00�
-100%�
�
Employees�
41.00�
13.99�
-66%�
�
Level crossing users�



522.00�



251.82�



-52%�
�
Unauthorised�
2050.00�
748.47�
-63%�
�
      Other �
11.60�
0.00�
-100%�
�
   Public�
2590.00�
1014.28�
-61%�
�






Numbers of significant traffic accidents 





�
�
�
�
�
�
�
�






Numbers of train collisions





�
�
�
�
�
�
�
�






Numbers of derailments 





Significant traffic accidents at level crossings





�
�
�
�
�
�
�
�






Numbers of significant accidents due to unauthorised persons 





�
�
�
�
�
�
�
�






Numbers of major fires in rolling stock





Numbers of deaths  





�
�
�
�
�
�
�
�






Numbers of deaths on level crossings 





Numbers of deaths  – unauthorised persons 





�
�
�
�
�
�
�
�






Numbers of injuries 





Numbers of injuries on level crossings





�
�
�
�
�
�
�
�






Numbers of injuries – unauthorised persons 





�
�
�
�
�
�
�
�






Numbers of accidents





Įvykių skaičius








