| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 5190200 (5.19.2: Limited Supervision is requested for current location (from modes different from Stand By and Post Trip)) Total: 6 Test case(s) |  |
| REF: | Subset-076-5-2-5190200 |
| ISSUE: | Version 3.2.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 0.0.1 23.03.2012 | All | First version/review | DLR |
| Version 2.9.0 31.03.2013 | ALL | First version/review for Baseline 3 | MULTITEL |
| Version 2.9.1 05.07.2013 | All | Template standardised with a dedicated tool | MULTITEL |
| Version 2.9.2 03.10.2013 | TC1, TC4 | Modifications requested by issues 10-11 of Review001_Subset076-5-2-5190200_v291 | MULTITEL |
| Version 2.9.3 30.10.2013 | All | Response to Review002_Subset-076-5-2-5190200_v291 | MULTITEL |
| Version 2.9.4 09.01.2014 | TC1,3-6 | Modifications requested by issue 1 of Review001_Subset076-5-2-5190200_v293 | MULTITEL |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.2.0 31.03.2017 | All | Baseline 3 1st maintenance release version - Bug fixing | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.19.2: Limited Supervision is requested for current location (from modes different from Stand By and Post Trip) |  | 5190200 |
| Test Case of Feature | Limited Supervision is requested for the current location and the driver acknowledges within the driver acknowledgement time (refer to Appendix A3.1) after the change to LS mode. |  | 1 |
| Applicable Mode/Level Combinations | L1: FS, OS, SR L2: FS, OS, SR L3: FS, OS, SR |  |  |
| Target of Test | The target of this test is to check that: After receiving the order to change to LS the ERTMS/ETCS on-board equipment shall switch immediately to LS mode and a request for acknowledgement shall be displayed to the driver The Driver acknowledges within the driver acknowledgement time (refer to Appendix A3.1) after the change to LS mode |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.4.19.1.2 |  |
|  |  | Subset-026-4.7.2 table - Input information Ackn of Limited Supervision mode |  |
|  |  | Subset-026-5.19.1.2 |  |
|  |  | Subset-026-5.19.2.1 |  |
|  |  | Subset-026-5.19.2.2 |  |
|  |  | Subset-026-5.19.2.3 |  |
|  |  | Subset-026-5.19.2.4 |  |
|  |  | Subset-026-7.4.2.26 Packet Number 80: Mode Profile |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The mode initially DISPLAYED changes to LS A Request for Acknowledgement is DISPLAYED Check on the JRU that : Transition to LS mode is RECORDED Driver Acknowledgement is RECORDED. |
| Constraints | The train is running under the permitted speed in LS Mode before reaching the LS area. If the Service Brake were not available for any reason, Emergency Brake shall be used. For LS mode, the mode profile shall define the entry and the length of the Limited Supervision area. To be in Limited Supervision mode, SSP and gradient shall be at least available from the FRONT END of the train. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 | FS / OS / SR |
| Radio communication session | ESTABLISHED | Only in L2 / L3. |
| Gradient Profile | STORED / NOT STORED | Shall be at least available from the FRONT END of the train If not stored, it shall be sent in this TC together with packet 12/15 and packet 80 |
| International SSP | STORED / NOT STORED | Shall be at least available from the FRONT END of the train If not stored, it shall be sent in this TC together with packet 12/15 and packet 80 |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Only in L2 / L3. |
| Service brake is NOT APPLIED | O | TIU | - |
| Current mode and level are DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 / L2 / L3 | FS / OS / SR | One Balise Group message containing packet 12, packet 80 (M_MAMODE=2) is received | I | BTM | One of the steps 1 or 2 will be chosen for testing Reception of the mode transition announcement (mode transition order at current location) If not yet stored, packet 21 gradient profile shall be sent in this message and gradient profile shall be at least available from the front end of the train If not yet stored, packet 27 ISSP shall be sent in this message and ISSP shall be at least available from the front end of the train | L1 / L2 / L3 | FS / OS / SR | - |
| 2 | L1 / L2 / L3 | FS / OS / SR | SA-DATA.Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 15, packet 80 (M_MAMODE=2) is received | I | RTM | One of the steps 1 or 2 will be chosen for testing Reception of the mode transition announcement (mode transition order at current location) If not yet stored, packet 21 gradient profile shall be sent in this message and gradient profile shall be at least available from the front end of the train If not yet stored, packet 27 ISSP shall be sent in this message and ISSP shall be at least available from the front end of the train | L1 / L2 / L3 | FS / OS / SR | - |
| 3 | L1 / L2 / L3 | FS / OS / SR | Complete message including Packet 80: Mode profile is RECORDED | O | JRU | - | L1 / L2 / L3 | FS / OS / SR | - |
| 4 | L1 / L2 / L3 | FS / OS / SR | The ERTMS/ETCS on-board equipment switches immediately to LS mode and a request for acknowledgement is displayed to the driver | O | DMI | - | L1 / L2 / L3 | LS | - |
| 5 | L1 / L2 / L3 | LS | Mode transition to LS is RECORDED | O | JRU | - | L1 / L2 / L3 | LS | - |
| 6 | L1 / L2 / L3 | LS | The on-board informs trackside about mode change Message 136: “Train Position Report” is SENT. | O | RTM | Only in L2 / L3. | L1 / L2 / L3 | LS | - |
| 7 | L1 / L2 / L3 | LS | Message 136: “Train position report” is RECORDED. | O | JRU | Only in L2 / L3. | L1 / L2 / L3 | LS | - |
| 8 | L1 / L2 / L3 | LS | Driver acknowledges within 5 sec after the change to LS | I | DMI | The driver acknowledgement time (refer to Appendix A3.1) | L1 / L2 / L3 | LS | - |
| 9 | L1 / L2 / L3 | LS | Driver Acknowledgement of transition to LS mode is RECORDED | O | JRU | - | L1 / L2 / L3 | LS | - |
| 10 | L1 / L2 / L3 | LS | Service brake is NOT APPLIED | O | TIU | Negative Testing | L1 / L2 / L3 | LS | - |
| 11 | L1 / L2 / L3 | LS | NO service Brake Indication is DISPLAYED | O | DMI | Negative Testing | L1 / L2 / L3 | LS | - |
| 12 | L1 / L2 / L3 | LS | NO Service Brake Intervention is RECORDED | O | JRU | Negative Testing | L1 / L2 / L3 | LS | - |
| 13 | L1 / L2 / L3 | LS | LS Mode indication is displayed | O | DMI | - | L1 / L2 / L3 | LS | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 0 | 1 of N |
| N_TOTAL | 3 | 1 | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 80 | Mode profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_MAMODE | 15 | 0 | Mode change starting in the current location |
| M_MAMODE | 2 | 10 | LS |
| V_MAMODE | 7 | FINITE VALUE | - |
| L_MAMODE | 15 | FINITE VALUE | - |
| L_ACKMAMODE | 15 | FINITE VALUE | - |
| Q_MAMODE | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_MAMODE(k) | 15 | FINITE VALUE | - |
| M_MAMODE(k) | 2 | FINITE VALUE | OS, LS, SH. |
| V_MAMODE(k) | 7 | FINITE VALUE | - |
| L_MAMODE(k) | 15 | FINITE VALUE | - |
| L_ACKMAMODE(k) | 15 | FINITE VALUE | - |
| Q_MAMODE(k) | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 12 | A Level 1 movement authority shall be sent together. |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out". |
| N_ITER | 5 | FINITE VALUE | Set to zero if V_MAIN = 0 or if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 1 | 2 of N |
| N_TOTAL | 3 | 1 | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 2: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | Message 3: Movement authority |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 10 + 14 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | Packet no 15: Level 2/3 MA |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 80 | Packet number 80: Mode profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_MAMODE | 15 | 0 | Mode transition for the current location |
| M_MAMODE | 2 | 10 | LS |
| V_MAMODE | 7 | FINITE VALUE | - |
| L_MAMODE | 15 | FINITE VALUE | - |
| L_ACKMAMODE | 15 | FINITE VALUE | - |
| Q_MAMODE | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_MAMODE(k) | 15 | FINITE VALUE | - |
| M_MAMODE(k) | 2 | FINITE VALUE | OS, LS, SH. |
| V_MAMODE(k) | 7 | FINITE VALUE | - |
| L_MAMODE(k) | 15 | FINITE VALUE | - |
| L_ACKMAMODE(k) | 15 | FINITE VALUE | - |
| Q_MAMODE(k) | 1 | FINITE VALUE | - |

| Step 6: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train position report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 10+14 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = 1 or 2 |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | 12 | LS |
| M_LEVEL | 3 | 3 / 4 | L2 / L3 |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 12 | LS |
| Radio communication session | ESTABLISHED | Only in level 2/3 |
| Gradient Profile | STORED UNCHANGED | Shall be at least available from the FRONT END of the train |
| International SSP | STORED UNCHANGED | Shall be at least available from the FRONT END of the train |
| ERTMS/ETCS level | 2 / 3 / 4 UNCHANGED | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Only in L2 / L3. |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.19.2: Limited Supervision is requested for current location (from modes different from Stand By and Post Trip) |  | 5190200 |
| Test Case of Feature | The on-board equipment is already in LS mode, when receiving the LS mode profile. |  | 2 |
| Applicable Mode/Level Combinations | L1: LS L2: LS L3: LS |  |  |
| Target of Test | The target of this test is to check that: After receiving the order to change to LS, no further acknowledgement shall be requested from the driver. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-5.19.2.7 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The mode initially DISPLAYED is still LS A Request for Acknowledgement is NOT DISPLAYED |
| Constraints | The train is already in an LS area. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 12 | LS |
| Radio communication session | ESTABLISHED | Only in L2 / L3. |
| Gradient Profile | STORED | Shall be at least available from the FRONT END of the train |
| International SSP | STORED | Shall be at least available from the FRONT END of the train |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Only in L2 / L3. |
| NOT RELEVANT | - | TIU | - |
| Current mode and level are DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message containing packet 12, packet 80 (M_MAMODE=2) is received | I | BTM | One of the steps 1 or 2 will be chosen for testing Reception of the mode transition announcement (mode transition order at current location) | N/A | N/A | - |
| 2 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 15, packet 80 (M_MAMODE=2) is received | I | RTM | One of the steps 1 or 2 will be chosen for testing Reception of the mode transition announcement (mode transition order at current location) | N/A | N/A | - |
| 3 | N/A | N/A | Complete message including Packet 80: Mode profile is RECORDED | O | JRU | - | N/A | N/A | - |
| 4 | N/A | N/A | NO Driver acknowledgement is requested | O | DMI | - | N/A | N/A | - |
| 5 | N/A | N/A | LS Mode indication is still displayed | O | DMI | - | N/A | N/A | - |
| 6 | N/A | N/A | The length of LS area is updated | O | DMI | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 0 | 1 of N |
| N_TOTAL | 3 | 1 | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 80 | Mode profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_MAMODE | 15 | 0 | Mode change starting in the current location |
| M_MAMODE | 2 | 10 | LS |
| V_MAMODE | 7 | FINITE VALUE | - |
| L_MAMODE | 15 | FINITE VALUE | - |
| L_ACKMAMODE | 15 | FINITE VALUE | - |
| Q_MAMODE | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_MAMODE(k) | 15 | FINITE VALUE | - |
| M_MAMODE(k) | 2 | FINITE VALUE | OS, LS, SH. |
| V_MAMODE(k) | 7 | FINITE VALUE | - |
| L_MAMODE(k) | 15 | FINITE VALUE | - |
| L_ACKMAMODE(k) | 15 | FINITE VALUE | - |
| Q_MAMODE(k) | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 12 | A Level 1 movement authority shall be sent together. |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out". |
| N_ITER | 5 | FINITE VALUE | Set to zero if V_MAIN = 0 or if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 1 | 2 of N |
| N_TOTAL | 3 | 1 | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 2: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | Message 3: Movement authority |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 10 + 14 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | Packet no 15: Level 2/3 MA |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 80 | Packet number 80: Mode profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_MAMODE | 15 | 0 | Mode transition for the current location |
| M_MAMODE | 2 | 10 | LS |
| V_MAMODE | 7 | FINITE VALUE | - |
| L_MAMODE | 15 | FINITE VALUE | - |
| L_ACKMAMODE | 15 | FINITE VALUE | - |
| Q_MAMODE | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_MAMODE(k) | 15 | FINITE VALUE | - |
| M_MAMODE(k) | 2 | FINITE VALUE | OS, LS, SH. |
| V_MAMODE(k) | 7 | FINITE VALUE | - |
| L_MAMODE(k) | 15 | FINITE VALUE | - |
| L_ACKMAMODE(k) | 15 | FINITE VALUE | - |
| Q_MAMODE(k) | 1 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 12 | LS |
| Radio communication session | ESTABLISHED | Only in level 2/3 |
| Gradient Profile | STORED UNCHANGED | Shall be at least available from the FRONT END of the train |
| International SSP | STORED UNCHANGED | Shall be at least available from the FRONT END of the train |
| ERTMS/ETCS level | 2 / 3 / 4 UNCHANGED | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Only in L2 / L3. |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.19.2: Limited Supervision is requested for current location (from modes different from Stand By and Post Trip) |  | 5190200 |
| Test Case of Feature | Limited Supervision is requested for the current location and the driver acknowledges within the driver acknowledgement time (refer to Appendix A3.1) after the change to LS mode. Overspeed Case. |  | 3 |
| Applicable Mode/Level Combinations | L1: FS, OS, SR L2: FS, OS, SR L3: FS, OS, SR |  |  |
| Target of Test | The target of this test is to check that: After receiving the order to change to LS the ERTMS/ETCS on-board equipment shall switch immediately to LS mode and a request for acknowledgement shall be displayed. Driver acknowledges within the driver acknowledgement time (refer to Appendix A3.1) after the change to LS mode In Limited Supervision mode, the speed supervision is such that the train speed cannot exceed the LS mode permitted speed limit. If the train speed was higher than the max. LS permitted speed because a higher speed was allowed in previous mode then a service/emergency brake command could be immediately triggered |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.4.19.1.2 |  |
|  |  | Subset-026-4.7.2 table - Input information Ackn of Limited Supervision mode |  |
|  |  | Subset-026-5.19.1.2 |  |
|  |  | Subset-026-5.19.2.1 |  |
|  |  | Subset-026-5.19.2.2 |  |
|  |  | Subset-026-5.19.2.3 |  |
|  |  | Subset-026-7.4.2.26 Packet Number 80: Mode Profile |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The LS mode indication is DISPLAYED A Request for Acknowledgement is DISPLAYED The SB indication in DISPLAYED The SB indication is REMOVED after speed becomes lower than or equal the permitted speed for LS Mode. Check on the JRU that : Transition to LS mode is RECORDED Application of Service Brake is RECORDED. Release of Service Brake is RECORDED |
| Constraints | The train is running over the permitted speed in LS Mode before reaching the LS area. If the Service Brake were not available for any reason, Emergency Brake shall be used. For LS mode, the mode profile shall define the entry and the length of the Limited Supervision area. To be in Limited Supervision mode, SSP and gradient shall be at least available from the FRONT END of the train. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 | FS / OS / SR |
| Radio communication session | ESTABLISHED | Only in L2 / L3. |
| Gradient Profile | STORED / NOT STORED | Shall be at least available from the FRONT END of the train If not stored, it shall be sent in this TC together with packet 12/15 and packet 80 |
| International SSP | STORED / NOT STORED | Shall be at least available from the FRONT END of the train If not stored, it shall be sent in this TC together with packet 12/15 and packet 80 |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Only in L2 / L3. |
| Service brake is NOT APPLIED | O | TIU | - |
| Current mode and level are DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 / L2 / L3 | FS / OS / SR | One Balise Group message containing packet 12, packet 80 (M_MAMODE=2) is received | I | BTM | One of the steps 2 or 3 will be chosen for testing Reception of the mode transition announcement (mode transition order at current location) If not yet stored, packet 21 gradient profile shall be sent in this message and gradient profile shall be at least available from the front end of the train If not yet stored, packet 27 ISSP shall be sent in this message and ISSP shall be at least available from the front end of the train | L1 / L2 / L3 | FS / OS / SR | - |
| 2 | L1 / L2 / L3 | FS / OS / SR | SA-DATA.Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 15, packet 80 (M_MAMODE=2) is received | I | RTM | One of the steps 2 or 3 will be chosen for testing Reception of the mode transition announcement (mode transition order at current location) If not yet stored, packet 21 gradient profile shall be sent in this message and gradient profile shall be at least available from the front end of the train If not yet stored, packet 27 ISSP shall be sent in this message and ISSP shall be at least available from the front end of the train | L1 / L2 / L3 | FS / OS / SR | - |
| 3 | L1 / L2 / L3 | FS / OS / SR | Complete message including Packet 80: Mode profile is RECORDED | O | JRU | - | L1 / L2 / L3 | FS / OS / SR | - |
| 4 | L1 / L2 / L3 | FS / OS / SR | The ERTMS/ETCS on-board equipment switches immediately to LS mode and a request for acknowledgement is displayed to the driver | O | DMI | - | L1 / L2 / L3 | LS | - |
| 5 | L1 / L2 / L3 | LS | Mode transition to LS is RECORDED | O | JRU | - | L1 / L2 / L3 | LS | - |
| 6 | L1 / L2 / L3 | LS | The on-board informs trackside about mode change Message 136: “Train Position Report” is SENT. | O | RTM | Only in L2 / L3. | L1 / L2 / L3 | LS | - |
| 7 | L1 / L2 / L3 | LS | Message 136: “Train position report” is RECORDED. | O | JRU | Only in L2 / L3. | L1 / L2 / L3 | LS | - |
| 8 | L1 / L2 / L3 | LS | Service brake is APPLIED | O | TIU | Due to over-speeding | L1 / L2 / L3 | LS | - |
| 9 | L1 / L2 / L3 | LS | Service Brake Indication is DISPLAYED | O | DMI | - | L1 / L2 / L3 | LS | - |
| 10 | L1 / L2 / L3 | LS | Service Brake Intervention is RECORDED | O | JRU | - | L1 / L2 / L3 | LS | - |
| 11 | L1 / L2 / L3 | LS | Driver acknowledges within 5 sec after the change to LS | I | DMI | The driver acknowledgement time (refer to Appendix A3.1) | L1 / L2 / L3 | LS | - |
| 12 | L1 / L2 / L3 | LS | Driver Acknowledgement of transition to LS mode is RECORDED | O | JRU | - | L1 / L2 / L3 | LS | - |
| 13 | L1 / L2 / L3 | LS | Service Brake is RELEASED | O | TIU | The speed becomes lower than or equal to permitted speed in LS mode | L1 / L2 / L3 | LS | - |
| 14 | L1 / L2 / L3 | LS | Service Brake Indication is REMOVED | O | DMI | - | L1 / L2 / L3 | LS | - |
| 15 | L1 / L2 / L3 | LS | Service Brake Release is RECORDED | O | JRU | - | L1 / L2 / L3 | LS | - |
| 16 | L1 / L2 / L3 | LS | LS mode indication is displayed | O | DMI | - | L1 / L2 / L3 | LS | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 0 | 1 of N |
| N_TOTAL | 3 | 1 | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 80 | Mode profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_MAMODE | 15 | 0 | Mode change for the current location |
| M_MAMODE | 2 | 10 | LS |
| V_MAMODE | 7 | FINITE VALUE | - |
| L_MAMODE | 15 | FINITE VALUE | - |
| L_ACKMAMODE | 15 | FINITE VALUE | - |
| Q_MAMODE | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_MAMODE(k) | 15 | FINITE VALUE | - |
| M_MAMODE(k) | 2 | FINITE VALUE | OS, LS, SH. |
| V_MAMODE(k) | 7 | FINITE VALUE | - |
| L_MAMODE(k) | 15 | FINITE VALUE | - |
| L_ACKMAMODE(k) | 15 | FINITE VALUE | - |
| Q_MAMODE(k) | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 12 | A Level 1 movement authority shall be sent together. |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out". |
| N_ITER | 5 | FINITE VALUE | Set to zero if V_MAIN = 0 or if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 1 | 2 of N |
| N_TOTAL | 3 | 1 | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 2: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | Message 3: Movement authority |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 10 + 14 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | Packet no 15: Level 2/3 MA |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 80 | Packet number 80: Mode profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_MAMODE | 15 | 0 | Mode change is requested for the current location |
| M_MAMODE | 2 | 10 | LS |
| V_MAMODE | 7 | FINITE VALUE | - |
| L_MAMODE | 15 | FINITE VALUE | - |
| L_ACKMAMODE | 15 | FINITE VALUE | - |
| Q_MAMODE | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_MAMODE(k) | 15 | FINITE VALUE | - |
| M_MAMODE(k) | 2 | FINITE VALUE | OS, LS, SH. |
| V_MAMODE(k) | 7 | FINITE VALUE | - |
| L_MAMODE(k) | 15 | FINITE VALUE | - |
| L_ACKMAMODE(k) | 15 | FINITE VALUE | - |
| Q_MAMODE(k) | 1 | FINITE VALUE | - |

| Step 6: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train position report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 10+14 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = 1 or 2 |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | 12 | LS |
| M_LEVEL | 3 | 3 / 4 | L2 / L3 |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 12 | LS |
| Radio communication session | ESTABLISHED | Only in level 2/3 |
| Gradient Profile | STORED UNCHANGED | Shall be at least available from the FRONT END of the train |
| International SSP | STORED UNCHANGED | Shall be at least available from the FRONT END of the train |
| ERTMS/ETCS level | 2 / 3 / 4 UNCHANGED | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Only in L2 / L3. |
| SB is RELEASED | O | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.19.2: Limited Supervision is requested for current location (from modes different from Stand By and Post Trip) |  | 5190200 |
| Test Case of Feature | Limited Supervision is requested for the current location and the driver doesn’t acknowledges within the driver acknowledgement time (refer to Appendix A3.1) after the change to LS mode |  | 4 |
| Applicable Mode/Level Combinations | L1: FS, OS, SR L2: FS, OS, SR L3: FS, OS, SR |  |  |
| Target of Test | The target of this test is to check that: After receiving the order to change to LS the ERTMS/ETCS on-board equipment shall switch immediately to LS mode and a request for acknowledgement shall be displayed to the driver And the driver doesn’t acknowledges within the driver acknowledgement time (refer to Appendix A3.1) after the change to LS mode, And the service brake is triggered. The command shall be released as soon as the driver acknowledges (unless the command was triggered also for other reasons) |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.4.19.1.2 |  |
|  |  | Subset-026-4.7.2 table - Input information Ackn of Limited Supervision mode |  |
|  |  | Subset-026-5.19.1.2 |  |
|  |  | Subset-026-5.19.2.1 |  |
|  |  | Subset-026-5.19.2.2 |  |
|  |  | Subset-026-5.19.2.3 |  |
|  |  | Subset-026-5.19.2.4 |  |
|  |  | Subset-026-7.4.2.26 Packet Number 80: Mode Profile |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The LS mode indication is DISPLAYED A Request for Acknowledgement is DISPLAYED The SB indication is DISPLAYED The SB indication is REMOVED after Drivers Acknowledgement Check on the JRU that : Transition to LS mode is RECORDED Application of Service Brake is RECORDED. Driver Acknowledgement is RECORDED Release of Service Brake is RECORDED |
| Constraints | In this TC the train will be running under the permitted speed in LS Mode before reaching the LS area If the Service Brake were not available for any reason, Emergency Brake shall be used. For LS mode, the mode profile shall define the entry and the length of the Limited Supervision area. To be in Limited Supervision mode, SSP and gradient shall be at least available from the FRONT END of the train. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 | FS / OS / SR |
| Radio communication session | ESTABLISHED | Only in L2 / L3. |
| Gradient Profile | STORED / NOT STORED | Shall be at least available from the FRONT END of the train If not stored, it shall be sent in this TC together with packet 12/15 and packet 80 |
| International SSP | STORED / NOT STORED | Shall be at least available from the FRONT END of the train If not stored, it shall be sent in this TC together with packet 12/15 and packet 80 |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Only in L2 / L3. |
| Service brake is NOT APPLIED | O | TIU | - |
| Current mode and level are DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 / L2 / L3 | FS / OS / SR | One Balise Group message containing packet 12, packet 80 (M_MAMODE=2) is received | I | BTM | One of the steps 1 or 2 will be chosen for testing Reception of the mode transition announcement (mode transition order at current location) If not yet stored, packet 21 gradient profile shall be sent in this message and gradient profile shall be at least available from the front end of the train If not yet stored, packet 27 ISSP shall be sent in this message and ISSP shall be at least available from the front end of the train | L1 / L2 / L3 | FS / OS / SR | - |
| 2 | L1 / L2 / L3 | FS / OS / SR | SA-DATA.Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 15, packet 80 (M_MAMODE=2) is received | I | RTM | One of the steps 1 or 2 will be chosen for testing Reception of the mode transition announcement (mode transition order at current location) If not yet stored, packet 21 gradient profile shall be sent in this message and gradient profile shall be at least available from the front end of the train If not yet stored, packet 27 ISSP shall be sent in this message and ISSP shall be at least available from the front end of the train | L1 / L2 / L3 | FS / OS / SR | - |
| 3 | L1 / L2 / L3 | FS / OS / SR | Complete message including Packet 80: Mode profile is RECORDED | O | JRU | - | L1 / L2 / L3 | FS / OS / SR | - |
| 4 | L1 / L2 / L3 | FS / OS / SR | The ERTMS/ETCS on-board equipment switches immediately to LS mode and a request for acknowledgement is displayed to the driver | O | DMI | - | L1 / L2 / L3 | LS | - |
| 5 | L1 / L2 / L3 | LS | Mode transition to LS is RECORDED | O | JRU | - | L1 / L2 / L3 | LS | - |
| 6 | L1 / L2 / L3 | LS | The on-board informs trackside about mode change Message 136: “Train Position Report” is SENT. | O | RTM | Only in L2 / L3. | L1 / L2 / L3 | LS | - |
| 7 | L1 / L2 / L3 | LS | Message 136: “Train position report” is RECORDED. | O | JRU | Only in L2 / L3. | L1 / L2 / L3 | LS | - |
| 8 | L1 / L2 / L3 | LS | Driver doesn’t acknowledges within 5 sec after the change to LS | I | DMI | the driver acknowledgement time (refer to Appendix A3.1) | L1 / L2 / L3 | LS | - |
| 9 | L1 / L2 / L3 | LS | Service Brake is APPLIED | O | TIU | - | L1 / L2 / L3 | LS | - |
| 10 | L1 / L2 / L3 | LS | Service Brake Indication is DISPLAYED | O | DMI | - | L1 / L2 / L3 | LS | - |
| 11 | L1 / L2 / L3 | LS | Service Brake Intervention is RECORDED | O | JRU | - | L1 / L2 / L3 | LS | - |
| 12 | L1 / L2 / L3 | LS | Driver acknowledges more than 5 sec after the change to LS | I | DMI | the driver acknowledgement time (refer to Appendix A3.1) | L1 / L2 / L3 | LS | - |
| 13 | L1 / L2 / L3 | LS | Driver Acknowledgement of transition to LS mode is RECORDED | O | JRU | - | L1 / L2 / L3 | LS | - |
| 14 | L1 / L2 / L3 | LS | Service Brake is RELEASED | O | TIU | - | L1 / L2 / L3 | LS | - |
| 15 | L1 / L2 / L3 | LS | Service Brake Indication is REMOVED | O | DMI | - | L1 / L2 / L3 | LS | - |
| 16 | L1 / L2 / L3 | LS | Service Brake Release is RECORDED | O | JRU | - | L1 / L2 / L3 | LS | - |
| 17 | L1 / L2 / L3 | LS | LS mode indication is DISPLAYED | O | DMI | - | L1 / L2 / L3 | LS | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 0 | 1 of N |
| N_TOTAL | 3 | 1 | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 80 | Mode profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_MAMODE | 15 | 0 | The mode change is required for the current location |
| M_MAMODE | 2 | 10 | LS |
| V_MAMODE | 7 | FINITE VALUE | - |
| L_MAMODE | 15 | FINITE VALUE | - |
| L_ACKMAMODE | 15 | FINITE VALUE | - |
| Q_MAMODE | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_MAMODE(k) | 15 | FINITE VALUE | - |
| M_MAMODE(k) | 2 | FINITE VALUE | OS, LS, SH. |
| V_MAMODE(k) | 7 | FINITE VALUE | - |
| L_MAMODE(k) | 15 | FINITE VALUE | - |
| L_ACKMAMODE(k) | 15 | FINITE VALUE | - |
| Q_MAMODE(k) | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 12 | A Level 1 movement authority, shall be sent together. |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out". |
| N_ITER | 5 | FINITE VALUE | Set to zero if V_MAIN = 0 or if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 1 | 2 of N |
| N_TOTAL | 3 | 1 | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 2: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | Message 3: Movement authority |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 10 + 14 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | Packet no 15: Level 2/3 MA |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 80 | Packet number 80: Mode profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_MAMODE | 15 | 0 | The mode transition is required for the current location |
| M_MAMODE | 2 | 10 | LS |
| V_MAMODE | 7 | FINITE VALUE | - |
| L_MAMODE | 15 | FINITE VALUE | - |
| L_ACKMAMODE | 15 | FINITE VALUE | - |
| Q_MAMODE | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_MAMODE(k) | 15 | FINITE VALUE | - |
| M_MAMODE(k) | 2 | FINITE VALUE | OS, LS, SH. |
| V_MAMODE(k) | 7 | FINITE VALUE | - |
| L_MAMODE(k) | 15 | FINITE VALUE | - |
| L_ACKMAMODE(k) | 15 | FINITE VALUE | - |
| Q_MAMODE(k) | 1 | FINITE VALUE | - |

| Step 6: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train position report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 10+14 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = 1 or 2 |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | 12 | LS |
| M_LEVEL | 3 | 3 / 4 | L2 / L3 |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 12 | LS |
| Radio communication session | ESTABLISHED | Only in L2 / L3. |
| Gradient Profile | STORED UNCHANGED | Shall be at least available from the FRONT END of the train |
| International SSP | STORED UNCHANGED | Shall be at least available from the FRONT END of the train |
| ERTMS/ETCS level | 2 / 3 / 4 UNCHANGED | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Only in L2 / L3. |
| SB is RELEASED | O | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 5

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.19.2: Limited Supervision is requested for current location (from modes different from Stand By and Post Trip) |  | 5190200 |
| Test Case of Feature | Limited Supervision is requested for the current location and the driver doesn’t acknowledges |  | 5 |
| Applicable Mode/Level Combinations | L1: FS, OS, SR L2: FS, OS, SR L3: FS, OS, SR |  |  |
| Target of Test | The target of this test is to check that: After receiving the order to change to LS the ERTMS/ETCS on-board equipment shall switch immediately to LS mode and a request for acknowledgement shall be displayed to the driver (refer to SRS chapter 4, transitions between modes). The driver does not acknowledge until the train reaches standstill And the service brake command is triggered. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.4.19.1.2 |  |
|  |  | Subset-026-4.7.2 table - Input information Ackn of Limited Supervision mode |  |
|  |  | Subset-026-5.19.1.2 |  |
|  |  | Subset-026-5.19.2.1 |  |
|  |  | Subset-026-5.19.2.2 |  |
|  |  | Subset-026-5.19.2.3 |  |
|  |  | Subset-026-5.19.2.4 |  |
|  |  | Subset-026-7.4.2.26 Packet Number 80: Mode Profile |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The LS mode indication is DISPLAYED A Request for Acknowledgement is DISPLAYED The SB indication in DISPLAYED Check on the JRU that : Transition to LS mode is RECORDED Application of Service Brake is RECORDED. The train reaches standstill |
| Constraints | LS is requested for the current location means that, according to the mode profile received the max safe front end of the train is at or in advance of the location for which switching to LS mode is requested. The train is running under the permitted speed in LS Mode before reaching the LS area. If the Service Brake were not available for any reason, Emergency Brake shall be used. For LS mode, the mode profile shall define the entry and the length of the LS area. To be in Limited Supervision mode, SSP and gradient shall be at least available from the FRONT END of the train. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 | FS / OS / SR |
| Radio communication session | ESTABLISHED | Only in L2 / L3. |
| Gradient Profile | STORED / NOT STORED | Shall be at least available from the FRONT END of the train If not stored, it shall be sent in this TC together with packet 12/15 and packet 80 |
| International SSP | STORED / NOT STORED | Shall be at least available from the FRONT END of the train If not stored, it shall be sent in this TC together with packet 12/15 and packet 80 |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Only in L2 / L3. |
| Service brake is NOT APPLIED | O | TIU | - |
| Current mode and level are DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 / L2 / L3 | FS / OS / SR | One Balise Group message containing packet 12, packet 80 (M_MAMODE=2) is received | I | BTM | One of the steps 2 or 3 will be chosen for testing Reception of the mode transition announcement (mode transition order at current location) If not yet stored, packet 21 gradient profile shall be sent in this message and gradient profile shall be at least available from the front end of the train If not yet stored, packet 27 ISSP shall be sent in this message and ISSP shall be at least available from the front end of the train | L1 / L2 / L3 | FS / OS / SR | - |
| 2 | L1 / L2 / L3 | FS / OS / SR | SA-DATA.Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 15, packet 80 (M_MAMODE=2) is received | I | RTM | One of the steps 2 or 3 will be chosen for testing Reception of the mode transition announcement (mode transition order at current location) If not yet stored, packet 21 gradient profile shall be sent in this message and gradient profile shall be at least available from the front end of the train If not yet stored, packet 27 ISSP shall be sent in this message and ISSP shall be at least available from the front end of the train | L1 / L2 / L3 | FS / OS / SR | - |
| 3 | L1 / L2 / L3 | FS / OS / SR | Complete message including Packet 80: Mode profile is RECORDED | O | JRU | - | L1 / L2 / L3 | FS / OS / SR | - |
| 4 | L1 / L2 / L3 | FS / OS / SR | The ERTMS/ETCS on-board equipment switches immediately to LS mode and a request for acknowledgement is displayed to the driver | O | DMI | - | L1 / L2 / L3 | LS | - |
| 5 | L1 / L2 / L3 | LS | Mode transition to LS is RECORDED | O | JRU | - | L1 / L2 / L3 | LS | - |
| 6 | L1 / L2 / L3 | LS | The on-board informs trackside about mode change Message 136: “Train Position Report” is SENT. | O | RTM | Only in L2 / L3. | L1 / L2 / L3 | LS | - |
| 7 | L1 / L2 / L3 | LS | Message 136: “Train position report” is RECORDED. | O | JRU | Only in L2 / L3. | L1 / L2 / L3 | LS | - |
| 8 | L1 / L2 / L3 | LS | DRIVER NEVER ACKNOWLEDGES | I | DMI | - | L1 / L2 / L3 | LS | - |
| 9 | L1 / L2 / L3 | LS | Service Brake is APPLIED | O | TIU | - | L1 / L2 / L3 | LS | - |
| 10 | L1 / L2 / L3 | LS | Service Brake Indication is DISPLAYED | O | DMI | - | L1 / L2 / L3 | LS | - |
| 11 | L1 / L2 / L3 | LS | Service Brake Intervention is RECORDED | O | JRU | - | L1 / L2 / L3 | LS | - |
| 12 | L1 / L2 / L3 | LS | Train is at standstill (V_TRAIN=0) | I | INT | - | L1 / L2 / L3 | LS | - |
| 13 | L1 / L2 / L3 | LS | Inform Trackside about standstill Message 136: “Train Position Report” is SENT. | O | RTM | Only in L2 / L3. | L1 / L2 / L3 | LS | - |
| 14 | L1 / L2 / L3 | LS | Message 136: “Train position report” is RECORDED. | O | JRU | Only in L2 / L3. | L1 / L2 / L3 | LS | - |
| 15 | L1 / L2 / L3 | LS | Driver acknowledges the change to LS | I | DMI | - | L1 / L2 / L3 | LS | - |
| 16 | L1 / L2 / L3 | LS | Driver Acknowledgement of transition to LS mode is RECORDED | O | JRU | - | L1 / L2 / L3 | LS | - |
| 17 | L1 / L2 / L3 | LS | Service Brake is RELEASED | O | TIU | - | L1 / L2 / L3 | LS | - |
| 18 | L1 / L2 / L3 | LS | Service Brake Indication is REMOVED | O | DMI | - | L1 / L2 / L3 | LS | - |
| 19 | L1 / L2 / L3 | LS | Service Brake Release is RECORDED | O | JRU | - | L1 / L2 / L3 | LS | - |
| 20 | L1 / L2 / L3 | LS | LS mode indication is DISPLAYED | O | DMI | - | L1 / L2 / L3 | LS | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 0 | 1 of N |
| N_TOTAL | 3 | 1 | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | Bga |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 80 | Mode profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_MAMODE | 15 | 0 | The mode change is required for the current location |
| M_MAMODE | 2 | 10 | LS |
| V_MAMODE | 7 | FINITE VALUE | - |
| L_MAMODE | 15 | FINITE VALUE | - |
| L_ACKMAMODE | 15 | FINITE VALUE | - |
| Q_MAMODE | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_MAMODE(k) | 15 | FINITE VALUE | - |
| M_MAMODE(k) | 2 | FINITE VALUE | OS, LS, SH. |
| V_MAMODE(k) | 7 | FINITE VALUE | - |
| L_MAMODE(k) | 15 | FINITE VALUE | - |
| L_ACKMAMODE(k) | 15 | FINITE VALUE | - |
| Q_MAMODE(k) | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 12 | A Level 1 movement authority, shall be sent also or together. |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out". |
| N_ITER | 5 | FINITE VALUE | Set to zero if V_MAIN = 0 or if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 1 | 2 of N |
| N_TOTAL | 3 | 1 | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | Bga |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 2: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | Message 3: Movement authority |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 10 + 14 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 80 | Packet number 80: Mode profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_MAMODE | 15 | 0 | The mode change is required for the current location |
| M_MAMODE | 2 | 10 | LS |
| V_MAMODE | 7 | FINITE VALUE | - |
| L_MAMODE | 15 | FINITE VALUE | - |
| L_ACKMAMODE | 15 | FINITE VALUE | - |
| Q_MAMODE | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_MAMODE(k) | 15 | FINITE VALUE | - |
| M_MAMODE(k) | 2 | FINITE VALUE | - |
| V_MAMODE(k) | 7 | FINITE VALUE | - |
| L_MAMODE(k) | 15 | FINITE VALUE | - |
| L_ACKMAMODE(k) | 15 | FINITE VALUE | - |
| Q_MAMODE (k) | 1 | FINITE VALUE | - |

| Step 6: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train position report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 10+14 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = 1 or 2 |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | 12 | LS |
| M_LEVEL | 3 | 3 / 4 | L2 / L3 |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC |

| Step 13: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train position report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 10+14 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = 1 or 2 |
| V_TRAIN | 7 | 0 | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | 12 | LS |
| M_LEVEL | 3 | 3 / 4 | L2 / L3 |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 12 | LS |
| Radio communication session | ESTABLISHED | Only in L2 / L3. |
| Gradient Profile | STORED UNCHANGED | Shall be at least available from the FRONT END of the train |
| International SSP | STORED UNCHANGED | Shall be at least available from the FRONT END of the train |
| ERTMS/ETCS level | 2 / 3 / 4 UNCHANGED | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Only in L2 / L3. |
| SB is RELEASED | O | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 6

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.19.2: Limited Supervision is requested for current location (from modes different from Stand By and Post Trip) |  | 5190200 |
| Test Case of Feature | Limited Supervision is requested for the current location and the driver acknowledges within the driver acknowledgement time (refer to Appendix A3.1) after the change to LS mode. |  | 6 |
| Applicable Mode/Level Combinations | L1: OS L2: OS L3: OS |  |  |
| Target of Test | The target of this test is to check that: After receiving the order to change to LS the ERTMS/ETCS on-board equipment shall switch immediately to LS mode and a request for acknowledgement shall be displayed to the driver The Driver acknowledges within the driver acknowledgement time (refer to Appendix A3.1) after the change to LS mode |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.4.19.1.2 |  |
|  |  | Subset-026-4.7.2 table - Input information Ackn of Limited Supervision mode |  |
|  |  | Subset-026-5.19.1.2 |  |
|  |  | Subset-026-5.19.2.1 |  |
|  |  | Subset-026-5.19.2.2 |  |
|  |  | Subset-026-5.19.2.3 |  |
|  |  | Subset-026-5.19.2.4 |  |
|  |  | Subset-026-7.4.2.26 Packet Number 80: Mode Profile |  |
|  |  | Subset-026-4.6.3 Condition Id [74] |  |
|  |  | Subset-026-4.6.2 row 6 column 8 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The mode initially DISPLAYED changes to LS A Request for Acknowledgement is DISPLAYED Check on the JRU that : Transition to LS mode is RECORDED Driver Acknowledgement is RECORDED. |
| Constraints | The train is running under the permitted speed in LS Mode before reaching the LS area. If the Service Brake were not available for any reason, Emergency Brake shall be used. For LS mode, the mode profile shall define the entry and the length of the Limited Supervision area. To be in Limited Supervision mode, SSP and gradient shall be at least available from the FRONT END of the train. The estimated front end of the train is not inside an OS acknowledgement area. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 1 | OS |
| Radio communication session | ESTABLISHED | Only in L2 / L3. |
| Gradient Profile | STORED | Shall be at least available from the FRONT END of the train |
| International SSP | STORED | Shall be at least available from the FRONT END of the train |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Only in L2 / L3. |
| Service brake is NOT APPLIED | O | TIU | - |
| Current mode and level are DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 / L2 / L3 | OS | One Balise Group message containing packet 12, packet 80 (M_MAMODE=2) is received | I | BTM | One of the steps 1 or 2 will be chosen for testing Reception of the mode transition announcement (mode transition order at current location) | L1 / L2 / L3 | OS | - |
| 2 | L1 / L2 / L3 | OS | SA-DATA.Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 15, packet 80 (M_MAMODE=2) is received | I | RTM | One of the steps 1 or 2 will be chosen for testing Reception of the mode transition announcement (mode transition order at current location) | L1 / L2 / L3 | OS | - |
| 3 | L1 / L2 / L3 | OS | Complete message including Packet 80: Mode profile is RECORDED | O | JRU | - | L1 / L2 / L3 | OS | - |
| 4 | L1 / L2 / L3 | OS | The ERTMS/ETCS on-board equipment switches immediately to LS mode and a request for acknowledgement is displayed to the driver | O | DMI | - | L1 / L2 / L3 | LS | - |
| 5 | L1 / L2 / L3 | LS | Mode transition to LS is RECORDED | O | JRU | - | L1 / L2 / L3 | LS | - |
| 6 | L1 / L2 / L3 | LS | The on-board informs trackside about mode change Message 136: “Train Position Report” is SENT. | O | RTM | Only in L2 / L3. | L1 / L2 / L3 | LS | - |
| 7 | L1 / L2 / L3 | LS | Message 136: “Train position report” is RECORDED. | O | JRU | Only in L2 / L3. | L1 / L2 / L3 | LS | - |
| 8 | L1 / L2 / L3 | LS | Driver acknowledges within 5 sec after the change to LS | I | DMI | The driver acknowledgement time (refer to Appendix A3.1) | L1 / L2 / L3 | LS | - |
| 9 | L1 / L2 / L3 | LS | Driver Acknowledgement of transition to LS mode is RECORDED | O | JRU | - | L1 / L2 / L3 | LS | - |
| 10 | L1 / L2 / L3 | LS | Service brake is NOT APPLIED | O | TIU | Negative Testing | L1 / L2 / L3 | LS | - |
| 11 | L1 / L2 / L3 | LS | NO service Brake Indication is DISPLAYED | O | DMI | Negative Testing | L1 / L2 / L3 | LS | - |
| 12 | L1 / L2 / L3 | LS | NO Service Brake Intervention is RECORDED | O | JRU | Negative Testing | L1 / L2 / L3 | LS | - |
| 13 | L1 / L2 / L3 | LS | LS Mode indication is displayed | O | DMI | - | L1 / L2 / L3 | LS | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 0 | 1 of N |
| N_TOTAL | 3 | 1 | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 80 | Mode profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_MAMODE | 15 | 0 | Mode change starting in the current location |
| M_MAMODE | 2 | 10 | LS |
| V_MAMODE | 7 | FINITE VALUE | - |
| L_MAMODE | 15 | FINITE VALUE | - |
| L_ACKMAMODE | 15 | FINITE VALUE | - |
| Q_MAMODE | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_MAMODE(k) | 15 | FINITE VALUE | - |
| M_MAMODE(k) | 2 | FINITE VALUE | OS, LS, SH. |
| V_MAMODE(k) | 7 | FINITE VALUE | - |
| L_MAMODE(k) | 15 | FINITE VALUE | - |
| L_ACKMAMODE(k) | 15 | FINITE VALUE | - |
| Q_MAMODE(k) | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 12 | A Level 1 movement authority shall be sent together. |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out". |
| N_ITER | 5 | FINITE VALUE | Set to zero if V_MAIN = 0 or if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 1 | 2 of N |
| N_TOTAL | 3 | 1 | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 2: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | Message 3: Movement authority |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 10 + 14 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | Packet no 15: Level 2/3 MA |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 80 | Packet number 80: Mode profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_MAMODE | 15 | 0 | Mode transition for the current location |
| M_MAMODE | 2 | 10 | LS |
| V_MAMODE | 7 | FINITE VALUE | - |
| L_MAMODE | 15 | FINITE VALUE | - |
| L_ACKMAMODE | 15 | FINITE VALUE | - |
| Q_MAMODE | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_MAMODE(k) | 15 | FINITE VALUE | - |
| M_MAMODE(k) | 2 | FINITE VALUE | OS, LS, SH. |
| V_MAMODE(k) | 7 | FINITE VALUE | - |
| L_MAMODE(k) | 15 | FINITE VALUE | - |
| L_ACKMAMODE(k) | 15 | FINITE VALUE | - |
| Q_MAMODE(k) | 1 | FINITE VALUE | - |

| Step 6: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train position report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 10+14 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = 1 or 2 |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | 12 | LS |
| M_LEVEL | 3 | 3 / 4 | L2 / L3 |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 12 | LS |
| Radio communication session | ESTABLISHED | Only in level 2/3 |
| Gradient Profile | STORED UNCHANGED | Shall be at least available from the FRONT END of the train |
| International SSP | STORED UNCHANGED | Shall be at least available from the FRONT END of the train |
| ERTMS/ETCS level | 2 / 3 / 4 UNCHANGED | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Only in L2 / L3. |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |