| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 3170200 (3.17.2: Determination of the operated system version) Total: 10 Test case(s) |  |
| REF: | Subset-076-5-2-3170200 |
| ISSUE: | Version 3.2.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 2.9.0 31.03.2013 | All | First version/New in Baseline 3 | INECO |
| Version 2.9.1 01.08.2013 | All | Second version/Lab Internal modification: TC_Editorv27b tool used; Correction of Requirements blank spaces. | INECO |
| Version 2.9.2 25.09.2013 | All | Third version/ UNISIG comments from 26.08.2013 included | INECO |
| Version 2.9.3 02.12.2013 | All | Fourth version/ Updated after bilateral meeting with UNISIG on 29.11.2013 | INECO |
| Version 2.9.4 26.12.2013 | All | Fifth version/ Internal version: cross-reference checked; old reference removed. | INECO |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.2.0 31.03.2017 | All | Baseline 3 1st maintenance release version - Bug fixing | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.17.2: Determination of the operated system version |  | 3170200 |
| Test Case of Feature | If the on-board equipment is in level 0, NTC or 1 (e.g. entrance in level 2/3 area), the RBC system version shall be operated when the transition to level 2/3 is executed. |  | 1 |
| Applicable Mode/Level Combinations | L0 : SH / UN / TR / NL LNTC : SH / TR / NL / SN L1: FS / LS / OS / SR / SH / TR / PT / NL / RV |  |  |
| Target of Test | The target of this test case is to check that: The RBC system version shall be operated when the transition to level 2/3 is executed. The on-board equipment shall operate with only one system version at a time. The system version of the RBC shall take precedence on the operated system version in relation to non-RBC constituents and system version ordered from balise group. The function "System Version Management" is active. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.17.2.1 |  |
|  |  | Subset-026-3.17.2.2 |  |
|  |  | Subset-026-3.17.2.8 |  |
|  |  | Subset-026-3.17.2.8 a) |  |
|  |  | Subset-026-4.5.2.1 table - System Version Management |  |
|  |  | Subset-026-4.7.2 table - Output information Operated System Version |  |
|  |  | Subset-026-4.8.3.1.1 Trackside constituent System Version |  |
|  |  | Subset-026-4.8.4.2 Trackside constituent System Version |  |
|  |  | Subset-026-4.8.3.1.1 System Version order |  |
|  |  | Subset-026-4.8.4.2 System Version order |  |
|  |  | Subset-026-7.4.2.1 Packet Number 2: System Version order |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: Operated system version is the same as the balise system version Operated system version after level transition is the same as the RBC system version Check on the RTM that: Message 32 RBC system version is RECEIVED. Check on the JRU that: Reception of message 32 is RECORDED General message is RECORDED |
| Constraints | The on-board equipment operates with one system version different to RBC system version but the same to non-RBC constituents and on system version ordered from BG (before level transition). NOTE: Rule used to define M_VERSION column: For RBC and BTM steps the M_VERSION represents the version of the components and in case of JRU steps the M_VERSION values is the CURRENTLY OPERATED VERSION. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2/ 3/ 4/ 7/ 8/ 11/12/ 13/ 14 | FS/ OS/ SR/ SH/ UN/ TR/ PT/ NL/ LS/ SN/ RV |
| Radio communication session | NOT ESTABLISHED | M_VERSION=010 0000 |
| ERTMS/ETCS level | 0/ 1/ 2 | L0/ LNTC/ L1 |
| Radio Network ID | STORED | - |
| RBC ID/Phone Number | STORED | - |
| Operated System Version | STORED | Version 1.0 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | M_VERSION | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  |  | Levels | Modes |  |
| 1 | L0/ LNTC/ L1 | FS/ OS/ SR/ SH/ UN/ TR/ PT/ NL/ LS/ SN/ RV | One Balise Group message (N_TOTAL=1) containing packet 42 is received | I | BTM | 001 0000 | - | L0/ LNTC/ L1 | FS/ OS/ SR/ SH/ UN/ TR/ PT/ NL/ LS/ SN/ RV | - |
| 2 | L0/ LNTC/ L1 | FS/ OS/ SR/ SH/ UN/ TR/ PT/ NL/ LS/ SN/ RV | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6; M_VERSION=001 0000) is recorded | O | JRU | 001 0000 | - | L0/ LNTC/ L1 | FS/ OS/ SR/ SH/ UN/ TR/ PT/ NL/ LS/ SN/ RV | - |
| 3 | L0/ LNTC/ L1 | FS/ OS/ SR/ SH/ UN/ TR/ PT/ NL/ LS/ SN/ RV | SA-CONNECT. Request is transmitted | O | RTM | - | Mobile terminal is registered | L0/ LNTC/ L1 | FS/ OS/ SR/ SH/ UN/ TR/ PT/ NL/ LS/ SN/ RV | - |
| 4 | L0/ LNTC/ L1 | FS/ OS/ SR/ SH/ UN/ TR/ PT/ NL/ LS/ SN/ RV | SA-CONNECT. Confirm is received | I | RTM | - | - | L0/ LNTC/ L1 | FS/ OS/ SR/ SH/ UN/ TR/ PT/ NL/ LS/ SN/ RV | - |
| 5 | L0/ LNTC/ L1 | FS/ OS/ SR/ SH/ UN/ TR/ PT/ NL/ LS/ SN/ RV | SA-DATA. Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted | O | RTM | - | - | L0/ LNTC/ L1 | FS/ OS/ SR/ SH/ UN/ TR/ PT/ NL/ LS/ SN/ RV | - |
| 6 | L0/ LNTC/ L1 | FS/ OS/ SR/ SH/ UN/ TR/ PT/ NL/ LS/ SN/ RV | MESSAGE TO RBC (NID_MESSAGE_JRU=10; M_VERSION=001 0000) is recorded | O | JRU | 001 0000 | - | L0/ LNTC/ L1 | FS/ OS/ SR/ SH/ UN/ TR/ PT/ NL/ LS/ SN/ RV | - |
| 7 | L0/ LNTC/ L1 | FS/ OS/ SR/ SH/ UN/ TR/ PT/ NL/ LS/ SN/ RV | SA-DATA. Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32; M_VERSION=010 0000) is received | I | RTM | 010 0000 | - | L0/ LNTC/ L1 | FS/ OS/ SR/ SH/ UN/ TR/ PT/ NL/ LS/ SN/ RV | - |
| 8 | L0/ LNTC/ L1 | FS/ OS/ SR/ SH/ UN/ TR/ PT/ NL/ LS/ SN/ RV | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; M_VERSION=001 0000) is recorded | O | JRU | 001 0000 | Communication session established for on-board. | L0/ LNTC/ L1 | FS/ OS/ SR/ SH/ UN/ TR/ PT/ NL/ LS/ SN/ RV | - |
| 9 | L0/ LNTC/ L1 | FS/ OS/ SR/ SH/ UN/ TR/ PT/ NL/ LS/ SN/ RV | SA-DATA. Request with Euroradio Message "Session Established" (NID_MESSAGE=159) is transmitted. | O | RTM | - | - | L0/ LNTC/ L1 | FS/ OS/ SR/ SH/ UN/ TR/ PT/ NL/ LS/ SN/ RV | - |
| 10 | L0/ LNTC/ L1 | FS/ OS/ SR/ SH/ UN/ TR/ PT/ NL/ LS/ SN/ RV | MESSAGE TO RBC (NID_MESSAGE_JRU=10; M_VERSION=001 0000) is recorded | O | JRU | 001 0000 | Communication session established for trackside RBC ID/Phone Number are valid | L0/ LNTC/ L1 | FS/ OS/ SR/ SH/ UN/ TR/ PT/ NL/ LS/ SN/ RV | - |
| 11 | L0/ LNTC/ L1 | FS/ OS/ SR/ SH/ UN/ TR/ PT/ NL/ LS/ SN/ RV | Train data send by on-board and ACK by RBC. | - | - | - | USE_FT4080401.1 Train data ACK by RBC | L0/ LNTC/ L1 | FS/ OS/ SR/ SH/ UN/ TR/ PT/ NL/ LS/ SN/ RV | - |
| 12 | L0/ LNTC/ L1 | FS/ OS/ SR/ SH/ UN/ TR/ PT/ NL/ LS/ SN/ RV | The driver presses "System version" | I | DMI | - | - | L0/ LNTC/ L1 | FS/ OS/ SR/ SH/ UN/ TR/ PT/ NL/ LS/ SN/ RV | - |
| 13 | L0/ LNTC/ L1 | FS/ OS/ SR/ SH/ UN/ TR/ PT/ NL/ LS/ SN/ RV | DMI shows the operated system version | O | DMI | - | Operated system version is 1.0 | L0/ LNTC/ L1 | FS/ OS/ SR/ SH/ UN/ TR/ PT/ NL/ LS/ SN/ RV | - |
| 14 | L0/ LNTC/ L1 | FS/ OS/ SR/ SH/ UN/ TR/ PT/ NL/ LS/ SN/ RV | Transition to level 2/3 is performed | - | - | - | USE_FT5100100.13 from L0 to L2 or USE_FT5100100.21 from LNTC to L2 or USE_FT5100100.37 from L1 to L2 or USE_FT5100100.26 from L0 to L3 or USE_FT5100100.20 from LNTC to L3 or USE_FT5100100.36 from L1 to L3 | L2 / L3 | FS/ OS/ SR/ SH/ TR/ PT/ NL/ LS/ RV | - |
| 15 | L2/ L3 | FS/ OS/ SR/ SH/ TR/ PT/ NL/ LS/ RV | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_VERSION=010 0000) is recorded | O | JRU | 010 0000 | - | L2/ L3 | FS/ OS/ SR/ SH/ TR/ PT/ NL/ LS/ RV | - |
| 16 | L2/ L3 | FS/ OS/ SR/ SH/ TR/ PT/ NL/ LS/ RV | The driver presses "System version" | I | DMI | - | - | L2/ L3 | FS/ OS/ SR/ SH/ TR/ PT/ NL/ LS/ RV | - |
| 17 | L2/ L3 | FS/ OS/ SR/ SH/ TR/ PT/ NL/ LS/ RV | DMI shows the operated system version | O | DMI | - | Operated system version is 2.0 | L2/ L3 | FS/ OS/ SR/ SH/ TR/ PT/ NL/ LS/ RV | - |
| 18 | L2/ L3 | FS/ OS/ SR/ SH/ TR/ PT/ NL/ LS/ RV | One Balise Group message (N_TOTAL=1) containing packet 2 (M_VERSION=001 0000) is received | I | BTM | 010 0000 | - | L2/ L3 | FS/ OS/ SR/ SH/ TR/ PT/ NL/ LS/ RV | - |
| 19 | L2/ L3 | FS/ OS/ SR/ SH/ TR/ PT/ NL/ LS/ RV | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6; M_VERSION=010 0000) is recorded | O | JRU | 010 0000 | - | L2/ L3 | FS/ OS/ SR/ SH/ TR/ PT/ NL/ LS/ RV | - |
| 20 | L2/ L3 | FS/ OS/ SR/ SH/ TR/ PT/ NL/ LS/ RV | The driver presses "System version" | I | DMI | - | - | L2/ L3 | FS/ OS/ SR/ SH/ TR/ PT/ NL/ LS/ RV | - |
| 21 | L2/ L3 | FS/ OS/ SR/ SH/ TR/ PT/ NL/ LS/ RV | DMI shows the operated system version | O | DMI | - | Operated system version is 2.0 Every JRU message recorded contains the currently operated system version M_VERSION=010 0000 | L2/ L3 | FS/ OS/ SR/ SH/ TR/ PT/ NL/ LS/ RV | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 001 0000 | Version 1.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | FINITE VALUE | - |
| N_TOTAL | 3 | FINITE VALUE | - |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BG 1 |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 42 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_RBC | 1 | 1 | - |
| NID_C | 10 | FINITE VALUE | RBC ETCS identity: NID_C not relevant if NID_RBC has value "Contact last known RBC". |
| NID_RBC | 14 | FINITE VALUE | - |
| NID_RADIO | 64 | FINITE VALUE | Not relevant if NID_RBC has value "Contact last known RBC". |
| Q_SLEEPSESSION | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | End of Information |

| Step 5: Radio Message 155 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 155 | Initiation of a communication session |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |

| Step 7: Radio Message 32 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 32 | RBC/RIU System Version |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| M_VERSION | 7 | 010 0000 | Version of the ERTMS/ETCS system |

| Step 9: Radio Message 159 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 159 | Session established |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |

| Step 18: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 010 0000 | Version 2.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | FINITE VALUE | - |
| N_TOTAL | 3 | FINITE VALUE | - |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BG 2 |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 2 | System version order |
| Q_DIR | 2 | FINITE VALUE | Relevant direction for the train |
| L_PACKET | 13 | FINITE VALUE | - |
| M_VERSION | 7 | 001 0000 | Version 1.0 |
| NID_PACKET | 8 | 255 | End of Information |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2/ 3/ 4/ 7/ 8/ 11/12/ 13/ 14 | FS/ OS/ SR/ SH/ UN/ TR/ PT/ NL/ LS/ SN/ RV |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Operated System Version | STORED | Version 2.0 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.17.2: Determination of the operated system version |  | 3170200 |
| Test Case of Feature | In case of SoM procedure or order received from trackside, if the on-board equipment is in level 2/3, the RBC system version shall be operated immediately. |  | 2 |
| Applicable Mode/Level Combinations | L2 / L3: SB |  |  |
| Target of Test | The target of this test case is to check that: If the on-board equipment is in level 2/3 (SoM procedure), the RBC system version shall be operated immediately The on-board equipment shall operate with only one system version at a time. The system version of the RBC shall take precedence on the operated system version in relation to non-RBC constituents and system version ordered from balise group. The function "System Version Management" is active. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.17.2.1 |  |
|  |  | Subset-026-3.17.2.2 |  |
|  |  | Subset-026-3.17.2.8 |  |
|  |  | Subset-026-3.17.2.8 b) |  |
|  |  | Subset-026-4.5.2.1 table - System Version Management |  |
|  |  | Subset-026-4.7.2 table - Output information Operated System Version |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: Operated system version is the same as the RBC system version Check on the RTM that: Message 32 RBC/RIU system version is RECEIVED. Check on the JRU that: Reception of message 32 is RECORDED General message is RECORDED at every operated system version change. |
| Constraints | The on-board equipment operates with one system version different to RBC system version. SoM procedure at S0. NOTE: Rule used to define M_VERSION column: For RBC and BTM steps the M_VERSION represents the version of the components and in case of JRU steps the M_VERSION values is the CURRENTLY OPERATED VERSION. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Radio communication session | NOT ESTABLISHED | - |
| Train Position | VALID | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| RBC/RIU System Version | NOT STORED | - |
| Operated System Version | STORED | Version 1.0 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | M_VERSION | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  |  | Levels | Modes |  |
| 1 | L2/L3 | SB | The Mode symbol "Stand By" is displayed. | O | DMI | - | Symbol MO13 | L2/L3 | SB | - |
| 2 | L2/L3 | SB | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit28=1>) is recorded | O | JRU | - | Check inside common header that SYSTEM_VERSION variable is version 1.0 | L2/L3 | SB | - |
| 3 | L2/L3 | SB | The driver re-validates the "Driver ID" | I | DMI | - | Driver ID - TBR Coming from SB | Na | Na | - |
| 4 | L2/L3 | SB | The driver re-validates the “Train Running Number” | I | DMI | - | Train running number -TBR. Coming from SB | L2/L3 | SB | - |
| 5 | L2/L3 | SB | ADDITIONAL DATA (NID_MESSAGE_JRU=24; DRIVER_ID; NID_OPERATIONAL) | O | JRU | - | Check that two variables are recorded. | L2/L3 | SB | - |
| 6 | L2/L3 | SB | The Level symbol "Level 2/Level 3" is displayed | O | DMI | - | Symbol LE04 or LE05 ERTMS/ETCS Level and train position are valid | L2/L3 | SB | - |
| 7 | L2/L3 | SB | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit4=1 or Bit5=1>) is recorded | O | JRU | - | - | L2/L3 | SB | - |
| 8 | L2/L3 | SB | SA-CONNECT. Request is transmitted | O | RTM | - | Mobile terminal is registered | L2/L3 | SB | - |
| 9 | L2/L3 | SB | SA-CONNECT. Confirm is received | I | RTM | - | - | L2/L3 | SB | - |
| 10 | L2/L3 | SB | SA-DATA. Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted | O | RTM | - | - | L2/L3 | SB | - |
| 11 | L2/L3 | SB | MESSAGE TO RBC (NID_MESSAGE_JRU=10; M_VERSION=001 0000) is recorded | O | JRU | 001 0000 | - | L2/L3 | SB | - |
| 12 | L2/L3 | SB | SA-DATA. Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32; M_VERSION=010 0000) is received | I | RTM | 010 0000 | - | L2/L3 | SB | - |
| 13 | L2/L3 | SB | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; 010 0000) is recorded | O | JRU | 010 0000 | Communication session established for on-board. | L2/L3 | SB | - |
| 14 | L2/L3 | SB | SA-DATA. Request with Euroradio Message "Session Established" (NID_MESSAGE=159) is transmitted. | O | RTM | - | - | L2/L3 | SB | - |
| 15 | L2/L3 | SB | MESSAGE TO RBC (NID_MESSAGE_JRU=10; M_VERSION=010 0000) is recorded | O | JRU | 010 0000 | Communication session established for trackside RBC ID/Phone Number are valid | L2/L3 | SB | - |
| 16 | L2/L3 | SB | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_VERSION=010 0000) is recorded | O | JRU | 010 0000 | - | L2/L3 | SB | - |
| 17 | L2/L3 | SB | SA-DATA. Request with Euroradio Message "SoM Position Report" (NID_MESSAGE=157, Q_STATUS=1) containing packet 0 is transmitted | O | RTM | 010 0000 | - | L2/L3 | SB | - |
| 18 | L2/L3 | SB | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=157; Q_STATUS=1; NID_PACKET=0) is recorded | O | JRU | 010 0000 | - | L2/L3 | SB | - |
| 19 | L2/L3 | SB | The driver presses "System version" | I | DMI | - | - | L2/L3 | SB | - |
| 20 | L2/L3 | SB | DMI shows the operated system version | O | DMI | - | Operated system version is 2.0 | L2/L3 | SB | - |

| Step 10: Radio Message 155 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 155 | Initiation of a communication session |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |

| Step 12: Radio Message 32 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 32 | RBC/RIU System Version |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| M_VERSION | 7 | 010 0000 | Version of the ERTMS/ETCS system |

| Step 14: Radio Message 159 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 159 | Session established |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |

| Step 17: Radio Message 157 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 157 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| Q_STATUS | 2 | 1 | VALID |
| NID_PACKET | 8 | 0 | Packet 1 must be used if position report is based on 2 BG |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | 6 | SB |
| M_LEVEL | 3 | 3 / 4 | L2 / L3 |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3/ 4 | L2/ L3 |
| Operated System Version | STORED | Version 2.0 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.17.2: Determination of the operated system version |  | 3170200 |
| Test Case of Feature | In case of session established with an accepting RBC (RBC/RBC Handover), the accepting RBC system version shall be operated as soon as the engine has passed the RBC/RBC border location with its maximum safe front end. |  | 3 |
| Applicable Mode/Level Combinations | L2 / L3 : FS / LS / OS / SR / TR / NL |  |  |
| Target of Test | The target of this test case is to check that: The accepting RBC system version shall be operated when the engine passes the RBC/RBC border location with its max safe front end. The on-board equipment shall operate with only one system version at a time. The system version of the RBC shall take precedence on the operated system version in relation to non-RBC constituents and system version ordered from balise group. The function "System Version Management" is active. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.17.2.1 |  |
|  |  | Subset-026-3.17.2.2 |  |
|  |  | Subset-026-3.17.2.8 |  |
|  |  | Subset-026-3.17.2.8 c) |  |
|  |  | Subset-026-4.5.2.1 table - System Version Management |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: Operated system version is the same as the Handing Over RBC system version Operated system version after handing over is the same as the Accepting RBC system version and the highest Operated system version number Y. Check on the RTM that: Message 32 RBC system version is RECEIVED. Check on the JRU that: Reception of message 32 is RECORDED General message is RECORDED at every operated system version change |
| Constraints | The on-board equipment operates with one system version different to accepting RBC system version Two communication sessions can be handled simultaneously. RBC Handover has a different system version than RBC accepting. NOTE: Rule used to define M_VERSION column: For RBC and BTM steps the M_VERSION represents the version of the components and in case of JRU steps the M_VERSION values is the CURRENTLY OPERATED VERSION. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2/ 7/ 11 /12 | FS/ OS/ SR/ TR/ NL/ LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2/ L3 |
| RBC/RIU System Version | STORED | Handing over RBC system version 2.0 |
| Operated System Version | STORED | Version 2.0 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Handing over RBC |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | M_VERSION | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA. Indication with Euroradio Message "General Message" (NID_MESSAGE=24) containing packet 131 is received. | I | RTM | 010 0000 | Handing Over RBC: The radio message contains the RBC transition border. (D_RBCTR ≠ 0) | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; M_VERSION=001 0000) is recorded | O | JRU | 010 0000 | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-CONNECT. Request is transmitted | O | RTM | - | Accepting RBC: A safe connection is established with the Accepting RBC | N/A | N/A | - |
| 4 | N/A | N/A | SA-CONNECT. Confirm is received | I | RTM | - | - | N/A | N/A | - |
| 5 | N/A | N/A | SA-DATA. Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted | O | RTM | - | to the Accepting RBC | N/A | N/A | - |
| 6 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10; M_VERSION=010 0000) is recorded | O | JRU | 010 0000 | - | N/A | N/A | - |
| 7 | N/A | N/A | The driver presses "System version" | I | DMI | - | - | N/A | N/A | - |
| 8 | N/A | N/A | DMI shows the operated system version | O | DMI | - | Operated system version is 2.0 Every JRU message recorded contains the currently operated system version M_VERSION=010 0000 | N/A | N/A | - |
| 9 | N/A | N/A | SA-DATA. Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32) is received | I | RTM | 001 0000 | Accepting RBC: The Version of the ERTMS/ETCS language is valid for the train and M_VERSION ( { 000 XXXX, 111 1111} | N/A | N/A | - |
| 10 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; M_VERSION=010 0000) is recorded | O | JRU | 010 0000 | - | N/A | N/A | - |
| 11 | N/A | N/A | SA-DATA. Request with Euroradio Message "Session Established" (NID_MESSAGE=159) is transmitted | O | RTM | - | Accepting RBC: The communication session with the Accepting RBC is established | N/A | N/A | - |
| 12 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10; M_VERSION=010 0000) is re0corded | O | JRU | 010 0000 | - | N/A | N/A | - |
| 13 | N/A | N/A | SA-DATA. Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) containing packet 0 is transmitted | O | RTM | - | Accepting RBC: The radio message contains also position report data. This step does not apply for SL or NL mode. | N/A | N/A | - |
| 14 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10; M_VERSION=010 0000) is recorded | O | JRU | 010 0000 | This step does not apply for SL or NL mode. | N/A | N/A | - |
| 15 | N/A | N/A | SA-DATA. Indication with Euroradio Message "Acknowledgement of Train Data" (NID_MESSAGE=8) is received | I | RTM | 001 0000 | Accepting RBC: This step does not apply for SL or NL mode. | N/A | N/A | - |
| 16 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; M_VERSION=010 0000) is recorded | O | JRU | 010 0000 | This step does not apply for SL or NL mode. | N/A | N/A | - |
| 17 | N/A | N/A | Train maximum safe front end location = LRBG + D_RBCTR | I | INT | - | Steps 18 and 20 are managed at the same time. | N/A | N/A | - |
| 18 | N/A | N/A | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted to Handing Over RBC. | O | RTM | - | Train location: The maximum safe front end of the train passes the location of the RBC transition. | N/A | N/A | - |
| 19 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10; M_VERSION=001 0000) is recorded | O | JRU | 001 0000 | - | N/A | N/A | - |
| 20 | N/A | N/A | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted to Accepting RBC. | O | RTM | - | Train position: The maximum safe front end of the train passes the location of the RBC transition | N/A | N/A | - |
| 21 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10 M_VERSION=001 0000) is recorded | O | JRU | 001 0000 | - | N/A | N/A | - |
| 22 | N/A | N/A | The On board is supervised by the Accepting RBC. Reception of an MA from the Accepting RBC | - | - | - | USE_FT4080425.3 Transmission of L2/L3 movement Authority from Accepting RBC | N/A | N/A | - |
| 23 | N/A | N/A | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_VERSION=001 0000) is recorded | O | JRU | - | Check inside common header that SYSTEM_VERSION variable is version 1.0 | N/A | N/A | - |
| 24 | N/A | N/A | The new MA is displayed | O | DMI | - | - | N/A | N/A | - |
| 25 | N/A | N/A | One Balise Group message (N_TOTAL=1) containing the order to execute immediately the handover packet 131 (Q_DIR=0; Q_LOCACC=10) is received | I | BTM | 010 0000 | The balise group is placed at the RBC/RBC transition border.(D_RBCTR = 0) | N/A | N/A | - |
| 26 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6; M_VERSION=001 0000) is recorded. | O | JRU | 001 0000 | - | N/A | N/A | - |
| 27 | N/A | N/A | The driver presses "System version" | I | DMI | - | - | N/A | N/A | - |
| 28 | N/A | N/A | DMI shows the operated system version | O | DMI | - | Operated system version is 1.0 Every JRU message recorded contains the currently operated system version M_VERSION=001 0000 | N/A | N/A | - |
| 29 | N/A | N/A | Train min safe rear end location = LRBG + D_RBCTR+L_TRAIN | I | INT | - | The minimum rear end of the train passes the location of the RBC transition | N/A | N/A | - |
| 30 | N/A | N/A | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted to Handing Over RBC. | O | RTM | - | - | N/A | N/A | - |
| 31 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10; M_VERSION=001 0000) is recorded | O | JRU | 001 0000 | - | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General Message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 131 | RBC transition order |
| Q_DIR | 2 | FINITE VALUE | Valid for current train direction |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_RBCTR | 15 | FINITE VALUE | ≠ 0 |
| NID_C | 10 | FINITE VALUE | Valid for the train |
| NID_RBC | 14 | FINITE VALUE | Accepting RBC |
| NID_RADIO | 64 | FINITE VALUE | Accepting RBC |
| Q_SLEEPSESSION | 1 | 0 | Sleeping equipment don’t consider session management information (to avoid TIU influence) |

| Step 5: Radio Message 155 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 155 | Initiation of a communication session |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 1 | FINITE VALUE | - |

| Step 9: Radio Message 32 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 32 | RBC/RIU System Version |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | 0 | To avoid a message 146 |
| NID_LRBG | 24 | FINITE VALUE | - |
| M_VERSION | 7 | 001 0000 | version 1.0 |

| Step 11: Radio Message 159 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 159 | Session established |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 1 | FINITE VALUE | - |

| Step 13: Radio Message 129 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 129 | Validated Train Data |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | < D_RBCTR |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | - |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_PACKET | 8 | 11 | Validated Train Data |
| L_PACKET | 13 | FINITE VALUE | - |
| NC_CDTRAIN | 4 | FINITE VALUE | - |
| NC_TRAIN | 15 | FINITE VALUE | - |
| L_TRAIN | 12 | FINITE VALUE | - |
| V_MAXTRAIN | 7 | FINITE VALUE | - |
| M_LOADINGGAUGE | 8 | FINITE VALUE | - |
| M_AXLELOADCAT | 7 | FINITE VALUE | - |
| M_AIRTIGHT | 2 | FINITE VALUE | - |
| N_AXLE | 10 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_VOLTAGE(k) | 4 | FINITE VALUE | Identity of the traction system. |
| NID_CTRACTION(k) | 10 | FINITE VALUE | NID_CTRACTION (k) given only if M_VOLTAGE (k) ≠ 0. |
| N_ITER | 5 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | Type of National System available. |

| Step 15: Radio Message 8 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 8 | Acknowledgement of Train Data |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | t0+ t’ with t’ > 0 |
| M_ACK | 1 | 0 | To avoid a message 146 |
| NID_LRBG | 24 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | t0 |

| Step 18: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 20 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | Max safe front end is considered (D_RBCTR - L_DOUBTUNDER) |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | - |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |

| Step 20: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | Max safe front end is considered (D_RBCTR - L_DOUBTUNDER) |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | - |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |

| Step 25: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 010 0000 | Version 2.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | FINITE VALUE | - |
| N_TOTAL | 3 | FINITE VALUE | - |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BG 1 |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 131 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_RBCTR | 15 | 0 | - |
| NID_C | 10 | FINITE VALUE | Accepting RBC identity. |
| NID_RBC | 14 | FINITE VALUE | - |
| NID_RADIO | 64 | FINITE VALUE | Accepting RBC radio subscriber number. |
| Q_SLEEPSESSION | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | End of Information |

| Step 30: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | BG 1 |
| D_LRBG | 15 | FINITE VALUE | Min safe rear end is considered (D_RBCTR + L_TRAIN + L_DOUBTOVER) |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | - |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2/ 7/ 11/ 12 | FS/ OS/ SR/ TR/ NL/ LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Operated System Version | STORED | Version 1.0 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Connected with Accepting RBC |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.17.2: Determination of the operated system version |  | 3170200 |
| Test Case of Feature | In case of the on-board equipment switches from level 2/3 to another level, the system version control in relation to non-RBC constituents shall be again applied and the balise group orders shall be again considered. |  | 4 |
| Applicable Mode/Level Combinations | L2 / L3 : FS / LS / OS / SR |  |  |
| Target of Test | The target of this test case is to check that: In case the on-board equipment switches from level 2/3 to another level, the system version control in relation to non-RBC constituents shall be again applied and the balise group orders shall be again considered. The on-board equipment shall operate with only one system version at a time. On receiving the order to operate system version from balise group, the on-board equipment shall immediately operate the system version given in the order. The function "System Version Management" is active. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.17.2.1 |  |
|  |  | Subset-026-3.17.2.2 |  |
|  |  | Subset-026-3.17.2.3 |  |
|  |  | Subset-026-3.17.2.5 |  |
|  |  | Subset-026-3.17.2.8 |  |
|  |  | Subset-026-3.17.2.8 d) |  |
|  |  | Subset-026-4.5.2.1 table - System Version Management |  |
|  |  | Subset-026-4.7.2 table - Output information Operated System Version |  |
|  |  | Subset-026-4.8.3.1.1 Trackside constituent System Version |  |
|  |  | Subset-026-4.8.4.2 Trackside constituent System Version |  |
|  |  | Subset-026-4.8.3.1.1 System Version order |  |
|  |  | Subset-026-4.8.4.2 System Version order |  |
|  |  | Subset-026-7.4.2.1 Packet Number 2: System Version order |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: Operated system version is the same as the RBC system version before transition to Level 1 Operated system version is the same as the BG system version after Level transition to Level 1 Operated system version is the same as ordered by track side Operated system version is the same as the BG system version Check on the BTM that: Packet 41 Level transition order is RECEIVED. Packet 2 System version order is RECEIVED. Packet 5 Linking is RECEIVED. Check on the JRU that: Reception of packet 41 is RECORDED Reception of message 136 is RECORDED Reception of packet 2 is RECORDED Reception of packet 5 is RECORDED General message is RECORDED at every operated system version change |
| Constraints | The on-board equipment operates with one system version different to System version order. The RBC operated system version should be different from the operated system version of Non-RBC constituents. The TC does not terminate the communication session. NOTE: Rule used to define M_VERSION column: For RBC and BTM steps the M_VERSION represents the version of the components and in case of JRU steps the M_VERSION values is the CURRENTLY OPERATED VERSION. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2 /12 | FS / OS/ SR / LS |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Level Transition Order | STORED | Transition to Level 1 at a further location |
| ERTMS/ETCS level | 3/ 4 | L2/ L3 |
| RBC/RIU System Version | STORED | Version 1.0 |
| Operated System Version | STORED | Version 1.0 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | RBC System version is 1.0 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | M_VERSION | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  |  | Levels | Modes |  |
| 1 | L2 / L3 | FS / LS / OS / SR | The driver presses "System version" | I | DMI | - | - | L2 / L3 | FS / LS / OS / SR | - |
| 2 | L2 / L3 | FS / LS / OS / SR | DMI shows the operated system version | O | DMI | - | Operated system version is 1.0 Every JRU message recorded contains the currently operated system version M_VERSION=001 0000 | L2 / L3 | FS / LS / OS / SR | - |
| 3 | L2 / L3 | FS / LS / OS / SR | One Balise Group message (N_TOTAL=1) containing packet 41 (M_LEVELTR=2) is received | I | BTM | 010 0000 | - | L2 / L3 | FS / LS / OS / SR | - |
| 4 | L2 / L3 | FS / LS / OS / SR | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6; M_VERSION=010 0000) is recorded | O | JRU | 010 0000 | - | L2 / L3 | FS / LS / OS / SR | - |
| 5 | L2 / L3 | FS / LS / OS / SR | The Level symbol "Level 1" is displayed | O | DMI | - | Level transition to level 1 is executed. DMI object: Symbol LE03 | L1 | FS / LS / OS / SR | - |
| 6 | L1 | FS / LS / OS / SR | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit3=1¦Bit4=0¦Bit5=0>) is recorded | O | JRU | - | - | L1 | FS / LS / OS / SR | - |
| 7 | L1 | FS / LS / OS / SR | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL= 2; M_VERSION=010 0000) is recorded | O | JRU | 010 0000 | - | L1 | FS / LS / OS / SR | - |
| 8 | L1 | FS / LS / OS / SR | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) is transmitted | O | RTM | - | Train passes the level transition border with its min safe rear end. | L1 | FS / LS / OS / SR | - |
| 9 | L1 | FS / LS / OS / SR | MESSAGE TO RBC (NID_MESSAGE_JRU=10; M_VERSION=010 0000) is recorded | O | JRU | 010 0000 | - | L1 | FS / LS / OS / SR | - |
| 10 | L1 | FS / LS / OS / SR | The driver presses "System version" | I | DMI | - | - | L1 | FS / LS / OS / SR | - |
| 11 | L1 | FS / LS / OS / SR | DMI shows the operated system version | O | DMI | - | Operated system version is 2.0 Every JRU message recorded contains the currently operated system version M_VERSION=010 0000 | L1 | FS / LS / OS / SR | - |
| 12 | L1 | FS / LS / OS / SR | One Balise Group message (N_TOTAL=1) containing packet 2 (M_VERSION=001 0000) is received | I | BTM | 001 0001 | - | L1 | FS / LS / OS / SR | - |
| 13 | L1 | FS / LS / OS / SR | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6; M_VERSION=001 0000) is recorded | O | JRU | 001 0000 | - | L1 | FS / LS / OS / SR | - |
| 14 | L1 | FS / LS / OS / SR | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_VERSION=001 0000) is recorded | O | JRU | 001 0000 | - | L1 | FS / LS / OS / SR | - |
| 15 | L1 | FS / LS / OS / SR | The driver presses "System version" | I | DMI | - | - | L1 | FS / LS / OS / SR | - |
| 16 | L1 | FS / LS / OS / SR | DMI shows the operated system version | O | DMI | - | Operated system version is 1.0 Every JRU message recorded contains the currently operated system version M_VERSION=001 0000 | L1 | FS / LS / OS / SR | - |
| 17 | L1 | FS / LS / OS / SR | One Balise Group message (N_TOTAL=1) containing packet 5 is received | I | BTM | 010 0000 | - | L1 | FS / LS / OS / SR | - |
| 18 | L1 | FS / LS / OS / SR | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6; M_VERSION=010 0000) is recorded | O | JRU | 010 0000 | - | L1 | FS / LS / OS / SR | - |
| 19 | L1 | FS / LS / OS / SR | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_VERSION=010 0000) is recorded | O | JRU | 010 0000 | - | L1 | FS / LS / OS / SR | - |
| 20 | L1 | FS / LS / OS / SR | The driver presses "System version" | I | DMI | - | - | L1 | FS / LS / OS / SR | - |
| 21 | L1 | FS / LS / OS / SR | DMI shows the operated system version | O | DMI | - | Operated system version is 2.0 Every JRU message recorded contains the currently operated system version M_VERSION=010 0000 | L1 | FS / LS / OS / SR | - |

| Step 3: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 010 0000 | Version 2.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | FINITE VALUE | - |
| N_TOTAL | 3 | FINITE VALUE | - |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BG 1 |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 41 | Level transition order |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | 32767 | - |
| M_LEVELTR | 3 | 2 | Level 1 |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | 0 | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | End of Information |

| Step 8: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | - |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | 2 | Transition to level 1 |

| Step 12: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 001 0000 | Version 1.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | FINITE VALUE | - |
| N_TOTAL | 3 | FINITE VALUE | - |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BG 1 |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 2 | System version order |
| Q_DIR | 2 | FINITE VALUE | Relevant direction for the train |
| L_PACKET | 13 | FINITE VALUE | - |
| M_VERSION | 7 | 001 0000 | Version 1.0 |
| NID_PACKET | 8 | 255 | End of Information |

| Step 17: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 010 0000 | Version 2.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | First balise |
| N_TOTAL | 3 | FINITE VALUE | N. of balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BG 1 |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 5 | Linking |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LINK | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION | 1 | FINITE VALUE | - |
| Q_LINKREACTION | 2 | FINITE VALUE | - |
| Q_LOCACC | 6 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_LINK(k) | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY (k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | End of Information |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ / 2/ 12 | FS/ OS/ SR/ LS |
| Radio communication session | ESTABLISHED | The TC does not terminate the communication session |
| ERTMS/ETCS level | 2 | L1 |
| Operated System Version | STORED | Version 2.0 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 5

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.17.2: Determination of the operated system version |  | 3170200 |
| Test Case of Feature | In case the engine passes the RBC/RBC border location with its maximum safe front end and no session is established with the accepting RBC, the system version control in relation to non-RBC constituents shall be again applied and the balise group orders shall be again considered. |  | 5 |
| Applicable Mode/Level Combinations | L2 / L3 : FS / LS / OS / SR / SH / TR / PT / NL / RV |  |  |
| Target of Test | The target of this test case is to check that: In case the engine passes the RBC/RBC border location with its max safe front end and no session is established with the accepting RBC, the system version control in relation to non-RBC constituents shall be again applied and the balise group orders shall be again considered. The on-board equipment shall operate with only one system version at a time. On receiving the order to operate system version from balise group, the on-board equipment shall immediately operate the system version given in the order. The function "System Version Management" is active. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.17.2.1 |  |
|  |  | Subset-026-3.17.2.2 |  |
|  |  | Subset-026-3.17.2.3 |  |
|  |  | Subset-026-3.17.2.5 |  |
|  |  | Subset-026-3.17.2.8 |  |
|  |  | Subset-026-3.17.2.8 e) |  |
|  |  | Subset-026-4.5.2.1 table - System Version Management |  |
|  |  | Subset-026-4.7.2 table - Output information Operated System Version |  |
|  |  | Subset-026-4.8.3.1.1 Trackside constituent System Version |  |
|  |  | Subset-026-4.8.4.2 Trackside constituent System Version |  |
|  |  | Subset-026-4.8.3.1.1 System Version order |  |
|  |  | Subset-026-4.8.4.2 System Version order |  |
|  |  | Subset-026-7.4.2.1 Packet Number 2: System Version order |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: Operated system version is the same as the RBC system version before the handover execution Operated system version is the same as the BG system version after passing RBC/RBC border. Operated system version is the same as ordered by trackside Check on the BTM that: Packet 131 RBC transition order is RECEIVED. Packet 2 System version order is RECEIVED. Check on the RTM that: Message 136 Train position report is SENT. Check on the JRU that: General message is RECORDED after operated system version is changed Reception of packet 131 is RECORDED Message 136 is RECORDED Reception of packet 2 is RECORDED |
| Constraints | No session is established with accepting RBC. Only one communication session can be handled. NOTE: Rule used to define M_VERSION column: For RBC and BTM steps the M_VERSION represents the version of the components and in case of JRU steps the M_VERSION values is the CURRENTLY OPERATED VERSION. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2/ 3/ 7/ 8/ 11/12/ 14 | FS/ OS/ SR/ SH/ TR/ PT/ NL/ LS/ RV |
| Radio communication session | ESTABLISHED | Session established with handing RBC. |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| RBC/RIU System Version | STORED | Version 2.0 |
| Operated System Version | STORED | Version 2.0 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | with handing RBC. |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | M_VERSION | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA. Indication with Euroradio Message "General Message" (NID_MESSAGE=24) containing packet 131 is received. | I | RTM | 010 0000 | The radio message from the Handing Over RBC contains the RBC transition border. D_RBCTR ≠ 0 | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; M_VERSION=010 0000) is recorded | O | JRU | 010 0000 | - | N/A | N/A | - |
| 3 | N/A | N/A | Train maximum safe front end location = DLRBG + D_RBCTR | I | INT | - | The maximum safe front end of the train passes the location of the RBC transition. | N/A | N/A | - |
| 4 | N/A | N/A | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted | O | RTM | - | Message transmitted to Handing Over RBC. | N/A | N/A | - |
| 5 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | 010 0000 | - | N/A | N/A | - |
| 6 | N/A | N/A | One Balise Group message (N_TOTAL=1) containing packet 131 is received. | I | BTM | 001 0000 | The balise group is placed at the RBC/RBC transition border. D_RBCTR = 32767 | N/A | N/A | - |
| 7 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6; M_VERSION=010 0000) is recorded | O | JRU | 010 0000 | - | N/A | N/A | - |
| 8 | N/A | N/A | Train min safe rear end location = LRBG + L_TRAIN + D_RBCTR | I | INT | - | The minimum rear end of the train passes the location of the RBC transition. | N/A | N/A | - |
| 9 | N/A | N/A | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted. | O | RTM | - | Message transmitted to Handing Over RBC. | N/A | N/A | - |
| 10 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10; M_VERSION=010 0000 ) is recorded | O | JRU | 010 0000 | - | N/A | N/A | - |
| 11 | N/A | N/A | The communication session with the Handing Over RBC is terminated and the safe connection is released. | - | - | - | USE_FT3050500.5 (termination of communication session ordered by RBC) | N/A | N/A | - |
| 12 | N/A | N/A | The Status symbol "Safe radio connection - Connection Up" is removed | O | DMI | - | ST03 is removed “No Connection”: absence of symbol. | N/A | N/A | - |
| 13 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit40=0>) | O | JRU | - | - | N/A | N/A | - |
| 14 | N/A | N/A | SA-CONNECT. Request is transmitted | O | RTM | - | - | N/A | N/A | - |
| 15 | N/A | N/A | SA-CONNECT. Confirm is not received | I | RTM | - | By any cause the communication session with Accepting RBC is not established. | N/A | N/A | - |
| 16 | N/A | N/A | One Balise Group message (N_TOTAL=1) containing packet 2 (M_VERSION=001 0000) is received | I | BTM | 010 0000 | - | N/A | N/A | - |
| 17 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6; M_VERSION=001 001) is recorded | O | JRU | 001 0000 | - | N/A | N/A | - |
| 18 | N/A | N/A | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_VERSION=001 0000) is recorded | O | JRU | 001 0000 | - | N/A | N/A | - |
| 19 | N/A | N/A | The driver presses "System version" | I | DMI | - | - | N/A | N/A | - |
| 20 | N/A | N/A | DMI shows the operated system version | O | DMI | - | Operated system version is 1.0 Every message recorded in JRU contains the currently operated system version M_VERSION=001 0000 | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General Message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 131 | RBC transition order |
| Q_DIR | 2 | FINITE VALUE | Valid for current train direction |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_RBCTR | 15 | FINITE VALUE | ≠ 0 |
| NID_C | 10 | FINITE VALUE | Valid for the train |
| NID_RBC | 14 | FINITE VALUE | Accepting RBC |
| NID_RADIO | 64 | FINITE VALUE | Accepting RBC |
| Q_SLEEPSESSION | 1 | 0 | Sleeping equipment don’t consider session management information (to avoid TIU influence) |

| Step 4: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 20 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | Max safe front end is considered (D_RBCTR - L_DOUBTUNDER) |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | - |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |

| Step 6: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 001 0000 | Version 1.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | First balise |
| N_TOTAL | 3 | FINITE VALUE | No. of balise |
| M_DUP | 2 | 00 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | Valid for the train |
| NID BG | 14 | BG1 | BG 1 |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 131 | RBC transition order |
| Q_DIR | 2 | FINITE VALUE | Valid for current train direction |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_RBCTR | 15 | 32767 | - |
| NID_C | 10 | FINITE VALUE | Valid for the train |
| NID_RBC | 14 | FINITE VALUE | Accepting RBC |
| NID_RADIO | 64 | FINITE VALUE | Accepting RBC |
| Q_SLEEPSESSION | 1 | 0 | Sleeping equipment don’t consider session management information (to avoid TIU influence) |
| NID_PACKET | 8 | 255 | End of Information |

| Step 6: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 001 0000 | Version 1.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 001 | Second balise |
| N_TOTAL | 3 | FINITE VALUE | - |
| M_DUP | 2 | 00 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | Valid for the train |
| NID BG | 14 | FINITE VALUE | BG 1 |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | End of Information |

| Step 9: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | BG 1 | - |
| D_LRBG | 15 | FINITE VALUE | Min safe rear end is considered (D_RBCTR + L_TRAIN + L_DOUBTOVER) |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | - |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |

| Step 16: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 010 0000 | Version 2.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | FINITE VALUE | - |
| N_TOTAL | 3 | FINITE VALUE | - |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BG 2 |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 2 | System version order |
| Q_DIR | 2 | FINITE VALUE | Relevant direction for the train |
| L_PACKET | 13 | FINITE VALUE | - |
| M_VERSION | 7 | 001 0000 | Version 1.0 |
| NID_PACKET | 8 | 255 | End of Information |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2/ 3/ 7/ 8/ 11/12/ 14 | FS/ OS/ SR/ SH/ TR/ PT/ NL/ LS/ RV |
| Radio communication session | NOT ESTABLISHED | With Accepting RBC |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Operated System Version | STORED | Version 1.0 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | No connection with Accepting RBC |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 6

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.17.2: Determination of the operated system version |  | 3170200 |
| Test Case of Feature | If a mismatch has been detected between the country or region identifier read from a balise/loop and the corresponding identifier for which a set of national values is used onboard, the on-board equipment shall consider the system version transmitted by balise/loop as the operated one. |  | 6 |
| Applicable Mode/Level Combinations | L1 : FS / LS / OS / SR / SH / TR / PT / NL / RV |  |  |
| Target of Test | The target of this test case is to check that: If a mismatch has been detected between the country or region identifier read from a balise/loop and the corresponding identifier for which a set of national values is used onboard, the on-board equipment shall operate the system version transmitted by balise/loop. The on-board equipment shall operate with only one system version at a time. The function "System Version Management" is active. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.17.2.1 |  |
|  |  | Subset-026-3.17.2.2 |  |
|  |  | Subset-026-3.17.2.6 |  |
|  |  | Subset-026-4.5.2.1 table - System Version Management |  |
|  |  | Subset-026-4.7.2 table - Output information Operated System Version |  |
|  |  | Subset-026-4.8.3.1.1 Trackside constituent System Version |  |
|  |  | Subset-026-4.8.4.2 Trackside constituent System Version |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: Operated system version is the same as the stored one before reading the BG Operated system version is the system version transmitted by balise after reading the BG Check on the BTM that: Packet 5 Linking is RECEIVED. Check on the JRU that: Reception of packet 5 is RECORDED General message is RECORDED after operated system version is changed |
| Constraints | The on-board equipment operates with one system version different to the system version transmitted by balise. NOTE: Rule used to define M_VERSION column: For RBC and BTM steps the M_VERSION represents the version of the components and in case of JRU steps the M_VERSION values is the CURRENTLY OPERATED VERSION. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2/ 3/ 6/ 7/ 8/ 11/12/ 14 | FS/ OS/ SR/ SH/ TR/ PT/ NL/ LS/ RV |
| National Values | STORED | NID_C (1) |
| ERTMS/ETCS level | 2 | L1 |
| Operated System Version | STORED | Version 2.0 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | M_VERSION | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The driver presses "System version" | I | DMI | - | - | N/A | N/A | - |
| 2 | N/A | N/A | DMI shows the operated system version | O | DMI | - | Operated system version is 2.0 Every JRU message recorded contains the currently operated system version M_VERSION=010 0000 | N/A | N/A | - |
| 3 | N/A | N/A | One Balise Group message (NID_C(2)) containing packet 5 is received | I | BTM | 001 0000 | NID_C(2) different from NID_C(1) stored | N/A | N/A | - |
| 4 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6; M_VERSION=001 0000) is recorded | O | JRU | 001 0000 | - | N/A | N/A | - |
| 5 | N/A | N/A | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_VERSION=001 0000) is recorded | O | JRU | 001 0000 | - | N/A | N/A | - |
| 6 | N/A | N/A | The driver presses "System version" | I | DMI | - | - | N/A | N/A | - |
| 7 | N/A | N/A | DMI shows the operated system version | O | DMI | - | Operated system version is 1.0 Every JRU message recorded contains the currently operated system version M_VERSION=001 0000 | N/A | N/A | - |

| Step 3: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 010 0000 | Version 2.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | FINITE VALUE | - |
| N_TOTAL | 3 | FINITE VALUE | - |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | NID_C(2) different from NID_C(1) stored |
| NID_BG | 14 | FINITE VALUE | BG 1 |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 5 | Linking |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LINK | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION | 1 | FINITE VALUE | - |
| Q_LINKREACTION | 2 | FINITE VALUE | - |
| Q_LOCACC | 6 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_LINK(k) | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY (k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | End of Information |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2/ 3/ 6/ 7/ 8/ 11/12/ 14 | FS/ OS/ SR/ SH/ TR/ PT/ NL/ LS/ RV |
| National Values | STORED | NID_C (2) |
| ERTMS/ETCS level | 2 | L1 |
| Operated System Version | STORED | Version 1.0 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 7

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.17.2: Determination of the operated system version |  | 3170200 |
| Test Case of Feature | If the on-board equipment does not support the system version transmitted by a non-RBC trackside constituent or the one specified in the balise group order, it shall consider the operated system version as unchanged. |  | 7 |
| Applicable Mode/Level Combinations | L1 : FS / LS / OS / SR / SH / TR / PT / NL / RV |  |  |
| Target of Test | The target of this test case is to check that: On-board equipment shall consider the operated system version as unchanged if the on-board equipment does not support the system version transmitted in the balise group order. The on-board equipment shall operate with only one system version at a time. The function "System Version Management" is active. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.17.2.1 |  |
|  |  | Subset-026-3.17.2.2 |  |
|  |  | Subset-026-3.17.2.7 |  |
|  |  | Subset-026-4.5.2.1 table - System Version Management |  |
|  |  | Subset-026-4.7.2 table - Output information Operated System Version |  |
|  |  | Subset-026-4.8.3.1.1 System Version order |  |
|  |  | Subset-026-4.8.4.2 System Version order |  |
|  |  | Subset-026-7.4.2.1 Packet Number 2: System Version order |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: Operated system version is the same as the stored one before reading the BG Operated system version remains unchanged after reading the BG Check on the BTM that: Packet 2 "System version order" is RECEIVED. Check on the JRU that: Reception of packet 2 is RECORDED A General Message ( M_VERSION changed) is NOT recorded |
| Constraints | The on-board equipment does not support the system version transmitted in the order by balise group order. The non-RBC constituent operates with same system version as the balise group order. NOTE: Rule used to define M_VERSION column: For RBC and BTM steps the M_VERSION represents the version of the components and in case of JRU steps the M_VERSION values is the CURRENTLY OPERATED VERSION. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2/ 3/ 6/ 7/ 8/ 11/12/ 14 | FS/ OS/ SR/ SH/ TR/ PT/ NL/ LS/ RV |
| ERTMS/ETCS level | 2 | L1 |
| Operated System Version | STORED | Version 2.0 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | M_VERSION | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The driver presses "System version" | I | DMI | - | - | N/A | N/A | - |
| 2 | N/A | N/A | DMI shows the operated system version | O | DMI | - | Operated system version is 2.0 Every message recorded in JRU contains the currently operated system version M_VERSION=010 0000 | N/A | N/A | - |
| 3 | N/A | N/A | One Balise Group message containing packet 2 (M_VERSION=100 0000) is received | I | BTM | 100 0000 | System version 4.0 ordered by the balise group version is not supported by the system. | N/A | N/A | - |
| 4 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6; M_VERSION=010 0000) is recorded | O | JRU | 010 0000 | - | N/A | N/A | - |
| 5 | N/A | N/A | The driver presses "System version" | I | DMI | - | - | N/A | N/A | - |
| 6 | N/A | N/A | DMI shows the operated system version | O | DMI | - | Operated system version remains unchanged 2.0 Every JRU message recorded contains the currently operated system version M_VERSION=010 0000 | N/A | N/A | - |

| Step 3: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 100 0000 | Version 4.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | FINITE VALUE | - |
| N_TOTAL | 3 | FINITE VALUE | - |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BG 1 |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 2 | System version order |
| Q_DIR | 2 | FINITE VALUE | Relevant direction for the train |
| L_PACKET | 13 | FINITE VALUE | - |
| M_VERSION | 7 | 100 0000 | Version 4.0 Version not supported |
| NID_PACKET | 8 | 255 | End of Information |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2/ 3/ 6/ 7/ 8/ 11/12/ 14 | FS/ OS/ SR/ SH/ TR/ PT/ NL/ LS/ RV |
| ERTMS/ETCS level | 2 | L1 |
| Operated System Version | STORED | Version 2.0 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 8

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.17.2: Determination of the operated system version |  | 3170200 |
| Test Case of Feature | The system version currently operated when the on-board equipment is switched off (i.e. enters No Power mode) shall be retained and re-used when powered on. |  | 8 |
| Applicable Mode/Level Combinations | L0 : SH / UN / SB / TR / NL LNTC : SH / SB / TR / NL / SN L1 / L2 / L3: FS / LS / OS / SR / SH / SB / TR / PT / NL / RV |  |  |
| Target of Test | The target of this test case is to check that: The system version operated when the on-board equipment is switched off shall be retained and re-used when powered on. The on-board equipment shall operate with only one system version at a time. The function "System Version Management" is active. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.17.2.1 |  |
|  |  | Subset-026-3.17.2.2 |  |
|  |  | Subset-026-3.17.2.9 |  |
|  |  | Subset-026-4.5.2.1 table - System Version Management |  |
|  |  | Subset-026-4.7.2 table - Output information Operated System Version |  |
|  |  | Subset-026-4.10.1.3 line Operated System Version |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: Operated system version is the same as the stored one before entering NP mode. Check on the JRU that: A general message indicating a change of operated System version changed is not recorded |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2/ 3/ 4/ 6/ 7/ 8/ 11/12/ 13/ 14 | FS/ OS/ SR/ SH/ SB/ UN/ TR/ PT/ NL/ LS/ SN/ RV |
| ERTMS/ETCS level | 0/ 1/ 2/ 3/ 4 | L0/ LNTC/ L1/ L2/ L3 |
| Operated System Version | STORED | Version 2.0 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0/ LNTC/ L1/ L2/ L3 | FS/ OS/ SR/ SH/ SB/ UN/ TR/ PT/ NL/ LS/ SN/ RV | The driver presses "System version" | I | DMI | - | L0/ LNTC/ L1/ L2/ L3 | FS/ OS/ SR/ SH/ SB/ UN/ TR/ PT/ NL/ LS/ SN/ RV | - |
| 2 | L0/ LNTC/ L1/ L2/ L3 | FS/ OS/ SR/ SH/ SB/ UN/ TR/ PT/ NL/ LS/ SN/ RV | DMI shows the operated system version | O | DMI | Operated system version is 2.0 Every message recorded in JRU contains the currently operated system version M_VERSION=010 0000 | L0/ LNTC/ L1/ L2/ L3 | FS/ OS/ SR/ SH/ SB/ UN/ TR/ PT/ NL/ LS/ SN/ RV | - |
| 3 | L0/ LNTC/ L1/ L2/ L3 | FS/ OS/ SR/ SH/ SB/ UN/ TR/ PT/ NL/ LS/ SN/ RV | The on-board switches to NP mode | - | - | USE_FT4060300.1 | L0/ LNTC/ L1/ L2/ L3 | NP | - |
| 4 | L0/ LNTC/ L1/ L2/ L3 | NP | The on-board switches to SB mode from NP mode. | - | - | USE_FT4060300.10 The system version shall be re-used when on-board equipment is powered on. | L0/ LNTC/ L1/ L2/ L3 | SB | - |
| 5 | L0/ LNTC/ L1/ L2/ L3 | SB | Desk is opened. | - | - | USE_FT5040300.24 | L0/ LNTC/ L1/ L2/ L3 | SB | - |
| 6 | L0/ LNTC/ L1/ L2/ L3 | SB | The driver presses "System version" | I | DMI | - | L0/ LNTC/ L1/ L2/ L3 | SB | - |
| 7 | L0/ LNTC/ L1/ L2/ L3 | SB | DMI shows the operated system version | O | DMI | Operated system version is 2.0 Every JRU message recorded contains the currently operated system version M_VERSION=010 0000 | L0/ LNTC/ L1/ L2/ L3 | SB | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| ERTMS/ETCS level | 0/ 1/ 2/ 3/ 4 | L0/ LNTC/ L1/ L2/ L3 |
| Operated System Version | STORED | Version 2.0 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 9

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.17.2: Determination of the operated system version |  | 3170200 |
| Test Case of Feature | Acceptance of System Version Order by modes and levels. |  | 9 |
| Applicable Mode/Level Combinations | L0 : SH/ UN/ PS/ SL/ SB/ TR/ NL LNTC: SH/ PS/ SL/ SB/ TR/ NL/ SN L1: FS/ LS/ OS/ SR/ SH/ PS/ SL/ SB/ TR/ PT/ NL/ RV L2: FS/ LS/ OS/ SR/ SH/ PS/ SL/ SB/ TR/ PT/ NL/ RV L3: FS/ LS/ OS/ SR/ SH/ PS/ SL/ SB/ TR/ PT/ NL/ RV |  |  |
| Target of Test | The target of this test case is to check that: The on-board equipment receives an order to change the Operating System version from one Balise group. The on-board equipment stores and accepts the information and operates immediately with the system version given in the order. Only data related with one system version can be contained in the message. The function "System Version Management" is active. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.17.2.1 |  |
|  |  | Subset-026-3.17.2.2 |  |
|  |  | Subset-026-3.17.2.5 |  |
|  |  | Subset-026-4.5.2.1 table - System Version Management |  |
|  |  | Subset-026-4.7.2 table - Output information Operated System Version |  |
|  |  | Subset-026-4.8.3.1.1 System Version order |  |
|  |  | Subset-026-4.8.4.2 System Version order |  |
|  |  | Subset-026-7.4.2.1 Packet Number 2: System Version order |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: Operating System version, same values as the one ordered by packet 2, is displayed. Check on the JRU: The balise message with packet 2 received by the on-board is RECORDED. New Operating system version is RECORDED. |
| Constraints | The on-board equipment operates with one system version different to System Version order. Packet 2 shall be set with a value for M_VERSION different from the currently operating one. NOTE: Rule used to define M_VERSION column: For RBC and BTM steps the M_VERSION represents the version of the components and in case of JRU steps the M_VERSION values is the CURRENTLY OPERATED VERSION. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2/ 3/ 4/ 5/ 6/ 7/ 8/ 11/ 12/ 13/ 14/ 15 | FS/ OS/ SR/ SH/ UN/ SL/ SB/ TR/ PT/ NL/ LS/ SN/ RV/ PS |
| ERTMS/ETCS level | 0/ 1/ 2/ 3 / 4 | L0/ LNTC/ L1/ L2 / L3 |
| Operated System Version | STORED | Version A |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | M_VERSION | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The driver presses "System version" | I | DMI | - | - | N/A | N/A | - |
| 2 | N/A | N/A | DMI shows the operated system version | O | DMI | - | Operated system version is A Every JRU message recorded contains the currently operating system version | N/A | N/A | - |
| 3 | N/A | N/A | One Balise Group message (N_TOTAL=1) containing packet 2 (M_VERSION=B) is received | I | BTM | A | M_VERSION contained in the order is different from the current operating one | N/A | N/A | - |
| 4 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6; M_VERSION=B) is recorded | O | JRU | B | - | N/A | N/A | - |
| 5 | N/A | N/A | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_VERSION=B) is recorded | O | JRU | B | - | N/A | N/A | - |
| 6 | N/A | N/A | The driver presses "System version" | I | DMI | - | - | N/A | N/A | - |
| 7 | N/A | N/A | DMI shows the operated system version | O | DMI | - | Check that Operated system version changed to M_VERSION value given in the order and is different from the initial one. Every JRU message recorded contains the currently operating system version | N/A | N/A | - |

| Step 3: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | A (Actual Operated System Version) |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | FINITE VALUE | - |
| N_TOTAL | 3 | FINITE VALUE | - |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BG 1 |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 2 | System version order |
| Q_DIR | 2 | FINITE VALUE | Relevant direction for the train |
| L_PACKET | 13 | FINITE VALUE | - |
| M_VERSION | 7 | FINITE VALUE | B (Different value from Operating System Version) |
| NID_PACKET | 8 | 255 | End of Information |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2/ 3/ 4/ 5/ 6/ 7/ 8/ 11/ 12/ 13/ 14/ 15 | FS/ OS/ SR/ SH/ UN/ SL/ SB/ TR/ PT/ NL/ LS/ SN/ RV/ PS |
| ERTMS/ETCS level | 0/ 1/ 2/ 3 / 4 | L0/ LNTC/ L1/ L2 / L3 |
| Operated System Version | STORED | Version B |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 10

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.17.2: Determination of the operated system version |  | 3170200 |
| Test Case of Feature | The system version currently operated when the on-board equipment loses the information (failure situation), the highest supported system version shall be used. |  | 10 |
| Applicable Mode/Level Combinations | L0 : SH / UN / SB / TR / NL LNTC : SH / SB / TR / NL / SN L1 / L2 / L3: FS / LS / OS / SR / SH / SB / TR / PT / NL / RV |  |  |
| Target of Test | The target of this test case is to check that: The system version operated when the on-board equipment loses the information (failure situation) shall be the highest supported system version. The on-board equipment shall operate with only one system version at a time. The function "System Version Management" is active. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.17.2.1 |  |
|  |  | Subset-026-3.17.2.2 |  |
|  |  | Subset-026-3.17.2.9.1 |  |
|  |  | Subset-026-4.5.2.1 table - System Version Management |  |
|  |  | Subset-026-4.7.2 table - Output information Operated System Version |  |
|  |  | Subset-026-4.10.1.3 line Operated System Version |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: Operated system version after on-board equipment loses the information (failure situation) is the highest supported system version. Check on the JRU that: A general message indicating a change of operated System version changed is recorded |
| Constraints | The on-board equipment operates with one system version different to the highest supported system version. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2/ 3/ 4/ 6/ 7/ 8/ 11/12/ 13/ 14 | FS/ OS/ SR/ SH/ SB/ UN/ TR/ PT/ NL/ LS/ SN/ RV |
| ERTMS/ETCS level | 0/ 1/ 2/ 3/ 4 | L0/ LNTC/ L1/ L2/ L3 |
| Operated System Version | STORED | Version 1.0 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0/ LNTC/ L1/ L2/ L3 | FS/ OS/ SR/ SH/ SB/ UN/ TR/ PT/ NL/ LS/ SN/ RV | The driver presses "System version" | I | DMI | - | L0/ LNTC/ L1/ L2/ L3 | FS/ OS/ SR/ SH/ SB/ UN/ TR/ PT/ NL/ LS/ SN/ RV | - |
| 2 | L0/ LNTC/ L1/ L2/ L3 | FS/ OS/ SR/ SH/ SB/ UN/ TR/ PT/ NL/ LS/ SN/ RV | DMI shows the operated system version | O | DMI | Operated system version is 1.0 Every message recorded in JRU contains the currently operated system version M_VERSION=001 0000 | L0/ LNTC/ L1/ L2/ L3 | FS/ OS/ SR/ SH/ SB/ UN/ TR/ PT/ NL/ LS/ SN/ RV | - |
| 3 | L0/ LNTC/ L1/ L2/ L3 | FS/ OS/ SR/ SH/ SB/ UN/ TR/ PT/ NL/ LS/ SN/ RV | The on-board switches to SF mode | - | - | USE_FT4060300.4 The system version shall be changed by the highest supported system version. | L0/ LNTC/ L1/ L2/ L3 | SF | - |
| 4 | L0/ LNTC/ L1/ L2/ L3 | SB | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=9) is recorded | O | JRU | - | L0/ LNTC/ L1/ L2/ L3 | SB | - |
| 5 | L0/ LNTC/ L1/ L2/ L3 | SF | The on-board switches to NP mode form SF mode | - | - | USE_FT4060300.1 | L0/ LNTC/ L1/ L2/ L3 | NP | - |
| 6 | L0/ LNTC/ L1/ L2/ L3 | NP | The on-board switches to SB mode from NP mode. | - | - | USE_FT4060300.10 The system version shall be re-used when on-board equipment is powered on. | L0/ LNTC/ L1/ L2/ L3 | SB | - |
| 7 | L0/ LNTC/ L1/ L2/ L3 | SB | Desk is opened. | - | - | USE_FT5040300.24 | L0/ LNTC/ L1/ L2/ L3 | SB | - |
| 8 | L0/ LNTC/ L1/ L2/ L3 | SB | The driver presses "System version" | I | DMI | - | L0/ LNTC/ L1/ L2/ L3 | SB | - |
| 9 | L0/ LNTC/ L1/ L2/ L3 | SB | DMI shows the operated system version | O | DMI | Operated system version is 2.0 (highest supported system version). Every JRU message recorded contains the currently operated system version M_VERSION=010 0000 | L0/ LNTC/ L1/ L2/ L3 | SB | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| ERTMS/ETCS level | 0/ 1/ 2/ 3/ 4 | L0/ LNTC/ L1/ L2/ L3 |
| Operated System Version | STORED | Version 2.0 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |