| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 5030100 (5.3.1: Scope and Purpose) Total: 1 Test case(s) |  |
| REF: | Subset-076-5-2-5030100 |
| ISSUE: | Version 3.2.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 2.9.0 31.03.2013 | All | First version/review | DLR |
| Version 2.9.1 30.08.2013 | All | review | DLR |
| Version 2.9.2 03.12.2013 | All | review | DLR |
| Version 2.9.3 10.01.2014 | All | review | DLR |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.2.0 31.03.2017 | All | Baseline 3 1st maintenance release version - Bug fixing | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.3.1: Scope and Purpose |  | 5030100 |
| Test Case of Feature | 5.3.1.3: Most "standard" transitions as defined within Chapter 4 "Modes and transitions", are not shown in the procedures. Nevertheless they possess the higher priority and would lead to exit the procedures immediately in most cases (e. g. cut off power of on-board equipment, isolation of on-board equipment). |  | 1 |
| Applicable Mode/Level Combinations | L0: SB LNTC: SB L1: SB L2: SB L3: SB |  |  |
| Target of Test | During the SoM procedure, if the driver isolates the ERTMS/ETCS on-board equipment, the procedure is exited. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-5.3.1.3 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Driver isolates the ERTMS/ETCS on-board equipment. |
| Constraints | The on-board equipment is in SoM procedure At start, V_TRAIN is 0, train is at standstill. Status of ERTMS/ETCS Level is VALID, there is a valid ERTMS/ETCS Level data stored on-board. The Desk is OPEN |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |
| Driver ID | VALID | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / LNTC / L1 / L2 / L3 | SB | The driver isolates the ERTMS/ETCS on-board equipment. | I | DMI | - | L0 / LNTC / L1 / L2 / L3 | IS | - |
| 2 | L0 / LNTC / L1 / L2 / L3 | IS | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0001 0010) is recorded | O | JRU | 0001 0010 = Isolation selected | L0 / LNTC / L1 / L2 / L3 | IS | - |
| 3 | L0 / LNTC / L1 / L2 / L3 | IS | ETCS isolated | O | TIU | - | L0 / LNTC / L1 / L2 / L3 | IS | - |
| 4 | L0 / LNTC / L1 / L2 / L3 | IS | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=10) is recorded | O | JRU | - | L0 / LNTC / L1 / L2 / L3 | IS | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 10 | IS |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 (UNCHANGED) |
| Driver ID | VALID | UNCHANGED |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |