| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 5100400 (5.10.4: Acknowledgement of the level transition) Total: 55 Test case(s) |  |
| REF: | Subset-076-5-2-5100400 |
| ISSUE: | Version 3.2.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 0.0.1 23.03.2012 | All | Creation | DLR |
| Version 2.9.0 05.10.2012 | All | First version/review for Baseline 3 | INECO |
| Version 2.9.1 01.08.2013 | All | Second version/Internal modification;TC_Editorv27; Correction of Requirements blank space | INECO |
| Version 2.9.2 11.11.2013 | All | Third version / UNISIG Review from 14.10.2013 included; TCEditor_v31tool used | INECO |
| Version 2.9.3 03.12.2013 | All | Fourth version/ TCs from 65 to 69 added new. | INECO |
| Version 2.9.4 03.01.2014 | All | Fifth version/ Internal version: Cross-reference checked; Old reference removed. | INECO |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.2.0 31.03.2017 | All | Baseline 3 1st maintenance release version - Bug fixing | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.4: Acknowledgement of the level transition |  | 5100400 |
| Test Case of Feature | The train enters in the acknowledgement window, and the driver is requested to acknowledge the transition from L0 to LNTC. The driver acknowledges before entering the LNTC area. |  | 1 |
| Applicable Mode/Level Combinations | L0 : UN |  |  |
| Target of Test | The target of this test case is to check that : The on-board equipment requests the driver to acknowledge the level transition L0/ LNTC through the DMI when the max safe front end of the train passes over the trackside defined location (defined in the level transition announcement with L_ACKLEVELTR). |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.7.2 table - Input information Ackn of level transition |  |
|  |  | Subset-026-5.10.4.1 |  |
|  |  | Subset-026-5.10.4.1 a) |  |
|  |  | Subset-026-5.10.4.4 |  |
|  |  | Subset-026-5.10.4.4 row L0 column L NTC |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The symbol LE01 (current level L0) is DISPLAYED. The symbol LE09 requesting the driver to acknowledge the Level transition when the max safe front end of the train reaches the location defined by L_ACKLEVELTR is DISPLAYED The driver acknowledges the Level transition on the DMI After driver acknowledgment, the symbol LE08 indicating the announcement of LNTC is DISPLAYED. Check on the JRU that: The displayed symbol LE09 is RECORDED. The driver acknowledgement of the level transition L0/LNTC is RECORDED. The displayed symbol LE08 is RECORDED. |
| Constraints | The driver acknowledges the level transition L0/LNTC inside the defined area (L_ACKLEVELTR: Length of the acknowledgement area in rear of the required level). |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 4 | UN |
| Level Transition Order | STORED | Transition to level NTC at further location with an acknowledgement window before the transition border |
| ERTMS/ETCS level | 0 | L0 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level 0" is displayed The Level symbol "Level NTC announcement" is displayed | O | DMI | DMI Object: Symbol LE01 DMI Object: Symbol LE08 (Except in SB mode) |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The Level symbol "Level NTC announcement" is removed | O | DMI | The max safe front end of the train enters the acknowledgement window (defined in the level transition announcement with L_ACKLEVELTR). Symbol LE08 | N/A | N/A | - |
| 2 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit08=0>) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The Level symbol "Acknowledge Level NTC announcement" is displayed | O | DMI | DMI Object: Symbol LE09 | N/A | N/A | - |
| 4 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit09=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | The driver acknowledges "Level NTC announcement" | I | DMI | The driver ACKNOWLEDGES the level transition | N/A | N/A | - |
| 6 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 1010) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | The Level symbol "Acknowledge Level NTC announcement" is removed | O | DMI | DMI Object: Symbol LE09 | N/A | N/A | - |
| 8 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit09=0>) is recorded | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | The Level symbol "Level NTC announcement" is displayed | O | DMI | DMI Object: Symbol LE08 | N/A | N/A | - |
| 10 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit08=1>) is recorded | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 4 | UN |
| Level Transition Order | STORED | Transition to level NTC |
| ERTMS/ETCS level | 0 | L0 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level NTC announcement" is displayed | O | DMI | DMI Object: Symbol LE08 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-Board equipment |  |  |
| Tested Feature | 5.10.4: Acknowledgement of the level transition |  | 5100400 |
| Test Case of Feature | The train has passed over the Level Transition border and the On-board equipment has switched from L0 to Level NTC upon receipt of the order. The driver acknowledges within the driver acknowledgement time [5 seconds] after the Level transition. |  | 2 |
| Applicable Mode/Level Combinations | LNTC: SN |  |  |
| Target of Test | The target of this test is to check that: The on-board does not apply the brakes if the driver acknowledges the Level Transition L0/LNTC within the driver acknowledgement time [5 seconds]after the Level Transition. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.7.2 table - Input information Ackn of level transition |  |
|  |  | Subset-026-5.10.4.1 |  |
|  |  | Subset-026-5.10.4.1 b) |  |
|  |  | Subset-026-5.10.4.4 row L0 column L NTC |  |
|  |  | Subset-026-5.10.4.2 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The symbol LE02 (current Level NTC) is DISPLAYED. The acknowledgement request symbol LE09 after level transition is DISPLAYED. After the driver acknowledgement the acknowledgement request symbol LE09 for Level Transition is REMOVED. Check on the TIU that : The service brake is NOT APPLIED. Check on the JRU that: The displayed symbol LE09 is RECORDED. The driver acknowledgement of the level transition L0/LNTC is RECORDED. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 13 | SN |
| ERTMS/ETCS level | 1 | LNTC |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Service brake not commanded | O | TIU | - |
| The Level symbol "Level NTC" is displayed | O | DMI | DMI Object: Symbol LE02 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The Level symbol "Acknowledge Level NTC announcement" is displayed | O | DMI | DMI Object: Symbol LE09 | N/A | N/A | - |
| 2 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit09=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The driver acknowledges "Level NTC announcement" | I | DMI | The driver acknowledges the Level Transition within the driver acknowledgement time [five seconds]after the Level Transition. | N/A | N/A | - |
| 4 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 1010) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | The Level symbol "Acknowledge Level NTC announcement" is removed | O | DMI | DMI Object: Symbol LE09 | N/A | N/A | - |
| 6 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit09=0>) is recorded | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 13 | SN |
| ERTMS/ETCS level | 1 | LNTC |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Service brake not commanded | O | TIU | - |
| The Level symbol "Level NTC" is displayed | O | DMI | DMI Object: Symbol LE02 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.4: Acknowledgement of the level transition |  | 5100400 |
| Test Case of Feature | The train has passed over the Level Transition border and the On-board equipment has switched from L0 to Level NTC, upon receipt of an order. The driver does not acknowledge within the driver acknowledgement time[5 seconds] after the Level transition. The service brake is applied. The driver acknowledges and the brakes are released. |  | 3 |
| Applicable Mode/Level Combinations | LNTC: SN |  |  |
| Target of Test | The target of this test is to check that: Since the driver does not acknowledge the Level Transition within the driver acknowledgement time [5 seconds]after the Level transition, Service Brake is APPLIED. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.14.1.2 |  |
|  |  | Subset-026-4.7.2 table - Input information Ackn of level transition |  |
|  |  | Subset-026-5.10.4.1 |  |
|  |  | Subset-026-5.10.4.1 b) |  |
|  |  | Subset-026-5.10.4.4 |  |
|  |  | Subset-026-5.10.4.4 row L0 column L NTC |  |
|  |  | Subset-026-5.10.4.2 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The symbol LE02 (current level NTC) is displayed The acknowledgement request symbol LE09 after level transition is DISPLAYED. Service brake application is DISPLAYED. After the driver acknowledges the acknowledgement, symbol LE09 is REMOVED.. Check on the TIU that : Service brake is APPLIED. Service brake is RELEASED when the driver acknowledges the Level Transition. Check on the JRU that: The displayed symbol LE09 is RECORDED. Service Brake intervention is RECORDED. The driver acknowledgement of the level transition L0/ LNTC is RECORDED. Service Brake release is RECORDED. |
| Constraints | If the Service Brake were not available for any reason, Emergency Brake shall be used. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 13 | SN |
| ERTMS/ETCS level | 1 | LNTC |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Service brake not commanded | O | TIU | - |
| The Level symbol "Level NTC" is displayed | O | DMI | DMI Object: Symbol LE02 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The Level symbol "Acknowledge Level NTC announcement" is displayed | O | DMI | DMI Object: Symbol LE09 | N/A | N/A | - |
| 2 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit09=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | NOT The driver acknowledges "Level NTC announcement" | I | DMI | The driver does not acknowledge the Level Transition within driver acknowledgement time[5 seconds] after the Level Transition. | N/A | N/A | - |
| 4 | N/A | N/A | Service brake commanded | O | TIU | - | N/A | N/A | - |
| 5 | N/A | N/A | The Status symbol "Service Brake intervention" is displayed | O | DMI | - | N/A | N/A | - |
| 6 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | The driver acknowledges "Level NTC announcement" | I | DMI | - | N/A | N/A | - |
| 8 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 1010) is recorded | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | Service brake not commanded | O | TIU | - | N/A | N/A | - |
| 10 | N/A | N/A | The Status symbol "Service Brake intervention" is removed | O | DMI | - | N/A | N/A | - |
| 11 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=0) is recorded | O | JRU | - | N/A | N/A | - |
| 12 | N/A | N/A | The Level symbol "Acknowledge Level NTC announcement" is removed | O | DMI | DMI Object: Symbol LE09 | N/A | N/A | - |
| 13 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit09=0>) is recorded | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 13 | SN |
| ERTMS/ETCS level | 1 | LNTC |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Service brake not commanded | O | TIU | - |
| The Level symbol "Level NTC" is displayed | O | DMI | DMI Object: Symbol LE02 The acknowledgement request for Level Transition, symbol LE09 is REMOVED. |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.4: Acknowledgement of the level transition |  | 5100400 |
| Test Case of Feature | The train enters in the acknowledgement window and the driver is requested to acknowledge the transition from L1 to L0. The driver acknowledges before entering the L0 area. |  | 4 |
| Applicable Mode/Level Combinations | L1 : FS / LS / OS / SR / SB / TR |  |  |
| Target of Test | The target of this test case is to check that : The on-board equipment requests the driver to acknowledge the level transition L1/ L0 through the DMI when the max safe front end of the train passes over the location in rear of the level transition border (defined in the level transition announcement with L_ACKLEVELTR). |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.7.2 table - Input information Ackn of level transition |  |
|  |  | Subset-026-5.10.4.1 |  |
|  |  | Subset-026-5.10.4.1 a) |  |
|  |  | Subset-026-5.10.4.4 |  |
|  |  | Subset-026-5.10.4.4 row L1 column L0 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The symbol LE03 (current level L1) is DISPLAYED. The symbol LE07 requesting the driver to acknowledge the Level transition when the max safe front end of the train reaches the location defined by L_ACKLEVELTR is DISPLAYED The driver acknowledges the Level transition on the DMI. After driver acknowledgment, the symbol LE06 indicating the announcement to L0 is DISPLAYED Check on the JRU that: Displayed symbols LE06 and LE07 are RECORDED. Driver’s acknowledgement of the level transition L1/L0 is RECORDED. |
| Constraints | The driver acknowledges the level transition L1/L0 inside the defined area (L_ACKLEVELTR: Length of the acknowledgement area in rear of the required level). |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2/ 6 / 7/ 12 | FS / OS / SR / SB / TR / LS |
| Level Transition Order | STORED | Level transition to L0 at further location with an acknowledgement window before the transition border |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level 1" is displayed The Level symbol "Level 0 announcement" is displayed | O | DMI | DMI Object: Symbol LE03 DMI Object: Symbol LE06 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The Level symbol "Level 0 announcement" is removed | O | DMI | The max safe front end of the train enters the acknowledgement window (defined in the level transition announcement with L_ACKLEVELTR). The symbol LE06 is REMOVED. | N/A | N/A | - |
| 2 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit06=0>) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The Level symbol "Acknowledge Level 0 announcement" is displayed | O | DMI | DMI Object: Symbol LE07 | N/A | N/A | - |
| 4 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit07=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | The driver acknowledges "Level 0 announcement" | I | DMI | The driver ACKNOWLEDGES the level transition | N/A | N/A | - |
| 6 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 0110) is recorded | O | JRU | The driver acknowledgement is RECORDED. | N/A | N/A | - |
| 7 | N/A | N/A | The Level symbol "Acknowledge Level 0 announcement" is removed | O | DMI | DMI Object: Symbol LE07 | N/A | N/A | - |
| 8 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit07=0>) is recorded | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | The Level symbol "Level 0 announcement" is displayed | O | DMI | DMI Object: Symbol LE06 | N/A | N/A | - |
| 10 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit06=1>) is recorded | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 7 / 12 | FS / OS / SR / SB / TR / LS |
| Level Transition Order | STORED | Level transition to L0 |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level 0 announcement" is displayed | O | DMI | DMI Object: Symbol LE06 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 5

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.4: Acknowledgement of the level transition |  | 5100400 |
| Test Case of Feature | The train enters in the acknowledgement window, and the driver is requested to acknowledge the transition from L1 to L0. The driver does not acknowledge before entering the L0 area. |  | 5 |
| Applicable Mode/Level Combinations | L1 : FS / LS / OS / SR / SB / TR |  |  |
| Target of Test | The target of this test case is to check that : The on-board equipment requests the driver to acknowledge the level transition L1/ L0 through the DMI when the max safe front end of the train passes over the location (defined in the level transition announcement with L_ACKLEVELTR). |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.7.2 table - Input information Ackn of level transition |  |
|  |  | Subset-026-5.10.4.1 |  |
|  |  | Subset-026-5.10.4.1 a) |  |
|  |  | Subset-026-5.10.4.4 |  |
|  |  | Subset-026-5.10.4.4 row L1 column L0 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The symbol LE07 requesting the driver to acknowledge the Level transition when the max safe front end of the train reaches the location defined by L_ACKLEVELTR is DISPLAYED The driver does not acknowledge the Level transition before entering the L0 area. Check on the JRU that: The displayed symbol LE07 requesting the acknowledgement is RECORDED |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2/ 6 / 7 / 12 | FS / OS / SR / SB / TR / LS |
| Level Transition Order | STORED | Level transition to L0 at further location with an acknowledgement window before the transition border |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level 1" is displayed The Level symbol "Level 0 announcement" is displayed | O | DMI | DMI Object: Symbol LE03 DMI Object: Symbol LE06 (Except in SB mode) |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The Level symbol "Level 0 announcement" is removed | O | DMI | The max safe front end of the train enters the acknowledgement window (defined in the level transition announcement with L_ACKLEVELTR). The symbol LE06 is REMOVED. | N/A | N/A | - |
| 2 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit06=0>) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The Level symbol "Acknowledge Level 0 announcement" is displayed | O | DMI | DMI Object: Symbol LE07 | N/A | N/A | - |
| 4 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit07=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | NOT The driver acknowledges "Level 0 announcement" | I | DMI | The driver does not acknowledge before entering the L0 area. | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 7 / 12 | FS / OS / SR / SB / TR / LS |
| Level Transition Order | STORED | Transition to level 0 |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Acknowledge Level 0 announcement" is displayed | O | DMI | DMI Object: Symbol LE07 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 6

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-Board equipment |  |  |
| Tested Feature | 5.10.4: Acknowledgement of the level transition |  | 5100400 |
| Test Case of Feature | The train has passed over the Level Transition border and the On-board equipment has switched from L1 to Level 0, upon receipt of an order . The driver acknowledges within the driver acknowledgement time[5 seconds] after the Level transition. |  | 6 |
| Applicable Mode/Level Combinations | L0: UN / SB |  |  |
| Target of Test | The target of this test is to check that: The on-board does not apply the brakes if the driver acknowledges the Level Transition L1 / L0 within the driver acknowledgement time [5seconds] after the Level Transition. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.7.2 table - Input information Ackn of level transition |  |
|  |  | Subset-026-5.10.4.1 |  |
|  |  | Subset-026-5.10.4.1 b) |  |
|  |  | Subset-026-5.10.4.4 row L1 column L0 |  |
|  |  | Subset-026-5.10.4.4 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The acknowledgement request symbol LE07 is DISPLAYED. After the driver acknowledgement, the acknowledgement request symbol LE07 for Level Transition is REMOVED. Check on the TIU that : The service brake is NOT APPLIED. Check on the JRU that: The displayed symbol LE07 is RECORDED. Driver´s acknowledgement of the level transition L1/L0 is RECORDED. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 4 | UN |
| ERTMS/ETCS level | 0 | L0 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Service brake not commanded | O | TIU | - |
| The Level symbol "Level 0" is displayed | O | DMI | DMI Object: Symbol LE01 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The Level symbol "Acknowledge Level 0 announcement" is displayed | O | DMI | DMI Object: Symbol LE07 | N/A | N/A | - |
| 2 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit07=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The driver acknowledges "Level 0 announcement" | I | DMI | The driver acknowledges the Level Transition within the driver acknowledgement time [5 seconds] after the Level Transition. | N/A | N/A | - |
| 4 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 0110) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | The Level symbol "Acknowledge Level 0 announcement" is removed | O | DMI | DMI Object: Symbol LE07 | N/A | N/A | - |
| 6 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit07=0>) is recorded | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 4 | UN |
| ERTMS/ETCS level | 0 | L0 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level 0" is displayed | O | DMI | DMI Object: Symbol LE01 The acknowledgement request for Level Transition, symbol LE07 is REMOVED. |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 7

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.4: Acknowledgement of the level transition |  | 5100400 |
| Test Case of Feature | The train has passed over the Level Transition border and the On-board equipment has switched from L1 to Level 0, upon receipt of an order. The driver does not acknowledge within the driver acknowledgement time[5 seconds] after the Level transition. The service brake is applied. The driver acknowledges and the brakes are released. |  | 7 |
| Applicable Mode/Level Combinations | L0: UN / SB |  |  |
| Target of Test | The target of this test is to check that: Since the driver does not acknowledge the Level Transition within the driver acknowledgement time[five seconds] after the Level transition, Service Brake is APPLIED. Service Brake is RELEASED when the driver acknowledges the Level Transition. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.14.1.2 |  |
|  |  | Subset-026-4.7.2 table - Input information Ackn of level transition |  |
|  |  | Subset-026-5.10.4.1 |  |
|  |  | Subset-026-5.10.4.1 b) |  |
|  |  | Subset-026-5.10.4.2 |  |
|  |  | Subset-026-5.10.4.4 |  |
|  |  | Subset-026-5.10.4.4 row L1 column L0 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The symbol LE01 (current level L0) is displayed The acknowledgement request symbol LE07 is DISPLAYED.. Service brake application is DISPLAYED. After the driver acknowledges, the acknowledgement request symbol LE07 is REMOVED. Check on the TIU that : Service brake is APPLIED. Service brake is RELEASED when the driver acknowledges the Level Transition. Check on the JRU that: The displayed symbol LE07 is RECORDED Service Brake intervention is RECORDED. Driver´s acknowledgement of the level transition L1/ L0 is RECORDED. Service Brake release is RECORDED. |
| Constraints | If the Service Brake were not available for any reason, Emergency Brake shall be used. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 4 / 6 | UN / SB |
| ERTMS/ETCS level | 0 | L0 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Service brake not commanded | O | TIU | - |
| The Level symbol "Level 0" is displayed | O | DMI | DMI Object: Symbol LE01 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The Level symbol "Acknowledge Level 0 announcement" is displayed | O | DMI | DMI Object: Symbol LE07 | N/A | N/A | - |
| 2 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit07=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | NOT The driver acknowledges "Level 0 announcement" | I | DMI | The driver does not acknowledge the Level Transition within the driver acknowledgement time [5 seconds] after the Level Transition. | N/A | N/A | - |
| 4 | N/A | N/A | Service brake commanded | O | TIU | - | N/A | N/A | - |
| 5 | N/A | N/A | The Status symbol "Service Brake intervention" is displayed | O | DMI | - | N/A | N/A | - |
| 6 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | The driver acknowledges "Level 0 announcement" | I | DMI | - | N/A | N/A | - |
| 8 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 0110) is recorded | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | Service brake not commanded | O | TIU | - | N/A | N/A | - |
| 10 | N/A | N/A | The Status symbol "Service Brake intervention" is removed | O | DMI | - | N/A | N/A | - |
| 11 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=0) is recorded | O | JRU | - | N/A | N/A | - |
| 12 | N/A | N/A | The Level symbol "Acknowledge Level 0 announcement" is removed | O | DMI | DMI Object: Symbol LE07 | N/A | N/A | - |
| 13 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit07=0>) is recorded | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 4 / 6 | UN / SB |
| ERTMS/ETCS level | 0 | L0 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Service brake not commanded | O | TIU | - |
| The Level symbol "Level 0" is displayed | O | DMI | DMI Object: Symbol LE01 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 8

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.4: Acknowledgement of the level transition |  | 5100400 |
| Test Case of Feature | The train enters in the acknowledgement window, and the driver is requested to acknowledge the transition from L1 to LNTC. The driver acknowledges before entering the LNTC area. |  | 8 |
| Applicable Mode/Level Combinations | L1 : FS/ LS/ OS/ SR/ SB/ TR |  |  |
| Target of Test | The target of this test case is to check that : The on-board equipment requests the driver to acknowledge the level transition L1/ LNTC through the DMI when the max safe front end of the train passes over the location (defined in the level transition announcement with L_ACKLEVELTR). |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.7.2 table - Input information Ackn of level transition |  |
|  |  | Subset-026-5.10.4.1 |  |
|  |  | Subset-026-5.10.4.1 a) |  |
|  |  | Subset-026-5.10.4.4 |  |
|  |  | Subset-026-5.10.4.4 row L1 column L NTC |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The symbol LE03 (current level L1) is DISPLAYED. The symbol LE09 requesting the driver to acknowledge the Level transition when the max safe front end of the train reaches the location defined by L_ACKLEVELTR. is DISPLAYED. The driver acknowledges the Level transition on the DMI After driver acknowledgment, the symbol LE08 indicating the announcement of level LNTC is DISPLAYED Check on the JRU that: The symbol LE09 is RECORDED. The driver´s acknowledgement of the level transition L1/LNTC is RECORDED. The symbol LE08 is RECORDED. |
| Constraints | The driver acknowledges the level transition L1/LNTC inside the defined area (L_ACKLEVELTR: Length of the acknowledgement area in rear of the required level). |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 7 / 12 | FS / OS / SR / SB / TR / LS |
| Level Transition Order | STORED | Level transition to LNTC at further location with an acknowledgement window before the transition border |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level 1" is displayed The Level symbol "Level NTC announcement" is displayed | O | DMI | DMI Object: Symbol LE03 DMI Object: Symbol LE08 (Except in SB mode) |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The Level symbol "Level NTC announcement" is removed | O | DMI | The max safe front end of the train enters the acknowledgement window (defined in the level transition announcement with L_ACKLEVELTR). Symbol LE08 is REMOVED | N/A | N/A | - |
| 2 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit08=0>) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The Level symbol "Acknowledge Level NTC announcement" is displayed | O | DMI | DMI Object: Symbol LE09 | N/A | N/A | - |
| 4 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit09=1>) is recorded | O | JRU | The displayed symbol LE09 is RECORDED. | N/A | N/A | - |
| 5 | N/A | N/A | The driver acknowledges "Level NTC announcement" | I | DMI | - | N/A | N/A | - |
| 6 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 1010) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | The Level symbol "Acknowledge Level NTC announcement" is removed | O | DMI | - | N/A | N/A | - |
| 8 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit09=0>) is recorded | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | The Level symbol "Level NTC announcement" is displayed | O | DMI | DMI Object: Symbol LE08 | N/A | N/A | - |
| 10 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit08=1>) is recorded | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 7 / 12 | FS / OS / SR / SB / TR / LS |
| Level Transition Order | STORED | Transition to LNTC |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level NTC announcement" is displayed | O | DMI | DMI Object: Symbol LE08 The acknowledgement request for the level transition, symbol LE09 is REMOVED. |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 9

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-Board equipment |  |  |
| Tested Feature | 5.10.4: Acknowledgement of the level transition |  | 5100400 |
| Test Case of Feature | The train has passed over the Level Transition border and the On-board equipment has switched from L1 to Level NTC, upon receipt of an order. The driver acknowledges within the driver acknowledgement time[5 seconds] after the Level transition. |  | 9 |
| Applicable Mode/Level Combinations | LNTC: SN |  |  |
| Target of Test | The target of this test is to check that: The on-board does not apply the brakes if the driver acknowledges the Level Transition L1/LNTC within the driver acknowledgement time[five seconds] after the Level Transition. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.7.2 table - Input information Ackn of level transition |  |
|  |  | Subset-026-5.10.4.1 |  |
|  |  | Subset-026-5.10.4.1 b) |  |
|  |  | Subset-026-5.10.4.4 |  |
|  |  | Subset-026-5.10.4.4 row L1 column L NTC |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The acknowledgement request symbol LE09 is DISPLAYED. After the driver acknowledgement the acknowledgement request for Level Transition, symbol LE09 is REMOVED. Check on the TIU that : The service brake is NOT APPLIED. Check on the JRU that: The displayed symbol LE09 is RECORDED. The driver acknowledgement of the level transition L1/LNTC is RECORDED. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 13 | SN |
| ERTMS/ETCS level | 1 | LNTC |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Service brake not commanded | O | TIU | - |
| The Level symbol "Level NTC" is displayed | O | DMI | DMI Object: Symbol LE02 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The Level symbol "Acknowledge Level NTC announcement" is displayed | O | DMI | DMI Object: Symbol LE09 | N/A | N/A | - |
| 2 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit09=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The driver acknowledges "Level NTC announcement" | I | DMI | The driver acknowledges the Level Transition within the driver acknowledgement time[5seconds] after the Level Transition. | N/A | N/A | - |
| 4 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 1010) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | The Level symbol "Acknowledge Level NTC announcement" is removed | O | DMI | DMI Object: Symbol LE09 | N/A | N/A | - |
| 6 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit09=0>) is recorded | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 13 | SN |
| ERTMS/ETCS level | 1 | LNTC |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level NTC" is displayed | O | DMI | DMI Object: Symbol LE02 The acknowledgement request for Level Transition, symbol LE09 is REMOVED |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 10

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.4: Acknowledgement of the level transition |  | 5100400 |
| Test Case of Feature | The train has passed over the Level Transition border and the On-board equipment has switched from L1 to Level NTC, upon receipt of an order. The driver does not acknowledge within the driver acknowledgement time[5 seconds] after the Level transition. The service brake is applied. The driver acknowledges and the brakes are released. |  | 10 |
| Applicable Mode/Level Combinations | LNTC: SN |  |  |
| Target of Test | The target of this test is to check that: Since the driver does not acknowledge the Level Transition within the driver acknowledgement time [5 seconds] after the Level transition, Service Brake is APPLIED. Service Brake is RELEASED when the driver acknowledges the Level Transition. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.14.1.2 |  |
|  |  | Subset-026-4.7.2 table - Input information Ackn of level transition |  |
|  |  | Subset-026-5.10.4.1 |  |
|  |  | Subset-026-5.10.4.1 b) |  |
|  |  | Subset-026-5.10.4.2 |  |
|  |  | Subset-026-5.10.4.4 |  |
|  |  | Subset-026-5.10.4.4 row L1 column L NTC |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The symbol LE02 (current level NTC) is displayed The acknowledgement request after level transition, symbol LE09 is DISPLAYED. Service brake application is DISPLAYED. After the driver acknowledges, the acknowledgement, symbol LE09 is REMOVED. Check on the TIU that : Service brake is APPLIED. Service brake is RELEASED when the driver acknowledges the Level Transition. Check on the JRU that: The displayed symbol LE09 is RECORDED Service Brake intervention is RECORDED. The driver acknowledgement of the level transition L1/ LNTC is RECORDED. Service Brake release is RECORDED. |
| Constraints | If the Service Brake were not available for any reason, Emergency Brake shall be used. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 13 | SN |
| ERTMS/ETCS level | 1 | LNTC |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Service brake not commanded | O | TIU | - |
| The Level symbol "Level NTC" is displayed | O | DMI | DMI Object: Symbol LE02 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The Level symbol "Acknowledge Level NTC announcement" is displayed | O | DMI | DMI Object: Symbol LE09 | N/A | N/A | - |
| 2 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit09=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | NOT The driver acknowledges "Level NTC announcement" | I | DMI | The driver does not acknowledge the Level Transition within the driver acknowledgement time[5 seconds] after the Level Transition. | N/A | N/A | - |
| 4 | N/A | N/A | Service brake commanded | O | TIU | - | N/A | N/A | - |
| 5 | N/A | N/A | The Status symbol "Service Brake intervention" is displayed | O | DMI | - | N/A | N/A | - |
| 6 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | The driver acknowledges "Level NTC announcement" | I | DMI | - | N/A | N/A | - |
| 8 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 1010) is recorded | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | Service brake not commanded | O | TIU | - | N/A | N/A | - |
| 10 | N/A | N/A | The Status symbol "Service Brake intervention" is removed | O | DMI | - | N/A | N/A | - |
| 11 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=0) is recorded | O | JRU | - | N/A | N/A | - |
| 12 | N/A | N/A | The Level symbol "Acknowledge Level NTC announcement" is removed | O | DMI | DMI Object: Symbol LE09 | N/A | N/A | - |
| 13 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit09=0>) is recorded | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 13 | SN |
| ERTMS/ETCS level | 1 | LNTC |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Service brake not commanded | O | TIU | - |
| The Level symbol "Level NTC" is displayed | O | DMI | DMI Object: Symbol LE02 The acknowledgement request for Level Transition, symbol LE09 is REMOVED. |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 11

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.4: Acknowledgement of the level transition |  | 5100400 |
| Test Case of Feature | The train enters in the acknowledgement window, and the driver is requested to acknowledge the transition from L2 to L0. The driver acknowledges before entering the L0 area. |  | 11 |
| Applicable Mode/Level Combinations | L2: FS / LS / OS / SR / SB / TR |  |  |
| Target of Test | The target of this test case is to check that : The on-board equipment requests the driver to acknowledge the level transition L2/ L0 through the DMI when the max safe front end of the train passes over the location (defined in the level transition announcement with L_ACKLEVELTR). |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.7.2 table - Input information Ackn of level transition |  |
|  |  | Subset-026-5.10.4.1 |  |
|  |  | Subset-026-5.10.4.1 a) |  |
|  |  | Subset-026-5.10.4.4 |  |
|  |  | Subset-026-5.10.4.4 row L2 column L0 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The symbol LE04 (current level L2) is DISPLAYED. The symbol LE07 requesting the driver to acknowledge the Level transition when the max safe front end of the train reaches the location defined by L_ACKLEVELTR is DISPLAYED. The driver acknowledges the Level transition on the DMI After driver acknowledgment, the symbol LE06 indicating the announcement to L0 is DISPLAYED. Check on the JRU that: Displayed symbol LE07 is RECORDED. The driver acknowledgement of the level transition L2/L0 is RECORDED Displayed symbol LE06 is RECORDED. |
| Constraints | The driver acknowledges the level transition L2/L0 inside the defined area (L_ACKLEVELTR: Length of the acknowledgement area in rear of the required level). |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 7 / 12 | FS / OS / SR / SB / TR / LS |
| Level Transition Order | STORED | Level transition to L0 at further location with an acknowledgement window before the transition border |
| ERTMS/ETCS level | 3 | L2 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level 2" is displayed The Level symbol "Level 0 announcement" is displayed | O | DMI | DMI Object: Symbol LE04 DMI Object: Symbol LE06 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The Level symbol "Level 0 announcement" is removed | O | DMI | The max safe front end of the train enters the acknowledgement window (defined in the level transition announcement with L_ACKLEVELTR). The symbol LE06 is REMOVED. | N/A | N/A | - |
| 2 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit06=0>) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The Level symbol "Acknowledge Level 0 announcement" is displayed | O | DMI | DMI Object: Symbol LE07 | N/A | N/A | - |
| 4 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit07=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | The driver acknowledges "Level 0 announcement" | I | DMI | - | N/A | N/A | - |
| 6 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 0110) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | The Level symbol "Acknowledge Level 0 announcement" is removed | O | DMI | DMI Object: Symbol LE07 | N/A | N/A | - |
| 8 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit07=0>) is recorded | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | The Level symbol "Level 0 announcement" is displayed | O | DMI | DMI Object: Symbol LE06 | N/A | N/A | - |
| 10 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit06=1>) is recorded | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 7 / 12 | FS / OS / SR / SB / TR / LS |
| Level Transition Order | STORED | Transition to L0 |
| ERTMS/ETCS level | 3 | L2 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level 0 announcement" is displayed | O | DMI | DMI Object: Symbol LE06 The acknowledgement request for the level transition, symbol LE07 is REMOVED. |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 12

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.4: Acknowledgement of the level transition |  | 5100400 |
| Test Case of Feature | The train enters in the acknowledgement window, and the driver is requested to acknowledge the transition from L2 to L0. The driver does not acknowledge before entering the L0 area. |  | 12 |
| Applicable Mode/Level Combinations | L2 : FS / LS / OS / SR /SB / TR |  |  |
| Target of Test | The target of this test case is to check that : The on-board equipment requests the driver to acknowledge the level transition L2/ L0 through the DMI when the max safe front end of the train passes over the location (defined in the level transition announcement with L_ACKLEVELTR). |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.7.2 table - Input information Ackn of level transition |  |
|  |  | Subset-026-5.10.4.1 |  |
|  |  | Subset-026-5.10.4.1 a) |  |
|  |  | Subset-026-5.10.4.4 |  |
|  |  | Subset-026-5.10.4.4 row L2 column L0 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The symbol LE04 (current level L2) is DISPLAYED The symbol LE07 requesting the driver to acknowledge the Level transition when the max safe front end of the train reaches the location defined by L_ACKLEVELTR. is DISPLAYED The driver does not acknowledge the Level transition before entering the L0 area. Check on the JRU that: The displayed symbol LE07 requesting the acknowledgement is RECORDED |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 7 / 12 | FS / OS / SR / SB / TR / LS |
| Level Transition Order | STORED | Level transition to L0 at further location with an acknowledgement window before the transition border |
| ERTMS/ETCS level | 3 | L2 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level 2" is displayed The Level symbol "Level 0 announcement" is displayed | O | DMI | DMI Object: Symbol LE04 DMI Object: Symbol LE06 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The Level symbol "Level 0 announcement" is removed | O | DMI | The max safe front end of the train enters the acknowledgement window (defined in the level transition announcement with L_ACKLEVELTR). The symbol LE06 is REMOVED. | N/A | N/A | - |
| 2 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit06=0>) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The Level symbol "Acknowledge Level 0 announcement" is displayed | O | DMI | DMI Object: Symbol LE07 | N/A | N/A | - |
| 4 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit07=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | NOT The driver acknowledges "Level 0 announcement" | I | DMI | The driver does not acknowledge before entering the L0 area. | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 7 / 12 | FS / OS / SR / SB / TR / LS |
| Level Transition Order | STORED | Transition to L0 |
| ERTMS/ETCS level | 3 | L2 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Acknowledge Level 0 announcement" is displayed | O | DMI | DMI Object: Symbol LE07 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 13

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.4: Acknowledgement of the level transition |  | 5100400 |
| Test Case of Feature | The train has passed over the Level Transition border and the on-board equipment has switched from L2 to Level 0, upon of an order. The driver acknowledges within the driver acknowledgement time[5 seconds] after the Level transition. |  | 13 |
| Applicable Mode/Level Combinations | L0: UN / SB |  |  |
| Target of Test | The target of this test is to check that: The on-board does not apply the service brake if the driver acknowledges the Level Transition within the driver acknowledgement time [5 seconds] after the Level Transition |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.7.2 table - Input information Ackn of level transition |  |
|  |  | Subset-026-5.10.4.1 |  |
|  |  | Subset-026-5.10.4.1 b) |  |
|  |  | Subset-026-5.10.4.4 |  |
|  |  | Subset-026-5.10.4.4 row L2 column L0 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The symbol LE01 (current Level 0) is DISPLAYED. The acknowledgement request symbol LE07 after level transition is DISPLAYED. After the driver acknowledges, the acknowledgement request for Level Transition symbol LE07 is REMOVED. Check on the TIU that : The service brake is NOT APPLIED. Check on the JRU that: The displayed symbol LE07 is RECORDED. Driver acknowledgement of the level transition L2/L0 is RECORDED. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 4 / 6 | UN / SB |
| ERTMS/ETCS level | 0 | L0 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Service brake not commanded | O | TIU | - |
| The Level symbol "Level 0" is displayed | O | DMI | DMI Object: Symbol LE01 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The Level symbol "Acknowledge Level 0 announcement" is displayed | O | DMI | DMI Object: Symbol LE07 | N/A | N/A | - |
| 2 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit07=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The driver acknowledges "Level 0 announcement" | I | DMI | The driver acknowledges the Level Transition within the driver acknowledgement time[5 seconds] after the Level Transition | N/A | N/A | - |
| 4 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 0110) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | The Level symbol "Acknowledge Level 0 announcement" is removed | O | DMI | DMI Object: Symbol LE07 | N/A | N/A | - |
| 6 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit07=0>) is recorded | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 4 / 6 | UN / SB |
| ERTMS/ETCS level | 0 | L0 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level 0" is displayed | O | DMI | DMI Object: Symbol LE01 The acknowledgement request for Level Transition, symbol LE07 is REMOVED |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 14

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.4: Acknowledgement of the level transition |  | 5100400 |
| Test Case of Feature | The train has passed over the Level Transition border and the On-board equipment has switched from L2 to Level 0, upon receipt of an order. The driver does not acknowledge within the driver acknowledgement time[5 seconds] after the Level transition. The service brake is applied. The driver acknowledges and the brakes are released. |  | 14 |
| Applicable Mode/Level Combinations | L0: UN / SB |  |  |
| Target of Test | The target of this test is to check that: Since the driver does not acknowledge the Level Transition within the driver acknowledgement time [5 seconds] after the Level transition, Service Brake is APPLIED. Service Brake is RELEASED when the driver acknowledges the Level Transition. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.14.1.2 |  |
|  |  | Subset-026-4.7.2 table - Input information Ackn of level transition |  |
|  |  | Subset-026-5.10.4.1 |  |
|  |  | Subset-026-5.10.4.1 b) |  |
|  |  | Subset-026-5.10.4.2 |  |
|  |  | Subset-026-5.10.4.4 |  |
|  |  | Subset-026-5.10.4.4 row L2 column L0 |  |
|  |  | Subset-026-8.6.4 Message 136: Train Position Report |  |
|  |  | Subset-026-7.4.3.1 Packet Number 0: Position Report |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The symbol LE01 (current level L0) is DISPLAYED. The acknowledgement request symbol LE07 is DISPLAYED. Service brake application is DISPLAYED. After the driver acknowledges, symbol LE07 is REMOVED.. Check on the TIU that : Service brake is APPLIED. Service brake is RELEASED when the driver acknowledges the Level Transition. Check on the JRU that: The displayed symbol LE07 is RECORDED. Service Brake intervention is RECORDED. Driver acknowledgement of the level transition L2/L0 is RECORDED. Service Brake release is RECORDED. |
| Constraints | If the Service Brake were not available for any reason, Emergency Brake shall be used. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 4 / 6 | UN / SB |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 0 | L0 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| Service brake not commanded | O | TIU | - |
| The Level symbol "Level 0" is displayed | O | DMI | DMI Object: Symbol LE01 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The Level symbol "Acknowledge Level 0 announcement" is displayed | O | DMI | DMI Object: Symbol LE07 | N/A | N/A | - |
| 2 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit07=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | NOT The driver acknowledges "Level 0 announcement" | I | DMI | The driver does not acknowledge the Level Transition within the driver acknowledgement time [5 seconds] after the Level Transition | N/A | N/A | - |
| 4 | N/A | N/A | Service brake commanded | O | TIU | - | N/A | N/A | - |
| 5 | N/A | N/A | The Status symbol "Service Brake intervention" is displayed | O | DMI | - | N/A | N/A | - |
| 6 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | Train is at standstill (V_TRAIN=0) | I | INT | - | N/A | N/A | - |
| 8 | N/A | N/A | SA-DATA.Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted | O | RTM | - | N/A | N/A | - |
| 9 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=136) is recorded | O | JRU | - | N/A | N/A | - |
| 10 | N/A | N/A | The driver acknowledges "Level 0 announcement" | I | DMI | - | N/A | N/A | - |
| 11 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 0110) is recorded | O | JRU | - | N/A | N/A | - |
| 12 | N/A | N/A | Service brake not commanded | O | TIU | - | N/A | N/A | - |
| 13 | N/A | N/A | The Status symbol "Service Brake intervention" is removed | O | DMI | - | N/A | N/A | - |
| 14 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=0) is recorded | O | JRU | - | N/A | N/A | - |
| 15 | N/A | N/A | The Level symbol "Acknowledge Level 0 announcement" is removed | O | DMI | DMI Object: Symbol LE07 | N/A | N/A | - |
| 16 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit07=0>) is recorded | O | JRU | - | N/A | N/A | - |

| Step 8: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 4 / 6 | UN / SB |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 0 | L0 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| Service brake not commanded | O | TIU | - |
| The Level symbol "Level 0" is displayed | O | DMI | DMI Object: Symbol LE01 The acknowledgement request for Level Transition, symbol LE07 is REMOVED. |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 15

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.4: Acknowledgement of the level transition |  | 5100400 |
| Test Case of Feature | The train enters in the acknowledgement window, and the driver is requested to acknowledge the transition from L2 to LNTC. The driver acknowledges before entering the LNTC area. |  | 15 |
| Applicable Mode/Level Combinations | L2 : FS / LS / OS / SR / SB / TR |  |  |
| Target of Test | The target of this test case is to check that : The on-board equipment requests the driver to acknowledge the level transition L2/ LNTC through the DMI when the max safe front end of the train passes over the location (defined in the level transition announcement with L_ACKLEVELTR). |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.7.2 table - Input information Ackn of level transition |  |
|  |  | Subset-026-5.10.4.1 |  |
|  |  | Subset-026-5.10.4.1 a) |  |
|  |  | Subset-026-5.10.4.4 |  |
|  |  | Subset-026-5.10.4.4 row L2 column L NTC |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The symbol LE04 (current level L2) is DISPLAYED The symbol LE09 requesting the driver to acknowledge the Level transition when the max safe front end of the train reaches the location defined by L_ACKLEVELTR is DISPLAYED. The driver acknowledges the Level transition on the DMI After driver acknowledgment, the symbol LE08 indicating the announcement of LNTC is DISPLAYED Check on the JRU that: The displayed symbol LE09 is RECORDED. The driver acknowledgement of the level transition L2/LNTC is RECORDED. The displayed symbol LE08 is RECORDED. |
| Constraints | The driver acknowledges the level transition L2/LNTC inside the defined area (L_ACKLEVELTR: Length of the acknowledgement area in rear of the required level). |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 7 / 12 | FS/ OS / SR / SB / TR / LS |
| Level Transition Order | STORED | Level transition to LNTC at further location with an acknowledgement window before the level transition border |
| ERTMS/ETCS level | 3 | L2 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level 2" is displayed The Level symbol "Level 0 announcement" is displayed | O | DMI | DMI Object: Symbol LE04 DMI Object: Symbol LE08 (Except in SB mode) |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The Level symbol "Level NTC announcement" is removed | O | DMI | The max safe front end of the train enters the acknowledgement window (defined in the level transition announcement with L_ACKLEVELTR). Symbol LE08 is REMOVED.. | N/A | N/A | - |
| 2 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit08=0>) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The Level symbol "Acknowledge Level NTC announcement" is displayed | O | DMI | DMI Object: Symbol LE09 | N/A | N/A | - |
| 4 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit09=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | The driver acknowledges "Level NTC announcement" | I | DMI | - | N/A | N/A | - |
| 6 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 1010) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | The Level symbol "Acknowledge Level NTC announcement" is removed | O | DMI | DMI Object: Symbol LE09 | N/A | N/A | - |
| 8 | N/A | N/A | The Level symbol "Level NTC announcement" is displayed | O | DMI | DMI Object: Symbol LE08 | N/A | N/A | - |
| 9 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit08=1>) is recorded | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 7 / 12 | FS/ OS / SR / SB / TR / LS |
| Level Transition Order | STORED | Transition to level NTC |
| ERTMS/ETCS level | 3 | L2 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level NTC announcement" is displayed | O | DMI | DMI Object: Symbol LE08 The acknowledgement request for the level transition, symbol LE09 is REMOVED |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 16

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-Board equipment |  |  |
| Tested Feature | 5.10.4: Acknowledgement of the level transition |  | 5100400 |
| Test Case of Feature | The train has passed over the Level Transition border and the On-board equipment has switched from L2 to Level NTC, upon receipt of an order. The driver acknowledges within the driver acknowledgement time[5 seconds] after the Level transition. |  | 16 |
| Applicable Mode/Level Combinations | LNTC: SB / TR / SN |  |  |
| Target of Test | The target of this test is to check that: The on-board does not apply the brakes if the driver acknowledges the Level Transition L2/LSTM within driver acknowledgement time[5 seconds] after the Level Transition. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.7.2 table - Input information Ackn of level transition |  |
|  |  | Subset-026-5.10.4.1 |  |
|  |  | Subset-026-5.10.4.1 b) |  |
|  |  | Subset-026-5.10.4.4 |  |
|  |  | Subset-026-5.10.4.4 row L2 column L NTC |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The symbol LE02 (current Level NTC) is DISPLAYED. The acknowledgement request symbol LE09 after level transition is DISPLAYED. After the driver acknowledgement, symbol LE09 is REMOVED. Check on the TIU that : The service brake is NOT APPLIED. Check on the JRU that: The displayed symbol LE09 is RECORDED. The driver acknowledgement of the level transition L2/LNTC is RECORDED. |
| Constraints | A cab needs to be active (SB Mode) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 / 7/ 13 | SB / TR / SN |
| ERTMS/ETCS level | 1 | LNTC |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level NTC" is displayed | O | DMI | DMI Object: Symbol LE02 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The Level symbol "Acknowledge Level NTC announcement" is displayed | O | DMI | DMI Object: Symbol LE09 | N/A | N/A | - |
| 2 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit09=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The driver acknowledges "Level NTC announcement" | I | DMI | The driver acknowledges the Level Transition within the driver acknowledgement time[5 seconds]after the Level Transition. | N/A | N/A | - |
| 4 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 1010) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | The Level symbol "Acknowledge Level NTC announcement" is removed | O | DMI | DMI Object: Symbol LE09 | N/A | N/A | - |
| 6 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit09=0>) is recorded | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 / 7/ 13 | SB / TR / SN |
| ERTMS/ETCS level | 1 | LNTC |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level NTC" is displayed | O | DMI | DMI Object: Symbol LE02 (current level ) The acknowledgement request for Level Transition, symbol LE09 is REMOVED. |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 17

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.4: Acknowledgement of the level transition |  | 5100400 |
| Test Case of Feature | The train has passed over the Level Transition border and the On-board equipment has switched from L2 to Level NTC, upon receipt of an order. The driver does not acknowledge within the driver acknowledgement time[5 seconds] after the Level transition. The service brake is applied. The driver acknowledges and the brakes are released. |  | 17 |
| Applicable Mode/Level Combinations | LNTC: SB / TR / SN |  |  |
| Target of Test | The target of this test is to check that: Since the driver does not acknowledge the Level Transition within the driver acknowledgement time [5 seconds] after the Level transition, Service Brake is APPLIED. Service Brake is RELEASED when the driver acknowledges the Level Transition. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.7.2 table - Input information Ackn of level transition |  |
|  |  | Subset-026-5.10.4.1 |  |
|  |  | Subset-026-5.10.4.1 b) |  |
|  |  | Subset-026-5.10.4.2 |  |
|  |  | Subset-026-5.10.4.4 |  |
|  |  | Subset-026-5.10.4.4 row L2 column L NTC |  |
|  |  | Subset-026-8.6.4 Message 136: Train Position Report |  |
|  |  | Subset-026-7.4.3.1 Packet Number 0: Position Report |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The symbol LE02 (current level NTC) is displayed The acknowledgement request symbol LE09 after level transition is DISPLAYED. Service brake application is DISPLAYED. After the driver acknowledges, the symbol LE09is REMOVED. Check on the TIU that : Service brake is APPLIED. Service brake is RELEASED when the driver acknowledges the Level Transition Check on the JRU that: The displayed symbol LE09 is RECORDED. Service Brake intervention is RECORDED. Driver acknowledgement of the level transition L2/ LNTC is RECORDED. Service Brake release is RECORDED. |
| Constraints | If the Service Brake were not available for any reason, Emergency Brake shall be used. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 / 7 / 13 | SB / TR / SN |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 1 | LNTC |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| Service brake not commanded | O | TIU | - |
| The Level symbol "Level NTC" is displayed | O | DMI | DMI Object: Symbol LE02 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The Level symbol "Acknowledge Level NTC announcement" is displayed | O | DMI | DMI Object: Symbol LE09 | N/A | N/A | - |
| 2 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit09=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | NOT The driver acknowledges "Level NTC announcement" | I | DMI | The driver does not acknowledge the Level Transition within the driver acknowledgement time [five seconds] after the Level Transition. | N/A | N/A | - |
| 4 | N/A | N/A | Service brake commanded | O | TIU | - | N/A | N/A | - |
| 5 | N/A | N/A | The Status symbol "Service Brake intervention" is displayed | O | DMI | - | N/A | N/A | - |
| 6 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | Train is at standstill (V_TRAIN=0) | I | INT | - | N/A | N/A | - |
| 8 | N/A | N/A | SA-DATA.Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted | O | RTM | - | N/A | N/A | - |
| 9 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=136) is recorded | O | JRU | - | N/A | N/A | - |
| 10 | N/A | N/A | The driver acknowledges "Level NTC announcement" | I | DMI | - | N/A | N/A | - |
| 11 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 1010) is recorded | O | JRU | - | N/A | N/A | - |
| 12 | N/A | N/A | Service brake not commanded | O | TIU | - | N/A | N/A | - |
| 13 | N/A | N/A | The Status symbol "Service Brake intervention" is removed | O | DMI | - | N/A | N/A | - |
| 14 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=0) is recorded | O | JRU | - | N/A | N/A | - |
| 15 | N/A | N/A | The Level symbol "Acknowledge Level NTC announcement" is removed | O | DMI | DMI Object: Symbol LE09 | N/A | N/A | - |
| 16 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit09=0>) is recorded | O | JRU | - | N/A | N/A | - |

| Step 8: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 / 7 / 13 | SB / TR / SN |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 1 | LNTC |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| Service brake not commanded | O | TIU | - |
| The Level symbol "Level NTC" is displayed | O | DMI | DMI Object: Symbol LE02 The acknowledgement request for Level Transition, symbol LE09 is REMOVED. |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 18

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.4: Acknowledgement of the level transition |  | 5100400 |
| Test Case of Feature | The train enters in the acknowledgement window, and the driver is requested to acknowledge the transition from L3 to L0. The driver acknowledges before entering the L0 area. |  | 18 |
| Applicable Mode/Level Combinations | L3 : FS / LS / OS / SR / SB / TR |  |  |
| Target of Test | The target of this test case is to check that : The on-board equipment requests the driver to acknowledge the level transition L3/L0 through the DMI when the max safe front end of the train passes over the location (defined in the level transition announcement with L_ACKLEVELTR). |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.7.2 table - Input information Ackn of level transition |  |
|  |  | Subset-026-5.10.4.1 |  |
|  |  | Subset-026-5.10.4.1 a) |  |
|  |  | Subset-026-5.10.4.4 |  |
|  |  | Subset-026-5.10.4.4 row L3 column L0 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The symbol LE07 requesting driver to acknowledge the Level transition when the max safe front end of the train reaches the location defined by L_ACKLEVELTR is DISPLAYED. The driver acknowledges the Level transition on the DMI After driver acknowledgment, the symbol LE06 indicating the announcement to L0 is DISPLAYED Check on the JRU that: The displayed symbol LE07 is RECORDED Driver acknowledgement of the level transition L3/L0 is RECORDED. The displayed symbol LE06 is RECORDED. |
| Constraints | The driver acknowledges the level transition L3/L0 inside the defined area (L_ACKLEVELTR: Length of the acknowledgement area in rear of the required level). |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 7 / 12 | FS / OS / SR / SB / TR / LS |
| Level Transition Order | STORED | Level transition to L0 at further location with an acknowledgement window before the level transition border |
| ERTMS/ETCS level | 4 | L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level 3" is displayed The Level symbol "Level 0 announcement" is displayed | O | DMI | DMI Object: Symbol LE05 DMI Object: Symbol LE06 (Except in SB mode) |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The Level symbol "Level 0 announcement" is removed | O | DMI | The max safe front end of the train enters the acknowledgement window (defined in the level transition announcement with L_ACKLEVELTR). Symbol LE06 is REMOVED. | N/A | N/A | - |
| 2 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit06=0>) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The Level symbol "Acknowledge Level 0 announcement" is displayed | O | DMI | DMI Object: Symbol LE07 | N/A | N/A | - |
| 4 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit07=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | The driver acknowledges "Level 0 announcement" | I | DMI | - | N/A | N/A | - |
| 6 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 0110) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | The Level symbol "Acknowledge Level 0 announcement" is removed | O | DMI | DMI Object: Symbol LE07 | N/A | N/A | - |
| 8 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit07=0>) is recorded | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | The Level symbol "Level 0 announcement" is displayed | O | DMI | DMI Object: Symbol LE06 | N/A | N/A | - |
| 10 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit06=1>) is recorded | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 7 / 12 | FS / OS / SR / SB / TR / LS |
| Level Transition Order | STORED | Transition to L0 |
| ERTMS/ETCS level | 4 | L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level 0 announcement" is displayed | O | DMI | DMI Object: Symbol LE06 is displayed The acknowledgement request for the level transition, symbol LE07 is REMOVED. |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 19

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-Board equipment |  |  |
| Tested Feature | 5.10.4: Acknowledgement of the level transition |  | 5100400 |
| Test Case of Feature | The train has passed over the Level Transition border and the On-board equipment has switched from L3 to Level 0, upon receipt of an order. The driver acknowledges within the driver acknowledgement time[5 seconds] after the Level transition. |  | 19 |
| Applicable Mode/Level Combinations | L0: UN / SB |  |  |
| Target of Test | The target of this test is to check that: The On-board does not apply the brakes if the driver acknowledges the Level Transition L3/L0 within the driver acknowledgement time [5seconds] after the Level Transition. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.7.2 table - Input information Ackn of level transition |  |
|  |  | Subset-026-5.10.4.1 |  |
|  |  | Subset-026-5.10.4.1 b) |  |
|  |  | Subset-026-5.10.4.4 |  |
|  |  | Subset-026-5.10.4.4 row L3 column L0 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The symbol LE01 (current Level 0) is DISPLAYED. The acknowledgement request symbol LE07 after level transition is DISPLAYED. After the driver acknowledgement the acknowledgement request for Level Transition, symbol LE07 is REMOVED. Check on the TIU that : The service brake is NOT APPLIED. Check on the JRU that: The displayed symbol LE07 is RECORDED. The driver acknowledgement of the level transition L3/L0 is RECORDED. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 4 / 6 | UN / SB |
| ERTMS/ETCS level | 0 | L0 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Service brake not commanded | O | TIU | - |
| The Level symbol "Level 0" is displayed | O | DMI | DMI Object: Symbol LE01 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The Level symbol "Acknowledge Level 0 announcement" is displayed | O | DMI | DMI Object: Symbol LE07 | N/A | N/A | - |
| 2 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit07=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The driver acknowledges "Level 0 announcement" | I | DMI | The driver acknowledges the Level Transition within the driver acknowledgement time [five seconds]after the Level Transition. | N/A | N/A | - |
| 4 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 0110) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | The Level symbol "Acknowledge Level 0 announcement" is removed | O | DMI | DMI Object: Symbol LE07 | N/A | N/A | - |
| 6 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit07=0>) is recorded | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 4 / 6 | UN / SB |
| ERTMS/ETCS level | 0 | L0 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level 0" is displayed | O | DMI | DMI Object: Symbol LE01 The acknowledgement request for Level Transition, symbol LE07 is REMOVED. |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 20

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.4: Acknowledgement of the level transition |  | 5100400 |
| Test Case of Feature | The train has passed over the Level Transition border and the On-board equipment has switched from L3 to Level 0, upon receipt of an order. The driver does not acknowledge within the driver acknowledgement time[5 seconds] after the Level transition. The service brake is applied. The driver acknowledges and the brakes are released. |  | 20 |
| Applicable Mode/Level Combinations | L0: UN / SB |  |  |
| Target of Test | The target of this test is to check that: Since the driver does not acknowledge the Level Transition within the driver acknowledgement time [five seconds] after the Level transition, Service Brake is APPLIED. Service Brake is RELEASED when the driver acknowledges the Level Transition. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.14.1.2 |  |
|  |  | Subset-026-4.7.2 table - Input information Ackn of level transition |  |
|  |  | Subset-026-5.10.4.1 |  |
|  |  | Subset-026-5.10.4.1 b) |  |
|  |  | Subset-026-5.10.4.2 |  |
|  |  | Subset-026-5.10.4.4 |  |
|  |  | Subset-026-5.10.4.4 row L3 column L0 |  |
|  |  | Subset-026-8.6.4 Message 136: Train Position Report |  |
|  |  | Subset-026-7.4.3.1 Packet Number 0: Position Report |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The symbol LE01 (current level L0) is DISPLAYED. The acknowledgement request symbol LE07 is DISPLAYED. Service brake application is DISPLAYED. After the driver acknowledges the acknowledgement, symbol LE07 is REMOVED.. Check on the TIU that : Service brake is APPLIED. Service brake is RELEASED when the driver acknowledges the Level Transition. Check on the JRU that: The displayed symbol LE07 is RECORDED. Service Brake intervention is RECORDED. The driver acknowledgement of the level transition L3/L0 is RECORDED. Service Brake release is RECORDED. |
| Constraints | If the Service Brake were not available for any reason, Emergency Brake shall be used. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 4 / 6 | UN / SB |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 0 | L0 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| Service brake not commanded | O | TIU | - |
| The Level symbol "Level 0" is displayed | O | DMI | DMI Object: Symbol LE01 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The Level symbol "Acknowledge Level 0 announcement" is displayed | O | DMI | DMI Object: Symbol LE07 | N/A | N/A | - |
| 2 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit07=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | NOT The driver acknowledges "Level 0 announcement" | I | DMI | The driver does not acknowledge the Level Transition within the driver acknowledgement time [5 seconds] after the Level Transition. | N/A | N/A | - |
| 4 | N/A | N/A | Service brake commanded | O | TIU | - | N/A | N/A | - |
| 5 | N/A | N/A | The Status symbol "Service Brake intervention" is displayed | O | DMI | - | N/A | N/A | - |
| 6 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | Train is at standstill (V_TRAIN=0) | I | INT | - | N/A | N/A | - |
| 8 | N/A | N/A | SA-DATA.Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted | O | RTM | - | N/A | N/A | - |
| 9 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=136) is recorded | O | JRU | - | N/A | N/A | - |
| 10 | N/A | N/A | The driver acknowledges "Level 0 announcement" | I | DMI | - | N/A | N/A | - |
| 11 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 0110) is recorded | O | JRU | - | N/A | N/A | - |
| 12 | N/A | N/A | Service brake not commanded | O | TIU | - | N/A | N/A | - |
| 13 | N/A | N/A | The Status symbol "Service Brake intervention" is removed | O | DMI | - | N/A | N/A | - |
| 14 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=0) is recorded | O | JRU | - | N/A | N/A | - |
| 15 | N/A | N/A | The Level symbol "Acknowledge Level 0 announcement" is removed | O | DMI | DMI Object: Symbol LE07 | N/A | N/A | - |
| 16 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit07=0>) is recorded | O | JRU | - | N/A | N/A | - |

| Step 8: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 4 / 6 | UN / SB |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 0 | L0 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| Service brake not commanded | O | TIU | - |
| The Level symbol "Level 0" is displayed | O | DMI | DMI Object: Symbol LE01 The acknowledgement request for Level Transition, symbol LE07 is REMOVED. |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 21

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.4: Acknowledgement of the level transition |  | 5100400 |
| Test Case of Feature | The train enters in the acknowledgement window, and the driver is requested to acknowledge the transition from L3 to LNTC. The driver acknowledges before entering the LNTC area. |  | 21 |
| Applicable Mode/Level Combinations | L3 : FS / LS / OS / SR / TR |  |  |
| Target of Test | The target of this test case is to check that : The on-board equipment requests the driver to acknowledge the level transition L3/LNTC through the DMI when the max safe front end of the train passes over the location (defined in the level transition announcement with L_ACKLEVELTR). |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.7.2 table - Input information Ackn of level transition |  |
|  |  | Subset-026-5.10.4.1 |  |
|  |  | Subset-026-5.10.4.1 a) |  |
|  |  | Subset-026-5.10.4.4 |  |
|  |  | Subset-026-5.10.4.4 row L3 column L NTC |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The symbol LE09 requesting driver to acknowledge the Level transition when the max safe front end of the train reaches the location defined by L_ACKLEVELTR is DISPLAYED.. The driver acknowledges the Level transition on the DMI After driver acknowledgment, the symbol LE08 indicating the announcement of LNTC is DISPLAYED Check on the JRU that: The displayed symbol LE09 is RECORDED. The driver acknowledgement of the level transition L3/LNTC is RECORDED. The displayed symbol LE08 is RECORDED. |
| Constraints | The driver acknowledges the level transition L3/LNTC inside the defined area (L_ACKLEVELTR: Length of the acknowledgement area in rear of the required level). |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 7 / 12 | FS / OS / SR / TR / LS |
| Level Transition Order | STORED | Level transition to LNTC at further location with an acknowledgement window before the transition border |
| ERTMS/ETCS level | 4 | L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level 3" is displayed The Level symbol "Level NTC announcement" is displayed | O | DMI | DMI Object: Symbol LE05 DMI Object: Symbol LE08 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The Level symbol "Level NTC announcement" is removed | O | DMI | The max safe front end of the train enters the acknowledgement window (defined in the level transition announcement with L_ACKLEVELTR). Symbol LE08 is REMOVED. | N/A | N/A | - |
| 2 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit08=0>) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The Level symbol "Acknowledge Level NTC announcement" is displayed | O | DMI | DMI Object: Symbol LE09 | N/A | N/A | - |
| 4 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit09=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | The driver acknowledges "Level NTC announcement" | I | DMI | - | N/A | N/A | - |
| 6 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 1010) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | The Level symbol "Acknowledge Level NTC announcement" is removed | O | DMI | DMI Object: Symbol LE09 | N/A | N/A | - |
| 8 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit09=0>) is recorded | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | The Level symbol "Level NTC announcement" is displayed | O | DMI | DMI Object: Symbol LE08 | N/A | N/A | - |
| 10 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit08=1>) is recorded | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 7 / 12 | FS / OS / SR / TR / LS |
| Level Transition Order | STORED | Transition to LNTC |
| ERTMS/ETCS level | 4 | L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level NTC announcement" is displayed | O | DMI | DMI Object: Symbol LE08 The acknowledgement request for the level transition, symbol LE09 is REMOVED |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 22

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-Board equipment |  |  |
| Tested Feature | 5.10.4: Acknowledgement of the level transition |  | 5100400 |
| Test Case of Feature | The train has passed over the Level Transition border and the On-board equipment has switched from L3 to Level NTC, upon receipt of an order. The driver acknowledges within the driver acknowledgement time[5 seconds] after the Level transition. |  | 22 |
| Applicable Mode/Level Combinations | LNTC: SN |  |  |
| Target of Test | The target of this test is to check that: The On-board does not apply the brakes if the driver acknowledges the Level Transition L3/LNTC within the driver acknowledgement time[5 seconds] after the Level Transition. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.7.2 table - Input information Ackn of level transition |  |
|  |  | Subset-026-5.10.4.1 |  |
|  |  | Subset-026-5.10.4.1 b) |  |
|  |  | Subset-026-5.10.4.4 |  |
|  |  | Subset-026-5.10.4.4 row L3 column L NTC |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The symbol LE02 (current Level NTC) is DISPLAYED. The acknowledgement request symbol LE09 after level transition is DISPLAYED. After the driver acknowledgement, symbol LE09 is REMOVED. Check on the TIU that : The service brake is NOT APPLIED. Check on the JRU that: The displayed symbol LE09 is RECORDED. The driver acknowledgement of the level transition L3/LNTC is RECORDED. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 13 | SN |
| ERTMS/ETCS level | 1 | LNTC |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Service brake not commanded | O | TIU | - |
| The Level symbol "Level NTC" is displayed | O | DMI | DMI Object: Symbol LE02 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The Level symbol "Acknowledge Level NTC announcement" is displayed | O | DMI | DMI Object: Symbol LE09 | N/A | N/A | - |
| 2 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit09=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The driver acknowledges "Level NTC announcement" | I | DMI | The driver acknowledges the Level Transition within the driver acknowledgement time [5 seconds]after the Level Transition. | N/A | N/A | - |
| 4 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 1010) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | The Level symbol "Acknowledge Level NTC announcement" is removed | O | DMI | DMI Object: Symbol LE09 | N/A | N/A | - |
| 6 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit09=0>) is recorded | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 13 | SN |
| ERTMS/ETCS level | 1 | LNTC |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level NTC" is displayed | O | DMI | DMI Object: Symbol LE02 The acknowledgement request for Level Transition, symbol LE09 is REMOVED. |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 23

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.4: Acknowledgement of the level transition |  | 5100400 |
| Test Case of Feature | The train has passed over the Level Transition border and the On-board equipment has switched from L3 to Level NTC, upon receipt of an order. The driver does not acknowledge within the driver acknowledgement time[5 seconds] after the Level transition. The service brake is applied. The driver acknowledges and the brakes are released. |  | 23 |
| Applicable Mode/Level Combinations | LNTC: SB / TR / SN |  |  |
| Target of Test | The target of this test is to check that: Since the driver does not acknowledge the Level Transition within the driver acknowledgement time[5 seconds] after the Level transition, Service Brake is APPLIED. Service Brake is RELEASED when the driver acknowledges the Level Transition. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-5.10.4.1 |  |
|  |  | Subset-026-5.10.4.1 b) |  |
|  |  | Subset-026-5.10.4.2 |  |
|  |  | Subset-026-5.10.4.4 |  |
|  |  | Subset-026-5.10.4.4 row L3 column L NTC |  |
|  |  | Subset-026-8.6.4 Message 136: Train Position Report |  |
|  |  | Subset-026-7.4.3.1 Packet Number 0: Position Report |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The symbol LE02 (current level NTC) is displayed The acknowledgement request after level transition, symbol LE09 is DISPLAYED. Service brake application is DISPLAYED. After the driver acknowledges, the acknowledgement is REMOVED.. Check on the TIU that : Service brake is APPLIED. Service brake is RELEASED when the driver acknowledges the Level Transition. Check on the JRU that: The displayed symbol LE09 is RECORDED Service Brake intervention is RECORDED. The acknowledgement of the level transition L3/LNTC is RECORDED. Service Brake release is RECORDED. |
| Constraints | If the Service Brake were not available for any reason, Emergency Brake shall be used. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 / 7 / 13 | SB / TR / SN |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 1 | LNTC |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| Service brake not commanded | O | TIU | - |
| The Level symbol "Level NTC" is displayed | O | DMI | DMI Object: Symbol LE02 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The Level symbol "Acknowledge Level NTC announcement" is displayed | O | DMI | DMI Object: Symbol LE09 | N/A | N/A | - |
| 2 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit09=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | NOT The driver acknowledges "Level NTC announcement" | I | DMI | The driver does not acknowledge the Level Transition within the driver acknowledgement time [5 seconds] after the Level Transition. | N/A | N/A | - |
| 4 | N/A | N/A | Service brake commanded | O | TIU | - | N/A | N/A | - |
| 5 | N/A | N/A | The Status symbol "Service Brake intervention" is displayed | O | DMI | - | N/A | N/A | - |
| 6 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | Train is at standstill (V_TRAIN=0) | I | INT | - | N/A | N/A | - |
| 8 | N/A | N/A | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted | O | RTM | - | N/A | N/A | - |
| 9 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=136) is recorded | O | JRU | - | N/A | N/A | - |
| 10 | N/A | N/A | The driver acknowledges "Level NTC announcement" | I | DMI | - | N/A | N/A | - |
| 11 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 1010) is recorded | O | JRU | - | N/A | N/A | - |
| 12 | N/A | N/A | Service brake not commanded | O | TIU | - | N/A | N/A | - |
| 13 | N/A | N/A | The Status symbol "Service Brake intervention" is removed | O | DMI | - | N/A | N/A | - |
| 14 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=0) is recorded | O | JRU | - | N/A | N/A | - |
| 15 | N/A | N/A | The Level symbol "Acknowledge Level NTC announcement" is removed | O | DMI | DMI Object: Symbol LE09 | N/A | N/A | - |
| 16 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit09=0>) is recorded | O | JRU | - | N/A | N/A | - |

| Step 8: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 / 7 / 13 | SB / TR / SN |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 1 | LNTC |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| Service brake not commanded | O | TIU | - |
| The Level symbol "Level NTC" is displayed | O | DMI | DMI Object: Symbol LE02 The acknowledgement request for Level Transition, symbol LE09 is REMOVED. |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 24

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.4: Acknowledgement of the level transition |  | 5100400 |
| Test Case of Feature | The train enters in the acknowledgement window, and the driver is requested to acknowledge the transition from LNTC to L0. The driver acknowledges before entering the L0 area. |  | 24 |
| Applicable Mode/Level Combinations | LNTC: TR / SN |  |  |
| Target of Test | The target of this test case is to check that : The on-board equipment requests the driver to acknowledge the level transition LNTC/ L0 through the DMI when the max safe front end of the train passes a trackside defined location in rear of the level transition border(defined in the level transition announcement with L_ACKLEVELTR). |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.7.2 table - Input information Ackn of level transition |  |
|  |  | Subset-026-5.10.4.1 |  |
|  |  | Subset-026-5.10.4.1 a) |  |
|  |  | Subset-026-5.10.4.4 |  |
|  |  | Subset-026-5.10.4.4 row L NTC column L0 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The symbol LE07 requesting driver to acknowledge the Level transition when the max safe front end of the train reaches the location defined by L_ACKLEVELTR is DISPLAYED. The driver acknowledges the Level transition on the DMI After driver acknowledgment, the symbol LE06 indicating the announcement to L0 is DISPLAYED and symbol LE07 is REMOVED. Check on the JRU that: The displayed symbol LE07 is RECORDED. The acknowledgement of the level transition LNTC/L0 is RECORDED The displayed symbol LE06 is RECORDED |
| Constraints | The driver acknowledges the level transition LNTC/L0 inside the defined area (L_ACKLEVELTR: Length of the acknowledgement area in rear of the required level). |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 / 13 | TR / SN |
| Level Transition Order | STORED | Level transition to L0 at further location with an acknowledgement window before the transition border |
| ERTMS/ETCS level | 1 | LNTC |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level NTC" is displayed The Level symbol "Level 0 announcement" is displayed | O | DMI | DMI Object: Symbol LE02 DMI Object: Symbol LE06 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The Level symbol "Level 0 announcement" is removed | O | DMI | The max safe front end of the train enters the acknowledgement window (defined in the level transition announcement with L_ACKLEVELTR). Symbol LE06 is REMOVED | N/A | N/A | - |
| 2 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit06=0>) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The Level symbol "Acknowledge Level 0 announcement" is displayed | O | DMI | DMI Object: Symbol LE07 | N/A | N/A | - |
| 4 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit07=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | The driver acknowledges "Level 0 announcement" | I | DMI | - | N/A | N/A | - |
| 6 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 0110) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | The Level symbol "Acknowledge Level 0 announcement" is removed | O | DMI | DMI Object: Symbol LE07 | N/A | N/A | - |
| 8 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit07=0>) is recorded | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | The Level symbol "Level 0 announcement" is displayed | O | DMI | DMI Object: Symbol LE06 | N/A | N/A | - |
| 10 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit06=1>) is recorded | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 / 13 | TR / SN |
| Level Transition Order | STORED | Level transition to L0 |
| ERTMS/ETCS level | 1 | LNTC |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level 0 announcement" is displayed | O | DMI | DMI Object: Symbol LE06 The acknowledgement request for the level transition, symbol LE07 is REMOVED. |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 25

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.4: Acknowledgement of the level transition |  | 5100400 |
| Test Case of Feature | The train has passed over the Level Transition border and the On-board equipment has switched from LNTC to Level 0, upon receipt of an order. The driver acknowledges within the driver acknowledgement time[5 seconds] after the Level transition. |  | 25 |
| Applicable Mode/Level Combinations | L0: UN / SB |  |  |
| Target of Test | The target of this test is to check that: The On-board does not apply the service brake if the driver acknowledges the Level Transition within the driver acknowledgement time [5 seconds] after the Level Transition |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.7.2 table - Input information Ackn of level transition |  |
|  |  | Subset-026-5.10.4.1 |  |
|  |  | Subset-026-5.10.4.1 b) |  |
|  |  | Subset-026-5.10.4.4 |  |
|  |  | Subset-026-5.10.4.4 row L NTC column L0 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The symbol LE01 (current Level 0) is DISPLAYED. The acknowledgement request symbol LE07 after level transition is DISPLAYED. After the driver acknowledges, the acknowledgement request for Level Transition symbol LE07 is REMOVED. Check on the TIU that : The service brake is NOT APPLIED. Check on the JRU that: The displayed symbol LE07 is RECORDED. The driver acknowledgement of the level transition LNTC/L0 is RECORDED. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 4 / 6 | UN / SB |
| ERTMS/ETCS level | 0 | L0 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Service brake not commanded | O | TIU | - |
| The Level symbol "Level 0" is displayed | O | DMI | DMI Object: Symbol LE01 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The Level symbol "Acknowledge Level 0 announcement" is displayed | O | DMI | DMI Object: Symbol LE07 | N/A | N/A | - |
| 2 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit07=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The driver acknowledges "Level 0 announcement" | I | DMI | The driver acknowledges the Level Transition within the driver acknowledgement time [five seconds]after the Level Transition | N/A | N/A | - |
| 4 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 0110) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | The Level symbol "Acknowledge Level 0 announcement" is removed | O | DMI | DMI Object: Symbol LE07 | N/A | N/A | - |
| 6 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit07=0>) is recorded | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 4 / 6 | UN / SB |
| ERTMS/ETCS level | 0 | L0 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level 0" is displayed | O | DMI | DMI Object: Symbol LE01 The acknowledgement request for Level Transition, symbol LE07 is REMOVED. |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 26

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.4: Acknowledgement of the level transition |  | 5100400 |
| Test Case of Feature | The train has passed over the Level Transition border and the On-board equipment has switched from LNTC to Level 0, upon receipt of the order. The driver does not acknowledge within the driver acknowledgement time[5 seconds] after the Level transition. The service brake is applied. The driver acknowledges and the brakes are released. |  | 26 |
| Applicable Mode/Level Combinations | L0: UN / SB |  |  |
| Target of Test | The target of this test is to check that: Since the driver does not acknowledge the Level Transition within the driver acknowledgement time [5 seconds] after the Level transition, Service Brake is APPLIED. Service Brake is RELEASED when the driver acknowledges the Level Transition. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-5.10.4.1 |  |
|  |  | Subset-026-5.10.4.1 b) |  |
|  |  | Subset-026-5.10.4.2 |  |
|  |  | Subset-026-5.10.4.4 |  |
|  |  | Subset-026-5.10.4.4 row L NTC column L0 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The symbol LE01 (current level L0) is DISPLAYED. The acknowledgement request symbol LE07 is DISPLAYED Service brake application is DISPLAYED. After the driver acknowledges the acknowledgement, symbol LE07 is REMOVED. Check on the TIU that : Service brake is APPLIED. Service brake is RELEASED when the driver acknowledges the Level Transition. Check on the JRU that: The displayed symbol LE07 is RECORDED. Service Brake intervention is RECORDED. The driver acknowledgement of the level transition LNTC/L0 is RECORDED. Service Brake release is RECORDED. |
| Constraints | If the Service Brake were not available for any reason, Emergency Brake shall be used. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 4 / 6 | UN / SB |
| ERTMS/ETCS level | 0 | L0 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Service brake not commanded | O | TIU | - |
| The Level symbol "Level 0" is displayed | O | DMI | DMI Object: Symbol LE01 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The Level symbol "Acknowledge Level 0 announcement" is displayed | O | DMI | DMI Object: Symbol LE07 | N/A | N/A | - |
| 2 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit07=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | NOT The driver acknowledges "Level 0 announcement" | I | DMI | The driver does not acknowledge the Level Transition within the driver acknowledgement time [five seconds] after the Level Transition | N/A | N/A | - |
| 4 | N/A | N/A | Service brake commanded | O | TIU | - | N/A | N/A | - |
| 5 | N/A | N/A | The Status symbol "Service Brake intervention" is displayed | O | DMI | - | N/A | N/A | - |
| 6 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | The driver acknowledges "Level 0 announcement" | I | DMI | - | N/A | N/A | - |
| 8 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 0110) is recorded | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | Service brake not commanded | O | TIU | - | N/A | N/A | - |
| 10 | N/A | N/A | The Status symbol "Service Brake intervention" is removed | O | DMI | - | N/A | N/A | - |
| 11 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=0) is recorded | O | JRU | - | N/A | N/A | - |
| 12 | N/A | N/A | The Level symbol "Acknowledge Level 0 announcement" is removed | O | DMI | DMI Object: Symbol LE07 | N/A | N/A | - |
| 13 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit07=0>) is recorded | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 4 / 6 | UN / SB |
| ERTMS/ETCS level | 0 | L0 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Service brake not commanded | O | TIU | - |
| The Level symbol "Level 0" is displayed | O | DMI | DMI Object: Symbol LE01 The acknowledgement request for Level Transition is REMOVED |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 27

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.4: Acknowledgement of the level transition |  | 5100400 |
| Test Case of Feature | The train enters in the acknowledgement window, and the driver is requested to acknowledge the transition from LNTC to L1. The driver acknowledges before entering the L1 area. |  | 27 |
| Applicable Mode/Level Combinations | LNTC: TR /SN |  |  |
| Target of Test | The target of this test case is to check that : The on-board equipment requests the driver to acknowledge the level transition LNTC/ L1 through the DMI when the max safe front end of the train passes location trackside defined location in rear of the level transition border (defined in the level transition announcement with L_ACKLEVELTR). |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.7.2 table - Input information Ackn of level transition |  |
|  |  | Subset-026-5.10.4.1 |  |
|  |  | Subset-026-5.10.4.1 a) |  |
|  |  | Subset-026-5.10.4.4 |  |
|  |  | Subset-026-5.10.4.4 row L NTC column L1 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The symbol LE11 requesting driver to acknowledge the Level transition when the max safe front end of the train reaches the location defined by L_ACKLEVELTR is DISPLAYED. The driver acknowledges the Level transition on the DMI After driver acknowledgment, the symbol LE10 indicating the announcement to L0 is DISPLAYED and symbol LE11 is REMOVED Check on the JRU that: The displayed symbol LE11 is RECORDED. The acknowledgement by the driver of the level transition LNTC/L1 is RECORDED. The displayed symbol LE10 is RECORDED. |
| Constraints | The driver acknowledges the level transition LNTC/L1 inside the defined area (L_ACKLEVELTR: Length of the acknowledgement area in rear of the required level). |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 / 13 | TR / SN |
| Level Transition Order | STORED | Level transition to L1 at further location with an acknowledgement window before the level transition border |
| ERTMS/ETCS level | 1 | LNTC |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level NTC" is displayed The Level symbol "Level 1 announcement" is displayed | O | DMI | DMI Object: Symbol LE02 DMI Object:, symbol LE10 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The Level symbol "Level 1 announcement" is removed | O | DMI | The max safe front end of the train enters the acknowledgement window (defined in the level transition announcement with L_ACKLEVELTR). Symbol LE10 is REMOVED. | N/A | N/A | - |
| 2 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit10=0>) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The Level symbol "Acknowledge Level 1 announcement" is displayed | O | DMI | DMI Object: Symbol LE11 | N/A | N/A | - |
| 4 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit11=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | The driver acknowledges "Level 1 announcement" | I | DMI | - | N/A | N/A | - |
| 6 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 0111) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | The Level symbol "Acknowledge Level 1 announcement" is removed | O | DMI | DMI Object: Symbol LE11 | N/A | N/A | - |
| 8 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit11=0>) is recorded | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | The Level symbol "Level 1 announcement" is displayed | O | DMI | DMI Object: Symbol LE10 | N/A | N/A | - |
| 10 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit10=1>) is recorded | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 / 13 | TR / SN |
| Level Transition Order | STORED | Level transition to L1 |
| ERTMS/ETCS level | 1 | LNTC |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level 1 announcement" is displayed | O | DMI | DMI Object: Symbol LE10 The acknowledgement request for the level transition, symbol LE11 is REMOVED |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 28

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.4: Acknowledgement of the level transition |  | 5100400 |
| Test Case of Feature | The train has passed over the Level Transition border and the On-board equipment has switched from LNTC to Level 1, upon receipt of an order. The driver acknowledges within the driver acknowledgement time[5 seconds] after the Level transition. |  | 28 |
| Applicable Mode/Level Combinations | L1: FS / LS / OS / SR / TR |  |  |
| Target of Test | The target of this test is to check that: The On-board does not apply the brakes if the driver acknowledges the Level Transition within the driver acknowledgement time [5 seconds] after the Level Transition |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.7.2 table - Input information Ackn of level transition |  |
|  |  | Subset-026-5.10.4.1 |  |
|  |  | Subset-026-5.10.4.1 b) |  |
|  |  | Subset-026-5.10.4.4 |  |
|  |  | Subset-026-5.10.4.4 row L NTC column L1 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The symbol LE03 (current Level 1) is DISPLAYED The acknowledgement request of level transition, symbol LE11 is DISPLAYED. After the driver acknowledgement, the acknowledgement request for Level Transition is REMOVED. Check on the TIU that : The service brake is RELEASED. Check on the JRU that: The driver acknowledgement of the level transition LNTC/L1 is RECORDED. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 7 / 12 | FS / OS / SR / TR / LS |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Service brake not commanded | O | TIU | - |
| The Level symbol "Level 1" is displayed | O | DMI | DMI Object: Symbol LE03 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The Level symbol "Acknowledge Level 1 announcement" is displayed | O | DMI | DMI Object: Symbol LE11 | N/A | N/A | - |
| 2 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit11=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The driver acknowledges "Level 1 announcement" | I | DMI | The driver acknowledges the Level Transition within the driver acknowledgement time [5 seconds] after the Level Transition | N/A | N/A | - |
| 4 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 0111) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | The Level symbol "Acknowledge Level 1 announcement" is removed | O | DMI | DMI Object: Symbol LE11 | N/A | N/A | - |
| 6 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit11=0>) is recorded | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 7 / 12 | FS / OS / SR / TR / LS |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Service brake not commanded | O | TIU | - |
| The Level symbol "Level 1" is displayed | O | DMI | DMI Object: Symbol LE03 The acknowledgement request for Level Transition, symbol LE11 is REMOVED. |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 29

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.4: Acknowledgement of the level transition |  | 5100400 |
| Test Case of Feature | The train has passed over the Level Transition border and the On-board equipment has switched from LNTC to Level 1, upon receipt of an order. The driver does not acknowledge within the acknowledgement time[5 seconds] after the Level transition. The service brake is applied. The driver acknowledges and the brakes are released. |  | 29 |
| Applicable Mode/Level Combinations | L1: FS / LS / OS / SR / TR |  |  |
| Target of Test | The target of this test is to check that: Since the driver does not acknowledge the Level Transition within the acknowledgement time [five seconds] after the Level transition, Service Brake is APPLIED. Service Brake is RELEASED when the driver acknowledges the Level Transition. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.14.1.2 |  |
|  |  | Subset-026-4.7.2 table - Input information Ackn of level transition |  |
|  |  | Subset-026-5.10.4.1 |  |
|  |  | Subset-026-5.10.4.1 a) |  |
|  |  | Subset-026-5.10.4.2 |  |
|  |  | Subset-026-5.10.4.4 |  |
|  |  | Subset-026-5.10.4.4 row L NTC column L1 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The symbol LE03 (current level L1) is DISPLAYED. The acknowledgement request symbol LE11 is DISPLAYED Service brake application is DISPLAYED. After the driver acknowledges the acknowledgement, symbol LE11 is REMOVED.. Check on the TIU that : Service brake is APPLIED. Service brake is RELEASED when the driver acknowledges the Level Transition. Check on the JRU that: The displayed symbol LE11 is RECORDED. Service Brake intervention is RECORDED. The driver acknowledgement of the level transition LNTC/L1 is RECORDED. Service Brake release is RECORDED. |
| Constraints | If the Service Brake were not available for any reason, Emergency Brake shall be used. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 7 / 12 | FS / OS / SR / TR / LS |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Service brake not commanded | O | TIU | - |
| The Level symbol "Level 1" is displayed | O | DMI | DMI Object: Symbol LE03 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The Level symbol "Acknowledge Level 1 announcement" is displayed | O | DMI | DMI Object: Symbol LE11 | N/A | N/A | - |
| 2 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit11=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | NOT The driver acknowledges "Level 1 announcement" | I | DMI | The driver does not acknowledge the Level Transition within the acknowledgement time [five seconds] after the Level Transition. | N/A | N/A | - |
| 4 | N/A | N/A | Service brake commanded | O | TIU | - | N/A | N/A | - |
| 5 | N/A | N/A | The Status symbol "Service Brake intervention" is displayed | O | DMI | - | N/A | N/A | - |
| 6 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | The driver acknowledges "Level 1 announcement" | I | DMI | - | N/A | N/A | - |
| 8 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 0111) is recorded | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | Service brake not commanded | O | TIU | - | N/A | N/A | - |
| 10 | N/A | N/A | The Status symbol "Service Brake intervention" is removed | O | DMI | - | N/A | N/A | - |
| 11 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=0) is recorded | O | JRU | - | N/A | N/A | - |
| 12 | N/A | N/A | The Level symbol "Acknowledge Level 1 announcement" is removed | O | DMI | DMI Object: Symbol LE11 | N/A | N/A | - |
| 13 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit11=0>) is recorded | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 7 / 12 | FS / OS / SR / TR / LS |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Service brake not commanded | O | TIU | - |
| The Level symbol "Level 1" is displayed | O | DMI | DMI Object: Symbol LE03 The request of acknowledgement is NOT DISPLAYED |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 30

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.4: Acknowledgement of the level transition |  | 5100400 |
| Test Case of Feature | The train enters in the acknowledgement window, and the driver is requested to acknowledge the transition from LNTC to L2. The driver acknowledges before entering the L2 area. |  | 30 |
| Applicable Mode/Level Combinations | LNTC : SL / SB / TR / SN |  |  |
| Target of Test | The target of this test case is to check that : The on-board equipment requests the driver to acknowledge the level transition LNTC/ L2 through the DMI when the max safe front end of the train passes a trackside defined location in rear of the level transition border (location defined in the level transition announcement with L_ACKLEVELTR). |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.7.2 table - Output information Level transition announcement |  |
|  |  | Subset-026-4.7.2 table - Input information Ackn of level transition |  |
|  |  | Subset-026-5.10.4.1 |  |
|  |  | Subset-026-5.10.4.1 a) |  |
|  |  | Subset-026-5.10.4.4 |  |
|  |  | Subset-026-5.10.4.4 row L NTC column L2 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The symbol LE13 requesting driver to acknowledge the Level transition when the max safe front end of the train reaches the location defined by L_ACKLEVELTR is DISPLAYED. The driver acknowledges the Level transition on the DMI After driver acknowledgment, the symbol LE12 indicating the announcement to L2 is DISPLAYED and symbol LE13 is REMOVED Check on the JRU that: The displayed symbol LE13 is RECORDED. The driver acknowledgement of the level transition LNTC/L2 is RECORDED The displayed symbol LE12 is RECORDED. |
| Constraints | The driver acknowledges the level transition LNTC/L2 inside the defined area (L_ACKLEVELTR: Length of the acknowledgement area in rear of the required level). |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 5 / 6 / 7 / 13 | SL / SB / TR / SN |
| Level Transition Order | STORED | Level transition to L2 at further location with an acknowledgement window before the transition level border |
| ERTMS/ETCS level | 1 | LNTC |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level 2 announcement" is displayed | O | DMI | DMI Object: Symbol LE12 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The Level symbol "Level 2 announcement" is removed | O | DMI | The max safe front end of the train enters the acknowledgement window (defined in the level transition announcement with L_ACKLEVELTR). Symbol LE12 is REMOVED. | N/A | N/A | - |
| 2 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit12=0>) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The Level symbol "Acknowledge Level 2 announcement" is displayed | O | DMI | DMI Object: Symbol LE13 Except in SL mode | N/A | N/A | - |
| 4 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit13=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | The driver acknowledges "Level 2 announcement" | I | DMI | Except in SL mode | N/A | N/A | - |
| 6 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 1000) is recorded | O | JRU | Except in SL mode | N/A | N/A | - |
| 7 | N/A | N/A | The Level symbol "Acknowledge Level 2 announcement" is removed | O | DMI | DMI Object: Symbol LE13 Except in SL mode | N/A | N/A | - |
| 8 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit13=0>) is recorded | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | The Level symbol " Level 2 announcement" is displayed | O | DMI | DMI Object: Symbol LE12 | N/A | N/A | - |
| 10 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit12=1>) is recorded | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 5 / 6 / 7 / 13 | SL / SB / TR / SN |
| Level Transition Order | STORED | Level transition to L2 |
| ERTMS/ETCS level | 1 | LNTC |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level 2 announcement" is displayed | O | DMI | DMI Object: Symbol LE12 The acknowledgement request for the level transition, symbol LE13 is REMOVED. |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 31

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-Board equipment |  |  |
| Tested Feature | 5.10.4: Acknowledgement of the level transition |  | 5100400 |
| Test Case of Feature | The train has passed over the Level Transition border and the On-board equipment has switched from LNTC to Level 2, upon receipt of an order. The driver acknowledges within the driver acknowledgement time[5 seconds] after the Level transition. |  | 31 |
| Applicable Mode/Level Combinations | L2: FS / LS / OS / SR / TR |  |  |
| Target of Test | The target of this test is to check that: The On-board does not apply the brakes if the driver acknowledges the Level Transition LNTC/L2 within the driver acknowledgement time [5 seconds] after the Level Transition. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.7.2 table - Input information Ackn of level transition |  |
|  |  | Subset-026-5.10.4.1 |  |
|  |  | Subset-026-5.10.4.1 a) |  |
|  |  | Subset-026-5.10.4.4 |  |
|  |  | Subset-026-5.10.4.4 row L NTC column L2 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The symbol LE04 (current Level 2) is DISPLAYED The acknowledgement request of level transition, symbol LE13 is DISPLAYED After the driver acknowledgement the acknowledgement request for Level Transition, symbol LE13 is REMOVED. Check on the TIU that : The service brake is RELEASED. Check on the JRU that: The displayed symbol LE13 is RECORDED. The driver acknowledgement of the level transition LNTC/L2 is RECORDED. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 7 / 12 | FS / OS / SR / TR / LS |
| ERTMS/ETCS level | 3 | L2 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Service brake not commanded | O | TIU | - |
| The Level symbol "Level 2" is displayed | O | DMI | DMI Object: Symbol LE04 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The Level symbol "Acknowledge Level 2 announcement" is displayed | O | DMI | DMI Object: Symbol LE13 | N/A | N/A | - |
| 2 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit13=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The driver acknowledges "Level 2 announcement" | I | DMI | The driver acknowledges the Level Transition within the driver acknowledgement time [5 seconds] after the Level Transition. | N/A | N/A | - |
| 4 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 1000) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | The Level symbol "Acknowledge Level 2 announcement" is removed | O | DMI | DMI Object: Symbol LE13 | N/A | N/A | - |
| 6 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit13=0>) is recorded | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 7 / 12 | FS / OS / SR / TR / LS |
| ERTMS/ETCS level | 3 | L2 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level 2" is displayed | O | DMI | DMI Object: Symbol LE04 The acknowledgement request for Level Transition, symbol LE13 is REMOVED. |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 32

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-Board equipment |  |  |
| Tested Feature | 5.10.4: Acknowledgement of the level transition |  | 5100400 |
| Test Case of Feature | The train has passed over the Level Transition border and the On-board equipment has switched from LNTC to Level 2, upon receipt of an order. The driver does not acknowledge within the driver acknowledgement time[5 seconds] after the Level transition. The service brake is applied. The driver acknowledges and the brakes are released. |  | 32 |
| Applicable Mode/Level Combinations | L2: FS / LS / OS / SR / TR |  |  |
| Target of Test | The target of this test is to check that: Since the driver does not acknowledge the Level Transition within the driver acknowledgement time [5 seconds] after the Level transition, Service Brake is APPLIED. Service Brake is RELEASED when the driver acknowledges the Level Transition. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.14.1.2 |  |
|  |  | Subset-026-4.7.2 table - Input information Ackn of level transition |  |
|  |  | Subset-026-5.10.4.1 |  |
|  |  | Subset-026-5.10.4.1 b) |  |
|  |  | Subset-026-5.10.4.2 |  |
|  |  | Subset-026-5.10.4.4 |  |
|  |  | Subset-026-5.10.4.4 row L NTC column L2 |  |
|  |  | Subset-026-8.6.4 Message 136: Train Position Report |  |
|  |  | Subset-026-7.4.3.1 Packet Number 0: Position Report |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The symbol LE04 (current level L2) is DISPLAYED. The acknowledgement request symbol LE13 is DISPLAYED Service brake application is DISPLAYED. After the driver acknowledges the acknowledgement, symbol LE13 is REMOVED.. Check on the TIU that : Service brake is APPLIED. Service brake is RELEASED when the driver acknowledges the Level Transition. Check on the JRU that: The displayed symbol LE13 is RECORDED Service Brake intervention is RECORDED. The driver acknowledgement of the level transition LNTC/L2 is RECORDED. Service Brake release is RECORDED. |
| Constraints | If the Service Brake were not available for any reason, Emergency Brake shall be used. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 7 / 12 | FS / OS / SR / TR / LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 | L2 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| Service brake not commanded | O | TIU | - |
| The Level symbol "Level 2" is displayed | O | DMI | DMI Object: Symbol LE04 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The Level symbol "Acknowledge Level 2 announcement" is displayed | O | DMI | DMI Object: Symbol LE13 | N/A | N/A | - |
| 2 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit13=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | NOT The driver acknowledges "Level 2 announcement" | I | DMI | The driver does not acknowledge the Level Transition within the driver acknowledgement time [5 seconds] after the Level Transition. | N/A | N/A | - |
| 4 | N/A | N/A | Service brake commanded | O | TIU | - | N/A | N/A | - |
| 5 | N/A | N/A | The Status symbol "Service Brake intervention" is displayed | O | DMI | - | N/A | N/A | - |
| 6 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | Train is at standstill (V_TRAIN=0) | I | INT | - | N/A | N/A | - |
| 8 | N/A | N/A | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted | O | RTM | - | N/A | N/A | - |
| 9 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=136) is recorded | O | JRU | - | N/A | N/A | - |
| 10 | N/A | N/A | The driver acknowledges "Level 2 announcement" | I | DMI | - | N/A | N/A | - |
| 11 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 1000) is recorded | O | JRU | - | N/A | N/A | - |
| 12 | N/A | N/A | Service brake not commanded | O | TIU | - | N/A | N/A | - |
| 13 | N/A | N/A | The Status symbol "Service Brake intervention" is removed | O | DMI | - | N/A | N/A | - |
| 14 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=0) is recorded | O | JRU | - | N/A | N/A | - |
| 15 | N/A | N/A | The Level symbol "Acknowledge Level 2 announcement" is removed | O | DMI | DMI Object: Symbol LE13 | N/A | N/A | - |
| 16 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit13=0>) is recorded | O | JRU | - | N/A | N/A | - |

| Step 8: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 7 / 12 | FS / OS / SR / TR / LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 | L2 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| Service brake not commanded | O | TIU | - |
| The Level symbol "Level 2" is displayed | O | DMI | DMI Object: Symbol LE04 The acknowledgement request for Level Transition, symbol LE13 is REMOVED. |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 33

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.4: Acknowledgement of the level transition |  | 5100400 |
| Test Case of Feature | The train enters in the acknowledgement window, and the driver is requested to acknowledge the transition from LNTC to L3. The driver acknowledges before entering the L3 area. |  | 33 |
| Applicable Mode/Level Combinations | LNTC : SL / TR / SN |  |  |
| Target of Test | The target of this test case is to check that : The on-board equipment requests the driver to acknowledge the level transition LNTC/ L3 through the DMI when the max safe front end of the train passes a trackside defined location in rear of the level transition(location defined in the level transition announcement with L_ACKLEVELTR). |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.7.2 table - Output information Level transition announcement |  |
|  |  | Subset-026-4.7.2 table - Input information Ackn of level transition |  |
|  |  | Subset-026-5.10.4.1 |  |
|  |  | Subset-026-5.10.4.1 a) |  |
|  |  | Subset-026-5.10.4.4 |  |
|  |  | Subset-026-5.10.4.4 row L NTC column L3 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The symbol LE15 requesting driver to acknowledge the Level transition when the max safe front end of the train reaches the location defined by L_ACKLEVELTR is DISPLAYED.. The driver acknowledges the Level transition on the DMI After driver acknowledgment the level 3 announcement, symbol LE14 is DISPLAYED. Check on the JRU that: The displayed symbol LE15 is RECORDED. The driver acknowledgement of the level transition LNTC/L3 is RECORDED. The displayed symbol LE14 is RECORDED. |
| Constraints | The driver acknowledges the level transition LNTC/L3 inside the defined area (L_ACKLEVELTR: Length of the acknowledgement area in rear of the required level). |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 5 / 7 / 13 | SL / TR / SN |
| Level Transition Order | STORED | Level transition to L3 at further location with an acknowledgement window before the level transition border |
| ERTMS/ETCS level | 1 | LNTC |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level NTC" is displayed The Level symbol "Level 3 announcement" is displayed | O | DMI | DMI Object: Symbol LE02 DMI Object: Symbol LE14 (Except in SB/SL mode) |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The Level symbol "Level 3 announcement" is removed | O | DMI | The max safe front end of the train enters the acknowledgement window (defined in the level transition announcement with L_ACKLEVELTR). Symbol LE14 is REMOVED Except in SL Mode | N/A | N/A | - |
| 2 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit14=0>) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The Level symbol "Acknowledge Level 3 announcement" is displayed | O | DMI | DMI Object: Symbol LE15 | N/A | N/A | - |
| 4 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit15=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | The driver acknowledges "Level 3 announcement" | I | DMI | - | N/A | N/A | - |
| 6 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 1001) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | The Level symbol "Acknowledge Level 3 announcement" is removed | O | DMI | DMI Object: Symbol LE15 | N/A | N/A | - |
| 8 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit15=0>) is recorded | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | The Level symbol "Level 3 announcement" is displayed | O | DMI | DMI Object: Symbol LE14 | N/A | N/A | - |
| 10 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit14=1>) is recorded | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 5 / 7 / 13 | SL / TR / SN |
| Level Transition Order | STORED | Level transition to L3 |
| ERTMS/ETCS level | 1 | LNTC |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level 3 announcement" is displayed | O | DMI | DMI Object: Symbol LE14 The acknowledgement request for the level transition, symbol LE15 is REMOVED. (Except in SL mode) |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 34

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-Board equipment |  |  |
| Tested Feature | 5.10.4: Acknowledgement of the level transition |  | 5100400 |
| Test Case of Feature | The train has passed over the Level Transition border and the On-board equipment has switched from LNTC to Level 3, upon receipt of the order. The driver acknowledges within the driver acknowledgement time [5 seconds] after the Level transition. |  | 34 |
| Applicable Mode/Level Combinations | L3: FS / LS / OS / SR / TR |  |  |
| Target of Test | The target of this test is to check that: The On-board does not apply the brakes if the driver acknowledges the Level Transition LNTC/L3 within the driver acknowledgement time [f5 seconds] after the Level Transition. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.7.2 table - Input information Ackn of level transition |  |
|  |  | Subset-026-5.10.4.1 |  |
|  |  | Subset-026-5.10.4.1 b) |  |
|  |  | Subset-026-5.10.4.4 |  |
|  |  | Subset-026-5.10.4.4 row L NTC column L3 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The symbol LE05 (current Level 3) is DISPLAYED The acknowledgement request of level transition, symbol LE15 is DISPLAYED After the driver acknowledgement the acknowledgement request for Level Transition, symbol LE15 is REMOVED. Check on the TIU that : The service brake is RELEASED. Check on the JRU that: The displayed symbol LE15 is RECORDED The driver acknowledgement of the level transition LNTC/L3 is RECORDED. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 7 / 12 | FS / OS / SR / TR / LS |
| ERTMS/ETCS level | 4 | L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Service brake not commanded | O | TIU | - |
| The Level symbol "Level 3" is displayed | O | DMI | DMI Object: Symbol LE05 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The Level symbol "Acknowledge Level 3 announcement" is displayed | O | DMI | DMI Object: Symbol LE15 | N/A | N/A | - |
| 2 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit15=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The driver acknowledges "Level 3 announcement" | I | DMI | The driver acknowledges the Level Transition within the driver acknowledgement time [5seconds] after the Level Transition. | N/A | N/A | - |
| 4 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 1001) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | The Level symbol "Acknowledge Level 3 announcement" is removed | O | DMI | DMI Object: Symbol LE15 | N/A | N/A | - |
| 6 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit15=0>) is recorded | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 7 / 12 | FS / OS / SR / TR / LS |
| ERTMS/ETCS level | 4 | L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level 3" is displayed | O | DMI | DMI Object: Symbol LE05 The acknowledgement request for Level Transition, symbol LE15 is REMOVED. |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 35

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.4: Acknowledgement of the level transition |  | 5100400 |
| Test Case of Feature | The train has passed over the Level Transition border and the On-board equipment has switched from LNTC to Level 3, upon receipt of the order. The driver does not acknowledge within the driver acknowledgement time [5 seconds] after the Level transition. The service brake is applied. The driver acknowledges and the brakes are released. |  | 35 |
| Applicable Mode/Level Combinations | L3: FS / LS / OS / SR / TR |  |  |
| Target of Test | The target of this test is to check that: Since the driver does not acknowledge the Level Transition within the driver acknowledgement time [f5seconds] after the Level transition, Service Brake is APPLIED. Service Brake is RELEASED when the driver acknowledges the Level Transition. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.14.1.2 |  |
|  |  | Subset-026-4.7.2 table - Input information Ackn of level transition |  |
|  |  | Subset-026-5.10.4.1 |  |
|  |  | Subset-026-5.10.4.1 b) |  |
|  |  | Subset-026-5.10.4.2 |  |
|  |  | Subset-026-5.10.4.4 |  |
|  |  | Subset-026-5.10.4.4 row L NTC column L3 |  |
|  |  | Subset-026-8.6.4 Message 136: Train Position Report |  |
|  |  | Subset-026-7.4.3.1 Packet Number 0: Position Report |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The symbol LE05 (current level L3) is DISPLAYED. The acknowledgement request symbol LE15 is DISPLAYED Service brake application is DISPLAYED. After the driver acknowledges the acknowledgement, symbol LE15 is REMOVED. Check on the TIU that : Service brake is APPLIED. Service brake is RELEASED when the driver acknowledges the Level Transition. Check on the JRU that: The displayed symbol LE15 is RECORDED Service Brake intervention is RECORDED. The driver acknowledgement of the level transition LNTC/L3 is RECORDED. Service Brake release is RECORDED. |
| Constraints | If the Service Brake were not available for any reason, Emergency Brake shall be used. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 7 / 12 | FS / OS / SR / TR / LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 4 | L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| Service brake not commanded | O | TIU | - |
| The Level symbol "Level 3" is displayed | O | DMI | DMI Object: Symbol LE05 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The Level symbol "Acknowledge Level 3 announcement" is displayed | O | DMI | DMI Object: Symbol LE15 | N/A | N/A | - |
| 2 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit15=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | NOT The driver acknowledges "Level 3 announcement" | I | DMI | The driver does not acknowledge the Level Transition within the driver acknowledgement time [5 seconds] after the Level Transition. | N/A | N/A | - |
| 4 | N/A | N/A | Service brake commanded | O | TIU | - | N/A | N/A | - |
| 5 | N/A | N/A | The Status symbol "Service Brake intervention" is displayed | O | DMI | - | N/A | N/A | - |
| 6 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | Train is at standstill (V_TRAIN=0) | I | INT | - | N/A | N/A | - |
| 8 | N/A | N/A | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted | O | RTM | - | N/A | N/A | - |
| 9 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=136) is recorded | O | JRU | - | N/A | N/A | - |
| 10 | N/A | N/A | The driver acknowledges "Level 3 announcement" | I | DMI | - | N/A | N/A | - |
| 11 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 1001) is recorded | O | JRU | - | N/A | N/A | - |
| 12 | N/A | N/A | Service brake not commanded | O | TIU | - | N/A | N/A | - |
| 13 | N/A | N/A | The Status symbol "Service Brake intervention" is removed | O | DMI | - | N/A | N/A | - |
| 14 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=0) is recorded | O | JRU | - | N/A | N/A | - |
| 15 | N/A | N/A | The Level symbol "Acknowledge Level 3 announcement" is removed | O | DMI | - | N/A | N/A | - |
| 16 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit15=0>) is recorded | O | JRU | - | N/A | N/A | - |

| Step 8: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 7 / 12 | FS / OS / SR / TR / LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 4 | L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| Service brake not commanded | O | TIU | - |
| The Level symbol "Level 3" is displayed | O | DMI | DMI Object: Symbol LE05 The acknowledgement request for Level Transition, symbol LE15 is REMOVED. |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 36

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.4: Acknowledgement of the level transition |  | 5100400 |
| Test Case of Feature | The train enters in the acknowledgement window, and the driver is requested to acknowledge the transition from LNTC [X] to LNTC[Y]. The driver acknowledges before entering the LNTC[Y] area. |  | 36 |
| Applicable Mode/Level Combinations | LNTC[X] : SN / SL / SB / TR |  |  |
| Target of Test | The target of this test case is to check that : The on-board equipment requests the driver to acknowledge the level transition LNTC[X]/ LNTC[Y] through the DMI when the max safe front end of the train passes a trackside defined location in rear of the level transition border(location defined in the level transition announcement with L_ACKLEVELTR). |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.7.2 table - Output information Level transition announcement |  |
|  |  | Subset-026-4.7.2 table - Input information Ackn of level transition |  |
|  |  | Subset-026-5.10.4.1 |  |
|  |  | Subset-026-5.10.4.1 a) |  |
|  |  | Subset-026-5.10.4.4 |  |
|  |  | Subset-026-5.10.4.4 row L NTC column L NTC |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The symbol LE09 requesting driver to acknowledge the Level transition when the max safe front end of the train reaches the location defined by L_ACKLEVELTR is DISPLAYED. The driver acknowledges the Level transition on the DMI After driver acknowledgment level NTC announcement, the symbol LE08 is DISPLAYED. Check on the JRU that: The displayed symbol LE09 is RECORDED. The driver acknowledgement of the level transition LNTC[X]/LNTC[Y] is RECORDED. The displayed symbol LE08 is RECORDED. |
| Constraints | The driver acknowledges the level transition LNTC[X]/LNTC[Y] inside the defined area (L_ACKLEVELTR: Length of the acknowledgement area in rear of the required level). |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 5 / 6 / 7 / 13 | SL / SB / TR / SN |
| Level Transition Order | STORED | Level transition to NTC[Y] at further location with an acknowledgement window before the level transition border |
| ERTMS/ETCS level | 1 | LNTC[X] |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level NTC" is displayed The Level symbol "Level NTC announcement" is displayed | O | DMI | DMI Object: Symbol LE02 DMI Object: Symbol LE08 (Except in SB/SL mode) |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The Level symbol "Level NTC announcement" is removed | O | DMI | The max safe front end of the train enters the acknowledgement window (defined in the level transition announcement with L_ACKLEVELTR). Symbol LE08 is REMOVED. Except if in SL mode | N/A | N/A | - |
| 2 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit08=0>) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The Level symbol "Acknowledge Level NTC announcement" is displayed | O | DMI | DMI Object: Symbol LE09 | N/A | N/A | - |
| 4 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit09=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | The driver acknowledges "Level NTC announcement" | I | DMI | - | N/A | N/A | - |
| 6 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 1010) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | The Level symbol "Acknowledge Level NTC announcement" is removed | O | DMI | DMI Object: Symbol LE09 | N/A | N/A | - |
| 8 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit09=0>) is recorded | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | The Level symbol "Level NTC announcement" is displayed | O | DMI | DMI Object: Symbol LE08 | N/A | N/A | - |
| 10 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit08=1>) is recorded | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 5 / 7 / 13 | SL / TR / SN |
| Level Transition Order | STORED | Level transition to NTC[Y] |
| ERTMS/ETCS level | 1 | LNTC[X] |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level NTC announcement" is displayed | O | DMI | DMI Object: Symbol LE08 The acknowledgement request for the level transition, symbol LE09 is REMOVED. (Except in SL mode) |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 37

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-Board equipment |  |  |
| Tested Feature | 5.10.4: Acknowledgement of the level transition |  | 5100400 |
| Test Case of Feature | The train has passed over the Level Transition border and the On-board equipment has switched from LNTC [X ] to Level NTC[Y] upon receipt of the order. The driver acknowledges within the driver acknowledgement time [5 seconds] after the Level transition. |  | 37 |
| Applicable Mode/Level Combinations | LNTC[Y]: TR / SN |  |  |
| Target of Test | The target of this test is to check that: The On-board does not apply the brakes if the driver acknowledges the Level Transition LNTC[X]/LNTC[Y] within the driver acknowledgement time [5seconds] after the Level Transition. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.7.2 table - Input information Ackn of level transition |  |
|  |  | Subset-026-5.10.4.1 |  |
|  |  | Subset-026-5.10.4.1 b) |  |
|  |  | Subset-026-5.10.4.4 |  |
|  |  | Subset-026-5.10.4.4 row L NTC column L NTC |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The symbol LE02 (current Level NTC [Y]) is DISPLAYED The acknowledgement request of level transition, symbol LE09 is DISPLAYED After the driver acknowledgement the acknowledgement request for Level Transition, symbol LE09 is REMOVED. Check on the TIU that : The service brake is NOT APPLIED. Check on the JRU that: The displayed symbol is RECORDED. The driver acknowledgement of the level transition LNTC[X]/LNTC[Y] is RECORDED. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 / 13 | TR / SN |
| ERTMS/ETCS level | 1 | LNTC[Y] |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Service brake not commanded | O | TIU | - |
| The Level symbol "Level NTC" is displayed | O | DMI | DMI Object: Symbol LE02 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The Level symbol "Acknowledge Level NTC announcement" is displayed | O | DMI | DMI Object: Symbol LE09 | N/A | N/A | - |
| 2 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit09=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The driver acknowledges "Level NTC announcement" | I | DMI | The driver acknowledges the Level Transition within the driver acknowledgement time [five seconds] after the Level Transition. | N/A | N/A | - |
| 4 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 1010) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | The Level symbol "Acknowledge Level NTC announcement" is removed | O | DMI | DMI Object: Symbol LE09 | N/A | N/A | - |
| 6 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit09=0>) is recorded | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 / 13 | TR / SN |
| ERTMS/ETCS level | 1 | LNTC[Y] |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level NTC" is displayed | O | DMI | DMI Object: Symbol LE02 The acknowledgement request for Level Transition, symbol LE09 is REMOVED. |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 38

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.4: Acknowledgement of the level transition |  | 5100400 |
| Test Case of Feature | The train has passed over the Level Transition border and the On-board equipment has switched from LNTC[X] to Level NTC[Y], upon receipt of the order. The driver does not acknowledge within the driver acknowledgement time [5 seconds] after the Level transition. The service brake is applied. The driver acknowledges and the brakes are released. |  | 38 |
| Applicable Mode/Level Combinations | LNTC[Y]: SN / TR |  |  |
| Target of Test | The target of this test is to check that: Since the driver does not acknowledge the Level Transition within the driver acknowledgement time [5 seconds] after the Level transition, Service Brake is APPLIED. Service Brake is RELEASED when the driver acknowledges the Level Transition. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.14.1.2 |  |
|  |  | Subset-026-4.7.2 table - Input information Ackn of level transition |  |
|  |  | Subset-026-5.10.4.1 |  |
|  |  | Subset-026-5.10.4.1 b) |  |
|  |  | Subset-026-5.10.4.2 |  |
|  |  | Subset-026-5.10.4.4 |  |
|  |  | Subset-026-5.10.4.4 row L NTC column L NTC |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The symbol LE02 (current level LNTC[Y]) is DISPLAYED. The acknowledgement request symbol LE09 is DISPLAYED Service brake application is DISPLAYED. After the driver acknowledges the acknowledgement, symbol LE09 is REMOVED. Check on the TIU that : Service brake is APPLIED. Service brake is RELEASED when the driver acknowledges the Level Transition. Check on the JRU that: The displayed symbol LE09 is RECORDED. Service Brake intervention is RECORDED. The driver acknowledgement of the level transition LNTC[X]/LNTC[Y] is RECORDED. Service Brake release is RECORDED. |
| Constraints | If the Service Brake were not available for any reason, Emergency Brake shall be used. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 / 13 | TR / SN |
| ERTMS/ETCS level | 1 | LNTC[Y] |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Service brake not commanded | O | TIU | - |
| The Level symbol "Level NTC" is displayed | O | DMI | DMI Object: Symbol LE02 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The Level symbol "Acknowledge Level NTC announcement" is displayed | O | DMI | DMI Object: Symbol LE09 | N/A | N/A | - |
| 2 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit09=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | NOT The driver acknowledges "Level NTC announcement" | I | DMI | The driver does not acknowledge the Level Transition within the driver acknowledgement time [f5 seconds] after the Level Transition. | N/A | N/A | - |
| 4 | N/A | N/A | Service brake commanded | O | TIU | - | N/A | N/A | - |
| 5 | N/A | N/A | The Status symbol "Service Brake intervention" is displayed | O | DMI | - | N/A | N/A | - |
| 6 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | The driver acknowledges "Level NTC announcement" | I | DMI | - | N/A | N/A | - |
| 8 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 1010) is recorded | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | Service brake not commanded | O | TIU | - | N/A | N/A | - |
| 10 | N/A | N/A | The Status symbol "Service Brake intervention" is removed | O | DMI | - | N/A | N/A | - |
| 11 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=0) is recorded | O | JRU | - | N/A | N/A | - |
| 12 | N/A | N/A | The Level symbol "Acknowledge Level NTC announcement" is removed | O | DMI | DMI Object: Symbol LE09 | N/A | N/A | - |
| 13 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit09=0>) is recorded | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 / 13 | TR / SN |
| ERTMS/ETCS level | 1 | LNTC[Y] |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Service brake not commanded | O | TIU | - |
| The Level symbol "Level NTC" is displayed | O | DMI | DMI Object: Symbol LE02 The acknowledgement request for Level Transition, symbol LE09 is REMOVED. |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 39

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.4: Acknowledgement of the level transition |  | 5100400 |
| Test Case of Feature | The train enters in the acknowledgement window. As the train is running in NL mode the driver is NOT requested to acknowledge the level transition |  | 39 |
| Applicable Mode/Level Combinations | L0 / L1 / L2 / L3 / LNTC : NL |  |  |
| Target of Test | The target of this test case is to check that : When the max safe front end of the train has passed a trackside defined location in rear of the level transition border than (location defined in the level transition announcement with L_ACKLEVELTR), The on-board equipment does NOT request the driver to acknowledge the level transition. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-5.10.4.1 |  |
|  |  | Subset-026-5.10.4.1 a) |  |
|  |  | Subset-026-5.10.4.1.1 |  |
|  |  | Subset-026-5.10.4.4 row L0 column L NTC |  |
|  |  | Subset-026-5.10.4.4 row L1 column L0 |  |
|  |  | Subset-026-5.10.4.4 row L1 column L NTC |  |
|  |  | Subset-026-5.10.4.4 row L2 column L0 |  |
|  |  | Subset-026-5.10.4.4 row L2 column L NTC |  |
|  |  | Subset-026-5.10.4.4 row L3 column L0 |  |
|  |  | Subset-026-5.10.4.4 row L3 column L NTC |  |
|  |  | Subset-026-5.10.4.4 row L NTC column L0 |  |
|  |  | Subset-026-5.10.4.4 row L NTC column L1 |  |
|  |  | Subset-026-5.10.4.4 row L NTC column L2 |  |
|  |  | Subset-026-5.10.4.4 row L NTC column L3 |  |
|  |  | Subset-026-5.10.4.4 row L NTC column L NTC |  |
|  |  | Subset-026-4.7.2 table - Output information Level transition announcement |  |
|  |  | Subset-026-4.7.2 table - Input information Ackn of level transition |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The driver is not requested to acknowledge the Level transition when the max safe front end of the train reaches the location defined by L_ACKLEVELTR. The level transition announcement symbol (one of these symbols LE06, LE08, LE10, LE12, LE14) is DISPLAYED Check on the JRU that : NOT The displayed symbol LE07, LE09, LE11, LE13 and LE15 is RECORDED. |
| Constraints | Depending on the required level transition, the acknowledgment symbols for the level transition, LE07, LE09, LE11, LE13 and LE15, are NOT DISPLAYED. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 11 | NL |
| Level Transition Order | STORED | Level transition at further location with an acknowledgement window before the level transition border |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0/ LNTC /L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level 0/ NTC / 1 / 2 / 3 " is displayed The Level symbol "Level 0/ NTC / 1 / 2 / 3 announcement" is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | NOT The Level symbol "Acknowledge Level NTC / 0 / 1 / 2 / 3 announcement" is displayed | O | DMI | The max safe front end of the train enters the acknowledgement window (defined in the level transition announcement with L_ACKLEVELTR). The on-board equipment does not require any acknowledgment from the drive. The symbol requesting driver acknowledgment for level transition is NOT DISPLAYED. Symbols as LE07, LE09, LE11, LE13 and LE15 are not displayed. | N/A | N/A | - |
| 2 | N/A | N/A | NOT DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit07/Bit09/Bit11/ Bit13/ Bit15=1>) is recorded | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 11 | NL |
| Level Transition Order | STORED | Level transition at further location |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0/ LNTC /L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level NTC / 0 / 1 / 2 / 3 / announcement" is displayed | O | DMI | One of the symbols LE06, LE08, LE10, LE12, LE14 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 40

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-Board equipment |  |  |
| Tested Feature | 5.10.4: Acknowledgement of the level transition |  | 5100400 |
| Test Case of Feature | The train has passed over the Level Transition border and the on-board equipment has switched to the next level upon receipt of the order to switch to the new level immediately. As the train is running in NL mode the driver is not requested to acknowledge the level transition. |  | 40 |
| Applicable Mode/Level Combinations | L0 / L1 / L2 / L3 / LNTC : NL |  |  |
| Target of Test | The target of this test is to check that: After level transition upon receipt of the order to switch to the new level immediately, the on-board equipment does NOT request the driver to acknowledge the level transition. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.7.2 table - Input information Ackn of level transition |  |
|  |  | Subset-026-5.10.4.1 |  |
|  |  | Subset-026-5.10.4.1 b) |  |
|  |  | Subset-026-5.10.4.1.1 |  |
|  |  | Subset-026-5.10.4.4 row L0 column L NTC |  |
|  |  | Subset-026-5.10.4.4 row L1 column L0 |  |
|  |  | Subset-026-5.10.4.4 row L1 column L NTC |  |
|  |  | Subset-026-5.10.4.4 row L2 column L0 |  |
|  |  | Subset-026-5.10.4.4 row L2 column L NTC |  |
|  |  | Subset-026-5.10.4.4 row L3 column L0 |  |
|  |  | Subset-026-5.10.4.4 row L3 column L NTC |  |
|  |  | Subset-026-5.10.4.4 row L NTC column L0 |  |
|  |  | Subset-026-5.10.4.4 row L NTC column L1 |  |
|  |  | Subset-026-5.10.4.4 row L NTC column L2 |  |
|  |  | Subset-026-5.10.4.4 row L NTC column L3 |  |
|  |  | Subset-026-5.10.4.4 row L NTC column L NTC |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The acknowledgement symbol for level transition is NOT DISPLAYED. Check on the JRU that : NOT The displayed symbol LE07, LE09, LE11, LE13 and LE15 is RECORDED. |
| Constraints | Depending on the required level transition, the acknowledgment symbols for the level transition are LE07, LE09, LE11, LE13, LE15 |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 11 | NL |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / L1 / L2 / L3 / LNTC |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level 0/ NTC / 1 / 2 / 3 " is displayed | O | DMI | One of the symbols LE01, LE02, LE03, LE04 and LE05 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | NOT The Level symbol "Acknowledge Level NTC / 0 / 1 / 2 / 3 announcement" is displayed | O | DMI | The train is inside the new level area after executing a level transition upon receipt of the order to switch to the new level immediately. The on-board equipment does not request any acknowledgment from the drive. The symbol requesting driver acknowledgment of level transition is NOT DISPLAYED. Symbols as LE07, LE09, LE11, LE13 and LE15 are not displayed. | N/A | N/A | - |
| 2 | N/A | N/A | NOT DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit07/Bit09/Bit11/ Bit13/ Bit15=1>) is recorded | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 11 | NL |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / L1 / L2 / L3 / LNTC |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level NTC / 0 / 1 / 2 / 3" is displayed | O | DMI | One of the symbols LE01, LE02, LE03, LE04 and LE05 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 41

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.4: Acknowledgement of the level transition |  | 5100400 |
| Test Case of Feature | The train enters in the acknowledgement window, and the driver is requested to acknowledge the transition from L0 to LNTC. The driver does not acknowledge before entering the LNTC area. |  | 41 |
| Applicable Mode/Level Combinations | L0 : UN |  |  |
| Target of Test | The target of this test case is to check that : The on-board equipment requests the driver to acknowledge the level transition L0 / LNTC through the DMI when the max safe front end of the train passes a trackside defined location in rear of the level transition border (location defined in the level transition announcement with L_ACKLEVELTR) The driver does not acknowledge the level transition announcement and no brakes are applied, |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.7.2 table - Output information Level transition announcement |  |
|  |  | Subset-026-5.10.4.1 |  |
|  |  | Subset-026-5.10.4.1 a) |  |
|  |  | Subset-026-5.10.4.4 |  |
|  |  | Subset-026-5.10.4.4 row L0 column L NTC |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The symbol LE01 (current level L0) is DISPLAYED The symbol LE09 requesting the driver to acknowledge the Level transition when the max safe front end of the train reaches the location defined by L_ACKLEVELTR is DISPLAYED. The driver does not acknowledge the Level transition before entering the LNTC area Check on the JRU that: The displayed symbol LE09 requesting the acknowledgement is RECORDED |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 4 | UN |
| Level Transition Order | STORED | Level transition to LNTC at further location with an acknowledgement window before the level transition border |
| ERTMS/ETCS level | 0 | L0 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Service brake not commanded | O | TIU | - |
| The Level symbol "Level 0" is displayed The Level symbol "Level NTC announcement" is displayed | O | DMI | DMI Object: Symbol LE01 DMI Object: Symbol LE08 (Except in SB mode) |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The Level symbol "Level NTC announcement" is removed | O | DMI | The max safe front end of the train enters the acknowledgement window (defined in the level transition announcement with L_ACKLEVELTR). The symbol LE08 is REMOVED. | N/A | N/A | - |
| 2 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit08=0>) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The Level symbol "Acknowledge Level NTC announcement" is displayed | O | DMI | DMI Object: Symbol LE09 | N/A | N/A | - |
| 4 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit09=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | NOT The driver acknowledges "Level NTC announcement" | I | DMI | The driver does not acknowledge before entering the LNTC area. | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 4 | UN |
| Level Transition Order | STORED | Level transition to LNTC |
| ERTMS/ETCS level | 0 | L0 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Service brake not commanded | O | TIU | - |
| The Level symbol "Acknowledge Level NTC announcement" is displayed | O | DMI | DMI Object: Symbol LE09 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 42

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.4: Acknowledgement of the level transition |  | 5100400 |
| Test Case of Feature | The train enters in the acknowledgement window, and the driver is requested to acknowledge the transition from L1 to LNTC. The driver does not acknowledge before entering the LNTC area. |  | 42 |
| Applicable Mode/Level Combinations | L1 : FS / LS / OS / SR / SB / TR |  |  |
| Target of Test | The target of this test case is to check that : The on-board equipment requests the driver to acknowledge the level transition L1 / LNTC through the DMI when the max safe front end of the train passes a trackside defined location in rear of the level transition border (location defined in the level transition announcement with L_ACKLEVELTR). The driver does not acknowledge the level transition announcement and no brakes are applied, |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.7.2 table - Output information Level transition announcement |  |
|  |  | Subset-026-5.10.4.1 |  |
|  |  | Subset-026-5.10.4.1 a) |  |
|  |  | Subset-026-5.10.4.4 |  |
|  |  | Subset-026-5.10.4.4 row L1 column L NTC |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The symbol LE03 (current level L1) is DISPLAYED The symbol LE09 requesting the driver to acknowledge the Level transition when the max safe front end of the train reaches the location defined by L_ACKLEVELTR is DISPLAYED. The driver does not acknowledge the Level transition before entering the LNTC area Check on the JRU that: The displayed symbol LE09 requesting the acknowledgement is RECORDED |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 7 / 12 | FS / OS / SR / SB / TR / LS |
| Level Transition Order | STORED | Level transition to LNTC at further location with an acknowledgement window before the level transition border |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level 1" is displayed The Level symbol "Level NTC announcement" is displayed | O | DMI | DMI Object: Symbol LE03 DMI Object: Symbol LE08 (Except in SB mode) |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The Level symbol "Level NTC announcement" is removed | O | DMI | The max safe front end of the train enters the acknowledgement window (defined in the level transition announcement with L_ACKLEVELTR). The symbol LE08 is REMOVED. | N/A | N/A | - |
| 2 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit08=0>) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The Level symbol "Acknowledge Level NTC announcement" is displayed | O | DMI | DMI Object: Symbol LE09 | N/A | N/A | - |
| 4 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit09=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | NOT The driver acknowledges "Level NTC announcement" | I | DMI | The driver does not acknowledge before entering the LNTC area. | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 7 / 12 | FS / OS / SR / SB / TR / LS |
| Level Transition Order | STORED | Level transition to LNTC |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Service brake not commanded | O | TIU | - |
| The Level symbol "Acknowledge Level NTC announcement" is displayed | O | DMI | DMI Object: Symbol LE09 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 43

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.4: Acknowledgement of the level transition |  | 5100400 |
| Test Case of Feature | The train enters in the acknowledgement window, and the driver is requested to acknowledge the transition from L2 to LNTC. The driver does not acknowledge before entering the LNTC area. |  | 43 |
| Applicable Mode/Level Combinations | L2 : FS / LS / OS / SR / SB / TR |  |  |
| Target of Test | The target of this test case is to check that : The on-board equipment requests the driver to acknowledge the level transition L2 / LNTC through the DMI when the max safe front end of the train passes a trackside defined location in rear of the level transition border (location defined in the level transition announcement with L_ACKLEVELTR). The driver does not acknowledge the level transition announcement and no brakes are applied, |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.7.2 table - Output information Level transition announcement |  |
|  |  | Subset-026-5.10.4.1 |  |
|  |  | Subset-026-5.10.4.1 a) |  |
|  |  | Subset-026-5.10.4.4 |  |
|  |  | Subset-026-5.10.4.4 row L2 column L NTC |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The symbol LE04 (current level L2) is DISPLAYED The symbol LE09 requesting the driver to acknowledge the Level transition when the max safe front end of the train reaches the location defined by L_ACKLEVELTR is DISPLAYED. The driver does not acknowledge the Level transition before entering the LNTC area Check on the JRU that: The displayed symbol LE09 requesting the acknowledgement is RECORDED |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 7 / 12 | FS / OS / SR / SB / TR / LS |
| Level Transition Order | STORED | Level transition to LNTC at further location with an acknowledgement window before the level transition border |
| ERTMS/ETCS level | 3 | L2 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Service brake not commanded | O | TIU | - |
| The Level symbol "Level 2" is displayed The Level symbol "Level NTC announcement" is displayed | O | DMI | DMI Object: Symbol LE04 DMI Object: Symbol LE08 (Except in SB mode) |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The Level symbol "Level NTC announcement" is removed | O | DMI | The max safe front end of the train enters the acknowledgement window (defined in the level transition announcement with L_ACKLEVELTR). The displayed symbol LE08 is REMOVED. | N/A | N/A | - |
| 2 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit08=0>) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The Level symbol "Acknowledge Level NTC announcement" is displayed | O | DMI | DMI Object: Symbol LE09 | N/A | N/A | - |
| 4 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit09=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | NOT The driver acknowledges "Level NTC announcement" | I | DMI | The driver does not acknowledge before entering the LNTC area. | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 7 / 12 | FS / OS / SR / SB / TR / LS |
| Level Transition Order | STORED | Level transition to LNTC |
| ERTMS/ETCS level | 3 | L2 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Service brake not commanded | O | TIU | - |
| The Level symbol "Acknowledge Level NTC announcement" is displayed | O | DMI | DMI Object: Symbol LE09 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 44

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.4: Acknowledgement of the level transition |  | 5100400 |
| Test Case of Feature | The train enters in the acknowledgement window, and the driver is requested to acknowledge the transition from L3 to L0. The driver does not acknowledge before entering the L0 area. |  | 44 |
| Applicable Mode/Level Combinations | L3 : FS / LS / OS / SR /SB / TR |  |  |
| Target of Test | The target of this test case is to check that : The on-board equipment requests the driver to acknowledge the level transition L3/ L0 through the DMI when the max safe front end of the train passes a trackside defined location in rear of the level transition border (location defined in the level transition announcement with L_ACKLEVELTR). The driver does not acknowledge the level transition announcement and no brakes are applied, |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.7.2 table - Output information Level transition announcement |  |
|  |  | Subset-026-5.10.4.1 |  |
|  |  | Subset-026-5.10.4.1 a) |  |
|  |  | Subset-026-5.10.4.4 |  |
|  |  | Subset-026-5.10.4.4 row L3 column L0 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The symbol LE05 (current level L3) is DISPLAYED The symbol LE07 requesting the driver to acknowledge the Level transition when the max safe front end of the train reaches the location defined by L_ACKLEVELTR. is DISPLAYED The driver does not acknowledge the Level transition before entering the L0 area. Check on the JRU that: The displayed symbol LE07requesting the acknowledgement is RECORDED |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 7 / 12 | FS / OS / SR / SB / TR / LS |
| Level Transition Order | STORED | Level transition to L0 at further location with an acknowledgement window before the level transition border |
| ERTMS/ETCS level | 3 | L2 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Service brake not commanded | O | TIU | - |
| The Level symbol "Level 3" is displayed The Level symbol "Level 0 announcement" is displayed | O | DMI | DMI Object: Symbol LE05 DMI Object: Symbol LE06 (Except in SB mode) |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The Level symbol "Level 0 announcement" is removed | O | DMI | The max safe front end of the train enters the acknowledgement window (defined in the level transition announcement with L_ACKLEVELTR). The symbol LE06 is REMOVED. | N/A | N/A | - |
| 2 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit06=0>) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The Level symbol "Acknowledge Level 0 announcement" is displayed | O | DMI | DMI Object: Symbol LE07 | N/A | N/A | - |
| 4 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit07=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | NOT The driver acknowledges "Level 0 announcement" | I | DMI | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 7 / 12 | FS / OS / SR / SB / TR / LS |
| Level Transition Order | STORED | Level transition to L0 |
| ERTMS/ETCS level | 3 | L2 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Acknowledge Level 0 announcement" is displayed | O | DMI | DMI Object: Symbol LE07 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 45

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.4: Acknowledgement of the level transition |  | 5100400 |
| Test Case of Feature | The train enters in the acknowledgement window, and the driver is requested to acknowledge the transition from L3 to LNTC. The driver does not acknowledge before entering the LNTC area. |  | 45 |
| Applicable Mode/Level Combinations | L3 : FS / LS / OS / SR / SB / TR |  |  |
| Target of Test | The target of this test case is to check that : The on-board equipment requests the driver to acknowledge the level transition L3 / LNTC through the DMI when the max safe front end of the train passes a trackside defined location in rear of the level transition border (location defined in the level transition announcement with L_ACKLEVELTR). The driver does not acknowledge the level transition announcement and no brakes are applied, |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.7.2 table - Output information Level transition announcement |  |
|  |  | Subset-026-5.10.4.1 |  |
|  |  | Subset-026-5.10.4.1 a) |  |
|  |  | Subset-026-5.10.4.4 |  |
|  |  | Subset-026-5.10.4.4 row L3 column L NTC |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The symbol LE05 (current level L3) is DISPLAYED The symbol LE09 requesting the driver to acknowledge the Level transition when the max safe front end of the train reaches the location defined by L_ACKLEVELTR is DISPLAYED. The driver does not acknowledge the Level transition before entering the LNTC area Check on the JRU that: The displayed symbol LE09 requesting the acknowledgement is RECORDED |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 7 / 12 | FS / OS / SR / SB / TR / LS |
| Level Transition Order | STORED | Level transition to LNTC at further location with an acknowledgement window before the level transition border |
| ERTMS/ETCS level | 4 | L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Service brake not commanded | O | TIU | - |
| The Level symbol "Level 3" is displayed The Level symbol "Level NTC announcement" is displayed | O | DMI | DMI Object: Symbol LE05 DMI Object: Symbol LE08 (Except in SB mode) |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The Level symbol "Level NTC announcement" is removed | O | DMI | The max safe front end of the train enters the acknowledgement window (defined in the level transition announcement with L_ACKLEVELTR). The displayed symbol LE08 is REMOVED. | N/A | N/A | - |
| 2 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit08=0>) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The Level symbol "Acknowledge Level NTC announcement" is displayed | O | DMI | DMI Object: Symbol LE09 | N/A | N/A | - |
| 4 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit09=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | NOT The driver acknowledges "Level NTC announcement" | I | DMI | The driver does not acknowledge before entering the LNTC area. | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 7 / 12 | FS / OS / SR / SB / TR / LS |
| Level Transition Order | STORED | Level transition to LNTC |
| ERTMS/ETCS level | 4 | L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Service brake not commanded | O | TIU | - |
| The Level symbol "Acknowledge Level NTC announcement" is displayed | O | DMI | DMI Object: Symbol LE09 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 46

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.4: Acknowledgement of the level transition |  | 5100400 |
| Test Case of Feature | The train enters in the acknowledgement window, and the driver is requested to acknowledge the transition from LNTC to L0. The driver does not acknowledge before entering the L0 area. |  | 46 |
| Applicable Mode/Level Combinations | LNTC : SN |  |  |
| Target of Test | The target of this test case is to check that : The on-board equipment requests the driver to acknowledge the level transition LNTC / L0 through the DMI when the max safe front end of the train passes a trackside defined location in rear of the level transition border (location defined in the level transition announcement with L_ACKLEVELTR). The driver does not acknowledge the level transition announcement and no brakes are applied, |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.7.2 table - Output information Level transition announcement |  |
|  |  | Subset-026-5.10.4.1 |  |
|  |  | Subset-026-5.10.4.1 a) |  |
|  |  | Subset-026-5.10.4.4 |  |
|  |  | Subset-026-5.10.4.4 row L NTC column L0 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The symbol LE02 (current level LNTC) is DISPLAYED The symbol LE07 requesting the driver to acknowledge the Level transition when the max safe front end of the train reaches the location defined by L_ACKLEVELTR. is DISPLAYED The driver does not acknowledge the Level transition before entering the L0 area. Check on the JRU that: The displayed symbol LE07 requesting the acknowledgement is RECORDED |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 13 | SN |
| Level Transition Order | STORED | Level transition to L0 at further location with an acknowledgement window before the level transition border |
| ERTMS/ETCS level | 1 | LNTC |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Service brake not commanded | O | TIU | - |
| The Level symbol "Level NTC" is displayed The Level symbol "Level 0 announcement" is displayed | O | DMI | DMI Object: Symbol LE02 DMI Object: Symbol LE06 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The Level symbol "Level 0 announcement" is removed | O | DMI | The max safe front end of the train enters the acknowledgement window (defined in the level transition announcement with L_ACKLEVELTR) The symbol LE06 is REMOVED. | N/A | N/A | - |
| 2 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit06=0>) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The Level symbol "Acknowledge Level 0 announcement" is displayed | O | DMI | DMI Object: Symbol LE07 | N/A | N/A | - |
| 4 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit07=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | NOT The driver acknowledges "Level 0 announcement" | I | DMI | The driver does not acknowledge before entering the L0 area. | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 13 | SN |
| Level Transition Order | STORED | Level transition to L0 |
| ERTMS/ETCS level | 1 | LNTC |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Acknowledge Level 0 announcement" is displayed | O | DMI | DMI Object: Symbol LE07 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 47

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.4: Acknowledgement of the level transition |  | 5100400 |
| Test Case of Feature | The train enters in the acknowledgement window, and the driver is requested to acknowledge the transition from LNTC to L1. The driver does not acknowledge before entering the L1 area. |  | 47 |
| Applicable Mode/Level Combinations | LNTC : SN |  |  |
| Target of Test | The target of this test case is to check that : The on-board equipment requests the driver to acknowledge the level transition LNTC / L1 through the DMI when the max safe front end of the train passes a trackside defined location in rear of the level transition border (location defined in the level transition announcement with L_ACKLEVELTR). The driver does not acknowledge the level transition announcement and no brakes are applied, |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.7.2 table - Output information Level transition announcement |  |
|  |  | Subset-026-5.10.4.1 |  |
|  |  | Subset-026-5.10.4.1 a) |  |
|  |  | Subset-026-5.10.4.4 |  |
|  |  | Subset-026-5.10.4.4 row L NTC column L1 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The symbol LE02 (current level LNTC) is DISPLAYED The symbol LE11 requesting the driver to acknowledge the Level transition when the max safe front end of the train reaches the location defined by L_ACKLEVELTR. is DISPLAYED The driver does not acknowledge the Level transition before entering the L1 area. Check on the JRU that: The displayed symbol LE11 requesting the acknowledgement is RECORDED |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 13 | SN |
| Level Transition Order | STORED | Level transition to L1 at further location with an acknowledgement window before the level transition border |
| ERTMS/ETCS level | 1 | LNTC |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Service brake not commanded | O | TIU | - |
| The Level symbol "Level NTC" is displayed The Level symbol "Level 1 announcement" is displayed | O | DMI | DMI Object: Symbol LE02 DMI Object: Symbol LE10 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The Level symbol "Level 1 announcement" is removed | O | DMI | The max safe front end of the train enters the acknowledgement window (defined in the level transition announcement with L_ACKLEVELTR). The symbol LE10 is REMOVED. | N/A | N/A | - |
| 2 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit10=0>) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The Level symbol "Acknowledge Level 1 announcement" is displayed | O | DMI | DMI Object: Symbol LE11 | N/A | N/A | - |
| 4 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit11=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | NOT The driver acknowledges "Level 1 announcement" | I | DMI | The driver does not acknowledge before entering the L1 area. | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 13 | SN |
| Level Transition Order | STORED | Level transition to L1 |
| ERTMS/ETCS level | 1 | LNTC |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Acknowledge Level 1 announcement" is displayed | O | DMI | DMI Object: Symbol LE11 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 48

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.4: Acknowledgement of the level transition |  | 5100400 |
| Test Case of Feature | The train enters in the acknowledgement window, and the driver is requested to acknowledge the transition from LNTC to L2. The driver does not acknowledge before entering the L2 area. |  | 48 |
| Applicable Mode/Level Combinations | LNTC : SN |  |  |
| Target of Test | The target of this test case is to check that : The on-board equipment requests the driver to acknowledge the level transition LNTC / L2 through the DMI when the max safe front end of the train passes a trackside defined location in rear of the level transition border (location defined in the level transition announcement with L_ACKLEVELTR). The driver does not acknowledge the level transition announcement and no brakes are applied, |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.7.2 table - Output information Level transition announcement |  |
|  |  | Subset-026-5.10.4.1 |  |
|  |  | Subset-026-5.10.4.1 a) |  |
|  |  | Subset-026-5.10.4.4 |  |
|  |  | Subset-026-5.10.4.4 row L NTC column L2 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The symbol LE02 (current level LNTC) is DISPLAYED The symbol LE13, requesting the driver to acknowledge the Level transition when the max safe front end of the train reaches the location defined by L_ACKLEVELTR, is DISPLAYED The driver does not acknowledge the Level transition before entering the L2 area. Check on the JRU that: The displayed symbol LE13 requesting the acknowledgement is RECORDED |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 13 | SN |
| Level Transition Order | STORED | Level transition to L2 at further location with an acknowledgement window before the level transition border |
| ERTMS/ETCS level | 1 | LNTC |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level NTC" is displayed The Level symbol "Level 2 announcement" is displayed | O | DMI | DMI Object: Symbol LE02 DMI Object: Symbol LE12 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The Level symbol "Level 2 announcement" is removed | O | DMI | The max safe front end of the train enters the acknowledgement window (defined in the level transition announcement with L_ACKLEVELTR). The symbol LE12 is REMOVED. | N/A | N/A | - |
| 2 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit12=0>) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The Level symbol "Acknowledge Level 2 announcement" is displayed | O | DMI | DMI Object: Symbol LE13 | N/A | N/A | - |
| 4 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit13=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | NOT The driver acknowledges "Level 2 announcement" | I | DMI | The driver does not acknowledge before entering the L2 area. | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 13 | SN |
| Level Transition Order | STORED | Level transition to L2 at further location with an acknowledgement window before the level transition border |
| ERTMS/ETCS level | 1 | LNTC |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Acknowledge Level 2 announcement" is displayed | O | DMI | DMI Object: Symbol LE13 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 49

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.4: Acknowledgement of the level transition |  | 5100400 |
| Test Case of Feature | The train enters in the acknowledgement window, and the driver is requested to acknowledge the transition from LNTC to L3. The driver does not acknowledge before entering the L3 area. |  | 49 |
| Applicable Mode/Level Combinations | LNTC : SN |  |  |
| Target of Test | The target of this test case is to check that : The on-board equipment requests the driver to acknowledge the level transition LNTC / L3 through the DMI when the max safe front end of the train passes a trackside defined location in rear of the level transition border (location defined in the level transition announcement with L_ACKLEVELTR). The driver does not acknowledge the level transition announcement and no brakes are applied, |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.7.2 table - Output information Level transition announcement |  |
|  |  | Subset-026-5.10.4.1 |  |
|  |  | Subset-026-5.10.4.1 a) |  |
|  |  | Subset-026-5.10.4.4 |  |
|  |  | Subset-026-5.10.4.4 row L NTC column L3 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The symbol LE02 (current level LNTC) is DISPLAYED The symbol LE15, requesting the driver to acknowledge the Level transition when the max safe front end of the train reaches the location defined by L_ACKLEVELTR, is DISPLAYED The driver does not acknowledge the Level transition before entering the L3 area. Check on the JRU that: The displayed symbol LE15 requesting the acknowledgement is RECORDED. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 13 | SN |
| Level Transition Order | STORED | Level transition to L3 at further location with an acknowledgement window before the level transition border |
| ERTMS/ETCS level | 1 | LNTC |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Service brake not commanded | O | TIU | - |
| The Level symbol "Level NTC" is displayed The Level symbol "Level 3 announcement" is displayed | O | DMI | DMI Object: Symbol LE02 DMI Object: Symbol LE14 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The Level symbol "Level 3 announcement" is removed | O | DMI | The max safe front end of the train enters the acknowledgement window (defined in the level transition announcement with L_ACKLEVELTR). The symbol LE14 is REMOVED. | N/A | N/A | - |
| 2 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit14=0>) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The Level symbol "Acknowledge Level 3 announcement" is displayed | O | DMI | DMI Object: Symbol LE15 | N/A | N/A | - |
| 4 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit15=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | NOT The driver acknowledges "Level 3 announcement" | I | DMI | The driver does not acknowledge before entering the L3 area. | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 13 | SN |
| Level Transition Order | STORED | Level transition to L3 |
| ERTMS/ETCS level | 1 | LNTC |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Acknowledge Level 2 announcement" is displayed | O | DMI | DMI Object: Symbol LE13 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 50

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.4: Acknowledgement of the level transition |  | 5100400 |
| Test Case of Feature | The train enters in the acknowledgement window, and the driver is requested to acknowledge the transition from LNTC[X] to LNTC[Y]. The driver does not acknowledge before entering the LNTC[Y] area. |  | 50 |
| Applicable Mode/Level Combinations | LNTC : SN |  |  |
| Target of Test | The target of this test case is to check that : When the train is inside the acknowledgement window within it’s max safe front end of the train, the on-board equipment requests the driver to acknowledge the level transition LNTC [X] / LNTC [Y] through the DMI. The driver does not acknowledges the level transition announcement and no brakes are applied. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.7.2 table - Output information Level transition announcement |  |
|  |  | Subset-026-5.10.4.1 |  |
|  |  | Subset-026-5.10.4.1 a) |  |
|  |  | Subset-026-5.10.4.4 |  |
|  |  | Subset-026-5.10.4.4 row L0 column L NTC |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The symbol LE02 (current level LNTC[X]) is DISPLAYED The symbol LE09 requesting the driver to acknowledge the Level transition when the max safe front end of the train reaches the location defined by L_ACKLEVELTR is DISPLAYED. The driver does not acknowledge the Level transition before entering the LNTC[Y] area Check on the JRU that: The displayed symbol LE09 requesting the acknowledgement is RECORDED |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 13 | SN |
| Level Transition Order | STORED | Level transition to LNTC at further location with an acknowledgement window before the level transition border |
| ERTMS/ETCS level | 1 | LNTC[X] |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Service brake not commanded | O | TIU | - |
| The Level symbol "Level NTC" is displayed The Level symbol "Level NTC announcement" is displayed | O | DMI | DMI Object: Symbol LE02 DMI Object: Symbol LE08 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The Level symbol "Level NTC announcement" is removed | O | DMI | The max safe front end of the train enters the acknowledgement window (defined in the level transition announcement with L_ACKLEVELTR). The symbol LE08 is REMOVED. | N/A | N/A | - |
| 2 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit08=0>) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The Level symbol "Acknowledge Level NTC announcement" is displayed | O | DMI | DMI Object: Symbol LE09 | N/A | N/A | - |
| 4 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit09=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | NOT The driver acknowledges "Level NTC announcement" | I | DMI | The driver does not acknowledge before entering the LNTC[Y] area. | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 13 | SN |
| Level Transition Order | STORED | Level transition to LNTC at further location with an acknowledgement window before the level transition border |
| ERTMS/ETCS level | 1 | LNTC[X] |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Service brake not commanded | O | TIU | - |
| The Level symbol "Acknowledge Level NTC announcement" is displayed | O | DMI | DMI Object: Symbol LE09 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 51

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.4: Acknowledgement of the level transition |  | 5100400 |
| Test Case of Feature | The train enters in the acknowledgement window, and the driver is NOT requested to acknowledge the transition from L0 to L1/ L2/ L3. |  | 51 |
| Applicable Mode/Level Combinations | L0 : UN |  |  |
| Target of Test | The target of this test case is to check that : The on-board equipment does NOT request the driver to acknowledge the level transition from L0 to L1/ L2/ L3 through the DMI when the max safe front end of the train passes over the trackside defined location (defined in the level transition announcement with L_ACKLEVELTR). |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-5.10.4.1 |  |
|  |  | Subset-026-5.10.4.1 a) |  |
|  |  | Subset-026-5.10.4.4 |  |
|  |  | Subset-026-5.10.4.4 row L0 column L1 |  |
|  |  | Subset-026-5.10.4.4 row L0 column L2 |  |
|  |  | Subset-026-5.10.4.4 row L0 column L3 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The symbol LE01 (current level L0) is DISPLAYED. The symbol LE10 / LE12 / LE14 indicating the announcement of L1 / L2 / L3 is DISPLAYED. NOT The symbol LE11 / LE13 / LE15 requesting the driver to acknowledge the Level transition when the max safe front end of the train reaches the location defined by L_ACKLEVELTR is DISPLAYED NOT The driver acknowledges the Level transition on the DMI Check on the JRU that: NOT The displayed symbol LE11 / LE13 / LE15 is RECORDED. NOT The driver acknowledgement of the level transition from L0 to L1 / L2 / L3 is RECORDED. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 4 | UN |
| Level Transition Order | STORED | Transition to level 1/ 2/ 3 at further location |
| ERTMS/ETCS level | 0 | L0 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level 0" is displayed The Level symbol "Level 1/ 2/ 3 announcement" is displayed | O | DMI | DMI Object: Symbol LE01 DMI Object: Symbol LE10/ LE12/ LE14 (Except in SB mode) |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is moving. | I | INT | The max safe front end of the train enters the acknowledgement window (defined in the level transition announcement with L_ACKLEVELTR). | N/A | N/A | - |
| 2 | N/A | N/A | NOT The Level symbol "Acknowledge Level 1/ 2/ 3 announcement" is displayed | O | DMI | DMI Object: Symbol LE1/ LE13/ LE15 | N/A | N/A | - |
| 3 | N/A | N/A | NOT DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit11/ Bit13/ Bit15=1>) is recorded | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 4 | UN |
| Level Transition Order | STORED | Transition to level 1/ 2/ 3 |
| ERTMS/ETCS level | 0 | L0 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level 1/ 2/ 3 announcement" is displayed | O | DMI | DMI Object: Symbol LE10/ LE12/ LE14 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 52

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.4: Acknowledgement of the level transition |  | 5100400 |
| Test Case of Feature | The train enters in the acknowledgement window and the driver is NOT requested to acknowledge the transition from L1 to L2/ L3. |  | 52 |
| Applicable Mode/Level Combinations | L1 : FS / LS / OS / SR / SB / TR |  |  |
| Target of Test | The target of this test case is to check that : The on-board equipment does NOT requests the driver to acknowledge the level transition L1/ L0 through the DMI when the max safe front end of the train passes over the location in rear of the level transition border (defined in the level transition announcement with L_ACKLEVELTR). |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-5.10.4.1 |  |
|  |  | Subset-026-5.10.4.1 a) |  |
|  |  | Subset-026-5.10.4.4 |  |
|  |  | Subset-026-5.10.4.4 row L1 column L2 |  |
|  |  | Subset-026-5.10.4.4 row L1 column L3 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The symbol LE03 (current level L1) is DISPLAYED. The symbol LE12/ LE14 indicating the announcement to L2/ L3 is DISPLAYED NOT The symbol LE13/ LE15 requesting the driver to acknowledge the Level transition when the max safe front end of the train reaches the location defined by L_ACKLEVELTR is DISPLAYED NOT The driver acknowledges the Level transition on the DMI. Check on the JRU that: NOT The displayed symbol LE13/ LE15 is RECORDED. NOT Driver’s acknowledgement of the level transition from L1 to L2/ L3 is RECORDED. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2/ 6 / 7/ 12 | FS / OS / SR / SB / TR / LS |
| Level Transition Order | STORED | Level transition to L2/ L3 at further location |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level 1" is displayed The Level symbol "Level 2/ 3 announcement" is displayed | O | DMI | DMI Object: Symbol LE03 DMI Object: Symbol LE12/ LE14 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The train is moving. | I | INT | The max safe front end of the train enters the acknowledgement window (defined in the level transition announcement with L_ACKLEVELTR). | N/A | N/A | - |
| 2 | N/A | N/A | NOT The Level symbol "Acknowledge Level 2/ 3 announcement" is displayed | O | DMI | DMI Object: Symbol LE13/ LE15 | N/A | N/A | - |
| 3 | N/A | N/A | NOT DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=< Bit13/ Bit15=1>) is recorded | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2/ 6 / 7/ 12 | FS / OS / SR / SB / TR / LS |
| Level Transition Order | STORED | Level transition to L2/ L3 |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level 2/ 3 announcement" is displayed | O | DMI | DMI Object: Symbol LE12/ LE14 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 53

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.4: Acknowledgement of the level transition |  | 5100400 |
| Test Case of Feature | The train enters in the acknowledgement window, and the driver is NOT requested to acknowledge the transition from L2 to L1/ L3. |  | 53 |
| Applicable Mode/Level Combinations | L2: FS / LS / OS / SR / SB / TR |  |  |
| Target of Test | The target of this test case is to check that : The on-board equipment does NOT request the driver to acknowledge the level transition from L2 to L1/ L3 through the DMI when the max safe front end of the train passes over the location (defined in the level transition announcement with L_ACKLEVELTR). |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-5.10.4.1 |  |
|  |  | Subset-026-5.10.4.1 a) |  |
|  |  | Subset-026-5.10.4.4 |  |
|  |  | Subset-026-5.10.4.4 row L2 column L1 |  |
|  |  | Subset-026-5.10.4.4 row L2 column L3 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The symbol LE04 (current level L2) is DISPLAYED. The symbol LE10/ LE14 indicating the announcement to L1/ L3 is DISPLAYED. NOT The symbol LE11/ LE15 requesting the driver to acknowledge the Level transition when the max safe front end of the train reaches the location defined by L_ACKLEVELTR is DISPLAYED. NOT The driver acknowledges the Level transition on the DMI Check on the JRU that: NOT Displayed symbol LE11/ LE13 is RECORDED. NOT The driver acknowledgement of the level transition from L2 to L1/ L3 is RECORDED |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2/ 6 / 7/ 12 | FS / OS / SR / SB / TR / LS |
| Level Transition Order | STORED | Level transition to L1/ L3 |
| ERTMS/ETCS level | 3 | L2 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level 2" is displayed The Level symbol "Level 1/ 3 announcement" is displayed | O | DMI | DMI Object: Symbol LE04 DMI Object: Symbol LE10/ 14 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The train is moving. | I | INT | The max safe front end of the train enters the acknowledgement window (defined in the level transition announcement with L_ACKLEVELTR). | N/A | N/A | - |
| 2 | N/A | N/A | NOT The Level symbol "Acknowledge Level 1/ 3 announcement" is displayed | O | DMI | DMI Object: Symbol LE11/ LE15 | N/A | N/A | - |
| 3 | N/A | N/A | NOT DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit11/ Bit15=1>) is recorded | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2/ 6 / 7/ 12 | FS / OS / SR / SB / TR / LS |
| Level Transition Order | STORED | Transition to L1/ L3 |
| ERTMS/ETCS level | 3 | L2 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level 1/ 3 announcement" is displayed | O | DMI | DMI Object: Symbol LE10/ LE14 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 54

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.4: Acknowledgement of the level transition |  | 5100400 |
| Test Case of Feature | The train enters in the acknowledgement window, and the driver is NOT requested to acknowledge the transition from L3 to L1/ L2. |  | 54 |
| Applicable Mode/Level Combinations | L3 : FS / LS / OS / SR / SB / TR |  |  |
| Target of Test | The target of this test case is to check that : The on-board equipment does NOT requests the driver to acknowledge the level transition from L3 to L1/ L2 through the DMI when the max safe front end of the train passes over the location (defined in the level transition announcement with L_ACKLEVELTR). |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-5.10.4.1 |  |
|  |  | Subset-026-5.10.4.1 a) |  |
|  |  | Subset-026-5.10.4.4 |  |
|  |  | Subset-026-5.10.4.4 row L3 column L1 |  |
|  |  | Subset-026-5.10.4.4 row L3 column L2 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The symbol LE05 (current level L3) is DISPLAYED. The symbol LE10/ LE12 indicating the announcement to L1/ L2 is DISPLAYED NOT The symbol LE11/ LE13 requesting driver to acknowledge the Level transition when the max safe front end of the train reaches the location defined by L_ACKLEVELTR is DISPLAYED. NOT The driver acknowledges the Level transition on the DMI After driver acknowledgment, the symbol LE06 indicating the announcement to L0 is DISPLAYED Check on the JRU that: NOT The displayed symbol LE11/ LE13 is RECORDED NOT Driver acknowledgement of the level transition L3/L0 is RECORDED. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 7 / 12 | FS / OS / SR / SB / TR / LS |
| Level Transition Order | STORED | Level transition to L1/ L2 at further location |
| ERTMS/ETCS level | 4 | L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level 3" is displayed The Level symbol "Level 1/ 2 announcement" is displayed | O | DMI | DMI Object: Symbol LE05 DMI Object: Symbol LE10/ LE12 (Except in SB mode) |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The train is moving. | I | INT | The max safe front end of the train enters the acknowledgement window (defined in the level transition announcement with L_ACKLEVELTR). | N/A | N/A | - |
| 2 | N/A | N/A | NOT The Level symbol "Acknowledge Level 1/ 2 announcement" is displayed | O | DMI | DMI Object: Symbol LE11/ LE13 | N/A | N/A | - |
| 3 | N/A | N/A | NOT DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit11/ Bit13=1>) is recorded | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 7 / 12 | FS / OS / SR / SB / TR / LS |
| Level Transition Order | STORED | Transition to L1/ L2 |
| ERTMS/ETCS level | 4 | L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level 1/ 2 announcement" is displayed | O | DMI | DMI Object: Symbol LE10/ LE12 is displayed |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 55

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-Board equipment |  |  |
| Tested Feature | 5.10.4: Acknowledgement of the level transition |  | 5100400 |
| Test Case of Feature | The train has passed over the Level Transition border and the On-board equipment has switched from L1 to Level 2/ 3, upon receipt of an order. The driver does NOT acknowledges the Level transition. |  | 55 |
| Applicable Mode/Level Combinations | L2 / L3: FS / LS / OS / SR / TR |  |  |
| Target of Test | The target of this test case is to check that : The on-board equipment does NOT requests the driver to acknowledge the level transition from L1 to L2/L3 upon receipt of order to switch to the new level immediately. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-5.10.4.1 |  |
|  |  | Subset-026-5.10.4.1 b) |  |
|  |  | Subset-026-5.10.4.4 |  |
|  |  | Subset-026-5.10.4.4 row L1 column L2 |  |
|  |  | Subset-026-5.10.4.4 row L1 column L3 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The Level symbol "Level 2/ 3" is displayed NOT The acknowledgement request symbol LE13/ LE15 is DISPLAYED. NOT The driver acknowledges the Level transition on the DMI. Check on the JRU that: NOT The displayed symbol LE13/ LE15 is RECORDED. NOT The driver acknowledgement of the level transition from L1 to Level 2/ 3 is RECORDED. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 7 / 12 | FS / OS / SR / TR / LS |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | O | TIU | - |
| The Level symbol "Level 2/ 3" is displayed | O | DMI | DMI Object: Symbol LE04/ LE05 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | NOT The Level symbol "Acknowledge Level 2/ 3 announcement" is displayed | O | DMI | DMI Object: Symbol LE13/ LE15 | N/A | N/A | - |
| 2 | N/A | N/A | NOT DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit13/ Bit15=1>) is recorded | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 7 / 12 | FS / OS / SR / TR / LS |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level 2/3" is displayed | O | DMI | DMI Object: Symbol LE04/ LE05 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |