| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 3130700 (3.13.7: Determination of Most Restrictive Speed Profile (MRSP)) Total: 7 Test case(s) |  |
| REF: | Subset-076-5-2-3130700 |
| ISSUE: | Version 3.2.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 2.9.1 31.03.2013 | All | First version/review | CEDEX |
| Version 2.9.2 21.11.2013 | ALL | REVIEW | CEDEX |
| Version 2.9.3 03.12.2013 | ALL | The requirement 3.11.1.1 has been added for coverage reasons. | CEDEX |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.2.0 31.03.2017 | All | Baseline 3 1st maintenance release version - Bug fixing | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.13.7: Determination of Most Restrictive Speed Profile (MRSP) |  | 3130700 |
| Test Case of Feature | The Most Restrictive Speed Profile shall be computed from all speed restrictions by selecting the most restrictive parts of each element, some elements being compensated by the train length when requested by trackside. Besides, the MRSP shall be recalculated when any of the elements it is built of is changed. |  | 1 |
| Applicable Mode/Level Combinations | L1: FS |  |  |
| Target of Test | The on-board calculates the most restricted speed taking into account the following speed restrictions: Static Speed Profile (SSP), Temporary Speed Restriction (TSR), Axle Load Speed Profile (ASP), Maximum Train Speed, Level Crossing speed restriction (LX SR), Signalling related speed restriction and Speed restriction to ensure a given permitted braking distance (PBD SR). Besides, the MRSP shall be recalculated when any of the elements it is built of is changed. The train length compensation shall be taken into account for each applicable speed restriction (SSP, ASP and TSR). |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.11.2.4 |  |
|  |  | Subset-026-3.13.7.2 |  |
|  |  | Subset-026-3.13.7.3 |  |
|  |  | Subset-026-3.11.1.1 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI and JRU interfaces that the appropriate change is displayed and registered each time the speed supervision limit changes to a less restrictive value. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 | FS |
| Linking | STORED | - |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Axle load speed profile | NOT STORED | - |
| STM max speed | NOT STORED | - |
| STM system speed/distance | NOT STORED | - |
| Temporary Speed Restrictions | NOT STORED | - |
| Signalling related Speed Restriction | STORED | - |
| Unconditional Emergency Stop | NOT STORED | - |
| Conditional Emergency Stop | NOT STORED | - |
| Train Position | STORED | - |
| Train Data | STORED | - |
| Adhesion factor | NOT STORED | - |
| ERTMS/ETCS level | 2 | L1 |
| Driver ID | STORED | - |
| Train Running Number | STORED | - |
| Track Ahead Free Request | NOT STORED | - |
| Level Crossing information | NOT STORED | - |
| Permitted Braking Distance Information | NOT STORED | - |
| Operated System Version | 010 0000 | Version 2.0, introduced in SRS 3.3.0 |
| Virtual Balise Covers | NOT STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is moving (V_TRAIN=<below V5>) | I | INT | See the meaning of V5 in the following step. | N/A | N/A | - |
| 2 | N/A | N/A | One Balise Group message containing packet 27 (D_STATIC=0, V_STATIC=<V1>, Q_FRONT=0, D_STATIC(1)=<L_AXLELOAD+L_0>, V_STATIC(1)=<Vmax>), packet 51 (D_AXLELOAD=0, V_AXLELOAD=<V2>, Q_FRONT=0), packet 65 (D_TSR=0, V_TSR=<V3>, Q_FRONT=0), packet 88 (D_LX=0, V_LX=<V4>), packet 52 (D_PBDSR=0), packet 5, packet 12 (V_MAIN = <V7>) and packet 21 is received. | I | BTM | The length of the MA shall be long enough to prevent the on-board equipment from braking to the EoA/SvL or LoA during the execution of this TC. The length value "L_0" will be a positive value and V_STATIC(1)=<Vmax> will be the highest speed value of all speed limits, in order to be able to see the supervision of V_STATIC = <V1> and V_MAIN = <V7>. V5 = the speed restriction to ensure permitted braking distance (D_PBD will be set so as to ensure V5 is the minimum of all the speed restrictions). V6 = maximum train speed. The relationship between the values of the speed limits is: V5 < V4 < V3 < V2 < V1 < V7 < V6 < Vmax. The relationships between the relevant length values are the following ones: L_PBDSR < L_LX < L_TSR L_TSR +L_TRAIN < L_AXLELOAD L_TRAIN < L_0 This location is at distance D0. | N/A | N/A | - |
| 3 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 4 | N/A | N/A | The speed and distance monitoring information is displayed (V_PERM = <V5>) | O | DMI | The PS is equal to <V5> (the speed restriction to ensure permitted braking distance). The DMI aspect is coherent with Normal Status in a Ceiling Speed Monitoring section. | N/A | N/A | - |
| 5 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=0; M_SDMSUPSTAT=0; V_PERM = <V5>) is recorded | O | JRU | V5 = the speed restriction to ensure permitted braking distance M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal Status) | N/A | N/A | - |
| 6 | N/A | N/A | The speed and distance monitoring information is displayed (V_PERM =<V4> ) | O | DMI | This location is at distance D0 + L_PBDSR. The PS is equal to <V4> (Permitted speed for the LX speed restriction). The DMI aspect is coherent with Normal Status in a Ceiling Speed Monitoring section. | N/A | N/A | - |
| 7 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=0; M_SDMSUPSTAT=0, V_PERM = <V4>) is recorded | O | JRU | V4 = Permitted speed for the LX speed restriction M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal Status) | N/A | N/A | - |
| 8 | N/A | N/A | The speed and distance monitoring information is displayed (V_PERM =<V3> ) | O | DMI | This location is at distance D0 + L_LX. The PS is equal to <V3> (=V_TSR). The DMI aspect is coherent with Normal Status in a Ceiling Speed Monitoring section. | N/A | N/A | - |
| 9 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=0; M_SDMSUPSTAT=0; V_PERM = <V3>) is recorded | O | JRU | V3 = V_TSR M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal Status) | N/A | N/A | - |
| 10 | N/A | N/A | The speed and distance monitoring information is displayed (V_PERM =<V2> ) | O | DMI | This location is at distance D0 + L_TSR+L_TRAIN. The PS is equal to <V2> (=V_AXLELOAD). The DMI aspect is coherent with Normal Status in a Ceiling Speed Monitoring section. | N/A | N/A | - |
| 11 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=0; M_SDMSUPSTAT=0; V_PERM = <V2>) is recorded | O | JRU | V2 = V_AXLELOAD M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal Status) | N/A | N/A | - |
| 12 | N/A | N/A | The speed and distance monitoring information is displayed (V_PERM =<V1> ) | O | DMI | This location is at distance D0 + L_AXLELOAD + L_TRAIN. The PS is equal to <V1> (=V_STATIC). The DMI aspect is coherent with Normal Status in a Ceiling Speed Monitoring section. | N/A | N/A | - |
| 13 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=0; M_SDMSUPSTAT=0; V_PERM = <V1>) is recorded | O | JRU | V1 = V_STATIC M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal Status) | N/A | N/A | - |
| 14 | N/A | N/A | The speed and distance monitoring information is displayed (V_PERM =<V7> ) | O | DMI | This location is at distance D0 + L_AXLELOAD + L_0 + L_TRAIN. The PS is equal to <V7> (=V_MAIN ). The DMI aspect is coherent with Normal Status in a Ceiling Speed Monitoring section. | N/A | N/A | - |
| 15 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=0; M_SDMSUPSTAT=0; V_PERM = <V7>) is recorded | O | JRU | V7 = V_MAIN M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal Status) | N/A | N/A | - |
| 16 | N/A | N/A | One Balise Group message containing packet 27 (D_STATIC=0, V_STATIC=<V8>), packet 12 (V_MAIN = <V9>) and packet 21 is received. | I | BTM | The length of the MA shall be long enough to prevent the on-board equipment from braking to the EoA/SvL or LoA during the execution of this TC. V6 = maximum train speed. The relation between the values of the speed limits is: V6 < V8 < V9 This location is at distance D1. | N/A | N/A | - |
| 17 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 18 | N/A | N/A | The speed and distance monitoring information is displayed (V_PERM = <V6>) | O | DMI | The PS is equal to V6 (maximum train speed). The DMI aspect is coherent with Normal Status in a Ceiling Speed Monitoring section. | N/A | N/A | - |
| 19 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=0; M_SDMSUPSTAT=0; V_PERM = <V6>) is recorded | O | JRU | V6 = maximum train speed M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal Status) | N/A | N/A | - |

| Step 2: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | 1 | According to SS-040, a BG message containing one Packet 5 must have Q_LINK=1. |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | 0 | - |
| V_STATIC | 7 | FINITE VALUE | V1 |
| Q_FRONT | 1 | 0 | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | 1 | - |
| D_STATIC(k) | 15 | FINITE VALUE | <L_AXLELOAD+L0> |
| V_STATIC(k) | 7 | FINITE VALUE | <Vmax> |
| Q_FRONT(k) | 1 | 0 | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 51 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | FINITE VALUE | - |
| D_TRACKINIT | 15 | FINITE VALUE | Only if Q_TRACKINIT = 1 |
| D_AXLELOAD | 15 | 0 | Only if Q_TRACKINIT = 0, D_AXLELOAD and the following variables follow. |
| L_AXLELOAD | 15 | FINITE VALUE | - |
| Q_FRONT | 1 | 0 | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_AXLELOADCAT(n) | 7 | FINITE VALUE | - |
| V_AXLELOAD(n) | 7 | FINITE VALUE | Speed restriction to be applied if the axle load category of the train ( M_AXLELOADCAT(n). |
| N_ITER | 5 | FINITE VALUE | - |
| D_AXLELOAD(k) | 15 | FINITE VALUE | V2 |
| L_AXLELOAD(k) | 15 | FINITE VALUE | - |
| Q_FRONT(k) | 1 | 0 | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| M_AXLELOADCAT(k,m) | 7 | FINITE VALUE | - |
| V_AXLELOAD(k,m) | 7 | FINITE VALUE | Speed restriction to be applied if the axle load category of the train ( M_AXLELOADCAT(k,m). |
| NID_PACKET | 8 | 65 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_TSR | 8 | FINITE VALUE | - |
| D_TSR | 15 | 0 | - |
| L_TSR | 15 | FINITE VALUE | - |
| Q_FRONT | 1 | 0 | - |
| V_TSR | 7 | FINITE VALUE | V3 |
| NID_PACKET | 8 | 88 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LX | 8 | FINITE VALUE | - |
| D_LX | 15 | 0 | - |
| L_LX | 15 | FINITE VALUE | V4 |
| Q_LXSTATUS | 1 | FINITE VALUE | - |
| V_LX | 7 | FINITE VALUE | Only if Q_LXSTATUS = 1. |
| Q_STOPLX | 1 | FINITE VALUE | Only if Q_LXSTATUS = 1. |
| L_STOPLX | 15 | FINITE VALUE | Only if Q_STOPLX = 1. |
| NID_PACKET | 8 | 52 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | FINITE VALUE | - |
| D_TRACKINIT | 15 | FINITE VALUE | Only if Q_TRACKINIT = 1 |
| D_PBD | 15 | FINITE VALUE | Only if Q_TRACKINIT = 0, D_PBD and the following variables follow. |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill, 1 = uphill. |
| G_PBDSR | 8 | FINITE VALUE | Gradient value to be used for the calculation. |
| Q_PBDSR | 1 | FINITE VALUE | - |
| D_PBDSR | 15 | 0 | - |
| L_PBDSR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_PBD(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill, 1 = uphill. |
| G_PBDSR(k) | 8 | FINITE VALUE | Gradient value to be used for the calculation. |
| Q_PBDSR(k) | 1 | FINITE VALUE | - |
| D_PBDSR(k) | 15 | FINITE VALUE | - |
| L_PBDSR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | 255 |

| Step 2: Eurobalise Telegram (balise 2/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | 1 | According to SS-040, a BG message containing one Packet 5 must have Q_LINK=1. |
| NID_PACKET | 8 | 5 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LINK | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION | 1 | FINITE VALUE | - |
| Q_LINKREACTION | 2 | FINITE VALUE | - |
| Q_LOCACC | 6 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_LINK(k) | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | FINITE VALUE | - |
| Q_LOCACC(k) | 6 | FINITE VALUE | - |
| NID_PACKET | 8 | 12 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | V7 |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out". |
| N_ITER | 5 | FINITE VALUE | Set to zero if V_MAIN = 0 or if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | 255 |

| Step 16: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | 1 | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | V8 |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 12 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | V9 |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out". |
| N_ITER | 5 | FINITE VALUE | Set to zero if V_MAIN = 0 or if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | 255 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 | FS |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| STM max speed | NOT STORED | - |
| STM system speed/distance | NOT STORED | - |
| Signalling related Speed Restriction | STORED | - |
| Unconditional Emergency Stop | NOT STORED | - |
| Conditional Emergency Stop | NOT STORED | - |
| Train Position | STORED | - |
| Train Data | STORED | - |
| Adhesion factor | NOT STORED | - |
| ERTMS/ETCS level | 2 | L1 |
| Driver ID | STORED | - |
| Train Running Number | STORED | - |
| Operated System Version | 010 0000 | Version 2.0, introduced in SRS 3.3.0 |
| Virtual Balise Covers | NOT STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.13.7: Determination of Most Restrictive Speed Profile (MRSP) |  | 3130700 |
| Test Case of Feature | The Most Restrictive Speed Profile shall be computed from all speed restrictions by selecting the most restrictive parts of each element, some elements being compensated by the train length when requested by trackside. Besides, the MRSP shall be recalculated when any of the elements it is built of is changed. |  | 2 |
| Applicable Mode/Level Combinations | L1: LS, OS |  |  |
| Target of Test | The on-board calculates the most restrictive speed taking into account the following speed restrictions: Static Speed Profile (SSP), Temporary Speed Restriction (TSR), Axle Load Speed Profile (ASP), Maximum Train Speed, Level Crossing speed restriction (LX SR), Signalling related speed restriction, Mode related Speed Restriction and Speed restriction to ensure a given permitted braking distance (PBD SR). Besides, the MRSP shall be recalculated when any of the elements it is built of is changed. The train length compensation shall be taken into account for each applicable speed restriction (SSP, ASP and TSR). |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.11.2.4 |  |
|  |  | Subset-026-3.13.7.2 |  |
|  |  | Subset-026-3.13.7.3 |  |
|  |  | Subset-026-3.11.1.1 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI and JRU interfaces that the appropriate change is displayed and registered each time the speed supervision limit changes to a less restrictive value. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 1/ 12 | OS/ LS |
| Linking | STORED | - |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Axle load speed profile | NOT STORED | - |
| STM max speed | NOT STORED | - |
| STM system speed/distance | NOT STORED | - |
| Temporary Speed Restrictions | NOT STORED | - |
| Signalling related Speed Restriction | STORED | - |
| Mode Profile | STORED | - |
| Unconditional Emergency Stop | NOT STORED | - |
| Conditional Emergency Stop | NOT STORED | - |
| Train Position | STORED | - |
| Train Data | STORED | - |
| Adhesion factor | NOT STORED | - |
| ERTMS/ETCS level | 2 | L1 |
| Driver ID | STORED | - |
| Train Running Number | STORED | - |
| Track Ahead Free Request | NOT STORED | - |
| Level Crossing information | NOT STORED | - |
| Permitted Braking Distance Information | NOT STORED | - |
| Operated System Version | 010 0000 | Version 2.0, introduced in SRS 3.3.0 |
| Virtual Balise Covers | NOT STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is moving (V_TRAIN=<below V5>) | I | INT | See the meaning of V5 in the following step. | N/A | N/A | - |
| 2 | N/A | N/A | The driver requests the speed and distance monitoring information to be displayed | I | DMI | Only if the current mode is OS mode. This action will be done once, since after it the displayed information will not disappear unless the driver requests it. | N/A | N/A | - |
| 3 | N/A | N/A | One Balise Group message containing packet 27 (D_STATIC=0, V_STATIC=<V1>, Q_FRONT=0, D_STATIC(1)=<L_AXLELOAD+L_0>, V_STATIC(1)=<Vmax>), packet 51 (D_AXLELOAD=0, V_AXLELOAD=<V2>, Q_FRONT=0), packet 65 (D_TSR=0, V_TSR=<V3>, Q_FRONT=0), packet 88 (D_LX=0, V_LX=<V4>), packet 52 (D_PBDSR=0), packet 5, packet 12 (V_MAIN = <V7>), packet 80 (D_MAMODE = 0, V_MAMODE = <V8>) and packet 21 is received. | I | BTM | The length of the MA shall be long enough to prevent the on-board equipment from braking to the EoA/SvL or LoA during the execution of this TC. The length value "L_0" will be a positive value and V_STATIC(1)=<Vmax> will be the highest speed value of all speed limits, in order to be able to see the supervision of V_STATIC=<V1> and V_MAIN = <V7>. V5 = the speed restriction to ensure permitted braking distance (D_PBD will be set so as to ensure V5 is the minimum of all the speed restrictions). V6 = maximum train speed. The relation between the values of the speed limits is: V5 < V4 < V3 < V2 < V1 < V7 < V8 < V6 < Vmax. The relationships between the relevant length values are the following ones: L_PBDSR < L_LX < L_TSR L_TSR +L_TRAIN < L_AXLELOAD L_TRAIN < L_0 This location is at distance D0. | N/A | N/A | - |
| 4 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | The speed and distance monitoring information is displayed (V_PERM = <V5>) | O | DMI | Only if not in LS mode: The PS is equal to <V5> (the speed restriction to ensure permitted braking distance). The DMI aspect is coherent with Normal Status in a Ceiling Speed Monitoring section. Only if in LS mode: The DMI aspect is coherent with Overspeed Status in a Ceiling Speed Monitoring section. V_TRAIN will be used instead of V_PERM | N/A | N/A | - |
| 6 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=0; M_SDMSUPSTAT=0; V_PERM = <V5>) is recorded | O | JRU | V5 = the speed restriction to ensure permitted braking distance Only if not in LS mode: M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal Status) Only if in LS mode: M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 2 (Overspeed Status) V_TRAIN will be used instead of V_PERM | N/A | N/A | - |
| 7 | N/A | N/A | The speed and distance monitoring information is displayed (V_PERM =<V4> ) | O | DMI | Only if not in LS mode: This location is at distance D0 + L_PBDSR. The PS is equal to <V4> (Permitted speed for the LX speed restriction). The DMI aspect is coherent with Normal Status in a Ceiling Speed Monitoring section. Only if in LS mode: The DMI aspect is coherent with Overspeed Status in a Ceiling Speed Monitoring section. V_TRAIN will be used instead of V_PERM | N/A | N/A | - |
| 8 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=0; M_SDMSUPSTAT=0; V_PERM = <V4>) is recorded | O | JRU | V4 = Permitted speed for the LX speed restriction Only if not in LS mode: M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal Status) Only if in LS mode: M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 2 (Overspeed Status) V_TRAIN will be used instead of V_PERM | N/A | N/A | - |
| 9 | N/A | N/A | The speed and distance monitoring information is displayed (V_PERM =<V3> ) | O | DMI | This location is at distance D0 + L_LX. Only if not in LS mode: The PS is equal to <V3> (=V_TSR). The DMI aspect is coherent with Normal Status in a Ceiling Speed Monitoring section. Only if in LS mode: The DMI aspect is coherent with Overspeed Status in a Ceiling Speed Monitoring section. V_TRAIN will be used instead of V_PERM | N/A | N/A | - |
| 10 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=0; M_SDMSUPSTAT=0; V_PERM = <V3>) is recorded | O | JRU | V3 = V_TSR Only if not in LS mode: M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal Status) Only if in LS mode: M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 2 (Overspeed Status) V_TRAIN will be used instead of V_PERM | N/A | N/A | - |
| 11 | N/A | N/A | The speed and distance monitoring information is displayed (V_PERM =<V2> ) | O | DMI | This location is at distance D0 + L_TSR+L_TRAIN. Only if not in LS mode: The PS is equal to <V2> (=V_AXLELOAD). The DMI aspect is coherent with Normal Status in a Ceiling Speed Monitoring section. Only if in LS mode: The DMI aspect is coherent with Overspeed Status in a Ceiling Speed Monitoring section. V_TRAIN will be used instead of V_PERM | N/A | N/A | - |
| 12 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=0; M_SDMSUPSTAT=0; V_PERM = <V2>) is recorded | O | JRU | V2 = V_AXLELOAD Only if not in LS mode: M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal Status) Only if in LS mode: M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 2 (Overspeed Status) V_TRAIN will be used instead of V_PERM | N/A | N/A | - |
| 13 | N/A | N/A | The speed and distance monitoring information is displayed (V_PERM =<V1> ) | O | DMI | This location is at distance D0 + L_AXLELOAD + L_TRAIN. Only if not in LS mode: The PS is equal to <V1> (=V_STATIC). The DMI aspect is coherent with Normal Status in a Ceiling Speed Monitoring section. Only if in LS mode: The DMI aspect is coherent with Overspeed Status in a Ceiling Speed Monitoring section. V_TRAIN will be used instead of V_PERM | N/A | N/A | - |
| 14 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=0; M_SDMSUPSTAT=0; V_PERM = <V1>) is recorded | O | JRU | V1 = V_STATIC Only if not in LS mode: M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal Status) Only if in LS mode: M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 2 (Overspeed Status) V_TRAIN will be used instead of V_PERM | N/A | N/A | - |
| 15 | N/A | N/A | The speed and distance monitoring information is displayed (V_PERM =<V7> ) | O | DMI | This location is at distance D0 + L_AXLELOAD + L_0 + L_TRAIN. Only if not in LS mode: The PS is equal to <V7> (=V_MAIN ). The DMI aspect is coherent with Normal Status in a Ceiling Speed Monitoring section. Only if in LS mode: The DMI aspect is coherent with Overspeed Status in a Ceiling Speed Monitoring section. V_TRAIN will be used instead of V_PERM | N/A | N/A | - |
| 16 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=0; M_SDMSUPSTAT=0; V_PERM = <V7>) is recorded | O | JRU | V7 = V_MAIN Only if not in LS mode: M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal Status) Only if in LS mode: M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 2 (Overspeed Status) V_TRAIN will be used instead of V_PERM | N/A | N/A | - |
| 17 | N/A | N/A | One Balise Group message containing packet 27 (D_STATIC=0, V_STATIC=<V9>), packet 12 (V_MAIN = <V10>), packet 80 (D_MAMODE = 0, V_MAMODE = <V8>) and packet 21 is received. | I | BTM | The length of the MA shall be long enough to prevent the on-board equipment from braking to the EoA/SvL or LoA during the execution of this TC. V6 = maximum train speed. The relation between the values of the speed limits is: V8 < V6 < V9 < V10 This location is at distance D1. | N/A | N/A | - |
| 18 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 19 | N/A | N/A | The speed and distance monitoring information is displayed (V_PERM = <V8>) | O | DMI | Only if not in LS mode: The PS is equal to V8 (Mode related Speed Restriction). The DMI aspect is coherent with Normal Status in a Ceiling Speed Monitoring section. Only if in LS mode: The DMI aspect is coherent with Overspeed Status in a Ceiling Speed Monitoring section. V_TRAIN will be used instead of V_PERM | N/A | N/A | - |
| 20 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=0; M_SDMSUPSTAT=0; V_PERM = <V8>) is recorded | O | JRU | V8 = Mode related Speed Restriction Only if not in LS mode: M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal Status) Only if in LS mode: M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 2 (Overspeed Status) V_TRAIN will be used instead of V_PERM | N/A | N/A | - |
| 21 | N/A | N/A | One Balise Group message containing packet 27 (D_STATIC=0, V_STATIC=<V9>), packet 12 (V_MAIN = <V10>), packet 80 (D_MAMODE = 0, V_MAMODE = <V8>) and packet 21 is received. | I | BTM | The length of the MA shall be long enough to prevent the on-board equipment from braking to the EoA/SvL or LoA during the execution of this TC. V6 = maximum train speed. The relation between the values of the speed limits is: V6 < V8 < V9 < V10 This location is at distance D2. | N/A | N/A | - |
| 22 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 23 | N/A | N/A | The speed and distance monitoring information is displayed (V_PERM = <V6>) | O | DMI | The PS is equal to V6 (maximum train speed). Only if not in LS mode: The DMI aspect is coherent with Normal Status in a Ceiling Speed Monitoring section. Only if in LS mode: The DMI aspect is coherent with Overspeed Status in a Ceiling Speed Monitoring section. V_TRAIN will be used instead of V_PERM | N/A | N/A | - |
| 24 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=0; M_SDMSUPSTAT=0; V_PERM = <V6>) is recorded | O | JRU | V6 = maximum train speed Only if not in LS mode: M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal Status) Only if in LS mode: M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 2 (Overspeed Status) V_TRAIN will be used instead of V_PERM | N/A | N/A | - |

| Step 3: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | 1 | According to SS-040, a BG message containing one Packet 5 must have Q_LINK=1. |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | 0 | - |
| V_STATIC | 7 | FINITE VALUE | V1 |
| Q_FRONT | 1 | 0 | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | 1 | - |
| D_STATIC(k) | 15 | FINITE VALUE | <L_AXLELOAD+L0> |
| V_STATIC(k) | 7 | FINITE VALUE | <Vmax> |
| Q_FRONT(k) | 1 | 0 | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 51 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | FINITE VALUE | - |
| D_TRACKINIT | 15 | FINITE VALUE | Only if Q_TRACKINIT = 1 |
| D_AXLELOAD | 15 | 0 | Only if Q_TRACKINIT = 0, D_AXLELOAD and the following variables follow. |
| L_AXLELOAD | 15 | FINITE VALUE | - |
| Q_FRONT | 1 | 0 | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_AXLELOADCAT(n) | 7 | FINITE VALUE | - |
| V_AXLELOAD(n) | 7 | FINITE VALUE | Speed restriction to be applied if the axle load category of the train ( M_AXLELOADCAT(n). |
| N_ITER | 5 | FINITE VALUE | - |
| D_AXLELOAD(k) | 15 | FINITE VALUE | V2 |
| L_AXLELOAD(k) | 15 | FINITE VALUE | - |
| Q_FRONT(k) | 1 | 0 | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| M_AXLELOADCAT(k,m) | 7 | FINITE VALUE | - |
| V_AXLELOAD(k,m) | 7 | FINITE VALUE | Speed restriction to be applied if the axle load category of the train ( M_AXLELOADCAT(k,m). |
| NID_PACKET | 8 | 65 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_TSR | 8 | FINITE VALUE | - |
| D_TSR | 15 | 0 | - |
| L_TSR | 15 | FINITE VALUE | - |
| Q_FRONT | 1 | 0 | - |
| V_TSR | 7 | FINITE VALUE | V3 |
| NID_PACKET | 8 | 88 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LX | 8 | FINITE VALUE | - |
| D_LX | 15 | 0 | - |
| L_LX | 15 | FINITE VALUE | V4 |
| Q_LXSTATUS | 1 | FINITE VALUE | - |
| V_LX | 7 | FINITE VALUE | Only if Q_LXSTATUS = 1. |
| Q_STOPLX | 1 | FINITE VALUE | Only if Q_LXSTATUS = 1. |
| L_STOPLX | 15 | FINITE VALUE | Only if Q_STOPLX = 1. |
| NID_PACKET | 8 | 52 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | FINITE VALUE | - |
| D_TRACKINIT | 15 | FINITE VALUE | Only if Q_TRACKINIT = 1 |
| D_PBD | 15 | FINITE VALUE | Only if Q_TRACKINIT = 0, D_PBD and the following variables follow. |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill, 1 = uphill. |
| G_PBDSR | 8 | FINITE VALUE | Gradient value to be used for the calculation. |
| Q_PBDSR | 1 | FINITE VALUE | - |
| D_PBDSR | 15 | 0 | - |
| L_PBDSR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_PBD(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill, 1 = uphill. |
| G_PBDSR(k) | 8 | FINITE VALUE | Gradient value to be used for the calculation. |
| Q_PBDSR(k) | 1 | FINITE VALUE | - |
| D_PBDSR(k) | 15 | FINITE VALUE | - |
| L_PBDSR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | 255 |

| Step 3: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | 1 | According to SS-040, a BG message containing one Packet 5 must have Q_LINK=1. |
| NID_PACKET | 8 | 5 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LINK | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION | 1 | FINITE VALUE | - |
| Q_LINKREACTION | 2 | FINITE VALUE | - |
| Q_LOCACC | 6 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_LINK(k) | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | FINITE VALUE | - |
| Q_LOCACC(k) | 6 | FINITE VALUE | - |
| NID_PACKET | 8 | 12 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | V7 |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out". |
| N_ITER | 5 | FINITE VALUE | Set to zero if V_MAIN = 0 or if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 80 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_MAMODE | 15 | FINITE VALUE | - |
| M_MAMODE | 2 | FINITE VALUE | OS, LS, SH. |
| V_MAMODE | 7 | FINITE VALUE | V8 |
| L_MAMODE | 15 | FINITE VALUE | - |
| L_ACKMAMODE | 15 | FINITE VALUE | - |
| Q_MAMODE | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_MAMODE(k) | 15 | FINITE VALUE | - |
| M_MAMODE(k) | 2 | FINITE VALUE | OS, LS, SH. |
| V_MAMODE(k) | 7 | FINITE VALUE | - |
| L_MAMODE(k) | 15 | FINITE VALUE | - |
| L_ACKMAMODE(k) | 15 | FINITE VALUE | - |
| Q_MAMODE(k) | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | 255 |

| Step 17: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | 1 | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | 0 | - |
| V_STATIC | 7 | FINITE VALUE | V9 |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 12 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | V10 |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out". |
| N_ITER | 5 | FINITE VALUE | Set to zero if V_MAIN = 0 or if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 80 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_MAMODE | 15 | FINITE VALUE | - |
| M_MAMODE | 2 | FINITE VALUE | OS, LS, SH. |
| V_MAMODE | 7 | FINITE VALUE | V8 |
| L_MAMODE | 15 | FINITE VALUE | - |
| L_ACKMAMODE | 15 | FINITE VALUE | - |
| Q_MAMODE | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_MAMODE(k) | 15 | FINITE VALUE | - |
| M_MAMODE(k) | 2 | FINITE VALUE | OS, LS, SH. |
| V_MAMODE(k) | 7 | FINITE VALUE | - |
| L_MAMODE(k) | 15 | FINITE VALUE | - |
| L_ACKMAMODE(k) | 15 | FINITE VALUE | - |
| Q_MAMODE(k) | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | 255 |

| Step 21: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | 1 | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | 0 | - |
| V_STATIC | 7 | FINITE VALUE | V9 |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 12 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | V10 |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out". |
| N_ITER | 5 | FINITE VALUE | Set to zero if V_MAIN = 0 or if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 80 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_MAMODE | 15 | 0 | - |
| M_MAMODE | 2 | FINITE VALUE | OS, LS, SH. |
| V_MAMODE | 7 | FINITE VALUE | V8 |
| L_MAMODE | 15 | FINITE VALUE | - |
| L_ACKMAMODE | 15 | FINITE VALUE | - |
| Q_MAMODE | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_MAMODE(k) | 15 | FINITE VALUE | - |
| M_MAMODE(k) | 2 | FINITE VALUE | OS, LS, SH. |
| V_MAMODE(k) | 7 | FINITE VALUE | - |
| L_MAMODE(k) | 15 | FINITE VALUE | - |
| L_ACKMAMODE(k) | 15 | FINITE VALUE | - |
| Q_MAMODE(k) | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | 255 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 1/ 12 | OS/ LS |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| STM max speed | NOT STORED | - |
| STM system speed/distance | NOT STORED | - |
| Signalling related Speed Restriction | STORED | - |
| Mode Profile | STORED | - |
| Unconditional Emergency Stop | NOT STORED | - |
| Conditional Emergency Stop | NOT STORED | - |
| Train Position | STORED | - |
| Train Data | STORED | - |
| Adhesion factor | NOT STORED | - |
| ERTMS/ETCS level | 2 | L1 |
| Driver ID | STORED | - |
| Train Running Number | STORED | - |
| Operated System Version | 010 0000 | Version 2.0, introduced in SRS 3.3.0 |
| Virtual Balise Covers | NOT STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.13.7: Determination of Most Restrictive Speed Profile (MRSP) |  | 3130700 |
| Test Case of Feature | The Most Restrictive Speed Profile shall be computed from all speed restrictions by selecting the most restrictive parts of each element, some elements being compensated by the train length when requested by trackside. Besides, the MRSP shall be recalculated when any of the elements it is built of is changed. |  | 3 |
| Applicable Mode/Level Combinations | L2 / L3: LS, OS |  |  |
| Target of Test | The on-board calculates the most restrictive speed taking into account the following speed restrictions: Static Speed Profile (SSP), Temporary Speed Restriction (TSR), Axle Load Speed Profile (ASP), Maximum Train Speed, Level Crossing speed restriction (LX SR), Mode related Speed Restriction and Speed restriction to ensure a given permitted braking distance (PBD SR). Besides, the MRSP shall be recalculated when any of the elements it is built of is changed. The train length compensation shall be taken into account for each applicable speed restriction (SSP, ASP and TSR). |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.11.2.4 |  |
|  |  | Subset-026-3.13.7.2 |  |
|  |  | Subset-026-3.13.7.3 |  |
|  |  | Subset-026-3.11.1.1 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI and JRU interfaces that the appropriate change is displayed and registered each time the speed supervision limit changes to a less restrictive value. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 1/ 12 | OS/ LS |
| Linking | STORED | - |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Axle load speed profile | NOT STORED | - |
| STM max speed | NOT STORED | - |
| STM system speed/distance | NOT STORED | - |
| Temporary Speed Restrictions | NOT STORED | - |
| Mode Profile | STORED | - |
| Unconditional Emergency Stop | NOT STORED | - |
| Conditional Emergency Stop | NOT STORED | - |
| Train Position | STORED | - |
| Train Data | STORED | - |
| Adhesion factor | NOT STORED | - |
| ERTMS/ETCS level | 3/ 4 | L2/ L3 |
| Driver ID | STORED | - |
| Radio Network ID | STORED | - |
| RBC ID/Phone Number | STORED | - |
| Train Running Number | STORED | - |
| Track Ahead Free Request | NOT STORED | - |
| Level Crossing information | NOT STORED | - |
| Permitted Braking Distance Information | NOT STORED | - |
| Operated System Version | 010 0000 | Version 2.0, introduced in SRS 3.3.0 |
| Virtual Balise Covers | NOT STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | Both | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is moving (V_TRAIN=<below V5>) | I | INT | See the meaning of V5 in the following step. | N/A | N/A | - |
| 2 | N/A | N/A | The driver requests the speed and distance monitoring information to be displayed | I | DMI | Only if the current mode is OS mode. This action will be done once, since after it the displayed information will not disappear unless the driver requests it. | N/A | N/A | - |
| 3 | N/A | N/A | One Balise Group message is received | I | BTM | This location is at distance D0. This BG message will be the LRBG after the on-board reads it. | N/A | N/A | - |
| 4 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 21, packet 27 (D_STATIC=0, V_STATIC=<V1>, Q_FRONT=0, D_STATIC(1)=<L_AXLELOAD+L_0>, V_STATIC(1)=<Vmax>), packet 51 (D_AXLELOAD=0, V_AXLELOAD=<V2>, Q_FRONT=0), packet 65 (D_TSR=0, V_TSR=<V3>, Q_FRONT=0), packet 88 (D_LX=0, V_LX=<V4>), packet 52 (D_PBDSR=0), packet 80 (D_MAMODE = 0, V_MAMODE = <V7>) and packet 5 is received | I | RTM | The length of the MA shall be long enough to prevent the on-board equipment from braking to the EoA/SvL or LoA during the execution of this TC. The length value "L_0" will be a positive value and V_STATIC(1)=<Vmax> will be the highest speed value of all speed limits, in order to be able to see the supervision of V_STATIC=<V1> and V_MAMODE = <V7>. V5 = the speed restriction to ensure permitted braking distance (D_PBD will be set so as to ensure V5 is the minimum of all the speed restrictions). V6 = maximum train speed. The relationship between the values of the speed limits is: V5 < V4 < V3 < V2 < V1 < V7 < V6 < Vmax. The relationships between the relevant length values are the following ones: L_PBDSR < L_LX < L_TSR L_TSR +L_TRAIN < L_AXLELOAD L_TRAIN < L_0 | N/A | N/A | - |
| 6 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | The speed and distance monitoring information is displayed (V_PERM = <V5>) | O | DMI | Only if not in LS mode: The PS is equal to <V5> (the speed restriction to ensure permitted braking distance). The DMI aspect is coherent with Normal Status in a Ceiling Speed Monitoring section. Only if in LS mode: The DMI aspect is coherent with Overspeed Status in a Ceiling Speed Monitoring section. V_TRAIN will be used instead of V_PERM | N/A | N/A | - |
| 8 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=0; M_SDMSUPSTAT=0; V_PERM = <V5>) is recorded | O | JRU | V5 = the speed restriction to ensure permitted braking distance Only if not in LS mode: M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal Status) Only if in LS mode: M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 2 (Overspeed Status) V_TRAIN will be used instead of V_PERM | N/A | N/A | - |
| 9 | N/A | N/A | The speed and distance monitoring information is displayed (V_PERM =<V4> ) | O | DMI | This location is at distance D0 + L_PBDSR. The PS is equal to <V4> (Permitted speed for the LX speed restriction). Only if not in LS mode: The DMI aspect is coherent with Normal Status in a Ceiling Speed Monitoring section. Only if in LS mode: The DMI aspect is coherent with Overspeed Status in a Ceiling Speed Monitoring section. V_TRAIN will be used instead of V_PERM | N/A | N/A | - |
| 10 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=0; M_SDMSUPSTAT=0; V_PERM = <V4>) is recorded | O | JRU | V4 = Permitted speed for the LX speed restriction Only if not in LS mode: M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal Status) Only if in LS mode: M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 2 (Overspeed Status) V_TRAIN will be used instead of V_PERM | N/A | N/A | - |
| 11 | N/A | N/A | The speed and distance monitoring information is displayed (V_PERM =<V3> ) | O | DMI | This location is at distance D0 + L_LX. The PS is equal to <V3> (=V_TSR). Only if not in LS mode: The DMI aspect is coherent with Normal Status in a Ceiling Speed Monitoring section. Only if in LS mode: The DMI aspect is coherent with Overspeed Status in a Ceiling Speed Monitoring section. V_TRAIN will be used instead of V_PERM | N/A | N/A | - |
| 12 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=0; M_SDMSUPSTAT=0; V_PERM = <V3>) is recorded | O | JRU | V3 = V_TSR Only if not in LS mode: M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal Status) Only if in LS mode: M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 2 (Overspeed Status) V_TRAIN will be used instead of V_PERM | N/A | N/A | - |
| 13 | N/A | N/A | The speed and distance monitoring information is displayed (V_PERM =<V2> ) | O | DMI | This location is at distance D0 + L_TSR+L_TRAIN. The PS is equal to <V2> (=V_AXLELOAD). Only if not in LS mode: The DMI aspect is coherent with Normal Status in a Ceiling Speed Monitoring section. Only if in LS mode: The DMI aspect is coherent with Overspeed Status in a Ceiling Speed Monitoring section. V_TRAIN will be used instead of V_PERM | N/A | N/A | - |
| 14 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=0; M_SDMSUPSTAT=0; V_PERM = <V2>) is recorded | O | JRU | V2 = V_AXLELOAD Only if not in LS mode: M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal Status) Only if in LS mode: M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 2 (Overspeed Status) V_TRAIN will be used instead of V_PERM | N/A | N/A | - |
| 15 | N/A | N/A | The speed and distance monitoring information is displayed (V_PERM =<V1> ) | O | DMI | This location is at distance D0 + L_AXLELOAD + L_TRAIN. The PS is equal to <V1> (=V_STATIC). Only if not in LS mode: The DMI aspect is coherent with Normal Status in a Ceiling Speed Monitoring section. Only if in LS mode: The DMI aspect is coherent with Overspeed Status in a Ceiling Speed Monitoring section. V_TRAIN will be used instead of V_PERM | N/A | N/A | - |
| 16 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=0; M_SDMSUPSTAT=0; V_PERM = <V1>) is recorded | O | JRU | V1 = V_STATIC Only if not in LS mode: M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal Status) Only if in LS mode: M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 2 (Overspeed Status) V_TRAIN will be used instead of V_PERM | N/A | N/A | - |
| 17 | N/A | N/A | The speed and distance monitoring information is displayed (V_PERM =<V7> ) | O | DMI | This location is at distance D0 + L_AXLELOAD + L_0 + L_TRAIN. The PS is equal to <V7> (=V_MAMODE ). Only if not in LS mode: The DMI aspect is coherent with Normal Status in a Ceiling Speed Monitoring section. Only if in LS mode: The DMI aspect is coherent with Overspeed Status in a Ceiling Speed Monitoring section. V_TRAIN will be used instead of V_PERM | N/A | N/A | - |
| 18 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=0; M_SDMSUPSTAT=0; V_PERM = <V7>) is recorded | O | JRU | V7 = V_MAMODE Only if not in LS mode: M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal Status) Only if in LS mode: M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 2 (Overspeed Status) V_TRAIN will be used instead of V_PERM | N/A | N/A | - |
| 19 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 21, packet 27 (D_STATIC=0, V_STATIC=<V1>), packet 80 (D_MAMODE = 0, V_MAMODE = <V7>) is received | I | RTM | The length of the MA shall be long enough to prevent the on-board equipment from braking to the EoA/SvL or LoA during the execution of this TC. V6 = maximum train speed. The relation between the values of the speed limits is: V6 < V7 < V1 | N/A | N/A | - |
| 20 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 21 | N/A | N/A | The speed and distance monitoring information is displayed (V_PERM = <V6>) | O | DMI | The PS is equal to V6 (maximum train speed). Only if not in LS mode: The DMI aspect is coherent with Normal Status in a Ceiling Speed Monitoring section. Only if in LS mode: The DMI aspect is coherent with Overspeed Status in a Ceiling Speed Monitoring section. V_TRAIN will be used instead of V_PERM | N/A | N/A | - |
| 22 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=0; M_SDMSUPSTAT=0; V_PERM = <V6>) is recorded | O | JRU | V6 = maximum train speed Only if not in LS mode: M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal Status) Only if in LS mode: M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 2 (Overspeed Status) V_TRAIN will be used instead of V_PERM | N/A | N/A | - |

| Step 3: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | 1 | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | 255 |

| Step 5: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | 0 | - |
| V_STATIC | 7 | FINITE VALUE | V1 |
| Q_FRONT | 1 | 0 | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | 1 | - |
| D_STATIC(k) | 15 | FINITE VALUE | <L_AXLELOAD+L0> |
| V_STATIC(k) | 7 | FINITE VALUE | <Vmax> |
| Q_FRONT(k) | 1 | 0 | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 51 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | FINITE VALUE | - |
| D_TRACKINIT | 15 | FINITE VALUE | Only if Q_TRACKINIT = 1 |
| D_AXLELOAD | 15 | 0 | Only if Q_TRACKINIT = 0, D_AXLELOAD and the following variables follow. |
| L_AXLELOAD | 15 | FINITE VALUE | - |
| Q_FRONT | 1 | 0 | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_AXLELOADCAT(n) | 7 | FINITE VALUE | - |
| V_AXLELOAD(n) | 7 | FINITE VALUE | Speed restriction to be applied if the axle load category of the train ( M_AXLELOADCAT(n). |
| N_ITER | 5 | FINITE VALUE | - |
| D_AXLELOAD(k) | 15 | FINITE VALUE | V2 |
| L_AXLELOAD(k) | 15 | FINITE VALUE | - |
| Q_FRONT(k) | 1 | 0 | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| M_AXLELOADCAT(k,m) | 7 | FINITE VALUE | - |
| V_AXLELOAD(k,m) | 7 | FINITE VALUE | Speed restriction to be applied if the axle load category of the train ( M_AXLELOADCAT(k,m). |
| NID_PACKET | 8 | 65 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_TSR | 8 | FINITE VALUE | - |
| D_TSR | 15 | 0 | - |
| L_TSR | 15 | FINITE VALUE | - |
| Q_FRONT | 1 | 0 | - |
| V_TSR | 7 | FINITE VALUE | V3 |
| NID_PACKET | 8 | 88 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LX | 8 | FINITE VALUE | - |
| D_LX | 15 | 0 | - |
| L_LX | 15 | FINITE VALUE | V4 |
| Q_LXSTATUS | 1 | FINITE VALUE | - |
| V_LX | 7 | FINITE VALUE | Only if Q_LXSTATUS = 1. |
| Q_STOPLX | 1 | FINITE VALUE | Only if Q_LXSTATUS = 1. |
| L_STOPLX | 15 | FINITE VALUE | Only if Q_STOPLX = 1. |
| NID_PACKET | 8 | 52 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | FINITE VALUE | - |
| D_TRACKINIT | 15 | FINITE VALUE | Only if Q_TRACKINIT = 1 |
| D_PBD | 15 | FINITE VALUE | Only if Q_TRACKINIT = 0, D_PBD and the following variables follow. |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill, 1 = uphill. |
| G_PBDSR | 8 | FINITE VALUE | Gradient value to be used for the calculation. |
| Q_PBDSR | 1 | FINITE VALUE | - |
| D_PBDSR | 15 | 0 | - |
| L_PBDSR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_PBD(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill, 1 = uphill. |
| G_PBDSR(k) | 8 | FINITE VALUE | Gradient value to be used for the calculation. |
| Q_PBDSR(k) | 1 | FINITE VALUE | - |
| D_PBDSR(k) | 15 | FINITE VALUE | - |
| L_PBDSR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 80 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_MAMODE | 15 | 0 | - |
| M_MAMODE | 2 | FINITE VALUE | OS, LS, SH. |
| V_MAMODE | 7 | FINITE VALUE | V7 |
| L_MAMODE | 15 | FINITE VALUE | - |
| L_ACKMAMODE | 15 | FINITE VALUE | - |
| Q_MAMODE | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_MAMODE(k) | 15 | FINITE VALUE | - |
| M_MAMODE(k) | 2 | FINITE VALUE | OS, LS, SH. |
| V_MAMODE(k) | 7 | FINITE VALUE | - |
| L_MAMODE(k) | 15 | FINITE VALUE | - |
| L_ACKMAMODE(k) | 15 | FINITE VALUE | - |
| Q_MAMODE(k) | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 5 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LINK | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION | 1 | FINITE VALUE | - |
| Q_LINKREACTION | 2 | FINITE VALUE | - |
| Q_LOCACC | 6 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_LINK(k) | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | FINITE VALUE | - |
| Q_LOCACC(k) | 6 | FINITE VALUE | - |

| Step 19: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | 0 | - |
| V_STATIC | 7 | FINITE VALUE | V1 |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 80 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_MAMODE | 15 | 0 | - |
| M_MAMODE | 2 | FINITE VALUE | OS, LS, SH. |
| V_MAMODE | 7 | FINITE VALUE | V8 |
| L_MAMODE | 15 | FINITE VALUE | - |
| L_ACKMAMODE | 15 | FINITE VALUE | - |
| Q_MAMODE | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_MAMODE(k) | 15 | FINITE VALUE | - |
| M_MAMODE(k) | 2 | FINITE VALUE | OS, LS, SH. |
| V_MAMODE(k) | 7 | FINITE VALUE | - |
| L_MAMODE(k) | 15 | FINITE VALUE | - |
| L_ACKMAMODE(k) | 15 | FINITE VALUE | - |
| Q_MAMODE(k) | 1 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 1/ 12 | OS/ LS |
| Linking | STORED | - |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| STM max speed | NOT STORED | - |
| STM system speed/distance | NOT STORED | - |
| Mode Profile | STORED | - |
| Unconditional Emergency Stop | NOT STORED | - |
| Conditional Emergency Stop | NOT STORED | - |
| Train Position | STORED | - |
| Train Data | STORED | - |
| Adhesion factor | NOT STORED | - |
| ERTMS/ETCS level | 3/ 4 | L2/ L3 |
| Driver ID | STORED | - |
| Radio Network ID | STORED | - |
| RBC ID/Phone Number | STORED | - |
| Train Running Number | STORED | - |
| Operated System Version | 010 0000 | Version 2.0, introduced in SRS 3.3.0 |
| Virtual Balise Covers | NOT STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | Both | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.13.7: Determination of Most Restrictive Speed Profile (MRSP) |  | 3130700 |
| Test Case of Feature | The Most Restrictive Speed Profile shall be computed from all speed restrictions by selecting the most restrictive parts of each element, some elements being compensated by the train length when requested by trackside. Besides, the MRSP shall be recalculated when any of the elements it is built of is changed. |  | 4 |
| Applicable Mode/Level Combinations | L2 / L3: FS |  |  |
| Target of Test | The on-board calculates the most restricted speed taking into account the following speed restrictions: Static Speed Profile (SSP), Temporary Speed Restriction (TSR), Axle Load Speed Profile (ASP), Maximum Train Speed, Level Crossing speed restriction (LX SR) and Speed restriction to ensure a given permitted braking distance (PBD SR). Besides, the MRSP shall be recalculated when any of the elements it is built of is changed. The train length compensation shall be taken into account for each applicable speed restriction (SSP, ASP and TSR). |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.11.2.4 |  |
|  |  | Subset-026-3.13.7.2 |  |
|  |  | Subset-026-3.13.7.3 |  |
|  |  | Subset-026-3.11.1.1 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI and JRU interfaces that the appropriate change is displayed and registered each time the speed supervision limit changes to a less restrictive value. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 | FS |
| Linking | STORED | - |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Axle load speed profile | NOT STORED | - |
| STM max speed | NOT STORED | - |
| STM system speed/distance | NOT STORED | - |
| Temporary Speed Restrictions | NOT STORED | - |
| Mode Profile | NOT STORED | - |
| Unconditional Emergency Stop | NOT STORED | - |
| Conditional Emergency Stop | NOT STORED | - |
| Train Position | STORED | - |
| Train Data | STORED | - |
| Adhesion factor | NOT STORED | - |
| ERTMS/ETCS level | 3/ 4 | L2/ L3 |
| Driver ID | STORED | - |
| Radio Network ID | STORED | - |
| RBC ID/Phone Number | STORED | - |
| Train Running Number | STORED | - |
| Track Ahead Free Request | NOT STORED | - |
| Level Crossing information | NOT STORED | - |
| Permitted Braking Distance Information | NOT STORED | - |
| Operated System Version | 010 0000 | Version 2.0, introduced in SRS 3.3.0 |
| Virtual Balise Covers | NOT STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | Both | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is moving (V_TRAIN=<below V5>) | I | INT | See the meaning of V5 in the following step. | N/A | N/A | - |
| 2 | N/A | N/A | One Balise Group message is received | I | BTM | This location is at distance D0. This BG message will be the LRBG after the on-board reads it. | N/A | N/A | - |
| 3 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 4 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 21, packet 27 (D_STATIC=0, V_STATIC=<V1>, Q_FRONT=0, D_STATIC(1)=<L_AXLELOAD+L_0>, V_STATIC(1)=<Vmax>), packet 51 (D_AXLELOAD=0, V_AXLELOAD=<V2>, Q_FRONT=0), packet 65 (D_TSR=0, V_TSR=<V3>, Q_FRONT=0), packet 88 (D_LX=0, V_LX=<V4>), packet 52 (D_PBDSR=0) and packet 5 is received | I | RTM | The length of the MA shall be long enough to prevent the on-board equipment from braking to the EoA/SvL or LoA during the execution of this TC. The length value "L_0" will be a positive value and V_STATIC(1)=<Vmax> will be the highest speed value of all speed limits, in order to be able to see the supervision of V_STATIC=<V1> and the maximum train speed (<V6>). V5 = the speed restriction to ensure permitted braking distance (D_PBD will be set so as to ensure V5 is the minimum of all the speed restrictions). V6 = maximum train speed. The relationship between the values of the speed limits is: V5 < V4 < V3 < V2 < V1 < V6 < Vmax. The relationships between the relevant length values are the following ones: L_PBDSR < L_LX < L_TSR L_TSR +L_TRAIN < L_AXLELOAD L_TRAIN < L_0 | N/A | N/A | - |
| 5 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 6 | N/A | N/A | The speed and distance monitoring information is displayed (V_PERM = <V5>) | O | DMI | The PS is equal to <V5> (the speed restriction to ensure permitted braking distance). The DMI aspect is coherent with Normal Status in a Ceiling Speed Monitoring section. | N/A | N/A | - |
| 7 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=0; M_SDMSUPSTAT=0; V_PERM = <V5>) is recorded | O | JRU | V5 = the speed restriction to ensure permitted braking distance M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal Status) | N/A | N/A | - |
| 8 | N/A | N/A | The speed and distance monitoring information is displayed (V_PERM =<V4> ) | O | DMI | This location is at distance D0 + L_PBDSR. The PS is equal to <V4> (Permitted speed for the LX speed restriction). The DMI aspect is coherent with Normal Status in a Ceiling Speed Monitoring section. | N/A | N/A | - |
| 9 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=0; M_SDMSUPSTAT=0; V_PERM = <V4>) is recorded | O | JRU | V4 = Permitted speed for the LX speed restriction M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal Status) | N/A | N/A | - |
| 10 | N/A | N/A | The speed and distance monitoring information is displayed (V_PERM =<V3> ) | O | DMI | This location is at distance D0 + L_LX. The PS is equal to <V3> (=V_TSR). The DMI aspect is coherent with Normal Status in a Ceiling Speed Monitoring section. | N/A | N/A | - |
| 11 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=0; M_SDMSUPSTAT=0; V_PERM = <V3>) is recorded | O | JRU | V3 = V_TSR M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal Status) | N/A | N/A | - |
| 12 | N/A | N/A | The speed and distance monitoring information is displayed (V_PERM =<V2> ) | O | DMI | This location is at distance D0 + L_TSR+L_TRAIN. The PS is equal to <V2> (=V_AXLELOAD). The DMI aspect is coherent with Normal Status in a Ceiling Speed Monitoring section. | N/A | N/A | - |
| 13 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=0; M_SDMSUPSTAT=0; V_PERM = <V2>) is recorded | O | JRU | V2 = V_AXLELOAD M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal Status) | N/A | N/A | - |
| 14 | N/A | N/A | The speed and distance monitoring information is displayed (V_PERM =<V1> ) | O | DMI | This location is at distance D0 + L_AXLELOAD + L_TRAIN. The PS is equal to <V1> (=V_STATIC). The DMI aspect is coherent with Normal Status in a Ceiling Speed Monitoring section. | N/A | N/A | - |
| 15 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=0; M_SDMSUPSTAT=0; V_PERM = <V1>) is recorded | O | JRU | V1 = V_STATIC M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal Status) | N/A | N/A | - |
| 16 | N/A | N/A | The speed and distance monitoring information is displayed (V_PERM =<V6> ) | O | DMI | This location is at distance D0 + L_AXLELOAD + L_0 + L_TRAIN. The PS is equal to <V6> (=maximum train speed ). The DMI aspect is coherent with Normal Status in a Ceiling Speed Monitoring section. | N/A | N/A | - |
| 17 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=0; M_SDMSUPSTAT=0; V_PERM = <V6>) is recorded | O | JRU | V6 = maximum train speed M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal Status) | N/A | N/A | - |

| Step 2: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | 1 | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | 255 |

| Step 4: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | 0 | - |
| V_STATIC | 7 | FINITE VALUE | V1 |
| Q_FRONT | 1 | 0 | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | 1 | - |
| D_STATIC(k) | 15 | FINITE VALUE | <L_AXLELOAD+L0> |
| V_STATIC(k) | 7 | FINITE VALUE | <Vmax> |
| Q_FRONT(k) | 1 | 0 | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 51 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | FINITE VALUE | - |
| D_TRACKINIT | 15 | FINITE VALUE | Only if Q_TRACKINIT = 1 |
| D_AXLELOAD | 15 | 0 | Only if Q_TRACKINIT = 0, D_AXLELOAD and the following variables follow. |
| L_AXLELOAD | 15 | FINITE VALUE | - |
| Q_FRONT | 1 | 0 | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_AXLELOADCAT(n) | 7 | FINITE VALUE | - |
| V_AXLELOAD(n) | 7 | FINITE VALUE | Speed restriction to be applied if the axle load category of the train ( M_AXLELOADCAT(n). |
| N_ITER | 5 | FINITE VALUE | - |
| D_AXLELOAD(k) | 15 | FINITE VALUE | V2 |
| L_AXLELOAD(k) | 15 | FINITE VALUE | - |
| Q_FRONT(k) | 1 | 0 | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| M_AXLELOADCAT(k,m) | 7 | FINITE VALUE | - |
| V_AXLELOAD(k,m) | 7 | FINITE VALUE | Speed restriction to be applied if the axle load category of the train ( M_AXLELOADCAT(k,m). |
| NID_PACKET | 8 | 65 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_TSR | 8 | FINITE VALUE | - |
| D_TSR | 15 | 0 | - |
| L_TSR | 15 | FINITE VALUE | - |
| Q_FRONT | 1 | 0 | - |
| V_TSR | 7 | FINITE VALUE | V3 |
| NID_PACKET | 8 | 88 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LX | 8 | FINITE VALUE | - |
| D_LX | 15 | 0 | - |
| L_LX | 15 | FINITE VALUE | V4 |
| Q_LXSTATUS | 1 | FINITE VALUE | - |
| V_LX | 7 | FINITE VALUE | Only if Q_LXSTATUS = 1. |
| Q_STOPLX | 1 | FINITE VALUE | Only if Q_LXSTATUS = 1. |
| L_STOPLX | 15 | FINITE VALUE | Only if Q_STOPLX = 1. |
| NID_PACKET | 8 | 52 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | FINITE VALUE | - |
| D_TRACKINIT | 15 | FINITE VALUE | Only if Q_TRACKINIT = 1 |
| D_PBD | 15 | FINITE VALUE | Only if Q_TRACKINIT = 0, D_PBD and the following variables follow. |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill, 1 = uphill. |
| G_PBDSR | 8 | FINITE VALUE | Gradient value to be used for the calculation. |
| Q_PBDSR | 1 | FINITE VALUE | - |
| D_PBDSR | 15 | 0 | - |
| L_PBDSR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_PBD(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill, 1 = uphill. |
| G_PBDSR(k) | 8 | FINITE VALUE | Gradient value to be used for the calculation. |
| Q_PBDSR(k) | 1 | FINITE VALUE | - |
| D_PBDSR(k) | 15 | FINITE VALUE | - |
| L_PBDSR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 5 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LINK | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION | 1 | FINITE VALUE | - |
| Q_LINKREACTION | 2 | FINITE VALUE | - |
| Q_LOCACC | 6 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_LINK(k) | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | FINITE VALUE | - |
| Q_LOCACC(k) | 6 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 | FS |
| Linking | STORED | - |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| STM max speed | NOT STORED | - |
| STM system speed/distance | NOT STORED | - |
| Mode Profile | NOT STORED | - |
| Unconditional Emergency Stop | NOT STORED | - |
| Conditional Emergency Stop | NOT STORED | - |
| Train Position | STORED | - |
| Train Data | STORED | - |
| Adhesion factor | NOT STORED | - |
| ERTMS/ETCS level | 3/ 4 | L2/ L3 |
| Driver ID | STORED | - |
| Radio Network ID | STORED | - |
| RBC ID/Phone Number | STORED | - |
| Train Running Number | STORED | - |
| Operated System Version | 010 0000 | Version 2.0, introduced in SRS 3.3.0 |
| Virtual Balise Covers | NOT STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | Both | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 5

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.13.7: Determination of Most Restrictive Speed Profile (MRSP) |  | 3130700 |
| Test Case of Feature | The Most Restrictive Speed Profile shall be computed from all speed restrictions by selecting the most restrictive parts of each element, some elements not being compensated by the train length because it is not requested by trackside. Besides, the MRSP shall be recalculated when any of the elements it is built of is changed. |  | 5 |
| Applicable Mode/Level Combinations | L1: FS L2 / L3: FS |  |  |
| Target of Test | The on-board calculates the most restricted speed taking into account the following speed restrictions as the minimum values to be supervised: Static Speed Profile (SSP), Temporary Speed Restriction (TSR), Axle Load Speed Profile (ASP) without taking into account the train length compensation. Besides, the MRSP shall be recalculated when any of the elements it is built of is changed. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.13.7.2 |  |
|  |  | Subset-026-3.13.7.3 |  |
|  |  | Subset-026-3.11.1.1 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI and JRU interfaces that the appropriate change is displayed and registered each time the speed supervision limit changes to a less restrictive value. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 | FS |
| Radio communication session | ESTABLISHED | Only in L2/L3 |
| Linking | STORED | - |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Axle load speed profile | NOT STORED | - |
| STM max speed | NOT STORED | - |
| STM system speed/distance | NOT STORED | - |
| Temporary Speed Restrictions | NOT STORED | - |
| Mode Profile | NOT STORED | - |
| Unconditional Emergency Stop | NOT STORED | - |
| Conditional Emergency Stop | NOT STORED | - |
| Train Position | STORED | - |
| Train Data | STORED | - |
| Adhesion factor | NOT STORED | - |
| ERTMS/ETCS level | 2/ 3/ 4 | L1/ L2/ L3 |
| Driver ID | STORED | - |
| Train Running Number | STORED | - |
| Level Crossing information | NOT STORED | - |
| Permitted Braking Distance Information | NOT STORED | - |
| Operated System Version | 010 0000 | Version 2.0, introduced in SRS 3.3.0 |
| Virtual Balise Covers | NOT STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | The state will be "SAFE CONNECTION SET-UP", in case the TC is executed in L2/L3 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is moving (V_TRAIN=<below V3>) | I | INT | See the meaning of V3 in the following step. | N/A | N/A | - |
| 2 | N/A | N/A | One Balise Group message is received | I | BTM | Only in L2/L3. This location is at distance D0. This BG message will be the LRBG after the on-board reads it. | N/A | N/A | - |
| 3 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | Only in L2/L3. | N/A | N/A | - |
| 4 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 21, packet 27 (D_STATIC=0, V_STATIC=<V1>, Q_FRONT=1, D_STATIC(1)=<L_AXLELOAD+L_0>, V_STATIC(1)=<Vmax>), packet 51 (D_AXLELOAD=0, V_AXLELOAD=<V2>, Q_FRONT=1), packet 65 (D_TSR=0, V_TSR=<V3>, Q_FRONT=1) and packet 5 is received | I | RTM | Only in L2/L3. The length of the MA shall be long enough to prevent the on-board equipment from braking to the EoA/SvL or LoA during the execution of this TC. The length value "L_0" will be a positive value and V_STATIC(1)=<Vmax> will be the highest speed value of all speed limits, in order to be able to see the supervision of V_STATIC=<V1> and the maximum train speed (<V4>). V4 = maximum train speed. The relation between the values of the speed limits is: V3 < V2 < V1 < V4 < Vmax. The relation between the relevant length values is: L_TSR < L_AXLELOAD. | N/A | N/A | - |
| 5 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | Only in L2/L3. | N/A | N/A | - |
| 6 | N/A | N/A | One Balise Group message containing packet 27 (D_STATIC=0, V_STATIC=<V1>, Q_FRONT=1, D_STATIC(1)=<L_AXLELOAD+L0>, V_STATIC(1)=<Vmax>), packet 51 (D_AXLELOAD=0, V_AXLELOAD=<V2>, Q_FRONT=1), packet 65 (D_TSR=0, V_TSR=<V3>, Q_FRONT=1), packet 5, packet 12 (V_MAIN = <V5>) and packet 21 is received. | I | BTM | Only in L1. This location is at distance D1. The length of the MA shall be long enough to prevent the on-board equipment from braking to the EoA/SvL or LoA during the execution of this TC. The length value "L0" will be a positive value and V_STATIC(1)=<Vmax> will be the highest speed value of all speed limits, in order to be able to see the supervision of V_STATIC=<V1> and the maximum train speed (<V4>). V4 = maximum train speed. The relation between the values of the speed limits is: V3 < V2 < V1 < V4 < V5 < Vmax. The relation between the relevant length values is: L_TSR < L_AXLELOAD. | N/A | N/A | - |
| 7 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | Only in L1. | N/A | N/A | - |
| 8 | N/A | N/A | The speed and distance monitoring information is displayed (V_PERM =<V3> ) | O | DMI | The PS is equal to <V3> (=V_TSR). The DMI aspect is coherent with Normal Status in a Ceiling Speed Monitoring section. | N/A | N/A | - |
| 9 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=0; M_SDMSUPSTAT=0; V_PERM = <V3>) is recorded | O | JRU | V3 = V_TSR M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal Status) | N/A | N/A | - |
| 10 | N/A | N/A | The speed and distance monitoring information is displayed (V_PERM =<V2> ) | O | DMI | In L1, this location is at distance D1 + L_TSR. In L2/L3, this location is at distance D0 + L_TSR. The PS is equal to <V2> (=V_AXLELOAD). The DMI aspect is coherent with Normal Status in a Ceiling Speed Monitoring section. | N/A | N/A | - |
| 11 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=0; M_SDMSUPSTAT=0; V_PERM = <V2>) is recorded | O | JRU | V2 = V_AXLELOAD M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal Status) | N/A | N/A | - |
| 12 | N/A | N/A | The speed and distance monitoring information is displayed (V_PERM =<V1> ) | O | DMI | In L1, this location is at distance D1 + L_AXLELOAD. In L2/L3, this location is at distance D0 + L_AXLELOAD. The PS is equal to <V1> (=V_STATIC). The DMI aspect is coherent with Normal Status in a Ceiling Speed Monitoring section. | N/A | N/A | - |
| 13 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=0; M_SDMSUPSTAT=0; V_PERM = <V1>) is recorded | O | JRU | V1 = V_STATIC M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal Status) | N/A | N/A | - |
| 14 | N/A | N/A | The speed and distance monitoring information is displayed (V_PERM =<V4> ) | O | DMI | In L1, this location is at distance D1 + L_AXLELOAD + L_0. In L2/L3, this location is at distance D0 + L_AXLELOAD + L_0. The PS is equal to <V4> (=maximum train speed ). The DMI aspect is coherent with Normal Status in a Ceiling Speed Monitoring section. | N/A | N/A | - |
| 15 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=0; M_SDMSUPSTAT=0; V_PERM = <V4>) is recorded | O | JRU | V4 = maximum train speed M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal Status) | N/A | N/A | - |

| Step 2: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | 1 | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | 255 |

| Step 4: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | 0 | - |
| V_STATIC | 7 | FINITE VALUE | V1 |
| Q_FRONT | 1 | 0 | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | 1 | - |
| D_STATIC(k) | 15 | FINITE VALUE | <L_AXLELOAD+L0> |
| V_STATIC(k) | 7 | FINITE VALUE | <Vmax> |
| Q_FRONT(k) | 1 | 0 | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 51 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | FINITE VALUE | - |
| D_TRACKINIT | 15 | FINITE VALUE | Only if Q_TRACKINIT = 1 |
| D_AXLELOAD | 15 | 0 | Only if Q_TRACKINIT = 0, D_AXLELOAD and the following variables follow. |
| L_AXLELOAD | 15 | FINITE VALUE | - |
| Q_FRONT | 1 | 0 | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_AXLELOADCAT(n) | 7 | FINITE VALUE | - |
| V_AXLELOAD(n) | 7 | FINITE VALUE | Speed restriction to be applied if the axle load category of the train ( M_AXLELOADCAT(n). |
| N_ITER | 5 | FINITE VALUE | - |
| D_AXLELOAD(k) | 15 | FINITE VALUE | V2 |
| L_AXLELOAD(k) | 15 | FINITE VALUE | - |
| Q_FRONT(k) | 1 | 0 | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| M_AXLELOADCAT(k,m) | 7 | FINITE VALUE | - |
| V_AXLELOAD(k,m) | 7 | FINITE VALUE | Speed restriction to be applied if the axle load category of the train ( M_AXLELOADCAT(k,m). |
| NID_PACKET | 8 | 65 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_TSR | 8 | FINITE VALUE | - |
| D_TSR | 15 | 0 | - |
| L_TSR | 15 | FINITE VALUE | - |
| Q_FRONT | 1 | 0 | - |
| V_TSR | 7 | FINITE VALUE | V3 |
| NID_PACKET | 8 | 5 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LINK | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION | 1 | FINITE VALUE | - |
| Q_LINKREACTION | 2 | FINITE VALUE | - |
| Q_LOCACC | 6 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_LINK(k) | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | FINITE VALUE | - |
| Q_LOCACC(k) | 6 | FINITE VALUE | - |

| Step 6: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | 1 | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | 0 | - |
| V_STATIC | 7 | FINITE VALUE | V1 |
| Q_FRONT | 1 | 0 | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | 1 | - |
| D_STATIC(k) | 15 | FINITE VALUE | <L_AXLELOAD+L0> |
| V_STATIC(k) | 7 | FINITE VALUE | <Vmax> |
| Q_FRONT(k) | 1 | 0 | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 51 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | FINITE VALUE | - |
| D_TRACKINIT | 15 | FINITE VALUE | Only if Q_TRACKINIT = 1 |
| D_AXLELOAD | 15 | 0 | Only if Q_TRACKINIT = 0, D_AXLELOAD and the following variables follow. |
| L_AXLELOAD | 15 | FINITE VALUE | - |
| Q_FRONT | 1 | 0 | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_AXLELOADCAT(n) | 7 | FINITE VALUE | - |
| V_AXLELOAD(n) | 7 | FINITE VALUE | Speed restriction to be applied if the axle load category of the train ( M_AXLELOADCAT(n). |
| N_ITER | 5 | FINITE VALUE | - |
| D_AXLELOAD(k) | 15 | FINITE VALUE | V2 |
| L_AXLELOAD(k) | 15 | FINITE VALUE | - |
| Q_FRONT(k) | 1 | 0 | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| M_AXLELOADCAT(k,m) | 7 | FINITE VALUE | - |
| V_AXLELOAD(k,m) | 7 | FINITE VALUE | Speed restriction to be applied if the axle load category of the train ( M_AXLELOADCAT(k,m). |
| NID_PACKET | 8 | 65 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_TSR | 8 | FINITE VALUE | - |
| D_TSR | 15 | 0 | - |
| L_TSR | 15 | FINITE VALUE | - |
| Q_FRONT | 1 | 0 | - |
| V_TSR | 7 | FINITE VALUE | V3 |
| NID_PACKET | 8 | 255 | 255 |

| Step 6: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | 1 | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 5 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LINK | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION | 1 | FINITE VALUE | - |
| Q_LINKREACTION | 2 | FINITE VALUE | - |
| Q_LOCACC | 6 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_LINK(k) | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | FINITE VALUE | - |
| Q_LOCACC(k) | 6 | FINITE VALUE | - |
| NID_PACKET | 8 | 12 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | V5 |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out". |
| N_ITER | 5 | FINITE VALUE | Set to zero if V_MAIN = 0 or if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | 255 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 | FS |
| Radio communication session | ESTABLISHED | Only in L2/L3 |
| Linking | STORED | - |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Mode Profile | NOT STORED | - |
| Unconditional Emergency Stop | NOT STORED | - |
| Conditional Emergency Stop | NOT STORED | - |
| Train Position | STORED | - |
| Train Data | STORED | - |
| Adhesion factor | NOT STORED | - |
| ERTMS/ETCS level | 2/ 3/ 4 | L1/ L2/ L3 |
| Driver ID | STORED | - |
| Train Running Number | STORED | - |
| Level Crossing information | NOT STORED | - |
| Permitted Braking Distance Information | NOT STORED | - |
| Operated System Version | 010 0000 | Version 2.0, introduced in SRS 3.3.0 |
| Virtual Balise Covers | NOT STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | The state will be "SAFE CONNECTION SET-UP", in case the TC is executed in L2/L3 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 6

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.13.7: Determination of Most Restrictive Speed Profile (MRSP) |  | 3130700 |
| Test Case of Feature | The Most Restrictive Speed Profile shall be computed from all speed restrictions by selecting the most restrictive parts of each element. |  | 6 |
| Applicable Mode/Level Combinations | L0: UN L1: SR L2/3: SR |  |  |
| Target of Test | The on-board calculates the most restricted speed taking into account the following speed restrictions: Temporary Speed Restriction (TSR), Maximum Train Speed, Mode related Speed Restriction and Override function related Speed Restriction. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.13.7.2 |  |
|  |  | Subset-026-3.11.1.1 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI and JRU interfaces that the appropriate change is displayed and registered each time the speed supervision limit changes to a less restrictive value. |
| Constraints | If the TC is tested in SR mode, it is necessary the SR mode speed limit at the beginning of the test has a higher value than the value of the V_TSR in the Packet 65 sent at the beginning. (Once the "Override EOA" procedure is triggered in the execution of this TC, the National values shall enter in force, see the requirement 4.4.11.1.6.5) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 / 4 | SR / UN |
| STM max speed | NOT STORED | - |
| STM system speed/distance | NOT STORED | - |
| Temporary Speed Restrictions | NOT STORED | - |
| Unconditional Emergency Stop | NOT STORED | - |
| Conditional Emergency Stop | NOT STORED | - |
| Train Position | STORED | - |
| Train Data | STORED | - |
| ERTMS/ETCS level | 0/ 2 / 3 / 4 | L0 / L1 / L2 / L3 |
| Driver ID | STORED | - |
| Train Running Number | STORED | - |
| Operated System Version | 010 0000 | Version 2.0, introduced in SRS 3.3.0 |
| Virtual Balise Covers | NOT STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | Both | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is moving (V_TRAIN=<below V1>) | I | INT | See the meaning of V1 in the following step. | N/A | N/A | - |
| 2 | N/A | N/A | The driver requests the speed and distance monitoring information to be displayed | I | DMI | This action will be done once, since after it the displayed information will not disappear unless the driver requests it. Only inSR mode. | N/A | N/A | - |
| 3 | N/A | N/A | One Balise Group message containing packet 65 (D_TSR=0, V_TSR = <V1>, Q_FRONT = 1), packet 3 (V_NVSUPOVTRP = V_NVALLOWOVTRP = <V2>, V_NVSTFF=<V3>, V_NVUNFIT = <V3>) is received. | I | BTM | In SR mode, the length of the given distance to run in SR shall be long enough to prevent the on-board equipment from braking to the end of that given distance during the execution of this TC (i.e., the on-board equipment will always be in a Ceiling Speed Monitoring section). V4 = maximum train speed. The relation between the values of the speed limits is: V1 < V2 < V3 < V4. This location is at distance D0. Inside Packet 3, the value of T_NVOVTRP has to be high enough in order to ensure the Override status ends due to the train having passed the distance D_NVOVTRP. | N/A | N/A | - |
| 4 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | The speed and distance monitoring information is displayed (V_PERM = <V1>) | O | DMI | The PS is equal to <V1> (the Temporary Speed Restriction). Only in SR mode. The DMI aspect is coherent with Normal Status in a Ceiling Speed Monitoring section. NOT IN UN MODE | N/A | N/A | - |
| 6 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=0; M_SDMSUPSTAT=0; V_PERM = <V1>) is recorded | O | JRU | V1 = V_TSR M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal Status) | N/A | N/A | - |
| 7 | N/A | N/A | Train is moving (V_TRAIN=<V1-10>) | I | INT | The train speed is lightly > V1 | N/A | N/A | - |
| 8 | N/A | N/A | The speed and distance monitoring information is displayed (V_PERM = <V1>) | O | DMI | The PS is equal to <V1> (the Temporary Speed Restriction). The DMI aspect is coherent with Over speed Status in a Ceiling Speed Monitoring section. | N/A | N/A | - |
| 9 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=0; M_SDMSUPSTAT=2; V_PERM = <V1>) is recorded | O | JRU | V1 = V_TSR M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 2 (Over Speed Status) | N/A | N/A | - |
| 10 | N/A | N/A | The speed and distance monitoring information is displayed (V_PERM = <V1>) | O | DMI | The PS is equal to <V1> (the Temporary Speed Restriction). Only in SR mode. The DMI aspect is coherent with Normal Status in a Ceiling Speed Monitoring section. NOT IN UN MODE | N/A | N/A | - |
| 11 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=0; M_SDMSUPSTAT=0; V_PERM = <V1>) is recorded | O | JRU | V1 = V_TSR M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal Status) | N/A | N/A | - |
| 12 | N/A | N/A | The driver selects "Override EoA" | I | DMI | This action is done before reaching the location: D0 + L_TSR. Since V_TRAIN < V_NVALLOWOVTRP, it is possible to select "Override EOA". This location is at D1. | N/A | N/A | - |
| 13 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 1110) is recorded | O | JRU | M_DRIVERACTIONS=0000 1110 (Override selected) | N/A | N/A | - |
| 14 | N/A | N/A | The Mode symbol "Override EOA is active" is displayed | O | DMI | - | N/A | N/A | - |
| 15 | N/A | N/A | The speed and distance monitoring information is displayed (V_PERM =<V1> ) | O | DMI | The PS is equal to <V1> (the Temporary Speed Restriction), since V_TSR < V_NVSUPOVTRP. The DMI aspect is coherent with Normal Status in a Ceiling Speed Monitoring section. | N/A | N/A | - |
| 16 | N/A | N/A | The speed and distance monitoring information is displayed (V_PERM =<V2> ) | O | DMI | This location is at distance D0 + L_TSR. The PS is equal to <V2> (= V_NVSUPOVTRP). The DMI aspect is coherent with Normal Status in a Ceiling Speed Monitoring section. | N/A | N/A | - |
| 17 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=0; M_SDMSUPSTAT=0; V_PERM = <V2>) is recorded | O | JRU | V2 = V_NVSUPOVTRP M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal Status) | N/A | N/A | - |
| 18 | N/A | N/A | The Mode symbol "Override EOA is active" is removed | O | DMI | This location is at distance D1 + D_NVOVTRP. | N/A | N/A | - |
| 19 | N/A | N/A | The speed and distance monitoring information is displayed (V_PERM =<V3> ) | O | DMI | The PS is equal to <V3> (=V_NVSTFF if in SR mode, or V_NVUNFIT if in UN mode). The DMI aspect is coherent with Normal Status in a Ceiling Speed Monitoring section. | N/A | N/A | - |
| 20 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=0; M_SDMSUPSTAT=0; V_PERM = <V3>) is recorded | O | JRU | V3 = V_NVSTFF, if in SR mode. V3 = V_NVUNFIT if in UN mode. M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal Status) | N/A | N/A | - |

| Step 3: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | 1 | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 65 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_TSR | 8 | FINITE VALUE | - |
| D_TSR | 15 | 0 | - |
| L_TSR | 15 | FINITE VALUE | - |
| Q_FRONT | 1 | 1 | - |
| V_TSR | 7 | FINITE VALUE | V1 |
| NID_PACKET | 8 | 3 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_VALIDNV | 15 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | Identification of national areas to which the set applies. |
| N_ITER | 5 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | Identification of additional national area(s) to which the set applies. |
| V_NVSHUNT | 7 | FINITE VALUE | V3 |
| V_NVSTFF | 7 | FINITE VALUE | V3 |
| V_NVONSIGHT | 7 | FINITE VALUE | - |
| V_NVLIMSUPERV | 7 | FINITE VALUE | - |
| V_NVUNFIT | 7 | FINITE VALUE | - |
| V_NVREL | 7 | FINITE VALUE | - |
| D_NVROLL | 15 | FINITE VALUE | - |
| Q_NVSBTSMPERM | 1 | FINITE VALUE | - |
| Q_NVEMRRLS | 1 | FINITE VALUE | - |
| Q_NVGUIPERM | 1 | FINITE VALUE | - |
| Q_NVSBFBPERM | 1 | FINITE VALUE | - |
| Q_NVINHSMICPERM | 1 | FINITE VALUE | - |
| V_NVALLOWOVTRP | 7 | FINITE VALUE | V2 |
| V_NVSUPOVTRP | 7 | FINITE VALUE | V2 |
| D_NVOVTRP | 15 | FINITE VALUE | - |
| T_NVOVTRP | 8 | FINITE VALUE | - |
| D_NVPOTRP | 15 | FINITE VALUE | - |
| M_NVCONTACT | 2 | FINITE VALUE | - |
| T_NVCONTACT | 8 | FINITE VALUE | - |
| M_NVDERUN | 1 | FINITE VALUE | - |
| D_NVSTFF | 15 | FINITE VALUE | - |
| Q_NVDRIVER_ADHES | 1 | FINITE VALUE | - |
| A_NVMAXREDADH1 | 6 | FINITE VALUE | - |
| A_NVMAXREDADH2 | 6 | FINITE VALUE | - |
| A_NVMAXREDADH3 | 6 | FINITE VALUE | - |
| Q_NVLOCACC | 6 | FINITE VALUE | - |
| M_NVAVADH | 5 | FINITE VALUE | - |
| M_NVEBCL | 4 | FINITE VALUE | - |
| Q_NVKINT | 1 | FINITE VALUE | - |
| Q_NVKVINTSET | 2 | FINITE VALUE | Only if Q_NVKINT = 1, Q_NVKVINTSET and the following variables follow. |
| A_NVP12 | 6 | FINITE VALUE | Only if Q_NVKVINTSET = 1 |
| A_NVP23 | 6 | FINITE VALUE | Only if Q_NVKVINTSET = 1 |
| V_NVKVINT(n) | 7 | FINITE VALUE | 0 km/h |
| M_NVKVINT(n) | 7 | FINITE VALUE | Valid between V_NVKVINT and V_NVKVINT(1. If Q_NVKVINTSET = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12. |
| M_NVKVINT(n) | 7 | FINITE VALUE | Only if Q_NVKVINTSET = 1. Valid between V_NVKVINT and V_NVKVINT(1). Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23. |
| N_ITER | 5 | FINITE VALUE | - |
| V_NVKVINT | 7 | FINITE VALUE | - |
| M_NVKVINT | 7 | FINITE VALUE | Valid between V_NVKVINT(n) and V_NVKVINT(n+1). If Q_NVKVINTSET = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP1. |
| M_NVKVINT(n) | 7 | FINITE VALUE | Only if Q_NVKVINTSET = 1.Valid between V_NVKVINT(n) and V_NVKVINT(n+1) Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP2. |
| N_ITER | 5 | FINITE VALUE | - |
| Q_NVKVINTSET(k) | 2 | FINITE VALUE | - |
| A_NVP12(k) | 6 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1 |
| A_NVP23(k) | 6 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1 |
| V_NVKVINT(k) | 7 | FINITE VALUE | 0 |
| M_NVKVINT(k) | 7 | FINITE VALUE | Valid between V_NVKVINT(k) and V_NVKVINT(k,1). If Q_NVKVINTSET(k) = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12(k. |
| M_NVKVINT(k) | 7 | FINITE VALUE | Only if Q_NVKVINTSET(k) = . Valid between V_NVKVINT(k) and V_NVKVINT(k,1) Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23(k). |
| N_ITER(k) | 5 | FINITE VALUE | - |
| V_NVKVINT(k,m) | 7 | FINITE VALUE | - |
| M_NVKVINT(k,m) | 7 | FINITE VALUE | Valid between V_NVKVINT(k,m) and V_NVKVINT(k,m+1). If Q_NVKVINTSET(k) = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12(k). |
| M_NVKVINT(k,m) | 7 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1.Valid between V_NVKVINT(k,m) and V_NVKVINT(k,m+1). Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23(k). |
| L_NVKRINT | 5 | FINITE VALUE | 0 m |
| M_NVKRINT | 5 | FINITE VALUE | Valid between L_NVKRINT and L_NVKRINT(1). |
| N_ITER | 5 | FINITE VALUE | - |
| L_NVKRINT(l) | 5 | FINITE VALUE | - |
| M_NVKRINT(l) | 5 | FINITE VALUE | Valid between L_NVKRINT(l) and L_NVKRINT(l+1). |
| M_NVKTINT | 5 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | 255 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 / 4 | SR / UN |
| National Values | STORED | - |
| STM max speed | NOT STORED | - |
| STM system speed/distance | NOT STORED | - |
| Temporary Speed Restrictions | NOT STORED | - |
| Unconditional Emergency Stop | NOT STORED | - |
| Conditional Emergency Stop | NOT STORED | - |
| Train Position | STORED | - |
| Train Data | STORED | - |
| ERTMS/ETCS level | 0 / 2 / 3 / 4 | L0 / L1 / L2 / L3 |
| Driver ID | STORED | - |
| Train Running Number | STORED | - |
| Operated System Version | 010 0000 | Version 2.0, introduced in SRS 3.3.0 |
| Virtual Balise Covers | NOT STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | Both | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 7

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.13.7: Determination of Most Restrictive Speed Profile (MRSP) |  | 3130700 |
| Test Case of Feature | The Most Restrictive Speed Profile shall be computed from all speed restrictions by selecting the most restrictive parts of each element. |  | 7 |
| Applicable Mode/Level Combinations | L1: RV L2/3: RV |  |  |
| Target of Test | The on-board calculates the most restricted speed taking into account the following speed restrictions: Maximum Train Speed and Mode related Speed Restriction. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.13.7.2 |  |
|  |  | Subset-026-3.11.1.1 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI and JRU interfaces that the appropriate change is displayed and registered each time the speed supervision limit changes to a less restrictive value. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 14 | RV |
| Radio communication session | ESTABLISHED | Only in L2/3 |
| Unconditional Emergency Stop | NOT STORED | - |
| Conditional Emergency Stop | NOT STORED | - |
| Train Position | STORED | - |
| Train Data | STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Driver ID | STORED | - |
| Train Running Number | STORED | - |
| Reversing Area Information | STORED | - |
| Reversing Supervision Information | STORED | - |
| Operated System Version | 010 0000 | Version 2.0, introduced in SRS 3.3.0 |
| Virtual Balise Covers | NOT STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | Both | RTM | Only in L2/3 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is moving (V_TRAIN=<below V1>) | I | INT | See the meaning of V1 in the following steps. | N/A | N/A | - |
| 2 | N/A | N/A | The speed and distance monitoring information to be displayed | o | DMI | - | N/A | N/A | - |
| 3 | N/A | N/A | One Balise Group message containing packet 139 (V_REVERSE = <V1>) is received. | I | BTM | Only in L1. The length of the given distance to run in RV (D_REVERSE) shall be long enough to prevent the on-board equipment from braking to the end of that given distance during the execution of this TC (i.e., the on-board equipment will always be in a Ceiling Speed Monitoring section). V2 = maximum train speed. The relation between the values of the speed limits is: V1 < V2 . This location is at distance D0. | N/A | N/A | - |
| 4 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | Only in L1. | N/A | N/A | - |
| 5 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 139 (V_REVERSE = <V1>) is received | I | RTM | Only in L2/L3. The length of the given distance to run in RV (D_REVERSE) shall be long enough to prevent the on-board equipment from braking to the end of that given distance during the execution of this TC (i.e., the on-board equipment will always be in a Ceiling Speed Monitoring section). V2 = maximum train speed. The relation between the values of the speed limits is: V1 < V2 . This location is at distance D0. | N/A | N/A | - |
| 6 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | Only in L2/L3. | N/A | N/A | - |
| 7 | N/A | N/A | The speed and distance monitoring information is displayed (V_PERM = <V1>) | O | DMI | V1 = V_REVERSE | N/A | N/A | - |
| 8 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=0; M_SDMSUPSTAT=0; V_PERM = <V1>) is recorded | O | JRU | V1 = V_REVERSE M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal Status) | N/A | N/A | - |

| Step 3: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | 1 | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 139 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_REVERSE | 15 | FINITE VALUE | - |
| V_REVERSE | 7 | FINITE VALUE | V1 |
| NID_PACKET | 8 | 255 | 255 |

| Step 5: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 139 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_REVERSE | 15 | FINITE VALUE | - |
| V_REVERSE | 7 | FINITE VALUE | V1 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 14 | RV |
| Radio communication session | ESTABLISHED | Only in L2/3 |
| Unconditional Emergency Stop | NOT STORED | - |
| Conditional Emergency Stop | NOT STORED | - |
| Train Position | STORED | - |
| Train Data | STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Driver ID | STORED | - |
| Train Running Number | STORED | - |
| Reversing Area Information | STORED | - |
| Reversing Supervision Information | STORED | - |
| Operated System Version | 010 0000 | Version 2.0, introduced in SRS 3.3.0 |
| Virtual Balise Covers | NOT STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | Both | RTM | Only in L2/3 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |