| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 5110400 (5.11.4: Degraded Situations) Total: 2 Test case(s) |  |
| REF: | Subset-076-5-2-5110400 |
| ISSUE: | Version 3.2.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 0.0.1 23.03.2012 | All | First version/creation | DLR |
| Version 2.9.0 31.03.2013 | All | Second version/review for Baseline 3; Format template updated; Dictionary steps used. | INECO |
| Version 2.9.1 01.08.2013 | All | Third version/Internal modification;TC_Editorv27; Correction of Requirements blank space | INECO |
| Version 2.9.2 15.11.2013 | All | Fourth version/ UNISIG Review from 03.10.2013 included; TCEditor_v31 tool used | INECO |
| Version 2.9.3 09.12.2013 | All | Fifth version/ UNISIG Review from 07.01.2014 included | INECO |
| Version 2.9.4 07.01.2014 | All | Sixth version/ Internal version: Cross-reference checked; Old reference removed. | INECO |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.2.0 31.03.2017 | All | Baseline 3 1st maintenance release version - Bug fixing | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.11.4: Degraded Situations |  | 5110400 |
| Test Case of Feature | Accidental loss of an already open session while ERTMS/ETCS on-board equipment is in level 2/3 and in PT mode. The driver shall have the possibility to select "Override" and the process shall go to the procedure "Override " Driver exit from PT mode with OVERRIDE EOA. |  | 1 |
| Applicable Mode/Level Combinations | L2 / L3: PT |  |  |
| Target of Test | The target of this test is to check that : Radio link connection is lost The on-board Equipment allows the driver to select “Override” Procedure Override is triggered. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-5.11.4.2 |  |
|  |  | Subset-026-5.8.2.1 |  |
|  |  | Subset-026-5.8.2.1 a) |  |
|  |  | Subset-026-5.8.2.1 b) |  |
|  |  | Subset-026-5.8.2.1 c) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI interface that: The Status symbol of safe radio connection switches from “Connection Up" to “Connection Lost”. The Override symbol , MO03 is displayed The mode symbol SR is displayed Check on the RTM interface that: Safe Connection is lost Check on the JRU interface that: The mode change to SR is recorded. |
| Constraints | The train is at standstill. Once the Override Procedure has been triggered the mode shall change to SR. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 8 | PT |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Mode symbol "Post Trip" is displayed. The Status symbol "Safe radio connection - Connection Up" is displayed. | O | DMI | DMI Object: MO06 DMI Object: ST03 |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill | I | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2 / L3 | PT | Accidental loss of radio contact | - | - | USE_ FT3050400.4 | L2 / L3 | PT | - |
| 2 | L2 / L3 | PT | The <Override> button in the <Default> window is enabled. | O | DMI | - | L2 / L3 | PT | - |
| 3 | L2 / L3 | PT | The driver presses "Override” button. | I | DMI | Override Procedure is triggered | L2 / L3 | PT | - |
| 4 | L2 / L3 | PT | The Override window is displayed | O | DMI | - | L2 / L3 | PT | - |
| 5 | L2 / L3 | PT | The driver selects "Override EoA" | I | DMI | - | L2 / L3 | PT | - |
| 6 | L2 / L3 | PT | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 1110) is recorded | O | JRU | - | L2 / L3 | PT | - |
| 7 | L2 / L3 | PT | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=2) is recorded | O | JRU | - | L2 / L3 | SR | - |
| 8 | L2 / L3 | SR | The override symbol is displayed. | O | DMI | DMI Object: MO03 | L2 / L3 | SR | - |
| 9 | L2 / L3 | SR | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit18=1>) is recorded | O | JRU | - | L2 / L3 | SR | - |
| 10 | L2 / L3 | SR | The mode symbol “SR” is displayed. | O | DMI | DMI Object: MO09 When one of the Override End Procedure is fulfilled (e.g. the max time T_NVOVTRP for train trip suppression elapses) | L2 / L3 | SR | - |
| 11 | L2 / L3 | SR | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit24=1>) is recorded | O | JRU | - | L2 / L3 | SR | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 | SR |
| Radio communication session | NOT ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The mode symbol “Staff Responsible” is displayed. The Status symbol "Safe radio connection - Connection Lost/Set-Up failed" is displayed. | O | DMI | DMI Object: MO09 DMI Object: ST04 |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill | I | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.11.4: Degraded Situations |  | 5110400 |
| Test Case of Feature | ERTMS/ETCS level 2 or 3: no acknowledge for PT mode is received from the RBC. The on-board equipment has entered in PT mode. It sends to the RBC a radio message 136 “Train Position Report” indicating the entry in PT mode. As it receives no answer, it repeats the PT mode report a defined number of times. After unanswered PT mode report, the on-board equipment sends a radio message 156 “Termination of a communication session” to the RBC. It considers the communication session terminated and releases the safe connection. |  | 2 |
| Applicable Mode/Level Combinations | L2: PT L3: PT |  |  |
| Target of Test | The target of this test is to check that : The on-board repeats a defined numbers of times (4 times each 15 s) the transmission of a radio message 136 “Train Position Report” indicating the entry in PT mode when no answer is received from the RBC. The termination of the communication session is initiated by the on-board equipment in 1 minute when no answer to the last send PT mode report message is received. It sends a radio message 156 “Termination of a communication session” to the RBC. No further message is sent by the on-board once the radio message 156 has been sent. The on-board considers the communication session terminated and releases the safe connection. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-5.11.4.1.1 |  |
|  |  | Subset-026-5.11.4.1.2 |  |
|  |  | Subset-026-8.6.4 Message 136: Train Position Report |  |
|  |  | Subset-026-8.6.14 Message 156: Termination of a communication session |  |
|  |  | Subset-026-A.3.1 - Waiting time before radio message repetition |  |
|  |  | Subset-026-A.3.1 - Additional delay time to disconnection on supervision of safe radio connection |  |
|  |  | Subset-026-4.8.4.2 Session management |  |
|  |  | Subset-026-4.8.3.1.1 Session management |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the RTM that: Four radio messages 136 (Train Position Report) are sent to the RBC. A radio message 156 (Termination of a communication session) is sent to the RBC. Check on the JRU that: The four radio messages 136 (Train Position Report) sent by the on-board are RECORDED. The radio message 156 (Termination of a communication session) sent by the on-board is RECORDED. Check on the DMI that: The Status symbol "Safe radio connection - Connection Up" is removed. |
| Constraints | ERTMS/ETCS level is 2 or 3: no acknowledge for PT mode is received from the RBC. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 8 | PT |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Mode symbol "Post Trip" is displayed. The Status symbol "Safe radio connection - Connection Up" is displayed. | O | DMI | DMI Object: MO06 DMI Object: ST03 |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill | I | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted. | O | RTM | - | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted. | O | RTM | No answer is received from the RBC and a time out occurs (15 s): The first repetition of position report message indicating the entry in PT mode is sent to the RBC. | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded. | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted. | O | RTM | No answer is received from the RBC and a time out occurs (15 s): The second repetition of position report message indicating the entry in PT mode is sent to the RBC. | N/A | N/A | - |
| 6 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded. | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted. | O | RTM | No answer is received from the RBC and a time out occurs (15 s): The third and last repetition of position report message indicating the entry in PT mode is sent to the RBC. | N/A | N/A | - |
| 8 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded. | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | SA-DATA. Request with Euroradio Message "Termination of a communication session" (NID_MESSAGE=156) is transmitted | O | RTM | No answer is received from the RBC and a time out occurs (60s ): It shall be checked that the on-board does not send more messages after this one. The on-board considers the communication session terminated. | N/A | N/A | - |
| 10 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 11 | N/A | N/A | SA-DISCONNECT. Request is transmitted | O | RTM | - | N/A | N/A | - |
| 12 | N/A | N/A | The Status symbol "Safe radio connection - Connection Up" is removed. | O | DMI | - | N/A | N/A | - |
| 13 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit40=0>) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE |  |
| M_MODE | 4 | 8 | PT |
| M_LEVEL | 3 | FINITE VALUE | - |

| Step 3: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE |  |
| M_MODE | 4 | 8 | PT |
| M_LEVEL | 3 | FINITE VALUE | - |

| Step 5: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE |  |
| M_MODE | 4 | 8 | PT |
| M_LEVEL | 3 | FINITE VALUE | - |

| Step 7: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE |  |
| M_MODE | 4 | 8 | PT |
| M_LEVEL | 3 | FINITE VALUE | - |

| Step 9: Radio Message 156 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 156 | Termination of a communication session |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 1 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 8 | PT |
| Radio communication session | NOT ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill | I | INT | - |