| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 6060304 (6.6.3.4 Messages transmitted to RBC/RIU) Total: 11 Test case(s) |  |
| REF: | Subset-076-5-2-6060304 |
| ISSUE: | Version 3.2.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 2.9.0 31.03.2013 | All | First version/New in Baseline 3 | INECO |
| Version 2.9.1 28.05.2013 | All | Second version/ UNISIG review comments from 13-05-2013 included; TC11 created new. | INECO |
| Version 2.9.2 15.07.2013 | TC11 | Third version/ line as 5,6,26,30A,35 and 36 from UNISIG review sheet(13-05-2013) answered; Step 10&11 from TC11 changed. | INECO |
| Version 2.9.3 01.08.2013 | All | Fourth version/Internal modification;TC_Editorv27; Correction of Requirements blank space. | INECO |
| Version 2.9.4 07.01.2014 | All | Fifth version/ Internal version: Cross-reference checked; Old reference removed. | INECO |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.2.0 31.03.2017 | All | Baseline 3 1st maintenance release version - Bug fixing | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 6.6.3.4 Messages transmitted to RBC/RIU |  | 6060304 |
| Test Case of Feature | Transmission of the message 132: “MA Request” by the on-board equipment to an RBC certified to the system version number X = 1. Tested action: Data is translated ” T[4]” & data is unchanged “U[1]” |  | 1 |
| Applicable Mode/Level Combinations | L2 / L3: FS/ OS |  |  |
| Target of Test | The target of this test is to check that: A situation which leads to shorten the current MA and track description to the current position of the on-board equipment or the new stop location. The on-board equipment informs the RBC/RIU by sending a radio message 132 “MA request” that is translated for the RBC system version X=1 as: the variable Q_MARQSTREASON is being replaced with Q_TRACKDEL. The transmitted value to X=1 RBC: Q_MARSQREASON = x1xxx into Q_TRACKDEL set to 1. The position report packet should be sent with M_MODE unchanged for FS or OS mode. The RBC/RIU understands correctly the received message and sends a valid Movement Authority. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-6.6.3.4.4 |  |
|  |  | Subset-026-6.6.3.4.5 Table - row 1 |  |
|  |  | Subset-026-6.6.3.4.5 Table - row 10 |  |
|  |  | Subset-026-6.6.3.4.5 [4] |  |
|  |  | Subset-026-7.4.3.1 Packet Number 0: Position Report |  |
|  |  | Subset-026-7.4.2.4 Packet Number 15: Level 2/3 Movement Authority |  |
|  |  | Subset-026-7.4.2.6 Packet Number 21: Gradient Profile |  |
|  |  | Subset-026-7.4.2.7 Packet Number 27: International Static Speed Profile |  |
|  |  | Subset-026-8.6.3 Message 132: MA Request |  |
|  |  | Subset-026-8.7.2 Message 3: Movement Authority |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the RTM that: Data from message 132 and packet 0 are translated correctly by the on-board and the trackside send the requested information.. Check on the JRU that: The Message 132 :MA Request is RECORDED. The Message 3: Movement Authority is RECORDED. |
| Constraints | This section applies for the parts of radio messages/packets, which are transmitted to an RBC or an RIU certified to the system version number X = 1. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 | FS / OS |
| Radio communication session | ESTABLISHED | System version X=1 |
| Linking | STORED | - |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | M_VERSION | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | No linked balise group message is received | I | BTM | - | The balise group missed was linked with service brake reaction | N/A | N/A | - |
| 2 | N/A | N/A | Service brake commanded. | O | TIU | - | - | N/A | N/A | - |
| 3 | N/A | N/A | The Status symbol "Service Brake " is displayed. | O | DMI | - | Brake command indication due to RAMS related supervision (service brake) Symbol ST01 | N/A | N/A | - |
| 4 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded. | O | JRU | - | - | N/A | N/A | - |
| 5 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; bit38=1) is recorded. | O | JRU | - | - | N/A | N/A | - |
| 6 | N/A | N/A | Train is at standstill (V_TRAIN=0; TIME= 60) | I | INT | - | - | N/A | N/A | - |
| 7 | N/A | N/A | The speed and distance monitoring information is removed | O | DMI | - | - | N/A | N/A | - |
| 8 | N/A | N/A | SA-DATA. Request with Euroradio Message "MA Request" (NID_MESSAGE=132) containing packet 0 is transmitted to a system version X=1 RBC. | O | RTM | 001 0000 | Q_TRACKDEL = 1 For Message132, data is translated as: Q_MARQSTREASON is replaced with Q_TRACKDEL And value x1xxx is set to 1 (track description deleted) | N/A | N/A | - |
| 9 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded. | O | JRU | - | - | N/A | N/A | - |
| 10 | N/A | N/A | SA-DATA. Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 15, packet 21 and packet 27 is received. | I | RTM | 001 0000 | The onboard should take into account exception (3) requirement 6.6.3.2.3 | N/A | N/A | - |
| 11 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded. | O | JRU | - | - | N/A | N/A | - |

| Step 8: Radio Message 132 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 132 | MA request |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 1 | FINITE VALUE | - |
| Q_TRACKDEL | 1 | 1 | Track description deleted |
| NID_PACKET | 8 | 0 | Position report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = “Train integrity confirmed by integrity monitoring device” or “Train integrity confirmed by driver” |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | 0 / 1 | 0 / 1 |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC |

| Step 10: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | Movement Authority |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | Level 2/3 Movement Authority |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to “no time-out” |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | Gradient Profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | International Static Speed Profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | - |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| NC_DIFF(n) | 4 | FINITE VALUE | - |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | - |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| NC_DIFF(k,m) | 4 | FINITE VALUE | - |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1 | FS/ OS |
| Radio communication session | ESTABLISHED | UNCHANGED |
| Movement Authority | STORED | CHANGED |
| Gradient Profile | STORED | CHANGED |
| International SSP | STORED | CHANGED |
| ERTMS/ETCS level | 3/ 4 | L2/ L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 6.6.3.4 Messages transmitted to RBC/RIU |  | 6060304 |
| Test Case of Feature | Transmission of the message 132: “MA Request” by the on-board equipment to an RBC certified to the system version number X = 1.Reception of the message 3:” Movement Authority” from the X=1 RBC. Tested action: Data is translated: T[4] & data is unchanged U[1] |  | 2 |
| Applicable Mode/Level Combinations | L2 / L3: FS/ OS |  |  |
| Target of Test | The target of this test is to check that: The on-board equipment requests a MA to the RBC/RIU by sending message 132 that is translated for the RBC system version X=1 as: the variable Q_MARQTREASON is being replaced with Q_TRACKDEL. The transmitted value to X=1 RBC: Q_MARSQREASON ≠ x1xxx into Q_TRACKDEL set to 0. The position report packet should be sent with M_MODE unchanged for FS or OS mode. The RBC/RIU understands the received message and sends a valid Movement Authority. Message 3 with packet 15 is received correctly and no translation is necessary. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-6.6.3.3.2 Table - row 2 |  |
|  |  | Subset-026-6.6.3.4.4 |  |
|  |  | Subset-026-6.6.3.4.5 Table - row 2 |  |
|  |  | Subset-026-6.6.3.4.5 Table - row 10 |  |
|  |  | Subset-026-6.6.3.4.5 [1] |  |
|  |  | Subset-026-6.6.3.4.5 [4] |  |
|  |  | Subset-026-7.4.3.2 Packet Number 1: Position Report based on two balise groups |  |
|  |  | Subset-026-7.4.2.4 Packet Number 15: Level 2/3 Movement Authority |  |
|  |  | Subset-026-8.6.3 Message 132: MA Request |  |
|  |  | Subset-026-8.7.2 Message 3: Movement Authority |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the RTM that: Data from message 132 is translated correctly by the on-board and the trackside send the requested information Data received from trackside, message 3 with packets 15 is used correctly by the on-board equipment. Check on the JRU that: The Message 132 :MA Request is RECORDED. The Message 3: Movement Authority with packets15 is RECORDED. |
| Constraints | This section applies for the parts of radio messages/packets, which are transmitted to an RBC or an RIU certified to the system version number X = 1. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1 | FS/ OS |
| Radio communication session | ESTABLISHED | System version X=1 |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| ERTMS/ETCS level | 3/ 4 | L2/ L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | M_VERSION | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA. Request with Euroradio Message "MA Request" (NID_MESSAGE=132) containing packet 1 is transmitted to a system version X=1 RBC. | O | RTM | 001 0000 | Q_TRACKDEL=0 For Message132, the on-board effectues the translation to system version X=1 as: Q_MARQSTREASON is replaced with Q_TRACKDEL And Value diferent from x1xxx is set to 0 (no track description deleted) | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded. | O | JRU | - | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA. Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) | I | RTM | 001 0000 | No data translation is necesary.Data from message and packet remains unchanged. | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded. | O | JRU | - | - | N/A | N/A | - |
| 5 | N/A | N/A | The speed and distance monitoring information is displayed. | O | DMI | - | MA and track description for level 2/3 is used. | N/A | N/A | - |
| 6 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | - | - | N/A | N/A | - |

| Step 1: Radio Message 132 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 132 | MA request |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 1 | FINITE VALUE | - |
| Q_TRACKDEL | 1 | 0 | No Track description deleted |
| NID_PACKET | 8 | 1 | Position report based on two balise groups |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PRVLRBG | - | - | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = “Train integrity confirmed by integrity monitoring device” or “Train integrity confirmed by driver” |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | 0 / 1 | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC |

| Step 3: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | Movement Authority |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | Level 2/3 Movement Authority |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to “no time-out” |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1 | FS/ OS |
| Radio communication session | ESTABLISHED | UNCHANGED |
| Movement Authority | STORED | New MA |
| Gradient Profile | STORED | UNCHANGED |
| International SSP | STORED | UNCHANGED |
| ERTMS/ETCS level | 3/ 4 | L2/ L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 6.6.3.4 Messages transmitted to RBC/RIU |  | 6060304 |
| Test Case of Feature | Transmission of the message 136: “Train Position report ” with packet 4: ”Error Reporting ” by the on-board equipment to an RBC/RIU certified to the system version number X = 1. Tested action: data is Unchanged “U[2]”. |  | 3 |
| Applicable Mode/Level Combinations | L2/ L3: SL/ NL |  |  |
| Target of Test | The target of this test is to check that : Packet 4: “Error Reporting” is properly translated and the error reported by the on-board equipment to the X=1 RBC/RIU is understood as a Fatal Error. Due to a Safety Critical Fault in a sleeping/non leading engine the train is not stopped, if possible, and the on-board equipment informs RBC/RIU about the error by sending the message 136 with packet 4. The on-board equipment sets the variable M_ERROR, from packet 4, from 6(X=2) to 7(X=1). The RBC if informed about the Fatal Error (Equipment in SL or NL mode). |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-6.6.3.4.4 |  |
|  |  | Subset-026-6.6.3.4.5 Table - row 4 |  |
|  |  | Subset-026-6.6.3.4.5 Table - row 11 |  |
|  |  | Subset-026-6.6.3.4.5 [2] |  |
|  |  | Subset-026-7.4.3.1 Packet Number 0: Position Report |  |
|  |  | Subset-026-7.4.3.4 Packet Number 4: Error reporting |  |
|  |  | Subset-026-8.6.4 Message 136: Train Position Report |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the RTM that: Data from packet 4 (M_ERROR) sent by the on-board equipment is understood by the RBC as a Fatal Error. Check on the JRU that: The radio message 136 (Train Position Report) with the packet 4 (Error Reporting) is RECORDED. Additional check: M_ERROR = 7 |
| Constraints | This section applies for the parts of radio messages/packets, which are transmitted to an RBC or an RIU certified to the system version number X = 1. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 5/ 11 | SL/ NL |
| Radio communication session | ESTABLISHED | System version X=1 |
| ERTMS/ETCS level | 3/ 4 | L2 /L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | M_VERSION | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | A safety critical fault occur | I | INT | - | - | N/A | N/A | - |
| 2 | N/A | N/A | SAFETY CRITICAL FAULT IN MODE SL, NL OR PS (NID_MESSAGE_JRU=27) is recorded | O | JRU | - | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 4 is transmitted to the X=1 RBC. | O | RTM | 001 0000 | Safety Critical failure For packet 4, data is translated as: M_ERROR value set from 6 to 7 The RBC understands the error as a Fatal Error (Equipment in SL or NL mode) | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded. | O | JRU | - | - | N/A | N/A | - |

| Step 3: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train position report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = “Train integrity confirmed by integrity monitoring device” or “Train integrity confirmed by driver” |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | 5 / 11 | SL / NL |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC |
| NID_PACKET | 8 | 4 | Error Reporting |
| L_PACKET | 13 | 29 | - |
| M_ERROR | 8 | 7 | Safety critical failure transmitted as Fatal Error Value 6 set to 7 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 5/ 11 | SL/ NL |
| Radio communication session | ESTABLISHED | UNCHANGED |
| ERTMS/ETCS level | 3/ 4 | L2 /L3 |
| Radio Network ID | STORED | UNCHANGED |
| RBC ID/Phone Number | STORED | UNCHANGED |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 6.6.3.4 Messages transmitted to RBC/RIU |  | 6060304 |
| Test Case of Feature | No transmission of packet 4: Error Reporting by the on-board equipment to an RBC certified to the system version number X = 1. Tested action: data is unchanged “U[2]”. |  | 4 |
| Applicable Mode/Level Combinations | L2/ L3: FS/ LS /OS |  |  |
| Target of Test | The target of this test is to check that: The on-board equipment deletes packet 4: “Error Reporting” from the message 136 when the error is due to a double linking error.(M_ERROR = 7) (X=2) The on-board equipment does not inform RBC/RIU about the error of not finding two balise groups included in the linking information. Message 136 is sent without packet 4. Reason: Value of M_ERROR = 7 not understood by the RBC certified with system version X=1. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-6.6.3.4.4 |  |
|  |  | Subset-026-6.6.3.4.5 Table - row 4 |  |
|  |  | Subset-026-6.6.3.4.5 Table - row 11 |  |
|  |  | Subset-026-6.6.3.4.5 [2] |  |
|  |  | Subset-026-3.16.2.7.1.1 |  |
|  |  | Subset-026-7.4.3.1 Packet Number 0: Position Report |  |
|  |  | Subset-026-8.6.4 Message 136: Train Position Report |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the RTM that: Packet 4 (M_ERROR=7) is not sent by the on-board equipment with message 136. Check on the JRU that: The radio message 136 (Train Position Report) without packet 4 (Error Reporting) is RECORDED. |
| Constraints | This section applies for the parts of radio messages/packets, which are transmitted to an RBC or an RIU certified to the system version number X = 1. Linking information stored on-board. The reference locations of the two consecutive BG (marked as linked and included in the linking information) are not found in the expectation windows. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 12 | FS/ OS /LS |
| Radio communication session | ESTABLISHED | System version X=1 |
| Linking | STORED | - |
| ERTMS/ETCS level | 3/ 4 | L2 /L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | M_VERSION | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | NOT One Balise Group message is received | I | BTM | - | Train passes the expectation window of the first announced BG A. The balise group is not detected. | N/A | N/A | - |
| 2 | N/A | N/A | NOT One Balise Group message is received | I | BTM | - | Train passes the expectation window of the second announced BG B. The balise group is not detected. | N/A | N/A | - |
| 3 | N/A | N/A | The system status message "Balise read error" is displayed | O | DMI | - | - | N/A | N/A | - |
| 4 | N/A | N/A | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23) is recorded | O | JRU | - | - | N/A | N/A | - |
| 5 | N/A | N/A | Service brake commanded. | O | TIU | - | - | N/A | N/A | - |
| 6 | N/A | N/A | The Status symbol "Service Brake " is displayed. | O | DMI | - | Brake command indication due to RAMS related supervision (service brake) Symbol ST01 | N/A | N/A | - |
| 7 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded. | O | JRU | - | - | N/A | N/A | - |
| 8 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; bit38=1) is recorded. | O | JRU | - | - | N/A | N/A | - |
| 9 | N/A | N/A | Train reaches standstill. Train is at standstill. | I | INT | - | V_MAIN = 0 | N/A | N/A | - |
| 10 | N/A | N/A | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) to the RBC X=1. | O | RTM | 001 0000 | Packet 4 is deleted because M_ERROR = 7 (X=2). The RBC not informed about the double linking error. | N/A | N/A | - |
| 11 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded. | O | JRU | - | - | N/A | N/A | - |
| 12 | N/A | N/A | Service brake not commanded. | O | TIU | - | - | N/A | N/A | - |
| 13 | N/A | N/A | The Status symbol "Service Brake intervention" is removed. | O | DMI | - | - | N/A | N/A | - |
| 14 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=0) is recorded. | O | JRU | - | - | N/A | N/A | - |
| 15 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; bit38=0) is recorded. | O | JRU | - | - | N/A | N/A | - |

| Step 10: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train position report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = “Train integrity confirmed by integrity monitoring device” or “Train integrity confirmed by driver” |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | 0 / 1 | 0 / 1 |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 12 | FS/ OS /LS |
| Radio communication session | ESTABLISHED | UNCHANGED |
| ERTMS/ETCS level | 2/ 3/ 4 | L1 /L2 /L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 5

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 6.6.3.4 Messages transmitted to RBC/RIU |  | 6060304 |
| Test Case of Feature | No transmission of packet 4: Error Reporting by the on-board equipment to an RBC certified to the system version number X = 1. Tested action: data is unchanged “U[2]”. |  | 5 |
| Applicable Mode/Level Combinations | L1 : FS/ OS |  |  |
| Target of Test | The target of this test is to check that: The on-board equipment deletes packet 4: “Error Reporting” from the message 136 when the identity of the error is due to a double repositioning error.(M_ERROR = 8) (X=2) The on-board equipment sends a position report to the RBC without sending the error of double repositioning error. Message 136 is send without packet 4. Reason: Value of M_ERROR = 8 not understood by the RBC certified with system version X=1. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-6.6.3.4.4 |  |
|  |  | Subset-026-6.6.3.4.5 Table - row 4 |  |
|  |  | Subset-026-6.6.3.4.5 Table - row 11 |  |
|  |  | Subset-026-6.6.3.4.5 [2] |  |
|  |  | Subset-026-3.16.2.7.2.2 |  |
|  |  | Subset-026-7.4.2.5 Packet Number 16: Repositioning Information |  |
|  |  | Subset-026-7.4.3.1 Packet Number 0: Position Report |  |
|  |  | Subset-026-8.6.4 Message 136: Train Position Report |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the RTM that: Packet 4 (M_ERROR=8) is not sent by the on-board equipment with message 136. Check on the JRU that: The radio message 136 (Train Position Report) without packet 4 (Error Reporting) is RECORDED. |
| Constraints | This section applies for the parts of radio messages/packets, which are transmitted to an RBC or an RIU certified to the system version number X = 1. Linking information with repositioning information (BG1 with NID_BG=16383) stored on-board. A second BG is found within the window of BG1 with the same criteria. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1 | FS/ OS |
| Radio communication session | ESTABLISHED | System version X=1 |
| Linking | STORED | BG1 marked as NID_BG=16383 BG2 not included in the linking information |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | M_VERSION | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Balise Group 1 message (N_TOTAL=1; NID_C=<country/region>) containing packet 16 is received | I | BTM | 001 0000 | The Balise group is passed before D_LINK | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | - | N/A | N/A | - |
| 3 | N/A | N/A | The distance monitoring information is displayed | O | DMI | - | TDM and MA length are updated. | N/A | N/A | - |
| 4 | N/A | N/A | DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | - | - | N/A | N/A | - |
| 5 | N/A | N/A | Balise Group 2 message (N_TOTAL=1; NID_C=<country/region>) containing packet 16 is received | I | BTM | 001 0000 | The on-board antenna hasn’t left the expectation window of the BG1. BG2 found satisfied the same criteria as BG1 | N/A | N/A | - |
| 6 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | - | N/A | N/A | - |
| 7 | N/A | N/A | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23) is recorded | O | JRU | - | - | N/A | N/A | - |
| 8 | N/A | N/A | Service brake commanded. | O | TIU | - | - | N/A | N/A | - |
| 9 | N/A | N/A | The Status symbol "Service Brake " is displayed. | O | DMI | - | Symbol ST01 | N/A | N/A | - |
| 10 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded. | O | JRU | - | - | N/A | N/A | - |
| 11 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; bit38=1) is recorded. | O | JRU | - | - | N/A | N/A | - |
| 12 | N/A | N/A | Train reaches standstill. Train is at standstill. | I | INT | - | V_MAIN = 0 | N/A | N/A | - |
| 13 | N/A | N/A | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) to the RBC X=1. | O | RTM | 001 0000 | Packet 4 is deleted because M_ERROR = 8 (X=2). The RBC not informed about the double repositioning error. | N/A | N/A | - |
| 14 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded. | O | JRU | - | - | N/A | N/A | - |
| 15 | N/A | N/A | The new speed and distance monitoring information is displayed. | O | DMI | - | MA and Track description are shortened to the current train position. Driver is informed. | N/A | N/A | - |
| 16 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | - | - | N/A | N/A | - |
| 17 | N/A | N/A | Service brake not commanded. | O | TIU | - | - | N/A | N/A | - |
| 18 | N/A | N/A | The Status symbol "Service Brake or Emergency Brake intervention" is removed. | O | DMI | - | - | N/A | N/A | - |
| 19 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=0) is recorded. | O | JRU | - | - | N/A | N/A | - |
| 20 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; bit38=0) is recorded. | O | JRU | - | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 010 0000 | Version 2.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | First |
| N_TOTAL | 3 | 1 | Two balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BG 1 |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 16 | Repositioning information |
| Q_DIR | 2 | 01 | Nominal direction |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| L_SECTION | 15 | FINITE VALUE | New length of movement authority |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 010 0000 | Version 2.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 001 | Second |
| N_TOTAL | 3 | 1 | Two balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BG 1 |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 5: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 010 0000 | Version 2.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | First |
| N_TOTAL | 3 | 1 | Two balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BG 2 |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 16 | Repositioning information |
| Q_DIR | 2 | 01 | Nominal direction |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| L_SECTION | 15 | FINITE VALUE | New length of movement authority |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 5: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 010 0000 | Version 2.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 001 | Second |
| N_TOTAL | 3 | 1 | Two balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BG 2 |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 13: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train position report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = “Train integrity confirmed by integrity monitoring device” or “Train integrity confirmed by driver” |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | 0 / 1 | 0 / 1 |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 12 | FS/ OS /LS |
| Radio communication session | ESTABLISHED | UNCHANGED |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 6

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 6.6.3.4 Messages transmitted to RBC/RIU |  | 6060304 |
| Test Case of Feature | Transmission and translation of packet 11: “Validated Train data” with message 129 by the on-board equipment to an RBC certified to the system version number X = 1. Tested action: Data is translated T[3] |  | 6 |
| Applicable Mode/Level Combinations | L0 : UN/ SB/ TR LNTC: SN/ SB/ TR L1: FS/ LS/ OS/ SR/ SB/ TR/ PT L2: FS/ LS/ OS/ SR/ SB/ TR/ PT L3: FS/ LS/ OS/ SR/ SB/ TR/ PT |  |  |
| Target of Test | The target of this test is to check that: The on-board equipment translates correctly packet 11: “Validated Train Data” to the X=1 RBC. Following any entry/ modification of TRAIN DATA when a communication session is already established or following the successful establishment of a communication session, the on-board equipment sends a valid set of Train Data to the RBC. Message 129 with packet 11. Translation applies to variables as: NID_OPERATIONAL[3a], NC_TRAIN [3b], M_LOADINGGAUGE [3c], M_AXLELOAD[3d] and N_ITER [3e] The RBC (X=1) understand correctly and acknowledges the reception of this set of Train Data with message 8: “Acknowledgment of Train Data”. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-6.6.3.3.2 Table - row 4 |  |
|  |  | Subset-026-6.6.3.4.4 |  |
|  |  | Subset-026-6.6.3.4.5 Table - row 7 |  |
|  |  | Subset-026-6.6.3.4.5 [3] |  |
|  |  | Subset-026-6.6.3.4.5 [3a] |  |
|  |  | Subset-026-6.6.3.4.5 [3b] |  |
|  |  | Subset-026-6.6.3.4.5 [3c] |  |
|  |  | Subset-026-6.6.3.4.5 [3d] |  |
|  |  | Subset-026-6.6.3.4.5 [3e] |  |
|  |  | Subset-026-3.18.3.4 |  |
|  |  | Subset-026-7.4.3.1 Packet Number 0: Position Report |  |
|  |  | Subset-026-7.4.3.5 Packet Number 11: Validated train data |  |
|  |  | Subset-026-8.6.1 Message 129: Validated Train Data |  |
|  |  | Subset-026-8.7.4 Message 8: Acknowledgement of Train Data |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : The transmitted radio message 129 with packet 11 is RECORDED. The acknowledgment of the RBC, message 8 is RECORDED |
| Constraints | Radio communication session is established Following any entry/modification of Train Data or following the successful establishment of a communication session A Cab is active (for SB mode) The train belongs to a “Cant Deficiency “ train category. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7/ 8/ 12/ 13 | FS/ OS/ SR/ UN/ SB/ TR/ PT/ LS/ SN |
| Radio communication session | ESTABLISHED | - |
| Train Data | STORED | The Stored Variables values for t0 (X=2) are: NC_CDTRAIN = 5 M_LOADINGGAUGE ≠0 M_AXLELOAD= D2/D3/D4/D4xL N_ITER ≠0 |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC/ L1 / L2 / L3 |
| Train Running Number | STORED | NID_OPERATIONAL = NID_OPERATIONAL1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | M_VERSION | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA. Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) containing packet 11 is transmitted to the X=1 RBC. | O | RTM | 001 0000 | For Message 129 no translation is necessary. For Packet 11 the translation is: NID_OPERATIONAL=NID_OPERTAIONAL1 NC_CDTRAIN translated to NC_TRAIN and set to xxx xxxx x1xx xxxx M_LOADINGGAUGE set to 0 M_AXLELOAD= 0101101 N_ITER set to 0 | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA. Indication with Euroradio Message "Acknowledgement of Train Data" (NID_MESSAGE=8) is received | I | RTM | 001 0000 | The RBC understands and acknowledges this set of Train Data. | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | - | N/A | N/A | - |

| Step 1: Radio Message 129 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 129 | Validated train data |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | t0 |
| NID_ENGINE | 24 | FINITE VALUE | Engine ID of the train |
| NID_PACKET | 8 | 0 | Position report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | BGa / SBGa |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = 1 or 2 |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | One of the applicable mode |
| M_LEVEL | 3 | FINITE VALUE | One of the applicable level |
| NID_PACKET | 8 | 11 | Validated train data |
| L_PACKET | 13 | FINITE VALUE | - |
| NID_OPERATIONAL | 32 | FINITE VALUE | NID_OPERATIONAL = NID_OPERATIONAL1 |
| NC_TRAIN | 15 | FINITE VALUE | International Train category |
| L_TRAIN | 12 | FINITE VALUE | - |
| V_MAXTRAIN | 7 | FINITE VALUE | - |
| M_LOADINGGAUGE | 8 | 0 | - |
| M_AXLELOAD | 7 | 0101101 | 22,5 t |
| M_AIRTIGHT | 2 | FINITE VALUE | Airtight system presence |
| N_ITER | 5 | 0 | - |
| M_TRACTION (k) | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | Type of National System available |

| Step 3: Radio Message 8 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 8 | Acknowledgement of Train Data |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | t0+ t’ with t’ > 0 |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | Reference to received train data message : t0 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7/ 8/ 12/ 13 | FS/ OS/ SR/ UN/ SB/ TR/ PT/ LS/ SN |
| Radio communication session | ESTABLISHED | UNCHANGED |
| Train Data | STORED | UNCHANGED |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC/ L1 / L2 / L3 |
| Train Running Number | STORED | UNCHANGED |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 7

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 6.6.3.4 Messages transmitted to RBC/RIU |  | 6060304 |
| Test Case of Feature | Transmission and translation of packet 11: “Validated Train data” with message 129 by the on-board equipment to an RBC certified to the system version number X = 1. Tested action: Data is translated T[3] |  | 7 |
| Applicable Mode/Level Combinations | L0 : UN/ SB/ TR LNTC: SN/ SB/ TR L1: FS/ LS/ OS/ SR/ SB/ TR/ PT L2: FS/ LS/ OS/ SR/ SB/ TR/ PT L3: FS/ LS/ OS/ SR/ SB/ TR/ PT |  |  |
| Target of Test | The target of this test is to check that: The on-board equipment translates correctly packet 11: “Validated Train Data” to the X=1 RBC. Following any entry/ modification of TRAIN DATA when a communication session is already established or following the successful establishment of a communication session, the on-board equipment sends a valid set of Train Data to the RBC. Message 129 with packet 11. Translation applies to variables as: NID_OPERATIONAL[3a], NC_TRAIN [3b], M_LOADINGGAUGE [3c], M_AXLELOAD[3d] and N_ITER [3e] The RBC (X=1) understand correctly and acknowledges the reception of this set of Train Data with message 8: “Acknowledgment of Train Data”. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-6.6.3.3.2 Table - row 4 |  |
|  |  | Subset-026-6.6.3.4.4 |  |
|  |  | Subset-026-6.6.3.4.5 Table - row 7 |  |
|  |  | Subset-026-6.6.3.4.5 [3] |  |
|  |  | Subset-026-6.6.3.4.5 [3a] |  |
|  |  | Subset-026-6.6.3.4.5 [3b] |  |
|  |  | Subset-026-6.6.3.4.5 [3c] |  |
|  |  | Subset-026-6.6.3.4.5 [3d] |  |
|  |  | Subset-026-6.6.3.4.5 [3e] |  |
|  |  | Subset-026-3.18.3.4 |  |
|  |  | Subset-026-7.4.3.1 Packet Number 0: Position Report |  |
|  |  | Subset-026-7.4.3.5 Packet Number 11: Validated train data |  |
|  |  | Subset-026-8.6.1 Message 129: Validated Train Data |  |
|  |  | Subset-026-8.7.4 Message 8: Acknowledgement of Train Data |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : The transmitted radio message 129 with packet 11 is RECORDED. The acknowledgment of the RBC, message 8 is RECORDED |
| Constraints | Radio communication session is established Following any entry/modification of Train Data or following the successful establishment of a communication session A Cab is active (for SB mode) The train belongs to “other international “ train category. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7/ 8/ 12/ 13 | FS/ OS/ SR/ UN/ SB/ TR/ PT/ LS/ SN |
| Radio communication session | ESTABLISHED | - |
| Train Data | STORED | The Stored Variables values for t0 (X=2) are: NC_TRAIN = xxx xxxx xxxx xx1x M_LOADINGGAUGE ≠0 M_AXLELOAD= HS17 N_ITER ≠0 |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC/ L1 / L2 / L3 |
| Train Running Number | STORED | NID_OPERATIONAL = NID_OPERATIONAL1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | M_VERSION | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA. Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) containing packet 11 is transmitted to the X=1 RBC. | O | RTM | 001 0000 | For Message 129 no translation is needed. For Packet 11 the translation is: NID_OPERATIONAL = NID_OPERATIONAL1 NC_TRAIN set to xxx x1xx xxxx xxxx M_LOADINGGAUGE set to 0 M_AXLELOAD= 0100010 N_ITER set to 0 | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA. Indication with Euroradio Message "Acknowledgement of Train Data" (NID_MESSAGE=8) is received | I | RTM | 001 0000 | The RBC understands and acknowledges this set of Train Data. | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | - | N/A | N/A | - |

| Step 1: Radio Message 129 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 129 | Validated train data |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | t0 |
| NID_ENGINE | 24 | FINITE VALUE | Engine ID of the train |
| NID_PACKET | 8 | 0 | Position report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | BGa / SBGa |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = 1 or 2 |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | One of the applicable mode |
| M_LEVEL | 3 | FINITE VALUE | One of the applicable level |
| NID_PACKET | 8 | 11 | Validated train data |
| L_PACKET | 13 | FINITE VALUE | - |
| NID_OPERATIONAL | 32 | FINITE VALUE | NID_OPERATIONAL = NID_OPERATIONAL1 |
| NC_TRAIN | 15 | FINITE VALUE | Other International Train category |
| L_TRAIN | 12 | FINITE VALUE | - |
| V_MAXTRAIN | 7 | FINITE VALUE | - |
| M_LOADINGGAUGE | 8 | 0 | - |
| M_AXLELOAD | 7 | 0100010 | 17 t |
| M_AIRTIGHT | 2 | FINITE VALUE | Airtight system presence |
| N_ITER | 5 | 0 | - |
| M_TRACTION (k) | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | Type of National System available |

| Step 3: Radio Message 8 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 8 | Acknowledgement of Train Data |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | t0+ t’ with t’ > 0 |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | t0 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7/ 8/ 12/ 13 | FS/ OS/ SR/ UN/ SB/ TR/ PT/ LS/ SN |
| Radio communication session | ESTABLISHED | UNCHANGED |
| Train Data | STORED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC/ L1 / L2 / L3 |
| Train Running Number | STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 8

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 6.6.3.4 Messages transmitted to RBC/RIU |  | 6060304 |
| Test Case of Feature | The transmission of Data Train (message 129 containing packet 11) by the on-board equipment to the RBC certified with X=1 instead of sending the Train running number (message 136 containing Packet 5) |  | 8 |
| Applicable Mode/Level Combinations | L2 / L3: FS/ LS/ OS/ SR/ SB/ NL |  |  |
| Target of Test | The target of this test is to check that: The on-board equipment sends the Train Running number to the X=1 RBC using message 129 with packet 11 instead of message 136 with packet 5. Clause 3.18.4.5.4 shall be replaced with: Only if valid Train Data is available: following any entry/modification of the train running number when a communication session is already established, the ERTMS/ETCS on-board equipment shall send the Train Data to the RBC. The RBC (X=1) understand correctly and acknowledges the reception of this set of Train Data with message 8: “Acknowledgment of Train Data”. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-6.6.3.4.2 |  |
|  |  | Subset-026-6.6.3.3.2 Table - row 4 |  |
|  |  | Subset-026-7.4.3.1 Packet Number 0: Position Report |  |
|  |  | Subset-026-7.4.3.5 Packet Number 11: Validated train data |  |
|  |  | Subset-026-8.6.1 Message 129: Validated Train Data |  |
|  |  | Subset-026-8.7.4 Message 8: Acknowledgement of Train Data |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : The transmitted radio message 129 with packet 11 is RECORDED. The acknowledgment of the RBC, message 8 is RECORDED. |
| Constraints | Radio communication session is established. Entry/modification of the Train Running Number by driver Only for SB mode: Train at standstill and A Cab is active . |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 11/ 12 | FS/ OS/ SR/ SB/ NL/ LS |
| Radio communication session | ESTABLISHED | - |
| Train Data | STORED | VALID |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Train Running Number | STORED | NID_OPERATIONAL1 = VALID |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | M_VERSION | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The driver enters/modifies the "Train Running Number" [MAIN]. (NID_OPERATIONAL2) | I | DMI | - | NID_OPERATIONAL1 ≠ NID_OPERATIONAL2 Only for SB mode: Train at standstill with a cab active. | N/A | N/A | - |
| 2 | N/A | N/A | TRAIN RUNNING NUMBER ENTERED BY THE DRIVER (NID_MESSAGE_JRU=24) is recorded | O | JRU | - | NID_OPERATIONAL2 | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA. Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) containing packet 11 is transmitted to the X=1 RBC. | O | RTM | 001 0000 | The RBC (X=1) is informed about the new train running number with packet 11 where NID_OPERATIONAL2 variable is included. Translation of packet 11 tested in FT6060304.6, 7. | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | Check that message 136 with packet 5 is not recorded. | N/A | N/A | - |
| 5 | N/A | N/A | SA-DATA. Indication with Euroradio Message "Acknowledgement of Train Data" (NID_MESSAGE=8) is received | I | RTM | 001 0000 | The RBC receives the information correctly and acknowledges this set of Train Data. | N/A | N/A | - |
| 6 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded. | O | JRU | - | - | N/A | N/A | - |

| Step 3: Radio Message 129 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 129 | Validated train data |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | t0 |
| NID_ENGINE | 24 | FINITE VALUE | Engine ID of the train |
| NID_PACKET | 8 | 0 | Position report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | BGa / SBGa |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = 1 or 2 |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | One of the applicable mode |
| M_LEVEL | 3 | FINITE VALUE | One of the applicable level |
| NID_PACKET | 8 | 11 | Validated train data |
| L_PACKET | 13 | FINITE VALUE | - |
| NID_OPERATIONAL | 32 | FINITE VALUE | = NID_OPERATIONAL2 |
| NC_TRAIN | 15 | FINITE VALUE | International Train category |
| L_TRAIN | 12 | FINITE VALUE | - |
| V_MAXTRAIN | 7 | FINITE VALUE | - |
| M_LOADINGGAUGE | 8 | 0 | - |
| M_AXLELOAD | 7 | FINITE VALUE | - |
| M_AIRTIGHT | 2 | FINITE VALUE | Airtight system presence |
| N_ITER | 5 | 0 | - |
| M_TRACTION (k) | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | Type of National System available |

| Step 5: Radio Message 8 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 8 | Acknowledgement of Train Data |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | t0+ t’ with t’ > 0 |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | t0 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 11/ 12 | FS/ OS/ SR/ SB/ NL/ LS |
| Radio communication session | ESTABLISHED | - |
| Train Data | STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Train Running Number | STORED | CHANGED ( VALID |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 9

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 6.6.3.4 Messages transmitted to RBC/RIU |  | 6060304 |
| Test Case of Feature | The transmission of Train Data (message 129 containing packet 11) by the on-board equipment to the RBC certified with X=1 instead of sending the Train running number (message 136 containing Packet 5) |  | 9 |
| Applicable Mode/Level Combinations | L2 / L3: FS/ LS/ OS/ SR/ SB/ NL |  |  |
| Target of Test | The target of this test is to check that: The on-board equipment sends the Train Running number to the X=1 RBC using message 129 with packet 11 instead of message 136 with packet 5. Clause 3.18.4.5.4 shall be replaced with: Only if valid Train Data is available: following the successful establishment of a communication session when valid train running number is already available, the ERTMS/ETCS on-board equipment shall send the Train Data to the RBC. The RBC (X=1) understand correctly and acknowledges the reception of this set of Train Data with message 8: “Acknowledgment of Train Data”. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-6.6.3.4.2 |  |
|  |  | Subset-026-6.6.3.3.2 Table - row 4 |  |
|  |  | Subset-026-6.6.3.3.2 Table - row 16 |  |
|  |  | Subset-026-6.6.3.4.5 Table - row 20 |  |
|  |  | Subset-026-6.6.3.4.5 Table - row 24 |  |
|  |  | Subset-026-7.4.3.1 Packet Number 0: Position Report |  |
|  |  | Subset-026-7.4.3.5 Packet Number 11: Validated train data |  |
|  |  | Subset-026-8.6.1 Message 129: Validated Train Data |  |
|  |  | Subset-026-8.7.4 Message 8: Acknowledgement of Train Data |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : The establishment of a radio communication session initiated by the on-board equipment is recorded. The transmitted radio message 129 with packet 11 is RECORDED. The acknowledgment of the RBC, message 8 is RECORDED. |
| Constraints | Radio communication session not yet established. A valid Train running number is available. Only for SB mode: Train at standstill and A Cab is active . |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 11/ 12 | FS/ OS/ SR/ SB/ NL/ LS |
| Radio communication session | NOT ESTABLISHED | - |
| Train Data | STORED | VALID |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Radio Network ID | STORED | - |
| RBC ID/Phone Number | STORED | - |
| Train Running Number | STORED | NID_OPERATIONAL = NID_OPERATIONAL1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | M_VERSION | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-CONNECT. Request is transmitted | O | RTM | - | - | N/A | N/A | - |
| 2 | N/A | N/A | SA-CONNECT. Confirm is received | I | RTM | - | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA. Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted | O | RTM | - | Type of action according to table 6.6.3.4.5 :[U] | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | - | N/A | N/A | - |
| 5 | N/A | N/A | SA-DATA. Indication with Euroradio Message "Configuration Determination" (NID_MESSAGE=32) is received | I | RTM | 001 0000 | Message 32: “RBC/RIU System Version” Type of action according to table 6.6.3.3.2:[U] | N/A | N/A | - |
| 6 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | Versions are compatible Communication session established for on-board. | N/A | N/A | - |
| 7 | N/A | N/A | SA-DATA. Request with Euroradio Message "Session Established" (NID_MESSAGE=159) is transmitted. | O | RTM | 001 0000 | Type of action according to table 6.6.3.4.5:[U] | N/A | N/A | - |
| 8 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | Communication session established for trackside. | N/A | N/A | - |
| 9 | N/A | N/A | SA-DATA. Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) containing packet 11 is transmitted to the X=1 RBC. | O | RTM | 001 0000 | The RBC (X=1) is informed about the new train running number with packet 11 where NID_OPERATIONAL = NID_OPERATIONAL1 variable is included. Translation of packet 11 tested in FT6060304.6 or 7. | N/A | N/A | - |
| 10 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | Check that message 136 with packet 5 is not recorded. | N/A | N/A | - |
| 11 | N/A | N/A | SA-DATA. Indication with Euroradio Message "Acknowledgement of Train Data" (NID_MESSAGE=8) is received | I | RTM | 001 0000 | The RBC receives the information correctly and acknowledges this set of Train Data. | N/A | N/A | - |
| 12 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded. | O | JRU | - | - | N/A | N/A | - |

| Step 3: Radio Message 155 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 155 | Initiation of a communication session |
| L_MESSAGE | 10 | 74 | - |
| T_TRAIN | 32 | FINITE VALUE | Time of the train |
| NID_ENGINE | 24 | FINITE VALUE | ETCS-ID of the Train |

| Step 5: Radio Message 32 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 32 | Configuration Determination |
| L_MESSAGE | 10 | 82 | - |
| T_TRAIN | 32 | FINITE VALUE | Time of the train approximated by RBC |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| M_VERSION | 7 | 001 0000 | Version of the ERTMS/ETCS language. |

| Step 7: Radio Message 159 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 159 | Session established |
| L_MESSAGE | 10 | 74 | In case of including optional packets, length would be greater |
| T_TRAIN | 32 | FINITE VALUE | Time of the train |
| NID_ENGINE | 24 | FINITE VALUE | ETCS-ID of the Train |

| Step 9: Radio Message 129 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 129 | Validated train data |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | t0 |
| NID_ENGINE | 24 | FINITE VALUE | Engine ID of the train |
| NID_PACKET | 8 | 0 | Position report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | BGa / SBGa |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = 1 or 2 |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | One of the applicable mode |
| M_LEVEL | 3 | FINITE VALUE | One of the applicable level |
| NID_PACKET | 8 | 11 | Validated train data |
| L_PACKET | 13 | FINITE VALUE | - |
| NID_OPERATIONAL | 32 | FINITE VALUE | NID_OPERATIONAL = NID_OPERATIONAL1 |
| NC_TRAIN | 15 | FINITE VALUE | International Train category |
| L_TRAIN | 12 | FINITE VALUE | - |
| V_MAXTRAIN | 7 | FINITE VALUE | - |
| M_LOADINGGAUGE | 8 | 0 | - |
| M_AXLELOAD | 7 | FINITE VALUE | - |
| M_AIRTIGHT | 2 | FINITE VALUE | Airtight system presence |
| N_ITER | 5 | 0 | - |
| M_TRACTION (k) | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | Type of National System available |

| Step 11: Radio Message 8 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 8 | Acknowledgement of Train Data |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | t0+ t’ with t’ > 0 |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | t0 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 11/ 12 | FS/ OS/ SR/ SB/ NL/ LS |
| Radio communication session | ESTABLISHED | CHANGED |
| Train Data | STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Radio Network ID | STORED | - |
| RBC ID/Phone Number | STORED | - |
| Train Running Number | STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 10

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 6.6.3.4 Messages transmitted to RBC/RIU |  | 6060304 |
| Test Case of Feature | Exception: If the train running number is received from the RBC (X=1), the Train Data is not sent back to the RBC by the on-board equipment. |  | 10 |
| Applicable Mode/Level Combinations | L2 / L3: FS/ LS/ OS |  |  |
| Target of Test | The target of this test is to check that : - The on-board equipment receives from the RBC a new Train Running Number. - Message 24 with packet 140 is sent by the X=1 RBC and NID_OPERATIONAL is translated according to T [10]. - The On-board equipment does not send back to the RBC the set of new Train Data. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-6.6.3.4.3 |  |
|  |  | Subset-026-6.6.3.2.3 Table - row 41 |  |
|  |  | Subset-026-6.6.3.2.3 [10] |  |
|  |  | Subset-026-7.4.2.36 Packet Number 140: Train running number from RBC |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : - The radio message 24 with packet 140 is RECORDED. - The recorded NID_OPERATIONAL has been translated (T[10]: FFFF1234 to 1234FFFF) - Message 129 with packet 11 is NOT RECORDED. |
| Constraints | Radio communication session is established |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS/ OS/ LS |
| Radio communication session | ESTABLISHED | - |
| Train Data | STORED | Train data is available |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Train Running Number | STORED | NID_OPERATIONAL1 = VALID |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | M_VERSION | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA. Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 140 (NID_OPERATIONAL2=FFFF1234) is received. | I | RTM | 001 0000 | NID_OPERATIONAL1 ≠ NID_OPERATIONAL2 | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | NID_OPERATIONAL2 is recorded following the translation T[10]: 1234FFFF | N/A | N/A | - |
| 3 | N/A | N/A | NOT SA-DATA.Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) containing packet 11 is transmitted. | O | RTM | - | The on-board does not send back to the RBC the Train Data. | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | t0 |
| M_ACK | 1 | 0 | No ACK required |
| NID_LRBG | 24 | FINITE VALUE | BGa / SBGa |
| NID_PACKET | 8 | 140 | Train Running Number from RBC |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| NID_OPERATIONAL | 32 | FFFF1234 | NID_OPERATIONAL2 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS/ OS/ LS |
| Radio communication session | ESTABLISHED | - |
| Train Data | STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Train Running Number | STORED | CHANGED (NID_OPERATIONAL2) |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 11

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 6.6.3.4 Messages transmitted to RBC/RIU |  | 6060304 |
| Test Case of Feature | Transmission of the message 132: “MA Request” by the on-board equipment to an RBC certified to the system version number X = 1. Tested action: Data is translated ” T[4]” & data is unchanged “U[1]” when EVC in LS mode. |  | 11 |
| Applicable Mode/Level Combinations | L2 / L3: LS |  |  |
| Target of Test | The target of this test is to check that when in LS mode: A situation which leads to shorten the current MA and track description to the current position of the on-board equipment or the new stop location. The on-board equipment informs the RBC/RIU by sending a radio message 132 “MA request” that is translated for the RBC system version X=1 as: the variable Q_MARQSTREASON is being replaced with Q_TRACKDEL. The transmitted value to X=1 RBC: Q_MARSQREASON = x1xxx into Q_TRACKDEL set to 1. The position report packet is sent with M_MODE=12. The RBC/RIU understands correctly the received message and sends a valid Movement Authority. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-6.6.3.4.4 |  |
|  |  | Subset-026-6.6.3.4.5 Table - row 1 |  |
|  |  | Subset-026-6.6.3.4.5 Table - row 10 |  |
|  |  | Subset-026-6.6.3.4.5 [1] |  |
|  |  | Subset-026-6.6.3.4.5 [4] |  |
|  |  | Subset-026-7.4.2.36 Packet Number 140: Train running number from RBC |  |
|  |  | Subset-026-7.4.2.4 Packet Number 15: Level 2/3 Movement Authority |  |
|  |  | Subset-026-7.4.2.6 Packet Number 21: Gradient Profile |  |
|  |  | Subset-026-7.4.2.7 Packet Number 27: International Static Speed Profile |  |
|  |  | Subset-026-7.4.2.26 Packet Number 80: Mode profile |  |
|  |  | Subset-026-8.6.3 Message 132: MA Request |  |
|  |  | Subset-026-8.7.2 Message 3: Movement Authority |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the RTM that: Data from message 132 and packet 0 are translated correctly by the on-board and the trackside send the requested information. Check on the JRU that: The Message 132 :MA Request with packet 0 is RECORDED. The Message 3: Movement Authority is RECORDED. |
| Constraints | This section applies for the parts of radio messages/packets, which are transmitted to an RBC or an RIU certified to the system version number X = 1. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 12 | LS |
| Radio communication session | ESTABLISHED | System version X=1 |
| Linking | STORED | - |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Mode Profile | STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | M_VERSION | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | No linked balise group message is received | I | BTM | - | The balise group missed was linked with service brake reaction | N/A | N/A | - |
| 2 | N/A | N/A | Service brake commanded. | O | TIU | - | - | N/A | N/A | - |
| 3 | N/A | N/A | The Status symbol "Service Brake " is displayed. | O | DMI | - | Symbol ST01 | N/A | N/A | - |
| 4 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded. | O | JRU | - | - | N/A | N/A | - |
| 5 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; bit38=1) is recorded. | O | JRU | - | - | N/A | N/A | - |
| 6 | N/A | N/A | Train is at standstill (V_TRAIN=0; TIME= 60) | I | INT | - | - | N/A | N/A | - |
| 7 | N/A | N/A | SA-DATA. Request with Euroradio Message "MA Request" (NID_MESSAGE=132) containing packet 0 is transmitted to a system version X=1 RBC. | O | RTM | 001 0000 | Q_TRACKDEL = 1 For Message132, data is translated as: Q_MARQSTREASON is replaced with Q_TRACKDEL And value x1xxx is set to 1 (track description deleted) When the applicable mode is LS, M_MAMODE=12 then, no translation is effected. | N/A | N/A | - |
| 8 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded. | O | JRU | - | - | N/A | N/A | - |
| 9 | N/A | N/A | SA-DATA. Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 15, packet 21, packet 27 and packet 80 is received | I | RTM | 001 0000 | Translation of Packets 21, 27 and 80 out of scope of test case target. | N/A | N/A | - |
| 10 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded. | O | JRU | - | - | N/A | N/A | - |

| Step 7: Radio Message 132 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 132 | MA request |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 1 | FINITE VALUE | - |
| Q_TRACKDEL | 1 | 1 | Track description deleted |
| NID_PACKET | 8 | 0 | Position report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = “Train integrity confirmed by integrity monitoring device” or “Train integrity confirmed by driver” |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | 12 | If the applicable mode is LS, M_MAMODE=12 then, no translation is effected |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC |

| Step 9: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | Movement Authority |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | Level 2/3 Movement Authority |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to “no time-out” |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | Gradient Profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | International Static Speed Profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | - |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| NC_DIFF(n) | 4 | FINITE VALUE | - |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | - |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| NC_DIFF(k,m) | 4 | FINITE VALUE | - |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 80 | Mode Profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_MAMODE | 15 | FINITE VALUE | - |
| M_MAMODE | 2 | 12 | If the applicable mode is LS, M_MAMODE=12 then, no translation is effected |
| V_MAMODE | 7 | FINITE VALUE | - |
| L_MAMODE | 15 | FINITE VALUE | - |
| L_ACKMAMODE | 15 | FINITE VALUE | - |
| Q_MAMODE | 1 | 1 | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_MAMODE(k) | 15 | FINITE VALUE | - |
| M_MAMODE(k) | 2 | FINITE VALUE | OS, LS, SH |
| V_MAMODE(k) | 7 | FINITE VALUE | - |
| L_MAMODE(k) | 15 | FINITE VALUE | - |
| Q_MAMODE(k) | 1 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 12 | LS |
| Radio communication session | ESTABLISHED | UNCHANGED |
| Movement Authority | STORED | UPDATED |
| Gradient Profile | STORED | UPDATED |
| International SSP | STORED | UPDATED |
| Mode Profile | STORED | UPDATED |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |