| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 3131050 (Requirements for Release speed monitoring (Tables 13, 14 and 15)) Total: 9 Test case(s) |  |
| REF: | Subset-076-5-2-3131050 |
| ISSUE: | Version 3.2.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 2.9.0 27.11.2013 | All | First version/review | CEDEX |
| Version 2.9.1 27.11.2013 | All | General review | CEDEX |
| Version 2.9.2 18.12.2013 | All | Review | CEDEX |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.2.0 31.03.2017 | All | Baseline 3 1st maintenance release version - Bug fixing | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | Requirements for Release speed monitoring (Tables 13, 14 and 15) |  | 3131050 |
| Test Case of Feature | Transitions between types of speed and distance monitoring (Table 16 Condition id [2] of TSM to RSM transition when RSM start location is calculated from EBD curve) AND Triggering condition t1of Table 13. Transition from Normal to Indication status of table 15, valid for Vest <= VRELEASE (not calculated on board and which does not exceed any MRSP in the area). When the current train speed is below or at the Release Speed (Vest <= VRELEASE) the Indication Status is active at the DMI.NV are stored with Q_NVINHSMICPERM=0 (No Permission to inhibit the compensation of the speed measurement inaccuracy) |  | 1 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS L2/L3: FS, LS, OS |  |  |
| Target of Test | The train enters from the area of target speed monitoring to the release speed monitoring one when max safe front end reaches the RSM start location calculated from EBD curve (condition id [2] of Subset-026-3.13.10.6.1) . The release speed,target distance and braking to target permitted speed supervision limit are displayed on the DMI. As Vest <= VRELEASE, the transition between the normal to Indication is shown on the DMI. Q_NVINHSMICPERM=0 (No permission to inhibit the compensation of the speed measurement inaccuracy) to test negative condition of Subset-026-3.13.9.4.6 b) Use of d_startRSM=dSB2(Vrelease) if dSBI2(Vrelease)-dSBI1(Vrelease) < dmax safefront -destfront equation of Subset-026-3.13.9.4.7 is tested. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.13.10.5.1 |  |
|  |  | Subset-026-3.13.10.5.2 |  |
|  |  | Subset-026-3.13.10.5.3 |  |
|  |  | Subset-026-3.13.10.5.4 table 13 |  |
|  |  | Subset-026-3.13.10.5.5 table 15 |  |
|  |  | Subset-026-3.13.9.4.6 a) |  |
|  |  | Subset-026-3.13.9.4.6 b) |  |
|  |  | Subset-026-3.13.9.4.7 |  |
|  |  | Subset-026-4.7.2 table - Output information Release speed |  |
|  |  | Subset-026-4.7.2 table - Output information Speed and distance monitoring supervision status |  |
|  |  | Subset-026-3.13.10.6.1 |  |
|  |  | Subset-026-3.13.10.5.5 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | The test case shall consider the train data and the National Values (either a valid Packet 3 or the Default Values are used) as preconditions to be known before setting the appropriate values for the rest of the trackside info and the speed profile. The required observables (speed supervision limits) should be checked using a suitable tool that performs fully the “speed and distance monitoring" functionality (by default use ERA Braking curves tool, which can be downloaded from this website: http://www.era.europa.eu/Core-Activities/ERTMS/Pages/Braking-Curves-Simulation-Tool.aspx). |
| Constraints | Distance between EoA and SvL together with location accuracy of the LRBG should have the appropiate values so that RSM start location is calculated from EBD curve. The release speed is not calculated on board and does not exceed any MRSP in the area. NV are stored with Q_NVINHSMICPERM=0 (No Permission to inhibit the compensation of the speed measurement inaccuracy) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | Only in L2/L3 |
| National Values | STORED | Q_NVINHSMICPERM=0 |
| Movement Authority | STORED | Release speed given by trackside directly or in terms of National Values |
| Gradient Profile | STORED | - |
| International SSP | STORED | No SSP element lower than release speed in the area of Release speed monitoring |
| Axle load speed profile | NOT STORED | - |
| STM max speed | NOT STORED | - |
| STM system speed/distance | NOT STORED | - |
| Temporary Speed Restrictions | NOT STORED | - |
| Signalling related Speed Restriction | NOT STORED | Only in L1 |
| Mode Profile | STORED | Only in OS or LS mode |
| Unconditional Emergency Stop | NOT STORED | - |
| Conditional Emergency Stop | NOT STORED | - |
| Train Position | STORED | - |
| Train Data | STORED | - |
| Adhesion factor | NOT STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Driver ID | STORED | - |
| Radio Network ID | STORED | Only in L2/L3 |
| RBC ID/Phone Number | STORED | Only in L2/L3 |
| Train Running Number | STORED | - |
| Track Ahead Free Request | NOT STORED | - |
| Level Crossing information | NOT STORED | - |
| Permitted Braking Distance Information | NOT STORED | - |
| Operated System Version | 010 0000 | Version 2.0, introduced in SRS 3.3.0 |
| Virtual Balise Covers | NOT STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is moving (V_TRAIN ≤ VRELEASE) | I | INT | The train is under target speed monitoring near the start of the release speed monitoring location. Train speed/position does not exceed the Indication supervision limit (I). | N/A | N/A | - |
| 2 | N/A | N/A | The driver requests the speed and distance monitoring information to be displayed | I | DMI | Only in OS mode. (The permitted speed shall be displayed until the driver requests to stop this its. See for example Req 4.4.12.1.4) | N/A | N/A | - |
| 3 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The Target speed montoring type is displayed. (As there is no change in the DMI, there is no JRU step with Target speed montoring type recorded) | N/A | N/A | - |
| 4 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The DMI aspect is coherent with Normal Status . Permitted speed is displayed (only if not in LS mode). (As there is no change in the DMI, there is not JRU step with Normal Status recorded) | N/A | N/A | - |
| 5 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The train has passed with its max safe front end the RSM start location The Release speed montoring type is displayed The release speed,target distance (only if not in LS mode) and braking to target permitted speed supervision limit (only if not in LS mode) are displayed | N/A | N/A | - |
| 6 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The DMI aspect is coherent with Indication Status . The Permitted Speed is displayed (only if not in LS mode). | N/A | N/A | - |
| 7 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=3; M_SDMSUPSTAT=1 V_PERM=V1; D_TARGET=D1; V_RELEASE=V2) is recorded | O | JRU | M_SDMTYPE (Speed and distance monitoring type) = 3 (Release speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 1 (Indication Status) V_PERM and D_TARGET will only be recorded in FS and OS mode. | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | Only in L2/L3 |
| National Values | STORED | Q_NVINHSMICPERM=0 |
| Movement Authority | STORED | Release speed given by trackside directly or in terms of National Values |
| Gradient Profile | STORED | - |
| International SSP | STORED | No SSP element lower than release speed in the area of Release speed monitoring |
| Axle load speed profile | NOT STORED | - |
| STM max speed | NOT STORED | - |
| STM system speed/distance | NOT STORED | - |
| Temporary Speed Restrictions | NOT STORED | - |
| Signalling related Speed Restriction | NOT STORED | Only in L1 |
| Mode Profile | STORED | Only in OS or LS mode |
| Unconditional Emergency Stop | NOT STORED | - |
| Conditional Emergency Stop | NOT STORED | - |
| Train Position | STORED | - |
| Train Data | STORED | - |
| Adhesion factor | NOT STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Driver ID | STORED | - |
| Radio Network ID | STORED | Only in L2/L3 |
| RBC ID/Phone Number | STORED | Only in L2/L3 |
| Train Running Number | STORED | - |
| Track Ahead Free Request | NOT STORED | - |
| Level Crossing information | NOT STORED | - |
| Permitted Braking Distance Information | NOT STORED | - |
| Operated System Version | 010 0000 | Version 2.0, introduced in SRS 3.3.0 |
| Virtual Balise Covers | NOT STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | Requirements for Release speed monitoring (Tables 13, 14 and 15) |  | 3131050 |
| Test Case of Feature | Transitions between types of speed and distance monitoring (Table 16 Condition id [2] of TSM to RSM transition when RSM start location is calculated from EBD curve) AND Triggering condition t1of Table 13. Transition from Normal to Indication status of table 15, valid for Vest <= VRELEASE (not calculated on board and which does not exceed any MRSP in the area). When the current train speed is below or at the Release Speed (Vest <= VRELEASE) the Indication Status is active at the DMI.NV are stored with Q_NVINHSMICPERM=1 (Permission to inhibit the compensation of the speed measurement inaccuracy) |  | 2 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS L2/L3: FS, LS, OS |  |  |
| Target of Test | The train enters from the area of target speed monitoring to the release speed monitoring one when max safe front end reaches the RSM start location calculated from EBD curve (condition id [2] of Subset-026-3.13.10.6.1) . The release speed,target distance and braking to target permitted speed supervision limit are displayed on the DMI. As Vest <= VRELEASE, the transition between the normal to Indication is shown on the DMI. Q_NVINHSMICPERM=1 (Permission to inhibit the compensation of the speed measurement inaccuracy) to test positive condition of Subset-026-3.13.9.4.6 b) Use of d_startRSM=dSB2(Vrelease) if dSBI2(Vrelease)-dSBI1(Vrelease) < dmax safefront -destfront equation of Subset-026-3.13.9.4.7 is tested. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.13.10.5.1 |  |
|  |  | Subset-026-3.13.10.5.2 |  |
|  |  | Subset-026-3.13.10.5.3 |  |
|  |  | Subset-026-3.13.10.5.4 table 13 |  |
|  |  | Subset-026-3.13.10.5.5 table 15 |  |
|  |  | Subset-026-3.13.9.4.6 a) |  |
|  |  | Subset-026-3.13.9.4.6 b) |  |
|  |  | Subset-026-3.13.9.4.7 |  |
|  |  | Subset-026-4.7.2 table - Output information Release speed |  |
|  |  | Subset-026-4.7.2 table - Output information Speed and distance monitoring supervision status |  |
|  |  | Subset-026-3.13.10.6.1 |  |
|  |  | Subset-026-3.13.10.5.5 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | The test case shall consider the train data and the National Values (either a valid Packet 3 or the Default Values are used) as preconditions to be known before setting the appropriate values for the rest of the trackside info and the speed profile. The required observables (speed supervision limits) should be checked using a suitable tool that performs fully the “speed and distance monitoring" functionality (by default use ERA Braking curves tool, which can be downloaded from this website: http://www.era.europa.eu/Core-Activities/ERTMS/Pages/Braking-Curves-Simulation-Tool.aspx). |
| Constraints | Distance between EoA and SvL together with location accuracy of the LRBG should have the appropiate values so that RSM start location is calculated from EBD curve. The release speed is not calculated on board and does not exceed any MRSP in the area. NV are stored with Q_NVINHSMICPERM=1 (Permission to inhibit the compensation of the speed measurement inaccuracy) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | Only in L2/L3 |
| National Values | STORED | Q_NVINHSMICPERM=1 |
| Movement Authority | STORED | Release speed given by trackside directly or in terms of National Values |
| Gradient Profile | STORED | - |
| International SSP | STORED | No SSP element lower than release speed in the area of Release speed monitoring |
| Axle load speed profile | NOT STORED | - |
| STM max speed | NOT STORED | - |
| STM system speed/distance | NOT STORED | - |
| Temporary Speed Restrictions | NOT STORED | - |
| Signalling related Speed Restriction | NOT STORED | Only in L1 |
| Mode Profile | STORED | Only in OS or LS mode |
| Unconditional Emergency Stop | NOT STORED | - |
| Conditional Emergency Stop | NOT STORED | - |
| Train Position | STORED | - |
| Train Data | STORED | - |
| Adhesion factor | NOT STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Driver ID | STORED | - |
| Radio Network ID | STORED | Only in L2/L3 |
| RBC ID/Phone Number | STORED | Only in L2/L3 |
| Train Running Number | STORED | - |
| Track Ahead Free Request | NOT STORED | - |
| Level Crossing information | NOT STORED | - |
| Permitted Braking Distance Information | NOT STORED | - |
| Operated System Version | 010 0000 | Version 2.0, introduced in SRS 3.3.0 |
| Virtual Balise Covers | NOT STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is moving (V_TRAIN ≤ VRELEASE) | I | INT | The train is under target speed monitoring near the start of the release speed monitoring location. Train speed/position does not exceed the Indication supervision limit (I). | N/A | N/A | - |
| 2 | N/A | N/A | The driver requests the speed and distance monitoring information to be displayed | I | DMI | Only in OS mode. (The permitted speed shall be displayed until the driver requests to stop this its. See for example Req 4.4.12.1.4) | N/A | N/A | - |
| 3 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The Target speed montoring type is displayed. (As there is no change in the DMI, there is no JRU step with Target speed montoring type recorded) | N/A | N/A | - |
| 4 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The DMI aspect is coherent with Normal Status . Permitted speed is displayed (only if not in LS mode). (As there is no change in the DMI, there is not JRU step with Normal Status recorded) | N/A | N/A | - |
| 5 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The train has passed with its max safe front end the RSM start location The Release speed montoring type is displayed The release speed,target distance (only if not in LS mode) and braking to target permitted speed supervision limit (only if not in LS mode) are displayed | N/A | N/A | - |
| 6 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The DMI aspect is coherent with Indication Status . Permitted speed is displayed (only if not in LS mode). | N/A | N/A | - |
| 7 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=3; M_SDMSUPSTAT=1 V_PERM=V1; D_TARGET=D1; V_RELEASE=V2) is recorded | O | JRU | M_SDMTYPE (Speed and distance monitoring type) = 3 (Release speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 1 (Indication Status) V_PERM and D_TARGET will only be recorded in FS and OS mode. | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | Only in L2/L3 |
| National Values | STORED | Q_NVINHSMICPERM=1 |
| Movement Authority | STORED | Release speed given by trackside directly or in terms of National Values |
| Gradient Profile | STORED | - |
| International SSP | STORED | No SSP element lower than release speed in the area of Release speed monitoring |
| Axle load speed profile | NOT STORED | - |
| STM max speed | NOT STORED | - |
| STM system speed/distance | NOT STORED | - |
| Temporary Speed Restrictions | NOT STORED | - |
| Signalling related Speed Restriction | NOT STORED | Only in L1 |
| Mode Profile | STORED | Only in OS or LS mode |
| Unconditional Emergency Stop | NOT STORED | - |
| Conditional Emergency Stop | NOT STORED | - |
| Train Position | STORED | - |
| Train Data | STORED | - |
| Adhesion factor | NOT STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Driver ID | STORED | - |
| Radio Network ID | STORED | Only in L2/L3 |
| RBC ID/Phone Number | STORED | Only in L2/L3 |
| Train Running Number | STORED | - |
| Track Ahead Free Request | NOT STORED | - |
| Level Crossing information | NOT STORED | - |
| Permitted Braking Distance Information | NOT STORED | - |
| Operated System Version | 010 0000 | Version 2.0, introduced in SRS 3.3.0 |
| Virtual Balise Covers | NOT STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | Requirements for Release speed monitoring (Tables 13, 14 and 15) |  | 3131050 |
| Test Case of Feature | Transitions between types of speed and distance monitoring (Table 16 Condition id [2] of TSM to RSM) AND Triggering condition t1 of Table 13. Transition from Normal to Indication status of table 15, valid for Vest <= VRELEASE (Calculated on board based on a speed restriction between EoA and SvL). When the current train speed is below or at the Release Speed (Vest <= VRELEASE) the Indication Status is active at the DMI. |  | 3 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS L2/L3: FS, LS, OS |  |  |
| Target of Test | The train enters from the area of target speed monitoring to the release speed monitoring one when max safe front end reaches the RSM start location calculated from EBD curve (condition id [2] of Subset-026-3.13.10.6.1) . The release speed,target distance and braking to target permitted speed supervision limit are displayed on the DMI. As Vest <= VRELEASE, the transition between the normal to Indication is shown on the DMI. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.13.10.5.1 |  |
|  |  | Subset-026-3.13.10.5.2 |  |
|  |  | Subset-026-3.13.10.5.3 |  |
|  |  | Subset-026-3.13.10.5.4 table 13 |  |
|  |  | Subset-026-3.13.10.5.5 table 15 |  |
|  |  | Subset-026-3.13.9.4.6 a) |  |
|  |  | Subset-026-3.13.9.4.7 |  |
|  |  | Subset-026-3.13.9.4.8 |  |
|  |  | Subset-026-3.13.9.4.8.1 |  |
|  |  | Subset-026-3.13.9.4.8.2 |  |
|  |  | Subset-026-4.7.2 table - Output information Release speed |  |
|  |  | Subset-026-4.7.2 table - Output information Speed and distance monitoring supervision status |  |
|  |  | Subset-026-3.13.10.6.1 |  |
|  |  | Subset-026-3.13.10.5.5 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | The test case shall consider the train data and the National Values (either a valid Packet 3 or the Default Values are used) as preconditions to be known before setting the appropriate values for the rest of the trackside info and the speed profile. The required observables (speed supervision limits) should be checked using a suitable tool that performs fully the “speed and distance monitoring" functionality (by default use ERA Braking curves tool, which can be downloaded from this website: http://www.era.europa.eu/Core-Activities/ERTMS/Pages/Braking-Curves-Simulation-Tool.aspx). |
| Constraints | The distance between EoA and SvL should be high enough, in order to put a target (based on a MRSP) between the Trip location related to the EOA and the SvL. The release speed is calculated on board basing on a speed restriction between EoA and SvL. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | Only in L2/L3 |
| National Values | STORED | - |
| Movement Authority | STORED | Release speed calculated on board |
| Gradient Profile | STORED | - |
| International SSP | STORED | SSP element between EoA and SvL. |
| Axle load speed profile | NOT STORED | - |
| STM max speed | NOT STORED | - |
| STM system speed/distance | NOT STORED | - |
| Temporary Speed Restrictions | NOT STORED | - |
| Signalling related Speed Restriction | NOT STORED | Only in L1 |
| Mode Profile | STORED | Only in OS or LS mode |
| Unconditional Emergency Stop | NOT STORED | - |
| Conditional Emergency Stop | NOT STORED | - |
| Train Position | STORED | - |
| Train Data | STORED | - |
| Adhesion factor | NOT STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Driver ID | STORED | - |
| Radio Network ID | STORED | Only in L2/L3 |
| RBC ID/Phone Number | STORED | Only in L2/L3 |
| Train Running Number | STORED | - |
| Track Ahead Free Request | NOT STORED | - |
| Level Crossing information | NOT STORED | - |
| Permitted Braking Distance Information | NOT STORED | - |
| Operated System Version | 010 0000 | Version 2.0, introduced in SRS 3.3.0 |
| Virtual Balise Covers | NOT STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is moving (V_TRAIN ≤ VRELEASE) | I | INT | The train is under target speed monitoring near the start of the release speed monitoring location. Train speed/position does not exceed the Indication supervision limit (I) | N/A | N/A | - |
| 2 | N/A | N/A | The driver requests the speed and distance monitoring information to be displayed | I | DMI | Only in OS mode. (The permitted speed shall be displayed until the driver requests to stop this its. See for example Req 4.4.12.1.4) | N/A | N/A | - |
| 3 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The Target speed montoring type is displayed. (As there is no change in the DMI, there is no JRU step with Target speed montoring type recorded) | N/A | N/A | - |
| 4 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The DMI aspect is coherent with Normal Status . Permitted speed is displayed (only if not in LS mode). (As there is no change in the DMI, there is not JRU step with Normal Status recorded) | N/A | N/A | - |
| 5 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The train has passed with its max safe front end the RSM start location The Release speed montoring type is displayed The release speed,target distance (only if not in LS mode) and braking to target permitted speed supervision limit (only if not in LS mode) are displayed | N/A | N/A | - |
| 6 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The DMI aspect is coherent with Indication Status . Permitted speed is displayed (only if not in LS mode). | N/A | N/A | - |
| 7 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=3; M_SDMSUPSTAT=1 is recorded | O | JRU | M_SDMTYPE (Speed and distance monitoring type) = 3 (Release speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 1 (Indication Status) | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | Only in L2/L3 |
| National Values | STORED | - |
| Movement Authority | STORED | On-board calculatec Release Speed |
| Gradient Profile | STORED | - |
| International SSP | STORED | No SSP element lower than release speed in the area of Release speed monitoring |
| Axle load speed profile | NOT STORED | - |
| STM max speed | NOT STORED | - |
| STM system speed/distance | NOT STORED | - |
| Temporary Speed Restrictions | NOT STORED | - |
| Signalling related Speed Restriction | NOT STORED | Only in L1 |
| Mode Profile | STORED | Only in OS or LS mode |
| Unconditional Emergency Stop | NOT STORED | - |
| Conditional Emergency Stop | NOT STORED | - |
| Train Position | STORED | - |
| Train Data | STORED | - |
| Adhesion factor | NOT STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Driver ID | STORED | - |
| Radio Network ID | STORED | Only in L2/L3 |
| RBC ID/Phone Number | STORED | Only in L2/L3 |
| Train Running Number | STORED | - |
| Track Ahead Free Request | NOT STORED | - |
| Level Crossing information | NOT STORED | - |
| Permitted Braking Distance Information | NOT STORED | - |
| Operated System Version | 010 0000 | Version 2.0, introduced in SRS 3.3.0 |
| Virtual Balise Covers | NOT STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | Requirements for Release speed monitoring (Tables 13, 14 and 15) |  | 3131050 |
| Test Case of Feature | Transitions between types of speed and distance monitoring (Table 16 Condition id [2] of TSM to RSM transition when RSM start location is calculated from EBD curve) AND Triggering condition t1of Table 13. Transition from Normal to Indication status of table 15, valid for Vest <= VRELEASE (not calculated on board and which exceeds the MRSP in the RSM area). When the current train speed is below or at the Release Speed (Vest <= VRELEASE) the Indication Status is active at the DMI.NV are stored with Q_NVINHSMICPERM=0 (No Permission to inhibit the compensation of the speed measurement inaccuracy) |  | 4 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS L2/L3: FS, LS, OS |  |  |
| Target of Test | The train enters from the area of target speed monitoring to the release speed monitoring one when max safe front end reaches the RSM start location calculated from EBD curve (condition id [2] of Subset-026-3.13.10.6.1) . The release speed,target distance and braking to target permitted speed supervision limit are displayed on the DMI. As Vest <= VRELEASE, the transition between the normal to Indication is shown on the DMI. Q_NVINHSMICPERM=0 (No permission to inhibit the compensation of the speed measurement inaccuracy) to test negative condition of Subset-026-3.13.9.4.6 b) Use of d_startRSM=dSB2(Vrelease) if dSBI2(Vrelease)-dSBI1(Vrelease) < dmax safefront -destfront equation of Subset-026-3.13.9.4.7 is tested. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.13.10.5.1 |  |
|  |  | Subset-026-3.13.10.5.2 |  |
|  |  | Subset-026-3.13.10.5.3 |  |
|  |  | Subset-026-3.13.10.5.4 table 13 |  |
|  |  | Subset-026-3.13.10.5.5 table 15 |  |
|  |  | Subset-026-3.13.9.4.6 a) |  |
|  |  | Subset-026-3.13.9.4.6 b) |  |
|  |  | Subset-026-3.13.9.4.7 |  |
|  |  | Subset-026-3.13.9.4.9 |  |
|  |  | Subset-026-4.7.2 table - Output information Release speed |  |
|  |  | Subset-026-4.7.2 table - Output information Speed and distance monitoring supervision status |  |
|  |  | Subset-026-3.13.10.6.1 |  |
|  |  | Subset-026-3.13.10.5.5 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | The test case shall consider the train data and the National Values (either a valid Packet 3 or the Default Values are used) as preconditions to be known before setting the appropriate values for the rest of the trackside info and the speed profile. The required observables (speed supervision limits) should be checked using a suitable tool that performs fully the “speed and distance monitoring" functionality (by default use ERA Braking curves tool, which can be downloaded from this website: http://www.era.europa.eu/Core-Activities/ERTMS/Pages/Braking-Curves-Simulation-Tool.aspx). |
| Constraints | Distance between EoA and SvL together with location accuracy of the LRBG should have the appropiate values so that RSM start location is calculated from EBD curve. The release speed exceeds the MRSP somewhere in the area delimited on one side by the presumed start location of the Release speed monitoring and on the other side by the trip location related to the EOA. NV are stored with Q_NVINHSMICPERM=0 (No Permission to inhibit the compensation of the speed measurement inaccuracy) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | Only in L2/L3 |
| National Values | STORED | Q_NVINHSMICPERM=0 |
| Movement Authority | STORED | Release speed given by trackside directly or in terms of National Values |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Axle load speed profile | - | - |
| STM max speed | NOT STORED | - |
| STM system speed/distance | NOT STORED | - |
| Temporary Speed Restrictions | - | - |
| Signalling related Speed Restriction | - | - |
| Mode Profile | STORED | Only in OS or LS mode |
| Unconditional Emergency Stop | NOT STORED | - |
| Conditional Emergency Stop | NOT STORED | - |
| Train Position | STORED | - |
| Train Data | STORED | - |
| Adhesion factor | NOT STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Driver ID | STORED | - |
| Radio Network ID | STORED | Only in L2/L3 |
| RBC ID/Phone Number | STORED | Only in L2/L3 |
| Train Running Number | STORED | - |
| Track Ahead Free Request | NOT STORED | - |
| Level Crossing information | NOT STORED | - |
| Permitted Braking Distance Information | - | - |
| Operated System Version | 010 0000 | Version 2.0, introduced in SRS 3.3.0 |
| Virtual Balise Covers | NOT STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is moving (V_TRAIN ≤ VRELEASE) | I | INT | The train is under target speed monitoring near the start of the release speed monitoring location. Train speed/position does not exceed the Indication supervision limit (I). | N/A | N/A | - |
| 2 | N/A | N/A | The driver requests the speed and distance monitoring information to be displayed | I | DMI | Only in OS mode. (The permitted speed shall be displayed until the driver requests to stop this its. See for example Req 4.4.12.1.4) | N/A | N/A | - |
| 3 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The Target speed montoring type is displayed. (As there is no change in the DMI, there is no JRU step with Target speed montoring type recorded) | N/A | N/A | - |
| 4 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The DMI aspect is coherent with Normal Status . Permitted speed is displayed (only if not in LS mode). (As there is no change in the DMI, there is not JRU step with Normal Status recorded) | N/A | N/A | - |
| 5 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The train has passed with its max safe front end the RSM start location The Release speed montoring type is displayed The release speed,target distance (only if not in LS mode) and braking to target permitted speed supervision limit (only if not in LS mode) are displayed | N/A | N/A | - |
| 6 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The DMI aspect is coherent with Indication Status . Permitted speed is displayed (only if not in LS mode). | N/A | N/A | - |
| 7 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=3; M_SDMSUPSTAT=1 is recorded | O | JRU | M_SDMTYPE (Speed and distance monitoring type) = 3 (Release speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 1 (Indication Status) | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | Only in L2/L3 |
| National Values | STORED | Q_NVINHSMICPERM=0 |
| Movement Authority | STORED | Release speed given by trackside directly or in terms of National Values |
| Gradient Profile | STORED | - |
| International SSP | STORED | SSP element lower than release speed in the area of Release speed monitoring |
| Axle load speed profile | NOT STORED | - |
| STM max speed | NOT STORED | - |
| STM system speed/distance | NOT STORED | - |
| Temporary Speed Restrictions | NOT STORED | - |
| Signalling related Speed Restriction | NOT STORED | Only in L1 |
| Mode Profile | STORED | Only in OS or LS mode |
| Unconditional Emergency Stop | NOT STORED | - |
| Conditional Emergency Stop | NOT STORED | - |
| Train Position | STORED | - |
| Train Data | STORED | - |
| Adhesion factor | NOT STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Driver ID | STORED | - |
| Radio Network ID | STORED | Only in L2/L3 |
| RBC ID/Phone Number | STORED | Only in L2/L3 |
| Train Running Number | STORED | - |
| Track Ahead Free Request | NOT STORED | - |
| Level Crossing information | NOT STORED | - |
| Permitted Braking Distance Information | NOT STORED | - |
| Operated System Version | 010 0000 | Version 2.0, introduced in SRS 3.3.0 |
| Virtual Balise Covers | NOT STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 5

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | Requirements for Release speed monitoring (Tables 13, 14 and 15) |  | 3131050 |
| Test Case of Feature | Transitions between types of speed and distance monitoring (Table 16 Condition id [2] of TSM to RSM transition when RSM start location is calculated from SBD curve) AND Triggering condition t1of Table 13. Transition from Normal to Indication status of table 15, valid for Vest <= VRELEASE (not calculated on board and which does not exceed any MRSP in the area). When the current train speed is below or at the Release Speed (Vest <= VRELEASE) the Indication Status is active at the DMI.NV are stored with Q_NVINHSMICPERM=0 (No Permission to inhibit the compensation of the speed measurement inaccuracy) |  | 5 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS L2/L3: FS, LS, OS |  |  |
| Target of Test | The train enters from the area of target speed monitoring to the release speed monitoring one when the estimated front end reaches the RSM start location calculated from SBD curve (condition id [2] of Subset-026-3.13.10.6.1) . The release speed,target distance and braking to target permitted speed supervision limit are displayed on the DMI. As Vest <= VRELEASE, the transition between the normal to Indication is shown on the DMI. Q_NVINHSMICPERM=0 (No permission to inhibit the compensation of the speed measurement inaccuracy) to test negative condition of Subset-026-3.13.9.4.6 b) Use of d_startRSM=dSB1(Vrelease) if dSBI2(Vrelease)-dSBI1(Vrelease) >= dmax safefront -destfront equation of Subset-026-3.13.9.4.7 is tested. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.13.10.5.1 |  |
|  |  | Subset-026-3.13.10.5.2 |  |
|  |  | Subset-026-3.13.10.5.3 |  |
|  |  | Subset-026-3.13.10.5.4 table 13 |  |
|  |  | Subset-026-3.13.10.5.5 table 15 |  |
|  |  | Subset-026-3.13.9.4.6 a) |  |
|  |  | Subset-026-3.13.9.4.6 b) |  |
|  |  | Subset-026-3.13.9.4.7 |  |
|  |  | Subset-026-4.7.2 table - Output information Release speed |  |
|  |  | Subset-026-4.7.2 table - Output information Speed and distance monitoring supervision status |  |
|  |  | Subset-026-3.13.10.6.1 |  |
|  |  | Subset-026-3.13.8.4.1 |  |
|  |  | Subset-026-3.13.6.3.1.3 |  |
|  |  | Subset-026-3.13.6.3.1.4 |  |
|  |  | Subset-026-3.13.10.5.5 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | The test case shall consider the train data and the National Values (either a valid Packet 3 or the Default Values are used) as preconditions to be known before setting the appropriate values for the rest of the trackside info and the speed profile. The required observables (speed supervision limits) should be checked using a suitable tool that performs fully the “speed and distance monitoring" functionality (by default use ERA Braking curves tool, which can be downloaded from this website: http://www.era.europa.eu/Core-Activities/ERTMS/Pages/Braking-Curves-Simulation-Tool.aspx). |
| Constraints | Distance between EoA and SvL together with location accuracy of the LRBG should have the appropiate values so that RSM start location is calculated from SBD curve. NV are stored with Q_NVINHSMICPERM=0 (No Permission to inhibit the compensation of the speed measurement inaccuracy) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | Only in L2/L3 |
| National Values | STORED | Q_NVINHSMICPERM=0 |
| Movement Authority | STORED | Release speed given by trackside directly or in terms of National Values |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Axle load speed profile | NOT STORED | - |
| STM max speed | NOT STORED | - |
| STM system speed/distance | NOT STORED | - |
| Temporary Speed Restrictions | NOT STORED | - |
| Signalling related Speed Restriction | NOT STORED | Only in L1 |
| Mode Profile | STORED | Only in OS or LS mode |
| Unconditional Emergency Stop | NOT STORED | - |
| Conditional Emergency Stop | NOT STORED | - |
| Train Position | STORED | - |
| Train Data | STORED | - |
| Adhesion factor | NOT STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Driver ID | STORED | - |
| Radio Network ID | STORED | Only in L2/L3 |
| RBC ID/Phone Number | STORED | Only in L2/L3 |
| Train Running Number | STORED | - |
| Track Ahead Free Request | NOT STORED | - |
| Level Crossing information | NOT STORED | - |
| Permitted Braking Distance Information | NOT STORED | - |
| Operated System Version | 010 0000 | Version 2.0, introduced in SRS 3.3.0 |
| Virtual Balise Covers | NOT STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is moving (V_TRAIN ≤ VRELEASE) | I | INT | The train is under target speed monitoring near the start of the release speed monitoring location. Train speed/position does not exceed the Indication supervision limit (I). | N/A | N/A | - |
| 2 | N/A | N/A | The driver requests the speed and distance monitoring information to be displayed | I | DMI | Only in OS mode. (The permitted speed shall be displayed until the driver requests to stop this its. See for example Req 4.4.12.1.4) | N/A | N/A | - |
| 3 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The Target speed montoring type is displayed. (As there is no change in the DMI, there is no JRU step with Target speed montoring type recorded) | N/A | N/A | - |
| 4 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The DMI aspect is coherent with Normal Status . Permitted speed is displayed (only if not in LS mode). (As there is no change in the DMI, there is not JRU step with Normal Status recorded) | N/A | N/A | - |
| 5 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The train has passed with its estimated front end the RSM start location The Release speed montoring type is displayed The release speed,target distance (only if not in LS mode) and braking to target permitted speed supervision limit (only if not in LS mode) are displayed | N/A | N/A | - |
| 6 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The DMI aspect is coherent with Indication Status . Permitted speed is displayed (only if not in LS mode). | N/A | N/A | - |
| 7 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=3; M_SDMSUPSTAT=1 is recorded | O | JRU | M_SDMTYPE (Speed and distance monitoring type) = 3 (Release speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 1 (Indication Status) | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | Only in L2/L3 |
| National Values | STORED | Q_NVINHSMICPERM=0 |
| Movement Authority | STORED | Release speed given by trackside directly or in terms of National Values |
| Gradient Profile | STORED | - |
| International SSP | STORED | No SSP element lower than release speed in the area of Release speed monitoring |
| Axle load speed profile | NOT STORED | - |
| STM max speed | NOT STORED | - |
| STM system speed/distance | NOT STORED | - |
| Temporary Speed Restrictions | NOT STORED | - |
| Signalling related Speed Restriction | NOT STORED | Only in L1 |
| Mode Profile | STORED | Only in OS or LS mode |
| Unconditional Emergency Stop | NOT STORED | - |
| Conditional Emergency Stop | NOT STORED | - |
| Train Position | STORED | - |
| Train Data | STORED | - |
| Adhesion factor | NOT STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Driver ID | STORED | - |
| Radio Network ID | STORED | Only in L2/L3 |
| RBC ID/Phone Number | STORED | Only in L2/L3 |
| Train Running Number | STORED | - |
| Track Ahead Free Request | NOT STORED | - |
| Level Crossing information | NOT STORED | - |
| Permitted Braking Distance Information | NOT STORED | - |
| Operated System Version | 010 0000 | Version 2.0, introduced in SRS 3.3.0 |
| Virtual Balise Covers | NOT STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 6

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | Requirements for Release speed monitoring (Tables 13, 14 and 15) |  | 3131050 |
| Test Case of Feature | Triggering condition t2 of Table 13 (valid for Vest > VRELEASE) and Revocation condition r0 of Table 14 (valid for train at standstill). Transition from Normal to Intervention status and subsequent Intervention to Indication one of table 15. |  | 6 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS L2/L3: FS, LS, OS |  |  |
| Target of Test | The train enters from the area of target speed monitoring to the release speed monitoring one when max safe front end has passed the RSM start location calculated from EBD (condition id [2] of Subset-026-3.13.10.6.1) and after receiving a new MA which allows this scenario. The release speed target distance and braking to target permitted speed supervision limit are displayed on the DMI. As Vest > VRELEASE, the transition from Normal to Intervention status is shown on the DMI and recorded in the JRU. Emergency brake command is triggered (TIU) and RECORDED. When train is at standstill Transition from Intervention to Indication status is shown on the DMI and RECORDED in the JRU. Emergency brake command is revoked (TIU) and RECORDED Q_NVINHSMICPERM=0 (No permission to inhibit the compensation of the speed measurement inaccuracy) to test positive condition of Subset-026-3.13.9.4.6 b) Use of d_startRSM=dSB2(Vrelease) if dSBI2(Vrelease)-dSBI1(Vrelease) < dmax safefront -destfront equation for based EBD curve respectively of Subset-026-3.13.9.4.7 is tested. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.13.10.5.1 |  |
|  |  | Subset-026-3.13.10.5.2 |  |
|  |  | Subset-026-3.13.10.5.3 |  |
|  |  | Subset-026-3.13.10.5.4 |  |
|  |  | Subset-026-3.13.10.5.4 table 13 |  |
|  |  | Subset-026-3.13.10.5.4 table 14 |  |
|  |  | Subset-026-3.13.10.5.5 table 15 |  |
|  |  | Subset-026-3.13.10.5.7 |  |
|  |  | Subset-026-3.13.9.4.6 a) |  |
|  |  | Subset-026-3.13.9.4.6 b) |  |
|  |  | Subset-026-3.13.9.4.7 |  |
|  |  | Subset-026-4.7.2 table - Output information Release speed |  |
|  |  | Subset-026-4.7.2 table - Output information Speed and distance monitoring supervision status |  |
|  |  | Subset-026-3.13.10.6.1 |  |
|  |  | Subset-026-3.13.10.5.5 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | The test case shall consider the train data and the National Values (either a valid Packet 3 or the Default Values are used) as preconditions to be known before setting the appropriate values for the rest of the trackside info and the speed profile. The required observables (speed supervision limits) should be checked using a suitable tool that performs fully the “speed and distance monitoring" functionality (by default use ERA Braking curves tool, which can be downloaded from this website: http://www.era.europa.eu/Core-Activities/ERTMS/Pages/Braking-Curves-Simulation-Tool.aspx). |
| Constraints | NV are stored with Q_NVINHSMICPERM=0 (No Permission to inhibit the compensation of the speed measurement inaccuracy) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | Only in L2/L3 |
| National Values | STORED | Q_NVINHSMICPERM=0 |
| Movement Authority | STORED | Release speed given by trackside directly or in terms of National Values |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Axle load speed profile | NOT STORED | - |
| STM max speed | NOT STORED | - |
| STM system speed/distance | NOT STORED | - |
| Temporary Speed Restrictions | NOT STORED | - |
| Signalling related Speed Restriction | NOT STORED | Only in L1 |
| Mode Profile | STORED | Only in OS or LS mode |
| Unconditional Emergency Stop | NOT STORED | - |
| Conditional Emergency Stop | NOT STORED | - |
| Train Position | STORED | - |
| Train Data | STORED | - |
| Adhesion factor | NOT STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Driver ID | STORED | - |
| Radio Network ID | STORED | Only in L2/L3 |
| RBC ID/Phone Number | STORED | Only in L2/L3 |
| Train Running Number | STORED | - |
| Track Ahead Free Request | NOT STORED | - |
| Level Crossing information | NOT STORED | - |
| Permitted Braking Distance Information | NOT STORED | - |
| Operated System Version | 010 0000 | Version 2.0, introduced in SRS 3.3.0 |
| Virtual Balise Covers | NOT STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is moving (V_TRAIN > VRELEASE) | I | INT | The train is under target speed monitoring near the start of the release speed monitoring location. Train speed/position does not exceed the Indication supervision limit (I) | N/A | N/A | - |
| 2 | N/A | N/A | The driver requests the speed and distance monitoring information to be displayed | I | DMI | Only in OS mode. (The permitted speed shall be displayed until the driver requests to stop this is. See for example Rerq4.4.12.1.4) | N/A | N/A | - |
| 3 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The Target speed monitoring type is displayed. (As there is no change in the DMI, there is no JRU step with Target speed monitoring type recorded) | N/A | N/A | - |
| 4 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The DMI aspect is coherent with Normal Status . Permitted speed is displayed (only if not in LS mode). (As there is no change in the DMI, there is not JRU step with Normal Status recorded) | N/A | N/A | - |
| 5 | N/A | N/A | One Balise Group message containing packet 12, packet 21 and packet 27 is received | I | BTM | Only in L1 | N/A | N/A | - |
| 6 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | Only in L1 | N/A | N/A | - |
| 7 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 21, packet 27 is received | I | RTM | Only in L2/L3 | N/A | N/A | - |
| 8 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=3) is recorded | O | JRU | Only in L2/L3 | N/A | N/A | - |
| 9 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The DMI aspect is coherent with Intervention Status . Permitted speed is displayed (only if not in LS mode). | N/A | N/A | - |
| 10 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=3; M_SDMSUPSTAT=4 is recorded | O | JRU | M_SDMTYPE (Speed and distance monitoring type) = 3 (Release speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 4 (Intervention Status) | N/A | N/A | - |
| 11 | N/A | N/A | Emergency brake commanded | O | TIU | - | N/A | N/A | - |
| 12 | N/A | N/A | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | M_BRAKE_COMMAND_STATE=1 (Command) | N/A | N/A | - |
| 13 | N/A | N/A | The Status symbol "Service Brake or Emergency Brake intervention" is displayed | O | DMI | - | N/A | N/A | - |
| 14 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=1>) is recorded | O | JRU | The Status symbol "Service Brake or Emergency Brake intervention" is recorded | N/A | N/A | - |
| 15 | N/A | N/A | Train is at standstill (V_TRAIN=0; TIME= 10) | I | INT | - | N/A | N/A | - |
| 16 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The DMI aspect is coherent with Indication Status . Permitted speed is displayed (only if not in LS mode). | N/A | N/A | - |
| 17 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMSUPSTAT=1 is recorded | O | JRU | M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 1 (Indication Status) | N/A | N/A | - |
| 18 | N/A | N/A | Emergency brake not commanded | O | TIU | The Emergency Brake has been revoked | N/A | N/A | - |
| 19 | N/A | N/A | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=0) is recorded | O | JRU | M_BRAKE_COMMAND_STATE=0 (Not Command) | N/A | N/A | - |

| Step 5: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 12 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out". |
| N_ITER | 5 | FINITE VALUE | Set to zero if V_MAIN = 0 or if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | 255 |

| Step 7: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | Only in L2/L3 |
| National Values | STORED | Q_NVINHSMICPERM=0 |
| Axle load speed profile | NOT STORED | - |
| STM max speed | NOT STORED | - |
| STM system speed/distance | NOT STORED | - |
| Temporary Speed Restrictions | NOT STORED | - |
| Signalling related Speed Restriction | NOT STORED | Only in L1 |
| Mode Profile | STORED | Only in OS or LS mode |
| Unconditional Emergency Stop | NOT STORED | - |
| Conditional Emergency Stop | NOT STORED | - |
| Train Position | STORED | - |
| Train Data | STORED | - |
| Adhesion factor | NOT STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Driver ID | STORED | - |
| Radio Network ID | STORED | Only in L2/L3 |
| RBC ID/Phone Number | STORED | Only in L2/L3 |
| Train Running Number | STORED | - |
| Track Ahead Free Request | NOT STORED | - |
| Level Crossing information | NOT STORED | - |
| Permitted Braking Distance Information | NOT STORED | - |
| Operated System Version | 010 0000 | Version 2.0, introduced in SRS 3.3.0 |
| Virtual Balise Covers | NOT STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 7

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | Requirements for Release speed monitoring (Tables 13, 14 and 15) |  | 3131050 |
| Test Case of Feature | Triggering condition t2 of Table 13 (valid for Vest > VRELEASE) and Revocation condition r0 of Table 14 (valid for train at standstill). Transition from Overspeed to Intervention status and then from Intervention to Indication status according to table 15. |  | 7 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS L2/L3: FS, LS, OS |  |  |
| Target of Test | The train enters from the area of target speed monitoring to the release speed monitoring one when max safe front end has passed the RSM start location calculated from EBD (condition id [2] of Subset-026-3.13.10.6.1) and after receiving a new MA which allows this scenario. The release speed,target distance and braking to target permitted speed supervision limit are displayed on the DMI. As Vest > VRELEASE, the transition from Overspeed to Intervention status is shown on the DMI and recorded in the JRU. Emergency brake command is triggered (TIU) and RECORDED. When train is at standstill Transition from Intervention to Indication status is shown on the DMI and RECORDED in the JRU.Emergency brake command is revoked (TIU) and RECORDED Q_NVINHSMICPERM=0 (No permission to inhibit the compensation of the speed measurement inaccuracy) to test positive condition of Subset-026-3.13.9.4.6 b) Use of d_startRSM=dSB2(Vrelease) if dSBI2(Vrelease)-dSBI1(Vrelease) < dmax safefront -destfront equation for based EBD curve respectively of Subset-026-3.13.9.4.7 is tested. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.13.10.5.1 |  |
|  |  | Subset-026-3.13.10.5.2 |  |
|  |  | Subset-026-3.13.10.5.3 |  |
|  |  | Subset-026-3.13.10.5.4 |  |
|  |  | Subset-026-3.13.10.5.4 table 13 |  |
|  |  | Subset-026-3.13.10.5.4 table 14 |  |
|  |  | Subset-026-3.13.10.5.5 table 15 |  |
|  |  | Subset-026-3.13.10.5.7 |  |
|  |  | Subset-026-3.13.9.4.6 a) |  |
|  |  | Subset-026-3.13.9.4.6 b) |  |
|  |  | Subset-026-3.13.9.4.7 |  |
|  |  | Subset-026-4.7.2 table - Output information Release speed |  |
|  |  | Subset-026-4.7.2 table - Output information Speed and distance monitoring supervision status |  |
|  |  | Subset-026-3.13.10.6.1 |  |
|  |  | Subset-026-3.13.10.5.5 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | The test case shall consider the train data and the National Values (either a valid Packet 3 or the Default Values are used) as preconditions to be known before setting the appropriate values for the rest of the trackside info and the speed profile. The required observables (speed supervision limits) should be checked using a suitable tool that performs fully the “speed and distance monitoring" functionality (by default use ERA Braking curves tool, which can be downloaded from this website: http://www.era.europa.eu/Core-Activities/ERTMS/Pages/Braking-Curves-Simulation-Tool.aspx). |
| Constraints | NV are stored with Q_NVINHSMICPERM=0 (No Permission to inhibit the compensation of the speed measurement inaccuracy) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | Only in L2/L3 |
| National Values | STORED | Q_NVINHSMICPERM=0 |
| Movement Authority | STORED | Release speed given by trackside directly or in terms of National Values |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Axle load speed profile | NOT STORED | - |
| STM max speed | NOT STORED | - |
| STM system speed/distance | NOT STORED | - |
| Temporary Speed Restrictions | NOT STORED | - |
| Signalling related Speed Restriction | NOT STORED | Only in L1 |
| Mode Profile | STORED | Only in OS or LS mode |
| Unconditional Emergency Stop | NOT STORED | - |
| Conditional Emergency Stop | NOT STORED | - |
| Train Position | STORED | - |
| Train Data | STORED | - |
| Adhesion factor | NOT STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Driver ID | STORED | - |
| Radio Network ID | STORED | Only in L2/L3 |
| RBC ID/Phone Number | STORED | Only in L2/L3 |
| Train Running Number | STORED | - |
| Track Ahead Free Request | NOT STORED | - |
| Level Crossing information | NOT STORED | - |
| Permitted Braking Distance Information | NOT STORED | - |
| Operated System Version | 010 0000 | Version 2.0, introduced in SRS 3.3.0 |
| Virtual Balise Covers | NOT STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is moving (V_TRAIN > VRELEASE) | I | INT | - | N/A | N/A | - |
| 2 | N/A | N/A | The driver requests the speed and distance monitoring information to be displayed | I | DMI | Only in OS mode. (The permitted speed shall be displayed until the driver requests to stop this its. See for example Req 4.4.12.1.4) | N/A | N/A | - |
| 3 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | When the train has passed with its max safe front end the location of the Overspeed supervision limit, valid for the estimated speed and calculated from the EBD, then the supervision status changes to "Overspeed Status" (t4 in 3.13.10.4.10 table 9 of the SRS). So, the DMI aspect is coherent with Overspeed Status in a Target Speed Monitoring section. | N/A | N/A | - |
| 4 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=2; M_SDMSUPSTAT=2) is recorded | O | JRU | M_SDMTYPE (Speed and distance monitoring type) = 2 (Target speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 2 (Overspeed Status) | N/A | N/A | - |
| 5 | N/A | N/A | One Balise Group message containing packet 12, packet 21 and packet 27 is received | I | BTM | Only in L1 | N/A | N/A | - |
| 6 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | Only in L1 | N/A | N/A | - |
| 7 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 21, packet 27 is received | I | RTM | Only in L2/L3 | N/A | N/A | - |
| 8 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=3) is recorded | O | JRU | Only in L2/L3 | N/A | N/A | - |
| 9 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The DMI aspect is coherent with Intervention Status . Permitted speed is displayed (only if not in LS mode). | N/A | N/A | - |
| 10 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=3; M_SDMSUPSTAT=4 is recorded | O | JRU | M_SDMTYPE (Speed and distance monitoring type) = 3 (Release speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 4 (Intervention Status) | N/A | N/A | - |
| 11 | N/A | N/A | Emergency brake commanded | O | TIU | - | N/A | N/A | - |
| 12 | N/A | N/A | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | M_BRAKE_COMMAND_STATE=1 (Command) | N/A | N/A | - |
| 13 | N/A | N/A | The Status symbol "Service Brake or Emergency Brake intervention" is displayed | O | DMI | - | N/A | N/A | - |
| 14 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=1>) is recorded | O | JRU | The Status symbol "Service Brake or Emergency Brake intervention" is recorded | N/A | N/A | - |
| 15 | N/A | N/A | Train is at standstill (V_TRAIN=0; TIME= 10) | I | INT | - | N/A | N/A | - |
| 16 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The DMI aspect is coherent with Indication Status . Permitted speed is displayed (only if not in LS mode). | N/A | N/A | - |
| 17 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMSUPSTAT=1 is recorded | O | JRU | M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 1 (Indication Status) | N/A | N/A | - |
| 18 | N/A | N/A | Emergency brake not commanded | O | TIU | The Emergency Brake has been revoked | N/A | N/A | - |
| 19 | N/A | N/A | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=0) is recorded | O | JRU | M_BRAKE_COMMAND_STATE=0 (Not Command) | N/A | N/A | - |

| Step 5: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 12 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out". |
| N_ITER | 5 | FINITE VALUE | Set to zero if V_MAIN = 0 or if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | 255 |

| Step 7: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | Only in L2/L3 |
| National Values | STORED | Q_NVINHSMICPERM=0 |
| Axle load speed profile | NOT STORED | - |
| STM max speed | NOT STORED | - |
| STM system speed/distance | NOT STORED | - |
| Temporary Speed Restrictions | NOT STORED | - |
| Signalling related Speed Restriction | NOT STORED | Only in L1 |
| Mode Profile | STORED | Only in OS or LS mode |
| Unconditional Emergency Stop | NOT STORED | - |
| Conditional Emergency Stop | NOT STORED | - |
| Train Position | STORED | - |
| Train Data | STORED | - |
| Adhesion factor | NOT STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Driver ID | STORED | - |
| Radio Network ID | STORED | Only in L2/L3 |
| RBC ID/Phone Number | STORED | Only in L2/L3 |
| Train Running Number | STORED | - |
| Track Ahead Free Request | NOT STORED | - |
| Level Crossing information | NOT STORED | - |
| Permitted Braking Distance Information | NOT STORED | - |
| Operated System Version | 010 0000 | Version 2.0, introduced in SRS 3.3.0 |
| Virtual Balise Covers | NOT STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 8

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | Requirements for Release speed monitoring (Tables 13, 14 and 15) |  | 3131050 |
| Test Case of Feature | When the speed and distance monitoring function becomes active and the release speed monitoring is the first one entered, the triggering condition t1 defined in Table 13 and since the Indication status does not apply, the on-board shall immediately set the supervision status to the Intervention status, applying a transition from the Indication status according to Table 15. |  | 8 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS L2/L3: FS, LS, OS |  |  |
| Target of Test | When the speed and distance monitoring function becomes active and the release speed monitoring is the first one entered, the triggering condition t1 defined in Table 13 and since the Indication status does not apply, the on-board shall immediately set the supervision status to the Intervention status, applying a transition from the Indication status according to Table 15. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.13.10.5.6 |  |
|  |  | Subset-026-3.13.10.5.4 table 13 |  |
|  |  | Subset-026-3.13.10.5.4 table 14 |  |
|  |  | Subset-026-3.13.10.5.5 table 15 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | The test case shall consider the train data and the National Values (either a valid Packet 3 or the Default Values are used) as preconditions to be known before setting the appropriate values for the rest of the trackside info and the speed profile. The required observables (speed supervision limits) should be checked using a suitable tool that performs fully the “speed and distance monitoring" functionality (by default use ERA Braking curves tool, which can be downloaded from this website: http://www.era.europa.eu/Core-Activities/ERTMS/Pages/Braking-Curves-Simulation-Tool.aspx). |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 4 | UN |
| Radio communication session | ESTABLISHED | Only if a transition to L2/L3 is stored on-board |
| Movement Authority | STORED | The variables D_DP and V_RELEASEDP (and/or D_OL and V_RELEASEOL) will be greater than zero. |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Mode Profile | STORED | Only if the on-board changes to OS/LS mode |
| Train Position | STORED | - |
| Train Data | STORED | - |
| ERTMS/ETCS level | 0 | L0 |
| Driver ID | STORED | - |
| Radio Network ID | STORED | Only if a transition to L2/L3 is stored on-board |
| RBC ID/Phone Number | STORED | Only if a transition to L2/L3 is stored on-board |
| Train Running Number | STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | Both | RTM | Only if a transition to L2/L3 is stored on-board |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 | UN | Train is moving (V_TRAIN > <V_RELEASE>) | I | INT | The train is under target speed monitoring near the start of the release speed monitoring location. The value of V_RELEASE will be set afterwards. | L0 | UN | - |
| 2 | L0 | UN | USE_FT4060300.19 for changing to FS mode or USE_FT4060300.55 for changing to OS mode or USE_FT5190300.4 for changing to LS mode | - | - | - | L1 / L2 / L3 | FS / LS / OS | - |
| 3 | L1 / L2 / L3 | FS / LS / OS | The driver requests the speed and distance monitoring information to be displayed | I | DMI | Only in OS mode. (The permitted speed shall be displayed until the driver requests to stop this its. See for example Req 4.4.12.1.4) | L1 / L2 / L3 | FS / LS / OS | - |
| 4 | L1 / L2 / L3 | FS / LS / OS | Emergency brake commanded | O | TIU | - | L1 / L2 / L3 | FS / LS / OS | - |
| 5 | L1 / L2 / L3 | FS / LS / OS | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | L1 / L2 / L3 | FS / LS / OS | - |
| 6 | L1 / L2 / L3 | FS / LS / OS | The Status symbol "Service Brake or Emergency Brake intervention" is displayed | O | DMI | - | L1 / L2 / L3 | FS / LS / OS | - |
| 7 | L1 / L2 / L3 | FS / LS / OS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=1>) is recorded | O | JRU | The Status symbol "Service Brake or Emergency Brake intervention" is recorded | L1 / L2 / L3 | FS / LS / OS | - |
| 8 | L1 / L2 / L3 | FS / LS / OS | The speed and distance monitoring information is displayed | O | DMI | The Release speed montoring type is displayed and the DMI aspect is coherent with Intervention Status. | L1 / L2 / L3 | FS / LS / OS | - |
| 9 | L1 / L2 / L3 | FS / LS / OS | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=3; M_SDMSUPSTAT=4) is recorded | O | JRU | M_SDMTYPE (Speed and distance monitoring type) = 3 (Release speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 4 (Intervention Status) | L1 / L2 / L3 | FS / LS / OS | - |
| 10 | L1 / L2 / L3 | FS / LS / OS | Train is at standstill (V_TRAIN=0) | I | INT | - | L1 / L2 / L3 | FS / LS / OS | - |
| 11 | L1 / L2 / L3 | FS / LS / OS | Emergency brake not commanded | O | TIU | The Emergency Brake has been revoked | L1 / L2 / L3 | FS / LS / OS | - |
| 12 | L1 / L2 / L3 | FS / LS / OS | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=0) is recorded | O | JRU | M_BRAKE_COMMAND_STATE=0 (Not Command) | L1 / L2 / L3 | FS / LS / OS | - |
| 13 | L1 / L2 / L3 | FS / LS / OS | The speed and distance monitoring information is displayed | O | DMI | The DMI aspect is coherent with Indication Status . Permitted speed is displayed (only if not in LS mode). | L1 / L2 / L3 | FS / LS / OS | - |
| 14 | L1 / L2 / L3 | FS / LS / OS | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=3; M_SDMSUPSTAT=4) is recorded | O | JRU | M_SDMTYPE (Speed and distance monitoring type) = 3 (Release speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 1 (Intervention Status) | L1 / L2 / L3 | FS / LS / OS | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | Only in L2/L3 |
| Train Position | STORED | - |
| Train Data | STORED | - |
| Adhesion factor | NOT STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Driver ID | STORED | - |
| Radio Network ID | STORED | Only in L2/L3 |
| RBC ID/Phone Number | STORED | Only in L2/L3 |
| Train Running Number | STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | Both | RTM | Only in L2/L3 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 9

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | Requirements for Release speed monitoring (Tables 13, 14 and 15) |  | 3131050 |
| Test Case of Feature | Triggering condition t2 of Table 13 (valid for Vest > VRELEASE) and Revocation condition r0 of Table 14 (valid for train at standstill). Transition from Warning to Intervention status and and then from Intervention to Indication status according to table 15. |  | 9 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS L2/L3: FS, LS, OS |  |  |
| Target of Test | The train enters from the area of target speed monitoring to the release speed monitoring one when max safe front end has passed the RSM start location calculated from EBD (condition id [2] of Subset-026-3.13.10.6.1) and after receiving a new MA which allows this scenario. The release speed target distance and braking to target permitted speed supervision limit are displayed on the DMI. As Vest > VRELEASE, the transition from Warning to Intervention status is shown on the DMI and recorded in the JRU. Emergency brake command is triggered (TIU) and RECORDED. When train is at standstill Transition from Intervention to Indication status is shown on the DMI and RECORDED in the JRU .Emergency brake command is revoked (TIU) and RECORDED Q_NVINHSMICPERM=0 (No permission to inhibit the compensation of the speed measurement inaccuracy) to test positive condition of Subset-026-3.13.9.4.6 b) Use of d_startRSM=dSB2(Vrelease) if dSBI2(Vrelease)-dSBI1(Vrelease) < dmax safefront -destfront equation for based EBD curve respectively of Subset-026-3.13.9.4.7 is tested. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.13.10.5.1 |  |
|  |  | Subset-026-3.13.10.5.2 |  |
|  |  | Subset-026-3.13.10.5.3 |  |
|  |  | Subset-026-3.13.10.5.4 |  |
|  |  | Subset-026-3.13.10.5.4 table 13 |  |
|  |  | Subset-026-3.13.10.5.4 table 14 |  |
|  |  | Subset-026-3.13.10.5.5 table 15 |  |
|  |  | Subset-026-3.13.10.5.7 |  |
|  |  | Subset-026-3.13.9.4.6 a) |  |
|  |  | Subset-026-3.13.9.4.6 b) |  |
|  |  | Subset-026-3.13.9.4.7 |  |
|  |  | Subset-026-4.7.2 table - Output information Release speed |  |
|  |  | Subset-026-4.7.2 table - Output information Speed and distance monitoring supervision status |  |
|  |  | Subset-026-3.13.10.6.1 |  |
|  |  | Subset-026-3.13.10.5.5 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | The test case shall consider the train data and the National Values (either a valid Packet 3 or the Default Values are used) as preconditions to be known before setting the appropriate values for the rest of the trackside info and the speed profile. The required observables (speed supervision limits) should be checked using a suitable tool that performs fully the “speed and distance monitoring" functionality (by default use ERA Braking curves tool, which can be downloaded from this website: http://www.era.europa.eu/Core-Activities/ERTMS/Pages/Braking-Curves-Simulation-Tool.aspx). |
| Constraints | NV are stored with Q_NVINHSMICPERM=0 (No Permission to inhibit the compensation of the speed measurement inaccuracy) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | Only in L2/L3 |
| National Values | STORED | Q_NVINHSMICPERM=0 |
| Movement Authority | STORED | Release speed given by trackside directly or in terms of National Values |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Axle load speed profile | NOT STORED | - |
| STM max speed | NOT STORED | - |
| STM system speed/distance | NOT STORED | - |
| Temporary Speed Restrictions | NOT STORED | - |
| Signalling related Speed Restriction | NOT STORED | Only in L1 |
| Mode Profile | STORED | Only in OS or LS mode |
| Unconditional Emergency Stop | NOT STORED | - |
| Conditional Emergency Stop | NOT STORED | - |
| Train Position | STORED | - |
| Train Data | STORED | - |
| Adhesion factor | NOT STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Driver ID | STORED | - |
| Radio Network ID | STORED | Only in L2/L3 |
| RBC ID/Phone Number | STORED | Only in L2/L3 |
| Train Running Number | STORED | - |
| Track Ahead Free Request | NOT STORED | - |
| Level Crossing information | NOT STORED | - |
| Permitted Braking Distance Information | NOT STORED | - |
| Operated System Version | 010 0000 | Version 2.0, introduced in SRS 3.3.0 |
| Virtual Balise Covers | NOT STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is moving (V_TRAIN > VRELEASE) | I | INT | - | N/A | N/A | - |
| 2 | N/A | N/A | The driver requests the speed and distance monitoring information to be displayed | I | DMI | Only in OS mode. (The permitted speed shall be displayed until the driver requests to stop this its. See for example Req 4.4.12.1.4) | N/A | N/A | - |
| 3 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | When the train has passed with its max safe front end the location of the Warning supervision limit, valid for the estimated speed and calculated from the EBD, then the supervision status changes to "Warning Status" (t7 in 3.13.10.4.10 table 9 of the SRS). So, the DMI aspect is coherent with Warning Status in a Target Speed Monitoring section. | N/A | N/A | - |
| 4 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=2; M_SDMSUPSTAT=3) is recorded | O | JRU | M_SDMTYPE (Speed and distance monitoring type) = 2 (Target speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 3 (Warning Status) | N/A | N/A | - |
| 5 | N/A | N/A | Cut off traction | O | TIU | - | N/A | N/A | - |
| 6 | N/A | N/A | TRACTION CUT OFF COMMAND STATE (NID_MESSAGE_JRU=43; M_TCO_COMMAND_STATE=1) is recorded | O | JRU | M_TCO_COMMAND_STATE = 1 (Commanded) | N/A | N/A | - |
| 7 | N/A | N/A | One Balise Group message containing packet 12, packet 21 and packet 27 is received | I | BTM | Only in L1 | N/A | N/A | - |
| 8 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | Only in L1 | N/A | N/A | - |
| 9 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 21, packet 27 is received | I | RTM | Only in L2/L3 | N/A | N/A | - |
| 10 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=3) is recorded | O | JRU | Only in L2/L3 | N/A | N/A | - |
| 11 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The DMI aspect is coherent with Intervention Status . Permitted speed is displayed (only if not in LS mode). | N/A | N/A | - |
| 12 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=3; M_SDMSUPSTAT=4 is recorded | O | JRU | M_SDMTYPE (Speed and distance monitoring type) = 3 (Release speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 4 (Intervention Status) | N/A | N/A | - |
| 13 | N/A | N/A | Emergency brake commanded | O | TIU | - | N/A | N/A | - |
| 14 | N/A | N/A | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | M_BRAKE_COMMAND_STATE=1 (Command) | N/A | N/A | - |
| 15 | N/A | N/A | The Status symbol "Service Brake or Emergency Brake intervention" is displayed | O | DMI | - | N/A | N/A | - |
| 16 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=1>) is recorded | O | JRU | The Status symbol "Service Brake or Emergency Brake intervention" is recorded | N/A | N/A | - |
| 17 | N/A | N/A | Train is at standstill (V_TRAIN=0; TIME= 10) | I | INT | - | N/A | N/A | - |
| 18 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The DMI aspect is coherent with Indication Status . Permitted speed is displayed (only if not in LS mode). | N/A | N/A | - |
| 19 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMSUPSTAT=1 is recorded | O | JRU | M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 1 (Indication Status) | N/A | N/A | - |
| 20 | N/A | N/A | Emergency brake not commanded | O | TIU | The Emergency Brake has been revoked | N/A | N/A | - |
| 21 | N/A | N/A | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=0) is recorded | O | JRU | M_BRAKE_COMMAND_STATE=0 (Not Command) | N/A | N/A | - |

| Step 7: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 12 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out". |
| N_ITER | 5 | FINITE VALUE | Set to zero if V_MAIN = 0 or if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | 255 |

| Step 9: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | Only in L2/L3 |
| National Values | STORED | Q_NVINHSMICPERM=0 |
| Axle load speed profile | NOT STORED | - |
| STM max speed | NOT STORED | - |
| STM system speed/distance | NOT STORED | - |
| Temporary Speed Restrictions | NOT STORED | - |
| Signalling related Speed Restriction | NOT STORED | Only in L1 |
| Mode Profile | STORED | Only in OS or LS mode |
| Unconditional Emergency Stop | NOT STORED | - |
| Conditional Emergency Stop | NOT STORED | - |
| Train Position | STORED | - |
| Train Data | STORED | - |
| Adhesion factor | NOT STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Driver ID | STORED | - |
| Radio Network ID | STORED | Only in L2/L3 |
| RBC ID/Phone Number | STORED | Only in L2/L3 |
| Train Running Number | STORED | - |
| Track Ahead Free Request | NOT STORED | - |
| Level Crossing information | NOT STORED | - |
| Permitted Braking Distance Information | NOT STORED | - |
| Operated System Version | 010 0000 | Version 2.0, introduced in SRS 3.3.0 |
| Virtual Balise Covers | NOT STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |