| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 4080408 (4.8.4: Accepted Information depending on the modes - Danger for SH information) Total: 4 Test case(s) |  |
| REF: | Subset-076-5-2-4080408 |
| ISSUE: | Version 3.2.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 0.0.1 23.03.2012 | All | First version/review | DLR |
| Version 2.9.0 31.03.2013 | All | First version/review for Baseline 3 | CEDEX |
| Version 2.9.1 04.10.2013 | All | Requirements corrections | CEDEX |
| Version 2.9.2 11.11.2013 | ALL | REVIEW ALL | CEDEX |
| Version 2.9.3 13.12.2013 | ALL | Second Review | CEDEX |
| Version 2.9.4 17.12.2013 | ALL | 2ND REVIEW | CEDEX |
| Version 2.9.5 17.12.2013 | ALL | 3RD REVIEW | CEDEX |
| Version 2.9.6 20.12.2013 | ALL | REVIEW | CEDEX |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.2.0 31.03.2017 | All | Baseline 3 1st maintenance release version - Bug fixing | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Danger for SH information |  | 4080408 |
| Test Case of Feature | The train is in shunting mode and overpasses a balise with the packet number 132 (danger for shunting information). The train will be tripped if it’s in shunting mode. |  | 1 |
| Applicable Mode/Level Combinations | L0: SH L1: SH L2: SH L3: SH LNTC: SH |  |  |
| Target of Test | The target of this test is to check that: When the train is in shunting mode and overpasses a balise with the packet number 132 (danger for shunting information) the train is tripped. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.4.8.1.1 c) |  |
|  |  | Subset-026-4.8.4.2 Danger for SH information |  |
|  |  | Subset-026-7.4.2.28 Packet Number 132: Danger for Shunting information |  |
|  |  | Subset-026-4.8.3.1.1 Exception [13] |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The displayed mode changes from SH to TR. Check on the JRU that: The message "Danger for SH information" is recorded. The transition to TR is recorded. |
| Constraints | Train Trip is not inhibited. In Level 0/ NTC "Danger for SH information" is received together with an immediate level transition order to L1/2/3. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 | SH |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / L1 / L2 / L3 / LNTC | SH | One Balise Group message (N_TOTAL=0; NID_C=<comment>) containing packet 132 "Danger for shunting information" (Q_ASPETC = 0) is received | I | BTM | One of the balises in BGa has a packet number 132 (Danger for shunting information) with the variable Q_ASPECT set to zero. If in L0/LNTC an immediate level transition to level 1/2/3 shall be also included (packet 41) | L0 / L1 / L2 / L3 / LNTC | SH | - |
| 2 | L0 / L1 / L2 / L3 / LNTC | SH | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L0 / L1 / L2 / L3 / LNTC | SH | - |
| 3 | L1 / L2 / L3 | SH | The Level symbol "Level <L1 / L2 / L3>" is displayed | O | DMI | Only if in Level 0/NTC | L1 / L2 / L3 | SH | - |
| 4 | L1 / L2 / L3 | SH | The level change is RECORDED | O | JRU | Only if in Level 0/NTC | L1 / L2 / L3 | SH | - |
| 5 | L1 / L2 / L3 | SH | USE_FT4060300.32 | - | - | Reception of “Stop if in shunting” information (Transition [49]) | L1 / L2 / L3 | TR | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 132 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_ASPECT | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 | TR |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On Board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Danger for SH information |  | 4080408 |
| Test Case of Feature | The train is in L1, L2 or L3, and overpasses a balise with the packet number 132 (danger for shunting information). The train will NOT be tripped because is running in a mode different to SH mode . |  | 2 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS, SR, PS, SL, SB, TR, PT, NL, RV L2: FS, LS, OS, SR, PS, SL, SB, TR, PT, NL, RV L3: FS, LS, OS, SR, PS, SL, SB, TR, PT, NL, RV |  |  |
| Target of Test | The target of this test is to check that: When the train is in L1, L2 or L3, and overpasses a balise with the packet number 132 (danger for shunting information). the train is NOT tripped because is running in a mode different to SH mode . |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.4.8.1.1 c) |  |
|  |  | Subset-026-4.8.4.2 Danger for SH information |  |
|  |  | Subset-026-7.4.2.28 Packet Number 132: Danger for Shunting information |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : No transition to TR is displayed Check on the JRU that: No transition to TR is recorded |
| Constraints | The reception of balise information in SB requires the train moving without an immediate movement protection. The national value D_NVROLL has to be set to a high value, enabling the train to read a BG. Train Trip is not inhibited. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 5 / 6 / 7 / 8 / 11 / 12 / 14 / 15 | FS / OS / SR/ SL / SB / TR / PT / NL / LS / RV / PS |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (N_TOTAL=0; NID_C=<comment>) containing packet 132 "Danger for shunting information" (Q_ASPETC = 0) is received | I | BTM | - | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | NOT The Mode symbol "Trip" is displayed | O | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | NOT GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=7) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 132 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_ASPECT | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 /1/ 2 / 5 / 6 / 7 / 8 / 11 / 12 / 14 / 15 | FS / OS / SR/ SL / SB / TR / PT / NL / LS / RV / PS |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Danger for SH information |  | 4080408 |
| Test Case of Feature | The train is in L0 or LNTC, and overpasses a balise with the packet number 132 (danger for shunting information). The train will NOT be tripped. |  | 3 |
| Applicable Mode/Level Combinations | L0: UN, PS, SL, SB, TR, NL LNTC: PS, SL, SB, TR, NL, SN |  |  |
| Target of Test | The target of this test is to check that: When the train is in L0 or LNTC, and overpasses a balise with the packet number 132 (danger for shunting information) the train is NOT tripped. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.4.8.1.1 c) |  |
|  |  | Subset-026-4.8.4.2 Danger for SH information |  |
|  |  | Subset-026-7.4.2.28 Packet Number 132: Danger for Shunting information |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : No transition to TR is displayed Check on the JRU that: No transition to TR is recorded |
| Constraints | Danger for SH information is received together with an immediate level transition order to L1/2/3. The reception of balise information in SB requires the train moving without an immediate movement protection. The national value D_NVROLL has to be set to a high value, enabling the train to read a BG. Train Trip is not inhibited. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 4 / 5 / 6 / 7 / 11 / 13 /15 | UN / SL / SB / TR / NL / SN / PS |
| ERTMS/ETCS level | 0/ 1 | L0 / LNTC |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0/LNTC | UN / SL / SB / TR / NL / SN / PS | One Balise Group message (N_TOTAL=0; NID_C=<comment>) containing packet 132 "Danger for shunting information" (Q_ASPETC = 0), packet 41 is received | I | BTM | One of the balises in BGa has a packet number 132 (Danger for shunting information) with the variable Q_ASPECT set to zero. an immediate level transition to level 1/2/3 shall be also included (packet 41) | L0/LNTC | UN / SL / SB / TR / NL / SN / PS | - |
| 2 | L0/LNTC | UN / SL / SB / TR / NL / SN / PS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L0/LNTC | UN / SL / SB / TR / NL / SN / PS | - |
| 3 | L0/LNTC | UN / SL / SB / TR / NL / SN / PS | The Level symbol "Level <L1 / L2 / L3>" is displayed | O | DMI | - | L1/L2/L3 | SL / SB / TR / NL / PS | - |
| 4 | L1/L2/L3 | SL / SB / TR / NL / PS | The level change is RECORDED | O | JRU | - | L1/L2/L3 | SL / SB / TR / NL / PS | - |
| 5 | L1/L2/L3 | SL / SB / TR / NL / PS | NOT The Mode symbol "Trip" is displayed | O | DMI | - | L1/L2/L3 | SL / SB / TR / NL / PS | - |
| 6 | L1/L2/L3 | SL / SB / TR / NL / PS | NOT GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=7) is recorded | O | JRU | - | L1/L2/L3 | SL / SB / TR / NL / PS | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 132 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_ASPECT | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 41 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | FINITE VALUE | - |
| M_LEVELTR | 3 | FINITE VALUE | L1/L2/L3 |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTR = 1 (NTC). |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 5 / 6 / 7 / 11 /15 | SL / SB / TR / NL / PS |
| ERTMS/ETCS level | 0/ 1 | L1/L2/L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Danger for SH information |  | 4080408 |
| Test Case of Feature | The train is in shunting mode and overpasses a balise with the packet number 132 (danger for shunting information) and packet 41 (with value different to immediate). The train will NOT be tripped. |  | 4 |
| Applicable Mode/Level Combinations | L0: SH LNTC: SH |  |  |
| Target of Test | The target of this test is to check that: When the train is in shunting mode and overpasses a balise with packet 132 (danger for shunting information) together with packet 41 (with value different to immediate) the train is NOT tripped. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.4.8.1.1 c) |  |
|  |  | Subset-026-4.8.4.2 Danger for SH information |  |
|  |  | Subset-026-7.4.2.28 Packet Number 132: Danger for Shunting information |  |
|  |  | Subset-026-4.8.3.1.1 Exception [13] |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The Train change to L1: (because in L0/LNTC "Danger for SH information" Is not managed. The displayed mode does NOT change from SH to TR. Check on the JRU that: The message "Danger for SH information" is recorded. The transition to TR is NOT recorded. |
| Constraints | Train Trip is not inhibited. In Level 0/ NTC "Danger for SH information" is received together with a NOT immediate level transition order to L1. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 | SH |
| ERTMS/ETCS level | 0 / 1 | L0 / LNTC |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message containing packet 132 "Danger for shunting information" (Q_ASPETC = 0), packet 41 is received | I | BTM | One of the balises in BGa has a packet number 132 (Danger for shunting information) with the variable Q_ASPECT set to zero. A NOT immediate level transition to level 1 shall be also included. | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | NOT The Mode symbol "Trip" is displayed | O | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | NOT GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=7) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 132 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_ASPECT | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 41 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | FINITE VALUE | D1 |
| M_LEVELTR | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTR = 1 (NTC). |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 | SH |
| ERTMS/ETCS level | 0 / 1 | L0 / LNTC |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |