| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 3070300 (3.7.3: Extension, replacement of track description and linking information) Total: 16 Test case(s) |  |
| REF: | Subset-076-5-2-3070300 |
| ISSUE: | Version 3.2.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 2.9.0 31.03.2013 | All | First version/review | DLR |
| Version 2.9.1 23.09.2013 | All | review | DLR |
| Version 2.9.2 02.12.2013 | All | added second BG | DLR |
| Version 2.9.3 08.01.2014 | All | review | DLR |
| Version 2.9.4 10.01.2014 | All | review | DLR |
| Version 2.9.5 14.01.2014 | All | review | DLR |
| Version 2.9.6 21.01.2014 | All | review | DLR |
| Version 2.9.7 22.01.2014 | All | review | DLR |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.2.0 31.03.2017 | All | Baseline 3 1st maintenance release version - Bug fixing | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.7.3: Extension, replacement of track description and linking information |  | 3070300 |
| Test Case of Feature | The train receives a new track description or a linking information and shall replace the stored information. |  | 1 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS L2: FS, LS, OS L3: FS, LS, OS |  |  |
| Target of Test | • The train receives a new packet 21 ‘Gradient’ information and shall replace all stored Gradient Profile information from the start location of the first element of the new information. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.7.3.1 |  |
|  |  | Subset-026-3.7.3.1 b) |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |
|  |  | Subset-026-7.4.2.6 Packet Number 21: Gradient Profile |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check that: • The train is NOT at standstill. • The On-board is in Level x (1, 2 or 3), and if in L2 or L3 the train has an active communication session to the RBC. • The train overpasses a Balise group (L1) or receives a radio message 24: ‘General Message’ (L2/L3) with a Packet Number 21: ‘Gradient Profile’. • The gradient profile is updated and replaces the stored gradient profile. |
| Constraints | The train has a valid MA with a Gradient profile stored On-board. If in L2 or L3 the train has an active communication session to the RBC. The current ETCS mode is FS: although gradient profile information is accepted in all three modes applicable for this test case, it is displayed in FS mode only, so the test shall be run in FS mode. The status of the planning information is “show planning information”. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | Only in L2 / L3 |
| Movement Authority | STORED | If run in OS or LS then the MA contains a mode profile changing to FS after reception of the new gradient profile. |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Only in L2 / L3 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 / L2 / L3 | FS / OS / LS | Train is moving (V_TRAIN=40) | I | INT | The train is NOT at standstill. | L1 / L2 / L3 | FS / OS / LS | - |
| 2 | L1 | FS / OS / LS | One Balise Group message (N_TOTAL=1; NID_BG=<BGa>) containing packet 21 () and packet 255 is received | I | BTM | If in L1; the Gradient Profile is different from the previous one and shall be valid | L1 | FS / OS / LS | - |
| 3 | L1 | FS / OS / LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | L1 only | L1 | FS / OS / LS | - |
| 4 | L2 / L3 | FS / OS / LS | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 21 () is received | I | RTM | If in L2/L3; the Gradient Profile is different from the previous one and shall valid | L2 / L3 | FS / OS / LS | - |
| 5 | L2 / L3 | FS / OS / LS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | L2/L3 only | L2 / L3 | FS / OS / LS | - |
| 6 | L1 / L2 / L3 | FS / OS / LS | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=0) is recorded | O | JRU | - | L1 / L2 / L3 | FS | - |
| 7 | L1 / L2 / L3 | FS | The Mode symbol "On Sight" is removed | O | DMI | Only if starting mode was OS. | L1 / L2 / L3 | FS | - |
| 8 | L1 / L2 / L3 | FS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit22=0>) is recorded | O | JRU | Only if starting mode was OS. | L1 / L2 / L3 | FS | - |
| 9 | L1 / L2 / L3 | FS | The Mode symbol "Limited Supervision" is removed | O | DMI | Only if starting mode was LS. | L1 / L2 / L3 | FS | - |
| 10 | L1 / L2 / L3 | FS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit36=0>) is recorded | O | JRU | Only if starting mode was LS. | L1 / L2 / L3 | FS | - |
| 11 | L1 / L2 / L3 | FS | The Mode symbol "Full Supervision" is displayed | O | DMI | - | L1 / L2 / L3 | FS | - |
| 12 | L1 / L2 / L3 | FS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit26=1>) is recorded | O | JRU | Only if starting mode was OS or LS. | L1 / L2 / L3 | FS | - |
| 13 | L1 / L2 / L3 | FS | In the <planning information> window, the gradient profile is updated | O | DMI | The new gradient profile is the one received from track | L1 / L2 / L3 | FS | - |

| Step 2: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | 0 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | "BGa" | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 21 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | 0 | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 4: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | 0 | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | 0 | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | L2 / L3 |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Only in L2 / L3 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.7.3: Extension, replacement of track description and linking information |  | 3070300 |
| Test Case of Feature | The train receives a new track description or a linking information and shall replace the stored information. |  | 2 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS, TR, PT, NL L2: FS, LS, OS, TR, PT, NL L3: FS, LS, OS, TR, PT, NL |  |  |
| Target of Test | • The train receives a new track condition Change of Traction System information shall replace all stored Change of Traction System information. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.7.3.1 |  |
|  |  | Subset-026-3.7.3.1 e) |  |
|  |  | Subset-026-5.18.10.2 |  |
|  |  | Subset-026-5.18.10.3 |  |
|  |  | Subset-026-5.18.10.5 |  |
|  |  | Subset-026-5.18.10.6 |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |
|  |  | Subset-026-7.4.2.8 Packet Number 39: Track Condition Change of traction system |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check that: • The train is NOT at standstill. • The On-board is in Level x (1, 2 or 3), and if in L2 or L3 the train has an active communication session to the RBC. • The train overpasses a Balise group (L1) or receives a radio message 24: ‘General Message’ (L2/L3) with a Packet Number 39: ‘Track Condition Change of traction system’. • The Track Condition Change of traction system is updated and replaces the stored Track Condition Change of traction system, if one was stored On-board. • The train overpasses a Balise group (L1) or receives a radio message 24: ‘General Message’ (L2/L3) with a Packet Number 39: ‘Track Condition Change of traction system’. • The Track Condition Change of traction system is updated and replaces the stored Track Condition Change of traction system. ↔ if one was stored on-board |
| Constraints | If in L2 or L3 the train has an active communication session to the RBC. If in TR mode, information “recognition of exit from TR” has been received” |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 7 / 8 / 11 / 12 | FS / OS / TR / PT / NL / LS |
| Radio communication session | ESTABLISHED | Only in L2 / L3 |
| Movement Authority | STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Only in L2 / L3 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 / L2 / L3 | FS / OS / TR / PT / NL / LS | Train is moving (V_TRAIN=40) | I | INT | The train is NOT at standstill. | L1 / L2 / L3 | FS / OS / TR / PT / NL / LS | - |
| 2 | L1 | FS / OS / TR / PT / NL / LS | One Balise Group message (N_TOTAL=1; NID_BG=<BGa>) containing packet 39 () and packet 255 is received | I | BTM | If in L1; M_VOLTAGE = 1 | L1 | FS / OS / TR / PT / NL / LS | - |
| 3 | L1 | FS / OS / TR / PT / NL / LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | L1 only | L1 | FS / OS / TR / PT / NL / LS | - |
| 4 | L2 / L3 | FS / OS / TR / PT / NL / LS | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 39 () is received | I | RTM | If in L2/L3; M_VOLTAGE = 1 | L2 / L3 | FS / OS / TR / PT / NL / LS | - |
| 5 | L2 / L3 | FS / OS / TR / PT / NL / LS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | L2/L3 only | L2 / L3 | FS / OS / TR / PT / NL / LS | - |
| 6 | L1 / L2 / L3 | FS / OS / TR / PT / NL / LS | The Track Condition symbol "Change of traction system: AC 25 kV 50 Hz announcement" is displayed | O | DMI | The driver is announced about the change of the traction system When different with the stored one on-board. | L1 / L2 / L3 | FS / OS / TR / PT / NL / LS | - |
| 7 | L1 / L2 / L3 | FS / OS / TR / PT / NL / LS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit69=1>) is recorded | O | JRU | Bit69 if driver action is requested. If, on the contrary, the traction change is made automatically, Bit68 is set. | L1 / L2 / L3 | FS / OS / TR / PT / NL / LS | - |
| 8 | L1 / L2 / L3 | FS / OS / TR / PT / NL / LS | Train is moving (V_TRAIN=40) | I | INT | The train enters with its max safe front end the “Change of traction system” location. | L1 / L2 / L3 | FS / OS / TR / PT / NL / LS | - |
| 9 | L1 / L2 / L3 | FS / OS / TR / PT / NL / LS | The Track Condition symbol "Change of traction system: AC 25 kV 50 Hz announcement" is removed | O | DMI | The ‘Track Condition Change of traction system’ is updated and the “New traction system” information shall be displayed to the driver. | L1 / L2 / L3 | FS / OS / TR / PT / NL / LS | - |
| 10 | L1 / L2 / L3 | FS / OS / TR / PT / NL / LS | The Track Condition symbol "Change of traction system: AC 25 kV 50 Hz" is displayed | O | DMI | The driver announcement is deleted | L1 / L2 / L3 | FS / OS / TR / PT / NL / LS | - |
| 11 | L1 / L2 / L3 | FS / OS / TR / PT / NL / LS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit68=1; Bit69=0>) is recorded | O | JRU | This step shall executed only if bit 69 has been set at step 7” | L1 / L2 / L3 | FS / OS / TR / PT / NL / LS | - |
| 12 | L1 | FS / OS / TR / PT / NL / LS | One Balise Group message (N_TOTAL=1; NID_BG=<BGb>) containing packet 39 () and packet 255 is received | I | BTM | If in L1; M_VOLTAGE = 2 | L1 | FS / OS / TR / PT / NL / LS | - |
| 13 | L1 | FS / OS / TR / PT / NL / LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | L1 only | L1 | FS / OS / TR / PT / NL / LS | - |
| 14 | L2 / L3 | FS / OS / TR / PT / NL / LS | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 39 () is received | I | RTM | If in L2/L3; M_VOLTAGE = 2 | L2 / L3 | FS / OS / TR / PT / NL / LS | - |
| 15 | L2 / L3 | FS / OS / TR / PT / NL / LS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | L2/L3 only | L2 / L3 | FS / OS / TR / PT / NL / LS | - |
| 16 | L1 / L2 / L3 | FS / OS / TR / PT / NL / LS | The Track Condition symbol "Change of traction system: <AC25/AC15> announcement" is displayed | O | DMI | The driver is announced about the change of the traction system. When different with the stored one on-board. | L1 / L2 / L3 | FS / OS / TR / PT / NL / LS | - |
| 17 | L1 / L2 / L3 | FS / OS / TR / PT / NL / LS | Train is moving (V_TRAIN=40) | I | INT | The train enters with its max safe front end the “Change of traction system” location. | L1 / L2 / L3 | FS / OS / TR / PT / NL / LS | - |
| 18 | L1 / L2 / L3 | FS / OS / TR / PT / NL / LS | The Track Condition symbol "Change of traction system: AC 15 kV 16.7 Hz" is displayed | O | DMI | The ‘Track Condition Change of traction system’ is updated and the “New traction system” information shall be displayed to the driver. | L1 / L2 / L3 | FS / OS / TR / PT / NL / LS | - |
| 19 | L1 / L2 / L3 | FS / OS / TR / PT / NL / LS | The Track Condition symbol "Change of traction system: <AC25/AC15> announcement" is removed | O | DMI | The driver announcement is deleted | L1 / L2 / L3 | FS / OS / TR / PT / NL / LS | - |
| 20 | L1 / L2 / L3 | FS / OS / TR / PT / NL / LS | Train is moving (V_TRAIN=40) | I | INT | The minimum safe rear end of the train is leaving a track condition area | L1 / L2 / L3 | FS / OS / TR / PT / NL / LS | - |
| 21 | L1 / L2 / L3 | FS / OS / TR / PT / NL / LS | The Track Condition symbol "Change of traction system: AC 15 kV 16.7 Hz" is removed | O | DMI | 5 seconds after last step 20 | L1 / L2 / L3 | FS / OS / TR / PT / NL / LS | - |

| Step 2: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 000 | Position in the group. Defines the position of the balise in the balise group. First balise |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group two balises |
| M_DUP | 2 | 00 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | "BGa" | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 39 | Track Condition Change of traction system |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | 1 | - |
| D_TRACTION | 15 | FINITE VALUE | Change to D_TRACTION>0, e.g. D_TRACTION=100, so that the announcement can also be tested. |
| M_VOLTAGE | 4 | 1 | Identity of the traction system. 1 = AC 25 kV 50 Hz |
| NID_CTRACTION | 10 | FINITE VALUE | NID_CTRACTION given only if M_VOLTAGE ≠ 0. |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 2: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | 0 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | "BGa" | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 4: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - General message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | Trainborne Clock |
| M_ACK | 1 | 0 | No acknowledgement required |
| NID_LRBG | 24 | FINITE VALUE | Identity of last relevant balisegroup |
| NID_PACKET | 8 | 39 | Track Condition Change of traction system |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | 0 | - |
| D_TRACTION | 15 | FINITE VALUE | Change to D_TRACTION>0, e.g. D_TRACTION=100, so that the announcement can also be tested. |
| M_VOLTAGE | 4 | 1 | Identity of the traction system. 1 = AC 25 kV 50 Hz |
| NID_CTRACTION | 10 | FINITE VALUE | NID_CTRACTION given only if M_VOLTAGE ≠ 0. |

| Step 12: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. First balise |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group two balises |
| M_DUP | 2 | 0 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | "BGb" | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 39 | Track Condition Change of traction system |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | 1 | - |
| D_TRACTION | 15 | FINITE VALUE | Change to D_TRACTION>0, e.g. D_TRACTION=100, so that the announcement can also be tested. |
| M_VOLTAGE | 4 | 2 | Identity of the traction system. 2 = AC 15 kV 16.7 Hz |
| NID_CTRACTION | 10 | FINITE VALUE | NID_CTRACTION given only if M_VOLTAGE ≠ 0. |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 12: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | 0 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | "BGb" | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 14: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - General message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | Trainborne Clock |
| M_ACK | 1 | 0 | No acknowledgement required |
| NID_LRBG | 24 | FINITE VALUE | Identity of last relevant balisegroup |
| NID_PACKET | 8 | 39 | Track Condition Change of traction system |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | 1 | - |
| D_TRACTION | 15 | FINITE VALUE | Change to D_TRACTION>0, e.g. D_TRACTION=100, so that the announcement can also be tested. |
| M_VOLTAGE | 4 | 2 | Identity of the traction system. 2 = AC 15 kV 16.7 Hz |
| NID_CTRACTION | 10 | FINITE VALUE | NID_CTRACTION given only if M_VOLTAGE ≠ 0. |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 7 / 8 / 11 / 12 | FS / OS / TR / PT / NL / LS |
| Radio communication session | ESTABLISHED | Only in L2 / L3 |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Only in L2 / L3 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.7.3: Extension, replacement of track description and linking information |  | 3070300 |
| Test Case of Feature | The train receives a new track description or a linking information and shall replace the stored information. |  | 3 |
| Applicable Mode/Level Combinations | L0: SH, UN, PS, SL, SB, TR, NL LNTC: SH, PS, SL, SB, TR, NL, SN L1: FS, LS, OS, SR, SH, PS, SL, SB, TR, NL L2: FS, LS, OS, SR, SH, PS, SL, SB, TR, PT, NL L3: FS, LS, OS, SR, SH, PS, SL, SB, TR, PT, NL |  |  |
| Target of Test | • The train receives a new track condition Big Metal Masses information shall replace all stored Big Metal Masses information from the start location of the first element of the new information. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.7.3.1 |  |
|  |  | Subset-026-3.7.3.1 f) |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |
|  |  | Subset-026-7.4.2.19 Packet Number 67: Track Conditions Big Metal Masses |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check that: • The train is NOT at standstill. • The train overpasses a BG with a Packet Number 67: ‘Track Condition Big Metal Masses’. • The Track Condition Big Metal Masses is updated and replaces the stored Track Condition Big Metal Masses, if one was stored On-board. • The train overpasses a BG with a Packet Number 67: ‘Track Condition Big Metal Masses’. • The Track Condition Big Metal Masses is updated and replaces the stored Track Condition Big Metal Masses. |
| Constraints | For PT mode: - for level 2/3: only if following the reception of the information “Recognition of Exit from TR mode” with a more recent time stamp; for level 1: rejected For SB mode: - only if a cab is active |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 4 / 5 / 6 / 7 / 8 / 11 / 12 / 13 / 15 | FS / OS / SR / SH / UN / SL / SB / TR / PT / NL / LS / SN / PS |
| Movement Authority | STORED | If applicable for operated mode |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / LNTC / L1 / L2 / L3 | FS / OS / SR / SH / UN / SL / SB / TR / PT / SF / NL / LS / SN / PS | Train is moving (V_TRAIN=40) | I | INT | The train is NOT at standstill. Design test sequence as follows (variable D is the distance travelled by the train since test beginning): The train passes a big metal object from D=0 to D=400. At D=0 a packet 67 is received with D_TRACKCOND=0 and L_TRACKCOND=400 | L0 / LNTC / L1 / L2 / L3 | FS / OS / SR / SH / UN / SL / SB / TR / PT / SF / NL / LS / SN / PS | - |
| 2 | L0 / LNTC / L1 / L2 / L3 | FS / OS / SR / SH / UN / SL / SB / TR / PT / SF / NL / LS / SN / PS | One Balise Group message (N_TOTAL=1; NID_BG=<BGa>; M_DUP=0; ) containing packet 67 (; D_TRACKCOND=0; L_TRACKCOND=400; N_ITER=0) and packet 255 is received | I | BTM | - | L0 / LNTC / L1 / L2 / L3 | FS / OS / SR / SH / UN / SL / SB / TR / PT / SF / NL / LS / SN / PS | - |
| 3 | L0 / LNTC / L1 / L2 / L3 | FS / OS / SR / SH / UN / SL / SB / TR / PT / SF / NL / LS / SN / PS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L0 / LNTC / L1 / L2 / L3 | FS / OS / SR / SH / UN / SL / SB / TR / PT / SF / NL / LS / SN / PS | - |
| 4 | L0 / LNTC / L1 / L2 / L3 | FS / OS / SR / SH / UN / SL / SB / TR / PT / SF / NL / LS / SN / PS | ‘Track Condition Big Metal Masses’ is updated | I | INT | Internal | L0 / LNTC / L1 / L2 / L3 | FS / OS / SR / SH / UN / SL / SB / TR / PT / SF / NL / LS / SN / PS | - |
| 5 | L0 / LNTC / L1 / L2 / L3 | FS / OS / SR / SH / UN / SL / SB / TR / PT / SF / NL / LS / SN / PS | One Balise Group message (N_TOTAL=1; NID_BG=<BGb>; M_DUP=0; ) containing packet 67 (; D_TRACKCOND=200; L_TRACKCOND=200; N_ITER=0) and packet 255 is received | I | BTM | At D=100 a second packet 67 is received with D_TRACKCOND=200 and L_TRACKCOND=200 | L0 / LNTC / L1 / L2 / L3 | FS / OS / SR / SH / UN / SL / SB / TR / PT / SF / NL / LS / SN / PS | - |
| 6 | L0 / LNTC / L1 / L2 / L3 | FS / OS / SR / SH / UN / SL / SB / TR / PT / SF / NL / LS / SN / PS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L0 / LNTC / L1 / L2 / L3 | FS / OS / SR / SH / UN / SL / SB / TR / PT / SF / NL / LS / SN / PS | - |
| 7 | L0 / LNTC / L1 / L2 / L3 | FS / OS / SR / SH / UN / SL / SB / TR / PT / SF / NL / LS / SN / PS | ‘Track Condition Big Metal Masses’ is replaced with the newer informations | I | INT | Internal | L0 / LNTC / L1 / L2 / L3 | FS / OS / SR / SH / UN / SL / SB / TR / PT / SF / NL / LS / SN / PS | - |
| 8 | L0 / LNTC / L1 / L2 / L3 | FS / OS / SR / SH / UN / SL / SB / TR / PT / SF / NL / LS / SN / PS | NOT BTM alarm due to big metal masses is received | I | INT | Somewhere between D=400m and D=500 an air-gap error occurs. A phantom signal (as may be caused by passing a metal bridge for example) shall be simulated by sending an incoherent signal over the air-gap. The On-board Transmission Equipment is allowed to give an alarm to the ERTMS/ETCS Kernel (Subset-036-6.2.1.7). Inside the big metal mass area the ERTMS/ETCS Kernel ignores the alarm. | L0 / LNTC / L1 / L2 / L3 | FS / OS / SR / SH / UN / SL / SB / TR / PT / SF / NL / LS / SN / PS | - |

| Step 2: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. First balise |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group Two balises |
| M_DUP | 2 | 0 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | "BGa" |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 67 | Track Condition Big Metal Masses |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | 1 | - |
| D_TRACKCOND | 15 | 0 | - |
| L_TRACKCOND | 15 | 400 | The distance for which integrity check alarms of balise transmission shall be ignored. Longer as the distance to the next Position where a newer Packet number 67 is received |
| N_ITER | 5 | 0 | - |
| D_TRACKCOND(k) | 15 | FINITE VALUE | - |
| L_TRACKCOND(k) | 15 | FINITE VALUE | The distance for which integrity check alarms of balise transmission shall be ignored. |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 2: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | 0 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | "BGa" |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 5: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. First balise |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group Two balises |
| M_DUP | 2 | 0 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | "BGb" | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 67 | Track Condition Big Metal Masses |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | 1 | - |
| D_TRACKCOND | 15 | 200 | - |
| L_TRACKCOND | 15 | 200 | The distance for which integrity check alarms of balise transmission shall be ignored. Longer as the distance to the next Position where a newer Packet number 67 is received |
| N_ITER | 5 | 0 | - |
| D_TRACKCOND(k) | 15 | FINITE VALUE | - |
| L_TRACKCOND(k) | 15 | FINITE VALUE | The distance for which integrity check alarms of balise transmission shall be ignored. |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 5: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | 0 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | "BGb" |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 4 / 5 / 6 / 7 / 8 / 11 / 12 / 13 / 15 | FS / OS / SR / SH / UN / SL / SB / TR / PT / NL / LS / SN / PS |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.7.3: Extension, replacement of track description and linking information |  | 3070300 |
| Test Case of Feature | The train receives a new track description or a linking information and shall replace the stored information. |  | 4 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS, TR L2: FS, LS, OS, TR L3: FS, LS, OS, TR |  |  |
| Target of Test | • The train receives a new track condition information of at least one of the types listed here, i.e., sound horn, non stopping area, tunnel stopping area, powerless section - lower pantograph, powerless section - switch off the main power switch, radio hole, air tightness, switch off regenerative brake, switch off eddy current brake for service brake, switch off eddy current brake for emergency brake, switch off magnetic shoe brake, shall replace all stored track condition information of the listed types from the start location of the first element of the new information. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.7.3.1 |  |
|  |  | Subset-026-3.7.3.1 g) |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |
|  |  | Subset-026-7.4.2.20 Packet Number 68: Track Condition |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check that: • The train is NOT at standstill. • The On-board is in Level x (1, 2 or 3), and if in L2 or L3 the train has an active communication session to the RBC. • The train overpasses a Balise group (L1) or receives a radio message 24: ‘General Message’ (L2/L3) with a Packet Number 68: ‘Track Condition’. • The Track Condition is updated and replaces the stored Track Condition, if one was stored On-board. • The train overpasses a Balise group (L1) or receives a radio message 24: ‘General Message’ (L2/L3) with a Packet Number 68: ‘Track Condition. • The Track Condition is updated and replaces the stored Track Condition. |
| Constraints | If in L2 or L3 the train has an active communication session to the RBC. For each possible Trackcondition (Lower pantograph announcement; Neutral section announcement; non stopping area announcement; Close air conditioning intake announcement; Regenerative Brake announcement; Tunnel stopping area announcement; change of traction announcement) one iteration should be performed. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 7 / 12 | FS / OS / TR / LS |
| Radio communication session | ESTABLISHED | Only in L2 / L3 |
| Movement Authority | STORED | If applicable for operated mode |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Only in L2 / L3 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 / L2 / L3 | FS / OS / TR / LS | Train is moving (V_TRAIN=40) | I | INT | The train is NOT at standstill. | L1 / L2 / L3 | FS / OS / TR / LS | - |
| 2 | L1 / L2 / L3 | FS / OS / TR / LS | One Balise Group message (N_TOTAL=1; NID_BG=<BGa>; ) containing packet 68 (; D_TRACKCOND>0; L_TRACKCOND=FINITE VALUE), and packet 255 is received | I | BTM | If in L1 Possible announcement are: Lower pantograph announcement; Neutral section announcement; non stopping area announcement; Close air conditioning intake announcement; Regenerative Brake announcement; Tunnel stopping area announcement; change of traction announcement | L1 / L2 / L3 | FS / OS / TR / LS | - |
| 3 | L1 / L2 / L3 | FS / OS / TR / LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | If in L1 | L1 / L2 / L3 | FS / OS / TR / LS | - |
| 4 | L2 / L3 | FS / OS / TR / LS | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 68 (; D_TRACKCOND>0; L_TRACKCOND=FINITE VALUE) is received | I | RTM | If in L2/L3 Possible announcement are: Lower pantograph announcement; Neutral section announcement; non stopping area announcement; Close air conditioning intake announcement; Regenerative Brake announcement; Tunnel stopping area announcement; change of traction announcement | L2 / L3 | FS / OS / TR / LS | - |
| 5 | L2 / L3 | FS / OS / TR / LS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | If in L2/L3 | L2 / L3 | FS / OS / TR / LS | - |
| 6 | L1 / L2 / L3 | FS / OS / TR / LS | The Track Condition symbol "X" is displayed | O | DMI | These track conditions are not applicable in NL and TR mode. An announcement for track condition "X" is displayed which is one of the following cases. Possible announcement are: Lower pantograph announcement; Neutral section announcement; non stopping area announcement; Close air conditioning intake announcement; Regenerative Brake announcement; Tunnel stopping area announcement; change of traction announcement | L1 / L2 / L3 | FS / OS / TR / LS | - |
| 7 | L1 / L2 / L3 | FS / OS / TR / LS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; bit_and[DMI_SYMB_STATUS,MASK]=MASK) | O | JRU | MASK depends on the announcement | L1 / L2 / L3 | FS / OS / TR / LS | - |
| 8 | L1 / L2 / L3 | FS / OS / TR / LS | Train is moving (V_TRAIN=40) | I | INT | The train reaches the Position where the ‘Track Condition’ begins. | L1 / L2 / L3 | FS / OS / TR / LS | - |
| 9 | L1 / L2 / L3 | FS / OS / TR / LS | The Track Condition symbol "X" is removed | O | DMI | - | L1 / L2 / L3 | FS / OS / TR / LS | - |
| 10 | L1 / L2 / L3 | FS / OS / TR / LS | The Track Condition symbol "X" is displayed | O | DMI | The train reaches the Position where the ‘Track Condition’ begins. The announcement is no longer displayed. | L1 / L2 / L3 | FS / OS / TR / LS | - |
| 11 | L1 / L2 / L3 | FS / OS / TR / LS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; bit_and[DMI_SYMB_STATUS,MASK]=MASK) | O | JRU | MASK depends on the announcement | L1 / L2 / L3 | FS / OS / TR / LS | - |
| 12 | L1 / L2 / L3 | FS / OS / TR / LS | One Balise Group message (N_TOTAL=1; NID_BG=<BGb>; ) containing packet 68 (; D_TRACKCOND=0; L_TRACKCOND=500), and packet 255 is received | I | BTM | If in L1, the BG message (400m behind the last BTM message) is received and the position of the new track condition begins before the last track condition area is left. | L1 / L2 / L3 | FS / OS / TR / LS | - |
| 13 | L1 / L2 / L3 | FS / OS / TR / LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L1 / L2 / L3 | FS / OS / TR / LS | - |
| 14 | L2 / L3 | FS / OS / TR / LS | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 68 (; D_TRACKCOND=0; L_TRACKCOND=500) is received | I | RTM | If in L2/L3, the Radio Message (400m behind the last RTM message) is received and the position of the new track condition begins before the last track condition area is left. | L2 / L3 | FS / OS / TR / LS | - |
| 15 | L2 / L3 | FS / OS / TR / LS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | L2 / L3 | FS / OS / TR / LS | - |
| 16 | L1 / L2 / L3 | FS / OS / TR / LS | The Track Condition symbol "X" is displayed | O | DMI | These track conditions are not applicable in NL and TR mode. An new announcement is displayed Possible announcement are: Lower pantograph announcement; Neutral section announcement; non stopping area announcement; Close air conditioning intake announcement; Regenerative Brake announcement; Tunnel stopping area announcement; change of traction announcement | L1 / L2 / L3 | FS / OS / TR / LS | - |
| 17 | L1 / L2 / L3 | FS / OS / TR / LS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; bit_and[DMI_SYMB_STATUS,MASK]=MASK) | O | JRU | MASK depends on the announcement | L1 / L2 / L3 | FS / OS / TR / LS | - |
| 18 | L1 / L2 / L3 | FS / OS / TR / LS | The Track Condition symbol "X" is removed | O | DMI | The train reaches the Position where the newer ‘Track Condition’ begins. The announcement is no longer displayed. | L1 / L2 / L3 | FS / OS / TR / LS | - |
| 19 | L1 / L2 / L3 | FS / OS / TR / LS | The Track Condition symbol "X" is displayed | O | DMI | The train reaches the Position where the newer ‘Track Condition’ begins. The track condition is displayed | L1 / L2 / L3 | FS / OS / TR / LS | - |
| 20 | L1 / L2 / L3 | FS / OS / TR / LS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; bit_and[DMI_SYMB_STATUS,MASK]=MASK) | O | JRU | MASK depends on the announcement | L1 / L2 / L3 | FS / OS / TR / LS | - |

| Step 2: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. First balise |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group Two balises |
| M_DUP | 2 | 0 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | "BGa" |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 68 | Track Condition |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | 1 | - |
| Q_TRACKINIT | 1 | 0 | - |
| D_TRACKCOND | 15 | FINITE VALUE | Only if Q_TRACKINIT = 0, D_TRACKCOND and the following variables follow. |
| L_TRACKCOND | 15 | FINITE VALUE | Longer than the distance to the next Position where a newer Packet number 68 is received |
| M_TRACKCOND | 4 | 0000 or 0001 or 0010 or 0011 or 0100 or 0101 or 0110 or 0111 or 1000 or 1001 or 1010 | 0000 Non stopping area. Initial state: stopping permitted 0001 Tunnel stopping area. Initial state: no tunnel stopping area 0010 Sound horn. Initial state: no request for sound horn 0011 Powerless section – lower pantograph. Initial state: not powerless section 0100 Radio hole (stop supervising T_NVCONTACT). Initial state: supervise T_NVCONTACT 0101 Air tightness. Initial state: no request for air tightness 0110 Switch off regenerative brake. Initial state: regenerative brake on 0111 Switch off eddy current brake for service brake. Initial state: eddy current brake for service brake on 1000 Switch off magnetic shoe brake. Initial state: magnetic shoe brake on 1001 Powerless section – switch off the main power switch. Initial state: not powerless section 1010 Switch off eddy current brake for emergency brake. Initial state: eddy current brake for emergency brake on |
| N_ITER | 5 | FINITE VALUE | - |
| D_TRACKCOND(k) | 15 | 0 | - |
| L_TRACKCOND(k) | 15 | 500 | - |
| M_TRACKCOND(k) | 4 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 2: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | 0 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | "BGa" |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 4: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - General message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | Trainborne Clock |
| M_ACK | 1 | 0 | No acknowledgement required |
| NID_LRBG | 24 | FINITE VALUE | Identity of last relevant balisegroup |
| NID_PACKET | 8 | 68 | Track Condition |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | 1 | - |
| Q_TRACKINIT | 1 | 0 | - |
| D_TRACKCOND | 15 | 0 | Only if Q_TRACKINIT = 0, D_TRACKCOND and the following variables follow. |
| L_TRACKCOND | 15 | 500 | Longer as the distance to the next Position where a newer Packet number 68 is received |
| M_TRACKCOND | 4 | 0000 or 0001 or 0010 or 0011 or 0100 or 0101 or 0110 or 0111 or 1000 or 1001 or 1010 | 0000 Non stopping area. Initial state: stopping permitted 0001 Tunnel stopping area. Initial state: no tunnel stopping area 0010 Sound horn. Initial state: no request for sound horn 0011 Powerless section – lower pantograph. Initial state: not powerless section 0100 Radio hole (stop supervising T_NVCONTACT). Initial state: supervise T_NVCONTACT 0101 Air tightness. Initial state: no request for air tightness 0110 Switch off regenerative brake. Initial state: regenerative brake on 0111 Switch off eddy current brake for service brake. Initial state: eddy current brake for service brake on 1000 Switch off magnetic shoe brake. Initial state: magnetic shoe brake on 1001 Powerless section – switch off the main power switch. Initial state: not powerless section 1010 Switch off eddy current brake for emergency brake. Initial state: eddy current brake for emergency brake on |
| N_ITER | 5 | FINITE VALUE | - |
| D_TRACKCOND(k) | 15 | FINITE VALUE | - |
| L_TRACKCOND(k) | 15 | FINITE VALUE | - |
| M_TRACKCOND(k) | 4 | FINITE VALUE | - |

| Step 12: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. First balise |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group Two balises |
| M_DUP | 2 | 0 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | "BGb" |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 68 | Track Condition |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | 1 | - |
| Q_TRACKINIT | 1 | 0 | - |
| D_TRACKCOND | 15 | 0 | Only if Q_TRACKINIT = 0, D_TRACKCOND and the following variables follow. |
| L_TRACKCOND | 15 | 500 | Longer than the distance to the next Position where a newer Packet number 68 is received |
| M_TRACKCOND | 4 | 0000 or 0001 or 0010 or 0011 or 0100 or 0101 or 0110 or 0111 or 1000 or 1001 or 1010 | 0000 Non stopping area. Initial state: stopping permitted 0001 Tunnel stopping area. Initial state: no tunnel stopping area 0010 Sound horn. Initial state: no request for sound horn 0011 Powerless section – lower pantograph. Initial state: not powerless section 0100 Radio hole (stop supervising T_NVCONTACT). Initial state: supervise T_NVCONTACT 0101 Air tightness. Initial state: no request for air tightness 0110 Switch off regenerative brake. Initial state: regenerative brake on 0111 Switch off eddy current brake for service brake. Initial state: eddy current brake for service brake on 1000 Switch off magnetic shoe brake. Initial state: magnetic shoe brake on 1001 Powerless section – switch off the main power switch. Initial state: not powerless section 1010 Switch off eddy current brake for emergency brake. Initial state: eddy current brake for emergency brake on |
| N_ITER | 5 | FINITE VALUE | - |
| D_TRACKCOND(k) | 15 | FINITE VALUE | - |
| L_TRACKCOND(k) | 15 | FINITE VALUE | - |
| M_TRACKCOND(k) | 4 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 12: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | 0 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | "BGb" |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 14: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - General message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | Trainborne Clock |
| M_ACK | 1 | 0 | No acknowledgement required |
| NID_LRBG | 24 | FINITE VALUE | Identity of last relevant balisegroup |
| NID_PACKET | 8 | 68 | Track Condition |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | 1 | - |
| Q_TRACKINIT | 1 | 0 | - |
| D_TRACKCOND | 15 | 0 | Only if Q_TRACKINIT = 0, D_TRACKCOND and the following variables follow. |
| L_TRACKCOND | 15 | 500 | Longer as the distance to the next Position where a newer Packet number 68 is received |
| M_TRACKCOND | 4 | 0000 or 0001 or 0010 or 0011 or 0100 or 0101 or 0110 or 0111 or 1000 or 1001 or 1010 | 0000 Non stopping area. Initial state: stopping permitted 0001 Tunnel stopping area. Initial state: no tunnel stopping area 0010 Sound horn. Initial state: no request for sound horn 0011 Powerless section – lower pantograph. Initial state: not powerless section 0100 Radio hole (stop supervising T_NVCONTACT). Initial state: supervise T_NVCONTACT 0101 Air tightness. Initial state: no request for air tightness 0110 Switch off regenerative brake. Initial state: regenerative brake on 0111 Switch off eddy current brake for service brake. Initial state: eddy current brake for service brake on 1000 Switch off magnetic shoe brake. Initial state: magnetic shoe brake on 1001 Powerless section – switch off the main power switch. Initial state: not powerless section 1010 Switch off eddy current brake for emergency brake. Initial state: eddy current brake for emergency brake on |
| N_ITER | 5 | FINITE VALUE | - |
| D_TRACKCOND(k) | 15 | FINITE VALUE | - |
| L_TRACKCOND(k) | 15 | FINITE VALUE | - |
| M_TRACKCOND(k) | 4 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 7 / 12 | FS / OS / TR / LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Only in L2 / L3 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 5

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.7.3: Extension, replacement of track description and linking information |  | 3070300 |
| Test Case of Feature | The train receives a new track description or a linking information and shall replace the stored information. |  | 5 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS L2: FS, LS, OS L3: FS, LS, OS |  |  |
| Target of Test | • The train receives a new route suitability line gauge information shall replace all stored route suitability line gauge information. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.7.3.1 |  |
|  |  | Subset-026-3.7.3.1 h) |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |
|  |  | Subset-026-7.4.2.21 Packet Number 70: route suitability data |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check that: • The train is NOT at standstill. • The On-board is in Level x (1, 2 or 3), and if in L2 or L3 the train has an active communication session to the RBC. • The train overpasses a Balise group (L1) or receives a radio message 24: ‘General Message’ (L2/L3) with a Packet Number 70: ‘Route Suitability Data’. • The Route Suitability Data is updated and replaces the stored Route Suitability Data, if one was stored On-board. • The train overpasses a Balise group (L1) or receives a radio message 24: ‘General Message’ (L2/L3) with a Packet Number 70: ‘Route Suitability Data’. • The Route Suitability Data is updated and replaces the stored Route Suitability Data. |
| Constraints | If in L2 or L3 the train has an active communication session to the RBC. In the list of train data shall not loading gauge category G1 included. The loading gauge profile of the train is GA |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | Only in L2 / L3 |
| Movement Authority | STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Only in L2 / L3 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 / L2 / L3 | FS / OS / LS | Train is moving (V_TRAIN=40) | I | INT | The train is NOT at standstill | L1 / L2 / L3 | FS / OS / LS | - |
| 2 | L1 / L2 / L3 | FS / OS / LS | One Balise Group message (N_TOTAL=1; NID_BG=<BGa>) containing packet 70 (; Q_SUITABILITY=00) and packet 255 is received | I | BTM | If in L1; Q_SUITABILITY = 00 (loading gauge); An unsuitability exists from location d1 on e.g. The loading gauge profile of the train is not included in the list of loading gauges accepted by trackside | L1 / L2 / L3 | FS / OS / LS | - |
| 3 | L1 / L2 / L3 | FS / OS / LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | If in L1 | L1 / L2 / L3 | FS / OS / LS | - |
| 4 | L2 / L3 | FS / OS / LS | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 70 (; Q_SUITABILITY=00) is received | I | RTM | If in L2/L3; Q_SUITABILITY = 00 (loading gauge); An unsuitability exists from location d1 on e.g. The loading gauge profile of the train is not included in the list of loading gauges accepted by trackside | L2 / L3 | FS / OS / LS | - |
| 5 | L2 / L3 | FS / OS / LS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | If in L2/L3 | L2 / L3 | FS / OS / LS | - |
| 6 | L1 / L2 / L3 | FS / OS / LS | The system status message "Route unsuitable – loading gauge" is displayed | O | DMI | The driver shall be informed about all unsuitabilities. The closest location corresponding to the unsuitability(ies) shall be considered as both the EOA and SvL (instead of the EOA and SvL given by the MA), with no Release Speed. | L1 / L2 / L3 | FS / OS / LS | - |
| 7 | L1 / L2 / L3 | FS / OS / LS | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23, SYSTEM_STATUS_MESSAGE=<Bit 21=1>) is recorded | O | JRU | - | L1 / L2 / L3 | FS / OS / LS | - |
| 8 | L1 / L2 / L3 | FS / OS / LS | One Balise Group message (N_TOTAL=1; NID_BG=<BGb>) containing packet 70 (; Q_SUITABILITY=00) and packet 255 is received | I | BTM | If in L1; Q_SUITABILITY = 00 (loading gauge); No unsuitability exists from location d1 on. | L1 / L2 / L3 | FS / OS / LS | - |
| 9 | L1 / L2 / L3 | FS / OS / LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | If in L1 | L1 / L2 / L3 | FS / OS / LS | - |
| 10 | L2 / L3 | FS / OS / LS | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 70 (; Q_SUITABILITY=00) is received | I | RTM | If in L2/L3; Q_SUITABILITY = 00 (loading gauge); No unsuitability exists from location d1 on. | L2 / L3 | FS / OS / LS | - |
| 11 | L2 / L3 | FS / OS / LS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | If in L2/L3 | L2 / L3 | FS / OS / LS | - |
| 12 | L1 / L2 / L3 | FS / OS / LS | Train is moving | I | INT | The train overpasses the location d1. | L1 / L2 / L3 | FS / OS / LS | - |
| 13 | L1 / L2 / L3 | FS / OS / LS | NOT GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=7) is recorded | O | JRU | The train does not switch to TRIP mode. | L1 / L2 / L3 | FS / OS / LS | - |

| Step 2: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. First balise |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group Two balises |
| M_DUP | 2 | 0 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | "BGa" | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 70 | Route Suitability Data |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | 1 | - |
| Q_TRACKINIT | 1 | 0 | - |
| D_SUITABILITY | 15 | 10000 | Only If Q_TRACKINIT = 0, D_SUITABILITY and the following variables follows. |
| Q_SUITABILITY | 2 | 0 | loading gauge |
| M_LINEGAUGE | 8 | xxxx xxx1 | If Q_SUITABILITY= loading gauge. xxxx xxx1=G1 |
| N_ITER | 5 | 0 | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 2: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | 0 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | "BGa" | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 4: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - General message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | Trainborne Clock |
| M_ACK | 1 | 0 | No acknowledgement required |
| NID_LRBG | 24 | FINITE VALUE | Identity of last relevant balisegroup |
| NID_PACKET | 8 | 70 | Route Suitability Data |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | 1 | - |
| Q_TRACKINIT | 1 | 0 | - |
| D_TRACKINIT | 15 | FINITE VALUE | Only if Q_TRACKINIT = 1. |
| D_SUITABILITY | 15 | 10000 | Only If Q_TRACKINIT = 0, D_SUITABILITY and the following variables follows. To be sure that the train will not stop |
| Q_SUITABILITY | 2 | 0 | loading gauge |
| M_LINEGAUGE | 8 | xxxx xxx1 | If Q_SUITABILITY= loading gauge. xxxx xxx1 = G1 |
| N_ITER | 5 | 0 | - |

| Step 8: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. First balise |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group Two balises |
| M_DUP | 2 | 0 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | "BGb" | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 70 | Route Suitability Data |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | 1 | - |
| Q_TRACKINIT | 1 | 0 | - |
| D_SUITABILITY | 15 | 0 | Only If Q_TRACKINIT = 0, D_SUITABILITY and the following variables follows. |
| Q_SUITABILITY | 2 | 0 | loading gauge |
| M_LINEGAUGE | 8 | xxxx xx1x | If Q_SUITABILITY= loading gauge. xxxx xx1x = GA |
| N_ITER | 5 | 0 | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 8: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | 0 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | "BGb" | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 10: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - General message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | Trainborne Clock |
| M_ACK | 1 | 0 | No acknowledgement required |
| NID_LRBG | 24 | FINITE VALUE | Identity of last relevant balisegroup |
| NID_PACKET | 8 | 70 | Route Suitability Data |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | 1 | - |
| Q_TRACKINIT | 1 | 0 | - |
| D_TRACKINIT | 15 | FINITE VALUE | Only if Q_TRACKINIT = 1. |
| D_SUITABILITY | 15 | 0 | Only If Q_TRACKINIT = 0, D_SUITABILITY and the following variables follows. |
| Q_SUITABILITY | 2 | 0 | loading gauge |
| M_LINEGAUGE | 8 | xxxx xx1x | If Q_SUITABILITY= loading gauge. xxxx xx1x = GA |
| N_ITER | 5 | 0 | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Only in L2 / L3 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 6

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.7.3: Extension, replacement of track description and linking information |  | 3070300 |
| Test Case of Feature | The train receives a new track description or a linking information and shall replace the stored information. |  | 6 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS L2: FS, LS, OS L3: FS, LS, OS |  |  |
| Target of Test | • The train receives a new route suitability traction system information shall replace all stored route suitability traction system information. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.7.3.1 |  |
|  |  | Subset-026-3.7.3.1 i) |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |
|  |  | Subset-026-7.4.2.21 Packet Number 70: route suitability data |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check that: • The train is NOT at standstill. • The On-board is in Level x (1, 2 or 3), and if in L2 or L3 the train has an active communication session to the RBC. • The train overpasses a Balise group (L1) or receives a radio message 24: ‘General Message’ (L2/L3) with a Packet Number 70: ‘Route Suitability Data’. • The Route Suitability Data is updated and replaces the stored Route Suitability Data, if one was stored On-board. • The train overpasses a Balise group (L1) or receives a radio message 24: ‘General Message’ (L2/L3) with a Packet Number 70: ‘Route Suitability Data’. • The Route Suitability Data is updated and replaces the stored Route Suitability Data. |
| Constraints | If in L2 or L3 the train has an active communication session to the RBC. In train data the following support of traction systems should be stored: traction system AC 25 kV 50 Hz is not supported by the train AND traction system DC 3 kV is supported by the train |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | Only in L2 / L3 |
| Movement Authority | STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Only in L2 / L3 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 / L2 / L3 | FS / OS / LS | Train is moving (V_TRAIN=40) | I | INT | The train is NOT at standstill. | L1 / L2 / L3 | FS / OS / LS | - |
| 2 | L1 / L2 / L3 | FS / OS / LS | One Balise Group message (N_TOTAL=1; NID_BG=<BGa>) containing packet 70 (; Q_SUITABILITY=10) and packet 255 is received | I | BTM | If in L1; Q_SUITABILITY = 10 (traction system); An unsuitability exists from location d1 on e.g. The list of traction systems accepted by the engine does not include the one received from trackside | L1 / L2 / L3 | FS / OS / LS | - |
| 3 | L1 / L2 / L3 | FS / OS / LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | If in L1 | L1 / L2 / L3 | FS / OS / LS | - |
| 4 | L2 / L3 | FS / OS / LS | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 70 (; Q_SUITABILITY=10) is received | I | RTM | If in L2/L3; Q_SUITABILITY = 10 (traction system); An unsuitability exists from location d1 on e.g. The list of traction systems accepted by the engine does not include the one received from trackside | L2 / L3 | FS / OS / LS | - |
| 5 | L2 / L3 | FS / OS / LS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | If in L2/L3 | L2 / L3 | FS / OS / LS | - |
| 6 | L1 / L2 / L3 | FS / OS / LS | The system status message "Route unsuitable – traction system" is displayed | O | DMI | The driver shall be informed about all unsuitabilities. The closest location corresponding to the unsuitability(ies) shall be considered as both the EOA and SvL (instead of the EOA and SvL given by the MA), with no Release Speed. | L1 / L2 / L3 | FS / OS / LS | - |
| 7 | L1 / L2 / L3 | FS / OS / LS | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23; SYSTEM_STATUS_MESSAGE=<Bit22=1>) is recorded | O | JRU | - | L1 / L2 / L3 | FS / OS / LS | - |
| 8 | L1 / L2 / L3 | FS / OS / LS | One Balise Group message (N_TOTAL=1; NID_BG=<BGb>) containing packet 70 (; Q_SUITABILITY=10) and packet 255 is received | I | BTM | If in L1; Q_SUITABILITY = 10 (traction system); No unsuitability exists from location d1 on. | L1 / L2 / L3 | FS / OS / LS | - |
| 9 | L1 / L2 / L3 | FS / OS / LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | If in L1 | L1 / L2 / L3 | FS / OS / LS | - |
| 10 | L2 / L3 | FS / OS / LS | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 70 (; Q_SUITABILITY=10) is received | I | RTM | If in L2/L3; Q_SUITABILITY = 10 (traction system); No unsuitability exists from location d1 on. | L2 / L3 | FS / OS / LS | - |
| 11 | L2 / L3 | FS / OS / LS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | If in L2/L3; | L2 / L3 | FS / OS / LS | - |
| 12 | L1 / L2 / L3 | FS / OS / LS | Train is moving | I | INT | The train overpasses the location d1. | L1 / L2 / L3 | FS / OS / LS | - |
| 13 | L1 / L2 / L3 | FS / OS / LS | NOT GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=7) is recorded | O | JRU | The train does not switch to TRIP mode. | L1 / L2 / L3 | FS / OS / LS | - |

| Step 2: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. First balise |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group Two balises |
| M_DUP | 2 | 0 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | "BGa" |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 70 | Route Suitability Data |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | 1 | - |
| Q_TRACKINIT | 1 | 0 | - |
| D_SUITABILITY | 15 | 10000 | Only If Q_TRACKINIT = 0, D_SUITABILITY and the following variables follows. To be sure that the train will not stop |
| Q_SUITABILITY | 2 | 10 | Traction system |
| M_VOLTAGE | 4 | 1 | 1 = AC 25 kV 50 Hz (incompatible with the train) |
| NID_CTRACTION | 10 | FINITE VALUE | - |
| N_ITER | 5 | 0 | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 2: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | 0 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | "BGa" |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 4: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - General message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | Trainborne Clock |
| M_ACK | 1 | 0 | No acknowledgement required |
| NID_LRBG | 24 | FINITE VALUE | Identity of last relevant balisegroup |
| NID_PACKET | 8 | 70 | Route Suitability Data |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | 1 | - |
| Q_TRACKINIT | 1 | 0 | - |
| D_SUITABILITY | 15 | 10000 | Only If Q_TRACKINIT = 0, D_SUITABILITY and the following variables follows. To be sure that the train will not stop |
| Q_SUITABILITY | 2 | 10 | Traction system |
| M_VOLTAGE | 4 | 1 | 1 = AC 25 kV 50 Hz (incompatible with the train) |
| NID_CTRACTION | 10 | FINITE VALUE | - |
| N_ITER | 5 | 0 | - |

| Step 8: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. First balise |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group Two balises |
| M_DUP | 2 | 0 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | "BGb" |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 70 | Route Suitability Data |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | 1 | - |
| Q_TRACKINIT | 1 | 0 | - |
| D_SUITABILITY | 15 | 0 | Only If Q_TRACKINIT = 0, D_SUITABILITY and the following variables follows. |
| Q_SUITABILITY | 2 | 10 | Traction system |
| M_LINEGAUGE | 8 | FINITE VALUE | If Q_SUITABILITY= loading gauge. |
| M_AXLELOADCAT | 7 | FINITE VALUE | If Q_SUITABILITY= Max axle load. |
| M_VOLTAGE | 4 | 3 | 3 = DC 3 kV (compatible with the train) |
| NID_CTRACTION | 10 | FINITE VALUE | Shall be one compatible with the train |
| N_ITER | 5 | 0 | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 8: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | 0 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | "BGb" |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 10: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - General message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | Trainborne Clock |
| M_ACK | 1 | 0 | No acknowledgement required |
| NID_LRBG | 24 | FINITE VALUE | Identity of last relevant balisegroup |
| NID_PACKET | 8 | 70 | Route Suitability Data |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | 1 | - |
| Q_TRACKINIT | 1 | 0 | - |
| D_SUITABILITY | 15 | 0 | Only If Q_TRACKINIT = 0, D_SUITABILITY and the following variables follows. |
| Q_SUITABILITY | 2 | 10 | Traction system |
| M_VOLTAGE | 4 | 3 | 3 = DC 3 kV (compatible with the train) |
| NID_CTRACTION | 10 | FINITE VALUE | Shall be one compatible with the train |
| N_ITER | 5 | 0 | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Only in L2 / L3 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 7

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.7.3: Extension, replacement of track description and linking information |  | 3070300 |
| Test Case of Feature | The train receives a new track description or a linking information and shall replace the stored information |  | 7 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS L2: FS, LS, OS L3: FS, LS, OS |  |  |
| Target of Test | • The train receives a new route suitability axle load information shall replace all stored route suitability axle load information. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.7.3.1 |  |
|  |  | Subset-026-3.7.3.1 j) |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |
|  |  | Subset-026-7.4.2.21 Packet Number 70: route suitability data |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check that: • The train is NOT at standstill. • The On-board is in Level x (1, 2 or 3), and if in L2 or L3 the train has an active communication session to the RBC. • The train overpasses a Balise group (L1) or receives a radio message 24: ‘General Message’ (L2/L3) with a Packet Number 70: ‘Route Suitability Data’. • The Route Suitability Data is updated and replaces the stored Route Suitability Data, if one was stored On-board. • The train overpasses a Balise group (L1) or receives a radio message 24: ‘General Message’ (L2/L3) with a Packet Number 70: ‘Route Suitability Data’. • The Route Suitability Data is updated and replaces the stored Route Suitability Data. |
| Constraints | If in L2 or L3 the train has an active communication session to the RBC. In the list of train data shall not axle load category C2 included. Train axle load category is C3 |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | Only in L2 / L3 |
| Movement Authority | STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Only in L2 / L3 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 / L2 / L3 | FS / OS / LS | Train is moving (V_TRAIN=40) | I | INT | The train is NOT at standstill. | L1 / L2 / L3 | FS / OS / LS | - |
| 2 | L1 / L2 / L3 | FS / OS / LS | One Balise Group message (N_TOTAL=1; NID_BG=<BGa>) containing packet 70 (; Q_SUITABILITY=01) and packet 255 is received | I | BTM | If in L1; Q_SUITABILITY = 01 (Max axle load); An unsuitability exists from location d1 on e.g. The axle load category of the train is higher than the permitted one received from trackside | L1 / L2 / L3 | FS / OS / LS | - |
| 3 | L1 / L2 / L3 | FS / OS / LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | If in L1; | L1 / L2 / L3 | FS / OS / LS | - |
| 4 | L2 / L3 | FS / OS / LS | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 70 (; Q_SUITABILITY=01) is received | I | RTM | If in L2/L3; Q_SUITABILITY = 01 (Max axle load); An unsuitability exists from location d1 on e.g. The axle load category of the train is higher than the permitted one received from trackside | L2 / L3 | FS / OS / LS | - |
| 5 | L2 / L3 | FS / OS / LS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | If in L2/L3 | L2 / L3 | FS / OS / LS | - |
| 6 | L1 / L2 / L3 | FS / OS / LS | The system status message "Route unsuitable – axle load category" is displayed | O | DMI | The driver shall be informed about all unsuitabilities. The closest location corresponding to the unsuitability(ies) shall be considered as both the EOA and SvL (instead of the EOA and SvL given by the MA), with no Release Speed. | L1 / L2 / L3 | FS / OS / LS | - |
| 7 | L1 / L2 / L3 | FS / OS / LS | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23; SYSTEM_STATUS_MESSAGE=<Bit20=1>) is recorded | O | JRU | - | L1 / L2 / L3 | FS / OS / LS | - |
| 8 | L1 / L2 / L3 | FS / OS / LS | One Balise Group message (N_TOTAL=1; NID_BG=<BGa>) containing packet 70 (; Q_SUITABILITY=10) and packet 255 is received | I | BTM | If in L1; Q_SUITABILITY = 10 (Traction system); An unsuitability exists from location d2>d1 on e.g. The Traction system of the train is higher than the permitted one received from trackside These step is for testing the negative test for the receiption and validation of packet 70. When the train receives a packet 70 which is different (7.5.1.135 Q_SUITABILITY) from that one which has been received earlier, it has no matter for the previous received one. | L1 / L2 / L3 | FS / OS / LS | - |
| 9 | L1 / L2 / L3 | FS / OS / LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | If in L1; | L1 / L2 / L3 | FS / OS / LS | - |
| 10 | L2 / L3 | FS / OS / LS | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 70 (; Q_SUITABILITY=10) is received | I | RTM | If in L2/L3; Q_SUITABILITY = 10 (Traction system); An unsuitability exists from location d2>d1 on e.g. The Traction system of the train is higher than the permitted one received from trackside These step is for testing the negative test for the receiption and validation of packet 70. When the train receives a packet 70 which is different (7.5.1.135 Q_SUITABILITY) from that one which has been received earlier, it has no matter for the previous received one. | L2 / L3 | FS / OS / LS | - |
| 11 | L2 / L3 | FS / OS / LS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | If in L2/L3 | L2 / L3 | FS / OS / LS | - |
| 12 | L1 / L2 / L3 | FS / OS / LS | The system status message "Route unsuitable – traction system" is displayed | O | DMI | The driver shall be informed about all unsuitabilities. The closest location corresponding to the unsuitability(ies) shall be considered as both the EOA and SvL (instead of the EOA and SvL given by the MA), with no Release Speed. | L1 / L2 / L3 | FS / OS / LS | - |
| 13 | L1 / L2 / L3 | FS / OS / LS | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23; SYSTEM_STATUS_MESSAGE=<Bit22=1>) is recorded | O | JRU | Steps 8 to 13 are inserted for testing the negative test for the receiption and validation of packet 70. When the train receives a packet 70 which is different (7.5.1.135 Q_SUITABILITY) from that one which has been received earlier, it has no matter for the previous received one. Nothing will change for packet 70 with Q_SUITABILITY = 01 | L1 / L2 / L3 | FS / OS / LS | - |
| 14 | L1 / L2 / L3 | FS / OS / LS | One Balise Group message (N_TOTAL=1; NID_BG=<BGb>) containing packet 70 (; Q_SUITABILITY=01) and packet 255 is received | I | BTM | If in L1; Q_SUITABILITY = 01 (Max axle load); No unsuitability exists from location d1 on. | L1 / L2 / L3 | FS / OS / LS | - |
| 15 | L1 / L2 / L3 | FS / OS / LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | If in L1; | L1 / L2 / L3 | FS / OS / LS | - |
| 16 | L2 / L3 | FS / OS / LS | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 70 (; Q_SUITABILITY=01) is received | I | RTM | If in L2/L3; Q_SUITABILITY = 01 (Max axle load); No unsuitability exists from location d1 on. | L2 / L3 | FS / OS / LS | - |
| 17 | L2 / L3 | FS / OS / LS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | If in L2/L3 | L2 / L3 | FS / OS / LS | - |
| 18 | L1 / L2 / L3 | FS / OS / LS | Train is moving | I | INT | The train overpasses the location d1. | L1 / L2 / L3 | FS / OS / LS | - |
| 19 | L1 / L2 / L3 | FS / OS / LS | NOT GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=7) is recorded | O | JRU | The train does not switch to TRIP mode. | L1 / L2 / L3 | FS / OS / LS | - |

| Step 2: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. First balise |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group Two balises |
| M_DUP | 2 | 0 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | "BGa" |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 70 | Route Suitability Data |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | 1 | - |
| Q_TRACKINIT | 1 | 0 | - |
| D_SUITABILITY | 15 | 10000 | Only If Q_TRACKINIT = 0, D_SUITABILITY and the following variables follows. To be sure that the train will not stop |
| Q_SUITABILITY | 2 | 1 | Max axle load |
| M_AXLELOADCAT | 7 | 4 | 4=C2 |
| N_ITER | 5 | 0 | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 2: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | 0 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | "BGa" |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 4: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - General message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | Trainborne Clock |
| M_ACK | 1 | 0 | No acknowledgement required |
| NID_LRBG | 24 | FINITE VALUE | Identity of last relevant balisegroup |
| NID_PACKET | 8 | 70 | Route Suitability Data |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | 1 | - |
| Q_TRACKINIT | 1 | 0 | - |
| D_SUITABILITY | 15 | 10000 | Only If Q_TRACKINIT = 0, D_SUITABILITY and the following variables follows. To be sure that the train will not stop |
| Q_SUITABILITY | 2 | 1 | Max axle load |
| M_AXLELOADCAT | 7 | 4 | If Q_SUITABILITY= Max axle load. 4=C2 |
| N_ITER | 5 | 0 | - |

| Step 8: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | "Bgx" |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 70 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | FINITE VALUE | - |
| D_TRACKINIT | 15 | FINITE VALUE | Only if Q_TRACKINIT = 1. |
| D_SUITABILITY | 15 | FINITE VALUE | Only If Q_TRACKINIT = 0, D_SUITABILITY and the following variables follows. |
| Q_SUITABILITY | 2 | 10 | - |
| M_LINEGAUGE | 8 | FINITE VALUE | If Q_SUITABILITY= loading gauge. |
| M_AXLELOADCAT | 7 | FINITE VALUE | If Q_SUITABILITY= Max axle load. |
| M_VOLTAGE | 4 | FINITE VALUE | If Q_SUITABILITY = traction system. |
| NID_CTRACTION | 10 | FINITE VALUE | If Q_SUITABILITY = traction system and M_VOLTAGE ≠ 0. |
| N_ITER | 5 | 0 | - |

| Step 8: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | "Bgx" |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 10: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - General message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | Trainborne Clock |
| M_ACK | 1 | 0 | No acknowledgement required |
| NID_LRBG | 24 | FINITE VALUE | Identity of last relevant balisegroup |
| NID_PACKET | 8 | 70 | Route Suitability Data |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | 1 | - |
| Q_TRACKINIT | 1 | 0 | - |
| D_SUITABILITY | 15 | 0 | Only If Q_TRACKINIT = 0, D_SUITABILITY and the following variables follows. |
| Q_SUITABILITY | 2 | 10 | Traction System |
| M_AXLELOADCAT | 7 | 5 | If Q_SUITABILITY= Max axle load. 5=C3 |
| N_ITER | 5 | 0 | - |

| Step 14: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. First balise |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group Two balises |
| M_DUP | 2 | 0 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | "BGb" | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 70 | Route Suitability Data |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | 1 | - |
| Q_TRACKINIT | 1 | 0 | - |
| D_SUITABILITY | 15 | 0 | - |
| Q_SUITABILITY | 2 | 1 | Max axle load |
| M_AXLELOADCAT | 7 | 5 | 5=C3 |
| N_ITER | 5 | 0 | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 14: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | 0 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | "BGb" |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 16: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - General message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | Trainborne Clock |
| M_ACK | 1 | 0 | No acknowledgement required |
| NID_LRBG | 24 | FINITE VALUE | Identity of last relevant balisegroup |
| NID_PACKET | 8 | 70 | Route Suitability Data |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | 1 | - |
| Q_TRACKINIT | 1 | 0 | - |
| D_SUITABILITY | 15 | 0 | Only If Q_TRACKINIT = 0, D_SUITABILITY and the following variables follows. |
| Q_SUITABILITY | 2 | 1 | Max axle load |
| M_AXLELOADCAT | 7 | 5 | If Q_SUITABILITY= Max axle load. 5=C3 |
| N_ITER | 5 | 0 | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Only in L2 / L3 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 8

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.7.3: Extension, replacement of track description and linking information |  | 3070300 |
| Test Case of Feature | The train receives a new track description or a linking information and shall replace the stored information. |  | 8 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS, SR, SB L2: FS, LS, OS, SR, SB, PT L3: FS, LS, OS, SR, SB, PT |  |  |
| Target of Test | • The train receives a new adhesion factor information shall replace all stored adhesion factor information from the start location of the new information. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.7.3.1 |  |
|  |  | Subset-026-3.7.3.1 l) |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |
|  |  | Subset-026-7.4.2.22 Packet Number 71: Adhesion factor |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check that: • The train is NOT at standstill. • The On-board is in Level x (1, 2 or 3), and if in L2 or L3 the train has an active communication session to the RBC. • The train overpasses a Balise group (L1) or receives a radio message 24: ‘General Message’ (L2/L3) with a Packet Number 71: ‘Adhesion factor’. • The Adhesion factor is updated and replaces the stored Adhesion factor, if one was stored On-board. • The train overpasses a Balise group (L1) or receives a radio message 24: ‘General Message’ (L2/L3) with a Packet Number 71: ‘Adhesion factor’. • The Adhesion factor is updated and replaced by “adhesion factor information from the start location of the new information ↔ if one was stored on-board |
| Constraints | If in L2 or L3 the train has an active communication session to the RBC. For PT mode: - for level 2/3: only if following the reception of the information “Recognition of Exit from TR mode” with a more recent time stamp; for level 1: rejected For SB mode: - only if a cab is active - for order to establish a communication session: RBC ID/phone number is stored without establishing the communication session |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 8 / 12 | FS / OS / SR / SB / PT / LS |
| Radio communication session | ESTABLISHED | Only in L2 / L3 |
| Movement Authority | STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Only in L2 / L3 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / LS / SN / RV | Train is moving (V_TRAIN=40) | I | INT | The train is NOT at standstill. | L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / LS / SN / RV | - |
| 2 | L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / LS / SN / RV | One Balise Group message (N_TOTAL=1; NID_BG=<BGv>) containing packet 71 () and packet 255 is received | I | BTM | If in L1 | L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / LS / SN / RV | - |
| 3 | L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / LS / SN / RV | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | If in L1 | L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / LS / SN / RV | - |
| 4 | L2 / L3 | FS / OS / SR / UN / SB / TR / PT / LS / SN / RV | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 71 () is received | I | RTM | If in L2/L3 | L2 / L3 | FS / OS / SR / UN / SB / TR / PT / LS / SN / RV | - |
| 5 | L2 / L3 | FS / OS / SR / UN / SB / TR / PT / LS / SN / RV | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | If in L2/L3 | L2 / L3 | FS / OS / SR / UN / SB / TR / PT / LS / SN / RV | - |
| 6 | L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / LS / SN / RV | One Balise Group message (N_TOTAL=1; NID_BG=<BGa>) containing packet 71 () and packet 255 is received | I | BTM | If in L1 The new adhesion factor information will replace the stored one of the last BG | L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / LS / SN / RV | - |
| 7 | L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / LS / SN / RV | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | If in L1 | L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / LS / SN / RV | - |
| 8 | L2 / L3 | FS / OS / SR / UN / SB / TR / PT / LS / SN / RV | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 71 () is received | I | RTM | If in L2/L3 The new adhesion factor information will replace the stored one of the last Radio message | L2 / L3 | FS / OS / SR / UN / SB / TR / PT / LS / SN / RV | - |
| 9 | L2 / L3 | FS / OS / SR / UN / SB / TR / PT / LS / SN / RV | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | If in L2/L3 | L2 / L3 | FS / OS / SR / UN / SB / TR / PT / LS / SN / RV | - |
| 10 | L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / LS / SN / RV | The Status symbol "Adhesion factor - slippery rail" is displayed | O | DMI | The driver is informed, that the adhesion factor is “slippery rail” | L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / LS / SN / RV | - |
| 11 | L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / LS / SN / RV | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit39=1>) is recorded | O | JRU | - | L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / LS / SN / RV | - |
| 12 | L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / LS / SN / RV | One Balise Group message (N_TOTAL=1; NID_BG=<BGb>) containing packet 71 () and packet 255 is received | I | BTM | If in L1 | L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / LS / SN / RV | - |
| 13 | L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / LS / SN / RV | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | If in L1 | L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / LS / SN / RV | - |
| 14 | L2 / L3 | FS / OS / SR / UN / SB / TR / PT / LS / SN / RV | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 71 () is received | I | RTM | If in L2/L3 | L2 / L3 | FS / OS / SR / UN / SB / TR / PT / LS / SN / RV | - |
| 15 | L2 / L3 | FS / OS / SR / UN / SB / TR / PT / LS / SN / RV | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | If in L2/L3 | L2 / L3 | FS / OS / SR / UN / SB / TR / PT / LS / SN / RV | - |
| 16 | L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / LS / SN / RV | The Status symbol "Adhesion factor - slippery rail" is removed | O | DMI | - | L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / LS / SN / RV | - |
| 17 | L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / LS / SN / RV | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit39=0>) is recorded | O | JRU | - | L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / LS / SN / RV | - |
| 18 | L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / LS / SN / RV | NOT The Status symbol "Adhesion factor - slippery rail" is displayed | O | DMI | NOT The driver is informed, that the adhesion factor is “slippery rail” | L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / LS / SN / RV | - |

| Step 2: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. First balise |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group Two balises |
| M_DUP | 2 | 0 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | "BGv" |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 71 | Adhesion factor |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | 1 | - |
| D_ADHESION | 15 | 1200 | - |
| L_ADHESION | 15 | 1000 | Longer as the distance to the next position where the new adhesion factor shall be valid |
| M_ADHESION | 1 | 0 | Slippery rail |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 2: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | 0 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | "BGv" |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 4: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - General Message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | Trainborne Clock |
| M_ACK | 1 | 0 | No acknowledgement required |
| NID_LRBG | 24 | FINITE VALUE | Identity of last relevant balisegroup |
| NID_PACKET | 8 | 71 | Adhesion factor |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | 1 | - |
| D_ADHESION | 15 | 1200 | - |
| L_ADHESION | 15 | 1000 | Longer as the distance to the next position where the new adhesion factor shall be valid |
| M_ADHESION | 1 | 0 | Slippery rail |

| Step 6: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. First balise |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group Two balises |
| M_DUP | 2 | 0 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | "BGa" |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 71 | Adhesion factor |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | 1 | - |
| D_ADHESION | 15 | 0 | - |
| L_ADHESION | 15 | 1000 | Longer as the distance to the next position where the new adhesion factor shall be valid |
| M_ADHESION | 1 | 0 | Slippery rail |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 6: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | 0 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | "BGa" |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 8: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - General Message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | Trainborne Clock |
| M_ACK | 1 | 0 | No acknowledgement required |
| NID_LRBG | 24 | FINITE VALUE | Identity of last relevant balisegroup |
| NID_PACKET | 8 | 71 | Adhesion factor |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | 1 | - |
| D_ADHESION | 15 | 0 | - |
| L_ADHESION | 15 | 1000 | Longer as the distance to the next position where the new adhesion factor shall be valid |
| M_ADHESION | 1 | 0 | Slippery rail |

| Step 12: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. First balise |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group Two balises |
| M_DUP | 2 | 0 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | "BGb" | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 71 | Adhesion factor |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | 1 | - |
| D_ADHESION | 15 | 0 | - |
| L_ADHESION | 15 | 1000 | - |
| M_ADHESION | 1 | 1 | Non slippery rail |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 12: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | 0 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | "BGb" |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 14: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - General Message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | Trainborne Clock |
| M_ACK | 1 | 0 | No acknowledgement required |
| NID_LRBG | 24 | FINITE VALUE | Identity of last relevant balisegroup |
| NID_PACKET | 8 | 71 | Adhesion factor |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | 1 | - |
| D_ADHESION | 15 | 0 | - |
| L_ADHESION | 15 | 1000 | - |
| M_ADHESION | 1 | 1 | Non slippery rail |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 8 / 12 | FS / OS / SR / SB / PT / LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Only in L2 / L3 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 9

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.7.3: Extension, replacement of track description and linking information |  | 3070300 |
| Test Case of Feature | The train receives a new track description or a linking information and shall replace the stored information. |  | 9 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS L2: FS, LS, OS L3: FS, LS, OS |  |  |
| Target of Test | The train receives a new linking information received as non-infill information shall replace all stored linking information from the LRBG. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.7.3.1 |  |
|  |  | Subset-026-3.7.3.1 m) |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |
|  |  | Subset-026-7.4.2.2 Packet Number 5: Linking |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check that: • The train is NOT at standstill. • The On-board is in Level x (1, 2 or 3), and if in L2 or L3 the train has an active communication session to the RBC. • The train overpasses a Balise group (L1) or receives a radio message 24: ‘General Message’ (L2/L3) with a Packet Number 5: ‘Linking’. • The Linking is updated and replaces the stored Linking, if one was stored On-board. The linking packet is announcing the next three Bg’s in front of the train. • The train overpasses a Balise group (L1, first one of the stored Linking packet) or receives a radio message 24: ‘General Message’ (L2/L3) with a Packet Number 5: ‘Linking’. • The Linking is updated and replaces the stored Linking. The linking packet is announcing only the last Bg of the last Linking package. |
| Constraints | If in L2 or L3 the train has an active communication session to the RBC. And the train has a valid LRBG. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | Only in L2 / L3 |
| Movement Authority | STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Only in L2 / L3 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 / L2 / L3 | FS / OS / LS | Train is moving (V_TRAIN=40) | I | INT | The train is NOT at standstill. | L1 / L2 / L3 | FS / OS / LS | - |
| 2 | L1 / L2 / L3 | FS / OS / LS | One Balise Group message (N_TOTAL=1; NID_BG=<BGa>) containing packet 5 () and packet 255 is received | I | BTM | If in L1; BGa with linking informations to BGb, BGc and BGd; For BGb and BGc Q_LINKREACTION = 01 (SB) and for BGd Q_LINKREACTION = 10 (NO Reaction) | L1 / L2 / L3 | FS / OS / LS | - |
| 3 | L1 / L2 / L3 | FS / OS / LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | If in L1 | L1 / L2 / L3 | FS / OS / LS | - |
| 4 | L2 / L3 | FS / OS / LS | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 5 () is received | I | RTM | If in L2/L3; with linking informations to BGc and BGd; For BGc Q_LINKREACTION = 01 (SB) and for BGd Q_LINKREACTION = 10 (NO Reaction) | L2 / L3 | FS / OS / LS | - |
| 5 | L2 / L3 | FS / OS / LS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | If in L2/L3 | L2 / L3 | FS / OS / LS | - |
| 6 | L1 / L2 / L3 | FS / OS / LS | One Balise Group message (N_TOTAL=1; NID_BG=<BGb>) containing packet 5 () and packet 255 is received | I | BTM | If in L1; BGb with linking informations to BGd and no more for BGc; for BGd Q_LINKREACTION = 10 (NO Reaction) | L1 / L2 / L3 | FS / OS / LS | - |
| 7 | L1 / L2 / L3 | FS / OS / LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | If in L1 | L1 / L2 / L3 | FS / OS / LS | - |
| 8 | L2 / L3 | FS / OS / LS | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 5 () is received | I | RTM | If in L2/L3; with linking informations to BGd and no more for BGc; for BGd Q_LINKREACTION = 10 (NO Reaction) | L2 / L3 | FS / OS / LS | - |
| 9 | L2 / L3 | FS / OS / LS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | If in L2/L3 | L2 / L3 | FS / OS / LS | - |
| 10 | L1 / L2 / L3 | FS / OS / LS | Train is moving (V_TRAIN=40) | I | INT | The train overpasses the position where BGc was announced by the linking packet of BGa, but no more by BGb . | L1 / L2 / L3 | FS / OS / LS | - |
| 11 | L1 / L2 / L3 | FS / OS / LS | NOT The system status message "Balise read error" is displayed | O | DMI | The train reaches the position of the BGc which was expected by the first packet 5 in step 2/4. No linking reaction due to BGc no longer being included in Linking packet and Interface to TIU and Comments to Neither SB nor EB are applied Nothing is displayed concerning BG no longer being included in Linking packet | L1 / L2 / L3 | FS / OS / LS | - |
| 12 | L1 / L2 / L3 | FS / OS / LS | NOT Service brake commanded | O | TIU | - | L1 / L2 / L3 | FS / OS / LS | - |
| 13 | L1 / L2 / L3 | FS / OS / LS | NOT The Status symbol "Service Brake or Emergency Brake intervention" is displayed | O | DMI | - | L1 / L2 / L3 | FS / OS / LS | - |
| 14 | L1 / L2 / L3 | FS / OS / LS | Train is moving (V_TRAIN=40) | I | INT | The train overpasses the position where BGd was announced by the linking packet of BGa and BGb. | L1 / L2 / L3 | FS / OS / LS | - |
| 15 | L1 / L2 / L3 | FS / OS / LS | NOT The system status message "Balise read error" is displayed | O | DMI | The train reaches the position of the BGd which was expected by the first packet 5 in step 2/4 and again by the second packet 5 in step 6/8 Nothing is displayed concerning Q_LINKREACTION = 10 (No Reaction) | L1 / L2 / L3 | FS / OS / LS | - |
| 16 | L1 / L2 / L3 | FS / OS / LS | NOT Service brake commanded | O | TIU | - | L1 / L2 / L3 | FS / OS / LS | - |
| 17 | L1 / L2 / L3 | FS / OS / LS | NOT The Status symbol "Service Brake or Emergency Brake intervention" is displayed | O | DMI | - | L1 / L2 / L3 | FS / OS / LS | - |

| Step 2: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. First balise |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group Two balises |
| M_DUP | 2 | 0 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | "BGa" |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 5 | Linking |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | 1 | - |
| D_LINK | 15 | 500 | - |
| Q_NEWCOUNTRY | 1 | 0 | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | "BGb" |
| Q_LINKORIENTATION | 1 | 1 | The balise group is seen by the train in nominal direction |
| Q_LINKREACTION | 2 | 1 | Apply service brake |
| Q_LOCACC | 6 | 20 | Accuracy of the balise location |
| N_ITER | 5 | 2 | - |
| D_LINK(k) | 15 | 500 | - |
| Q_NEWCOUNTRY(k) | 1 | 0 | - |
| NID_BG(k) | 14 | FINITE VALUE | "BGc" |
| Q_LINKORIENTATION(k) | 1 | 1 | The balise group is seen by the train in nominal direction |
| Q_LINKREACTION(k) | 2 | 1 | Apply service brake |
| Q_LOCACC(k) | 6 | 20 | Accuracy of the balise location |
| D_LINK(k) | 15 | 500 | - |
| Q_NEWCOUNTRY(k) | 1 | 0 | - |
| NID_BG(k) | 14 | FINITE VALUE | "BGd" |
| Q_LINKORIENTATION(k) | 1 | 1 | The balise group is seen by the train in nominal direction |
| Q_LINKREACTION(k) | 2 | 10 | No Reaction |
| Q_LOCACC(k) | 6 | 20 | Accuracy of the balise location |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 2: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | 0 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | "BGa" |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 4: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - General Message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | Trainborne Clock |
| M_ACK | 1 | 0 | No acknowledgement required |
| NID_LRBG | 24 | FINITE VALUE | Identity of last relevant balisegroup |
| NID_PACKET | 8 | 5 | Linking |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | 1 | - |
| D_LINK | 15 | 500 | - |
| Q_NEWCOUNTRY | 1 | 0 | - |
| NID_BG | 14 | FINITE VALUE | "BGc" |
| Q_LINKORIENTATION | 1 | 1 | The balise group is seen by the train in nominal direction |
| Q_LINKREACTION | 2 | 1 | Apply service brake |
| Q_LOCACC | 6 | 20 | Accuracy of the balise location |
| N_ITER | 5 | 1 | - |
| D_LINK(k) | 15 | 500 | - |
| Q_NEWCOUNTRY(k) | 1 | 0 | - |
| NID_BG(k) | 14 | FINITE VALUE | "BGd" |
| Q_LINKORIENTATION(k) | 1 | 1 | The balise group is seen by the train in nominal direction |
| Q_LINKREACTION(k) | 2 | 10 | No Reaction |
| Q_LOCACC(k) | 6 | 20 | Accuracy of the balise location |

| Step 6: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. First balise |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group Two balises |
| M_DUP | 2 | 0 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | "BGb" |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 5 | Linking |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | 1 | - |
| D_LINK | 15 | 1000 | - |
| Q_NEWCOUNTRY | 1 | 0 | - |
| NID_BG | 14 | FINITE VALUE | "BGd" |
| Q_LINKORIENTATION | 1 | 1 | The balise group is seen by the train in nominal direction |
| Q_LINKREACTION | 2 | 10 | No Reaction |
| Q_LOCACC | 6 | 20 | Accuracy of the balise location |
| N_ITER | 5 | 0 | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 6: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | 0 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | "BGb" |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 8: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - General Message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | Trainborne Clock |
| M_ACK | 1 | 0 | No acknowledgement required |
| NID_LRBG | 24 | FINITE VALUE | Identity of last relevant balisegroup |
| NID_PACKET | 8 | 5 | Linking |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | 1 | - |
| D_LINK | 15 | FINITE VALUE | depends on LRBG |
| Q_NEWCOUNTRY | 1 | 0 | - |
| NID_BG | 14 | "BGd" | - |
| Q_LINKORIENTATION | 1 | 1 | The balise group is seen by the train in nominal direction |
| Q_LINKREACTION | 2 | 10 | No reaction |
| Q_LOCACC | 6 | 20 | Accuracy of the balise location |
| N_ITER | 5 | 0 | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 2 / 3 / 3 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Only in L2 / L3 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 10

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.7.3: Extension, replacement of track description and linking information |  | 3070300 |
| Test Case of Feature | When requested by trackside, the ERTMS/ETCS on-board equipment shall resume initial states beyond a given location individually |  | 10 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS, L2: FS, LS, OS, L3: FS, LS, OS, |  |  |
| Target of Test | • When requested by trackside, the ERTMS/ETCS on-board equipment shall resume initial states beyond a given location individually for stored axle load speed profile information - A given non-continuous ASP shall be interpreted according Paragraph 3.6.3.2.4. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.7.3.1 |  |
|  |  | Subset-026-3.7.3.1 c) |  |
|  |  | Subset-026-3.7.3.2 |  |
|  |  | Subset-026-3.7.3.2 b) |  |
|  |  | Subset-026-3.11.4.5 |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |
|  |  | Subset-026-7.4.2.13 Packet Number 51: axle load speed profile |  |
|  |  | Subset-026-3.6.3.2.4 a) |  |
|  |  | Subset-026-3.6.3.2.4 b) |  |
|  |  | Subset-026-3.6.3.2.4 c) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check that: • The train is NOT at standstill. • The On-board is in Level x (1, 2 or 3), and if in L2 or L3 the train has an active communication session to the RBC. • The train overpasses a Balise group (L1) or receives a radio message 24: ‘General Message’ (L2/L3) with a Packet Number 51: ‘Axle Load Speed Profile’. • The Axle Load Speed Profile is updated and replaces the stored Axle Load Speed Profile, if one was stored On-board. • The train overpasses a Balise group (L1) or receives a radio message 24: ‘General Message’ (L2/L3) with a Packet Number 51: ‘Axle Load Speed Profile’. With Q_TRACKINIT = ‘1’ • The Axle Load Speed Profile is updated and replaces it with its initial state for Axle load Speed Profile which shall be “no restriction due to axle load”. ↔ if one was stored on-board |
| Constraints | If in L2 or L3 the train has an active communication session to the RBC. Train axle load category is D2 (M_AXLELOADCAT=7) For PT mode: - for level 2/3: only if following the reception of the information “Recognition of Exit from TR mode” with a more recent time stamp; for level 1: rejected For SB mode: - only if a cab is active - for order to establish a communication session: RBC ID/phone number is stored without establishing the communication session |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 8 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | Only in L2 / L3 |
| Movement Authority | STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Only in L2 / L3 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 / L2 / L3 | FS / OS / LS / | Train is moving (V_TRAIN=40) | I | INT | The train is NOT at standstill. | L1 / L2 / L3 | FS / OS / LS / | - |
| 2 | L1 / L2 / L3 | FS / OS / LS / | One Balise Group message (N_TOTAL=1; NID_BG=<BGa>) containing packet 51 (Q_TRACKINIT=0; N_ITER>0) and packet 255 is received | I | BTM | If in L1; Q_TRACKINIT = 0 ; Ensure N_ITER>1 (second N_ITER variable in packet 51) and ensure the ASP to include non-continuous data. I.e. there have to be parts of the track without given ASP-Restriction to check that the OBU uses the default or previously given data here. | L1 / L2 / L3 | FS / OS / LS / | - |
| 3 | L1 / L2 / L3 | FS / OS / LS / | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | If in L1 | L1 / L2 / L3 | FS / OS / LS / | - |
| 4 | L2 / L3 | FS / OS / LS / | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 51 (Q_TRACKINIT=0; N_ITER>1) is received | I | RTM | If in L2/L3; Q_TRACKINIT = 0; Ensure N_ITER>1 (second N_ITER variable in packet 51) and ensure the ASP to include non-continuous data. I.e. there have to be parts of the track without given ASP-Restriction to check that the OBU uses the default or previously given data here. | L2 / L3 | FS / OS / LS / | - |
| 5 | L2 / L3 | FS / OS / LS / | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | If in L2/L3 | L2 / L3 | FS / OS / LS / | - |
| 6 | L2 / L3 | FS / OS / LS / | The speed and distance monitoring information is displayed | O | DMI | The permitted speed is updated and equal to the ‘Axle Load Speed Profile’ When different with the stored one onboard. As N_ITER in the received Axle load profile has to be greater than 1 the DMI shows all values of the ASP in the planning area, including the default/privious values in that parts of the area where no ASP is given (non-continuous). This Step does not apply in LS mode. | L1 / L2 / L3 | FS / OS / LS / | - |
| 7 | L1 / L2 / L3 | FS / OS / LS / | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | - | L1 / L2 / L3 | FS / OS / LS / | - |
| 8 | L1 / L2 / L3 | FS / OS / LS / | One Balise Group message (N_TOTAL=1; NID_BG=<BGb>) containing packet 51 (; Q_TRACKINIT=1) and packet 255 is received | I | BTM | If in L1; Q_TRACKINIT = 1; The difference of the position between the first and the second packet 51 is less than the value of L_AXLELOAD of the first packet 51 | L1 / L2 / L3 | FS / OS / LS / | - |
| 9 | L1 / L2 / L3 | FS / OS / LS / | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | If in L1 | L1 / L2 / L3 | FS / OS / LS / | - |
| 10 | L2 / L3 | FS / OS / LS / | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 51 (; Q_TRACKINIT=1) is received | I | RTM | If in L2/L3; Q_TRACKINIT = 1; The difference of the position between the first and the second packet 51 is less than L_AXLELOAD of the first packet 51 | L2 / L3 | FS / OS / LS / | - |
| 11 | L2 / L3 | FS / OS / LS / | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | If in L2/L3 | L2 / L3 | FS / OS / LS / | - |
| 12 | L1 / L2 / L3 | FS / OS / LS / | The speed and distance monitoring information is updated | O | DMI | The Axle Load Speed Profile is updated and replaces it with its initial state for Axle load Speed Profile which shall be “no restriction due to axle load”. | L1 / L2 / L3 | FS / OS / LS / | - |
| 13 | L1 / L2 / L3 | FS / OS / LS / | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | - | L1 / L2 / L3 | FS / OS / LS / | - |

| Step 2: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. First balise |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group Two balises |
| M_DUP | 2 | 0 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | "BGa" | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 51 | Axle Load Speed Profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | 1 | - |
| Q_TRACKINIT | 1 | 0 | - |
| D_AXLELOAD | 15 | FINITE VALUE | Only if Q_TRACKINIT = 0, D_AXLELOAD and the following variables follow. |
| L_AXLELOAD | 15 | 1000 | - |
| Q_FRONT | 1 | 1 | - |
| N_ITER | 5 | 1 | - |
| M_AXLELOADCAT(n) | 7 | 4 | 7=D2 |
| V_AXLELOAD(n) | 7 | FINITE VALUE | Speed restriction to be applied if the axle load category of the train ( M_AXLELOADCAT(n). Less than the permitted speed |
| N_ITER | 5 | FINITE VALUE | - |
| D_AXLELOAD(k) | 15 | FINITE VALUE | - |
| L_AXLELOAD(k) | 15 | FINITE VALUE | - |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| M_AXLELOADCAT(k,m) | 7 | FINITE VALUE | - |
| V_AXLELOAD(k,m) | 7 | FINITE VALUE | Speed restriction to be applied if the axle load category of the train ( M_AXLELOADCAT(k,m). |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 2: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | 0 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | "BGa" | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 4: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General Message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | Trainborne Clock |
| M_ACK | 1 | 0 | No acknowledgement required |
| NID_LRBG | 24 | FINITE VALUE | Identity of last relevant balisegroup |
| NID_PACKET | 8 | 51 | Axle Load Speed Profile’ |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | 1 | - |
| Q_TRACKINIT | 1 | 0 | - |
| D_AXLELOAD | 15 | FINITE VALUE | Only if Q_TRACKINIT = 0, D_AXLELOAD and the following variables follow. |
| L_AXLELOAD | 15 | 1000 | - |
| Q_FRONT | 1 | 1 | - |
| N_ITER | 5 | 1 | - |
| M_AXLELOADCAT(n) | 7 | 4 | 4=C2 |
| V_AXLELOAD(n) | 7 | FINITE VALUE | Speed restriction to be applied if the axle load category of the train ( M_AXLELOADCAT(n). Less than the permitted speed |
| N_ITER | 5 | FINITE VALUE | - |
| D_AXLELOAD(k) | 15 | FINITE VALUE | - |
| L_AXLELOAD(k) | 15 | FINITE VALUE | - |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| M_AXLELOADCAT(k,m) | 7 | FINITE VALUE | - |
| V_AXLELOAD(k,m) | 7 | FINITE VALUE | Speed restriction to be applied if the axle load category of the train ( M_AXLELOADCAT(k,m). |

| Step 8: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. First balise |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group Two balises |
| M_DUP | 2 | 0 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | "BGb" | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 51 | Axle Load Speed Profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | 1 | - |
| Q_TRACKINIT | 1 | 1 | “no restriction due to axle load” |
| D_TRACKINIT | 15 | FINITE VALUE | Only if Q_TRACKINIT = 1 |
| D_AXLELOAD | 15 | 0 | Only if Q_TRACKINIT = 0, D_AXLELOAD and the following variables follow. |
| L_AXLELOAD | 15 | FINITE VALUE | - |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | 0 | - |
| M_AXLELOADCAT(n) | 7 | FINITE VALUE | - |
| V_AXLELOAD(n) | 7 | FINITE VALUE | Speed restriction to be applied if the axle load category of the train ( M_AXLELOADCAT(n). |
| N_ITER | 5 | FINITE VALUE | - |
| D_AXLELOAD(k) | 15 | FINITE VALUE | - |
| L_AXLELOAD(k) | 15 | FINITE VALUE | - |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| M_AXLELOADCAT(k,m) | 7 | FINITE VALUE | - |
| V_AXLELOAD(k,m) | 7 | FINITE VALUE | Speed restriction to be applied if the axle load category of the train ( M_AXLELOADCAT(k,m). |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 8: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | 0 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | "BGb" | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 10: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - General Message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | Trainborne Clock |
| M_ACK | 1 | 0 | No acknowledgement required |
| NID_LRBG | 24 | FINITE VALUE | Identity of last relevant balisegroup |
| NID_PACKET | 8 | 51 | Axle Load Speed Profile’ |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | 1 | - |
| Q_TRACKINIT | 1 | 1 | no restriction due to axle load” |
| D_TRACKINIT | 15 | FINITE VALUE | Only if Q_TRACKINIT = 1 |
| L_AXLELOAD | 15 | FINITE VALUE | - |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_AXLELOADCAT(n) | 7 | FINITE VALUE | - |
| V_AXLELOAD(n) | 7 | FINITE VALUE | Speed restriction to be applied if the axle load category of the train ( M_AXLELOADCAT(n). |
| N_ITER | 5 | FINITE VALUE | - |
| D_AXLELOAD(k) | 15 | FINITE VALUE | - |
| L_AXLELOAD(k) | 15 | FINITE VALUE | - |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| M_AXLELOADCAT(k,m) | 7 | FINITE VALUE | - |
| V_AXLELOAD(k,m) | 7 | FINITE VALUE | Speed restriction to be applied if the axle load category of the train ( M_AXLELOADCAT(k,m). |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 8 / 12 | FS / OS / SR / SB / PT / LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Only in L2 / L3 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 11

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.7.3: Extension, replacement of track description and linking information |  | 3070300 |
| Test Case of Feature | When requested by trackside, the ERTMS/ETCS on-board equipment shall resume initial states beyond a given location individually. |  | 11 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS, TR, NL L2: FS, LS, OS, TR, PT, NL L3: FS, LS, OS, TR, PT, NL |  |  |
| Target of Test | • When requested by trackside, the ERTMS/ETCS on-board equipment shall resume initial states beyond a given location individually through a single request, for all stored track condition information of the following types: sound horn, non stopping area, tunnel stopping area, powerless section – lower pantograph, powerless section – switch off the main power switch, radio hole, air tightness, switch off regenerative brake, switch off eddy current brake for service brake, switch off eddy current brake for emergency brake and switch off magnetic shoe brake (for initial states, refer to 3.12.1.3) |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.7.3.2 |  |
|  |  | Subset-026-3.7.3.2 c) |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |
|  |  | Subset-026-7.4.2.20 Packet Number 68: Track Condition |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check that: • The train is NOT at standstill. • The On-board is in Level x (1, 2 or 3), and if in L2 or L3 the train has an active communication session to the RBC. • The train overpasses a Balise group (L1) or receives a radio message 24: ‘General Message’ (L2/L3) with a Packet Number 68: ‘Track Condition’. • The Track Condition is updated and replaces the stored Track Condition, if one was stored On-board. • The train overpasses a Balise group (L1) or receives a radio message 24: ‘General Message’ (L2/L3) with a Packet Number 68: ‘Track Condition. With Q_TRACKINIT = ‘1’ • The Track Condition is updated and replaces it with its initial state. |
| Constraints | If in L2 or L3 the train has an active communication session to the RBC. For each possible Track condition (Lower pantograph announcement; Neutral section announcement; non stopping area announcement; Close air conditioning intake announcement; Regenerative Brake announcement; Tunnel stopping area announcement; change of traction announcement) one iteration should be performed. For PT mode: - for level 2/3: only if following the reception of the information “Recognition of Exit from TR mode” with a more recent time stamp; for level 1: rejected |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 7 / 8 / 9 / 11 / 12 | FS / OS / TR / PT / NL / LS |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | STORED | if applicable for current mode |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Only in L2 / L3 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 / L2 / L3 | FS / OS / TR / PT / NL / LS | Train is moving (V_TRAIN=40) | I | INT | The train is NOT at standstill. | L1 / L2 / L3 | FS / OS / TR / PT / NL / LS | - |
| 2 | L1 / L2 / L3 | FS / OS / TR / PT / NL / LS | One Balise Group message (N_TOTAL=1; NID_BG=<BGa>) containing packet 68 (; Q_TRACKINIT=0) and packet 255 is received | I | BTM | If in L1; Q_TRACKINIT = 0 Possible announcement are: Only for: Lower pantograph announcement; Neutral section announcement; non stopping area announcement; Close air conditioning intake announcement; Regenerative Brake announcement; Tunnel stopping area announcement; change of traction announcement | L1 / L2 / L3 | FS / OS / TR / PT / NL / LS | - |
| 3 | L1 / L2 / L3 | FS / OS / TR / PT / NL / LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | If in L1 | L1 / L2 / L3 | FS / OS / TR / PT / NL / LS | - |
| 4 | L2 / L3 | FS / OS / TR / PT / NL / LS | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 68 (; Q_TRACKINIT=0) is received | I | RTM | If in L2/L3; Q_TRACKINIT = 0 Possible announcement are: Only for: Lower pantograph announcement; Neutral section announcement; non stopping area announcement; Close air conditioning intake announcement; Regenerative Brake announcement; Tunnel stopping area announcement; change of traction announcement | L2 / L3 | FS / OS / TR / PT / NL / LS | - |
| 5 | L2 / L3 | FS / OS / TR / PT / NL / LS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | If in L2/L3 | L2 / L3 | FS / OS / TR / PT / NL / LS | - |
| 6 | L1 / L2 / L3 | FS / OS / TR / PT / NL / LS | Announcement of track condition symbol “X” is displayed | O | DMI | These indication of track conditions are not applicable in NL and TR mode. An announcement for track condition "X" is displayed which is one of the following cases. Possible announcement are: Only for: Lower pantograph announcement; Neutral section announcement; non stopping area announcement; Close air conditioning intake announcement; Regenerative Brake announcement; Tunnel stopping area announcement; change of traction announcement | L1 / L2 / L3 | FS / OS / TR / PT / NL / LS | - |
| 7 | L1 / L2 / L3 | FS / OS / TR / PT / NL / LS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit50=1>) is recorded | O | JRU | Assuming that driver intervention is required. Otherwise Bit49=1 | L1 / L2 / L3 | FS / OS / TR / PT / NL / LS | - |
| 8 | L1 / L2 / L3 | FS / OS / TR / PT / NL / LS | Train is moving (V_TRAIN=40) | I | INT | The train reaches the Position where the ‘Track Condition’ begins. | L1 / L2 / L3 | FS / OS / TR / PT / NL / LS | - |
| 9 | L1 / L2 / L3 | FS / OS / TR / PT / NL / LS | Announcement of track condition symbol “X” is removed | O | DMI | The announcement is no longer displayed. | L1 / L2 / L3 | FS / OS / TR / PT / NL / LS | - |
| 10 | L1 / L2 / L3 | FS / OS / TR / PT / NL / LS | The Track Condition symbol "X" is displayed | O | DMI | The track condition is displayed. | L1 / L2 / L3 | FS / OS / TR / PT / NL / LS | - |
| 11 | L1 / L2 / L3 | FS / OS / TR / PT / NL / LS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit50=0¦Bit49=1>) is recorded | O | JRU | Depends on the announcement MASK=0000000001000000000000 | L1 / L2 / L3 | FS / OS / TR / PT / NL / LS | - |
| 12 | L1 / L2 / L3 | FS / OS / TR / PT / NL / LS | One Balise Group message (N_TOTAL=1; NID_BG=<BGb>) containing packet 68 (; Q_TRACKINIT=1) and packet 255 is received | I | BTM | If in L1; Q_TRACKINIT = 1; The difference of the position between the first and the second packet 68 is less than the value of L_TRACKCOND of the first packet 68 | L1 / L2 / L3 | FS / OS / TR / PT / NL / LS | - |
| 13 | L1 / L2 / L3 | FS / OS / TR / PT / NL / LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | If in L1 | L1 / L2 / L3 | FS / OS / TR / PT / NL / LS | - |
| 14 | L2 / L3 | FS / OS / TR / PT / NL / LS | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 68 (; Q_TRACKINIT=1) is received | I | RTM | If in L2/L3; Q_TRACKINIT = 1; The difference of the position between the first and the second packet 68 is less than the value of L_TRACKCOND of the first packet 68 | L2 / L3 | FS / OS / TR / PT / NL / LS | - |
| 15 | L2 / L3 | FS / OS / TR / PT / NL / LS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | If in L2/L3 | L2 / L3 | FS / OS / TR / PT / NL / LS | - |
| 16 | L1 / L2 / L3 | FS / OS / TR / PT / NL / LS | The Track Condition symbol "X" is removed | O | DMI | Depends on the announcement. The Track Condition is updated and replaces it with its initial state. Nothing is displayed | L1 / L2 / L3 | FS / OS / TR / PT / NL / LS | - |
| 17 | L1 / L2 / L3 | FS / OS / TR / PT / NL / LS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit50=0¦Bit49=0>) is recorded | O | JRU | - | L1 / L2 / L3 | FS / OS / TR / PT / NL / LS | - |

| Step 2: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. First balise |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group Two balises |
| M_DUP | 2 | 0 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | "BGa" | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 68 | Track Condition |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | 1 | - |
| Q_TRACKINIT | 1 | 0 | - |
| D_TRACKCOND | 15 | 0 | Only if Q_TRACKINIT = 0, D_TRACKCOND and the following variables follow. |
| L_TRACKCOND | 15 | FINITE VALUE | Longer as the distance to the next Position where a newer Packet number 68 is received |
| M_TRACKCOND | 4 | 0000 or 0001 or 0010 or 0011 or 0100 or 0101 or 0110 or 0111 or 1000 or 1001 or 1010 | 0000 Non stopping area. Initial state: stopping permitted 0001 Tunnel stopping area. Initial state: no tunnel stopping area 0010 Sound horn. Initial state: no request for sound horn 0011 Powerless section – lower pantograph. Initial state: not powerless section 0100 Radio hole (stop supervising T_NVCONTACT). Initial state: supervise T_NVCONTACT 0101 Air tightness. Initial state: no request for air tightness 0110 Switch off regenerative brake. Initial state: regenerative brake on 0111 Switch off eddy current brake for service brake. Initial state: eddy current brake for service brake on 1000 Switch off magnetic shoe brake. Initial state: magnetic shoe brake on 1001 Powerless section – switch off the main power switch. Initial state: not powerless section 1010 Switch off eddy current brake for emergency brake. Initial state: eddy current brake for emergency brake on |
| N_ITER | 5 | 0 | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 2: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | 0 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | "BGa" | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 4: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - General Message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | Trainborne Clock |
| M_ACK | 1 | 0 | No acknowledgement required |
| NID_LRBG | 24 | FINITE VALUE | Identity of last relevant balisegroup |
| NID_PACKET | 8 | 68 | Track Condition |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | 1 | - |
| Q_TRACKINIT | 1 | 0 | - |
| D_TRACKCOND | 15 | 0 | Only if Q_TRACKINIT = 0, D_TRACKCOND and the following variables follow. |
| L_TRACKCOND | 15 | FINITE VALUE | Longer as the distance to the next Position where a newer Packet number 68 is received |
| M_TRACKCOND | 4 | 0000 or 0001 or 0010 or 0011 or 0100 or 0101 or 0110 or 0111 or 1000 or 1001 or 1010 | 0000 Non stopping area. Initial state: stopping permitted 0001 Tunnel stopping area. Initial state: no tunnel stopping area 0010 Sound horn. Initial state: no request for sound horn 0011 Powerless section – lower pantograph. Initial state: not powerless section 0100 Radio hole (stop supervising T_NVCONTACT). Initial state: supervise T_NVCONTACT 0101 Air tightness. Initial state: no request for air tightness 0110 Switch off regenerative brake. Initial state: regenerative brake on 0111 Switch off eddy current brake for service brake. Initial state: eddy current brake for service brake on 1000 Switch off magnetic shoe brake. Initial state: magnetic shoe brake on 1001 Powerless section – switch off the main power switch. Initial state: not powerless section 1010 Switch off eddy current brake for emergency brake. Initial state: eddy current brake for emergency brake on |
| N_ITER | 5 | 0 | - |
| D_TRACKCOND(k) | 15 | FINITE VALUE | - |
| L_TRACKCOND(k) | 15 | FINITE VALUE | - |
| M_TRACKCOND(k) | 4 | FINITE VALUE | - |

| Step 12: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. First balise |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group Two balises |
| M_DUP | 2 | 0 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | "BGb" | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 68 | Track Condition |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | 1 | - |
| Q_TRACKINIT | 1 | 1 | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 12: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | 0 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | "BGb" | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 14: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - General Message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | Trainborne Clock |
| M_ACK | 1 | 0 | No acknowledgement required |
| NID_LRBG | 24 | FINITE VALUE | Identity of last relevant balisegroup |
| NID_PACKET | 8 | 68 | Track Condition |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | 1 | - |
| Q_TRACKINIT | 1 | 1 | - |
| D_TRACKINIT | 15 | 0 | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 7 / 8 / 9 / 11 / 12 | FS / OS / TR / PT / NL / LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Only in L2 / L3 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 12

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.7.3: Extension, replacement of track description and linking information |  | 3070300 |
| Test Case of Feature | When requested by trackside, the ERTMS/ETCS on-board equipment shall resume initial states beyond a given location individually. |  | 12 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS L2: FS, LS, OS L3: FS, LS, OS |  |  |
| Target of Test | When requested by trackside, the ERTMS/ETCS on-board equipment shall resume initial states beyond a given location individually through a single request, for all stored route suitability information (for initial state, refer to 3.12.2.10). |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.7.3.2 |  |
|  |  | Subset-026-3.7.3.2 d) |  |
|  |  | Subset-026-3.12.2.10 |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |
|  |  | Subset-026-7.4.2.21 Packet Number 70: route suitability data |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check that: • The train is NOT at standstill. • The On-board is in Level x (1, 2 or 3), and if in L2 or L3 the train has an active communication session to the RBC. • The train overpasses a Balise group (L1) or receives a radio message 24: ‘General Message’ (L2/L3) with a Packet Number 70: ‘Route Suitability Data’. • The Route Suitability Data is updated and replaces the stored Route Suitability Data, if one was stored On-board. • The train overpasses a Balise group (L1) or receives a radio message 24: ‘General Message’ (L2/L3) with a Packet Number 70: ‘Route Suitability Data’. With Q_TRACKINIT = ‘1’ • The Route Suitability Data is updated and the on-board equipment shall resume the initial states |
| Constraints | If in L2 or L3 the train has an active communication session to the RBC. a) The loading gauge profile of the train is not G1 b) The list of traction systems accepted by the engine does not include AC25kV c) The axle load category of the train is higher than C2 |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / LS / OS |
| Radio communication session | ESTABLISHED | Only in L2 / L3 |
| Movement Authority | STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Only in L2 / L3 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 / L2 / L3 | FS / LS / OS | Train is moving (V_TRAIN=40) | I | INT | The train is NOT at standstill. | L1 / L2 / L3 | FS / LS / OS | - |
| 2 | L1 / L2 / L3 | FS / LS / OS | One Balise Group message (N_TOTAL=1; NID_BG=<BGa>) containing packet 70 (; TRACKINIT=0) and packet 255 is received | I | BTM | If in L1; Q_TRACKINIT = 0, An unsuitability exists, Max axle load | L1 / L2 / L3 | FS / LS / OS | - |
| 3 | L1 / L2 / L3 | FS / LS / OS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | If in L1 | L1 / L2 / L3 | FS / LS / OS | - |
| 4 | L2 / L3 | FS / LS / OS | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 70 (; Q_TRACKINIT=0) is received | I | RTM | If in L2/L3; Q_TRACKINIT = 0, An unsuitability exists, Max axle load | L2 / L3 | FS / LS / OS | - |
| 5 | L2 / L3 | FS / LS / OS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | If in L2/L3 | L2 / L3 | FS / LS / OS | - |
| 6 | L1 / L2 / L3 | FS / LS / OS | The system status message "Route unsuitable – axle load category" is displayed | O | DMI | The driver shall be informed about all unsuitabilities (Max axle load). The closest location corresponding to the unsuitability(ies) shall be considered as both the EOA and SvL (instead of the EOA and SvL given by the MA), with no Release Speed. | L1 / L2 / L3 | FS / LS / OS | - |
| 7 | L1 / L2 / L3 | FS / LS / OS | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23) is recorded | O | JRU | - | L1 / L2 / L3 | FS / LS / OS | - |
| 8 | L1 / L2 / L3 | FS / LS / OS | One Balise Group message (N_TOTAL=1; NID_BG=<BGb>) containing packet 70 (; TRACKINIT=1) and packet 255 is received | I | BTM | If in L1; Q_TRACKINIT = 1, No unsuitability exists | L1 / L2 / L3 | FS / LS / OS | - |
| 9 | L1 / L2 / L3 | FS / LS / OS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | If in L1 | L1 / L2 / L3 | FS / LS / OS | - |
| 10 | L2 / L3 | FS / LS / OS | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 70 (; Q_TRACKINIT=1) is received | I | RTM | If in L2/L3; Q_TRACKINIT = 1, No unsuitability exists | L2 / L3 | FS / LS / OS | - |
| 11 | L2 / L3 | FS / LS / OS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | If in L2/L3 | L2 / L3 | FS / LS / OS | - |
| 12 | L1 / L2 / L3 | FS / LS / OS | The system status message "Route unsuitable – axle load category" is removed | O | DMI | ‘Route Suitability Data’ is updated for Max axle load | L1 / L2 / L3 | FS / LS / OS | - |
| 13 | L1 / L2 / L3 | FS / LS / OS | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23) is recorded | O | JRU | - | L1 / L2 / L3 | FS / LS / OS | - |

| Step 2: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. First balise |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group Two balises |
| M_DUP | 2 | 0 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | "BGa" | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 70 | Route Suitability Data |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | 1 | - |
| Q_TRACKINIT | 1 | 0 | - |
| D_SUITABILITY | 15 | 0 | Only If Q_TRACKINIT = 0, D_SUITABILITY and the following variables follows. |
| Q_SUITABILITY | 2 | 00 or 01 or 10 | 00 =Loading gauge or 01=Max axle load or 10=Traction system |
| M_LINEGAUGE | 8 | xxxx xxx1 | If Q_SUITABILITY= loading gauge. xxxx xxx1 = G1 |
| M_AXLELOADCAT | 7 | 4 | If Q_SUITABILITY= Max axle load. 4=C2 |
| M_VOLTAGE | 4 | 1 | If Q_SUITABILITY = traction system. 1 = AC 25 kV 50 Hz |
| NID_CTRACTION | 10 | FINITE VALUE | If Q_SUITABILITY = traction system and M_VOLTAGE ≠ 0. |
| N_ITER | 5 | 0 | - |
| D_SUITABILITY(k) | 15 | FINITE VALUE | - |
| Q_SUITABILITY(k) | 2 | FINITE VALUE | - |
| M_LINEGAUGE(k) | 8 | FINITE VALUE | If Q_SUITABILITY(k) = loading gauge. |
| M_AXLELOADCAT(k) | 7 | FINITE VALUE | If Q_SUITABILITY(k) = Max axle load. |
| M_VOLTAGE(k) | 4 | FINITE VALUE | If Q_SUITABILITY(k) = traction system. |
| NID_CTRACTION(k) | 10 | FINITE VALUE | If Q_SUITABILITY(k) = traction system and M_VOLTAGE(k) ≠ 0. |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 2: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | 0 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | "BGa" | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 4: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - General Message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | Trainborne Clock |
| M_ACK | 1 | 0 | No acknowledgement required |
| NID_LRBG | 24 | FINITE VALUE | Identity of last relevant balisegroup |
| NID_PACKET | 8 | 70 | Route Suitability Data |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | 1 | - |
| Q_TRACKINIT | 1 | 0 | - |
| D_SUITABILITY | 15 | 0 | Only If Q_TRACKINIT = 0, D_SUITABILITY and the following variables follows. |
| Q_SUITABILITY | 2 | 00 or 01 or 10 | 00 =Loading gauge or 01=Max axle load or 10=Traction system |
| M_LINEGAUGE | 8 | xxxx xxx1 | If Q_SUITABILITY= loading gauge. xxxx xxx1 = G1 |
| M_AXLELOADCAT | 7 | 4 | If Q_SUITABILITY= Max axle load. 4=C2 |
| M_VOLTAGE | 4 | 1 | If Q_SUITABILITY = traction system. 1= AC 25 kV 50 Hz |
| NID_CTRACTION | 10 | FINITE VALUE | If Q_SUITABILITY = traction system and M_VOLTAGE ≠ 0. |
| N_ITER | 5 | FINITE VALUE | - |
| D_SUITABILITY(k) | 15 | FINITE VALUE | - |
| Q_SUITABILITY(k) | 2 | FINITE VALUE | - |
| M_LINEGAUGE(k) | 8 | FINITE VALUE | If Q_SUITABILITY(k) = loading gauge. |
| M_AXLELOADCAT(k) | 7 | FINITE VALUE | If Q_SUITABILITY(k) = Max axle load. |
| M_VOLTAGE(k) | 4 | FINITE VALUE | If Q_SUITABILITY(k) = traction system. |
| NID_CTRACTION(k) | 10 | FINITE VALUE | If Q_SUITABILITY(k) = traction system and M_VOLTAGE(k) ≠ 0. |

| Step 8: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. First balise |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group Two balises |
| M_DUP | 2 | 0 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | "BGb" | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 70 | Route Suitability Data |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | 1 | - |
| Q_TRACKINIT | 1 | 1 | 1 = initial state; the initial state is that no restrictions related to route suitability exists |
| D_TRACKINIT | 15 | 0 | Only if Q_TRACKINIT = 1. |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 8: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | 0 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | "BGb" | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 10: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - General Message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | Trainborne Clock |
| M_ACK | 1 | 0 | No acknowledgement required |
| NID_LRBG | 24 | FINITE VALUE | Identity of last relevant balisegroup |
| NID_PACKET | 8 | 70 | Route Suitability Data |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | 1 | 1 = initial state; the initial state is that no restrictions related to route suitability exists |
| D_TRACKINIT | 15 | 0 | Only if Q_TRACKINIT = 1. |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / LS / OS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Only in L2 / L3 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 13

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.7.3: Extension, replacement of track description and linking information |  | 3070300 |
| Test Case of Feature | New Speed Restriction to ensure Permitted Braking Distance information shall replace all stored Speed Restriction to ensure Permitted Braking Distance information from the start location of the first element of the new information |  | 13 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS L2: FS, LS, OS L3: FS, LS, OS |  |  |
| Target of Test | When the train receives a new packet 52 (Permitted Braking Distance Information), it shall replace all stored Speed Restriction to ensure Permitted Braking Distance information from the start location of the first element of the new information |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.7.3.1 |  |
|  |  | Subset-026-3.7.3.1 d) |  |
|  |  | Subset-026-3.7.3.2 |  |
|  |  | Subset-026-3.7.3.2 a) |  |
|  |  | Subset-026-7.4.2.13.1 Packet Number 52: Permitted Braking Distance Information |  |
|  |  | Subset-026-3.11.11.11 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check that: • The train receives first BG (BGa) message containing a packet 52 • The Permitted Braking Distance Information is stored • The train receives second BG (BGb) message containing a packet 52 • The Permitted Braking Distance Information is updated and replaces the first one from the start location of the first element of the new information • The train receives third BG (BGc) message containing a packet 52 with Q_TRACKINIT= 1 (Empty profile, initial states to be resumed) • All speed restrictions derived from packets 52 in BGa and BGb are RELEASED |
| Constraints | At test beginning, there is no Permitted Braking Distance Information |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Only in L2 / L3 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 / L2 / L3 | FS / OS / LS | One Balise Group message (N_TOTAL=1; NID_BG=<BGa>) containing packet 52 and packet 255 is received | I | BTM | L1: The train overpasses a balise group (BGa). The BGa contains a packet 52 | L1 / L2 / L3 | FS / OS / LS | - |
| 2 | L1 / L2 / L3 | FS / OS / LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L1 / L2 / L3 | FS / OS / LS | - |
| 3 | L2 / L3 | FS / OS / LS | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 52 () is received | I | RTM | L2: The radio message contains a packet 52 | L2 / L3 | FS / OS / LS | - |
| 4 | L2 / L3 | FS / OS / LS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | L2 / L3 | FS / OS / LS | - |
| 5 | L1 / L2 / L3 | FS / OS / LS | The speed and distance monitoring information is displayed | O | DMI | The corresponding speed restriction is DISPLAYED This Step does not apply in LS mode. | L1 / L2 / L3 | FS / OS / LS | - |
| 6 | L1 / L2 / L3 | FS / OS / LS | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | - | L1 / L2 / L3 | FS / OS / LS | - |
| 7 | L1 / L2 / L3 | FS / OS / LS | One Balise Group message (N_TOTAL=1; NID_BG=<BGb>) containing packet 52 and packet 255 is received | I | BTM | L1: The train overpasses a second balise group (BGb). The BGb contains a packet 52 As BGb is overpassed, the Permitted Braking Distance of BGa is still effective | L1 / L2 / L3 | FS / OS / LS | - |
| 8 | L1 / L2 / L3 | FS / OS / LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L1 / L2 / L3 | FS / OS / LS | - |
| 9 | L2 / L3 | FS / OS / LS | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 52 () is received | I | RTM | L2: The radio message contains a packet 52 | L2 / L3 | FS / OS / LS | - |
| 10 | L2 / L3 | FS / OS / LS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | L2 / L3 | FS / OS / LS | - |
| 11 | L1 / L2 / L3 | FS / OS / LS | The speed and distance monitoring information is displayed | O | DMI | The speed and distance monitoring information is displayed. Check that the Permitted Braking Distance Information is updated and replaces the first one from the start location of the first element of the new information. This Step does not apply in LS mode. | L1 / L2 / L3 | FS / OS / LS | - |
| 12 | L1 / L2 / L3 | FS / OS / LS | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | - | L1 / L2 / L3 | FS / OS / LS | - |
| 13 | L1 / L2 / L3 | FS / OS / LS | One Balise Group message (N_TOTAL=1; NID_BG=<BGv>) containing packet 52 (Q_TRACKINIT=1) and packet 255 is received | I | BTM | L1: The train overpasses a third BG (BGc) The BGc contains a packet 52 with Q_TRACKINIT= 1 (Empty profile, initial states to be resumed) As BGc is overpassed, the Permitted Braking Distance of BGb is still effective | L1 / L2 / L3 | FS / OS / LS | - |
| 14 | L1 / L2 / L3 | FS / OS / LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L1 / L2 / L3 | FS / OS / LS | - |
| 15 | L2 / L3 | FS / OS / LS | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 52 () is received | I | RTM | L2: The radio message contains a packet 52 | L2 / L3 | FS / OS / LS | - |
| 16 | L2 / L3 | FS / OS / LS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | L2 / L3 | FS / OS / LS | - |
| 17 | L1 / L2 / L3 | FS / OS / LS | The speed and distance monitoring information is displayed | O | DMI | All speed restrictions derived from packets 52 in BGa and BGb are RELEASED This Step does not apply in LS mode. | L1 / L2 / L3 | FS / OS / LS | - |
| 18 | L1 / L2 / L3 | FS / OS / LS | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | - | L1 / L2 / L3 | FS / OS / LS | - |

| Step 1: Eurobalise Telegram (balise 1/N) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | 010 0000 | Version 2.0, introduced in SRS 3.3.0 |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. 1 of N |
| N_TOTAL | 3 | N | Total number of balises in the balise group N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group Except 254 |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | "BGa" | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 52 | Permitted Braking Distance Information |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | 0 | No initial states to be resumed, profile to follow |
| D_PBD | 15 | 0 | Only if Q_TRACKINIT = 0, D_PBD and the following variables follow. |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill, 1 = uphill. |
| G_PBDSR | 8 | FINITE VALUE | Gradient value to be used for the calculation. |
| Q_PBDSR | 1 | FINITE VALUE | - |
| D_PBDSR | 15 | FINITE VALUE | - |
| L_PBDSR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_PBD(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill, 1 = uphill. |
| G_PBDSR(k) | 8 | FINITE VALUE | Gradient value to be used for the calculation. |
| Q_PBDSR(k) | 1 | FINITE VALUE | - |
| D_PBDSR(k) | 15 | FINITE VALUE | - |
| L_PBDSR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 3: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 52 | Permitted Braking Distance Information |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | 0 | No initial states to be resumed, profile to follow |
| D_PBD | 15 | 0 | Only if Q_TRACKINIT = 0, D_PBD and the following variables follow. |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill, 1 = uphill. |
| G_PBDSR | 8 | FINITE VALUE | Gradient value to be used for the calculation. |
| Q_PBDSR | 1 | FINITE VALUE | - |
| D_PBDSR | 15 | FINITE VALUE | - |
| L_PBDSR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_PBD(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill, 1 = uphill. |
| G_PBDSR(k) | 8 | FINITE VALUE | Gradient value to be used for the calculation. |
| Q_PBDSR(k) | 1 | FINITE VALUE | - |
| D_PBDSR(k) | 15 | FINITE VALUE | - |
| L_PBDSR(k) | 15 | FINITE VALUE | - |

| Step 7: Eurobalise Telegram (balise 1/N) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | 010 0000 | Version 2.0, introduced in SRS 3.3.0 |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. 1 of N |
| N_TOTAL | 3 | N | Total number of balises in the balise group N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group Except 254 |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | "BGb" | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 52 | Permitted Braking Distance Information |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | 0 | No initial states to be resumed, profile to follow |
| D_PBD | 15 | 0 | Only if Q_TRACKINIT = 0, D_PBD and the following variables follow. |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill, 1 = uphill. |
| G_PBDSR | 8 | FINITE VALUE | Gradient value to be used for the calculation. |
| Q_PBDSR | 1 | FINITE VALUE | - |
| D_PBDSR | 15 | FINITE VALUE | - |
| L_PBDSR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_PBD(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill, 1 = uphill. |
| G_PBDSR(k) | 8 | FINITE VALUE | Gradient value to be used for the calculation. |
| Q_PBDSR(k) | 1 | FINITE VALUE | - |
| D_PBDSR(k) | 15 | FINITE VALUE | - |
| L_PBDSR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 9: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 52 | Permitted Braking Distance Information |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | 0 | No initial states to be resumed, profile to follow |
| D_PBD | 15 | 0 | Only if Q_TRACKINIT = 0, D_PBD and the following variables follow. |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill, 1 = uphill. |
| G_PBDSR | 8 | FINITE VALUE | Gradient value to be used for the calculation. |
| Q_PBDSR | 1 | FINITE VALUE | - |
| D_PBDSR | 15 | FINITE VALUE | - |
| L_PBDSR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_PBD(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill, 1 = uphill. |
| G_PBDSR(k) | 8 | FINITE VALUE | Gradient value to be used for the calculation. |
| Q_PBDSR(k) | 1 | FINITE VALUE | - |
| D_PBDSR(k) | 15 | FINITE VALUE | - |
| L_PBDSR(k) | 15 | FINITE VALUE | - |

| Step 13: Eurobalise Telegram (balise 1/N) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | 010 0000 | Version 2.0, introduced in SRS 3.3.0 |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. 1 of N |
| N_TOTAL | 3 | N | Total number of balises in the balise group N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group Except 254 |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | "BGc" | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 52 | Permitted Braking Distance Information |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | 1 | - |
| D_TRACKINIT | 15 | 0 | Only if Q_TRACKINIT = 1. |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 15: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 52 | Permitted Braking Distance Information |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | 1 | - |
| D_TRACKINIT | 15 | 0 | Only if Q_TRACKINIT = 1. |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Only in L2 / L3 |
| NOT RELEVANT | - | TIU | - |
| The speed and distance monitoring information is displayed | O | DMI | All speed restrictions derived from packets 52 in BGa and BGb (or received from RBC) are RELEASED |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 14

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.7.3: Extension, replacement of track description and linking information |  | 3070300 |
| Test Case of Feature | The train receives new Static Speed Profile information which replaces all stored Static Speed Profile information from the start location of the new information. |  | 14 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS L2: FS, LS, OS L3: FS, LS, OS |  |  |
| Target of Test | • The train receives a new packet 27 'International Static Speed Profile' information and shall replace all stored International Static Speed Profile information from the start location of the first element of the new information. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.7.3.1 |  |
|  |  | Subset-026-3.7.3.1 a) |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |
|  |  | Subset-026-7.4.2.7 Packet Number 27: International Static Speed Profile |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check that: • The train receives first BG (L1) or RBC message (L2/L3) message containing a packet 27 • The International Static Speed Profile Information is stored and replaces the previous International Static Speed Profile Information (if any) from the start location of the first element of the new information |
| Constraints | An SSP is stored on-board which ends beyond the start location of SSP received from BGa or RBC |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | Only in L2 / L3 |
| Movement Authority | STORED | - |
| International SSP | STORED | Ends beyond the start location of SSP received from BGa or RBC |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Only in L2 / L3 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | Speed and distance information is DISPLAYED, which reflects the currently stored SSP |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 / L2 / L3 | FS / OS / LS | Train is moving (V_TRAIN=40) | I | INT | The train is NOT at standstill. | L1 / L2 / L3 | FS / OS / LS | - |
| 2 | L1 / L2 / L3 | FS / OS / LS | One Balise Group message (N_TOTAL=1; NID_BG=<BGa>) containing packet 27 () and packet 255 is received | I | BTM | If in L1; the SSP is different from the previous one and shall be valid immediately | L1 / L2 / L3 | FS / OS / LS | - |
| 3 | L1 / L2 / L3 | FS / OS / LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | If in L1 | L1 / L2 / L3 | FS / OS / LS | - |
| 4 | L2 / L3 | FS / OS / LS | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 27 () is received | I | RTM | If in L2/L3; the SSP is different from the previous one and shall be valid immediately | L2 / L3 | FS / OS / LS | - |
| 5 | L2 / L3 | FS / OS / LS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | If in L2/L3 | L2 / L3 | FS / OS / LS | - |
| 6 | L1 / L2 / L3 | FS / OS / LS | The speed and distance monitoring information is displayed | O | DMI | The new SSP is the one recieved from track This Step does not apply in LS mode. | L1 / L2 / L3 | FS / OS / LS | - |

| Step 2: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. First balise |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group 2 balises in group |
| M_DUP | 2 | 0 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | "BGa" | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 2: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | 0 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | "BGa" | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 4: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - General Message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | Trainborne Clock |
| M_ACK | 1 | 0 | No acknowledgement required |
| NID_LRBG | 24 | FINITE VALUE | Identity of last relevant balisegroup |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | Only in L2 / L3 |
| Movement Authority | STORED | - |
| International SSP | STORED | New SSP |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Only in L2 / L3 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | Speed and distance information is DISPLAYED, which reflects the new SSP |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 15

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.7.3: Extension, replacement of track description and linking information |  | 3070300 |
| Test Case of Feature | The train receives a linking information and shall replace the stored information. |  | 15 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS |  |  |
| Target of Test | Target of Test • The train receives a new linking information received as infill information shall replace all stored linking information from the LRBG. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.7.3.1 |  |
|  |  | Subset-026-3.7.3.1 n) |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |
|  |  | Subset-026-7.4.2.2 Packet Number 5: Linking |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check that: • The train is NOT at standstill. • The On-board is in Level 1 • The train received from Eurobalise a message with a Packet Number 5: ‘Linking’. • The Linking is updated and replaces the stored Linking, if one was stored On-board. The linking packet is announcing the next three Bg’s in front of the train. • The train receives a RIU message with a Packet Number 5: ‘Linking’. • The Linking is updated and replaces the stored Linking. The linking packet is announcing only the last Bg of the last Linking package. ↔ if one was stored on-board |
| Constraints | The train has a valid LRBG. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Linking | STORED | - |
| Movement Authority | STORED | - |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Connection to RIU established |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 | FS / OS / LS | Train is moving (V_TRAIN=40) | I | INT | The train is NOT at standstill. | L1 | FS / OS / LS | - |
| 2 | L1 | FS / OS / LS | One Balise Group message containing packet 5 is received | I | BTM | This is Bga With linking informations to Bgb, BGc and BGd; For BGb Q_LINKREACTION = 10 (NO Reaction), for BGc Q_LINKREACTION = 01 (SB) and for BGd Q_LINKREACTION = 10 (NO Reaction) | L1 | FS / OS / LS | - |
| 3 | L1 | FS / OS / LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L1 | FS / OS / LS | - |
| 4 | L1 | FS / OS / LS | SA-DATA.Indication with Euroradio Message"Infill MA" (NID_MESSAGE=37) containing packet 12, packet 136, packet 5 (Q_DIR=1) is received | I | RTM | With linking informations to BGd and no more for BGc; for BGd Q_LINKREACTION = 10 (NO Reaction) | L1 | FS / OS / LS | - |
| 5 | L1 | FS / OS / LS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | L1 | FS / OS / LS | - |
| 6 | L1 | FS / OS / LS | Train is moving (V_TRAIN=40) | I | INT | The train overpasses the position of the main Bgb, no linking reaction is triggered | L1 | FS / OS / LS | - |
| 7 | L1 | FS / OS / LS | Train is moving (V_TRAIN=40) | I | INT | The train overpasses the position where BGc was announced by the linking packet of step 2, but no more by packet of step 4 . | L1 | FS / OS / LS | - |
| 8 | L1 | FS / OS / LS | NOT Service brake commanded | O | TIU | Service brake is not commanded because the second packet 5 has replaced the information of the first one | L1 | FS / OS / LS | - |
| 9 | L1 | FS / OS / LS | One Balise Group message is received | I | BTM | The train overpasses the position where BGd was announced by the linking packet of step 2 and step 4 | L1 | FS / OS / LS | - |

| Step 2: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | "Bga" | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 5 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LINK | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_BG | 14 | "Bgb" | - |
| Q_LINKORIENTATION | 1 | FINITE VALUE | - |
| Q_LINKREACTION | 2 | 10 | - |
| Q_LOCACC | 6 | FINITE VALUE | - |
| N_ITER | 5 | 2 | - |
| D_LINK | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_BG | 14 | "BGc" | - |
| Q_LINKORIENTATION | 1 | FINITE VALUE | - |
| Q_LINKREACTION | 2 | 1 | - |
| Q_LOCACC | 6 | FINITE VALUE | - |
| D_LINK(k) | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_BG(k) | 14 | "BGd" | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | 10 | - |
| Q_LOCACC(k) | 6 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 4: Radio Message 37 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 37 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | "Bga" | - |
| NID_PACKET | 8 | 12 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out". |
| N_ITER | 5 | FINITE VALUE | Set to zero if V_MAIN = 0 or if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 136 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | 0 | - |
| NID_BG | 14 | "Bgb" | - |
| NID_PACKET | 8 | 5 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LINK | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | 0 | - |
| NID_BG | 14 | "Bgd" | - |
| Q_LINKORIENTATION | 1 | FINITE VALUE | - |
| Q_LINKREACTION | 2 | 10 | - |
| Q_LOCACC | 6 | FINITE VALUE | - |
| N_ITER | 5 | 0 | - |

| Step 9: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Linking | STORED | - |
| Movement Authority | STORED | - |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Connection to RIU established |
| NOT RELEVANT | - | TIU | NOT Service brake commanded |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 16

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.7.3: Extension, replacement of track description and linking information |  | 3070300 |
| Test Case of Feature | New Speed Restriction to ensure Permitted Braking Distance information shall replace all stored Speed Restriction to ensure Permitted Braking Distance information from the start location of the first element of the new information |  | 16 |
| Applicable Mode/Level Combinations | L1: FS, LS |  |  |
| Target of Test | When the train receives a new packet 52 (Permitted Braking Distance Information), it shall replace all stored Speed Restriction to ensure Permitted Braking Distance information from the start location of the first element of the new information. The first packet 52 is received via Message 37 (Infill MA) |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.7.3.1 |  |
|  |  | Subset-026-3.7.3.1 d) |  |
|  |  | Subset-026-3.7.3.2 |  |
|  |  | Subset-026-3.7.3.2 a) |  |
|  |  | Subset-026-7.4.2.13.1 Packet Number 52: Permitted Braking Distance Information |  |
|  |  | Subset-026-3.11.11.7 |  |
|  |  | Subset-026-4.8.4.1.1 |  |
|  |  | Subset-026-8.7.15 Message 37: Infill MA |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check that: • The train receives the Infill MA containing a packet 52 • The Permitted Braking Distance Information is stored • The train receives first BG (BGa) message containing a packet 52 • The Permitted Braking Distance Information is stored • The train receives second BG (BGb) message containing a packet 52 • The Permitted Braking Distance Information is updated and replaces the first one from the start location of the first element of the new information • The train receives third BG (BGc) message containing a packet 52 with Q_TRACKINIT= 1 (Empty profile, initial states to be resumed) • All speed restrictions derived from packets 52 in BGa and BGb are RELEASED |
| Constraints | • At test beginning, there is no Permitted Braking Distance Information • The train is inside an Infill Area with Radio infill • Linking information including Main balise group is stored. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| Linking | STORED | Linking information is stored. |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 | FS / LS | Train is moving (V_TRAIN=<V_PERMITTED>) | I | INT | (The onbard has established a radio communication session to radio infill unit.) | L1 | FS / LS | - |
| 2 | L1 | FS / LS | SA-DATA.Indication with Euroradio Message "Infill MA" (NID_MESSAGE=37) containing packet 12 (MA), packet 136 (infill location reference), packet 52 (Permitted Braking Distance Information), is received. | I | RTM | - | L1 | FS / LS | - |
| 3 | L1 | FS / LS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | L1 | FS / LS | - |
| 4 | L1 | FS / LS | The speed and distance monitoring information is displayed | O | DMI | New MA is displayed. Complete message is accepted and suitable information is presented to driver. The information is valid from the main balise group. The corresponding speed restriction is DISPLAYED | L1 | FS / LS | - |
| 5 | L1 | FS / LS | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | - | L1 | FS / LS | - |
| 6 | L1 | FS / LS | One Balise Group message (N_TOTAL=1; NID_BG=<BGa>) containing packet 52 and packet 255 is received | I | BTM | L1: The train overpasses a balise group (BGa). The BGa contains a packet 52 BGa is the Main BG for infill message in step 2 | L1 | FS / LS | - |
| 7 | L1 | FS / LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L1 | FS / LS | - |
| 8 | L1 | FS / LS | The speed and distance monitoring information is displayed | O | DMI | The corresponding speed restriction is DISPLAYED | L1 | FS / LS | - |
| 9 | L1 | FS / LS | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | - | L1 | FS / LS | - |
| 10 | L1 | FS / LS | One Balise Group message (N_TOTAL=1; NID_BG=<BGb>) containing packet 52 and packet 255 is received | I | BTM | L1: The train overpasses a second balise group (BGb). The BGb contains a packet 52 As BGb is overpassed, the Permitted Braking Distance of BGa is still effective | L1 | FS / LS | - |
| 11 | L1 | FS / LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L1 | FS / LS | - |
| 12 | L1 | FS / LS | The speed and distance monitoring information is displayed | O | DMI | The speed and distance monitoring information is displayed. Check that the Permitted Braking Distance Information is updated and replaces the first one from the start location of the first element of the new information. | L1 | FS / LS | - |
| 13 | L1 | FS / LS | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | - | L1 | FS / LS | - |
| 14 | L1 | FS / LS | One Balise Group message (N_TOTAL=1; NID_BG=<BGv>) containing packet 52 (Q_TRACKINIT=1) and packet 255 is received | I | BTM | L1: The train overpasses a third BG (BGc) The BGc contains a packet 52 with Q_TRACKINIT= 1 (Empty profile, initial states to be resumed) As BGc is overpassed, the Permitted Braking Distance of BGb is still effective | L1 | FS / LS | - |
| 15 | L1 | FS / LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L1 | FS / LS | - |
| 16 | L1 | FS / LS | The speed and distance monitoring information is displayed | O | DMI | All speed restrictions derived from packets 52 in BGa and BGb are RELEASED | L1 | FS / LS | - |
| 17 | L1 | FS / LS | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | - | L1 | FS / LS | - |

| Step 2: Radio Message 37 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 37 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 136 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | If Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| NID_PACKET | 8 | 12 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out". |
| N_ITER | 5 | FINITE VALUE | Set to zero if V_MAIN = 0 or if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 52 | Permitted Braking Distance Information |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | 0 | No initial states to be resumed, profile to follow |
| D_PBD | 15 | 0 | Only if Q_TRACKINIT = 0, D_PBD and the following variables follow. |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill, 1 = uphill. |
| G_PBDSR | 8 | FINITE VALUE | Gradient value to be used for the calculation. |
| Q_PBDSR | 1 | FINITE VALUE | - |
| D_PBDSR | 15 | FINITE VALUE | - |
| L_PBDSR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_PBD(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill, 1 = uphill. |
| G_PBDSR(k) | 8 | FINITE VALUE | Gradient value to be used for the calculation. |
| Q_PBDSR(k) | 1 | FINITE VALUE | - |
| D_PBDSR(k) | 15 | FINITE VALUE | - |
| L_PBDSR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | End of Information |

| Step 6: Eurobalise Telegram (balise 1/N) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | 010 0000 | Version 2.0, introduced in SRS 3.3.0 |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. 1 of N |
| N_TOTAL | 3 | N | Total number of balises in the balise group N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group Except 254 |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | "BGa" | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 52 | Permitted Braking Distance Information |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | 0 | No initial states to be resumed, profile to follow |
| D_PBD | 15 | 0 | Only if Q_TRACKINIT = 0, D_PBD and the following variables follow. |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill, 1 = uphill. |
| G_PBDSR | 8 | FINITE VALUE | Gradient value to be used for the calculation. |
| Q_PBDSR | 1 | FINITE VALUE | - |
| D_PBDSR | 15 | FINITE VALUE | - |
| L_PBDSR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_PBD(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill, 1 = uphill. |
| G_PBDSR(k) | 8 | FINITE VALUE | Gradient value to be used for the calculation. |
| Q_PBDSR(k) | 1 | FINITE VALUE | - |
| D_PBDSR(k) | 15 | FINITE VALUE | - |
| L_PBDSR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 10: Eurobalise Telegram (balise 1/N) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | 010 0000 | Version 2.0, introduced in SRS 3.3.0 |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. 1 of N |
| N_TOTAL | 3 | N | Total number of balises in the balise group N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group Except 254 |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | "BGb" | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 52 | Permitted Braking Distance Information |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | 0 | No initial states to be resumed, profile to follow |
| D_PBD | 15 | 0 | Only if Q_TRACKINIT = 0, D_PBD and the following variables follow. |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill, 1 = uphill. |
| G_PBDSR | 8 | FINITE VALUE | Gradient value to be used for the calculation. |
| Q_PBDSR | 1 | FINITE VALUE | - |
| D_PBDSR | 15 | FINITE VALUE | - |
| L_PBDSR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_PBD(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill, 1 = uphill. |
| G_PBDSR(k) | 8 | FINITE VALUE | Gradient value to be used for the calculation. |
| Q_PBDSR(k) | 1 | FINITE VALUE | - |
| D_PBDSR(k) | 15 | FINITE VALUE | - |
| L_PBDSR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 14: Eurobalise Telegram (balise 1/N) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | 010 0000 | Version 2.0, introduced in SRS 3.3.0 |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. 1 of N |
| N_TOTAL | 3 | N | Total number of balises in the balise group N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group Except 254 |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | "BGc" | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 52 | Permitted Braking Distance Information |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | 1 | - |
| D_TRACKINIT | 15 | 0 | Only if Q_TRACKINIT = 1. |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| Linking | 1 | Linking is stored. |
| Movement Authority | STORED | Movement Authority has been received. |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The speed and distance monitoring information is displayed | O | DMI | All speed restrictions derived from packets 52 in BGa and BGb (or received from RBC) are RELEASED |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |