| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 4080415 (4.8.4: Accepted Information depending on the modes - Geographical Position) Total: 7 Test case(s) |  |
| REF: | Subset-076-5-2-4080415 |
| ISSUE: | Version 3.2.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 0.0.1 23.03.2012 | All | First version/review | DLR |
| Version 2.9.0 31.03.2013 | All | First version/review for Baseline 3 | CEDEX |
| Version 2.9.1 04.10.2013 | All | First version/review for Baseline 3 | CEDEX |
| Version 2.9.2 21.11.2013 | All | General review | CEDEX |
| Version 2.9.3 12.12.2013 | TC 2, 6 | Editorial changes | CEDEX |
| Version 2.9.4 18.12.2013 | TC 1,2,3,4,6,7,8 | General review | CEDEX |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.2.0 31.03.2017 | All | Baseline 3 1st maintenance release version - Bug fixing | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Geographical Position |  | 4080415 |
| Test Case of Feature | Transmission of list geographical position information from RBC, together with standard message 24. |  | 1 |
| Applicable Mode/Level Combinations | L2 : FS, LS, OS, SR, SB, TR, PT, NL L3 : FS, LS, OS, SR, SB, TR, PT, NL |  |  |
| Target of Test | The target of this test is to check the correct working, that is: A geographical position information is received and accepted by the on-board system. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.3.1.1 Geographical Position |  |
|  |  | Subset-026-4.8.4.2 Geographical Position |  |
|  |  | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-4.8.4.2 [1] |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |
|  |  | Subset-026-7.4.2.25 Packet Number 79: Geographical Position Information |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check that : The on-board accepts the message Check on JRU that : A message sent from RBC containing a geographical position information list has been received and is recorded. |
| Constraints | In SB mode, a cab needs to be active. In PT mode, only if following the reception of the information “Recognition of Exit from TR mode” with a more recent time stamp. (Req. 3.6.6.4) Geographical position information shall always use a balise group as geographical position reference balise group and if needed an offset from that balise group. A geographical position reference balise group shall be either: a) part of the last reported balise groups memorised on-board, in case the information is received from radio, OR b) the balise group transmitting the information, in case the information is received from balise group, OR c) any balise group not yet passed at the time of reception of the information. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/1/2/6/7/8/11/12 | FS/OS/SR/ SB/TR/PT/NL/LS |
| ERTMS/ETCS level | 3 / 4 | L2 /L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 79 is received | I | RTM | The complete message is accepted by on-board. | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | Message is recorded | N/A | N/A | - |
| 3 | N/A | N/A | The Driver Request symbol "Geographical Position" is displayed | O | DMI | After the train front end has passed over the location specified by D_POSOFF value. | N/A | N/A | - |
| 4 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit83=1>) is recorded | O | JRU | Bit83: Symbol DR03 (the symbol DR03 shall be shown in G12 if the geographical position is toggled off) | N/A | N/A | - |
| 5 | N/A | N/A | The driver presses "Geographical Position" | I | DMI | - | N/A | N/A | - |
| 6 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS = 0001 1110) is recorded | O | JRU | M_DRIVERACTIONS = 0001 1110: Request to show geographical position | N/A | N/A | - |
| 7 | N/A | N/A | The "Geographical Position" is displayed | O | DMI | According to the M_POSITION | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 79 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | Geographical Position Reference Balise Group. |
| D_POSOFF | 15 | FINITE VALUE | - |
| Q_MPOSITION | 1 | FINITE VALUE | Geographical Position counting direction. |
| M_POSITION | 24 | FINITE VALUE | Track kilometre reference value. |
| N_ITER | 5 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | Geographical Position Reference Balise Group. |
| D_POSOFF(k) | 15 | FINITE VALUE | - |
| Q_MPOSITION(k) | 1 | FINITE VALUE | Geographical Position counting direction. |
| M_POSITION(k) | 24 | FINITE VALUE | Track kilometre reference value. |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/1/2/6/7/8/11/12 | FS/OS/SR/ SB/TR/PT/NL/LS |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Geographical Position |  | 4080415 |
| Test Case of Feature | The onboard equipment receives a list of geographical position information from RBC. The packet is REJECTED, because of the current mode. |  | 2 |
| Applicable Mode/Level Combinations | L2 : SH, PS, SL, RV L3 : SH, PS, SL, RV |  |  |
| Target of Test | The target of this test is to check the correct working, that is: A geographical position information is received and rejected by the on-board system. No list of geographical position information is stored onboard. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.3.1.1 Geographical Position |  |
|  |  | Subset-026-4.8.4.2 Geographical Position |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |
|  |  | Subset-026-7.4.2.25 Packet Number 79: Geographical Position Information |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on JRU that : A message sent from RBC or BG containing a geographical position information list has been received and is recorded. The on-board equipment changes to SB mode. Check on the DMI that : No geographical position is diplayed after the driver requests it |
| Constraints | (Req. 3.6.6.4) Geographical position information shall always use a balise group as geographical position reference balise group and if needed an offset from that balise group. A geographical position reference balise group shall be either: a) part of the last reported balise groups memorised on-board, in case the information is received from radio, OR b) the balise group transmitting the information, in case the information is received from balise group, OR c) any balise group not yet passed at the time of reception of the information. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 / 5 / 14 / 15 | SH / SL / RV / PS |
| ERTMS/ETCS level | 3 / 4 | L2 /L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2/L3 | SH/PS/SL/RV | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 79 is received | I | RTM | The packet is REJECTED. | L2/L3 | SH/PS/SL/RV | - |
| 2 | L2/L3 | SH/PS/SL/RV | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | Message is recorded | L2/L3 | SH/PS/SL/RV | - |
| 3 | L2/L3 | SH/PS/SL/RV | USE_FT4060300.18: The EVC changes to SB mode after the train front end has passed over the location specified by D_POSOFF value | - | - | If transition from SH mode | L2/L3 | SB | - |
| 4 | L2/L3 | SH/PS/SL/RV | USE_FT3060300.9: The EVC changes to SB mode after the train front end has passed over the location specified by D_POSOFF value | - | - | If transition from PS mode | L2/L3 | SB | - |
| 5 | L2/L3 | SH/PS/SL/RV | USE_FT4060300.2,7: The EVC changes to SB mode after the train front end has passed over the location specified by D_POSOFF value | - | - | If transition from SL mode | L2/L3 | SB | - |
| 6 | L2/L3 | SH/PS/SL/RV | USE_FT4060300.8: The EVC changes to SB mode after the train front end has passed over the location specified by D_POSOFF value | - | - | If transition from RV mode | L2/L3 | SB | - |
| 7 | L2/L3 | SB | Cab active | I | TIU | If cab is not active | L2/L3 | SB | - |
| 8 | L2/L3 | SB | NOT The Driver Request symbol "Geographical Position" is displayed | O | DMI | - | L2/L3 | SB | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 79 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | Geographical Position Reference Balise Group. |
| D_POSOFF | 15 | FINITE VALUE | - |
| Q_MPOSITION | 1 | FINITE VALUE | Geographical Position counting direction. |
| M_POSITION | 24 | FINITE VALUE | Track kilometre reference value. |
| N_ITER | 5 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | Geographical Position Reference Balise Group. |
| D_POSOFF(k) | 15 | FINITE VALUE | - |
| Q_MPOSITION(k) | 1 | FINITE VALUE | Geographical Position counting direction. |
| M_POSITION(k) | 24 | FINITE VALUE | Track kilometre reference value. |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Geographical Position |  | 4080415 |
| Test Case of Feature | Transmission of geographical position information via balise group Together with standard balise telegram. |  | 3 |
| Applicable Mode/Level Combinations | L0 : UN, SB, NL LNTC: SB, TR, NL, SN L1 : FS, LS, OS, SR, SB, TR, NL L2 : FS, LS, OS, SR, SB, TR, NL L3 : FS, LS, OS, SR, SB, TR,,NL |  |  |
| Target of Test | The target of this test is to check that the: Geographical position information via balise group or an empty list is received and accepted by the on-board system. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.3.1.1 Geographical Position |  |
|  |  | Subset-026-4.8.3.1.1 Exception [1] |  |
|  |  | Subset-026-4.8.3.1.1 Exception [2] |  |
|  |  | Subset-026-4.8.4.2 Geographical Position |  |
|  |  | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-7.4.2.25 Packet Number 79: Geographical Position Information |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check that : The on-board accepts the Telegram M_POSITION<>16777215 in packet 79 Check on JRU that : A Geographical position information via balise group containing a list of balises (or empty list) has been received and is recorded. |
| Constraints | A cab needs to be active (SB Mode) In LNTC, an order to switch to L1 or to L2/3 at a further location has been received. (Req. 3.6.6.4) Geographical position information shall always use a balise group as geographical position reference balise group and if needed an offset from that balise group. A geographical position reference balise group shall be either: a) part of the last reported balise groups memorised on-board, in case the information is received from radio, OR b) the balise group transmitting the information, in case the information is received from balise group, OR c) any balise group not yet passed at the time of reception of the information. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 11 / 12 / 13 | FS / OS / SR / UN / SB / TR / NL / LS / SN |
| Level Transition Order | STORED | L1 / L2 / L3 (Only in LNTC) |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0/LNTC/L1 /L2/L3 | FS/OS/SR/UN/SB/TR/NL/LS/SN | One Balise Group message (N_TOTAL=1; NID_C=<comment>) containing packet 79 : ‘geographical position information’ is received | I | BTM | Telegram is received by on-board | L0/LNTC/L1 /L2/L3 | FS/OS/SR/UN/SB/TR/NL/LS/SN | - |
| 2 | L0/LNTC/L1 /L2/L3 | FS/OS/SR/UN/SB/TR/NL/LS/SN | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | Packet and telegram are recorded. | L0/LNTC/L1 /L2/L3 | FS/OS/SR/UN/SB/TR/NL/LS/SN | - |
| 3 | L0/LNTC/L1 /L2/L3 | FS/OS/SR/UN/SB/TR/NL/LS/SN | USE_FT5100100.23,21,20: for changing to L1/2/3 | - | - | If the current level is LNTC | L0/L1 /L2/L3 | FS/OS/SR/UN/SB/TR/NL/LS/SN | - |
| 4 | L0/L1 /L2/L3 | FS/OS/SR/UN/SB/TR/NL/LS | The Driver Request symbol "Geographical Position" is displayed | O | DMI | After the train front end has passed over the location specified by D_POSOFF value. | L0/L1 /L2/L3 | FS/OS/SR/UN/SB/TR/NL/LS | - |
| 5 | L0/L1 /L2/L3 | FS/OS/SR/UN/SB/TR/NL/LS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit83=1>) is recorded | O | JRU | Bit83: Symbol DR03 (the symbol DR03 shall be shown in G12 if the geographical position is toggled off) | L0/L1 /L2/L3 | FS/OS/SR/UN/SB/TR/NL/LS | - |
| 6 | L0/L1 /L2/L3 | FS/OS/SR/UN/SB/TR/NL/LS | The driver presses "Geographical Position" | I | DMI | Only if the previous status of the geographical position is toggled off | L0/L1 /L2/L3 | FS/OS/SR/UN/SB/TR/NL/LS | - |
| 7 | L0/L1 /L2/L3 | FS/OS/SR/UN/SB/TR/NL/LS | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS = 0001 1110) is recorded | O | JRU | Only if the previous status of the geographical position is toggled off M_DRIVERACTIONS = 0001 1110: Request to show geographical position | L0/L1 /L2/L3 | FS/OS/SR/UN/SB/TR/NL/LS | - |
| 8 | L0/L1 /L2/L3 | FS/OS/SR/UN/SB/TR/NL/LS | The "Geographical Position" is displayed | O | DMI | According to the M_POSITION | L0/L1 /L2/L3 | FS/OS/SR/UN/SB/TR/NL/LS | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0) Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system. |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M_MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of the balise group. |
| NID_C | 10 | FINITE VALUE | Country or region. |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group. |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 79 | Any optional packet and common optional packets ..referring to SRS SRS 8.4.4.4.1 and 8.4.4.6.1 Packet 79 in this feature |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | 0 cm, 1m or 10 m depends on Q_SCALE |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1 |
| NID_BG | 14 | FINITE VALUE | Geographical Position Reference Balise Group |
| D_POSOFF | 15 | FINITE VALUE | - |
| Q_MPOSITION | 1 | FINITE VALUE | Geographical Position counting direction |
| M_POSITION | 24 | FINITE VALUE | Track kilometre reference value |
| N_ITER | 5 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1 |
| NID_BG(k) | 14 | FINITE VALUE | Geographical Position Reference Balise Group |
| D_POSOFF(k) | 15 | FINITE VALUE | - |
| Q_MPOSITION(k) | 1 | FINITE VALUE | Geographical Position counting direction |
| M_POSITION(k) | 24 | FINITE VALUE | Track kilometre reference value |
| NID_PACKET | 8 | 255 | 255 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 11 / 12 | FS / OS / SR / UN / SB / TR / NL / LS |
| ERTMS/ETCS level | 0 / 2 / 3 / 4 | L0 / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Geographical Position |  | 4080415 |
| Test Case of Feature | The onboard equipment receives a list of geographical position information via balise group. The packet is REJECTED because of the current mode. |  | 4 |
| Applicable Mode/Level Combinations | L0 : SH, PS, SL LNTC: SL L1 : SH, PS, SL, PT, RV L2 : SH, PS, SL, PT, RV L3 : SH, PS, SL, PT, RV |  |  |
| Target of Test | The target of this test is to check that the: Geographical position information via balise group or an empty list is received and rejected by the on-board system, because the current mode obliges to reject this information. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.3.1.1 Geographical Position |  |
|  |  | Subset-026-4.8.4.2 Geographical Position |  |
|  |  | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-4.8.4.2 [1] |  |
|  |  | Subset-026-7.4.2.25 Packet Number 79: Geographical Position Information |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check that : No geographical position information is stored onboard Check on JRU that : A Geographical position information via balise group containing a list of balises (or empty list) has been received and is recorded. The on-board equipment changes to one of the following modes: SB, FS, LS, OS or SR. Check on the DMI that : No geographical position is diplayed after the driver requests it |
| Constraints | In LNTC, an order to switch to L1 or to L2/3 at a further location has been received. In PT mode, the “Recognition of Exit from TR mode” has not been received yet. (Req. 3.6.6.4) Geographical position information shall always use a balise group as geographical position reference balise group and if needed an offset from that balise group. A geographical position reference balise group shall be either: a) part of the last reported balise groups memorised on-board, in case the information is received from radio, OR b) the balise group transmitting the information, in case the information is received from balise group, OR c) any balise group not yet passed at the time of reception of the information. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 / 5 / 8 / 14 / 15 | SH / SL / PT / RV / PS |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0/LNTC/L1/L2/L3 | SH/SL/PT/RV/PS | One Balise Group message (N_TOTAL=1; NID_C=<comment>) containing packet 79 : ‘geographical position information’ is received | I | BTM | The packet is REJECTED. | L0/LNTC/L1/L2/L3 | SH/SL/PT/RV/PS | - |
| 2 | L0/LNTC/L1/L2/L3 | SH/SL/PT/RV/PS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | Packet and telegram are recorded. | L0/LNTC/L1/L2/L3 | SH/SL/PT/RV/PS | - |
| 3 | L0/LNTC/L1/L2/L3 | SH/SL/PT/RV/PS | USE_FT4060300.18: The EVC changes to SB mode after the train front end has passed over the location specified by D_POSOFF value | - | - | If transition from SH mode | L0/L1/L2/L3 | SB | - |
| 4 | L0/LNTC/L1/L2/L3 | SH/SL/PT/RV/PS | USE_FT4060300.2,7: The EVC changes to SB mode after the train front end has passed over the location specified by D_POSOFF value | - | - | If transition from SL mode | L0/L1/L2/L3 | SB | - |
| 5 | L0/LNTC/L1/L2/L3 | SH/SL/PT/RV/PS | USE_FT4060300.8: The EVC changes to SB mode after the train front end has passed over the location specified by D_POSOFF value | - | - | If transition from RV mode | L0/L1/L2/L3 | SB | - |
| 6 | L0/LNTC/L1/L2/L3 | SH/SL/PT/RV/PS | The EVC changes to SB mode after the train front end has passed over the location specified by D_POSOFF value | - | - | If transition from PS mode | L0/L1/L2/L3 | SB | - |
| 7 | L0/L1/L2/L3 | SB/PT | Cab active | I | TIU | If cab is not active | L0/L1/L2/L3 | SB/PT | - |
| 8 | L0/L1/L2/L3 | SB/PT | NOT The Driver Request symbol "Geographical Position" is displayed | O | DMI | - | L0/L1/L2/L3 | SB/PT | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0) Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system. |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M_MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of the balise group. |
| NID_C | 10 | FINITE VALUE | Country or region. |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group. |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 79 | Any optional packet and common optional packets ..referring to SRS SRS 8.4.4.4.1 and 8.4.4.6.1 Packet 79 in this feature |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | 0 cm, 1m or 10 m depends on Q_SCALE |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1 |
| NID_BG | 14 | FINITE VALUE | Geographical Position Reference Balise Group |
| D_POSOFF | 15 | FINITE VALUE | - |
| Q_MPOSITION | 1 | FINITE VALUE | Geographical Position counting direction |
| M_POSITION | 24 | FINITE VALUE | Track kilometre reference value |
| N_ITER | 5 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1 |
| NID_BG(k) | 14 | FINITE VALUE | Geographical Position Reference Balise Group |
| D_POSOFF(k) | 15 | FINITE VALUE | - |
| Q_MPOSITION(k) | 1 | FINITE VALUE | Geographical Position counting direction |
| M_POSITION(k) | 24 | FINITE VALUE | Track kilometre reference value |
| NID_PACKET | 8 | 255 | 255 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 / 8 | SB / PT |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 5

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Geographical Position |  | 4080415 |
| Test Case of Feature | The onboard equipment receives a list of geographical position information from the RBC. The packet is REJECTED, because the current level is not L2/3 and no announcement to change to L2/3 has been received on-board. |  | 5 |
| Applicable Mode/Level Combinations | L0 : UN, SB, NL LNTC: SB, TR, NL, SN L1 : FS, LS, OS, SR, SB, TR, NL |  |  |
| Target of Test | The target of this test is to check the correct working, that is: A geographical position information is received and rejected by the on-board system. No list of geographical position information is stored onboard. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.3.1.1 Geographical Position |  |
|  |  | Subset-026-4.8.4.2 Geographical Position |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |
|  |  | Subset-026-7.4.2.25 Packet Number 79: Geographical Position Information |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check that : The on-board rejects the message No list of geographical position information is stored onboard Check on JRU that : A message sent from RBC containing a geographical position information list has been received and is recorded. Check on the DMI that : No geographical position is diplayed after the driver requests it |
| Constraints | No announcement of transition level is stored onboard. (Req. 3.6.6.4) Geographical position information shall always use a balise group as geographical position reference balise group and if needed an offset from that balise group. A geographical position reference balise group shall be either: a) part of the last reported balise groups memorised on-board, in case the information is received from radio, OR b) the balise group transmitting the information, in case the information is received from balise group, OR c) any balise group not yet passed at the time of reception of the information. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 11 / 12 / 13 | FS / OS / SR / UN / SB / TR / NL / LS / SN |
| Radio communication session | ESTABLISHED | - |
| Level Transition Order | NOT STORED | L2 / L3 |
| ERTMS/ETCS level | 0 / 1 / 2 | L0 / LNTC / L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0/LNTC/L1 | FS/OS/SR/UN/SB/TR/ NL/LS/SN | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 79 is received | I | RTM | The packet is REJECTED, since no order to switch to L2/3 at a further location has been received | L0/LNTC/L1 | FS/OS/SR/UN/SB/TR/ NL/LS/SN | - |
| 2 | L0/LNTC/L1 | FS/OS/SR/UN/SB/TR/ NL/LS/SN | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | Message is recorded | L0/LNTC/L1 | FS/OS/SR/UN/SB/TR/ NL/LS/SN | - |
| 3 | L0/LNTC/L1 | FS/OS/SR/UN/SB/TR/ NL/LS/SN | USE_FT4060300.19 | - | - | Only UN or SN. The train changes to L1/FS | L0/LNTC/L1 | FS/OS/SR/UN/SB/TR/ NL/LS/SN | - |
| 4 | L1 | FS/OS/SR/SB/TR/ NL/LS | One Balise Group message containing packet 41 (D_LEVELTR=0; M_LEVELTR=3) is received | I | BTM | If comminig from LNTC. Transition to L2 | L2 | FS/OS/SR/SB/TR/ NL/LS | - |
| 5 | L2 | FS/OS/SR/SB/TR/ NL/LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | If comminig from LNTC. Transition to L2 | L2 | FS/OS/SR/SB/TR/ NL/LS | - |
| 6 | L2 | FS/OS/SR/SB/TR/ NL/LS | The Level symbol "Level 2" is displayed | O | DMI | If comminig from LNTC. Transition to L2 | L2 | FS/OS/SR/SB/TR/ NL/LS | - |
| 7 | L2 | FS/OS/SR/SB/TR/ NL/LS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit04=1>) is recorded | O | JRU | If comminig from LNTC. Transition to L2 | L2 | FS/OS/SR/SB/TR/ NL/LS | - |
| 8 | L2 | FS/OS/SR/SB/TR/ NL/LS | NOT The Driver Request symbol "Geographical Position" is displayed | O | DMI | - | L2 | FS/OS/SR/SB/TR/ NL/LS | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 79 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | Geographical Position Reference Balise Group. |
| D_POSOFF | 15 | FINITE VALUE | - |
| Q_MPOSITION | 1 | FINITE VALUE | Geographical Position counting direction. |
| M_POSITION | 24 | FINITE VALUE | Track kilometre reference value. |
| N_ITER | 5 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | Geographical Position Reference Balise Group. |
| D_POSOFF(k) | 15 | FINITE VALUE | - |
| Q_MPOSITION(k) | 1 | FINITE VALUE | Geographical Position counting direction. |
| M_POSITION(k) | 24 | FINITE VALUE | Track kilometre reference value. |

| Step 4: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 41 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | 0 | - |
| M_LEVELTR | 3 | 3 | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTR = 1 (NTC). |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | 255 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 7 / 11 / 12 | FS / OS / SR / SB / TR / NL / LS |
| ERTMS/ETCS level | 3 | L2 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 6

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Geographical Position |  | 4080415 |
| Test Case of Feature | Transmission of a list geographical position information from the RBC together with standard message 24. The packet is ACCEPTED, because the current level is not L2/3 and an announcement to change to L2/3 has been received on-board. |  | 6 |
| Applicable Mode/Level Combinations | L0 : UN, SB, NL LNTC: SB, TR, NL, SN L1 : FS, LS, OS, SR, SB, TR, NL |  |  |
| Target of Test | The target of this test is to check the correct working, that is: A geographical position information is received and accepted by the on-board system. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.3.1.1 Geographical Position |  |
|  |  | Subset-026-4.8.4.2 Geographical Position |  |
|  |  | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-4.8.3.1.1 Exception [2] |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |
|  |  | Subset-026-7.4.2.25 Packet Number 79: Geographical Position Information |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check that : The on-board accepts the message Check on JRU that : A message sent from RBC containing a geographical position information list has been received and is recorded. |
| Constraints | In SB mode, a cab needs to be active. An announcement to change to L2/3 has been received on-board. (Req. 3.6.6.4) Geographical position information shall always use a balise group as geographical position reference balise group and if needed an offset from that balise group. A geographical position reference balise group shall be either: a) part of the last reported balise groups memorised on-board, in case the information is received from radio, OR b) the balise group transmitting the information, in case the information is received from balise group, OR c) any balise group not yet passed at the time of reception of the information. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 11 / 12 / 13 | FS / OS / SR / UN / SB / TR / NL / LS /SN |
| Level Transition Order | STORED | L2 / L3 |
| ERTMS/ETCS level | 0 / 1 / 2 | L0 / LNTC / L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0/LNTC/L1 | FS/OS/SR/SB/UN/TR/NL/LS/SN | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 79 is received | I | RTM | The complete message is accepted by the on-board equipment. | L0/LNTC/L1 | FS/OS/SR/SB/UN/TR/NL/LS/SN | - |
| 2 | L0/LNTC/L1 | FS/OS/SR/SB/UN/TR/NL/LS/SN | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | Message is recorded | L0/LNTC/L1 | FS/OS/SR/SB/UN/TR/NL/LS/SN | - |
| 3 | L0 | SB/UN/TR/NL | USE_FT5100100.13,26: for changing to L2/3 | - | - | If the current level is L0 | L2/L3 | FS/OS/SR/SB/TR/NL/LS | - |
| 4 | LNTC | SB/TR/NL/SN | USE_FT5100100.21,20: for changing to L2/3 | - | - | If the current level is LNTC | L2/L3 | FS/OS/SR/SB/TR/NL/LS | - |
| 5 | L1 | FS/OS/SR/SB/TR/NL/LS | USE_FT5100100.37,36: for changing to L2/3 | - | - | If the current level is L1 | L2/L3 | FS/OS/SR/SB/TR/NL/LS | - |
| 6 | L2/L3 | FS/OS/SR/SB/TR/NL/LS | The Driver Request symbol "Geographical Position" is displayed | O | DMI | After the train front end has passed over the location specified by D_POSOFF value. | L2/L3 | FS/OS/SR/SB/TR/NL/LS | - |
| 7 | L2/L3 | FS/OS/SR/SB/TR/NL/LS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit83=1>) is recorded | O | JRU | Bit83: Symbol DR03 (the symbol DR03 shall be shown in G12 if the geographical position is toggled off) | L2/L3 | FS/OS/SR/SB/TR/NL/LS | - |
| 8 | L2/L3 | FS/OS/SR/SB/TR/NL/LS | The driver presses "Geographical Position" | I | DMI | - | L2/L3 | FS/OS/SR/SB/TR/NL/LS | - |
| 9 | L2/L3 | FS/OS/SR/SB/TR/NL/LS | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS = 0001 1110) is recorded | O | JRU | M_DRIVERACTIONS = 0001 1110: Request to show geographical position | L2/L3 | FS/OS/SR/SB/TR/NL/LS | - |
| 10 | L2/L3 | FS/OS/SR/SB/TR/NL/LS | The "Geographical Position" is displayed | O | DMI | According to the M_POSITION | L2/L3 | FS/OS/SR/SB/TR/NL/LS | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 79 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | Geographical Position Reference Balise Group. |
| D_POSOFF | 15 | FINITE VALUE | - |
| Q_MPOSITION | 1 | FINITE VALUE | Geographical Position counting direction. |
| M_POSITION | 24 | FINITE VALUE | Track kilometre reference value. |
| N_ITER | 5 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | Geographical Position Reference Balise Group. |
| D_POSOFF(k) | 15 | FINITE VALUE | - |
| Q_MPOSITION(k) | 1 | FINITE VALUE | Geographical Position counting direction. |
| M_POSITION(k) | 24 | FINITE VALUE | Track kilometre reference value. |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 7 / 8 / 11 / 12 | FS / OS / SR / SB / TR / PT / NL / LS |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 7

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Geographical Position |  | 4080415 |
| Test Case of Feature | The onboard equipment receives a list of geographical position information via balise group. The packet is REJECTED because the current level is LNTC and no announcement to change to L1 nor L2/3 has been received on-board |  | 7 |
| Applicable Mode/Level Combinations | LNTC: SB, TR, NL, SN |  |  |
| Target of Test | The target of this test is to check that the: Geographical position information via balise group or an empty list is received and rejected by the on-board system, because the current level obliges to reject this information. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.3.1.1 Geographical Position |  |
|  |  | Subset-026-4.8.4.2 Geographical Position |  |
|  |  | Subset-026-7.4.2.25 Packet Number 79: Geographical Position Information |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check that : No geographical position information is stored onboard Check on JRU that : A Geographical position information via balise group containing a list of balises (or empty list) has been received and is recorded. If the current mode is SN, the on-board equipment changes to SB. Check on the DMI that : No geographical position is diplayed after the driver requests it |
| Constraints | No order to switch to L1 nor L2/3 at a further location has been received. (Req. 3.6.6.4) Geographical position information shall always use a balise group as geographical position reference balise group and if needed an offset from that balise group. A geographical position reference balise group shall be either: a) part of the last reported balise groups memorised on-board, in case the information is received from radio, OR b) the balise group transmitting the information, in case the information is received from balise group, OR c) any balise group not yet passed at the time of reception of the information. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 / 7 / 11 / 13 | SB / TR / NL / SN |
| Level Transition Order | NOT STORED | L1 / L2 / L3 |
| ERTMS/ETCS level | 1 | LNTC |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | LNTC | SB/TR/NL/SN | One Balise Group message (N_TOTAL=1; NID_C=<comment>) containing packet 79 : ‘geographical position information’ is received | I | BTM | The packet is REJECTED. | LNTC | SB/TR/NL/SN | - |
| 2 | LNTC | SB/TR/NL/SN | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | Packet and telegram are recorded. | LNTC | SB/TR/NL/SN | - |
| 3 | LNTC | SB/TR/NL/SN | USE_FT4060300.9: The EVC changes to SB mode after the train front end has passed over the location specified by D_POSOFF value | . | . | If the current mode is SN, the on-board equipment changes to SB (Transition [28]) | LNTC | SB | - |
| 4 | LNTC | SB | Cab active | I | TIU | If cab is not active | LNTC | SB | - |
| 5 | LNTC | SB | NOT The Driver Request symbol "Geographical Position" is displayed | O | DMI | - | LNTC | SB | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0) Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system. |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M_MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of the balise group. |
| NID_C | 10 | FINITE VALUE | Country or region. |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group. |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 79 | Any optional packet and common optional packets ..referring to SRS SRS 8.4.4.4.1 and 8.4.4.6.1 Packet 79 in this feature |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | 0 cm, 1m or 10 m depends on Q_SCALE |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1 |
| NID_BG | 14 | FINITE VALUE | Geographical Position Reference Balise Group |
| D_POSOFF | 15 | FINITE VALUE | - |
| Q_MPOSITION | 1 | FINITE VALUE | Geographical Position counting direction |
| M_POSITION | 24 | FINITE VALUE | Track kilometre reference value |
| N_ITER | 5 | 1 | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1 |
| NID_BG(k) | 14 | FINITE VALUE | Geographical Position Reference Balise Group |
| D_POSOFF(k) | 15 | FINITE VALUE | - |
| Q_MPOSITION(k) | 1 | FINITE VALUE | Geographical Position counting direction |
| M_POSITION(k) | 24 | FINITE VALUE | Track kilometre reference value |
| NID_PACKET | 8 | 255 | 255 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| ERTMS/ETCS level | 1 | LNTC |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |