| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 3080400 (3.8.4: Use of the MA on-board the train) Total: 10 Test case(s) |  |
| REF: | Subset-076-5-2-3080400 |
| ISSUE: | Version 3.2.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 2.9.0 31.03.2013 | All | First version/review | DLR / CEDEX |
| Version 2.9.1 06.09.2013 | All | review | DLR / CEDEX |
| Version 2.9.2 12.12.2013 | All | review | DLR / CEDEX |
| Version 2.9.3 17.12.2013 | All | review | DLR / CEDEX |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.2.0 31.03.2017 | All | Baseline 3 1st maintenance release version - Bug fixing | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.8.4: Use of the MA on-board the train |  | 3080400 |
| Test Case of Feature | The on board starts a timer for the speed at the LoA when it detects that the train is passing over the first encountered balise of the balise group giving the MA or at the value of the time stamp of the radio message that includes the MA. When the timer expires the speed associated to the Limit of Authority is set to zero. |  | 1 |
| Applicable Mode/Level Combinations | L1: FS/LS/OS L2: FS/LS/OS L3: FS/LS/OS |  |  |
| Target of Test | The target of this test is to check that: • When the timer related to the LoA expires, the speed associated to the End of Authority is set to zero, so the LoA becomes the EoA |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.8.4.3.1 |  |
|  |  | Subset-026-3.8.4.3.1 a) |  |
|  |  | Subset-026-3.8.4.3.1 b) |  |
|  |  | Subset-026-3.8.4.3.2 |  |
|  |  | Subset-026-4.5.2.1 table - Determine EOA/LOA, SvL, Danger Point, etc… |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : • When the timer expires, the target speed is updated and set to zero. (This Step does not apply in LS mode.) Check on the JRU that: • When the timer expires, the target speed recorded is updated and set to zero. • The time difference between the reception of the MA and the expiration of the timer is the value received in the message. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| ERTMS/ETCS level | 2 / 3 | L1 / L2 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Target Speed is DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (N_TOTAL>=1; M_DUP=0; Q_LINK=1) containing packet 12 (Q_DIR=10; V_LOA≠0) is received | I | BTM | The test sequence shall be run twice: First run starting with step 1 (for ETCS Level 1) and proceeding with step 3. A timer T_LOA is started at the time of passage over the first encountered balise of the balise group giving the MA. t=to | N/A | N/A | - |
| 2 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 15 (V_LOA≠0) | I | RTM | The test sequence shall be run twice: Second run starting with step 2 (for ETCS Level 2) and proceeding with step 4. A timer T_LOA is started at the time stamp included in the MA. t=to | N/A | N/A | - |
| 3 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | This step shall be excecuted only in the first run, after step 1 has been excecuted | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | This step shall be excecuted only in the second run after step 2 has been excecuted | N/A | N/A | - |
| 5 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | At t=to+T_LOA, the timer expires, so the speed associated to the Limit of Authority is set to zero This Step does not apply in LS mode. | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/>001) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | >001 | Total number of balises in the balise group |
| M_DUP | 2 | 0 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 12 | Level 1 Movement Authority |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | - |
| V_LOA | 7 | > 0 | MA includes a LoA |
| T_LOA | 10 | < 1023 | T_LOA shall be greater than 0 but smaller than 1023 |
| N_ITER | 5 | > 0 | At least the current section where the train is running and another following one. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 2: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | Level 2/3 Movement Authority |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_LOA | 7 | > 0 | MA includes a LoA |
| T_LOA | 10 | < 1023 | T_LOA shall be greater than 0 but smaller than 1023 |
| N_ITER | 5 | FINITE VALUE | At least the current section where the train is running and another following one. |
| L_SECTION(k) | 15 | FINITE VALUE | The next section |
| Q_SECTIONTIMER(k) | 1 | 1 | Section information timer to follow |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| ERTMS/ETCS level | 2 / 3 | L1 / L2 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The speed and distance monitoring information is displayed | O | DMI | This Step does not apply in LS mode. |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.8.4: Use of the MA on-board the train |  | 3080400 |
| Test Case of Feature | A train with a valid MA including N sections with time outs, proceeding or standing in section k, in advance of the section k Time-out stop location (k ranges from 1 to N-1) requests a new MA a defined time before the time out of section k+1 expires and continue to request it after that time out expired (until the train receives a new MA from RBC) |  | 2 |
| Applicable Mode/Level Combinations | L2 :FS/LS/OS |  |  |
| Target of Test | The target of this test is to check that a train with a MA of N sections (N= K+1) with time outs, proceeding or standing in section k. The front end has passed the section k Time-out stop location. At the proper moment the on-board starts to request new MAs cyclically (each T_CYCRQST SECONDS), and continues to request it even after the time out of section k+1 expired |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.8.2.1 |  |
|  |  | Subset-026-3.8.2.3 |  |
|  |  | Subset-026-3.8.2.3 b) |  |
|  |  | Subset-026-3.8.2.3.1 |  |
|  |  | Subset-026-3.8.2.8 |  |
|  |  | Subset-026-3.8.2.8 c) |  |
|  |  | Subset-026-3.8.2.8 d) |  |
|  |  | Subset-026-3.8.4.2.1 |  |
|  |  | Subset-026-3.8.4.2.1 a) |  |
|  |  | Subset-026-7.4.2.14 Packet Number 57: MA request Parameters |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the Juridical recorder messages exchanged with RBC before the starting of the test Calculate the time [T_SECTIONTIMER (K+1) - T_TIMEOUTRQST ], drive the train in such a way that the min front end has passed the section k Time-out stop location when the [T_SECTIONTIMER (K+1) – T_TIMEOUTRQST] expires - Check on JRU that the message 132 is sent by the train to RBC when the train time reaches the value T = [ T_TRAINlast received MA + T_SECTIONTIMER(K+1) - T_TIMEOUTRQST] - Check on JRU that the message 132 is repeated each interval T_CYCRQST until other condition (MA received) breaks the cyclic request |
| Constraints | The test is preceded by a situation with the following conditions: • the reception onboard of a MA of N sections (message 3) • the reception onboard from RBC of packet 57 (MA Request parameters)... • onboard reports valid position to RBC (messages recorded on JRU) • a counter starting jointly with the time out of section k+1 reaches the value : [T_SECTION TIMER (K+1) - T_TIMEOUTRQST] : • the train has passed the section k Time-out stop location and the section Time-out stop location of section k+1 is in front of the min safe front end of the train. • No time out related to the previous sections (ranging from 1 to k) expired, so the track description has not been deleted and the MA has not been shortened • The Timer for section k+1 expires before the train has passed the section Time-out stop location of section k+1. • The On-board has deleted the track-side description (Q_MARQSTREASON = x1xxx) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 | L2 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3; T_TRAIN=<T_0>) containing packet 15, packet 57 is received | I | RTM | At T_0 the on-board starts a section timer for each section. | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | Train is moving (V_TRAIN > 0) | I | INT | Train has passed the section k Time-out stop location, but not the Time-out stop location for section k+1. No time-out related to the sections ranging from 1 to k expired. The section timer for section k+1 reaches the value [T_SECTION TIMER (K+1) - T_TIMEOUTRQST] | N/A | N/A | - |
| 4 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The target distance value must be consistent with the assumption: the train is in section k, has passed the section k Time-out stop location and the min safe front end is in front of the section k+1 Time-out stop location. This Step does not apply in LS mode. | N/A | N/A | - |
| 5 | N/A | N/A | SA-DATA.Request with Euroradio Message "MA Request" (NID_MESSAGE=132; Q_MARQSTREASON=<xx1xx>) containing packet 0 is transmitted | O | RTM | MA Request according to the description of step 3 is sent here. Track description has not been deleted yet. | N/A | N/A | - |
| 6 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | SA-DATA.Request with Euroradio Message "MA Request" (NID_MESSAGE=132; Q_MARQSTREASON=<xx1xx>) containing packet 0 is transmitted | O | RTM | Check that the difference of time between the first (step 5) and the second (step 7) MA requests is effectively equal to T_CYRSQST. | N/A | N/A | - |
| 8 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | Train is moving (V_TRAIN > 0) | I | INT | T_SECTIONTIMER (K+1) expires MA is shortened to the entry point of section k+1. Target distance, permanently displayed, changes suddenly its value (to the entry point of section k+1) The train has not passed the section k+1 Time-our stop location. Use_FT9990400.19 for the exhaustive list of location based information stored on-board, which shall be deleted accordingly | N/A | N/A | - |
| 10 | N/A | N/A | SA-DATA.Request with Euroradio Message "MA Request" (NID_MESSAGE=132) containing packet 0 is transmitted | O | RTM | As intervals of T_CYCRQST seconds expire, the train continues to request MA in L2 until it receives a MA or until the driver selects override. Check that the difference of time between two consecutive MA requests is effectively equal to T_CYRSQST. The value for Q_MARQSTREASON is to be clarified by ERA clarification request 147. | N/A | N/A | - |
| 11 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | 1 | with time out, at least for i= k and i = k+1 with k ε {1,..,N-1,empty} |
| T_SECTIONTIMER(k) | 10 | KNOWN | at least for i= k and i = k+1 with k ε {1,..,N-1,empty} |
| D_SECTIONTIMERSTOPLOC(k) | 15 | KNOWN | at least for i= k and i = k+1 with k ε {1,..,N-1,empty} |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 57 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| T_MAR | 8 | 255 | To aviod MA requests for other conditions |
| T_TIMEOUTRQST | 10 | < 1023 | - |
| T_CYCRQST | 8 | <255 | To avoid use of fixed value from A3.1 (T_CYCRQST ≠ 60), to ensure the use of stored MA request parameter |

| Step 5: Radio Message 132 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 132 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | Check that ME requests are sent periodically every T_CYCRQST seconds |
| NID_ENGINE | 24 | FINITE VALUE | - |
| Q_MARQSTREASON | 5 | xx1xx | - |
| NID_PACKET | 8 | 0 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | The min safe front end has not passed the Time-out stop location of End Section |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | 0 / 1 / 12 | FS / OS / LS |
| M_LEVEL | 3 | 3 | L2 |

| Step 7: Radio Message 132 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 132 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | Check that ME requests are sent periodically every T_CYCRQST seconds |
| NID_ENGINE | 24 | FINITE VALUE | - |
| Q_MARQSTREASON | 5 | xx1xx | - |
| NID_PACKET | 8 | 0 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | The min safe front end has not passed the Time-out stop location of End Section |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | 0 / 1 / 12 | FS / OS / LS |
| M_LEVEL | 3 | 3 | L2 |

| Step 10: Radio Message 132 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 132 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | Check that ME requests are sent periodically every T_CYCRQST seconds |
| NID_ENGINE | 24 | FINITE VALUE | - |
| Q_MARQSTREASON | 5 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | The min safe front end has not passed the Time-out stop location of End Section |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | 0 / 1 / 12 | FS / OS / LS |
| M_LEVEL | 3 | 3 | L2 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | STORED | Consistent with the shortened MA to the entry point of section k+1 |
| Train Position | STORED | - |
| ERTMS/ETCS level | 3 | L2 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The speed and distance monitoring information is displayed | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.8.4: Use of the MA on-board the train |  | 3080400 |
| Test Case of Feature | A train with a valid MA including N sections with time outs has passed the entry point of section k and the min safe front end has not passed the section k Time-out stop location (k ranges from 1 to N), stops to request a new MA when the time out of section k expires. EB is applied and the TR mode is entered |  | 3 |
| Applicable Mode/Level Combinations | L2 :FS/LS/OS |  |  |
| Target of Test | The target of this test is to check that if a train with a MA of N sections with time outs, is standing in section k. The front end has passed the section k entry point and the min safe front end has not passed the section k Time-out stop location. At the proper time (according to MA request parameters) the on-board starts to request MAs cyclically, and continue until the time out of section k expires. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.8.2.1 |  |
|  |  | Subset-026-3.8.2.3 |  |
|  |  | Subset-026-3.8.2.3 b) |  |
|  |  | Subset-026-3.8.2.8 |  |
|  |  | Subset-026-3.8.2.8 c) |  |
|  |  | Subset-026-3.8.4.2.1 |  |
|  |  | Subset-026-3.8.4.2.1 a) |  |
|  |  | Subset-026-7.4.2.14 Packet Number 57: MA request Parameters |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Calculate the time T = [ T_TRAINlast received MA + T_SECTIONTIMER (K) - T_TIMEOUTRQST SEC ]. The simulators of track and of train motion will drive the train in order to locate it in section k the section k Time-out stop location is not passed with the min safe front endat the starting time of the test. When the time T = [ T_TRAINlast received MA + T_SECTIONTIMER (K) - T_TIMEOUTRQST SEC ]. is reached - Check on RTM and JRU that the message 132 is sent by the train to RBC - Check on RTM and JRU that the message 132 is repeated all T_CYCRQST seconds until the time out of section k expires and the min safe frnt end of the train has not passed the Time-out stop location of section k |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 | L2 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2 | FS / OS / LS | SA-DATA.Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 15, packet 57 is received | I | RTM | MA request parameter stored on-board and T_MAR = 255 to make it not relevant in this test case | L2 | FS / OS / LS | - |
| 2 | L2 | FS / OS / LS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | L2 | FS / OS / LS | - |
| 3 | L2 | FS / OS / LS | Train is moving (V_TRAIN > 0) | I | INT | A counter starting jointly with the time out of section k reaches the value =[Q_SECTION TIMER (K) - 60 SEC] - the train has entered the section k, and of the min safe front end has not passed the section k Time-out stop location - No time out related to the previous sections ranging from 1 to the k-1 expired, so the track description has not been deleted and the MA has not been shortened - The On-board has not deleted the track-side description (Q_TRACKDEL is not 01000) | L2 | FS / OS / LS | - |
| 4 | L2 | FS / OS / LS | The speed and distance monitoring information is displayed | O | DMI | Target distance value must be consistent with the assumption that the train has entered section k and the min safe front end has not passed the Time-out stop location of section k . This Step does not apply in LS mode. | L2 | FS / OS / LS | - |
| 5 | L2 | FS / OS / LS | SA-DATA.Request with Euroradio Message "MA Request" (NID_MESSAGE=132) containing packet 0 is transmitted | O | RTM | As [T_SECTIONTIMER (K) - time interval set for starting to require cyclically the MA] expires, the onboard. Thereafter, new MA requests are issued every T_CYCRQST seconds. | L2 | FS / OS / LS | - |
| 6 | L2 | FS / OS / LS | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | This step shall be executed every for every MA request issued by the on-board. | L2 | FS / OS / LS | - |
| 7 | L2 | FS / OS / LS | Train is moving (V_TRAIN > 0) | I | INT | T_SECTIONTIMER (K) expires MA is shortened to the entry point of section k, because the min front end of the train has not passed the Time-out stop location: On-board equipment goes in TR mode Use FT9990400.19: Reset of supervision data when the stored MA is shortened due to a section time out. | L2 | TR | - |
| 8 | L2 | TR | The Mode symbol "Trip" is displayed | O | DMI | - | L2 | TR | - |
| 9 | L2 | TR | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit19=1>) is recorded | O | JRU | - | L2 | TR | - |
| 10 | L2 | TR | SA-DATA.Request with Euroradio message (NID_MESSAGE=136) containing packet 0 is transmitted | O | RTM | - | L2 | TR | - |
| 11 | L2 | TR | MESSAGETO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | L2 | TR | - |

| Step 1: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | 1 | - |
| T_SECTIONTIMER(k) | 10 | KNOWN | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | KNOWN | with k ε {1,..,N-1,empty} |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 57 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| T_MAR | 8 | 255 | To aviod MA requests for other conditions |
| T_TIMEOUTRQST | 10 | FINITE VALUE | - |
| T_CYCRQST | 8 | ≠ 60 | ≠ 60 to avoid the use of default value, because the packet 57 was received. |

| Step 5: Radio Message 132 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 132 | MA request |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| Q_MARQSTREASON | 5 | 100 | Time before a section timer/LOA speed timer expires reached |
| NID_PACKET | 8 | 0 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | 3 | L2 |
| M_LEVEL | 3 | 0 / 1 / 12 | FS / OS / LS |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| Step 10: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 | TR |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | UNKNOWN | Deleted |
| ERTMS/ETCS level | 3 | L2 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.8.4: Use of the MA on-board the train |  | 3080400 |
| Test Case of Feature | A train with a valid MA including N sections with time outs. The train is entering the end section and after the End Section timer value becomes greater than the time-out value given by trackside. The EOA/LOA shall be withdrawn to the current position of the train and a non-zero target speed value at the EOA/LOA shall be set to zero. |  | 4 |
| Applicable Mode/Level Combinations | L2: FS/LS/OS L3: FS/LS/OS |  |  |
| Target of Test | The target of this test is to check that a train with a MA of N sections (N= K+1) with time outs, the on-board shall withdraw the EOA/LOA to the current position of the train and a non-zero target speed value at the EOA/LOA shall be set to zero when the end section timer expires. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.8.4.1.2 |  |
|  |  | Subset-026-3.8.4.1.2 a) |  |
|  |  | Subset-026-3.8.4.1.2 b) |  |
|  |  | Subset-026-4.5.2.1 table - Determine EOA/LOA, SvL, Danger Point, etc… |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | The train has an MA stored Onboard with several sections within the MA. - The Variable Q_ENDTIMER is set, that the Onboard shall consider the Endtimer is set. The End Section timer value becomes greater than the time-out value given by trackside. The EOA/LOA shall be withdrawn to the current position of the train A non-zero target speed value at the EOA/LOA shall be set to zero. The train applies the brakes - Check on DMI that the End section value becomes greater than the time-out value by trackside. (Except in LS) - Check on DMI that the EOA/LOA will be withdrawn to the actual train position (Except in LS) - Check on DMI that the train applies the brakes and switches to TRIP Mode |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 | L2 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2 | FS / OS / LS | SA-DATA.Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) with packet 57 is received | I | RTM | - | L2 | FS / OS / LS | - |
| 2 | L2 | FS / OS / LS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | L2 | FS / OS / LS | - |
| 3 | L2 | FS / OS / LS | Train is moving (V_TRAIN > 0) | I | INT | A counter starting jointly with the time out of Endsection k reaches the value = [Q_ENDTIMER (K) - 60 SEC] The End Section timer value onboard becomes greater than the time-out value given by trackside. The EOA/LOA shall be withdrawn to the current position of the train. A non-zero target speed value at the EOA/LOA shall be set to zero | L2 | FS / OS / LS | - |
| 4 | L2 | TR | The distance monitoring information is removed | O | DMI | Target distance is removed This Step does not apply in LS mode. | L2 | FS / OS / LS | - |
| 5 | L2 | TR | The Mode symbol "Trip" is displayed @MinFront | O | DMI | On-board equipment goes in TR mode after the min safe front has passed the location where the ENDTIMER expired. | L2 | TR | - |
| 6 | L2 | TR | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; bit_and[DMI_SYMB_STATUS,MASK]=MASK) is recorded | O | JRU | MASK=0000000000000200000008 | L2 | TR | - |
| 7 | L2 | TR | SA-DATA.Request with Euroradio message (NID_MESSAGE=136) containing packet 0 is transmitted | O | RTM | - | L2 | TR | - |
| 8 | L2 | TR | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | L2 | TR | - |

| Step 1: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | Non-zero target speed value at the EOA/LOA |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | with time out, at least for i= k and i = k+1 with k ε {1,..,N-1,empty} |
| T_ENDTIMER | 10 | FINITE VALUE | at least for i= k and i = k+1 with k ε {1,..,N-1,empty} |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | at least for i= k and i = k+1 with k ε {1,..,N-1,empty} |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 57 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| T_MAR | 8 | FINITE VALUE | - |
| T_TIMEOUTRQST | 10 | FINITE VALUE | - |
| T_CYCRQST | 8 | FINITE VALUE | - |

| Step 7: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 | TR |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 | L2 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 5

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.8.4: Use of the MA on-board the train |  | 3080400 |
| Test Case of Feature | An on-board unit has a valid MA including N sections with no end section timer. The on-board receives a new MA with an End Section timer start location in rear of the max safe front end of the train. The EOA/LOA shall be withdrawn to the current position of the train and a non-zero target speed value at the EOA/LOA shall be set to zero. |  | 5 |
| Applicable Mode/Level Combinations | L2: FS/LS/OS |  |  |
| Target of Test | The target of this test is to check that If a train with a MA of N sections (N= K+1) and no End Section timer is running and the on-board receives a new MA with an End Section timer start location in rear of the max safe front end of the train, then the on-board unit shall withdraw The EOA/LOA to the current position of the train and a non-zero target speed value at the EOA/LOA, if any, shall be set to zero. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.8.4.1.3 |  |
|  |  | Subset-026-3.8.4.1.2 |  |
|  |  | Subset-026-3.8.4.1.2 a) |  |
|  |  | Subset-026-3.8.4.1.2 b) |  |
|  |  | Subset-026-4.6.3 Condition Id [16] |  |
|  |  | Subset-026-8.7.2 Message 3: Movement Authority |  |
|  |  | Subset-026-4.5.2.1 table - Determine EOA/LOA, SvL, Danger Point, etc… |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | The train has an MA stored Onboard with several sections within the MA. - The Variable Q_ENDTIMER is NOT SET, so that the Onboard shall consider the End Section timer is NOT SET. The on-board receives a new MA with an End Section timer start location in rear of the max safe front end of the trainThe EOA/LOA shall be withdrawn to the current position of the train. A non-zero target speed value at the EOA/LOA shall be set to zero. The train applies the brakes - Check on DMI that the End section value becomes greater than the time-out value by trackside. - Check on DMI that the EOA/LOA will be withdrawn to the actual train position - Check on DMI that the train applies the brakes and switches to TRIP Mode |
| Constraints | The test is preceded by: • the reception onboard of a MA of N sections (message 3) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | STORED Q_ENDTIMER(i)=0 | No End section timer information |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Train Position | STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Target Speed is DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 15 (V_LOA≠0) | I | RTM | The on-board receives a new MA (message 3) with an End Section timer start location in rear of the max safe front end of the train | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | FS / OS / LS | Use_FT4060300.24: The train is in OS/LS/FS and the train/engine overpasses the EOA/LOA of its MA. The system switches to TR Mode. Transition Id [16] | - | - | Immediately, the EOA/LOA shall be withdrawn to the current position of the train. A non-zero target speed value at the EOA/LOA shall be set to zero On-board equipment goes in TR mode Target distance disappears on the screen TR mode is displayed. | N/A | N/A | - |

| Step 1: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_LOA | 7 | >0 | Non-zero target speed value at the EOA/LOA |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | >0 | At least the current section where the train is running and another following one. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | Distance from beginning of section to the Section Time-out stop location. End Section timer start location must be in rear of the max safe front end of the train |
| Q_ENDTIMER | 1 | 1 | End section timer information to follow |
| T_ENDTIMER | 10 | 1023 | No time out |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | In rear of the max safe front end of the train |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 | TR |
| Radio communication session | ESTABLISHED | - |
| Geographical Position | VALID V_LOA=0 | The non-zero target speed value at the EOA/LOA has been set to zero |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| TR mode is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 6

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.8.4: Use of the MA on-board the train |  | 3080400 |
| Test Case of Feature | In case an End Section timer is already running when the on-board receives a new MA with an End Section timer start location in rear of the max safe front end of the train, the on-board shall keep the End Section timer running but replace the time-out value with the value received with the new MA. |  | 6 |
| Applicable Mode/Level Combinations | L2: FS/LS/OS |  |  |
| Target of Test | In case an End Section timer is already running and the on-board receives a new MA with an End Section timer start location in rear of the max safe front end of the train, then the on-board shall keep the End Section timer running but replace the time-out value with the value received with the new MA. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.8.4.1.4 |  |
|  |  | Subset-026-3.8.4.1.2 |  |
|  |  | Subset-026-3.8.4.1.2 a) |  |
|  |  | Subset-026-3.8.4.1.2 b) |  |
|  |  | Subset-026-4.6.3 Condition Id [16] |  |
|  |  | Subset-026-8.7.2 Message 3: Movement Authority |  |
|  |  | Subset-026-4.5.2.1 table - Determine EOA/LOA, SvL, Danger Point, etc… |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | - Check on DMI that, 5 seconds after reception of second MA (step 2), the End section value becomes greater than the time-out value by trackside so that - The EOA/LOA is withdrawn to the current position of the train. - A non-zero target speed value at the EOA/LOA is be set to zero |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Target Distance is DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 15 (T_ENDTIMER=1022) | I | RTM | The on-board receives a new MA (message 3) with a End Section timer time-out value of 1022. | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | L2 | FS / OS / LS | Train is moving (V_TRAIN > 0) @MaxFront | I | INT | The train reaches the End Section timer start location defined by D_ENDTIMERSTARTLOC in Step 1. | N/A | N/A | - |
| 4 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 15 (T_ENDTIMER=10) | I | RTM | Just 5 seconds after Step 3, the on-board receives a new MA (message 3) with an End Section timer start location in rear of the max safe front end of the train and with an End Section timer time-out value of 10. The on-board keeps the End Section timer running but replaces the time-out value with the value received with the new MA. | N/A | N/A | - |
| 5 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 6 | N/A | FS / OS / LS | Use_FT4060300.24: The train is in OS/LS/FS and the train/engine overpasses the EOA/LOA of its MA. The system switches to TR Mode. Transition Id [16] | - | - | 5 seconds after Step 4, the End Section timer value on-board becomes greater than the time-out value given by trackside. The EOA/LOA shall be withdrawn to the current position of the train. A non-zero target speed value at the EOA/LOA shall be set to zero On-board equipment goes in TR mode Target distance disappears on the screen TR mode is displayed | N/A | TR | - |

| Step 1: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | > 0 | At least the current section where the train is running and another following one. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | 1 | - |
| T_ENDTIMER | 10 | 1022 | Maximum value, in order to be able to check that it is overwritten at Step 3 |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | Distance from End section timer start location to End of Authority End Section timer start location MUST NOT be in rear of the max safe front end of the train |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |

| Step 4: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_LOA | 7 | >0 | Non-zero target speed value at the EOA/LOA |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | > 0 | At least the current section where the train is running and another following one. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | 1 | - |
| T_ENDTIMER | 10 | 10 | Just 10 seconds, so that the replacement of the old value by this one can be tested |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | Distance from End section timer start location to End of Authority End Section timer start location MUST be in rear of the max safe front end of the train |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 | TR |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | VALID V_LOA=0 | The non-zero target speed value at the EOA/LOA has been set to zero |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| TR mode is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 7

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.8.4: Use of the MA on-board the train |  | 3080400 |
| Test Case of Feature | 3.8.4.5.2 As long as a Limit of Authority is supervised, no SvL shall be defined on-board. |  | 7 |
| Applicable Mode/Level Combinations | L1: FS / LS / OS L2: FS / LS / OS L3: FS / LS / OS |  |  |
| Target of Test | If a Limit of Authority is supervised, no supervised location shall be display on the DMI in case no Danger Point and no overlap is stored onbard. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.8.4.5.2 |  |
|  |  | Subset-026-4.5.2.1 table - Determine EOA/LOA, SvL, Danger Point, etc… |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: -Current train speed pointer has the color of NoS (Normal Status information) |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Movement Authority | STORED | V_LOA > 0; Q_DANGERPOINT = 0; Q_OVERLAP = 0 |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 / L2 / L3 | FS / OS / LS | Train is moving (0 < V_TRAIN <= V_LOA) | I | INT | The train has passed the location from which on the color of the current train speed pointer would change from NoS (Normal Status information) to another, if an End Of Authority (EOA) would be defined (V_LOA = 0). The train did not pass the LOA location. | L1 / L2 / L3 | FS / OS / LS | - |
| 2 | L1 / L2 / L3 | FS / OS / LS | The current train speed pointer has the color for NoS. | O | DMI | - | L1 / L2 / L3 | FS / OS / LS | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Movement Authority | STORED | V_LOA > 0; Q_DANGERPOINT = 0; Q_OVERLAP = 0 |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 8

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.8.4: Use of the MA on-board the train |  | 3080400 |
| Test Case of Feature | The Supervised Location (SvL) shall be defined on-board as the end of overlap, when there is one. |  | 8 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS L2/L3: FS, LS, OS |  |  |
| Target of Test | The Supervised Location (SvL) shall be defined on-board as the end of overlap. The MA stored (and used) on-board will have a danger point as well, but the supervised release speed will be the one related to the overlap. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.8.4.5.1 |  |
|  |  | Subset-026-3.8.4.5.1 a) |  |
|  |  | Subset-026-4.5.2.1 table - Determine EOA/LOA, SvL, Danger Point, etc… |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | The DMI displays the correct value for the Release Speed. (Does not apply in LS mode.) |
| Constraints | The stored MA will have Q_DANGERPOINT=1, Q_OVERLAP=1 and V_RELEASEDP=127 (use national value V_NVREL). The stored V_RELEASEOL is different as the stored V_NVREL. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| National Values | STORED | V_NVREL = 40km/h (default value) |
| Movement Authority | STORED | Q_DANGERPOINT=1; Q_OVERLAP=1; V_RELEASEDP=127; V_RELEASEOL ≠ V_NVREL |
| Unconditional Emergency Stop | NOT STORED | - |
| Conditional Emergency Stop | NOT STORED | - |
| Adhesion factor | NOT STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is moving (V_TRAIN=<below V_RELEASEOL>) | I | INT | - | N/A | N/A | - |
| 2 | N/A | N/A | The driver requests the speed and distance monitoring information to be displayed | I | DMI | Only in OS mode | N/A | N/A | - |
| 3 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0001 1011) is recorded | O | JRU | Only in OS mode | N/A | N/A | - |
| 4 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The displayed release speed is equal to V_RELEASEOL. Check that the SvL is equal to the end of overlap and different as the danger point and to the EoA. This Step does not apply in LS mode. | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Movement Authority | STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 9

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.8.4: Use of the MA on-board the train |  | 3080400 |
| Test Case of Feature | The Supervised Location (SvL) shall be defined on-board as the Danger Point, when there is one and there is no Overlap. |  | 9 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS L2/L3: FS, LS, OS |  |  |
| Target of Test | The Supervised Location (SvL) shall be defined on-board as the Danger Point. The MA stored (and used) on-board will not have an Overlap, and the supervised release speed will be the one related to the Danger Point. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.8.4.5.1 |  |
|  |  | Subset-026-3.8.4.5.1 b) |  |
|  |  | Subset-026-4.5.2.1 table - Determine EOA/LOA, SvL, Danger Point, etc… |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | The DMI displays the correct value for the Release Speed. Does not apply in LS mode. |
| Constraints | The stored MA will have Q_DANGERPOINT=1, Q_OVERLAP=0 and V_RELEASEDP=127 (use national value V_NVREL). |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| National Values | STORED | V_NVREL = 40km/h (default value) |
| Movement Authority | STORED | Q_DANGERPOINT=1 and Q_OVERLAP=0; V_RELEASEDP=127 |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Virtual Balise Covers | NOT STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is moving (V_TRAIN=<below V_RELEASEDP>) | I | INT | - | N/A | N/A | - |
| 2 | N/A | N/A | The driver requests the speed and distance monitoring information to be displayed | I | DMI | Only in OS mode | N/A | N/A | - |
| 3 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0001 1011) is recorded | O | JRU | Only in OS mode | N/A | N/A | - |
| 4 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The displayed release speed is equal to V_NVREL (40 km/h). Check that the SvL is equal to the danger point and different as the EoA. This Step does not apply in LS mode. | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Movement Authority | STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 10

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.8.4: Use of the MA on-board the train |  | 3080400 |
| Test Case of Feature | The Supervised Location (SvL) shall be defined on-board as the the End Of Authority, when there is no Danger Point nor Overlap. |  | 10 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS L2/L3: FS, LS, OS |  |  |
| Target of Test | The Supervised Location (SvL) shall be defined on-board as the EoA. The MA stored (and used) on-board will not have a Danger Point nor Overlap, and there will not be supervised release speed. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.8.4.5.1 |  |
|  |  | Subset-026-3.8.4.5.1 c) |  |
|  |  | Subset-026-4.5.2.1 table - Determine EOA/LOA, SvL, Danger Point, etc… |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | The DMI displays no Release Speed. |
| Constraints | The stored MA will have Q_DANGERPOINT=0 and Q_OVERLAP=0. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Movement Authority | STORED | Q_DANGERPOINT=0; Q_OVERLAP=0 |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is moving (V_TRAIN=<V1>) | I | INT | - | N/A | N/A | - |
| 2 | N/A | N/A | The driver requests the speed and distance monitoring information to be displayed | I | DMI | Only in OS mode | N/A | N/A | - |
| 3 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0001 1011) is recorded | O | JRU | Only in OS mode | N/A | N/A | - |
| 4 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | No release speed is displayed This Step does not apply in LS mode. | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Movement Authority | STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |