| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 4080100 (4.8.1: Acceptance of received information) Total: 8 Test case(s) |  |
| REF: | Subset-076-5-2-4080100 |
| ISSUE: | Version 3.2.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 0.0.1 31.03.2013 | All | Creation | DLR |
| Version 2.9.0 31.03.2013 | All | First version/review for Baseline 3 | INECO |
| Version 2.9.1 01.08.2013 | All | Second version/Internal modification;TC_Editorv28; Correction of Requirements blank space | INECO |
| Version 2.9.2 24.09.2013 | All | Third version/ UNISIG review comments from 11.09.2013 included; FT renumbered. | INECO |
| Version 2.9.3 09.12.2013 | All | Fourth version/ UNISIG review comments from 28/11/2013included | INECO |
| Version 2.9.4 02.01.2014 | All | Fifth version/ Internal version: Cross-reference checked; Old reference removed. | INECO |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.2.0 31.03.2017 | All | Baseline 3 1st maintenance release version - Bug fixing | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.1: Acceptance of received information |  | 4080100 |
| Test Case of Feature | When evaluating trackside information received by radio or when re-evaluating a set of information released from the transition buffer, linking information, if any, shall be evaluated prior to any other location related information. (Linking evaluation). |  | 1 |
| Applicable Mode/Level Combinations | L0: UN L1: FS/ LS/ OS/ SR |  |  |
| Target of Test | The target of this test is to check that: A level transition to Level 2 is announced in BGa. No linking information is currently used. The on-board equipment receives some track description and linking information via radio, referring to BGa as LRBG. The length of the ISSP and GP is 10500m (this value is for clarity purposes, but it can be chosen freely). According to the L2 linking information (not yet used), the next BGb is 500m away from BGa. The on-board equipment passes over BGb when it has travelled 520m from BGa, and it receives an MA via radio referring to this BGb as LRBG. The length of the MA is 10000m The on-board reads the next announced BG (c), containing an order to switch to Level 2 immediately. At this point, the end of the MA, counted from BGb is, at least, at 10520m from the BGa, but the track description ends at 10500m from the BGa. So, the information would be REJECTED. Nevertheless, the linking information must be evaluated first, and then the MA and track description are recalculated and their lengths compared. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.1.6 |  |
|  |  | Subset-026-7.4.2.2 Packet Number 5: Linking |  |
|  |  | Subset-026-7.4.2.6 Packet Number 21: Gradient Profile |  |
|  |  | Subset-026-7.4.2.7 Packet Number 27: International Static Speed Profile |  |
|  |  | Subset-026-7.4.2.9 Packet Number 41: Level Transition Order |  |
|  |  | Subset-026-7.4.2.10 Packet Number 42: Session Management |  |
|  |  | Subset-026-8.7.2 Message 3: Movement Authority |  |
|  |  | Subset-026-8.7.4 Message 8: Acknowledgement of Train Data |  |
|  |  | Subset-026-8.7.9 Message 24: General Message |  |
|  |  | Subset-026-8.6.1 Message 129: Validated Train Data |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: Level transition announcement and order to Level 2 are DISPLAYED New MA is DISPLAYED Check on the RTM that: Radio communication session is ESTABLISHED Track description and linking information are RECEIVED and STORED till level transition execution Movement authority is RECEIVED and STORED till level transition execution Check on the JRU that : BGa message is RECORDED. Track description and linking information (sent via radio) are RECORDED after reading BGa. BGb is recorded a few meters away from the announced location. The MA (sent via radio) is RECORDED after reading BGb. BGc message is RECORDED. Level transition is RECORDED |
| Constraints | No linking information can be used until the train changes to Level 2. This is to prevent the train from relocating when passing the Balise Groups. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 12 | FS / OS / SR / UN / LS |
| Radio communication session | NOT ESTABLISHED | - |
| Linking | NOT STORED | - |
| ERTMS/ETCS level | 0 / 2 | L0 / L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / L1 | FS / OS / SR / UN / LS | D = d0 m One Balise Group message (N_TOTAL=1) containing packet 41 and packet 42 is received | I | BTM | Order to switch to level 2 at further location and order to establish a communication session. An announcement of level transition to Level 2 (Packet 41) received from Balise Group BGa contains: D_LEVLETR = 1040 m M_LEVELTR = 3 (Level 2) The value 1040m for D_LEVLETR is not compulsory, but it makes the TC more understandable (see the following steps). This BG becomes the LRBG. | L0 / L1 | FS / OS / SR / UN / LS | - |
| 2 | L0 / L1 | FS / OS / SR / UN / LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | Balise Group BGa message, including Packet 41(”Level transition order”), and Packet 42 (“Session management”) is RECORDED. | L0 / L1 | FS / OS / SR / UN / LS | - |
| 3 | L0 / L1 | FS / OS / SR / UN / LS | The Level symbol "Level 2 announcement" is displayed | O | DMI | DMI Object: Symbol LE12 | L0 / L1 | FS / OS / SR / UN / LS | - |
| 4 | L0 / L1 | FS / OS / SR / UN / LS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit12=1>) is recorded | O | JRU | - | L0 / L1 | FS / OS / SR / UN / LS | - |
| 5 | L0 / L1 | FS / OS / SR / UN / LS | SA-CONNECT. Request is transmitted | O | RTM | Mobile terminal is registered | L0 / L1 | FS / OS / SR / UN / LS | - |
| 6 | L0 / L1 | FS / OS / SR / UN / LS | SA-CONNECT. Confirm is received | I | RTM | - | L0 / L1 | FS / OS / SR / UN / LS | - |
| 7 | L0 / L1 | FS / OS / SR / UN / LS | SA-DATA. Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted | O | RTM | - | L0 / L1 | FS / OS / SR / UN / LS | - |
| 8 | L0 / L1 | FS / OS / SR / UN / LS | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | L0 / L1 | FS / OS / SR / UN / LS | - |
| 9 | L0 / L1 | FS / OS / SR / UN / LS | SA-DATA. Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32) is received | I | RTM | - | L0 / L1 | FS / OS / SR / UN / LS | - |
| 10 | L0 / L1 | FS / OS / SR / UN / LS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | Communication session established for on-board. | L0 / L1 | FS / OS / SR / UN / LS | - |
| 11 | L0 / L1 | FS / OS / SR / UN / LS | SA-DATA. Request with Euroradio Message "Session Established" (NID_MESSAGE=159) is transmitted. | O | RTM | - | L0 / L1 | FS / OS / SR / UN / LS | - |
| 12 | L0 / L1 | FS / OS / SR / UN / LS | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | Communication session established for trackside RBC ID/Phone Number are valid | L0 / L1 | FS / OS / SR / UN / LS | - |
| 13 | L0 / L1 | FS / OS / SR / UN / LS | SA-DATA. Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) is transmitted | O | RTM | - | L0 / L1 | FS / OS / SR / UN / LS | - |
| 14 | L0 / L1 | FS / OS / SR / UN / LS | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | L0 / L1 | FS / OS / SR / UN / LS | - |
| 15 | L0 / L1 | FS / OS / SR / UN / LS | SA-DATA. Indication with Euroradio Message "Acknowledgement of Train Data" (NID_MESSAGE=8) is received | I | RTM | The RBC acknowledges train data. | L0 / L1 | FS / OS / SR / UN / LS | - |
| 16 | L0 / L1 | FS / OS / SR / UN / LS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | L0 / L1 | FS / OS / SR / UN / LS | - |
| 17 | L0 / L1 | FS / OS / SR / UN / LS | SA-DATA. Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 21, packet 27, packet 5 is received | I | RTM | This message refers to BGa as LRBG. Packet 27 (finite profile: 10500m) Packet 21 (finite profile: 10500m) Packet 5, containing BGb with D_LINK = 500 BGc with D_LINK = 500 | L0 / L1 | FS / OS / SR / UN / LS | - |
| 18 | L0 / L1 | FS / OS / SR / UN / LS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | L0 / L1 | FS / OS / SR / UN / LS | - |
| 19 | L0 / L1 | FS / OS / SR / UN / LS | D = d0 + 520 m One Balise Group message (BGb; N_TOTAL=1) is received | I | BTM | No linking consistency is being checked at this point, but the BG is found in the expectation window - as if it was supervised according to the stored L2 linking information, although not yet used. | L0 / L1 | FS / OS / SR / UN / LS | - |
| 20 | L0 / L1 | FS / OS / SR / UN / LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L0 / L1 | FS / OS / SR / UN / LS | - |
| 21 | L0 / L1 | FS / OS / SR / UN / LS | SA-DATA. Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 15 is received | I | RTM | This message refers to BGb as LRBG. For FS / LS mode, Packet 80: Mode Profile is not included. For OS mode, Packet 80: Mode Profile is included. The train receives Message 3: Movement Authority, without any track description. (nominal length of the MA: 10000m) | L0 / L1 | FS / OS / SR / UN / LS | - |
| 22 | L0 / L1 | FS / OS / SR / UN / LS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | L0 / L1 | FS / OS / SR / UN / LS | - |
| 23 | L0 / L1 | FS / OS / SR / UN / LS | D = d0 + 1040 m One Balise Group message (BGc; N_TOTAL=1) containing packet 41 is received. | I | BTM | An order of level transition to Level 2 Packet 41: D_LEVELTR = 32767 (now) M_LEVELTR = 3 (Level 2) | L0 / L1 | FS / OS / SR / UN / LS | - |
| 24 | L0 / L1 | FS / OS / SR / UN / LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L0 / L1 | FS / OS / SR / UN / LS | - |
| 25 | L0 / L1 | FS / OS / SR / UN / LS | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=3) is recorded | O | JRU | - | L2 | FS / OS / LS | - |
| 26 | L0 / L1 | FS / OS / SR / UN / LS | The Level symbol "Level <L2>" is displayed | O | DMI | DMI Object: Symbol LE04 | L2 | FS / OS / LS | - |
| 27 | L2 | FS / OS / LS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit04=1>) is recorded | O | JRU | - | L2 | FS / OS / LS | - |
| 28 | L2 | FS / OS / LS | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=0/ 1/ 12) is recorded. | O | JRU | Optional step: The on-board equipment switches to FS (or LS or OS) mode if different than the previous one. From the set of information released from the transition buffer, linking information is evaluated prior to any other location related information. | L2 | FS / OS / LS | - |
| 29 | L2 | FS / OS / LS | The Mode symbol "FS or LS or OS" is displayed. | O | DMI | Optional step: This is only necessary if the previous mode is different than the applicable one. DMI Object: Symbol MO11/ MO21/MO07 | L2 | FS / OS / LS | - |
| 30 | L2 | FS / OS / LS | The new MA and track description is displayed. | O | DMI | The MA is recalculated, and its full length does not exceed the track description. | L2 | FS / OS / LS | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 010 0000 | Version 2.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | First |
| N_TOTAL | 3 | 1 | Two balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 41 | Level transition order |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | 1 | - |
| D_LEVELTR | 15 | 1040 | - |
| M_LEVELTR | 3 | 3 | Level 2 |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTR = 1 (NTC). |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 42 | Session Management |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_RBC | 1 | 1 | - |
| NID_C | 10 | FINITE VALUE | RBC ETCS identity : NID_C not relevant if NID_RBC has value "Contact last known RBC". |
| NID_RBC | 14 | FINITE VALUE | - |
| NID_RADIO | 64 | FINITE VALUE | Not relevant if NID_RBC has value "Contact last known RBC". |
| Q_SLEEPSESSION | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 1 | Second |
| N_TOTAL | 3 | 1 | Two balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 7: Radio Message 155 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 155 | Initiation of a communication session |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |

| Step 9: Radio Message 32 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 32 | RBC/RIU System Version |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |

| Step 11: Radio Message 159 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 159 | Session established |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |

| Step 13: Radio Message 129 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 129 | Validated Train data |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | This value is used by the trackside equipment to know each train in its area |
| NID_PACKET | 8 | 0 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |
| NID_PACKET | 8 | 11 | Validated train data |
| L_PACKET | 13 | FINITE VALUE | Related to NID_PACKET |
| NID_OPERATIONAL | 32 | FINITE VALUE | - |
| NC_TRAIN | 15 | FINITE VALUE | Default value Basic Static speed profile |
| L_TRAIN | 12 | FINITE VALUE | Default value unknown |
| V_MAXTRAIN | 7 | FINITE VALUE | - |
| M_LOADINGGAUGE | 2 | FINITE VALUE | One of 4 different profiles |
| M_AXLELOAD | 7 | FINITE VALUE | - |
| M_AIRTIGHT | 2 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_TRACTION | 8 | FINITE VALUE | Type of traction |
| M_VOLTAGE | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| NID_STM | 8 | FINITE VALUE | Type of STM available |

| Step 15: Radio Message 8 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 8 | Acknowledgement of Train data |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | 0 | No acknowledgement required |
| NID_LRBG | 10+14 | FINITE VALUE | BGa |

| Step 17: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | BGa |
| NID_PACKET | 8 | 21 | Gradient Profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | 0 | Incremental distance to next change of gradient |
| Q_GDIR | 1 | FINITE VALUE | - |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | 1 | - |
| D_GRADIENT(k) | 15 | 10500 | Incremental distance to next change of gradient |
| Q_GDIR(k) | 1 | FINITE VALUE | - |
| G_A(k) | 8 | 255 | Non numerical value telling that the current gradient description ends at D_GRADIENT (n) |
| NID_PACKET | 8 | 27 | Static Speed Profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | 0 | Incremental distance to next discontinuity in an ISSP |
| V_STATIC | 7 | FINITE VALUE | - |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| NC_DIFF(n) | 4 | FINITE VALUE | - |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | 1 | - |
| D_STATIC(k) | 15 | 10500 | Incremental distance to next discontinuity in an ISSP |
| V_STATIC(k) | 7 | 127 | Non numerical value telling that the ISSP description ends at D_STATIC (n) |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| NC_DIFF(k,m) | 4 | FINITE VALUE | - |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 5 | Linking |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LINK | 15 | 500 | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | If Q_NEWCOUNTRY = 1 |
| NID_BG | 14 | BGb | - |
| Q_LINKORIENTATION | 1 | FINITE VALUE | - |
| Q_LINKREACTION | 2 | FINITE VALUE | - |
| Q_LOCACC | 6 | 20 | High enough |
| N_ITER | 5 | FINITE VALUE | - |
| D_LINK (k) | 15 | 500 | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C (k) | 10 | FINITE VALUE | If Q_NEWCOUNTRY = 1 |
| NID_BG (k) | 14 | BGc | - |
| Q_LINKORIENTATION (k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION (k) | 2 | FINITE VALUE | - |
| Q_LOCACC (k) | 6 | 20 | High enough |

| Step 19: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 010 0000 | Version 2.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 0 | First |
| N_TOTAL | 3 | 1 | Two balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGb |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 19: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 010 0000 | Version 2.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 1 | Second |
| N_TOTAL | 3 | 1 | Two balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGb |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 21: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | Movement Authority |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | 0 | No ack |
| NID_LRBG | 24 | BGb | - |
| NID_PACKET | 8 | 15 | Level 2/3 MA |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| L_SECTION (k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | 10000 | As an example |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |

| Step 23: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 010 0000 | Version 2.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | First |
| N_TOTAL | 3 | 1 | Two balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGc |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 41 | Level transition order |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | 32767 | - |
| M_LEVELTR | 3 | 3 | Level 2 |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTR = 1 (NTC). |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 23: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 010 0000 | Version 2.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 001 | Second |
| N_TOTAL | 3 | 1 | Two balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGc |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / LS / OS |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| ERTMS/ETCS level | 3 | L2 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Level 2 and mode symbol displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.1: Acceptance of received information |  | 4080100 |
| Test Case of Feature | When evaluating trackside information received by radio or when re-evaluating a set of information released from the transition buffer, linking information, if any, shall be evaluated prior to any other location related information. (Linking evaluation). |  | 2 |
| Applicable Mode/Level Combinations | L0: UN L1: FS/ LS/ OS/ SR |  |  |
| Target of Test | The target of this test is to check that: A level transition to Level 2 is announced in BGa. No linking information is currently used. The on-board equipment receives an MA and linking information via radio, referring to BGa as LRBG. The length of this MA is 10500m (this value is for clarity purposes, but it can be chosen freely). According to the L2 linking information (not yet used), the next BGb is 480m away from BGa. The on-board equipment passes over BGb when it has travelled 500m from BGa, and it receives some track description via radio, referring to this BGb as LRBG. The length of the profiles (ISSP and GP) is 10000m. The on-board reads the next announced BG (c), containing an order to switch to Level 2 immediately. At this point, the end of the MA, counted from BGa, is at 10500m from the BGa, and the track description ends also at 10500m from the BGa. So, the information would be ACCEPTED. Nevertheless, the linking information must be evaluated first, and then the MA and track description are recalculated and their lengths compared (the MA ends at 10500m and the track description at 10480m from the BGa), so the train switches to TR mode if this TC is tested in L0/UN, but it will continue without changes if it is tested in FS, LS, OS or SR. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.1.6 |  |
|  |  | Subset-026-7.4.2.2 Packet Number 5: Linking |  |
|  |  | Subset-026-7.4.2.9 Packet Number 41: Level Transition Order |  |
|  |  | Subset-026-7.4.2.10 Packet Number 42: Session Management |  |
|  |  | Subset-026-8.7.2 Message 3: Movement Authority |  |
|  |  | Subset-026-8.7.4 Message 8: Acknowledgement of Train Data |  |
|  |  | Subset-026-8.7.9 Message 24: General Message |  |
|  |  | Subset-026-8.6.1 Message 129: Validated Train Data |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: Level transition announcement and order to Level 2 are DISPLAYED. The train switches to TR only if the mode is UN. If it is not the case, the train continues without any change. No new MA is DISPLAYED (depending on level/mode combination) Check on the RTM that: Radio communication session is ESTABLISHED Track description and linking information are RECEIVED and STORED till level transition execution Movement authority is RECEIVED and STORED till level transition execution Check on the JRU that : BGa message is RECORDED. The MA and linking information (sent via radio) are RECORDED after reading BGa. Track description information (sent via radio) is RECORDED after reading BGb. BGb is recorded a few meters away from the announced location. BGc message is RECORDED. Level transition is RECORDED |
| Constraints | The L1 MA being used (if any) will have its LoA some meters further than the level transition border. No linking information can be used until the train changes to Level 2. This is to prevent the train from relocating when passing the Balise Groups. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 12 | FS / OS / SR / UN / LS |
| Radio communication session | NOT ESTABLISHED | - |
| Linking | NOT STORED | - |
| ERTMS/ETCS level | 0 / 2 | L0 / L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / L1 | FS / OS / SR / UN / LS | D = d0 m One Balise Group message (N_TOTAL=1) containing packet 41 and packet 42 is received | I | BTM | Order to switch to level 2 at further location and order to establish a communication session. Packet 41: D_LEVLETR = 1000 m M_LEVELTR = 3 (Level 2) The value 1000m for D_LEVLETR is not compulsory, but it makes the TC more understandable (see the following steps). The BGa becomes the LRBG. | L0 / L1 | FS / OS / SR / UN / LS | - |
| 2 | L0 / L1 | FS / OS / SR / UN / LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L0 / L1 | FS / OS / SR / UN / LS | - |
| 3 | L0 / L1 | FS / OS / SR / UN / LS | The Level symbol "Level 2 announcement" is displayed | O | DMI | DMI Object: Symbol LE12 | L0 / L1 | FS / OS / SR / UN / LS | - |
| 4 | L0 / L1 | FS / OS / SR / UN / LS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit12=1>) is recorded | O | JRU | - | L0 / L1 | FS / OS / SR / UN / LS | - |
| 5 | L0 / L1 | FS / OS / SR / UN / LS | SA-CONNECT. Request is transmitted | O | RTM | Mobile terminal is registered | L0 / L1 | FS / OS / SR / UN / LS | - |
| 6 | L0 / L1 | FS / OS / SR / UN / LS | SA-CONNECT. Confirm is received | I | RTM | - | L0 / L1 | FS / OS / SR / UN / LS | - |
| 7 | L0 / L1 | FS / OS / SR / UN / LS | SA-DATA. Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted | O | RTM | - | L0 / L1 | FS / OS / SR / UN / LS | - |
| 8 | L0 / L1 | FS / OS / SR / UN / LS | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | L0 / L1 | FS / OS / SR / UN / LS | - |
| 9 | L0 / L1 | FS / OS / SR / UN / LS | SA-DATA. Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32) is received | I | RTM | - | L0 / L1 | FS / OS / SR / UN / LS | - |
| 10 | L0 / L1 | FS / OS / SR / UN / LS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | Communication session established for on-board. | L0 / L1 | FS / OS / SR / UN / LS | - |
| 11 | L0 / L1 | FS / OS / SR / UN / LS | SA-DATA. Request with Euroradio Message "Session Established" (NID_MESSAGE=159) is transmitted. | O | RTM | - | L0 / L1 | FS / OS / SR / UN / LS | - |
| 12 | L0 / L1 | FS / OS / SR / UN / LS | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | Communication session established for trackside RBC ID/Phone Number are valid | L0 / L1 | FS / OS / SR / UN / LS | - |
| 13 | L0 / L1 | FS / OS / SR / UN / LS | SA-DATA. Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) is transmitted | O | RTM | - | L0 / L1 | FS / OS / SR / UN / LS | - |
| 14 | L0 / L1 | FS / OS / SR / UN / LS | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | L0 / L1 | FS / OS / SR / UN / LS | - |
| 15 | L0 / L1 | FS / OS / SR / UN / LS | SA-DATA. Indication with Euroradio Message "Acknowledgement of Train Data" (NID_MESSAGE=8) is received | I | RTM | The RBC acknowledges train data. | L0 / L1 | FS / OS / SR / UN / LS | - |
| 16 | L0 / L1 | FS / OS / SR / UN / LS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | L0 / L1 | FS / OS / SR / UN / LS | - |
| 17 | L0 / L1 | FS / OS / SR / UN / LS | SA-DATA. Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 15 and packet 5 is received | I | RTM | The train receives Message 3: Movement Authority, without any track description. (nominal length of the MA: 10500m) Packet 5 is also included, containing BGb with D_LINK = 480 BGc with D_LINK = 480 This message refers to BGa as LRBG. For OS mode, Packet 80: Mode Profile is included. | L0 / L1 | FS / OS / SR / UN / LS | - |
| 18 | L0 / L1 | FS / OS / SR / UN / LS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | L0 / L1 | FS / OS / SR / UN / LS | - |
| 19 | L0 / L1 | FS / OS / SR / UN / LS | D = d0 + 500 m One Balise Group message ( BGb; N_TOTAL=1) is received | I | BTM | No linking consistency is being checked at this point, but the BG is found in the expectation window - as if it was supervised according to the stored L2 linking information, although not yet used. | L0 / L1 | FS / OS / SR / UN / LS | - |
| 20 | L0 / L1 | FS / OS / SR / UN / LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L0 / L1 | FS / OS / SR / UN / LS | - |
| 21 | L0 / L1 | FS / OS / SR / UN / LS | SA-DATA. Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 21 and packet 27 is received | I | RTM | This message refers to BGb as LRBG. Packet 27 (finite profile: 10000m) Packet 21 (finite profile: 10000m) | L0 / L1 | FS / OS / SR / UN / LS | - |
| 22 | L0 / L1 | FS / OS / SR / UN / LS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | L0 / L1 | FS / OS / SR / UN / LS | - |
| 23 | L0 / L1 | FS / OS / SR / UN / LS | D = d0 + 1000 m One Balise Group message ( BGc; N_TOTAL=1) containing packet 41 is received | I | BTM | Packet 41: D_LEVLETR = 32767 (now) M_LEVELTR = 3 (Level 2) | L0 / L1 | FS / OS / SR / UN / LS | - |
| 24 | L0 / L1 | FS / OS / SR / UN / LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L0 / L1 | FS / OS / SR / UN / LS | - |
| 25 | L0 / L1 | FS / OS / SR / UN / LS | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=3) is recorded | O | JRU | - | L2 | FS / OS / SR / UN / LS | - |
| 26 | L2 | FS / OS / SR/ UN / LS | The Level symbol "Level <L2>" is displayed | O | DMI | DMI Object: Symbol LE04 | L2 | FS / OS / UN/ LS | - |
| 27 | L2 | FS / OS / UN / LS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit04=1>) is recorded | O | JRU | - | L2 | FS / OS / UN/ LS | - |
| 28 | L2 | FS / OS / LS | The same Mode symbol "FS/ LS / OS" is displayed. | O | DMI | The train continues being supervised without changing its mode (FS, OS or LS) Same DMI Symbol (MO11 or MO21 or MO07) From the set of information released from the transition buffer, linking information is evaluated prior to any other location related information. | L2 | FS / OS / LS | - |
| 29 | L2 | FS / OS / LS | The MA and track description is displayed without changes in the supervision limits. | O | DMI | The MA is recalculated, and its full length exceeds the track description. This means that the Level 2 MA is not used after changing to Level 2. Step not applicable in LS mode. | L2 | FS / OS / LS | - |
| 30 | L2 | UN | The on-board equipment switches to TR mode (only if the current mode is UN). | - | - | Optional step. Only if previous level/mode was L0 /UN USE_FT4060300. 28,29 | L2 | TR | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 010 0000 | Version 2.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | First |
| N_TOTAL | 3 | 1 | Two balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 41 | Level transition order |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | 1 | - |
| D_LEVELTR | 15 | 1000 | 1000m |
| M_LEVELTR | 3 | 3 | Level 2 |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTR = 1 (NTC). |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 42 | Session Management |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_RBC | 1 | 1 | - |
| NID_C | 10 | FINITE VALUE | RBC ETCS identity : NID_C not relevant if NID_RBC has value "Contact last known RBC". |
| NID_RBC | 14 | FINITE VALUE | - |
| NID_RADIO | 64 | FINITE VALUE | Not relevant if NID_RBC has value "Contact last known RBC". |
| Q_SLEEPSESSION | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 1 | Second |
| N_TOTAL | 3 | 1 | Two balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 7: Radio Message 155 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 155 | Initiation of a communication session |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |

| Step 9: Radio Message 32 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 32 | RBC/RIU System Version |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |

| Step 11: Radio Message 159 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 159 | Session established |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |

| Step 13: Radio Message 129 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 129 | Validated Train data |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | This value is used by the trackside equipment to know each train in its area |
| NID_PACKET | 8 | 0 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |
| NID_PACKET | 8 | 11 | Validated train data |
| L_PACKET | 13 | FINITE VALUE | Related to NID_PACKET |
| NID_OPERATIONAL | 32 | FINITE VALUE | - |
| NC_TRAIN | 15 | FINITE VALUE | Default value Basic Static speed profile |
| L_TRAIN | 12 | FINITE VALUE | Default value unknown |
| V_MAXTRAIN | 7 | FINITE VALUE | - |
| M_LOADINGGAUGE | 2 | FINITE VALUE | One of 4 different profiles |
| M_AXLELOAD | 7 | FINITE VALUE | - |
| M_AIRTIGHT | 2 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_TRACTION | 8 | FINITE VALUE | Type of traction |
| M_VOLTAGE | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| NID_STM | 8 | FINITE VALUE | Type of STM available |

| Step 15: Radio Message 8 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 8 | Acknowledgement of Train data |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | 0 | No acknowledgement required |
| NID_LRBG | 10+14 | FINITE VALUE | BGa |

| Step 17: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | Movement Authority |
| L_MESSAGE | 10 | RESERVED | - |
| T_TRAIN | 32 | RESERVED | - |
| M_ACK | 1 | 0 | No ack |
| NID_LRBG | 24 | BGb | - |
| NID_PACKET | 8 | 15 | Level 2/3 MA |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | RESERVED | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| L_SECTION (k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | 10500 | As an example |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 5 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LINK | 15 | 480 | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | If Q_NEWCOUNTRY = 1 |
| NID_BG | 14 | BGb | - |
| Q_LINKORIENTATION | 1 | FINITE VALUE | - |
| Q_LINKREACTION | 2 | FINITE VALUE | - |
| Q_LOCACC | 6 | 20 | High enough |
| N_ITER | 5 | FINITE VALUE | - |
| D_LINK (k) | 15 | 480 | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C (k) | 10 | FINITE VALUE | If Q_NEWCOUNTRY = 1 |
| NID_BG (k) | 14 | BGc | - |
| Q_LINKORIENTATION (k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION (k) | 2 | FINITE VALUE | - |
| Q_LOCACC (k) | 6 | 20 | High enough |

| Step 19: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 010 0000 | Version 2.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | First |
| N_TOTAL | 3 | 1 | Two balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGb |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 19: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 1 | Second |
| N_TOTAL | 3 | 1 | Two balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGb |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 21: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | Gradient Profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | 0 | Incremental distance to next change of gradient |
| Q_GDIR | 1 | FINITE VALUE | - |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | 1 | - |
| D_GRADIENT(k) | 15 | 10000 | Incremental distance to next change of gradient |
| Q_GDIR(k) | 1 | FINITE VALUE | - |
| G_A(k) | 8 | 255 | Non numerical value telling that the current gradient description ends at D_GRADIENT (n) |
| NID_PACKET | 8 | 27 | Static Speed Profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | 0 | Incremental distance to next discontinuity in an ISSP |
| V_STATIC | 7 | FINITE VALUE | - |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| NC_DIFF(n) | 4 | FINITE VALUE | - |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | 1 | - |
| D_STATIC(k) | 15 | 10000 | Incremental distance to next discontinuity in an ISSP |
| V_STATIC(k) | 7 | 127 | Non numerical value telling that the ISSP description ends at D_STATIC (n) |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| NC_DIFF(k,m) | 4 | FINITE VALUE | - |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |

| Step 23: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 010 0000 | Version 2.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | First |
| N_TOTAL | 3 | 1 | Two balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGc |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 41 | Level transition order |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | 1 | - |
| D_LEVELTR | 15 | 32767 | 1000m |
| M_LEVELTR | 3 | 3 | Level 2 |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTR = 1 (NTC). |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 23: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 1 | Second |
| N_TOTAL | 3 | 1 | Two balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGc |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 7 /12 | FS / OS / TR / LS |
| Radio communication session | ESTABLISHED | - |
| Linking | STORED | - |
| ERTMS/ETCS level | 3 | L2 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Level 2 and mode symbol is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.1: Acceptance of received information |  | 4080100 |
| Test Case of Feature | Handling of transition buffer in case of level transition announcement. If a message contains level transition information, any other information in that message shall be evaluated considering the level transition information. |  | 3 |
| Applicable Mode/Level Combinations | L1: FS/ LS/ OS |  |  |
| Target of Test | The target of this test is to check that: When the on board equipment receives a level transition order for the current location and, in the same message, it receives information that would be rejected due to the current level (if the information had been received without the immediate transition order), the transition buffer is able to store this information, until the level transition is effective. After the level transition is done, the stored information is re-evaluated and then used. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.1.3 |  |
|  |  | Subset-026-4.8.1.3.1 |  |
|  |  | Subset-026-8.7.9 Message 24: General Message |  |
|  |  | Subset-026-7.4.2.9 Packet Number 41: Level Transition Order |  |
|  |  | Subset-026-7.4.2.23 Packet Number 72: Packet for sending plain text messages |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the RTM: An immediate level transition order to the current location is received together with plain text message information. Check on the DMI that: Train Level changes to L2/L3 when it receives the transition order. The Plain Text is DISPLAYED after the level change (General Message, with packet 72, was stored in the buffer) Check on the JRU that : RBC Messages 24 (general message), including packets 41 and 72, is RECORDED. The new level is RECORDED. The start displaying plain text is RECORDED |
| Constraints | MA and track description covering at least the first section of the level 2 / 3 area are being used. No level transition announcement is stored on-board. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Level Transition Order | NOT STORED | - |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 | FS / OS / LS | SA-DATA. Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 41 and packet 72 is received. | I | RTM | Immediate level transition (D_LEVELTR = 32767) Plain Text Message: Start to be displayed only in L2/L3 area. | L1 | FS / OS / LS | - |
| 2 | L1 | FS / OS / LS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | L1 | FS / OS / LS | - |
| 3 | L1 | FS / OS / LS | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=3/4) is recorded | O | JRU | - | L2 / L3 | FS / OS / LS | - |
| 4 | L1 | FS / OS / LS | The Level symbol "Level <L2/L3>" is displayed | O | DMI | DMI Object: Symbol LE04/LE05 | L2 / L3 | FS / OS / LS | - |
| 5 | L2/ L3 | FS / OS / LS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit04=1¦Bit05=1>) is recorded | O | JRU | - | L2/ L3 | FS / OS / LS | - |
| 6 | L2/ L3 | FS / OS / LS | The train continues being supervised without changing its mode (FS, OS or LS) and without changes in the supervision limits. | O | DMI | DMI Object: Symbol MO11 (or symbol MO21 or MO07) Information received in the same message as an immediate level transition order or a conditional level transition order that causes a level transition shall be evaluated first considering the on-board currently operated level, as if a level transition order for further location had been received. From the set of information released from the transition buffer, linking information is evaluated prior to any other location related information. | L2/ L3 | FS / OS / LS | - |
| 7 | L2/ L3 | FS / OS / LS | The "plain text message" is displayed | O | DMI | “TEST OK” | L2/ L3 | FS / OS / LS | - |
| 8 | L2/ L3 | FS / OS / LS | START DISPLAYING PLAIN TEXT MESSAGE (NID_MESSAGE_JRU=18) is recorded | O | JRU | - | L2/ L3 | FS / OS / LS | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 41 | Level Transition Order |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | 32767 | - |
| M_LEVELTR | 3 | 3 / 4 | Transition to level 2 or 3 |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| NID_PACKET | 8 | 72 | Packet for sending plain text messages |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TEXTCLASS | 2 | FINITE VALUE | - |
| Q_TEXTDISPLAY | 1 | 1 | Start/end events relation. |
| D_TEXTDISPLAY | 15 | FINITE VALUE | Start event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | Start event. |
| M_LEVELTEXTDISPLAY | 3 | 3 / 4 | Start event. Only in Level 2/3 |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| L_TEXTDISPLAY | 15 | FINITE VALUE | End event. |
| T_TEXTDISPLAY | 10 | FINITE VALUE | End event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | End event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | End event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| Q_TEXTCONFIRM | 2 | FINITE VALUE | - |
| Q_CONFTEXTDISPLAY | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| Q_TEXTREPORT | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| NID_TEXTMESSAGE | 8 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_C | 10 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_RBC | 14 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| L_TEXT | 8 | FINITE VALUE | - |
| X_TEXT(L_TEXT) | 8 | TEST OK | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Level Transition Order | STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level <L2/L3>" is displayed The "plain text message" is displayed | O | DMI | DMI Object: LE12/LE14 “TEST OK” |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.1: Acceptance of received information |  | 4080100 |
| Test Case of Feature | Handling of transition buffer in case of level transition announcement. If a message contains level transition information, any other information in that message shall be evaluated considering the level transition information. |  | 4 |
| Applicable Mode/Level Combinations | L2 / L3: FS / LS / OS |  |  |
| Target of Test | The target of this test is to check that: When the on board equipment receives a level transition order for current location and, in the same message, it receives information that would be rejected due to the current level (if the information had been received without the immediate transition order), the transition buffer is able to store this information, until the level transition is effective. After the level transition is done, the stored information is re-evaluated and then used. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.1.3.1 |  |
|  |  | Subset-026-4.8.1.3 |  |
|  |  | Subset-026-7.4.2.9 Packet Number 41: Level Transition Order |  |
|  |  | Subset-026-7.4.2.20 Packet Number 68: Track Condition |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI: Train Level changes to L1 when it receives the transition order. Track condition is DISPLAYED after the level change (Packet 68 was stored in the buffer). Check on the JRU: The BG message, including packets 41 and 68, is RECORDED. The new level is RECORDED. DMI symbols status are RECORDED |
| Constraints | MA and track description covering at least the first section of the level 1 area are being used No level transition announcement is stored on-board. No |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 /12 | FS / OS / LS |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Level Transition Order | NOT STORED | - |
| Track conditions excluding big metal masses | NOT STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2/L3 | FS / OS / LS | One Balise Group message (N_TOTAL=1) containing packet 41 and packet 68 is received | I | BTM | Immediate level transition (D_LEVELTR = 32767) Packet 68: M_TRACKCOND= 0001 | L2/L3 | FS / OS / LS | - |
| 2 | L2/L3 | FS / OS / LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L2/L3 | FS / OS / LS | - |
| 3 | L2/L3 | FS / OS / LS | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=2) is recorded | O | JRU | - | L1 | FS / OS / LS | - |
| 4 | L1 | FS / OS / LS | The Level symbol "Level <L1>" is displayed | O | DMI | DMI Object: Symbol LE03 | L1 | FS / OS / LS | - |
| 5 | L1 | FS / OS / LS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit03=1>) is recorded | O | JRU | - | L1 | FS / OS / LS | - |
| 6 | L1 | FS / OS / LS | The train continues being supervised without changing its mode (FS, OS or LS) and without changes in the supervision limits. | O | DMI | Symbol MO11 (or symbol MO21 or MO07) Information received in the same message as an immediate level transition order or a conditional level transition order that causes a level transition shall be evaluated first considering the on-board currently operated level, as if a level transition order for further location had been received. From the set of information released from the transition buffer, linking information is evaluated prior to any other location related information. | L1 | FS / OS / LS | - |
| 7 | L1 | FS / OS / LS | The Driver Request symbol " Toggling function for tunnel stopping area " is displayed | O | DMI | DMI Object: Symbol DR05 | L1 | FS / OS / LS | - |
| 8 | L1 | FS / OS / LS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit85=1>) is recorded | O | JRU | - | L1 | FS / OS / LS | - |
| 9 | L1 | FS / OS / LS | The driver requests to show tunnel stopping area information | I | DMI | - | L1 | FS / OS / LS | - |
| 10 | L1 | FS / OS / LS | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; MDRIVERATIONS = 0010 0111) is recorded | O | JRU | - | L1 | FS / OS / LS | - |
| 11 | L1 | FS / OS / LS | The Track Condition symbol "Tunnel stopping area announcement" is displayed | O | DMI | DMI Object: Symbol TC37 | L1 | FS / OS / LS | - |
| 12 | L1 | FS / OS / LS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit80=1>) is recorded | O | JRU | - | L1 | FS / OS / LS | - |
| 13 | L1 | FS / OS / LS | The Track Condition symbol "Tunnel stopping area" is displayed | O | DMI | DMI Object: Symbol TC36 | L1 | FS / OS / LS | - |
| 14 | L1 | FS / OS / LS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit79=1¦Bit80=0>) is recorded | O | JRU | - | L1 | FS / OS / LS | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 010 0000 | Version 2.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | First |
| N_TOTAL | 3 | 1 | Two balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 41 | Level Transition Order |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | 32767 | - |
| M_LEVELTR | 3 | 2 | Transition to level 1 |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| NID_PACKET | 8 | 68 | Track Condition |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | FINITE VALUE | - |
| D_TRACKINIT | 15 | FINITE VALUE | Only if Q_TRACKINIT = 1. |
| D_TRACKCOND | 15 | FINITE VALUE | Only if Q_TRACKINIT = 0, D_TRACKCOND and the following variables follow. |
| L_TRACKCOND | 15 | FINITE VALUE | - |
| M_TRACKCOND | 4 | 0001 | Tunnel stopping area. |
| N_ITER | 5 | FINITE VALUE | - |
| D_TRACKCOND(k) | 15 | FINITE VALUE | - |
| L_TRACKCOND(k) | 15 | FINITE VALUE | - |
| M_TRACKCOND(k) | 4 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 010 0000 | Version 2.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 001 | Second |
| N_TOTAL | 3 | 1 | Two balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Level Transition Order | STORED | - |
| Track conditions excluding big metal masses | NOT STORED | - |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level <L1>" is displayed The Track Condition symbol "Tunnel stopping area" is displayed | O | DMI | DMI Object: LE10 DMI Object: TC36 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 5

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.1: Acceptance of received information |  | 4080100 |
| Test Case of Feature | Infill information evaluated with other non-infill information. If a message contains infill information, this latter shall be evaluated considering all other non-infill information in that message. |  | 5 |
| Applicable Mode/Level Combinations | L1: FS / LS |  |  |
| Target of Test | The target of this test is to check that: The on-board equipment receives some non- infill (Packet 27) and infill information (Packet 12 and Packet 21) about track description via balises (BGa), referring to infill BGb . The track description covers full MA length. This infill information shall be evaluated considering other non-infill information in this message. When the new MA is given as infill information all data beyond the announced balise group at the next main signal shall be replaced. The on-board equipment passes over BGb and receives only Packet 255. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.8.4.6.2 |  |
|  |  | Subset-026-3.8.4.6.4 |  |
|  |  | Subset-026-4.8.1.5 |  |
|  |  | Subset-026-7.4.2.3 Packet Number 12: Level 1 Movement Authority |  |
|  |  | Subset-026-7.4.2.6 Packet Number 21: Gradient Profile |  |
|  |  | Subset-026-7.4.2.7 Packet Number 27: International Static Speed Profile |  |
|  |  | Subset-026-7.4.2.32 Packet Number 136: Infill location reference |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The non-infill track description is replaced by the information received from BGa The new MA and Gradient Profile (infill information) will replaced the data beyond the next main signal (BGb). Check on the JRU that : Track description and infill information are RECORDED after reading BGa. The displayed information is RECORDED. BGb message is RECORDED. |
| Constraints | Track description and infill information should be different from previous MA and Track description already stored. Linking information contained BGa and BGb shall be stored. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| Linking | STORED | About BGa and BGb |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (N_TOTAL=1) containing packet 27, packet 136, packet 12 and packet 21 is received. | I | BTM | The train receives a message with non-infill and infill information (referred to BGb) from BGa, containing several packets in the following order: Packet 27, Packet 136, Packet 12 and Packet 21. | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The driver is informed about the new MA and track description. | O | DMI | The non-infill track description is replaced by the information received from BGa now. The new MA and Gradient Profile (infill information) will replaced the data beyond the next main signal (BGb). Step not applicable in LS mode | N/A | N/A | - |
| 4 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded. | O | JRU | Step not applicable in LS mode | N/A | N/A | - |
| 5 | N/A | N/A | One Balise Group message (N_TOTAL=1) is received | I | BTM | The BGb message received contains only Packet 255. | N/A | N/A | - |
| 6 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | The train is running according to information given in BGa. | I | INT | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 010 0000 | Version 2.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | First |
| N_TOTAL | 3 | 1 | Two balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 27 | International Static Speed Profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 136 | Infill location reference |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | If Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | BGb |
| NID_PACKET | 8 | 12 | Level 1 Movement Authority |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out". |
| N_ITER | 5 | FINITE VALUE | Set to zero if V_MAIN = 0 or if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | Gradient Profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 010 0000 | Version 2.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 001 | Second |
| N_TOTAL | 3 | 1 | Two balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGb |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 5: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 010 0000 | Version 2.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | First |
| N_TOTAL | 3 | 1 | Two balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGb |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 5: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 010 0000 | Version 2.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 001 | Second |
| N_TOTAL | 3 | 1 | Two balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| Linking | STORED | - |
| Movement Authority | STORED | Stored but different from previous MA |
| Gradient Profile | STORED | Stored but different from previous gradient profile |
| International SSP | STORED | Stored but different from previous SSP |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Updated speed and distance monitoring information is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 6

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.1: Acceptance of received information |  | 4080100 |
| Test Case of Feature | In levels 2/3, if information is received from the “Accepting” RBC while the “Handing Over” RBC is still responsible, it is stored onboard until the RBC transition is performed with the exception for the information “Revocation of Emergency Stop” that shall be rejected. |  | 6 |
| Applicable Mode/Level Combinations | L2 / L3: FS / LS / OS / SR |  |  |
| Target of Test | The target of this test is to check that: The on board equipment receives the information “Revocation of Emergency Stop” from the Accepting RBC while the Handing Over is still responsible. The received information should be rejected. After the RBC/ RBC handover the information from the buffer is re-evaluated and the train is tripped. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.2.1 |  |
|  |  | Subset-026-4.8.2.1 c) |  |
|  |  | Subset-026-8.7.7 Message 16: Unconditional Emergency Stop |  |
|  |  | Subset-026-8.7.8 Message 18: Revocation of Emergency Stop |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: Train is tripped. Check on JRU that: RBC Messages 16 (Unconditional Emergency Stop) received from the Accepting RBC is recorded. RBC Messages 18 (Revocation of Emergency Stop) received from the Accepting RBC is recorded. Mode change to TR is recorded. |
| Constraints | The on-board equipment is able to manage two communication sessions. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Radio communication session | ESTABLISHED | With Handing Over RBC |
| Position Report parameters | STORED | Handing over RBC |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | With Handing Over RBC |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2 / L3 | FS / LS / OS / SR | SA-DATA. Indication with Euroradio Message "General Message" (NID_MESSAGE=24) containing packet 131 is received. | I | RTM | Received from the Handing Over RBC. D_RBCTR ≠ 0. | L2 / L3 | FS / LS / OS / SR | - |
| 2 | L2 / L3 | FS / LS / OS / SR | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=24) is recorded | O | JRU | - | L2 / L3 | FS / LS / OS / SR | - |
| 3 | L2 / L3 | FS / LS / OS / SR | SA-CONNECT. Request is transmitted | O | RTM | A connect request to the Accepting RBC is sent. A safe connection is established with the Accepting RBC | L2 / L3 | FS / LS / OS / SR | - |
| 4 | L2 / L3 | FS / LS / OS / SR | SA-CONNECT .Indication is received | I | RTM | A connection indication from the Accepting RBC is received on board | L2 / L3 | FS / LS / OS / SR | - |
| 5 | L2 / L3 | FS / LS / OS / SR | SA-DATA. Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted | O | RTM | A radio message 155 (initiation of communication session) is sent to the Accepting RBC. | L2 / L3 | FS / LS / OS / SR | - |
| 6 | L2 / L3 | FS / LS / OS / SR | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=155) is recorded | O | JRU | - | L2 / L3 | FS / LS / OS / SR | - |
| 7 | L2 / L3 | FS / LS / OS / SR | SA-DATA. Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32) is received | I | RTM | A radio message 32 (configuration determination) is received from the Accepting RBC. The Version of the ERTMS/ETCS language is valid for the train and M_VERSION <> { 000 XXXX, 111 1111} | L2 / L3 | FS / LS / OS / SR | - |
| 8 | L2 / L3 | FS / LS / OS / SR | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=32) is recorded | O | JRU | - | L2 / L3 | FS / LS / OS / SR | - |
| 9 | L2 / L3 | FS / LS / OS / SR | SA-DATA. Request with Euroradio Message "Session Established" (NID_MESSAGE=159) is transmitted | O | RTM | The communication session with the Accepting RBC A is established | L2 / L3 | FS / LS / OS / SR | - |
| 10 | L2 / L3 | FS / LS / OS / SR | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=159) is recorded | O | JRU | - | L2 / L3 | FS / LS / OS / SR | - |
| 11 | L2 / L3 | FS / LS / OS / SR | SA-DATA. Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) containing packet 0, packet 11 is transmitted | O | RTM | A radio message 129 (Validated train data) is sent to the Accepting RBC . The radio message contains also position report data. | L2 / L3 | FS / LS / OS / SR | - |
| 12 | L2 / L3 | FS / LS / OS / SR | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=129) is recorded | O | JRU | - | L2 / L3 | FS / LS / OS / SR | - |
| 13 | L2 / L3 | FS / LS / OS / SR | SA-DATA. Indication with Euroradio Message "Acknowledgement of Train Data" (NID_MESSAGE=8) is received | I | RTM | From Accepting RBC | L2 / L3 | FS / LS / OS / SR | - |
| 14 | L2 / L3 | FS / LS / OS / SR | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=8) is recorded | O | JRU | - | L2 / L3 | FS / LS / OS / SR | - |
| 15 | L2 / L3 | FS / LS / OS / SR | SA-DATA. Indication with Euroradio Message "Unconditional Emergency Stop" (NID_MESSAGE=16) is received | I | RTM | The ERTMS/ETCS on-board equipment stores the set of information in a transition buffer. | L2 / L3 | FS / LS / OS / SR | - |
| 16 | L2 / L3 | FS / LS / OS / SR | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=16) is recorded | O | JRU | - | L2 / L3 | FS / LS / OS / SR | - |
| 17 | L2 / L3 | FS / LS / OS / SR | Train is moving. | I | INT | Train location: in rear of the RBC/RBC handover area | L2 / L3 | FS / LS / OS / SR | - |
| 18 | L2 / L3 | FS / LS / OS / SR | SA-DATA. Indication with Euroradio Message "Revocation of Emergency Stop" (NID_MESSAGE=18) is received | I | RTM | The ERTMS/ETCS on-board equipment rejects the information. | L2 / L3 | FS / LS / OS / SR | - |
| 19 | L2 / L3 | FS / LS / OS / SR | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=18) is recorded | O | JRU | - | L2 / L3 | FS / LS / OS / SR | - |
| 20 | L2 / L3 | FS / LS / OS / SR | Train is moving. | I | INT | Train Location: Train maximum safe front end location = LRBG + D_RBCTR The maximum safe front end of the train passes the location of the RBC transition. The on-board is supervised by the Accepting RBC. | L2 / L3 | FS / LS / OS / SR | - |
| 21 | L2 / L3 | FS / LS / OS / SR | SA-DATA. Request with Euroradio message (NID_MESSAGE=136) containing packet 0 is transmitted | O | RTM | A radio message 136 (train position report) is sent to the Handing Over RBC. | L2 / L3 | FS / LS / OS / SR | - |
| 22 | L2 / L3 | FS / LS / OS / SR | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=136) is recorded | O | JRU | - | L2 / L3 | FS / LS / OS / SR | - |
| 23 | L2 / L3 | FS / LS / OS / SR | SA-DATA. Request with Euroradio message (NID_MESSAGE=136) containing packet 0 is transmitted | O | RTM | A radio message 136 (train position report) is sent to the Accepting RBC. | L2 / L3 | FS / LS / OS / SR | - |
| 24 | L2 / L3 | FS / LS / OS / SR | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=136) is recorded | O | JRU | - | L2 / L3 | FS / LS / OS / SR | - |
| 25 | L2 / L3 | FS / LS / OS / SR | The system status message "Emergency stop" is displayed | O | DMI | The driver is informed that the unconditional emergency stop is accepted. | L2 / L3 | FS / LS / OS / SR | - |
| 26 | L2 / L3 | FS / LS / OS / SR | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23) is recorded | O | JRU | - | L2 / L3 | FS / LS / OS / SR | - |
| 27 | L2 / L3 | FS / LS / OS / SR | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=7) is recorded | O | JRU | The train is Tripped TRIGGERING EVENT: When the mode changes | L2 / L3 | TR | - |
| 28 | L2 / L3 | TR | The Mode symbol "Trip" is displayed. The system status message "Emergency stop" is displayed. | O | DMI | The driver is informed on the DMI of the change to TR mode and trip reason. DMI Object: MO04 | L2 / L3 | TR | - |
| 29 | L2 / L3 | TR | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit19=1>) is recorded | O | JRU | - | L2 / L3 | TR | - |
| 30 | L2 / L3 | TR | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23) is recorded | O | JRU | - | L2 / L3 | TR | - |
| 31 | L2 / L3 | TR | Report mode change to RBC | - | - | USE_FT3060500.4 | L2 / L3 | TR | - |
| 32 | L2 / L3 | TR | Emergency brake commanded | O | TIU | - | L2 / L3 | TR | - |
| 33 | L2 / L3 | TR | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | L2 / L3 | TR | - |
| 34 | L2 / L3 | TR | SA-DATA .Request with Euroradio Message "Acknowledgement of Emergency Stop" (NID_MESSAGE=147) containing packet 0 is transmitted. | O | RTM | Indicating that the train has received the Unconditional Emergency Stop. Check that the messages 147 is send over the normal priority channel. | L2 / L3 | TR | - |
| 35 | L2 / L3 | TR | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | L2 / L3 | TR | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General Message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 131 | RBC transition order |
| Q_DIR | 2 | FINITE VALUE | Valid for current train direction |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_RBCTR | 15 | FINITE VALUE | ≠ 0 |
| NID_C | 10 | FINITE VALUE | Valid for the train |
| NID_RBC | 14 | FINITE VALUE | Accepting RBC |
| NID_RADIO | 64 | FINITE VALUE | Accepting RBC |
| Q_SLEEPSESSION | 1 | 0 | Sleeping equipment don’t consider session management information (to avoid TIU influence) |

| Step 5: Radio Message 155 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 155 | Initiation of a communication session |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 1 | FINITE VALUE | - |

| Step 7: Radio Message 32 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 32 | Configuration Determination |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| M_VERSION | 7 | 010 0000 | Version 2.0 |

| Step 9: Radio Message 159 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 159 | Session established |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 1 | FINITE VALUE | - |

| Step 11: Radio Message 129 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 129 | Validated Train Data |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | < D_RBCTR |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | - |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | 0 / 1 / 2 / 5 / 7 / 11 | - |
| M_LEVEL | 3 | 3 / 4 | - |
| NID_PACKET | 8 | 11 | Validated Train Data |
| L_PACKET | 13 | FINITE VALUE | - |
| NC_CDTRAIN | 4 | FINITE VALUE | - |
| NC_TRAIN | 15 | FINITE VALUE | - |
| L_TRAIN | 12 | FINITE VALUE | - |
| V_MAXTRAIN | 7 | FINITE VALUE | - |
| M_LOADINGGAUGE | 8 | FINITE VALUE | - |
| M_AXLELOADCAT | 7 | FINITE VALUE | - |
| M_AIRTIGHT | 2 | FINITE VALUE | - |
| N_AXLE | 10 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_VOLTAGE(k) | 4 | FINITE VALUE | Identity of the traction system. |
| NID_CTRACTION(k) | 10 | FINITE VALUE | NID_CTRACTION(k) given only if M_VOLTAGE(k) ≠ 0. |
| N_ITER | 5 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | Type of National System available. |

| Step 13: Radio Message 8 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 8 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | Not: 4294967295 (unknown) |
| M_ACK | 1 | 0 | No acknowledgement required |
| NID_LRBG | 24 | FINITE VALUE | Not: 16777215 (unknown) |

| Step 15: Radio Message 16 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 16 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | 0/1 | 0: No acknowledgement required 1: Acknowledgement required |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_EM | 4 | FINITE VALUE | Emergency Stop Identification Number |

| Step 18: Radio Message 18 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 18 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | 1 | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_EM | 4 | FINITE VALUE | Same Identification number of the emergency stop message ( from message 16) |

| Step 21: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report (To Handing Over RBC) |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | End of Information |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| Step 23: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report (To Accepting RBC) |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| Step 34: Radio Message 147 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 147 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_EM | 4 | FINITE VALUE | Identification Number of the acknowledged Emergency Stop Message |
| Q_EMERGENCYSTOP | 2 | 10 | Unconditional Emergency Stop |
| Packet 0 or 1 | - | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 | TR |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Trip mode and reason is DISPLAYED. | O | DMI | DMI Object: Symbol MO04 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 7

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.1: Acceptance of received information |  | 4080100 |
| Test Case of Feature | In levels 2/3, if the information “LSSMA display toggle on order “ is received from the “Accepting” RBC while the “Handing Over” RBC is still responsible, ” that information shall be rejected immediately and not stored on-board. |  | 7 |
| Applicable Mode/Level Combinations | L2 / L3: LS |  |  |
| Target of Test | The target of this test is to check that: The on board equipment receives the information “LSSMA display toggle on order” from the Accepting RBC while the Handing Over is still responsible. The received information is rejected. The LSSMA speed is not displayed. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.2.1 |  |
|  |  | Subset-026-4.8.2.1 c) |  |
|  |  | Subset-026-8.7.9 Message 24: General Message |  |
|  |  | Subset-026-7.4.2.37.3 Packet Number 180: LSSMA display toggle order |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: LSSMA speed is not displayed and not shown when toggle on order is received. |
| Constraints | The on-board equipment is able to manage two communication sessions. No active Emergency stops are pending on-board. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 12 | LS |
| Radio communication session | ESTABLISHED | With Handing Over RBC |
| Position Report parameters | STORED | Handing over RBC |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | With Handing Over RBC |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2 / L3 | LS | SA-DATA. Indication with Euroradio Message "General Message" (NID_MESSAGE=24) containing packet 131 is received. | I | RTM | Received from the Handing Over RBC. D_RBCTR ≠ 0. | L2 / L3 | LS | - |
| 2 | L2 / L3 | LS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=24) is recorded | O | JRU | - | L2 / L3 | LS | - |
| 3 | L2 / L3 | LS | SA-CONNECT. Request is transmitted | O | RTM | A connect request to the Accepting RBC is sent. A safe connection is established with the Accepting RBC | L2 / L3 | LS | - |
| 4 | L2 / L3 | LS | SA-CONNECT .Indication is received | I | RTM | A connection indication from the Accepting RBC is received on board | L2 / L3 | LS | - |
| 5 | L2 / L3 | LS | SA-DATA. Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted | O | RTM | A radio message 155 (initiation of communication session) is sent to the Accepting RBC. | L2 / L3 | LS | - |
| 6 | L2 / L3 | LS | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=155) is recorded | O | JRU | - | L2 / L3 | LS | - |
| 7 | L2 / L3 | LS | SA-DATA. Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32) is received | I | RTM | A radio message 32 (configuration determination) is received from the Accepting RBC. The Version of the ERTMS/ETCS language is valid for the train and M_VERSION <> { 000 XXXX, 111 1111} | L2 / L3 | LS | - |
| 8 | L2 / L3 | FS / LS / OS / SR | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=32) is recorded | O | JRU | - | L2 / L3 | LS | - |
| 9 | L2 / L3 | LS | SA-DATA. Request with Euroradio Message "Session Established" (NID_MESSAGE=159) is transmitted | O | RTM | The communication session with the Accepting RBC A is established | L2 / L3 | LS | - |
| 10 | L2 / L3 | LS | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=159) is recorded | O | JRU | - | L2 / L3 | LS | - |
| 11 | L2 / L3 | LS | SA-DATA. Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) containing packet 0, packet 11 is transmitted | O | RTM | A radio message 129 (Validated train data) is sent to the Accepting RBC . The radio message contains also position report data. | L2 / L3 | LS | - |
| 12 | L2 / L3 | LS | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=129) is recorded | O | JRU | - | L2 / L3 | LS | - |
| 13 | L2 / L3 | LS | SA-DATA. Indication with Euroradio Message "Acknowledgement of Train Data" (NID_MESSAGE=8) is received | I | RTM | From Accepting RBC | L2 / L3 | LS | - |
| 14 | L2 / L3 | LS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=8) is recorded | O | JRU | - | L2 / L3 | LS | - |
| 15 | L2 / L3 | LS | SA-DATA. Indication with Euroradio Message "General message” with packet 180 “LSSMA display toggle order" (NID_MESSAGE=24; Q_LSSMA=1) is received | I | RTM | The ERTMS/ETCS on-board equipment rejects the information (Information received from Accepting RBC) | L2 / L3 | LS | - |
| 16 | L2 / L3 | LS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=24) is recorded | O | JRU | - | L2 / L3 | LS | - |
| 17 | L2 / L3 | LS | NOT LSSMA number is displayed | O | DMI | - | L2 / L3 | LS | - |
| 18 | L2 / L3 | LS | Train is moving. | I | INT | Train location: Train maximum safe front end location = LRBG + D_RBCTR | L2 / L3 | LS | - |
| 19 | L2 / L3 | LS | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted. | O | RTM | To both RBCs (Handing Over) when the maximum safe front end of the train passes the location of the RBC transition. | L2 / L3 | LS | - |
| 20 | L2 / L3 | LS | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | L2 / L3 | LS | - |
| 21 | L2 / L3 | LS | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted. | O | RTM | To both RBCs (Accepting) when the maximum safe front end of the train passes the location of the RBC transition. | L2 / L3 | LS | - |
| 22 | L2 / L3 | LS | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | L2 / L3 | LS | - |
| 23 | L2 / L3 | LS | Balise Group message containing packet 131 is received | I | BTM | D_RBCTR = 0 The balise group containing the order to execute immediately the handover is placed at the RBC/RBC transition border. | L2 / L3 | LS | - |
| 24 | L2 / L3 | LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L2 / L3 | LS | - |
| 25 | L2 / L3 | LS | Train is moving. | I | INT | Train min safe rear end location = LRBG + D_RBCTR+L_TRAIN The minimum safe rear end of the train passes the location of the RBC transition | L2 / L3 | LS | - |
| 26 | L2 / L3 | LS | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted. | O | RTM | To the Handing Over RBC | L2 / L3 | LS | - |
| 27 | L2 / L3 | LS | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | L2 / L3 | LS | - |
| 28 | L2 / L3 | LS | NOT LSSMA number is displayed | O | DMI | - | L2 / L3 | LS | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General Message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 131 | RBC transition order |
| Q_DIR | 2 | FINITE VALUE | Valid for current train direction |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_RBCTR | 15 | FINITE VALUE | ≠ 0 |
| NID_C | 10 | FINITE VALUE | Valid for the train |
| NID_RBC | 14 | FINITE VALUE | Accepting RBC |
| NID_RADIO | 64 | FINITE VALUE | Accepting RBC |
| Q_SLEEPSESSION | 1 | 0 | Sleeping equipment don’t consider session management information (to avoid TIU influence) |

| Step 5: Radio Message 155 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 155 | Initiation of a communication session |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 1 | FINITE VALUE | - |

| Step 7: Radio Message 32 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 32 | Configuration Determination |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| M_VERSION | 7 | 010 0000 | Version 2.0 |

| Step 9: Radio Message 159 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 159 | Session established |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 1 | FINITE VALUE | - |

| Step 11: Radio Message 129 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 129 | Validated Train Data |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | < D_RBCTR |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | - |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | 0 / 1 / 2 / 5 / 7 / 11 | - |
| M_LEVEL | 3 | 3 / 4 | - |
| NID_PACKET | 8 | 11 | Validated Train Data |
| L_PACKET | 13 | FINITE VALUE | - |
| NC_CDTRAIN | 4 | FINITE VALUE | - |
| NC_TRAIN | 15 | FINITE VALUE | - |
| L_TRAIN | 12 | FINITE VALUE | - |
| V_MAXTRAIN | 7 | FINITE VALUE | - |
| M_LOADINGGAUGE | 8 | FINITE VALUE | - |
| M_AXLELOADCAT | 7 | FINITE VALUE | - |
| M_AIRTIGHT | 2 | FINITE VALUE | - |
| N_AXLE | 10 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_VOLTAGE(k) | 4 | FINITE VALUE | Identity of the traction system. |
| NID_CTRACTION(k) | 10 | FINITE VALUE | NID_CTRACTION(k) given only if M_VOLTAGE(k) ≠ 0. |
| N_ITER | 5 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | Type of National System available. |

| Step 13: Radio Message 8 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 8 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | Not: 4294967295 (unknown) |
| M_ACK | 1 | 0 | No acknowledgement required |
| NID_LRBG | 24 | FINITE VALUE | Not: 16777215 (unknown) |

| Step 15: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General Message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 180 | LSSMA display toggle order |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_LSSMA | 1 | 1 | Toggle on |
| T_LSSMA | 8 | FINITE VALUE | Only if Q_LSSMA = 1 |

| Step 19: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report (To Handing Over RBC) |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | - |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |

| Step 21: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report (To Accepting RBC) |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | - |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |

| Step 23: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | - |
| M_VERSION | 7 | 010 0000 | Version 2.0 |
| Q_MEDIA | 1 | 0 | - |
| N_PIG | 3 | FINITE VALUE | - |
| N_TOTAL | 3 | FINITE VALUE | - |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINK | 1 | 1 | - |
| NID_PACKET | 8 | 131 | RBC Transition order |
| Q_DIR | 2 | FINITE VALUE | Valid for current train direction |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_RBCTR | 15 | 0 | - |
| NID_C | 10 | FINITE VALUE | Valid for the train |
| NID_RBC | 14 | FINITE VALUE | Accepting RBC B |
| NID_RADIO | 64 | FINITE VALUE | Accepting RBC B |
| Q_SLEEPSESSION | 1 | 0 | Sleeping equipment don’t consider session management information (to avoid TIU influence) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 26: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report (to Handing Over RBC) |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | - |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 12 | LS |
| Radio communication session | ESTABLISHED | Both communications |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| LSSMA display toggle on order | NOT STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT LS frame symbol is displayed | O | DMI | DMI Object: Absence of symbol ??? or LS01 yes??? |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 8

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.1: Acceptance of received information |  | 4080100 |
| Test Case of Feature | In levels 2/3, if the information “LSSMA display toggle off order “ is received from the “Accepting” RBC while the “Handing Over” RBC is still responsible, that information shall be rejected immediately and not stored on-board. |  | 8 |
| Applicable Mode/Level Combinations | L2 / L3: LS |  |  |
| Target of Test | The target of this test is to check that: The on board equipment receives the information “LSSMA display toggle off order” from the Accepting RBC while the Handing Over is still responsible. The received information is rejected. The LSSMA speed is still displayed. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.2.1 |  |
|  |  | Subset-026-4.8.2.1 c) |  |
|  |  | Subset-026-8.7.9 Message 24: General Message |  |
|  |  | Subset-026-7.4.2.37.3 Packet Number 180: LSSMA display toggle order |  |
|  |  | Subset-026-4.7.2 table - Output information LSSMA |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: LSSMA speed is displayed and not removed when toggle off order is received. |
| Constraints | The on-board equipment is able to manage two communication sessions. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 12 | LS |
| Radio communication session | ESTABLISHED | With Handing Over RBC |
| Position Report parameters | STORED | Handing over RBC |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | With Handing Over RBC |
| NOT RELEVANT | - | TIU | - |
| LS frame and number are displayed. | O | DMI | DMI Object: LS01+ V_LSSMA |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2 / L3 | LS | SA-DATA. Indication with Euroradio Message "General Message" (NID_MESSAGE=24) containing packet 131 is received. | I | RTM | Received from the Handing Over RBC. D_RBCTR ≠ 0. | L2 / L3 | LS | - |
| 2 | L2 / L3 | LS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=24) is recorded | O | JRU | - | L2 / L3 | LS | - |
| 3 | L2 / L3 | LS | SA-CONNECT. Request is transmitted | O | RTM | A connect request to the Accepting RBC is sent. A safe connection is established with the Accepting RBC | L2 / L3 | LS | - |
| 4 | L2 / L3 | LS | SA-CONNECT .Indication is received | I | RTM | A connection indication from the Accepting RBC is received on board | L2 / L3 | LS | - |
| 5 | L2 / L3 | LS | SA-DATA. Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted | O | RTM | A radio message 155 (initiation of communication session) is sent to the Accepting RBC. | L2 / L3 | LS | - |
| 6 | L2 / L3 | LS | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=155) is recorded | O | JRU | - | L2 / L3 | LS | - |
| 7 | L2 / L3 | LS | SA-DATA. Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32) is received | I | RTM | A radio message 32 (configuration determination) is received from the Accepting RBC. The Version of the ERTMS/ETCS language is valid for the train and M_VERSION <> { 000 XXXX, 111 1111} | L2 / L3 | LS | - |
| 8 | L2 / L3 | FS / LS / OS / SR | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=32) is recorded | O | JRU | - | L2 / L3 | LS | - |
| 9 | L2 / L3 | LS | SA-DATA. Request with Euroradio Message "Session Established" (NID_MESSAGE=159) is transmitted | O | RTM | The communication session with the Accepting RBC A is established | L2 / L3 | LS | - |
| 10 | L2 / L3 | LS | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=159) is recorded | O | JRU | - | L2 / L3 | LS | - |
| 11 | L2 / L3 | LS | SA-DATA. Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) containing packet 0, packet 11 is transmitted | O | RTM | A radio message 129 (Validated train data) is sent to the Accepting RBC . The radio message contains also position report data. | L2 / L3 | LS | - |
| 12 | L2 / L3 | LS | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=129) is recorded | O | JRU | - | L2 / L3 | LS | - |
| 13 | L2 / L3 | LS | SA-DATA. Indication with Euroradio Message "Acknowledgement of Train Data" (NID_MESSAGE=8) is received | I | RTM | From Accepting RBC | L2 / L3 | LS | - |
| 14 | L2 / L3 | LS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=8) is recorded | O | JRU | - | L2 / L3 | LS | - |
| 15 | L2 / L3 | LS | SA-DATA. Indication with Euroradio Message "General message” with packet 180 “LSSMA display toggle order" (NID_MESSAGE=24; Q_LSSMA=0) is received | I | RTM | The ERTMS/ETCS on-board equipment rejects the information (Information received from Accepting RBC) | L2 / L3 | LS | - |
| 16 | L2 / L3 | LS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=24) is recorded | O | JRU | - | L2 / L3 | LS | - |
| 17 | L2 / L3 | LS | The Limited supervision symbol “ Limited Supervision frame” is displayed. | O | DMI | DMI Object: Symbol LS01 | L2 / L3 | LS | - |
| 18 | L2 / L3 | LS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit87=1>) is recorded | O | JRU | - | L2 / L3 | LS | - |
| 19 | L2 / L3 | LS | The LSSMA number is displayed. | O | DMI | - | L2 / L3 | LS | - |
| 20 | L2 / L3 | LS | LOWEST SUPERVISED SPEED WITHIN THE MOVEMENT AUTHORITY (NID_MESSAGE_JRU=44; <V_LSSMA>) is recorded | O | JRU | - | L2 / L3 | LS | - |
| 21 | L2 / L3 | LS | Train is moving. | I | INT | Train location: Train maximum safe front end location = LRBG + D_RBCTR | L2 / L3 | LS | - |
| 22 | L2 / L3 | LS | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted. | O | RTM | To both RBCs (Handing Over) when the maximum safe front end of the train passes the location of the RBC transition. | L2 / L3 | LS | - |
| 23 | L2 / L3 | LS | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | L2 / L3 | LS | - |
| 24 | L2 / L3 | LS | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted. | O | RTM | To both RBCs (Accepting) when the maximum safe front end of the train passes the location of the RBC transition. | L2 / L3 | LS | - |
| 25 | L2 / L3 | LS | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | L2 / L3 | LS | - |
| 26 | L2 / L3 | LS | Balise Group message containing packet 131 is received | I | BTM | D_RBCTR = 0 The balise group containing the order to execute immediately the handover is placed at the RBC/RBC transition border. | L2 / L3 | LS | - |
| 27 | L2 / L3 | LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L2 / L3 | LS | - |
| 28 | L2 / L3 | LS | Train is moving. | I | INT | Train min safe rear end location = LRBG + D_RBCTR+L_TRAIN The minimum safe rear end of the train passes the location of the RBC transition | L2 / L3 | LS | - |
| 29 | L2 / L3 | LS | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted. | O | RTM | To the Handing Over RBC | L2 / L3 | LS | - |
| 30 | L2 / L3 | LS | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | L2 / L3 | LS | - |
| 31 | L2 / L3 | LS | The Limited supervision symbol “ Limited Supervision frame” is displayed. | O | DMI | DMI Object: Symbol LS01 | L2 / L3 | LS | - |
| 32 | L2 / L3 | LS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit87=1>) is recorded | O | JRU | - | L2 / L3 | LS | - |
| 33 | L2 / L3 | LS | The LSSMA number is displayed. | O | DMI | - | L2 / L3 | LS | - |
| 34 | L2 / L3 | LS | LOWEST SUPERVISED SPEED WITHIN THE MOVEMENT AUTHORITY (NID_MESSAGE_JRU=44; <V_LSSMA>) is recorded | O | JRU | - | L2 / L3 | LS | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General Message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 131 | RBC transition order |
| Q_DIR | 2 | FINITE VALUE | Valid for current train direction |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_RBCTR | 15 | FINITE VALUE | ≠ 0 |
| NID_C | 10 | FINITE VALUE | Valid for the train |
| NID_RBC | 14 | FINITE VALUE | Accepting RBC |
| NID_RADIO | 64 | FINITE VALUE | Accepting RBC |
| Q_SLEEPSESSION | 1 | 0 | Sleeping equipment don’t consider session management information (to avoid TIU influence) |

| Step 5: Radio Message 155 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 155 | Initiation of a communication session |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 1 | FINITE VALUE | - |

| Step 7: Radio Message 32 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 32 | Configuration Determination |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| M_VERSION | 7 | 010 0000 | Version 2.0 |

| Step 9: Radio Message 159 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 159 | Session established |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 1 | FINITE VALUE | - |

| Step 11: Radio Message 129 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 129 | Validated Train Data |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | < D_RBCTR |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | - |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | 0 / 1 / 2 / 5 / 7 / 11 | - |
| M_LEVEL | 3 | 3 / 4 | - |
| NID_PACKET | 8 | 11 | Validated Train Data |
| L_PACKET | 13 | FINITE VALUE | - |
| NC_CDTRAIN | 4 | FINITE VALUE | - |
| NC_TRAIN | 15 | FINITE VALUE | - |
| L_TRAIN | 12 | FINITE VALUE | - |
| V_MAXTRAIN | 7 | FINITE VALUE | - |
| M_LOADINGGAUGE | 8 | FINITE VALUE | - |
| M_AXLELOADCAT | 7 | FINITE VALUE | - |
| M_AIRTIGHT | 2 | FINITE VALUE | - |
| N_AXLE | 10 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_VOLTAGE(k) | 4 | FINITE VALUE | Identity of the traction system. |
| NID_CTRACTION(k) | 10 | FINITE VALUE | NID_CTRACTION(k) given only if M_VOLTAGE(k) ≠ 0. |
| N_ITER | 5 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | Type of National System available. |

| Step 13: Radio Message 8 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 8 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | Not: 4294967295 (unknown) |
| M_ACK | 1 | 0 | No acknowledgement required |
| NID_LRBG | 24 | FINITE VALUE | Not: 16777215 (unknown) |

| Step 15: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General Message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 180 | LSSMA display toggle order |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_LSSMA | 1 | 0 | Toggle off |
| T_LSSMA | 8 | FINITE VALUE | Only if Q_LSSMA = 1 |

| Step 22: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report (To Handing Over RBC) |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | - |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |

| Step 24: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report (To Accepting RBC) |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | - |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |

| Step 26: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | - |
| M_VERSION | 7 | 010 0000 | Version 2.0 |
| Q_MEDIA | 1 | 0 | - |
| N_PIG | 3 | FINITE VALUE | - |
| N_TOTAL | 3 | FINITE VALUE | - |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINK | 1 | 1 | - |
| NID_PACKET | 8 | 131 | RBC Transition order |
| Q_DIR | 2 | FINITE VALUE | Valid for current train direction |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_RBCTR | 15 | 0 | - |
| NID_C | 10 | FINITE VALUE | Valid for the train |
| NID_RBC | 14 | FINITE VALUE | Accepting RBC B |
| NID_RADIO | 64 | FINITE VALUE | Accepting RBC B |
| Q_SLEEPSESSION | 1 | 0 | Sleeping equipment don’t consider session management information (to avoid TIU influence) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 29: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report (to Handing Over RBC) |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | - |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 12 | LS |
| Radio communication session | ESTABLISHED | Both communications |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| LSSMA display toggle on order | STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| LS frame and number are displayed | O | DMI | DMI Object: LS01+actual V_LSSMA |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |