| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 3130235 (3.13.2.3.5 Reduced Adhesion Conditions) Total: 3 Test case(s) |  |
| REF: | Subset-076-5-2-3130235 |
| ISSUE: | Version 3.2.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 2.9.1 31.03.2013 | All | First version/review | CEDEX |
| Version 2.9.2 28.11.2013 | All | General review | CEDEX |
| Version 2.9.3 16.12.2013 | TC2 | An uncovered requirement has been added | CEDEX |
| Version 2.9.3 19.12.2013 | All | Review | CEDEX |
| Version 2.9.4 20.12.2013 | All | Review | CEDEX |
| Version 2.9.5 08.01.2014 | All | Review | CEDEX |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 14.01.2015 | All | SRS 340 | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.2.0 31.03.2017 | All | Baseline 3 1st maintenance release version - Bug fixing | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.13.2.3.5 Reduced Adhesion Conditions |  | 3130235 |
| Test Case of Feature | Use of the track condition: Adhesion Factor in the determination of supervision limits for target speed monitoring (Adhesion Factor is given by the trackside) |  | 1 |
| Applicable Mode/Level Combinations | L1: FS, LS, SR, OS L2/L3: FS, LS, SR, OS |  |  |
| Target of Test | The target of this test is to evaluate the use of the track condition: Adhesion Factor by the on board equipment to determine the speed supervision limits for target speed monitoring (Adhesion Factor is given by the trackside) |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.13.2.3.5.1 |  |
|  |  | Subset-026-3.13.5.1 |  |
|  |  | Subset-026-3.13.6.2.1.1 |  |
|  |  | Subset-026-3.13.6.2.1.3 |  |
|  |  | Subset-026-3.13.6.2.1.6 |  |
|  |  | Subset-026-3.18.4.6.1 |  |
|  |  | Subset-026-3.18.4.6.2 |  |
|  |  | Subset-026-3.18.4.6.2.1 |  |
|  |  | Subset-026-3.18.4.6.2.3 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | The test case shall consider the train data and the National Values (either a valid Packet 3 or the Default Values are used) as preconditions to be known before setting the appropriate values for the rest of the trackside info and the speed profile. The required observables (speed supervision limits) should be checked using a suitable tool that performs fully the “speed and distance monitoring" functionality (by default use ERA Braking curves tool, which can be downloaded from this website: http://www.era.europa.eu/Core-Activities/ERTMS/Pages/Braking-Curves-Simulation-Tool.aspx).. The design of the Test Case requires a decrease in the permitted speed within the braking curve. When adhesion condition is applied the system has already to be in a target speed monitoring section and the train speed will be low for no warning or brake application |
| Constraints | The target cannot be a EoA (use, for example, LoA o MRSP), in order to prevent the on-board from supervising the SBD curve, since, in this case, no adhesion conditions need to be taken into account for its calculation (see the requirements 3.13.6.2.1.1, 3.13.6.3.1.1 and 3.13.8.4.1). |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 /1 / 2 /12 | FS, OS, SR, LS |
| Radio communication session | ESTABLISHED | Only in L2 and L3 |
| Movement Authority | STORED | Only in FS, OS, LS |
| Gradient Profile | STORED | Only in FS, OS, LS |
| International SSP | STORED | Only in FS, OS, LS |
| Axle load speed profile | NOT STORED | - |
| Route Suitability Data | NOT STORED | - |
| Mode Profile | STORED | Only in OS and LS mode |
| Track conditions excluding big metal masses | NOT STORED | - |
| Track condition big metal masses | NOT STORED | - |
| Unconditional Emergency Stop | NOT STORED | - |
| Conditional Emergency Stop | NOT STORED | - |
| Train Data | STORED | - |
| Adhesion factor | NOT STORED | - |
| ERTMS/ETCS level | 2/3/4 | Level 1, 2, 3 |
| Driver ID | STORED | - |
| Train Running Number | STORED | - |
| Reversing Area Information | NOT STORED | - |
| Reversing Supervision Information | NOT STORED | - |
| Track Ahead Free Request | NOT STORED | - |
| Operated System Version | 010 0000 | Version 2.0, introduced in SRS 3.3.0 |
| Virtual Balise Covers | NOT STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is moving (V_TRAIN=<V_PERMITTED) | I | INT | - | N/A | N/A | - |
| 2 | N/A | N/A | The driver requests the speed and distance monitoring information to be displayed | I | DMI | Only in OS or SR mode. | N/A | N/A | - |
| 3 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | Only for OS, SR | N/A | N/A | - |
| 4 | N/A | N/A | One Balise Group message (N_TOTAL=1) containing packet 71 (M_ADHESION= 0) is received | I | BTM | Only for L1 | N/A | N/A | - |
| 5 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | Only for L1 | N/A | N/A | - |
| 6 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 71 (M_ADHESION= 0) is received | I | RTM | Only for L2 /L3 | N/A | N/A | - |
| 7 | N/A | N/A | TELEGRAM FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | Only for L2 /L3 | N/A | N/A | - |
| 8 | N/A | N/A | One Balise Group message (N_TOTAL=1) is received | I | BTM | This balise group has to be the LRBG after the on-board reads it. This way the location related information (the MA, among others) shall be relocated and it will be possible to check the last two steps of this TC with great accuracy. This group has to be in the same place as the position marked by D_ADHESION + 20m approximately | N/A | N/A | - |
| 9 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 10 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | Only in FS, OS or SR mode. The DMI aspect is coherent with Normal Status when braking to a target. Permitted speed displayed is compared to ERA tool calculation with the new condition (Adhesion Factor - Slippery Rail). | N/A | N/A | - |
| 11 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=2; M_SDMSUPSTAT=0; V_PERM=<V1>) is recorded | O | JRU | Only in FS, OS or SR mode. M_SDMTYPE (Speed and distance monitoring type) = 2 (Target speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal Status) V_PERM is compared to ERA tool calculation | N/A | N/A | - |

| Step 4: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 71 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_ADHESION | 15 | FINITE VALUE | - |
| L_ADHESION | 15 | FINITE VALUE | - |
| M_ADHESION | 1 | 0 | Slippery Rail |
| NID_PACKET | 8 | 71 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_ADHESION | 15 | FINITE VALUE | - |
| L_ADHESION | 15 | FINITE VALUE | - |
| M_ADHESION | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | 255 |

| Step 4: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | 255 |

| Step 6: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 71 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_ADHESION | 15 | FINITE VALUE | - |
| L_ADHESION | 15 | FINITE VALUE | - |
| M_ADHESION | 1 | 0 | Slippery Rail |

| Step 8: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | 255 |

| Step 8: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | 255 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 /1 / 2 /12 | FS, OS, SR, LS |
| Radio communication session | ESTABLISHED | Only in L2 and L3 |
| Movement Authority | STORED | Only in FS, OS, LS |
| Gradient Profile | STORED | Only in FS, OS, LS |
| International SSP | STORED | Only in FS, OS, LS |
| Axle load speed profile | NOT STORED | - |
| Route Suitability Data | NOT STORED | - |
| Mode Profile | STORED | Only in OS and LS mode |
| Track conditions excluding big metal masses | NOT STORED | - |
| Track condition big metal masses | NOT STORED | - |
| Unconditional Emergency Stop | NOT STORED | - |
| Conditional Emergency Stop | NOT STORED | - |
| Train Data | STORED | - |
| ERTMS/ETCS level | 1/2/3 | Level 1, 2, 3 |
| Driver ID | STORED | - |
| Train Running Number | STORED | - |
| Reversing Area Information | NOT STORED | - |
| Reversing Supervision Information | NOT STORED | - |
| Track Ahead Free Request | NOT STORED | - |
| Operated System Version | STORED | - |
| Virtual Balise Covers | NOT STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.13.2.3.5 Reduced Adhesion Conditions |  | 3130235 |
| Test Case of Feature | Use of the track condition: Adhesion Factor in the determination of supervision limits for target speed monitoring (Adhesion Factor is modified by the driver) |  | 2 |
| Applicable Mode/Level Combinations | L1: FS, LS, SR, OS L2/L3: FS, LS, SR, OS |  |  |
| Target of Test | The target of this test is to evaluate the use of the track condition: - The selection of the adhesion value by driver entry shall be limited to the options slippery rail/ non slippery rail - Adhesion Factor by the on board equipment to determine the speed supervision limits for target speed monitoring (Adhesion Factor is modified by the driver) |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.13.5.3 |  |
|  |  | Subset-026-3.13.5.4 |  |
|  |  | Subset-026-3.13.6.2.1.1 |  |
|  |  | Subset-026-3.13.6.2.1.6 |  |
|  |  | Subset-026-3.18.4.6.1 |  |
|  |  | Subset-026-3.18.4.6.2.1 |  |
|  |  | Subset-026-3.18.4.6.2.3 |  |
|  |  | Subset-026-3.18.4.6.3 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | The test case shall consider the train data and the National Values (either a valid Packet 3 or the Default Values are used) as preconditions to be known before setting the appropriate values for the rest of the trackside info and the speed profile. The required observables (speed supervision limits) should be checked using a suitable tool that performs fully the “speed and distance monitoring" functionality (by default use ERA Braking curves tool, which can be downloaded from this website: http://www.era.europa.eu/Core-Activities/ERTMS/Pages/Braking-Curves-Simulation-Tool.aspx).. The design of the Test Case requires a decrease in the permitted speed within the braking curve. When adhesion condition is applied the system has already to be in a target speed monitoring section and the train speed will be low for no warning or brake application |
| Constraints | The target cannot be a EoA (use, for example, LoA o MRSP), in order to prevent the on-board from supervising the SBD curve, since, in this case, no adhesion conditions need to be taken into account for its calculation (see the requirements 3.13.6.2.1.1, 3.13.6.3.1.1 and 3.13.8.4.1). |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 /1 / 2 / 12 | FS, OS, SR, LS |
| Radio communication session | ESTABLISHED | Only in L2 and L3 |
| National Values | STORED | Q_NVDRIVER_ADHES=1 |
| Not yet applicable National Values | NOT STORED | - |
| Movement Authority | STORED | Only in FS, OS, LS |
| Gradient Profile | STORED | Only in FS, OS, LS |
| International SSP | STORED | Only in FS, OS, LS |
| Axle load speed profile | NOT STORED | - |
| Route Suitability Data | NOT STORED | - |
| Mode Profile | STORED | Only in OS and LS mode |
| Track conditions excluding big metal masses | NOT STORED | - |
| Track condition big metal masses | NOT STORED | - |
| Unconditional Emergency Stop | NOT STORED | - |
| Conditional Emergency Stop | NOT STORED | - |
| Train Data | STORED | - |
| Adhesion factor | NOT STORED | - |
| ERTMS/ETCS level | 2/3/4 | Level 1, 2, 3 |
| Driver ID | STORED | - |
| Train Running Number | STORED | - |
| Reversing Area Information | NOT STORED | - |
| Reversing Supervision Information | NOT STORED | - |
| Track Ahead Free Request | NOT STORED | - |
| Operated System Version | 010 0000 | Version 2.0, introduced in SRS 3.3.0 |
| Virtual Balise Covers | NOT STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is stopped (V_TRAIN=0) | I | INT | - | N/A | N/A | - |
| 2 | N/A | N/A | The driver requests the speed and distance monitoring information to be displayed | I | DMI | Only in OS or SR mode. | N/A | N/A | - |
| 3 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | Only for OS, SR | N/A | Na | - |
| 4 | N/A | N/A | The driver modifies "Adhesion Factor" | I | DMI | The driver selects Slippery rail | N/A | N/A | - |
| 5 | N/A | N/A | The <Adhesion> window is displayed. | O | DMI | The selection of the adhesion value shall be limited to the options slippery rail/ non slippery rai | N/A | N/A | - |
| 6 | N/A | N/A | The driver confirms "Adhesion Factor" | I | DMI | The driver confirms Slippery rail | N/A | N/A | - |
| 7 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTION=0010 0000) is recorded | O | JRU | The selection of slippery rail by the driver is now recorded with M_DRIVERACTION = 0010 0000 | N/A | N/A | - |
| 8 | N/A | N/A | The Status symbol "Adhesion factor - slippery rail" is displayed | O | DMI | ST02: Adhesion factor “slippery rail" | N/A | N/A | - |
| 9 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit39=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 10 | N/A | N/A | Train is moving (V_TRAIN=<V_PERMITTED>) | I | INT | - | N/A | N/A | - |
| 11 | N/A | N/A | One Balise Group message (N_TOTAL=1) is received | I | BTM | This balise group is used to reset the odometry. The stopping point should be near the balise group. | N/A | N/A | - |
| 12 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 13 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | Only in FS, OS or SR mode. The DMI aspect is coherent with Normal Status when braking to a target. Permitted speed displayed is compared to ERA tool calculation with the new condition (Adhesion Factor - Slippery Rail). | N/A | N/A | - |
| 14 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=2; M_SDMSUPSTAT=0; V_PERM=<V1>) is recorded | O | JRU | Only in FS, OS or SR mode. M_SDMTYPE (Speed and distance monitoring type) = 2 (Target speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal Status) V_PERM is compared to ERA tool calculation | N/A | N/A | - |

| Step 11: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | 255 |

| Step 11: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | 255 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 /1 / 2 / 12 | FS, OS, SR, LS |
| Radio communication session | ESTABLISHED | Only in L2 and L3 |
| National Values | STORED | Q_NVDRIVER_ADHES=1 |
| Movement Authority | STORED | Only in FS, OS, LS |
| Gradient Profile | STORED | Only in FS, OS, LS |
| International SSP | STORED | Only in FS, OS, LS |
| Axle load speed profile | NOT STORED | - |
| Route Suitability Data | NOT STORED | - |
| Mode Profile | STORED | Only in OS and LS mode |
| Track conditions excluding big metal masses | NOT STORED | - |
| Track condition big metal masses | NOT STORED | - |
| Unconditional Emergency Stop | NOT STORED | - |
| Conditional Emergency Stop | NOT STORED | - |
| Train Data | STORED | - |
| ERTMS/ETCS level | 2/3/4 | Level 1, 2, 3 |
| Driver ID | STORED | - |
| Train Running Number | STORED | - |
| Reversing Area Information | NOT STORED | - |
| Reversing Supervision Information | NOT STORED | - |
| Track Ahead Free Request | NOT STORED | - |
| Operated System Version | STORED | - |
| Virtual Balise Covers | NOT STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.13.2.3.5 Reduced Adhesion Conditions |  | 3130235 |
| Test Case of Feature | Use of the track condition: Adhesion Factor in the determination of supervision limits for target speed monitoring (Adhesion Factor is modified by the driver and received by trackside) |  | 3 |
| Applicable Mode/Level Combinations | L1: FS, LS, SR, OS L2/L3: FS, LS, SR, OS |  |  |
| Target of Test | The target of this test is to evaluate the use of the track condition: Adhesion Factor by the on board equipment to determine the speed supervision limits for target speed monitoring (Adhesion Factor is modified by the driver and received by trackside) |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.13.5.3 |  |
|  |  | Subset-026-3.13.5.4 |  |
|  |  | Subset-026-3.13.5.5 |  |
|  |  | Subset-026-3.13.6.2.1.1 |  |
|  |  | Subset-026-3.13.6.2.1.6 |  |
|  |  | Subset-026-3.18.4.6.1 |  |
|  |  | Subset-026-3.18.4.6.2 |  |
|  |  | Subset-026-3.18.4.6.2.1 |  |
|  |  | Subset-026-3.18.4.6.2.3 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | The test case shall consider the train data and the National Values (either a valid Packet 3 or the Default Values are used) as preconditions to be known before setting the appropriate values for the rest of the trackside info and the speed profile. The required observables (speed supervision limits) should be checked using a suitable tool that performs fully the “speed and distance monitoring" functionality (by default use ERA Braking curves tool, which can be downloaded from this website: http://www.era.europa.eu/Core-Activities/ERTMS/Pages/Braking-Curves-Simulation-Tool.aspx).. The design of the Test Case requires a decrease in the permitted speed within the braking curve. When adhesion condition is applied the system has already to be in a target speed monitoring section and the train speed will be low for no warning or brake application |
| Constraints | The target cannot be a EoA (use, for example, LoA o MRSP), in order to prevent the on-board from supervising the SBD curve, since, in this case, no adhesion conditions need to be taken into account for its calculation (see the requirements 3.13.6.2.1.1, 3.13.6.3.1.1 and 3.13.8.4.1). |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 /1 / 2 /12 | FS, OS, SR, LS |
| Radio communication session | ESTABLISHED | Only in L2 and L3 |
| National Values | STORED | Q_NVDRIVER_ADHES=1 |
| Not yet applicable National Values | NOT STORED | - |
| Movement Authority | STORED | Only in FS, OS, LS |
| Gradient Profile | STORED | Only in FS, OS, LS |
| International SSP | STORED | Only in FS, OS, LS |
| Axle load speed profile | NOT STORED | - |
| Route Suitability Data | NOT STORED | - |
| Mode Profile | STORED | Only in OS and LS mode |
| Track conditions excluding big metal masses | NOT STORED | - |
| Track condition big metal masses | NOT STORED | - |
| Unconditional Emergency Stop | NOT STORED | - |
| Conditional Emergency Stop | NOT STORED | - |
| Train Data | STORED | - |
| Adhesion factor | NOT STORED | - |
| ERTMS/ETCS level | 2/3/4 | Level 1, 2, 3 |
| Driver ID | STORED | - |
| Train Running Number | STORED | - |
| Reversing Area Information | NOT STORED | - |
| Reversing Supervision Information | NOT STORED | - |
| Track Ahead Free Request | NOT STORED | - |
| Operated System Version | 010 0000 | Version 2.0, introduced in SRS 3.3.0 |
| Virtual Balise Covers | NOT STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is moving (V_TRAIN=<V_PERMITTED) | I | INT | - | N/A | N/A | - |
| 2 | N/A | N/A | The driver requests the speed and distance monitoring information to be displayed | I | DMI | Only in OS or SR mode. | N/A | N/A | - |
| 3 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | Only for OS, SR | N/A | N/A | - |
| 4 | N/A | N/A | One Balise Group message (N_TOTAL=1) containing packet 71 (M_ADHESION= 1) is received | I | BTM | Only for L1 | N/A | N/A | - |
| 5 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | Only for L1 | N/A | N/A | - |
| 6 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 71 (M_ADHESION= 0) is received | I | RTM | Only for L2 /L3 | N/A | N/A | - |
| 7 | N/A | N/A | TELEGRAM FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | Only for L2 /L3 | N/A | N/A | - |
| 8 | N/A | N/A | Train is stopped (V_TRAIN=0) | I | INT | The train stops in the Adhesion factor - Slippery Rail area | N/A | N/A | - |
| 9 | N/A | N/A | The driver modifies "Adhesion Factor" | I | DMI | The driver selects Slippery rail | N/A | N/A | - |
| 10 | N/A | N/A | The driver confirms "Adhesion Factor" | I | DMI | The driver confirms Slippery rail | N/A | N/A | - |
| 11 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTION=0010 0000) is recorded | O | JRU | The selection of slippery rail by the driver is now recorded with M_DRIVERACTION = 0010 0000 | N/A | N/A | - |
| 12 | N/A | N/A | The Status symbol "Adhesion factor - slippery rail" is displayed | O | DMI | ST02: Adhesion factor “slippery rail" | N/A | N/A | - |
| 13 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit39=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 14 | N/A | N/A | Train is moving (V_TRAIN=<V_PERMITTED) | I | INT | - | N/A | N/A | - |
| 15 | N/A | N/A | One Balise Group message (N_TOTAL=1) is received | I | BTM | This balise group is used to reset the odometry. This balise group is used to reset the odometry. The balise group should be near the stopping point. | N/A | N/A | - |
| 16 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 17 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | Only in FS, OS or SR mode. The DMI aspect is coherent with Normal Status when braking to a target. Permitted speed displayed is compared to ERA tool calculation whit the condition (Adhesion Factor - Slippery Rail). The train has to be used the most restrictive value of the adhesion conditions selected by the driver and the adhesion conditions calculated from the trackside profile. | N/A | N/A | - |
| 18 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=2; M_SDMSUPSTAT=0; V_PERM=<V1>) is recorded | O | JRU | Only in FS, OS or SR mode. M_SDMTYPE (Speed and distance monitoring type) = 2 (Target speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal Status) V_PERM is compared to ERA tool calculation | N/A | N/A | - |

| Step 4: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 71 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_ADHESION | 15 | FINITE VALUE | - |
| L_ADHESION | 15 | FINITE VALUE | - |
| M_ADHESION | 1 | 0 | - |
| NID_PACKET | 8 | 255 | 255 |

| Step 4: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | 255 |

| Step 6: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 71 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_ADHESION | 15 | FINITE VALUE | - |
| L_ADHESION | 15 | FINITE VALUE | - |
| M_ADHESION | 1 | 0 | Slippery Rail |

| Step 15: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | 255 |

| Step 15: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | 255 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 /1 / 2 /12 | FS, OS, SR, LS |
| Radio communication session | ESTABLISHED | Only in L2 and L3 |
| Movement Authority | STORED | Only in FS, OS, LS |
| Gradient Profile | STORED | Only in FS, OS, LS |
| International SSP | STORED | Only in FS, OS, LS |
| Axle load speed profile | NOT STORED | - |
| Route Suitability Data | NOT STORED | - |
| Mode Profile | STORED | Only in OS and LS mode |
| Track conditions excluding big metal masses | NOT STORED | - |
| Track condition big metal masses | NOT STORED | - |
| Unconditional Emergency Stop | NOT STORED | - |
| Conditional Emergency Stop | NOT STORED | - |
| Train Data | STORED | - |
| ERTMS/ETCS level | 1/2/3 | Level 1, 2, 3 |
| Driver ID | STORED | - |
| Train Running Number | STORED | - |
| Reversing Area Information | NOT STORED | - |
| Reversing Supervision Information | NOT STORED | - |
| Track Ahead Free Request | NOT STORED | - |
| Operated System Version | STORED | - |
| Virtual Balise Covers | NOT STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |