| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 5180900 (5.18.9: Sounding the horn) Total: 2 Test case(s) |  |
| REF: | Subset-076-5-2-5180900 |
| ISSUE: | Version 3.2.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 2.9.0 31.03.2013 | All | First version/review | MULTITEL |
| Version 2.9.1 05.07.2013 | All | Template standardised with a dedicated tool, Answer to Review001 | MULTITEL |
| Version 2.9.2 15.10.2013 | All | Answer to Review001_Subset076-5-2-5180900_v291 | MULTITEL |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.2.0 31.03.2017 | All | Baseline 3 1st maintenance release version - Bug fixing | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.18.9: Sounding the horn |  | 5180900 |
| Test Case of Feature | “Sound horn” is displayed to the driver when approaching a “sound horn” area. |  | 1 |
| Applicable Mode/Level Combinations | L1 : FS, LS, OS L2 : FS, LS, OS L3 : FS, LS, OS |  |  |
| Target of Test | The target of this test is to check that “Sound horn” is displayed to the driver when the estimated front end of the train reaches a location in rear of the beginning of the “sound horn” area and taking into account the time necessary for the driver to perform the required action (see Appendix A3.1) and the current train speed. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.7.2 table - Output information Track conditions sound horn, non stopping areas, tunnel stopping areas |  |
|  |  | Subset-026-A.3.1 - Driver reaction time before sounding the horn |  |
|  |  | Subset-026-5.18.9.2 |  |
|  |  | Subset-026-5.18.9.2.1 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | When the train reaches a location in rear of the beginning of the “sound horn” area and taking into account the time necessary for the driver to perform the required action (see Appendix A3.1), check that “sound horn” is displayed on the DMI |
| Constraints | There is a “sound horn” area stored on-board and the estimated front end of the train is in rear of it. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Track conditions excluding big metal masses | STORED | A “sound horn” area stored on-board and the estimated front end of the train is in rear of it. |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Track Condition symbol "Sound horn" is not displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The Track Condition symbol "Sound horn" is displayed | O | DMI | The train reaches a location in rear of the beginning of the “sound horn” area with its estimated front end and taking into account the time necessary for the driver to perform the required action (4s) and the current train speed. | N/A | N/A | - |
| 2 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit78=1>) is recorded | O | JRU | Symbol TC35 | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Track Condition symbol "Sound horn" is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.18.9: Sounding the horn |  | 5180900 |
| Test Case of Feature | “Sound horn” is NO LONGER displayed when the train reaches the end location of the “sound horn” area. |  | 2 |
| Applicable Mode/Level Combinations | L1 : FS, LS, OS L2 : FS, LS, OS L3 : FS, LS, OS |  |  |
| Target of Test | The target of this test is to check that when the estimated front end of the train reaches the end location of the “sound horn” area, the indication “Sound horn” information is no longer displayed. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.12.1.2.1.4 |  |
|  |  | Subset-026-4.7.2 table - Output information Track conditions sound horn, non stopping areas, tunnel stopping areas |  |
|  |  | Subset-026-5.18.9.3 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | When the estimated front end of the train reaches the end location of the “sound horn” area, check that the indication “Sound horn” information is no longer displayed. |
| Constraints | “Sound horn” is displayed to the driver. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Track conditions excluding big metal masses | STORED | A “sound horn” area stored on-board |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Track Condition symbol "Sound horn" is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The Track Condition symbol "Sound horn" is removed | O | DMI | The train reaches the end location of the “sound horn” area with its estimated front end. | N/A | N/A | - |
| 2 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit78=0>) is recorded | O | JRU | Symbol TC35 | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Track Condition symbol "Sound horn" is not displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |