| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 5060200 (5.6.2: Table of requirements for “Shunting Initiated by Driver” procedure) Total: 7 Test case(s) |  |
| REF: | Subset-076-5-2-5060200 |
| ISSUE: | Version 3.2.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 0.0.1 23.03.2012 | All | First version/review | DLR |
| Version 2.9.0 31.03.2013 | All | First version/review for Baseline 3 | INECO |
| Version 2.9.1 05.07.2013 | All | Template standardised with a dedicated tool, TCs renumbered | MULTITEL |
| Version 2.9.2 04.10.2013 | TC3, TC4, TC6 | Modifications requested by issues 7 to 15 of Review002_Subset-076-5-2-5060200v291 | MULTITEL |
| Version 2.9.3 11.10.2013 | All | Response to Review003_Subset-076-5-2-5060200_v291 | MULTITEL |
| Version 2.9.4 05.12.2013 | TC1,6 | Response to issue 25 from Review003_Subset-076-5-2-5060200_v293 | MULTITEL |
| Version 2.9.5 17.01.2014 | TC6 | Response to issue 1 of Review001_Subset-076-5-2-5060200_v294 | MULTITEL |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.2.0 31.03.2017 | All | Baseline 3 1st maintenance release version - Bug fixing | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.6.2: Table of requirements for “Shunting Initiated by Driver” procedure |  | 5060200 |
| Test Case of Feature | Driver selects “Shunting” and train is at standstill, “Shunting granted by RBC” is received and the system switches to SH mode. The previous list of balise groups for SH area is replaced by a new list of balise groups for SH area. |  | 1 |
| Applicable Mode/Level Combinations | L2: FS, LS, OS, SR, SB, PT L3: FS, LS, OS, SR, SB, PT |  |  |
| Target of Test | The target of this test is to check that: If the driver selects SH mode and the current level is L2 or L3 then after the on–board has asked the RBC for authorisation and after having received the authorisation, the change of mode takes place and the previous list of balise groups for SH area is replaced by a new list of balise groups for SH area. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-5.6.2.2 A050 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The train switches to TR when it passes over a BG included in the old list of balise groups for SH area. |
| Constraints | A mission is on-going. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2/ 8/ 12 | FS/ OS/ SR/ PT/ LS |
| Radio communication session | ESTABLISHED | - |
| List of Balises for SH area | STORED | Not empty: with at least BG a |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill | O | INT | V_TRAIN=0 |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2 / L3 | FS / OS / SR / SB / PT / LS | The driver selects "SHUNTING" | I | DMI | - | L2 / L3 | FS / OS / SR / SB / PT / LS | - |
| 2 | L2 / L3 | FS / OS / SR / SB / PT / LS | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=11) is recorded | O | JRU | - | L2 / L3 | FS / OS / SR / SB / PT / LS | - |
| 3 | L2 / L3 | FS / OS / SR / SB / PT / LS | SA-DATA.Request with Euroradio Message "Request for Shunting" (NID_MESSAGE=130) | O | RTM | - | L2 / L3 | FS / OS / SR / SB / PT / LS | - |
| 4 | L2 / L3 | FS / OS / SR / SB / PT / LS | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=130) is recorded | O | JRU | - | L2 / L3 | FS / OS / SR / SB / PT / LS | - |
| 5 | L2 / L3 | FS / OS / SR / SB / PT / LS | SA-DATA.Indication with Euroradio Message "SH Authorised" (NID_MESSAGE=28) containing packet 49 is received | I | RTM | - | L2 / L3 | FS / OS / SR / SB / PT / LS | - |
| 6 | L2 / L3 | FS / OS / SR / SB / PT / LS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=28;NID_PACKET=49) is recorded | O | JRU | - | L2 / L3 | FS / OS / SR / SB / PT / LS | - |
| 7 | L2 / L3 | FS / OS / SR / SB / PT / LS | The Mode symbol "Shunting" is displayed | O | DMI | The system switches to SH mode Any previous list of balise groups for SH area is replaced by the new list | L2 / L3 | SH | - |
| 8 | L2 / L3 | SH | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21;Bit16=1) is recorded | O | JRU | MO01 displayed | L2 / L3 | SH | - |
| 9 | L2 / L3 | SH | SA-DATA.Request with Euroradio Message "End of Mission" (NID_MESSAGE=150) is transmitted | O | RTM | End of Mission Procedure | L2 / L3 | SH | - |
| 10 | L2 / L3 | SH | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=150) is recorded | O | JRU | - | L2 / L3 | SH | - |
| 11 | L2 / L3 | SH | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 42 (Q_RBC=0) is received | I | RTM | - | L2 / L3 | SH | - |
| 12 | L2 / L3 | SH | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=24;NID_PACKET=42;Q_RBC=0) is recorded | O | JRU | - | L2 / L3 | SH | - |
| 13 | L2 / L3 | SH | SA-DATA.Request with Euroradio Message "Termination of a communication session" (NID_MESSAGE=156) is transmitted | O | RTM | - | L2 / L3 | SH | - |
| 14 | L2 / L3 | SH | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=156) is recorded | O | JRU | - | L2 / L3 | SH | - |
| 15 | L2 / L3 | SH | SA-DATA.Indication with Euroradio Message "Acknowledgement of termination of a communication session" (NID_MESSAGE=39) is received | I | RTM | - | L2 / L3 | SH | - |
| 16 | L2 / L3 | SH | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=39) is recorded | O | JRU | - | L2 / L3 | SH | - |
| 17 | L2 / L3 | SH | SA-DISCONNECT.Request is transmitted | O | RTM | - | L2 / L3 | SH | - |
| 18 | L2 / L3 | SH | SA-DISCONNECT.Indication is received | I | RTM | - | L2 / L3 | SH | - |
| 19 | L2 / L3 | SH | Train is moving | I | INT | - | L2 / L3 | SH | - |
| 20 | L2 / L3 | SH | One Balise Group message is received | I | BTM | BG b included in the new list of balises for SH area | L2 / L3 | SH | - |
| 21 | L2 / L3 | SH | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L2 / L3 | SH | - |
| 22 | L2 / L3 | SH | The Mode symbol "Shunting" is displayed | O | DMI | No Trip after a BG b in the new list of balises for SH area | L2 / L3 | SH | - |
| 23 | L2 / L3 | SH | One Balise Group message is received | I | BTM | BG a included in the old list of balises for SH area | L2 / L3 | SH | - |
| 24 | L2 / L3 | SH | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L2 / L3 | SH | - |
| 25 | L2 / L3 | SH | The Mode symbol "Trip" is displayed | O | DMI | - | L2 / L3 | TR | - |
| 26 | L2 / L3 | TR | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21;Bit19=1) is recorded | O | JRU | MO04 displayed | L2 / L3 | TR | - |

| Step 3: Radio Message 130 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 130 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | Not: 4294967295 (unknown) |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Packet 0 must not be used if position report is based on 2 BG |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC |
| NID_PACKET | 8 | 1 | Packet 1 must be used if position report is based on 2 BG |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PRVLRBG | 24 | FINITE VALUE | Used as reference for all directional information in the packet: a move from PRVLRBG towards the LRBG defines the “nominal” direction |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | Train orientation according to reference direction |
| Q_DLRBG | 2 | FINITE VALUE | Train front position according to reference direction |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = “Train integrity confirmed by integrity monitoring device” or “Train integrity confirmed by driver” |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | Actual running direction according to reference direction |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC |

| Step 5: Radio Message 28 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 28 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | Not: 4294967295 (unknown) |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_PACKET | 8 | 49 | List of balises for SH Area |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | BG b | - |

| Step 9: Radio Message 150 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 150 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Packet 0 must not be used if position report is based on 2 BG |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC |
| NID_PACKET | 8 | 1 | Packet 1 must be used if position report is based on 2 BG |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PRVLRBG | 24 | FINITE VALUE | Used as reference for all directional information in the packet: a move from PRVLRBG towards the LRBG defines the “nominal” direction |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | Train orientation according to reference direction |
| Q_DLRBG | 2 | FINITE VALUE | Train front position according to reference direction |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = “Train integrity confirmed by integrity monitoring device” or “Train integrity confirmed by driver” |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | Actual running direction according to reference direction |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC |

| Step 11: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 42 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_RBC | 1 | 0 | Terminate communication session. |
| NID_C | 10 | FINITE VALUE | RBC ETCS identity : NID_C not relevant if NID_RBC has value "Contact last known RBC". |
| NID_RBC | 14 | FINITE VALUE | - |
| NID_RADIO | 64 | FINITE VALUE | Not relevant if NID_RBC has value "Contact last known RBC". |
| Q_SLEEPSESSION | 1 | FINITE VALUE | - |

| Step 13: Radio Message 156 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 156 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | t0 |
| NID_ENGINE | 24 | FINITE VALUE | - |

| Step 15: Radio Message 39 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 39 | - |
| L_MESSAGE | 10 | 75 | - |
| T_TRAIN | 32 | FINITE VALUE | t0 + t’ with t’ > 0 |
| M_ACK | 1 | FINITE VALUE | always set to 0 |
| NID_LRBG | 24 | FINITE VALUE | - |

| Step 20: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 010 0000 | - |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | 1 of N |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BG b |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 20: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 010 0000 | - |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 1 | N of N |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BG b |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 23: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 010 0000 | - |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 0 | 1 of N |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BG a |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 23: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 010 0000 | - |
| Q_MEDIA | 1 | 1 | Balise |
| N_PIG | 3 | 1 | N of N |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BG a |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 | TR |
| Radio communication session | NOT ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.6.2: Table of requirements for “Shunting Initiated by Driver” procedure |  | 5060200 |
| Test Case of Feature | Driver selects “Shunting”, train is at standstill (Transition [5]) and the system switches to SH mode. An on-going mission shall NOT exist. |  | 2 |
| Applicable Mode/Level Combinations | L0: SB L1: SB, PT LNTC: SB, SN |  |  |
| Target of Test | The target of this test is to check that: The on-board equipment switches to SH mode after the driver selects “Shunting” and train is at standstill In case of selection of SH mode by the driver, the switch to shunting is always accepted by the on – board equipment The entry of the ERTMS / ETCS on–board equipment in SH mode is not considered as an end of mission because an on-going mission doesn’t exist. The process ends. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.4.8.1.6 |  |
|  |  | Subset-026-4.4.8.1.7 |  |
|  |  | Subset-026-4.4.8.1.7 a) |  |
|  |  | Subset-026-4.6.2 row 4 column 2 |  |
|  |  | Subset-026-4.6.2 row 4 column 13 |  |
|  |  | Subset-026-4.6.3 Condition Id [5] |  |
|  |  | Subset-026-5.6.2.2 E015 |  |
|  |  | Subset-026-5.6.2.2 D020 |  |
|  |  | Subset-026-5.6.2.2 A050 |  |
|  |  | Subset-026-5.6.2.2 D040 |  |
|  |  | Subset-026-5.6.2.2 D080 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: SH mode symbol is DISPLAYED The permitted speed is DISPLAYED Check on the JRU that: The Driver selection is RECORDED. The transition to SH mode is RECORDED. The change of permitted speed is RECORDED. Driver request for permitted speed is RECORDED |
| Constraints | The train is at standstill. The ERTMS/ETCS level is 0, NTC or 1. The driver selects SH mode. An on-going mission shall NOT exist. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 / 8 | SB / PT |
| ERTMS/ETCS level | 0 / 1 / 2 | L0 / LNTC / L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The option for selection of SH is AVAILABLE. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill | O | INT | V_TRAIN=0 |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / LNTC / L1 | SB / PT / SN | The driver presses “Shunting” button. | I | DMI | - | L0 / LNTC / L1 | SB / PT / SN | - |
| 2 | L0 / LNTC / L1 | SB / PT / SN | Driver selection is RECORDED. | O | JRU | - | L0 / LNTC / L1 | SB / PT / SN | - |
| 3 | L0 / LNTC / L1 | SB / PT / SN | The SH symbol, MO01 is DISPLAYED. | O | DMI | - | L0 / LNTC / L1 | SH | - |
| 4 | L0 / LNTC / L1 | SH | The displayed symbol for SH mode is RECORDED | O | JRU | - | L0 / LNTC / L1 | SH | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 | SH |
| ERTMS/ETCS level | 0 / 1 / 2 | L0 / LNTC / L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Indication of SH mode, symbol MO01 is DISPLAYED. New permitted speed is DISPLAYED. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill | I | INT | V_TRAIN=0 |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.6.2: Table of requirements for “Shunting Initiated by Driver” procedure |  | 5060200 |
| Test Case of Feature | Driver selects “Shunting” and train is at standstill, “Shunting granted by RBC” is received and the system switches to SH mode. No on-going mission exists. |  | 3 |
| Applicable Mode/Level Combinations | L2: SB, PT L3: SB, PT |  |  |
| Target of Test | The target of this test is to check that: If the driver selects SH mode and the current level is L2 or L3 then the on–board shall ask the RBC for authorisation, after receiving the authorisation the change of mode takes place, the ERTMS / ETCS on–board equipment enters in the SH there is No on-going mission, the level is still L2/L3, so a position report is send to RBC to report mode change the on-board equipment waits the RBC order to terminate the communication session. The order is received and the on-board ends the communication session. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.4.8.1.6 |  |
|  |  | Subset-026-4.4.8.1.7 |  |
|  |  | Subset-026-4.4.8.1.7 b) |  |
|  |  | Subset-026-4.6.2 row 4 column 2 |  |
|  |  | Subset-026-4.6.2 row 4 column 13 |  |
|  |  | Subset-026-4.6.3 Condition Id [6] |  |
|  |  | Subset-026-4.7.2 table - Input information Shunting request |  |
|  |  | Subset-026-4.8.3.1.1 SH authorised + (optional) List of Balises for SH area |  |
|  |  | Subset-026-4.8.4.2 SH authorised + (optional) List of Balises for SH Area |  |
|  |  | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-5.6.2.2 A045 |  |
|  |  | Subset-026-5.6.2.2 S050 |  |
|  |  | Subset-026-5.6.2.2 A050 |  |
|  |  | Subset-026-5.6.2.2 D040 |  |
|  |  | Subset-026-5.6.2.2 D080 |  |
|  |  | Subset-026-5.6.2.2 A095 |  |
|  |  | Subset-026-5.6.2.2 S100 |  |
|  |  | Subset-026-5.6.2.2 A115 |  |
|  |  | Subset-026-8.6.2 Message 130: Request for Shunting |  |
|  |  | Subset-026-8.7.11 Message 28: SH Authorised |  |
|  |  | Subset-026-8.6.4 Message 136: Train Position Report |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The hourglass symbol is DISPLAYED. SH mode symbol is DISPLAYED Check on the JRU that: The messages 130, 28, transition to SH mode and 136 are RECORDED Check on the RTM that: A position report is sent |
| Constraints | The train is at standstill The ERTMS/ ETCS level is 2 or 3 and a communication session in on-going The information “Shunting granted by RBC”, due to a Shunting request from the driver is received. No on-going mission shall exist. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 / 8 | SB / PT |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2/ L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The option for selection of SH is AVAILABLE. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill | I | INT | V_TRAIN=0 |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2 / L3 | SB / PT | The driver presses “Shunting” button. | I | DMI | - | L2 / L3 | SB / PT | - |
| 2 | L2 / L3 | SB / PT | Driver action is RECORDED | O | JRU | - | L2 / L3 | SB / PT | - |
| 3 | L2 / L3 | SB / PT | On board sends message 130 :“Request for Shunting” | O | RTM | - | L2 / L3 | SB / PT | - |
| 4 | L2 / L3 | SB / PT | Message 130 is RECORDED | O | JRU | - | L2 / L3 | SB / PT | - |
| 5 | L2 / L3 | SB / PT | In the main window, the hourglass symbol is DISPLAYED. | O | DMI | This hourglass symbol is ST05 (according to the document ERA_ERTMS_015560 v330), which shall be displayed to show that the onboard awaits an answer from the RBC | L2 / L3 | SB / PT | - |
| 6 | L2 / L3 | SB / PT | Message 28 (“SH authorised”) is received by the on- board system | I | RTM | - | L2 / L3 | SB / PT | - |
| 7 | L2 / L3 | SB / PT | Message 28 is RECORDED | O | JRU | - | L2 / L3 | SH | - |
| 8 | L2 / L3 | SB / PT | The SH symbol is DISPLAYED. | O | DMI | - | L2 / L3 | SH | - |
| 9 | L2 / L3 | SH | The displayed symbol MO01 forSH mode is RECORDED. | O | JRU | - | L2 / L3 | SH | - |
| 10 | L2 / L3 | SH | The on-board reports mode change to RBC by sending message 136: “Train Position Report” | O | RTM | - | L2 / L3 | SH | - |
| 11 | L2 / L3 | SH | Message 136 is RECORDED | O | JRU | - | L2 / L3 | SH | - |
| 12 | L2 / L3 | SH | Use_FT3050500.4,5: Termination of a communication session ordered by RBC | - | - | The on-board waits for the order to terminate session. | L2 / L3 | SH | - |

| Step 3: Radio Message 130 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 130 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | Not: 4294967295 (unknown) |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | packet 1 must be used if position report is based on 2 BG |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| Step 6: Radio Message 28 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 28 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | Not: 4294967295 (unknown) |

| Step 10: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | packet 1 must be used if position report is based on 2 BG |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 | SH |
| Radio communication session | NOT ESTABLISHED | - |
| Movement Authority | NOT STORED | deleted |
| List of Balises for SH area | NOT STORED / STORED | DELETED or REPLACED |
| Train Data | NOT STORED | deleted |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| SH symbol mode is DISPLAYED. The hourglass symbol ST05 is DISPLAYED. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.6.2: Table of requirements for “Shunting Initiated by Driver” procedure |  | 5060200 |
| Test Case of Feature | The train is at standstill and the driver selects shunting. A radio message 130 (request for shunting) is sent to the RBC together with a position report. The on-board equipment awaits the replay to the SH request. When the radio message 27 (shunting refused) is received from the RBC, the driver is informed that the shunting request has been refused by the RBC and the process ends. |  | 4 |
| Applicable Mode/Level Combinations | L2: FS, LS, OS, SR, SB, PT L3: FS, LS, OS, SR, SB, PT |  |  |
| Target of Test | The target of this test is to check that : The “shunting request” can be selected by the driver when the train is at standstill. A radio message 130 (request for shunting) together with a position report ( with special value “position unknown” if the position is not known) is sent to the RBC when a shunting request is received from the driver The driver is informed that the shunting request has been refused by the RBC when the radio message 27 (shunting refused) is received from the RBC. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.4.8.1.7 b) |  |
|  |  | Subset-026-4.4.8.1.7 |  |
|  |  | Subset-026-4.4.8.1.6 |  |
|  |  | Subset-026-4.7.2 table - Input information Shunting request |  |
|  |  | Subset-026-4.7.2 table - Output information Shunting refused by RBC |  |
|  |  | Subset-026-4.8.3.1.1 SH refused |  |
|  |  | Subset-026-4.8.4.2 SH refused |  |
|  |  | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-4.8.4.2 [1] |  |
|  |  | Subset-026-5.6.2.2 E015 |  |
|  |  | Subset-026-5.6.2.2 D020 |  |
|  |  | Subset-026-5.6.2.2 A045 |  |
|  |  | Subset-026-5.6.2.2 S050 |  |
|  |  | Subset-026-5.6.2.2 A220 |  |
|  |  | Subset-026-8.6.2 Message 130: Request for Shunting |  |
|  |  | Subset-026-8.7.10 Message 27: SH refused |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on DMI that: The hourglass symbol is DISPLAYED. The text message “Shunting Refused ” is displayed when the radio message 27 (shunting refused) is received from the RBC Check on RTM that: The radio message 130 (request for shunting) is sent to the RBC. The radio message 27 (shunting refused) is received from the RBC Check on JRU: The driver selection of “Shunting” is Recorded The sent radio message 130 is Recorded. The received radio message 27 is Recorded. |
| Constraints | A cab needs to be active (SB Mode). Only if following the reception of the information “Recognition of Exit from TR mode” with a more recent time stamp. (for PT mode) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 8 / 12 | FS / OS / SR / SB / PT / LS |
| Radio communication session | ESTABLISHED | - |
| Train Data | STORED | Validated train data is STORED. |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| "Shunting" button is AVAILABLE | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill | I | INT | V_TRAIN=0 |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The driver presses “shunting” button. | I | DMI | - | N/A | N/A | - |
| 2 | N/A | N/A | The driver’s action “Shunting selected” is RECORDED | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The radio message 130 (request for shunting) is sent to the RBC | O | RTM | - | N/A | N/A | - |
| 4 | N/A | N/A | The radio message 130 sent by the on-board is RECORDED | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | In the main window, the hourglass symbol is DISPLAYED | O | DMI | This hourglass symbol is ST05 (according to the document ERA_ERTMS_015560 v330), which shall be displayed to show that the onboard awaits an answer from the RBC | N/A | N/A | - |
| 6 | N/A | N/A | The radio message 27 (shunting refused) is received from RBC | I | RTM | - | N/A | N/A | - |
| 7 | N/A | N/A | The radio message 27 received from RBC is RECORDED | O | JRU | - | N/A | N/A | - |
| 8 | N/A | N/A | The text message “Shunting refused” is DISPLAYED | O | DMI | Inform driver | N/A | N/A | - |
| 9 | N/A | N/A | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23) is recorded | O | JRU | - | N/A | N/A | - |

| Step 3: Radio Message 130 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 130 | Request for shunting |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | packet 1 must be used if position report is based on 2 BG |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| Step 6: Radio Message 27 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 27 | SH refused |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| T_TRAIN | 32 | T0 | Time stamp of the shunting request |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 8 / 12 | FS / OS / SR / SB / PT / LS |
| Radio communication session | ESTABLISHED | - |
| Train Data | STORED | and VALID |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The text message "shunting refused" is DISPLAYED. The hourglass symbol is DISPLAYED. Main window dialog sequence is DISPLAYED (only in SB mode). | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill | I | INT | V_TRAIN=0 |

## Test case 5

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.6.2: Table of requirements for “Shunting Initiated by Driver” procedure |  | 5060200 |
| Test Case of Feature | Driver selects “Shunting”, train is at standstill (Transition [5]) and the system switches to SH mode. An on-going mission exists.“End of Mission“ procedure triggered. |  | 5 |
| Applicable Mode/Level Combinations | L0: UN L1: FS, LS, OS, SR, PT |  |  |
| Target of Test | The target of this test is to check that: The on-board equipment switches to SH mode after the driver selects “Shunting” and train is at standstill In case of selection of SH mode by the driver, the switch to shunting is always accepted by the on – board equipment The entry of the ERTMS / ETCS on – board equipment in SH mode shall be considered as an end of mission. Exception: from PT mode only if there was an on-going mission. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.4.8.1.6 |  |
|  |  | Subset-026-4.4.8.1.7 |  |
|  |  | Subset-026-4.4.8.1.7 a) |  |
|  |  | Subset-026-4.6.2 row 4 column 5 |  |
|  |  | Subset-026-4.6.2 row 4 column 6 |  |
|  |  | Subset-026-4.6.2 row 4 column 7 |  |
|  |  | Subset-026-4.6.2 row 4 column 8 |  |
|  |  | Subset-026-4.6.2 row 4 column 11 |  |
|  |  | Subset-026-4.6.2 row 4 column 13 |  |
|  |  | Subset-026-4.6.3 Condition Id [5] |  |
|  |  | Subset-026-5.6.2.2 E015 |  |
|  |  | Subset-026-5.6.2.2 D020 |  |
|  |  | Subset-026-5.6.2.2 A050 |  |
|  |  | Subset-026-5.6.2.2 D040 |  |
|  |  | Subset-026-5.6.2.2 A100 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: SH mode symbol is DISPLAYED The permitted speed is DISPLAYED Check on the JRU that: The Driver selection is RECORDED The transition to SH mode is RECORDED The change of permitted speed is RECORDED. Driver request for permitted speed is RECORDED. |
| Constraints | The train is at standstill. The ERTMS/ETCS level is 0 or 1. The driver selects SH mode. The entry of the ERTMS/ETCS on-board equipment into the SH mode, from PT mode is considered as an EoM if there was an on-going mission. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 8 / 12 | FS, OS, SR, UN, PT, LS |
| ERTMS/ETCS level | 0 / 2 | L0 / L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The option for selection of SH is AVAILABLE. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill | I | INT | V_TRAIN=0 |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / L1 | UN / FS / LS / OS / SR / PT | The driver presses “shunting” button | I | DMI | - | L0 / L1 | UN / FS / LS / OS / SR / PT | - |
| 2 | L0 / L1 | UN / FS / LS / OS / SR / PT | Driver selection is RECORDED | O | JRU | - | L0 / L1 | UN / FS / LS / OS / SR / PT | - |
| 3 | L0 / L1 | UN / FS / LS / OS / SR / PT | The SH symbol is DISPLAYED | O | DMI | The system switches to SH SH symbol is MO01( according to the document ERA_ERTMS_015560 v330). | L0 / L1 | SH | - |
| 4 | L0 / L1 | SH | The transition to SH mode is RECORDED | O | JRU | - | L0 / L1 | SH | - |
| 5 | L0 / L1 | SH | The change of permitted speed is RECORDED | O | JRU | - | L0 / L1 | SH | - |
| 6 | L0 / L1 | SH | Driver requests the permitted speed | I | DMI | - | L0 / L1 | SH | - |
| 7 | L0 / L1 | SH | Driver request is RECORDED | O | JRU | - | L0 / L1 | SH | - |
| 8 | L0 / L1 | SH | The permitted speed is DISPLAYED | O | DMI | - | L0 / L1 | SH | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 | SH |
| ERTMS/ETCS level | 0 / 2 | L0 / L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The SH mode symbol is DISPLAYED. New permitted speed is DISPLAYED. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill | O | INT | V_TRAIN=0 |

## Test case 6

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.6.2: Table of requirements for “Shunting Initiated by Driver” procedure |  | 5060200 |
| Test Case of Feature | Driver selects “Shunting” and train is at standstill, “Shunting granted by RBC” is received and the system switches to SH mode. |  | 6 |
| Applicable Mode/Level Combinations | L2: FS, LS, OS, SR, PT L3: FS, LS, OS, SR, PT |  |  |
| Target of Test | The target of this test is to check that: If the driver selects SH mode and the current level is L2 or L3 then the on–board shall ask the RBC for authorisation, after receiving the authorisation the change of mode takes place, the entry of the ERTMS / ETCS on–board equipment in the SH mode shall be considered as an end of mission. Exception: from PT mode only if there was an on-going mission. End of mission procedure is triggered. The previous list of balise groups for SH area shall be deleted. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.4.8.1.6 |  |
|  |  | Subset-026-4.4.8.1.7 |  |
|  |  | Subset-026-4.4.8.1.7 b) |  |
|  |  | Subset-026-4.6.2 row 4 column 8 |  |
|  |  | Subset-026-4.6.2 row 4 column 5 |  |
|  |  | Subset-026-4.6.2 row 4 column 13 |  |
|  |  | Subset-026-4.6.2 row 4 column 7 |  |
|  |  | Subset-026-4.6.2 row 4 column 6 |  |
|  |  | Subset-026-4.6.3 Condition Id [6] |  |
|  |  | Subset-026-4.7.2 table - Input information Shunting request |  |
|  |  | Subset-026-4.10.1.3 line List of balises for SH area |  |
|  |  | Subset-026-4.8.3.1.1 SH authorised + (optional) List of Balises for SH area |  |
|  |  | Subset-026-4.8.4.2 SH authorised + (optional) List of Balises for SH Area |  |
|  |  | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-5.6.2.2 E015 |  |
|  |  | Subset-026-5.6.2.2 D020 |  |
|  |  | Subset-026-5.6.2.2 A045 |  |
|  |  | Subset-026-5.6.2.2 A100 |  |
|  |  | Subset-026-5.6.2.2 S050 |  |
|  |  | Subset-026-5.6.2.2 A050 |  |
|  |  | Subset-026-5.6.2.2 D040 |  |
|  |  | Subset-026-5.11.2.2 S140 c) |  |
|  |  | Subset-026-8.6.2 Message 130: Request for Shunting |  |
|  |  | Subset-026-8.7.11 Message 28: SH Authorised |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: SH mode symbol is DISPLAYED. The hourglass symbol is DISPLAYED The train does not switch to TR when it passes over a BG included in the old list of balise groups for SH area. Check on the JRU that: The messages 130, 28 and transition to SH mode are RECORDED |
| Constraints | The train is at standstill in L2/L3.Communication session established. Driver selects SH mode. The entry of the ERTMS/ETCS on-board equipment into the SH mode, from PT mode is considered as an EoM if there is an on-going mission. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1/ 2/ 8 / 12 | FS, OS, SR, PT, LS |
| Radio communication session | ESTABLISHED | - |
| List of Balises for SH area | STORED | Not empty: with at least BG a |
| ERTMS/ETCS level | 3 / 4 | L2/ L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The option for selection of SH is AVAILABLE. | O | DMI | At the main window |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill | I | INT | V_TRAIN=0 |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2 / L3 | FS / LS / OS / SR / PT | The driver presses “Shunting” button | I | DMI | - | L2 / L3 | FS / LS / OS / SR / PT | - |
| 2 | L2 / L3 | FS / LS / OS / SR / PT | Driver action is RECORDED. | O | JRU | - | L2 / L3 | FS / LS / OS / SR / PT | - |
| 3 | L2 / L3 | FS / LS / OS / SR / PT | SA-DATA.Request with Euroradio Message "Request for Shunting" (NID_MESSAGE=130) | O | RTM | - | L2 / L3 | FS / LS / OS / SR / PT | - |
| 4 | L2 / L3 | FS / LS / OS / SR / PT | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=130) is recorded | O | JRU | - | L2 / L3 | FS / LS / OS / SR / PT | - |
| 5 | L2 / L3 | FS / LS / OS / SR / PT | In the main window, the hourglass symbol is DISPLAYED | O | DMI | This hourglass symbol is ST05 (according to the document ERA_ERTMS_015560 v330), which shall be displayed to show that the onboard awaits an answer from the RBC | L2 / L3 | FS / LS / OS / SR / PT | - |
| 6 | L2 / L3 | FS / LS / OS / SR / PT | Message 28 (“SH Authorised”) is received by the on- board system | I | RTM | - | L2 / L3 | FS / LS / OS / SR / PT | - |
| 7 | L2 / L3 | FS / LS / OS / SR / PT | Message 28 is RECORDED | O | JRU | - | L2 / L3 | FS / LS / OS / SR / PT | - |
| 8 | L2 / L3 | FS / LS / OS / SR / PT | The SH symbol is DISPLAYED. | O | DMI | SH symbol is MO01( according to the document ERA_ERTMS_015560 v330). The system switches to SH mode | L2 / L3 | SH | - |
| 9 | L2 / L3 | SH | The displayed symbol MO01 for SH mode is RECORDED. | O | JRU | - | L2 / L3 | SH | - |
| 10 | L2 / L3 | SH | Use_FT5050300.3 | - | - | Inform track-side about End of Mission procedure. Termination of communication session ordered by RBC | L2 / L3 | SH | - |
| 11 | L2 / L3 | SH | Train is moving | I | INT | - | L2 / L3 | SH | - |
| 12 | L2 / L3 | SH | One Balise Group message is received | I | BTM | BG b was not included in the old list of balises for SH area | L2 / L3 | SH | - |
| 13 | L2 / L3 | SH | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L2 / L3 | SH | - |
| 14 | L2 / L3 | SH | The Mode symbol "Shunting" is displayed | O | DMI | No Trip after a BG b not in the old list of balises for SH area, because the list was deleted | L2 / L3 | SH | - |

| Step 3: Radio Message 130 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 130 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | Not: 4294967295 (unknown) |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Packet 0 must not be used if position report is based on 2 BG |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC |
| NID_PACKET | 8 | 1 | Packet 1 must be used if position report is based on 2 BG |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PRVLRBG | 24 | FINITE VALUE | Used as reference for all directional information in the packet: a move from PRVLRBG towards the LRBG defines the “nominal” direction |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | Train orientation according to reference direction |
| Q_DLRBG | 2 | FINITE VALUE | Train front position according to reference direction |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = “Train integrity confirmed by integrity monitoring device” or “Train integrity confirmed by driver” |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | Actual running direction according to reference direction |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC |

| Step 6: Radio Message 28 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 28 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | Not: 4294967295 (unknown) |

| Step 12: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 010 0000 | - |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 0 | 1 of N |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BG b |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 12: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 010 0000 | - |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 1 | N of N |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BG b |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 | SH |
| Radio communication session | NOT ESTABLISHED | - |
| Movement Authority | NOT STORED | deleted |
| List of Balises for SH area | NOT STORED | deleted |
| Train Data | NOT STORED | deleted |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| SH symbol mode is DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is NOT at standstill | I | INT | - |

## Test case 7

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.6.2: Table of requirements for “Shunting Initiated by Driver” procedure |  | 5060200 |
| Test Case of Feature | Driver selects “Shunting” and train is at standstill, the system switches to SH mode. An on-going mission exists. |  | 7 |
| Applicable Mode/Level Combinations | L0: UN LNTC: SN L1: FS, LS, OS, SR L2: FS, LS, OS, SR L3: FS, LS, OS, SR |  |  |
| Target of Test | The target of this test is to check that: If the driver selects SH mode, the ERTMS / ETCS on–board equipment enters in the SH mode. The entry of the ERTMS/ETCS on–board equipment in SH mode shall be considered as an end of mission. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.7.2 table - Input information Shunting request |  |
|  |  | Subset-026-5.6.2.2 A100 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The hourglass symbol is DISPLAYED in L2/L3. SH mode symbol is DISPLAYED Check on the JRU that: The messages 130 and 28 (in L2/L3) and transition to SH mode are RECORDED Check on the RTM that: The messages 130 and 28 are exchanged in L2/L3 |
| Constraints | For Level NTC no national trip procedure is active. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/1/2/4/12/13 | FS/OS/SR/UN/LS/SN |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0/LNTC/L1/L2/ L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The option for selection of SH is AVAILABLE. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill | I | INT | V_TRAIN=0 |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0/LNTC/L1/L2/ L3 | FS/OS/SR/UN/LS/SN | The driver selects "SHUNTING" | I | DMI | - | L0/LNTC/L1/L2/ L3 | FS/OS/SR/UN/LS/SN | - |
| 2 | L0/LNTC/L1/L2/ L3 | FS/OS/SR/UN/LS/SN | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=11) is recorded | O | JRU | - | L0/LNTC/L1/L2/ L3 | FS/OS/SR/UN/LS/SN | - |
| 3 | L0/LNTC/L1/L2/ L3 | FS/OS/SR/UN/LS/SN | SA-DATA.Request with Euroradio Message "Request for Shunting" (NID_MESSAGE=130) is transmitted | O | RTM | Only if in L2 or L3. | L0/LNTC/L1/L2/ L3 | FS/OS/SR/UN/LS/SN | - |
| 4 | L0/LNTC/L1/L2/ L3 | FS/OS/SR/UN/LS/SN | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=130) is recorded | O | JRU | Only if in L2 or L3. | L0/LNTC/L1/L2/ L3 | FS/OS/SR/UN/LS/SN | - |
| 5 | L0/LNTC/L1/L2/ L3 | FS/OS/SR/UN/LS/SN | The main window is displayed. | O | DMI | Only if in L2 or L3. The hourglass symbol ST05 (according to the document ERA_ERTMS_015560 v330), is displayed to show that the onboard awaits an answer from the RBC. | L0/LNTC/L1/L2/ L3 | FS/OS/SR/UN/LS/SN | - |
| 6 | L0/LNTC/L1/L2/ L3 | FS/OS/SR/UN/LS/SN | SA-DATA.Indication with Euroradio Message "SH Authorised" (NID_MESSAGE=28) is received | I | RTM | Only if in L2 or L3. | L0/LNTC/L1/L2/ L3 | FS/OS/SR/UN/LS/SN | - |
| 7 | L0/LNTC/L1/L2/ L3 | FS/OS/SR/UN/LS/SN | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=28) is recorded | O | JRU | Only if in L2 or L3. | L0/LNTC/L1/L2/ L3 | FS/OS/SR/UN/LS/SN | - |
| 8 | L0/LNTC/L1/L2/ L3 | FS/OS/SR/UN/LS/SN | The Mode symbol "Shunting" is displayed. | O | DMI | MO01 | L0/LNTC/L1/L2/ L3 | SH | - |
| 9 | L0/LNTC/L1/L2/ L3 | SH | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit16=1>) is recorded | O | JRU | MO01 | L0/LNTC/L1/L2/ L3 | SH | - |
| 10 | L0/LNTC/L1/L2/ L3 | SH | Use_FT5050300.3: End of Mission Procedure. | - | - | - | L0/LNTC/L1/L2/ L3 | SH | - |

| Step 3: Radio Message 130 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 130 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | Not: 4294967295 (unknown) |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | packet 1 must be used if position report is based on 2 BG |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| Step 6: Radio Message 28 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 28 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | Not: 4294967295 (unknown) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 | SH |
| Radio communication session | NOT ESTABLISHED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0/LNTC/L1/L2/ L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill | I | INT | V_TRAIN=0 |