| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 3131030 (Requirements for Ceiling speed monitoring (Tables 5, 6 and 7)) Total: 8 Test case(s) |  |
| REF: | Subset-076-5-2-3131030 |
| ISSUE: | Version 3.2.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 2.9.2 18.11.2013 | All | First version/review | CEDEX |
| Version 2.9.1 30.06.2013 | All | First version/review | CEDEX |
| Version 2.9.2 18.11.2013 | All | Review | CEDEX |
| Version 2.9.3 03.12.2013 | All | Creation of TC 8 | CEDEX |
| Version 2.9.4 21.01.2014 | All | Review | CEDEX |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 14.01.2015 | All | SRS 340 | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.2.0 31.03.2017 | All | Baseline 3 1st maintenance release version - Bug fixing | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | Requirements for Ceiling speed monitoring (Tables 5, 6 and 7) |  | 3131030 |
| Test Case of Feature | Triggering condition t1 of Table 5 (Normal Status). The permitted speed is displayed. |  | 1 |
| Applicable Mode/Level Combinations | L0: SH, UN LNTC: SH, L1: SH, FS, LS, SR, OS, RV L2/L3: SH, FS, LS, SR, OS, RV |  |  |
| Target of Test | The target of this test is to check that: When the train is running in the Ceiling speed monitoring section with Vest < VMRSP, no intervention is commanded and Normal Status is displayed. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.13.10.3.1 |  |
|  |  | Subset-026-3.13.10.3.3 |  |
|  |  | Subset-026-3.13.10.3.3 table 5 |  |
|  |  | Subset-026-3.13.10.3.5 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: - The train speed is below the permitted speed. - Normal Status in Ceiling Speed Monitoring Section is displayed. Check on the TIU that: - No intervention by the on-board equipment is commanded. Check on the JRU that: - The train speed is below the permitted speed. - No intervention by the on-board equipment is recorded. |
| Constraints | The speed and distance monitoring function is active within a ceiling speed monitoring section. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 4 / 12 / 14 | FS / OS / SR / SH / UN / LS / RV |
| Movement Authority | STORED | Only in FS, OS, LS |
| Gradient Profile | STORED | Only in FS, OS, LS |
| International SSP | STORED | Only in FS, OS, LS |
| List of Balises in SR Authority + SR mode speed limit and distance | STORED | Only in SR |
| Mode Profile | STORED | Only in OS, LS, SH |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |
| Reversing Area Information | STORED | Only in RV |
| Reversing Supervision Information | STORED | Only in RV |
| Operated System Version | 010 0000 | Version 2.0, introduced in SRS 3.3.0 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is moving (V_TRAIN ≤ VMRSP) | I | INT | - | N/A | N/A | - |
| 2 | N/A | N/A | The driver requests the speed and distance monitoring information to be displayed | I | DMI | Only in SH, SR and OS mode. (The permitted speed shall be displayed until the driver requests to stop this. See for example Req 4.4.12.1.4) | N/A | N/A | - |
| 3 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The DMI aspect is coherent with Normal Status . The Permitted Speed is displayed (only if not in LS or UN mode). (As there is no change in the DMI, there is not JRU step) | N/A | N/A | - |
| 4 | N/A | N/A | NOT Service brake commanded | O | TIU | - | N/A | N/A | - |
| 5 | N/A | N/A | NOT SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 4 / 12 / 14 | FS / OS / SR / SH / UN / LS / RV |
| Movement Authority | STORED | Only in FS, OS, LS |
| Gradient Profile | STORED | Only in FS, OS, LS |
| International SSP | STORED | Only in FS, OS, LS |
| List of Balises in SR Authority + SR mode speed limit and distance | STORED | Only in SR |
| Mode Profile | STORED | Only in OS, LS, SH |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |
| Reversing Area Information | STORED | Only in RV |
| Reversing Supervision Information | STORED | Only in RV |
| Operated System Version | 010 0000 | Version 2.0, introduced in SRS 3.3.0 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| The brakes are not commanded | O | TIU | - |
| Normal Status is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | Requirements for Ceiling speed monitoring (Tables 5, 6 and 7) |  | 3131030 |
| Test Case of Feature | Triggering condition t2 of Table 5 (Overspeed Status). Revoking condition r1 of Table 6 (Normal Status). Transitions of Table 7, Normal status --> Overspeed status --> Normal status. |  | 2 |
| Applicable Mode/Level Combinations | L0: SH, UN LNTC: SH L1: SH, FS, LS, SR, OS, RV L2/L3: SH, FS, LS, SR, OS, RV |  |  |
| Target of Test | The target of this test is to check that: When the train is running in the Ceiling speed monitoring section with Vest > VMRSP, no intervention is commanded and Overspeed Status is displayed. Once Vest < VMRSP, the Normal Status is restored. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.13.10.3.1 |  |
|  |  | Subset-026-3.13.10.3.2 |  |
|  |  | Subset-026-3.13.10.3.3 |  |
|  |  | Subset-026-3.13.10.3.3 table 5 |  |
|  |  | Subset-026-3.13.10.3.3 table 6 |  |
|  |  | Subset-026-3.13.10.3.4 |  |
|  |  | Subset-026-3.13.10.3.4 table 7 |  |
|  |  | Subset-026-3.13.10.3.5 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: - The train speed is between the permitted speed and the Warning limit. - Overspeed Status in Ceiling Speed Monitoring Section is displayed. - Once the train speed is below the permitted speed, Normal Status is displayed. Check on the TIU that: - No intervention by the on-board equipment is commanded. Check on the JRU that: - The train speed is between the permitted speed and the Warning limit. - The Overspeed Status is recorded. - No intervention by the on-board equipment is recorded. - The train speed is below the permitted speed. - The Normal Status is recorded. |
| Constraints | The speed and distance monitoring function is active within a ceiling speed monitoring section. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 4 / 12 / 14 | FS / OS / SR / SH / UN / LS / RV |
| Movement Authority | STORED | Only in FS, OS, LS |
| Gradient Profile | STORED | Only in FS, OS, LS |
| International SSP | STORED | Only in FS, OS, LS |
| List of Balises in SR Authority + SR mode speed limit and distance | STORED | Only in SR |
| Mode Profile | STORED | Only in OS, LS, SH |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |
| Reversing Area Information | STORED | Only in RV |
| Reversing Supervision Information | STORED | Only in RV |
| Operated System Version | 010 0000 | Version 2.0, introduced in SRS 3.3.0 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is moving (V_TRAIN ≤ VMRSP) | I | INT | - | N/A | N/A | - |
| 2 | N/A | N/A | The driver requests the speed and distance monitoring information to be displayed | I | DMI | Only in SH, SR and OS mode. (The permitted speed shall be displayed until the driver requests to stop this. See for example Req 4.4.12.1.4) | N/A | N/A | - |
| 3 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The DMI aspect is coherent with Normal Status . The Permitted Speed is displayed (only if not in LS or UN mode). (As there is no change in the DMI, there is not JRU step with Normal Status recorded) | N/A | N/A | - |
| 4 | N/A | N/A | Train is moving (V_TRAIN > VMRSP) | I | INT | - | N/A | N/A | - |
| 5 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | When the V_TRAIN (estimated speed) > Vmrsp, then the supervision status changes to "Overspeed Status" (t2 in 3.13.10.3.3 table 5 of the SRS). So, the DMI aspect is coherent with Overspeed Status in a Ceiling Speed Monitoring section. Only in FS mode: the DMI shall display the SBI Speed (3.13.10.3.2) | N/A | N/A | - |
| 6 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=0; M_SDMSUPSTAT=2) is recorded | O | JRU | M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 2 (Overspeed Status) | N/A | N/A | - |
| 7 | N/A | N/A | NOT Service brake commanded | O | TIU | - | N/A | N/A | - |
| 8 | N/A | N/A | NOT SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | Train is moving (V_TRAIN ≤ VMRSP) | I | INT | - | N/A | N/A | - |
| 10 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | When the V_TRAIN (estimated speed) ≤ Vmrsp, then the supervision status changes to "Normal Status" (r1 in 3.13.10.3.3 table 6 of the SRS). | N/A | N/A | - |
| 11 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=0; M_SDMSUPSTAT=0) is recorded | O | JRU | M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal Status) | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 4 / 12 / 14 | FS / OS / SR / SH / UN / LS / RV |
| Movement Authority | STORED | Only in FS, OS, LS |
| Gradient Profile | STORED | Only in FS, OS, LS |
| International SSP | STORED | Only in FS, OS, LS |
| List of Balises in SR Authority + SR mode speed limit and distance | STORED | Only in SR |
| Mode Profile | STORED | Only in OS, LS, SH |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |
| Reversing Area Information | STORED | Only in RV |
| Reversing Supervision Information | STORED | Only in RV |
| Operated System Version | 010 0000 | Version 2.0, introduced in SRS 3.3.0 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| The brakes are not commanded | O | TIU | - |
| Normal Status is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | Requirements for Ceiling speed monitoring (Tables 5, 6 and 7) |  | 3131030 |
| Test Case of Feature | Triggering condition t3 of Table 5 (Warning Status). Revoking condition r1 of Table 6 (Normal Status). Transitions of Table 7, Normal status --> Warning status --> Normal status. |  | 3 |
| Applicable Mode/Level Combinations | L0: SH, UN LNTC: SH L1: SH, FS, LS, SR, OS, RV L2/L3: SH, FS, LS, SR, OS, RV |  |  |
| Target of Test | The target of this test is to check that: When the train is running in the Ceiling speed monitoring section with Vest > VMRSP + dVWarning(VMRSP), no intervention is commanded and Warning Status is displayed. Once Vest < VMRSP, the Normal Status is restored. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.13.10.3.1 |  |
|  |  | Subset-026-3.13.10.3.2 |  |
|  |  | Subset-026-3.13.10.3.3 |  |
|  |  | Subset-026-3.13.10.3.3 table 5 |  |
|  |  | Subset-026-3.13.10.3.3 table 6 |  |
|  |  | Subset-026-3.13.10.3.4 |  |
|  |  | Subset-026-3.13.10.3.4 table 7 |  |
|  |  | Subset-026-3.13.10.3.5 |  |
|  |  | Subset-026-3.13.9.2.3 |  |
|  |  | Subset-026-3.13.9.2.6 |  |
|  |  | Subset-026-A.3.1 - Speed difference between Permitted speed and Warning supervision limits, minimum value |  |
|  |  | Subset-026-A.3.1 - Speed difference between Permitted speed and Warning supervision limits, maximum value |  |
|  |  | Subset-026-A.3.1 - Value of MRSP where dV_warning starts to increase to dV_warning_max |  |
|  |  | Subset-026-A.3.1 - Value of MRSP where dV_warning stops to increase to dV_warning_max |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: - The train speed is between the Warning Limit and the SBI speed. - Warning Status in Ceiling Speed Monitoring Section is displayed. - Once the train speed is below the permitted speed, Normal Status is displayed. Check on the TIU that: - No intervention by the on-board equipment is commanded. Check on the JRU that: - The train speed is between the Warning Limit and the SBI speed. - The Warning Status is recorded. - No intervention by the on-board equipment is recorded. - The train speed is below the permitted speed. - The Normal Status is recorded. |
| Constraints | The speed and distance monitoring function is active within a ceiling speed monitoring section. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 4 / 12 / 14 | FS / OS / SR / SH / UN / LS / RV |
| Movement Authority | STORED | Only in FS, OS, LS |
| Gradient Profile | STORED | Only in FS, OS, LS |
| International SSP | STORED | Only in FS, OS, LS |
| List of Balises in SR Authority + SR mode speed limit and distance | STORED | Only in SR |
| Mode Profile | STORED | Only in OS, LS, SH |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |
| Reversing Area Information | STORED | Only in RV |
| Reversing Supervision Information | STORED | Only in RV |
| Operated System Version | 010 0000 | Version 2.0, introduced in SRS 3.3.0 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is moving (V_TRAIN ≤ VMRSP) | I | INT | - | N/A | N/A | - |
| 2 | N/A | N/A | The driver requests the speed and distance monitoring information to be displayed | I | DMI | Only in SH, SR and OS mode. (The permitted speed shall be displayed until the driver requests to stop this. See for example Req 4.4.12.1.4) | N/A | N/A | - |
| 3 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The DMI aspect is coherent with Normal Status . The Permitted Speed is displayed (only if not in LS, or UN mode). (As there is no change in the DMI, there is not JRU step with Normal Status recorded) | N/A | N/A | - |
| 4 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | When the V_TRAIN (estimated speed) > Vmrsp, then the supervision status changes to "Overspeed Status" (t2 in 3.13.10.3.3 table 5 of the SRS). So, the DMI aspect is coherent with Overspeed Status in a Ceiling Speed Monitoring section. Only in FS mode: the DMI shall display the SBI Speed (3.13.10.3.2) | N/A | N/A | - |
| 5 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=0; M_SDMSUPSTAT=2) is recorded | O | JRU | M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 2 (Overspeed Status) | N/A | N/A | - |
| 6 | N/A | N/A | Train is moving (V_TRAIN > Vmrsp + dVwarning(Vmrsp)) | I | INT | - | N/A | N/A | - |
| 7 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | When the V_TRAIN (estimated speed) > Vmrsp + dVwarning(Vmrsp), then the supervision status changes to "Warning Status" (t3 in 3.13.10.3.3 table 5 of the SRS). So, the DMI aspect is coherent with Warning Status in a Ceiling Speed Monitoring section. Only in FS mode: the DMI shall display the SBI Speed (3.13.10.3.2) No “SPEED AND DISTANCE MONITORING INFORMATION“ message is recorded on the JRU after this step, since there is no change on the DMI when the status changes from “Overspeed” to “Warning”. | N/A | N/A | - |
| 8 | N/A | N/A | NOT Service brake commanded | O | TIU | - | N/A | N/A | - |
| 9 | N/A | N/A | NOT SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | N/A | N/A | - |
| 10 | N/A | N/A | Train is moving (V_TRAIN ≤ VMRSP) | I | INT | - | N/A | N/A | - |
| 11 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | When the V_TRAIN (estimated speed) < Vmrsp, then the supervision status changes to "Normal Status" (r1 in 3.13.10.3.3. table 6 and transition: Warning Status --> Normal Status in 3.13.10.3.4 table 7 of the SRS). So, the DMI aspect is coherent with Normal Status in a Ceiling Speed Monitoring section. | N/A | N/A | - |
| 12 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=0; M_SDMSUPSTAT=0) is recorded | O | JRU | M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal Status) | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 4 / 12 / 14 | FS / OS / SR / SH / UN / LS / RV |
| Movement Authority | STORED | Only in FS, OS, LS |
| Gradient Profile | STORED | Only in FS, OS, LS |
| International SSP | STORED | Only in FS, OS, LS |
| List of Balises in SR Authority + SR mode speed limit and distance | STORED | Only in SR |
| Mode Profile | STORED | Only in OS, LS, SH |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |
| Reversing Area Information | STORED | Only in RV |
| Reversing Supervision Information | STORED | Only in RV |
| Operated System Version | 010 0000 | Version 2.0, introduced in SRS 3.3.0 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| The brakes are not commanded | O | TIU | - |
| Normal Status is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | Requirements for Ceiling speed monitoring (Tables 5, 6 and 7) |  | 3131030 |
| Test Case of Feature | Triggering condition t4 of Table 5 (Intervention Status). Revoking condition r1 of Table 6 (Normal Status). Transitions of Table 7, Normal status --> Intervention status --> Normal status. |  | 4 |
| Applicable Mode/Level Combinations | L0: SH, UN LNTC: SH L1: SH, FS, LS, SR, OS L2/L3: SH, FS, LS, SR, OS |  |  |
| Target of Test | The target of this test is to check that: When the train is running in the Ceiling speed monitoring section with Vest > VMRSP + dVsbi(VMRSP), the SB intervention is commanded and Intervention Status is displayed. Once Vest < VMRSP, the SB is released and the Normal Status is displayed. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.13.10.3.1 |  |
|  |  | Subset-026-3.13.10.3.2 |  |
|  |  | Subset-026-3.13.10.3.3 |  |
|  |  | Subset-026-3.13.10.3.3 table 5 |  |
|  |  | Subset-026-3.13.10.3.3 table 6 |  |
|  |  | Subset-026-3.13.10.3.4 |  |
|  |  | Subset-026-3.13.10.3.4 table 7 |  |
|  |  | Subset-026-3.13.10.3.5 |  |
|  |  | Subset-026-3.13.9.2.3 |  |
|  |  | Subset-026-3.13.9.2.5 |  |
|  |  | Subset-026-A.3.1 - Speed difference between Permitted speed and Service Brake Intervention supervision limits, minimum value |  |
|  |  | Subset-026-A.3.1 - Speed difference between Permitted speed and Service Brake Intervention supervision limits, maximum value |  |
|  |  | Subset-026-A.3.1 - Value of MRSP where dV_sbi starts to increase to dV_sbi_max |  |
|  |  | Subset-026-A.3.1 - Value of MRSP where dV_sbi stops to increase to dV_sbi_max |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: - The train speed is between the SBI and the EBI speed. - Intervention Status in Ceiling Speed Monitoring Section is displayed. - Once the train speed is below the permitted speed, Normal Status is displayed. Check on the TIU that: - Service Brake by the on-board equipment is commanded. - Once the train speed is below the permitted speed, Service Brake is released. Check on the JRU that: - The train speed is between the SBI and the EBI speed. - Service Brake command by the on-board equipment is recorded. - The Intervention Status is recorded. - The train speed is below the permitted speed. - Service Brake release by the on-board equipment is recorded. - The Normal Status is recorded. |
| Constraints | The speed and distance monitoring function is active within a ceiling speed monitoring section. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 4 / 12 | FS / OS / SR / SH / UN / LS |
| Movement Authority | STORED | Only in FS, OS, LS |
| Gradient Profile | STORED | Only in FS, OS, LS |
| International SSP | STORED | Only in FS, OS, LS |
| List of Balises in SR Authority + SR mode speed limit and distance | STORED | Only in SR |
| Mode Profile | STORED | Only in OS, LS, SH |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |
| Operated System Version | 010 0000 | Version 2.0, introduced in SRS 3.3.0 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is moving (V_TRAIN ≤ VMRSP) | I | INT | - | N/A | N/A | - |
| 2 | N/A | N/A | The driver requests the speed and distance monitoring information to be displayed | I | DMI | Only in SH, SR and OS mode. (The permitted speed shall be displayed until the driver requests to stop this. See for example Req 4.4.12.1.4) | N/A | N/A | - |
| 3 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The DMI aspect is coherent with Normal Status . Permitted speed is displayed (only if not in LS or UN mode). (As there is no change in the DMI, there is not JRU step with Normal Status recorded) | N/A | N/A | - |
| 4 | N/A | N/A | Train is moving (V_TRAIN > Vmrsp + dVsbi(Vmrsp)) | I | INT | - | N/A | N/A | - |
| 5 | N/A | N/A | Service brake commanded | O | TIU | - | N/A | N/A | - |
| 6 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | The Status symbol "Service Brake or Emergency Brake intervention" is displayed | O | DMI | - | N/A | N/A | - |
| 8 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=1) is recorded | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | When the V_TRAIN (estimated speed) > Vmrsp + dVsbi(Vmrsp), then the supervision status changes to "Intervention Status" (t4 in 3.13.10.3.3 table 5 of the SRS). So, the DMI aspect is coherent with Intervention Status in a Ceiling Speed Monitoring section. Only in FS mode: the DMI shall display the SBI Speed (3.13.10.3.2) | N/A | N/A | - |
| 10 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=0; M_SDMSUPSTAT=4) is recorded | O | JRU | M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 4 (Intervention Status) | N/A | N/A | - |
| 11 | N/A | N/A | Train is moving (V_TRAIN < Vmrsp) | I | INT | - | N/A | N/A | - |
| 12 | N/A | N/A | Service brake not commanded | O | TIU | - | N/A | N/A | - |
| 13 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=0) is recorded | O | JRU | - | N/A | N/A | - |
| 14 | N/A | N/A | The Status symbol "Service Brake or Emergency Brake intervention" is removed | O | DMI | - | N/A | N/A | - |
| 15 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=0) is recorded | O | JRU | - | N/A | N/A | - |
| 16 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | When the V_TRAIN (estimated speed) < Vmrsp, then the supervision status changes to "Normal Status" (r1 in 3.13.10.3.3. table 6 and transition: Intervention Status --> Normal Status in 3.13.10.3.4 table 7 of the SRS). So, the DMI aspect is coherent with Normal Status in a Ceiling Speed Monitoring section. | N/A | N/A | - |
| 17 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=0; M_SDMSUPSTAT=0) is recorded | O | JRU | M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal Status) | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 4 / 12 | FS / OS / SR / SH / UN / LS |
| Movement Authority | STORED | Only in FS, OS, LS |
| Gradient Profile | STORED | Only in FS, OS, LS |
| International SSP | STORED | Only in FS, OS, LS |
| List of Balises in SR Authority + SR mode speed limit and distance | STORED | Only in SR |
| Mode Profile | STORED | Only in OS, LS, SH |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |
| Operated System Version | 010 0000 | Version 2.0, introduced in SRS 3.3.0 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| The brakes are not commanded | O | TIU | - |
| Normal Status is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 5

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | Requirements for Ceiling speed monitoring (Tables 5, 6 and 7) |  | 3131030 |
| Test Case of Feature | Triggering condition t5 of Table 5 (Intervention Status). Revoking condition r0 of Table 6 (Normal Status). Transitions of Table 7, Normal status --> Intervention status --> Normal status. |  | 5 |
| Applicable Mode/Level Combinations | L0: SH, UN LNTC: SH L1: SH, FS, LS, SR, OS, RV L2/L3: SH, FS, LS, SR, OS, RV |  |  |
| Target of Test | The target of this test is to check that: When the train is running in the Ceiling speed monitoring section with Vest > VMRSP + dVebi(VMRSP), the EB intervention is commanded and Intervention Status is displayed. Once the train is at standstill, the EB is released and the Normal Status is displayed. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.13.10.3.1 |  |
|  |  | Subset-026-3.13.10.3.2 |  |
|  |  | Subset-026-3.13.10.3.3 |  |
|  |  | Subset-026-3.13.10.3.3 table 5 |  |
|  |  | Subset-026-3.13.10.3.3 table 6 |  |
|  |  | Subset-026-3.13.10.3.4 |  |
|  |  | Subset-026-3.13.10.3.4 table 7 |  |
|  |  | Subset-026-3.13.10.3.5 |  |
|  |  | Subset-026-3.13.9.2.3 |  |
|  |  | Subset-026-A.3.1 - Speed difference between Permitted speed and Emergency Brake Intervention supervision limits, minimum value |  |
|  |  | Subset-026-A.3.1 - Speed difference between Permitted speed and Emergency Brake Intervention supervision limits, maximum value |  |
|  |  | Subset-026-A.3.1 - Value of MRSP where dV_ebi starts to increase to dV_ebi_max |  |
|  |  | Subset-026-A.3.1 - Value of MRSP where dV_ebi stops to increase to dV_ebi_max |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: - The train speed is over the EBI speed. - Intervention Status in Ceiling Speed Monitoring Section is displayed. - Once the train reaches standstill, Normal Status is displayed. Check on the TIU that: - Emergency Brake by the on-board equipment is commanded. - Once the train reaches standstill, Emergency Brake is released. Check on the JRU that: - The train speed is over the EBI speed. - Emergency Brake command by the on-board equipment is recorded. - The Intervention Status is recorded. - The train speed is equal to zero. - Emergency Brake release by the on-board equipment is recorded. - The Normal Status is recorded. |
| Constraints | The speed and distance monitoring function is active within a ceiling speed monitoring section. In order to reach the EBI limit, it shall be necessary either to send a SSP information before testing this TC that obliges the on-board to be above the EBI limit after accepting the message, or either to previously disable the reaction to the TCO and SB while testing this TC. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 4 / 12 / 14 | FS / OS / SR / SH / UN / LS / RV |
| National Values | STORED | Q_NVEMRRLS= 0 |
| Movement Authority | STORED | Only in FS, OS, LS |
| Gradient Profile | STORED | Only in FS, OS, LS |
| International SSP | STORED | Only in FS, OS, LS |
| List of Balises in SR Authority + SR mode speed limit and distance | STORED | Only in SR |
| Mode Profile | STORED | Only in OS, LS, SH |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |
| Reversing Area Information | STORED | Only in RV |
| Reversing Supervision Information | STORED | Only in RV |
| Operated System Version | 010 0000 | Version 2.0, introduced in SRS 3.3.0 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is moving (V_TRAIN ≤ VMRSP) | I | INT | - | N/A | N/A | - |
| 2 | N/A | N/A | The driver requests the speed and distance monitoring information to be displayed | I | DMI | Only in SH, SR and OS mode. (The permitted speed shall be displayed until the driver requests to stop this. See for example Req 4.4.12.1.4) | N/A | N/A | - |
| 3 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | Optional step (this step is only needed if the TC is not tested by means of sending a SSP information before testing this TC that obliges the on-board to be above the EBI limit after accepting the message): The DMI aspect is coherent with Normal Status . Permitted speed is displayed (only if not in LS or UN mode). (As there is no change in the DMI, there is not JRU step with Normal Status recorded) | N/A | N/A | - |
| 4 | N/A | N/A | Train is moving (V_TRAIN > Vmrsp + dVebi(Vmrsp)) | I | INT | - | N/A | N/A | - |
| 5 | N/A | N/A | Emergency brake commanded | O | TIU | - | N/A | N/A | - |
| 6 | N/A | N/A | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | The Status symbol "Service Brake or Emergency Brake intervention" is displayed | O | DMI | - | N/A | N/A | - |
| 8 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=1) is recorded | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | When the V_TRAIN (estimated speed) > Vmrsp + dVebi(Vmrsp), then the supervision status changes to "Intervention Status" (t5 in 3.13.10.3.3 table 5 of the SRS). So, the DMI aspect is coherent with Intervention Status in a Ceiling Speed Monitoring section. Only in FS mode: the DMI shall display the SBI Speed (3.13.10.3.2) | N/A | N/A | - |
| 10 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=0; M_SDMSUPSTAT=4) is recorded | O | JRU | M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 4 (Intervention Status) | N/A | N/A | - |
| 11 | N/A | N/A | Train is at standstill (V_TRAIN=0; TIME= 60) | I | INT | National Value Q_NVEMRRLS= 0. The Emergency Brake is released only at standstill. | N/A | N/A | - |
| 12 | N/A | N/A | Emergency brake not commanded | O | TIU | - | N/A | N/A | - |
| 13 | N/A | N/A | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=0) is recorded | O | JRU | - | N/A | N/A | - |
| 14 | N/A | N/A | The Status symbol "Service Brake or Emergency Brake intervention" is removed | O | DMI | - | N/A | N/A | - |
| 15 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=0) is recorded | O | JRU | - | N/A | N/A | - |
| 16 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | When the V_TRAIN (estimated speed) < Vmrsp, then the supervision status changes to "Normal Status" (r0 in 3.13.10.3.3. table 6 and transition: Intervention Status --> Normal Status in 3.13.10.3.4 table 7 of the SRS). So, the DMI aspect is coherent with Normal Status in a Ceiling Speed Monitoring section. | N/A | N/A | - |
| 17 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=0; M_SDMSUPSTAT=0) is recorded | O | JRU | M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal Status) | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 4 / 12 / 14 | FS / OS / SR / SH / UN / LS / RV |
| National Values | STORED | Q_NVEMRRLS= 0 |
| Movement Authority | STORED | Only in FS, OS, LS |
| Gradient Profile | STORED | Only in FS, OS, LS |
| International SSP | STORED | Only in FS, OS, LS |
| List of Balises in SR Authority + SR mode speed limit and distance | STORED | Only in SR |
| Mode Profile | STORED | Only in OS, LS, SH |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |
| Reversing Area Information | STORED | Only in RV |
| Reversing Supervision Information | STORED | Only in RV |
| Operated System Version | 010 0000 | Version 2.0, introduced in SRS 3.3.0 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| The brakes are not commanded | O | TIU | - |
| Normal Status is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill | I | INT | - |

## Test case 6

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | Requirements for Ceiling speed monitoring (Tables 5, 6 and 7) |  | 3131030 |
| Test Case of Feature | Transition of Table 7, Indication status --> Normal status. |  | 6 |
| Applicable Mode/Level Combinations | L1: FS, SR, OS L2/L3: FS, SR, OS |  |  |
| Target of Test | The target of this test is to check that: When the train is running in the Target speed monitoring section with Indication Status and enters in the Ceiling speed monitoring section with Vest < VMRSP, the Normal Status is displayed. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.13.10.3.1 |  |
|  |  | Subset-026-3.13.10.3.3 |  |
|  |  | Subset-026-3.13.10.3.3 table 6 |  |
|  |  | Subset-026-3.13.10.3.4 |  |
|  |  | Subset-026-3.13.10.3.4 table 7 |  |
|  |  | Subset-026-3.13.10.3.6 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: - The train speed is below the permitted speed. - Indication Status in Target Speed Monitoring Section is displayed. - Normal Status in Ceiling Speed Monitoring Section is displayed. Check on the JRU that: - The train speed is below the permitted speed. - The Indication Status in Target Speed Monitoring Section is recorded. - Track information triggering the change from Target Speed Monitoring to Ceiling Speed Monitoring is recorded. - The Normal Status in Ceiling Speed Monitoring Section is recorded. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 | FS / OS / SR |
| Radio communication session | ESTABLISHED | Only in L2 / L3 |
| Movement Authority | STORED | Only in FS, OS |
| Gradient Profile | STORED | Only in FS, OS |
| International SSP | STORED | Only in FS, OS |
| List of Balises in SR Authority + SR mode speed limit and distance | STORED | Only in SR |
| Mode Profile | STORED | Only in OS |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Operated System Version | 010 0000 | Version 2.0, introduced in SRS 3.3.0 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | Both | RTM | Only in L2 / L3 |
| The brakes are not commanded | O | TIU | - |
| Normal Status in Target Speed Monitoring Section is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is moving (V_TRAIN ≤ VMRSP) | I | INT | The train is in a target speed monitoring section and the estimated speed is < Vmrsp (It has to be ensured the brakes are not applied) | N/A | N/A | - |
| 2 | N/A | N/A | The driver requests the speed and distance monitoring information to be displayed | I | DMI | Only in SR and OS mode. (The permitted speed shall be displayed until the driver requests to stop this. See for example Req 4.4.12.1.4) | N/A | N/A | - |
| 3 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The DMI aspect is coherent with Indication Status. Permitted speed is displayed. (As there is no change in the DMI, there is not JRU step with Indication Status recorded) | N/A | N/A | - |
| 4 | N/A | N/A | One Balise Group message containing , packet 12, packet 27, packet 21 is received | I | BTM | Only in L1. The MA is extended in order to change to a Ceiling Speed Monitoring Section. | N/A | N/A | - |
| 5 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | Only in L1. | N/A | N/A | - |
| 6 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 21 and packet 27 is received | I | RTM | Only in L2/L3. The MA is extended in order to change to a Ceiling Speed Monitoring Section. | N/A | N/A | - |
| 7 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | Only in L2/L3 | N/A | N/A | - |
| 8 | N/A | N/A | Train is moving (V_TRAIN ≤ VMRSP) | I | INT | The train is now in a ceiling speed monitoring section and the estimated speed is < Vmrsp. | N/A | N/A | - |
| 9 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | When the V_TRAIN (estimated speed) ≤ Vmrsp, then the supervision status is "Normal Status" (3.13.10.3.4 table 7 transition: Indication Status --> Normal status ). | N/A | N/A | - |
| 10 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=0; M_SDMSUPSTAT=0) is recorded | O | JRU | M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal Status) | N/A | N/A | - |

| Step 4: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 12 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out". |
| N_ITER | 5 | FINITE VALUE | Set to zero if V_MAIN = 0 or if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | 255 |

| Step 6: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 | FS / OS / SR |
| Radio communication session | ESTABLISHED | Only in L2 / L3 |
| Movement Authority | STORED | Only in FS, OS |
| Gradient Profile | STORED | Only in FS, OS |
| International SSP | STORED | Only in FS, OS |
| List of Balises in SR Authority + SR mode speed limit and distance | STORED | Only in SR |
| Mode Profile | STORED | Only in OS |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Operated System Version | 010 0000 | Version 2.0, introduced in SRS 3.3.0 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | Both | RTM | Only in L2 / L3 |
| The brakes are not commanded | O | TIU | - |
| Normal Status in Ceiling Speed Monitoring Section is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 7

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | Requirements for Ceiling speed monitoring (Tables 5, 6 and 7) |  | 3131030 |
| Test Case of Feature | A increase of the MRSP shall be supervised by the equipment taking into account the min safe front end of the train. |  | 7 |
| Applicable Mode/Level Combinations | L0: UN L1: FS, LS, SR, OS L2/L3: FS, LS, SR, OS |  |  |
| Target of Test | The target of this test is to check that: When the train is running in the Ceiling speed monitoring section, the MRSP increments are supervised with the min safe front end of the train. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.13.10.2.8 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | This functionality shall be tested by means of a TSR, as an appropiate method (for the current level/mode combinations) to trigger an increase in the MRSP when leaving the TSR area. Check on the DMI that: - The train speed is below the permitted speed. - Normal Status in Ceiling Speed Monitoring Section is displayed. - The permitted speed is only updated when the minimum safe front end of the train has entered into the new MRSP section Check on the JRU that: - The train speed is below the permitted speed. - With a delay corresponding to the train confidence interval, the permitted speed change is recorded. |
| Constraints | It is recommended to implement this TC with a high confidence interval, in order to make noticeable the delay in the update of the permitted speed. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 12 | FS / OS / SR / UN / LS |
| Movement Authority | STORED | Only in FS, OS, LS |
| Gradient Profile | STORED | Only in FS, OS, LS |
| International SSP | STORED | Only in FS, OS, LS |
| List of Balises in SR Authority + SR mode speed limit and distance | STORED | Only in SR |
| Mode Profile | STORED | Only in OS, LS |
| ERTMS/ETCS level | 0 / 2 / 3 / 4 | L0 / L1 / L2 / L3 |
| Operated System Version | 010 0000 | Version 2.0, introduced in SRS 3.3.0 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is moving (V_TRAIN ≤ VTSR) | I | INT | - | N/A | N/A | - |
| 2 | N/A | N/A | The driver requests the speed and distance monitoring information to be displayed | I | DMI | Only in SR and OS mode. (The permitted speed shall be displayed until the driver requests to stop this. See for example Req 4.4.12.1.4) | N/A | N/A | - |
| 3 | N/A | N/A | One Balise Group message (N_TOTAL=1) containing packet 65 (D_TSR=0; V_TSR<VMRSP(L_TSR); Q_FRONT=1) is received | I | BTM | Q_FRONT= 1 (no train length delay), D_TSR= 0 and L_TSR > 0. The value of V_TSR shall be chosen to be less than the V_MRSP at L_TSR, in order to have an increase in the permitted speed once the TSR area is left. | N/A | N/A | - |
| 4 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The DMI aspect is coherent with Normal Status . Permitted speed is updated (only if not in LS or UN mode). Only if in LS or UN mode: the train speed will be a bit over the PS, in order to see the changes below, since the PS is not displayed in these modes. | N/A | N/A | - |
| 6 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM = VTSR) is recorded | O | JRU | Only if not in LS or UN mode: M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal Status) Only if in LS or UN mode: M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 2 (Overspeed Status) V_TRAIN will be used instead of V_PERM for comparing with V_TSR | N/A | N/A | - |
| 7 | N/A | N/A | Train is moving (V_TRAIN ≤ VTSR) | I | INT | The train is at the end of the TSR (L_TSR). It has to be ensured the equipment remains in ceiling speed area (it depends on mode for example with a low speed or with a long MA for example) | N/A | N/A | - |
| 8 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The supervision status is "Normal Status". The MRSP increases. Check that the train supervises this increase with the min safe front end (min safe front end: req 3.6.4.4 of the SRS) | N/A | N/A | - |
| 9 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; TRAIN_POSITION=L_TSR+L_DOUBTUNDER; V_PERM > VTSR) is recorded | O | JRU | Only if not in LS or UN mode: M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal Status) Only if in LS or UN mode: M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 2 (Overspeed Status) V_TRAIN will be used instead of V_PERM for comparing with V_TSR | N/A | N/A | - |

| Step 3: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 65 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_TSR | 8 | FINITE VALUE | - |
| D_TSR | 15 | 0 | - |
| L_TSR | 15 | FINITE VALUE | The value shall be chosen in agreement with V_TSR, in order to fulfill the condition V_TSR<V_MRSP(L_TSR) |
| Q_FRONT | 1 | 1 | - |
| V_TSR | 7 | FINITE VALUE | The value shall be chosen in agreement with L_TSR, in order to fulfill the condition V_TSR<V_MRSP(L_TSR) |
| NID_PACKET | 8 | 255 | 255 |

| Step 3: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | 255 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 12 | FS / OS / SR / UN / LS |
| Movement Authority | STORED | Only in FS, OS, LS |
| Gradient Profile | STORED | Only in FS, OS, LS |
| International SSP | STORED | Only in FS, OS, LS |
| List of Balises in SR Authority + SR mode speed limit and distance | STORED | Only in SR |
| Mode Profile | STORED | Only in OS, LS |
| ERTMS/ETCS level | 0 / 2 / 3 / 4 | L0 / L1 / L2 / L3 |
| Operated System Version | 010 0000 | Version 2.0, introduced in SRS 3.3.0 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| The brakes are not commanded | O | TIU | - |
| Normal Status is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 8

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | Requirements for Ceiling speed monitoring (Tables 5, 6 and 7) |  | 3131030 |
| Test Case of Feature | The SBI speed is displayed when the supervision status is Overspeed. Triggering condition t2 of Table 5 (Overspeed Status). Revoking condition r1 of Table 6 (Normal Status). |  | 8 |
| Applicable Mode/Level Combinations | L1: FS L2/L3: FS |  |  |
| Target of Test | The target of this test is to check that: When the train is running in the Ceiling speed monitoring section with Vest > VMRSP, no intervention is commanded and Overspeed Status is displayed, which includes the SBI speed is displayed. Once Vest < VMRSP, the Normal Status is restored. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.13.10.3.1 |  |
|  |  | Subset-026-3.13.10.3.2 |  |
|  |  | Subset-026-3.13.10.3.3 |  |
|  |  | Subset-026-3.13.10.3.3 table 5 |  |
|  |  | Subset-026-3.13.10.3.3 table 6 |  |
|  |  | Subset-026-3.13.10.3.4 |  |
|  |  | Subset-026-3.13.10.3.4 table 7 |  |
|  |  | Subset-026-3.13.10.3.5 |  |
|  |  | Subset-026-4.7.2 table - Output information SBI Speed |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: - The train speed is between the permitted speed and the Warning limit. - Overspeed Status in Ceiling Speed Monitoring Section is displayed. - The SBI speed is displayed. - Once the train speed is below the permitted speed, Normal Status is displayed. Check on the JRU that: - The train speed is between the permitted speed and the Warning limit. - The Overspeed Status is recorded. - The SBI speed is recorded. - The train speed is below the permitted speed. - The Normal Status is recorded. |
| Constraints | The speed and distance monitoring function is active within a ceiling speed monitoring section. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 | FS |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Operated System Version | 010 0000 | Version 2.0, introduced in SRS 3.3.0 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is moving (V_TRAIN ≤ VMRSP) | I | INT | - | N/A | N/A | - |
| 2 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The DMI aspect is coherent with Normal Status . Permitted speed is displayed. (As there is no change in the DMI, there is not JRU step with Normal Status recorded) | N/A | N/A | - |
| 3 | N/A | N/A | Train is moving (V_TRAIN > VMRSP) | I | INT | - | N/A | N/A | - |
| 4 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | When the V_TRAIN (estimated speed) > Vmrsp, then the supervision status changes to "Overspeed Status" (t2 in 3.13.10.3.3 table 5 of the SRS). So, the DMI aspect is coherent with Overspeed Status in a Ceiling Speed Monitoring section. The DMI shall display the SBI Speed (3.13.10.3.2). | N/A | N/A | - |
| 5 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=0; M_SDMSUPSTAT=2; V_SBI=<V1>) is recorded | O | JRU | M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 2 (Overspeed Status) V1 will be the corresponding value for V_SBI. | N/A | N/A | - |
| 6 | N/A | N/A | Train is moving (V_TRAIN ≤ VMRSP) | I | INT | - | N/A | N/A | - |
| 7 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | When the V_TRAIN (estimated speed) ≤ Vmrsp, then the supervision status changes to "Normal Status" (r1 in 3.13.10.3.3 table 6 of the SRS). | N/A | N/A | - |
| 8 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=0; M_SDMSUPSTAT=0) is recorded | O | JRU | M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal Status) | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 | FS |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Operated System Version | 010 0000 | Version 2.0, introduced in SRS 3.3.0 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Normal Status is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |