| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 3131020 (General requirements) Total: 6 Test case(s) |  |
| REF: | Subset-076-5-2-3131020 |
| ISSUE: | Version 3.2.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 2.9.0 31.03.2013 | All | First version/review | CEDEX |
| Version 2.9.1 18.11.2013 | All | Inclusion of uncovered requirements | CEDEX |
| Version 2.9.2 20.12.2013 | All | Version/review | CEDEX |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.2.0 31.03.2017 | All | Baseline 3 1st maintenance release version - Bug fixing | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | General requirements |  | 3131020 |
| Test Case of Feature | The use of the service brake command is not allowed by a National Value (only in Target speed monitoring) and the requirements specified for the revocation of service brake command are followed for revoking the emergency brake. |  | 1 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS L2/L3: FS, LS, OS |  |  |
| Target of Test | When the use of the service brake command is not allowed by a National Value (only in Target speed monitoring), whenever a service brake command is specified, the emergency brake command shall be triggered instead. The emergency brake command, which is triggered instead of the service brake command when an SBI supervision limit is exceeded, shall be revoked according to the requirements specified for the revocation of service brake command. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.13.10.2.3 |  |
|  |  | Subset-026-3.13.10.2.4 |  |
|  |  | Subset-026-3.13.2.3.7.1 |  |
|  |  | Subset-026-3.13.10.4.9 |  |
|  |  | Subset-026-3.13.10.4.9 b) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | The test case shall consider the train data as preconditions to be known before setting the appropriate values for the rest of the trackside info and the speed profile. The required observables (speed supervision limits) should be checked using a suitable tool that performs fully the “speed and distance monitoring" functionality (by default use ERA Braking curves tool, which can be downloaded from this website: http://www.era.europa.eu/Core-Activities/ERTMS/Pages/Braking-Curves-Simulation-Tool.aspx). |
| Constraints | The location accuracy of the LRBG has to be high enough, in order to ensure the train never leaves with its estimated front end the location of the Permitted Speed calculated from the SBD, valid for the estimated speed, before the train has left with its maximum safe front end the location of the same supervision limits calculated from the EBD, valid for the estimated speed. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Linking | STORED | - |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Mode Profile | STORED | Only in OS or LS mode |
| Unconditional Emergency Stop | NOT STORED | - |
| Conditional Emergency Stop | NOT STORED | - |
| Train Position | STORED | - |
| Train Data | STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Driver ID | STORED | - |
| Train Running Number | STORED | - |
| Operated System Version | 010 0000 | Version 2.0, introduced in SRS 3.3.0 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is moving (V_TRAIN=<below the SBI supervision limit (V_MRSP + dV_SBI (V_MRSP))>) | I | INT | - | N/A | N/A | - |
| 2 | N/A | N/A | The driver requests the speed and distance monitoring information to be displayed | I | DMI | Only in OS mode | N/A | N/A | - |
| 3 | N/A | N/A | One Balise Group message containing packet 5, packet 12 (Q_DANGERPOINT=0; Q_OVERLAP=0; V_LOA=0), packet 27, packet 21, packet 3 (Q_NVSBTSMPERM=0; Q_NVEMRRLS=0) is received | I | BTM | Only in L1. Q_NVSBTSMPERM=0: No permission to use service brake in target speed monitoring Q_NVEMRRLS=0: Revoke emergency brake command at standstill | N/A | N/A | - |
| 4 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | Only in L1 | N/A | N/A | - |
| 5 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 15 (Q_DANGERPOINT=0; Q_OVERLAP=0; V_LOA=0), packet 21, packet 27, packet 5, packet 3 (Q_NVSBTSMPERM=0; Q_NVEMRRLS=0) is received | I | RTM | Only in L2/L3. Q_NVSBTSMPERM=0: No permission to use service brake in target speed monitoring Q_NVEMRRLS=0: Revoke emergency brake command at standstill | N/A | N/A | - |
| 6 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | Only in L2/L3 | N/A | N/A | - |
| 7 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The train has passed with its maximum safe front end the location of the Indication supervision limit, valid for V_MRSP and calculated from the EBD, and it has not passed the location of the EBI supervision limit, valid for the estimated speed. The train accelerates until the estimated speed is above the SBI supervision limit (V_MRSP + dV_SBI (V_MRSP)) but below the EBI supervision limit (V_MRSP + dV_EBI (V_MRSP)). Then, the supervision status is "Intervention Status" (t12 in 3.13.10.4.10 table 9 of the SRS). So, the DMI aspect is coherent with Intervention Status in a Target speed monitoring section. | N/A | N/A | - |
| 8 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=2; M_SDMSUPSTAT=4) is recorded | O | JRU | M_SDMTYPE (Speed and distance monitoring type) = 2 (Target speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 4 (Intervention Status) | N/A | N/A | - |
| 9 | N/A | N/A | Emergency brake commanded | O | TIU | - | N/A | N/A | - |
| 10 | N/A | N/A | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | M_BRAKE_COMMAND_STATE = 1 (Command) | N/A | N/A | - |
| 11 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | When the train has left with its maximum safe front end the location of the Permitted Speed supervision limit, valid for the estimated speed and calculated from the EBD, then the supervision status changes to "Indication Status" (r3 in 3.13.10.4.12 table 11 of the SRS). So, the DMI aspect is coherent with Indication Status in a Target speed Monitoring section. | N/A | N/A | - |
| 12 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=2; M_SDMSUPSTAT=1) is recorded | O | JRU | M_SDMTYPE (Speed and distance monitoring type) = 2 (Target speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 1 (Indication Status) | N/A | N/A | - |
| 13 | N/A | N/A | Emergency brake not commanded | O | TIU | - | N/A | N/A | - |
| 14 | N/A | N/A | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=0) is recorded | O | JRU | M_BRAKE_COMMAND_STATE = 0 (Release) | N/A | N/A | - |

| Step 3: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 5 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LINK | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION | 1 | FINITE VALUE | - |
| Q_LINKREACTION | 2 | FINITE VALUE | - |
| Q_LOCACC | 6 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_LINK(k) | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | FINITE VALUE | - |
| Q_LOCACC(k) | 6 | FINITE VALUE | - |
| NID_PACKET | 8 | 12 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | - |
| V_LOA | 7 | 0 | - |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out". |
| N_ITER | 5 | FINITE VALUE | Set to zero if V_MAIN = 0 or if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | 0 | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | 0 | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 3 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_VALIDNV | 15 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | Identification of national areas to which the set applies. |
| N_ITER | 5 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | Identification of additional national area(s) to which the set applies. |
| V_NVSHUNT | 7 | FINITE VALUE | - |
| V_NVSTFF | 7 | FINITE VALUE | - |
| V_NVONSIGHT | 7 | FINITE VALUE | - |
| V_NVLIMSUPERV | 7 | FINITE VALUE | - |
| V_NVUNFIT | 7 | FINITE VALUE | - |
| V_NVREL | 7 | FINITE VALUE | - |
| D_NVROLL | 15 | FINITE VALUE | - |
| Q_NVSBTSMPERM | 1 | 0 | - |
| Q_NVEMRRLS | 1 | 0 | - |
| Q_NVGUIPERM | 1 | FINITE VALUE | - |
| Q_NVSBFBPERM | 1 | FINITE VALUE | - |
| Q_NVINHSMICPERM | 1 | FINITE VALUE | - |
| V_NVALLOWOVTRP | 7 | FINITE VALUE | - |
| V_NVSUPOVTRP | 7 | FINITE VALUE | - |
| D_NVOVTRP | 15 | FINITE VALUE | - |
| T_NVOVTRP | 8 | FINITE VALUE | - |
| D_NVPOTRP | 15 | FINITE VALUE | - |
| M_NVCONTACT | 2 | FINITE VALUE | - |
| T_NVCONTACT | 8 | FINITE VALUE | - |
| M_NVDERUN | 1 | FINITE VALUE | - |
| D_NVSTFF | 15 | FINITE VALUE | - |
| Q_NVDRIVER_ADHES | 1 | FINITE VALUE | - |
| A_NVMAXREDADH1 | 6 | FINITE VALUE | - |
| A_NVMAXREDADH2 | 6 | FINITE VALUE | - |
| A_NVMAXREDADH3 | 6 | FINITE VALUE | - |
| Q_NVLOCACC | 6 | FINITE VALUE | - |
| M_NVAVADH | 5 | FINITE VALUE | - |
| M_NVEBCL | 4 | FINITE VALUE | - |
| Q_NVKINT | 1 | FINITE VALUE | - |
| Q_NVKVINTSET | 2 | FINITE VALUE | Only if Q_NVKINT = 1, Q_NVKVINTSET and the following variables follow. |
| A_NVP12 | 6 | FINITE VALUE | Only if Q_NVKVINTSET = 1 |
| A_NVP23 | 6 | FINITE VALUE | Only if Q_NVKVINTSET = 1 |
| V_NVKVINT(n) | 7 | FINITE VALUE | 0 km/h |
| M_NVKVINT(n) | 7 | FINITE VALUE | Valid between V_NVKVINT and V_NVKVINT(1. If Q_NVKVINTSET = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12. |
| M_NVKVINT(n) | 7 | FINITE VALUE | Only if Q_NVKVINTSET = 1. Valid between V_NVKVINT and V_NVKVINT(1). Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23. |
| N_ITER | 5 | FINITE VALUE | - |
| V_NVKVINT | 7 | FINITE VALUE | - |
| M_NVKVINT | 7 | FINITE VALUE | Valid between V_NVKVINT(n) and V_NVKVINT(n+1). If Q_NVKVINTSET = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP1. |
| M_NVKVINT(n) | 7 | FINITE VALUE | Only if Q_NVKVINTSET = 1.Valid between V_NVKVINT(n) and V_NVKVINT(n+1) Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP2. |
| N_ITER | 5 | FINITE VALUE | - |
| Q_NVKVINTSET(k) | 2 | FINITE VALUE | - |
| A_NVP12(k) | 6 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1 |
| A_NVP23(k) | 6 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1 |
| V_NVKVINT(k) | 7 | FINITE VALUE | 0 |
| M_NVKVINT(k) | 7 | FINITE VALUE | Valid between V_NVKVINT(k) and V_NVKVINT(k,1). If Q_NVKVINTSET(k) = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12(k. |
| M_NVKVINT(k) | 7 | FINITE VALUE | Only if Q_NVKVINTSET(k) = . Valid between V_NVKVINT(k) and V_NVKVINT(k,1) Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23(k). |
| N_ITER(k) | 5 | FINITE VALUE | - |
| V_NVKVINT(k,m) | 7 | FINITE VALUE | - |
| M_NVKVINT(k,m) | 7 | FINITE VALUE | Valid between V_NVKVINT(k,m) and V_NVKVINT(k,m+1). If Q_NVKVINTSET(k) = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12(k). |
| M_NVKVINT(k,m) | 7 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1.Valid between V_NVKVINT(k,m) and V_NVKVINT(k,m+1). Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23(k). |
| L_NVKRINT | 5 | FINITE VALUE | 0 m |
| M_NVKRINT | 5 | FINITE VALUE | Valid between L_NVKRINT and L_NVKRINT(1). |
| N_ITER | 5 | FINITE VALUE | - |
| L_NVKRINT(l) | 5 | FINITE VALUE | - |
| M_NVKRINT(l) | 5 | FINITE VALUE | Valid between L_NVKRINT(l) and L_NVKRINT(l+1). |
| M_NVKTINT | 5 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | 255 |

| Step 5: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_LOA | 7 | 0 | - |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 5 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LINK | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION | 1 | FINITE VALUE | - |
| Q_LINKREACTION | 2 | FINITE VALUE | - |
| Q_LOCACC | 6 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_LINK(k) | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | FINITE VALUE | - |
| Q_LOCACC(k) | 6 | FINITE VALUE | - |
| NID_PACKET | 8 | 3 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_VALIDNV | 15 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | Identification of national areas to which the set applies. |
| N_ITER | 5 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | Identification of additional national area(s) to which the set applies. |
| V_NVSHUNT | 7 | FINITE VALUE | - |
| V_NVSTFF | 7 | FINITE VALUE | - |
| V_NVONSIGHT | 7 | FINITE VALUE | - |
| V_NVLIMSUPERV | 7 | FINITE VALUE | - |
| V_NVUNFIT | 7 | FINITE VALUE | - |
| V_NVREL | 7 | FINITE VALUE | - |
| D_NVROLL | 15 | FINITE VALUE | - |
| Q_NVSBTSMPERM | 1 | 0 | - |
| Q_NVEMRRLS | 1 | 0 | - |
| Q_NVGUIPERM | 1 | FINITE VALUE | - |
| Q_NVSBFBPERM | 1 | FINITE VALUE | - |
| Q_NVINHSMICPERM | 1 | FINITE VALUE | - |
| V_NVALLOWOVTRP | 7 | FINITE VALUE | - |
| V_NVSUPOVTRP | 7 | FINITE VALUE | - |
| D_NVOVTRP | 15 | FINITE VALUE | - |
| T_NVOVTRP | 8 | FINITE VALUE | - |
| D_NVPOTRP | 15 | FINITE VALUE | - |
| M_NVCONTACT | 2 | FINITE VALUE | - |
| T_NVCONTACT | 8 | FINITE VALUE | - |
| M_NVDERUN | 1 | FINITE VALUE | - |
| D_NVSTFF | 15 | FINITE VALUE | - |
| Q_NVDRIVER_ADHES | 1 | FINITE VALUE | - |
| A_NVMAXREDADH1 | 6 | FINITE VALUE | - |
| A_NVMAXREDADH2 | 6 | FINITE VALUE | - |
| A_NVMAXREDADH3 | 6 | FINITE VALUE | - |
| Q_NVLOCACC | 6 | FINITE VALUE | - |
| M_NVAVADH | 5 | FINITE VALUE | - |
| M_NVEBCL | 4 | FINITE VALUE | - |
| Q_NVKINT | 1 | FINITE VALUE | - |
| Q_NVKVINTSET | 2 | FINITE VALUE | Only if Q_NVKINT = 1, Q_NVKVINTSET and the following variables follow. |
| A_NVP12 | 6 | FINITE VALUE | Only if Q_NVKVINTSET = 1 |
| A_NVP23 | 6 | FINITE VALUE | Only if Q_NVKVINTSET = 1 |
| V_NVKVINT(n) | 7 | FINITE VALUE | 0 km/h |
| M_NVKVINT(n) | 7 | FINITE VALUE | Valid between V_NVKVINT and V_NVKVINT(1. If Q_NVKVINTSET = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12. |
| M_NVKVINT(n) | 7 | FINITE VALUE | Only if Q_NVKVINTSET = 1. Valid between V_NVKVINT and V_NVKVINT(1). Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23. |
| N_ITER | 5 | FINITE VALUE | - |
| V_NVKVINT | 7 | FINITE VALUE | - |
| M_NVKVINT | 7 | FINITE VALUE | Valid between V_NVKVINT(n) and V_NVKVINT(n+1). If Q_NVKVINTSET = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP1. |
| M_NVKVINT(n) | 7 | FINITE VALUE | Only if Q_NVKVINTSET = 1.Valid between V_NVKVINT(n) and V_NVKVINT(n+1) Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP2. |
| N_ITER | 5 | FINITE VALUE | - |
| Q_NVKVINTSET(k) | 2 | FINITE VALUE | - |
| A_NVP12(k) | 6 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1 |
| A_NVP23(k) | 6 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1 |
| V_NVKVINT(k) | 7 | FINITE VALUE | 0 |
| M_NVKVINT(k) | 7 | FINITE VALUE | Valid between V_NVKVINT(k) and V_NVKVINT(k,1). If Q_NVKVINTSET(k) = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12(k. |
| M_NVKVINT(k) | 7 | FINITE VALUE | Only if Q_NVKVINTSET(k) = . Valid between V_NVKVINT(k) and V_NVKVINT(k,1) Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23(k). |
| N_ITER(k) | 5 | FINITE VALUE | - |
| V_NVKVINT(k,m) | 7 | FINITE VALUE | - |
| M_NVKVINT(k,m) | 7 | FINITE VALUE | Valid between V_NVKVINT(k,m) and V_NVKVINT(k,m+1). If Q_NVKVINTSET(k) = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12(k). |
| M_NVKVINT(k,m) | 7 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1.Valid between V_NVKVINT(k,m) and V_NVKVINT(k,m+1). Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23(k). |
| L_NVKRINT | 5 | FINITE VALUE | 0 m |
| M_NVKRINT | 5 | FINITE VALUE | Valid between L_NVKRINT and L_NVKRINT(1). |
| N_ITER | 5 | FINITE VALUE | - |
| L_NVKRINT(l) | 5 | FINITE VALUE | - |
| M_NVKRINT(l) | 5 | FINITE VALUE | Valid between L_NVKRINT(l) and L_NVKRINT(l+1). |
| M_NVKTINT | 5 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| National Values | STORED | - |
| Linking | STORED | - |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Mode Profile | STORED | Only in OS or LS mode |
| Unconditional Emergency Stop | NOT STORED | - |
| Conditional Emergency Stop | NOT STORED | - |
| Train Position | STORED | - |
| Train Data | STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Driver ID | STORED | - |
| Train Running Number | STORED | - |
| Operated System Version | 010 0000 | Version 2.0, introduced in SRS 3.3.0 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | General requirements |  | 3131020 |
| Test Case of Feature | The use of the service brake command is not allowed by a National Value (only in Target speed monitoring), but the requirements specified for the revocation of service brake command are not followed. |  | 2 |
| Applicable Mode/Level Combinations | L1: SR L2/L3: SR |  |  |
| Target of Test | The use of the service brake command is not allowed by a National Value (only in Target speed monitoring). Nevertheless, when the emergency brake command has been also triggered due to an EBI supervision limit, the condition for revoking the emergency brake command due to EBI supervision limit shall prevail over the requirements specified for the revocation of service brake command. The National Values shall state the emergency brake command has to be revoked at standstill, so the on-board equipment will release the EB at standstill. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.13.10.2.3 |  |
|  |  | Subset-026-3.13.10.2.4 |  |
|  |  | Subset-026-3.13.2.3.7.2 |  |
|  |  | Subset-026-3.13.2.3.7.2 b) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | The test case shall consider the train data as preconditions to be known before setting the appropriate values for the rest of the trackside info and the speed profile. The required observables (speed supervision limits) should be checked using a suitable tool that performs fully the “speed and distance monitoring" functionality (by default use ERA Braking curves tool, which can be downloaded from this website: http://www.era.europa.eu/Core-Activities/ERTMS/Pages/Braking-Curves-Simulation-Tool.aspx). |
| Constraints | The value applicable for the SR mode speed limit and the distance to run in SR have not been previously entered by the driver. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 | SR |
| Radio communication session | ESTABLISHED | Only in L2/L3 |
| Axle load speed profile | NOT STORED | - |
| STM max speed | NOT STORED | - |
| STM system speed/distance | NOT STORED | - |
| List of Balises in SR Authority + SR mode speed limit and distance | STORED | Only in L2/L3 |
| Temporary Speed Restrictions | NOT STORED | - |
| Train Position | STORED | - |
| Train Data | STORED | - |
| Adhesion factor | NOT STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Driver ID | STORED | - |
| Radio Network ID | STORED | Only in L2/L3 |
| RBC ID/Phone Number | STORED | Only in L2/L3 |
| Train Running Number | STORED | - |
| Track Ahead Free Request | NOT STORED | - |
| Operated System Version | 010 0000 | Version 2.0, introduced in SRS 3.3.0 |
| Virtual Balise Covers | NOT STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is moving (V_TRAIN=<below the SBI supervision limit (V_MRSP + dV_SBI (V_MRSP))>) | I | INT | - | N/A | N/A | - |
| 2 | N/A | N/A | The driver requests the speed and distance monitoring information to be displayed | I | DMI | - | N/A | N/A | - |
| 3 | N/A | N/A | One Balise Group message containing packet 3 (V_NVSTFF=<V1>; D_NVSTFF=<D1>; Q_NVSBTSMPERM=0; Q_NVEMRRLS=0) is received | I | BTM | V1 and D1 will be adjusted in order to properly check the following steps. Q_NVSBTSMPERM=0: No permission to use service brake in target speed monitoring Q_NVEMRRLS=0: Revoke emergency brake command at standstill | N/A | N/A | - |
| 4 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | SA-DATA.Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted | O | RTM | Only in L2/L3. | N/A | N/A | - |
| 6 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | Only in L2/L3. | N/A | N/A | - |
| 7 | N/A | N/A | SA-DATA.Indication with Euroradio Message "SR Authorisation" (NID_MESSAGE=2; D_SR=<D2>) is received | I | RTM | Only in L2/L3. D2 will be adjusted in order to properly check the following steps D_SR=<D2> | N/A | N/A | - |
| 8 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | Only in L2/L3 | N/A | N/A | - |
| 9 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The train has passed with its maximum safe front end the location of the EBI supervision limit, valid for the estimated speed. The estimated speed is above zero and below the EBI supervision limit (V_MRSP + dV_EBI (V_MRSP)). Then, the supervision status is "Intervention Status" (t13 in 3.13.10.4.10 table 9 of the SRS). So, the DMI aspect is coherent with Intervention Status in a Target speed monitoring section. | N/A | N/A | - |
| 10 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=2; M_SDMSUPSTAT=4) is recorded | O | JRU | M_SDMTYPE (Speed and distance monitoring type) = 2 (Target monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 4 (Intervention Status) | N/A | N/A | - |
| 11 | N/A | N/A | Emergency brake commanded | O | TIU | - | N/A | N/A | - |
| 12 | N/A | N/A | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | M_BRAKE_COMMAND_STATE = 1 (Command) | N/A | N/A | - |
| 13 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | When the train has left with its maximum safe front end the location of the Permitted Speed supervision limit, valid for the estimated speed and calculated from the EBD, then the supervision status does not change to "Indication Status", since it is not allowed by National Value (r3 in 3.13.10.4.12 table 11 of the SRS). So, the DMI aspect is coherent with Intervention Status in a Target Speed Monitoring section. | N/A | N/A | - |
| 14 | N/A | N/A | Emergency brake not commanded | O | TIU | - | N/A | N/A | - |
| 15 | N/A | N/A | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=0) is recorded | O | JRU | M_BRAKE_COMMAND_STATE = 0 (Release) | N/A | N/A | - |

| Step 3: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 3 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_VALIDNV | 15 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | Identification of national areas to which the set applies. |
| N_ITER | 5 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | Identification of additional national area(s) to which the set applies. |
| V_NVSHUNT | 7 | FINITE VALUE | - |
| V_NVSTFF | 7 | FINITE VALUE | - |
| V_NVONSIGHT | 7 | FINITE VALUE | - |
| V_NVLIMSUPERV | 7 | FINITE VALUE | - |
| V_NVUNFIT | 7 | FINITE VALUE | - |
| V_NVREL | 7 | FINITE VALUE | - |
| D_NVROLL | 15 | FINITE VALUE | - |
| Q_NVSBTSMPERM | 1 | 0 | - |
| Q_NVEMRRLS | 1 | 0 | - |
| Q_NVGUIPERM | 1 | FINITE VALUE | - |
| Q_NVSBFBPERM | 1 | FINITE VALUE | - |
| Q_NVINHSMICPERM | 1 | FINITE VALUE | - |
| V_NVALLOWOVTRP | 7 | FINITE VALUE | - |
| V_NVSUPOVTRP | 7 | FINITE VALUE | - |
| D_NVOVTRP | 15 | FINITE VALUE | - |
| T_NVOVTRP | 8 | FINITE VALUE | - |
| D_NVPOTRP | 15 | FINITE VALUE | - |
| M_NVCONTACT | 2 | FINITE VALUE | - |
| T_NVCONTACT | 8 | FINITE VALUE | - |
| M_NVDERUN | 1 | FINITE VALUE | - |
| D_NVSTFF | 15 | FINITE VALUE | - |
| Q_NVDRIVER_ADHES | 1 | FINITE VALUE | - |
| A_NVMAXREDADH1 | 6 | FINITE VALUE | - |
| A_NVMAXREDADH2 | 6 | FINITE VALUE | - |
| A_NVMAXREDADH3 | 6 | FINITE VALUE | - |
| Q_NVLOCACC | 6 | FINITE VALUE | - |
| M_NVAVADH | 5 | FINITE VALUE | - |
| M_NVEBCL | 4 | FINITE VALUE | - |
| Q_NVKINT | 1 | FINITE VALUE | - |
| Q_NVKVINTSET | 2 | FINITE VALUE | Only if Q_NVKINT = 1, Q_NVKVINTSET and the following variables follow. |
| A_NVP12 | 6 | FINITE VALUE | Only if Q_NVKVINTSET = 1 |
| A_NVP23 | 6 | FINITE VALUE | Only if Q_NVKVINTSET = 1 |
| V_NVKVINT(n) | 7 | FINITE VALUE | 0 km/h |
| M_NVKVINT(n) | 7 | FINITE VALUE | Valid between V_NVKVINT and V_NVKVINT(1. If Q_NVKVINTSET = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12. |
| M_NVKVINT(n) | 7 | FINITE VALUE | Only if Q_NVKVINTSET = 1. Valid between V_NVKVINT and V_NVKVINT(1). Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23. |
| N_ITER | 5 | FINITE VALUE | - |
| V_NVKVINT | 7 | FINITE VALUE | - |
| M_NVKVINT | 7 | FINITE VALUE | Valid between V_NVKVINT(n) and V_NVKVINT(n+1). If Q_NVKVINTSET = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP1. |
| M_NVKVINT(n) | 7 | FINITE VALUE | Only if Q_NVKVINTSET = 1.Valid between V_NVKVINT(n) and V_NVKVINT(n+1) Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP2. |
| N_ITER | 5 | FINITE VALUE | - |
| Q_NVKVINTSET(k) | 2 | FINITE VALUE | - |
| A_NVP12(k) | 6 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1 |
| A_NVP23(k) | 6 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1 |
| V_NVKVINT(k) | 7 | FINITE VALUE | 0 |
| M_NVKVINT(k) | 7 | FINITE VALUE | Valid between V_NVKVINT(k) and V_NVKVINT(k,1). If Q_NVKVINTSET(k) = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12(k. |
| M_NVKVINT(k) | 7 | FINITE VALUE | Only if Q_NVKVINTSET(k) = . Valid between V_NVKVINT(k) and V_NVKVINT(k,1) Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23(k). |
| N_ITER(k) | 5 | FINITE VALUE | - |
| V_NVKVINT(k,m) | 7 | FINITE VALUE | - |
| M_NVKVINT(k,m) | 7 | FINITE VALUE | Valid between V_NVKVINT(k,m) and V_NVKVINT(k,m+1). If Q_NVKVINTSET(k) = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12(k). |
| M_NVKVINT(k,m) | 7 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1.Valid between V_NVKVINT(k,m) and V_NVKVINT(k,m+1). Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23(k). |
| L_NVKRINT | 5 | FINITE VALUE | 0 m |
| M_NVKRINT | 5 | FINITE VALUE | Valid between L_NVKRINT and L_NVKRINT(1). |
| N_ITER | 5 | FINITE VALUE | - |
| L_NVKRINT(l) | 5 | FINITE VALUE | - |
| M_NVKRINT(l) | 5 | FINITE VALUE | Valid between L_NVKRINT(l) and L_NVKRINT(l+1). |
| M_NVKTINT | 5 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | 255 |

| Step 5: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| Step 7: Radio Message 2 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 2 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_SR | 15 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 | SR |
| Radio communication session | ESTABLISHED | Only in L2/L3 |
| National Values | STORED | - |
| Axle load speed profile | NOT STORED | - |
| STM max speed | NOT STORED | - |
| STM system speed/distance | NOT STORED | - |
| List of Balises in SR Authority + SR mode speed limit and distance | STORED | Only in L2/L3 |
| Temporary Speed Restrictions | NOT STORED | - |
| Train Position | STORED | - |
| Train Data | STORED | - |
| Adhesion factor | NOT STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Driver ID | STORED | - |
| Radio Network ID | STORED | Only in L2/L3 |
| RBC ID/Phone Number | STORED | Only in L2/L3 |
| Train Running Number | STORED | - |
| Track Ahead Free Request | NOT STORED | - |
| Operated System Version | 010 0000 | Version 2.0, introduced in SRS 3.3.0 |
| Virtual Balise Covers | NOT STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | General requirements |  | 3131020 |
| Test Case of Feature | The use of the service brake command is not allowed by a National Value (only in Target speed monitoring), but the requirements specified for the revocation of service brake command are not followed. |  | 3 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS L2/L3: FS, LS, OS |  |  |
| Target of Test | The use of the service brake command is not allowed by a National Value (only in Target speed monitoring). Nevertheless, when the emergency brake command has been also triggered due to an EBI supervision limit, the condition for revoking the emergency brake command due to EBI supervision limit shall prevail over the requirements specified for the revocation of service brake command. The National Values shall state the emergency brake command has to be revoked at standstill, so the on-board equipment will release the EB at standstill. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.13.10.2.3 |  |
|  |  | Subset-026-3.13.10.2.4 |  |
|  |  | Subset-026-3.13.2.3.7.2 |  |
|  |  | Subset-026-3.13.2.3.7.2 b) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | The test case shall consider the train data as preconditions to be known before setting the appropriate values for the rest of the trackside info and the speed profile. The required observables (speed supervision limits) should be checked using a suitable tool that performs fully the “speed and distance monitoring" functionality (by default use ERA Braking curves tool, which can be downloaded from this website: http://www.era.europa.eu/Core-Activities/ERTMS/Pages/Braking-Curves-Simulation-Tool.aspx). |
| Constraints | The location accuracy of the LRBG has to be high enough, in order to ensure the train never leaves with its estimated front end the location of the Permitted Speed calculated from the SBD, valid for the estimated speed, before the train has left with its maximum safe front end the location of the same supervision limits calculated from the EBD, valid for the estimated speed. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Linking | STORED | - |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Mode Profile | STORED | Only in OS or LS mode |
| Unconditional Emergency Stop | NOT STORED | - |
| Conditional Emergency Stop | NOT STORED | - |
| Train Position | STORED | - |
| Train Data | STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Driver ID | STORED | - |
| Train Running Number | STORED | - |
| Operated System Version | 010 0000 | Version 2.0, introduced in SRS 3.3.0 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is moving (V_TRAIN=<below the SBI supervision limit (V_MRSP + dV_SBI (V_MRSP))>) | I | INT | - | N/A | N/A | - |
| 2 | N/A | N/A | The driver requests the speed and distance monitoring information to be displayed | I | DMI | Only in OS mode | N/A | N/A | - |
| 3 | N/A | N/A | One Balise Group message containing packet 5, packet 12 (Q_DANGERPOINT=0; Q_OVERLAP=0; V_LOA=0), packet 27, packet 21, packet 3 (Q_NVSBTSMPERM=0; Q_NVEMRRLS=0) is received | I | BTM | Only in L1. Q_NVSBTSMPERM=0: No permission to use service brake in target speed monitoring Q_NVEMRRLS=0: Revoke emergency brake command at standstill | N/A | N/A | - |
| 4 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | Only in L1 | N/A | N/A | - |
| 5 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 15 (Q_DANGERPOINT=0; Q_OVERLAP=0; V_LOA=0), packet 21, packet 27, packet 5, packet 3 (Q_NVSBTSMPERM=0; Q_NVEMRRLS=0) is received | I | RTM | Only in L2/L3. Q_NVSBTSMPERM=0: No permission to use service brake in target speed monitoring Q_NVEMRRLS=0: Revoke emergency brake command at standstill | N/A | N/A | - |
| 6 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | Only in L2/L3 | N/A | N/A | - |
| 7 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The train has passed with its maximum safe front end the location of the EBI supervision limit, valid for the estimated speed. The estimated speed is above zero and below the EBI supervision limit (V_MRSP + dV_EBI (V_MRSP)). Then, the supervision status is "Intervention Status" (t13 in 3.13.10.4.10 table 9 of the SRS). So, the DMI aspect is coherent with Intervention Status in a Target speed monitoring section. | N/A | N/A | - |
| 8 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=2; M_SDMSUPSTAT=4) is recorded | O | JRU | M_SDMTYPE (Speed and distance monitoring type) = 2 (Target monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 4 (Intervention Status) | N/A | N/A | - |
| 9 | N/A | N/A | Emergency brake commanded | O | TIU | - | N/A | N/A | - |
| 10 | N/A | N/A | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | M_BRAKE_COMMAND_STATE = 1 (Command) | N/A | N/A | - |
| 11 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | When the train has left with its maximum safe front end the location of the Permitted Speed supervision limit, valid for the estimated speed and calculated from the EBD, then the supervision status does not change to "Indication Status", since it is not allowed by National Value (r3 in 3.13.10.4.12 table 11 of the SRS). So, the DMI aspect is coherent with Intervention Status in a Target Speed Monitoring section. | N/A | N/A | - |
| 12 | N/A | N/A | Train is at standstill (V_TRAIN=0) | I | INT | - | N/A | N/A | - |
| 13 | N/A | N/A | Emergency brake not commanded | O | TIU | - | N/A | N/A | - |
| 14 | N/A | N/A | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=0) is recorded | O | JRU | M_BRAKE_COMMAND_STATE = 0 (Release) | N/A | N/A | - |

| Step 3: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 5 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LINK | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION | 1 | FINITE VALUE | - |
| Q_LINKREACTION | 2 | FINITE VALUE | - |
| Q_LOCACC | 6 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_LINK(k) | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | FINITE VALUE | - |
| Q_LOCACC(k) | 6 | FINITE VALUE | - |
| NID_PACKET | 8 | 12 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | - |
| V_LOA | 7 | 0 | - |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out". |
| N_ITER | 5 | FINITE VALUE | Set to zero if V_MAIN = 0 or if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | 0 | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | 0 | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 3 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_VALIDNV | 15 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | Identification of national areas to which the set applies. |
| N_ITER | 5 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | Identification of additional national area(s) to which the set applies. |
| V_NVSHUNT | 7 | FINITE VALUE | - |
| V_NVSTFF | 7 | FINITE VALUE | - |
| V_NVONSIGHT | 7 | FINITE VALUE | - |
| V_NVLIMSUPERV | 7 | FINITE VALUE | - |
| V_NVUNFIT | 7 | FINITE VALUE | - |
| V_NVREL | 7 | FINITE VALUE | - |
| D_NVROLL | 15 | FINITE VALUE | - |
| Q_NVSBTSMPERM | 1 | 0 | - |
| Q_NVEMRRLS | 1 | 0 | - |
| Q_NVGUIPERM | 1 | FINITE VALUE | - |
| Q_NVSBFBPERM | 1 | FINITE VALUE | - |
| Q_NVINHSMICPERM | 1 | FINITE VALUE | - |
| V_NVALLOWOVTRP | 7 | FINITE VALUE | - |
| V_NVSUPOVTRP | 7 | FINITE VALUE | - |
| D_NVOVTRP | 15 | FINITE VALUE | - |
| T_NVOVTRP | 8 | FINITE VALUE | - |
| D_NVPOTRP | 15 | FINITE VALUE | - |
| M_NVCONTACT | 2 | FINITE VALUE | - |
| T_NVCONTACT | 8 | FINITE VALUE | - |
| M_NVDERUN | 1 | FINITE VALUE | - |
| D_NVSTFF | 15 | FINITE VALUE | - |
| Q_NVDRIVER_ADHES | 1 | FINITE VALUE | - |
| A_NVMAXREDADH1 | 6 | FINITE VALUE | - |
| A_NVMAXREDADH2 | 6 | FINITE VALUE | - |
| A_NVMAXREDADH3 | 6 | FINITE VALUE | - |
| Q_NVLOCACC | 6 | FINITE VALUE | - |
| M_NVAVADH | 5 | FINITE VALUE | - |
| M_NVEBCL | 4 | FINITE VALUE | - |
| Q_NVKINT | 1 | FINITE VALUE | - |
| Q_NVKVINTSET | 2 | FINITE VALUE | Only if Q_NVKINT = 1, Q_NVKVINTSET and the following variables follow. |
| A_NVP12 | 6 | FINITE VALUE | Only if Q_NVKVINTSET = 1 |
| A_NVP23 | 6 | FINITE VALUE | Only if Q_NVKVINTSET = 1 |
| V_NVKVINT(n) | 7 | FINITE VALUE | 0 km/h |
| M_NVKVINT(n) | 7 | FINITE VALUE | Valid between V_NVKVINT and V_NVKVINT(1. If Q_NVKVINTSET = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12. |
| M_NVKVINT(n) | 7 | FINITE VALUE | Only if Q_NVKVINTSET = 1. Valid between V_NVKVINT and V_NVKVINT(1). Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23. |
| N_ITER | 5 | FINITE VALUE | - |
| V_NVKVINT | 7 | FINITE VALUE | - |
| M_NVKVINT | 7 | FINITE VALUE | Valid between V_NVKVINT(n) and V_NVKVINT(n+1). If Q_NVKVINTSET = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP1. |
| M_NVKVINT(n) | 7 | FINITE VALUE | Only if Q_NVKVINTSET = 1.Valid between V_NVKVINT(n) and V_NVKVINT(n+1) Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP2. |
| N_ITER | 5 | FINITE VALUE | - |
| Q_NVKVINTSET(k) | 2 | FINITE VALUE | - |
| A_NVP12(k) | 6 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1 |
| A_NVP23(k) | 6 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1 |
| V_NVKVINT(k) | 7 | FINITE VALUE | 0 |
| M_NVKVINT(k) | 7 | FINITE VALUE | Valid between V_NVKVINT(k) and V_NVKVINT(k,1). If Q_NVKVINTSET(k) = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12(k. |
| M_NVKVINT(k) | 7 | FINITE VALUE | Only if Q_NVKVINTSET(k) = . Valid between V_NVKVINT(k) and V_NVKVINT(k,1) Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23(k). |
| N_ITER(k) | 5 | FINITE VALUE | - |
| V_NVKVINT(k,m) | 7 | FINITE VALUE | - |
| M_NVKVINT(k,m) | 7 | FINITE VALUE | Valid between V_NVKVINT(k,m) and V_NVKVINT(k,m+1). If Q_NVKVINTSET(k) = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12(k). |
| M_NVKVINT(k,m) | 7 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1.Valid between V_NVKVINT(k,m) and V_NVKVINT(k,m+1). Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23(k). |
| L_NVKRINT | 5 | FINITE VALUE | 0 m |
| M_NVKRINT | 5 | FINITE VALUE | Valid between L_NVKRINT and L_NVKRINT(1). |
| N_ITER | 5 | FINITE VALUE | - |
| L_NVKRINT(l) | 5 | FINITE VALUE | - |
| M_NVKRINT(l) | 5 | FINITE VALUE | Valid between L_NVKRINT(l) and L_NVKRINT(l+1). |
| M_NVKTINT | 5 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | 255 |

| Step 5: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_LOA | 7 | 0 | - |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | 0 | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | 0 | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 5 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LINK | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION | 1 | FINITE VALUE | - |
| Q_LINKREACTION | 2 | FINITE VALUE | - |
| Q_LOCACC | 6 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_LINK(k) | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | FINITE VALUE | - |
| Q_LOCACC(k) | 6 | FINITE VALUE | - |
| NID_PACKET | 8 | 3 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_VALIDNV | 15 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | Identification of national areas to which the set applies. |
| N_ITER | 5 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | Identification of additional national area(s) to which the set applies. |
| V_NVSHUNT | 7 | FINITE VALUE | - |
| V_NVSTFF | 7 | FINITE VALUE | - |
| V_NVONSIGHT | 7 | FINITE VALUE | - |
| V_NVLIMSUPERV | 7 | FINITE VALUE | - |
| V_NVUNFIT | 7 | FINITE VALUE | - |
| V_NVREL | 7 | FINITE VALUE | - |
| D_NVROLL | 15 | FINITE VALUE | - |
| Q_NVSBTSMPERM | 1 | 0 | - |
| Q_NVEMRRLS | 1 | 0 | - |
| Q_NVGUIPERM | 1 | FINITE VALUE | - |
| Q_NVSBFBPERM | 1 | FINITE VALUE | - |
| Q_NVINHSMICPERM | 1 | FINITE VALUE | - |
| V_NVALLOWOVTRP | 7 | FINITE VALUE | - |
| V_NVSUPOVTRP | 7 | FINITE VALUE | - |
| D_NVOVTRP | 15 | FINITE VALUE | - |
| T_NVOVTRP | 8 | FINITE VALUE | - |
| D_NVPOTRP | 15 | FINITE VALUE | - |
| M_NVCONTACT | 2 | FINITE VALUE | - |
| T_NVCONTACT | 8 | FINITE VALUE | - |
| M_NVDERUN | 1 | FINITE VALUE | - |
| D_NVSTFF | 15 | FINITE VALUE | - |
| Q_NVDRIVER_ADHES | 1 | FINITE VALUE | - |
| A_NVMAXREDADH1 | 6 | FINITE VALUE | - |
| A_NVMAXREDADH2 | 6 | FINITE VALUE | - |
| A_NVMAXREDADH3 | 6 | FINITE VALUE | - |
| Q_NVLOCACC | 6 | FINITE VALUE | - |
| M_NVAVADH | 5 | FINITE VALUE | - |
| M_NVEBCL | 4 | FINITE VALUE | - |
| Q_NVKINT | 1 | FINITE VALUE | - |
| Q_NVKVINTSET | 2 | FINITE VALUE | Only if Q_NVKINT = 1, Q_NVKVINTSET and the following variables follow. |
| A_NVP12 | 6 | FINITE VALUE | Only if Q_NVKVINTSET = 1 |
| A_NVP23 | 6 | FINITE VALUE | Only if Q_NVKVINTSET = 1 |
| V_NVKVINT(n) | 7 | FINITE VALUE | 0 km/h |
| M_NVKVINT(n) | 7 | FINITE VALUE | Valid between V_NVKVINT and V_NVKVINT(1. If Q_NVKVINTSET = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12. |
| M_NVKVINT(n) | 7 | FINITE VALUE | Only if Q_NVKVINTSET = 1. Valid between V_NVKVINT and V_NVKVINT(1). Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23. |
| N_ITER | 5 | FINITE VALUE | - |
| V_NVKVINT | 7 | FINITE VALUE | - |
| M_NVKVINT | 7 | FINITE VALUE | Valid between V_NVKVINT(n) and V_NVKVINT(n+1). If Q_NVKVINTSET = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP1. |
| M_NVKVINT(n) | 7 | FINITE VALUE | Only if Q_NVKVINTSET = 1.Valid between V_NVKVINT(n) and V_NVKVINT(n+1) Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP2. |
| N_ITER | 5 | FINITE VALUE | - |
| Q_NVKVINTSET(k) | 2 | FINITE VALUE | - |
| A_NVP12(k) | 6 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1 |
| A_NVP23(k) | 6 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1 |
| V_NVKVINT(k) | 7 | FINITE VALUE | 0 |
| M_NVKVINT(k) | 7 | FINITE VALUE | Valid between V_NVKVINT(k) and V_NVKVINT(k,1). If Q_NVKVINTSET(k) = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12(k. |
| M_NVKVINT(k) | 7 | FINITE VALUE | Only if Q_NVKVINTSET(k) = . Valid between V_NVKVINT(k) and V_NVKVINT(k,1) Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23(k). |
| N_ITER(k) | 5 | FINITE VALUE | - |
| V_NVKVINT(k,m) | 7 | FINITE VALUE | - |
| M_NVKVINT(k,m) | 7 | FINITE VALUE | Valid between V_NVKVINT(k,m) and V_NVKVINT(k,m+1). If Q_NVKVINTSET(k) = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12(k). |
| M_NVKVINT(k,m) | 7 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1.Valid between V_NVKVINT(k,m) and V_NVKVINT(k,m+1). Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23(k). |
| L_NVKRINT | 5 | FINITE VALUE | 0 m |
| M_NVKRINT | 5 | FINITE VALUE | Valid between L_NVKRINT and L_NVKRINT(1). |
| N_ITER | 5 | FINITE VALUE | - |
| L_NVKRINT(l) | 5 | FINITE VALUE | - |
| M_NVKRINT(l) | 5 | FINITE VALUE | Valid between L_NVKRINT(l) and L_NVKRINT(l+1). |
| M_NVKTINT | 5 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| National Values | STORED | - |
| Linking | STORED | - |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Mode Profile | STORED | Only in OS or LS mode |
| Unconditional Emergency Stop | NOT STORED | - |
| Conditional Emergency Stop | NOT STORED | - |
| Train Position | STORED | - |
| Train Data | STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Driver ID | STORED | - |
| Train Running Number | STORED | - |
| Operated System Version | 010 0000 | Version 2.0, introduced in SRS 3.3.0 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | General requirements |  | 3131020 |
| Test Case of Feature | The use of the service brake command is not allowed by a National Value (only in Target speed monitoring) and the requirements specified for the revocation of service brake command are followed for revoking the emergency brake. |  | 4 |
| Applicable Mode/Level Combinations | L1: SR L2/L3: SR |  |  |
| Target of Test | When the use of the service brake command is not allowed by a National Value (only in Target speed monitoring), whenever a service brake command is specified, the emergency brake command shall be triggered instead. The emergency brake command, which is triggered instead of the service brake command when an SBI supervision limit is exceeded, shall be revoked according to the requirements specified for the revocation of service brake command. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.13.10.2.3 |  |
|  |  | Subset-026-3.13.10.2.4 |  |
|  |  | Subset-026-3.13.2.3.7.1 |  |
|  |  | Subset-026-3.13.10.4.9 |  |
|  |  | Subset-026-3.13.10.4.9 b) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | The test case shall consider the train data as preconditions to be known before setting the appropriate values for the rest of the trackside info and the speed profile. The required observables (speed supervision limits) should be checked using a suitable tool that performs fully the “speed and distance monitoring" functionality (by default use ERA Braking curves tool, which can be downloaded from this website: http://www.era.europa.eu/Core-Activities/ERTMS/Pages/Braking-Curves-Simulation-Tool.aspx). |
| Constraints | The value applicable for the SR mode speed limit and the distance to run in SR have not been previously entered by the driver. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 | SR |
| Radio communication session | ESTABLISHED | Only in L2/L3 |
| Axle load speed profile | NOT STORED | - |
| STM max speed | NOT STORED | - |
| STM system speed/distance | NOT STORED | - |
| List of Balises in SR Authority + SR mode speed limit and distance | STORED | Only in L2/L3 |
| Temporary Speed Restrictions | NOT STORED | - |
| Train Position | STORED | - |
| Train Data | STORED | - |
| Adhesion factor | NOT STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Driver ID | STORED | - |
| Radio Network ID | STORED | Only in L2/L3 |
| RBC ID/Phone Number | STORED | Only in L2/L3 |
| Train Running Number | STORED | - |
| Track Ahead Free Request | NOT STORED | - |
| Operated System Version | 010 0000 | Version 2.0, introduced in SRS 3.3.0 |
| Virtual Balise Covers | NOT STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is moving (V_TRAIN=<below the SBI supervision limit (V_MRSP + dV_SBI (V_MRSP))>) | I | INT | - | N/A | N/A | - |
| 2 | N/A | N/A | The driver requests the speed and distance monitoring information to be displayed | I | DMI | - | N/A | N/A | - |
| 3 | N/A | N/A | One Balise Group message containing packet 3 (V_NVSTFF=<V1>; D_NVSTFF=<D1>; Q_NVSBTSMPERM=0; Q_NVEMRRLS=0) is received | I | BTM | V1 and D1 will be adjusted in order to properly check the following steps. Q_NVSBTSMPERM=0: No permission to use service brake in target speed monitoring Q_NVEMRRLS=0: Revoke emergency brake command at standstill | N/A | N/A | - |
| 4 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | SA-DATA.Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted | O | RTM | Only in L2/L3. | N/A | N/A | - |
| 6 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | Only in L2/L3. | N/A | N/A | - |
| 7 | N/A | N/A | SA-DATA.Indication with Euroradio Message "SR Authorisation" (NID_MESSAGE=2; D_SR=<D2>) is received | I | RTM | Only in L2/L3. D2 will be adjusted in order to properly check the following steps D_SR=<D2> | N/A | N/A | - |
| 8 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | Only in L2/L3 | N/A | N/A | - |
| 9 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The train has passed with its maximum safe front end the location of the Indication supervision limit, valid for V_MRSP and calculated from the EBD, and it has not passed the location of the EBI supervision limit, valid for the estimated speed. The train accelerates until the estimated speed is above the SBI supervision limit (V_MRSP + dV_SBI (V_MRSP)) but below the EBI supervision limit (V_MRSP + dV_EBI (V_MRSP)). Then, the supervision status is "Intervention Status" (t12 in 3.13.10.4.10 table 9 of the SRS). So, the DMI aspect is coherent with Intervention Status in a Target speed monitoring section. | N/A | N/A | - |
| 10 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=2; M_SDMSUPSTAT=4) is recorded | O | JRU | M_SDMTYPE (Speed and distance monitoring type) = 2 (Target speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 4 (Intervention Status) | N/A | N/A | - |
| 11 | N/A | N/A | Emergency brake commanded | O | TIU | - | N/A | N/A | - |
| 12 | N/A | N/A | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | M_BRAKE_COMMAND_STATE = 1 (Command) | N/A | N/A | - |
| 13 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | When the train has left with its maximum safe front end the location of the Permitted Speed supervision limit, valid for the estimated speed and calculated from the EBD, then the supervision status changes to "Indication Status" (r3 in 3.13.10.4.12 table 11 of the SRS). So, the DMI aspect is coherent with Indication Status in a Target speed Monitoring section. | N/A | N/A | - |
| 14 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=2; M_SDMSUPSTAT=1) is recorded | O | JRU | M_SDMTYPE (Speed and distance monitoring type) = 2 (Target speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 1 (Indication Status) | N/A | N/A | - |
| 15 | N/A | N/A | Emergency brake not commanded | O | TIU | - | N/A | N/A | - |
| 16 | N/A | N/A | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=0) is recorded | O | JRU | M_BRAKE_COMMAND_STATE = 0 (Release) | N/A | N/A | - |

| Step 3: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 3 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_VALIDNV | 15 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | Identification of national areas to which the set applies. |
| N_ITER | 5 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | Identification of additional national area(s) to which the set applies. |
| V_NVSHUNT | 7 | FINITE VALUE | - |
| V_NVSTFF | 7 | FINITE VALUE | - |
| V_NVONSIGHT | 7 | FINITE VALUE | - |
| V_NVLIMSUPERV | 7 | FINITE VALUE | - |
| V_NVUNFIT | 7 | FINITE VALUE | - |
| V_NVREL | 7 | FINITE VALUE | - |
| D_NVROLL | 15 | FINITE VALUE | - |
| Q_NVSBTSMPERM | 1 | 0 | - |
| Q_NVEMRRLS | 1 | 0 | - |
| Q_NVGUIPERM | 1 | FINITE VALUE | - |
| Q_NVSBFBPERM | 1 | FINITE VALUE | - |
| Q_NVINHSMICPERM | 1 | FINITE VALUE | - |
| V_NVALLOWOVTRP | 7 | FINITE VALUE | - |
| V_NVSUPOVTRP | 7 | FINITE VALUE | - |
| D_NVOVTRP | 15 | FINITE VALUE | - |
| T_NVOVTRP | 8 | FINITE VALUE | - |
| D_NVPOTRP | 15 | FINITE VALUE | - |
| M_NVCONTACT | 2 | FINITE VALUE | - |
| T_NVCONTACT | 8 | FINITE VALUE | - |
| M_NVDERUN | 1 | FINITE VALUE | - |
| D_NVSTFF | 15 | FINITE VALUE | - |
| Q_NVDRIVER_ADHES | 1 | FINITE VALUE | - |
| A_NVMAXREDADH1 | 6 | FINITE VALUE | - |
| A_NVMAXREDADH2 | 6 | FINITE VALUE | - |
| A_NVMAXREDADH3 | 6 | FINITE VALUE | - |
| Q_NVLOCACC | 6 | FINITE VALUE | - |
| M_NVAVADH | 5 | FINITE VALUE | - |
| M_NVEBCL | 4 | FINITE VALUE | - |
| Q_NVKINT | 1 | FINITE VALUE | - |
| Q_NVKVINTSET | 2 | FINITE VALUE | Only if Q_NVKINT = 1, Q_NVKVINTSET and the following variables follow. |
| A_NVP12 | 6 | FINITE VALUE | Only if Q_NVKVINTSET = 1 |
| A_NVP23 | 6 | FINITE VALUE | Only if Q_NVKVINTSET = 1 |
| V_NVKVINT(n) | 7 | FINITE VALUE | 0 km/h |
| M_NVKVINT(n) | 7 | FINITE VALUE | Valid between V_NVKVINT and V_NVKVINT(1. If Q_NVKVINTSET = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12. |
| M_NVKVINT(n) | 7 | FINITE VALUE | Only if Q_NVKVINTSET = 1. Valid between V_NVKVINT and V_NVKVINT(1). Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23. |
| N_ITER | 5 | FINITE VALUE | - |
| V_NVKVINT | 7 | FINITE VALUE | - |
| M_NVKVINT | 7 | FINITE VALUE | Valid between V_NVKVINT(n) and V_NVKVINT(n+1). If Q_NVKVINTSET = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP1. |
| M_NVKVINT(n) | 7 | FINITE VALUE | Only if Q_NVKVINTSET = 1.Valid between V_NVKVINT(n) and V_NVKVINT(n+1) Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP2. |
| N_ITER | 5 | FINITE VALUE | - |
| Q_NVKVINTSET(k) | 2 | FINITE VALUE | - |
| A_NVP12(k) | 6 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1 |
| A_NVP23(k) | 6 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1 |
| V_NVKVINT(k) | 7 | FINITE VALUE | 0 |
| M_NVKVINT(k) | 7 | FINITE VALUE | Valid between V_NVKVINT(k) and V_NVKVINT(k,1). If Q_NVKVINTSET(k) = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12(k. |
| M_NVKVINT(k) | 7 | FINITE VALUE | Only if Q_NVKVINTSET(k) = . Valid between V_NVKVINT(k) and V_NVKVINT(k,1) Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23(k). |
| N_ITER(k) | 5 | FINITE VALUE | - |
| V_NVKVINT(k,m) | 7 | FINITE VALUE | - |
| M_NVKVINT(k,m) | 7 | FINITE VALUE | Valid between V_NVKVINT(k,m) and V_NVKVINT(k,m+1). If Q_NVKVINTSET(k) = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12(k). |
| M_NVKVINT(k,m) | 7 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1.Valid between V_NVKVINT(k,m) and V_NVKVINT(k,m+1). Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23(k). |
| L_NVKRINT | 5 | FINITE VALUE | 0 m |
| M_NVKRINT | 5 | FINITE VALUE | Valid between L_NVKRINT and L_NVKRINT(1). |
| N_ITER | 5 | FINITE VALUE | - |
| L_NVKRINT(l) | 5 | FINITE VALUE | - |
| M_NVKRINT(l) | 5 | FINITE VALUE | Valid between L_NVKRINT(l) and L_NVKRINT(l+1). |
| M_NVKTINT | 5 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | 255 |

| Step 5: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| Step 7: Radio Message 2 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 2 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_SR | 15 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 | SR |
| Radio communication session | ESTABLISHED | Only in L2/L3 |
| National Values | STORED | - |
| Axle load speed profile | NOT STORED | - |
| STM max speed | NOT STORED | - |
| STM system speed/distance | NOT STORED | - |
| List of Balises in SR Authority + SR mode speed limit and distance | STORED | Only in L2/L3 |
| Temporary Speed Restrictions | NOT STORED | - |
| Train Position | STORED | - |
| Train Data | STORED | - |
| Adhesion factor | NOT STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Driver ID | STORED | - |
| Radio Network ID | STORED | Only in L2/L3 |
| RBC ID/Phone Number | STORED | Only in L2/L3 |
| Train Running Number | STORED | - |
| Track Ahead Free Request | NOT STORED | - |
| Operated System Version | 010 0000 | Version 2.0, introduced in SRS 3.3.0 |
| Virtual Balise Covers | NOT STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 5

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | General requirements |  | 3131020 |
| Test Case of Feature | There is no on-board interface with the service brake, so the requirements specified for the revocation of service brake command are followed for revoking the emergency brake. |  | 5 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS L2/L3: FS, LS, OS |  |  |
| Target of Test | When there is no on-board interface with the service brake, whenever a service brake command is specified, the emergency brake command shall be triggered instead. The emergency brake command, which is triggered instead of the service brake command when an SBI supervision limit is exceeded, shall be revoked according to the requirements specified for the revocation of service brake command. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.13.10.2.3 |  |
|  |  | Subset-026-3.13.10.2.4 |  |
|  |  | Subset-026-3.13.10.4.9 |  |
|  |  | Subset-026-3.13.10.4.9 a) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | The test case shall consider the train data as preconditions to be known before setting the appropriate values for the rest of the trackside info and the speed profile. The required observables (speed supervision limits) should be checked using a suitable tool that performs fully the “speed and distance monitoring" functionality (by default use ERA Braking curves tool, which can be downloaded from this website: http://www.era.europa.eu/Core-Activities/ERTMS/Pages/Braking-Curves-Simulation-Tool.aspx). |
| Constraints | The location accuracy of the LRBG has to be high enough, in order to ensure the train never leaves with its estimated front end the location of the Permitted Speed calculated from the SBD, valid for the estimated speed, before the train has left with its maximum safe front end the location of the same supervision limits calculated from the EBD, valid for the estimated speed. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | Only in L2/L3 |
| National Values | STORED | Q_NVSBTSMPERM=1: (Yes) permission to use service brake in target speed monitoring |
| Linking | STORED | - |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Mode Profile | STORED | Only in OS or LS mode |
| Unconditional Emergency Stop | NOT STORED | - |
| Conditional Emergency Stop | NOT STORED | - |
| Train Position | STORED | - |
| Train Data | STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Driver ID | STORED | - |
| Train Running Number | STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is moving (V_TRAIN=<below the SBI supervision limit (V_MRSP + dV_SBI (V_MRSP))>) | I | INT | - | N/A | N/A | - |
| 2 | N/A | N/A | The driver requests the speed and distance monitoring information to be displayed | I | DMI | Only in OS mode | N/A | N/A | - |
| 3 | N/A | N/A | One Balise Group message containing packet 5, packet 12 (Q_DANGERPOINT=0; Q_OVERLAP=0; V_LOA=0), packet 27, packet 21 is received | I | BTM | Only in L1 | N/A | N/A | - |
| 4 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | Only in L1 | N/A | N/A | - |
| 5 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 15 (Q_DANGERPOINT=0; Q_OVERLAP=0; V_LOA=0), packet 21, packet 27, packet 5 is received | I | RTM | Only in L2/L3. | N/A | N/A | - |
| 6 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | Only in L2/L3 | N/A | N/A | - |
| 7 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The train has passed with its maximum safe front end the location of the Indication supervision limit, valid for V_MRSP and calculated from the EBD, and it has not passed the location of the EBI supervision limit, valid for the estimated speed. The train accelerates until the estimated speed is above the SBI supervision limit (V_MRSP + dV_SBI (V_MRSP)) but below the EBI supervision limit (V_MRSP + dV_EBI (V_MRSP)). Then, the supervision status is "Intervention Status" (t12 in 3.13.10.4.10 table 9 of the SRS). So, the DMI aspect is coherent with Intervention Status in a Target speed monitoring section. | N/A | N/A | - |
| 8 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=2; M_SDMSUPSTAT=4) is recorded | O | JRU | M_SDMTYPE (Speed and distance monitoring type) = 2 (Target speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 4 (Intervention Status) | N/A | N/A | - |
| 9 | N/A | N/A | Emergency brake commanded | O | TIU | - | N/A | N/A | - |
| 10 | N/A | N/A | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | M_BRAKE_COMMAND_STATE = 1 (Command) | N/A | N/A | - |
| 11 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | When the train has left with its maximum safe front end the location of the Permitted Speed supervision limit, valid for the estimated speed and calculated from the EBD, then the supervision status changes to "Indication Status" (r3 in 3.13.10.4.12 table 11 of the SRS). So, the DMI aspect is coherent with Indication Status in a Target speed Monitoring section. | N/A | N/A | - |
| 12 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=2; M_SDMSUPSTAT=1) is recorded | O | JRU | M_SDMTYPE (Speed and distance monitoring type) = 2 (Target speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 1 (Indication Status) | N/A | N/A | - |
| 13 | N/A | N/A | Emergency brake not commanded | O | TIU | - | N/A | N/A | - |
| 14 | N/A | N/A | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=0) is recorded | O | JRU | M_BRAKE_COMMAND_STATE = 0 (Release) | N/A | N/A | - |

| Step 3: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 5 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LINK | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION | 1 | FINITE VALUE | - |
| Q_LINKREACTION | 2 | FINITE VALUE | - |
| Q_LOCACC | 6 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_LINK(k) | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | FINITE VALUE | - |
| Q_LOCACC(k) | 6 | FINITE VALUE | - |
| NID_PACKET | 8 | 12 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| V_LOA | 10 | FINITE VALUE | Can be set to "no time-out". |
| N_ITER | 5 | FINITE VALUE | Set to zero if V_MAIN = 0 or if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | 255 |

| Step 5: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| V_LOA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 5 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LINK | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION | 1 | FINITE VALUE | - |
| Q_LINKREACTION | 2 | FINITE VALUE | - |
| Q_LOCACC | 6 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_LINK(k) | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | FINITE VALUE | - |
| Q_LOCACC(k) | 6 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | Only in L2/L3 |
| Linking | STORED | - |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Mode Profile | STORED | Only in OS or LS mode |
| Unconditional Emergency Stop | NOT STORED | - |
| Conditional Emergency Stop | NOT STORED | - |
| Train Position | STORED | - |
| Train Data | STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Driver ID | STORED | - |
| Train Running Number | STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 6

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | General requirements |  | 3131020 |
| Test Case of Feature | There is no on-board interface with the service brake, so the requirements specified for the revocation of service brake command are followed for revoking the emergency brake. |  | 6 |
| Applicable Mode/Level Combinations | L1: SR L2/L3: SR |  |  |
| Target of Test | When there is no on-board interface with the service brake, whenever a service brake command is specified, the emergency brake command shall be triggered instead. The emergency brake command, which is triggered instead of the service brake command when an SBI supervision limit is exceeded, shall be revoked according to the requirements specified for the revocation of service brake command. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.13.10.2.3 |  |
|  |  | Subset-026-3.13.10.2.4 |  |
|  |  | Subset-026-3.13.10.4.9 |  |
|  |  | Subset-026-3.13.10.4.9 a) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | The test case shall consider the train data as preconditions to be known before setting the appropriate values for the rest of the trackside info and the speed profile. The required observables (speed supervision limits) should be checked using a suitable tool that performs fully the “speed and distance monitoring" functionality (by default use ERA Braking curves tool, which can be downloaded from this website: http://www.era.europa.eu/Core-Activities/ERTMS/Pages/Braking-Curves-Simulation-Tool.aspx). |
| Constraints | The value applicable for the SR mode speed limit and the distance to run in SR have not been previously entered by the driver. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 | SR |
| Radio communication session | ESTABLISHED | Only in L2/L3 |
| Axle load speed profile | NOT STORED | - |
| STM max speed | NOT STORED | - |
| STM system speed/distance | NOT STORED | - |
| List of Balises in SR Authority + SR mode speed limit and distance | STORED | Only in L2/L3 |
| Temporary Speed Restrictions | NOT STORED | - |
| Train Position | STORED | - |
| Train Data | STORED | - |
| Adhesion factor | NOT STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Driver ID | STORED | - |
| Radio Network ID | STORED | Only in L2/L3 |
| RBC ID/Phone Number | STORED | Only in L2/L3 |
| Train Running Number | STORED | - |
| Track Ahead Free Request | NOT STORED | - |
| Virtual Balise Covers | NOT STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is moving (V_TRAIN=<below the SBI supervision limit (V_MRSP + dV_SBI (V_MRSP))>) | I | INT | - | N/A | N/A | - |
| 2 | N/A | N/A | The driver requests the speed and distance monitoring information to be displayed | I | DMI | - | N/A | N/A | - |
| 3 | N/A | N/A | One Balise Group message containing packet 3 (V_NVSTFF=<V1>; D_NVSTFF=<D1>; Q_NVSBTSMPERM=1; Q_NVEMRRLS=0) is received | I | BTM | V1 and D1 will be adjusted in order to properly check the following steps. Q_NVSBTSMPERM=1: (Yes) permission to use service brake in target speed monitoring Q_NVEMRRLS=0: Revoke emergency brake command at standstill | N/A | N/A | - |
| 4 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | SA-DATA.Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted | O | RTM | Only in L2/L3. | N/A | N/A | - |
| 6 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | Only in L2/L3. | N/A | N/A | - |
| 7 | N/A | N/A | SA-DATA.Indication with Euroradio Message "SR Authorisation" (NID_MESSAGE=2; D_SR=<D2>) is received | I | RTM | Only in L2/L3. D2 will be adjusted in order to properly check the following steps D_SR=<D2> | N/A | N/A | - |
| 8 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | Only in L2/L3 | N/A | N/A | - |
| 9 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The train has passed with its maximum safe front end the location of the Indication supervision limit, valid for V_MRSP and calculated from the EBD, and it has not passed the location of the EBI supervision limit, valid for the estimated speed. The train accelerates until the estimated speed is above the SBI supervision limit (V_MRSP + dV_SBI (V_MRSP)) but below the EBI supervision limit (V_MRSP + dV_EBI (V_MRSP)). Then, the supervision status is "Intervention Status" (t12 in 3.13.10.4.10 table 9 of the SRS). So, the DMI aspect is coherent with Intervention Status in a Target speed monitoring section. | N/A | N/A | - |
| 10 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=2; M_SDMSUPSTAT=4) is recorded | O | JRU | M_SDMTYPE (Speed and distance monitoring type) = 2 (Target speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 4 (Intervention Status) | N/A | N/A | - |
| 11 | N/A | N/A | Emergency brake commanded | O | TIU | - | N/A | N/A | - |
| 12 | N/A | N/A | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | M_BRAKE_COMMAND_STATE = 1 (Command) | N/A | N/A | - |
| 13 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | When the train has left with its maximum safe front end the location of the Permitted Speed supervision limit, valid for the estimated speed and calculated from the EBD, then the supervision status changes to "Indication Status" (r3 in 3.13.10.4.12 table 11 of the SRS). So, the DMI aspect is coherent with Indication Status in a Target speed Monitoring section. | N/A | N/A | - |
| 14 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=2; M_SDMSUPSTAT=1) is recorded | O | JRU | M_SDMTYPE (Speed and distance monitoring type) = 2 (Target speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 1 (Indication Status) | N/A | N/A | - |
| 15 | N/A | N/A | Emergency brake not commanded | O | TIU | - | N/A | N/A | - |
| 16 | N/A | N/A | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=0) is recorded | O | JRU | M_BRAKE_COMMAND_STATE = 0 (Release) | N/A | N/A | - |

| Step 3: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 3 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_VALIDNV | 15 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | Identification of national areas to which the set applies. |
| N_ITER | 5 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | Identification of additional national area(s) to which the set applies. |
| V_NVSHUNT | 7 | FINITE VALUE | - |
| V_NVSTFF | 7 | FINITE VALUE | - |
| V_NVONSIGHT | 7 | FINITE VALUE | - |
| V_NVLIMSUPERV | 7 | FINITE VALUE | - |
| V_NVUNFIT | 7 | FINITE VALUE | - |
| V_NVREL | 7 | FINITE VALUE | - |
| D_NVROLL | 15 | FINITE VALUE | - |
| Q_NVSBTSMPERM | 1 | 1 | - |
| Q_NVEMRRLS | 1 | 0 | - |
| Q_NVGUIPERM | 1 | FINITE VALUE | - |
| Q_NVSBFBPERM | 1 | FINITE VALUE | - |
| Q_NVINHSMICPERM | 1 | FINITE VALUE | - |
| V_NVALLOWOVTRP | 7 | FINITE VALUE | - |
| V_NVSUPOVTRP | 7 | FINITE VALUE | - |
| D_NVOVTRP | 15 | FINITE VALUE | - |
| T_NVOVTRP | 8 | FINITE VALUE | - |
| D_NVPOTRP | 15 | FINITE VALUE | - |
| M_NVCONTACT | 2 | FINITE VALUE | - |
| T_NVCONTACT | 8 | FINITE VALUE | - |
| M_NVDERUN | 1 | FINITE VALUE | - |
| D_NVSTFF | 15 | FINITE VALUE | - |
| Q_NVDRIVER_ADHES | 1 | FINITE VALUE | - |
| A_NVMAXREDADH1 | 6 | FINITE VALUE | - |
| A_NVMAXREDADH2 | 6 | FINITE VALUE | - |
| A_NVMAXREDADH3 | 6 | FINITE VALUE | - |
| Q_NVLOCACC | 6 | FINITE VALUE | - |
| M_NVAVADH | 5 | FINITE VALUE | - |
| M_NVEBCL | 4 | FINITE VALUE | - |
| Q_NVKINT | 1 | FINITE VALUE | - |
| Q_NVKVINTSET | 2 | FINITE VALUE | Only if Q_NVKINT = 1, Q_NVKVINTSET and the following variables follow. |
| A_NVP12 | 6 | FINITE VALUE | Only if Q_NVKVINTSET = 1 |
| A_NVP23 | 6 | FINITE VALUE | Only if Q_NVKVINTSET = 1 |
| V_NVKVINT(n) | 7 | FINITE VALUE | 0 km/h |
| M_NVKVINT(n) | 7 | FINITE VALUE | Valid between V_NVKVINT and V_NVKVINT(1. If Q_NVKVINTSET = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12. |
| M_NVKVINT(n) | 7 | FINITE VALUE | Only if Q_NVKVINTSET = 1. Valid between V_NVKVINT and V_NVKVINT(1). Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23. |
| N_ITER | 5 | FINITE VALUE | - |
| V_NVKVINT | 7 | FINITE VALUE | - |
| M_NVKVINT | 7 | FINITE VALUE | Valid between V_NVKVINT(n) and V_NVKVINT(n+1). If Q_NVKVINTSET = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP1. |
| M_NVKVINT(n) | 7 | FINITE VALUE | Only if Q_NVKVINTSET = 1.Valid between V_NVKVINT(n) and V_NVKVINT(n+1) Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP2. |
| N_ITER | 5 | FINITE VALUE | - |
| Q_NVKVINTSET(k) | 2 | FINITE VALUE | - |
| A_NVP12(k) | 6 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1 |
| A_NVP23(k) | 6 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1 |
| V_NVKVINT(k) | 7 | FINITE VALUE | 0 |
| M_NVKVINT(k) | 7 | FINITE VALUE | Valid between V_NVKVINT(k) and V_NVKVINT(k,1). If Q_NVKVINTSET(k) = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12(k. |
| M_NVKVINT(k) | 7 | FINITE VALUE | Only if Q_NVKVINTSET(k) = . Valid between V_NVKVINT(k) and V_NVKVINT(k,1) Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23(k). |
| N_ITER(k) | 5 | FINITE VALUE | - |
| V_NVKVINT(k,m) | 7 | FINITE VALUE | - |
| M_NVKVINT(k,m) | 7 | FINITE VALUE | Valid between V_NVKVINT(k,m) and V_NVKVINT(k,m+1). If Q_NVKVINTSET(k) = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12(k). |
| M_NVKVINT(k,m) | 7 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1.Valid between V_NVKVINT(k,m) and V_NVKVINT(k,m+1). Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23(k). |
| L_NVKRINT | 5 | FINITE VALUE | 0 m |
| M_NVKRINT | 5 | FINITE VALUE | Valid between L_NVKRINT and L_NVKRINT(1). |
| N_ITER | 5 | FINITE VALUE | - |
| L_NVKRINT(l) | 5 | FINITE VALUE | - |
| M_NVKRINT(l) | 5 | FINITE VALUE | Valid between L_NVKRINT(l) and L_NVKRINT(l+1). |
| M_NVKTINT | 5 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | 255 |

| Step 5: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| Step 7: Radio Message 2 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 2 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_SR | 15 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 | SR |
| Radio communication session | ESTABLISHED | Only in L2/L3 |
| National Values | STORED | - |
| Axle load speed profile | NOT STORED | - |
| STM max speed | NOT STORED | - |
| STM system speed/distance | NOT STORED | - |
| List of Balises in SR Authority + SR mode speed limit and distance | STORED | Only in L2/L3 |
| Temporary Speed Restrictions | NOT STORED | - |
| Train Position | STORED | - |
| Train Data | STORED | - |
| Adhesion factor | NOT STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Driver ID | STORED | - |
| Radio Network ID | STORED | Only in L2/L3 |
| RBC ID/Phone Number | STORED | Only in L2/L3 |
| Train Running Number | STORED | - |
| Track Ahead Free Request | NOT STORED | - |
| Virtual Balise Covers | NOT STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |