| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 4080426 (4.8.4: Accepted Information depending on the modes - National Values) Total: 6 Test case(s) |  |
| REF: | Subset-076-5-2-4080426 |
| ISSUE: | Version 3.2.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 0.0.1 23.03.2012 | All | First version/review | DLR |
| Version 2.9.0 31.03.2013 | All | First version/review for Baseline 3 | CEDEX |
| Version 2.9.1 27.08.2013 | All | General revision | CEDEX |
| Version 2.9.2 25.10.2013 | All | Requirements correction | CEDEX |
| Version 2.9.3 03.11.2013 | TC3 | DELETE 3 LAST STEPS | CEDEX |
| Version 2.9.4 23.12.2013 | TC 4 and TC 6 | Substitution of the "Followed by" step by a "USE_FT" step | CEDEX |
| Version 2.9.5 08.01.2014 | All | Editorial changes | CEDEX |
| Version 2.9.6 22.01.2014 | TC 6 | In the Method field, the reference to "Followed by" step has been changed to "Use_FT". | CEDEX |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.2.0 31.03.2017 | All | Baseline 3 1st maintenance release version - Bug fixing | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - National Values |  | 4080426 |
| Test Case of Feature | Acceptance of National Values information. When a new value for M_NVDERUN is received on-board (by means of Packet 3) and it becomes applicable, the on-board will use this new value and Entry of Driver ID while running shall be not permitted. |  | 1 |
| Applicable Mode/Level Combinations | L0: SB, SH, NL, UN L1: SB, SH, FS, LS, SR, OS, NL L2: SB, SH, FS, LS, SR, OS, NL L3: SB, SH, FS, LS, SR, OS, NL LNTC: SB, NL, SH, SN |  |  |
| Target of Test | The target of this test is to check that: The train is moving and the entry of Driver ID while running shall be permitted. The train receives a newer set of National Values that shall be valid and applicable inmediatelly. The value of M_NVDERUN is updated and the on-board will use this new value so that the entry of Driver ID while running shall not be permitted. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.3.1.1 National Values |  |
|  |  | Subset-026-4.8.4.2 National Values |  |
|  |  | Subset-026-7.4.2.1.1 Packet Number 3: National Values |  |
|  |  | Subset-026-4.8.4.2 [2] |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : Before receiving Packet 3 the entry of Driver ID while running is permitted After receiving Packet 3, the on-board will use this new value so that the entry of Driver ID while running shall not be permitted. Check on the JRU that : Packet 3 National Values, Including M_NVDERUN & correct value of NID_C is RECORDED |
| Constraints | Packet 3 will be sent to the onboard with M_NVDERUN= 0, different from the value currently being used. A cab needs to be active (SB Mode) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 4 / 6 / 11 / 12 / 13 | FS / OS / SR / SH / UN / SB / NL / LS / SN |
| Radio communication session | ESTABLISHED | Only necessary if Packet 3 : National Values is received via radio |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | LNTC / L0 / L1 / L2 / L3 |
| Radio Network ID | STORED | Only necessary if Packet 3 : National Values is received via radio |
| RBC ID/Phone Number | STORED | Only necessary if Packet 3 : National Values is received via radio |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | Both | RTM | Only in L2 / L3 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is moving | I | INT | - | N/A | N/A | - |
| 2 | N/A | N/A | The driver modifies the "Driver ID" | I | DMI | - | N/A | N/A | - |
| 3 | N/A | N/A | One Balise Group message (N_TOTAL=1) containing packet 3 (M_NVDERUN=0) is received | I | BTM | This step will be used if steps 5 and 6 are not used. M_NVDERUN=0, different from the one currently being used The set of National Values that shall be valid and applicable inmediatelly | N/A | N/A | - |
| 4 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | This step will be used if steps 5 and 6 are not used. | N/A | N/A | - |
| 5 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 3 (M_NVDERUN=0) is received | I | RTM | This step will be used if steps 3 and 4 are not used and the current level is L2/ L3 or also in level 0/NTC/1 if a level 2 transition announcement is stored on board. M_NVDERUN=0,different from the one currently being used The set of National Values that shall be valid and applicable inmediatelly | N/A | N/A | - |
| 6 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | This step will be used if steps 3 and 4 are not used and the current level is L2/ L3 or also in level 0/NTC/1 if a level 2 transition announcement is stored on board. | N/A | N/A | - |
| 7 | N/A | N/A | The Level symbol "Level 2" is displayed | O | DMI | Only if coming from step 5 and being in level 0/NTC/1 | N/A | N/A | - |
| 8 | N/A | N/A | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=3) is recorded | O | JRU | Only if coming from THE PREVIOUS STEP | N/A | N/A | - |
| 9 | N/A | N/A | Train is moving | I | INT | - | N/A | N/A | - |
| 10 | N/A | N/A | The <Driver ID> button in the <Main> window is disabled | O | DMI | - | N/A | N/A | - |

| Step 3: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 3 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_VALIDNV | 15 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | Identification of national areas to which the set applies. |
| N_ITER | 5 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | Identification of additional national area(s) to which the set applies. |
| V_NVSHUNT | 7 | FINITE VALUE | - |
| V_NVSTFF | 7 | FINITE VALUE | - |
| V_NVONSIGHT | 7 | FINITE VALUE | - |
| V_NVLIMSUPERV | 7 | FINITE VALUE | - |
| V_NVUNFIT | 7 | FINITE VALUE | - |
| V_NVREL | 7 | FINITE VALUE | - |
| D_NVROLL | 15 | FINITE VALUE | - |
| Q_NVSBTSMPERM | 1 | FINITE VALUE | - |
| Q_NVEMRRLS | 1 | FINITE VALUE | - |
| Q_NVGUIPERM | 1 | FINITE VALUE | - |
| Q_NVSBFBPERM | 1 | FINITE VALUE | - |
| Q_NVINHSMICPERM | 1 | FINITE VALUE | - |
| V_NVALLOWOVTRP | 7 | FINITE VALUE | - |
| V_NVSUPOVTRP | 7 | FINITE VALUE | - |
| D_NVOVTRP | 15 | FINITE VALUE | - |
| T_NVOVTRP | 8 | FINITE VALUE | - |
| D_NVPOTRP | 15 | FINITE VALUE | - |
| M_NVCONTACT | 2 | FINITE VALUE | - |
| T_NVCONTACT | 8 | FINITE VALUE | - |
| M_NVDERUN | 1 | 0 | - |
| D_NVSTFF | 15 | FINITE VALUE | - |
| Q_NVDRIVER_ADHES | 1 | FINITE VALUE | - |
| A_NVMAXREDADH1 | 6 | FINITE VALUE | - |
| A_NVMAXREDADH2 | 6 | FINITE VALUE | - |
| A_NVMAXREDADH3 | 6 | FINITE VALUE | - |
| Q_NVLOCACC | 6 | FINITE VALUE | - |
| M_NVAVADH | 5 | FINITE VALUE | - |
| M_NVEBCL | 4 | FINITE VALUE | - |
| Q_NVKINT | 1 | FINITE VALUE | - |
| Q_NVKVINTSET | 2 | FINITE VALUE | Only if Q_NVKINT = 1, Q_NVKVINTSET and the following variables follow. |
| A_NVP12 | 6 | FINITE VALUE | Only if Q_NVKVINTSET = 1 |
| A_NVP23 | 6 | FINITE VALUE | Only if Q_NVKVINTSET = 1 |
| V_NVKVINT | 7 | FINITE VALUE | 0 km/h |
| M_NVKVINT | 7 | FINITE VALUE | Valid between V_NVKVINT and V_NVKVINT(1. If Q_NVKVINTSET = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12. |
| M_NVKVINT | 7 | FINITE VALUE | Only if Q_NVKVINTSET = 1. Valid between V_NVKVINT and V_NVKVINT(1). Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23. |
| N_ITER | 5 | FINITE VALUE | - |
| V_NVKVINT(n) | 7 | FINITE VALUE | - |
| M_NVKVINT(n) | 7 | FINITE VALUE | Valid between V_NVKVINT(n) and V_NVKVINT(n+1). If Q_NVKVINTSET = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP1. |
| M_NVKVINT(n) | 7 | FINITE VALUE | Only if Q_NVKVINTSET = 1.Valid between V_NVKVINT(n) and V_NVKVINT(n+1) Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP2. |
| N_ITER | 5 | FINITE VALUE | - |
| Q_NVKVINTSET(k) | 2 | FINITE VALUE | - |
| A_NVP12(k) | 6 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1 |
| A_NVP23(k) | 6 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1 |
| V_NVKVINT(k) | 7 | FINITE VALUE | 0 |
| M_NVKVINT(k) | 7 | FINITE VALUE | Valid between V_NVKVINT(k) and V_NVKVINT(k,1). If Q_NVKVINTSET(k) = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12(k. |
| M_NVKVINT(k) | 7 | FINITE VALUE | Only if Q_NVKVINTSET(k) = . Valid between V_NVKVINT(k) and V_NVKVINT(k,1) Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23(k). |
| N_ITER(k) | 5 | FINITE VALUE | - |
| V_NVKVINT(k,m) | 7 | FINITE VALUE | - |
| M_NVKVINT(k,m) | 7 | FINITE VALUE | Valid between V_NVKVINT(k,m) and V_NVKVINT(k,m+1). If Q_NVKVINTSET(k) = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12(k). |
| M_NVKVINT(k,m) | 7 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1.Valid between V_NVKVINT(k,m) and V_NVKVINT(k,m+1). Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23(k). |
| L_NVKRINT | 5 | FINITE VALUE | 0 m |
| M_NVKRINT | 5 | FINITE VALUE | Valid between L_NVKRINT and L_NVKRINT(1). |
| N_ITER | 5 | FINITE VALUE | - |
| L_NVKRINT(l) | 5 | FINITE VALUE | - |
| M_NVKRINT(l) | 5 | FINITE VALUE | Valid between L_NVKRINT(l) and L_NVKRINT(l+1). |
| M_NVKTINT | 5 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | 255 |

| Step 3: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | 255 |

| Step 5: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 3 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_VALIDNV | 15 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | Identification of national areas to which the set applies. |
| N_ITER | 5 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | Identification of additional national area(s) to which the set applies. |
| V_NVSHUNT | 7 | FINITE VALUE | - |
| V_NVSTFF | 7 | FINITE VALUE | - |
| V_NVONSIGHT | 7 | FINITE VALUE | - |
| V_NVLIMSUPERV | 7 | FINITE VALUE | - |
| V_NVUNFIT | 7 | FINITE VALUE | - |
| V_NVREL | 7 | FINITE VALUE | - |
| D_NVROLL | 15 | FINITE VALUE | - |
| Q_NVSBTSMPERM | 1 | FINITE VALUE | - |
| Q_NVEMRRLS | 1 | FINITE VALUE | - |
| Q_NVGUIPERM | 1 | FINITE VALUE | - |
| Q_NVSBFBPERM | 1 | FINITE VALUE | - |
| Q_NVINHSMICPERM | 1 | FINITE VALUE | - |
| V_NVALLOWOVTRP | 7 | FINITE VALUE | - |
| V_NVSUPOVTRP | 7 | FINITE VALUE | - |
| D_NVOVTRP | 15 | FINITE VALUE | - |
| T_NVOVTRP | 8 | FINITE VALUE | - |
| D_NVPOTRP | 15 | FINITE VALUE | - |
| M_NVCONTACT | 2 | FINITE VALUE | - |
| T_NVCONTACT | 8 | FINITE VALUE | - |
| M_NVDERUN | 1 | 0 | - |
| D_NVSTFF | 15 | FINITE VALUE | - |
| Q_NVDRIVER_ADHES | 1 | FINITE VALUE | - |
| A_NVMAXREDADH1 | 6 | FINITE VALUE | - |
| A_NVMAXREDADH2 | 6 | FINITE VALUE | - |
| A_NVMAXREDADH3 | 6 | FINITE VALUE | - |
| Q_NVLOCACC | 6 | FINITE VALUE | - |
| M_NVAVADH | 5 | FINITE VALUE | - |
| M_NVEBCL | 4 | FINITE VALUE | - |
| Q_NVKINT | 1 | FINITE VALUE | - |
| Q_NVKVINTSET | 2 | FINITE VALUE | Only if Q_NVKINT = 1, Q_NVKVINTSET and the following variables follow. |
| A_NVP12 | 6 | FINITE VALUE | Only if Q_NVKVINTSET = 1 |
| A_NVP23 | 6 | FINITE VALUE | Only if Q_NVKVINTSET = 1 |
| V_NVKVINT | 7 | FINITE VALUE | 0 km/h |
| M_NVKVINT | 7 | FINITE VALUE | Valid between V_NVKVINT and V_NVKVINT(1. If Q_NVKVINTSET = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12. |
| M_NVKVINT | 7 | FINITE VALUE | Only if Q_NVKVINTSET = 1. Valid between V_NVKVINT and V_NVKVINT(1). Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23. |
| N_ITER | 5 | FINITE VALUE | - |
| V_NVKVINT(n) | 7 | FINITE VALUE | - |
| M_NVKVINT(n) | 7 | FINITE VALUE | Valid between V_NVKVINT(n) and V_NVKVINT(n+1). If Q_NVKVINTSET = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP1. |
| M_NVKVINT(n) | 7 | FINITE VALUE | Only if Q_NVKVINTSET = 1.Valid between V_NVKVINT(n) and V_NVKVINT(n+1) Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP2. |
| N_ITER | 5 | FINITE VALUE | - |
| Q_NVKVINTSET(k) | 2 | FINITE VALUE | - |
| A_NVP12(k) | 6 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1 |
| A_NVP23(k) | 6 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1 |
| V_NVKVINT(k) | 7 | FINITE VALUE | 0 |
| M_NVKVINT(k) | 7 | FINITE VALUE | Valid between V_NVKVINT(k) and V_NVKVINT(k,1). If Q_NVKVINTSET(k) = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12(k. |
| M_NVKVINT(k) | 7 | FINITE VALUE | Only if Q_NVKVINTSET(k) = . Valid between V_NVKVINT(k) and V_NVKVINT(k,1) Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23(k). |
| N_ITER(k) | 5 | FINITE VALUE | - |
| V_NVKVINT(k,m) | 7 | FINITE VALUE | - |
| M_NVKVINT(k,m) | 7 | FINITE VALUE | Valid between V_NVKVINT(k,m) and V_NVKVINT(k,m+1). If Q_NVKVINTSET(k) = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12(k). |
| M_NVKVINT(k,m) | 7 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1.Valid between V_NVKVINT(k,m) and V_NVKVINT(k,m+1). Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23(k). |
| L_NVKRINT | 5 | FINITE VALUE | 0 m |
| M_NVKRINT | 5 | FINITE VALUE | Valid between L_NVKRINT and L_NVKRINT(1). |
| N_ITER | 5 | FINITE VALUE | - |
| L_NVKRINT(l) | 5 | FINITE VALUE | - |
| M_NVKRINT(l) | 5 | FINITE VALUE | Valid between L_NVKRINT(l) and L_NVKRINT(l+1). |
| M_NVKTINT | 5 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 4 / 6 / 11 / 12 / 13 | FS / OS / SR / SH / UN / SB / NL / LS / SN |
| Radio communication session | ESTABLISHED | Only necessary if Packet 3 : National Values is received via radio |
| National Values | STORED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | LNTC / L0 / L1 / L2 / L3 |
| Radio Network ID | STORED | Only necessary if Packet 3 : National Values is received via radio |
| RBC ID/Phone Number | STORED | Only necessary if Packet 3 : National Values is received via radio |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | Both | RTM | Only in L2 / L3 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - National Values |  | 4080426 |
| Test Case of Feature | Acceptance of National Values information. When a new value for D_NVPOTRP is received on-board (by means of Packet 3) and it becomes applicable, the on-board will use this new value to supervise the on-going movement in PT mode. However, the starting location shall remain unchanged. |  | 2 |
| Applicable Mode/Level Combinations | L2: PT L3: PT |  |  |
| Target of Test | The target of this test is to check that: The train is moving backwards in PT mode and being supervised against D_NVPOTRP. The train receives a newer set of National Values that shall be valid and applicable inmediatelly. The value of D_NVPOTRP is updated and the train is supervised against a new value that has its origin at the location where the train started to move in PT mode. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.18.2.2 |  |
|  |  | Subset-026-3.18.2.3 |  |
|  |  | Subset-026-3.18.2.4 |  |
|  |  | Subset-026-3.18.2.8.1 |  |
|  |  | Subset-026-3.18.2.11 |  |
|  |  | Subset-026-4.8.3.1.1 National Values |  |
|  |  | Subset-026-4.8.4.2 National Values |  |
|  |  | Subset-026-4.4.14.1.3 |  |
|  |  | Subset-026-7.4.2.1.1 Packet Number 3: National Values |  |
|  |  | Subset-026-4.8.4.2 [1] |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The train is in PT mode. After receiving Packet 3, the train is supervised against the new value for D_NVPOTRP, but the starting location remains unchanged. Check on the JRU that : Packet 3 National Values, Including D_NVOVTRP & correct value of NID_C is RECORDED The brakes are applied after exceeding the allowed distance for running in PT mode. |
| Constraints | Packet 3 will be sent to the onboard with D_NVPOTRP different from the value currently being used. National Values information is sent following the reception of the information “Recognition of Exit from TR mode” with a more recent time stamp |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 8 | PT |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Radio Network ID | STORED | - |
| RBC ID/Phone Number | STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | Both | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is moving | I | INT | The train is running backwards in PT mode. | N/A | N/A | - |
| 2 | N/A | N/A | The Mode symbol "Post trip" is displayed | O | DMI | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 3 (D_NVPOTRP=<different from the previously stored value>) is received | I | RTM | D_NVPOTRP will be different from the one currently being used | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | Service brake commanded | O | TIU | The train exceeds the distance d = d0 - D_NVPOTRP d0 is the position of the train when the the train started to run in PT mode | N/A | N/A | - |
| 6 | N/A | N/A | The Status symbol "Service Brake intervention" is displayed | O | DMI | - | N/A | N/A | - |
| 7 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | N/A | N/A | - |
| 8 | N/A | N/A | Train is at standstill (V_TRAIN=0) | I | INT | - | N/A | N/A | - |

| Step 3: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 3 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_VALIDNV | 15 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | Identification of national areas to which the set applies. |
| N_ITER | 5 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | Identification of additional national area(s) to which the set applies. |
| V_NVSHUNT | 7 | FINITE VALUE | - |
| V_NVSTFF | 7 | FINITE VALUE | - |
| V_NVONSIGHT | 7 | FINITE VALUE | - |
| V_NVLIMSUPERV | 7 | FINITE VALUE | - |
| V_NVUNFIT | 7 | FINITE VALUE | - |
| V_NVREL | 7 | FINITE VALUE | - |
| D_NVROLL | 15 | FINITE VALUE | - |
| Q_NVSBTSMPERM | 1 | FINITE VALUE | - |
| Q_NVEMRRLS | 1 | FINITE VALUE | - |
| Q_NVGUIPERM | 1 | FINITE VALUE | - |
| Q_NVSBFBPERM | 1 | FINITE VALUE | - |
| Q_NVINHSMICPERM | 1 | FINITE VALUE | - |
| V_NVALLOWOVTRP | 7 | FINITE VALUE | - |
| V_NVSUPOVTRP | 7 | FINITE VALUE | - |
| D_NVOVTRP | 15 | FINITE VALUE | - |
| T_NVOVTRP | 8 | FINITE VALUE | - |
| D_NVPOTRP | 15 | FINITE VALUE | - |
| M_NVCONTACT | 2 | FINITE VALUE | - |
| T_NVCONTACT | 8 | FINITE VALUE | - |
| M_NVDERUN | 1 | FINITE VALUE | - |
| D_NVSTFF | 15 | FINITE VALUE | - |
| Q_NVDRIVER_ADHES | 1 | FINITE VALUE | - |
| A_NVMAXREDADH1 | 6 | FINITE VALUE | - |
| A_NVMAXREDADH2 | 6 | FINITE VALUE | - |
| A_NVMAXREDADH3 | 6 | FINITE VALUE | - |
| Q_NVLOCACC | 6 | FINITE VALUE | - |
| M_NVAVADH | 5 | FINITE VALUE | - |
| M_NVEBCL | 4 | FINITE VALUE | - |
| Q_NVKINT | 1 | FINITE VALUE | - |
| Q_NVKVINTSET | 2 | FINITE VALUE | Only if Q_NVKINT = 1, Q_NVKVINTSET and the following variables follow. |
| A_NVP12 | 6 | FINITE VALUE | Only if Q_NVKVINTSET = 1 |
| A_NVP23 | 6 | FINITE VALUE | Only if Q_NVKVINTSET = 1 |
| V_NVKVINT | 7 | FINITE VALUE | 0 km/h |
| M_NVKVINT | 7 | FINITE VALUE | Valid between V_NVKVINT and V_NVKVINT(1. If Q_NVKVINTSET = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12. |
| M_NVKVINT | 7 | FINITE VALUE | Only if Q_NVKVINTSET = 1. Valid between V_NVKVINT and V_NVKVINT(1). Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23. |
| N_ITER | 5 | FINITE VALUE | - |
| V_NVKVINT(n) | 7 | FINITE VALUE | - |
| M_NVKVINT(n) | 7 | FINITE VALUE | Valid between V_NVKVINT(n) and V_NVKVINT(n+1). If Q_NVKVINTSET = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP1. |
| M_NVKVINT(n) | 7 | FINITE VALUE | Only if Q_NVKVINTSET = 1.Valid between V_NVKVINT(n) and V_NVKVINT(n+1) Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP2. |
| N_ITER | 5 | FINITE VALUE | - |
| Q_NVKVINTSET(k) | 2 | FINITE VALUE | - |
| A_NVP12(k) | 6 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1 |
| A_NVP23(k) | 6 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1 |
| V_NVKVINT(k) | 7 | FINITE VALUE | 0 |
| M_NVKVINT(k) | 7 | FINITE VALUE | Valid between V_NVKVINT(k) and V_NVKVINT(k,1). If Q_NVKVINTSET(k) = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12(k. |
| M_NVKVINT(k) | 7 | FINITE VALUE | Only if Q_NVKVINTSET(k) = . Valid between V_NVKVINT(k) and V_NVKVINT(k,1) Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23(k). |
| N_ITER(k) | 5 | FINITE VALUE | - |
| V_NVKVINT(k,m) | 7 | FINITE VALUE | - |
| M_NVKVINT(k,m) | 7 | FINITE VALUE | Valid between V_NVKVINT(k,m) and V_NVKVINT(k,m+1). If Q_NVKVINTSET(k) = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12(k). |
| M_NVKVINT(k,m) | 7 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1.Valid between V_NVKVINT(k,m) and V_NVKVINT(k,m+1). Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23(k). |
| L_NVKRINT | 5 | FINITE VALUE | 0 m |
| M_NVKRINT | 5 | FINITE VALUE | Valid between L_NVKRINT and L_NVKRINT(1). |
| N_ITER | 5 | FINITE VALUE | - |
| L_NVKRINT(l) | 5 | FINITE VALUE | - |
| M_NVKRINT(l) | 5 | FINITE VALUE | Valid between L_NVKRINT(l) and L_NVKRINT(l+1). |
| M_NVKTINT | 5 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 8 | PT |
| Radio communication session | ESTABLISHED | - |
| National Values | STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Radio Network ID | STORED | - |
| RBC ID/Phone Number | STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | Both | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - National Values |  | 4080426 |
| Test Case of Feature | On-board Equipment accepts NATIONAL VALUES transmitted by RBC |  | 3 |
| Applicable Mode/Level Combinations | L2 : FS/LS/OS/SR/SH/PS/SL/SB/TR/PT/NL/RV L3 : FS/LS/OS/SR/SH/PS/SL/SB/TR/PTNL/RV |  |  |
| Target of Test | The test is aimed to check that the Onboard Equipment accepts and stores the National Values |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.4.2 [1] |  |
|  |  | Subset-026-4.8.3.1.1 National Values |  |
|  |  | Subset-026-4.8.4.2 National Values |  |
|  |  | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |
|  |  | Subset-026-7.4.2.1.1 Packet Number 3: National Values |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on JRU that RBC has sent message 24 containing packet 3 [National values] Check that the On-board Equipment accepts and stores National Values. Select and use a suitable test case in order to observe the packet 3 dependent reaction on available interfaces |
| Constraints | In PT mode the RBC message including the packet of National Values is accepted only if following the reception of the information ”Recognition of Exit from TR mode” with a more recent time stamp. (see comments below the table 4.8.4.2 in SRS) A cab needs to be active (SB Mode) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | FS/LS/OS/SR/SH/PS/SL/SB/TR/PT/NL/RV | 0/12/1/2/3/15/5/6/7/8/11/14 |
| Radio communication session | ESTABLISHED | - |
| Train Position | STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2/ L3 |
| Radio Network ID | STORED | - |
| RBC ID/Phone Number | STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | Both | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 3 is received | I | RTM | Other combinations for acceptance are possible | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | Use_FT4060300: mode profile requires OS (Transition [40]) and Use_FT3130237: check the supervision of a speed limit. | - | - | These FTs are just examples. A suitable test case will be used in order to cause some onboard reaction that can be related to packet 3 (dependent observables on available interfaces). | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 3 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_VALIDNV | 15 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | Identification of national areas to which the set applies. |
| N_ITER | 5 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | Identification of additional national area(s) to which the set applies. |
| V_NVSHUNT | 7 | FINITE VALUE | - |
| V_NVSTFF | 7 | FINITE VALUE | - |
| V_NVONSIGHT | 7 | FINITE VALUE | - |
| V_NVLIMSUPERV | 7 | FINITE VALUE | - |
| V_NVUNFIT | 7 | FINITE VALUE | - |
| V_NVREL | 7 | FINITE VALUE | - |
| D_NVROLL | 15 | FINITE VALUE | - |
| Q_NVSBTSMPERM | 1 | FINITE VALUE | - |
| Q_NVEMRRLS | 1 | FINITE VALUE | - |
| Q_NVGUIPERM | 1 | FINITE VALUE | - |
| Q_NVSBFBPERM | 1 | FINITE VALUE | - |
| Q_NVINHSMICPERM | 1 | FINITE VALUE | - |
| V_NVALLOWOVTRP | 7 | FINITE VALUE | - |
| V_NVSUPOVTRP | 7 | FINITE VALUE | - |
| D_NVOVTRP | 15 | FINITE VALUE | - |
| T_NVOVTRP | 8 | FINITE VALUE | - |
| D_NVPOTRP | 15 | FINITE VALUE | - |
| M_NVCONTACT | 2 | FINITE VALUE | - |
| T_NVCONTACT | 8 | FINITE VALUE | - |
| M_NVDERUN | 1 | FINITE VALUE | - |
| D_NVSTFF | 15 | FINITE VALUE | - |
| Q_NVDRIVER_ADHES | 1 | FINITE VALUE | - |
| A_NVMAXREDADH1 | 6 | FINITE VALUE | - |
| A_NVMAXREDADH2 | 6 | FINITE VALUE | - |
| A_NVMAXREDADH3 | 6 | FINITE VALUE | - |
| Q_NVLOCACC | 6 | FINITE VALUE | - |
| M_NVAVADH | 5 | FINITE VALUE | - |
| M_NVEBCL | 4 | FINITE VALUE | - |
| Q_NVKINT | 1 | FINITE VALUE | - |
| Q_NVKVINTSET | 2 | FINITE VALUE | Only if Q_NVKINT = 1, Q_NVKVINTSET and the following variables follow. |
| A_NVP12 | 6 | FINITE VALUE | Only if Q_NVKVINTSET = 1 |
| A_NVP23 | 6 | FINITE VALUE | Only if Q_NVKVINTSET = 1 |
| V_NVKVINT | 7 | FINITE VALUE | 0 km/h |
| M_NVKVINT | 7 | FINITE VALUE | Valid between V_NVKVINT and V_NVKVINT(1. If Q_NVKVINTSET = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12. |
| M_NVKVINT | 7 | FINITE VALUE | Only if Q_NVKVINTSET = 1. Valid between V_NVKVINT and V_NVKVINT(1). Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23. |
| N_ITER | 5 | FINITE VALUE | - |
| V_NVKVINT(n) | 7 | FINITE VALUE | - |
| M_NVKVINT(n) | 7 | FINITE VALUE | Valid between V_NVKVINT(n) and V_NVKVINT(n+1). If Q_NVKVINTSET = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP1. |
| M_NVKVINT(n) | 7 | FINITE VALUE | Only if Q_NVKVINTSET = 1.Valid between V_NVKVINT(n) and V_NVKVINT(n+1) Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP2. |
| N_ITER | 5 | FINITE VALUE | - |
| Q_NVKVINTSET(k) | 2 | FINITE VALUE | - |
| A_NVP12(k) | 6 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1 |
| A_NVP23(k) | 6 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1 |
| V_NVKVINT(k) | 7 | FINITE VALUE | 0 |
| M_NVKVINT(k) | 7 | FINITE VALUE | Valid between V_NVKVINT(k) and V_NVKVINT(k,1). If Q_NVKVINTSET(k) = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12(k. |
| M_NVKVINT(k) | 7 | FINITE VALUE | Only if Q_NVKVINTSET(k) = . Valid between V_NVKVINT(k) and V_NVKVINT(k,1) Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23(k). |
| N_ITER(k) | 5 | FINITE VALUE | - |
| V_NVKVINT(k,m) | 7 | FINITE VALUE | - |
| M_NVKVINT(k,m) | 7 | FINITE VALUE | Valid between V_NVKVINT(k,m) and V_NVKVINT(k,m+1). If Q_NVKVINTSET(k) = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12(k). |
| M_NVKVINT(k,m) | 7 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1.Valid between V_NVKVINT(k,m) and V_NVKVINT(k,m+1). Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23(k). |
| L_NVKRINT | 5 | FINITE VALUE | 0 m |
| M_NVKRINT | 5 | FINITE VALUE | Valid between L_NVKRINT and L_NVKRINT(1). |
| N_ITER | 5 | FINITE VALUE | - |
| L_NVKRINT(l) | 5 | FINITE VALUE | - |
| M_NVKRINT(l) | 5 | FINITE VALUE | Valid between L_NVKRINT(l) and L_NVKRINT(l+1). |
| M_NVKTINT | 5 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | FS/LS/OS/SR/SH/PS/SL/SB/TR/PT/NL/RV | 0/12/1/2/3/15/5/6/7/8/11/14 |
| Radio communication session | ESTABLISHED | - |
| National Values | STORED | - |
| Train Position | STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2/ L3 |
| Radio Network ID | STORED | - |
| RBC ID/Phone Number | STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | Both | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - National Values |  | 4080426 |
| Test Case of Feature | Transmission of National Values via balise group |  | 4 |
| Applicable Mode/Level Combinations | LNTC: SL/SB/NL/SH/PS/SN L 0 : SL/ SB/ SH/PS/UN/NL L 1: FS/LS/OS/SR/SH/PS/SL/ SB/TR/NL/ RV L 2: FS/LS/OS/SR/SH/PS/SL/ SB/TR/NL/ RV L 3: FS/LS/OS/SR/SH/PS/SL/ SB/TR/NL/ RV |  |  |
| Target of Test | The test is aimed to check that the Onboard Equipment accepts National Values transmitted via BG (single BG or not). |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.4.17.1.2 |  |
|  |  | Subset-026-4.8.3.1.1 National Values |  |
|  |  | Subset-026-4.8.4.2 National Values |  |
|  |  | Subset-026-7.4.2.1.1 Packet Number 3: National Values |  |
|  |  | Subset-026-4.8.4.2 [2] |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on JRU that Onboard equipment received National Values by means of the balise group. Check on all available interfaces that the Onboard Equipment accepts, stores and uses the National Values, by means of the sequence of feature indicated in step "Use_FT" of the table "SEQUENCE OF TEST".Check on JRU that Onboard equipment received National Values by means of the balise group. Check on all available interfaces that the Onboard Equipment accepts, stores and uses the National Values, by means of the sequence of feature indicated in step "Use_FT" of the table "SEQUENCE OF TEST".Check on JRU that Onboard equipment received National Values by means of the balise group. Check on all available interfaces that the Onboard Equipment accepts, stores and uses the National Values, by means of the sequence of feature indicated in step "Use_FT" of the table "SEQUENCE OF TEST".Check on JRU that Onboard equipment received National Values by means of the balise group. Check on all available interfaces that the Onboard Equipment accepts, stores and uses the National Values, by means of the sequence of feature indicated in step "Use_FT" of the table "SEQUENCE OF TEST". |
| Constraints | A cab needs to be active (SB Mode) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | FS/LS/OS/SR/SH/PS/SL/SB/TR/NL/RV/UN/SN | 0/12/1/2/3/15/5/6/7/11/14/4/13 |
| Train Position | STORED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | LNTC / L0 / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message containing packet 3 is received | I | BTM | - | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | Use_FT4060300: mode profile requires OS (Transition [40]) and Use_FT3130237: check the supervision of a speed limit. | - | - | These FTs are just examples. A suitable test case will be used in order to cause some onboard reaction that can be related to packet 3 (dependent observables on available interfaces). | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 3 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_VALIDNV | 15 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | Identification of national areas to which the set applies. |
| N_ITER | 5 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | Identification of additional national area(s) to which the set applies. |
| V_NVSHUNT | 7 | FINITE VALUE | - |
| V_NVSTFF | 7 | FINITE VALUE | - |
| V_NVONSIGHT | 7 | FINITE VALUE | - |
| V_NVLIMSUPERV | 7 | FINITE VALUE | - |
| V_NVUNFIT | 7 | FINITE VALUE | - |
| V_NVREL | 7 | FINITE VALUE | - |
| D_NVROLL | 15 | FINITE VALUE | - |
| Q_NVSBTSMPERM | 1 | FINITE VALUE | - |
| Q_NVEMRRLS | 1 | FINITE VALUE | - |
| Q_NVGUIPERM | 1 | FINITE VALUE | - |
| Q_NVSBFBPERM | 1 | FINITE VALUE | - |
| Q_NVINHSMICPERM | 1 | FINITE VALUE | - |
| V_NVALLOWOVTRP | 7 | FINITE VALUE | - |
| V_NVSUPOVTRP | 7 | FINITE VALUE | - |
| D_NVOVTRP | 15 | FINITE VALUE | - |
| T_NVOVTRP | 8 | FINITE VALUE | - |
| D_NVPOTRP | 15 | FINITE VALUE | - |
| M_NVCONTACT | 2 | FINITE VALUE | - |
| T_NVCONTACT | 8 | FINITE VALUE | - |
| M_NVDERUN | 1 | FINITE VALUE | - |
| D_NVSTFF | 15 | FINITE VALUE | - |
| Q_NVDRIVER_ADHES | 1 | FINITE VALUE | - |
| A_NVMAXREDADH1 | 6 | FINITE VALUE | - |
| A_NVMAXREDADH2 | 6 | FINITE VALUE | - |
| A_NVMAXREDADH3 | 6 | FINITE VALUE | - |
| Q_NVLOCACC | 6 | FINITE VALUE | - |
| M_NVAVADH | 5 | FINITE VALUE | - |
| M_NVEBCL | 4 | FINITE VALUE | - |
| Q_NVKINT | 1 | FINITE VALUE | - |
| Q_NVKVINTSET | 2 | FINITE VALUE | Only if Q_NVKINT = 1, Q_NVKVINTSET and the following variables follow. |
| A_NVP12 | 6 | FINITE VALUE | Only if Q_NVKVINTSET = 1 |
| A_NVP23 | 6 | FINITE VALUE | Only if Q_NVKVINTSET = 1 |
| V_NVKVINT | 7 | FINITE VALUE | 0 km/h |
| M_NVKVINT | 7 | FINITE VALUE | Valid between V_NVKVINT and V_NVKVINT(1. If Q_NVKVINTSET = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12. |
| M_NVKVINT | 7 | FINITE VALUE | Only if Q_NVKVINTSET = 1. Valid between V_NVKVINT and V_NVKVINT(1). Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23. |
| N_ITER | 5 | FINITE VALUE | - |
| V_NVKVINT(n) | 7 | FINITE VALUE | - |
| M_NVKVINT(n) | 7 | FINITE VALUE | Valid between V_NVKVINT(n) and V_NVKVINT(n+1). If Q_NVKVINTSET = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP1. |
| M_NVKVINT(n) | 7 | FINITE VALUE | Only if Q_NVKVINTSET = 1.Valid between V_NVKVINT(n) and V_NVKVINT(n+1) Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP2. |
| N_ITER | 5 | FINITE VALUE | - |
| Q_NVKVINTSET(k) | 2 | FINITE VALUE | - |
| A_NVP12(k) | 6 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1 |
| A_NVP23(k) | 6 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1 |
| V_NVKVINT(k) | 7 | FINITE VALUE | 0 |
| M_NVKVINT(k) | 7 | FINITE VALUE | Valid between V_NVKVINT(k) and V_NVKVINT(k,1). If Q_NVKVINTSET(k) = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12(k. |
| M_NVKVINT(k) | 7 | FINITE VALUE | Only if Q_NVKVINTSET(k) = . Valid between V_NVKVINT(k) and V_NVKVINT(k,1) Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23(k). |
| N_ITER(k) | 5 | FINITE VALUE | - |
| V_NVKVINT(k,m) | 7 | FINITE VALUE | - |
| M_NVKVINT(k,m) | 7 | FINITE VALUE | Valid between V_NVKVINT(k,m) and V_NVKVINT(k,m+1). If Q_NVKVINTSET(k) = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12(k). |
| M_NVKVINT(k,m) | 7 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1.Valid between V_NVKVINT(k,m) and V_NVKVINT(k,m+1). Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23(k). |
| L_NVKRINT | 5 | FINITE VALUE | 0 m |
| M_NVKRINT | 5 | FINITE VALUE | Valid between L_NVKRINT and L_NVKRINT(1). |
| N_ITER | 5 | FINITE VALUE | - |
| L_NVKRINT(l) | 5 | FINITE VALUE | - |
| M_NVKRINT(l) | 5 | FINITE VALUE | Valid between L_NVKRINT(l) and L_NVKRINT(l+1). |
| M_NVKTINT | 5 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | 255 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | FS/LS/OS/SR/SH/PS/SL/SB/TR/NL/RV/UN/SN | 0/12/1/2/3/15/5/6/7/11/14/4/13 |
| National Values | STORED | - |
| Train Position | STORED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | LNTC / L0 / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 5

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - National Values |  | 4080426 |
| Test Case of Feature | Rejection of National Values information. When a new value for D_NVPOTRP is received on-board (by means of Packet 3), the on-board will reject this information to supervise the on-going movement in PT mode. The current value of D_NVPOTRP will be used instead to supervise the on-going movement in PT mode. |  | 5 |
| Applicable Mode/Level Combinations | L1: PT |  |  |
| Target of Test | The target of this test is to check that: The train is moving backwards in PT mode and being supervised against D_NVPOTRP. The train receives a newer set of National Values applicable inmediatelly. National Values information is rejected and the value of D_NVPOTRP is not updated. The train is supervised against the current value that has its origin at the location where the train started to move in PT mode. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.18.2.2 |  |
|  |  | Subset-026-3.18.2.3 |  |
|  |  | Subset-026-3.18.2.4 |  |
|  |  | Subset-026-3.18.2.8.1 |  |
|  |  | Subset-026-3.18.2.11 |  |
|  |  | Subset-026-4.8.3.1.1 National Values |  |
|  |  | Subset-026-4.8.4.2 National Values |  |
|  |  | Subset-026-4.4.14.1.3 |  |
|  |  | Subset-026-7.4.2.1.1 Packet Number 3: National Values |  |
|  |  | Subset-026-4.8.4.2 [1] |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The train is in PT mode. After receiving Packet 3, which is rejected, the train is supervised against the current value for D_NVPOTRP, and the starting location remains unchanged. Check on the JRU that : Packet 3 National Values, Including D_NVOVTRP & correct value of NID_C is RECORDED The brakes are applied after exceeding the allowed distance for running in PT mode. |
| Constraints | Packet 3 will be sent to the onboard with D_NVPOTRP different from the value currently being used. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 8 | PT |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is moving | I | INT | The train is running backwards in PT mode and supervising the train movements against the value D_NVPOTRP’. | N/A | N/A | - |
| 2 | N/A | N/A | The Mode symbol "Post trip" is displayed/removed | O | DMI | - | N/A | N/A | - |
| 3 | N/A | N/A | One Balise Group message containing packet 3 (D_NVPOTRP=<different from the previously stored value>) is received | I | BTM | This step will be used if steps 5 and 6 are not used. D_NVPOTRP will be different from the one currently being used (D_NVPOTRP’) | N/A | N/A | - |
| 4 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | This step will be used if steps 5 and 6 are not used. | N/A | N/A | - |
| 5 | N/A | N/A | Train is moving | I | INT | The train is running backwards in PT mode and supervising the train movements against the value D_NVPOTRP’. | N/A | N/A | - |
| 6 | N/A | N/A | Service brake commanded | O | TIU | The train exceeds the distance d = d0 - D_NVPOTRP' d0 is the position of the train when the the train started to run in PT mode | N/A | N/A | - |
| 7 | N/A | N/A | The Status symbol "Service Brake intervention" is displayed | O | DMI | - | N/A | N/A | - |
| 8 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | Train is at standstill (V_TRAIN=0) | I | INT | - | N/A | N/A | - |
| 10 | N/A | N/A | Service brake not commanded | O | TIU | - | N/A | N/A | - |
| 11 | N/A | N/A | The Status symbol "Service Brake intervention" is removed | O | DMI | - | N/A | N/A | - |
| 12 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=0) is recorded | O | JRU | - | N/A | N/A | - |

| Step 3: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 3 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_VALIDNV | 15 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | Identification of national areas to which the set applies. |
| N_ITER | 5 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | Identification of additional national area(s) to which the set applies. |
| V_NVSHUNT | 7 | FINITE VALUE | - |
| V_NVSTFF | 7 | FINITE VALUE | - |
| V_NVONSIGHT | 7 | FINITE VALUE | - |
| V_NVLIMSUPERV | 7 | FINITE VALUE | - |
| V_NVUNFIT | 7 | FINITE VALUE | - |
| V_NVREL | 7 | FINITE VALUE | - |
| D_NVROLL | 15 | FINITE VALUE | - |
| Q_NVSBTSMPERM | 1 | FINITE VALUE | - |
| Q_NVEMRRLS | 1 | FINITE VALUE | - |
| Q_NVGUIPERM | 1 | FINITE VALUE | - |
| Q_NVSBFBPERM | 1 | FINITE VALUE | - |
| Q_NVINHSMICPERM | 1 | FINITE VALUE | - |
| V_NVALLOWOVTRP | 7 | FINITE VALUE | - |
| V_NVSUPOVTRP | 7 | FINITE VALUE | - |
| D_NVOVTRP | 15 | FINITE VALUE | - |
| T_NVOVTRP | 8 | FINITE VALUE | - |
| D_NVPOTRP | 15 | FINITE VALUE | - |
| M_NVCONTACT | 2 | FINITE VALUE | - |
| T_NVCONTACT | 8 | FINITE VALUE | - |
| M_NVDERUN | 1 | FINITE VALUE | - |
| D_NVSTFF | 15 | FINITE VALUE | - |
| Q_NVDRIVER_ADHES | 1 | FINITE VALUE | - |
| A_NVMAXREDADH1 | 6 | FINITE VALUE | - |
| A_NVMAXREDADH2 | 6 | FINITE VALUE | - |
| A_NVMAXREDADH3 | 6 | FINITE VALUE | - |
| Q_NVLOCACC | 6 | FINITE VALUE | - |
| M_NVAVADH | 5 | FINITE VALUE | - |
| M_NVEBCL | 4 | FINITE VALUE | - |
| Q_NVKINT | 1 | FINITE VALUE | - |
| Q_NVKVINTSET | 2 | FINITE VALUE | Only if Q_NVKINT = 1, Q_NVKVINTSET and the following variables follow. |
| A_NVP12 | 6 | FINITE VALUE | Only if Q_NVKVINTSET = 1 |
| A_NVP23 | 6 | FINITE VALUE | Only if Q_NVKVINTSET = 1 |
| V_NVKVINT | 7 | FINITE VALUE | 0 km/h |
| M_NVKVINT | 7 | FINITE VALUE | Valid between V_NVKVINT and V_NVKVINT(1. If Q_NVKVINTSET = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12. |
| M_NVKVINT | 7 | FINITE VALUE | Only if Q_NVKVINTSET = 1. Valid between V_NVKVINT and V_NVKVINT(1). Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23. |
| N_ITER | 5 | FINITE VALUE | - |
| V_NVKVINT(n) | 7 | FINITE VALUE | - |
| M_NVKVINT(n) | 7 | FINITE VALUE | Valid between V_NVKVINT(n) and V_NVKVINT(n+1). If Q_NVKVINTSET = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP1. |
| M_NVKVINT(n) | 7 | FINITE VALUE | Only if Q_NVKVINTSET = 1.Valid between V_NVKVINT(n) and V_NVKVINT(n+1) Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP2. |
| N_ITER | 5 | FINITE VALUE | - |
| Q_NVKVINTSET(k) | 2 | FINITE VALUE | - |
| A_NVP12(k) | 6 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1 |
| A_NVP23(k) | 6 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1 |
| V_NVKVINT(k) | 7 | FINITE VALUE | 0 |
| M_NVKVINT(k) | 7 | FINITE VALUE | Valid between V_NVKVINT(k) and V_NVKVINT(k,1). If Q_NVKVINTSET(k) = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12(k. |
| M_NVKVINT(k) | 7 | FINITE VALUE | Only if Q_NVKVINTSET(k) = . Valid between V_NVKVINT(k) and V_NVKVINT(k,1) Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23(k). |
| N_ITER(k) | 5 | FINITE VALUE | - |
| V_NVKVINT(k,m) | 7 | FINITE VALUE | - |
| M_NVKVINT(k,m) | 7 | FINITE VALUE | Valid between V_NVKVINT(k,m) and V_NVKVINT(k,m+1). If Q_NVKVINTSET(k) = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12(k). |
| M_NVKVINT(k,m) | 7 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1.Valid between V_NVKVINT(k,m) and V_NVKVINT(k,m+1). Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23(k). |
| L_NVKRINT | 5 | FINITE VALUE | 0 m |
| M_NVKRINT | 5 | FINITE VALUE | Valid between L_NVKRINT and L_NVKRINT(1). |
| N_ITER | 5 | FINITE VALUE | - |
| L_NVKRINT(l) | 5 | FINITE VALUE | - |
| M_NVKRINT(l) | 5 | FINITE VALUE | Valid between L_NVKRINT(l) and L_NVKRINT(l+1). |
| M_NVKTINT | 5 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | 255 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 8 | PT |
| National Values | STORED | - |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 6

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - National Values |  | 4080426 |
| Test Case of Feature | Rejection of National Values information. When a new value for M_NVDERUN is received on-board (by means of Packet 3 received by RBC) and the information is applicable, due to the rejection of the information the on-board will use the current value of M_NVDERUN. The Entry of Driver ID while running shall continue to be permitted. |  | 6 |
| Applicable Mode/Level Combinations | L0: SB, SH, NL, UN L1: SB, SH, FS, LS, SR, OS, NL LNTC: SN |  |  |
| Target of Test | The target of this test is to check that: The train is moving and the entry of Driver ID while running shall be permitted. The train receives a newer set of National Values that applicable immediately. The value of M_NVDERUN is not updated due to the rejection of National Values information and the on-board will use the current value so that the entry of Driver ID while running shall continue to be permitted. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.3.1.1 National Values |  |
|  |  | Subset-026-4.8.4.2 National Values |  |
|  |  | Subset-026-7.4.2.1.1 Packet Number 3: National Values |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : Before receiving Packet 3 the entry of Driver ID while running is permitted After receiving Packet 3, the on-board will not use this new value so that the entry of Driver ID while running shall continue to be permitted. Check on the JRU that : Packet 3 National Values, Including M_NVDERUN & correct value of NID_C is RECORDED |
| Constraints | Packet 3 will be sent to the onboard with M_NVDERUN= 0, different from the value currently being used. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6/ 3/ 0/ 12 / 2 / 1/ 11/ 4/ 13 | SB/ SH/ FS/ LS/ SR/ OS/ NL/ UN/ SN |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 0 / 1 / 2 | LNTC/ L0/ L1 |
| Driver ID | STORED | - |
| Radio Network ID | STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | Both | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is moving | I | INT | - | N/A | N/A | - |
| 2 | N/A | N/A | Entry of Driver ID while running is permitted. | O | DMI | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 3 (M_NVDERUN=0) is received | I | RTM | M_NVDERUN=0 is different from the one currently being used The set of National Values that shall be applicable inmediatelly but the information is rejected | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | Train is moving | I | INT | - | N/A | N/A | - |
| 6 | N/A | N/A | The driver modifies the "Driver ID" | I | DMI | - | N/A | N/A | - |
| 7 | N/A | N/A | GENERAL MESSAGE (NID_MESSAGE_JRU=1; DRIVER_ID=<new driver id>) is recorded | O | JRU | - | N/A | N/A | - |

| Step 3: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 3 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_VALIDNV | 15 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | Identification of national areas to which the set applies. |
| N_ITER | 5 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | Identification of additional national area(s) to which the set applies. |
| V_NVSHUNT | 7 | FINITE VALUE | - |
| V_NVSTFF | 7 | FINITE VALUE | - |
| V_NVONSIGHT | 7 | FINITE VALUE | - |
| V_NVLIMSUPERV | 7 | FINITE VALUE | - |
| V_NVUNFIT | 7 | FINITE VALUE | - |
| V_NVREL | 7 | FINITE VALUE | - |
| D_NVROLL | 15 | FINITE VALUE | - |
| Q_NVSBTSMPERM | 1 | FINITE VALUE | - |
| Q_NVEMRRLS | 1 | FINITE VALUE | - |
| Q_NVGUIPERM | 1 | FINITE VALUE | - |
| Q_NVSBFBPERM | 1 | FINITE VALUE | - |
| Q_NVINHSMICPERM | 1 | FINITE VALUE | - |
| V_NVALLOWOVTRP | 7 | FINITE VALUE | - |
| V_NVSUPOVTRP | 7 | FINITE VALUE | - |
| D_NVOVTRP | 15 | FINITE VALUE | - |
| T_NVOVTRP | 8 | FINITE VALUE | - |
| D_NVPOTRP | 15 | FINITE VALUE | - |
| M_NVCONTACT | 2 | FINITE VALUE | - |
| T_NVCONTACT | 8 | FINITE VALUE | - |
| M_NVDERUN | 1 | FINITE VALUE | - |
| D_NVSTFF | 15 | FINITE VALUE | - |
| Q_NVDRIVER_ADHES | 1 | FINITE VALUE | - |
| A_NVMAXREDADH1 | 6 | FINITE VALUE | - |
| A_NVMAXREDADH2 | 6 | FINITE VALUE | - |
| A_NVMAXREDADH3 | 6 | FINITE VALUE | - |
| Q_NVLOCACC | 6 | FINITE VALUE | - |
| M_NVAVADH | 5 | FINITE VALUE | - |
| M_NVEBCL | 4 | FINITE VALUE | - |
| Q_NVKINT | 1 | FINITE VALUE | - |
| Q_NVKVINTSET | 2 | FINITE VALUE | Only if Q_NVKINT = 1, Q_NVKVINTSET and the following variables follow. |
| A_NVP12 | 6 | FINITE VALUE | Only if Q_NVKVINTSET = 1 |
| A_NVP23 | 6 | FINITE VALUE | Only if Q_NVKVINTSET = 1 |
| V_NVKVINT | 7 | FINITE VALUE | 0 km/h |
| M_NVKVINT | 7 | FINITE VALUE | Valid between V_NVKVINT and V_NVKVINT(1. If Q_NVKVINTSET = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12. |
| M_NVKVINT | 7 | FINITE VALUE | Only if Q_NVKVINTSET = 1. Valid between V_NVKVINT and V_NVKVINT(1). Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23. |
| N_ITER | 5 | FINITE VALUE | - |
| V_NVKVINT(n) | 7 | FINITE VALUE | - |
| M_NVKVINT(n) | 7 | FINITE VALUE | Valid between V_NVKVINT(n) and V_NVKVINT(n+1). If Q_NVKVINTSET = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP1. |
| M_NVKVINT(n) | 7 | FINITE VALUE | Only if Q_NVKVINTSET = 1.Valid between V_NVKVINT(n) and V_NVKVINT(n+1) Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP2. |
| N_ITER | 5 | FINITE VALUE | - |
| Q_NVKVINTSET(k) | 2 | FINITE VALUE | - |
| A_NVP12(k) | 6 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1 |
| A_NVP23(k) | 6 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1 |
| V_NVKVINT(k) | 7 | FINITE VALUE | 0 |
| M_NVKVINT(k) | 7 | FINITE VALUE | Valid between V_NVKVINT(k) and V_NVKVINT(k,1). If Q_NVKVINTSET(k) = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12(k. |
| M_NVKVINT(k) | 7 | FINITE VALUE | Only if Q_NVKVINTSET(k) = . Valid between V_NVKVINT(k) and V_NVKVINT(k,1) Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23(k). |
| N_ITER(k) | 5 | FINITE VALUE | - |
| V_NVKVINT(k,m) | 7 | FINITE VALUE | - |
| M_NVKVINT(k,m) | 7 | FINITE VALUE | Valid between V_NVKVINT(k,m) and V_NVKVINT(k,m+1). If Q_NVKVINTSET(k) = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12(k). |
| M_NVKVINT(k,m) | 7 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1.Valid between V_NVKVINT(k,m) and V_NVKVINT(k,m+1). Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23(k). |
| L_NVKRINT | 5 | FINITE VALUE | 0 m |
| M_NVKRINT | 5 | FINITE VALUE | Valid between L_NVKRINT and L_NVKRINT(1). |
| N_ITER | 5 | FINITE VALUE | - |
| L_NVKRINT(l) | 5 | FINITE VALUE | - |
| M_NVKRINT(l) | 5 | FINITE VALUE | Valid between L_NVKRINT(l) and L_NVKRINT(l+1). |
| M_NVKTINT | 5 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6/ 3/ 0/ 12 / 2 / 1/ 11/ 4/ 13 | SB/ SH/ FS/ LS/ SR/ OS/ NL/ UN/ SN |
| Radio communication session | ESTABLISHED | - |
| National Values | STORED | - |
| ERTMS/ETCS level | 0 / 1 / 2 | LNTC/ L0/ L1 |
| Driver ID | STORED | - |
| Radio Network ID | STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | Both | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |