| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 5060400 (5.6.4: Degraded Situation) Total: 1 Test case(s) |  |
| REF: | Subset-076-5-2-5060400 |
| ISSUE: | Version 3.2.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 0.0.1 23.03.2012 | All | First version/review | DLR |
| Version 2.9.0 31.03.2013 | All | First version/review for Baseline 3 | MULTITEL |
| Version 2.9.1 05.07.2013 | All | Template standardised with a dedicated tool | MULTITEL |
| Version 2.9.2 24.09.2013 | All | Modifications following "Review004_Subset076-5-2-3-5060400_v290_MULTITEL" | MULTITEL |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.2.0 31.03.2017 | All | Baseline 3 1st maintenance release version - Bug fixing | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.6.4: Degraded Situation |  | 5060400 |
| Test Case of Feature | Driver selects “Shunting” and train is at standstill, “Shunting granted by RBC” is received and the system switches to SH mode. No mission is on-going and no order to terminate the session is received from the RBC. |  | 1 |
| Applicable Mode/Level Combinations | L2 / L3 : SB/ PT |  |  |
| Target of Test | The target of this test is to check that: If the driver selects SH mode and the current level is L2 or L3 then after the on–board has asked the RBC for authorisation and after having received the authorisation, the change of mode takes place and as no mission is on-going, the on-board reports the mode change to the RBC, but the RBC send no order to terminate the session. Thus, the on-board repeats its reports a defined number of times with a fixed waiting time and finally terminates the communication session. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-A.3.1 - Repetition of radio messages (i.e. excluding the first sending) |  |
|  |  | Subset-026-A.3.1 - Waiting time before radio message repetition |  |
|  |  | Subset-026-5.6.4.2 |  |
|  |  | Subset-026-5.6.4.3 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that: After having switched to SH, the on-boards: Sends 4 times (first issue + 3 repetitions) a message 136 Sends a message 156 (termination of a communication session) after the 4th message 136 Waits 15s between each message repetition and before sending the message 156 |
| Constraints | No mission is on-going. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6/ 8 | SB/ PT |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill (V_TRAIN=0) | I | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2/ L3 | SB/ PT | The driver selects "SHUNTING" | I | DMI | - | L2/ L3 | SB/ PT | - |
| 2 | L2/ L3 | SB/ PT | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=11) is recorded | O | JRU | - | L2/ L3 | SB/ PT | - |
| 3 | L2/ L3 | SB/ PT | SA-DATA.Request with Euroradio Message "Request for Shunting" (NID_MESSAGE=130; NID_PACKET=0) | O | RTM | If Position Report based on two balise groups, packet 1 is used instead of packet 0 | L2/ L3 | SB/ PT | - |
| 4 | L2/ L3 | SB/ PT | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=130; NID_PACKET=0) is recorded | O | JRU | If Position Report based on two balise groups, packet 1 is used instead of packet 0 | L2/ L3 | SB/ PT | - |
| 5 | L2/ L3 | SB/ PT | SA-DATA.Indication with Euroradio Message "SH Authorised" (NID_MESSAGE=28) is received | I | RTM | - | L2/ L3 | SB/ PT | - |
| 6 | L2/ L3 | SB/ PT | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=28) is recorded | O | JRU | - | L2/ L3 | SB/ PT | - |
| 7 | L2/ L3 | SB/ PT | The Mode symbol "Shunting" is displayed | O | DMI | The system switches to SH mode | L2/ L3 | SH | - |
| 8 | L2/ L3 | SH | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21;Bit16=1) is recorded | O | JRU | MO01 displayed | L2/ L3 | SH | - |
| 9 | L2/ L3 | SH | SA-DATA.Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136; NID_PACKET=0) is transmitted | O | RTM | Report mode change T_TRAIN=t0 If Position Report based on two balise groups, packet 1 is used instead of packet 0 | L2/ L3 | SH | - |
| 10 | L2/ L3 | SH | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=136; NID_PACKET=0) is recorded | O | JRU | If Position Report based on two balise groups, packet 1 is used instead of packet 0 | L2/ L3 | SH | - |
| 11 | L2/ L3 | SH | SA-DATA.Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136; NID_PACKET=0) is transmitted | O | RTM | First repetition T_TRAIN=t0+15s If Position Report based on two balise groups, packet 1 is used instead of packet 0 | L2/ L3 | SH | - |
| 12 | L2/ L3 | SH | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=136; NID_PACKET=0) is recorded | O | JRU | If Position Report based on two balise groups, packet 1 is used instead of packet 0 | L2/ L3 | SH | - |
| 13 | L2/ L3 | SH | SA-DATA.Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136; NID_PACKET=0) is transmitted | O | RTM | Second repetition T_TRAIN=t0+30s If Position Report based on two balise groups, packet 1 is used instead of packet 0 | L2/ L3 | SH | - |
| 14 | L2/ L3 | SH | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=136; NID_PACKET=0) is recorded | O | JRU | If Position Report based on two balise groups, packet 1 is used instead of packet 0 | L2/ L3 | SH | - |
| 15 | L2/ L3 | SH | SA-DATA.Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136; NID_PACKET=0) is transmitted | O | RTM | Third repetition T_TRAIN=t0+45s If Position Report based on two balise groups, packet 1 is used instead of packet 0 | L2/ L3 | SH | - |
| 16 | L2/ L3 | SH | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=136; NID_PACKET=0) is recorded | O | JRU | If Position Report based on two balise groups, packet 1 is used instead of packet 0 | L2/ L3 | SH | - |
| 17 | L2/ L3 | SH | SA-DATA.Request with Euroradio Message "Termination of a communication session" (NID_MESSAGE=156) is transmitted | O | RTM | T_TRAIN=t0+60s | L2/ L3 | SH | - |
| 18 | L2/ L3 | SH | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=156) is recorded | O | JRU | - | L2/ L3 | SH | - |
| 19 | L2/ L3 | SH | SA-DATA.Indication with Euroradio Message "Acknowledgement of termination of a communication session" (NID_MESSAGE=39) is received | I | RTM | - | L2/ L3 | SH | - |
| 20 | L2/ L3 | SH | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=39) is recorded | O | JRU | - | L2/ L3 | SH | - |
| 21 | L2/ L3 | SH | SA-DISCONNECT.Request is transmitted | O | RTM | - | L2/ L3 | SH | - |
| 22 | L2/ L3 | SH | SA-DISCONNECT.Indication is received | I | RTM | - | L2/ L3 | SH | - |

| Step 3: Radio Message 130 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 130 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | Not: 4294967295 (unknown) |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| Step 5: Radio Message 28 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 28 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | Not: 4294967295 (unknown) |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |

| Step 9: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | t0 |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| Step 11: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | t0+15s |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| Step 13: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | t0+30s |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| Step 15: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | t0+45s |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| Step 17: Radio Message 156 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 156 | - |
| L_MESSAGE | 10 | 74 | - |
| T_TRAIN | 32 | FINITE VALUE | t0+60s |
| NID_ENGINE | 24 | FINITE VALUE | - |

| Step 19: Radio Message 39 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 39 | - |
| L_MESSAGE | 10 | 75 | - |
| T_TRAIN | 32 | FINITE VALUE |  |
| M_ACK | 1 | FINITE VALUE | always set to 0 |
| NID_LRBG | 24 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 | SH |
| Radio communication session | NOT ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |