| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 4040800 (4.4.8: SHUNTING) Total: 18 Test case(s) |  |
| REF: | Subset-076-5-2-4040800 |
| ISSUE: | Version 3.2.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 2.9.0 31.03.2013 | All | First version/review | DLR |
| Version 2.9.1 23.09.2013 | All | review | DLR |
| Version 2.9.2 24.01.2014 | TC2 | “Followed by” step changed to “Use_”. End conditions changed accordingly | DLR |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.2.0 31.03.2017 | All | Baseline 3 1st maintenance release version - Bug fixing | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.4.8: SHUNTING |  | 4040800 |
| Test Case of Feature | On-board supervision of shunting speed defined by trackside |  | 1 |
| Applicable Mode/Level Combinations | LNTC: SH L0: SH L1: SH L2: SH L3: SH |  |  |
| Target of Test | The target of test is to check that: • the ERTMS/ETCS on-board equipment supervises the train movements against the shunting mode permitted speed limit defined by trackside. • Service brake is applied when supervised speed is higher than shunting speed defined by trackside. • Service brake is released when speed is lower or equal to shunting speed defined by trackside. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.11.1.2 |  |
|  |  | Subset-026-3.11.7.1 |  |
|  |  | Subset-026-3.11.7.1.1 |  |
|  |  | Subset-026-4.4.8.1.1 |  |
|  |  | Subset-026-4.4.8.1.1 a) |  |
|  |  | Subset-026-4.4.8.1.10 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: • current train speed is DISPLAYED. • maximum shunting speed is DISPLAYED. • permitted speed is DISPLAYED on driver request. • permitted speed displayed on driver request is lower or equal to maximum shunting speed. • Service Brake Indication is DISPLAYED and REMOVED. Check on the JRU that : • Application of Service Brake is RECORDED. • Release of Service Brake is RECORDED. |
| Constraints | • The train has previously received Packet Number 80: “Mode profile” including variable V_MAMODE (required mode related speed) and M_MAMODE (required mode for a part of the MA). • There are no temporary speed restrictions. • The mode has just changed to shunting (SH). The transition to shunting mode is not tested in this feature. • EB will be applied if SB is not available |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 | SH |
| Mode Profile | STORED | Packet Number 80: Mode profile; 0 km/h <= V_MAMODE * 5 km/h <= 600 km/h; M_MAMODE = 01 (Shunting) |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | LNTC / L0 / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Current train speed is DISPLAYED. Maximum shunting speed is DISPLAYED. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Use_FT4070201.37 DMI versus Mode Table - Output | - | - | "Indication of allowed speed in SH after driver request" | N/A | N/A | - |
| 2 | N/A | N/A | Train is moving (V_TRAIN=<V_PERMITTED>) | I | INT | - | N/A | N/A | - |
| 3 | N/A | N/A | Train is moving (V_TRAIN=<V_PERMITTED+10>) | I | INT | - | N/A | N/A | - |
| 4 | N/A | N/A | Use_FT3131030.4 Requirements for Ceiling speed monitoring (Tables 5, 6 and 7) | - | - | Indication of warning due to over-speeding and Service brake intervention due to over-speeding” | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 | SH |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | LNTC / L0 / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.4.8: SHUNTING |  | 4040800 |
| Test Case of Feature | The on-board system supervises the list of expected balises in SH mode, if such list was sent by trackside |  | 2 |
| Applicable Mode/Level Combinations | LNTC: SH L0: SH L1: SH L2: SH L3: SH |  |  |
| Target of Test | The target of this test case is to check that: - the on-board equipment supervises the list of expected balises in SH mode because such list was sent by trackside and - the train pass a balise group, contained in the list, without reaction. - The function Supervise “danger for shunting” information and list of expected balises for shunting is active. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.5.2.1 table - Supervise “danger for shunting” information and list of expected balises for shunting |  |
|  |  | Subset-026-4.4.8.1.1 |  |
|  |  | Subset-026-4.4.8.1.1 b) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that - the train has passed balises that are listed in list of expected balises for shunting (balise telegram is recorded). Check on the TIU that - there is no reaction of on-board equipment during passage of balise that is listed in list of expected balises for shunting (no braking command). Check on the DMI that TRIP is applied if the train passes a balise that is not listed in list of expected balises for shunting (Use_FT4060300.TC34). |
| Constraints | For testing the reaction during passage of balise that is not listed in list of expected balises for shunting The balises passed in the shunting area do not contain the packet 132 “Danger for Shunting information”. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 | SH |
| List of Balises for SH area | STORED | Received with packet 49, must contain all the balises that are to be passed during this test except the last one. |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | LNTC / L0 / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Current train speed is DISPLAYED. Maximum shunting speed is DISPLAYED. | O | DMI | indication of SH mode |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (N_TOTAL=0) received | I | BTM | Train passes balises that are included in the list of expected balises for shunting | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | no reaction was applied | N/A | N/A | - |
| 3 | N/A | N/A | Emergency brake not commanded | O | TIU | passage is allowed | N/A | N/A | - |
| 4 | N/A | N/A | Use_FT4060300.TC34 | - | - | When the train passes over the first balise group (BGa) not included in the list, it switches to TRIP mode and the emergency brakes are applied | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 | TR |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | LNTC / L0 / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| EB is APPLIED | O | TIU | - |
| The mode TR indication, symbol MO04 is DISPLAYED. Text message “SH stop order“ is DISPLAYED. The EB application indication is DISPLAYED. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.4.8: SHUNTING |  | 4040800 |
| Test Case of Feature | The on-board system supervises the list of expected balises in SH mode, if an empty list was sent by trackside |  | 3 |
| Applicable Mode/Level Combinations | LNTC: SH L0: SH L1: SH L2: SH L3: SH |  |  |
| Target of Test | The target of this test case is to check that: - the on-board equipment supervises the received empty list of expected balises in SH mode correctly and - the train is not allowed to pass any balise group. - The function Supervise “danger for shunting” information and list of expected balises for shunting is active. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.5.2.1 table - Supervise “Stop if in SR” information and list of expected balises for Staff Responsible |  |
|  |  | Subset-026-4.4.8.1.1 |  |
|  |  | Subset-026-4.4.8.1.1 b) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the TIU that - EB is applied of on-board equipment during passage of any balise. Check on the DMI that - TRIP is applied if the train passes a balise (Use_FT4060300.TC34). |
| Constraints | For testing the reaction during passage of balise that is not listed in list of expected balises for shunting The balises passed in the shunting area do not contain the packet 132 “Danger for Shunting information”. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 | SH |
| List of Balises for SH area | STORED | The list is empty |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | LNTC / L0 / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Current train speed is DISPLAYED. Maximum shunting speed is DISPLAYED. | O | DMI | indication of SH mode |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (N_TOTAL=0) received | I | BTM | - | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | Use_FT4060300.TC34 | - | - | transition to TR [52] | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 | TR |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | LNTC / L0 / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Emergency brake commanded | O | TIU | - |
| The Mode symbol "Trip" is displayed. The Status symbol "Emergency Brake intervention" is displayed. | O | DMI | MO04 ST01 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.4.8: SHUNTING |  | 4040800 |
| Test Case of Feature | The train is in shunting mode and overpasses a balise without the packet number 132 (danger for shunting information). |  | 4 |
| Applicable Mode/Level Combinations | LNTC: SH L0: SH L1: SH L2: SH L3: SH |  |  |
| Target of Test | The target of this test is to check that: • The train overpasses a balise without the packet number 132 (danger for shunting information), the identity of the overpassed balise is in the list of expected balises to SH mode and the ETCS language is compatibel (only required for L1, L2, L3). • Then the train will not be tripped. • The function Supervise “danger for shunting” information and list of expected balises for shunting is active. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.5.2.1 table - Supervise “Stop if in SR” information and list of expected balises for Staff Responsible |  |
|  |  | Subset-026-4.4.8.1.1 |  |
|  |  | Subset-026-4.4.8.1.1 c) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : • The mode displayed is not TR Check on the JRU that: • No transition to TR is recorded |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 | SH |
| Mode Profile | STORED | Received with packet 49, must contain all the balises that are to be passed during this test except the last one. |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | LNTC / L0 / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Current train speed is DISPLAYED. Maximum shunting speed is DISPLAYED. | O | DMI | indication of SH mode |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (N_TOTAL=0) received | I | BTM | The train overpasses a balise • without the packet number 132 (danger for shunting information), • the identity of the overpassed balise is in the list of expected balises to SH mode and • the ETCS language is compatibel (only required for L1, L2, L3). | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | NOT The Mode symbol "Trip" is displayed | O | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | NOT DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit19=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | NOT GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=7) is recorded | O | JRU | - | N/A | N/A | - |
| 6 | N/A | N/A | NOT Emergency brake commanded | O | TIU | - | N/A | N/A | - |
| 7 | N/A | N/A | NOT EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 | SH |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | LNTC / L0 / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 5

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.4.8: SHUNTING |  | 4040800 |
| Test Case of Feature | The train receives an order to switch immediately from L0 to L1 at the current position. As the on-board equipment is running in SH mode the system has not to switch to L1. |  | 5 |
| Applicable Mode/Level Combinations | L0: SH, PS |  |  |
| Target of Test | The target of this test is to check that: • The system does not switches to level 1 when the order to switch immediately to Level 1 is received on-board. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.4.8.1.5 |  |
|  |  | Subset-026-4.4.20.1.11 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: • Level 0 indication is DISPLAYED. Check on the JRU that: • Packet 41: “Level transition order” is RECORDED with M_LEVELTR = 2, D_LEVELTR = 0 or = 32767. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 / 15 | SH / PS |
| ERTMS/ETCS level | 0 | L0 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Level 0 is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (N_TOTAL=0) containing packet 41 (D_LEVELTR= 0; M_LEVELTR=2) is received | I | BTM | - | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | The On-board equipment does not switch to Level 1. as long as the train is in SH or PS mode. | N/A | N/A | - |
| 3 | N/A | N/A | NOT The Level symbol "Level 1" is displayed | O | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | NOT DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit03=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | NOT Any message (NID_MESSAGE_JRU=ALL; M_LEVEL=2) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 41 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | 0 | - |
| M_LEVELTR | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTR = 1 (NTC). |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | 2 | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 / 15 | SH / PS |
| ERTMS/ETCS level | 0 | L0 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| LEVEL 0 is DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 6

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.4.8: SHUNTING |  | 4040800 |
| Test Case of Feature | The train receives an order to switch immediately from L0 to L2 at the current position. As the on-board equipment is running in SH mode the system has not to switch to L2. |  | 6 |
| Applicable Mode/Level Combinations | L0: SH, PS |  |  |
| Target of Test | The target of this test is to check that: • The system does not switches to level 2 when the order to switch immediately to Level 2 is received on-board. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.4.8.1.5 |  |
|  |  | Subset-026-4.4.20.1.11 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: • Level 0 indication is DISPLAYED. Check on the JRU that: • Packet 41: “Level transition order” is RECORDED with M_LEVELTR = 3, D_LEVELTR = 0 or = 32767. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 / 15 | SH / PS |
| ERTMS/ETCS level | 0 | L0 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Level 0 is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (N_TOTAL=0) containing packet 41 (D_LEVELTR= 0; M_LEVELTR=3) is received | I | BTM | - | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | The On-board equipment does not switch to Level 2. as long as the train is in SH or PS mode. | N/A | N/A | - |
| 3 | N/A | N/A | NOT The Level symbol "Level 2" is displayed | O | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | NOT DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit04=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | NOT Any message (NID_MESSAGE_JRU=ALL; M_LEVEL=3) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 41 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | 0 | - |
| M_LEVELTR | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTR = 1 (NTC). |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | 3 | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 / 15 | SH / PS |
| ERTMS/ETCS level | 0 | L0 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| LEVEL 0 is DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 7

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.4.8: SHUNTING |  | 4040800 |
| Test Case of Feature | The train receives an order to switch immediately from L1 to L0 at the current position. As the on-board equipment is running in SH mode the system has not to switch to L0. |  | 7 |
| Applicable Mode/Level Combinations | L1: SH, PS |  |  |
| Target of Test | The target of this test is to check that: • The system does not switches to level 0 when the order to switch immediately to Level 0 is received on-board. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.4.8.1.5 |  |
|  |  | Subset-026-4.4.20.1.11 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: • Level 1 indication is DISPLAYED. Check on the JRU that: • Packet 41: “Level transition order” is RECORDED with M_LEVELTR = 0, D_LEVELTR = 0 or = 32767. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 / 15 | SH / PS |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Level 1 is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (N_TOTAL=0) containing packet 41 (D_LEVELTR= 0; M_LEVELTR=0) is received | I | BTM | - | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | The On-board equipment does not switch to Level 0. as long as the train is in SH or PS mode. | N/A | N/A | - |
| 3 | N/A | N/A | NOT The Level symbol "Level 0" is displayed | O | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | NOT DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit01=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | NOT Any message (NID_MESSAGE_JRU=ALL; M_LEVEL=0) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 41 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | 0 | - |
| M_LEVELTR | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTR = 1 (NTC). |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | 0 | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 / 15 | SH / PS |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| LEVEL 1 is DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 8

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.4.8: SHUNTING |  | 4040800 |
| Test Case of Feature | The train receives an order to switch immediately from L1 to L2 at the current position. As the on-board equipment is running in SH mode the system has not to switch to L2. |  | 8 |
| Applicable Mode/Level Combinations | L1: SH, PS |  |  |
| Target of Test | The target of this test is to check that: • The system does not switches to level 2 when the order to switch immediately to Level 2 is received on-board. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.4.8.1.5 |  |
|  |  | Subset-026-4.4.20.1.11 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: • Level 1 indication is DISPLAYED. Check on the JRU that: • Packet 41: “Level transition order” is RECORDED with M_LEVELTR = 3, D_LEVELTR = 0 or = 32767. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 / 15 | SH / PS |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Level 1 is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (N_TOTAL=0) containing packet 41 (D_LEVELTR= 0; M_LEVELTR=3) is received | I | BTM | - | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | The On-board equipment does not switch to Level 2. as long as the train is in SH or PS mode. | N/A | N/A | - |
| 3 | N/A | N/A | NOT The Level symbol "Level 2" is displayed | O | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | NOT DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit04=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | NOT Any message (NID_MESSAGE_JRU=ALL; M_LEVEL=3) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 41 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | 0 | - |
| M_LEVELTR | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTR = 1 (NTC). |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | 3 | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 / 15 | SH / PS |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| LEVEL 1 is DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 9

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.4.8: SHUNTING |  | 4040800 |
| Test Case of Feature | The train receives an order to switch immediately from L2 to L0 at the current position. As the on-board equipment is running in SH mode the system has not to switch to L0. |  | 9 |
| Applicable Mode/Level Combinations | L2: SH, PS |  |  |
| Target of Test | The target of this test is to check that: • The system does not switches to level 0 when the order to switch immediately to Level 0 is received on-board. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.4.8.1.5 |  |
|  |  | Subset-026-4.4.20.1.11 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: • Level 2 indication is DISPLAYED. Check on the JRU that: • Packet 41: “Level transition order” is RECORDED with M_LEVELTR = 0, D_LEVELTR = 0 or = 32767. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 / 15 | SH / PS |
| ERTMS/ETCS level | 3 | L2 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Level 2 is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (N_TOTAL=0) containing packet 41 (D_LEVELTR= 0; M_LEVELTR=0) is received | I | BTM | - | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | The On-board equipment does not switch to Level 0. as long as the train is in SH or PS mode. | N/A | N/A | - |
| 3 | N/A | N/A | NOT The Level symbol "Level 0" is displayed | O | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | NOT DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit01=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | NOT Any message (NID_MESSAGE_JRU=ALL; M_LEVEL=0) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 41 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | 0 | - |
| M_LEVELTR | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTR = 1 (NTC). |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | 0 | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 / 15 | SH / PS |
| ERTMS/ETCS level | 3 | L2 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| LEVEL 2 is DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 10

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.4.8: SHUNTING |  | 4040800 |
| Test Case of Feature | The train receives an order to switch immediately from L2 to L1 at the current position. As the on-board equipment is running in SH mode the system has not to switch to L1 |  | 10 |
| Applicable Mode/Level Combinations | L2: SH, PS |  |  |
| Target of Test | The target of this test is to check that: • The system does not switches to level 1 when the order to switch immediately to Level 1 is received on-board. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.4.8.1.5 |  |
|  |  | Subset-026-4.4.20.1.11 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: • Level 2 indication is DISPLAYED. Check on the JRU that: • Packet 41: “Level transition order” is RECORDED with M_LEVELTR = 2, D_LEVELTR = 0 or = 32767. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 / 15 | SH / PS |
| ERTMS/ETCS level | 3 | L2 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Level 2 is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (N_TOTAL=0) containing packet 41 (D_LEVELTR= 0; M_LEVELTR=2) is received | I | BTM | - | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | The On-board equipment does not switch to Level 1. as long as the train is in SH or PS mode. | N/A | N/A | - |
| 3 | N/A | N/A | NOT The Level symbol "Level 1" is displayed | O | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | NOT DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit03=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | NOT Any message (NID_MESSAGE_JRU=ALL; M_LEVEL=2) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 41 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | 0 | - |
| M_LEVELTR | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTR = 1 (NTC). |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | 2 | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 / 15 | SH / PS |
| ERTMS/ETCS level | 3 | L2 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| LEVEL 2 is DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 11

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.4.8: SHUNTING |  | 4040800 |
| Test Case of Feature | The OBU receives a communication session establishment order, the ERTMS/ETCS on-board in Shunting mode does not establish the communication session, but stores the RBC ID/phone number. |  | 11 |
| Applicable Mode/Level Combinations | LNTC: SH L0: SH L1: SH |  |  |
| Target of Test | The target of this test is to check that when the OBU receives a communication session establishment order • does not establish the communication session and • stores the RBC ID/phone number. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX / INECO / DLR / MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.4.8.1.5.1 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: • When the driver selects Level 2/3, the RBC contact Window is displayed Check on the JRU that: • Packet Number 42 - Session Management is RECORDED Check on the RTM that: • A communication session is NOT established |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 | SH |
| Level Transition Order | NOT STORED | Only for level M_LEVEL= 3 or 4 |
| Train Data | VALID | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |
| Radio Network ID | VALID | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | Both | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | NTC / 0 / 1 | SH | One Balise Group message (N_TOTAL=0) containing packet 42 (Q_RBC = 1) is received | I | BTM | RBC Data has to be differend from any currently stored values. | NTC / 0 / 1 | SH | - |
| 2 | NTC / 0 / 1 | SH | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | NTC / 0 / 1 | SH | - |
| 3 | NTC / 0 / 1 | SH | NOT SA-CONNECT.Request is transmitted | O | RTM | Train DOES NOT try to set up a new communication session. The order to contact the RBC is ignored because the current mode is SH. However, RBC ID/phone number is stored. | NTC / 0 / 1 | SH | - |
| 4 | NTC / 0 / 1 | SH | NOT SA-DATA.Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted | O | RTM | - | NTC / 0 / 1 | SH | - |
| 5 | NTC / 0 / 1 | SH | NOT SA.DATA.Request with Euroradio Message "Session Established" (NID_Message = 159) is transmitted | O | RTM | - | NTC / 0 / 1 | SH | - |
| 6 | NTC / 0 / 1 | SH | Train is at standstill (V_TRAIN=0) | I | INT | - | NTC / 0 / 1 | SH | - |
| 7 | NTC / 0 / 1 | SH | The driver selects "EXIT SHUNTING" | I | DMI | - | NTC / 0 / 1 | SB | - |
| 8 | NTC / 0 / 1 | SB | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=6) is recorded | O | JRU | - | NTC / 0 / 1 | SB | - |
| 9 | NTC / 0 / 1 | SB | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | Exit of Shunting selected | NTC / 0 / 1 | SB | - |
| 10 | NTC / 0 / 1 | SB | Use_FT5040300.7,8 "Request for Driver ID in scope of Start of Mission" | - | - | - | NTC / 0 / 1 | SB | - |
| 11 | NTC / 0 / 1 | SB | The driver selects "Level 2" and confirms "RBC Data" | I | DMI | The NID_RBC and NID_RADIO are equal to the variables of packet 42 in step 1 | 2 | SB | - |
| 12 | 2 | SB | Any message (NID_MESSAGE_JRU=ALL; M_LEVEL=3) is recorded | O | JRU | - | 2 | SB | - |
| 13 | 2 | SB | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | - | 2 | SB | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 42 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_RBC | 1 | 1 | - |
| NID_C | 10 | FINITE VALUE | RBC ETCS identity : _x005F_x000D_NID_C not relevant if NID_RBC has value "Contact last known RBC". |
| NID_RBC | 14 | FINITE VALUE | - |
| NID_RADIO | 64 | FINITE VALUE | Not relevant if NID_RBC has value "Contact last known RBC". |
| Q_SLEEPSESSION | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L2 |
| Radio Network ID | VALID | - |
| RBC ID/Phone Number | VALID | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 12

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.4.8: SHUNTING |  | 4040800 |
| Test Case of Feature | When in Shunting mode, the ERTMS/ETCS on-board shall not manage RBC-RBC handover, except for storing the RBC ID/phone number given at the RBC/RBC border. |  | 12 |
| Applicable Mode/Level Combinations | L2: SH |  |  |
| Target of Test | The target of this test is to check that when the OBU receives an RBC transition order at the RBC/RBC border • does not manage any handover • stores the RBC ID/phone number. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX / INECO / DLR / MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.4.8.1.5.2 |  |
|  |  | Subset-026-4.8.4.2 [8] |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: • When the driver selects Level 2/3, the RBC contact Window is displayed Check on the JRU that: • Packet Number 42 - Session Management is RECORDED Check on the RTM that: Train DOES NOT try to set up a new communication session with RBCy while in Shunting mode |
| Constraints | The on-board previously had a connection with RBCx before entering Shunting mode. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 | SH |
| Radio communication session | NOT ESTABLISHED | - |
| Train Position | VALID | - |
| ERTMS/ETCS level | 3 | L2 |
| Radio Network ID | VALID | . |
| RBC ID/Phone Number | VALID | RBCx |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | Both | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | SH | One Balise Group message (N_TOTAL=0) containing packet 131 (D_RBCTR=0) is received | I | BTM | The NID_RBC and NID_RADIO of RBCy differ from those of RBCx | N/A | SH | - |
| 2 | N/A | SH | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | SH | - |
| 3 | N/A | SH | NOT SA-CONNECT.Request is transmitted | O | RTM | Train DOES NOT try to establish a communication session with RBCy | N/A | SH | - |
| 4 | N/A | SH | NOT SA.DATA.Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted | O | RTM | - | N/A | SH | - |
| 5 | N/A | SH | NOT SA.DATA.Request with Euroradio Message "Session Established" (NID_Message = 159) is transmitted | O | RTM | - | N/A | SH | - |
| 6 | N/A | SH | Train is at standstill (V_TRAIN=0) | I | INT | - | N/A | SH | - |
| 7 | N/A | SH | The driver selects "EXIT SHUNTING" | I | DMI | - | N/A | SB | - |
| 8 | N/A | SB | Use_FT5040300.7,8 "Request for Driver ID in scope of Start of Mission" | - | - | TC to use depends on state of driver id | N/A | SB | - |
| 9 | N/A | SB | Use_FT3050300.28 "A communication session initiated by the on-board unit is established successfully" | - | - | Check that the connection is set up to RBCy with data from packet 131, step 1. Check NID_RBC and NID_RADIO | N/A | SB | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 131 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_RBCTR | 15 | 0 | - |
| NID_C | 10 | FINITE VALUE | Accepting RBC identity. |
| NID_RBC | 14 | FINITE VALUE | - |
| NID_RADIO | 64 | FINITE VALUE | Accepting RBC radio subscriber number. |
| Q_SLEEPSESSION | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Radio communication session | NOT ESTABLISHED | - |
| ERTMS/ETCS level | 3 | L2 |
| Radio Network ID | STORED and VALID | - |
| RBC ID/Phone Number | STORED and VALID | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | Both | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 13

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.4.8: SHUNTING |  | 4040800 |
| Test Case of Feature | The train receives an conditional order to switch from L0 to L1 at the current position. Since the on-board equipment is running in SH mode the system does not to switch to L1. |  | 13 |
| Applicable Mode/Level Combinations | L0: SH, PS |  |  |
| Target of Test | The target of this test is to check that: • The system does not switch to level 1 when the order to switch immediately to Level 1 is received on-board. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.4.8.1.5 |  |
|  |  | Subset-026-4.4.20.1.11 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: • Level 0 indication is DISPLAYED. Check on the JRU that: • Packet 46: “Conditional level transition order” is RECORDED with M_LEVELTR = 2. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 / 15 | SH / PS |
| ERTMS/ETCS level | 0 | L0 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Level 0 is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (N_TOTAL=0) containing packet 46 (M_LEVELTR=2) is received | I | BTM | - | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | The On-board equipment does not switch to Level 1. as long as the train is in SH or PS mode. | N/A | N/A | - |
| 3 | N/A | N/A | NOT The Level symbol "Level 1" is displayed | O | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | NOT DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit03=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | NOT Any message (NID_MESSAGE_JRU=ALL; M_LEVEL=2) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 46 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| M_LEVELTR | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTR = 1 (NTC). |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | 2 | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 / 15 | SH / PS |
| ERTMS/ETCS level | 0 | L0 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| LEVEL 0 is DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 14

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.4.8: SHUNTING |  | 4040800 |
| Test Case of Feature | The train receives a conditional order to switch immediately from L0 to L2 at the current position. As the on-board equipment is running in SH mode the system has not to switch to L2. |  | 14 |
| Applicable Mode/Level Combinations | L0: SH, PS |  |  |
| Target of Test | The target of this test is to check that: • The system does not switches to level 2 when the order to switch immediately to Level 2 is received on-board. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.4.8.1.5 |  |
|  |  | Subset-026-4.4.20.1.11 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: • Level 0 indication is DISPLAYED. Check on the JRU that: • Packet 46: “Conditional Level transition order” is RECORDED with M_LEVELTR = 3. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 / 15 | SH / PS |
| ERTMS/ETCS level | 0 | L0 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Level 0 is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (N_TOTAL=0) containing packet 46 (M_LEVELTR=3) is received | I | BTM | - | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | The On-board equipment does not switch to Level 1. as long as the train is in SH or PS mode. | N/A | N/A | - |
| 3 | N/A | N/A | NOT The Level symbol "Level 2" is displayed | O | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | NOT DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit04=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | NOT Any message (NID_MESSAGE_JRU=ALL; M_LEVEL=3) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 46 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| M_LEVELTR | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTR = 1 (NTC). |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | 3 | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 / 15 | SH / PS |
| ERTMS/ETCS level | 0 | L0 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| LEVEL 0 is DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 15

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.4.8: SHUNTING |  | 4040800 |
| Test Case of Feature | The train receives a conditional order to switch immediately from L1 to L0 at the current position. As the on-board equipment is running in SH or PS mode the system has not to switch to L0. |  | 15 |
| Applicable Mode/Level Combinations | L1: SH, PS |  |  |
| Target of Test | The target of this test is to check that: • The system does not switches to level 0 when the order to switch immediately to Level 0 is received on-board. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.4.8.1.5 |  |
|  |  | Subset-026-4.4.20.1.11 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: • Level 1 indication is DISPLAYED. Check on the JRU that: • Packet 46: “Conditional level transition order” is RECORDED with M_LEVELTR = 0. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 / 15 | SH / PS |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Level 1 is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (N_TOTAL=0) containing packet 46 (M_LEVELTR=0) is received | I | BTM | - | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | The On-board equipment does not switch to Level 1. as long as the train is in SH or PS mode. | N/A | N/A | - |
| 3 | N/A | N/A | NOT The Level symbol "Level 0" is displayed | O | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | NOT DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit01=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | NOT Any message (NID_MESSAGE_JRU=ALL; M_LEVEL=0) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 46 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| M_LEVELTR | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTR = 1 (NTC). |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | 0 | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 / 15 | SH / PS |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| LEVEL 1 is DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 16

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.4.8: SHUNTING |  | 4040800 |
| Test Case of Feature | The train receives a conditional order to switch immediately from L1 to L2 at the current position. As the on-board equipment is running in SH mode the system has not to switch to L2. |  | 16 |
| Applicable Mode/Level Combinations | L1: SH, PS |  |  |
| Target of Test | The target of this test is to check that: • The system does not switches to level 2 when the order to switch immediately to Level 2 is received on-board. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.4.8.1.5 |  |
|  |  | Subset-026-4.4.20.1.11 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: • Level 1 indication is DISPLAYED. Check on the JRU that: • Packet 46: “Conditional Level transition order” is RECORDED with M_LEVELTR = 3. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 / 15 | SH / PS |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Level 1 is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (N_TOTAL=0) containing packet 46 (M_LEVELTR=3) is received | I | BTM | - | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | The On-board equipment does not switch to Level 2 as long as the train is in SH or PS mode. | N/A | N/A | - |
| 3 | N/A | N/A | NOT The Level symbol "Level 2" is displayed | O | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | NOT DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit04=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | NOT Any message (NID_MESSAGE_JRU=ALL; M_LEVEL=3) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 46 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| M_LEVELTR | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTR = 1 (NTC). |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | 3 | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 / 15 | SH / PS |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| LEVEL 1 is DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 17

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.4.8: SHUNTING |  | 4040800 |
| Test Case of Feature | The train receives a conditional order to switch immediately from L2 to L0 at the current position. As the on-board equipment is running in SH or PS mode the system has not to switch to L0. |  | 17 |
| Applicable Mode/Level Combinations | L2: SH, PS |  |  |
| Target of Test | The target of this test is to check that: • The system does not switches to level 0 when the order to switch immediately to Level 0 is received on-board. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.4.8.1.5 |  |
|  |  | Subset-026-4.4.20.1.11 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: • Level 2 indication is DISPLAYED. Check on the JRU that: • Packet 46: “Conditional level transition order” is RECORDED with M_LEVELTR = 0. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 / 15 | SH / PS |
| ERTMS/ETCS level | 3 | L2 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Level 2 is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (N_TOTAL=0) containing packet 46 (M_LEVELTR=0) is received | I | BTM | - | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | The On-board equipment does not switch to Level 0 as long as the train is in SH or PS mode. | N/A | N/A | - |
| 3 | N/A | N/A | NOT The Level symbol "Level 0" is displayed | O | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | NOT DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit01=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | NOT Any message (NID_MESSAGE_JRU=ALL; M_LEVEL=0) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 46 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| M_LEVELTR | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTR = 1 (NTC). |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | 0 | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 / 15 | SH / PS |
| ERTMS/ETCS level | 3 | L2 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| LEVEL 2 is DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 18

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.4.8: SHUNTING |  | 4040800 |
| Test Case of Feature | The train receives a conditional order to switch immediately from L2 to L1 at the current position. As the on-board equipment is running in SH or PS mode the system has not to switch to L1 |  | 18 |
| Applicable Mode/Level Combinations | L2: SH, PS |  |  |
| Target of Test | The target of this test is to check that: • The system does not switches to level 1 when the order to switch immediately to Level 1 is received on-board. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.4.8.1.5 |  |
|  |  | Subset-026-4.4.20.1.11 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: • Level 2 indication is DISPLAYED. Check on the JRU that: • Packet 46: “Conditional Level transition order” is RECORDED with M_LEVELTR = 2. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 / 15 | SH / PS |
| ERTMS/ETCS level | 3 | L2 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Level 2 is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (N_TOTAL=0) containing packet 46 (M_LEVELTR=2) is received | I | BTM | - | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | The On-board equipment does not switch to Level 1. as long as the train is in SH or PS mode. | N/A | N/A | - |
| 3 | N/A | N/A | NOT The Level symbol "Level 1" is displayed | O | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | NOT DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit03=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | NOT Any message (NID_MESSAGE_JRU=ALL; M_LEVEL=2) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 46 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| M_LEVELTR | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTR = 1 (NTC). |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | 2 | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 / 15 | SH / PS |
| ERTMS/ETCS level | 3 | L2 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| LEVEL 2 is DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |