| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 4080451 (4.8.4: Accepted Information depending on the modes - SH authorised + (optional) List of Balises for SH Area)) Total: 2 Test case(s) |  |
| REF: | Subset-076-5-2-4080451 |
| ISSUE: | Version 3.2.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 0.0.1 23.03.2012 | All | First version/review | CEDEX |
| Version 2.9.0 31.03.2013 | All | First version/review for Baseline 3 | CEDEX |
| Version 2.9.1 21.10.2013 | All | Requirements correction/Review for Baseline 3 | CEDEX |
| Version 2.9.2 10.12.2013 | TC 2 | Editorial change (packet 255 has been added in the BG) | CEDEX |
| Version 2.9.3 23.12.2013 | All | Editorial changes. | CEDEX |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.2.0 31.03.2017 | All | Baseline 3 1st maintenance release version - Bug fixing | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - SH authorised + (optional) List of Balises for SH Area) |  | 4080451 |
| Test Case of Feature | Transmission of list of balises for SH area from RBC (independent of an empty or not empty list) in a “SH authorised” message. All the information is rejected because train data has been sent to the RBC and the acknowledgement of train data has not been received. |  | 1 |
| Applicable Mode/Level Combinations | L2: FS, LS, OS, SR, SB, PT L3: FS, LS, OS, SR, SB, PT |  |  |
| Target of Test | The target of this test is to check that: When the on-board receives a “SH authorised” message with a “List of Balises for SH mode” packet, both the message and the packet are rejected, since no acknowledgement of train data has been received before |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.4.2 SH authorised + (optional) List of Balises for SH Area |  |
|  |  | Subset-026-4.8.3.1.1 SH authorised + (optional) List of Balises for SH area |  |
|  |  | Subset-026-4.8.4.2 [1] |  |
|  |  | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-4.8.3.1.1 Exception [3] |  |
|  |  | Subset-026-7.4.2.12 Packet Number 49: List of Balises for SH area |  |
|  |  | Subset-026-8.6.2 Message 130: Request for shunting |  |
|  |  | Subset-026-8.7.11 Message 28: SH Authorised |  |
|  |  | Subset-026-8.6.4 Message 136: Train Position Report |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: No indication of SH mode is DISPLAYED after receiving message 28 Check on the JRU that: No transition to SH mode is recorded after receiving message 28 The messages 130, 28, transition to SH mode and 136 are RECORDED Check on the RTM that: The messages 130 and 28 are sent and received |
| Constraints | The train is at standstill In PT mode the information “Recognition of Exit of TR mode” has been received previously A cab needs to be active in SB Mode |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 8 / 12 | FS / OS / SR / SB / PT / LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Radio Network ID | STORED | - |
| RBC ID/Phone Number | STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | Both | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN=0 | O | INT | Train is at standstill |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2/ L3 | FS / LS / OS / SR/ PT / SB | SA-DATA.Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) containing packet 11 is transmitted | O | RTM | - | L2/ L3 | FS / LS / OS / SR/ PT / SB | - |
| 2 | L2/ L3 | FS / LS / OS / SR/ PT / SB | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=129) is recorded | O | JRU | - | L2/ L3 | FS / LS / OS / SR/ PT / SB | - |
| 3 | L2/ L3 | FS / LS / OS / SR/ PT / SB | The driver selects "SHUNTING" | I | DMI | - | L2/ L3 | FS / LS / OS / SR/ PT / SB | - |
| 4 | L2/ L3 | FS / LS / OS / SR/ PT / SB | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | - | L2/ L3 | FS / LS / OS / SR/ PT / SB | - |
| 5 | L2/ L3 | FS / LS / OS / SR/ PT / SB | SA-DATA.Request with Euroradio Message "Request for Shunting" (NID_MESSAGE=130) is transmitted | O | RTM | - | L2/ L3 | FS / LS / OS / SR/ PT / SB | - |
| 6 | L2/ L3 | FS / LS / OS / SR/ PT / SB | MESSAGE TO RBC (NID_MESSAGE_JRU=10;NID_MESSAGE=130) is recorded | O | JRU | - | L2/ L3 | FS / LS / OS / SR/ PT / SB | - |
| 7 | L2/ L3 | FS / LS / OS / SR/ PT / SB | SA-DATA.Indication with Euroradio Message "SH Authorised" (NID_MESSAGE=28) containing packet 49 is received | I | RTM | The list of balises of the packet 49 will be empty | L2/ L3 | FS / LS / OS / SR/ PT / SB | - |
| 8 | L2/ L3 | FS / LS / OS / SR/ PT / SB | MESSAGE FROM RBC (NID_MESSAGE_JRU=9;NID_MESSAGE=28) is recorded | O | JRU | - | L2/ L3 | FS / LS / OS / SR/ PT / SB | - |
| 9 | L2/ L3 | FS / LS / OS / SR/ PT / SB | NOT The Mode symbol "Shunting" is displayed | O | DMI | The message SH authorised has been rejected. | L2/ L3 | FS / LS / OS / SR/ PT / SB | - |
| 10 | L2/ L3 | FS / LS / OS / SR/ PT / SB | NOT GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=3) is recorded | O | JRU | The message "SH authorised" has been rejected. | L2/ L3 | FS / LS / OS / SR/ PT / SB | - |
| 11 | L2/ L3 | FS / LS / OS / SR/ PT / SB | SA-DATA.Indication with Euroradio Message "Acknowledgement of Train Data" (NID_MESSAGE=8) is received | I | RTM | - | L2/ L3 | FS / LS / OS / SR/ PT / SB | - |
| 12 | L2/ L3 | FS / LS / OS / SR/ PT / SB | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=8) is recorded | O | JRU | - | L2/ L3 | FS / LS / OS / SR/ PT / SB | - |
| 13 | L2/ L3 | FS / LS / OS / SR/ PT / SB | Use_FT3050500.11 | - | - | The first 5 steps of this TC11 will have to be removed when using it in a TS, but the rest of it will be used to terminate the communication session. | L2/ L3 | FS / LS / OS / SR/ PT / SB | - |
| 14 | L2/ L3 | FS / LS / OS / SR/ PT / SB | The driver selects "Level 1" | I | DMI | This is done to test later the rejection of the list of balises for SH mode. | L1 | FS / LS / OS / SR/ PT / SB | - |
| 15 | L1 | FS / LS / OS / SR/ PT / SB | The Level symbol "Level 1" is displayed | O | DMI | - | L1 | FS / LS / OS / SR/ PT / SB | - |
| 16 | L1 | FS / LS / OS / SR/ PT / SB | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=2) is recorded | O | JRU | - | L1 | FS / LS / OS / SR/ PT / SB | - |
| 17 | L1 | FS / LS / OS / SR/ PT / SB | The on-board changes to SH mode: Use_FT5060200.5 if there is an ongoing mission. Use_FT5060200.2 if there is not an ongoing mission. | - | - | Use this step only if there is an ongoing mission | L1 | FS / LS / OS / SR/ PT / SB | - |
| 18 | L1 | SH | The Mode symbol "Shunting" is displayed | O | DMI | - | L1 | SH | - |
| 19 | L1 | SH | One Balise Group message is received | I | BTM | NID_C+NID_BG can be anything if the packet 49 was received empty. NID_C+NID_BG will not coincide with any of the values given in packet 49, if it was not sent empty. | L1 | SH | - |
| 20 | L1 | SH | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L1 | SH | - |
| 21 | L1 | SH | Not emergency brake commanded | O | TIU | This way the train not changing to TR mode is tested | L1 | SH | - |

| Step 1: Radio Message 129 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 129 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 11 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| NC_CDTRAIN | 4 | FINITE VALUE | - |
| NC_TRAIN | 15 | FINITE VALUE | - |
| L_TRAIN | 12 | FINITE VALUE | - |
| V_MAXTRAIN | 7 | FINITE VALUE | - |
| M_LOADINGGAUGE | 8 | FINITE VALUE | - |
| M_AXLELOADCAT | 7 | FINITE VALUE | - |
| M_AIRTIGHT | 2 | FINITE VALUE | - |
| N_AXLE | 10 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_VOLTAGE(k) | 4 | FINITE VALUE | Identity of the traction system. |
| NID_CTRACTION(k) | 10 | FINITE VALUE | NID_CTRACTION(k) given only if M_VOLTAGE(k) ≠ 0. |
| N_ITER | 5 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | Type of National System available. |

| Step 5: Radio Message 130 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 130 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |

| Step 7: Radio Message 28 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 28 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_PACKET | 8 | 49 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | - |

| Step 11: Radio Message 8 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 8 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 | SH |
| List of Balises for SH area | NOT STORED | - |
| Train Data | INVALID | - |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | Both | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Mode symbol "Shunting" is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - SH authorised + (optional) List of Balises for SH Area) |  | 4080451 |
| Test Case of Feature | Transmission of list of balises for SH area from RBC (independent of an empty or not empty list) in a “SH Authorised” message |  | 2 |
| Applicable Mode/Level Combinations | L2 : FS, LS, OS, SR, SB, PT L3 : FS, LS, OS, SR, SB, PT |  |  |
| Target of Test | The target of this test is to check the correct working, that is: A list of balises or an empty list is received and accepted by the on-board system. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.3.1.1 SH authorised + (optional) List of Balises for SH area |  |
|  |  | Subset-026-4.8.4.2 SH authorised + (optional) List of Balises for SH area |  |
|  |  | Subset-026-4.8.4.2 [1] |  |
|  |  | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-8.7.11 Message 28: SH Authorised |  |
|  |  | Subset-026-7.4.2.12 Packet Number 49: List of Balises for SH area |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on JRU that : A message sent from RBC containing a list of balises (or empty list) has been received and is recorded. Check that : The on-board accepts the message using the appropriate TCs at the end |
| Constraints | In PT mode the information “Recognition of Exit of TR mode” have been received previously A cab needs to be active (if in SB Mode) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 8 / 12 | FS / OS / SR / SB / PT / LS |
| Radio communication session | ESTABLISHED | - |
| List of Balises for SH area | NOT STORED | - |
| ERTMS/ETCS level | 3/ 4 | L2 / L3 |
| Radio Network ID | STORED | - |
| RBC ID/Phone Number | STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | Both | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2/ L3 | FS / LS / OS / SR / SB / PT | The driver selects "SHUNTING" | I | DMI | - | L2/ L3 | FS / LS / OS / SR / SB / PT | - |
| 2 | L2/ L3 | FS / LS / OS / SR / SB / PT | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | - | L2/ L3 | FS / LS / OS / SR / SB / PT | - |
| 3 | L2/ L3 | FS / LS / OS / SR / SB / PT | SA-DATA.Request with Euroradio Message "Request for Shunting" (NID_MESSAGE=130) is transmitted | O | RTM | - | L2/ L3 | FS / LS / OS / SR / SB / PT | - |
| 4 | L2/ L3 | FS / LS / OS / SR / SB / PT | MESSAGE TO RBC (NID_MESSAGE_JRU=10;NID_MESSAGE=130) is recorded | O | JRU | - | L2/ L3 | FS / LS / OS / SR / SB / PT | - |
| 5 | L2/ L3 | FS / LS / OS / SR / SB / PT | SA-DATA.Indication with Euroradio Message "SH Authorised" (NID_MESSAGE=28) containing packet 49 is received | I | RTM | - | L2/ L3 | FS / LS / OS / SR / SB / PT | - |
| 6 | L2/ L3 | FS / LS / OS / SR / SB / PT | MESSAGE FROM RBC (NID_MESSAGE_JRU=9;NID_MESSAGE=28;NID_PACKET=49) is recorded | O | JRU | Either an empty list or "BG A" will be recieved with packet 49 | L2/ L3 | FS / LS / OS / SR / SB / PT | - |
| 7 | L2/ L3 | FS / LS / OS / SR / SB / PT | The Mode symbol "Shunting" is displayed | O | DMI | SH symbol is MO01( according to the document ERA_ERTMS_015560 v330). | L2/ L3 | SH | - |
| 8 | L2/ L3 | SH | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit16=1>) is recorded | O | JRU | - | L2/ L3 | SH | - |
| 9 | L2/ L3 | SH | SA-DATA.Request with Euroradio Message "End of Mission" (NID_MESSAGE=150) is transmitted | O | RTM | End of Mission Procedure | L2/ L3 | SH | - |
| 10 | L2/ L3 | SH | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=150) is recorded | O | JRU | - | L2/ L3 | SH | - |
| 11 | L2/ L3 | SH | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 42 (Q_RBC=0) is received | I | RTM | - | L2/ L3 | SH | - |
| 12 | L2/ L3 | SH | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=24;NID_PACKET=42;Q_RBC=0) is recorded | O | JRU | - | L2/ L3 | SH | - |
| 13 | L2/ L3 | SH | SA-DATA.Request with Euroradio Message "Termination of a communication session" (NID_MESSAGE=156) is transmitted | O | RTM | - | L2/ L3 | SH | - |
| 14 | L2/ L3 | SH | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=156) is recorded | O | JRU | - | L2/ L3 | SH | - |
| 15 | L2/ L3 | SH | SA-DATA.Indication with Euroradio Message "Acknowledgement of termination of a communication session" (NID_MESSAGE=39) is received | I | RTM | - | L2/ L3 | SH | - |
| 16 | L2/ L3 | SH | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=39) is recorded | O | JRU | - | L2/ L3 | SH | - |
| 17 | L2/ L3 | SH | SA-DISCONNECT.Request is transmitted | O | RTM | - | L2/ L3 | SH | - |
| 18 | L2/ L3 | SH | SA-DISCONNECT.Indication is received | I | RTM | - | L2/ L3 | SH | - |
| 19 | L2/ L3 | SH | Train is moving | I | INT | - | L2/ L3 | SH | - |
| 20 | L2/ L3 | SH | One Balise Group message is received | I | BTM | "BG A", NOT included in the list of balises for SH area that was received before (if the list was empty, any BG can be recieved) | L2/ L3 | SH | - |
| 21 | L2/ L3 | SH | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L2/ L3 | SH | - |
| 22 | L2/ L3 | SH | The Mode symbol "Trip" is displayed | O | DMI | TR symbol is MO04( according to the document ERA_ERTMS_015560 v330). | L2/ L3 | TR | - |
| 23 | L2/ L3 | TR | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit19=1>) is recorded | O | JRU | - | L2/ L3 | TR | - |
| 24 | L2/ L3 | TR | Emergency brake commanded | O | TIU | - | L2/ L3 | TR | - |
| 25 | L2/ L3 | TR | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | L2/ L3 | TR | - |
| 26 | L2/ L3 | TR | The Status symbol "Service Brake or Emergency Brake intervention" is displayed | O | DMI | - | L2/ L3 | TR | - |

| Step 3: Radio Message 130 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 130 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |

| Step 5: Radio Message 28 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 28 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_PACKET | 8 | 49 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | - |

| Step 9: Radio Message 150 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 150 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |

| Step 11: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 42 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_RBC | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | RBC ETCS identity : NID_C not relevant if NID_RBC has value "Contact last known RBC". |
| NID_RBC | 14 | FINITE VALUE | - |
| NID_RADIO | 64 | FINITE VALUE | Not relevant if NID_RBC has value "Contact last known RBC". |
| Q_SLEEPSESSION | 1 | FINITE VALUE | - |

| Step 13: Radio Message 156 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 156 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |

| Step 15: Radio Message 39 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 39 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | always set to 0 |
| NID_LRBG | 24 | FINITE VALUE | - |

| Step 20: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | 255 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 | TR |
| Radio communication session | NOT ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Radio Network ID | STORED | - |
| RBC ID/Phone Number | STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | Both | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |