| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 4080441 (4.8.4: Accepted Information depending on the modes - Temporary Speed Restriction Revocation) Total: 10 Test case(s) |  |
| REF: | Subset-076-5-2-4080441 |
| ISSUE: | Version 3.2.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 0.0.1 23.03.2012 | All | First version/review | DLR |
| Version 2.9.0 31.03.2013 | All | First version/review for Baseline 3 | CEDEX |
| Version 2.9.1 31.03.2013 | All | EDITORIAL CHANGES/CHANGES IN DMI ROW IN START CONDITIONS | CEDEX |
| Version 2.9.2 25.10.2013 | All | Requirements correction + REVIEW N/A | CEDEX |
| Version 2.9.3 11.12.2013 | TCs 1, 3, 6, 7, 8, 9, 10 | TC 1 has been removed. EDITORIAL CHANGES | CEDEX |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.2.0 31.03.2017 | All | Baseline 3 1st maintenance release version - Bug fixing | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Temporary Speed Restriction Revocation |  | 4080441 |
| Test Case of Feature | Rejection of Temporary Speed Restriction Revocation (packet 66) by on-board if Level 2 or Level 3 is active and train data has been sent to the RBC and the “Acknowledgement of Train Data” has not been received yet. |  | 1 |
| Applicable Mode/Level Combinations | L2 / L3: FS / LS / OS / SR / SB / TR / PT |  |  |
| Target of Test | The target of this test case is to check that: on-board equipment rejects the packet 66 (transmitted in General Radio Message 24). |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.3.1.1 Temporary Speed Restriction Revocation |  |
|  |  | Subset-026-4.8.3.1.1 Exception [3] |  |
|  |  | Subset-026-4.8.4.2 Temporary Speed Restriction Revocation |  |
|  |  | Subset-026-4.8.4.2 [1] |  |
|  |  | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-4.8.4.2 [4] |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |
|  |  | Subset-026-7.4.2.18 Packet Number 66: Temporary Speed Restriction Revocation |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that the permitted speed does not change to less restrictive value after receiving TSR Revocation (for FS, LS, OS and SR this check can be done inmetiately, but for SB, TR and PT, a mode transition has to be performed first) Check on the JRU that message 24 with packet 66 is recorded. |
| Constraints | The data of the stored TSR (packet 65) has to be the most restrictive speed profile (for the location of TSR) and the Temporary Speed Restriction is not marked as non-revocable. Recognition of exit from TR mode has been received (PT mode) A cab needs to be active and valid Train Data are stored on-board (SB Mode) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 7 / 8 / 12 | FS / OS / SR / SB / TR / PT / LS |
| Radio communication session | ESTABLISHED | - |
| Temporary Speed Restrictions | STORED | revocable and send by RBC, that is value 127....253 |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2 / L3 | SR / FS / OS / TR / PT / LS / SB | SA-DATA.Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) containing packet 11 is transmitted | O | RTM | Validated train data | L2 / L3 | SR / FS / OS / TR / PT / LS / SB | - |
| 2 | L2 / L3 | SR / FS / OS / TR / PT / LS / SB | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=129) is recorded | O | JRU | - | L2 / L3 | SR / FS / OS / TR / PT / LS / SB | - |
| 3 | L2 / L3 | SR / FS / OS / TR / PT / LS / SB | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 66 is received | I | RTM | Data will be rejected because Message 8 has not been received yet. | L2 / L3 | SR / FS / OS / TR / PT / LS / SB | - |
| 4 | L2 / L3 | SR / FS / OS / TR / PT / LS / SB | MESSAGE FROM RBC (NID_MESSAGE_JRU=9;NID_MESSAGE=24;NID_PACKET=66) is recorded | O | JRU | Reception of TSR Revocation order | L2 / L3 | SR / FS / OS / TR / PT / LS / SB | - |
| 5 | L2 / L3 | SR / FS / OS / TR / PT / LS / SB | SA-DATA.Indication with Euroradio Message "Acknowledgement of Train Data" (NID_MESSAGE=8) is received | I | RTM | - | L2 / L3 | SR / FS / OS / TR / PT / LS / SB | - |
| 6 | L2 / L3 | SR / FS / OS / TR / PT / LS / SB | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=8) is recorded | O | JRU | - | L2 / L3 | SR / FS / OS / TR / PT / LS / SB | - |
| 7 | L2 / L3 | SR / FS / OS / TR / PT / LS / SB | USE_FT5110200.13 | - | - | Only in TR mode. This is for changing to PT mode. | L2 / L3 | SR / FS / OS / TR / PT / LS / SB | - |
| 8 | L2 / L3 | SR / FS / OS / TR / PT / LS / SB | USE_FT5110200.5 | - | - | Only in PT mode. This is for changing to FS mode. | L2 / L3 | SR / FS / OS / TR / PT / LS / SB | - |
| 9 | L2 / L3 | SR / FS / OS / TR / PT / LS / SB | USE_FT5040300.19 | - | - | Only in SB mode. This is for changing to FS mode. | L2 / L3 | SR / FS / OS / TR / PT / LS / SB | - |
| 10 | L2 / L3 | SR / FS / OS / LS | The driver requests the speed and distance monitoring information to be displayed | I | DMI | If the current mode is SR or OS mode. | L2 / L3 | SR / FS / OS / LS | - |
| 11 | L2 / L3 | SR / FS / OS / LS | The speed and distance monitoring information is displayed | O | DMI | The train position is located in the area of TSR, given in packet 65, which was received before this TC. If in FS, OS or SR mode: The Permitted Speed is the one previously received in packet 65. If in LS mode: The train speed will be over the value of V_TSR sent in the packet 65, and the supervision status is OvS or WaS. | L2 / L3 | SR / FS / OS / LS | - |

| Step 1: Radio Message 129 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 129 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 11 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| NC_CDTRAIN | 4 | FINITE VALUE | - |
| NC_TRAIN | 15 | FINITE VALUE | - |
| L_TRAIN | 12 | FINITE VALUE | - |
| V_MAXTRAIN | 7 | FINITE VALUE | - |
| M_LOADINGGAUGE | 8 | FINITE VALUE | - |
| M_AXLELOADCAT | 7 | FINITE VALUE | - |
| M_AIRTIGHT | 2 | FINITE VALUE | - |
| N_AXLE | 10 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_VOLTAGE(k) | 4 | FINITE VALUE | Identity of the traction system. |
| NID_CTRACTION(k) | 10 | FINITE VALUE | NID_CTRACTION(k) given only if M_VOLTAGE(k) ≠ 0. |
| N_ITER | 5 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | Type of National System available. |

| Step 3: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 66 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| NID_TSR | 8 | FINITE VALUE | Identity of TSR to be revoked. |

| Step 5: Radio Message 8 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 8 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | Reference to received train data message |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Radio communication session | ESTABLISHED | - |
| Temporary Speed Restrictions | STORED | revocable and send by RBC, that is value 127...253 |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Temporary Speed Restriction Revocation |  | 4080441 |
| Test Case of Feature | Reception and acceptance of Temporary Speed Restriction Revocation (packet 66) by the on-board when it is received via infill BG |  | 2 |
| Applicable Mode/Level Combinations | L1: FS, LS |  |  |
| Target of Test | The target of this test case is to check that: - the on-board equipment has a TSR stored on-board that starts beyond the main signal on-board equipment receives and accepts the packet 66 (transmitted by In-fill Balise Group) the BG message at the main signal is never found (the linking reaction stored on.board should be “No reaction”) on-board equipment revokes the stored temporary speed restriction correctly (the Temporary Speed Restriction is revocable), i.e., no TSR will be supervised when reaching the location for the beginning of the TSR. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.3.1.1 Temporary Speed Restriction Revocation |  |
|  |  | Subset-026-4.8.4.2 Temporary Speed Restriction Revocation |  |
|  |  | Subset-026-7.4.2.18 Packet Number 66: Temporary Speed Restriction Revocation |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that the permitted speed does not change to less restrictive value when reaching the start location of the TSR. Check on the JRU that balise telegram with packet 66 is recorded |
| Constraints | A TSR starting beyond the main signal must be stored on-board The data of the stored TSR (packet 65) has to be the most restrictive speed profile (for the location of TSR) and the Temporary Speed Restriction is not marked as non-revocable. No BG message is received at the main signal, but the linking reaction stored on.board should be “No reaction” |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 | FS |
| Linking | STORED | - |
| Temporary Speed Restrictions | STORED | TSR is stored, starting beyond the location of the main signal. Revocable and send by non-RBC, that are values 0....126 |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message containing packet 136, packet 66 is received | I | BTM | - | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6;NID_PACKET=136;NID_PACKET=66) is recorded | O | JRU | Reception of TSR Revocation order is attested | N/A | N/A | - |
| 3 | N/A | N/A | Train is moving (V_TRAIN= <above zero>) | I | INT | The train reaches the location of the main signal, and no BG message is read. No reaction is applied on-board. | N/A | N/A | - |
| 4 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | If in FS mode: The permitted speed does not change to a less restrictive value when reaching the location of the revoked TSR, i.e., the PS is higher than the value of V_TSR. The train is located in the area of the revoked TSR, given in packet 65 that was stored on-board. If in LS mode: The train speed is higher than the value of V_TSR that was stored on-board, and the current supervision status is NoS. | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 136 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | If Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| NID_PACKET | 8 | 66 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| NID_TSR | 8 | FINITE VALUE | Identity of TSR to be revoked. |
| NID_PACKET | 8 | 255 | 255 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| Linking | STORED | Including the main balise group |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Temporary Speed Restriction Revocation |  | 4080441 |
| Test Case of Feature | Reception and acceptance of Temporary Speed Restriction Revocation (packet 66) via radio, and Level 0, NTC or 1are the current levels. The telegram is ACCEPTED, because a level 2/3 transition order is stored onboard |  | 3 |
| Applicable Mode/Level Combinations | L0: SB, UN, TR LNTC: SN L1: FS / LS / OS / SR / SB / TR |  |  |
| Target of Test | The target of this test case is to check that: on-board equipment receives and accepts the packet 66 (transmitted in General Radio Message 24) when a Level 2/3 announcement is stored onboard. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.3.1.1 Temporary Speed Restriction Revocation |  |
|  |  | Subset-026-4.8.4.2 Temporary Speed Restriction Revocation |  |
|  |  | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-4.8.4.2 [4] |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |
|  |  | Subset-026-4.8.3.1.1 Exception [2] |  |
|  |  | Subset-026-7.4.2.18 Packet Number 66: Temporary Speed Restriction Revocation |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that message 24 with packet 66 is recorded The on-board equipment changes to L2/3, and possibly to FS, LS, OS or SR. Check on the DMI that the permitted speed is higher than the previously stored value of V_TSR (for the location of TSR) |
| Constraints | The data of the stored TSR (packet 65) has to be the most restrictive speed profile (for the location of TSR) and the Temporary Speed Restriction is not marked as non-revocable. A level 2/3 transition order is stored onboard. A cab needs to be active and valid Train Data are stored on-board (SB Mode) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 12 / 13 | FS / OS / SR / UN / SB / TR / LS / SN |
| Radio communication session | ESTABLISHED | - |
| Level Transition Order | STORED | Level 2/3 announcement message |
| Temporary Speed Restrictions | STORED | TSR is stored. revocable and send by RBC, that is value 127...253 |
| ERTMS/ETCS level | 0 / 1 / 2 | L0 / LNTC / L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0/LNTC/L1 | UN / SN / FS / LS / OS / SR / SB / TR | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 66 is received | I | RTM | - | L0/LNTC/L1 | UN / SN / FS / LS / OS / SR / SB / TR | - |
| 2 | L0/LNTC/L1 | UN / SN / FS / LS / OS / SR / SB / TR | MESSAGE FROM RBC (NID_MESSAGE_JRU=9;NID_MESSAGE=24;NID_PACKET=66) is recorded | O | JRU | Reception of TSR Revocation order | L0/LNTC/L1 | UN / SN / FS / LS / OS / SR / SB / TR | - |
| 3 | L0/LNTC/L1 | UN / SN / FS / LS / OS / SR / SB / TR | USE_FT5100100 | - | - | (for changing to L2/3) And The on-board equipment changes to either FS, LS, SR or OS mode, if it is not in any of these modes. | L2/L3 | FS / LS / OS / SR | - |
| 4 | L2/L3 | FS / LS / OS / SR | The driver requests the speed and distance monitoring information to be displayed | I | DMI | If the current mode is OS or SR. | L2/L3 | FS / LS / OS / SR | - |
| 5 | L2/L3 | FS / LS / OS / SR | The speed and distance monitoring information is displayed | O | DMI | If in FS, OS or SR mode: The PS is higher than the previously received value of V_TSR. The train is located in the area of the previously stored TSR, given in packet 65. If in LS mode: The train speed is higher than the previously received value of V_TSR and the supervision status is NoS. | L2/L3 | FS / LS / OS / SR | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | Message 24: General message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | 0 | No acknowledgement required |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 66 | Packet Number 66: Temporary Speed Restriction Revocation |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| NID_TSR | 8 | FINITE VALUE | Identity of TSR to be revoked (Identity of stored TSR) Value <> 1111 1111 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Radio communication session | ESTABLISHED | - |
| Temporary Speed Restrictions | STORED | revocable and send by RBC, that is value 127...253 |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Temporary Speed Restriction Revocation |  | 4080441 |
| Test Case of Feature | Reception and rejection of Temporary Speed Restriction Revocation (packet 66) via radio, because Level 0, NTC or 1are the current levels. |  | 4 |
| Applicable Mode/Level Combinations | L0: SB, UN, TR LNTC: SN L1: FS / LS / OS / SR / SB / TR |  |  |
| Target of Test | The target of this test case is to check that: on-board equipment receives and rejects the packet 66 (transmitted in General Radio Message 24) when no Level 2/3 announcement is stored onboard. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.3.1.1 Temporary Speed Restriction Revocation |  |
|  |  | Subset-026-4.8.4.2 Temporary Speed Restriction Revocation |  |
|  |  | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-4.8.4.2 [4] |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |
|  |  | Subset-026-7.4.2.18 Packet Number 66: Temporary Speed Restriction Revocation |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that message 24 with packet 66 is recorded The on-board equipment changes to L2/3, and possibly to FS, LS, OS or SR. Check on the DMI that the permitted speed is equal to the previously stored value of V_TSR (for the location of TSR) |
| Constraints | The data of the stored TSR (packet 65) has to be the most restrictive speed profile (for the location of TSR) and the Temporary Speed Restriction is not marked as non-revocable A cab needs to be active and if valid Train Data are stored on-board (SB Mode) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 12 / 13 | FS / OS / SR / UN / SB / TR / LS / SN |
| Radio communication session | ESTABLISHED | - |
| Temporary Speed Restrictions | STORED | TSR is stored. Revocable and send by RBC, that is value 127...253 |
| ERTMS/ETCS level | 0 / 1 / 2 | L0 / LNTC / L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0/LNTC/L1 | UN / SN / FS / LS / OS / SR / SB / TR | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 66 is received | I | RTM | - | L0/LNTC/L1 | UN / SN / FS / LS / OS / SR / SB / TR | - |
| 2 | L0/LNTC/L1 | UN / SN / FS / LS / OS / SR / SB / TR | MESSAGE FROM RBC (NID_MESSAGE_JRU=9;NID_MESSAGE=24;NID_PACKET=66) is recorded | O | JRU | Reception of TSR Revocation order | L0/LNTC/L1 | UN / SN / FS / LS / OS / SR / SB / TR | - |
| 3 | L0/LNTC/L1 | UN / SN / FS / LS / OS / SR / SB / TR | USE_FT5100100 | - | - | (for changing to L2/3) And The on-board equipment changes to either FS, LS, SR or OS mode, if it is not in any of these modes. | L2/L3 | FS / LS / OS / SR | - |
| 4 | L2/L3 | FS / LS / OS / SR | The driver requests the speed and distance monitoring information to be displayed | I | DMI | If the current mode is OS or SR mode. | L2/L3 | FS / LS / OS / SR | - |
| 5 | L2/L3 | FS / LS / OS / SR | The speed and distance monitoring information is displayed | O | DMI | If in FS, OS or SR mode: The PS is equal to the previously received value of V_TSR. The train is located in the area of the previously stored TSR, given in packet 65. If in LS mode: The train speed is higher than the previously received value of V_TSR and the supervision status is OvS or WaS. | L2/L3 | FS / LS / OS / SR | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | Message 24: General message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | 0 | No acknowledgement required |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 66 | Packet Number 66: Temporary Speed Restriction Revocation |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| NID_TSR | 8 | FINITE VALUE | Identity of TSR to be revoked (Identity of stored TSR) Value <> 1111 1111 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Radio communication session | ESTABLISHED | - |
| Temporary Speed Restrictions | STORED | revocable and send by RBC, that is value 127...253 |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 5

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Temporary Speed Restriction Revocation |  | 4080441 |
| Test Case of Feature | Reception and acceptance of Temporary Speed Restriction Revocation (packet 66) by on-board via BG message or via radio for PT mode. |  | 5 |
| Applicable Mode/Level Combinations | L0: SB / TR L1: SB / TR L2 / L3: SB / TR / PT |  |  |
| Target of Test | The target of this test case is to check that: on-board equipment receives and accepts the packet 66 (transmitted in a BG message or in a radio message for PT mode) |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.3.1.1 Temporary Speed Restriction Revocation |  |
|  |  | Subset-026-4.8.4.2 Temporary Speed Restriction Revocation |  |
|  |  | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-4.8.4.2 [4] |  |
|  |  | Subset-026-7.4.2.18 Packet Number 66: Temporary Speed Restriction Revocation |  |
|  |  | Subset-026-4.8.4.2 [1] |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that packet 66 inside a BG message is recorded packet 66 inside a radio message is recorded (if in PT mode) Check on the DMI that the permitted speed changes to less restrictive value after revocation of TSR (for the location of TSR) (a mode transition and a level transition, if the level is L0, has to be performed first) |
| Constraints | The data of the stored TSR (packet 65) has to be the most restrictive speed profile (for the location of TSR) and the Temporary Speed Restriction is not marked as non-revocable. A cab needs to be active and if valid Train Data are stored on-board (SB Mode). A message 6 must have been received before. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 / 7 / 8 | SB / TR / PT |
| Radio communication session | ESTABLISHED | For L2/3 and PT mode |
| Temporary Speed Restrictions | STORED | TSR is stored |
| ERTMS/ETCS level | 0 / 2 / 3 / 4 | L0 / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / L1 / L2 / L3 | SB / TR / PT | One Balise Group message containing packet 66 is received | I | BTM | Only in SB or TR mode. Reception of TSR Revocation order | L0 / L1 / L2 / L3 | SB / TR / PT | - |
| 2 | L0 / L1 / L2 / L3 | SB / TR / PT | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6;NID_PACKET=66) is recorded | O | JRU | Only in SB oir TR mode | L0 / L1 / L2 / L3 | SB / TR / PT | - |
| 3 | L0 / L1 / L2 / L3 | SB / TR / PT | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 66 is received | I | RTM | Only in PT mode. A message 6 must have been received before. | L0 / L1 / L2 / L3 | SB / TR / PT | - |
| 4 | L0 / L1 / L2 / L3 | SB / TR / PT | MESSAGE FROM RBC (NID_MESSAGE_JRU=9;NID_MESSAGE=24) is recorded | O | JRU | Only in PT mode. | L0 / L1 / L2 / L3 | SB / TR / PT | - |
| 5 | L0 / L1 / L2 / L3 | SB / TR / PT | USE_FT5110200.13 | - | - | Only in L1/L2/L3 and TR mode. This is for changing to PT mode. | L0 / L1 / L2 / L3 | SB / TR / PT | - |
| 6 | L0 / L1 / L2 / L3 | SB / TR / PT | The driver selects "Start" | I | DMI | Only in L2/3 and PT or SB mode. | L0 / L1 / L2 / L3 | SB / TR / PT | - |
| 7 | L0 / L1 / L2 / L3 | SB / TR / PT | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11;M_DRIVERACTIONS=0001 0011) is recorded | O | JRU | Only in L2/3 and PT or SB mode. | L0 / L1 / L2 / L3 | SB / TR / PT | - |
| 8 | L0 / L1 / L2 / L3 | SB / TR / PT | SA-DATA.Request with Euroradio Message "MA Request" (NID_MESSAGE=132) is transmitted | O | RTM | Only in L2/3 and PT or SB mode. | L0 / L1 / L2 / L3 | SB / TR / PT | - |
| 9 | L0 / L1 / L2 / L3 | SB / TR / PT | MESSAGE TO RBC (NID_MESSAGE_JRU=10;NID_MESSAGE=132) is recorded | O | JRU | Only in L2/3 and PT or SB mode. | L0 / L1 / L2 / L3 | SB / TR / PT | - |
| 10 | L0 / L1 / L2 / L3 | SB / TR / PT | SA-DATA.Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 21, packet 27 is received | I | RTM | Only in L2/3 and PT or SB mode. | L0 / L1 / L2 / L3 | SB / TR / PT | - |
| 11 | L0 / L1 / L2 / L3 | SB / TR / PT | MESSAGE FROM RBC (NID_MESSAGE_JRU=9;NID_MESSAGE=3) is recorded | O | JRU | Only in L2/3 and PT or SB mode. | L0 / L1 / L2 / L3 | SB / TR / PT | - |
| 12 | L0 / L1 / L2 / L3 | SB / TR / PT | The system status message "Entering FS" is displayed | O | DMI | Only in L2/3 and PT or SB mode. | L0 / L1 / L2 / L3 | SB / TR / PT | - |
| 13 | L0 / L1 / L2 / L3 | SB / TR / PT | USE_FT5110200.7 | - | - | Only in L1 and PT mode. | L0 / L1 / L2 / L3 | SB / TR / PT | - |
| 14 | L0 / L1 / L2 / L3 | SB / TR / PT | USE_FT5040300.16 | - | - | Only in L1 and SB mode. | L0 / L1 / L2 / L3 | SB / TR / PT | - |
| 15 | L0 / L1 / L2 / L3 | SB / TR / PT | USE_FT4060300.49 | - | - | Only in L1 and PT or SB mode. This is for changing to SR mode. | L0 / L1 / L2 / L3 | SB / TR / PT | - |
| 16 | L0 / L1 / L2 / L3 | SB / TR / PT | USE_FT5110200.12 | - | - | Only in L0 and TR mode. This is for changing to UN mode. | L0 / L1 / L2 / L3 | SB / TR / PT | - |
| 17 | L0 / L1 / L2 / L3 | SB / TR / PT | USE_FT5040300.27 and USE_FT4060300.58 | - | - | Only in L0 and SB mode. This is for changing to L0 and UN mode. | L0 / L1 / L2 / L3 | SB / TR / PT | - |
| 18 | L0 / L1 / L2 / L3 | SB / TR / PT | USE_FT4060300.52 | - | - | Only in L0 and UN mode. This is for changing to L1 and SR mode. | L0 / L1 / L2 / L3 | SB / TR / PT | - |
| 19 | L1 / L2 / L3 | FS / SR | The driver requests the speed and distance monitoring information to be displayed | I | DMI | If the current mode is different from FS | L1 / L2 / L3 | FS / SR | - |
| 20 | L1 / L2 / L3 | FS / SR | The speed and distance monitoring information is displayed | O | DMI | The PS is higher than the previously received value of V_TSR.The train is located in the area of TSR, given in packet 65. (This will test the packet 66 was accepted before) | L1 / L2 / L3 | FS / SR | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 66 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| NID_TSR | 8 | FINITE VALUE | Identity of TSR to be revoked. |
| NID_PACKET | 8 | 255 | 255 |

| Step 3: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 66 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| NID_TSR | 8 | FINITE VALUE | Identity of TSR to be revoked. |

| Step 8: Radio Message 132 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 132 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| Q_MARQSTREASON | 5 | FINITE VALUE | - |

| Step 10: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 2 | FS / SR |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 6

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Temporary Speed Restriction Revocation |  | 4080441 |
| Test Case of Feature | Intentionally deleted |  | Intentionally deleted |
| Applicable Mode/Level Combinations | Intentionally deleted |  |  |
| Target of Test | Intentionally deleted |  |  |
| Version | Intentionally deleted | Intentionally deleted |  |
| Author | Intentionally deleted |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Intentionally deleted |  |

## Test case 7

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Temporary Speed Restriction Revocation |  | 4080441 |
| Test Case of Feature | Intentionally deleted |  | Intentionally deleted |
| Applicable Mode/Level Combinations | Intentionally deleted |  |  |
| Target of Test | Intentionally deleted |  |  |
| Version | Intentionally deleted | Intentionally deleted |  |
| Author | Intentionally deleted |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Intentionally deleted |  |

## Test case 8

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Temporary Speed Restriction Revocation |  | 4080441 |
| Test Case of Feature | Intentionally deleted |  | Intentionally deleted |
| Applicable Mode/Level Combinations | Intentionally deleted |  |  |
| Target of Test | Intentionally deleted |  |  |
| Version | Intentionally deleted | Intentionally deleted |  |
| Author | Intentionally deleted |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Intentionally deleted |  |

## Test case 9

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Temporary Speed Restriction Revocation |  | 4080441 |
| Test Case of Feature | The onboard equipment receives a Temporary Speed Restriction Revocation (packet 66). The packet is REJECTED because of the current level/ and mode are L1 and PT mode. |  | 9 |
| Applicable Mode/Level Combinations | L1: PT |  |  |
| Target of Test | The target of this test case is to check that: The on-board equipment receives and rejects the packet 66 (transmitted in a BG message) and The on-board equipment does NOT revoke the stored temporary speed restriction (the Temporary Speed Restriction is revocable). |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.3.1.1 Temporary Speed Restriction Revocation |  |
|  |  | Subset-026-4.8.4.2 Temporary Speed Restriction Revocation |  |
|  |  | Subset-026-4.8.4.2 [1] |  |
|  |  | Subset-026-7.4.2.18 Packet Number 66: Temporary Speed Restriction Revocation |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that the permitted speed does NOT change to a less restrictive value (for the location of TSR) after the train changes to FS, LS, OS or SR Check on the JRU that a BG message with packet 66 is recorded |
| Constraints | The data of the stored TSR (packet 65) has to be the most restrictive speed profile (for the location of TSR) and the Temporary Speed Restriction is not marked as non-revocable |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 8 | PT |
| Temporary Speed Restrictions | STORED | revocable |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message containing packet 66 is received | I | BTM | The packet is REJECTED. | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6;NID_PACKET=66) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | A transition has to be performed to one of the following modes: FS, LS, OS or SR. | - | - | - | N/A | N/A | - |
| 4 | N/A | N/A | The driver requests the speed and distance monitoring information to be displayed | I | DMI | If the current mode is different from FS | N/A | N/A | - |
| 5 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The Permitted Speed is is equal to the previously received value of V_TSR.The train is located in the area of TSR, given in a previously stored packet 65. | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 66 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| NID_TSR | 8 | FINITE VALUE | Identity of TSR to be revoked. |
| NID_PACKET | 8 | 255 | 255 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Temporary Speed Restrictions | STORED | Initial revocable TSR |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 10

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Temporary Speed Restriction Revocation |  | 4080441 |
| Test Case of Feature | The onboard equipment receives a Temporary Speed Restriction Revocation (packet 66). The packet is REJECTED because the current level and mode is L2 and PT, and the information “Recognition of Exit from TR mode” with a more recent time stamp has not been received. |  | 10 |
| Applicable Mode/Level Combinations | L2: PT L3: PT |  |  |
| Target of Test | The target of this test case is to check that: The on-board equipment receives and rejects the packet 66 (transmitted in a General Radio Message 24) and The on-board equipment does NOT revoke the stored temporary speed restriction (the Temporary Speed Restriction is revocable). |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.3.1.1 Temporary Speed Restriction Revocation |  |
|  |  | Subset-026-4.8.4.2 Temporary Speed Restriction Revocation |  |
|  |  | Subset-026-4.8.4.2 [1] |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |
|  |  | Subset-026-7.4.2.18 Packet Number 66: Temporary Speed Restriction Revocation |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that the permitted speed does NOT change to a less restrictive value (for the location of TSR) after the train changes to FS, LS, OS or SR Check on the JRU that message 24 with packet 66 is recorded |
| Constraints | The data of the stored TSR (packet 65) has to be the most restrictive speed profile (for the location of TSR) and the Temporary Speed Restriction is not marked as non-revocable The information “Recognition of Exit from TR mode” with a more recent time stamp has not been received |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 8 | PT |
| Temporary Speed Restrictions | STORED | revocable and sent by RBC, that is value 127...253 |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 66 is received | I | RTM | The packet is REJECTED because the information “Recognition of Exit from TR mode” with a more recent time stamp has not been received | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9;NID_MESSAGE=24;NID_PACKET=66) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | A transition has to be performed to one of the following modes: FS, LS, OS or SR. | - | - | - | N/A | N/A | - |
| 4 | N/A | N/A | The driver requests the speed and distance monitoring information to be displayed | I | DMI | If the current mode is different from FS | N/A | N/A | - |
| 5 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The Permitted Speed is is equal to the previously received value of V_TSR. The train is located in the area of TSR, given in packet 65. | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | Message 24: General message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | 0 | No acknowledgement required |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 66 | Packet Number 66: Temporary Speed Restriction Revocation |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| NID_TSR | 8 | FINITE VALUE | Identity of TSR to be revoked (Identity of stored TSR) Value <> 1111 1111 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Radio communication session | ESTABLISHED | - |
| Temporary Speed Restrictions | STORED | Initial TSR revocable and that has a value 127...253 |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |