| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 4080404 (4.8.4: Accepted Information depending on the modes - Axle Load speed profile) Total: 9 Test case(s) |  |
| REF: | Subset-076-5-2-4080404 |
| ISSUE: | Version 3.2.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 0.0.1 23.03.2012 | All | First version/review | DLR |
| Version 2.9.0 31.03.2013 | All | First version/review for Baseline 3 | CEDEX |
| Version 2.9.1 04.10.2013 | All | Requirements corrections | CEDEX |
| Version 2.9.2 11.11.2013 | ALL | REVIEW | CEDEX |
| Version 2.9.3 11.12.2013 | ALL | 2ND REVIEW. CHANGE IN USE_FT REFRENCES | CEDEX |
| Version 2.9.4 17.12.2013 | ALL | 3ND REVIEW. | CEDEX |
| Version 2.9.5 20.12.2013 | ALL | REVIEW. | CEDEX |
| Version 2.9.6 21.01.2014 | ALL | CHANGE FOLLOWED STEP TO USE STEP | CEDEX |
| Version 2.9.7 04.02.2014 | TCs 1, 2, 6, 8, 15-18 | Review (some “Use FT” steps have been substituted for explicit steps) | CEDEX |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.2.0 31.03.2017 | All | Baseline 3 1st maintenance release version - Bug fixing | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Axle Load speed profile |  | 4080404 |
| Test Case of Feature | Transmission of an Axle load profile to the on-board equipment from RBC. The train accepts the information. |  | 1 |
| Applicable Mode/Level Combinations | L2: FS / OS / SR / SB / PT /LS L3: FS / OS / SR / SB / PT /LS |  |  |
| Target of Test | The target of test is to check that: The onboard equipment has received the axle load speed profile from RBC. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.6.2.2.2 |  |
|  |  | Subset-026-3.6.2.2.2 c) |  |
|  |  | Subset-026-3.6.3.2.4 |  |
|  |  | Subset-026-4.8.3.1.1 Axle Load speed profile |  |
|  |  | Subset-026-4.8.4.2 Axle Load speed profile |  |
|  |  | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |
|  |  | Subset-026-4.8.4.2 [1] |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : the receipt of an axle load speed profile (packet 51) sent by the RBC is RECORDED. |
| Constraints | A cab needs to be active (SB Mode) The onboard equipment has sent the identity numbers from LRBGONB to the RBC. In PT mode the information “Recognition of Exit from TR mode” has to be received before "ACK of train data" has been already received |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 8 /12 | FS / OS / SR / SB / PT /LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2/L3 | FS / OS / SR / SB / PT /LS | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 51 is received | I | RTM | The axle load speed profile sent by the RBC is RECEIVED. | L2/L3 | FS / OS / SR / SB / PT /LS | - |
| 2 | L2/L3 | FS / OS / SR / SB / PT /LS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | The RBC send a axle load speed profile to the on-board equipment. The axle load speed profile include non-continuous speed profile data. The on-board equipment has to accept information referring to one of at least eight LRBGONB last reported to the RBC. | L2/L3 | FS / OS / SR / SB / PT /LS | - |
| 3 | L2/L3 | FS / OS / SR / SB / PT /LS | USE_FT4060300.20 or USE_FT4060300.21 or USE_FT4060300.48 or USE_FT4060300.59 | - | - | CHANGE TO FS IF MODE IS PT,SR, SB or LS | L2/L3 | FS /OS | - |
| 4 | L2/L3 | FS /OS | Train is moving | I | INT | - | L2/L3 | FS /OS | - |
| 5 | L2/L3 | FS /OS | The driver requests the speed and distance monitoring information to be displayed | I | DMI | Optional step. Applies only if the applicable mode is OS | L2/L3 | FS /OS | - |
| 6 | L2/L3 | FS /OS | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | Optional step. Applies only if the applicable mode is OS | L2/L3 | FS /OS | - |
| 7 | L2/L3 | FS /OS | The speed and distance monitoring information is displayed. | O | DMI | The train has reached the location given by D_AXLELOAD and the displayed Permitted Speed is equal to the value of V_AXLELOAD, which was received at the beginning of this TC. It is also displayed in LS mode because the Permitted Speed is lower than the LS mode speed limit. | L2/L3 | FS /OS | - |
| 8 | L2/L3 | FS /OS | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM=V_AXELOAD) is recorded. | O | JRU | - | L2/L3 | FS /OS | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 51 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | FINITE VALUE | - |
| D_TRACKINIT | 15 | FINITE VALUE | Only if Q_TRACKINIT = 1 |
| D_AXLELOAD | 15 | FINITE VALUE | Only if Q_TRACKINIT = 0, D_AXLELOAD and the following variables follow. |
| L_AXLELOAD | 15 | FINITE VALUE | - |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_AXLELOADCAT(n) | 7 | FINITE VALUE | - |
| V_AXLELOAD(n) | 7 | FINITE VALUE | Speed restriction to be applied if the axle load category of the train ( M_AXLELOADCAT(n). |
| N_ITER | 5 | FINITE VALUE | - |
| D_AXLELOAD(k) | 15 | FINITE VALUE | - |
| L_AXLELOAD(k) | 15 | FINITE VALUE | - |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| M_AXLELOADCAT(k,m) | 7 | FINITE VALUE | - |
| V_AXLELOAD(k,m) | 7 | FINITE VALUE | Speed restriction to be applied if the axle load category of the train ( M_AXLELOADCAT(k,m). |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 /1 | FS /OS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Axle Load speed profile |  | 4080404 |
| Test Case of Feature | Transmission of an Axle load profile to the on-board equipment from RBC. The train rejects the Axle load profile because train data has been sent to the RBC and the acknowledgement of train data has not been received. |  | 2 |
| Applicable Mode/Level Combinations | L2 / L3: FS / OS / SR / SB / PT /LS |  |  |
| Target of Test | The target of test is to check that: The onboard equipment has received the axle load speed profile from RBC. The onboard equipment has rejected the axle load speed profile because the “Acknowledgement of Train Data” has not been received |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.6.2.2.2 |  |
|  |  | Subset-026-3.6.2.2.2 c) |  |
|  |  | Subset-026-3.6.3.2.4 |  |
|  |  | Subset-026-4.8.3.1.1 Axle Load speed profile |  |
|  |  | Subset-026-4.8.3.1.1 Exception [3] |  |
|  |  | Subset-026-4.8.4.2 Axle Load speed profile |  |
|  |  | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : the receipt of an axle load speed profile (packet 51) sent by the RBC is RECORDED. |
| Constraints | A cab needs to be active (SB Mode) The onboard equipment has sent the identity numbers from LRBGONB to the RBC. The Acknowledgement of Train Data has not been receive d from RBC The reception of balise information in SB requires the train moving without an immediate movement protection. The national value D_NVROLL has to be set to a high value, enabling the train to read a BG |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 8 / 12 | FS / OS / SR / SB / PT /LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2/L3 | FS / OS / SR / SB / PT /LS | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 51 is received | I | RTM | The axle load speed profile send by the RBC is RECEIVED. | L2 /L3 | FS / OS / SR / SB / PT /LS | - |
| 2 | L2/L3 | FS / OS / SR / SB / PT /LS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | The RBC send an axle load speed profile to the on-board equipment. The axle load speed profile include non-continuous speed profile data. The on-board equipment has to accept information referring to one of at least eight LRBGONB last reported to the RBC. | L2 /L3 | FS / OS / SR / SB / PT /LS | - |
| 3 | L2/L3 | FS / OS / SR / SB / PT /LS | USE_FT4060300.20 or USE_FT4060300.21 or USE_FT4060300.48 or USE_FT4060300.59 | - | - | CHANGE TO FS IF MODE IS PT,SR, SB or LS | L2 /L3 | FS /OS | - |
| 4 | L2/L3 | FS /OS | Train is moving | I | INT | - | L2/L3 | FS /OS | - |
| 5 | L2/L3 | FS /OS | The driver requests the speed and distance monitoring information to be displayed | I | DMI | Optional step. Applies only if the applicable mode is OS | L2/L3 | FS /OS | - |
| 6 | L2/L3 | FS /OS | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | Optional step. Applies only if the applicable mode is OS | L2/L3 | FS /OS | - |
| 7 | L2/L3 | FS /OS | The speed and distance monitoring information is displayed. | O | DMI | The train has not yet arrived to D_AXLELOAD. The displayed Permitted Speed is different from the value of V_AXLELOAD, which was received at the beginning of this TC. It is also displayed in LS mode because the Permitted Speed is lower than the LS mode speed limit. | L2/L3 | FS /OS | - |
| 8 | L2/L3 | FS /OS | The speed and distance monitoring information is displayed. | O | DMI | The train reaches the location given by D_AXLELOAD. The displayed Permitted Speed is different from the value of V_AXLELOAD, which was received at the beginning of this TC. It is also displayed in LS mode because the Permitted Speed is lower than the LS mode speed limit. | L2/L3 | FS /OS | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 51 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | FINITE VALUE | - |
| D_TRACKINIT | 15 | FINITE VALUE | Only if Q_TRACKINIT = 1 |
| D_AXLELOAD | 15 | FINITE VALUE | Only if Q_TRACKINIT = 0, D_AXLELOAD and the following variables follow. |
| L_AXLELOAD | 15 | FINITE VALUE | - |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_AXLELOADCAT(n) | 7 | FINITE VALUE | - |
| V_AXLELOAD(n) | 7 | FINITE VALUE | Speed restriction to be applied if the axle load category of the train ( M_AXLELOADCAT(n). |
| N_ITER | 5 | FINITE VALUE | - |
| D_AXLELOAD(k) | 15 | FINITE VALUE | - |
| L_AXLELOAD(k) | 15 | FINITE VALUE | - |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| M_AXLELOADCAT(k,m) | 7 | FINITE VALUE | - |
| V_AXLELOAD(k,m) | 7 | FINITE VALUE | Speed restriction to be applied if the axle load category of the train ( M_AXLELOADCAT(k,m). |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 /1 | FS /OS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Axle Load speed profile |  | 4080404 |
| Test Case of Feature | The onboard device receives a message with an axle load speed profile from RBC. The message is rejected. |  | 3 |
| Applicable Mode/Level Combinations | L2: TR, PT L3: TR, PT |  |  |
| Target of Test | The target of test is to check that: The onboard equipment has received the axle load speed profile from RBC. The onboard equipment has rejected the axle load speed profile, because the applicable mode combination is not allowed. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.3.1.1 Axle Load speed profile |  |
|  |  | Subset-026-4.8.4.2 Axle Load speed profile |  |
|  |  | Subset-026-7.4.2.13 Packet Number 51: axle load speed profile |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |
|  |  | Subset-026-4.8.4.2 [1] |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : the receipt of an axle load speed profile (packet 51) sent by the RBC is RECORDED. after the D_AXLEOAD distance No changes in Permitted Speed. |
| Constraints | Recognition of Exit from TR mode” with a more recent time stamp has not been received. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 / 8 | TR /PT |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2 / L3 | TR / PT | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 51 is received | I | RTM | The message is REJECTED | L2 / L3 | TR / PT | - |
| 2 | L2 / L3 | TR / PT | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | L2 / L3 | TR / PT | - |
| 3 | L2 / L3 | TR / PT | Train is moving (V_TRAIN=<>) | I | INT | The train arrive to D_AXELOAD | L2 / L3 | TR / PT | - |
| 4 | L2 / L3 | TR / PT | USE_FT5110200.13 | - | - | Only if in TR mode | L2 / L3 | PT | - |
| 5 | L2 / L3 | PT | USE_FT5110200.5 | - | - | The train changes to FS mode | L2 / L3 | FS | - |
| 6 | L2 / L3 | FS | The speed and distance monitoring information is displayed | O | DMI | No changes of indication are displayed after arrive D_AXLELOAD The information is Rejected and will not be used. | L2 / L3 | FS | - |
| 7 | L2 / L3 | FS | NOT SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | The information is Rejected and will not be used. No changes in DMI | L2 / L3 | FS | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 51 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | FINITE VALUE | - |
| D_TRACKINIT | 15 | FINITE VALUE | Only if Q_TRACKINIT = 1 |
| D_AXLELOAD | 15 | FINITE VALUE | Only if Q_TRACKINIT = 0, D_AXLELOAD and the following variables follow. |
| L_AXLELOAD | 15 | FINITE VALUE | - |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_AXLELOADCAT(n) | 7 | FINITE VALUE | - |
| V_AXLELOAD(n) | 7 | FINITE VALUE | Speed restriction to be applied if the axle load category of the train ( M_AXLELOADCAT(n). |
| N_ITER | 5 | FINITE VALUE | - |
| D_AXLELOAD(k) | 15 | FINITE VALUE | - |
| L_AXLELOAD(k) | 15 | FINITE VALUE | - |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| M_AXLELOADCAT(k,m) | 7 | FINITE VALUE | - |
| V_AXLELOAD(k,m) | 7 | FINITE VALUE | Speed restriction to be applied if the axle load category of the train ( M_AXLELOADCAT(k,m). |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 | FS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Axle Load speed profile |  | 4080404 |
| Test Case of Feature | Transmission of an Axle load profile to the on-board equipment via radio infill device. |  | 4 |
| Applicable Mode/Level Combinations | L1: FS ,LS |  |  |
| Target of Test | The target of test is to check that: The onboard equipment has received the axle load speed profile via radio infill device. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.6.3.2.4 |  |
|  |  | Subset-026-3.9.1.3 |  |
|  |  | Subset-026-4.8.3.1.1 Axle Load speed profile |  |
|  |  | Subset-026-4.8.4.2 Axle Load speed profile |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |
|  |  | Subset-026-7.4.2.13 Packet Number 51: axle load speed profile |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : the receipt of an axle load speed profile (packet 51) sent via radio infill device is RECORDED. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 /12 | FS /LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 | FS / LS | SA-DATA.Indication with Euroradio Message "Infill MA" (NID_MESSAGE=37) containing packet 51 (Q_DIR=1)is received | I | RTM | The axle load speed profile sent via radio infill device is RECEIVED. | L1 | FS / LS | - |
| 2 | L1 | FS / LS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | The axle load speed profile include non-continuous speed profile data. | L1 | FS / LS | - |
| 3 | L1 | FS / LS | Train is moving | I | INT | The train passes over the location of the main balise without reading any message. | L1 | FS / LS | - |
| 4 | L1 | FS / LS | USE_FT4060300.44 | - | - | Only if in LS mode. Change to FS if the current mode is LS mode. | L1 | FS | - |
| 5 | L1 | FS | The speed and distance monitoring information is displayed. | O | DMI | The train has reached the location given by D_AXLELOAD and the displayed Permitted Speed is equal to the value of V_AXLELOAD, which was received at the beginning of this TC. It is also displayed in LS mode because the Permitted Speed is lower than the LS mode speed limit. | L1 | FS | - |
| 6 | L1 | FS | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM=V_AXELOAD) is recorded. | O | JRU | - | L1 | FS | - |

| Step 1: Radio Message 37 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 37 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 136 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | If Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| NID_PACKET | 8 | 12 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out". |
| N_ITER | 5 | FINITE VALUE | Set to zero if V_MAIN = 0 or if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 51 | - |
| Q_DIR | 2 | 1 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | FINITE VALUE | - |
| D_TRACKINIT | 15 | FINITE VALUE | Only if Q_TRACKINIT = 1 |
| D_AXLELOAD | 15 | FINITE VALUE | Only if Q_TRACKINIT = 0, D_AXLELOAD and the following variables follow. |
| L_AXLELOAD | 15 | FINITE VALUE | - |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_AXLELOADCAT(n) | 7 | FINITE VALUE | - |
| V_AXLELOAD(n) | 7 | FINITE VALUE | Speed restriction to be applied if the axle load category of the train ( M_AXLELOADCAT(n). |
| N_ITER | 5 | FINITE VALUE | - |
| D_AXLELOAD(k) | 15 | FINITE VALUE | - |
| L_AXLELOAD(k) | 15 | FINITE VALUE | - |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| M_AXLELOADCAT(k,m) | 7 | FINITE VALUE | - |
| V_AXLELOAD(k,m) | 7 | FINITE VALUE | Speed restriction to be applied if the axle load category of the train ( M_AXLELOADCAT(k,m). |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 | FS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 5

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Axle Load speed profile |  | 4080404 |
| Test Case of Feature | Reception of an Axle load speed profile (packet 51) via balise group. The received message is ACCEPTED. |  | 5 |
| Applicable Mode/Level Combinations | L1: FS / OS / SR / SB /LS |  |  |
| Target of Test | The target of test is to check that: The onboard equipment has received the axle load speed profile (packet 51) via balise group. The message is accepted. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.6.3.2.4 |  |
|  |  | Subset-026-4.8.3.1.1 Axle Load speed profile |  |
|  |  | Subset-026-4.8.4.2 Axle Load speed profile |  |
|  |  | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-7.4.2.13 Packet Number 51: axle load speed profile |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : the receipt of an axle load speed profile (packet 51) sent via balise group is RECORDED. |
| Constraints | A cab needs to be active (SB Mode) The reception of balise information in SB requires the train moving without an immediate movement protection. The national value D_NVROLL has to be set to a high value, enabling the train to read a BG |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 /12 | FS / OS / SR / SB /LS |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 | FS / OS / SR / SB /LS | One Balise Group message (N_TOTAL=0) containing packet 51 is received | I | BTM | The axle load speed profile sent via balise group is RECEIVED and ACCEPTED. | L1 | FS / OS / SR / SB /LS | - |
| 2 | L1 | FS / OS / SR / SB /LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L1 | FS / OS / SR / SB /LS | - |
| 3 | L1 | FS / OS / SR / SB /LS | USE_FT4060300.44 or USE_FT4060300.48 or USE_FT4060300.44 | - | - | CHANGE TO FS IF MODE IS SR, SB or LS | L1 | FS /OS | - |
| 4 | L1 | FS /OS | Train is moving | I | INT | - | L1 | FS /OS | - |
| 5 | L1 | FS /OS | The driver requests the speed and distance monitoring information to be displayed | I | DMI | Optional step. Applies only if the applicable mode is OSS | L1 | FS /OS | - |
| 6 | L1 | FS /OS | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | Optional step. Applies only if the applicable mode is OS | L1 | FS /OS | - |
| 7 | L1 | FS /OS | The speed and distance monitoring information is displayed. | O | DMI | The train has reached the location given by D_AXLELOAD and the displayed Permitted Speed is equal to the value of V_AXLELOAD, which was received at the beginning of this TC. It is also displayed in LS mode because the Permitted Speed is lower than the LS mode speed limit. | L1 | FS /OS | - |
| 8 | L1 | FS /OS | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM=V_AXELOAD) is recorded. | O | JRU | - | L1 | FS /OS | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 51 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | FINITE VALUE | - |
| D_TRACKINIT | 15 | FINITE VALUE | Only if Q_TRACKINIT = 1 |
| D_AXLELOAD | 15 | FINITE VALUE | Only if Q_TRACKINIT = 0, D_AXLELOAD and the following variables follow. |
| L_AXLELOAD | 15 | FINITE VALUE | - |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_AXLELOADCAT(n) | 7 | FINITE VALUE | - |
| V_AXLELOAD(n) | 7 | FINITE VALUE | Speed restriction to be applied if the axle load category of the train ( M_AXLELOADCAT(n). |
| N_ITER | 5 | FINITE VALUE | - |
| D_AXLELOAD(k) | 15 | FINITE VALUE | - |
| L_AXLELOAD(k) | 15 | FINITE VALUE | - |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| M_AXLELOADCAT(k,m) | 7 | FINITE VALUE | - |
| V_AXLELOAD(k,m) | 7 | FINITE VALUE | Speed restriction to be applied if the axle load category of the train ( M_AXLELOADCAT(k,m). |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 /1 | FS /OS |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 6

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Axle Load speed profile |  | 4080404 |
| Test Case of Feature | Reception of an Axle load speed profile (packet 51) via balise group. The received message is REJECTED because no level transition is stored onboard. |  | 6 |
| Applicable Mode/Level Combinations | L2:L3: FS / OS / SR / SB / LS L0: SB / UN LNTC: SN |  |  |
| Target of Test | The target of test is to check that: The onboard equipment has received the axle load speed profile (packet 51) via balise group. The message is REJECTED. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.6.3.2.4 |  |
|  |  | Subset-026-4.8.3.1.1 Axle Load speed profile |  |
|  |  | Subset-026-4.8.4.2 Axle Load speed profile |  |
|  |  | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-7.4.2.13 Packet Number 51: axle load speed profile |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : the receipt of an axle load speed profile (packet 51) sent via balise group is RECORDED. No changes of axel load is NOT DISPLAYED in DMI |
| Constraints | A cab needs to be active (SB Mode) No Level Transition is stored onbard The reception of balise information in SB requires the train moving without an immediate movement protection. The national value D_NVROLL has to be set to a high value, enabling the train to read a BG |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 /4 / 6 / 7 /12 /13 | FS / OS / SR / SB/ TR /LS /UN /SN |
| Level Transition Order | NOT STORED | Level transition order to L1 |
| ERTMS/ETCS level | 3/4 / 1 / 0 | L2/ L3 /LNTC /L0 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0/L2/L3/LNTC | FS / OS / SR / SB/ TR /LS /UN /SN | One Balise Group message (N_TOTAL=0) containing packet 51 is received | I | BTM | The axle load speed profile sent via balise group is RECEIVED and REJECTED | L0/L2/L3/LNTC | FS / OS / SR / SB/ TR /LS /UN /SN | - |
| 2 | L0/L2/L3/LNTC | FS / OS / SR / SB/ TR /LS /UN /SN | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L0/L2/L3/LNTC | FS / OS / SR / SB/ TR /LS /UN /SN | - |
| 3 | L0/L2/L3/LNTC | FS / OS / SR / SB/ TR /LS /UN /SN | USE_FT5100100,15,23,28,32 | - | - | The train receives the transition level only L1 | L1 | FS / OS / SR / SB / LS | - |
| 4 | L1 | FS / OS / SR / SB / LS | USE_FT4060300.21 or USE_FT4060300.48 or USE_FT4060300.59 | - | - | CHANGE TO FS IF MODE IS IN SR, SB or LS | L1 | FS /OS | - |
| 5 | L1 | FS /OS | Train is moving | I | INT | - | L1 | FS /OS | - |
| 6 | L1 | FS /OS | The driver requests the speed and distance monitoring information to be displayed | I | DMI | Optional step. Applies only if the applicable mode is OS/ LS | L1 | FS /OS | - |
| 7 | L1 | FS /OS | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | Optional step. Applies only if the applicable mode is OS/ LS | L1 | FS /OS | - |
| 8 | L1 | FS /OS | The speed and distance monitoring information is displayed. | O | DMI | The train has not yet arrived to D_AXLELOAD. The displayed Permitted Speed is different from the value of V_AXLELOAD, which was received at the beginning of this TC. It is also displayed in LS mode because the Permitted Speed is lower than the LS mode speed limit. | L1 | FS /OS | - |
| 9 | L1 | FS /OS | The speed and distance monitoring information is displayed. | O | DMI | The train reaches the location given by D_AXLELOAD. The displayed Permitted Speed is different from the value of V_AXLELOAD, which was received at the beginning of this TC. It is also displayed in LS mode because the Permitted Speed is lower than the LS mode speed limit. | L1 | FS /OS | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 51 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | FINITE VALUE | - |
| D_TRACKINIT | 15 | FINITE VALUE | Only if Q_TRACKINIT = 1 |
| D_AXLELOAD | 15 | FINITE VALUE | Only if Q_TRACKINIT = 0, D_AXLELOAD and the following variables follow. |
| L_AXLELOAD | 15 | FINITE VALUE | - |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_AXLELOADCAT(n) | 7 | FINITE VALUE | - |
| V_AXLELOAD(n) | 7 | FINITE VALUE | Speed restriction to be applied if the axle load category of the train ( M_AXLELOADCAT(n). |
| N_ITER | 5 | FINITE VALUE | - |
| D_AXLELOAD(k) | 15 | FINITE VALUE | - |
| L_AXLELOAD(k) | 15 | FINITE VALUE | - |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| M_AXLELOADCAT(k,m) | 7 | FINITE VALUE | - |
| V_AXLELOAD(k,m) | 7 | FINITE VALUE | Speed restriction to be applied if the axle load category of the train ( M_AXLELOADCAT(k,m). |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 | FS / OS |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 7

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Axle Load speed profile |  | 4080404 |
| Test Case of Feature | Transmission of an Axle load profile to the on-board equipment from RBC. The train accepts the Axle load profile because Transition order to L2/L3 is stored onboard |  | 7 |
| Applicable Mode/Level Combinations | L0: SB, UN L1: SB, SR, FS, OS, TR, LS LNTC:SN |  |  |
| Target of Test | The target of test is to check that: The onboard equipment has received the axle load speed profile from RBC. The onboard equipment has accepted the axle load speed because Transition order to L2/L3 is stored onboard |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.6.2.2.2 |  |
|  |  | Subset-026-3.6.2.2.2 c) |  |
|  |  | Subset-026-3.6.3.2.4 |  |
|  |  | Subset-026-4.8.3.1.1 Axle Load speed profile |  |
|  |  | Subset-026-4.8.3.1.1 Exception [2] |  |
|  |  | Subset-026-4.8.4.2 Axle Load speed profile |  |
|  |  | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : the receipt of an axle load speed profile (packet 51) sent by the RBC is RECORDED. |
| Constraints | A transition level to L2/3 has been received A cab needs to be active (SB Mode) The reception of balise information in SB requires the train moving without an immediate movement protection. The national value D_NVROLL has to be set to a high value, enabling the train to read a BG |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 15 /13 | FS / OS / SR / SB / TR / LS /UN /SN |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 1/2/3 | L0/L1/LNTC |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0/L1/LNTC | SB /UN /SR/ FS/ OS /TR / LS /SN | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 51 is received | I | RTM | The axle load speed profile send by the RBC is RECEIVED. | L0/L1/LNTC | FS / OS / SR / SB / TR / LS /UN /SN | - |
| 2 | L0/L1/LNTC | SB /UN /SR/ FS/ OS /TR / LS /SN | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | The RBC send an axle load speed profile to the on-board equipment. The axle load speed profile include non-continuous speed profile data. The on-board equipment has to accept information referring to one of at least eight LRBGONB last reported to the RBC. | L0/L1/LNTC | FS / OS / SR / SB / TR / LS /UN /SN | - |
| 3 | L0/L1/LNTC | SB /UN /SR/ FS/ OS /TR / LS /SN | USE_FT5100100,13,21,37,26,20,36 | - | - | The train reach the transition level L2, L3 | L2/L3 | FS / OS / SR / SB / TR / LS | - |
| 4 | L2, L3 | SB /SR/ FS/ OS /TR / LS | USE_FT5110200.13 | - | - | Only in TR mode: IF MODE IS TR, The train change to PT | L2/L3 | SB /SR/ FS/ OS /PT / LS | - |
| 5 | L2, L3 | SB /SR/ FS/ OS /PT / LS | USE_FT4060300.20 or USE_FT4060300.21 or USE_FT4060300.48 or USE_FT4060300.59 | - | - | CHANGE TO FS IF MODE IS PT,SR, SB or LS | L2/L3 | FS/ OS | - |
| 6 | L2/L3 | FS/ OS | Train is moving | I | INT | - | L2/L3 | FS/ OS | - |
| 7 | L2/L3 | FS/ OS | The driver requests the speed and distance monitoring information to be displayed | I | DMI | Optional step. Applies only if the applicable mode is OS | L2/L3 | FS/ OS | - |
| 8 | L2/L3 | FS/ OS | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | Optional step. Applies only if the applicable mode is OS | L2/L3 | FS/ OS | - |
| 9 | L2/L3 | FS/ OS | The speed and distance monitoring information is displayed. | O | DMI | The train has reached the location given by D_AXLELOAD and the displayed Permitted Speed is equal to the value of V_AXLELOAD, which was received at the beginning of this TC. It is also displayed in LS mode because the Permitted Speed is lower than the LS mode speed limit. | L2/L3 | FS/ OS | - |
| 10 | L2/L3 | FS/ OS | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM=V_AXELOAD) is recorded. | O | JRU | - | L2/L3 | FS/ OS | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 51 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | FINITE VALUE | - |
| D_TRACKINIT | 15 | FINITE VALUE | Only if Q_TRACKINIT = 1 |
| D_AXLELOAD | 15 | FINITE VALUE | Only if Q_TRACKINIT = 0, D_AXLELOAD and the following variables follow. |
| L_AXLELOAD | 15 | FINITE VALUE | - |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_AXLELOADCAT(n) | 7 | FINITE VALUE | - |
| V_AXLELOAD(n) | 7 | FINITE VALUE | Speed restriction to be applied if the axle load category of the train ( M_AXLELOADCAT(n). |
| N_ITER | 5 | FINITE VALUE | - |
| D_AXLELOAD(k) | 15 | FINITE VALUE | - |
| L_AXLELOAD(k) | 15 | FINITE VALUE | - |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| M_AXLELOADCAT(k,m) | 7 | FINITE VALUE | - |
| V_AXLELOAD(k,m) | 7 | FINITE VALUE | Speed restriction to be applied if the axle load category of the train ( M_AXLELOADCAT(k,m). |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 /1 | FS /OS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 /4 | L2, L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 8

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Axle Load speed profile |  | 4080404 |
| Test Case of Feature | Transmission of an Axle load profile to the on-board equipment from RBC. The train REJECTS the Axle load profile because No transition level to L2/3 has been received. |  | 8 |
| Applicable Mode/Level Combinations | L0: SB, UN L1: SB, SR, FS, OS, TR, LS LNTC:SN |  |  |
| Target of Test | The target of test is to check that: The onboard equipment has received the axle load speed profile from RBC. The onboard equipment has REJECTED the axle load speed profile |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.6.2.2.2 |  |
|  |  | Subset-026-3.6.2.2.2 c) |  |
|  |  | Subset-026-3.6.3.2.4 |  |
|  |  | Subset-026-4.8.3.1.1 Axle Load speed profile |  |
|  |  | Subset-026-4.8.3.1.1 Exception [2] |  |
|  |  | Subset-026-4.8.4.2 Axle Load speed profile |  |
|  |  | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : The receipt of an axle load speed profile (packet 51) sent by the RBC is RECORDED. No change in axel load is DISPLAYED in DMI |
| Constraints | No transition level to L2/3 has been received The reception of balise information in SB requires the train moving without an immediate movement protection. The national value D_NVROLL has to be set to a high value, enabling the train to read a BG |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4/ 6 / 7 /13 / 15 | FS / OS / SR /UN/ SB / TR / SN / LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 1/2/3 | L0/L1/LNTC |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0/LNTC/L1 | FS / OS / SR /UN/ SB / TR / SN / LS | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 51 is received | I | RTM | The axle load speed profile send by the RBC is RECEIVED. | L0/LNTC/L1 | FS / OS / SR /UN/ SB / TR / SN / LS | - |
| 2 | L0/LNTC/L1 | FS / OS / SR /UN/ SB / TR / SN / LS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=24) is recorded | O | JRU | The RBC send an axle load speed profile to the on-board equipment. The axle load speed profile include non-continuous speed profile data. | L0/LNTC/L1 | FS / OS / SR /UN/ SB / TR / SN / LS | - |
| 3 | L0/LNTC/L1 | FS / OS / SR /UN/ SB / TR / SN / LS | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 41 (D_LEVELTR=<different from 0>; M_LEVELTR=<2¦3>) is received | I | RTM | Only if in L0 / LNTC and UN / SN mode | L0/LNTC/L1 | FS / OS / SR /UN/ SB / TR / SN / LS | - |
| 4 | L0/LNTC/L1 | FS / OS / SR /UN/ SB / TR / SN / LS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=24) is recorded | O | JRU | Only if in L0 / LNTC and UN / SN mode | L0/LNTC/L1 | FS / OS / SR /UN/ SB / TR / SN / LS | - |
| 5 | L0/LNTC/L1 | FS / OS / SR /UN/ SB / TR / SN / LS | SA-DATA.Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 21, packet 27 is received | I | RTM | Only if in L0 / LNTC and UN / SN mode | L0/LNTC/L1 | FS / OS / SR /UN/ SB / TR / SN / LS | - |
| 6 | L0/LNTC/L1 | FS / OS / SR /UN/ SB / TR / SN / LS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=3) is recorded | O | JRU | Only if in L0 / LNTC and UN / SN mode | L0/LNTC/L1 | FS / OS / SR /UN/ SB / TR / SN / LS | - |
| 7 | L0/LNTC/L1 | FS / OS / SR /UN/ SB / TR / SN / LS | USE_FT5100100.36 or USE_FT5100100.37 | - | - | For changing immediately to L3 or L2 | L2/L3 | FS / OS / SR /UN/ SB / TR / SN / LS | - |
| 8 | L2/L3 | FS / OS / SR /UN/ SB / TR / SN / LS | USE_FT5110200.13 | - | - | Only if in TR mode, then the train changes to PT. | L2/L3 | FS / OS / SR /UN/ SB / PT / SN / LS | - |
| 9 | L2/L3 | FS / OS / SR /UN/ SB / PT / SN / LS | USE_FT4060300.19 | - | - | Change to FS if the current mode is UN or SN. | L2/L3 | FS / OS / SR / SB / PT / LS | - |
| 10 | L2/L3 | FS / OS / SR / SB / PT / LS | USE_FT5040300.19 | - | - | Change to FS if the current mode is SB. | L2/L3 | FS / OS / SR / PT / LS | - |
| 11 | L2/L3 | FS / OS / SR / PT / LS | USE_FT4060300.20 (for PT) or USE_FT4060300.21 (for SR) or USE_FT4060300.59 (for LS) | - | - | Change to FS if the current mode is PT,SR or LS. | L2/L3 | FS / OS | - |
| 12 | L2/L3 | FS / OS | Train is moving | I | INT | - | L2/L3 | FS / OS | - |
| 13 | L2/L3 | FS / OS | The driver requests the speed and distance monitoring information to be displayed | I | DMI | Optional step. Applies only if the applicable mode is OS | L2/L3 | FS / OS | - |
| 14 | L2/L3 | FS / OS | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | Optional step. Applies only if the applicable mode is OS | L2/L3 | FS / OS | - |
| 15 | L2/L3 | FS / OS | The speed and distance monitoring information is displayed. | O | DMI | The train has not yet arrived to D_AXLELOAD. The displayed Permitted Speed is different from the value of V_AXLELOAD, which was received at the beginning of this TC. It is also displayed in LS mode because the Permitted Speed is lower than the LS mode speed limit. | L2/L3 | FS / OS | - |
| 16 | L2/L3 | FS / OS | The speed and distance monitoring information is displayed. | O | DMI | The train reaches the location given by D_AXLELOAD. The displayed Permitted Speed is different from the value of V_AXLELOAD, which was received at the beginning of this TC. It is also displayed in LS mode because the Permitted Speed is lower than the LS mode speed limit. | L2/L3 | FS / OS | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 51 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | FINITE VALUE | - |
| D_TRACKINIT | 15 | FINITE VALUE | Only if Q_TRACKINIT = 1 |
| D_AXLELOAD | 15 | FINITE VALUE | Only if Q_TRACKINIT = 0, D_AXLELOAD and the following variables follow. |
| L_AXLELOAD | 15 | FINITE VALUE | - |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_AXLELOADCAT(n) | 7 | FINITE VALUE | - |
| V_AXLELOAD(n) | 7 | FINITE VALUE | Speed restriction to be applied if the axle load category of the train ( M_AXLELOADCAT(n). |
| N_ITER | 5 | FINITE VALUE | - |
| D_AXLELOAD(k) | 15 | FINITE VALUE | - |
| L_AXLELOAD(k) | 15 | FINITE VALUE | - |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| M_AXLELOADCAT(k,m) | 7 | FINITE VALUE | - |
| V_AXLELOAD(k,m) | 7 | FINITE VALUE | Speed restriction to be applied if the axle load category of the train ( M_AXLELOADCAT(k,m). |

| Step 3: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 41 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | FINITE VALUE | Different from 0 |
| M_LEVELTR | 3 | 3 / 4 | L2 / L3 |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTR = 1 (NTC). |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |

| Step 5: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 /1 | FS /OS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 9

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Axle Load speed profile |  | 4080404 |
| Test Case of Feature | Reception of an Axle load speed profile (packet 51) via balise group. The received message is ACCEPTED because a level transition to L1 order is stored onboard. |  | 9 |
| Applicable Mode/Level Combinations | L2:L3: FS /OS /SR /SB/ LS L0: SB, UN LNTC:SN |  |  |
| Target of Test | The target of test is to check that: The onboard equipment has received the axle load speed profile (packet 51) via balise group. The message is accepted. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.6.3.2.4 |  |
|  |  | Subset-026-4.8.3.1.1 Axle Load speed profile |  |
|  |  | Subset-026-4.8.4.2 Axle Load speed profile |  |
|  |  | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-7.4.2.13 Packet Number 51: axle load speed profile |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : the receipt of an axle load speed profile (packet 51) sent via balise group is RECORDED. |
| Constraints | A cab needs to be active (SB Mode) A Level Transition to L1 is stored onbard The reception of balise information in SB requires the train moving without an immediate movement protection. The national value D_NVROLL has to be set to a high value, enabling the train to read a BG |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 /4 / 6 /12 /13 | FS / OS / SR / UN /SB /LS / SN |
| Level Transition Order | STORED | Level transition order to L1. |
| ERTMS/ETCS level | 3/4 / 1 /0 | L2/ L3 /LNTC /L0 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0, LNTC, L2, L3 | FS / OS / SR / SB / LS /UN /SN | One Balise Group message (N_TOTAL=0) containing packet 51 is received | I | BTM | The axle load speed profile sent via balise group is RECEIVED and ACCEPTED. | L0, LNTC.L2, L3 | FS / OS / SR / SB / LS /UN /SN | - |
| 2 | L0, LNTC.L2, L3 | FS / OS / SR / SB / LS /UN /SN | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L0, LNTC.L2, L3 | FS / OS / SR / SB / LS /UN /SN | - |
| 3 | L0, LNTC.L2, L3 | FS / OS / SR / SB / LS /UN /SN | USE_FT5100100,15,23,28,32 | - | - | The train reach the transition level only L1 | L1 | FS / OS / SR / SB / LS | - |
| 4 | L1 | FS / OS / SR / SB / LS | USE_FT4060300.21 or USE_FT4060300.48 or USE_FT4060300.59 | - | - | CHANGE TO FS IF MODE IS IN SR, SB or LS | L1 | FS /OS | - |
| 5 | L1 | FS /OS | Train is moving | I | INT | - | L1 | FS /OS | - |
| 6 | L1 | FS /OS | The driver requests the speed and distance monitoring information to be displayed | I | DMI | Optional step. Applies only if the applicable mode is OS | L1 | FS /OS | - |
| 7 | L1 | FS /OS | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | Optional step. Applies only if the applicable mode is OS | L1 | FS /OS | - |
| 8 | L1 | FS /OS | The speed and distance monitoring information is displayed. | O | DMI | The train has reached the location given by D_AXLELOAD and the displayed Permitted Speed is equal to the value of V_AXLELOAD, which was received at the beginning of this TC. It is also displayed in LS mode because the Permitted Speed is lower than the LS mode speed limit. | L1 | FS /OS | - |
| 9 | L1 | FS /OS | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM=V_AXELOAD) is recorded. | O | JRU | - | L1 | FS /OS | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 51 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | FINITE VALUE | - |
| D_TRACKINIT | 15 | FINITE VALUE | Only if Q_TRACKINIT = 1 |
| D_AXLELOAD | 15 | FINITE VALUE | Only if Q_TRACKINIT = 0, D_AXLELOAD and the following variables follow. |
| L_AXLELOAD | 15 | FINITE VALUE | - |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_AXLELOADCAT(n) | 7 | FINITE VALUE | - |
| V_AXLELOAD(n) | 7 | FINITE VALUE | Speed restriction to be applied if the axle load category of the train ( M_AXLELOADCAT(n). |
| N_ITER | 5 | FINITE VALUE | - |
| D_AXLELOAD(k) | 15 | FINITE VALUE | - |
| L_AXLELOAD(k) | 15 | FINITE VALUE | - |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| M_AXLELOADCAT(k,m) | 7 | FINITE VALUE | - |
| V_AXLELOAD(k,m) | 7 | FINITE VALUE | Speed restriction to be applied if the axle load category of the train ( M_AXLELOADCAT(k,m). |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 | FS / OS |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |