| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 4041300 (4.4.13: TRIP) Total: 2 Test case(s) |  |
| REF: | Subset-076-5-2-4041300 |
| ISSUE: | Version 3.2.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 2.9.0 31.03.2013 | All | First version/review | DLR |
| Version 2.9.1 03.12.2013 | All | review | DLR |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.2.0 31.03.2017 | All | Baseline 3 1st maintenance release version - Bug fixing | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.4.13: TRIP |  | 4041300 |
| Test Case of Feature | Permanent application of emergency brakes |  | 1 |
| Applicable Mode/Level Combinations | L0 / L1 / L2 / L3 / L NTC: TR |  |  |
| Target of Test | The target of this test case is to check that: • the emergency brake command is permanently transmitted from TIU to train as long as in TR mode. • the emergency brake command is recorded in JRU. • no emergency brake revocation is recorded in JRU as long as in TR mode. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.4.13.1.2 |  |
|  |  | Subset-026-4.5.2.1 table - Command Emergency Brake |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the TIU that: - the “apply emergency brake”-command is permanently transmitted from TIU to train equipment (emergency brake) as long as in TR mode. Check on the JRU that: - the emergency brake command is recorded. - no emergency brake revocation is recorded as long as in TR mode. |
| Constraints | - The TR mode is indicated on DMI. - mode changes to and from TR are not tested in The present feature. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 | TR |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1/ L2/ L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Current mode is DISPLAYED. | O | DMI | The transition to TR mode has just taken place. So the EB is not jet applied. The EB will apply in Step 1. |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Emergency brake commanded | O | TIU | - | N/A | N/A | - |
| 2 | N/A | N/A | The Status symbol "Service Brake or Emergency Brake intervention" is displayed | O | DMI | - | N/A | N/A | - |
| 3 | N/A | N/A | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | N/A | N/A | - |
| 4 | N/A | N/A | NOT Emergency brake not commanded | O | TIU | - | N/A | N/A | - |
| 5 | N/A | N/A | NOT The Status symbol "Service Brake or Emergency Brake intervention" is removed | O | DMI | - | N/A | N/A | - |
| 6 | N/A | N/A | NOT EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=0) is recorded | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 | TR |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1/ L2/ L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Current mode is DISPLAYED. EB application is DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.4.13: TRIP |  | 4041300 |
| Test Case of Feature | Closing the desk in Trip mode doesn’t allow any action by driver, except mode change to isolation |  | 2 |
| Applicable Mode/Level Combinations | L0 / L1 / L2 / L3 / L NTC: TR / IS |  |  |
| Target of Test | The target of this test case is to check that: • No mode transition or driver interaction is possible after closing the desk in Trip mode, except change to isolation |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.4.13.1.6 |  |
|  |  | Subset-026-4.4.3.1.2 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the TIU that: - the “apply emergency brake”-command is permanently transmitted from TIU to train equipment (emergency brake) as long as in TR mode. - Closing the desk of the engine is APPLIED Check on the DMI that: - Mode change from Trip to Isolation is DISPLAYED Check on the JRU that: - the emergency brake command is RECORDED. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 | TR |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1/ L2/ L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| EB is APPLIED | O | TIU | - |
| Current mode is DISPLAYED. | O | DMI | Indication of TR mode |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / LNTC / L1 / L2 / L3 | TR | Cab not active | I | TIU | Closing the desk while being in Trip mode will not cause a mode change but no interaction with the driver shall be possible as long as the desk is closed. | L0 / LNTC / L1 / L2 / L3 | TR | - |
| 2 | L0 / LNTC / L1 / L2 / L3 | TR | CAB STATUS (NID_MESSAGE_JRU=38; M_MODE=7) is recorded | O | JRU | - | L0 / LNTC / L1 / L2 / L3 | TR | - |
| 3 | L0 / LNTC / L1 / L2 / L3 | TR | The driver isolates the ERTMS/ETCS on-board equipment. | I | DMI | - | L0 / LNTC / L1 / L2 / L3 | IS | - |
| 4 | L0 / LNTC / L1 / L2 / L3 | IS | ETCS isolated | O | TIU | - | L0 / LNTC / L1 / L2 / L3 | IS | - |
| 5 | L0 / LNTC / L1 / L2 / L3 | IS | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=< 0001 0010>) is recorded | O | JRU | - | L0 / LNTC / L1 / L2 / L3 | IS | - |
| 6 | L0 / LNTC / L1 / L2 / L3 | IS | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=10) is recorded | O | JRU | - | L0 / LNTC / L1 / L2 / L3 | IS | - |
| 7 | L0 / LNTC / L1 / L2 / L3 | IS | Indication of IS mode. | O | DMI | From ERA_ERTMS_015560 v330.pdf DMI specification: 5.6.1.1 The ERTMS/ETCS DMI shall offer a means to isolate the ERTMS/ETCS on-board equipment. 5.6.1.1.1 Note: The location and form of this are implementation dependent. Permanent indication of Isolation status (The way of indication has to be clarified with the OBU supplier). No display of current mode while system is in mode “isolation” | L0 / LNTC / L1 / L2 / L3 | IS | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 10 | IS |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1/ L2/ L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| No Display | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |