| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 5100315 (5.10.3.15: Transition initiated by driver) Total: 4 Test case(s) |  |
| REF: | Subset-076-5-2-5100315 |
| ISSUE: | Version 3.2.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 2.9.0 31.03.2013 | All | Creation / New in BS3 | INECO |
| Version 2.9.1 16.05.2013 | All | First version / TCs Template updated; TCs not affected by UNISIG remarks from 30/04/2013 - “Followed By” and “Proceeded By” links kept. | INECO |
| Version 2.9.2 01.08.2013 | All | Second version/Internal modification; TC_Editorv27; Correction of Requirements blank space; Requirements related to table 4.7.2- Additional Data removed from TC1. | INECO |
| Version 2.9.3 03.01.2014 | TC1 | Third version/ Requirement related with table 4.5.2 inserted. | INECO |
| Version 2.9.4 03.01.2014 | All | Fourth version/ Internal version: Cross-reference checked; Old reference removed. | INECO |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.2.0 31.03.2017 | All | Baseline 3 1st maintenance release version - Bug fixing | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.3.15: Transition initiated by driver |  | 5100315 |
| Test Case of Feature | Train is at standstill and a radio communication session is opened. The driver changes the level from 2 or 3 to any other stored in the table of priority. The system switches to selected level. On-board equipment reports the new level to the RBC. When receiving the level change report, the RBC orders the communication session to be terminated. |  | 1 |
| Applicable Mode/Level Combinations | L2: FS/ LS/ SR/ OS/ NL L3: FS/ LS/ SR/ OS/ NL |  |  |
| Target of Test | The target of this test is to check that: The on-board switches to selected level after driver selection amongst those stored in the table of priority, The on-board equipment reports the new level to the RBC. RBC orders the communication session to be terminated when the report has been received. The on-board switches to the level selected by the driver. The function “Report train position when change of level due to driver request“is active. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.18.4.2.4 |  |
|  |  | Subset-026-3.18.4.2.5 |  |
|  |  | Subset-026-5.10.3.15.3 |  |
|  |  | Subset-026-4.5.2.1 table - Report train position when change of level due to driver request |  |
|  |  | Subset-026-8.6.4 Message 136: Train Position Report |  |
|  |  | Subset-026-7.4.3.1 Packet Number 0: Position Report |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The level initially displayed on the DMI changes to the one selected by the driver amongst those stored in the table of priority. Check on the JRU that : The new current level is RECORDED Position report (message 136) is RECORDED. Termination of the communication session is RECORDED. (only if the selected level is different from L2 or 3) |
| Constraints | Train at standstill.(V_TRAIN=0) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 11 / 12 | FS/ OS/ SR/ NL/ LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2/L3 |
| Table of priority of trackside supported levels | STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level 2" or “Level 3” is displayed | O | DMI | Symbols LE04/LE05 |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill. | I | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2 / L3 | FS / LS / SR / OS / NL | Train is at standstill. | I | INT | (V_TRAIN=0; TIME= 60) | L2 / L3 | FS / LS / SR / OS / NL | - |
| 2 | L2 / L3 | FS / LS / SR / OS / NL | Driver selects "level X". | I | DMI | Level X different from the applicable one and among those stored in the table of priority.(L0/LNTC/L1) | L2 / L3 | FS / LS / SR / OS / NL | - |
| 3 | L2 / L3 | FS / LS / SR / OS / NL | The driver confirms "Level" | I | DMI | - | L2 / L3 | FS / LS / SR / OS / NL | - |
| 4 | L2 / L3 | FS / LS / SR / OS / NL | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | - | L2 / L3 | FS / LS / SR / OS / NL | - |
| 5 | L2 / L3 | FS / LS / SR / OS / NL | The Level symbol "Level 0/NTC/1" is displayed | O | DMI | One of the Symbols LE01/LE02/LE03 | LX | FS / LS / SR / OS / NL | - |
| 6 | LX | FS / LS / SR / OS / NL | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; Bit01=1/ Bit02=1/Bit03=1) is recorded | O | JRU | - | LX | FS / LS / SR / OS / NL | - |
| 7 | LX | FS / LS / SR / OS / NL | SA-DATA. Request with Euroradio message (NID_MESSAGE=136) containing packet 0 is transmitted. | O | RTM | Level change report | LX | FS / LS / SR / OS / NL | - |
| 8 | LX | FS / LS / SR / OS / NL | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | LX | FS / LS / SR / OS / NL | - |
| 9 | LX | FS / LS / SR / OS / NL | RBC orders the communication session to be terminated | - | - | USE_FT3050500.5 | LX | FS / LS / SR / OS / NL | - |

| Step 7: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | - |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | L0 / LNTC / L1 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 11 / 12 | FS/ OS/ SR/ NL/ LS |
| Radio communication session | NOT ESTABLISHED | - |
| ERTMS/ETCS level | FINITE VALUE | CHANGED to LX (X = different from the initial one) |
| Table of priority of trackside supported levels | STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol “Level X” is displayed | O | DMI | new selected level (symbol LE0X) |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill. | I | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.3.15: Transition initiated by driver |  | 5100315 |
| Test Case of Feature | After SoM, the Driver manually changes level to L2/3 and one Mobile Terminal is registered to a Radio Network and a valid RBC ID/ phone number is available. Immediately after the ERTMS/ETCS on-board equipment establishes a communication session with the RBC. |  | 2 |
| Applicable Mode/Level Combinations | L0 : UN/ SB/ NL LNTC : SB/ NL/ SN L1: FS/ LS/ OS/ SR/ SB/ TR/ NL/ RV |  |  |
| Target of Test | The target of this test is to check that (after a Start of Mission): The driver manually changes to level 2 or 3 On-board Equipment immediately establishes a communication session with the available RBC |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.5.3.4 |  |
|  |  | Subset-026-3.5.3.4 d) |  |
|  |  | Subset-026-5.10.3.15.2 |  |
|  |  | Subset-026-5.10.3.15.2 a) |  |
|  |  | Subset-026-8.6.13 Message 155: Initiation of a communication session |  |
|  |  | Subset-026-8.7.12 Message 32: RBC/RIU System Version |  |
|  |  | Subset-026-8.6.17 Message 159: Session established |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The level transition to L2/L3 initiated by the driver is DISPLAYED New selected level (2 or 3) is DISPLAYED Check on the JRU that: DRIVER actions are RECORDED. New current level is RECORDED. The on-board establishing a Radio communication session is RECORDED. |
| Constraints | Train is at standstill. Driver ID is valid Mobile TERMINAL registered to a RADIO NETWORK. RBC IDENTITY + PHONE NUMBER STATUS are VALID. Radio link is not Connected. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 11 / 12/ 13 / 14 | FS/ OS/ SR/ UN/ SB/ TR/ NL/ LS/ SN/ RV |
| Radio communication session | NOT ESTABLISHED | - |
| ERTMS/ETCS level | 0/ 1 / 2 | L0/ LNTC / L1 |
| Table of priority of trackside supported levels | STORED | - |
| Driver ID | STORED | Entered after the desk is opened VALID |
| Radio Network ID | STORED | VALID |
| RBC ID/Phone Number | STORED | VALID |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level X" is displayed | O | DMI | Level X = LE01/ 02 /03 |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill. | I | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0/ LNTC/ L1 | FS/ LS/ OS/ SR/ UN/ SB/ TR/ NL/ SN/ RV | Train is at standstill. | I | INT | (V_TRAIN=0; TIME= 60) | L0/ LNTC/ L1 | FS/ LS/ OS/ SR/ UN/ SB/ TR/ NL/ SN/ RV | - |
| 2 | L0/ LNTC/ L1 | FS/ LS/ OS/ SR/ UN/ SB/ TR/ NL/ SN/ RV | The driver selects "Level 2 or “Level 3" and "Contact last RBC" | I | DMI | Level different from the applicable one and among those stored in the table of priority. | L0/ LNTC/ L1 | FS/ LS/ OS/ SR/ UN/ SB/ TR/ NL/ SN/ RV | - |
| 3 | L0/ LNTC/ L1 | FS/ LS/ OS/ SR/ UN/ SB/ TR/ NL/ SN/ RV | The driver confirms "Level" | I | DMI | - | L0/ LNTC/ L1 | FS/ LS/ OS/ SR/ UN/ SB/ TR/ NL/ SN/ RV | - |
| 4 | L0/ LNTC/ L1 | FS/ LS/ OS/ SR/ UN/ SB/ TR/ NL/ SN/ RV | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | - | L0/ LNTC/ L1 | FS/ LS/ OS/ SR/ UN/ SB/ TR/ NL/ SN/ RV | - |
| 5 | L0/ LNTC/ L1 | FS/ LS/ OS/ SR/ UN/ SB/ TR/ NL/ SN/ RV | The Level symbol "Level 2"/ "Level 3" is displayed | O | DMI | One of the Symbols LE04 or LE05 | L2/L3 | FS/ LS/ OS/ SR/ UN/ SB/ TR/ NL/ SN/ RV | - |
| 6 | L2/L3 | FS/ LS/ OS/ SR/ UN/ SB/ TR/ NL/ SN/ RV | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; Bit04=1/ Bit05=1) is recorded | O | JRU | - | L2/L3 | FS/ LS/ OS/ SR/ UN/ SB/ TR/ NL/ SN/ RV | - |
| 7 | L2/L3 | FS/ LS/ OS/ SR/ UN/ SB/ TR/ NL/ SN/ RV | SA-CONNECT. Request is transmitted | O | RTM | Immediately, the on-board establishes a communication session with the RBC | L2/L3 | FS/ LS/ OS/ SR/ UN/ SB/ TR/ NL/ SN/ RV | - |
| 8 | L2/L3 | FS/ LS/ OS/ SR/ UN/ SB/ TR/ NL/ SN/ RV | SA-CONNECT. Confirm is received | I | RTM | - | L2/L3 | FS/ LS/ OS/ SR/ UN/ SB/ TR/ NL/ SN/ RV | - |
| 9 | L2/L3 | FS/ LS/ OS/ SR/ UN/ SB/ TR/ NL/ SN/ RV | SA-DATA. Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted | O | RTM | - | L2/L3 | FS/ LS/ OS/ SR/ UN/ SB/ TR/ NL/ SN/ RV | - |
| 10 | L2/L3 | FS/ LS/ OS/ SR/ UN/ SB/ TR/ NL/ SN/ RV | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | L2/L3 | FS/ LS/ OS/ SR/ UN/ SB/ TR/ NL/ SN/ RV | - |
| 11 | L2/L3 | FS/ LS/ OS/ SR/ UN/ SB/ TR/ NL/ SN/ RV | SA-DATA. Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32) is received | I | RTM | - | L2/L3 | FS/ LS/ OS/ SR/ UN/ SB/ TR/ NL/ SN/ RV | - |
| 12 | L2/L3 | FS/ LS/ OS/ SR/ UN/ SB/ TR/ NL/ SN/ RV | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | Communication session established for on-board. | L2/L3 | FS/ LS/ OS/ SR/ UN/ SB/ TR/ NL/ SN/ RV | - |
| 13 | L2/L3 | FS/ LS/ OS/ SR/ UN/ SB/ TR/ NL/ SN/ RV | SA-DATA. Request with Euroradio Message "Session Established" (NID_MESSAGE=159) is transmitted. | O | RTM | - | L2/L3 | FS/ LS/ OS/ SR/ UN/ SB/ TR/ NL/ SN/ RV | - |
| 14 | L2/L3 | FS/ LS/ OS/ SR/ UN/ SB/ TR/ NL/ SN/ RV | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | Communication session established for trackside | L2/L3 | FS/ LS/ OS/ SR/ UN/ SB/ TR/ NL/ SN/ RV | - |

| Step 9: Radio Message 155 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 155 | Initiation of a communication session |
| L_MESSAGE | 10 | 74 | - |
| T_TRAIN | 32 | FINITE VALUE | Time of the train |
| NID_ENGINE | 24 | FINITE VALUE | ETCS-ID of the Train |

| Step 11: Radio Message 32 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 32 | RBC/RIU System Version |
| L_MESSAGE | 10 | 82 | - |
| T_TRAIN | 32 | FINITE VALUE | Time of the train approximated by RBC |
| M_ACK | 1 | 0 | No ack |
| NID_LRBG | 24 | FINITE VALUE | - |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |

| Step 13: Radio Message 159 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 159 | Session established |
| L_MESSAGE | 10 | 74 | In case of including optional packets, length would be greater |
| T_TRAIN | 32 | FINITE VALUE | Time of the train |
| NID_ENGINE | 24 | FINITE VALUE | ETCS-ID of the Train |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 11 / 12/ 13 / 14 | FS/ OS/ SR/ UN/ SB/ TR/ NL/ LS/ SN/ RV |
| Radio communication session | ESTABLISHED | CHANGED |
| ERTMS/ETCS level | 3 / 4 | CHANGED to L2/L3 VALID |
| Table of priority of trackside supported levels | STORED | - |
| Driver ID | STORED | - |
| Radio Network ID | STORED | - |
| RBC ID/Phone Number | STORED | - |
| RBC/RIU System Version | STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level 2/ Level 3" is displayed | O | DMI | new selected level (symbol LE04 or LE05) |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill. | I | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.3.15: Transition initiated by driver |  | 5100315 |
| Test Case of Feature | The driver enters/validates the RBC contact information (RBC Identity + Phone Number Status).The ERTMS/ETCS on-board equipment establishes a communication session with the RBC. After SoM and after the driver has changed manually the level to L2/3. (covered by FT3180400.1) |  | 3 |
| Applicable Mode/Level Combinations | L2 : FS/ LS/ OS/ SR / PT /NL L3: FS/ LS /OS /SR / PT /NL |  |  |
| Target of Test | The target of this test is to check that, after a Start of Mission after the driver has changed manually to level to L2/3 : The driver selects “Radio data” from Radio data window list, because no valid RBC ID/Phone Number stored on-board The driver enters and confirms the RBC ID/Phone Number The on-board equipment establishes a radio communication session with the registered RBC. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-5.10.3.15.2 |  |
|  |  | Subset-026-5.10.3.15.2 b) |  |
|  |  | Subset-026-8.6.13 Message 155: Initiation of a communication session |  |
|  |  | Subset-026-8.7.12 Message 32: RBC/RIU System Version |  |
|  |  | Subset-026-8.6.17 Message 159: Session established |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: “RBC data” is available for driver selection Check on the JRU that: The on-board establishing a Radio communication session is RECORDED. |
| Constraints | Train is at standstill. If the level transition leads to TR mode, the request for RBC contact information is only displayed once the ERTMS/ETCS on-board equipment is in PT mode. Driver ID is valid. LEVEL status is Valid. Mobile TERMINAL is registered to RADIO NETWORK and RBC IDENTITY + PHONE NUMBER STATUS are INVALID. Radio communication session is not established. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 8 / 11 / 12 | FS / OS / SR / PT / NL / LS |
| Radio communication session | NOT ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2/ L3 |
| Table of priority of trackside supported levels | STORED | - |
| Driver ID | STORED | Entered after the desk is opened VALID |
| Radio Network ID | STORED | VALID |
| RBC ID/Phone Number | NOT STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The data from "RBC contact" window are displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill. | I | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is at standstill. | I | INT | (V_TRAIN=0; TIME= 60) | N/A | N/A | - |
| 2 | N/A | N/A | The driver presses "RBC data". | I | DMI | - | N/A | N/A | - |
| 3 | N/A | N/A | The driver enters "RBC data". | I | DMI | RBC ID/ Phone number | N/A | N/A | - |
| 4 | N/A | N/A | The driver confirms "RBC data". | I | DMI | RBC ID/ Phone number | N/A | N/A | - |
| 5 | N/A | N/A | ADDITIONAL DATA (NID_MESSAGE_JRU=24) is recorded. | O | JRU | The variable Q_RBCENTRY=2 (Enter RBC data) and the variables NID_C, NID_RBC and NID_RADIO identical to the data entered by the driver. | N/A | N/A | - |
| 6 | N/A | N/A | SA-CONNECT. Request is transmitted | O | RTM | the on-board initiates a radio communication session with the RBC | N/A | N/A | - |
| 7 | N/A | N/A | SA-CONNECT. Confirm is received | I | RTM | - | N/A | N/A | - |
| 8 | N/A | N/A | SA-DATA. Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted | O | RTM | - | N/A | N/A | - |
| 9 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 10 | N/A | N/A | SA-DATA. Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32) is received | I | RTM | - | N/A | N/A | - |
| 11 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | Communication session established for on-board. | N/A | N/A | - |
| 12 | N/A | N/A | SA-DATA. Request with Euroradio Message "Session Established" (NID_MESSAGE=159) is transmitted. | O | RTM | - | N/A | N/A | - |
| 13 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | Communication session established for trackside | N/A | N/A | - |

| Step 8: Radio Message 155 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 155 | Initiation of a communication session |
| L_MESSAGE | 10 | 74 | - |
| T_TRAIN | 32 | FINITE VALUE | Time of the train |
| NID_ENGINE | 24 | FINITE VALUE | ETCS-ID of the Train |

| Step 10: Radio Message 32 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 32 | RBC/RIU System Version |
| L_MESSAGE | 10 | 82 | - |
| T_TRAIN | 32 | FINITE VALUE | Time of the train approximated by RBC |
| M_ACK | 1 | 0 | No ack |
| NID_LRBG | 24 | FINITE VALUE | - |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |

| Step 12: Radio Message 159 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 159 | Session established |
| L_MESSAGE | 10 | 74 | In case of including optional packets, length would be greater |
| T_TRAIN | 32 | FINITE VALUE | Time of the train |
| NID_ENGINE | 24 | FINITE VALUE | ETCS-ID of the Train |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 8 / 11 / 12 | FS / OS / SR / PT / NL / LS |
| Radio communication session | ESTABLISHED | CHANGED |
| ERTMS/ETCS level | 3/4 | L2/L3 |
| Table of priority of trackside supported levels | STORED | - |
| Driver ID | STORED | - |
| Radio Network ID | STORED | VALID |
| RBC ID/Phone Number | STORED | VALID CHANGED |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill. | I | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.3.15: Transition initiated by driver |  | 5100315 |
| Test Case of Feature | The driver enters/validates the RBC contact information (Radio Network ID).The ERTMS/ETCS on-board equipment establishes a communication session with the RBC. After SoM and after the driver has changed manually to level to L2/3. (covered by FT3180400.2) |  | 4 |
| Applicable Mode/Level Combinations | L2 : FS/ LS/ OS/ SR/ PT /NL L3: FS/ LS /OS /SR / PT /NL |  |  |
| Target of Test | The target of this test is to check that, after a Start of Mission after the driver has changed manually to level to L2/3 : The driver selects “Radio network ID” from RBC contact information window, because no Mobile Terminal is registered to a Radio Network The driver enters the Radio network ID information The on-board equipment establishes a radio communication session with the registered RBC. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-5.10.3.15.2 |  |
|  |  | Subset-026-5.10.3.15.2 b) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: “RBC network ID” is available for driver selection Check on the JRU that: The on-board establishing a Radio communication session is RECORDED. |
| Constraints | Train is at standstill. If the level transition leads to TR mode, the request for RBC contact information is only displayed once the ERTMS/ETCS on-board equipment is in PT mode. Driver ID is valid. LEVEL status is Valid. No Mobile TERMINAL is registered to the RADIO NETWORK. Radio communication session is not established. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 8 / 11 / 12 | FS / OS / SR / PT / NL / LS |
| Radio communication session | NOT ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2/ L3 |
| Table of priority of trackside supported levels | STORED | - |
| Driver ID | STORED | Entered after the desk is opened VALID |
| Radio Network ID | NOT STORED | - |
| RBC ID/Phone Number | NOT STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill. | I | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2/L3 | FS / OS / SR / PT / NL / LS | Train is at standstill. | I | INT | (V_TRAIN=0; TIME= 60) | L2/L3 | FS / OS / SR / PT / NL / LS | - |
| 2 | L2/L3 | FS / OS / SR / PT / NL / LS | The driver presses "Radio network ID". | I | DMI | - | L2/L3 | FS / OS / SR / PT / NL / LS | - |
| 3 | L2/L3 | FS / OS / SR / PT / NL / LS | The driver selects a new “Radio network ID ". | I | DMI | - | L2/L3 | FS / OS / SR / PT / NL / LS | - |
| 4 | L2/L3 | FS / OS / SR / PT / NL / LS | The driver confirms “Radio network ID ". | I | DMI | Validates the Radio network ID | L2/L3 | FS / OS / SR / PT / NL / LS | - |
| 5 | L2/L3 | FS / OS / SR / PT / NL / LS | ADDITIONAL DATA (NID_MESSAGE_JRU=24) is recorded. | O | JRU | The variable NID_MN identical to the data selected by the driver. | L2/L3 | FS / OS / SR / PT / NL / LS | - |
| 6 | L2/L3 | FS / OS / SR / PT / NL / LS | The driver enters the RBC contact information (RBC Identity + Phone Number Status).The ERTMS/ETCS on-board equipment establishes a communication session with the RBC. | - | - | USE_FT5100315.3 | L2/L3 | FS / OS / SR / PT / NL / LS | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 8 / 11 / 12 | FS / OS / SR / SB / PT / NL / LS |
| Radio communication session | ESTABLISHED | CHANGED |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Table of priority of trackside supported levels | STORED | - |
| Driver ID | STORED | - |
| Radio Network ID | STORED | VALID |
| RBC ID/Phone Number | STORED | VALID |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill. | I | INT | - |