| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 5100100 (5.10.1: Level Transitions General requirements) Total: 38 Test case(s) |  |
| REF: | Subset-076-5-2-5100100 |
| ISSUE: | Version 3.2.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 0.0.1 23.03.2012 | All | First version/review | DLR |
| Version 2.9.0 31.03.2013 | All | First version/review Baseline 3 | MULTITEL |
| Version 2.9.1 05.07.2013 | All | Template standardised with a dedicated tool, TCs renumbered, Answer to Review001, Add Test Cases 1-2 | MULTITEL |
| Version 2.9.2 03.10.2013 | TC1, TC2, TC3, TC4, TC6, TC12, TC18, TC22, TC33, TC35 | Modifications requested by Review001_Subset076-5-2-5100100_v291 | MULTITEL |
| Version 2.9.3 24.10.2013 | All | Modifications related to issue 8 from Review002_Subset-076-5-2-5100100_v291 | MULTITEL |
| Version 2.9.4 05.12.2013 | All | Modifications related to issue 9 from Review002_Subset-076-5-2-5100100_v291 | MULTITEL |
| Version 2.9.5 20.12.2013 | All | Modifications related to issue 10 and 11 from Review003_Subset076-5-2-3-5100100_v294 | MULTITEL |
| Version 2.9.6 09.01.2014 | TC3,6,18,35 | Modifications related to issue 1 from Review001_Subset076-5-2-3-5100100_v295 | MULTITEL |
| Version 2.9.7 10.01.2014 | TC35 | Modifications related to issue 2 from Review001_Subset076-5-2-3-5100100_v296 | MULTITEL |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.2.0 31.03.2017 | All | Baseline 3 1st maintenance release version - Bug fixing | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.1: Level Transitions General requirements |  | 5100100 |
| Test Case of Feature | The train is running in PS mode and receives multiples orders to switch immediately or conditionnaly from LX at the current position. Only the last set of Level Transition Information is stored AND the system switch only when another mode than SH or PS has been entered. |  | 1 |
| Applicable Mode/Level Combinations | L0 : PS LNTC : PS L1 : PS L2 : PS L3 : PS |  |  |
| Target of Test | The target of this test is to check that: Only the last set of Level Transition Information is stored The system switch only when another mode than SH or PS has been entered. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-5.10.1.6.1 |  |
|  |  | Subset-026-4.4.20.1.11 |  |
|  |  | Subset-026-4.8.4.2 Level Transition Order and Conditional Level Transition Order |  |
|  |  | Subset-026-4.8.4.2 [7] |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: Level Y indication is DISPLAYED only when another mode than SH or PS has been entered. Check on the JRU that: Two immediate or conditional Level Transition Orders are received : LX/LW and LX/LY. They could indifferently be transmitted in 2 different types of packets: Packet 41: “Level transition order” is RECORDED with M_LEVELTR >< X, D_LEVELTR = 0 or = 32767. Packet 46: “Conditional Level Transition Order” is RECORDED with M_LEVELTR >< X. Level X is RECORDED while the train is still in PS mode. New Level Y is RECORDED only when SB mode has been entered. |
| Constraints | The train is running in PS A “Stop Shunting on desk opening” information is stored onboard |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 15 | PS |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | LX | PS | The train receives an order to switch immediately to Level W from balise group BGa. | I | BTM | Packet 41: “Level Transition Order” is RECEIVED. D_LEVELTR= 0 or = 32767 M_LEVELTR= LVL_W Or Packet 46: “Conditional Level Transition Order” is RECEIVED. M_LEVELTR= LVL_W | LX | PS | - |
| 2 | LX | PS | Balise Group message, including Packet 41 or 46, is RECORDED. | O | JRU | - | LX | PS | - |
| 3 | LX | PS | The train receives an order to switch immediately to Level Y from balise group BGa. | I | BTM | Packet 41: “Level Transition Order” is RECEIVED. D_LEVELTR= 0 or = 32767 M_LEVELTR= LVL_Y Or Packet 46: “Conditional Level Transition Order” is RECEIVED. M_LEVELTR= LVL_Y | LX | PS | - |
| 4 | LX | PS | Balise Group message, including Packet 41 or 46, is RECORDED. | O | JRU | - | LX | PS | - |
| 5 | LX | PS | The Level X is RECORDED | O | JRU | - | LX | PS | - |
| 6 | LX | PS | The driver opens the desk | I | TIU | - | LX | PS | - |
| 7 | LX | PS | The train switches to SB | O | DMI | - | LX | SB | - |
| 8 | LX | SB | Level Y is DISPLAYED. | O | DMI | The On-board equipment switches to Level Y | LY | SB | - |
| 9 | LY | SB | Level Y is RECORDED | O | JRU | - | LY | SB | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | First |
| N_TOTAL | 3 | 001 | 2 balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 41 | Level Transition Order (an alternative option is to use packet 46 Conditional Level Transition Order) |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | 0 or = 32767 | Switch immediately |
| M_LEVELTR | 3 | LVL_W | Transition to level W |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL_TR = 1 (NTC) |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 001 | Second |
| N_TOTAL | 3 | FINITE VALUE | 2 balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 3: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | First |
| N_TOTAL | 3 | 001 | 2 balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 41 | Level Transition Order (an alternative option is to use packet 46 Conditional Level Transition Order) |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | 0 or = 32767 | Switch immediately |
| M_LEVELTR | 3 | LVL_Y | Transition to level Y |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL_TR = 1 (NTC) |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 3: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 001 | Second |
| N_TOTAL | 3 | 001 | 2 balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | SB | 6 |
| ERTMS/ETCS level | LVL_Y | LY |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.1: Level Transitions General requirements |  | 5100100 |
| Test Case of Feature | The train is running in SH mode and receives multiples orders to switch immediately or conditionnaly from LX at the current position. Only the last set of Level Transition Information is stored AND the system switch only when another mode than SH or PS has been entered. |  | 2 |
| Applicable Mode/Level Combinations | L0 : SH LNTC : SH L1 : SH L2 : SH L3 : SH |  |  |
| Target of Test | The target of this test is to check that: Only the last set of Level Transition Information is stored The system switch only when another mode than SH or PS has been entered. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-5.10.1.6.1 |  |
|  |  | Subset-026-4.8.4.2 Level Transition Order and Conditional Level Transition Order |  |
|  |  | Subset-026-4.8.4.2 [7] |  |
|  |  | Subset-026-4.4.8.1.5 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: Level X indication is DISPLAYED. Level Y indication is DISPLAYED only when another mode than SH or PS has been entered. Check on the JRU that: Two immediate or conditional Level Transition Orders are received : LX/LW and LX/LY. They could indifferently be transmitted in 2 different types of packets: Packet 41: “Level transition order” is RECORDED with M_LEVELTR >< X, D_LEVELTR = 0 or = 32767. Packet 46: “Conditional Level Transition Order” is RECORDED with M_LEVELTR >< X. New Level Y is RECORDED only when SB mode has been entered. |
| Constraints | The train is running in SH |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 | SH |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Level X is DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | LX | SH | Level X is DISPLAYED. | O | DMI | - | LX | SH | - |
| 2 | LX | SH | The train receives an order to switch immediately to Level W from balise group BGa. | I | BTM | Packet 41: “Level Transition Order” is RECEIVED. D_LEVELTR= 0 or = 32767 M_LEVELTR= LVL_W Or Packet 46: “Conditional Level Transition Order” is RECEIVED. M_LEVELTR= LVL_W | LX | SH | - |
| 3 | LX | SH | Balise Group message, including Packet 41 or 46, is RECORDED. | O | JRU | - | LX | SH | - |
| 4 | LX | SH | The train receives an order to switch immediately to Level Y from balise group BGa. | I | BTM | Packet 41: “Level Transition Order” is RECEIVED. D_LEVELTR= 0 or = 32767 M_LEVELTR= LVL_Y Or Packet 46: “Conditional Level Transition Order” is RECEIVED. M_LEVELTR= LVL_Y | LX | SH | - |
| 5 | LX | SH | Balise Group message, including Packet 41 or 46, is RECORDED. | O | JRU | - | LX | SH | - |
| 6 | LX | SH | Level X is (still) DISPLAYED. | O | DMI | - | LX | SH | - |
| 7 | LX | SH | The train reaches standstill. | O | DMI | - | LX | SH | - |
| 8 | LX | SH | The driver selects “exit shunting” | I | DMI | - | LX | SH | - |
| 9 | LX | SH | The driver’s selection of “exit shunting” is RECORDED | O | JRU | - | LX | SH | - |
| 10 | LX | SH | SB is DISPLAYED | O | DMI | The train switches to SB | LX | SB | - |
| 11 | LX | SB | SB is RECORDED | O | JRU | - | LX | SB | - |
| 12 | LX | SB | Level Y is DISPLAYED. | O | DMI | The On-board equipment switches to Level Y. | LY | SB | - |
| 13 | LY | SB | Level Y is RECORDED | O | JRU | - | LY | SB | - |

| Step 2: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | First |
| N_TOTAL | 3 | 001 | 2 balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 41 | Level Transition Order (an alternative option is to use packet 46 Conditional Level Transition Order) |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | 0 or = 32767 | Switch immediately |
| M_LEVELTR | 3 | LVL_W | Transition to level W |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL_TR = 1 (NTC) |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 2: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 001 | Second |
| N_TOTAL | 3 | 001 | 2 balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 4: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | First |
| N_TOTAL | 3 | 001 | 2 balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 41 | Level Transition Order (an alternative option is to use packet 46 Conditional Level Transition Order) |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | 0 or = 32767 | Switch immediately |
| M_LEVELTR | 3 | LVL_Y | Transition to level Y |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL_TR = 1 (NTC) |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 4: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 001 | Second |
| N_TOTAL | 3 | 001 | 2 balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | SB | 6 |
| ERTMS/ETCS level | LVL_Y | LY |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.1: Level Transitions General requirements |  | 5100100 |
| Test Case of Feature | The train receives from an infill balise group the Level Transition announcement L1/L1. The driver is NOT informed. |  | 3 |
| Applicable Mode/Level Combinations | L1 : FS, LS |  |  |
| Target of Test | The target of this test case is to check that: The system receives an announcement to switch to level 1, as infill information, i.e., after Packet 136 The on-board equipment DOESN’T displays the announcement of the Level Transition to the same level L1 after receiving the announcement of the transition from the infill balise group BG |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.6.3.2.3 |  |
|  |  | Subset-026-4.7.2 table - Output information Level transition announcement |  |
|  |  | Subset-026-4.8.3.1.1 Level Transition Order |  |
|  |  | Subset-026-4.8.4.2 Level Transition Order and Conditional Level Transition Order |  |
|  |  | Subset-026-5.10.1.3 |  |
|  |  | Subset-026-7.4.2.9 Packet Number 41: Level Transition Order |  |
|  |  | Subset-026-7.4.2.32 Packet Number 136: Infill location reference |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that The Level Transition announcement is NOT DISPLAYED after Packet 41: “Level Transition Order” is received on-board. Check on the JRU that Infill Balise Group message, including the announcement of the Level Transition L1/L1 is RECORDED. No indication is recorded. |
| Constraints | Unique announcement of level transition (L1/L1 as described in the Packet 41 defined in this test case with N_ITER = 0). No announcement of level transition is displayed to the driver. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 | - | An announcement of level transition to Level 1 (Packet 41) is RECEIVED from an infill Balise Group: D_LEVELTR <> 0 and <> 32767 M_LEVELTR= 2 (Level 1) | I | BTM | Order to switch to level 1 at further location and this announcement will not result in a change of the on-board level. | Same | Same | - |
| 2 | L1 | - | NO indication of Announcement of Level 1 is DISPLAYED. | O | DMI | - | Same | Same | - |
| 3 | L1 | - | NO indication of Announcement of Level 1 is RECORDED | O | JRU | - | Same | Same | - |
| 4 | L1 | - | The infill Balise Group message, including Packets 136 and 41,”Level transition order”, is RECORDED. | O | JRU | - | Same | Same | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | First |
| N_TOTAL | 3 | 001 | 2 balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 136 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | 0 | - |
| NIG_BG | 14 | RESERVED | - |
| NID_PACKET | 8 | 41 | Level Transition Order |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | FINITE VALUE | <> 0 and <>32767 |
| M_LEVELTR | 3 | 2 | Transition to level 1 |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL_Y = 1 (NTC) |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 001 | Second |
| N_TOTAL | 3 | 001 | 2 balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 UNCHANGED | FS / LS |
| ERTMS/ETCS level | 2 UNCHANGED | L1 (no change after announcement) |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.1: Level Transitions General requirements |  | 5100100 |
| Test Case of Feature | The train receives from the RBC the Level Transition announcement LX/LX. The driver is NOT informed. |  | 4 |
| Applicable Mode/Level Combinations | L2: FS, LS, OS, SR, SL, SB, TR, PT, NL L3: FS, LS, OS, SR, SL, SB, TR, PT, NL |  |  |
| Target of Test | The target of this test is to check that: The on-board system DOESN’T displays the announcement of the Level Transition to the same level LX after receiving the announcement of the transition from the RBC. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.7.2 table - Output information Level transition announcement |  |
|  |  | Subset-026-4.8.3.1.1 Level Transition Order |  |
|  |  | Subset-026-4.8.4.2 Level Transition Order and Conditional Level Transition Order |  |
|  |  | Subset-026-5.10.1.3 |  |
|  |  | Subset-026-7.4.2.9 Packet Number 41: Level Transition Order |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The Level Transition announcement is NOT DISPLAYED after the train receives from the RBC Packet 41: “Level Transition Order”. Check on the JRU that : The radio message including the announcement of the Level Transition LX/LX is RECORDED. Level transition is announced by means of Packet 41 “Level Transition Order” with M_LEVELTR=M_LEVEL_X and D_LEVELTR <> 0 and <> 32767. |
| Constraints | A cab needs to be active (SB and TR Mode) Unique announcement of level transition (LX/LX as described in the Packet 41 defined in this test case with N_ITER = 0) and this announcement will NOT result in a change of the on-board level. If in PT mode, the packet 41 follows the reception of the information “Recognition of Exit from TR mode” with a more recent time stamp. If the level transition to LNTC is announced, the NID_NTC from Packet 41 is the same that the current NID_NTC. No announcement of level transition is displayed to the driver. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 5 / 6 / 7 / 8 / 11 / 12 / 13 | FS / OS / SR / UN / SL / SB / TR / PT / NL / LS / SN |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | M_LEVEL_X | Level X |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | X | - | An announcement of Level Transition to Level X (radio message with Packet 41) is received from RBC: D_LEVLETR <> 0 and <> 32767 M_LEVELTR= M_LEVEL_X (Level X) | I | RTM | Order to switch to level X at further location and this announcement will not result in a change of the on-board level. | Same | Same | - |
| 2 | X | - | NO indication of announcement of Level X is DISPLAYED. | O | DMI | - | Same | Same | - |
| 3 | X | - | NO indication is RECORDED | O | JRU | - | Same | Same | - |
| 4 | X | - | Radio message , including Packet 41”Level transition order” is RECORDED. | O | JRU | - | Same | Same | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 41 | Level Transition Order |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | FINITE VALUE | <> 0 and <> 32767 |
| M_LEVELTR | 3 | M_LEVEL_X | Transition to level X |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL_X = 1 (NTC), NID_NTC = NID_NTC current |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 5 / 6 / 7 / 8 / 11 / 12 / 13 UNCHANGED | FS / OS / SR / UN / SL / SB / TR / PT / NL / LS / SN |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | M_LEVEL_X UNCHANGED | Level X |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 5

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.1: Level Transitions General requirements |  | 5100100 |
| Test Case of Feature | The train receives from the balise group BGa the Level Transition announcement LX/LX. The driver is NOT informed. |  | 5 |
| Applicable Mode/Level Combinations | L0: UN, SL, SB, TR, NL LNTC: SL, SB, TR, NL, SN L1: FS, LS, OS, SR, SL, SB, TR, NL L2: FS, LS, OS, SR, SL, SB, TR, PT, NL L3: FS, LS, OS, SR, SL, SB, TR, PT, NL |  |  |
| Target of Test | The target of this test is to check that: The on-board equipment DOESN’T displays the announcement of the Level Transition to the same level LX after receiving the announcement of the transition from the balise group BGa. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.7.2 table - Output information Level transition announcement |  |
|  |  | Subset-026-4.8.3.1.1 Level Transition Order |  |
|  |  | Subset-026-4.8.4.2 Level Transition Order and Conditional Level Transition Order |  |
|  |  | Subset-026-5.10.1.3 |  |
|  |  | Subset-026-7.4.2.9 Packet Number 41: Level Transition Order |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The Level Transition announcement is NOT DISPLAYED after Packet 41: “Level Transition Order” is received on-board Check on the JRU that : Balise Group BGa message, including the announcement of the Level Transition LX/LX is RECORDED. Level transition is announced by means of Packet 41 “Level Transition Order” with M_LEVELTR=M_LEVEL_X and D_LEVELTR <> 0 and <> 32767. |
| Constraints | A cab needs to be active (SB and TR Mode) Unique announcement of level transition (LX/LX as described in the Packet 41 defined in this test case with N_ITER = 0) and this announcement will NOT result in a change of the on-board level. If in PT mode, the packet 41 follows the reception of the information “Recognition of Exit from TR mode” with a more recent time stamp. If the level transition to LNTC is announced, the NID_NTC from Packet 41 is the same that the current NID_NTC. No announcement of level transition is displayed to the driver. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 5 / 6 / 7 / 8 / 11 / 12 / 13 | FS / OS / SR / UN / SL / SB / TR / PT / NL / LS / SN |
| ERTMS/ETCS level | M_LEVEL_X | Level X |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | X | - | An announcement of level transition to Level X (Packet 41) is RECEIVED from Balise Group BGa: D_LEVELTR <> 0 and <> 32767 M_LEVELTR= M_LEVEL_X (Level X) | I | BTM | Order to switch to level X at further location and this announcement will not result in a change of the on-board level. | Same | Same | - |
| 2 | X | - | NO indication of Announcement of Level X is DISPLAYED. | O | DMI | - | Same | Same | - |
| 3 | X | - | NO indication is recorded | O | JRU | - | Same | Same | - |
| 4 | X | - | Balise Group BGa message, including Packet 41”Level transition order”, is RECORDED. | O | JRU | - | Same | Same | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | First |
| N_TOTAL | 3 | 001 | 2 balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 41 | Level Transition Order |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | FINITE VALUE | <> 0 and <> 32767 |
| M_LEVELTR | 3 | M_LEVEL_X | Transition to level Y |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL_X = 1 (NTC), NID_NTC = NID_NTC current |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 001 | Second |
| N_TOTAL | 3 | 001 | 2 balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 5 / 6 / 7 / 8 / 11 / 12 / 13 UNCHANGED | FS / OS / SR / UN / SL / SB / TR / PT / NL / LS / SN UNCHANGED |
| ERTMS/ETCS level | M_LEVEL_X UNCHANGED | Level X UNCHANGED |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 6

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.1: Level Transitions General requirements |  | 5100100 |
| Test Case of Feature | The train receives an order to switch immediately from L1 to L0 via infill BG. The driver is informed. |  | 6 |
| Applicable Mode/Level Combinations | L1 : FS, LS |  |  |
| Target of Test | The target of this test is to check that: The system receives an order to switch to level 0 by means of D_LEVELTR = 0, as infill information, i.e., after Packet 136. The on-board equipment displays the announcement of the Level Transition to L0 |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.6.3.2.3 |  |
|  |  | Subset-026-4.8.3.1.1 Level Transition Order |  |
|  |  | Subset-026-4.8.4.2 Level Transition Order and Conditional Level Transition Order |  |
|  |  | Subset-026-7.4.2.32 Packet Number 136: Infill location reference |  |
|  |  | Subset-026-7.4.2.9 Packet Number 41: Level Transition Order |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: Level 0 indication is DISPLAYED. Check on the JRU that: Packet 41: “Level transition order” is RECORDED with M_LEVELTR = 0 and D_LEVELTR = 0. The indication to the driver is recorded |
| Constraints | There must be linking information stored on-board with linking reaction for the following main BG set to “No reaction”. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| Linking | STORED | The linking reaction for the next main BG must be “No reaction” |
| Movement Authority | STORED | Level 1 Movement Authority |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Level 1 is DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 | - | An order to switch to Level 0 is received from an infill balise group Packet 41: “Level Transition Order” is RECEIVED. D_LEVELTR= 0 M_LEVELTR=0 | I | BTM | - | L1 | Same | - |
| 2 | L1 | - | Balise Group message, including Packet 41 ”Level transition order”, is RECORDED. | O | JRU | - | L1 | Same | - |
| 3 | L1 | - | An indication of Announcement of Level 0 is DISPLAYED. | O | DMI | - | L1 | Same | - |
| 4 | L1 | - | An indication of Announcement of Level 0 is RECORDED | O | JRU | - | L1 | Same | - |
| 5 | L1 | - | Level 0 is DISPLAYED. | O | DMI | The train reaches the location of the main BG announced in the linking information, but no BG message is read. The linking reaction for this main BG must be “No reaction”. The On-board equipment switches to Level 0. | L0 | Same | - |
| 6 | L0 | - | New Level 0 is RECORDED. | O | JRU | - | L0 | Same | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | First |
| N_TOTAL | 3 | 001 | 2 balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 136 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | 0 | - |
| NIG_BG | 14 | RESERVED | - |
| NID_PACKET | 8 | 41 | Level Transition Order |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | 0 | - |
| M_LEVELTR | 3 | 0 | Transition to level 0 |
| NID_NTC | 8 | FINITE VALUE | - |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 001 | Second |
| N_TOTAL | 3 | 001 | 2 balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 UNCHANGED | FS / LS UNCHANGED |
| ERTMS/ETCS level | 0 | L0 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| LEVEL 0 is DISPLAYED | O | DMI | The announcement of the level transition L1/L0 at a further position is DELETED from the DMI |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 7

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.1: Level Transitions General requirements |  | 5100100 |
| Test Case of Feature | The train has received one level transition announcement (to Level Y), and it receives a second (to Level W) for the same location. The train executes the last received level transition announcement to level W at the announced location. Comment: This test case is applicable for all possible combinations of Level transitions |  | 7 |
| Applicable Mode/Level Combinations | L0: UN, SL, SB, TR, NL L1: FS, LS, OS, SR, SB, SL, TR, PT, NL L2: FS, LS, OS, SR, SB, SL, TR, PT, NL L3: FS, LS, OS, SR, SB, SL, TR, PT, NL LNTC: SL, SB, TR, NL, SN |  |  |
| Target of Test | The target of this test is to check that: The on-board system executes the last level transition announcement received on-board, i.e. the last announcement of level transition replaces the previous level transition announcement data stored on-board |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-5.10.1.6 |  |
|  |  | Subset-026-5.10.1.5 |  |
|  |  | Subset-026-7.4.2.9 Packet Number 41: Level Transition Order |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The on-board executes only the last received level transition announcement Check on the JRU that : The new level (of the last received level transition announcement) is recorded when the on-board executes the level transition |
| Constraints | The on-board equipment received first the packet 41 with D_LEVELTR_1 and M_LEVELTR_1 (Level Y). Then the on-board equipment receives the second level transition announcement for the same location and with M_LEVELTR_2 (Level W) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| Level Transition Order | STORED | D_LEVELTR_1 <> 0 and <> 32767 M_LEVELTR_1 = Y (Transition to Level Y (First announcement received)) |
| ERTMS/ETCS level | X | LX |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The message “Announcement of Level Y” is DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | X | - | An announcement of level transition to Level W (Packet 41) is RECEIVED from Balise Group BGa: D_LEVELTR_2 <> 0 and <> 32767 M_LEVELTR_2 = W (Level W) | I | BTM | D_LEVELTR_1 and D_LEVELTR_2 point to the same location. | X | Same | - |
| 2 | X | - | Balise Group BGa message, including Packet 41”Level transition order”, is RECORDED. | O | JRU | - | X | Same | - |
| 3 | X | - | An indication of Announcement of Level Y is REMOVED | O | DMI | ('Except in SL and SB modes') | X | Same | - |
| 4 | X | - | Removed indication is recorded | O | JRU | ('Except in SL and SB modes') | X | Same | - |
| 5 | X | - | An indication of Announcement of Level W is DISPLAYED. | O | DMI | ('Except in SL and SB modes') | X | Same | - |
| 6 | X | - | Displayed indication is recorded | O | JRU | ('Except in SL and SB modes') | X | Same | - |
| 7 | X | - | Level W is DISPLAYED | O | DMI | ('Except in SL ') The train is running and reaches the level border location. The border is located by both D_LEVELTR_1 and D_LEVELTR_2. The train switches to Level W | W | Same | - |
| 8 | W | - | New Level is RECORDED | O | JRU | - | W | Same | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | First |
| N_TOTAL | 3 | 001 | 2 balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 41 | Level Transition Order |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | D_LEVELTR_2 | <> 0 and <> 32767 D_LEVELTR_2 points to the same location as D_LEVELTR_1 |
| M_LEVELTR | 3 | M_LEVEL_W | Transition to level W |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL_TR = 1 (NTC) |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 001 | Second |
| N_TOTAL | 3 | 001 | 2 balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS level | W | LW |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 8

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.1: Level Transitions General requirements |  | 5100100 |
| Test Case of Feature | The train has received one level transition announcement at a location, and it receives a second to the same level, but for a different location. The train executes the last received level transition announcement at the announced location. Comment: This test case is applicable for all possible combinations of Level transitions |  | 8 |
| Applicable Mode/Level Combinations | L0: UN, SL, SB, TR, NL L1: FS, LS, OS, SR, SB, SL, TR, PT, NL L2: FS, LS, OS, SR, SB, SL, TR, PT, NL L3: FS, LS, OS, SR, SB, SL, TR, PT, NL LNTC: SL, SB, TR, NL, SN |  |  |
| Target of Test | The target of this test is to check that: The on-board system executes the last level transition announcement received on-board, i.e. the last announcement of level transition replaces the previous level transition announcement data stored on-board |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-5.10.1.6 |  |
|  |  | Subset-026-5.10.1.5 |  |
|  |  | Subset-026-7.4.2.9 Packet Number 41: Level Transition Order |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The on-board executes only the last received level transition announcement Check on the JRU that: The new level is recorded at the new location |
| Constraints | The on-board equipment received first the packet 41 with D_LEVELTR_1 and M_LEVELTR_1 (Level Y). Then the on-board equipment receives the second level transition announcement for another location and with M_LEVELTR_2 (Level Y) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| Level Transition Order | STORED | M_LEVELTR_1 = Y (Transition to Level Y) D_LEVELTR_1 <> 0 and <> 32767 |
| ERTMS/ETCS level | X | LX |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Message “Announcement of Level Y” is DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | X | - | An announcement of level transition to Level Y (Packet 41) is RECEIVED from Balise Group BGa: D_LEVELTR_2 <> 0 and <> 32767 M_LEVELTR_2 = Y (Level Y) | I | BTM | D_LEVELTR_1 and D_LEVELTR_2 point to a different location. | X | Same | - |
| 2 | X | - | Balise Group BGa message, including Packet 41”Level transition order”, is RECORDED. | O | JRU | - | X | Same | - |
| 3 | X | - | No level transition is performed | O | DMI | ('Except in SL ') The train is running and reaches the level border location. The border location is pointed by D_LEVELTR_1. | X | Same | - |
| 4 | X | - | Level Y is DISPLAYED | O | DMI | ('Except in SL ') The train is running and reaches the level border location. The border location is pointed by D_LEVELTR_2. The train switches to Level Y | Y | Same | - |
| 5 | Y | - | New Level is RECORDED | O | JRU | - | Y | Same | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | First |
| N_TOTAL | 3 | 001 | 2 balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 41 | Level Transition Order |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | D_LEVELTR_2 | <> 0 and <> 32767 D_LEVELTR_2 points to another location than D_LEVELTR_1 |
| M_LEVELTR | 3 | M_LEVEL_Y | Transition to level Y |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL_TR = 1 (NTC) |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 001 | Second |
| N_TOTAL | 3 | 001 | 2 balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS level | Y | LY |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 9

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.1: Level Transitions General requirements |  | 5100100 |
| Test Case of Feature | The train has received one level transition announcement to Level Y at a location, and it receives a second to Level W for another location. The train executes the last received level transition announcement at the announced location. Comment: This test case is applicable for all possible combinations of Level transitions |  | 9 |
| Applicable Mode/Level Combinations | L0: UN, SL, SB, TR, NL L1: FS, LS, OS, SR, SB, SL, TR, PT, NL L2: FS, LS, OS, SR, SB, SL, TR, PT, NL L3: FS, LS, OS, SR, SB, SL, TR, PT, NL LNTC: SL, SB, TR, NL, SN |  |  |
| Target of Test | The target of this test is to check that: The on-board system executes the last level transition announcement received on-board, i.e. the last announcement of level transition replaces the previous level transition announcement data stored on-board |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-5.10.1.6 |  |
|  |  | Subset-026-5.10.1.5 |  |
|  |  | Subset-026-7.4.2.9 Packet Number 41: Level Transition Order |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The on-board executes only the last received level transition announcement Check on the JRU that : The new level (of the last received level transition announcement) is recorded when the on-board executes the level transition |
| Constraints | The on-board equipment received first the packet 41 with D_LEVELTR_1 and M_LEVELTR_1 (Level Y). Then the on-board equipment receives the second level transition announcement for another level and another location. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| Level Transition Order | STORED | D_LEVELTR_1 <> 0 and <> 32767 M_LEVELTR_1 = Y (Transition to Level Y) |
| ERTMS/ETCS level | X | LX |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The message “Announcement of Level Y” is DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | X | - | An announcement of level transition to Level W (Packet 41) is RECEIVED from Balise Group BGa: D_LEVELTR_2 <> 0 and <> 32767 M_LEVELTR_2 = W (Level W) | I | BTM | D_LEVELTR_1 and D_LEVELTR_2 point to a different location. | X | Same | - |
| 2 | X | - | Balise Group BGa message, including Packet 41”Level transition order”, is RECORDED. | O | JRU | - | X | Same | - |
| 3 | X | - | An indication of Announcement of Level Y is REMOVED | O | DMI | ('Except in SL and SB modes') | X | Same | - |
| 4 | X | - | Removed indication is recorded | O | JRU | ('Except in SL and SB modes') | X | Same | - |
| 5 | X | - | An indication of Announcement of Level W is DISPLAYED. | O | DMI | ('Except in SL and SB modes') | X | Same | - |
| 6 | X | - | Displayed indication is recorded | O | JRU | ('Except in SL and SB modes') | X | Same | - |
| 7 | X | - | No level transition is performed | O | DMI | ('Except in SL ') The train is running and reaches the level border location. The border location is pointed by D_LEVELTR_1. | X | Same | - |
| 8 | X | - | Level W is DISPLAYED | O | DMI | ('Except in SL ') The train is running and reaches the level border location. The border location is pointed by D_LEVELTR_2. The train switches to Level W | W | Same | - |
| 9 | W | - | New Level is RECORDED | O | JRU | - | W | Same | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | First |
| N_TOTAL | 3 | 001 | 2 balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 41 | Level Transition Order |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | D_LEVELTR_2 | <> 0 and <> 32767 D_LEVELTR_2 points to another location than D_LEVELTR_1 |
| M_LEVELTR | 3 | M_LEVEL_W | Transition to level W |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL_TR = 1 (NTC) |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 001 | Second |
| N_TOTAL | 3 | 001 | 2 balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS level | W | LW |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 10

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.1: Level Transitions General requirements |  | 5100100 |
| Test Case of Feature | The train has received one level transition announcement to Level Y at a location, and it receives a second to the same level and location. The train executes the last received level transition announcement at the announced location. Comment: This test case is applicable for all possible combinations of Level transitions |  | 10 |
| Applicable Mode/Level Combinations | L0: UN, SL, SB, TR, NL L1: FS, LS, OS, SR, SB, SL, TR, PT, NL L2: FS, LS, OS, SR, SB, SL, TR, PT, NL L3: FS, LS, OS, SR, SB, SL, TR, PT, NL LNTC: SL, SB, TR, NL, SN |  |  |
| Target of Test | The target of this test is to check that: The on-board system executes the last level transition announcement received on-board, i.e. the last announcement of level transition replaces the previous level transition announcement data stored on-board |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-5.10.1.6 |  |
|  |  | Subset-026-5.10.1.5 |  |
|  |  | Subset-026-7.4.2.9 Packet Number 41: Level Transition Order |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The on-board executes only the last received level transition announcement Check on the JRU that: The new level is recorded at the new location |
| Constraints | The on-board equipment received first the packet 41 with D_LEVELTR_1 and M_LEVELTR_1 (Level Y).Then the on-board equipment received the second level transition announcement for the same level and location. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| Level Transition Order | STORED | M_LEVELTR_1 = Y (Transition to Level Y) D_LEVELTR_1 <> 0 and <> 32767 |
| ERTMS/ETCS level | X | LX |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Message “Announcement of Level Y” is DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | X | - | An announcement of level transition to Level W (Packet 41) is RECEIVED from Balise Group BGa: D_LEVELTR_2 <> 0 and <> 32767 M_LEVELTR_2 = Y (Level Y) | I | BTM | D_LEVELTR_1 and D_LEVELTR_2 point to the same location. | X | Same | - |
| 2 | X | - | Balise Group BGa message, including Packet 41”Level transition order”, is RECORDED. | O | JRU | - | X | Same | - |
| 3 | X | - | Level Y is DISPLAYED | O | DMI | ('Except in SL ') The train is running and reaches the level border location. The border location is pointed by both D_LEVELTR_1 and D_LEVELTR_2. The train switches to Level Y | Y | Same | - |
| 4 | Y | - | New Level is RECORDED | O | JRU | - | Y | Same | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | First |
| N_TOTAL | 3 | 001 | 2 balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 41 | Level Transition Order |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | D_LEVELTR_2 | <> 0 and <> 32767 D_LEVELTR_2 points to the same location as D_LEVELTR_1 |
| M_LEVELTR | 3 | M_LEVEL_Y | Transition to level Y |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL_TR = 1 (NTC) |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 001 | Second |
| N_TOTAL | 3 | 001 | 2 balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS level | Y | LY |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 11

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.1: Level Transitions General requirements |  | 5100100 |
| Test Case of Feature | The train receives an order to switch immediately from L2 to L0 at the current position. on-board equipment immediately switches to L0. |  | 11 |
| Applicable Mode/Level Combinations | L2: FS, LS, OS, SR, SL, SB, TR, NL |  |  |
| Target of Test | The target of this test is to check that: The system switches to level 0 when the order to switch immediately to Level 0 is received on-board |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.3.1.1 Level Transition Order |  |
|  |  | Subset-026-4.8.4.2 Level Transition Order and Conditional Level Transition Order |  |
|  |  | Subset-026-7.4.2.9 Packet Number 41: Level Transition Order |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI (Except in SL mode) that: Level 0 indication is DISPLAYED. Check on the JRU that: Packet 41: “Level transition order” is RECORDED with M_LEVELTR = 0, D_LEVELTR = 0 or = 32767. New Level 0 is RECORDED. |
| Constraints | A cab needs to be active (SB and TR Mode) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 5 / 6 / 8 / 11 / 12 | FS / OS / SR / SL / SB / TR / NL / LS |
| ERTMS/ETCS level | 3 | L2 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Level 2 is DISPLAYED | O | DMI | Except in SL mode – desk is closed |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2 | - | The train reaches the Level transition border and an order to switch immediately to Level 0 is received from balise group BGa. Packet 41: “Level Transition Order” is RECEIVED. D_LEVELTR= 0 or = 32767 M_LEVELTR=0 | I | BTM | - | L2 | Same | - |
| 2 | L2 | - | Balise Group message, including Packet 41 ”Level transition order”, is RECORDED. | O | JRU | - | L2 | Same | - |
| 3 | L2 | - | Level 0 is DISPLAYED. | O | DMI | (Except in SL mode) The On-board equipment switches to Level 0. | L0 | Same | - |
| 4 | L0 | - | New Level 0 is RECORDED. | O | JRU | - | L0 | Same | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | First |
| N_TOTAL | 3 | 001 | 2 balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 41 | Level Transition Order |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | 0 or = 32767 | Switch immediately |
| M_LEVELTR | 3 | 0 | Transition to level 0 |
| NID_NTC | 8 | FINITE VALUE | - |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 001 | Second |
| N_TOTAL | 3 | 001 | 2 balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 5 / 6 / 7 / 11 / 12 | FS / OS / SR / SL / SB / TR / NL / LS |
| ERTMS/ETCS level | 0 | L0 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 12

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.1: Level Transitions General requirements |  | 5100100 |
| Test Case of Feature | The train receives an order to switch immediately from L1 to L0 at the current position. on-board equipment immediately switches to L0. |  | 12 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS, SR, SL, SB, TR, NL |  |  |
| Target of Test | The target of this test is to check that: The system switches to level 0 when the order to switch immediately to Level 0 is received on-board. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.3.1.1 Level Transition Order |  |
|  |  | Subset-026-4.8.4.2 Level Transition Order and Conditional Level Transition Order |  |
|  |  | Subset-026-7.4.2.9 Packet Number 41: Level Transition Order |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that (Except in SL mode – desk is closed): Level 0 indication is DISPLAYED. Check on the JRU that: Packet 41: “Level transition order” is RECORDED with M_LEVELTR = 0, D_LEVELTR = 0 or = 32767. New Level 0 is RECORDED. |
| Constraints | A cab needs to be active (SB and TR Mode) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 5 / 6 / 7 / 11 / 12 | FS / OS / SR / SL / SB / TR / NL / LS |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Level 1 is DISPLAYED | O | DMI | Except in SL mode – desk is closed |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 | - | The train reaches the Level transition border and an order to switch immediately to Level 0 is received from balise group BGa. Packet 41: “Level Transition Order” is RECEIVED. D_LEVELTR= 0 or = 32767 M_LEVELTR=0 | I | BTM | - | L1 | Same | - |
| 2 | L1 | - | Balise Group message, including Packet 41 ”Level transition order”, is RECORDED. | O | JRU | - | L1 | Same | - |
| 3 | L1 | - | Level 0 is DISPLAYED. | O | DMI | (Except in SL mode) The On-board equipment switches to Level 0. | L0 | Same | - |
| 4 | L0 | - | New Level 0 is RECORDED. | O | JRU | - | L0 | Same | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | First |
| N_TOTAL | 3 | 001 | 2 balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 41 | Level Transition Order |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | 0 or = 32767 | Switch immediately |
| M_LEVELTR | 3 | 0 | Transition to level 0 |
| NID_NTC | 8 | FINITE VALUE | - |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 001 | Second |
| N_TOTAL | 3 | 001 | 2 balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 5 / 6 / 7 / 11 / 12 | FS / OS / SR / SL / SB / TR / NL / LS |
| ERTMS/ETCS level | 0 | L0 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| LEVEL 0 is DISPLAYED | O | DMI | The announcement of the level transition L1/L0 at a further position is DELETED from the DMI in mode SL no indication – desk is closed |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 13

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.1: Level Transitions General requirements |  | 5100100 |
| Test Case of Feature | The train receives an order to switch immediately from L0 to L2 at the current position. on-board equipment immediately switches to L2. |  | 13 |
| Applicable Mode/Level Combinations | L0: UN, SL, SB, TR, NL |  |  |
| Target of Test | The target of this test is to check that: The system switches to level when the order to switch immediately to Level 2 is received on-board |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.3.1.1 Level Transition Order |  |
|  |  | Subset-026-4.8.4.2 Level Transition Order and Conditional Level Transition Order |  |
|  |  | Subset-026-7.4.2.9 Packet Number 41: Level Transition Order |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that (Except in SL mode – desk is closed): Level 2 indication is DISPLAYED. Check on the JRU that: Packet 41: “Level transition order” is RECORDED with M_LEVELTR = 3, D_LEVELTR = 0 or = 32767. New Level 2 is RECORDED. |
| Constraints | A Cab needs to be active (SB and TR Mode) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 4 / 5 / 6 / 7 / 11 | UN / SL / SB / TR / NL |
| ERTMS/ETCS level | 0 | L0 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Level 0 is DISPLAYED | O | DMI | Except in SL mode – desk is closed |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 | - | The train reaches the Level transition border and an order to switch immediately to Level 2 is received from balise group BGa : Packet 41: “Level Transition Order” is RECEIVED. D_LEVELTR=0 M_LEVELTR=3 | I | BTM | - | L0 | Same | - |
| 2 | L0 | - | Balise Group message, including Packet 41 ”Level transition order”, is RECORDED | O | JRU | - | L0 | Same | - |
| 3 | L0 | - | Level 2 is DISPLAYED | O | DMI | (Except in SL mode – desk is closed) The on-board system switches to Level 2 | L2 | Same | - |
| 4 | L2 | - | New Level L2 is RECORDED | O | JRU | - | L2 | Same | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | First |
| N_TOTAL | 3 | 001 | 2 balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | BGa | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 41 | Level Transition Order |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | 0 | Switch immediately |
| M_LEVELTR | 3 | 3 | Transition to level 2 |
| NID_NTC | 8 | FINITE VALUE | - |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 001 | Second |
| N_TOTAL | 3 | 001 | 2 balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | BGa | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 4 / 5 / 6 / 7 / 11 | UN / SL / SB / TR / NL |
| ERTMS/ETCS level | 3 | L2 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 14

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.1: Level Transitions General requirements |  | 5100100 |
| Test Case of Feature | The train receives from the balise group BGa the Level Transition announcement LX/LY. The driver is informed. |  | 14 |
| Applicable Mode/Level Combinations | L0: UN, SL, SB, TR, NL LNTC: SL, SB, TR, NL, SN L1: FS, LS, OS, SR, SL, SB, TR, NL L2: FS, LS, OS, SR, SL, SB, TR, PT, NL L3: FS, LS, OS, SR, SL, SB, TR, PT, NL |  |  |
| Target of Test | The target of this test is to check that: The on-board equipment displays the announcement of the Level Transition to LY after receiving the announcement of the transition from the balise group BGa. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.7.2 table - Output information Level transition announcement |  |
|  |  | Subset-026-4.8.3.1.1 Level Transition Order |  |
|  |  | Subset-026-4.8.4.2 Level Transition Order and Conditional Level Transition Order |  |
|  |  | Subset-026-5.10.1.3 |  |
|  |  | Subset-026-7.4.2.9 Packet Number 41: Level Transition Order |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The Level Transition announcement is DISPLAYED after Packet 41: “Level Transition Order” is received on-board ('Except in SL and SB modes') Check on the JRU that : Balise Group BGa message, including the announcement of the Level Transition LX/LY is RECORDED. Level transition is announced by means of Packet 41 “Level Transition Order” with M_LEVELTR=M_LEVEL_Y and D_LEVELTR <> 0 and <> 32767. |
| Constraints | A cab needs to be active (SB and TR Mode) Unique announcement of level transition (LX/LY as described in the Packet 41 defined in this test case with N_ITER = 0) and this announcement will result in a change of the on-board level. If in PT mode, the packet 41 follows the reception of the information “Recognition of Exit from TR mode” with a more recent time stamp. If the level transition to LNTC is announced, the train shall be equipped with the NTC module compatible to the one defined with NID_NTC in the Level transition announcement. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 5 / 6 / 7 / 8 / 11 / 12 / 13 | FS / OS / SR / UN / SL / SB / TR / PT / NL / LS / SN |
| ERTMS/ETCS level | M_LEVEL_X | Level X |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | X | - | An announcement of level transition to Level Y (Packet 41) is RECEIVED from Balise Group BGa: D_LEVELTR <> 0 and <> 32767 M_LEVELTR= M_LEVEL_Y (Level Y) | I | BTM | Order to switch to level Y at further location and this announcement will result in a change of the on-board level. | Same | Same | - |
| 2 | X | - | An indication of Announcement of Level Y is DISPLAYED. | O | DMI | ('Except in SL and SB modes') | Same | Same | - |
| 3 | X | - | Displayed indication is recorded | O | JRU | ('Except in SL and SB modes') | Same | Same | - |
| 4 | X | - | Balise Group BGa message, including Packet 41”Level transition order”, is RECORDED. | O | JRU | - | Same | Same | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | First |
| N_TOTAL | 3 | 001 | 2 balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 41 | Level Transition Order |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | FINITE VALUE | <> 0 and <> 32767 |
| M_LEVELTR | 3 | M_LEVEL_Y | Transition to level Y |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL_Y = 1 (NTC) |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 001 | Second |
| N_TOTAL | 3 | 001 | 2 balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 5 / 6 / 7 / 8 / 11 / 12 / 13 UNCHANGED | FS / OS / SR / UN / SL / SB / TR / PT / NL / LS / SN UNCHANGED |
| Level Transition Order | STORED | M_LEVELTR = M_LEVEL_Y (Transition to level Y) NID_NTC = FINITE VALUE (If M_LEVEL_Y = 1 (NTC)) D_LEVELTR = FINITE VALUE (<> 0 and <> 32767) |
| ERTMS/ETCS level | M_LEVEL_X UNCHANGED | Level X UNCHANGED |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| An indication of announcement of Level Y is DISPLAYED. | O | DMI | ('Except in SL and SB modes') |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 15

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.1: Level Transitions General requirements |  | 5100100 |
| Test Case of Feature | The train receives an order to switch immediately from L0 to L1 at the current position. on-board equipment immediately switches to L1. |  | 15 |
| Applicable Mode/Level Combinations | L0: UN, SL, SB, TR, NL |  |  |
| Target of Test | The target of this test is to check that: The system switches to level 1 when the order to switch immediately to Level 1 is received on-board |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.3.1.1 Level Transition Order |  |
|  |  | Subset-026-4.8.4.2 Level Transition Order and Conditional Level Transition Order |  |
|  |  | Subset-026-7.4.2.9 Packet Number 41: Level Transition Order |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that (Except in SL mode – desk is closed): Level 1 indication is DISPLAYED. Check on the JRU that: Packet 41: “Level transition order” is RECORDED with M_LEVELTR = 2, D_LEVELTR = 0 or = 32767. New Level 1 is RECORDED. |
| Constraints | A cab needs to be active (SB and TR Mode) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 4 / 5 / 6 / 7 / 11 | UN / SL / SB / TR / NL |
| ERTMS/ETCS level | 0 | L0 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Level 0 ( symbol LE01) is DISPLAYED. | O | DMI | (Except in SL mode) |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 | - | The train reaches the Level transition border and an order to switch immediately to Level 1 is received from balise group BGa : Packet 41: “Level Transition Order” is RECEIVED. D_LEVELTR=0 M_LEVELTR=2 | I | BTM | - | L0 | Same | - |
| 2 | L0 | - | Balise Group message, including Packet 41 ”Level transition order”, is RECORDED | O | JRU | - | L0 | Same | - |
| 3 | L0 | - | Level 1 is DISPLAYED | O | DMI | (Except in SL mode – desk is closed) The on-board system switches to Level 1 | L1 | Same | - |
| 4 | L1 | - | New Level L1 is RECORDED | O | JRU | - | L1 | Same | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | First |
| N_TOTAL | 3 | 001 | 2 balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | BGa | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 41 | Level Transition Order |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | 0 | Switch immediately |
| M_LEVELTR | 3 | 2 | Transition to level 1 |
| NID_NTC | 8 | FINITE VALUE | - |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 001 | Second |
| N_TOTAL | 3 | 001 | 2 balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | BGa | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 4 / 5 / 6 / 7 / 11 | UN / SL / SB / TR / NL |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 16

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.1: Level Transitions General requirements |  | 5100100 |
| Test Case of Feature | The train receives an order to switch immediately from L0 to LNTC at the current position. on-board equipment immediately switches to LNTC. |  | 16 |
| Applicable Mode/Level Combinations | L0: UN, SL, SB, TR, NL |  |  |
| Target of Test | The target of this test is to check that: The system switches to Level NTC when the order to switch immediately to Level NTC is received on-board |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.3.1.1 Level Transition Order |  |
|  |  | Subset-026-4.8.4.2 Level Transition Order and Conditional Level Transition Order |  |
|  |  | Subset-026-7.4.2.9 Packet Number 41: Level Transition Order |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI (Except in SL mode) that: Level NTC indication is DISPLAYED. Check on the JRU that: Packet 41: “Level transition order” is RECORDED with M_LEVELTR = 1, D_LEVELTR = 0 or = 32767. New Level NTC is RECORDED. |
| Constraints | A cab needs to be active (SB Mode) The train shall be equipped with the NTC module compatible to the one defined with NID_NTC in the Level transition announcement |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 4 / 5 / 6 / 7 / 11 | UN / SL / SB / TR / NL |
| ERTMS/ETCS level | 0 | L0 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Level 0 ( symbol LE01) is DISPLAYED. | O | DMI | (Except in SL mode) |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 | - | The train reaches the Level transition border and an order to switch immediately to Level NTC is received from balise group BG1 (packet 41): D_LEVELTR=0 M_LEVELTR=1 NID_STMNTC | I | BTM | “The onboard equipment sends a signal to the appropriate NTC, in order to be prepared for immediate activation” | L0 | Same | - |
| 2 | L0 | - | Balise Group message, including Packet 41 ”Level transition order”, is RECORDED. | O | JRU | - | L0 | Same | - |
| 3 | L0 | - | Level NTC is DISPLAYED. | O | DMI | (Except in SL mode) The On-board equipment switches to Level NTC. | NTC | Same | - |
| 4 | NTC | - | New Level NTC is RECORDED. | O | JRU | - | NTC | Same | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | First |
| N_TOTAL | 3 | 001 | 2 balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | BGa | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 41 | Level Transition Order |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | 0 | Switch immediately |
| M_LEVELTR | 3 | 1 | Transition to Level NTC |
| NID_NTC | 8 | FINITE VALUE | - |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 001 | - |
| N_TOTAL | 3 | 001 | - |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | BGa | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 4 / 5 / 6 / 7 / 11 | UN / SL / SB / TR / NL |
| ERTMS/ETCS level | 1 | LNTC |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 17

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.1: Level Transitions General requirements |  | 5100100 |
| Test Case of Feature | The train is in Level X and reaches the level transition border with the Announcement of Level Transition to Level Y. The on-board equipment does not receive any order to switch immediately at the level border. Nevertheless it switches to LY as required at the Level Transition announcement to LY |  | 17 |
| Applicable Mode/Level Combinations | L0: UN, SL, SB, TR, NL LNTC: SL, SB, TR, NL, SN L1: FS, LS, OS, SR, SL, SB, TR, NL L2: FS, LS, OS, SR, SL, SB, TR, PT, NL L3: FS, LS, OS, SR, SL, SB, TR, PT, NL |  |  |
| Target of Test | The target of this test is to check that: The on-board equipment switches from level X to level Y although no order to switch immediately at the announced position is received. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.6.4.4 a) |  |
|  |  | Subset-026-5.10.1.5 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI (Except in SL mode) that: Level Y indication is DISPLAYED. Check on the JRU that: New Level Y is RECORDED. |
| Constraints | A cab needs to be active (SB and TR Mode) If LNTC was announced in the level transition announcement, the train shall be equipped with the NTC module compatible to the one defined with NID_NTC in the Level transition announcement |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 5 / 6 / 7 / 8 / 11 / 12 / 13 / | FS / OS / SR / UN / SL / SB / TR / PT / NL / LS / SN |
| Level Transition Order | STORED | M_LEVELTR = M_LEVEL_Y (Transition to level Y) NID_NTC = FINITE VALUE (If M_LEVEL_Y = 1 (NTC)) D_LEVELTR = FINITE VALUE |
| ERTMS/ETCS level | M_LEVEL_X | Level X |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Level X is DISPLAYED. An indication of announcement of Level Y is DISPLAYED | O | DMI | Except in SL mode Except in SL/SB modes |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | X | - | Level Y is DISPLAYED. | O | DMI | (Except in SL mode) The estimated front-end train reaches de Level transition location (D_LEVELTR) the On-board equipment switches to Level Y. | Y | Same | - |
| 2 | Y | - | Announcement of Level Y is NOT DISPLAYED any more. | O | DMI | - | Same | Same | - |
| 3 | Y | - | New Level Y is RECORDED. | O | JRU | - | Same | Same | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 5 / 6 / 7 / 8 / 11 / 12 / 13 / | FS / OS / SR / UN / SL / SB / TR / PT / NL / LS / SN |
| ERTMS/ETCS level | M_LEVEL_Y | Level Y |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| New Level Y is DISPLAYED. Announcement of Level Y is NOT DISPLAYED any more. | O | DMI | Except in SL mode |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 18

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.1: Level Transitions General requirements |  | 5100100 |
| Test Case of Feature | Reception and acceptance of Temporary Speed Restriction Revocation (packet 66) by on-board if an announcement to Level 2 or Level 3 has been received |  | 18 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS, SR, SB, TR, PT L0: UN LNTC: SN |  |  |
| Target of Test | The target of this test case is to check that: on-board equipment receives and accepts the packet 66 (transmitted in General Radio Message 24) and on-board equipment revokes the stored temporary speed restriction correctly (the Temporary Speed Restriction is revocable). |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-5.10.1.7 |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |
|  |  | Subset-026-7.4.2.18 Packet Number 66: Temporary Speed Restriction Revocation |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that the permitted speed or LSSMA changes to less restrictive value after revocation of TSR (for the location of TSR) Check on the JRU that message 24 with packet 66 is recorded |
| Constraints | The data of the stored TSR (packet 65) has to be the most restrictive speed profile (for the location of TSR) and the Temporary Speed Restriction is not marked as non-revocable A cab needs to be active and valid Train Data are stored on-board (if in SB Mode) If PT mode, recognition of exit from TR Mode is stored If LS mode, the LSSMA is displayed |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 8 / 12 / 13 | FS / OS / SR / UN / SB / TR / PT / LS / SN |
| Radio communication session | ESTABLISHED | - |
| Level Transition Order | STORED | Level transition to Levels 2 or 3 |
| Temporary Speed Restrictions | STORED | revocable and send by RBC, that is value 127...254 |
| ERTMS/ETCS level | 0 / 1 / 2 | L0 / LNTC / L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Current level and mode are DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | X | - | The RBC sends a message 24 (General message) including packet 66 (Temporary Speed Restriction Revocation) to the train. | I | RTM | - | Same | Same | - |
| 2 | X | - | The message 24 including packet 66 is RECORDED | O | JRU | Reception of TSR Revocation order | Same | Same | - |
| 3 | X | - | Use_FT5100100.17 for a level transition to level 2 | - | - | - | Same | Same | - |
| 4 | X | - | The driver requests the speed and distance monitoring information to be displayed | I | DMI | Not for LS mode, LSSMA is already displayed | Same | Same | - |
| 5 | X | - | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | Not for LS mode, LSSMA is already displayed The driver request for permitted speed is RECORDED | Same | Same | - |
| 6 | X | - | The speed and distance monitoring information is displayed | O | DMI | The train is located in the area of TSR, given in packet 65. The permitted speed or LSSMA changes to less restrictive values after revocation of TSR Only in SB mode (Train at standstill!): The train position is located in the area of TSR, given in packet 65. | Same | Same | - |
| 7 | X | - | The “SPEED AND DISTANCE MONITORING INFORMATION” message is RECORDED | O | JRU | V_PERM changes to less restrictive values after revocation of TSR | Same | Same | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | Message 24: General message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | 0 | No acknowledgement required |
| NID_LRBG | 10 + 14 | FINITE VALUE | - |
| NID_PACKET | 8 | 66 | Packet Number 66: Temporary Speed Restriction Revocation |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| NID_TSR | 8 | FINITE VALUE | Identity of TSR to be revoked (Identity of stored TSR) Value ≠ 255 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 8 / 12 / 13 UNCHANGED | FS / OS / SR / UN / SB / TR / PT / LS / SN |
| Radio communication session | ESTABLISHED | - |
| Level Transition Order | STORED | Level transition to Levels 2 or 3 |
| Temporary Speed Restrictions | STORED | revocable and send by RBC, that is value 127...254 |
| ERTMS/ETCS level | 0 / 1 / 2 UNCHANGED | L0 / LNTC / L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Current level and mode are DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 19

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.1: Level Transitions General requirements |  | 5100100 |
| Test Case of Feature | The train receives an order to switch immediately from LNTC[X] to LNTC[Y] at the current position. on-board equipment immediately switches to LNTC[Y]. |  | 19 |
| Applicable Mode/Level Combinations | LNTC : SL, SB, TR, NL, SN |  |  |
| Target of Test | The target of this test is to check that: The system switches to level NTC [Y] when the order to switch immediately to Level NTC [Y] is received on-board |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.3.1.1 Level Transition Order |  |
|  |  | Subset-026-4.8.4.2 Level Transition Order and Conditional Level Transition Order |  |
|  |  | Subset-026-7.4.2.9 Packet Number 41: Level Transition Order |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that (Except in SL mode – desk is closed): Level NTC[Y] indication is DISPLAYED. Check on the JRU that: Packet 41: “Level transition order” is RECORDED with M_LEVELTR = 1,NID_ NTC and D_LEVELTR = 0 or = 32767. New Level NTC[Y] is RECORDED. |
| Constraints | A cab needs to be active (SB Mode) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 5 / 6 / 7 / 11 / 13 | SL / SB / TR / NL / SN |
| ERTMS/ETCS level | 1 | LNTC[X] |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Level NTC[X] is DISPLAYED. | O | DMI | Except if in SL Mode |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | NTC[X] | - | The train reaches the Level transition border and an order to switch immediately to Level NTC[Y] is received from balise group BGa. Packet 41: “Level Transition Order” is RECEIVED. D_LEVELTR= 0 or = 32767 M_LEVELTR=1 (Level NTC) NID_NTC=FINITE VALUE (NTC[Y]) | I | BTM | The on-board equipment sends a signal to deactivate the appropriate NTC[X] | Same | Same | - |
| 2 | NTC[X] | - | Balise Group message, including Packet 41 ”Level transition order”, is RECORDED. | O | JRU | - | NTC[X] | Same | - |
| 3 | NTC[X] | - | Level NTC[Y] is DISPLAYED. | O | DMI | (Except if in SL Mode) The On-board equipment switches to Level NTC[Y]. | NTC[Y] | Same | - |
| 4 | NTC[Y] | - | New Level NTC[Y] is RECORDED. | O | JRU | - | Same | Same | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | First |
| N_TOTAL | 3 | 001 | 2 balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 41 | Level Transition Order |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | 0 or = 32767 | Switch immediately |
| M_LEVELTR | 3 | 1 | Transition to level NTC |
| NID_NTC | 8 | FINITE VALUE | NTC [Y] |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 001 | Second |
| N_TOTAL | 3 | 001 | 2 balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 5 / 6 / 7 / 11 / 13 | SL / SB / TR / NL / SN |
| ERTMS/ETCS level | 4 | LNTC[Y] |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 20

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.1: Level Transitions General requirements |  | 5100100 |
| Test Case of Feature | The train receives an order to switch immediately from LNTC to L3 at the current position. on-board equipment immediately switches to L3. |  | 20 |
| Applicable Mode/Level Combinations | LNTC : SL, SB, TR, NL, SN |  |  |
| Target of Test | The target of this test is to check that: The system switches to level 3 when the order to switch immediately to Level 3 is received on-board |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.3.1.1 Level Transition Order |  |
|  |  | Subset-026-4.8.4.2 Level Transition Order and Conditional Level Transition Order |  |
|  |  | Subset-026-7.4.2.9 Packet Number 41: Level Transition Order |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that (Except in SL mode – desk is closed): Level 3 indication is DISPLAYED. Check on the JRU that: Packet 41: “Level transition order” is RECORDED with M_LEVELTR = 4, D_LEVELTR = 0 or = 32767. New Level 3 is RECORDED. |
| Constraints | A cab needs to be active (SB and TR Mode) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 5 / 6 / 7 / 11 / 13 | SL / SB / TR / NL / SN |
| ERTMS/ETCS level | 1 | LNTC |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Level NTC is DISPLAYED | O | DMI | Except in SL mode – desk is closed |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | NTC | - | The train reaches the Level transition border and an order to switch immediately to Level 3 is received from balise group BGa. Packet 41: “Level Transition Order” is RECEIVED. D_LEVELTR= 0 or = 32767 M_LEVELTR=4 | I | BTM | - | NTC | Same | - |
| 2 | NTC | - | Balise Group message, including Packet 41 ”Level transition order”, is RECORDED. | O | JRU | - | NTC | Same | - |
| 3 | NTC | - | Level 3 is DISPLAYED. | O | DMI | (Except if in SL Mode) The On-board equipment switches to Level 3. The on-board equipment sends a signal to deactivate the appropriate NTC | L3 | Same | - |
| 4 | L3 | - | New Level 3 is RECORDED. | O | JRU | - | L3 | Same | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | First |
| N_TOTAL | 3 | 001 | 2 balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 41 | Level Transition Order |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | 0 or = 32767 | Switch immediately |
| M_LEVELTR | 3 | 4 | Transition to level 3 |
| NID_NTC | 8 | FINITE VALUE | - |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 001 | Second |
| N_TOTAL | 3 | 001 | 2 balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 5 / 6 / 7 / 11 / 13 | SL / SB / TR / NL / SN |
| ERTMS/ETCS level | 4 | L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 21

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.1: Level Transitions General requirements |  | 5100100 |
| Test Case of Feature | The train receives an order to switch immediately from LNTC to L2 at the current position. on-board equipment immediately switches to L2. |  | 21 |
| Applicable Mode/Level Combinations | LNTC : SL, SB, TR, NL, SN |  |  |
| Target of Test | The target of this test is to check that: The system switches to level 2 when the order to switch immediately to Level 2 is received on-board |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.3.1.1 Level Transition Order |  |
|  |  | Subset-026-4.8.4.2 Level Transition Order and Conditional Level Transition Order |  |
|  |  | Subset-026-7.4.2.9 Packet Number 41: Level Transition Order |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that (Except in SL mode – desk is closed): Level 2 indication is DISPLAYED. Check on the JRU that: Packet 41: “Level transition order” is RECORDED with M_LEVELTR = 3, D_LEVELTR = 0 or = 32767. New Level 2 is RECORDED. |
| Constraints | A cab needs to be active (SB and TR Mode) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 5 / 6 / 7 / 11 / 13 | SL / SB / TR / NL / SN |
| ERTMS/ETCS level | 1 | LNTC |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Level NTC is DISPLAYED | O | DMI | Except in SL mode – desk is closed |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | NTC | - | The train reaches the Level transition border and an order to switch immediately to Level 2 is received from balise group BGa. Packet 41: “Level Transition Order” is RECEIVED. D_LEVELTR= 0 or = 32767 M_LEVELTR=3 | I | BTM | - | NTC | Same | - |
| 2 | NTC | - | Balise Group message, including Packet 41 ”Level transition order”, is RECORDED. | O | JRU | - | NTC | Same | - |
| 3 | NTC | - | Level 2 is DISPLAYED. | O | DMI | (Except if in SL Mode) The On-board equipment switches to Level 2. The on-board equipment sends a signal to deactivate the appropriate NTC | L2 | Same | - |
| 4 | L2 | - | New Level 2 is RECORDED. | O | JRU | - | L2 | Same | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | First |
| N_TOTAL | 3 | 001 | 2 balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 41 | Level Transition Order |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | 0 or = 32767 | Switch immediately |
| M_LEVELTR | 3 | 3 | Transition to level 2 |
| NID_NTC | 8 | FINITE VALUE | - |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 001 | Second |
| N_TOTAL | 3 | 001 | 2 balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 5 / 6 / 7 / 11 / 13 | SL / SB / TR / NL / SN |
| ERTMS/ETCS level | 3 | L2 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 22

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.1: Level Transitions General requirements |  | 5100100 |
| Test Case of Feature | The train receives from the RBC the Level Transition announcement LX/LY. The driver is informed. |  | 22 |
| Applicable Mode/Level Combinations | L0: UN, SL, SB, TR, NL LNTC: SL, SB, TR, NL, SN L1: FS, LS, OS, SR, SL, SB, TR, NL L2: FS, LS, OS, SR, SL, SB, TR, PT, NL L3: FS, LS, OS, SR, SL, SB, TR, PT, NL |  |  |
| Target of Test | The target of this test is to check that: The on-board system displays the announcement of the Level Transition to LY after receiving the announcement of the transition from the RBC. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.7.2 table - Output information Level transition announcement |  |
|  |  | Subset-026-4.8.3.1.1 Level Transition Order |  |
|  |  | Subset-026-4.8.4.2 Level Transition Order and Conditional Level Transition Order |  |
|  |  | Subset-026-5.10.1.3 |  |
|  |  | Subset-026-7.4.2.9 Packet Number 41: Level Transition Order |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The Level Transition announcement is DISPLAYED after the train receives from the RBC Packet 41: “Level Transition Order”. ('Except in SL and SB modes') Check on the JRU that : The radio message including the announcement of the Level Transition LX/LY is RECORDED. Level transition is announced by means of Packet 41 “Level Transition Order” with M_LEVELTR=M_LEVEL_Y and D_LEVELTR <> 0 and <> 32767. |
| Constraints | A cab needs to be active (SB and TR Mode) Unique announcement of level transition (LX/LY as described in the Packet 41 defined in this test case with N_ITER = 0) and this announcement will result in a change of the on-board level. If in PT mode, the packet 41 follows the reception of the information “Recognition of Exit from TR mode” with a more recent time stamp. If the level transition to LNTC is announced, the train shall be equipped with the NTC module compatible to the one defined with NID_NTC in the Level transition announcement. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 5 / 6 / 7 / 8 / 11 / 12 / 13 / | FS / OS / SR / UN / SL / SB / TR / PT / NL / LS / SN |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | M_LEVEL_X | Level X |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | An announcement of Level Transition to Level Y (radio message with Packet 41) is received from RBC: D_LEVLETR <> 0 and <> 32767 M_LEVELTR= M_LEVEL_Y (Level Y) | I | RTM | Order to switch to level Y at further location and this announcement will result in a change of the on-board level. | N/A | N/A | - |
| 2 | N/A | N/A | Radio message , including Packet 41”Level transition order” is RECORDED. | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | An indication of announcement of Level Y is DISPLAYED. | O | DMI | ('Except in SL and SB modes') | N/A | N/A | - |
| 4 | N/A | N/A | Displayed indication is RECORDED | O | JRU | ('Except in SL and SB modes') | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 41 | Level Transition Order |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | FINITE VALUE | <> 0 and <> 32767 |
| M_LEVELTR | 3 | M_LEVEL_Y | Transition to level Y |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL_Y = 1 (NTC) |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 5 / 6 / 7 / 8 / 11 / 12 / 13 UNCHANGED | FS / OS / SR / UN / SL / SB / TR / PT / NL / LS / SN |
| Radio communication session | ESTABLISHED | - |
| Level Transition Order | STORED | M_LEVELTR = M_LEVEL_Y (Transition to level Y) NID_NTC = FINITE VALUE (If M_LEVEL_Y = 1 (NTC)) D_LEVELTR = FINITE VALUE (<> 0 and <> 32767) |
| ERTMS/ETCS level | M_LEVEL_X UNCHANGED | Level X |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| An indication of announcement of level Y is DISPLAYED. | O | DMI | ('Except in SL and SB modes') |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 23

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.1: Level Transitions General requirements |  | 5100100 |
| Test Case of Feature | The train receives an order to switch immediately from LNTC to L1 at the current position. on-board equipment immediately switches to L1. |  | 23 |
| Applicable Mode/Level Combinations | LNTC : SL, SB, TR, NL, SN |  |  |
| Target of Test | The target of this test is to check that: The system switches to level 1 when the order to switch immediately to Level 1 is received on-board. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.3.1.1 Level Transition Order |  |
|  |  | Subset-026-4.8.4.2 Level Transition Order and Conditional Level Transition Order |  |
|  |  | Subset-026-7.4.2.9 Packet Number 41: Level Transition Order |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that (Except in SL mode – desk is closed): Level 1 indication is DISPLAYED. Check on the JRU that: Packet 41: “Level transition order” is RECORDED with M_LEVELTR = 2, D_LEVELTR = 0 or = 32767. New Level 1 is RECORDED. |
| Constraints | A cab needs to be active (SB and TR Mode). If tested in SN mode, level transition announcement at further location and track description information about next L1 area should be received previously to avoid the train to be tripped. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 5 / 6 / 7 / 11 / 13 | SL / SB / TR / NL / SN |
| ERTMS/ETCS level | 1 | LNTC |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Level NTC is DISPLAYED | O | DMI | Except in SL mode – desk is closed |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | LNTC | SL / SB / TR / NL / SN | The train reaches the Level transition border and an order to switch immediately to Level 1 is received from balise group BGa. Packet 41: “Level Transition Order” is RECEIVED. D_LEVELTR= 0 or = 32767 M_LEVELTR=2 | I | BTM | - | LNTC | SL / SB / TR / NL / SN | - |
| 2 | LNTC | SL / SB / TR / NL / SN | Balise Group message, including Packet 41 ”Level transition order”, is RECORDED. | O | JRU | - | LNTC | SL / SB / TR / NL / SN | - |
| 3 | LNTC | SL / SB / TR / NL / SN | Level 1 is DISPLAYED. | O | DMI | (Except in SL mode – desk is closed) The On-board equipment switches to Level 1. | L1 | SL / SB / TR / NL / SN | - |
| 4 | L1 | SL / SB / TR / NL / SN | New Level 1 is RECORDED. | O | JRU | - | L1 | SL / SB / TR / NL / SN | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | First |
| N_TOTAL | 3 | 001 | 2 balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | BGa | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 41 | Level Transition Order |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | 0 | Switch immediately |
| M_LEVELTR | 3 | 2 | Transition to level 1 |
| NID_NTC | 8 | FINITE VALUE | - |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 001 | Second |
| N_TOTAL | 3 | 001 | 2 balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | BGa | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 5 / 6 / 7 / 11 / 13 UNCHANGED | SL / SB / TR / NL / SN The modes are present to describe the Start Conditions of the Mode Change in the TC which follows, they may be incompatible with the End level of this TC. |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Level 1 is DISPLAYED | O | DMI | Except in SL mode – desk is closed |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 24

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.1: Level Transitions General requirements |  | 5100100 |
| Test Case of Feature | The train receives an order to switch immediately from LNTC to L0 at the current position. on-board equipment immediately switches to L0. |  | 24 |
| Applicable Mode/Level Combinations | LNTC : SL, SB, TR, NL, SN |  |  |
| Target of Test | The target of this test case is to check that : The system switches to level 0 when the order to switch immediately to Level 0 is received on-board |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.3.1.1 Level Transition Order |  |
|  |  | Subset-026-4.8.4.2 Level Transition Order and Conditional Level Transition Order |  |
|  |  | Subset-026-7.4.2.9 Packet Number 41: Level Transition Order |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that (Except in SL mode – desk is closed): Level 0 indication is DISPLAYED. Check on the JRU that: Packet 41: “Level transition order” is RECORDED with M_LEVELTR = 0, D_LEVELTR = 0 or = 32767. New Level 0 is RECORDED. |
| Constraints | A cab needs to be active (SB and TR Mode) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 5 / 6 / 7 / 11 / 13 | SL / SB / TR / NL / SN |
| ERTMS/ETCS level | 1 | LNTC |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Level NTC is DISPLAYED | O | DMI | Except in SL mode – desk is closed |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | NTC | N/A | The train reaches the Level transition border and an order to switch immediately to Level 0 is received from balise group BGa. Packet 41: “Level Transition Order” is RECEIVED. D_LEVELTR= 0 or = 32767 M_LEVELTR=0 | I | BTM | Reception of level transition order (Packet 41) | NTC | N/A | - |
| 2 | NTC | N/A | Balise Group message, including Packet 41 ”Level transition order”, is RECORDED. | O | JRU | - | NTC | N/A | - |
| 3 | NTC | N/A | Level 0 is DISPLAYED | O | DMI | (Except in SL mode – desk is closed) The On-board equipment switches to Level 0. | L0 | N/A | - |
| 4 | L0 | N/A | New Level 0 is RECORDED | O | JRU | - | L0 | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 001 | Second |
| N_TOTAL | 3 | 001 | 2 balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 41 | Level Transition Order |
| Q_DIR | 2 | 0 / 1 | FINITE VALUE |
| L_PACKET | 13 | 63 | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | 0 or = 32767 | - |
| M_LEVELTR | 3 | 0 | Level 0 |
| NID_NTC | 8 | FINITE VALUE | - |
| L_ACKLEVELTR | 15 | 0 | no data is following |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 001 | Second |
| N_TOTAL | 3 | 001 | 2 balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 5 / 6 / 7 / 11 / 13 | SL / SB / TR / NL / SN |
| ERTMS/ETCS level | 0 | L0 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 25

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.1: Level Transitions General requirements |  | 5100100 |
| Test Case of Feature | The train receives an order to switch immediately from L3 to LNTC at the current position. on-board equipment immediately switches to LNTC. |  | 25 |
| Applicable Mode/Level Combinations | L3: FS, LS, OS, SR, SL, SB, TR, NL |  |  |
| Target of Test | The target of this test is to check that: The system switches to Level NTC when the order to switch immediately to Level NTC is received on-board. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.3.1.1 Level Transition Order |  |
|  |  | Subset-026-4.8.4.2 Level Transition Order and Conditional Level Transition Order |  |
|  |  | Subset-026-7.4.2.9 Packet Number 41: Level Transition Order |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI (Except in SL mode) that: Level NTC indication is DISPLAYED. Check on the JRU that: Packet 41: “Level transition order” is RECORDED with M_LEVELTR = 3, D_LEVELTR = 0 or = 32767. New Level NTC is RECORDED. |
| Constraints | The train shall be equipped with the NTC module compatible to the one defined with NID_NTC in the Level transition announcement A cab needs to be active (SB and TR Mode) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 5 / 6 / 7 / 11 / 12 | FS / OS / SR / SL / SB / TR / NL / LS |
| Linking | STORED | If Linking information is stored, Balise Group BGa shall be included if BGa is marked as linked. (The onboard equipment could not have Linking information). |
| MA Request Parameters | STORED | If previously recevied |
| Position Report parameters | STORED | If previously received |
| ERTMS/ETCS level | 4 | L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Level 3 is DISPLAYED. | O | DMI | Except in SL mode – desk is closed |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L3 | - | The train reaches the Level transition border and an order to switch immediately to Level NTC is received from balise group BGa. Packet 41: “Level Transition Order” is RECEIVED. D_LEVELTR= 0 or = 32767 M_LEVELTR=1 NID_NTC=FINITE VALUE | I | BTM | The on-board equipment sends a signal to the appropriate NTC, in order to be prepared for immediate activation | L3 | Same | - |
| 2 | L3 | - | Balise Group message, including Packet 41 ”Level transition order”, is RECORDED. | O | JRU | - | L3 | Same | - |
| 3 | L3 | - | Level NTC is DISPLAYED. | O | DMI | (Except if in SL mode) The On-board equipment switches to Level NTC. | NTC | Same | - |
| 4 | NTC | - | New Level NTC is RECORDED. | O | JRU | - | NTC | Same | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | First |
| N_TOTAL | 3 | 001 | 2 balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 41 | Level Transition Order |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | 0 or = 32767 | Switch immediately |
| M_LEVELTR | 3 | 1 | Transition to Level NTC |
| NID_NTC | 8 | FINITE VALUE | - |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 001 | Second |
| N_TOTAL | 3 | 001 | 2 balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 5 / 6 / 7 / 11 / 12 | FS / OS / SR / SL / SB / TR / NL / LS The modes are present to describe the Start Conditions of the Mode Change in the TC which follows, they may be incompatible with the End level of this TC. |
| MA Request Parameters | FINITE VALUE | Set to default value (if previously received) |
| Position Report parameters | FINITE VALUE | Set to default value (if previously received) |
| ERTMS/ETCS level | 1 | LNTC |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 26

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.1: Level Transitions General requirements |  | 5100100 |
| Test Case of Feature | The train receives an order to switch immediately from L0 to L3 at the current position. on-board equipment immediately switches to L3. |  | 26 |
| Applicable Mode/Level Combinations | L0: UN, SL, SB, TR, NL |  |  |
| Target of Test | The target of this test is to check that: The system switches to level 3 when the order to switch immediately to Level 3 is received on-board |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.3.1.1 Level Transition Order |  |
|  |  | Subset-026-4.8.4.2 Level Transition Order and Conditional Level Transition Order |  |
|  |  | Subset-026-7.4.2.9 Packet Number 41: Level Transition Order |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that (Except in SL mode – desk is closed): Level 3 indication is DISPLAYED. Check on the JRU that: Packet 41: “Level transition order” is RECORDED with M_LEVELTR = 4, D_LEVELTR = 0 or = 32767. New Level 3 is RECORDED. |
| Constraints | A cab needs to be active (SB and TR Mode) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 4 / 5 / 6 / 7 / 11 | UN / SL / SB / TR / NL |
| ERTMS/ETCS level | 0 | L0 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Level 0 is DISPLAYED | O | DMI | Except in SL mode – desk is closed |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 | - | The train reaches the Level transition border and an order to switch immediately to Level 3 is received from balise group BGa : Packet 41: “Level Transition Order” is RECEIVED. D_LEVELTR=0 M_LEVELTR=4 | I | BTM | - | L0 | Same | - |
| 2 | L0 | - | Balise Group message, including Packet 41 ”Level transition order”, is RECORDED | O | JRU | - | L0 | Same | - |
| 3 | L0 | - | Level 3 is DISPLAYED | O | DMI | (Except in SL mode – desk is closed) The on-board system switches to Level 3 | L3 | Same | - |
| 4 | L3 | - | New Level L3 is RECORDED | O | JRU | - | L3 | Same | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | First |
| N_TOTAL | 3 | 001 | 2 Balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | BGa | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 41 | Level Transition Order |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | 0 | Switch immediately |
| M_LEVELTR | 3 | 4 | Transition to level 3 |
| NID_NTC | 8 | FINITE VALUE | - |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 001 | Second |
| N_TOTAL | 3 | 001 | 2 balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | BGa | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 4 / 5 / 6 / 7 / 11 | UN / SL / SB / TR / NL |
| ERTMS/ETCS level | 4 | L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 27

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.1: Level Transitions General requirements |  | 5100100 |
| Test Case of Feature | The train receives an order to switch immediately from L3 to L2 at the current position. On-board equipment immediately switches to L2. |  | 27 |
| Applicable Mode/Level Combinations | L3: FS, LS, OS, SR, SL, SB, TR, NL |  |  |
| Target of Test | The target of this test is to check that: The system switches to Level 2 when the order to switch immediately to Level 2 is received on-board. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.3.1.1 Level Transition Order |  |
|  |  | Subset-026-4.8.4.2 Level Transition Order and Conditional Level Transition Order |  |
|  |  | Subset-026-7.4.2.9 Packet Number 41: Level Transition Order |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI (Except in SL mode) that: Level 2 indication is DISPLAYED. Check on the JRU that: Packet 41: “Level transition order” is RECORDED with M_LEVELTR = 3, D_LEVELTR = 0 or = 32767. New Level 2 is RECORDED. |
| Constraints | A cab needs to be active (SB and TR Mode) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 5 / 6 / 7 / 11 / 12 | FS / OS / SR / SL / SB / TR / NL / LS |
| Linking | STORED | If Linking information is stored, Balise Group BGa shall be included if BGa is marked as linked. (The onboard equipment could not have Linking information). |
| MA Request Parameters | STORED | If previously recevied |
| Position Report parameters | STORED | If previously received |
| ERTMS/ETCS level | 4 | L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Level 3 is DISPLAYED. | O | DMI | (Except in SL mode) |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L3 | - | The train reaches the Level transition border and an order to switch immediately to Level 2 is received from balise group BGa. Packet 41: “Level Transition Order” is RECEIVED. D_LEVELTR= 0 or = 32767 M_LEVELTR=3 | I | BTM | - | L3 | Same | - |
| 2 | L3 | - | Balise Group message, including Packet 41 ”Level transition order”, is RECORDED. | O | JRU | - | L3 | Same | - |
| 3 | L3 | - | Level 2 is DISPLAYED. | O | DMI | (Except if in SL mode) The On-board equipment switches to Level 2. | L2 | Same | - |
| 4 | L2 | - | New Level 2 is RECORDED. | O | JRU | - | L2 | Same | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | First |
| N_TOTAL | 3 | 001 | 2 balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 41 | Level Transition Order |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | 0 or = 32767 | Switch immediately |
| M_LEVELTR | 3 | 3 | Transition to Level 2 |
| NID_NTC | 8 | FINITE VALUE | - |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 001 | Second |
| N_TOTAL | 3 | 001 | 2 balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 5 / 6 / 7 / 11 / 12 | FS / OS / SR / SL / SB / TR / NL / LS |
| MA Request Parameters | UNCHANGED | (if previously received) |
| Position Report parameters | UNCHANGED | (if previously received) |
| ERTMS/ETCS level | 3 | L2 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 28

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.1: Level Transitions General requirements |  | 5100100 |
| Test Case of Feature | The train receives an order to switch immediately from L3 to L1 at the current position. on-board equipment immediately switches to L1. |  | 28 |
| Applicable Mode/Level Combinations | L3: FS, LS, OS, SR, SL, SB, TR, NL |  |  |
| Target of Test | The target of this test is to check that: The system switches to Level 1 when the order to switch immediately to Level 1 is received on-board. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.3.1.1 Level Transition Order |  |
|  |  | Subset-026-4.8.4.2 Level Transition Order and Conditional Level Transition Order |  |
|  |  | Subset-026-7.4.2.9 Packet Number 41: Level Transition Order |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI (Except in SL mode) that: Level 1 indication is DISPLAYED. Check on the JRU that: Packet 41: “Level transition order” is RECORDED with M_LEVELTR = 2, D_LEVELTR = 0 or = 32767. New Level 1 is RECORDED. |
| Constraints | A cab needs to be active (SB and TR Mode) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 5 / 6 / 7 / 11 / 12 | FS / OS / SR / SL / SB / TR / NL / LS |
| Linking | STORED | If Linking information is stored, Balise Group BGa shall be included if BGa is marked as linked. (The onboard equipment could not have Linking information). |
| MA Request Parameters | STORED | If previously recevied |
| Position Report parameters | STORED | If previously received |
| ERTMS/ETCS level | 4 | L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Level 3 is DISPLAYED. | O | DMI | (Except in SL mode) |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L3 | - | The train reaches the Level transition border and an order to switch immediately to Level 1 is received from balise group BGa. Packet 41: “Level Transition Order” is RECEIVED. D_LEVELTR= 0 or = 32767 M_LEVELTR=2 | I | BTM | - | L3 | Same | - |
| 2 | L3 | - | Balise Group message, including Packet 41 ”Level transition order”, is RECORDED. | O | JRU | - | L3 | Same | - |
| 3 | L3 | - | Level 1 is DISPLAYED. | O | DMI | (Except if in SL mode) The On-board equipment switches to Level 1. | L1 | Same | - |
| 4 | L1 | - | New Level 1 is RECORDED. | O | JRU | - | L1 | Same | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | First |
| N_TOTAL | 3 | 001 | 2 balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 41 | Level Transition Order |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | 0 or = 32767 | Switch immediately |
| M_LEVELTR | 3 | 2 | Transition to Level 1 |
| NID_NTC | 8 | FINITE VALUE | - |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 001 | Second |
| N_TOTAL | 3 | 001 | 2 balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 5 / 6 / 7 / 11 / 12 | FS / OS / SR / SL / SB / TR / NL / LS |
| MA Request Parameters | FINITE VALUE | Set to default value (if previously received) |
| Position Report parameters | FINITE VALUE | Set to default value (if previously received) |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 29

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.1: Level Transitions General requirements |  | 5100100 |
| Test Case of Feature | The train receives an order to switch immediately from L3 to L0 at the current position. on-board equipment immediately switches to L0. |  | 29 |
| Applicable Mode/Level Combinations | L3: FS, LS, OS, SR, SL, SB, TR, NL |  |  |
| Target of Test | The target of this test is to check that: The system switches to level 0 when the order to switch immediately to Level 0 is received on-board. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.3.1.1 Level Transition Order |  |
|  |  | Subset-026-4.8.4.2 Level Transition Order and Conditional Level Transition Order |  |
|  |  | Subset-026-7.4.2.9 Packet Number 41: Level Transition Order |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI (Except in SL mode) that: Level 0 indication is DISPLAYED. Check on the JRU that: Packet 41: “Level transition order” is RECORDED with M_LEVELTR = 0, D_LEVELTR = 0 or = 32767. New Level 0 is RECORDED. |
| Constraints | A cab needs to be active (SB and TR Mode) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 5 / 6 / 7 / 11 / 12 | FS / OS / SR / SL / SB / TR / NL / LS |
| Linking | STORED | If Linking information is stored, Balise Group BGa shall be included if BGa is marked as linked. (The onboard equipment could not have Linking information). |
| MA Request Parameters | STORED | If previously recevied |
| Position Report parameters | STORED | If previously received |
| ERTMS/ETCS level | 4 | L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Level 3 is DISPLAYED. | O | DMI | (Except in SL mode) |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L3 | - | The train reaches the Level transition border and an order to switch immediately to Level 0 is received from balise group BGa. Packet 41: “Level Transition Order” is RECEIVED. D_LEVELTR= 0 or = 32767 M_LEVELTR=0 | I | BTM | - | L3 | Same | - |
| 2 | L3 | - | Balise Group message, including Packet 41 ”Level transition order”, is RECORDED. | O | JRU | - | L3 | Same | - |
| 3 | L3 | - | Level 0 is DISPLAYED. | O | DMI | (Except in SL mode) The On-board equipment switches to Level 0. | L0 | Same | - |
| 4 | L0 | - | New Level 0 is RECORDED. | O | JRU | - | L0 | Same | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | First |
| N_TOTAL | 3 | 001 | 2 balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 41 | Level Transition Order |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | 0 or = 32767 | Switch immediately |
| M_LEVELTR | 3 | 0 | Transition to level 0 |
| NID_NTC | 8 | FINITE VALUE | - |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 001 | Second |
| N_TOTAL | 3 | 001 | 2 balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 5 / 6 / 7 / 11 / 12 | FS / OS / SR / SL / SB / TR / NL / LS |
| MA Request Parameters | FINITE VALUE | Set to default value (if previously received) |
| Position Report parameters | FINITE VALUE | Set to default value (if previously received) |
| ERTMS/ETCS level | 0 | L0 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 30

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.1: Level Transitions General requirements |  | 5100100 |
| Test Case of Feature | The train receives an order to switch immediately from L2 to LNTC at the current position. on-board equipment immediately switches to LNTC. |  | 30 |
| Applicable Mode/Level Combinations | L2: FS, LS, OS, SR, SL, SB, TR, NL |  |  |
| Target of Test | The target of this test is to check that: The system switches to Level NTC when the order to switch immediately to Level NTC is received on-board |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.3.1.1 Level Transition Order |  |
|  |  | Subset-026-4.8.4.2 Level Transition Order and Conditional Level Transition Order |  |
|  |  | Subset-026-7.4.2.9 Packet Number 41: Level Transition Order |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI (Except in SL mode) that: Level NTC indication is DISPLAYED. Check on the JRU that: Packet 41: “Level transition order” is RECORDED with M_LEVELTR = 1, D_LEVELTR = 0 or = 32767. New Level NTC is RECORDED. |
| Constraints | The train shall be equipped with the NTC module compatible to the one defined with NID_NTC in the Level transition announcement A cab needs to be active (SB and TR Mode) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 5 / 6 / 7 / 11 / 12 | FS / OS / SR / SL / SB / TR / NL / LS |
| Linking | STORED | If Linking information is stored, Balise Group BGa shall be included if BGa is marked as linked. (The onboard equipment could not have Linking information). |
| MA Request Parameters | STORED | If previously recevied |
| Position Report parameters | STORED | If previously received |
| ERTMS/ETCS level | 3 | L2 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Level 2 is DISPLAYED. | O | DMI | (Except in SL mode) |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2 | - | The train reaches the Level transition border and an order to switch immediately to Level NTC is received from balise group BG1 (packet 41): D_LEVELTR=0 M_LEVELTR=1 NID_NTC | I | BTM | “The on-board equipment sends a signal to the appropriate NTC, in order to be prepared for immediate activation” | L2 | Same | - |
| 2 | L2 | - | Balise Group message, including Packet 41 ”Level transition order”, is RECORDED. | O | JRU | - | L2 | Same | - |
| 3 | L2 | - | Level NTC is DISPLAYED. | O | DMI | (Except in SL mode) The On-board equipment switches to Level NTC. | NTC | Same | - |
| 4 | NTC | N/A | New Level NTC is RECORDED. | O | JRU | - | NTC | Same | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | - |
| N_TOTAL | 3 | 001 | - |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | 255 | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | BGa | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 41 | Level Transition Order |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | 0 | Switch immediately |
| M_LEVELTR | 3 | 1 | Transition to Level NTC |
| NID_NTC | 8 | FINITE VALUE | - |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 001 | - |
| N_TOTAL | 3 | 001 | - |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | BGa | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 5 / 6 / 7 / 11 / 12 | FS / OS / SR / SL / SB / TR / NL / LS |
| MA Request Parameters | UNKNOWN | DELETED |
| Position Report parameters | UNKNOWN | DELETED |
| ERTMS/ETCS level | 1 | LNTC |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 31

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.1: Level Transitions General requirements |  | 5100100 |
| Test Case of Feature | The train receives an order to switch immediately from L2 to L3 at the current position. on-board equipment immediately switches to L3. |  | 31 |
| Applicable Mode/Level Combinations | L2: FS, LS, OS, SR, SL, SB, TR, NL |  |  |
| Target of Test | The target of this test is to check that: The system switches to level 3 when the order to switch immediately to Level 3 is received on-board |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.3.1.1 Level Transition Order |  |
|  |  | Subset-026-4.8.4.2 Level Transition Order and Conditional Level Transition Order |  |
|  |  | Subset-026-7.4.2.9 Packet Number 41: Level Transition Order |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI (Except if in SL mode) that: Level 3 indication is DISPLAYED. Check on the JRU that: Packet 41: “Level transition order” is RECORDED with M_LEVELTR = 4, D_LEVELTR = 0 or = 32767. New Level 3 is RECORDED. |
| Constraints | A cab needs to be active (SB and TR Mode) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 5 / 6 / 7 / 11 / 12 | FS / OS / SR / SL / SB / TR / NL / LS |
| Linking | STORED | If Linking information is stored, Balise Group BGa shall be included if BGa is marked as linked. (The onboard equipment could not have Linking information). |
| MA Request Parameters | STORED | If previously received |
| Position Report parameters | STORED | If previously received |
| ERTMS/ETCS level | 3 | L2 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Level 2 is DISPLAYED. | O | DMI | Except if in SL mode |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2 | - | The train reaches the Level transition border and an order to switch immediately to Level 3 is received from balise group BGa. Packet 41: “Level Transition Order” is RECEIVED. D_LEVELTR= 0 or = 32767 M_LEVELTR=4 | I | BTM | - | L2 | Same | - |
| 2 | L2 | - | Balise Group message, including Packet 41 ”Level transition order”, is RECORDED. | O | JRU | - | L2 | Same | - |
| 3 | L2 | - | Level 3 is DISPLAYED. | O | DMI | (Except if in SL and TR mode) The On-board equipment switches to Level 3. | L3 | Same | - |
| 4 | L3 | N/A | New Level 3 is RECORDED. | O | JRU | - | L3 | Same | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | First |
| N_TOTAL | 3 | 001 | 2 balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | 255 | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 41 | Level Transition Order |
| Q_DIR | 2 | 10 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | 0 or 32767 | Switch immediately |
| M_LEVELTR | 3 | 4 | Transition to level 3 |
| NID_NTC | 8 | FINITE VALUE | - |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 001 | Second |
| N_TOTAL | 3 | 001 | 2 balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | BGa | BG a |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 5 / 6 / 7 / 11 / 12 | FS / OS / SR / SL / SB / TR / NL / LS |
| MA Request Parameters | UNCHANGED | If previously received |
| Position Report parameters | UNCHANGED | If previously received |
| ERTMS/ETCS level | 4 | L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 32

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.1: Level Transitions General requirements |  | 5100100 |
| Test Case of Feature | The train receives an order to switch immediately from L2 to L1 at the current position. on-board equipment immediately switches to L1. |  | 32 |
| Applicable Mode/Level Combinations | L2: FS, LS, OS, SR, SL, SB, TR, NL |  |  |
| Target of Test | The target of this test is to check that: The system switches to level 1 when the order to switch immediately to Level 1 is received on-board |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.3.1.1 Level Transition Order |  |
|  |  | Subset-026-4.8.4.2 Level Transition Order and Conditional Level Transition Order |  |
|  |  | Subset-026-7.4.2.9 Packet Number 41: Level Transition Order |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI (Except in SL mode) that: Level 1 indication is DISPLAYED. Check on the JRU that: Packet 41: “Level transition order” is RECORDED with M_LEVELTR = 2, D_LEVELTR = 0 or = 32767. New Level 1 is RECORDED. |
| Constraints | A cab needs to be active (SB and TR Mode) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 5 / 6 / 7 / 11 / 12 | FS / OS / SR / SL / SB / TR / NL / LS |
| ERTMS/ETCS level | 3 | L2 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Level 2 is DISPLAYED. | O | DMI | (Except in SL mode) |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2 | N/A | The train reaches the Level transition border and an order to switch immediately to Level 1 is received from balise group BGa. Packet 41: “Level Transition Order” is RECEIVED. • D_LEVELTR= 0 or = 32767 M_LEVELTR=2 | I | BTM | - | L2 | N/A | - |
| 2 | L2 | N/A | Balise Group message, including Packet 41 ”Level transition order”, is RECORDED. | O | JRU | - | L2 | N/A | - |
| 3 | L2 | N/A | Level 1 is DISPLAYED. | O | DMI | The On-board equipment switches to Level 1. | L1 | N/A | - |
| 4 | L1 | N/A | New Level 1 is RECORDED. | O | JRU | - | L1 | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | First |
| N_TOTAL | 3 | 001 | 2 balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 41 | Level Transition Order |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | 0 or = 32767 | Switch immediately |
| M_LEVELTR | 3 | 2 | Transition to level 1 |
| NID_NTC | 8 | FINITE VALUE | - |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 001 | Second |
| N_TOTAL | 3 | 001 | 2 balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 5 / 6 / 7 / 11 / 12 | FS / OS / SR / SL / SB / TR / NL / LS |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 33

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.1: Level Transitions General requirements |  | 5100100 |
| Test Case of Feature | The on-board has switched to Level Y and a new MA related to level 2/3 is received. The new MA is rejected. |  | 33 |
| Applicable Mode/Level Combinations | L0: UN, SB LNTC: SB, SN L1: FS, LS, OS, SR, SB |  |  |
| Target of Test | The target of this test is to check that: The on-board rejects MA related to Level 2/3 when the Level Transition to Level Y has been performed. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-5.10.1.8 |  |
|  |  | Subset-026-4.8.3.1.1 Movement Authority + (optional) Mode Profile + (optional) List of Balises for SH area |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : There is no change when the new MA is received. Check on the JRU that: The new MA is RECORDED. |
| Constraints | Cab is active and valid Train Data are stored on-board if in SB |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 12 / 13 | FS / OS / SR / UN / SB / LS / SN |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | M_LEVEL_Y | Level Y |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | Y | - | SA-DATA.Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) is received | I | RTM | - | Same | Same | - |
| 2 | Y | - | The new MA is rejected. No changes in the DMI are DISPLAYED. | O | DMI | - | Same | Same | - |
| 3 | Y | - | Message 3: “Movement Authority” is RECORDED. | O | JRU | - | Same | Same | - |

| Step 1: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 12 / 13 UNCHANGED | FS / OS / SR / UN / SB / LS / SN |
| ERTMS/ETCS level | M_LEVEL_Y UNCHANGED | Level Y |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 34

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.1: Level Transitions General requirements |  | 5100100 |
| Test Case of Feature | The train receives an order to switch immediately from L1 to LNTC at the current position. on-board equipment immediately switches to LNTC. |  | 34 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS, SR, SL, SB, TR, NL |  |  |
| Target of Test | The target of this test is to check that: The system switches to Level NTC when the order to switch immediately to Level NTC is received on-board |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.3.1.1 Level Transition Order |  |
|  |  | Subset-026-4.8.4.2 Level Transition Order and Conditional Level Transition Order |  |
|  |  | Subset-026-7.4.2.9 Packet Number 41: Level Transition Order |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI (Except in SL mode) that: Level NTC indication is DISPLAYED. Check on the JRU that: Packet 41: “Level transition order” is RECORDED with M_LEVELTR = 1, D_LEVELTR = 0 or = 32767. New Level NTC is RECORDED. |
| Constraints | A cab needs to be active (SB and TR Mode) The train shall be equipped with the NTC module compatible to the one defined with NID_NTC in the Level transition announcement |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 5 / 6 / 7 / 11 / 12 | FS / OS / SR / SL / SB / TR / NL / LS |
| Linking | STORED | If Linking information is stored, Balise Group BGa shall be included if BGa is marked as linked. (The on-board equipment could not have Linking information). |
| Signalling related Speed Restriction | STORED | If previously received |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Level 1 is DISPLAYED | O | DMI | (Except in SL mode) |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 | - | The train reaches the Level transition border and an order to switch immediately to Level NTC is received from balise group BG1 (packet 41): D_LEVELTR=0 M_LEVELTR=1 NID_NTC | I | BTM | “The on-board equipment sends a signal to the appropriate NTC, in order to be prepared for immediate activation” | Same | Same | - |
| 2 | L1 | - | Balise Group message, including Packet 41 ”Level transition order”, is RECORDED. | O | JRU | - | L1 | Same | - |
| 3 | L1 | - | Level NTC is DISPLAYED. | O | DMI | (Except in SL mode) The On-board equipment switches to Level NTC. | NTC | Same | - |
| 4 | NTC | N/A | New Level NTC is RECORDED. | O | JRU | - | NTC | Same | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | First |
| N_TOTAL | 3 | 001 | 2 balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | BGa | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 41 | Level Transition Order |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | 0 | Switch immediately |
| M_LEVELTR | 3 | 1 | Transition to Level NTC |
| NID_NTC | 8 | FINITE VALUE | - |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 001 | - |
| N_TOTAL | 3 | 001 | - |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | BGa | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 5 / 6 / 7 / 11 / 12 | FS / OS / SR / SL / SB / TR / NL / LS |
| Signalling related Speed Restriction | UNKNOWN | If previously received, it is deleted. |
| ERTMS/ETCS level | 1 | LNTC |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 35

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.1: Level Transitions General requirements |  | 5100100 |
| Test Case of Feature | The train receives from an infill balise group the Level Transition announcement L1/LY. The driver is informed. |  | 35 |
| Applicable Mode/Level Combinations | L1 : FS, LS |  |  |
| Target of Test | The target of this test case is to check that: The system receives an announcement to switch to level Y, as infill information, i.e., after Packet 136 The on-board equipment displays the announcement of the Level Transition to LY after receiving the announcement of the transition from the infill balise group BG |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-5.10.1.3 |  |
|  |  | Subset-026-7.4.2.9 Packet Number 41: Level Transition Order |  |
|  |  | Subset-026-7.4.2.32 Packet Number 136: Infill location reference |  |
|  |  | Subset-026-3.6.3.2.3 |  |
|  |  | Subset-026-4.7.2 table - Output information Level transition announcement |  |
|  |  | Subset-026-4.8.3.1.1 Level Transition Order |  |
|  |  | Subset-026-4.8.4.2 Level Transition Order and Conditional Level Transition Order |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that The Level Transition announcement is DISPLAYED after Packet 41: “Level Transition Order” is received on-board. Check on the JRU that Infill Balise Group message, including the announcement of the Level Transition L1/LY is RECORDED. The displayed indication is recorded. |
| Constraints | Unique announcement of level transition (L1/LY as described in the Packet 41 defined in this test case with N_ITER = 0). If the level transition to LNTC is announced, the train shall be equipped with the NTC module compatible to the one defined with NID_NTC in the Level transition announcement. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| Linking | STORED | - |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 | - | An announcement of level transition to Level Y (Packet 41) is RECEIVED from an infill Balise Group: D_LEVELTR <> 0 and <> 32767 M_LEVELTR= M_LEVEL_Y (Level Y) | I | BTM | Order to switch to level Y at further location and this announcement will result in a change of the on-board level. | Same | Same | - |
| 2 | L1 | - | An indication of Announcement of Level Y is DISPLAYED. | O | DMI | - | Same | Same | - |
| 3 | L1 | - | An indication of Announcement of Level Y is RECORDED | O | JRU | - | Same | Same | - |
| 4 | L1 | - | The infill Balise Group message, including Packets 136 and 41,”Level transition order”, is RECORDED. | O | JRU | - | Same | Same | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | First |
| N_TOTAL | 3 | 001 | 2 balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 136 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | 0 | - |
| NIG_BG | 14 | RESERVED | - |
| NID_PACKET | 8 | 41 | Level Transition Order |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | FINITE VALUE | <> 0 and <>32767 |
| M_LEVELTR | 3 | M_LEVEL_Y | Transition to level Y |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL_Y = 1 (NTC) |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 001 | Second |
| N_TOTAL | 3 | 001 | 2 balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 UNCHANGED | FS / LS |
| Level Transition Order | STORED | M_LEVELTR = M_LEVEL_Y (Transition to level Y) NID_NTC = FINITE VALUE (If M_LEVEL_Y = 1 (NTC)) D_LEVELTR = FINITE VALUE (<> 0 and <> 32767) |
| ERTMS/ETCS level | 2 UNCHANGED | L1 (no change after announcement) |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| An indication of announcement of Level Y is DISPLAYED. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 36

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.1: Level Transitions General requirements |  | 5100100 |
| Test Case of Feature | The train receives an order to switch immediately from L1 to L3 at the current position. On-board equipment immediately switches to L3. |  | 36 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS, SR, SL, SB, TR, NL |  |  |
| Target of Test | The target of this test is to check that: The system switches to level 3 when the order to switch immediately to Level 3 is received on-board. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.3.1.1 Level Transition Order |  |
|  |  | Subset-026-4.8.4.2 Level Transition Order and Conditional Level Transition Order |  |
|  |  | Subset-026-7.4.2.9 Packet Number 41: Level Transition Order |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI (Except in SL mode) that: Level 3 indication is DISPLAYED. Check on the JRU that: Packet 41: “Level transition order” is RECORDED with M_LEVELTR = 4, D_LEVELTR = 0 or = 32767. New Level 3 is RECORDED |
| Constraints | A cab needs to be active (SB and TR Mode) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 5 / 6 / 7 / 11 / 12 | FS / OS / SR / SL / SB / TR / NL / LS |
| Linking | STORED | If Linking information is stored, Balise Group BGa shall be included if BGa is marked as linked. (The on-board equipment could not have Linking information). |
| Signalling related Speed Restriction | STORED | If previously received |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Level 1 is DISPLAYED | O | DMI | (Except in SL mode) |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 | - | The train reaches the Level transition border and an order to switch immediately to Level 3 is received from balise group BGa: Packet 41: “Level Transition Order” is RECEIVED. D_LEVELTR= 0 or = 32767 M_LEVELTR=4 | I | BTM | - | L1 | Same | - |
| 2 | L1 | - | Balise Group message, including Packet 41 ”Level transition order”, is RECORDED | O | JRU | - | L1 | Same | - |
| 3 | L1 | - | Level 3 is DISPLAYED | O | DMI | (Except in SL mode) The On-board equipment switches to Level 3. | L3 | Same | - |
| 4 | L3 | N/A | The new Level L3 is RECORDED | O | JRU | - | L3 | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | First |
| N_TOTAL | 3 | 001 | 2 balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 41 | Level Transition Order |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | 0 | Switch immediately |
| M_LEVELTR | 3 | 4 | Transition to level 3 |
| NID_NTC | 8 | FINITE VALUE | - |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 001 | Second |
| N_TOTAL | 3 | 001 | 2 balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | BGa | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 5 / 6 / 7 / 11 / 12 | FS / OS / SR / SL / SB / TR / NL / LS |
| Signalling related Speed Restriction | UNKNOWN | If previously received, it is deleted. |
| ERTMS/ETCS level | 4 | L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 37

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.1: Level Transitions General requirements |  | 5100100 |
| Test Case of Feature | The train receives an order to switch immediately from L1 to L2 at the current position. On-board equipment immediately switches to L2. |  | 37 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS, SR, SL, SB, TR, NL |  |  |
| Target of Test | The target of this test is to check that: The system switches to level 2 when the order to switch immediately to Level 2 is received on-board |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.3.1.1 Level Transition Order |  |
|  |  | Subset-026-4.8.4.2 Level Transition Order and Conditional Level Transition Order |  |
|  |  | Subset-026-7.4.2.9 Packet Number 41: Level Transition Order |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI (Except in SL mode) that: Level 2 indication is DISPLAYED. Check on the JRU that: Packet 41: “Level transition order” is RECORDED with M_LEVELTR = 3, D_LEVELTR = 0 or = 32767. New Level 2 is RECORDED. |
| Constraints | A cab needs to be active (SB and TR Mode) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 5 / 6 / 7 / 11 / 12 | FS / OS / SR / SL / SB / TR / NL / LS |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Level 1 is DISPLAYED. | O | DMI | Except in SL mode |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 | - | The train reaches the Level transition border and an order to switch immediately to Level 2 is received from balise group BGa. Packet 41: “Level Transition Order” is RECEIVED. D_LEVELTR= 0 or = 32767 M_LEVELTR=3 | I | BTM | - | Same | Same | - |
| 2 | L1 | - | Balise Group message, including Packet 41 ”Level transition order”, is RECORDED. | O | JRU | - | Same | Same | - |
| 3 | L1 | - | Level 2 is DISPLAYED. | O | DMI | (Except in SL mode) The On-board equipment switches to Level 2. | L2 | Same | - |
| 4 | L2 | - | New Level 2 is RECORDED. | O | JRU | - | Same | Same | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | First |
| N_TOTAL | 3 | 001 | 2 balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 41 | Level Transition Order |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | 0 or = 32767 | Switch immediately |
| M_LEVELTR | 3 | 3 | Transition to level 2 |
| NID_NTC | 8 | FINITE VALUE | - |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 001 | Second |
| N_TOTAL | 3 | 001 | 2 balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 5 / 6 / 7 / 11 / 12 | FS / OS / SR / SL / SB / TR / NL / LS |
| ERTMS/ETCS level | 3 | L2 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| New Level 2 is DISPLAYED. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 38

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.1: Level Transitions General requirements |  | 5100100 |
| Test Case of Feature | The on-board has switched to Level Y and a new MA related to level 1 is received. The new MA is rejected. |  | 38 |
| Applicable Mode/Level Combinations | L0: UN, SB LNTC: SB, SN L2: FS, LS, OS, SR, SB, PT L3: FS, LS, OS, SR, SB, PT |  |  |
| Target of Test | The target of this test is to check that: The on-board rejects a MA related to Level 1 when the Level Transition to Level Y has been performed |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-5.10.1.8 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : There is no change when the new MA is received Check on the JRU that: The new MA is received and rejected. |
| Constraints | Cab is active and valid Train Data are stored on-board if in SB The MA must follow the reception of the information “Recognition of Exit from TR mode” with a more recent time stamp if in PT |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 8 / 12 / 13 | FS / OS / SR / UN / SB / PT / LS / SN |
| ERTMS/ETCS level | M_LEVEL_Y | Level Y |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | Y | - | Onboard has just changed to LY . The train passes over a balise group BG 1 giving a Level 1 MA | I | BTM | - | Same | Same | - |
| 2 | Y | - | The new MA is rejected. No changes in the DMI are DISPLAYED | O | DMI | - | Same | Same | - |
| 3 | Y | - | The balise telegram is RECORDED | O | JRU | - | Same | Same | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | First |
| N_TOTAL | 3 | 001 | Two or more balises |
| M_DUP | 2 | 00 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | BG1 | BG 1 |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 12 | Level 1 Movement Authority |
| Q_DIR | 2 | 10 | Both directions |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | <> o |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out". |
| N_ITER | 5 | FINITE VALUE | Set to zero if V_MAIN = 0 or if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 001 | Second |
| N_TOTAL | 3 | 001 | 2 balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 8 / 12 / 13 UNCHANGED | FS / OS / SR / UN / SB / PT / LS / SN |
| ERTMS/ETCS level | M_LEVEL_Y UNCHANGED | Level Y |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |