| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 3160300 (3.16.3: Radio) Total: 14 Test case(s) |  |
| REF: | Subset-076-5-2-3160300 |
| ISSUE: | Version 3.2.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 2.9.0 31.03.2013 | All | First version/review | DLR |
| Version 2.9.1 17.09.2013 | All | review | DLR |
| Version 2.9.2 26.11.2013 | All | review | DLR |
| Version 2.9.3 05.12.2013 | All | review | DLR |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.2.0 31.03.2017 | All | Baseline 3 1st maintenance release version - Bug fixing | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.16.3: Radio |  | 3160300 |
| Test Case of Feature | Monitoring of radio message sequence from RBC |  | 1 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS, SR, SB, TR, PT, NL, RV L2: FS, LS, OS, SR, SB, TR, PT, NL, RV L3: FS, LS, OS, SR, SB, TR, PT, NL, RV |  |  |
| Target of Test | The target of this test is to check that: When a radio message is received and it has a time stamp lower than the time stamp of the last received message, the message is rejected. Since the message 34 is used, no request is displayed. The function check radio sequence is active |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.16.3.3.3 |  |
|  |  | Subset-026-3.16.1.1 |  |
|  |  | Subset-026-3.16.1.1 b) |  |
|  |  | Subset-026-3.16.3.1.4 |  |
|  |  | Subset-026-3.16.3.1.3 |  |
|  |  | Subset-026-3.16.3.3.2 |  |
|  |  | Subset-026-4.5.2.1 table - Check radio sequence |  |
|  |  | Subset-026-4.8.3.1.1 Track Ahead Free Request |  |
|  |  | Subset-026-4.8.4.2 Track Ahead Free Request |  |
|  |  | Subset-026-4.8.2.1 |  |
|  |  | Subset-026-4.8.2.1 a) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that: • Message 136: "Train Position Report" containing packet 4 (M_ERROR = 4) is transmitted Check on the DMI that: • No track ahead free request message is displayed |
| Constraints | The messages received shall only be data with normal priority. A time stamp wrap around shall not occur between the two received Euroradio messages in order that the condition T_TRAIN_1 < T_TRAIN_2 is valid The variables of the track ahead free request message will be chosen so as to allow the display of the message if the radio message was accepted. Mode SB is applicable only if a cab is active. Mode PT is applicable only if following the reception of the information “Recognition of Exit from TR mode” with a more recent time stamp. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 1 / 2 / 6 / 12 | OS/ SR /SB/LS |
| ERTMS/ETCS level | 2 / 3 / 4 | L1/ L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| L2/L3 : Radio link is CONNECTED | Both | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) is received | I | RTM | A first message is received from the RBC with T_TRAIN_1: | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 72 is received | I | RTM | A message 24 containing plain text (packet 72) is received from the RBC with time stamp T_TRAIN_2. The time stamp of this message is smaller than the time stamp of the first one: T_TRAIN_2<T_TRAIN_1 | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | NOT The "plain text message" is displayed | O | DMI | - | N/A | N/A | - |
| 6 | N/A | N/A | SA-DATA.Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 4 (M_ERROR = 4) is transmitted | O | RTM | Radio sequence error is reported | N/A | N/A | - |
| 7 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | 24 is an example. Can be other. Has to be unique |
| L_MESSAGE | 10 | FINITE VALUE | Must correspond to the computed record size(on-board) |
| T_TRAIN | 32 | T_TRAIN 1 | - |
| M_ACK | 1 | 0 | Acknowledge of message required = 1, Not required =0 |
| NID_LRBG | 24 | FINITE VALUE | ID number of LRBG |

| Step 3: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | Must correspond to the computed record size(on-board) |
| T_TRAIN | 32 | T_TRAIN 2 | T_TRAIN 2<T_TRAIN 1 |
| M_ACK | 1 | 0 | Acknowledge of message required = 1, Not required =0 |
| NID_LRBG | 24 | FINITE VALUE | ID number of LRBG |
| NID_PACKET | 8 | 72 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TEXTCLASS | 2 | FINITE VALUE | - |
| Q_TEXTDISPLAY | 1 | FINITE VALUE | Start/end events relation. |
| D_TEXTDISPLAY | 15 | FINITE VALUE | Start event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | Start event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | Start event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| L_TEXTDISPLAY | 15 | FINITE VALUE | End event. |
| T_TEXTDISPLAY | 10 | FINITE VALUE | End event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | End event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | End event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| Q_TEXTCONFIRM | 2 | FINITE VALUE | - |
| Q_CONFTEXTDISPLAY | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| Q_TEXTREPORT | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| NID_TEXTMESSAGE | 8 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_C | 10 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_RBC | 14 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| L_TEXT | 8 | FINITE VALUE | - |
| X_TEXT(L_TEXT) | 8 | FINITE VALUE | - |

| Step 6: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |
| NID_PACKET | 8 | 4 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| M_ERROR | 8 | 4 | Error type identifier: Radio sequence error |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 1 / 2 / 6 / 12 UNCHANGED | OS/ SR /SB /LS |
| ERTMS/ETCS level | 2 /3 / 4 UNCHANGED | L1/ L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The track ahead free request message is not displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.16.3: Radio |  | 3160300 |
| Test Case of Feature | Monitoring of radio message sequence from RBC |  | 2 |
| Applicable Mode/Level Combinations | L2: FS, LS, OS, SR, L3: FS, LS, OS, SR, |  |  |
| Target of Test | The target of this test is to check that: When the time stamp of an unconditional emergency stop message received as high priority data, is lower than the time stamp of the radio message received just before the train shall accept the radio message The function check radio sequence is active |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.16.3.1.4 |  |
|  |  | Subset-026-3.16.1.1 |  |
|  |  | Subset-026-3.16.1.1 b) |  |
|  |  | Subset-026-3.16.3.1.2 |  |
|  |  | Subset-026-3.16.3.3.2 |  |
|  |  | Subset-026-3.16.3.3.3 |  |
|  |  | Subset-026-4.5.2.1 table - Check radio sequence |  |
|  |  | Subset-026-4.8.2.1 |  |
|  |  | Subset-026-4.8.2.1 a) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that: • Message 147: "Acknowledgement of Emergency Stop" containing packet 0 is transmitted |
| Constraints | The second messages received shall be an unconditional emergency stop received as high priority data. A time stamp wrap around shall not occur between the two received Euroradio messages in order that the condition T_TRAIN_2 < T_TRAIN_1 is valid |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| L2/L3 : Radio link is CONNECTED | Both | RTM | - |
| NOT RELEVANT | O | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) is received | I | RTM | A first message is received from the RBC with T_TRAIN_1: | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-HP-DATA.Indication with Euroradio Message "Unconditional Emergency Stop" (NID_MESSAGE=16) is received | I | RTM | A second message is received: including T_TRAIN_2 (This radio message shall be an unconditional emergency message send as high priority data) T_TRAIN_2 < T_TRAIN_1 | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=7) is recorded | O | JRU | - | N/A | TR | - |
| 6 | N/A | TR | The Mode symbol "Trip" is displayed | O | DMI | - | N/A | TR | - |
| 7 | N/A | TR | The system status message "Emergency stop" is displayed | O | DMI | - | N/A | TR | - |
| 8 | N/A | TR | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23) is recorded | O | JRU | - | N/A | TR | - |
| 9 | N/A | TR | Emergency brake commanded | O | TIU | - | N/A | TR | - |
| 10 | N/A | TR | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | N/A | TR | - |
| 11 | N/A | TR | SA-DATA.Request with Euroradio Message "Acknowledgement of Emergency Stop" (NID_MESSAGE=147) containing packet 0 is transmitted | O | RTM | This is to check emergency stop message acceptance | N/A | TR | - |
| 12 | N/A | TR | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | TR | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | 24 is an example. Can be other. Has to be unique |
| L_MESSAGE | 10 | FINITE VALUE | Must correspond to the computed record size(on-board) |
| T_TRAIN | 32 | T_TRAIN_1 | - |
| M_ACK | 1 | FINITE VALUE | Acknowledge of message required = 1, Not required =0 |
| NID_LRBG | 24 | FINITE VALUE | ID number of LRBG |

| Step 3: Radio Message 16 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 16 | - |
| L_MESSAGE | 10 | FINITE VALUE | Must correspond to the computed record size(on-board) |
| T_TRAIN | 32 | T_TRAIN_2 | T_TRAIN_2 < T_TRAIN_1 |
| M_ACK | 1 | FINITE VALUE | Acknowledge of message required = 1, Not required =0 |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_EM | 4 | FINITE VALUE | Identification number of the emergency stop message |

| Step 11: Radio Message 147 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 147 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_EM | 4 | FINITE VALUE | Identification number of the acknowledged emergency message |
| Q_EMERGENCYSTOP | 2 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | 7 | Trip Mode |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 | TR |
| ERTMS/ETCS level | 3 / 4 UNCHANGED | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.16.3: Radio |  | 3160300 |
| Test Case of Feature | Monitoring of radio message sequence from RBC |  | 3 |
| Applicable Mode/Level Combinations | L2: FS, LS, OS L3: FS, LS, OS |  |  |
| Target of Test | The target of this test is to check that: When the time stamp of a conditional emergency stop message received as high priority data, is lower than the time stamp of the radio message received just before the train shall accept the radio message since time stamps of high priority messages shall not be used for the supervision of the sequence. The function check radio sequence is active |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.16.3.1.4 |  |
|  |  | Subset-026-3.16.1.1 |  |
|  |  | Subset-026-3.16.1.1 b) |  |
|  |  | Subset-026-3.16.3.3.2 |  |
|  |  | Subset-026-3.16.3.3.3 |  |
|  |  | Subset-026-3.16.3.3.3.1 |  |
|  |  | Subset-026-4.5.2.1 table - Check radio sequence |  |
|  |  | Subset-026-4.8.2.1 |  |
|  |  | Subset-026-4.8.2.1 a) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that: • Message 147: "Acknowledgement of Emergency Stop" containing packet 0 is transmitted |
| Constraints | The second message received shall be a conditional emergency stop send as high priority data. When evaluating the conditional emergency stop, the train has not jet passed with its min. safe front end the new stop location A time stamp wrap around shall not occur between the two received Euroradio messages in order that the condition T_TRAIN_2 < T_TRAIN_1 is valid |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 /12 | FS / OS / LS |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| L2/L3 : Radio link is CONNECTED | Both | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) is received | I | RTM | A first radio message is received from RBC with TRAIN 1: | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-HP-DATA.Indication with Euroradio Message "Conditional Emergency Stop" (NID_MESSAGE=15) is received | I | RTM | A second message is received: including T_TRAIN_2 (This radio message shall be a conditional emergency message) The time stamp of this message is smaller than the time stamp of the first one: T_TRAIN_2<T_TRAIN_1 | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The acceptance of the conditional emergency stop shall be tested by observing the shortening of the new stop location | N/A | N/A | - |
| 6 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | SA-DATA.Request with Euroradio Message "Acknowledgement of Emergency Stop" (NID_MESSAGE=147) containing packet 0 is transmitted | O | RTM | The radio message 147: Acknowledgement of Emergency Stop is sent to the RBC The Emergency Stop is accepted. If the new stop location is not beyond, the current EOA/ LOA (as defined in the MA) the EOA/ SVL is shortened to the new stop location (as defined by the conditional emergency stop). | N/A | N/A | - |
| 8 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | T_TRAIN_1 | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |

| Step 3: Radio Message 15 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 15 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | T_TRAIN_2 | T_TRAIN_2 < T_TRAIN_1 |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_EM | 4 | FINITE VALUE | Identification number of the emergency stop message |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_REF | 16 | FINITE VALUE | - |
| Q_DIR | 2 | FINITE VALUE | - |
| D_EMERGENCYSTOP | 15 | FINITE VALUE | Distance between LRBG and the position reference to the emergency stop |

| Step 7: Radio Message 147 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 147 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | T_TRAIN | On-board train time |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_EM | 4 | FINITE VALUE | Identification number of the acknowledged emergency message |
| Q_EMERGENCYSTOP | 2 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 UNCHANGED | FS / OS / LS |
| ERTMS/ETCS level | 3 / 4 UNCHANGED | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.16.3: Radio |  | 3160300 |
| Test Case of Feature | Detection of loss of radio link and initiation of the installed reaction |  | 4 |
| Applicable Mode/Level Combinations | L2: FS, LS, OS L3: FS, LS, OS |  |  |
| Target of Test | The target of this test is to check that: • When the difference between the time stamp of the latest consistent received message and the current on-board time is greater than the T_NVCONTACT parameter (national value) • the on-board shall apply NO REACTION, as defined in the M_NVCONTACT parameter. • The function check safe radio connection (only level 2/3) is active |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.16.3.4.1 |  |
|  |  | Subset-026-3.16.3.4.3 |  |
|  |  | Subset-026-4.5.2.1 table - Check safe radio connection (only level 2/3) |  |
|  |  | Subset-026-A.3.1 - Additional delay time to disconnection on supervision of safe radio connection |  |
|  |  | Subset-026-A.3.2 - Maximum time since creation in the RBC of last received telegram. |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : • Radio link supervision error (maximum time since the reception of the previous message) is RECORDED. |
| Constraints | Packet 3: National Values has been previously received with • T_NVCONTACT= FINITE VALUE and • M_NVCONTACT = 10 |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 /12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | - |
| National Values | STORED | Variable T_NVCONTACT: FINITE VALUE Maximum time since creation in the RBC of last received telegram (Default value ‘1111 1111=’ , Finite Values ‘0s’ to ‘254s’) Variable M_NVCONTACT: 10 System reaction if radio channel monitoring expires (T_NVCONTACT=10-No Reaction) |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| Radio link is CONNECTED. | Both | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) is received | I | RTM | RBC sends a general message to the On-board with period lower than T_NVCONTACT. | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The train does not receive any message in the period given by T_NVCONTACT | I | INT | No reception of messages due to errors of radio link. | N/A | N/A | - |
| 4 | N/A | N/A | RADIO ERROR (NID_MESSAGE_JRU=13, M_ERROR=5) is recorded | O | JRU | Safe radio connection error | N/A | N/A | - |
| 5 | N/A | N/A | No reaction due to safe radio connection error | I | INT | As T_NVCONTACT expires the defined reaction (no reaction) occurs | N/A | N/A | - |
| 6 | N/A | N/A | Additional time delay (60s) expires | I | INT | The on-board shall release the safe radio connection and then set-up it again | N/A | N/A | - |
| 7 | N/A | N/A | SA-DISCONNECT.Request is transmitted | O | RTM | The on-board releases the safe connection | N/A | N/A | - |
| 8 | N/A | N/A | SA-CONNECT.Request is transmitted | O | RTM | The on-board tries to set-up the connection again | N/A | N/A | - |
| 9 | N/A | N/A | SA-CONNECT.Confirm is received | I | RTM | The connection is set up | N/A | N/A | - |
| 10 | N/A | N/A | Steps 3 to 9 will repeat until newer message is received within T_NVCONTACT | I | INT | Train movement is supervised to stop at the end of the current MA (EoA or SvL) if no newer MA is received. | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 UNCHANGED | FS / OS / LS |
| ERTMS/ETCS level | 3 / 4 UNCHANGED | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION IS SET-UP | I/O | RTM | Radio link is reconnected during step 9 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 5

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.16.3: Radio |  | 3160300 |
| Test Case of Feature | Plausibility check for variables transmitted from RBC |  | 5 |
| Applicable Mode/Level Combinations | L0: UN, SH, PS, NL, SL L1: FS, LS, OS, SR, SH, PS, SL, TR, PT, SB, NL, RV L2: FS, LS, OS, SR, SH, PS, SL, TR, PT, SB, NL, RV L3: FS, LS, OS, SR, SH, PS, SL, TR, PT, SB, NL, RV LNTC:SN, NL, SL |  |  |
| Target of Test | The target of this test is to check that: • The radio message 24 is correct according to variables values( i.e. plausibility check ) • The whole message will be accepted by the on-board when the variables are correct. • Packet 42 contained in radio message 24 is accepted and managed • The function check the correctness of radio messages is correct |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.16.1.1 |  |
|  |  | Subset-026-3.16.1.1 a) |  |
|  |  | Subset-026-4.5.2.1 table - Check correctness of radio messages |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : • Message 24: “General Message”, including Packet 42 “Session Management”, is RECORDED. • Message 156: “Termination of a communication session” is RECORDED. • Message 39 “Acknowledgement of termination of a communication session” is RECORDED |
| Constraints | • Radio message 24 with a packet 42 with Q_RBC=0 (Terminate communication session) and Q_SLEEPSESSION=1 (Sleeping equipment shall manage session management information) • PT mode: radio message 24 is received after the information “Recognition of Exit from TR mode” with a more recent time stamp |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 4 / 5 / 6 / 7 / 8 / 11 /12 / 13 / 14 / 15 | FS / OS / SR / SH / UN / SL / SB / TR / PT / NL / LS /SN / RV / PS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| Radio communication session is ESTABLISED | Both | RTM | - |
| NOT RELEVANT | O | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 42 is received | I | RTM | The train receives a consistent radio message 24 with packet 42 - Variable values are accepted by the on-board. i.e. checking of plausibility and correctness | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA.Request with Euroradio Message "Termination of a communication session" (NID_MESSAGE=156) is transmitted | O | RTM | Radio message 156 ‘Termination of a communication session’ is sent As soon as this message is received by trackside (RBC), the RBC shall consider the communication session terminated and it shall send the acknowledgement to the on-board. It shall be checked that no more messages are sent by the onboard | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Acknowledgement of termination of a communication session" (NID_MESSAGE=39) is received | I | RTM | Message 39: “Acknowledgement of termination of a communication session” is RECEIVED from the RBC and the on-board equipment considers the communication session terminated. When the acknowledgement is received, the on-board shall consider the communication session terminated and it shall begin the release of the safe connection with trackside. | N/A | N/A | - |
| 6 | N/A | N/A | Message 39: “Acknowledgement of termination of a communication session” is RECORDED. | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | the train tries to release the safe connection | O | RTM | - | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 42 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_RBC | 1 | 0 | Terminate communication session. |
| NID_C | 10 | FINITE VALUE | RBC ETCS identity : NID_C not relevant if NID_RBC has value "Contact last known RBC". |
| NID_RBC | 14 | FINITE VALUE | - |
| NID_RADIO | 64 | FINITE VALUE | Not relevant if NID_RBC has value "Contact last known RBC". |
| Q_SLEEPSESSION | 1 | 1 | Order to contact RBC also in SL mode |

| Step 3: Radio Message 156 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 156 | - |
| L_MESSAGE | 10 | 74 | - |
| T_TRAIN | 32 | FINITE VALUE | t0 |
| NID_ENGINE | 24 | FINITE VALUE | Engine ID of the train |

| Step 5: Radio Message 39 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 39 | - |
| L_MESSAGE | 10 | 75 | - |
| T_TRAIN | 32 | FINITE VALUE | t0 + t’ with t’ > 0 |
| M_ACK | 1 | 0 | always set to 0 |
| NID_LRBG | 24 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 4 / 5 / 6 / 7 / 8 / 11 12 / 13 / 14 / 15 | FS / OS / SR / SH / UN / SL / SB / TR / PT / NL / LS SN / RV / PS |
| Radio communication session | TERMINATED | - |
| ERTMS/ETCS level | 3 / 4 0 / 2 / 1 | L2 / L3 L0 / L1/ LNTC |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION IS NOT SET-UP | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 6

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.16.3: Radio |  | 3160300 |
| Test Case of Feature | Plausibility check for variables transmitted from RBC |  | 6 |
| Applicable Mode/Level Combinations | L0: UN, SH, PS, NL, SL L1: FS, LS, OS, SR, SH, PS, SL, TR, PT, SB, NL, RV L2: FS, LS, OS, SR, SH, PS, SL, TR, PT, SB, NL, RV L3: FS, LS, OS, SR, SH, PS, SL, TR, PT, SB, NL, RV LNTC: SN, NL, SL |  |  |
| Target of Test | When the train receives a inconsistent radio message 24 with an incorrect value for a variable, which contains a packet 42 with a terminate communication session order: - The message is rejected - the trackside is informed - Terminate communication session order is not managed |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.16.3.1.1.1 |  |
|  |  | Subset-026-3.16.3.1.1.2 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that: • The radio message 24 with packet 42 is RECORDED. • No radio message 156 is RECORDED |
| Constraints | One of the variables in the radio message has an invalid value Radio message 24 with a packet 42 with Q_RBC=0 (Terminate communication session) and Q_SLEEPSESSION=1 (Sleeping equipment shall manage session management information) PT mode: radio message 24 is received after the information “Recognition of Exit from TR mode” with a more recent time stamp |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 4 / 5 / 6 / 7 / 8 / 11 /12 / 13 / 14 / 15 | FS / OS / SR / SH / UN / SL / SB / TR / PT / NL / LS /SN / RV / PS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| Radio communication session is ESTABLISED | Both | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 42 is received | I | RTM | The train receives an inconsistent radio message 24 with packet 42 One of the variables in the radio message has an invalid value A wrong value for L_MESSAGE, or a spare value in a variable, for example | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | Use_FT3060100.4 | - | - | An error report is sent: Message 136 with packet 4 M_ERROR=3 | N/A | N/A | - |
| 4 | N/A | N/A | No radio message 156 is sent | O | RTM | Terminate communication session order shall not be managed | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 42 | Session Management |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_RBC | 1 | 0 | Terminate communication session. |
| NID_C | 10 | FINITE VALUE | - |
| NID_RBC | 14 | FINITE VALUE | - |
| NID_RADIO | 64 | FINITE VALUE | - |
| Q_SLEEPSESSION | 1 | 1 | Order to contact RBC also in SL mode |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 4 / 5 / 6 / 7 / 8 / 11 12 / 13 / 14 / 15 | FS / OS / SR / SH / UN / SL / SB / TR / PT / NL / LS SN / RV / PS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 0 / 2 / 1 | L2 / L3 L0 / L1/ LNTC |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION IS SET-UP | I/O | RTM | - |
| NOT AFFECTED | O | TIU | - |
| NOT AFFECTED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 7

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.16.3: Radio |  | 3160300 |
| Test Case of Feature | Radio message acknowledgement for radio link to RBC |  | 7 |
| Applicable Mode/Level Combinations | L0: UN, SL, NL (if Level Transition to L2/L3 is received) L1: FS / LS / OS / SR / SL / NL LNTC: SN / SL / NL L2: FS / LS / OS / SR / SB / TR / PT / RV / SH / PS / SL / NL L3: FS / LS / OS / SR / SB / TR / PT / RV / SH / PS / SL / NL |  |  |
| Target of Test | The target of this test is to check that : • The on-board equipment acknowledges any consistent radio messages that request to be acknowledged |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.16.3.5.1 |  |
|  |  | Subset-026-3.16.3.5.3 |  |
|  |  | Subset-026-8.6.7 Message 146: Acknowledgement |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the RTM that : • A consistent radio message requiring to be acknowledged by the on-board equipment is received from the RBC. • The radio message 146 is sent by the on-board equipment to acknowledge the radio message previously received that requested to be acknowledged. Check on the JRU that : • A radio message requiring to be acknowledged by the on-board equipment is recorded. • The radio message 146 (Acknowledgement of radio message) sent by the on-board equipment is recorded |
| Constraints | • The radio message sent by the RBC requires to be acknowledged by the on-board equipment. • Radio messages to be acknowledged by the driver are NOT tested within the scope of this test case (e.g. General message 24 + text message to be displayed to the driver). • It is assumed that the radio message sent from the RBC is valid. • The radio message sent by the RBC is NOT an Emergency Stop message (covered with test cases of features 235 and 236). |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 4 / 5 / 6 / 7 / 8 / 10 / 11 / 12 / 13 / 14 / 15 | FS / OS / SR / SH / UN / SL / SB / TR / PT / IS / NL / LS / SN / RV / PS |
| Radio communication session | ESTABLISHED | A radio communication session is established with the RBC. |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| Radio link is CONNECTED | Both | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) is received | I | RTM | t = t0 The on-board equipment receives a consistent radio message from the RBC with: • M_ACK = 1 • T_TRAIN = t0 Any consistent radio message except emergency stop messages and messages that require to be acknowledged by the driver can be received. The radio message requires to be acknowledged since M_ACK is set to 1. | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | Recorded when received on-board. | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA.Request with Euroradio Message "Acknowledgement" (NID_MESSAGE=146) is transmitted | O | RTM | t = t0 + t’ The on-board equipment acknowledges the received radio message by sending the radio message 146 to the RBC with : • T_TRAIN = t0 The value of T_TRAIN in the message 146 is equal to the value of T_TRAIN in the message that required to be acknowledged (i.e. t0). | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | Except emergency stop messages 15 and 16. Except general message 24 + text information that requests acknowledgement by the driver. |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | t0 | - |
| M_ACK | 1 | 1 | The trackside requires the on-board equipment to acknowledge this radio message |
| NID_LRBG | 24 | FINITE VALUE | - |

| Step 3: Radio Message 146 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 146 | - |
| L_MESSAGE | 10 | 106 | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | Engine ID of the train |
| T_TRAIN | 32 | t0 | Same as the acknowledged message |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 4 / 5 / 6 / 7 / 8 / 10 / 11 / 12 / 13 / 14 / 15 UNCHANGED | FS / OS / SR / SH / UN / SL / SB / TR / PT / IS / NL / LS / SN / RV / PS |
| Radio communication session | UNCHANGED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 UNCHANGED | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION IS SET-UP | I/O | RTM | Radio link is still connected |
| NOT AFFECTED | O | TIU | - |
| NOT AFFECTED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 8

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.16.3: Radio |  | 3160300 |
| Test Case of Feature | Radio message acknowledgement for radio link to RBC |  | 8 |
| Applicable Mode/Level Combinations | L0: UN, SL, NL if Level Transition to L2/L3 is received) L1: FS / LS / OS / SR / SL / NL LNTC: SN / SL / NL L2: FS / LS / OS / SR / SB / TR / PT / RV / SH / PS / SL / NL L3: FS / LS / OS / SR / SB / TR / PT / RV / SH / PS / SL / NL |  |  |
| Target of Test | The target of this test is to check that : • The on-board equipment does not acknowledge inconsistent radio messages that request to be acknowledged. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.16.3.5.1 |  |
|  |  | Subset-026-8.6.7 Message 146: Acknowledgement |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the RTM that : • An inconsistent radio message requiring to be acknowledged by the on-board equipment is received from the RBC. • The radio message is NOT acknowledged Check on the JRU that : • A radio message requiring to be acknowledged by the on-board equipment is recorded. • The radio message 146 (Acknowledgement of radio message) is NOT recorded. |
| Constraints | • The radio message sent by the RBC is inconsistent. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 4 / 5 / 6 / 7 / 8 / 10 / 11 / 12 / 13 / 14 / 15 | FS / OS / SR / SH / UN / SL / SB / TR / PT / IS / NL / LS / SN / RV / PS |
| Radio communication session | ESTABLISHED | A radio communication session is established with the RBC. |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| Radio link is CONNECTED | Both | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) is received | I | RTM | The on-board equipment receives an inconsistent radio message, e.g. with a time stamp lower than that of a previously received message or an in-valid value, from the RBC with: • M_ACK = 1 | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | Recorded when received on-board. | N/A | N/A | - |
| 3 | N/A | N/A | Use_FT3060100.4 | - | - | The on-board equipment detects an inconsistency in the received radio message | N/A | N/A | - |
| 4 | N/A | N/A | NOT SA-DATA.Request with Euroradio Message "Acknowledgement" (NID_MESSAGE=146) is transmitted | O | RTM | - | N/A | N/A | - |
| 5 | N/A | N/A | NOT MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | 1 | The trackside requires the on-board equipment to acknowledge this radio message |
| NID_LRBG | 24 | FINITE VALUE | ID of LRBG |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 4 / 5 / 6 / 7 / 8 / 10 / 11 / 12 / 13 / 14 / 15 UNCHANGED | FS / OS / SR / SH / UN / SL / SB / TR / PT / IS / NL / LS / SN / RV / PS |
| Radio communication session | UNCHANGED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 UNCHANGED | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION IS SET-UP | I/O | RTM | Radio link is still connected |
| NOT AFFECTED | O | TIU | - |
| NOT AFFECTED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 9

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.16.3: Radio |  | 3160300 |
| Test Case of Feature | Radio message acknowledgement for link to Radio infill unit |  | 9 |
| Applicable Mode/Level Combinations | L1: FS, LS |  |  |
| Target of Test | The target of this test is to check that : • The on-board equipment acknowledges any consistent radio messages that request to be acknowledged |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.16.3.5.1 |  |
|  |  | Subset-026-3.16.3.5.3 |  |
|  |  | Subset-026-8.6.7 Message 146: Acknowledgement |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the RTM that : • The radio message 146 is sent by the on-board equipment to acknowledge the radio message previously received that required to be acknowledged. Check on the JRU that : • A radio message requiring to be acknowledged by the on-board equipment is recorded. • The radio message 146 (Acknowledgement of radio message) sent by the on-board equipment is recorded. |
| Constraints | • The radio message sent by the RIU requires to be acknowledged by the on-board equipment. • It is assumed that the radio message sent by the RIU is consistent. • The received radio message can be any message except emergency stop messages. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| Radio communication session | ESTABLISHED | A radio communication session is established with the RBC. |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| Radio link is CONNECTED | Both | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) is received | I | RTM | t ≈ t0 The on-board equipment receives a radio message from the RIU with: • M_ACK = 1 • T_TRAIN = t0 Any consistent radio message can be received. The radio message requires to be acknowledged since M_ACK is set to 1. | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | Recorded when received on-board. | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA.Request with Euroradio Message "Acknowledgement" (NID_MESSAGE=146) is transmitted | O | RTM | t = t1, t1 > t0 The on-board equipment acknowledges the received radio message by sending the radio message 146 to the RIU with : • T_TRAIN = t1 (time stamp of radio message) • T_TRAIN = t0 (time stamp contained in the message that is acknowledged) The value of T_TRAIN in the message 146 is equal to the value of T_TRAIN in the message that required to be acknowledged (i.e. t0). | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | Except emergency stop messages 15 and 16. |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | t0 | - |
| M_ACK | 1 | 1 | The trackside requires the on-board equipment to acknowledge this radio message |
| NID_LRBG | 24 | FINITE VALUE | - |

| Step 3: Radio Message 146 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 146 | Acknowledgement of radio message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | t1 > t0 |
| NID_ENGINE | 24 | FINITE VALUE | Engine ID of the train |
| T_TRAIN | 32 | FINITE VALUE | t0: Time stamp contained in the radio message that is acknowledged |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 UNCHANGED | FS / LS |
| Radio communication session | ESTABLISHED UNCHANGED | - |
| ERTMS/ETCS level | 2 UNCHANGED | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION IS SET-UP | I/O | RTM | - |
| NOT AFFECTED | O | TIU | - |
| NOT AFFECTED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 10

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.16.3: Radio |  | 3160300 |
| Test Case of Feature | Radio message acknowledgement for link to radio infill unit |  | 10 |
| Applicable Mode/Level Combinations | L1: FS, LS |  |  |
| Target of Test | The target of this test is to check that : • The on-board equipment does not send an acknowledgement radio message for radio messages that require to be acknowledged but are inconsistent. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.16.3.5.1 |  |
|  |  | Subset-026-8.6.7 Message 146: Acknowledgement |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : • A radio message requiring acknowledgement by the on-board equipment is recorded. • The acknowledgement message 146 is not recorded. Check on the RTM that : • The acknowledgement message 146 is not sent to the RIU. |
| Constraints | The radio message sent by the RIU is inconsistent. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| Radio communication session | ESTABLISHED | A radio communication session is established with the RIU. |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| Radio link is CONNECTED | Both | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) is received | I | RTM | The on-board equipment receives an inconsistent radio message from the RIU with: • M_ACK = 1 | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | NOT SA-DATA.Request with Euroradio Message "Acknowledgement" (NID_MESSAGE=146) is transmitted | O | RTM | The on-board equipment detects an inconsistency in the received radio message | N/A | N/A | - |
| 4 | N/A | N/A | NOT MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | 1 | The trackside requires the on-board equipment to acknowledge this radio message |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 72 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TEXTCLASS | 2 | FINITE VALUE | - |
| Q_TEXTDISPLAY | 1 | FINITE VALUE | Start/end events relation. |
| D_TEXTDISPLAY | 15 | FINITE VALUE | Start event. |
| M_MODETEXTDISPLAY | 4 | 3 | Spare |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | Start event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| L_TEXTDISPLAY | 15 | FINITE VALUE | End event. |
| T_TEXTDISPLAY | 10 | FINITE VALUE | End event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | End event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | End event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| Q_TEXTCONFIRM | 2 | FINITE VALUE | - |
| Q_CONFTEXTDISPLAY | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| Q_TEXTREPORT | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| NID_TEXTMESSAGE | 8 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_C | 10 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_RBC | 14 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| L_TEXT | 8 | FINITE VALUE | - |
| X_TEXT(L_TEXT) | 8 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 UNCHANGED | FS / LS |
| Radio communication session | ESTABLISHED UNCHANGED | - |
| ERTMS/ETCS level | 2 UNCHANGED | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION IS SET-UP | I/O | RTM | - |
| NOT AFFECTED | - | TIU | - |
| NOT AFFECTED | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 11

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.16.3: Radio |  | 3160300 |
| Test Case of Feature | Check for completeness of messages from RBC |  | 11 |
| Applicable Mode/Level Combinations | L2: FS, LS, OS, SR, SL, SB, TR, SH, PS, PT, NL, RV L3: FS, LS, OS, SR, SL, SB, TR, SH, PS, PT, NL, RV LNTC: SN |  |  |
| Target of Test | The target of this test is to check that: - When the on-board receives a radio message that cannot be decoded, it rejects it - The train receives a level transition information (order to switch immediately) but the message cannot be decoded so the transition does not take place |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.16.3.1.1.1 |  |
|  |  | Subset-026-3.16.3.1.1.2 |  |
|  |  | Subset-026-4.8.3.1.1 Level Transition Order |  |
|  |  | Subset-026-4.8.4.2 Level Transition Order and Conditional Level Transition Order |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : An error message is sent. Check on the DMI that: No Level transition is displayed |
| Constraints | The message sent shall be modified to make that the checks perfomed by Euroradio protocol have been failed (see Subset-037 MAC-Error) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/1/2/3/4/5/6/7/8/11/12/13/14 / 15 | FS/OS/SR/SH/UN/SL/SB/TR/PT/NL/LS/SN/RV/PS |
| ERTMS/ETCS level | 1 / 3 / 4 | LNTC / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| Radio link is CONNECTED | Both | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 41 is received | I | RTM | A radio message with a level transition information (Packet 41) (order to switch immediately) is received but it cannot be decoded | N/A | N/A | - |
| 2 | N/A | N/A | The RBC message is rejected by the on-board | I | INT | The packets included in the message received should be selected in order to be able to test this step | N/A | N/A | - |
| 3 | N/A | N/A | An error message is sent to the RBC | - | - | Use_FT3060100.4 | N/A | N/A | - |
| 4 | N/A | N/A | The error message is RECORDED. | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | No level transition is displayed | O | DMI | - | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | RESERVED | - |
| NID_PACKET | 8 | 41 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | FINITE VALUE | <> 0 and <> 32767 |
| M_LEVELTR | 3 | 0,1,2,3,4 | Transition to different of actual level |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTR = 1 (NTC). |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/1/2/3/4/5/6/7/8/11/12/13/14/15 UNCHANGED | FS/OS/SR/SH/UN /SL/SB/TR/PT/NL/LS/SN/RV/PS |
| ERTMS/ETCS level | 1/3/4 UNCHANGED | LNTC/L2/L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION IS SET-UP | I/O | RTM | If L2/L3 |
| NOT AFFECTED | - | TIU | - |
| NOT AFFECTED | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 12

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.16.3: Radio |  | 3160300 |
| Test Case of Feature | Consistency check of radio messages |  | 12 |
| Applicable Mode/Level Combinations | L2: SB / FS / LS / SR / OS / PT L3: SB / FS / LS / SR / OS / PT |  |  |
| Target of Test | The target of this test case is to check that : - When a radio message is received, the supervision of sequence uses only time stamps of message received as normal priority data |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.16.3.3.3 |  |
|  |  | Subset-026-3.16.3.3.3.1 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: • The third received message is accepted, i.e. the plain text message is displayed Check on the JRU that : • all three messages are recorded |
| Constraints | The first message received shall be a normal priority message. The second one, a high priority message. The third one is a normal priority message with time stamp lower than the one of the second message but higher than the one of the first message. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 8 / 12 | FS / OS / SR / SB / PT / LS |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Mode Profile | STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| Radio link is CONNECTED | Both | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) is received | I | RTM | A first radio message is received from RBC with T_TRAIN_1: | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-HP-DATA.Indication with Euroradio Message "Conditional Emergency Stop" (NID_MESSAGE=15) is received | I | RTM | A second message is received: including T_TRAIN_2 (This radio message shall be a conditional emergency message new stop location is not beyond the current EOA, so that EOA and SvL are moved accordingly) The time stamp of this message is bigger than the time stamp of the first one: T_TRAIN_2 > T_TRAIN_1 | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | SA-DATA.Request with Euroradio Message "Acknowledgement of Emergency Stop" (NID_MESSAGE=147, Q_EMERGENCYSTOP=0) including packet 0 is transmitted | O | RTM | The radio message 147: Acknowledgement of Emergency Stop is sent to the RBC The MA is shortened to the new location included in the conditional emergency stop | N/A | N/A | - |
| 6 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | MA is withdrawn to the new location of EoA/LOA | O | DMI | Restricted in LS. Some Information is available on DMI under certain conditions | N/A | N/A | - |
| 8 | N/A | N/A | The new target speed and target position is RECORDED | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | The system status message "Emergency stop" is displayed | O | DMI | - | - | - | - |
| 10 | N/A | N/A | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23, SYSTEM_STATUS_MESSAGE=<Bit17>) is recorded | O | JRU | - | - | - | - |
| 11 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 72 (Q_DIR=1, X_TEXT=”NP MSG display correct 3.16.3.3.3.1”) is received | I | RTM | Before the train reaches the emergency stop location, a third radio message, namely message 24, is received from RBC with T_TRAIN_3 The time stamp of this message is lower than the one of the second message but higher than the one of the first message: T_TRAIN_1 < T_TRAIN_2 > T_TRAIN 3 | N/A | N/A | - |
| 12 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | The third message is accepted | N/A | N/A | - |
| 13 | N/A | N/A | The "NP MSG display correct 3.16.3.3.3.1" is displayed | O | DMI | - | N/A | N/A | - |
| 14 | N/A | N/A | START DISPLAYING PLAIN TEXT MESSAGE (NID_MESSAGE_JRU=18) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | T_TRAIN_1 | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |

| Step 3: Radio Message 15 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 15 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_EM | 4 | FINITE VALUE | Identification number of the emergency stop message |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_REF | 16 | FINITE VALUE | Emergency stop location is in from of current EOA |
| Q_DIR | 2 | FINITE VALUE | - |
| D_EMERGENCYSTOP | 15 | FINITE VALUE | Distance between LRBG and the position reference to the emergency stop |

| Step 5: Radio Message 147 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 147 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | T_TRAIN | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_EM | 4 | FINITE VALUE | Identification number of the acknowledged emergency message |
| Q_EMERGENCYSTOP | 2 | 0 | Conditional Emergency Stop accepted, with update of current EOA/LOA |
| NID_PACKET | 8 | 0 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| Step 11: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | T_TRAIN_3 | T_TRAIN_1 < T_TRAIN_3 < T_TRAIN_2 |
| M_ACK | 1 | 0 | Acknowledge of message required = 1, Not required =0 |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 72 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TEXTCLASS | 2 | FINITE VALUE | - |
| Q_TEXTDISPLAY | 1 | FINITE VALUE | Start/end events relation. |
| D_TEXTDISPLAY | 15 | FINITE VALUE | Start event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | Start event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | Start event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| L_TEXTDISPLAY | 15 | FINITE VALUE | End event. |
| T_TEXTDISPLAY | 10 | FINITE VALUE | End event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | End event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | End event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| Q_TEXTCONFIRM | 2 | FINITE VALUE | - |
| Q_CONFTEXTDISPLAY | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| Q_TEXTREPORT | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| NID_TEXTMESSAGE | 8 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_C | 10 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_RBC | 14 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| L_TEXT | 8 | FINITE VALUE | - |
| X_TEXT(L_TEXT) | 8 | “The "NP MSG display correct 3.16.3.3.3.1" is displayed” | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 8 / 12 | FS / OS / SR / SB / PT / LS |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT AFFECTED | - | TIU | - |
| The "NP MSG display correct 3.16.3.3.3.1" is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 13

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.16.3: Radio |  | 3160300 |
| Test Case of Feature | Consistency check of radio messages |  | 13 |
| Applicable Mode/Level Combinations | L2: FS / LS / OS / PT L3: FS / LS / OS / PT |  |  |
| Target of Test | The target of this test case is to check that : - When a radio message is received, the supervision of sequence uses only time stamps of message received as normal priority data |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.16.3.3.3 |  |
|  |  | Subset-026-3.16.3.3.3.1 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the interfaces that: • The third received message is accepted, i.e. the plain text message is displayed Check on the JRU that : • all three messages are recorded |
| Constraints | The first message received shall be a normal priority message. The second one, a high priority message. The third one is a normal priority message with time stamp lower than the one of the second message but higher than the one of the first message. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 8 / 12 | FS / OS / PT / LS |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Mode Profile | STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| Radio link is CONNECTED | Both | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) is received | I | RTM | A first radio message is received from RBC with T_TRAIN_1: | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-HP-DATA.Indication with Euroradio Message "Conditional Emergency Stop" (NID_MESSAGE=15) is received | I | RTM | A second message is received: including T_TRAIN_2 (This radio message shall be a conditional emergency message) The time stamp of this message is greater than the time stamp of the first one: T_TRAIN_2 > T_TRAIN_1 | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | SA-DATA.Request with Euroradio Message "Acknowledgement of Emergency Stop" (NID_MESSAGE=147) is transmitted | O | RTM | The radio message 147: Acknowledgement of Emergency Stop is sent to the RBC If the Emergency Stop is accepted, the MA is shortened to the new location included in the conditional emergency stop | N/A | N/A | - |
| 6 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | Shortening MA shall be withdrawn to the new location of EoA/LOA | O | DMI | Restricted in LS. Some Information is available on DMI under certain conditions | N/A | N/A | - |
| 8 | N/A | N/A | The new target speed and target position is RECORDED | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 72 (Q_DIR=1, X_TEXT=”NP MSG display correct 3.16.3.3.3.1”) is received | I | RTM | A third radio message, namely message 24, is received from RBC with T_TRAIN_3 The time stamp of this message is lower than the one of the second message but higher than the one of the first message: T_TRAIN_1 < T_TRAIN_3 < T_TRAIN_2 | N/A | N/A | - |
| 10 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 11 | N/A | N/A | The "NP MSG display correct 3.16.3.3.3.1" is displayed | O | DMI | The third message is accepted | N/A | N/A | - |
| 12 | N/A | N/A | START DISPLAYING PLAIN TEXT MESSAGE (NID_MESSAGE_JRU=18) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | T_TRAIN_1 | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |

| Step 3: Radio Message 15 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 15 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | T_TRAIN_2 | T_TRAIN_2 > T_TRAIN_1 |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_EM | 4 | FINITE VALUE | Identification number of the emergency stop message |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_REF | 16 | FINITE VALUE | - |
| Q_DIR | 2 | FINITE VALUE | - |
| D_EMERGENCYSTOP | 15 | FINITE VALUE | Distance between LRBG and the position reference to the emergency stop |

| Step 5: Radio Message 147 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 147 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | T_TRAIN | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_EM | 4 | FINITE VALUE | Identification number of the acknowledged emergency message |
| Q_EMERGENCYSTOP | 2 | FINITE VALUE | - |

| Step 9: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | T_TRAIN_3 | T_TRAIN_1 < T_TRAIN_3 < T_TRAIN_2 |
| M_ACK | 1 | 0 | Acknowledge of message required = 1, Not required =0 |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 72 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TEXTCLASS | 2 | FINITE VALUE | - |
| Q_TEXTDISPLAY | 1 | FINITE VALUE | Start/end events relation. |
| D_TEXTDISPLAY | 15 | FINITE VALUE | Start event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | Start event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | Start event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| L_TEXTDISPLAY | 15 | FINITE VALUE | End event. |
| T_TEXTDISPLAY | 10 | FINITE VALUE | End event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | End event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | End event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| Q_TEXTCONFIRM | 2 | FINITE VALUE | - |
| Q_CONFTEXTDISPLAY | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| Q_TEXTREPORT | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| NID_TEXTMESSAGE | 8 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_C | 10 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_RBC | 14 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| L_TEXT | 8 | FINITE VALUE | - |
| X_TEXT(L_TEXT) | 8 | “The "NP MSG display correct 3.16.3.3.3.1" is displayed” | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 8 / 12 | FS / OS / PT / LS |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT AFFECTED | - | TIU | - |
| The "NP MSG display correct 3.16.3.3.3.1" is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 14

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.16.3: Radio |  | 3160300 |
| Test Case of Feature | The onboard equipment has received a message which is NOT an emergency message as high priority data |  | 14 |
| Applicable Mode/Level Combinations | L2: FS, LS, OS, SR, SB, TR, PT, NL, RV L3: FS, LS, OS, SR, SB, TR, PT, NL, RV |  |  |
| Target of Test | The target of this test is to check that: • The message including packet 57 is received as high priority data on-board • The message is NOT acknowledged • The message is REJECTED |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.16.3.1.3 |  |
|  |  | Subset-026-3.16.3.1.3.1 |  |
|  |  | Subset-026-4.8.3.1.1 MA Request Parameters |  |
|  |  | Subset-026-4.8.4.2 MA Request Parameters |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |
|  |  | Subset-026-7.4.2.14 Packet Number 57: MA request Parameters |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that: • Message including packet 57 (MA request prameters) is recorded. • Message 147: "Acknowledgement of Emergency Stop" is NOT recorded |
| Constraints | Mode SB is applicable only if a cab is active. Mode PT is applicable only if following the reception of the information “Recognition of Exit from TR mode” with a more recent time stamp. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 | FS / OS |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| Radio link is CONNECTED | Both | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-HP-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 72 is received | I | RTM | The RBC sends a message 24 (General Message) including packet 72 (plain text message) as high priority data The message 24 and packet 72 are NOT accepted by the on-board Complete message is NOT accepted The length of this message must not exceed 25 bytes (see SUBSET-037 v320, 5.6.1.3). | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | Complete message is recorded | N/A | N/A | - |
| 3 | N/A | N/A | NOT An acknowledgement message is sent. | O | RTM | Accepted high priority messages are acknowledged. This message is rejected so no acknowledgement shall be sent | N/A | N/A | - |
| 4 | N/A | N/A | NOT The "plain text message" is displayed | O | DMI | Plain text message is rejected, so it is not displayed | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 72 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TEXTCLASS | 2 | FINITE VALUE | - |
| Q_TEXTDISPLAY | 1 | FINITE VALUE | Start/end events relation. |
| D_TEXTDISPLAY | 15 | FINITE VALUE | Start event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | Start event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | Start event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| L_TEXTDISPLAY | 15 | FINITE VALUE | End event. |
| T_TEXTDISPLAY | 10 | FINITE VALUE | End event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | End event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | End event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| Q_TEXTCONFIRM | 2 | FINITE VALUE | - |
| Q_CONFTEXTDISPLAY | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| Q_TEXTREPORT | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| NID_TEXTMESSAGE | 8 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_C | 10 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_RBC | 14 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| L_TEXT | 8 | FINITE VALUE | - |
| X_TEXT(L_TEXT) | 8 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 UNCHANGED | FS / OS |
| Radio communication session | STORED UNCHANGED | - |
| Linking | STORED UNCHANGED | - |
| Movement Authority | STORED UNCHANGED | - |
| Gradient Profile | STORED UNCHANGED | - |
| International SSP | STORED UNCHANGED | - |
| ERTMS/ETCS level | 3 / 4 UNCHANGED | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION IS SET-UP | I/O | RTM | - |
| NOT AFFECTED | - | TIU | - |
| NOT AFFECTED | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |