| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 5070300 (5.7.3: Shunting is requested for a further location) Total: 4 Test case(s) |  |
| REF: | Subset-076-5-2-5070300 |
| ISSUE: | Version 3.2.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 2.9.0 31.03.2013 | All | First version/review for Baseline 3 | MULTITEL |
| Version 2.9.1 05.07.2013 | All | Template standardised with a dedicated tool, TCs renumbered, Answer to Review001_Subset-076-5-2-5070300_v290 | MULTITEL |
| Version 2.9.2 03.12.2013 | All | Apply of Review001_Subset-076-5-2-5070300_v291 | MULTITEL |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.2.0 31.03.2017 | All | Baseline 3 1st maintenance release version - Bug fixing | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.7.3: Shunting is requested for a further location |  | 5070300 |
| Test Case of Feature | The beginning of the SH area is considered as the EOA (keeping the SvL given by the MA),with no release speed. The driver does not acknowledge the Shunting area and the system applies the brakes. After the switch to SH, acknowledgement for shunting is NOT acknowledged within the driver acknowledgement time. The system applies the service brake. |  | 1 |
| Applicable Mode/Level Combinations | L1: FS, OS L2: FS, OS L3: FS, OS |  |  |
| Target of Test | The target of this test is to check that: The beginning of SH area is supervised as EOA (the train keeps the SvL given by the MA) The on-board equipment reaches the max. safe front end of SH area. The on-board switches to SH mode (There is an end of mission (in levels 2 and 3)) The driver does not acknowledge within the driver acknowledgement time The service brake is applied. The service brake is released when driver acknowledges |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.6.2 row 4 column 5 |  |
|  |  | Subset-026-4.6.2 row 4 column 8 |  |
|  |  | Subset-026-4.6.3 Condition Id [51] |  |
|  |  | Subset-026-4.7.2 table - Input information Ackn of Shunting mode |  |
|  |  | Subset-026-5.7.3.2 |  |
|  |  | Subset-026-5.7.3.2 a) |  |
|  |  | Subset-026-5.7.3.2 b) |  |
|  |  | Subset-026-5.7.3.4 |  |
|  |  | Subset-026-5.7.3.6 |  |
|  |  | Subset-026-5.7.3.7 |  |
|  |  | Subset-026-A.3.1 - Mode transitions: Driver acknowledgement time |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: Indication SH mode is DISPLAYED Acknowledgement for SH is DISPLAYED Check on the JRU that: The transition to SH mode is RECORDED Termination of communication session is RECORDED (in levels 2 and 3) The triggering of Service Brake is RECORDED The driver request is RECORDED The release of Service Brake is RECORDED |
| Constraints | The speed is lower than the Shunting mode permitted speed No brake command is triggered due to other reasons than shunting acknowledgement. The distance between the estimated front end of the train and the beginning of shunting area is longer than L_ACKMAMODE |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 | FS / OS |
| Radio communication session | ESTABLISHED | In levels 2 and 3 |
| Mode Profile | STORED | M_MAMODE=01 (The mode profile requires Shunting) Q_MAMODE=0 (The beginning of the mode profile is considered as the EOA (keeping the SvL given by the MA)) |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | In levels 2 and 3 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train Speed: V | I | INT | The train speed is lower than the Shunting mode speed limit (National Value, or value given in the mode profile) |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | FS / OS | A target distance to EOA is DISPLAYED | O | DMI | - | N/A | FS / OS | - |
| 2 | N/A | FS / OS | The train speed is lower than the Shunting mode speed limit (National Value, or value given in the mode profile) | O | DMI | - | N/A | FS / OS | - |
| 3 | N/A | FS / OS | The Mode symbol "Acknowledge Shunting" is displayed | O | DMI | The distance between the estimated front end of the train and the beginning of shunting area becomes shorter than L_ACKMAMODE | N/A | FS / OS | - |
| 4 | N/A | FS / OS | Service brake commanded | O | TIU | The Service Brake is applied because of the EOA Until the switch to SH, the beginning of the Shunting area is considered as an EOA keeping the SvL given by the MA, with no release speed. The train is allowed to enter in the SH area by a distance corresponding to the length of the confidence interval | N/A | FS / OS | - |
| 5 | N/A | FS / OS | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | Application of Service Brake is RECORDED | N/A | FS / OS | - |
| 6 | N/A | FS / OS | The on-board equipment reaches the SH area with its max. safe front end. The system switches to SH mode | O | DMI | - | N/A | SH | - |
| 7 | N/A | SH | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=3) is recorded | O | JRU | The transition to SH mode is RECORDED | N/A | SH | - |
| 8 | N/A | SH | Service brake not commanded | O | TIU | The Service Brake is released | N/A | SH | - |
| 9 | N/A | SH | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=0) is recorded | O | JRU | Release of Service Brake is RECORDED. | N/A | SH | - |
| 10 | N/A | SH | The Mode symbol "Acknowledge Shunting" is displayed | O | DMI | The request for acknowledgement for shunting mode is still displayed | N/A | SH | - |
| 11 | N/A | SH | “Inform trackside about end of Mission” | - | - | Use_FT5050300.3 : “Inform trackside about end of Mission” (for levels 2 and 3) | N/A | SH | - |
| 12 | N/A | SH | NOT The driver acknowledges "Shunting" | O | DMI | Driver acknowledgement time The driver has NOT acknowledged within TACK after the change to SH | N/A | SH | - |
| 13 | N/A | SH | Service brake commanded | O | TIU | The Service Brake is applied | N/A | SH | - |
| 14 | N/A | SH | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | The Service Brake is applied is RECORDED | N/A | SH | - |
| 15 | N/A | SH | The driver acknowledges "Shunting" | I | DMI | The driver acknowledges "Shunting" | N/A | SH | - |
| 16 | N/A | SH | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | The driver action is RECORDED | N/A | SH | - |
| 17 | N/A | SH | Service brake not commanded | O | TIU | The Service Brake is released | N/A | SH | - |
| 18 | N/A | SH | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=0) is recorded | O | JRU | Release of Service Brake is RECORDED. | N/A | SH | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 | SH |
| Radio communication session | ESTABLISHED | In levels 2 and 3 |
| ERTMS/ETCS level | 2 / 3 / 4 UNCHANGED | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | In levels 2 and 3 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.7.3: Shunting is requested for a further location |  | 5070300 |
| Test Case of Feature | The On-Board equipment is in FS or OS mode and an SH area starts at a further location. The beginning of the SH area is considered as both the EOA and SvL (instead of the EOA and SvL given by the MA),with no release speed. The driver does not acknowledge the Shunting area |  | 2 |
| Applicable Mode/Level Combinations | L1: FS, OS L2: FS, OS L3: FS, OS |  |  |
| Target of Test | The target of this test is to check that the beginning of the shunting area is considered as both the EOA and SvL (instead of the EOA and SvL given by the MA),with no release speed as long as the driver does not acknowledge the Request for Shunting. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-5.7.3.2 |  |
|  |  | Subset-026-5.7.3.2 a) |  |
|  |  | Subset-026-5.7.3.2 b) |  |
|  |  | Subset-026-5.7.3.4 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: acknowledgement request for SH is displayed |
| Constraints | The speed is lower than the Shunting mode permitted speed Q_MAMODE=1 The distance between the estimated front end of the train and the beginning of shunting area is longer than L_ACKMAMODE |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 | FS / OS |
| Mode Profile | STORED | M_MAMODE=01 (The mode profile requires Shunting) D_MAMODE >0 (SH is requested for further location) Q_MAMODE=1 (The beginning of the mode profile is considered as both the EOA and SvL (instead of the EOA and SvL given by the MA)) |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train Speed: V | I | INT | The train speed is lower than the Shunting mode speed limit (National Value, or value given in the mode profile) |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Driver asks for the Target Distance. | I | DMI | Only if the Target Distance is not yet displayed | N/A | N/A | - |
| 2 | N/A | N/A | The Target Distance is displayed and it corresponds to the distance to the beginning of the Shunting area. | O | DMI | - | N/A | N/A | - |
| 3 | N/A | N/A | The train speed is lower than the Shunting mode speed limit (National Value, or value given in the mode profile) | O | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | The Mode symbol "Acknowledge Shunting" is displayed | O | DMI | The distance between the estimated front end of the train and the beginning of shunting area becomes shorter than L_ACKMAMODE | N/A | N/A | - |
| 5 | N/A | N/A | NOT The driver acknowledges "Shunting" | I | DMI | - | N/A | N/A | - |
| 6 | N/A | N/A | Service brake commanded | O | TIU | The beginning of the Shunting area is considered as an EOA and SvL with no release speed. The train APPLIES the service brake to reach a speed of zero at the beginning of the shunting area. | N/A | N/A | - |
| 7 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 UNCHANGED | FS / OS / LS |
| ERTMS/ETCS level | 2 / 3 / 4 UNCHANGED | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Service brake commanded | O | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.7.3: Shunting is requested for a further location |  | 5070300 |
| Test Case of Feature | Once the request for acknowledgement is displayed, it shall not be taken back, even if the related conditions are no more fulfilled e.g. the train accelerates |  | 3 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS L2: FS, LS, OS L3: FS, LS, OS |  |  |
| Target of Test | The target of this test is to check that once the request for acknowledgement is displayed, it remains displayed if the train speed becomes greater than the maximum shunting speed |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-5.7.3.2 |  |
|  |  | Subset-026-5.7.3.2 a) |  |
|  |  | Subset-026-5.7.3.2 b) |  |
|  |  | Subset-026-5.7.3.3 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The acknowledgement request for SH is displayed The acknowledgement for SH is not taken back when the train speed becomes greater than the Shunting mode speed limit (National Value, or value given in the mode profile) |
| Constraints | The speed is lower than the Shunting mode permitted speed before speed increase and after speed decrease The distance between the estimated front end of the train and the beginning of shunting area is longer than L_ACKMAMODE |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Mode Profile | STORED | M_MAMODE=01 (The mode profile requires Shunting) D_MAMODE >0 (SH is requested for further location) |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train Speed: V | I | INT | The train speed is lower than the Shunting mode speed limit (National Value, or value given in the mode profile) |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The train speed is lower than the Shunting mode speed limit (National Value, or value given in the mode profile) | O | DMI | - | N/A | N/A | - |
| 2 | N/A | N/A | Shunting acknowledgement is DISPLAYED | O | DMI | The distance between the estimated front end of the train and the beginning of shunting area becomes shorter than L_ACKMAMODE | N/A | N/A | - |
| 3 | N/A | N/A | The Train speed increases so that it is greater than the Shunting mode speed limit (National Value, or value given in the mode profile) | O | DMI | The driver does not acknowledge the Shunting The Shunting acknowledgement remains displayed | N/A | N/A | - |
| 4 | N/A | N/A | The acknowledgement for SH is not taken back. It is still displayed | O | DMI | - | N/A | N/A | - |
| 5 | N/A | N/A | The train speed decreases so that it is lower than the Shunting mode speed limit (National Value, or value given in the mode profile) | O | DMI | The driver does not acknowledge the Shunting The Shunting acknowledgement remains displayed | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 UNCHANGED | FS / OS / LS |
| ERTMS/ETCS level | 2 / 3 / 4 UNCHANGED | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.7.3: Shunting is requested for a further location |  | 5070300 |
| Test Case of Feature | Acknowledgement for shunting is requested, driver acknowledges, the system switches to SH mode, the communication session is not established and the train don´t try to establish a communication session to inform the trackside |  | 4 |
| Applicable Mode/Level Combinations | L2: FS, LS, OS L3: FS, LS, OS |  |  |
| Target of Test | The target of this test is to check that: The on-board equipment switches to SH mode after the acknowledgement of the driver takes place. The train didn’t try to establish the communication to inform the trackside |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.6.2 row 4 column 5 |  |
|  |  | Subset-026-4.6.2 row 4 column 6 |  |
|  |  | Subset-026-4.6.2 row 4 column 8 |  |
|  |  | Subset-026-4.6.3 Condition Id [50] |  |
|  |  | Subset-026-4.7.2 table - Input information Ackn of Shunting mode |  |
|  |  | Subset-026-5.7.3.2 |  |
|  |  | Subset-026-5.7.3.2 a) |  |
|  |  | Subset-026-5.7.3.2 b) |  |
|  |  | Subset-026-5.5.4.1.2 |  |
|  |  | Subset-026-5.7.3.5 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: Indication SH mode is DISPLAYED Check on the JRU that: The transition to SH mode is RECORDED Establishment of communication session is not recorded |
| Constraints | The speed is lower than the Shunting mode permitted speed The communication session is not established when the acknowledgement area is reached The distance between the estimated front end of the train and the beginning of shunting area is longer than L_ACKMAMODE |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | Not Established | - |
| Mode Profile | STORED | M_MAMODE=01 (The mode profile requires Shunting) |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | I | INT | The train speed is lower than the Shunting mode speed limit (National Value, or value given in the mode profile) |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2 / L3 | FS / OS / LS | The train speed is lower than the Shunting mode speed limit (National Value, or value given in the mode profile) | O | DMI | - | L2 / L3 | FS / OS / LS | - |
| 2 | L2 / L3 | FS / OS / LS | The request for acknowledgement for shunting mode is displayed | O | DMI | The distance between the estimated front end of the train and the beginning of shunting area becomes shorter than L_ACKMAMODE | L2 / L3 | FS / OS / LS | - |
| 3 | L2 / L3 | FS / OS / LS | The text message of acknowledgment for SH mode is acknowledged | I | DMI | - | L2 / L3 | SH | - |
| 4 | L2 / L3 | SH | Use_FT4070201.20 : When the onboard is in Shunting mode a permanent indication of Shunting is displayed. | O | DMI | Indication of SH mode is DISPLAYED | L2 / L3 | SH | - |
| 5 | L2 / L3 | SH | The transition to SH mode is RECORDED | O | JRU | - | L2 / L3 | SH | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 | SH |
| Radio communication session | Not established | - |
| ERTMS/ETCS level | 3 / 4 UNCHANGED | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |