| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 3120200 (3.12.2: Route Suitability) Total: 3 Test case(s) |  |
| REF: | Subset-076-5-2-3120200 |
| ISSUE: | Version 3.2.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 2.9.0 31.03.2013 | All | First version/review | DLR |
| Version 2.9.1 15.10.2013 | All | review | DLR |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.2.0 31.03.2017 | All | Baseline 3 1st maintenance release version - Bug fixing | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.12.2: Route Suitability |  | 3120200 |
| Test Case of Feature | Supervision of loading gauge profile. |  | 1 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS L2: FS, LS, OS L3: FS, LS, OS |  |  |
| Target of Test | The target of this test is to check that: - The train supervises train movements against a loading gauge profile - The information of unsuitability is displayed to the driver. - The corresponding location is supervised like EOA and SvL with no release speed - The on-board shall trip, when the location is overpassed. - The function manage Route Suitability is active |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.5.2.1 table - Manage Route Suitability |  |
|  |  | Subset-026-4.7.2 table - Output information Route unsuitability(ies) |  |
|  |  | Subset-026-3.12.2.3 |  |
|  |  | Subset-026-3.12.2.3 a) |  |
|  |  | Subset-026-3.12.2.4 |  |
|  |  | Subset-026-3.12.2.8 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on DMI that: • Information about unsuitability is DISPLAYED. Check on JRU that: • The train is not allowed to enter the route because the profile is not compatible with the train data (D_TRAIN< D_SUITABILITY). |
| Constraints | Before execution of test case the following preconditions have to be satisfied: - The RECEIVED Line gauge profile is incompatible with the train data. - The driver does not choose a location to stop the train. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Position Report parameters | STORED | V_TRAIN≠0 (Train running) |
| Route Suitability Data | STORED | Packet 70 D_SUITABILITY=FINITE VALUE Q_SUITABILITY=0 (Loading gauge Profile) M_LINEGAUGE=FINITE VALUE |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The system status message "Route unsuitable – loading gauge" is displayed | O | DMI | The system status message "Route unsuitable – loading gauge" is displayed. This information shall be DISPLAYED when RECEIVED. | N/A | N/A | - |
| 2 | N/A | N/A | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23) is recorded | O | JRU | Route unsuitable – loading gauge | N/A | N/A | - |
| 3 | N/A | N/A | Train is moving (V_TRAIN=40) | I | INT | The train overpasses the location | N/A | N/A | - |
| 4 | N/A | FS / OS / LS | The Mode symbol "Trip" is displayed | O | DMI | - | N/A | TR | - |
| 5 | N/A | N/A | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=7) is recorded | O | JRU | Mode change is recorded | N/A | N/A | - |
| 6 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit19=1>) is recorded | O | JRU | MASK=0000000000000000040000 | N/A | N/A | - |
| 7 | N/A | N/A | Emergency brake commanded | O | TIU | - | N/A | N/A | - |
| 8 | N/A | N/A | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | The system status message "Emergency stop" is displayed | O | DMI | - | N/A | N/A | - |
| 10 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=1>) is recorded | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 | TR |
| Position Report parameters | STORED | V_TRAIN=0 (Train at standstill) |
| Route Suitability Data | STORED | Q_SUITABILITY=0 (Loading gauge Profile) |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Emergency brake commanded | O | TIU | - |
| The system status message "Route unsuitable – loading gauge" is displayed The Mode symbol "Trip" is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.12.2: Route Suitability |  | 3120200 |
| Test Case of Feature | Supervision of power supply profile. |  | 2 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS L2: FS, LS, OS L3: FS, LS, OS |  |  |
| Target of Test | The target of this test is to check that: - The train supervises train movements against power supply profile - The information of unsuitability is displayed to the driver - The corresponding location is supervised like EOA and SvL with no release speed - The on-board shall trip, when the location is overpassed. - The function manage Route Suitability is active |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.5.2.1 table - Manage Route Suitability |  |
|  |  | Subset-026-4.7.2 table - Output information Route unsuitability(ies) |  |
|  |  | Subset-026-3.12.2.3 |  |
|  |  | Subset-026-3.12.2.3 b) |  |
|  |  | Subset-026-3.12.2.4 |  |
|  |  | Subset-026-3.12.2.8 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on DMI that: • Information about unsuitability is DISPLAYED. Check on JRU that: • The train is not allowed to enter the route because the profile is not compatible with the train data (D_TRAIN< D_SUITABILITY) |
| Constraints | Before execution of test case the following preconditions have to be satisfied: - The RECEIVED power supply profile are incompatible to the train data. - The driver does not choose a location to stop the train. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Position Report parameters | STORED | V_TRAIN≠0 (Train running) |
| Route Suitability Data | STORED | Power supply Profile D_SUITABILITY=FINITE VALUE Q_SUITABILITY=10 (Traction system) M_VOLTAGE=FINITE VALUE (Traction System voltage) NID_CTRACTION=FINITE VALUE if M_VOLTAGE ≠ 0: Country identifier of the traction system |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The system status message "Route unsuitable – traction system" is displayed | O | DMI | The system status message "Route unsuitable – traction system" is displayed . This information shall be DISPLAYED when RECEIVED. | N/A | N/A | - |
| 2 | N/A | N/A | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23) is recorded | O | JRU | Route unsuitable – traction system | N/A | N/A | - |
| 3 | N/A | N/A | Train is moving (V_TRAIN=40) | I | INT | The train overpasses the location | N/A | N/A | - |
| 4 | N/A | FS / OS / LS | The Mode symbol "Trip" is displayed | O | DMI | - | N/A | TR | - |
| 5 | N/A | N/A | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=7) is recorded | O | JRU | Mode change is recorded | N/A | N/A | - |
| 6 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit19=1>) is recorded | O | JRU | MASK=0000000000000000040000 | N/A | N/A | - |
| 7 | N/A | N/A | Emergency brake commanded | O | TIU | - | N/A | N/A | - |
| 8 | N/A | N/A | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | The system status message "Emergency stop" is displayed | O | DMI | - | N/A | N/A | - |
| 10 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=1>) is recorded | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 | TR |
| Position Report parameters | STORED | V_TRAIN=0 (Train at standstill) |
| Route Suitability Data | STORED | Q_SUITABILITY=10 (Traction system) |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Emergency brake commanded | O | TIU | - |
| The system status message "Route unsuitable – traction system" is displayed The Mode symbol "Trip" is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.12.2: Route Suitability |  | 3120200 |
| Test Case of Feature | Supervision of power supply profile. |  | 3 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS L2: FS, LS, OS L3: FS, LS, OS |  |  |
| Target of Test | The target of this test is to check that: - The train supervises train movements against an axle load profile - The information of unsuitability is displayed - The corresponding location is supervised like EOA and SvL with no release speed - The on-board shall trip, when the location is over passed. - The function manage Route Suitability is active |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.5.2.1 table - Manage Route Suitability |  |
|  |  | Subset-026-4.7.2 table - Output information Route unsuitability(ies) |  |
|  |  | Subset-026-3.12.2.3 |  |
|  |  | Subset-026-3.12.2.3 c) |  |
|  |  | Subset-026-3.12.2.4 |  |
|  |  | Subset-026-3.12.2.8 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on DMI that: • Information about unsuitability is DISPLAYED. Check on JRU that: • The train is not allowed to enter the route because the profile is not compatible with the train data (D_TRAIN< D_SUITABILITY) |
| Constraints | Before execution of test case the following preconditions have to be satisfied : The RECEIVED axle load profile are incompatible to the train data. The driver does not choose a location to stop the train. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Position Report parameters | STORED | V_TRAIN≠0 (Train running) |
| Route Suitability Data | STORED | Power supply Profile D_SUITABILITY=FINITE VALUE Q_SUITABILITY=01 (Max axle load) M_AXLELOADCAT=FINITE VALUE |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The system status message "Route unsuitable – axle load category" is displayed | O | DMI | The system status message "Route unsuitable – axle load category" is displayed . This information shall be DISPLAYED when RECEIVED. | N/A | N/A | - |
| 2 | N/A | N/A | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23) is recorded | O | JRU | Route unsuitable – axle load category | N/A | N/A | - |
| 3 | N/A | N/A | Train is moving (V_TRAIN=40) | I | INT | The train overpasses the location | N/A | N/A | - |
| 4 | N/A | FS / OS / LS | The Mode symbol "Trip" is displayed | O | DMI | - | N/A | TR | - |
| 5 | N/A | N/A | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=7) is recorded | O | JRU | Mode change is recorded | N/A | N/A | - |
| 6 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit19=1>) is recorded | O | JRU | MASK=0000000000000000040000 | N/A | N/A | - |
| 7 | N/A | N/A | Emergency brake commanded | O | TIU | - | N/A | N/A | - |
| 8 | N/A | N/A | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | The system status message "Emergency stop" is displayed | O | DMI | - | N/A | N/A | - |
| 10 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=1>) is recorded | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 | TR |
| Position Report parameters | STORED | V_TRAIN=0 (Train at standstill) |
| Route Suitability Data | STORED | Q_SUITABILITY=01 (Max axle load) |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Emergency brake commanded | O | TIU | - |
| The Mode symbol "Trip" is displayed | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |