| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 5080200 (5.8.2: Selection of “Override”) Total: 6 Test case(s) |  |
| REF: | Subset-076-5-2-5080200 |
| ISSUE: | Version 3.2.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 0.0.1 23.03.2012 | All | First version/review | DLR |
| Version 2.9.0 31.03.2013 | All | First version/review for Baseline 3 | MULTITEL |
| Version 2.9.1 05.07.2013 | All | Template standardised with a dedicated tool, TCs renumbered | MULTITEL |
| Version 2.9.2 25.10.2013 | TC1, TC2, TC3 | Modification requested by issue 28 of Review002_Subset-076-5-2-5080200 | MULTITEL |
| Version 2.9.3 25.10.2013 | All | Modifications requested by Review002_Subset-076-5-2-5080200_v291 | MULTITEL |
| Version 2.9.4 09.01.2014 | TC1-3,5 | Modifications requested by Review001_Subset-076-5-2-5080200_v293 | MULTITEL |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.2.0 31.03.2017 | All | Baseline 3 1st maintenance release version - Bug fixing | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.8.2: Selection of “Override” |  | 5080200 |
| Test Case of Feature | The ERTMS/ETCS on-board equipment allows the driver to select “Override”. The speed limit for triggering the “override” function is a national value and should be replaced when a new set of NVs is available. |  | 1 |
| Applicable Mode/Level Combinations | L0: UN, SH, SB L1: FS, LS, OS, SR, SH, SL, TR, PT, NL, RV L2: FS, LS, OS, SR, SH, SL, SB, TR, PT, NL, RV L3: FS, LS, OS, SR, SH, SL, SB, TR, PT, NL, RV LNTC: SH, SN |  |  |
| Target of Test | The target of this test is to check that: Trains shall be supervised according to the National Values of the current infrastructure if they are available on-board. The train will receive a newer set of National Values that shall be applicable at a defined location. Evaluating a balise group message, balise identity information shall be used to ensure that correct NV are used. (Compare NID_C) If the newer set of National Values is valid, i.e, if NID_C of received NV is equal to country or region identifier from the LRBG the train will apply it at the defined location. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.18.2.2 |  |
|  |  | Subset-026-3.18.2.3 |  |
|  |  | Subset-026-3.18.2.4 |  |
|  |  | Subset-026-4.8.3.1.1 National Values |  |
|  |  | Subset-026-4.8.4.2 National Values |  |
|  |  | Subset-026-5.8.2.1 |  |
|  |  | Subset-026-5.8.2.1 a) |  |
|  |  | Subset-026-5.8.2.1 b) |  |
|  |  | Subset-026-5.8.2.1 c) |  |
|  |  | Subset-026-7.4.2.1.1 Packet Number 3: National Values |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The “override request” is AVAILABLE or NOT AVAILABLE following the value of V_Train and V_NVALLOWOVTRP (Override request is not available in NL, SL, RV and TR) Check on the JRU that : Packet 3 National Values, Including V_NVALLOWOVTRP, D_VALIDNV & NID_C is RECORDED |
| Constraints | If the train is in SB mode, a cab is active. Packet 3 will be sent to the on-board with V_NVALLOWOVTRP different from its default value (0km/h) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/1/2/3/4/5/6/7/8/11/12/13/14 | FS/OS/SR/SH/UN/SL/SB/TR/PT/NL/LS/SN/RV |
| Radio communication session | ESTABLISHED | Only necessary if Packet 3 : National Values is received via radio |
| Train Data | STORED | Validated train data is STORED. |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Only necessary if Packet 3 : National Values is received via radio. |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The train runs with speed higher than V_NVALLOWOVTRP (included in the default value) (e.g. V_Train = 30km/h). The “override request” is NOT AVAILABLE. | O | DMI | - | N/A | N/A | - |
| 2 | N/A | N/A | Packet 3: National Values, Including V_NVALLOWOVTRP, D_VALIDNV & NID_C is RECEIVED. | I | BTM | Use the steps 2/3 or 4/5 for testing” Evaluating a balise group message, balise identity information shall be used to ensure that correct NV are used. (Compare NID_C) Balise group is taken as a new LRBG. New V_NVALLOWOVTRP > default value (0 km/h) | N/A | N/A | - |
| 3 | N/A | N/A | Packet 3: National Values, Including V_NVALLOWOVTRP, D_VALIDNV & NID_C is RECORDED | O | JRU | Use the steps 2/3 or 4/5 for testing” | N/A | N/A | - |
| 4 | N/A | N/A | Packet 3: National Values, Including V_NVALLOWOVTRP, D_VALIDNV & NID_C is RECEIVED. | I | RTM | Use the steps 2/3 or 4/5 for testing” Evaluating a radio message, related balise identity information shall be used to ensure that correct NV are used. (Compare NID_C) New V_NVALLOWOVTRP > default value (0 km/h) | N/A | N/A | - |
| 5 | N/A | N/A | Packet 3: National Values, Including V_NVALLOWOVTRP, D_VALIDNV & NID_C is RECORDED | O | JRU | Use the steps 2/3 or 4/5 for testing” | N/A | N/A | - |
| 6 | N/A | N/A | The override button is activated | I | DMI | if the menu with override button is not activated, the driver selects the right menu in order to see the override button | N/A | N/A | - |
| 7 | N/A | N/A | The train does not reach the location indicated in D_VALIDNV, D_Train < D_VALIDNV The “override request” is NOT AVAILABLE | O | DMI | As long as the train has not reached the location indicated in D_VALIDNV, the national values shall still not be applicable After this one steps 8 or 9 will be chosen for testing depending on design conditions | N/A | N/A | - |
| 8 | N/A | N/A | The train reaches the distance indicated in D_VALIDNV, D_Train > D_VALIDNV; And the speed of the train V_Train is less or equal to V_NVALLOWOVTRP, V_Train < V_NVALLOWOVTRP The “override request” is AVAILABLE. | O | DMI | Only If NID_C of received NV is equal to country or region identifier of the LRBG, National values shall become applicable. Override request is not available in NL, SL, RV and TR | N/A | N/A | - |
| 9 | N/A | N/A | The train reaches the distance indicated in D_VALIDNV, D_Train > D_VALIDNV; And the speed of the train V_Train is greater than V_NVALLOWOVTRP, V_Train > V_NVALLOWOVTRP The “override request” is NOT AVAILABLE | O | DMI | Only If NID_C of received NV is different from country or region identifier of the LRBG. National Values shall still not be applicable. The set of NV keeps stored | N/A | N/A | - |
| 10 | N/A | N/A | Onboard receives a balise group message. | I | BTM | Only if coming from step 9 Balise group is taken as a new LRBG. NID_C identifier of received BG is equal to the area (NID_C) in which stored NV are applicable. | N/A | N/A | - |
| 11 | N/A | N/A | The “override request” is AVAILABLE. | O | DMI | Only if coming from step 10 National values are now applicable. Override request is not available in NL, SL, RV and TR | N/A | N/A | - |

| Step 2: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | FINITE VALUE | Type of media |
| N_PIG | 3 | FINITE VALUE | - |
| N_TOTAL | 3 | FINITE VALUE | 1 <= N <= 7 i.e. from 2 to 8 balises in the group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Identical Value for all balises of the group |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | ID of Balise Group |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 3 | Packet 3 |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_VALIDNV | 15 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | Identification of national areas to which the set applies. |
| N_ITER | 5 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | Identification of additional national area(s) to which the set applies. |
| V_NVSHUNT | 7 | FINITE VALUE | - |
| V_NVSTFF | 7 | FINITE VALUE | - |
| V_NVONSIGHT | 7 | FINITE VALUE | - |
| V_NVLIMSUPERV | 7 | FINITE VALUE | - |
| V_NVUNFIT | 7 | FINITE VALUE | - |
| V_NVREL | 7 | FINITE VALUE | - |
| D_NVROLL | 15 | FINITE VALUE | - |
| Q_NVSBTSMPERM | 1 | FINITE VALUE | - |
| Q_NVEMRRLS | 1 | FINITE VALUE | - |
| Q_NVGUIPERM | 1 | FINITE VALUE | - |
| Q_NVSBFBPERM | 1 | FINITE VALUE | - |
| Q_NVINHSMICPERM | 1 | FINITE VALUE | - |
| V_NVALLOWOVTRP | 7 | FINITE VALUE | - |
| V_NVSUPOVTRP | 7 | FINITE VALUE | - |
| D_NVOVTRP | 15 | FINITE VALUE | - |
| T_NVOVTRP | 8 | FINITE VALUE | - |
| D_NVPOTRP | 15 | FINITE VALUE | - |
| M_NVCONTACT | 2 | FINITE VALUE | - |
| T_NVCONTACT | 8 | FINITE VALUE | - |
| M_NVDERUN | 1 | FINITE VALUE | - |
| D_NVSTFF | 15 | FINITE VALUE | - |
| Q_NVDRIVER_ADHES | 1 | FINITE VALUE | - |
| A_NVMAXREDADH1 | 6 | FINITE VALUE | - |
| A_NVMAXREDADH2 | 6 | FINITE VALUE | - |
| A_NVMAXREDADH3 | 6 | FINITE VALUE | - |
| Q_NVLOCACC | 6 | FINITE VALUE | - |
| M_NVAVADH | 5 | FINITE VALUE | - |
| M_NVEBCL | 4 | FINITE VALUE | - |
| Q_NVKINT | 1 | FINITE VALUE | - |
| Q_NVKVINTSET | 2 | FINITE VALUE | Only if Q_NVKINT = 1, Q_NVKVINTSET and the following variables follow. |
| A_NVP12 | 6 | FINITE VALUE | Only if Q_NVKVINTSET = 1 |
| A_NVP23 | 6 | FINITE VALUE | Only if Q_NVKVINTSET = 1 |
| V_NVKVINT(n) | 7 | FINITE VALUE | 0 km/h |
| M_NVKVINT(n) | 7 | FINITE VALUE | Valid between V_NVKVINT and V_NVKVINT(1. If Q_NVKVINTSET = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12. |
| M_NVKVINT(n) | 7 | FINITE VALUE | Only if Q_NVKVINTSET = 1. Valid between V_NVKVINT and V_NVKVINT(1). Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23. |
| N_ITER | 5 | FINITE VALUE | - |
| V_NVKVINT | 7 | FINITE VALUE | - |
| M_NVKVINT | 7 | FINITE VALUE | Valid between V_NVKVINT(n) and V_NVKVINT(n+1). If Q_NVKVINTSET = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP1. |
| M_NVKVINT(n) | 7 | FINITE VALUE | Only if Q_NVKVINTSET = 1.Valid between V_NVKVINT(n) and V_NVKVINT(n+1) Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP2. |
| N_ITER | 5 | FINITE VALUE | - |
| Q_NVKVINTSET(k) | 2 | FINITE VALUE | - |
| A_NVP12(k) | 6 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1 |
| A_NVP23(k) | 6 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1 |
| V_NVKVINT(k) | 7 | FINITE VALUE | 0 |
| M_NVKVINT(k) | 7 | FINITE VALUE | Valid between V_NVKVINT(k) and V_NVKVINT(k,1). If Q_NVKVINTSET(k) = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12(k. |
| M_NVKVINT(k) | 7 | FINITE VALUE | Only if Q_NVKVINTSET(k) = . Valid between V_NVKVINT(k) and V_NVKVINT(k,1) Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23(k). |
| N_ITER(k) | 5 | FINITE VALUE | - |
| V_NVKVINT(k,m) | 7 | FINITE VALUE | - |
| M_NVKVINT(k,m) | 7 | FINITE VALUE | Valid between V_NVKVINT(k,m) and V_NVKVINT(k,m+1). If Q_NVKVINTSET(k) = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12(k). |
| M_NVKVINT(k,m) | 7 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1.Valid between V_NVKVINT(k,m) and V_NVKVINT(k,m+1). Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23(k). |
| L_NVKRINT | 5 | FINITE VALUE | 0 m |
| M_NVKRINT | 5 | FINITE VALUE | Valid between L_NVKRINT and L_NVKRINT(1). |
| N_ITER | 5 | FINITE VALUE | - |
| L_NVKRINT(l) | 5 | FINITE VALUE | - |
| M_NVKRINT(l) | 5 | FINITE VALUE | Valid between L_NVKRINT(l) and L_NVKRINT(l+1). |
| M_NVKTINT | 5 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 4: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General message |
| L_MESSAGE | 10 | FINITE VALUE | Must correspond to the computed record size(on-board) |
| T_TRAIN | 32 | FINITE VALUE | On-board train time |
| M_ACK | 1 | FINITE VALUE | Acknowledge of message required = 1, Not required =0 |
| T_TRAIN | 1 | 1 | On-board train time |
| NID_LRBG | 10+14 | FINITE VALUE | ID number of LRBG |
| NID_PACKET | 8 | 3 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_VALIDNV | 15 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | Identification of national areas to which the set applies. |
| N_ITER | 5 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | Identification of additional national area(s) to which the set applies. |
| V_NVSHUNT | 7 | FINITE VALUE | - |
| V_NVSTFF | 7 | FINITE VALUE | - |
| V_NVONSIGHT | 7 | FINITE VALUE | - |
| V_NVLIMSUPERV | 7 | FINITE VALUE | - |
| V_NVUNFIT | 7 | FINITE VALUE | - |
| V_NVREL | 7 | FINITE VALUE | - |
| D_NVROLL | 15 | FINITE VALUE | - |
| Q_NVSBTSMPERM | 1 | FINITE VALUE | - |
| Q_NVEMRRLS | 1 | FINITE VALUE | - |
| Q_NVGUIPERM | 1 | FINITE VALUE | - |
| Q_NVSBFBPERM | 1 | FINITE VALUE | - |
| Q_NVINHSMICPERM | 1 | FINITE VALUE | - |
| V_NVALLOWOVTRP | 7 | FINITE VALUE | - |
| V_NVSUPOVTRP | 7 | FINITE VALUE | - |
| D_NVOVTRP | 15 | FINITE VALUE | - |
| T_NVOVTRP | 8 | FINITE VALUE | - |
| D_NVPOTRP | 15 | FINITE VALUE | - |
| M_NVCONTACT | 2 | FINITE VALUE | - |
| T_NVCONTACT | 8 | FINITE VALUE | - |
| M_NVDERUN | 1 | FINITE VALUE | - |
| D_NVSTFF | 15 | FINITE VALUE | - |
| Q_NVDRIVER_ADHES | 1 | FINITE VALUE | - |
| A_NVMAXREDADH1 | 6 | FINITE VALUE | - |
| A_NVMAXREDADH2 | 6 | FINITE VALUE | - |
| A_NVMAXREDADH3 | 6 | FINITE VALUE | - |
| Q_NVLOCACC | 6 | FINITE VALUE | - |
| M_NVAVADH | 5 | FINITE VALUE | - |
| M_NVEBCL | 4 | FINITE VALUE | - |
| Q_NVKINT | 1 | FINITE VALUE | - |
| Q_NVKVINTSET | 2 | FINITE VALUE | Only if Q_NVKINT = 1, Q_NVKVINTSET and the following variables follow. |
| A_NVP12 | 6 | FINITE VALUE | Only if Q_NVKVINTSET = 1 |
| A_NVP23 | 6 | FINITE VALUE | Only if Q_NVKVINTSET = 1 |
| V_NVKVINT(n) | 7 | FINITE VALUE | 0 km/h |
| M_NVKVINT(n) | 7 | FINITE VALUE | Valid between V_NVKVINT and V_NVKVINT(1. If Q_NVKVINTSET = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12. |
| M_NVKVINT(n) | 7 | FINITE VALUE | Only if Q_NVKVINTSET = 1. Valid between V_NVKVINT and V_NVKVINT(1). Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23. |
| N_ITER | 5 | FINITE VALUE | - |
| V_NVKVINT | 7 | FINITE VALUE | - |
| M_NVKVINT | 7 | FINITE VALUE | Valid between V_NVKVINT(n) and V_NVKVINT(n+1). If Q_NVKVINTSET = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP1. |
| M_NVKVINT(n) | 7 | FINITE VALUE | Only if Q_NVKVINTSET = 1.Valid between V_NVKVINT(n) and V_NVKVINT(n+1) Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP2. |
| N_ITER | 5 | FINITE VALUE | - |
| Q_NVKVINTSET(k) | 2 | FINITE VALUE | - |
| A_NVP12(k) | 6 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1 |
| A_NVP23(k) | 6 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1 |
| V_NVKVINT(k) | 7 | FINITE VALUE | 0 |
| M_NVKVINT(k) | 7 | FINITE VALUE | Valid between V_NVKVINT(k) and V_NVKVINT(k,1). If Q_NVKVINTSET(k) = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12(k. |
| M_NVKVINT(k) | 7 | FINITE VALUE | Only if Q_NVKVINTSET(k) = . Valid between V_NVKVINT(k) and V_NVKVINT(k,1) Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23(k). |
| N_ITER(k) | 5 | FINITE VALUE | - |
| V_NVKVINT(k,m) | 7 | FINITE VALUE | - |
| M_NVKVINT(k,m) | 7 | FINITE VALUE | Valid between V_NVKVINT(k,m) and V_NVKVINT(k,m+1). If Q_NVKVINTSET(k) = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12(k). |
| M_NVKVINT(k,m) | 7 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1.Valid between V_NVKVINT(k,m) and V_NVKVINT(k,m+1). Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23(k). |
| L_NVKRINT | 5 | FINITE VALUE | 0 m |
| M_NVKRINT | 5 | FINITE VALUE | Valid between L_NVKRINT and L_NVKRINT(1). |
| N_ITER | 5 | FINITE VALUE | - |
| L_NVKRINT(l) | 5 | FINITE VALUE | - |
| M_NVKRINT(l) | 5 | FINITE VALUE | Valid between L_NVKRINT(l) and L_NVKRINT(l+1). |
| M_NVKTINT | 5 | FINITE VALUE | - |

| Step 10: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Type of media |
| N_PIG | 3 | FINITE VALUE | - |
| N_TOTAL | 3 | FINITE VALUE | 1 <= N <= 7 i.e. from 2 to 8 balises in the group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Identical Value for all balises of the group |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | ID of Balise Group |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/1/2/3/4/5/6/7/8/11/12/13/14 UNCHANGED | FS/OS/SR/SH/UN/SL/SB/TR/PT/NL/LS/SN/RV |
| Radio communication session | ESTABLISHED | Only necessary if Packet 3 : National Values is received via radio |
| National Values | STORED | Packet 3 including V_NVALLOWOVTRP, D_VALIDNV and a correct value of NID_C. |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 UNCHANGED | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Only necessary if Packet 3 : National Values is received via radio. |
| NOT RELEVANT | - | TIU | - |
| The "EOA" button in the "Override" window is enabled. | O | DMI | Except in NL, SL, RV and TR. |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.8.2: Selection of “Override” |  | 5080200 |
| Test Case of Feature | The ERTMS/ETCS on-board equipment allows the driver to select “Override”. The speed limit for triggering the “override” function is a national value and Default Values shall be used as fall back values |  | 2 |
| Applicable Mode/Level Combinations | L0: UN, SH, SB L1: FS, LS, OS, SR, SH, PT L2: FS, LS, OS, SR, SH, SB, PT L3: FS, LS, OS, SR, SH, SB, PT LNTC: SH, SN |  |  |
| Target of Test | The target of this test is to check that: Trains shall be supervised according to the National Values of the current infrastructure if they are available on-board. Evaluating a balise group message, balise identity information shall be used to ensure that correct NV are used. (Compare NID_C) Default Values shall be used as fall back values if a mismatch has been detected between the country or region identifier read from a balise group and the corresponding identifier(s) for which a set of national values is used onboard. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.18.2.2 |  |
|  |  | Subset-026-3.18.2.3 |  |
|  |  | Subset-026-3.18.2.4 |  |
|  |  | Subset-026-3.18.2.5 b) |  |
|  |  | Subset-026-4.8.3.1.1 National Values |  |
|  |  | Subset-026-4.8.4.2 National Values |  |
|  |  | Subset-026-5.8.2.1 |  |
|  |  | Subset-026-5.8.2.1 a) |  |
|  |  | Subset-026-5.8.2.1 b) |  |
|  |  | Subset-026-5.8.2.1 c) |  |
|  |  | Subset-026-7.4.2.1.1 Packet Number 3: National Values |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The “override request” is AVAILABLE following the value of V_Train and V_NVALLOWOVTRP. Check on the JRU that : Packet 3: National Values, Including V_NVALLOWOVTRP, D_VALIDNV & incorrect value of NID_C is RECORDED |
| Constraints | If the train is in SB mode, then a cab is active. Packet 3 will be sent to the onboard with V_NVALLOWOVTRP different from its default value (0km/h) Packet 3 will be received onboard with NID_C invalid |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/1/2/3/4/6/8/12/13 | FS/OS/SR/SH/UN/SB/PT/LS/SN |
| Radio communication session | ESTABLISHED | Only necessary if Packet 3 : National Values is received via radio |
| Train Data | STORED | Validated train data is STORED. |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Only necessary if Packet 3 : National Values is received via radio. |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The train runs with speed lower than V_NVALLOWOVTRP (included in the previous set of NV) (e.g. V_Train = 30km/h). The “override request” is AVAILABLE. | O | DMI | National values are applicable and used by the train. Override request is not available in NL, SL, RV and TR | N/A | N/A | - |
| 2 | N/A | N/A | Onboard receives a balise group message. | I | BTM | Balise group is taken as a new LRBG. There is a mismatch between country or region identifier NID_C of the Balise Group and the corresponding identifier for which a set of national values is used on-board | N/A | N/A | - |
| 3 | N/A | N/A | Balise group message is RECORDED | O | JRU | - | N/A | N/A | - |
| 4 | N/A | N/A | The override button is activated | I | DMI | if the menu with override button is not activated, the driver selects the right menu in order to see the override button | N/A | N/A | - |
| 5 | N/A | N/A | The “override request” is NOT AVAILABLE. | O | DMI | Default Values shall be used as fall back values (e.g. V_Train = 30km/h). | N/A | N/A | - |
| 6 | N/A | N/A | The train decelerates to standstill and when it has reached standstill the “override request” is AVAILABLE again. | O | DMI | - | N/A | N/A | - |

| Step 2: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Type of media |
| N_PIG | 3 | FINITE VALUE | - |
| N_TOTAL | 3 | FINITE VALUE | 1 <= N <= 7 i.e. from 2 to 8 balises in the group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Identical Value for all balises of the group |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | ID of Balise Group |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/1/2/3/4/6/8/12/13 | FS/OS/SR/SH/UN/SB/PT/LS/SN |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The "EOA" button in the "Override" window is enabled. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.8.2: Selection of “Override” |  | 5080200 |
| Test Case of Feature | The ERTMS/ETCS on-board equipment allows the driver to select “Override”. The speed limit for triggering the “override” function is a national value and should be replaced when a new set of NVs is available. A previously received set of National Values not yet applicable should be deleted. |  | 3 |
| Applicable Mode/Level Combinations | L0: UN, SH, SB L1: FS, LS, OS, SR, SH, SL, TR, PT, NL, RV L2: FS, LS, OS, SR, SH, SL, SB, TR, PT, NL, RV L3: FS, LS, OS, SR, SH, SL, SB, TR, PT, NL, RV LNTC: SH, SN |  |  |
| Target of Test | The target of this test is to check that: If a new set of National Values is received, a previously received set which is not yet valid shall be deleted. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.18.2.2 |  |
|  |  | Subset-026-3.18.2.3 |  |
|  |  | Subset-026-3.18.2.4 |  |
|  |  | Subset-026-3.18.2.9 |  |
|  |  | Subset-026-3.18.2.9 a) |  |
|  |  | Subset-026-3.18.2.10 |  |
|  |  | Subset-026-4.8.3.1.1 National Values |  |
|  |  | Subset-026-4.8.4.2 National Values |  |
|  |  | Subset-026-5.8.2.1 |  |
|  |  | Subset-026-5.8.2.1 a) |  |
|  |  | Subset-026-5.8.2.1 b) |  |
|  |  | Subset-026-5.8.2.1 c) |  |
|  |  | Subset-026-7.4.2.1.1 Packet Number 3: National Values |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The “override request” is AVAILABLE or NOT AVAILABLE following the value of V_Train and V_NVALLOWOVTRP (Override request is not available in NL, SL, RV and TR) Check on the JRU that : Two messages including Packet 3 National Values, Including V_NVALLOWOVTRP, D_VALIDNV , NID_C is RECORDED The on-board equipment has a current used set of NV (valid). During the Test Case: The on-board equipment receives a first message including packet 3 (National Values), which does not become valid The on-board equipment receives a second message including packet 3 (National Values), which replace the first and become valid The first set of NV is deleted which is checked when the train uses default Values after receiving a balise message including a NID_C = 000001. |
| Constraints | If the train is in SB mode, then a cab is active. Initial packet 3 (National Values) includes the variable V_NVALLOWOVTRP which is set to 30km/h - NID_C = (000001) First received packet 3 (National Values) includes the variable V_NVALLOWOVTRP which is set to 40km/h - NID_C = (000011) is different from initial balise. Second received packet 3 (National Values) includes the variable V_NVALLOWOVTRP which is set to 50km/h - NID_C = (000011) is different from initial balise. New Region or Country is announced - NID_C = 000011 |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/1/2/3/4/5/6/7/8/11/12/13/14 | FS/OS/SR/SH/UN/SL/SB/TR/PT/NL/LS/SN/RV |
| Radio communication session | ESTABLISHED | Only necessary if Packet 3 : National Values is received via radio |
| National Values | STORED | V_NVALLOWOVTRP = 30km/h for NID_C = 000001 b |
| Train Data | STORED | Validated train data is STORED. |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Only necessary if Packet 3 : National Values is received via radio. |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | First message including Packet 3: National Values, Including V_NVALLOWOVTRP (40 km/h) is RECEIVED. | I | BTM | NID_C (000011) is different from initial balise (NID_C = (000001)). The received V_NVALLOWOVTRP is different than the initial value of V_NVALLOWOVTRP (30 km/h). | N/A | N/A | - |
| 2 | N/A | N/A | First message including Packet 3: National Values, Including V_NVALLOWOVTRP (40 km/h) is RECORDED | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | Second message including Packet 3: National Values, Including V_NVALLOWOVTRP (50 km/h) and Linking Information is RECEIVED. | I | BTM | NID_C (000011) is different from initial balise / new Region or Country is announced. | N/A | N/A | - |
| 4 | N/A | N/A | Second message including Packet 3: National Values, Including V_NVALLOWOVTRP (50 km/h) is RECORDED | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | The train receives a balise message including a NID_C = 000011 | I | BTM | New Region or Country is announced. | N/A | N/A | - |
| 6 | N/A | N/A | The train receives a balise message including a NID_C = 000011 is RECORDED | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | The Train Speed is between 40 and 50 km/h | O | DMI | Second message including Packet 3: National Values including V_NVALLOWOVTRP (50 km/h) gets VALID. Initial set of NVs gets INVALID and is DELETED. | N/A | N/A | - |
| 8 | N/A | N/A | The override button is activated | I | DMI | if the menu with override button is not activated, the driver selects the right menu in order to see the override button | N/A | N/A | - |
| 9 | N/A | N/A | Override button IS ACTIVATED. | O | DMI | Override request is not available in NL, SL, RV and TR | N/A | N/A | - |
| 10 | N/A | N/A | The train speed > 50 km/h | O | DMI | - | N/A | N/A | - |
| 11 | N/A | N/A | The driver selects the right menu in order to see the override button, | I | DMI | (override request is not available in NL, SL and TR) | N/A | N/A | - |
| 12 | N/A | N/A | Override button is NOT ACTIVATED. | O | DMI | - | N/A | N/A | - |
| 13 | N/A | N/A | The train receives a balise message including a NID_C = 000001 | I | BTM | The Message does NOT contain National Values. Due to the Fact that the last uses NV of Region 000001 should be deleted as invalid, train uses default Values. | N/A | N/A | - |
| 14 | N/A | N/A | Use_FT9990200.41 : The train uses default national Values | - | - | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Type of media |
| N_PIG | 3 | FINITE VALUE | - |
| N_TOTAL | 3 | FINITE VALUE | 1 <= N <= 7 i.e. from 2 to 8 balises in the group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Identical Value for all balises of the group |
| NID_C | 10 | FINITE VALUE | 000011 |
| NID_BG | 14 | FINITE VALUE | ID of Balise Group |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 3 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_VALIDNV | 15 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | Identification of national areas to which the set applies. |
| N_ITER | 5 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | Identification of additional national area(s) to which the set applies. |
| V_NVSHUNT | 7 | FINITE VALUE | - |
| V_NVSTFF | 7 | FINITE VALUE | - |
| V_NVONSIGHT | 7 | FINITE VALUE | - |
| V_NVLIMSUPERV | 7 | FINITE VALUE | - |
| V_NVUNFIT | 7 | FINITE VALUE | - |
| V_NVREL | 7 | FINITE VALUE | - |
| D_NVROLL | 15 | FINITE VALUE | - |
| Q_NVSBTSMPERM | 1 | FINITE VALUE | - |
| Q_NVEMRRLS | 1 | FINITE VALUE | - |
| Q_NVGUIPERM | 1 | FINITE VALUE | - |
| Q_NVSBFBPERM | 1 | FINITE VALUE | - |
| Q_NVINHSMICPERM | 1 | FINITE VALUE | - |
| V_NVALLOWOVTRP | 7 | FINITE VALUE | 40 km/h |
| V_NVSUPOVTRP | 7 | FINITE VALUE | - |
| D_NVOVTRP | 15 | FINITE VALUE | - |
| T_NVOVTRP | 8 | FINITE VALUE | - |
| D_NVPOTRP | 15 | FINITE VALUE | - |
| M_NVCONTACT | 2 | FINITE VALUE | - |
| T_NVCONTACT | 8 | FINITE VALUE | - |
| M_NVDERUN | 1 | FINITE VALUE | - |
| D_NVSTFF | 15 | FINITE VALUE | - |
| Q_NVDRIVER_ADHES | 1 | FINITE VALUE | - |
| A_NVMAXREDADH1 | 6 | FINITE VALUE | - |
| A_NVMAXREDADH2 | 6 | FINITE VALUE | - |
| A_NVMAXREDADH3 | 6 | FINITE VALUE | - |
| Q_NVLOCACC | 6 | FINITE VALUE | - |
| M_NVAVADH | 5 | FINITE VALUE | - |
| M_NVEBCL | 4 | FINITE VALUE | - |
| Q_NVKINT | 1 | FINITE VALUE | - |
| Q_NVKVINTSET | 2 | FINITE VALUE | Only if Q_NVKINT = 1, Q_NVKVINTSET and the following variables follow. |
| A_NVP12 | 6 | FINITE VALUE | Only if Q_NVKVINTSET = 1 |
| A_NVP23 | 6 | FINITE VALUE | Only if Q_NVKVINTSET = 1 |
| V_NVKVINT(n) | 7 | FINITE VALUE | 0 km/h |
| M_NVKVINT(n) | 7 | FINITE VALUE | Valid between V_NVKVINT and V_NVKVINT(1. If Q_NVKVINTSET = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12. |
| M_NVKVINT(n) | 7 | FINITE VALUE | Only if Q_NVKVINTSET = 1. Valid between V_NVKVINT and V_NVKVINT(1). Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23. |
| N_ITER | 5 | FINITE VALUE | - |
| V_NVKVINT | 7 | FINITE VALUE | - |
| M_NVKVINT | 7 | FINITE VALUE | Valid between V_NVKVINT(n) and V_NVKVINT(n+1). If Q_NVKVINTSET = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP1. |
| M_NVKVINT(n) | 7 | FINITE VALUE | Only if Q_NVKVINTSET = 1.Valid between V_NVKVINT(n) and V_NVKVINT(n+1) Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP2. |
| N_ITER | 5 | FINITE VALUE | - |
| Q_NVKVINTSET(k) | 2 | FINITE VALUE | - |
| A_NVP12(k) | 6 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1 |
| A_NVP23(k) | 6 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1 |
| V_NVKVINT(k) | 7 | FINITE VALUE | 0 |
| M_NVKVINT(k) | 7 | FINITE VALUE | Valid between V_NVKVINT(k) and V_NVKVINT(k,1). If Q_NVKVINTSET(k) = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12(k. |
| M_NVKVINT(k) | 7 | FINITE VALUE | Only if Q_NVKVINTSET(k) = . Valid between V_NVKVINT(k) and V_NVKVINT(k,1) Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23(k). |
| N_ITER(k) | 5 | FINITE VALUE | - |
| V_NVKVINT(k,m) | 7 | FINITE VALUE | - |
| M_NVKVINT(k,m) | 7 | FINITE VALUE | Valid between V_NVKVINT(k,m) and V_NVKVINT(k,m+1). If Q_NVKVINTSET(k) = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12(k). |
| M_NVKVINT(k,m) | 7 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1.Valid between V_NVKVINT(k,m) and V_NVKVINT(k,m+1). Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23(k). |
| L_NVKRINT | 5 | FINITE VALUE | 0 m |
| M_NVKRINT | 5 | FINITE VALUE | Valid between L_NVKRINT and L_NVKRINT(1). |
| N_ITER | 5 | FINITE VALUE | - |
| L_NVKRINT(l) | 5 | FINITE VALUE | - |
| M_NVKRINT(l) | 5 | FINITE VALUE | Valid between L_NVKRINT(l) and L_NVKRINT(l+1). |
| M_NVKTINT | 5 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 3: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Type of media |
| N_PIG | 3 | FINITE VALUE | - |
| N_TOTAL | 3 | FINITE VALUE | 1 <= N <= 7 i.e. from 2 to 8 balises in the group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Identical Value for all balises of the group |
| NID_C | 10 | FINITE VALUE | 000011 |
| NID_BG | 14 | FINITE VALUE | ID of Balise Group |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 3 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_VALIDNV | 15 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | Identification of national areas to which the set applies. |
| N_ITER | 5 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | Identification of additional national area(s) to which the set applies. |
| V_NVSHUNT | 7 | FINITE VALUE | - |
| V_NVSTFF | 7 | FINITE VALUE | - |
| V_NVONSIGHT | 7 | FINITE VALUE | - |
| V_NVLIMSUPERV | 7 | FINITE VALUE | - |
| V_NVUNFIT | 7 | FINITE VALUE | - |
| V_NVREL | 7 | FINITE VALUE | - |
| D_NVROLL | 15 | FINITE VALUE | - |
| Q_NVSBTSMPERM | 1 | FINITE VALUE | - |
| Q_NVEMRRLS | 1 | FINITE VALUE | - |
| Q_NVGUIPERM | 1 | FINITE VALUE | - |
| Q_NVSBFBPERM | 1 | FINITE VALUE | - |
| Q_NVINHSMICPERM | 1 | FINITE VALUE | - |
| V_NVALLOWOVTRP | 7 | FINITE VALUE | 50 km/h |
| V_NVSUPOVTRP | 7 | FINITE VALUE | - |
| D_NVOVTRP | 15 | FINITE VALUE | - |
| T_NVOVTRP | 8 | FINITE VALUE | - |
| D_NVPOTRP | 15 | FINITE VALUE | - |
| M_NVCONTACT | 2 | FINITE VALUE | - |
| T_NVCONTACT | 8 | FINITE VALUE | - |
| M_NVDERUN | 1 | FINITE VALUE | - |
| D_NVSTFF | 15 | FINITE VALUE | - |
| Q_NVDRIVER_ADHES | 1 | FINITE VALUE | - |
| A_NVMAXREDADH1 | 6 | FINITE VALUE | - |
| A_NVMAXREDADH2 | 6 | FINITE VALUE | - |
| A_NVMAXREDADH3 | 6 | FINITE VALUE | - |
| Q_NVLOCACC | 6 | FINITE VALUE | - |
| M_NVAVADH | 5 | FINITE VALUE | - |
| M_NVEBCL | 4 | FINITE VALUE | - |
| Q_NVKINT | 1 | FINITE VALUE | - |
| Q_NVKVINTSET | 2 | FINITE VALUE | Only if Q_NVKINT = 1, Q_NVKVINTSET and the following variables follow. |
| A_NVP12 | 6 | FINITE VALUE | Only if Q_NVKVINTSET = 1 |
| A_NVP23 | 6 | FINITE VALUE | Only if Q_NVKVINTSET = 1 |
| V_NVKVINT(n) | 7 | FINITE VALUE | 0 km/h |
| M_NVKVINT(n) | 7 | FINITE VALUE | Valid between V_NVKVINT and V_NVKVINT(1. If Q_NVKVINTSET = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12. |
| M_NVKVINT(n) | 7 | FINITE VALUE | Only if Q_NVKVINTSET = 1. Valid between V_NVKVINT and V_NVKVINT(1). Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23. |
| N_ITER | 5 | FINITE VALUE | - |
| V_NVKVINT | 7 | FINITE VALUE | - |
| M_NVKVINT | 7 | FINITE VALUE | Valid between V_NVKVINT(n) and V_NVKVINT(n+1). If Q_NVKVINTSET = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP1. |
| M_NVKVINT(n) | 7 | FINITE VALUE | Only if Q_NVKVINTSET = 1.Valid between V_NVKVINT(n) and V_NVKVINT(n+1) Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP2. |
| N_ITER | 5 | FINITE VALUE | - |
| Q_NVKVINTSET(k) | 2 | FINITE VALUE | - |
| A_NVP12(k) | 6 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1 |
| A_NVP23(k) | 6 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1 |
| V_NVKVINT(k) | 7 | FINITE VALUE | 0 |
| M_NVKVINT(k) | 7 | FINITE VALUE | Valid between V_NVKVINT(k) and V_NVKVINT(k,1). If Q_NVKVINTSET(k) = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12(k. |
| M_NVKVINT(k) | 7 | FINITE VALUE | Only if Q_NVKVINTSET(k) = . Valid between V_NVKVINT(k) and V_NVKVINT(k,1) Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23(k). |
| N_ITER(k) | 5 | FINITE VALUE | - |
| V_NVKVINT(k,m) | 7 | FINITE VALUE | - |
| M_NVKVINT(k,m) | 7 | FINITE VALUE | Valid between V_NVKVINT(k,m) and V_NVKVINT(k,m+1). If Q_NVKVINTSET(k) = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12(k). |
| M_NVKVINT(k,m) | 7 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1.Valid between V_NVKVINT(k,m) and V_NVKVINT(k,m+1). Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23(k). |
| L_NVKRINT | 5 | FINITE VALUE | 0 m |
| M_NVKRINT | 5 | FINITE VALUE | Valid between L_NVKRINT and L_NVKRINT(1). |
| N_ITER | 5 | FINITE VALUE | - |
| L_NVKRINT(l) | 5 | FINITE VALUE | - |
| M_NVKRINT(l) | 5 | FINITE VALUE | Valid between L_NVKRINT(l) and L_NVKRINT(l+1). |
| M_NVKTINT | 5 | FINITE VALUE | - |
| NID_PACKET | 8 | 5 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LINK | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | 1 | - |
| NID_C | 10 | 000011 | - |
| NID_BG | 14 | FINITE VALUE | ID of Balise Group (same as Step 5) |
| Q_LINKORIENTATION | 1 | FINITE VALUE | - |
| Q_LINKREACTION | 2 | FINITE VALUE | - |
| Q_LOCACC | 6 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_LINK(k) | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | FINITE VALUE | - |
| Q_LOCACC(k) | 6 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 5: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Type of media |
| N_PIG | 3 | FINITE VALUE | - |
| N_TOTAL | 3 | FINITE VALUE | 1 <= N <= 7 i.e. from 2 to 8 balises in the group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Identical Value for all balises of the group |
| NID_C | 10 | FINITE VALUE | 000011 |
| NID_BG | 14 | FINITE VALUE | ID of Balise Group |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 5 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LINK | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | 1 | - |
| NID_C | 10 | 000001 | - |
| NID_BG | 14 | Finite Value | ID of Balise Group (same as Step 13) |
| Q_LINKORIENTATION | 1 | FINITE VALUE | - |
| Q_LINKREACTION | 2 | FINITE VALUE | - |
| Q_LOCACC | 6 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_LINK(k) | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | FINITE VALUE | - |
| Q_LOCACC(k) | 6 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 13: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Type of media |
| N_PIG | 3 | FINITE VALUE | - |
| N_TOTAL | 3 | FINITE VALUE | 1 <= N <= 7 i.e. from 2 to 8 balises in the group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Identical Value for all balises of the group |
| NID_C | 10 | FINITE VALUE | 000001 |
| NID_BG | 14 | FINITE VALUE | ID of Balise Group |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/1/2/3/4/5/6/7/8/11/12/13/14 UNCHANGED | FS/OS/SR/SH/UN/SL/SB/TR/PT/NL/LS/SN/RV |
| Radio communication session | ESTABLISHED | Only necessary if Packet 3 : National Values is received via radio |
| National Values | STORED | V_NVALLOWOVTRP = 50km/h for NID_C = 000011 b |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 UNCHANGED | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The "EOA" button in the "Override" window is enabled. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.8.2: Selection of “Override” |  | 5080200 |
| Test Case of Feature | On-board equipment allows the driver to select override request only when the speed is under or equal to V_NVALLOWOVTRP: national value received from trackside AND Validated Train Data is available |  | 4 |
| Applicable Mode/Level Combinations | L0: UN L1: FS / LS / OS / SR L2: FS / LS / OS / SR L3: FS / LS / OS / SR LNTC: SN |  |  |
| Target of Test | The target of the test is to check that the on-board equipment supervises: a speed limit to trigger the override EoA function override EoA function can be triggered only if train speed is not higher than speed limit defined by National value sent from trackside |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-5.8.2.1 |  |
|  |  | Subset-026-5.8.2.1 a) |  |
|  |  | Subset-026-5.8.2.1 b) |  |
|  |  | Subset-026-5.8.2.1 c) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: Validated Train Data is available The override request is DISPLAYED only when the train is running under (V_NVALLOWOVTRP : national value) The override request appears when speed comes under V_NVALLOWOVTRP and disappears when train overpasses V_NVALLOWOVTRP |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 12 / 13 | FS / OS / SR / UN / LS / SN |
| National Values | STORED | V_NVALLOWOVTRP sent from trackside and different from the default value (0 km/h). |
| Train Data | STORED | Validated train data is STORED |
| ERTMS/ETCS level | 0 / 1/ 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The speed and distance monitoring information is displayed. The "EOA" button in the "Override" window is disabled. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is moving (V_TRAIN > V_NVALLOWOVTRP). | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The driver presses "Data view" | I | DMI | - | N/A | N/A | - |
| 2 | N/A | N/A | The data from "Data View" window are displayed | O | DMI | Train Data and Train running number are displayed (thus their status is valid) | N/A | N/A | - |
| 3 | N/A | N/A | The driver presses "Override" | I | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | The <EOA> button in the <Override> window is enabled | O | DMI | - | N/A | N/A | - |
| 5 | N/A | N/A | The train runs with speed higher than V_NVALLOWOVTRP. Current speed is DISPLAYED. | O | DMI | V_TRAIN > V_NVALLOWOVTRP | N/A | N/A | - |
| 6 | N/A | N/A | The <EOA> button in the <Override> window is disabled | O | DMI | - | N/A | N/A | - |
| 7 | N/A | N/A | The train decreases speed - train speed < V_NVALLOWOVTRP. Current speed is DISPLAYED | O | DMI | V_TRAIN < V_NVALLOWOVTRP | N/A | N/A | - |
| 8 | N/A | N/A | The <EOA> button in the <Override> window is enabled | O | DMI | - | N/A | N/A | - |
| 9 | N/A | N/A | The train accelerates until speed is higher than V_NVALLOWOVTRP. Current speed is DISPLAYED. | O | DMI | V_TRAIN > V_NVALLOWOVTRP | N/A | N/A | - |
| 10 | N/A | N/A | The <EOA> button in the <Override> window is disabled | O | DMI | - | N/A | N/A | - |
| 11 | N/A | N/A | The train decreases speed train speed < V_NVALLOWOVTRP. Current speed is DISPLAYED | O | DMI | V_TRAIN < V_NVALLOWOVTRP | N/A | N/A | - |
| 12 | N/A | N/A | The <EOA> button in the <Override> window is enabled | O | DMI | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 12 / 13 | FS / OS / SR / UN / LS / SN |
| National Values | STORED | V_NVALLOWOVTRP sent from trackside and different from the default value (0 km/h). |
| Train Data | STORED | Validated train data is STORED. |
| ERTMS/ETCS level | 0 / 1/ 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The speed and distance monitoring information is displayed. The "EOA" button in the "Override" window is enabled. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is moving (V_TRAIN > V_NVALLOWOVTRP). | - | INT | - |

## Test case 5

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.8.2: Selection of “Override” |  | 5080200 |
| Test Case of Feature | The "Override request" is not available in SB for L1, NL, SL, RV and TR modes. |  | 5 |
| Applicable Mode/Level Combinations | L1: SL, SB, TR, NL, RV L2: SL, TR, NL, RV L3: SL, TR, NL, RV |  |  |
| Target of Test | The target of this test is to check that: Trains shall be supervised according to the National Values of the current infrastructure if they are available on-board. Evaluating a balise group message, balise identity information shall be used to ensure that correct NV are used. (Compare NID_C) Default Values shall be used as fall back values if a mismatch has been detected between the country or region identifier read from a balise group and the corresponding identifier(s) for which a set of national values is used onboard. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.18.2.2 |  |
|  |  | Subset-026-3.18.2.3 |  |
|  |  | Subset-026-3.18.2.4 |  |
|  |  | Subset-026-3.18.2.5 b) |  |
|  |  | Subset-026-4.8.3.1.1 National Values |  |
|  |  | Subset-026-4.8.4.2 National Values |  |
|  |  | Subset-026-5.8.2.1 |  |
|  |  | Subset-026-5.8.2.1 a) |  |
|  |  | Subset-026-5.8.2.1 b) |  |
|  |  | Subset-026-5.8.2.1 c) |  |
|  |  | Subset-026-7.4.2.1.1 Packet Number 3: National Values |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The “override request” is NOT AVAILABLE in NL, SL, RV and TR (no influence of the values of V_Train and V_NVALLOWOVTRP). Check on the JRU that : Packet 3: National Values, Including V_NVALLOWOVTRP, D_VALIDNV & incorrect value of NID_C is RECORDED |
| Constraints | If the train is in SB mode, then a cab is active. Packet 3 will be sent to the onboard with V_NVALLOWOVTRP different from its default value (0km/h) Packet 3 will be received onboard with NID_C invalid |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 5/6/7/11/13 | SL/SB/TR/NL/SN |
| Radio communication session | ESTABLISHED | Only necessary if Packet 3 : National Values is received via radio |
| Train Data | STORED | Validated train data is STORED. |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Only necessary if Packet 3 : National Values is received via radio. |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The train runs with speed lower than V_NVALLOWOVTRP (included in the previous set of NV) (e.g. V_Train = 30km/h). The “override request” is NOT AVAILABLE. | O | DMI | National values are applicable and used by the train. Override request is not available in SB for L1, NL, SL, RV and TR | N/A | N/A | - |
| 2 | N/A | N/A | Onboard receives a balise group message. | I | BTM | Balise group is taken as a new LRBG. There is a mismatch between country or region identifier NID_C of the Balise Group and the corresponding identifier for which a set of national values is used on-board | N/A | N/A | - |
| 3 | N/A | N/A | Balise group message is RECORDED | O | JRU | - | N/A | N/A | - |
| 4 | N/A | N/A | The train decelerates to standstill and when it has reached standstill the “override request” is still NOT AVAILABLE. | O | DMI | Default Values shall be used as fall back values (e.g. V_Train = 30km/h). Override request is not available in SB for L1, NL, SL, RV and TR | N/A | N/A | - |

| Step 2: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Type of media |
| N_PIG | 3 | FINITE VALUE | - |
| N_TOTAL | 3 | FINITE VALUE | 1 <= N <= 7 i.e. from 2 to 8 balises in the group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Identical Value for all balises of the group |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | ID of Balise Group |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 5/6/7/11/13 | SL/SB/TR/NL/SN |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 6

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.8.2: Selection of “Override” |  | 5080200 |
| Test Case of Feature | On-board equipment allows the driver to select override request only when the speed is under or equal to V_NVALLOWOVTRP: national value received from trackside. No Validated Train Data available is necessary when already in Shunting mode. |  | 6 |
| Applicable Mode/Level Combinations | L0: SH L1: SH L2: SH L3: SH |  |  |
| Target of Test | The target of the test is to check that the on-board equipment supervises: a speed limit to trigger the override EoA function override EoA function can be triggered only if train speed is not higher than speed limit defined by National value sent from trackside |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-5.8.2.1 |  |
|  |  | Subset-026-5.8.2.1 a) |  |
|  |  | Subset-026-5.8.2.1 b) |  |
|  |  | Subset-026-5.8.2.1 c) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The override request is DISPLAYED only when the train is running under (V_NVALLOWOVTRP : national value) The override request appears when speed comes under V_NVALLOWOVTRP and disappears when train overpasses V_NVALLOWOVTRP |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 | SH |
| National Values | STORED | V_NVALLOWOVTRP sent from trackside and different from the default value (0 km/h). |
| ERTMS/ETCS level | 0 / 2 / 3 / 4 | L0 / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The speed and distance monitoring information is displayed. The "EOA" button in the "Override" window is disabled. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is moving (V_TRAIN > V_NVALLOWOVTRP). | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The override button is activated | I | DMI | if the menu with override button is not activated, the driver selects the right menu in order to see the override button | N/A | N/A | - |
| 2 | N/A | N/A | The train runs with speed higher than V_NVALLOWOVTRP. Current speed is DISPLAYED. The “override request” is not DISPLAYED | O | DMI | V_TRAIN > V_NVALLOWOVTRP | N/A | N/A | - |
| 3 | N/A | N/A | The train decreases speed - train speed < V_NVALLOWOVTRP. Current speed is DISPLAYED "Override request" is DISPLAYED | O | DMI | V_TRAIN < V_NVALLOWOVTRP | N/A | N/A | - |
| 4 | N/A | N/A | The train accelerates until speed is higher than V_NVALLOWOVTRP. Current speed is DISPLAYED. The “override request” is not DISPLAYED | O | DMI | V_TRAIN > V_NVALLOWOVTRP | N/A | N/A | - |
| 5 | N/A | N/A | The train decreases speed train speed < V_NVALLOWOVTRP. Current speed is DISPLAYED "Override request" is DISPLAYED | O | DMI | V_TRAIN < V_NVALLOWOVTRP | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 | SH |
| National Values | STORED | V_NVALLOWOVTRP sent from trackside and different from the default value (0 km/h). |
| ERTMS/ETCS level | 0 / 2 / 3 / 4 | L0 / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The speed and distance monitoring information is displayed. The "EOA" button in the "Override" window is enabled. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is moving (V_TRAIN > V_NVALLOWOVTRP). | - | INT | - |