| - ERTMS/ETCS |  |
| --- | --- |
| Subset-076-6-3_9990200_02_v310.mdb State : Ordered list of Test Case Steps with communication information |  |
| REF : | Subset-076-6-3_9990200_02_v310.mdb |
| ISSUE : | 3.1.0 |
| DATE : | 19/04/17 13:23 |

| Company | Technical Approval | Management approval |
| --- | --- | --- |
| European Union Agency for Railways | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 3.1.0 19.04.2017 | All |  | MLT |

# Sequence description

Tested Feature A3.2: List of National / Default Data.

# Test cases

Important:	All columns called ‘Test Results’ are only placeholders for results of the test execution. They shall not be used for the description of test cases. 
A test is passed if all cells of ‘Test Result’ are passed.

|  | SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Step | Row |  |  | Dist. (m) |  | Previous |  |  |  |  | Description of Events |  | I/O |  | Interface |  | Comments |  |  | Next |  |  |  |  | Test Result |  |  |
|  |  |  |  |  |  |  | Levels |  | Mode |  |  |  |  |  |  |  |  |  |  |  | Levels |  |  | Mode |  |  |  |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 10: The ETCS on-board equipment is powered .Transition Condition Id [4]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 |  | 1 |  |  | 0.00 |  | N/A |  | NP |  |  | ERTMS/ETCS on-board unit is powered up |  | I |  | INT |  | The on-board equipment changes to SB mode. |  |  | N/A |  |  | NP |  | - |  |
|  | 2 |  | 2 |  |  | 0.00 |  | N/A |  | NP |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=6) is recorded |  | O |  | JRU |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 24: Start of Mission procedure is engaged |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 3 |  | 1 |  |  | 0.00 |  | N/A |  | SB |  |  | Driver opens desk |  | I |  | TIU |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 4 |  | 2 |  |  | 0.00 |  | N/A |  | SB |  |  | The 'CAB STATUS' message is RECORDED with the variable M_CAB_A_STATUS=1 (Active). |  | O |  | JRU |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 5 |  | 3 |  |  | 0.00 |  | N/A |  | SB |  |  | The actual mode SB is DISPLAYED |  | O |  | DMI |  | The SB indication after opening the desk may not be recorded. Because the change of mode was already recorded while desk was closed |  |  | N/A |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 7: The on-board equipment requests the driver to enter a driver ID within the scope of the Start of Mission procedure in SB mode when the status of the driver ID is unknown. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 6 |  | 1 |  |  | 0.00 |  | N/A |  | SB |  |  | The on-board equipment requires the driver to enter a Driver ID There is NO initial Driver ID value displayed |  | O |  | DMI |  | Starting conditions : There is no value initially displayed since the status of the driver ID is unknown. |  |  | N/A |  |  | SB |  | - |  |
|  | 7 |  | 2 |  |  | 0.00 |  | N/A |  | SB |  |  | The on-board equipment prevents the driver from continuing the procedure before the validation of the Driver ID is done. |  | O |  | DMI |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 8 |  | 3 |  |  | 0.00 |  | N/A |  | SB |  |  | The driver enters and validates a correct Driver ID |  | I |  | DMI |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 9 |  | 4 |  |  | 0.00 |  | N/A |  | SB |  |  | The validated Driver ID is recorded on the JRU with the DRIVER_ID variable in a 'GENERAL MESSAGE'. |  | O |  | JRU |  | A 'GENERAL MESSAGE' is recorded every 5s by the on-board equipment. |  |  | N/A |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 10: Within the scope of the Start of Mission procedure in SB mode, the status of the ERTMS/ETCS Level data is INVALID. The driver selects a level (the existing ERTMS/ETCS Level or a new one). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 10 |  | 1 |  |  | 0.00 |  | N/A |  | SB |  |  | The driver selects "Level <L0/LNTC/L1/L2/L3>" |  | I |  | DMI |  | Depending on the implementation, a list of available levels may be presented to the driver on the DMI. The driver is then allowed to select one level among the proposed levels. For NTC levels, the abbreviations of the concerned National Systems shall be presented to the driver. _____ The Driver selects L1 |  |  | L1 |  |  | SB |  | - |  |
|  | 11 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=<34/35/36/37/38>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 12 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=<0/1/2/3/4>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 6: Within the scope of the Start of Mission procedure in SB mode, the On-Board equipment offers the driver to select either SH or NL or ‘Train Data Entry’ before the on-board equipment requests the driver to enter a complete set of Train Data. The driver selects ‘Train Data Entry’. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 13 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver can select either SH or NL or ‘Train Data Entry’ |  | O |  | DMI |  | Starting conditions The 'Non-Leading' button is enabled if the 'non leading' input signal is received. |  |  | L1 |  |  | SB |  | - |  |
|  | 14 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver selects ‘Train Data Entry’ |  | I |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 15 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | A Train data window is is displayed on the DMI. |  | O |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 16 |  | 4 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver’s selection is recorded on the JRU. |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 47: The on-board equipment requests the driver to enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when the status of the Train Data is unknown (S12) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 18 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The on-board equipment requires the driver to enter all Train Data unknown by the on-board equipment and prevents the driver from performing any other actions. |  | O |  | DMI |  | The driver might be requested to enter the 'Train category(ies)', 'Train length', 'Traction / brake parameters', 'Maximum train speed', 'Loading gauge', 'Axle load category', 'Train fitted with airtight system', but the driver shall never be requested to enter the 'Traction system(s) accepted by the engine', 'List of National Systems available on-board' or 'Axle number'. |  |  | L1 |  |  | SB |  | - |  |
|  | 19 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver enters each Train data unknown on-board |  | I |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 20 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver is allowed to validate the train data |  | O |  | DMI |  | A complete set of valid Train Data is available |  |  | L1 |  |  | SB |  | - |  |
|  | 21 |  | 4 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver validates the train data. |  | I |  | DMI |  | The on-board equipment allows the driver to validate as soon as a valid and complete set of Train Data is entered. |  |  | L1 |  |  | SB |  | - |  |
|  | 22 |  | 5 |  |  | 0.00 |  | L1 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded |  | O |  | JRU |  | Triggered by : The driver validation. |  |  | L1 |  |  | SB |  | - |  |
|  | 23 |  | 6 |  |  | 0.00 |  | L1 |  | SB |  |  | TRAIN DATA (NID_MESSAGE_JRU=2) is recorded |  | O |  | JRU |  | Triggered by : Data Entry completed |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 46: After the set of Train Data has been entered/re-validated by the driver (in S12), if the status of the Train running number is 'unknown', the ERTMS/ETCS on-board equipment requests the driver to enter the Train running number (S13) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 25 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver is requested to enter the Train running number. |  | O |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 26 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver enters the "Train Running Number" [MAIN] |  | I |  | DMI |  | Entered Train Running Number: NID_OPERATIONAL_1 |  |  | L1 |  |  | SB |  | - |  |
|  | 27 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | ADDITIONAL DATA (NID_MESSAGE_JRU=24; NID_OPERATIONAL=NID_OPERATIONAL_1) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 1: The on-board equipment requests the driver to re-validate/enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when level is 0/1/NTC (Subset-026-5.4.3.2 S12). The driver is requested to enter/re-validate the Train running number if the status of the Train running number is 'unknown' or 'invalid'. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 28 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | The DMI allows the driver to select the option ‘START’. |  | O |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 16: The driver selects 'Start' within the scope of the Start of Mission procedure in SB mode and in ERTMS Level 1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 29 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver selects 'Start'. |  | I |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 30 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | Driver’s selection of 'Start' is recorded on the JRU |  | O |  | JRU |  | Triggered by: The driver acts on the on-board DMI. |  |  | L1 |  |  | SB |  | - |  |
|  | 31 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | An acknowledgement request for running in Staff Responsible mode is displayed on the DMI |  | O |  | DMI |  | The on-board equipment will then waits for the driver to acknowledge the acknowledgement request for running in Staff Responsible mode |  |  | L1 |  |  | SB |  | - |  |
|  | 32 |  | 4 |  |  | 0.00 |  | L1 |  | SB |  |  | The acknowledgement request for running in Staff Responsible mode is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 4070200: 4.7.2: DMI versus Mode Table - Input Test Case 9: In normal operation after the start of mission, the driver shall have no further possibility to modify the RBC contact information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 33 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver selects "Main" |  | I |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 34 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | The RBC contact window is not attainable |  | O |  | DMI |  | The display of the RBC contact window via the Level window must not be tested. |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 16: The driver selects 'Start' within the scope of the Start of Mission procedure in SB mode and in ERTMS Level 1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 35 |  | 5 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver acknowledges 'Staff Responsible' |  | I |  | DMI |  | Symbol MO10 |  |  | L1 |  |  | SB |  | - |  |
|  | 36 |  | 6 |  |  | 0.00 |  | L1 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS= 0000 0011) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 37 |  | 7 |  |  | 0.00 |  | L1 |  | SB |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=2) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 38 |  | 8 |  |  | 0.00 |  | L1 |  | SR |  |  | The Mode symbol 'Staff Responsible' is displayed |  | O |  | DMI |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 39 |  | 9 |  |  | 0.00 |  | L1 |  | SR |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit24=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 40 |  | 10 |  |  | 0.00 |  | L1 |  | SR |  |  | Any message (NID_MESSAGE_JRU=ALL; NID_LRBG=16777215) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | Feature 5080400: 5.8.4: End of Override procedure Test Case 2: The driver selects Override. When a new value for T_NVOVTRP is received on-board (by means of Packet 3) and it becomes applicable, the ongoing 'Override' supervision will use this new value. However, the starting time shall remain unchanged. The Override procedure ends when the "max. time for train trip suppression when Override function is triggered" elapses |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 41 |  | 1 |  |  | 0.00 |  | L1 |  | SR |  |  | The driver selects "Override EOA" |  | I |  | DMI |  | The train speed is below max. speed for override (V_NVALLOWOVTRP) _____ V=0 |  |  | L1 |  |  | SR |  | - |  |
|  | 42 |  | 2 |  |  | 0.00 |  | L1 |  | SR |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=14) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 43 |  | 3 |  |  | 0.00 |  | L1 |  | SR |  |  | The Mode symbol "Override EOA is active" is displayed |  | O |  | DMI |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 44 |  | 4 |  |  | 0.00 |  | L1 |  | SR |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit18=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | Feature 9990200: A3.2: List of National / Default Data Test Case 7: On-board equipment supervises train movements against overpassing default speed values of National Data valid for Override function |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 51 |  | 1 |  |  | 50.00 |  | L1 |  | SR |  |  | Train is moving (V_TRAIN=V_NVSUPOVTRP ) |  | I |  | INT |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 52 |  | 2 |  |  | 50.00 |  | L1 |  | SR |  |  | Driver requests permitted speed. |  | I |  | DMI |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 53 |  | 3 |  |  | 50.00 |  | L1 |  | SR |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 54 |  | 4 |  |  | 50.00 |  | L1 |  | SR |  |  | The speed and distance monitoring information is displayed |  | O |  | DMI |  | Indication of V_TRAIN |  |  | L1 |  |  | SR |  | - |  |
|  | 55 |  | 5 |  |  | 50.00 |  | L1 |  | SR |  |  | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 56 |  | 6 |  |  | 125.00 |  | L1 |  | SR |  |  | Train is moving (V_TRAIN>V_NVSUPOVTRP ) |  | I |  | INT |  | The train runs and overpasses the permitted speed limit when the override function is active. _____ Default value V_NVSUPOVTRP = 30 |  |  | L1 |  |  | SR |  | - |  |
|  | 57 |  | 7 |  |  | 125.00 |  | L1 |  | SR |  |  | Service brake commanded |  | O |  | TIU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 58 |  | 8 |  |  | 125.00 |  | L1 |  | SR |  |  | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 59 |  | 9 |  |  | 125.00 |  | L1 |  | SR |  |  | The Status symbol "Service Brake or Emergency Brake intervention" is displayed |  | O |  | DMI |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | Feature 9990200: A3.2: List of National / Default Data Test Case 29: When the train is NOT at standstill, the driver is NOT allowed by the on-board equipment to modify the current Driver ID when such modification is NOT permitted (national value M_NVDERUN set to 0). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 61 |  | 1 |  |  | 200.00 |  | L1 |  | SR |  |  | Train is moving |  | I |  | INT |  | - _____ V_TRAIN<V_NVSUPOVTRP |  |  | L1 |  |  | SR |  | - |  |
|  | Feature 9990200: A3.2: List of National / Default Data Test Case 7: On-board equipment supervises train movements against overpassing default speed values of National Data valid for Override function |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 63 |  | 11 |  |  | 200.00 |  | L1 |  | SR |  |  | Service brake not commanded |  | O |  | TIU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 64 |  | 12 |  |  | 200.00 |  | L1 |  | SR |  |  | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=0) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 65 |  | 13 |  |  | 200.00 |  | L1 |  | SR |  |  | The Status symbol "Service Brake or Emergency Brake intervention" is removed |  | O |  | DMI |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | Feature 4080426: 4.8.4: Accepted Information depending on the modes - National Values Test Case 4: Transmission of National Values via balise group |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 66 |  | 1 |  |  | 300.00 |  | L1 |  | SR |  |  | One Balise Group message containing packet 3 is received |  | I |  | BTM |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 67 |  | 2 |  |  | 300.00 |  | L1 |  | SR |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | Feature 4080424: 4.8.4: Accepted Information depending on the modes - Mode Profile Test Case 5: The train receives a message including packet 80 (Mode Profile) that is transmitted via balise group. The message is ACCEPTED. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 69 |  | 1 |  |  | 300.00 |  | L1 |  | SR |  |  | One Balise Group message containing packet 12, packet 21, packet 27, packet 80 is received |  | I |  | BTM |  | Complete message including Mode Profile is accepted |  |  | L1 |  |  | SR |  | - |  |
|  | 70 |  | 2 |  |  | 300.00 |  | L1 |  | SR |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | Complete message including Mode Profile is recorded |  |  | L1 |  |  | SR |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 57: Mode profile requires shunting at current location in level 1. Transition Condition Id [51]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 72 |  | 1 |  |  | 300.00 |  | L1 |  | SR |  |  | Train is moving. |  | I |  | INT |  | The train is at the location defined by the variable D_MAMODE (in packet 80) D_MAMODE = 0 |  |  | L1 |  |  | SR |  | - |  |
|  | 73 |  | 2 |  |  | 300.00 |  | L1 |  | SR |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=3) is recorded |  | O |  | JRU |  | The system switches to SH mode immediately |  |  | L1 |  |  | SH |  | - |  |
|  | 74 |  | 3 |  |  | 300.00 |  | L1 |  | SH |  |  | The Mode symbol 'Mode <SH>' is displayed |  | O |  | DMI |  | DMI Object: MO01 Or USE_FT4070201.20 |  |  | L1 |  |  | SH |  | - |  |
|  | 75 |  | 4 |  |  | 300.00 |  | L1 |  | SH |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit16=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 77 |  | 6 |  |  | 300.00 |  | L1 |  | SH |  |  | The Mode symbol 'Acknowledge Shunting' is displayed |  | O |  | DMI |  | DMI Object: MO02 |  |  | L1 |  |  | SH |  | - |  |
|  | 78 |  | 7 |  |  | 300.00 |  | L1 |  | SH |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit17=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 79 |  | 8 |  |  | 300.00 |  | L1 |  | SH |  |  | The driver acknowledges 'Shunting' |  | I |  | DMI |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 80 |  | 9 |  |  | 300.00 |  | L1 |  | SH |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 0001) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | Feature 4070201: 4.7.2: DMI versus Mode Table - Output Test Case 37: The on-board equipment is in SH mode and indicates the permitted speed on the DMI after a driver request. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 82 |  | 1 |  |  | 300.00 |  | L1 |  | SH |  |  | The permitted speed (V_NVSHUNT) is NOT DISPLAYED |  | O |  | DMI |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 83 |  | 2 |  |  | 300.00 |  | L1 |  | SH |  |  | The speed and distance monitoring information is displayed |  | O |  | DMI |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 84 |  | 3 |  |  | 300.00 |  | L1 |  | SH |  |  | The driver requests to show supervision limits |  | I |  | DMI |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 85 |  | 4 |  |  | 300.00 |  | L1 |  | SH |  |  | The driver action ‘Request to show supervision limits’ is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 86 |  | 5 |  |  | 300.00 |  | L1 |  | SH |  |  | The permitted speed (V_NVSHUNT) is DISPLAYED |  | O |  | DMI |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 87 |  | 6 |  |  | 300.00 |  | L1 |  | SH |  |  | The driver requests to hide supervision limits |  | I |  | DMI |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 88 |  | 7 |  |  | 300.00 |  | L1 |  | SH |  |  | The driver action ‘Request to hide supervision limits’ is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 89 |  | 8 |  |  | 300.00 |  | L1 |  | SH |  |  | The permitted speed (V_NVSHUNT) is NOT DISPLAYED |  | O |  | DMI |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | Feature 9990200: A3.2: List of National / Default Data Test Case 1: On-board supervision of shunting speed defined by national / default value in an area equipped with ETCS level 1, 2 or 3. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 90 |  | 2 |  |  | 375.00 |  | L1 |  | SH |  |  | Train is moving (V_TRAIN=<V_PERMITTED>) |  | I |  | INT |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 91 |  | 3 |  |  | 425.00 |  | L1 |  | SH |  |  | Train is moving (V_TRAIN=<V_PERMITTED+10>) |  | I |  | INT |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 92 |  | 4 |  |  | 425.00 |  | L1 |  | SH |  |  | The speed and distance monitoring information is displayed |  | O |  | DMI |  | Indication of warning due to over-speeding |  |  | L1 |  |  | SH |  | - |  |
|  | 93 |  | 5 |  |  | 425.00 |  | L1 |  | SH |  |  | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMSUPSTAT=3) is recorded |  | O |  | JRU |  | Indication of warning due to over-speeding |  |  | L1 |  |  | SH |  | - |  |
|  | 94 |  | 6 |  |  | 425.00 |  | L1 |  | SH |  |  | The Status symbol "Service Brake or Emergency Brake intervention" is displayed |  | O |  | DMI |  | Service brake intervention due to over-speeding |  |  | L1 |  |  | SH |  | - |  |
|  | 95 |  | 7 |  |  | 425.00 |  | L1 |  | SH |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=1>) is recorded |  | O |  | JRU |  | Service brake intervention due to over-speeding |  |  | L1 |  |  | SH |  | - |  |
|  | 96 |  | 8 |  |  | 425.00 |  | L1 |  | SH |  |  | Service brake commanded |  | O |  | TIU |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 97 |  | 9 |  |  | 425.00 |  | L1 |  | SH |  |  | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 98 |  | 10 |  |  | 475.00 |  | L1 |  | SH |  |  | Train is moving (V_TRAIN=<V_PERMITTED>) |  | I |  | INT |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 99 |  | 11 |  |  | 475.00 |  | L1 |  | SH |  |  | Service brake not commanded |  | O |  | TIU |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 100 |  | 12 |  |  | 475.00 |  | L1 |  | SH |  |  | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=0) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 101 |  | 13 |  |  | 475.00 |  | L1 |  | SH |  |  | The Status symbol "Service Brake or Emergency Brake intervention" is removed |  | O |  | DMI |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 102 |  | 14 |  |  | 475.00 |  | L1 |  | SH |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=0>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | Feature 4042000: 4.4.20: PASSIVE SHUNTING Test Case 1: As the engine is coupled to a leading engine, its ERTMS/ETCS on-board equipment shall not perform any train movement supervision. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 103 |  | 1 |  |  | 550.00 |  | L1 |  | SH |  |  | Train is at standstill (V_TRAIN=0) |  | I |  | INT |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 104 |  | 2 |  |  | 550.00 |  | L1 |  | SH |  |  | The Mode symbol "Shunting" is displayed |  | O |  | DMI |  | Indication of SH mode |  |  | L1 |  |  | SH |  | - |  |
|  | 105 |  | 3 |  |  | 550.00 |  | L1 |  | SH |  |  | Passive shunting permitted |  | I |  | TIU |  | 'passive shunting input signal' is received from the train interface |  |  | L1 |  |  | SH |  | - |  |
|  | 106 |  | 4 |  |  | 550.00 |  | L1 |  | SH |  |  | The driver selects "MAINTAIN SHUNTING" |  | I |  | DMI |  | The driver selects "Continue Shunting on desk closure" |  |  | L1 |  |  | SH |  | - |  |
|  | 107 |  | 5 |  |  | 550.00 |  | L1 |  | SH |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded |  | O |  | JRU |  | The driver selects "Continue Shunting on desk closure" is recorded |  |  | L1 |  |  | SH |  | - |  |
|  | 108 |  | 6 |  |  | 550.00 |  | L1 |  | SH |  |  | Cab not active |  | I |  | TIU |  | The driver closes the desk |  |  | L1 |  |  | SH |  | - |  |
|  | 109 |  | 7 |  |  | 550.00 |  | L1 |  | SH |  |  | CAB STATUS (NID_MESSAGE_JRU=38) is recorded |  | O |  | JRU |  | Closing of desk is recorded |  |  | L1 |  |  | SH |  | - |  |
|  | 110 |  | 8 |  |  | 550.00 |  | L1 |  | SH |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=15) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | N/A |  | - |  |
|  | 111 |  | 9 |  |  | 600.00 |  | L1 |  | N/A |  |  | Train is moving (V_TRAIN=40) |  | I |  | INT |  | - |  |  | L1 |  |  | N/A |  | - |  |
|  | Feature 5100100: 5.10.1: Level Transitions General requirements Test Case 1: The train is running in PS mode and receives multiples orders to switch immediately or conditionnaly from LX at the current position. Only the last set of Level Transition Information is stored AND the system switch only when another mode than SH or PS has been entered. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 112 |  | 1 |  |  | 620.00 |  | L1 |  | N/A |  |  | The train receives an order to switch immediately to Level W from balise group BGa. |  | I |  | BTM |  | Packet 41: 'Level Transition Order' is RECEIVED. D_LEVELTR= 0 or = 32767 M_LEVELTR= LVL_W Or Packet 46: 'Conditional Level Transition Order' is RECEIVED. M_LEVELTR= LVL_W |  |  | L1 |  |  | N/A |  | - |  |
|  | 113 |  | 2 |  |  | 620.00 |  | L1 |  | N/A |  |  | Balise Group message, including Packet 41 or 46, is RECORDED. |  | O |  | JRU |  | - |  |  | L1 |  |  | N/A |  | - |  |
|  | 114 |  | 3 |  |  | 675.00 |  | L1 |  | N/A |  |  | The train receives an order to switch immediately to Level Y from balise group BGa. |  | I |  | BTM |  | Packet 41: 'Level Transition Order' is RECEIVED. D_LEVELTR= 0 or = 32767 M_LEVELTR= LVL_Y Or Packet 46: 'Conditional Level Transition Order' is RECEIVED. M_LEVELTR= LVL_Y |  |  | L1 |  |  | N/A |  | - |  |
|  | 115 |  | 4 |  |  | 675.00 |  | L1 |  | N/A |  |  | Balise Group message, including Packet 41 or 46, is RECORDED. |  | O |  | JRU |  | - |  |  | L1 |  |  | N/A |  | - |  |
|  | 116 |  | 5 |  |  | 675.00 |  | L1 |  | N/A |  |  | The Level X is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  |  | N/A |  | - |  |
|  | Feature 4042000: 4.4.20: PASSIVE SHUNTING Test Case 1: As the engine is coupled to a leading engine, its ERTMS/ETCS on-board equipment shall not perform any train movement supervision. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 117 |  | 10 |  |  | 675.00 |  | L1 |  | N/A |  |  | One Balise Group message (N_TOTAL=0) containing packet 132 (Q_ASPECT=0) is received |  | I |  | BTM |  | Packet 132 (danger for shunting), Q_ASPECT = 0 (Stop if in SH mode) |  |  | L1 |  |  | N/A |  | - |  |
|  | 118 |  | 11 |  |  | 675.00 |  | L1 |  | N/A |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | N/A |  | - |  |
|  | 119 |  | 12 |  |  | 675.00 |  | L1 |  | N/A |  |  | Emergency brake not commanded |  | O |  | TIU |  | - |  |  | L1 |  |  | N/A |  | - |  |
|  | Feature 3060300: 3.6.3: Validity direction of transmitted Information Test Case 9: In PS mode, the ERTMS/ETCS on- board equipment shall only take into account information valid for its movement direction |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 120 |  | 1 |  |  | 725.00 |  | L1 |  | N/A |  |  | One Balise Group message (N_TOTAL=1) containing packet 135 (Q_DIR=0) and packet 255 is received |  | I |  | BTM |  | The packet 135 „Stop Shunting on desk opening' is valid for reverse direction only |  |  | L1 |  |  | N/A |  | - |  |
|  | 121 |  | 2 |  |  | 725.00 |  | L1 |  | N/A |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | N/A |  | - |  |
|  | Feature 4042000: 4.4.20: PASSIVE SHUNTING Test Case 1: As the engine is coupled to a leading engine, its ERTMS/ETCS on-board equipment shall not perform any train movement supervision. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 122 |  | 13 |  |  | 775.00 |  | L1 |  | N/A |  |  | Train is at standstill (V_TRAIN=0) |  | I |  | INT |  | The driver stops the train |  |  | L1 |  |  | N/A |  | - |  |
|  | Feature 5100100: 5.10.1: Level Transitions General requirements Test Case 1: The train is running in PS mode and receives multiples orders to switch immediately or conditionnaly from LX at the current position. Only the last set of Level Transition Information is stored AND the system switch only when another mode than SH or PS has been entered. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 126 |  | 6 |  |  | 775.00 |  | L1 |  | N/A |  |  | The driver opens the desk |  | I |  | TIU |  | - |  |  | L1 |  |  | N/A |  | - |  |
|  | Feature 4042000: 4.4.20: PASSIVE SHUNTING Test Case 1: As the engine is coupled to a leading engine, its ERTMS/ETCS on-board equipment shall not perform any train movement supervision. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 127 |  | 15 |  |  | 775.00 |  | L1 |  | N/A |  |  | CAB STATUS (NID_MESSAGE_JRU=38) is recorded |  | O |  | JRU |  | Opening of desk is recorded |  |  | L1 |  |  | SH |  | - |  |
|  | 128 |  | 16 |  |  | 775.00 |  | L1 |  | SH |  |  | The driver selects "Main" |  | I |  | DMI |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 129 |  | 17 |  |  | 775.00 |  | L1 |  | SH |  |  | The Maintain Shunting button in the Main window is enabled |  | O |  | DMI |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 130 |  | 18 |  |  | 775.00 |  | L1 |  | SH |  |  | Cab not active |  | I |  | TIU |  | The driver closes the desk |  |  | L1 |  |  | SH |  | - |  |
|  | 131 |  | 19 |  |  | 775.00 |  | L1 |  | SH |  |  | CAB STATUS (NID_MESSAGE_JRU=38) is recorded |  | O |  | JRU |  | Closing of desk is recorded |  |  | L1 |  |  | SH |  | - |  |
|  | 132 |  | 20 |  |  | 775.00 |  | L1 |  | SH |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | Feature 3060300: 3.6.3: Validity direction of transmitted Information Test Case 9: In PS mode, the ERTMS/ETCS on- board equipment shall only take into account information valid for its movement direction |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 133 |  | 5 |  |  | 775.00 |  | L1 |  | SH |  |  | The Mode symbol "Shunting" is displayed |  | O |  | DMI |  | Symbol MO01 „Shunting' is displayed |  |  | L1 |  |  | SH |  | - |  |
|  | 134 |  | 6 |  |  | 775.00 |  | L1 |  | SH |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit16=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 135 |  | 7 |  |  | 775.00 |  | L1 |  | SH |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=3) is recorded |  | O |  | JRU |  | New mode SH is recorded |  |  | L1 |  |  | SH |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 18: The on-board is in SH mode and the driver selects 'exit shunting'. The on-board equipment immediately switches to SB mode if the train is at standstill. Transition Condition Id [19]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 136 |  | 1 |  |  | 775.00 |  | L1 |  | SH |  |  | The Train is at standstill |  | I |  | INT |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 137 |  | 2 |  |  | 775.00 |  | L1 |  | SH |  |  | The driver selects 'EXIT SHUNTING' |  | I |  | DMI |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 138 |  | 3 |  |  | 775.00 |  | L1 |  | SH |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0001 0001) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 139 |  | 4 |  |  | 775.00 |  | L1 |  | SH |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=6) is recorded |  | O |  | JRU |  | The on-board equipment switches to SB mode. |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5100100: 5.10.1: Level Transitions General requirements Test Case 1: The train is running in PS mode and receives multiples orders to switch immediately or conditionnaly from LX at the current position. Only the last set of Level Transition Information is stored AND the system switch only when another mode than SH or PS has been entered. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 141 |  | 7 |  |  | 775.00 |  | L1 |  | SB |  |  | The train switches to SB |  | O |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 18: The on-board is in SH mode and the driver selects 'exit shunting'. The on-board equipment immediately switches to SB mode if the train is at standstill. Transition Condition Id [19]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 142 |  | 6 |  |  | 775.00 |  | L1 |  | SB |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit28=1>) is recorded |  | O |  | JRU |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | Feature 5100100: 5.10.1: Level Transitions General requirements Test Case 1: The train is running in PS mode and receives multiples orders to switch immediately or conditionnaly from LX at the current position. Only the last set of Level Transition Information is stored AND the system switch only when another mode than SH or PS has been entered. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 143 |  | 8 |  |  | 775.00 |  | L0 |  | SB |  |  | Level Y is DISPLAYED. |  | O |  | DMI |  | The On-board equipment switches to Level Y |  |  | L0 |  |  | SB |  | - |  |
|  | 144 |  | 9 |  |  | 775.00 |  | L0 |  | SB |  |  | Level Y is RECORDED |  | O |  | JRU |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 8: The on-board equipment allows the driver to validate an existing Driver ID within the scope of the Start of Mission procedure in SB mode when the status of the driver ID is invalid. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 145 |  | 1 |  |  | 775.00 |  | L0 |  | SB |  |  | There is an initial Driver ID displayed. The on-board equipment allows the driver to simply revalidate it. |  | O |  | DMI |  | A stored on-board Driver ID is displayed to the driver since the status of the Driver ID is INVALID. |  |  | L0 |  |  | SB |  | - |  |
|  | 146 |  | 2 |  |  | 775.00 |  | L0 |  | SB |  |  | The driver validates the Driver Id. |  | I |  | DMI |  | The on-board equipment allows the driver to simply revalidate an existing Driver ID without entering a new Driver ID. |  |  | L0 |  |  | SB |  | - |  |
|  | 147 |  | 3 |  |  | 775.00 |  | L0 |  | SB |  |  | The validated Driver ID is recorded on the JRU with the DRIVER_ID variable in a 'GENERAL MESSAGE'. |  | O |  | JRU |  | A 'GENERAL MESSAGE' is recorded every 5s by the on-board equipment. |  |  | L0 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 11: The on-board equipment allows the driver to validate an existing ERTMS/ETCS Level data within the scope of the Start of Mission procedure in SB mode when the status of the ERTMS/ETCS Level data is INVALID. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 148 |  | 1 |  |  | 775.00 |  | L0 |  | SB |  |  | The driver validates the ERMS/ETCS level. |  | I |  | DMI |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | 149 |  | 2 |  |  | 775.00 |  | L0 |  | SB |  |  | The initial ERTMS/ETCS Level is still displayed on the DMI. |  | O |  | DMI |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | 150 |  | 3 |  |  | 775.00 |  | L0 |  | SB |  |  | Driver’s validation action is recorded on the JRU. |  | O |  | JRU |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 6: Within the scope of the Start of Mission procedure in SB mode, the On-Board equipment offers the driver to select either SH or NL or ‘Train Data Entry’ before the on-board equipment requests the driver to enter a complete set of Train Data. The driver selects ‘Train Data Entry’. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 151 |  | 1 |  |  | 775.00 |  | L0 |  | SB |  |  | The driver can select either SH or NL or ‘Train Data Entry’ |  | O |  | DMI |  | Starting conditions The 'Non-Leading' button is enabled if the 'non leading' input signal is received. |  |  | L0 |  |  | SB |  | - |  |
|  | 152 |  | 2 |  |  | 775.00 |  | L0 |  | SB |  |  | The driver selects ‘Train Data Entry’ |  | I |  | DMI |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | 153 |  | 3 |  |  | 775.00 |  | L0 |  | SB |  |  | A Train data window is is displayed on the DMI. |  | O |  | DMI |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | 154 |  | 4 |  |  | 775.00 |  | L0 |  | SB |  |  | The driver’s selection is recorded on the JRU. |  | O |  | JRU |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 2: The on-board equipment requests the driver to validate an existing complete set of Train Data within the scope of the Start of Mission procedure in SB mode when the status of the Train Data is invalid (S12) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 156 |  | 1 |  |  | 775.00 |  | L0 |  | SB |  |  | A complete set of Train Data is displayed to the driver The driver is allowed to validate the train data. |  | O |  | DMI |  | The driver might be requested to validate the 'Train category(ies)', 'Train length', 'Traction / brake parameters', 'Maximum train speed', 'Loading gauge', 'Axle load category', 'Train fitted with airtight system', but not the 'Traction system(s) accepted by the engine', 'List of National Systems available on-board' or 'Axle number'. |  |  | L0 |  |  | SB |  | - |  |
|  | 157 |  | 2 |  |  | 775.00 |  | L0 |  | SB |  |  | The driver validates the train data. |  | I |  | DMI |  | The on-board equipment allows the driver to simply validate the existing set of Train Data. |  |  | L0 |  |  | SB |  | - |  |
|  | 158 |  | 3 |  |  | 775.00 |  | L0 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded |  | O |  | JRU |  | Triggered by : The driver validation. |  |  | L0 |  |  | SB |  | - |  |
|  | 159 |  | 4 |  |  | 775.00 |  | L0 |  | SB |  |  | TRAIN DATA (NID_MESSAGE_JRU=2) is recorded |  | O |  | JRU |  | Triggered by : Data Entry completed |  |  | L0 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 45: After the set of Train Data has been entered/re-validated by the driver (in S12), if the status of the Train running number is 'invalid', the ERTMS/ETCS on-board equipment requests the driver to re-validate the Train running number (S13) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 161 |  | 1 |  |  | 775.00 |  | L0 |  | SB |  |  | The driver is requested to re-validate the Train running number. |  | O |  | DMI |  | Displayed Train Running Number: NID_OPERATIONAL_1 |  |  | L0 |  |  | SB |  | - |  |
|  | 162 |  | 2 |  |  | 775.00 |  | L0 |  | SB |  |  | The driver confirms the "Train Running Number" [MAIN] |  | I |  | DMI |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | 163 |  | 3 |  |  | 775.00 |  | L0 |  | SB |  |  | ADDITIONAL DATA (NID_MESSAGE_JRU=24; NID_OPERATIONAL=NID_OPERATIONAL_1) is recorded |  | O |  | JRU |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 1: The on-board equipment requests the driver to re-validate/enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when level is 0/1/NTC (Subset-026-5.4.3.2 S12). The driver is requested to enter/re-validate the Train running number if the status of the Train running number is 'unknown' or 'invalid'. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 164 |  | 3 |  |  | 775.00 |  | L0 |  | SB |  |  | The DMI allows the driver to select the option ‘START’. |  | O |  | DMI |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | Feature 5060200: 5.6.2: Table of requirements for 'Shunting Initiated by Driver' procedure Test Case 2: Driver selects 'Shunting', train is at standstill (Transition [5]) and the system switches to SH mode. An on-going mission shall NOT exist. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 165 |  | 1 |  |  | 775.00 |  | L0 |  | SB |  |  | The driver presses 'Shunting' button. |  | I |  | DMI |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | 166 |  | 2 |  |  | 775.00 |  | L0 |  | SB |  |  | Driver selection is RECORDED. |  | O |  | JRU |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | 167 |  | 3 |  |  | 775.00 |  | L0 |  | SB |  |  | The SH symbol, MO01 is DISPLAYED. |  | O |  | DMI |  | - |  |  | L0 |  |  | SH |  | - |  |
|  | 168 |  | 4 |  |  | 775.00 |  | L0 |  | SH |  |  | The displayed symbol for SH mode is RECORDED |  | O |  | JRU |  | - |  |  | L0 |  |  | SH |  | - |  |
|  | Feature 4070201: 4.7.2: DMI versus Mode Table - Output Test Case 37: The on-board equipment is in SH mode and indicates the permitted speed on the DMI after a driver request. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 170 |  | 1 |  |  | 800.00 |  | L0 |  | SH |  |  | The permitted speed (V_NVSHUNT) is NOT DISPLAYED |  | O |  | DMI |  | - |  |  | L0 |  |  | SH |  | - |  |
|  | 171 |  | 2 |  |  | 800.00 |  | L0 |  | SH |  |  | The speed and distance monitoring information is displayed |  | O |  | DMI |  | - |  |  | L0 |  |  | SH |  | - |  |
|  | 172 |  | 3 |  |  | 800.00 |  | L0 |  | SH |  |  | The driver requests to show supervision limits |  | I |  | DMI |  | - |  |  | L0 |  |  | SH |  | - |  |
|  | 173 |  | 4 |  |  | 800.00 |  | L0 |  | SH |  |  | The driver action ‘Request to show supervision limits’ is RECORDED |  | O |  | JRU |  | - |  |  | L0 |  |  | SH |  | - |  |
|  | 174 |  | 5 |  |  | 800.00 |  | L0 |  | SH |  |  | The permitted speed (V_NVSHUNT) is DISPLAYED |  | O |  | DMI |  | - |  |  | L0 |  |  | SH |  | - |  |
|  | 175 |  | 6 |  |  | 800.00 |  | L0 |  | SH |  |  | The driver requests to hide supervision limits |  | I |  | DMI |  | - |  |  | L0 |  |  | SH |  | - |  |
|  | 176 |  | 7 |  |  | 800.00 |  | L0 |  | SH |  |  | The driver action ‘Request to hide supervision limits’ is RECORDED |  | O |  | JRU |  | - |  |  | L0 |  |  | SH |  | - |  |
|  | 177 |  | 8 |  |  | 800.00 |  | L0 |  | SH |  |  | The permitted speed (V_NVSHUNT) is NOT DISPLAYED |  | O |  | DMI |  | - |  |  | L0 |  |  | SH |  | - |  |
|  | Feature 9990200: A3.2: List of National / Default Data Test Case 2: On-board supervision of shunting speed defined by national / default value in an area not equipped with ETCS. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 178 |  | 2 |  |  | 825.00 |  | L0 |  | SH |  |  | Train is moving (V_TRAIN=<V_PERMITTED>) |  | I |  | INT |  | - |  |  | L0 |  |  | SH |  | - |  |
|  | 179 |  | 3 |  |  | 875.00 |  | L0 |  | SH |  |  | Train is moving (V_TRAIN=<V_PERMITTED+10>) |  | I |  | INT |  | - |  |  | L0 |  |  | SH |  | - |  |
|  | 180 |  | 4 |  |  | 875.00 |  | L0 |  | SH |  |  | The speed and distance monitoring information is displayed |  | O |  | DMI |  | Indication of warning due to over-speeding |  |  | L0 |  |  | SH |  | - |  |
|  | 181 |  | 5 |  |  | 875.00 |  | L0 |  | SH |  |  | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMSUPSTAT=3) is recorded |  | O |  | JRU |  | Indication of warning due to over-speeding |  |  | L0 |  |  | SH |  | - |  |
|  | 182 |  | 6 |  |  | 875.00 |  | L0 |  | SH |  |  | The Status symbol "Service Brake or Emergency Brake intervention" is displayed |  | O |  | DMI |  | Service brake intervention due to over-speeding |  |  | L0 |  |  | SH |  | - |  |
|  | 183 |  | 7 |  |  | 875.00 |  | L0 |  | SH |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=1>) is recorded |  | O |  | JRU |  | Service brake intervention due to over-speeding |  |  | L0 |  |  | SH |  | - |  |
|  | 184 |  | 8 |  |  | 875.00 |  | L0 |  | SH |  |  | Service brake commanded |  | O |  | TIU |  | - |  |  | L0 |  |  | SH |  | - |  |
|  | 185 |  | 9 |  |  | 875.00 |  | L0 |  | SH |  |  | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded |  | O |  | JRU |  | - |  |  | L0 |  |  | SH |  | - |  |
|  | 186 |  | 10 |  |  | 950.00 |  | L0 |  | SH |  |  | Train is moving (V_TRAIN=<V_PERMITTED>) |  | I |  | INT |  | - |  |  | L0 |  |  | SH |  | - |  |
|  | 187 |  | 11 |  |  | 950.00 |  | L0 |  | SH |  |  | Service brake not commanded |  | O |  | TIU |  | - |  |  | L0 |  |  | SH |  | - |  |
|  | 188 |  | 12 |  |  | 950.00 |  | L0 |  | SH |  |  | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=0) is recorded |  | O |  | JRU |  | - |  |  | L0 |  |  | SH |  | - |  |
|  | 189 |  | 13 |  |  | 950.00 |  | L0 |  | SH |  |  | The Status symbol "Service Brake or Emergency Brake intervention" is removed |  | O |  | DMI |  | - |  |  | L0 |  |  | SH |  | - |  |
|  | 190 |  | 14 |  |  | 950.00 |  | L0 |  | SH |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=0>) is recorded |  | O |  | JRU |  | - |  |  | L0 |  |  | SH |  | - |  |
|  | Feature 4080426: 4.8.4: Accepted Information depending on the modes - National Values Test Case 4: Transmission of National Values via balise group |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 191 |  | 1 |  |  | 950.00 |  | L0 |  | SH |  |  | One Balise Group message containing packet 3 is received |  | I |  | BTM |  | - _____ Restore default NV |  |  | L0 |  |  | SH |  | - |  |
|  | 192 |  | 2 |  |  | 950.00 |  | L0 |  | SH |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L0 |  |  | SH |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 1: The ETCS on-board system is NOT powered. Transition Condition Id [29]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 194 |  | 1 |  |  | 1000.00 |  | L0 |  | SH |  |  | The ERTMS/ETCS on-board unit is powered down |  | I |  | INT |  | - _____ The train reached standstill. |  |  | L0 |  |  | NP |  | - |  |
|  | 195 |  | 2 |  |  | 1000.00 |  | L0 |  | NP |  |  | NOT GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=-) is recorded |  | O |  | JRU |  | There are no messages recorded in the JRU during a period of time |  |  | L0 |  |  | NP |  | - |  |
| Step 10: DMI Events Distance 0.00 m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Event Id. |  |  |  | Event Name |  |  |  |  |  |  | Data |  |  |  |  |  |  |  |  | Delay |  |  |  |  |  |  |  |  |
| 29 |  |  |  | Request Change Level |  |  |  |  |  |  | LEVEL=1 |  |  |  |  |  |  |  |  | 10 s |  |  |  |  |  |  |  |  |

| - Step 66 : Balise Group BG 101 d 1/2 - Distance 300.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 101 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 12 d | Level 1 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 73 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| V_MAIN | 7 | 30 d |  |  |
| V_LOA | 7 | 0 d |  |  |
| T_LOA | 10 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_ENDSECTION | 15 | 1000 d |  |  |
| Q_SECTIONTIMER | 1 | 0 b |  |  |
| Q_ENDTIMER | 1 | 0 b |  |  |
| Q_DANGERPOINT | 1 | 0 b |  |  |
| Q_OVERLAP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 21 d | Gradient profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 54 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_GRADIENT | 15 | 0 d |  |  |
| Q_GDIR | 1 | 0 b |  |  |
| G_A | 8 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 27 d | International static speed profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 58 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_STATIC | 15 | 0 d |  |  |
| V_STATIC | 7 | 30 d |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 80 d | Mode profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 85 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_MAMODE | 15 | 0 d |  |  |
| M_MAMODE | 2 | 01 b |  |  |
| V_MAMODE | 7 | 6 d |  |  |
| L_MAMODE | 15 | 500 d |  |  |
| L_ACKMAMODE | 15 | 0 d |  |  |
| Q_MAMODE | 1 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 66 : Balise Group BG 101 d 2/2 - Distance 305.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 101 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 3 d | National values |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 230 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_VALIDNV | 15 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| V_NVSHUNT | 7 | 6 d |  |  |
| V_NVSTFF | 7 | 8 d |  |  |
| V_NVONSIGHT | 7 | 6 d |  |  |
| V_NVLIMSUPERV | 7 | 20 d |  |  |
| V_NVUNFIT | 7 | 20 d |  |  |
| V_NVREL | 7 | 8 d |  |  |
| D_NVROLL | 15 | 2 d |  |  |
| Q_NVSBTSMPERM | 1 | 1 d |  |  |
| Q_NVEMRRLS | 1 | 0 b |  |  |
| Q_NVGUIPERM | 1 | 0 b |  |  |
| Q_NVSBFBPERM | 1 | 0 b |  |  |
| Q_NVINHSMICPERM | 1 | 0 b |  |  |
| V_NVALLOWOVTRP | 7 | 0 d |  |  |
| V_NVSUPOVTRP | 7 | 6 d |  |  |
| D_NVOVTRP | 15 | 200 d |  |  |
| T_NVOVTRP | 8 | 60 d |  |  |
| D_NVPOTRP | 15 | 200 d |  |  |
| M_NVCONTACT | 2 | 10 b |  |  |
| T_NVCONTACT | 8 | 255 d |  |  |
| M_NVDERUN | 1 | 1 b |  |  |
| D_NVSTFF | 15 | 500 d |  |  |
| Q_NVDRIVER_ADHES | 1 | 0 b |  |  |
| A_NVMAXREDADH1 | 6 | 010100 b |  |  |
| A_NVMAXREDADH2 | 6 | 001110 b |  |  |
| A_NVMAXREDADH3 | 6 | 001110 b |  |  |
| Q_NVLOCACC | 6 | 12 d |  |  |
| M_NVAVADH | 5 | 1 d |  |  |
| M_NVEBCL | 4 | 0001 b |  |  |
| Q_NVKINT | 1 | 0 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 112 : Balise Group BG 110 d 1/2 - Distance 620.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 110 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 41 d | Level transition order |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 63 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_LEVELTR | 15 | 0 d |  |  |
| M_LEVELTR | 3 | 3 d |  |  |
| L_ACKLEVELTR | 15 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 112 : Balise Group BG 110 d 2/2 - Distance 625.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 110 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 114 : Balise Group BG 111 d 1/2 - Distance 675.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 111 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 41 d | Level transition order |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 63 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_LEVELTR | 15 | 0 d |  |  |
| M_LEVELTR | 3 | 0 d |  |  |
| L_ACKLEVELTR | 15 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 114 : Balise Group BG 111 d 2/2 - Distance 680.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 111 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 132 d | Danger for shunting information |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 24 d |  |  |
| Q_ASPECT | 1 | 0 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 120 : Balise Group BG 112 d 1/2 - Distance 725.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 112 d |  |  |
| Q_LINK | 1 | 0 b |  |  |
| NID_PACKET | 8 | 135 d | Stop shunting on desk opening |  |
| Q_DIR | 2 | 00 b |  |  |
| L_PACKET | 13 | 23 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 120 : Balise Group BG 112 d 2/2 - Distance 730.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 112 d |  |  |
| Q_LINK | 1 | 0 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 191 : Balise Group BG 113 d 1/2 - Distance 950.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 113 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 191 : Balise Group BG 113 d 2/2 - Distance 955.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 113 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 3 d | National values |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 230 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_VALIDNV | 15 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| V_NVSHUNT | 7 | 6 d |  |  |
| V_NVSTFF | 7 | 8 d |  |  |
| V_NVONSIGHT | 7 | 6 d |  |  |
| V_NVLIMSUPERV | 7 | 20 d |  |  |
| V_NVUNFIT | 7 | 20 d |  |  |
| V_NVREL | 7 | 8 d |  |  |
| D_NVROLL | 15 | 2 d |  |  |
| Q_NVSBTSMPERM | 1 | 1 d |  |  |
| Q_NVEMRRLS | 1 | 0 b |  |  |
| Q_NVGUIPERM | 1 | 0 b |  |  |
| Q_NVSBFBPERM | 1 | 0 b |  |  |
| Q_NVINHSMICPERM | 1 | 0 b |  |  |
| V_NVALLOWOVTRP | 7 | 0 d |  |  |
| V_NVSUPOVTRP | 7 | 6 d |  |  |
| D_NVOVTRP | 15 | 200 d |  |  |
| T_NVOVTRP | 8 | 60 d |  |  |
| D_NVPOTRP | 15 | 200 d |  |  |
| M_NVCONTACT | 2 | 10 b |  |  |
| T_NVCONTACT | 8 | 255 d |  |  |
| M_NVDERUN | 1 | 1 b |  |  |
| D_NVSTFF | 15 | 32767 d |  |  |
| Q_NVDRIVER_ADHES | 1 | 0 b |  |  |
| A_NVMAXREDADH1 | 6 | 010100 b |  |  |
| A_NVMAXREDADH2 | 6 | 001110 b |  |  |
| A_NVMAXREDADH3 | 6 | 001110 b |  |  |
| Q_NVLOCACC | 6 | 12 d |  |  |
| M_NVAVADH | 5 | 0 d |  |  |
| M_NVEBCL | 4 | 1001 b |  |  |
| Q_NVKINT | 1 | 0 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |