| - ERTMS/ETCS |  |
| --- | --- |
| Subset-076-6-3_9990200_03_v310.mdb State : Ordered list of Test Case Steps with communication information |  |
| REF : | Subset-076-6-3_9990200_03_v310.mdb |
| ISSUE : | 3.1.0 |
| DATE : | 19/04/17 13:25 |

| Company | Technical Approval | Management approval |
| --- | --- | --- |
| European Union Agency for Railways | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 3.1.0 19.04.2017 | All |  | MLT |

# Sequence description

Tested Feature A3.2 : List of National / Default Data

# Test cases

Important:	All columns called ‘Test Results’ are only placeholders for results of the test execution. They shall not be used for the description of test cases. 
A test is passed if all cells of ‘Test Result’ are passed.

|  | SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Step | Row |  | Dist. (m) |  | Previous |  |  |  |  |  | Description of Events |  |  | I/O |  | Interface |  |  | Comments |  | Next |  |  |  |  | Test Result |  |  |
|  |  |  |  |  |  | Levels |  |  | Mode |  |  |  |  |  |  |  |  |  |  |  |  | Levels |  |  | Mode |  |  |  |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 10: The ETCS on-board equipment is powered .Transition Condition Id [4]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 |  | 1 |  | 0.00 |  | N/A |  |  | NP |  |  | ERTMS/ETCS on-board unit is powered up |  |  | I |  | INT |  |  | The on-board equipment changes to SB mode. |  | N/A |  |  | NP |  | - |  |
|  | 2 |  | 2 |  | 0.00 |  | N/A |  |  | NP |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=6) is recorded |  |  | O |  | JRU |  |  | - |  | N/A |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 24: Start of Mission procedure is engaged |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 3 |  | 1 |  | 0.00 |  | N/A |  |  | SB |  |  | Driver opens desk |  |  | I |  | TIU |  |  | - |  | N/A |  |  | SB |  | - |  |
|  | 4 |  | 2 |  | 0.00 |  | N/A |  |  | SB |  |  | The 'CAB STATUS' message is RECORDED with the variable M_CAB_A_STATUS=1 (Active). |  |  | O |  | JRU |  |  | - |  | N/A |  |  | SB |  | - |  |
|  | 5 |  | 3 |  | 0.00 |  | N/A |  |  | SB |  |  | The actual mode SB is DISPLAYED |  |  | O |  | DMI |  |  | The SB indication after opening the desk may not be recorded. Because the change of mode was already recorded while desk was closed |  | N/A |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 7: The on-board equipment requests the driver to enter a driver ID within the scope of the Start of Mission procedure in SB mode when the status of the driver ID is unknown. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 6 |  | 1 |  | 0.00 |  | N/A |  |  | SB |  |  | The on-board equipment requires the driver to enter a Driver ID There is NO initial Driver ID value displayed |  |  | O |  | DMI |  |  | Starting conditions : There is no value initially displayed since the status of the driver ID is unknown. |  | N/A |  |  | SB |  | - |  |
|  | 7 |  | 2 |  | 0.00 |  | N/A |  |  | SB |  |  | The on-board equipment prevents the driver from continuing the procedure before the validation of the Driver ID is done. |  |  | O |  | DMI |  |  | - |  | N/A |  |  | SB |  | - |  |
|  | 8 |  | 3 |  | 0.00 |  | N/A |  |  | SB |  |  | The driver enters and validates a correct Driver ID |  |  | I |  | DMI |  |  | - |  | N/A |  |  | SB |  | - |  |
|  | 9 |  | 4 |  | 0.00 |  | N/A |  |  | SB |  |  | The validated Driver ID is recorded on the JRU with the DRIVER_ID variable in a 'GENERAL MESSAGE'. |  |  | O |  | JRU |  |  | A 'GENERAL MESSAGE' is recorded every 5s by the on-board equipment. |  | N/A |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 10: Within the scope of the Start of Mission procedure in SB mode, the status of the ERTMS/ETCS Level data is INVALID. The driver selects a level (the existing ERTMS/ETCS Level or a new one). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 10 |  | 1 |  | 0.00 |  | N/A |  |  | SB |  |  | The driver selects "Level <L0/LNTC/L1/L2/L3>" |  |  | I |  | DMI |  |  | Depending on the implementation, a list of available levels may be presented to the driver on the DMI. The driver is then allowed to select one level among the proposed levels. For NTC levels, the abbreviations of the concerned National Systems shall be presented to the driver. _____ The Driver selects L1 |  | L1 |  |  | SB |  | - |  |
|  | 11 |  | 2 |  | 0.00 |  | L1 |  |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=<34/35/36/37/38>) is recorded |  |  | O |  | JRU |  |  | - |  | L1 |  |  | SB |  | - |  |
|  | 12 |  | 3 |  | 0.00 |  | L1 |  |  | SB |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=<0/1/2/3/4>) is recorded |  |  | O |  | JRU |  |  | - |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 6: Within the scope of the Start of Mission procedure in SB mode, the On-Board equipment offers the driver to select either SH or NL or ‘Train Data Entry’ before the on-board equipment requests the driver to enter a complete set of Train Data. The driver selects ‘Train Data Entry’. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 13 |  | 1 |  | 0.00 |  | L1 |  |  | SB |  |  | The driver can select either SH or NL or ‘Train Data Entry’ |  |  | O |  | DMI |  |  | Starting conditions The 'Non-Leading' button is enabled if the 'non leading' input signal is received. |  | L1 |  |  | SB |  | - |  |
|  | 14 |  | 2 |  | 0.00 |  | L1 |  |  | SB |  |  | The driver selects ‘Train Data Entry’ |  |  | I |  | DMI |  |  | - |  | L1 |  |  | SB |  | - |  |
|  | 15 |  | 3 |  | 0.00 |  | L1 |  |  | SB |  |  | A Train data window is is displayed on the DMI. |  |  | O |  | DMI |  |  | - |  | L1 |  |  | SB |  | - |  |
|  | 16 |  | 4 |  | 0.00 |  | L1 |  |  | SB |  |  | The driver’s selection is recorded on the JRU. |  |  | O |  | JRU |  |  | - |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 47: The on-board equipment requests the driver to enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when the status of the Train Data is unknown (S12) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 18 |  | 1 |  | 0.00 |  | L1 |  |  | SB |  |  | The on-board equipment requires the driver to enter all Train Data unknown by the on-board equipment and prevents the driver from performing any other actions. |  |  | O |  | DMI |  |  | The driver might be requested to enter the 'Train category(ies)', 'Train length', 'Traction / brake parameters', 'Maximum train speed', 'Loading gauge', 'Axle load category', 'Train fitted with airtight system', but the driver shall never be requested to enter the 'Traction system(s) accepted by the engine', 'List of National Systems available on-board' or 'Axle number'. |  | L1 |  |  | SB |  | - |  |
|  | 19 |  | 2 |  | 0.00 |  | L1 |  |  | SB |  |  | The driver enters each Train data unknown on-board |  |  | I |  | DMI |  |  | - |  | L1 |  |  | SB |  | - |  |
|  | 20 |  | 3 |  | 0.00 |  | L1 |  |  | SB |  |  | The driver is allowed to validate the train data |  |  | O |  | DMI |  |  | A complete set of valid Train Data is available |  | L1 |  |  | SB |  | - |  |
|  | 21 |  | 4 |  | 0.00 |  | L1 |  |  | SB |  |  | The driver validates the train data. |  |  | I |  | DMI |  |  | The on-board equipment allows the driver to validate as soon as a valid and complete set of Train Data is entered. |  | L1 |  |  | SB |  | - |  |
|  | 22 |  | 5 |  | 0.00 |  | L1 |  |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded |  |  | O |  | JRU |  |  | Triggered by : The driver validation. |  | L1 |  |  | SB |  | - |  |
|  | 23 |  | 6 |  | 0.00 |  | L1 |  |  | SB |  |  | TRAIN DATA (NID_MESSAGE_JRU=2) is recorded |  |  | O |  | JRU |  |  | Triggered by : Data Entry completed |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 46: After the set of Train Data has been entered/re-validated by the driver (in S12), if the status of the Train running number is 'unknown', the ERTMS/ETCS on-board equipment requests the driver to enter the Train running number (S13) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 25 |  | 1 |  | 0.00 |  | L1 |  |  | SB |  |  | The driver is requested to enter the Train running number. |  |  | O |  | DMI |  |  | - |  | L1 |  |  | SB |  | - |  |
|  | 26 |  | 2 |  | 0.00 |  | L1 |  |  | SB |  |  | The driver enters the "Train Running Number" [MAIN] |  |  | I |  | DMI |  |  | Entered Train Running Number: NID_OPERATIONAL_1 |  | L1 |  |  | SB |  | - |  |
|  | 27 |  | 3 |  | 0.00 |  | L1 |  |  | SB |  |  | ADDITIONAL DATA (NID_MESSAGE_JRU=24; NID_OPERATIONAL=NID_OPERATIONAL_1) is recorded |  |  | O |  | JRU |  |  | - |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 16: The driver selects 'Start' within the scope of the Start of Mission procedure in SB mode and in ERTMS Level 1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 29 |  | 1 |  | 0.00 |  | L1 |  |  | SB |  |  | The driver selects 'Start'. |  |  | I |  | DMI |  |  | - |  | L1 |  |  | SB |  | - |  |
|  | 30 |  | 2 |  | 0.00 |  | L1 |  |  | SB |  |  | Driver’s selection of 'Start' is recorded on the JRU |  |  | O |  | JRU |  |  | Triggered by: The driver acts on the on-board DMI. |  | L1 |  |  | SB |  | - |  |
|  | 31 |  | 3 |  | 0.00 |  | L1 |  |  | SB |  |  | An acknowledgement request for running in Staff Responsible mode is displayed on the DMI |  |  | O |  | DMI |  |  | The on-board equipment will then waits for the driver to acknowledge the acknowledgement request for running in Staff Responsible mode |  | L1 |  |  | SB |  | - |  |
|  | 32 |  | 4 |  | 0.00 |  | L1 |  |  | SB |  |  | The acknowledgement request for running in Staff Responsible mode is recorded |  |  | O |  | JRU |  |  | - |  | L1 |  |  | SB |  | - |  |
|  | 33 |  | 5 |  | 0.00 |  | L1 |  |  | SB |  |  | The driver acknowledges 'Staff Responsible' |  |  | I |  | DMI |  |  | Symbol MO10 |  | L1 |  |  | SB |  | - |  |
|  | 34 |  | 6 |  | 0.00 |  | L1 |  |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS= 0000 0011) is recorded |  |  | O |  | JRU |  |  | - |  | L1 |  |  | SB |  | - |  |
|  | 35 |  | 7 |  | 0.00 |  | L1 |  |  | SB |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=2) is recorded |  |  | O |  | JRU |  |  | - |  | L1 |  |  | SR |  | - |  |
|  | 36 |  | 8 |  | 0.00 |  | L1 |  |  | SR |  |  | The Mode symbol 'Staff Responsible' is displayed |  |  | O |  | DMI |  |  | - |  | L1 |  |  | SR |  | - |  |
|  | 37 |  | 9 |  | 0.00 |  | L1 |  |  | SR |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit24=1>) is recorded |  |  | O |  | JRU |  |  | - |  | L1 |  |  | SR |  | - |  |
|  | 38 |  | 10 |  | 0.00 |  | L1 |  |  | SR |  |  | Any message (NID_MESSAGE_JRU=ALL; NID_LRBG=16777215) is recorded |  |  | O |  | JRU |  |  | - |  | L1 |  |  | SR |  | - |  |
|  | Feature 9990200: A3.2: List of National / Default Data Test Case 40: The train uses the default location accuracy of a balise group |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 39 |  | 1 |  | 100.00 |  | L1 |  |  | SR |  |  | One Balise Group message (NID_BG = BGa) containing packet 5 (NID_BG=BGb; Q_LOCACC=5) is received |  |  | I |  | BTM |  |  | Packets inside telegram include packet 5 : linking |  | L1 |  |  | SR |  | - |  |
|  | 40 |  | 2 |  | 100.00 |  | L1 |  |  | SR |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  |  | O |  | JRU |  |  | BGa |  | L1 |  |  | SR |  | - |  |
|  | 41 |  | 3 |  | 100.00 |  | L1 |  |  | SR |  |  | GENERAL MESSAGE(NID_MESSAGE=1;L_DOUBTOVER= L_DOUBTOVER(1);L_DOUBTUNDER= L_DOUBTUNDER(1)) |  |  | O |  | JRU |  |  | L_DOUBTUNDER(1) and L_DOUBTOVER(1) have reduced their values after having recorded the Balise Group BGa. |  | L1 |  |  | SR |  | - |  |
|  | 42 |  | 4 |  | 200.00 |  | L1 |  |  | SR |  |  | One Balise Group message (NID_BG = BGb) is received |  |  | I |  | BTM |  |  | Packets inside telegram are not relevant. |  | L1 |  |  | SR |  | - |  |
|  | 43 |  | 5 |  | 200.00 |  | L1 |  |  | SR |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  |  | O |  | JRU |  |  | BGb |  | L1 |  |  | SR |  | - |  |
|  | 44 |  | 6 |  | 200.00 |  | L1 |  |  | SR |  |  | GENERAL MESSAGE(NID_MESSAGE=1;L_DOUBTOVER= L_DOUBTOVER(2);L_DOUBTUNDER= L_DOUBTUNDER(2)) |  |  | O |  | JRU |  |  | L_DOUBTUNDER(2) and L_DOUBTOVER(2) have reduced their values after having recorded the Balise Group BGb. L_DOUBTUNDER(2) < L_DOUBTUNDER(1), L_DOUBTOVER(2) < L_DOUBTOVER(1) |  | L1 |  |  | SR |  | - |  |
|  | Feature 5100100: 5.10.1: Level Transitions General requirements Test Case 37: The train receives an order to switch immediately from L1 to L2 at the current position. On-board equipment immediately switches to L2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 45 |  | 1 |  | 300.00 |  | L1 |  |  | SR |  |  | The train reaches the Level transition border and an order to switch immediately to Level 2 is received from balise group BGa. Packet 41: 'Level Transition Order' is RECEIVED. D_LEVELTR= 0 or = 32767 M_LEVELTR=3 |  |  | I |  | BTM |  |  | - |  | L1 |  |  | SR |  | - |  |
|  | 46 |  | 2 |  | 300.00 |  | L1 |  |  | SR |  |  | Balise Group message, including Packet 41 'Level transition order', is RECORDED. |  |  | O |  | JRU |  |  | - |  | L1 |  |  | SR |  | - |  |
|  | 47 |  | 3 |  | 300.00 |  | L1 |  |  | SR |  |  | Level 2 is DISPLAYED. |  |  | O |  | DMI |  |  | (Except in SL mode) The On-board equipment switches to Level 2. |  | L2 |  |  | SR |  | - |  |
|  | 48 |  | 4 |  | 300.00 |  | L2 |  |  | SR |  |  | New Level 2 is RECORDED. |  |  | O |  | JRU |  |  | - |  | L2 |  |  | SR |  | - |  |
|  | Feature 4080426: 4.8.4: Accepted Information depending on the modes - National Values Test Case 4: Transmission of National Values via balise group |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 49 |  | 1 |  | 300.00 |  | L2 |  |  | SR |  |  | One Balise Group message containing packet 3 is received |  |  | I |  | BTM |  |  | - |  | L2 |  |  | SR |  | - |  |
|  | 50 |  | 2 |  | 300.00 |  | L2 |  |  | SR |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  |  | O |  | JRU |  |  | - |  | L2 |  |  | SR |  | - |  |
|  | Feature 3050300: 3.5.3: Establishing a communication session Test Case 22: System version is compatible and session is established. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 52 |  | 1 |  | 300.00 |  | L2 |  |  | SR |  |  | One Balise Group message (NID_RBC?16383) containing packet 42 (NID_RADIO ? FFFF FFFF FFFF FFFF) is received |  |  | I |  | BTM |  |  | Not use last known RBC and don’t use short number. |  | L2 |  |  | SR |  | - |  |
|  | 53 |  | 2 |  | 300.00 |  | L2 |  |  | SR |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  |  | O |  | JRU |  |  | - |  | L2 |  |  | SR |  | - |  |
|  | 54 |  | 3 |  | 300.00 |  | L2 |  |  | SR |  |  | SA-CONNECT.Request is transmitted |  |  | O |  | RTM |  |  | According to EURORADIO specifications. This attempt shall be repeated until successful. The onboard tries to set up a connection with the phone number included in the session management packet. |  | L2 |  |  | SR |  | - |  |
|  | 55 |  | 4 |  | 300.00 |  | L2 |  |  | SR |  |  | SA-CONNECT.Confirm is received |  |  | I |  | RTM |  |  | - |  | L2 |  |  | SR |  | - |  |
|  | 56 |  | 5 |  | 300.00 |  | L2 |  |  | SR |  |  | The Status symbol "Safe radio connection - Connection Up" is displayed |  |  | O |  | DMI |  |  | - |  | L2 |  |  | SR |  | - |  |
|  | 57 |  | 6 |  | 300.00 |  | L2 |  |  | SR |  |  | JRU step 'DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit40=1>) is recorded'. |  |  | O |  | JRU |  |  | - |  | L2 |  |  | SR |  | - |  |
|  | 58 |  | 7 |  | 300.00 |  | L2 |  |  | SR |  |  | SA-DATA.Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted |  |  | O |  | RTM |  |  | - |  | L2 |  |  | SR |  | - |  |
|  | 59 |  | 8 |  | 300.00 |  | L2 |  |  | SR |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  |  | O |  | JRU |  |  | - |  | L2 |  |  | SR |  | - |  |
|  | 60 |  | 9 |  | 300.00 |  | L2 |  |  | SR |  |  | SA-DATA.Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32) is received |  |  | I |  | RTM |  |  | System versions M_VERSION (Version of the ERTMS/ETCS language) are compatible, so that the communication session established for on-board |  | L2 |  |  | SR |  | - |  |
|  | 61 |  | 10 |  | 300.00 |  | L2 |  |  | SR |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  |  | O |  | JRU |  |  | - |  | L2 |  |  | SR |  | - |  |
|  | 62 |  | 11 |  | 300.00 |  | L2 |  |  | SR |  |  | SA-DATA.Request with Euroradio Message "Session Established" (NID_MESSAGE=159) is transmitted |  |  | O |  | RTM |  |  | - |  | L2 |  |  | SR |  | - |  |
|  | 63 |  | 12 |  | 300.00 |  | L2 |  |  | SR |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  |  | O |  | JRU |  |  | - |  | L2 |  |  | SR |  | - |  |
|  | Feature 4080401: 4.8.4: Accepted Information depending on the modes - Acknowledgement of train data Test Case 1: Radio message with new Train Data and acknowledge of the RBC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 65 |  | 1 |  | 300.00 |  | L2 |  |  | SR |  |  | SA-DATA.Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) is transmitted |  |  | O |  | RTM |  |  | Validated train data |  | L2 |  |  | SR |  | - |  |
|  | 66 |  | 2 |  | 300.00 |  | L2 |  |  | SR |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=129) is recorded |  |  | O |  | JRU |  |  | - |  | L2 |  |  | SR |  | - |  |
|  | 67 |  | 3 |  | 300.00 |  | L2 |  |  | SR |  |  | SA-DATA.Indication with Euroradio Message "Acknowledgement of Train Data" (NID_MESSAGE=8) is received |  |  | I |  | RTM |  |  | - |  | L2 |  |  | SR |  | - |  |
|  | 68 |  | 4 |  | 300.00 |  | L2 |  |  | SR |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=8) is recorded |  |  | O |  | JRU |  |  | - |  | L2 |  |  | SR |  | - |  |
|  | Feature 4080425: 4.8.4: Accepted Information depending on the modes - Movement Authority Test Case 3: New Level 2/3 MA is ACCEPTED by the on board equipment because the SSP (Static Speed Profile) and GP (Gradient Profile) sent together with the MA on-board covers the whole length of the MA. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 69 |  | 1 |  | 550.00 |  | L2 |  |  | SR |  |  | SA-Data.Indication with Euroradio message (NID_MESSAGE=3) containing packet 15 (MA), packet 21 (Gradient), packet 27 (SSP) is received |  |  | I |  | RTM |  |  | Track description sending is optional New track description (or stored one if not sent) covers the movement authority distance |  | L2 |  |  | SR |  | - |  |
|  | 70 |  | 2 |  | 550.00 |  | L2 |  |  | SR |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  |  | O |  | JRU |  |  | - |  | L2 |  |  | SR |  | - |  |
|  | 71 |  | 3 |  | 550.00 |  | L2 |  |  | SR |  |  | The speed and distance monitoring information is displayed |  |  | O |  | DMI |  |  | - |  | L2 |  |  | FS |  | - |  |
|  | 72 |  | 4 |  | 550.00 |  | L2 |  |  | FS |  |  | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded |  |  | O |  | JRU |  |  | - |  | L2 |  |  | FS |  | - |  |
|  | Feature 9990200: A3.2: List of National / Default Data Test Case 42: Permission to release emergency brake defined by default national value |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 73 |  | 1 |  | 750.00 |  | L2 |  |  | FS |  |  | Train is moving (V_TRAIN < V_MRSP + dV_EBI (V_MRSP)) |  |  | I |  | INT |  |  | Train speed is below the EBI supervision limit |  | L2 |  |  | FS |  | - |  |
|  | Feature 4080440: 4.8.4: Accepted Information depending on the modes - Temporary Speed Restriction Test Case 1: The trackside generates and sends a radio message including packet 65(TSR) to a train. On-board checks and verifies and then accepts the message. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 74 |  | 1 |  | 775.00 |  | L2 |  |  | FS |  |  | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 65 is received |  |  | I |  | RTM |  |  | Complete message is accepted and stored |  | L2 |  |  | FS |  | - |  |
|  | 75 |  | 2 |  | 775.00 |  | L2 |  |  | FS |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9;NID_PACKET=65) is recorded |  |  | O |  | JRU |  |  | Complete message is recorded |  | L2 |  |  | FS |  | - |  |
|  | Feature 9990200: A3.2: List of National / Default Data Test Case 42: Permission to release emergency brake defined by default national value |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 77 |  | 2 |  | 800.00 |  | L2 |  |  | FS |  |  | Train is moving (V_TRAIN > V_MRSP + dV_EBI (V_MRSP)) |  |  | I |  | INT |  |  | The train exceeds the EBI supervision limit and the emergency brake is commanded |  | L2 |  |  | FS |  | - |  |
|  | 78 |  | 3 |  | 800.00 |  | L2 |  |  | FS |  |  | Emergency brake commanded |  |  | O |  | TIU |  |  | - |  | L2 |  |  | FS |  | - |  |
|  | 79 |  | 4 |  | 800.00 |  | L2 |  |  | FS |  |  | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=1) is recorded |  |  | O |  | JRU |  |  | M_BRAKE_COMMAND_STATE = 1 (Commanded) |  | L2 |  |  | FS |  | - |  |
|  | 80 |  | 5 |  | 1000.00 |  | L2 |  |  | FS |  |  | Train is moving (V_TRAIN < V_MRSP + dV_EBI (V_MRSP)) |  |  | I |  | INT |  |  | Train speed is again below the EBI supervision limit. Emergency brake is STILL commanded |  | L2 |  |  | FS |  | - |  |
|  | 81 |  | 6 |  | 1100.00 |  | L2 |  |  | FS |  |  | Train is at standstill (V_TRAIN=0) |  |  | I |  | INT |  |  | - |  | L2 |  |  | FS |  | - |  |
|  | 82 |  | 7 |  | 1100.00 |  | L2 |  |  | FS |  |  | Emergency brake NOT commanded |  |  | O |  | TIU |  |  | Check that the emergency brake is release AFTER train reaches stillstand |  | L2 |  |  | FS |  | - |  |
|  | 83 |  | 8 |  | 1100.00 |  | L2 |  |  | FS |  |  | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=0) is recorded |  |  | O |  | JRU |  |  | M_BRAKE_COMMAND_STATE = 0 (Not Commanded) |  | L2 |  |  | FS |  | - |  |
|  | Feature 9990200: A3.2: List of National / Default Data Test Case 18: When an RBC/RBC handover has been announced, the current onboard time shall be compared with the time stamp of the latest consistent message from the Handing over RBC until the train considers the Accepting RBC as the supervising one. From then on the current onboard time shall be compared with the time stamp of the latest received consistent message from the Accepting RBC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 84 |  | 1 |  | 1300.00 |  | L2 |  |  | FS |  |  | SA-DATA.Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 15, packet 131 (D_RBCTR ? 0) is received |  |  | I |  | RTM |  |  | A radio message 3 (Movement authority) containing an announcement of the RBC transition (packet 131) is received from the Handing Over RBC. • D_RBCTR ? 0 The radio message contains a movement authority including the RBC transition border. All other RTM messages are received from or sent to the accepting RBC. |  | L2 |  |  | FS |  | - |  |
|  | 85 |  | 2 |  | 1300.00 |  | L2 |  |  | FS |  |  | The radio message 3 with packets 15 & 131 is RECORDED |  |  | O |  | JRU |  |  | - |  | L2 |  |  | FS |  | - |  |
|  | 86 |  | 3 |  | 1300.00 |  | L2 |  |  | FS |  |  | SA-CONNECT.Request is transmitted |  |  | O |  | RTM |  |  | A safe connection is established with the Accepting RBC |  | L2 |  |  | FS |  | - |  |
|  | 87 |  | 4 |  | 1300.00 |  | L2 |  |  | FS |  |  | SA-CONNECT.Confirm is received |  |  | I |  | RTM |  |  | - |  | L2 |  |  | FS |  | - |  |
|  | 88 |  | 5 |  | 1300.00 |  | L2 |  |  | FS |  |  | SA-DATA.Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted |  |  | O |  | RTM |  |  | - |  | L2 |  |  | FS |  | - |  |
|  | 89 |  | 6 |  | 1300.00 |  | L2 |  |  | FS |  |  | The radio message 155 is RECORDED |  |  | O |  | JRU |  |  | - |  | L2 |  |  | FS |  | - |  |
|  | 90 |  | 7 |  | 1300.00 |  | L2 |  |  | FS |  |  | SA-DATA.Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32) is received |  |  | I |  | RTM |  |  | The Version of the ERTMS/ETCS language is valid for the train and M_VERSION is Version 1.0, 1.1 or 2.0 |  | L2 |  |  | FS |  | - |  |
|  | 91 |  | 8 |  | 1300.00 |  | L2 |  |  | FS |  |  | The radio message 32 is RECORDED |  |  | O |  | JRU |  |  | - |  | L2 |  |  | FS |  | - |  |
|  | 92 |  | 9 |  | 1300.00 |  | L2 |  |  | FS |  |  | SA-DATA.Request with Euroradio Message "Session Established" (NID_MESSAGE=159) is transmitted |  |  | O |  | RTM |  |  | The communication session with the Accepting RBC is established |  | L2 |  |  | FS |  | - |  |
|  | 93 |  | 10 |  | 1300.00 |  | L2 |  |  | FS |  |  | The radio message 159 is RECORDED |  |  | O |  | JRU |  |  | - |  | L2 |  |  | FS |  | - |  |
|  | 94 |  | 11 |  | 1300.00 |  | L2 |  |  | FS |  |  | SA-DATA.Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) containing packet 0 is transmitted |  |  | O |  | RTM |  |  | The radio message contains also position report data. |  | L2 |  |  | FS |  | - |  |
|  | 95 |  | 12 |  | 1300.00 |  | L2 |  |  | FS |  |  | The radio message 129 is RECORDED |  |  | O |  | JRU |  |  | - |  | L2 |  |  | FS |  | - |  |
|  | 96 |  | 13 |  | 1300.00 |  | L2 |  |  | FS |  |  | SA-DATA.Indication with Euroradio Message "Acknowledgement of Train Data" (NID_MESSAGE=8) is received |  |  | I |  | RTM |  |  | - |  | L2 |  |  | FS |  | - |  |
|  | 97 |  | 14 |  | 1300.00 |  | L2 |  |  | FS |  |  | The radio message 8 is RECORDED |  |  | O |  | JRU |  |  | - |  | L2 |  |  | FS |  | - |  |
|  | 98 |  | 15 |  | 1550.00 |  | L2 |  |  | FS |  |  | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 3 is received |  |  | I |  | RTM |  |  | T_NVCONTACT with a value lower than the T_NVCONTACT of the Handing over RBC area (currently being supervised) M_NVCONTACT will be set to 'Apply SB'. D_VALIDNV should be short enough to start being valid from this location, at least. |  | L2 |  |  | FS |  | - |  |
|  | 99 |  | 16 |  | 1550.00 |  | L2 |  |  | FS |  |  | The radio message 24 with Packet 3 is RECORDED |  |  | O |  | JRU |  |  | - |  | L2 |  |  | FS |  | - |  |
|  | 100 |  | 17 |  | 1650.00 |  | L2 |  |  | FS |  |  | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing is received |  |  | I |  | RTM |  |  | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24, with period lower than T_NVCONTACT) is received |  | L2 |  |  | FS |  | - |  |
|  | 101 |  | 18 |  | 1650.00 |  | L2 |  |  | FS |  |  | The radio message 24 is RECORDED |  |  | O |  | JRU |  |  | - |  | L2 |  |  | FS |  | - |  |
|  | 102 |  | 19 |  | 1850.00 |  | L2 |  |  | FS |  |  | NOT SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) is received |  |  | I |  | RTM |  |  | The train does not receive any message from the Accepting RBC, for a period equal to the value of T_NVCONTACT of the Accepting RBC area. The value of T_NVCONTACT of the Accepting RBC area must be lower than the T_NVCONTACT of the Handing over RBC area. |  | L2 |  |  | FS |  | - |  |
|  | 103 |  | 20 |  | 1850.00 |  | L2 |  |  | FS |  |  | No radio link supervision error is recorded. |  |  | O |  | JRU |  |  | - |  | L2 |  |  | FS |  | - |  |
|  | 104 |  | 21 |  | 1850.00 |  | L2 |  |  | FS |  |  | The Service Brake is not applied, after the expiration of the T_NVCONTACT of the Accepting RBC. |  |  | O |  | TIU |  |  | - |  | L2 |  |  | FS |  | - |  |
|  | 105 |  | 22 |  | 1950.00 |  | L2 |  |  | FS |  |  | NOT SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) is received |  |  | I |  | RTM |  |  | The train does not receive any message from the Accepting RBC, for a period equal to the value of T_NVCONTACT of the Handing over RBC area. This is to check that the current onboard time shall NOT be compared with the time stamp of the latest consistent message from the Accepting RBC |  | L2 |  |  | FS |  | - |  |
|  | 106 |  | 23 |  | 1950.00 |  | L2 |  |  | FS |  |  | No radio link supervision error is recorded. |  |  | O |  | JRU |  |  | - |  | L2 |  |  | FS |  | - |  |
|  | 107 |  | 24 |  | 1950.00 |  | L2 |  |  | FS |  |  | NOT SA-DISCONNECT.Request is transmitted |  |  | O |  | RTM |  |  | - |  | L2 |  |  | FS |  | - |  |
|  | Feature 4080427: 4.8.4: Accepted Information depending on the modes - Plain Text Information Test Case 1: Transmission of a plain text message to the on-board equipment from RBC referring to train position. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 108 |  | 1 |  | 2000.00 |  | L2 |  |  | FS |  |  | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 72 is received |  |  | I |  | RTM |  |  | - _____ Message received from the Handing over RBC |  | L2 |  |  | FS |  | - |  |
|  | 109 |  | 2 |  | 2000.00 |  | L2 |  |  | FS |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  |  | O |  | JRU |  |  | The RBC send a plain text message to the on-board equipment. The plain text message include information about the train position where the message has to be displayed. The on-board equipment has to accept information referring to one of at least eight LRBGONB last reported to the RBC. |  | L2 |  |  | FS |  | - |  |
|  | 110 |  | 3 |  | 2000.00 |  | L2 |  |  | FS |  |  | The "plain text message" is displayed |  |  | O |  | DMI |  |  | - |  | L2 |  |  | FS |  | - |  |
|  | Feature 9990200: A3.2: List of National / Default Data Test Case 18: When an RBC/RBC handover has been announced, the current onboard time shall be compared with the time stamp of the latest consistent message from the Handing over RBC until the train considers the Accepting RBC as the supervising one. From then on the current onboard time shall be compared with the time stamp of the latest received consistent message from the Accepting RBC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 111 |  | 25 |  | 2200.00 |  | L2 |  |  | FS |  |  | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) is received |  |  | I |  | RTM |  |  | A radio message 24( General Message) is received from the Accepting RBC. The difference between the time stamp of this message and the latest message received from the Handing over RBC must be lower than the value of T_NVCONTACT of the Handing over RBC area. This is to check that the EVC does not take into account the messages coming from the Accepting RBC for the supervision of the T_NVCONTACT that corresponds to the Handing over RBC area. |  | L2 |  |  | FS |  | - |  |
|  | 112 |  | 26 |  | 2200.00 |  | L2 |  |  | FS |  |  | The radio message 24 is RECORDED |  |  | O |  | JRU |  |  | - |  | L2 |  |  | FS |  | - |  |
|  | 113 |  | 27 |  | 2300.00 |  | L2 |  |  | FS |  |  | NOT SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) is received |  |  | I |  | RTM |  |  | The train does not receive any message from the Handing over RBC, for a period equal to the value of T_NVCONTACT of the Handing over RBC area. |  | L2 |  |  | FS |  | - |  |
|  | 114 |  | 28 |  | 2300.00 |  | L2 |  |  | FS |  |  | Radio link supervision error is RECORDED. |  |  | O |  | JRU |  |  | - |  | L2 |  |  | FS |  | - |  |
|  | 115 |  | 29 |  | 2300.00 |  | L2 |  |  | FS |  |  | No reaction due to loss of Radio Link |  |  | I |  | INT |  |  | T_NVCONTACT expires à Defined reaction occurs |  | L2 |  |  | FS |  | - |  |
|  | 116 |  | 30 |  | 2300.00 |  | L2 |  |  | FS |  |  | SA-DISCONNECT.Request is transmitted |  |  | O |  | RTM |  |  | The on-board releases the safe connection |  | L2 |  |  | FS |  | - |  |
|  | 117 |  | 31 |  | 2300.00 |  | L2 |  |  | FS |  |  | SA-CONNECT.Request is transmitted |  |  | O |  | RTM |  |  | The on-board tries to set-up the connection again |  | L2 |  |  | FS |  | - |  |
|  | 118 |  | 32 |  | 2300.00 |  | L2 |  |  | FS |  |  | SA-CONNECT.Confirm is received |  |  | I |  | RTM |  |  | The connection is set up |  | L2 |  |  | FS |  | - |  |
|  | 119 |  | 33 |  | 2300.00 |  | L2 |  |  | FS |  |  | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) is received |  |  | I |  | RTM |  |  | A radio message 24 is received from the Handing over RBC with period lower than the value of T_NVCONTACT |  | L2 |  |  | FS |  | - |  |
|  | 120 |  | 34 |  | 2300.00 |  | L2 |  |  | FS |  |  | The radio message 24 is RECORDED |  |  | O |  | JRU |  |  | - |  | L2 |  |  | FS |  | - |  |
|  | 121 |  | 35 |  | 2400.00 |  | L2 |  |  | FS |  |  | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) is received |  |  | I |  | RTM |  |  | A radio message 24( General Message) is received from the Accepting RBC with period lower than the value of T_NVCONTACT of the Accepting RBC area. |  | L2 |  |  | FS |  | - |  |
|  | 122 |  | 36 |  | 2400.00 |  | L2 |  |  | FS |  |  | The radio message 24 is RECORDED |  |  | O |  | JRU |  |  | - |  | L2 |  |  | FS |  | - |  |
|  | 123 |  | 37 |  | 2500.00 |  | L2 |  |  | FS |  |  | SA-DATA.Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containint packet 0 is transmitted |  |  | O |  | RTM |  |  | Train maximum safe front end location = LRBG + D_RBCTR A radio message 136 (train position report) is sent to the Handing Over RBC and the Accepting RBC. The maximum safe front end of the train passes the location of the RBC transition |  | L2 |  |  | FS |  | - |  |
|  | 124 |  | 38 |  | 2500.00 |  | L2 |  |  | FS |  |  | The radio message 136 is RECORDED |  |  | O |  | JRU |  |  | - |  | L2 |  |  | FS |  | - |  |
|  | 125 |  | 39 |  | 2500.00 |  | L2 |  |  | FS |  |  | One Balise Group message (N_TOTAL=0) containing packet 131 (D_RBCTR = 0) is received |  |  | I |  | BTM |  |  | The balise group is placed at the RBC transition border. |  | L2 |  |  | FS |  | - |  |
|  | 126 |  | 40 |  | 2500.00 |  | L2 |  |  | FS |  |  | The balise message with packet 131 is RECORDED |  |  | O |  | JRU |  |  | - |  | L2 |  |  | FS |  | - |  |
|  | Feature 4080425: 4.8.4: Accepted Information depending on the modes - Movement Authority Test Case 3: New Level 2/3 MA is ACCEPTED by the on board equipment because the SSP (Static Speed Profile) and GP (Gradient Profile) sent together with the MA on-board covers the whole length of the MA. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 128 |  | 1 |  | 2500.00 |  | L2 |  |  | FS |  |  | SA-Data.Indication with Euroradio message (NID_MESSAGE=3) containing packet 15 (MA), packet 21 (Gradient), packet 27 (SSP) is received |  |  | I |  | RTM |  |  | Track description sending is optional New track description (or stored one if not sent) covers the movement authority distance |  | L2 |  |  | FS |  | - |  |
|  | 129 |  | 2 |  | 2500.00 |  | L2 |  |  | FS |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  |  | O |  | JRU |  |  | - |  | L2 |  |  | FS |  | - |  |
|  | 130 |  | 3 |  | 2500.00 |  | L2 |  |  | FS |  |  | The speed and distance monitoring information is displayed |  |  | O |  | DMI |  |  | - |  | L2 |  |  | FS |  | - |  |
|  | 131 |  | 4 |  | 2500.00 |  | L2 |  |  | FS |  |  | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded |  |  | O |  | JRU |  |  | - |  | L2 |  |  | FS |  | - |  |
|  | Feature 9990200: A3.2: List of National / Default Data Test Case 18: When an RBC/RBC handover has been announced, the current onboard time shall be compared with the time stamp of the latest consistent message from the Handing over RBC until the train considers the Accepting RBC as the supervising one. From then on the current onboard time shall be compared with the time stamp of the latest received consistent message from the Accepting RBC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 132 |  | 42 |  | 2500.00 |  | L2 |  |  | FS |  |  | The new MA is displayed |  |  | O |  | DMI |  |  | - |  | L2 |  |  | FS |  | - |  |
|  | 133 |  | 54 |  | 2540.00 |  | L2 |  |  | FS |  |  | SA-DATA.Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) is transmitted |  |  | O |  | RTM |  |  | Train min safe rear end location = LRBG + D_RBCTR+L_TRAIN A radio message 136 (train position report) is sent to the Handing Over RBC. The minimum rear end of the train passes the location of the RBC transition |  | L2 |  |  | FS |  | - |  |
|  | 134 |  | 55 |  | 2540.00 |  | L2 |  |  | FS |  |  | The radio message 136 is RECORDED |  |  | O |  | JRU |  |  | - |  | L2 |  |  | FS |  | - |  |
|  | 135 |  | 56 |  | 2540.00 |  | L2 |  |  | FS |  |  | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) is received |  |  | I |  | RTM |  |  | A radio message 24( General Message) is received from the Accepting RBC. Message is received before on-board releases the safe radio connection. |  | L2 |  |  | FS |  | - |  |
|  | 136 |  | 57 |  | 2540.00 |  | L2 |  |  | FS |  |  | The radio message 24 is RECORDED |  |  | O |  | JRU |  |  | - |  | L2 |  |  | FS |  | - |  |
|  | 137 |  | 58 |  | 2550.00 |  | L2 |  |  | FS |  |  | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) is received |  |  | I |  | RTM |  |  | A radio message 24( General Message) is received from the Accepting RBC with period lower than the value of T_NVCONTACT of the Accepting RBC area. |  | L2 |  |  | FS |  | - |  |
|  | 138 |  | 59 |  | 2550.00 |  | L2 |  |  | FS |  |  | The radio message 24 is RECORDED |  |  | O |  | JRU |  |  | - |  | L2 |  |  | FS |  | - |  |
|  | Feature 3050500: 3.5.5: Terminating a communication session Test Case 5: The on-board equipment terminates a communication session ordered by RBC. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 140 |  | 1 |  | 2550.00 |  | L2 |  |  | FS |  |  | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 42 (Q_RBC=0) is received |  |  | I |  | RTM |  |  | Q_RBC = 0 (Terminate communication session) |  | L2 |  |  | FS |  | - |  |
|  | 141 |  | 2 |  | 2550.00 |  | L2 |  |  | FS |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  |  | O |  | JRU |  |  | - |  | L2 |  |  | FS |  | - |  |
|  | 142 |  | 3 |  | 2550.00 |  | L2 |  |  | FS |  |  | SA-DATA.Request with Euroradio Message "Termination of a communication session" (NID_MESSAGE=156) is transmitted |  |  | O |  | RTM |  |  | As soon as this message is received by trackside (RBC), the RBC shall consider the communication session terminated and it shall send the acknowledgement to the on-board. It shall be checked that no more messages are sent by the onboard |  | L2 |  |  | FS |  | - |  |
|  | 143 |  | 4 |  | 2550.00 |  | L2 |  |  | FS |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  |  | O |  | JRU |  |  | - |  | L2 |  |  | FS |  | - |  |
|  | 144 |  | 5 |  | 2550.00 |  | L2 |  |  | FS |  |  | SA-DATA.Indication with Euroradio Message "Acknowledgement of termination of a communication session" (NID_MESSAGE=39) is received |  |  | I |  | RTM |  |  | Message 39: 'Acknowledgement of termination of a communication session' is RECEIVED from the RBC and the on-board equipment considers the communication session terminated. |  | L2 |  |  | FS |  | - |  |
|  | 145 |  | 6 |  | 2550.00 |  | L2 |  |  | FS |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  |  | O |  | JRU |  |  | When the acknowledgement is received, the onboard shall consider the communication session terminated and it shall begin the release of the safe connection with trackside. |  | L2 |  |  | FS |  | - |  |
|  | 146 |  | 7 |  | 2550.00 |  | L2 |  |  | FS |  |  | SA-DISCONNECT.Request is transmitted |  |  | O |  | RTM |  |  | The train tries to release the safe connection. |  | L2 |  |  | FS |  | - |  |
|  | Feature 9990200: A3.2: List of National / Default Data Test Case 18: When an RBC/RBC handover has been announced, the current onboard time shall be compared with the time stamp of the latest consistent message from the Handing over RBC until the train considers the Accepting RBC as the supervising one. From then on the current onboard time shall be compared with the time stamp of the latest received consistent message from the Accepting RBC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 147 |  | 43 |  | 2590.00 |  | L2 |  |  | FS |  |  | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) is received |  |  | I |  | RTM |  |  | A radio message 24( General Message) is received from the Accepting RBC with period lower than the value of T_NVCONTACT of the Accepting RBC area. |  | L2 |  |  | FS |  | - |  |
|  | 148 |  | 44 |  | 2590.00 |  | L2 |  |  | FS |  |  | The radio message 24 is RECORDED |  |  | O |  | JRU |  |  | - |  | L2 |  |  | FS |  | - |  |
|  | 149 |  | 45 |  | 2790.00 |  | L2 |  |  | FS |  |  | NOT SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) is received |  |  | I |  | RTM |  |  | The train does not receive any message from the Handing over RBC, for a period equal to the value of T_NVCONTACT of the Accepting RBC area. This time will be counted from the time of receiving the latest message from the Handing over RBC. This is to check that the current onboard time shall NOT be compared with the time stamp of the latest consistent message from the Handing over RBC |  | L2 |  |  | FS |  | - |  |
|  | 150 |  | 46 |  | 2790.00 |  | L2 |  |  | FS |  |  | No radio link supervision error is recorded. |  |  | O |  | JRU |  |  | - |  | L2 |  |  | FS |  | - |  |
|  | 151 |  | 47 |  | 2790.00 |  | L2 |  |  | FS |  |  | NOT SA-DISCONNECT.Request is transmitted |  |  | O |  | RTM |  |  | The on-board does not release the safe connection |  | L2 |  |  | FS |  | - |  |
|  | 152 |  | 48 |  | 2790.00 |  | L2 |  |  | FS |  |  | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) is received |  |  | I |  | RTM |  |  | A radio message 24( General Message) is received from the Accepting RBC with period lower than the value of T_NVCONTACT of the Accepting RBC area. |  | L2 |  |  | FS |  | - |  |
|  | 153 |  | 49 |  | 2790.00 |  | L2 |  |  | FS |  |  | The radio message 24 is RECORDED |  |  | O |  | JRU |  |  | - |  | L2 |  |  | FS |  | - |  |
|  | 154 |  | 50 |  | 2890.00 |  | L2 |  |  | FS |  |  | NOT SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) is received |  |  | I |  | RTM |  |  | The train does not receive any message from the Handing over RBC, for a period equal to the value of T_NVCONTACT of the Handing over RBC area. This time will be counted from the time of receiving the latest message from the Handing over RBC. This step and the following ones have been added to test that the current onboard time shall only be compared with the time stamp of the latest received consistent message from the Accepting RBC and with the value given by T_NVCONTACT of the Accepting RBC area. The train might need to be stopped in order to make the time defined by T_NVCONTACT expire before reaching the location for diconnection with the Handing over RBC. |  | L2 |  |  | FS |  | - |  |
|  | 155 |  | 51 |  | 2890.00 |  | L2 |  |  | FS |  |  | No radio link supervision error is recorded. |  |  | O |  | JRU |  |  | - |  | L2 |  |  | FS |  | - |  |
|  | 156 |  | 52 |  | 2890.00 |  | L2 |  |  | FS |  |  | NOT SA-DISCONNECT.Request is transmitted |  |  | O |  | RTM |  |  | The on-board does not release the safe connection |  | L2 |  |  | FS |  | - |  |
|  | Feature 9990200: A3.2: List of National / Default Data Test Case 16: Reaction set to APPLY SERVICE BRAKE by National Values. Newer valid message is further received. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 158 |  | 1 |  | 2890.00 |  | L2 |  |  | FS |  |  | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) is received |  |  | I |  | RTM |  |  | RBC sends a general message to the On-board with period lower than T_NVCONTACT. |  | L2 |  |  | FS |  | - |  |
|  | 159 |  | 2 |  | 2890.00 |  | L2 |  |  | FS |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  |  | O |  | JRU |  |  | - |  | L2 |  |  | FS |  | - |  |
|  | 160 |  | 3 |  | 3090.00 |  | L2 |  |  | FS |  |  | NOT SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) is received |  |  | I |  | RTM |  |  | No reception of messages due to errors of radio link The train does not receive any message in the period given by T_NVCONTACT |  | L2 |  |  | FS |  | - |  |
|  | 161 |  | 4 |  | 3090.00 |  | L2 |  |  | FS |  |  | An indication of the loss of radio link to the driver is DISPLAYED. |  |  | O |  | DMI |  |  | - |  | L2 |  |  | FS |  | - |  |
|  | 162 |  | 5 |  | 3090.00 |  | L2 |  |  | FS |  |  | Radio link supervision error is RECORDED. |  |  | O |  | JRU |  |  | - |  | L2 |  |  | FS |  | - |  |
|  | 163 |  | 6 |  | 3090.00 |  | L2 |  |  | FS |  |  | Service Brake is APPLIED due to loss of Radio Link. |  |  | O |  | TIU |  |  | - |  | L2 |  |  | FS |  | - |  |
|  | 164 |  | 7 |  | 3090.00 |  | L2 |  |  | FS |  |  | Service Brake Indication is DISPLAYED. |  |  | O |  | DMI |  |  | - |  | L2 |  |  | FS |  | - |  |
|  | 165 |  | 8 |  | 3090.00 |  | L2 |  |  | FS |  |  | Service Brake application is RECORDED. |  |  | O |  | JRU |  |  | - |  | L2 |  |  | FS |  | - |  |
|  | 166 |  | 9 |  | 3090.00 |  | L2 |  |  | FS |  |  | SA-DISCONNECT.Request is transmitted |  |  | O |  | RTM |  |  | - |  | L2 |  |  | FS |  | - |  |
|  | 167 |  | 10 |  | 3090.00 |  | L2 |  |  | FS |  |  | NOT SA-DISCONNECT.Indication is received |  |  | I |  | RTM |  |  | - |  | L2 |  |  | FS |  | - |  |
|  | 168 |  | 11 |  | 3090.00 |  | L2 |  |  | FS |  |  | SA-DISCONNECT.Request is transmitted |  |  | O |  | RTM |  |  | - |  | L2 |  |  | FS |  | - |  |
|  | 169 |  | 12 |  | 3090.00 |  | L2 |  |  | FS |  |  | NOT SA-DISCONNECT.Indication is received |  |  | I |  | RTM |  |  | - |  | L2 |  |  | FS |  | - |  |
|  | 170 |  | 13 |  | 3090.00 |  | L2 |  |  | FS |  |  | SA-DISCONNECT.Request is transmitted |  |  | O |  | RTM |  |  | - |  | L2 |  |  | FS |  | - |  |
|  | 171 |  | 14 |  | 3090.00 |  | L2 |  |  | FS |  |  | NOT SA-DISCONNECT.Indication is received |  |  | I |  | RTM |  |  | - |  | L2 |  |  | FS |  | - |  |
|  | 172 |  | 15 |  | 3090.00 |  | L2 |  |  | FS |  |  | SA-DISCONNECT.Request is transmitted |  |  | O |  | RTM |  |  | - |  | L2 |  |  | FS |  | - |  |
|  | 173 |  | 16 |  | 3090.00 |  | L2 |  |  | FS |  |  | NOT SA-DISCONNECT.Indication is received |  |  | I |  | RTM |  |  | - |  | L2 |  |  | FS |  | - |  |
|  | 174 |  | 17 |  | 3090.00 |  | L2 |  |  | FS |  |  | SA-CONNECT.Request is transmitted |  |  | O |  | RTM |  |  | - |  | L2 |  |  | FS |  | - |  |
|  | 175 |  | 18 |  | 3090.00 |  | L2 |  |  | FS |  |  | SA-CONNECT.Confirm is received |  |  | I |  | RTM |  |  | - |  | L2 |  |  | FS |  | - |  |
|  | Feature 4080426: 4.8.4: Accepted Information depending on the modes - National Values Test Case 4: Transmission of National Values via balise group |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 176 |  | 1 |  | 3100.00 |  | L2 |  |  | FS |  |  | One Balise Group message containing packet 3 is received |  |  | I |  | BTM |  |  | - |  | L2 |  |  | FS |  | - |  |
|  | 177 |  | 2 |  | 3100.00 |  | L2 |  |  | FS |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  |  | O |  | JRU |  |  | - |  | L2 |  |  | FS |  | - |  |
|  | Feature 9990200: A3.2: List of National / Default Data Test Case 16: Reaction set to APPLY SERVICE BRAKE by National Values. Newer valid message is further received. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 179 |  | 19 |  | 3130.00 |  | L2 |  |  | FS |  |  | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) is received |  |  | I |  | RTM |  |  | Newer message received before standstill |  | L2 |  |  | FS |  | - |  |
|  | 180 |  | 20 |  | 3130.00 |  | L2 |  |  | FS |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  |  | O |  | JRU |  |  | - |  | L2 |  |  | FS |  | - |  |
|  | 181 |  | 21 |  | 3130.00 |  | L2 |  |  | FS |  |  | Service Brake is RELEASED due to reception of newer message |  |  | O |  | TIU |  |  | - |  | L2 |  |  | FS |  | - |  |
|  | 182 |  | 22 |  | 3130.00 |  | L2 |  |  | FS |  |  | Service Brake Indication is REMOVED. |  |  | O |  | DMI |  |  | Indication of the loss of radio link to the driver is REMOVED. |  | L2 |  |  | FS |  | - |  |
|  | 183 |  | 23 |  | 3130.00 |  | L2 |  |  | FS |  |  | Service Brake releasing is RECORDED. |  |  | O |  | JRU |  |  | - |  | L2 |  |  | FS |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 1: The ETCS on-board system is NOT powered. Transition Condition Id [29]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 184 |  | 1 |  | 3250.00 |  | L2 |  |  | FS |  |  | The ERTMS/ETCS on-board unit is powered down |  |  | I |  | INT |  |  | - _____ The train reached standstill. |  | L2 |  |  | NP |  | - |  |
|  | 185 |  | 2 |  | 3250.00 |  | L2 |  |  | NP |  |  | NOT GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=-) is recorded |  |  | O |  | JRU |  |  | There are no messages recorded in the JRU during a period of time |  | L2 |  |  | NP |  | - |  |
| - Step 39 : Balise Group BG 100 d 1/2 - Distance 100.00 m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Variable |  |  |  |  |  |  |  | Length |  |  | Value |  |  | Comment |  |  |  |  | User comment |  |  |  |  |  |  |  |  |  |  |
| Q_UPDOWN |  |  |  |  |  |  |  | 1 |  |  | 1 b |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| M_VERSION |  |  |  |  |  |  |  | 7 |  |  | 0100000 b |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Q_MEDIA |  |  |  |  |  |  |  | 1 |  |  | 0 b |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| N_PIG |  |  |  |  |  |  |  | 3 |  |  | 0 d |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| N_TOTAL |  |  |  |  |  |  |  | 3 |  |  | 1 d |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| M_DUP |  |  |  |  |  |  |  | 2 |  |  | 00 b |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| M_MCOUNT |  |  |  |  |  |  |  | 8 |  |  | 0 d |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NID_C |  |  |  |  |  |  |  | 10 |  |  | 64 d |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NID_BG |  |  |  |  |  |  |  | 14 |  |  | 100 d |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Q_LINK |  |  |  |  |  |  |  | 1 |  |  | 1 b |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NID_PACKET |  |  |  |  |  |  |  | 8 |  |  | 5 d |  |  | Linking |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Q_DIR |  |  |  |  |  |  |  | 2 |  |  | 01 b |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| L_PACKET |  |  |  |  |  |  |  | 13 |  |  | 69 d |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Q_SCALE |  |  |  |  |  |  |  | 2 |  |  | 01 b |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| D_LINK |  |  |  |  |  |  |  | 15 |  |  | 100 d |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Q_NEWCOUNTRY |  |  |  |  |  |  |  | 1 |  |  | 0 b |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NID_BG |  |  |  |  |  |  |  | 14 |  |  | 101 d |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Q_LINKORIENTATION |  |  |  |  |  |  |  | 1 |  |  | 1 b |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Q_LINKREACTION |  |  |  |  |  |  |  | 2 |  |  | 00 b |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Q_LOCACC |  |  |  |  |  |  |  | 6 |  |  | 5 d |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| N_ITER |  |  |  |  |  |  |  | 5 |  |  | 0 d |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NID_PACKET |  |  |  |  |  |  |  | 8 |  |  | 255 d |  |  | End of information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| - Step 39 : Balise Group BG 100 d 2/2 - Distance 105.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 100 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 42 : Balise Group BG 101 d 1/2 - Distance 200.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 101 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 42 : Balise Group BG 101 d 2/2 - Distance 205.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 101 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 45 : Balise Group BG 1 d 1/2 - Distance 300.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 1 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 41 d | Level transition order |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 63 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_LEVELTR | 15 | 0 d |  |  |
| M_LEVELTR | 3 | 3 d |  |  |
| L_ACKLEVELTR | 15 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 42 d | Session management |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 113 d |  |  |
| Q_RBC | 1 | 1 b |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_RBC | 14 | 1515 d |  |  |
| NID_RADIO | 64 | 003265342101FFFF h |  |  |
| Q_SLEEPSESSION | 1 | 0 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 45 : Balise Group BG 1 d 2/2 - Distance 305.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 1 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 3 d | National values |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 230 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_VALIDNV | 15 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| V_NVSHUNT | 7 | 6 d |  |  |
| V_NVSTFF | 7 | 8 d |  |  |
| V_NVONSIGHT | 7 | 6 d |  |  |
| V_NVLIMSUPERV | 7 | 20 d |  |  |
| V_NVUNFIT | 7 | 20 d |  |  |
| V_NVREL | 7 | 8 d |  |  |
| D_NVROLL | 15 | 2 d |  |  |
| Q_NVSBTSMPERM | 1 | 1 d |  |  |
| Q_NVEMRRLS | 1 | 0 b |  |  |
| Q_NVGUIPERM | 1 | 0 b |  |  |
| Q_NVSBFBPERM | 1 | 0 b |  |  |
| Q_NVINHSMICPERM | 1 | 0 b |  |  |
| V_NVALLOWOVTRP | 7 | 0 d |  |  |
| V_NVSUPOVTRP | 7 | 6 d |  |  |
| D_NVOVTRP | 15 | 200 d |  |  |
| T_NVOVTRP | 8 | 60 d |  |  |
| D_NVPOTRP | 15 | 200 d |  |  |
| M_NVCONTACT | 2 | 10 b |  |  |
| T_NVCONTACT | 8 | 30 d |  |  |
| M_NVDERUN | 1 | 1 b |  |  |
| D_NVSTFF | 15 | 32767 d |  |  |
| Q_NVDRIVER_ADHES | 1 | 0 b |  |  |
| A_NVMAXREDADH1 | 6 | 010100 b |  |  |
| A_NVMAXREDADH2 | 6 | 001110 b |  |  |
| A_NVMAXREDADH3 | 6 | 001110 b |  |  |
| Q_NVLOCACC | 6 | 12 d |  |  |
| M_NVAVADH | 5 | 1 d |  |  |
| M_NVEBCL | 4 | 0001 b |  |  |
| Q_NVKINT | 1 | 0 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 58 : Message 155 - Initiation of a Communication Session (RBC n? 1) - Distance 300.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 155 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |

| - Step 60 : Message 32 - RBC/RIU System Version (RBC n? 1) - Distance 300.00 m Delay : 5.00 s Conditional Message Id : 155 Back Delay : 5.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 32 d |  |  |
| L_MESSAGE | 10 | 11 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 16777215 d |  |  |
| M_VERSION | 7 | 0100000 b |  |  |

| - Step 62 : Message 159 - Session Established (RBC n? 1) - Distance 300.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 159 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |

| - Step 65 : Message 129 - Validated Train Data (RBC n? 1) - Distance 300.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 129 d |  |  |
| L_MESSAGE | 10 | 38 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 114 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 1048577 d |  |  |
| D_LRBG | 15 | 0 d |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 0 d |  |  |
| L_DOUBTUNDER | 15 | 0 d |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 7 d |  |  |
| Q_DIRTRAIN | 2 | 01 b |  |  |
| M_MODE | 4 | 0010 b |  |  |
| M_LEVEL | 3 | 3 d |  |  |
| NID_PACKET | 8 | 11 d | Validated train data |  |
| L_PACKET | 13 | 110 d |  |  |
| NC_CDTRAIN | 4 | 7 d |  |  |
| NC_TRAIN | 15 | 4 d |  |  |
| L_TRAIN | 12 | 40 d |  |  |
| V_MAXTRAIN | 7 | 80 d |  |  |
| M_LOADINGGAUGE | 8 | 00000001 b |  |  |
| M_AXLELOADCAT | 7 | 7 d |  |  |
| M_AIRTIGHT | 2 | 00 b |  |  |
| N_AXLE | 10 | 4 d |  |  |
| N_ITER | 5 | 1 d |  |  |
| M_VOLTAGE | 4 | 3 d |  |  |
| NID_CTRACTION | 10 | 15 d |  |  |
| N_ITER | 5 | 0 d |  |  |

| - Step 67 : Message 8 - Acknowledgement of Train Data (RBC n? 1) - Distance 300.00 m Delay : 5.00 s Conditional Message Id : 129 Back Delay : 5.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 8 d |  |  |
| L_MESSAGE | 10 | 14 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048577 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |

| - Step 69 : Message 3 - Movement Authority (RBC n? 1) - Distance 550.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 3 d |  |  |
| L_MESSAGE | 10 | 32 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048577 d |  |  |
| NID_PACKET | 8 | 15 d | Level 2/3 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 66 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| V_LOA | 7 | 0 d |  |  |
| T_LOA | 10 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_ENDSECTION | 15 | 5000 d |  |  |
| Q_SECTIONTIMER | 1 | 0 b |  |  |
| Q_ENDTIMER | 1 | 0 b |  |  |
| Q_DANGERPOINT | 1 | 0 b |  |  |
| Q_OVERLAP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 21 d | Gradient profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 54 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_GRADIENT | 15 | 0 d |  |  |
| Q_GDIR | 1 | 0 b |  |  |
| G_A | 8 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 27 d | International static speed profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 58 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_STATIC | 15 | 0 d |  |  |
| V_STATIC | 7 | 20 d |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |

| - Step 74 : Message 24 - General Message (RBC n? 1) - Distance 775.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 24 d |  |  |
| L_MESSAGE | 10 | 19 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048577 d |  |  |
| NID_PACKET | 8 | 65 d | Temporary speed restriction |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 71 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_TSR | 8 | 100 d |  |  |
| D_TSR | 15 | 500 d |  |  |
| L_TSR | 15 | 200 d |  |  |
| Q_FRONT | 1 | 1 b |  |  |
| V_TSR | 7 | 10 d |  |  |

| - Step 84 : Message 3 - Movement Authority (RBC n? 1) - Distance 1300.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 3 d |  |  |
| L_MESSAGE | 10 | 34 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048577 d |  |  |
| NID_PACKET | 8 | 15 d | Level 2/3 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 66 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| V_LOA | 7 | 0 d |  |  |
| T_LOA | 10 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_ENDSECTION | 15 | 5000 d |  |  |
| Q_SECTIONTIMER | 1 | 0 b |  |  |
| Q_ENDTIMER | 1 | 0 b |  |  |
| Q_DANGERPOINT | 1 | 0 b |  |  |
| Q_OVERLAP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 131 d | RBC transition order |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 129 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_RBCTR | 15 | 2200 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_RBC | 14 | 1616 d |  |  |
| NID_RADIO | 64 | 003265342102FFFF h |  |  |
| Q_SLEEPSESSION | 1 | 0 b |  |  |

| - Step 88 : Message 155 - Initiation of a Communication Session (RBC n? 2) - Distance 1300.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 155 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |

| - Step 90 : Message 32 - RBC/RIU System Version (RBC n? 2) - Distance 1300.00 m Delay : 5.00 s Conditional Message Id : 155 Back Delay : 5.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 32 d |  |  |
| L_MESSAGE | 10 | 11 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 16777215 d |  |  |
| M_VERSION | 7 | 0100000 b |  |  |

| - Step 92 : Message 159 - Session Established (RBC n? 2) - Distance 1300.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 159 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |

| - Step 94 : Message 129 - Validated Train Data (RBC n? 2) - Distance 1300.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 129 d |  |  |
| L_MESSAGE | 10 | 38 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 114 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 1048577 d |  |  |
| D_LRBG | 15 | 1000 d |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 0 d |  |  |
| L_DOUBTUNDER | 15 | 0 d |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 10 d |  |  |
| Q_DIRTRAIN | 2 | 01 b |  |  |
| M_MODE | 4 | 0000 b |  |  |
| M_LEVEL | 3 | 3 d |  |  |
| NID_PACKET | 8 | 11 d | Validated train data |  |
| L_PACKET | 13 | 110 d |  |  |
| NC_CDTRAIN | 4 | 7 d |  |  |
| NC_TRAIN | 15 | 4 d |  |  |
| L_TRAIN | 12 | 40 d |  |  |
| V_MAXTRAIN | 7 | 80 d |  |  |
| M_LOADINGGAUGE | 8 | 00000001 b |  |  |
| M_AXLELOADCAT | 7 | 7 d |  |  |
| M_AIRTIGHT | 2 | 00 b |  |  |
| N_AXLE | 10 | 4 d |  |  |
| N_ITER | 5 | 1 d |  |  |
| M_VOLTAGE | 4 | 3 d |  |  |
| NID_CTRACTION | 10 | 15 d |  |  |
| N_ITER | 5 | 0 d |  |  |

| - Step 96 : Message 8 - Acknowledgement of Train Data (RBC n? 2) - Distance 1300.00 m Delay : 5.00 s Conditional Message Id : 129 Back Delay : 5.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 8 d |  |  |
| L_MESSAGE | 10 | 14 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048577 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |

| - Step 98 : Message 24 - General Message (RBC n? 2) - Distance 1550.00 m Delay : 3.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 24 d |  |  |
| L_MESSAGE | 10 | 39 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048577 d |  |  |
| NID_PACKET | 8 | 3 d | National values |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 230 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_VALIDNV | 15 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| V_NVSHUNT | 7 | 6 d |  |  |
| V_NVSTFF | 7 | 8 d |  |  |
| V_NVONSIGHT | 7 | 6 d |  |  |
| V_NVLIMSUPERV | 7 | 20 d |  |  |
| V_NVUNFIT | 7 | 20 d |  |  |
| V_NVREL | 7 | 8 d |  |  |
| D_NVROLL | 15 | 2 d |  |  |
| Q_NVSBTSMPERM | 1 | 1 b |  |  |
| Q_NVEMRRLS | 1 | 0 b |  |  |
| Q_NVGUIPERM | 1 | 0 b |  |  |
| Q_NVSBFBPERM | 1 | 0 b |  |  |
| Q_NVINHSMICPERM | 1 | 0 b |  |  |
| V_NVALLOWOVTRP | 7 | 0 d |  |  |
| V_NVSUPOVTRP | 7 | 6 d |  |  |
| D_NVOVTRP | 15 | 200 d |  |  |
| T_NVOVTRP | 8 | 60 d |  |  |
| D_NVPOTRP | 15 | 200 d |  |  |
| M_NVCONTACT | 2 | 01 b |  |  |
| T_NVCONTACT | 8 | 20 d |  |  |
| M_NVDERUN | 1 | 1 b |  |  |
| D_NVSTFF | 15 | 32767 d |  |  |
| Q_NVDRIVER_ADHES | 1 | 0 b |  |  |
| A_NVMAXREDADH1 | 6 | 010100 b |  |  |
| A_NVMAXREDADH2 | 6 | 001110 b |  |  |
| A_NVMAXREDADH3 | 6 | 001110 b |  |  |
| Q_NVLOCACC | 6 | 12 d |  |  |
| M_NVAVADH | 5 | 00001 b |  |  |
| M_NVEBCL | 4 | 0001 b |  |  |
| Q_NVKINT | 1 | 0 b |  |  |

| - Step 100 : Message 24 - General Message (RBC n? 2) - Distance 1650.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 24 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048577 d |  |  |

| - Step 108 : Message 24 - General Message (RBC n? 1) - Distance 2000.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 24 d |  |  |
| L_MESSAGE | 10 | 55 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048577 d |  |  |
| NID_PACKET | 8 | 72 d | Packet for sending plain text messages |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 364 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| Q_TEXTCLASS | 2 | 00 b |  |  |
| Q_TEXTDISPLAY | 1 | 0 b |  |  |
| D_TEXTDISPLAY | 15 | 32767 d |  |  |
| M_MODETEXTDISPLAY | 4 | 15 d |  |  |
| M_LEVELTEXTDISPLAY | 3 | 101 b |  |  |
| L_TEXTDISPLAY | 15 | 32767 d |  |  |
| T_TEXTDISPLAY | 10 | 5 d |  |  |
| M_MODETEXTDISPLAY | 4 | 15 d |  |  |
| M_LEVELTEXTDISPLAY | 3 | 101 b |  |  |
| Q_TEXTCONFIRM | 2 | 00 b |  |  |
| L_TEXT | 8 | 34 d |  |  |
| X_TEXT | 8 | M |  |  |
| X_TEXT | 8 | e |  |  |
| X_TEXT | 8 | s |  |  |
| X_TEXT | 8 | s |  |  |
| X_TEXT | 8 | a |  |  |
| X_TEXT | 8 | g |  |  |
| X_TEXT | 8 | e |  |  |
| X_TEXT | 8 |  |  |  |
| X_TEXT | 8 | f |  |  |
| X_TEXT | 8 | r |  |  |
| X_TEXT | 8 | o |  |  |
| X_TEXT | 8 | m |  |  |
| X_TEXT | 8 |  |  |  |
| X_TEXT | 8 | t |  |  |
| X_TEXT | 8 | h |  |  |
| X_TEXT | 8 | e |  |  |
| X_TEXT | 8 |  |  |  |
| X_TEXT | 8 | H |  |  |
| X_TEXT | 8 | a |  |  |
| X_TEXT | 8 | n |  |  |
| X_TEXT | 8 | d |  |  |
| X_TEXT | 8 | i |  |  |
| X_TEXT | 8 | n |  |  |
| X_TEXT | 8 | g |  |  |
| X_TEXT | 8 |  |  |  |
| X_TEXT | 8 | o |  |  |
| X_TEXT | 8 | v |  |  |
| X_TEXT | 8 | e |  |  |
| X_TEXT | 8 | r |  |  |
| X_TEXT | 8 |  |  |  |
| X_TEXT | 8 | R |  |  |
| X_TEXT | 8 | B |  |  |
| X_TEXT | 8 | C |  |  |
| X_TEXT | 8 | . |  |  |

| - Step 111 : Message 24 - General Message (RBC n? 2) - Distance 2200.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 24 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048577 d |  |  |

| - Step 119 : Message 24 - General Message (RBC n? 1) - Distance 2300.00 m Delay : 3.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 24 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048577 d |  |  |

| - Step 121 : Message 24 - General Message (RBC n? 2) - Distance 2400.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 24 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048577 d |  |  |

| - Step 123 : Message 136 - Train Position Report (RBC n? 1) - Distance 2500.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 136 d |  |  |
| L_MESSAGE | 10 | 24 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 114 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 1048577 d |  |  |
| D_LRBG | 15 | 2200 d |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 0 d |  |  |
| L_DOUBTUNDER | 15 | 0 d |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 7 d |  |  |
| Q_DIRTRAIN | 2 | 01 b |  |  |
| M_MODE | 4 | 0000 b |  |  |
| M_LEVEL | 3 | 3 d |  |  |

| - Step 125 : Balise Group BG 02 d 1/1 - Distance 2500.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 0 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 02 d |  |  |
| Q_LINK | 1 | 0 b |  |  |
| NID_PACKET | 8 | 131 d | RBC transition order |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 129 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_RBCTR | 15 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_RBC | 14 | 1616 d |  |  |
| NID_RADIO | 64 | 003265342102FFFF h |  |  |
| Q_SLEEPSESSION | 1 | 0 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 128 : Message 3 - Movement Authority (RBC n? 2) - Distance 2500.00 m Delay : 2.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 3 d |  |  |
| L_MESSAGE | 10 | 32 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048577 d |  |  |
| NID_PACKET | 8 | 15 d | Level 2/3 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 66 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| V_LOA | 7 | 0 d |  |  |
| T_LOA | 10 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_ENDSECTION | 15 | 7000 d |  |  |
| Q_SECTIONTIMER | 1 | 0 b |  |  |
| Q_ENDTIMER | 1 | 0 b |  |  |
| Q_DANGERPOINT | 1 | 0 b |  |  |
| Q_OVERLAP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 21 d | Gradient profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 54 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_GRADIENT | 15 | 0 d |  |  |
| Q_GDIR | 1 | 0 b |  |  |
| G_A | 8 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 27 d | International static speed profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 58 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_STATIC | 15 | 0 d |  |  |
| V_STATIC | 7 | 40 d |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |

| - Step 133 : Message 136 - Train Position Report (RBC n? 1) - Distance 2540.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 136 d |  |  |
| L_MESSAGE | 10 | 24 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 114 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 1048577 d |  |  |
| D_LRBG | 15 | 2240 d |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 0 d |  |  |
| L_DOUBTUNDER | 15 | 0 d |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 7 d |  |  |
| Q_DIRTRAIN | 2 | 01 b |  |  |
| M_MODE | 4 | 0000 b |  |  |
| M_LEVEL | 3 | 3 d |  |  |

| - Step 135 : Message 24 - General Message (RBC n? 2) - Distance 2540.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 24 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048577 d |  |  |

| - Step 137 : Message 24 - General Message (RBC n? 2) - Distance 2550.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 24 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048577 d |  |  |

| - Step 140 : Message 24 - General Message (RBC n? 1) - Distance 2550.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 24 d |  |  |
| L_MESSAGE | 10 | 24 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048577 d |  |  |
| NID_PACKET | 8 | 42 d | Session management |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 113 d |  |  |
| Q_RBC | 1 | 0 b |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_RBC | 14 | 1515 d |  |  |
| NID_RADIO | 64 | 003265342101FFFF h |  |  |
| Q_SLEEPSESSION | 1 | 0 b |  |  |

| - Step 142 : Message 156 - Termination of a Communication Session (RBC n? 1) - Distance 2550.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 156 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |

| - Step 144 : Message 39 - Acknowledgement of Termination of Communication Session (RBC n? 1) - Distance 2550.00 m Delay : 5.00 s Conditional Message Id : 156 Back Delay : 5.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 39 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048577 d |  |  |

| - Step 147 : Message 24 - General Message (RBC n? 2) - Distance 2590.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 24 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048577 d |  |  |

| - Step 152 : Message 24 - General Message (RBC n? 2) - Distance 2790.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 24 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048577 d |  |  |

| - Step 158 : Message 24 - General Message (RBC n? 2) - Distance 2890.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 24 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048577 d |  |  |

| - Step 176 : Balise Group BG 99 d 1/2 - Distance 3100.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 99 d |  |  |
| Q_LINK | 1 | 0 b |  |  |
| NID_PACKET | 8 | 3 d | National values |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 230 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_VALIDNV | 15 | 0 d |  |  |
| NID_C | 10 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| V_NVSHUNT | 7 | 6 d |  |  |
| V_NVSTFF | 7 | 8 d |  |  |
| V_NVONSIGHT | 7 | 6 d |  |  |
| V_NVLIMSUPERV | 7 | 20 d |  |  |
| V_NVUNFIT | 7 | 20 d |  |  |
| V_NVREL | 7 | 8 d |  |  |
| D_NVROLL | 15 | 2 d |  |  |
| Q_NVSBTSMPERM | 1 | 1 d |  |  |
| Q_NVEMRRLS | 1 | 0 b |  |  |
| Q_NVGUIPERM | 1 | 0 b |  |  |
| Q_NVSBFBPERM | 1 | 0 b |  |  |
| Q_NVINHSMICPERM | 1 | 0 b |  |  |
| V_NVALLOWOVTRP | 7 | 0 d |  |  |
| V_NVSUPOVTRP | 7 | 6 d |  |  |
| D_NVOVTRP | 15 | 200 d |  |  |
| T_NVOVTRP | 8 | 60 d |  |  |
| D_NVPOTRP | 15 | 200 d |  |  |
| M_NVCONTACT | 2 | 10 b |  |  |
| T_NVCONTACT | 8 | 255 d |  |  |
| M_NVDERUN | 1 | 1 b |  |  |
| D_NVSTFF | 15 | 32767 d |  |  |
| Q_NVDRIVER_ADHES | 1 | 0 b |  |  |
| A_NVMAXREDADH1 | 6 | 010100 b |  |  |
| A_NVMAXREDADH2 | 6 | 001110 b |  |  |
| A_NVMAXREDADH3 | 6 | 001110 b |  |  |
| Q_NVLOCACC | 6 | 12 d |  |  |
| M_NVAVADH | 5 | 0 d |  |  |
| M_NVEBCL | 4 | 1001 b |  |  |
| Q_NVKINT | 1 | 0 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 176 : Balise Group BG 99 d 2/2 - Distance 3105.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 99 d |  |  |
| Q_LINK | 1 | 0 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 179 : Message 24 - General Message (RBC n? 2) - Distance 3130.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 24 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048577 d |  |  |