| - ERTMS/ETCS |  |
| --- | --- |
| Subset-076-6-3_9990500_02_v310.mdb State : Ordered list of Test Case Steps with communication information |  |
| REF : | Subset-076-6-3_9990500_02_v310.mdb |
| ISSUE : | 3.1.0 |
| DATE : | 19/04/17 14:31 |

| Company | Technical Approval | Management approval |
| --- | --- | --- |
| European Union Agency for Railways | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 3.1.0 19.04.2017 | All | Renamed from Subset-076-6-3_9990500_02_L3 to Subset-076-6-3_9990500_02 | RINA |

# Sequence description

TC06

# Test cases

Important:	All columns called ‘Test Results’ are only placeholders for results of the test execution. They shall not be used for the description of test cases. 
A test is passed if all cells of ‘Test Result’ are passed.

|  | SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Step | Row |  | Dist. (m) | Previous |  |  | Description of Events | I/O | Interface | Comments |  | Next |  |  | Test Result |
|  |  |  |  |  | Levels | Mode |  |  |  |  |  |  | Levels |  | Mode |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 10: The ETCS on-board equipment is powered .Transition Condition Id [4]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | 1 |  | 0.00 | N/A | NP |  | ERTMS/ETCS on-board unit is powered up | I | INT | The on-board equipment changes to SB mode. |  | N/A |  | NP | - |
|  | 2 | 2 |  | 0.00 | N/A | NP |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=6) is recorded | O | JRU | - |  | N/A |  | NP | - |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 24: Start of Mission procedure is engaged |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 3 | 1 |  | 0.00 | N/A | NP |  | Driver opens desk | I | TIU | - |  | N/A |  | NP | - |
|  | 4 | 2 |  | 0.00 | N/A | NP |  | The 'CAB STATUS' message is RECORDED with the variable M_CAB_A_STATUS=1 (Active). | O | JRU | - |  | N/A |  | SB | - |
|  | 5 | 3 |  | 0.00 | N/A | SB |  | The actual mode SB is DISPLAYED | O | DMI | The SB indication after opening the desk may not be recorded. Because the change of mode was already recorded while desk was closed |  | N/A |  | SB | - |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 7: The on-board equipment requests the driver to enter a driver ID within the scope of the Start of Mission procedure in SB mode when the status of the driver ID is unknown. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 6 | 1 |  | 0.00 | N/A | SB |  | The on-board equipment requires the driver to enter a Driver ID There is NO initial Driver ID value displayed | O | DMI | Starting conditions : There is no value initially displayed since the status of the driver ID is unknown. |  | N/A |  | SB | - |
|  | 7 | 2 |  | 0.00 | N/A | SB |  | The on-board equipment prevents the driver from continuing the procedure before the validation of the Driver ID is done. | O | DMI | - |  | N/A |  | SB | - |
|  | 8 | 3 |  | 0.00 | N/A | SB |  | The driver enters and validates a correct Driver ID | I | DMI | - |  | N/A |  | SB | - |
|  | 9 | 4 |  | 0.00 | N/A | SB |  | The validated Driver ID is recorded on the JRU with the DRIVER_ID variable in a 'GENERAL MESSAGE'. | O | JRU | A 'GENERAL MESSAGE' is recorded every 5s by the on-board equipment. |  | N/A |  | SB | - |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 10: Within the scope of the Start of Mission procedure in SB mode, the status of the ERTMS/ETCS Level data is INVALID. The driver selects a level (the existing ERTMS/ETCS Level or a new one). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 10 | 1 |  | 0.00 | N/A | SB |  | The driver selects "Level <L0/LNTC/L1/L2/L3>" | I | DMI | Depending on the implementation, a list of available levels may be presented to the driver on the DMI. The driver is then allowed to select one level among the proposed levels. For NTC levels, the abbreviations of the concerned National Systems shall be presented to the driver. _____ [59.-1.1] |  | L1 |  | SB | - |
|  | 11 | 2 |  | 0.00 | L1 | SB |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=<34/35/36/37/38>) is recorded | O | JRU | - |  | L1 |  | SB | - |
|  | 12 | 3 |  | 0.00 | L1 | SB |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=<0/1/2/3/4>) is recorded | O | JRU | - |  | L1 |  | SB | - |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 6: Within the scope of the Start of Mission procedure in SB mode, the On-Board equipment offers the driver to select either SH or NL or ‘Train Data Entry’ before the on-board equipment requests the driver to enter a complete set of Train Data. The driver selects ‘Train Data Entry’. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 13 | 1 |  | 0.00 | L1 | SB |  | The driver can select either SH or NL or ‘Train Data Entry’ | O | DMI | Starting conditions The 'Non-Leading' button is enabled if the 'non leading' input signal is received. |  | L1 |  | SB | - |
|  | 14 | 2 |  | 0.00 | L1 | SB |  | The driver selects ‘Train Data Entry’ | I | DMI | - |  | L1 |  | SB | - |
|  | 15 | 3 |  | 0.00 | L1 | SB |  | A Train data window is is displayed on the DMI. | O | DMI | - |  | L1 |  | SB | - |
|  | 16 | 4 |  | 0.00 | L1 | SB |  | The driver’s selection is recorded on the JRU. | O | JRU | - |  | L1 |  | SB | - |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 47: The on-board equipment requests the driver to enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when the status of the Train Data is unknown (S12) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 18 | 1 |  | 0.00 | L1 | SB |  | The on-board equipment requires the driver to enter all Train Data unknown by the on-board equipment and prevents the driver from performing any other actions. | O | DMI | The driver might be requested to enter the 'Train category(ies)', 'Train length', 'Traction / brake parameters', 'Maximum train speed', 'Loading gauge', 'Axle load category', 'Train fitted with airtight system', but the driver shall never be requested to enter the 'Traction system(s) accepted by the engine', 'List of National Systems available on-board' or 'Axle number'. |  | L1 |  | SB | - |
|  | 19 | 2 |  | 0.00 | L1 | SB |  | The driver enters each Train data unknown on-board | I | DMI | - |  | L1 |  | SB | - |
|  | 20 | 3 |  | 0.00 | L1 | SB |  | The driver is allowed to validate the train data | O | DMI | A complete set of valid Train Data is available |  | L1 |  | SB | - |
|  | 21 | 4 |  | 0.00 | L1 | SB |  | The driver validates the train data. | I | DMI | The on-board equipment allows the driver to validate as soon as a valid and complete set of Train Data is entered. |  | L1 |  | SB | - |
|  | 22 | 5 |  | 0.00 | L1 | SB |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | Triggered by : The driver validation. |  | L1 |  | SB | - |
|  | 23 | 6 |  | 0.00 | L1 | SB |  | TRAIN DATA (NID_MESSAGE_JRU=2) is recorded | O | JRU | Triggered by : Data Entry completed |  | L1 |  | SB | - |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 46: After the set of Train Data has been entered/re-validated by the driver (in S12), if the status of the Train running number is 'unknown', the ERTMS/ETCS on-board equipment requests the driver to enter the Train running number (S13) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 25 | 1 |  | 0.00 | L1 | SB |  | The driver is requested to enter the Train running number. | O | DMI | - |  | L1 |  | SB | - |
|  | 26 | 2 |  | 0.00 | L1 | SB |  | The driver enters the "Train Running Number" [MAIN] | I | DMI | Entered Train Running Number: NID_OPERATIONAL_1 |  | L1 |  | SB | - |
|  | 27 | 3 |  | 0.00 | L1 | SB |  | ADDITIONAL DATA (NID_MESSAGE_JRU=24; NID_OPERATIONAL=NID_OPERATIONAL_1) is recorded | O | JRU | - |  | L1 |  | SB | - |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 1: The on-board equipment requests the driver to re-validate/enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when level is 0/1/NTC (Subset-026-5.4.3.2 S12). The driver is requested to enter/re-validate the Train running number if the status of the Train running number is 'unknown' or 'invalid'. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 28 | 3 |  | 0.00 | L1 | SB |  | The DMI allows the driver to select the option ‘START’. | O | DMI | - |  | L1 |  | SB | - |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 16: The driver selects 'Start' within the scope of the Start of Mission procedure in SB mode and in ERTMS Level 1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 29 | 1 |  | 0.00 | L1 | SB |  | The driver selects 'Start'. | I | DMI | - |  | L1 |  | SB | - |
|  | 30 | 2 |  | 0.00 | L1 | SB |  | Driver’s selection of 'Start' is recorded on the JRU | O | JRU | Triggered by: The driver acts on the on-board DMI. |  | L1 |  | SB | - |
|  | 31 | 3 |  | 0.00 | L1 | SB |  | An acknowledgement request for running in Staff Responsible mode is displayed on the DMI | O | DMI | The on-board equipment will then waits for the driver to acknowledge the acknowledgement request for running in Staff Responsible mode |  | L1 |  | SB | - |
|  | 32 | 4 |  | 0.00 | L1 | SB |  | The acknowledgement request for running in Staff Responsible mode is recorded | O | JRU | - |  | L1 |  | SB | - |
|  | 33 | 5 |  | 0.00 | L1 | SB |  | The driver acknowledges 'Staff Responsible' | I | DMI | Symbol MO10 |  | L1 |  | SB | - |
|  | 34 | 6 |  | 0.00 | L1 | SB |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS= 0000 0011) is recorded | O | JRU | - |  | L1 |  | SB | - |
|  | 35 | 7 |  | 0.00 | L1 | SB |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=2) is recorded | O | JRU | - |  | L1 |  | SR | - |
|  | 36 | 8 |  | 0.00 | L1 | SR |  | The Mode symbol 'Staff Responsible' is displayed | O | DMI | - |  | L1 |  | SR | - |
|  | 37 | 9 |  | 0.00 | L1 | SR |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit24=1>) is recorded | O | JRU | - |  | L1 |  | SR | - |
|  | 38 | 10 |  | 0.00 | L1 | SR |  | Any message (NID_MESSAGE_JRU=ALL; NID_LRBG=16777215) is recorded | O | JRU | - |  | L1 |  | SR | - |
|  | Feature 5100100: 5.10.1: Level Transitions General requirements Test Case 14: The train receives from the balise group BGa the Level Transition announcement LX/LY. The driver is informed. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 39 | 1 |  | 80.00 | L1 | SR |  | An announcement of level transition to Level Y (Packet 41) is RECEIVED from Balise Group BGa: D_LEVELTR <> 0 and <> 32767 M_LEVELTR= M_LEVEL_Y (Level Y) | I | BTM | Order to switch to level Y at further location and this announcement will result in a change of the on-board level. |  | L1 |  | SR | - |
|  | 40 | 2 |  | 80.00 | L1 | SR |  | An indication of Announcement of Level Y is DISPLAYED. | O | DMI | ('Except in SL and SB modes') |  | L1 |  | SR | - |
|  | 41 | 3 |  | 80.00 | L1 | SR |  | Displayed indication is recorded | O | JRU | ('Except in SL and SB modes') |  | L1 |  | SR | - |
|  | 42 | 4 |  | 80.00 | L1 | SR |  | Balise Group BGa message, including Packet 41'Level transition order', is RECORDED. | O | JRU | - |  | L1 |  | SR | - |
|  | Feature 9990500: A3.5: Handling of Actions in Specific Situations Test Case 9: The on-board equipment shall ensure that a level transition order is executed once, and it is not reverted when moving backwards and passing over the defined location again. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 43 | 1 |  | 130.00 | L1 | SR |  | The train passes over the location where the stored level transition order has to be executed. | I | INT | - |  | L1 |  | SR | - |
|  | 44 | 2 |  | 130.00 | L1 | SR |  | The train changes from Level X to Level Y. The new Level Y is displayed | O | DMI | Levels X and Y will depend on the actual test. |  | L2 |  | SR | - |
|  | 45 | 3 |  | 130.00 | L2 | SR |  | The new Level Y is recorded. | O | JRU | - |  | L2 |  | SR | - |
|  | 46 | 4 |  | 160.00 | L2 | SR |  | The driver stops the train. | I | INT | The train will have to be stopped very close to the location defined by the stored value of D_LEVELTR, since it will pass over this location in backwards direction. Backwards movements are limited by the variable D_NVROLL. |  | L2 |  | SR | - |
|  | 47 | 5 |  | 130.00 | L2 | SR |  | The train moves backwards and overpasses the location of the already executed level transition order. | I | INT | - |  | L2 |  | SR | - |
|  | 48 | 6 |  | 130.00 | L2 | SR |  | The level is not reverted. The current level is still Level Y. | O | DMI | - |  | L2 |  | SR | - |
|  | 49 | 7 |  | 130.00 | L2 | SR |  | No level change is recorded. | O | JRU | - |  | L2 |  | SR | - |
|  | 50 | 8 |  | 100.00 | L2 | SR |  | The train is stopped. | I | INT | The brakes might have been applied. |  | L2 |  | SR | - |
|  | Feature 3050300: 3.5.3: Establishing a communication session Test Case 22: System version is compatible and session is established. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 51 | 1 |  | 150.00 | L2 | SR |  | One Balise Group message (NID_RBC?16383) containing packet 42 (NID_RADIO ? FFFF FFFF FFFF FFFF) is received | I | BTM | Not use last known RBC and don’t use short number. |  | L2 |  | SR | - |
|  | 52 | 2 |  | 150.00 | L2 | SR |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - |  | L2 |  | SR | - |
|  | 53 | 3 |  | 150.00 | L2 | SR |  | SA-CONNECT.Request is transmitted | O | RTM | According to EURORADIO specifications. This attempt shall be repeated until successful. The onboard tries to set up a connection with the phone number included in the session management packet. |  | L2 |  | SR | - |
|  | 54 | 4 |  | 150.00 | L2 | SR |  | SA-CONNECT.Confirm is received | I | RTM | - |  | L2 |  | SR | - |
|  | 55 | 5 |  | 150.00 | L2 | SR |  | The Status symbol "Safe radio connection - Connection Up" is displayed | O | DMI | - |  | L2 |  | SR | - |
|  | 56 | 6 |  | 150.00 | L2 | SR |  | JRU step 'DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit40=1>) is recorded'. | O | JRU | - |  | L2 |  | SR | - |
|  | 57 | 7 |  | 150.00 | L2 | SR |  | SA-DATA.Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted | O | RTM | - |  | L2 |  | SR | - |
|  | 58 | 8 |  | 150.00 | L2 | SR |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - |  | L2 |  | SR | - |
|  | 59 | 9 |  | 150.00 | L2 | SR |  | SA-DATA.Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32) is received | I | RTM | System versions M_VERSION (Version of the ERTMS/ETCS language) are compatible, so that the communication session established for on-board |  | L2 |  | SR | - |
|  | 60 | 10 |  | 150.00 | L2 | SR |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - |  | L2 |  | SR | - |
|  | 61 | 11 |  | 150.00 | L2 | SR |  | SA-DATA.Request with Euroradio Message "Session Established" (NID_MESSAGE=159) is transmitted | O | RTM | - |  | L2 |  | SR | - |
|  | 62 | 12 |  | 150.00 | L2 | SR |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - |  | L2 |  | SR | - |
|  | Feature 4080401: 4.8.4: Accepted Information depending on the modes - Acknowledgement of train data Test Case 1: Radio message with new Train Data and acknowledge of the RBC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 64 | 1 |  | 150.00 | L2 | SR |  | SA-DATA.Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) is transmitted | O | RTM | Validated train data |  | L2 |  | SR | - |
|  | 65 | 2 |  | 150.00 | L2 | SR |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=129) is recorded | O | JRU | - |  | L2 |  | SR | - |
|  | 66 | 3 |  | 150.00 | L2 | SR |  | SA-DATA.Indication with Euroradio Message "Acknowledgement of Train Data" (NID_MESSAGE=8) is received | I | RTM | - |  | L2 |  | SR | - |
|  | 67 | 4 |  | 150.00 | L2 | SR |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=8) is recorded | O | JRU | - |  | L2 |  | SR | - |
|  | Feature 3060100: 3.6.1: Location Principles and Train Position and Train Orientation Test Case 5: An error in the radio messages sequence has been detected. The onboard equipment sends a radio message 136 'Train Position Report' containing the packet 4 'Error reporting' to the RBC. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 70 | 3 |  | 350.00 | L2 | SR |  | SA-DATA.Indication with Euroradio Message "Track Ahead Free Request" (NID_MESSAGE=34) is received | I | RTM | A message 34 is received from the RBC with T_TRAIN 2; The time stamp of this message is smaller than the time stamp of the first one: T_TRAIN 2<T_TRAIN_1 _____ T_TRAIN is CORRECT Message 34 is ACCEPTED |  | L2 |  | SR | - |
|  | 71 | 4 |  | 350.00 | L2 | SR |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - |  | L2 |  | SR | - |
|  | Feature 9990500: A3.5: Handling of Actions in Specific Situations Test Case 6: The on-board equipment shall ensure that the information about track ahead free request is removed once and for all, although the train passes over the location where the information has to be removed more than once. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 75 | 1 |  | 350.00 | L2 | SR |  | ‘Track Ahead Free Request’ is being displayed to the driver. | O | DMI | The driver does not acknowledge the ‘Track Ahead Free Request’. |  | L2 |  | SR | - |
|  | 76 | 2 |  | 450.00 | L2 | SR |  | The train overpasses the end of the 'Track Ahead Free Request' area. | I | INT | - |  | L2 |  | SR | - |
|  | 77 | 3 |  | 450.00 | L2 | SR |  | ‘Track Ahead Free Request’ is removed. | O | DMI | - |  | L2 |  | SR | - |
|  | 78 | 4 |  | 450.00 | L2 | SR |  | Removal of ‘Track Ahead Free Request’ is RECORDED | O | JRU | - |  | L2 |  | SR | - |
|  | 79 | 5 |  | 500.00 | L2 | SR |  | The driver stops the train. | I | INT | The train will have to be stopped very close to the end of the TAF area, since it will pass over this location in backwards direction. Backwards movements are limited by the variable D_NVROLL. |  | L2 |  | SR | - |
|  | 80 | 6 |  | 450.00 | L2 | SR |  | The train moves backwards and overpasses the location of the end of the TAF area again. | I | INT | - |  | L2 |  | SR | - |
|  | 81 | 7 |  | 450.00 | L2 | SR |  | No ‘Track Ahead Free Request’ is displayed. | O | DMI | - |  | L2 |  | SR | - |
|  | 82 | 8 |  | 450.00 | L2 | SR |  | No ‘Track Ahead Free Request’ is recorded. | O | JRU | - |  | L2 |  | SR | - |
|  | 83 | 9 |  | 400.00 | L2 | SR |  | The driver stops the train. | I | INT | The brakes might have been applied. |  | L2 |  | SR | - |
|  | 84 | 10 |  | 450.00 | L2 | SR |  | The train moves forward and overpasses the location of the end of the TAF area again. | I | INT | - |  | L2 |  | SR | - |
|  | 85 | 11 |  | 450.00 | L2 | SR |  | No ‘Track Ahead Free Request’ is displayed. | O | DMI | - |  | L2 |  | SR | - |
|  | 86 | 12 |  | 450.00 | L2 | SR |  | No ‘Track Ahead Free Request’ is recorded. | O | JRU | - |  | L2 |  | SR | - |
|  | Feature 9990500: A3.5: Handling of Actions in Specific Situations Test Case 7: The on-board equipment shall ensure that the location to start calculating the geographical position is taken into account only once, although the train passes over the location more than once |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 87 | 1 |  | 600.00 | L2 | SR |  | The train reaches the location where the geographical position starts to be calculated. | I | INT | - |  | L2 |  | SR | - |
|  | 88 | 2 |  | 620.00 | L2 | SR |  | The driver requests the DISPLAY of the Geographical Position | I | DMI | After the train front end has passed over the location specified in the stored Packet 79. |  | L2 |  | SR | - |
|  | 89 | 3 |  | 620.00 | L2 | SR |  | The driver requests of The Geographical Position DISPLAY is recorded in the JRU | O | JRU | - |  | L2 |  | SR | - |
|  | 90 | 4 |  | 620.00 | L2 | SR |  | The Geographical position is DISPLAYED according to the stored value of M_POSITION | O | DMI | Route kilometre (variable M_POSITION) is based on the received packet 79. |  | L2 |  | SR | - |
|  | 91 | 5 |  | 650.00 | L2 | SR |  | The driver stops the train. | I | INT | The train will have to be stopped very close to the beginning of the area where the geographical position starts to be calculated, since it will pass over this location in backwards direction. Backwards movements are limited by the variable D_NVROLL. |  | L2 |  | SR | - |
|  | 92 | 6 |  | 630.00 | L2 | SR |  | The train moves backwards and overpasses the location where the geographical position started to be calculated. | I | INT | - |  | L2 |  | SR | - |
|  | 93 | 7 |  | 630.00 | L2 | SR |  | The geographical position continues to be displayed as normal. | O | DMI | - |  | L2 |  | SR | - |
|  | 94 | 8 |  | 540.00 | L2 | SR |  | The driver stops the train. | I | INT | The brakes might have been applied. |  | L2 |  | SR | - |
|  | 95 | 9 |  | 580.00 | L2 | SR |  | The train moves forward and overpasses the location where the geographical position started to be calculated. | I | INT | - |  | L2 |  | SR | - |
|  | 96 | 10 |  | 580.00 | L2 | SR |  | The geographical position continues to be displayed as normal. | O | DMI | - |  | L2 |  | SR | - |
|  | Feature 4080415: 4.8.4: Accepted Information depending on the modes - Geographical Position Test Case 3: Transmission of geographical position information via balise group Together with standard balise telegram. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 97 | 1 |  | 660.00 | L2 | SR |  | One Balise Group message (N_TOTAL=1; NID_C=<comment>) containing packet 79 : ‘geographical position information’ is received | I | BTM | Telegram is received by on-board |  | L2 |  | SR | - |
|  | 98 | 2 |  | 660.00 | L2 | SR |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | Packet and telegram are recorded. |  | L2 |  | SR | - |
|  | 104 | 8 |  | 740.00 | L2 | SR |  | The "Geographical Position" is displayed | O | DMI | According to the M_POSITION |  | L2 |  | SR | - |
|  | Feature 9990500: A3.5: Handling of Actions in Specific Situations Test Case 8: The on-board equipment shall ensure that the location to stop calculating the geographical position is taken into account only once, although the train passes over the location more than once |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 105 | 1 |  | 790.00 | L2 | SR |  | The train reaches the location where the geographical position stops to be calculated. | I | INT | - |  | L2 |  | SR | - |
|  | 106 | 2 |  | 790.00 | L2 | SR |  | The driver requests the DISPLAY of the Geographical Position | I | DMI | After the train front end has passed over the location specified in the stored Packet 79. |  | L2 |  | SR | - |
|  | 107 | 3 |  | 790.00 | L2 | SR |  | No Geographical position is DISPLAYED | O | DMI | - |  | L2 |  | SR | - |
|  | 108 | 4 |  | 810.00 | L2 | SR |  | The driver stops the train. | I | INT | The train will have to be stopped very close to the end of the area where the geographical position is calculated, since it will pass over this location in backwards direction. Backwards movements are limited by the variable D_NVROLL. |  | L2 |  | SR | - |
|  | 109 | 5 |  | 760.00 | L2 | SR |  | The train moves backwards and overpasses the location where the geographical position stopped to be calculated. | I | INT | - |  | L2 |  | SR | - |
|  | 110 | 6 |  | 760.00 | L2 | SR |  | The driver requests the DISPLAY of the Geographical Position | I | DMI | - |  | L2 |  | SR | - |
|  | 111 | 7 |  | 760.00 | L2 | SR |  | No Geographical position is DISPLAYED | O | DMI | - |  | L2 |  | SR | - |
|  | 112 | 8 |  | 740.00 | L2 | SR |  | The driver stops the train. | I | INT | The brakes might have been applied. |  | L2 |  | SR | - |
|  | Feature 5100100: 5.10.1: Level Transitions General requirements Test Case 22: The train receives from the RBC the Level Transition announcement LX/LY. The driver is informed. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 113 | 1 |  | 900.00 | L2 | SR |  | An announcement of Level Transition to Level Y (radio message with Packet 41) is received from RBC: D_LEVLETR <> 0 and <> 32767 M_LEVELTR= M_LEVEL_Y (Level Y) | I | RTM | Order to switch to level Y at further location and this announcement will result in a change of the on-board level. |  | L2 |  | SR | - |
|  | 114 | 2 |  | 900.00 | L2 | SR |  | Radio message , including Packet 41'Level transition order' is RECORDED. | O | JRU | - |  | L2 |  | SR | - |
|  | 115 | 3 |  | 900.00 | L2 | SR |  | An indication of announcement of Level Y is DISPLAYED. | O | DMI | ('Except in SL and SB modes') |  | L2 |  | SR | - |
|  | 116 | 4 |  | 900.00 | L2 | SR |  | Displayed indication is RECORDED | O | JRU | ('Except in SL and SB modes') |  | L2 |  | SR | - |
|  | Feature 9990500: A3.5: Handling of Actions in Specific Situations Test Case 10: The on-board equipment shall ensure that an acknowledgement for a level transition announcement is displayed and acknowledged only once, although it passes over the beginning of the acknowledgement area twice, because it moves backwards and forwards. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 117 | 1 |  | 960.00 | L2 | SR |  | The train passes over the beginning of the acknowledgement area for the level transition. | I | INT | - |  | L2 |  | SR | - |
|  | 118 | 2 |  | 960.00 | L2 | SR |  | An acknowledgement for level transition is displayed | O | DMI | - |  | L2 |  | SR | - |
|  | 119 | 3 |  | 960.00 | L2 | SR |  | The acknowledgement request is recorded. | O | JRU | - |  | L2 |  | SR | - |
|  | 120 | 4 |  | 1010.00 | L2 | SR |  | The driver acknowledges the level transition announcement. | I | DMI | - |  | L0 |  | UN | - |
|  | 121 | 5 |  | 1010.00 | L0 | UN |  | The driver’s action is recorded. | O | JRU | - |  | L0 |  | UN | - |
|  | 122 | 6 |  | 1090.00 | L0 | UN |  | The driver stops the train. | I | INT | The train will have to be stopped very close to the beginning of the acknowledgement area, since it will pass over this location in backwards direction. Backwards movements are limited by the variable D_NVROLL. |  | L0 |  | UN | - |
|  | 123 | 7 |  | 960.00 | L0 | UN |  | The train moves backwards and overpasses the beginning of the acknowledgement area. | I | INT | - |  | L0 |  | UN | - |
|  | 124 | 8 |  | 960.00 | L0 | UN |  | No acknowledgement for level transition is displayed. | O | DMI | - |  | L0 |  | UN | - |
|  | 125 | 9 |  | 960.00 | L0 | UN |  | No acknowledgement request is recorded. | O | JRU | - |  | L0 |  | UN | - |
|  | 126 | 10 |  | 935.00 | L0 | UN |  | The driver stops the train. | I | INT | The brakes might have been applied. |  | L0 |  | UN | - |
|  | 127 | 11 |  | 960.00 | L0 | UN |  | The train moves forward. | I | INT | - |  | L0 |  | UN | - |
|  | 128 | 12 |  | 960.00 | L0 | UN |  | The train passes over the beginning of the acknowledgement area for the level transition again. | I | INT | - |  | L0 |  | UN | - |
|  | 129 | 13 |  | 960.00 | L0 | UN |  | No acknowledgement for level transition is displayed. | O | DMI | - |  | L0 |  | UN | - |
|  | 130 | 14 |  | 960.00 | L0 | UN |  | No acknowledgement request is recorded. | O | JRU | - |  | L0 |  | UN | - |
|  | Feature 3160200: 3.16.2: Balises Test Case 17: No linking information is used on-board -A BG marked as unlinked is found -No message consistency problem The message is accepted. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 131 | 1 |  | 1150.00 | L0 | UN |  | One Balise Group message (N_TOTAL=1;Q_LINK=0) containing packet 72, packet 255 is received | I | BTM | No linking information is used on-board The train overpasses a BG marked as unlinked No message consistency problem |  | L0 |  | UN | - |
|  | 132 | 2 |  | 1150.00 | L0 | UN |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded. | O | JRU | - |  | L0 |  | UN | - |
|  | 133 | 3 |  | 1150.00 | L0 | UN |  | The "plain text message" is displayed | O | DMI | Only if mode not PS, SH, SL or PT (with exception of L2/L3 and following reception of 'Recognition of Exit from TR mode' with a more recent time stamp). The BG message is accepted. |  | L0 |  | UN | - |
|  | 134 | 4 |  | 1150.00 | L0 | UN |  | START DISPLAYING PLAIN TEXT MESSAGE (NID_MESSAGE_JRU=18) is recorded | O | JRU | Only if mode not PS, SH, SL or PT (with exception of L2/L3 and following reception of 'Recognition of Exit from TR mode' with a more recent time stamp). |  | L0 |  | UN | - |
|  | 135 | 5 |  | 1170.00 | L0 | UN |  | The "plain text message" is removed | O | DMI | Only if mode not PS, SH, SL or PT (with exception of L2/L3 and following reception of 'Recognition of Exit from TR mode' with a more recent time stamp). |  | L0 |  | UN | - |
|  | 136 | 6 |  | 1170.00 | L0 | UN |  | STOP DISPLAYING PLAIN TEXT MESSAGE (NID_MESSAGE_JRU=19) is recorded | O | JRU | Only if mode not PS, SH, SL or PT (with exception of L2/L3 and following reception of 'Recognition of Exit from TR mode' with a more recent time stamp). |  | L0 |  | UN | - |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 1: The ETCS on-board system is NOT powered. Transition Condition Id [29]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 137 | 1 |  | 1220.00 | L0 | UN |  | The ERTMS/ETCS on-board unit is powered down | I | INT | - |  | L0 |  | NP | - |
|  | 138 | 2 |  | 1220.00 | L0 | NP |  | NOT GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=-) is recorded | O | JRU | There are no messages recorded in the JRU during a period of time |  | L0 |  | NP | - |
| Step 10: DMI Events Distance 0.00 m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Event Id. |  |  | Event Name |  |  |  | Data |  |  |  |  | Delay |  |  |  |  |
| 29 |  |  | Request Change Level |  |  |  | LEVEL=1 |  |  |  |  | 0 s |  |  |  |  |

| - Step 39 : Balise Group BG 5 d 1/2 - Distance 80.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 |  |  |
| NID_BG | 14 | 5 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 41 d | Level transition order |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 63 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_LEVELTR | 15 | 50 d |  |  |
| M_LEVELTR | 3 | 3 |  |  |
| L_ACKLEVELTR | 15 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 3 d | National values |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 230 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_VALIDNV | 15 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| V_NVSHUNT | 7 | 6 d |  |  |
| V_NVSTFF | 7 | 8 d |  |  |
| V_NVONSIGHT | 7 | 6 d |  |  |
| V_NVLIMSUPERV | 7 | 20 d |  |  |
| V_NVUNFIT | 7 | 20 d |  |  |
| V_NVREL | 7 | 8 d |  |  |
| D_NVROLL | 15 | 200 d |  |  |
| Q_NVSBTSMPERM | 1 | 1 d |  |  |
| Q_NVEMRRLS | 1 | 0 b |  |  |
| Q_NVGUIPERM | 1 | 0 b |  |  |
| Q_NVSBFBPERM | 1 | 0 b |  |  |
| Q_NVINHSMICPERM | 1 | 0 b |  |  |
| V_NVALLOWOVTRP | 7 | 0 d |  |  |
| V_NVSUPOVTRP | 7 | 6 d |  |  |
| D_NVOVTRP | 15 | 200 d |  |  |
| T_NVOVTRP | 8 | 60 d |  |  |
| D_NVPOTRP | 15 | 200 d |  |  |
| M_NVCONTACT | 2 | 10 b |  |  |
| T_NVCONTACT | 8 | 255 d |  |  |
| M_NVDERUN | 1 | 1 b |  |  |
| D_NVSTFF | 15 | 32767 d |  |  |
| Q_NVDRIVER_ADHES | 1 | 0 b |  |  |
| A_NVMAXREDADH1 | 6 | 10100 b |  |  |
| A_NVMAXREDADH2 | 6 | 1110 b |  |  |
| A_NVMAXREDADH3 | 6 | 1110 b |  |  |
| Q_NVLOCACC | 6 | 12 d |  |  |
| M_NVAVADH | 5 | 1 d |  |  |
| M_NVEBCL | 4 | 1 b |  |  |
| Q_NVKINT | 1 | 0 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 39 : Balise Group BG 5 d 2/2 - Distance 85.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 |  |  |
| NID_BG | 14 | 5 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 51 : Balise Group BG 2 d 1/2 - Distance 150.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 |  |  |
| NID_BG | 14 | 2 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 42 d | Session management |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 113 d |  |  |
| Q_RBC | 1 | 1 b |  |  |
| NID_C | 10 | 352 d |  |  |
| NID_RBC | 14 | 1515 d |  |  |
| NID_RADIO | 64 | 003265342101FFFF h |  |  |
| Q_SLEEPSESSION | 1 | 0 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 51 : Balise Group BG 2 d 2/2 - Distance 155.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 |  |  |
| NID_BG | 14 | 2 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 3 d | National values |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 230 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_VALIDNV | 15 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| V_NVSHUNT | 7 | 6 d |  |  |
| V_NVSTFF | 7 | 8 d |  |  |
| V_NVONSIGHT | 7 | 6 d |  |  |
| V_NVLIMSUPERV | 7 | 20 d |  |  |
| V_NVUNFIT | 7 | 20 d |  |  |
| V_NVREL | 7 | 8 d |  |  |
| D_NVROLL | 15 | 200 d |  |  |
| Q_NVSBTSMPERM | 1 | 1 d |  |  |
| Q_NVEMRRLS | 1 | 0 b |  |  |
| Q_NVGUIPERM | 1 | 0 b |  |  |
| Q_NVSBFBPERM | 1 | 0 b |  |  |
| Q_NVINHSMICPERM | 1 | 0 b |  |  |
| V_NVALLOWOVTRP | 7 | 0 d |  |  |
| V_NVSUPOVTRP | 7 | 6 d |  |  |
| D_NVOVTRP | 15 | 200 d |  |  |
| T_NVOVTRP | 8 | 60 d |  |  |
| D_NVPOTRP | 15 | 200 d |  |  |
| M_NVCONTACT | 2 | 10 b |  |  |
| T_NVCONTACT | 8 | 255 d |  |  |
| M_NVDERUN | 1 | 1 b |  |  |
| D_NVSTFF | 15 | 32767 d |  |  |
| Q_NVDRIVER_ADHES | 1 | 0 b |  |  |
| A_NVMAXREDADH1 | 6 | 10100 b |  |  |
| A_NVMAXREDADH2 | 6 | 1110 b |  |  |
| A_NVMAXREDADH3 | 6 | 1110 b |  |  |
| Q_NVLOCACC | 6 | 12 d |  |  |
| M_NVAVADH | 5 | 1 d |  |  |
| M_NVEBCL | 4 | 1 b |  |  |
| Q_NVKINT | 1 | 0 b |  |  |
| NID_PACKET | 8 | 79 d | Geographical position information |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 85 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| Q_NEWCOUNTRY | 1 | 0 b |  |  |
| NID_BG | 14 | 2 d |  |  |
| D_POSOFF | 15 | 450 d |  |  |
| Q_MPOSITION | 1 | 1 b |  |  |
| M_POSITION | 24 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 57 : Message 155 - Initiation of a Communication Session (RBC n? 1) - Distance 150.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 155 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |

| - Step 59 : Message 32 - RBC/RIU System Version (RBC n? 1) - Distance 150.00 m Delay : 5.00 s Conditional Message Id : 155 Back Delay : 5.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 32 d |  |  |
| L_MESSAGE | 10 | 11 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048578 d |  |  |
| M_VERSION | 7 | 010 0000 b |  |  |

| - Step 61 : Message 159 - Session Established (RBC n? 1) - Distance 150.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 159 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |

| - Step 64 : Message 129 - Validated Train Data (RBC n? 1) - Distance 150.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 129 d |  |  |
| L_MESSAGE | 10 | 40 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 114 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 1048578 d |  |  |
| D_LRBG | 15 | 0 d |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 12 d |  |  |
| L_DOUBTUNDER | 15 | 12 d |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 4 d |  |  |
| Q_DIRTRAIN | 2 | 01 b |  |  |
| M_MODE | 4 | 2 d |  |  |
| M_LEVEL | 3 | 011 b |  |  |
| NID_PACKET | 8 | 11 d | Validated train data |  |
| L_PACKET | 13 | 126 d |  |  |
| NC_CDTRAIN | 4 | 7 d |  |  |
| NC_TRAIN | 15 | 4 d |  |  |
| L_TRAIN | 12 | 40 d |  |  |
| V_MAXTRAIN | 7 | 80 d |  |  |
| M_LOADINGGAUGE | 8 | 0000 0001 b |  |  |
| M_AXLELOADCAT | 7 | 7 d |  |  |
| M_AIRTIGHT | 2 | 01 b |  |  |
| N_AXLE | 10 | 4 d |  |  |
| N_ITER | 5 | 1 d |  |  |
| M_VOLTAGE | 4 | 3 |  |  |
| NID_CTRACTION | 10 | 15 d |  |  |
| N_ITER | 5 | 2 d |  |  |
| NID_NTC | 8 | 2 d |  |  |
| NID_NTC | 8 | 3 d |  |  |

| - Step 66 : Message 8 - Acknowledgement of Train Data (RBC n? 1) - Distance 150.00 m Delay : 5.00 s Conditional Message Id : 129 Back Delay : 5.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 8 d |  |  |
| L_MESSAGE | 10 | 14 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048578 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |

| - Step 70 : Message 34 - Track Ahead Free Request (RBC n? 1) - Distance 350.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 34 d |  |  |
| L_MESSAGE | 10 | 16 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048578 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_REF | 16 | 0 d |  |  |
| Q_DIR | 2 | 01 b |  |  |
| D_TAFDISPLAY | 15 | 200 d |  |  |
| L_TAFDISPLAY | 15 | 100 d |  |  |

| - Step 97 : Balise Group BG 3 d 1/2 - Distance 660.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 |  |  |
| NID_BG | 14 | 3 d |  |  |
| Q_LINK | 1 | 1 d |  |  |
| NID_PACKET | 8 | 79 d | Geographical position information |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 140 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| Q_NEWCOUNTRY | 1 | 0 b |  |  |
| NID_BG | 14 | 3 d |  |  |
| D_POSOFF | 15 | 80 d |  |  |
| Q_MPOSITION | 1 | 1 b |  |  |
| M_POSITION | 24 | 0 d |  |  |
| N_ITER | 5 | 1 d |  |  |
| Q_NEWCOUNTRY | 1 | 0 b |  |  |
| NID_BG | 14 | 3 d |  |  |
| D_POSOFF | 15 | 130 d |  |  |
| Q_MPOSITION | 1 | 1 b |  |  |
| M_POSITION | 24 | 16777215 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 97 : Balise Group BG 3 d 2/2 - Distance 665.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 |  |  |
| NID_BG | 14 | 3 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 113 : Message 24 - General Message (RBC n? 1) - Distance 900.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 24 d |  |  |
| L_MESSAGE | 10 | 18 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 1 |  |  |
| NID_LRBG | 24 | 1048579 d |  |  |
| NID_PACKET | 8 | 41 d | Level transition order |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 63 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_LEVELTR | 15 | 440 d |  |  |
| M_LEVELTR | 3 | 0 |  |  |
| L_ACKLEVELTR | 15 | 100 d |  |  |
| N_ITER | 5 | 0 d |  |  |

| - Step 131 : Balise Group BG 5 d 1/2 - Distance 1150.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 999 d |  |  |
| NID_BG | 14 | 5 d |  |  |
| Q_LINK | 1 | 0 b |  |  |
| NID_PACKET | 8 | 72 d | Packet for sending plain text messages |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 276 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| Q_TEXTCLASS | 2 | 00 b |  |  |
| Q_TEXTDISPLAY | 1 | 0 b |  |  |
| D_TEXTDISPLAY | 15 | 0 d |  |  |
| M_MODETEXTDISPLAY | 4 | 15 d |  |  |
| M_LEVELTEXTDISPLAY | 3 | 101 b |  |  |
| L_TEXTDISPLAY | 15 | 20 d |  |  |
| T_TEXTDISPLAY | 10 | 1023 d |  |  |
| M_MODETEXTDISPLAY | 4 | 15 d |  |  |
| M_LEVELTEXTDISPLAY | 3 | 101 b |  |  |
| Q_TEXTCONFIRM | 2 | 00 b |  |  |
| L_TEXT | 8 | 23 d |  |  |
| X_TEXT | 8 | N |  |  |
| X_TEXT | 8 | a |  |  |
| X_TEXT | 8 | t |  |  |
| X_TEXT | 8 | i |  |  |
| X_TEXT | 8 | o |  |  |
| X_TEXT | 8 | n |  |  |
| X_TEXT | 8 | a |  |  |
| X_TEXT | 8 | l |  |  |
| X_TEXT | 8 |  |  |  |
| X_TEXT | 8 | v |  |  |
| X_TEXT | 8 | a |  |  |
| X_TEXT | 8 | l |  |  |
| X_TEXT | 8 | u |  |  |
| X_TEXT | 8 | e |  |  |
| X_TEXT | 8 | s |  |  |
| X_TEXT | 8 |  |  |  |
| X_TEXT | 8 | d |  |  |
| X_TEXT | 8 | e |  |  |
| X_TEXT | 8 | l |  |  |
| X_TEXT | 8 | e |  |  |
| X_TEXT | 8 | t |  |  |
| X_TEXT | 8 | e |  |  |
| X_TEXT | 8 | d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 131 : Balise Group BG 5 d 2/2 - Distance 1155.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 999 d |  |  |
| NID_BG | 14 | 5 d |  |  |
| Q_LINK | 1 | 0 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |