| - ERTMS/ETCS |  |
| --- | --- |
| Subset-076-6-3_5080200_01_v320.mdb State : Ordered list of Test Case Steps with communication information |  |
| REF : | Subset-076-6-3_5080200_01_v320.mdb |
| ISSUE : | 3.2.0 |
| DATE : | 10/04/17 12:09 |

| Company | Technical Approval | Management approval |
| --- | --- | --- |
| European Union Agency for Railways | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 3.2.0 10.04.2017 | All |  | MLT |

# Sequence description

Tested Feature: 5.8.2: Selection of Override

# Test cases

Important:	All columns called ‘Test Results’ are only placeholders for results of the test execution. They shall not be used for the description of test cases. 
A test is passed if all cells of ‘Test Result’ are passed.

|  | SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Step | Row |  |  | Dist. (m) |  | Previous |  |  |  |  | Description of Events |  | I/O |  | Interface |  | Comments |  |  | Next |  |  |  |  | Test Result |  |  |
|  |  |  |  |  |  |  | Levels |  | Mode |  |  |  |  |  |  |  |  |  |  |  | Levels |  |  | Mode |  |  |  |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 10: The ETCS on-board equipment is powered .Transition Condition Id [4]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 |  | 1 |  |  | 0.00 |  | N/A |  | NP |  |  | ERTMS/ETCS on-board unit is powered up |  | I |  | INT |  | The on-board equipment changes to SB mode. |  |  | N/A |  |  | NP |  | - |  |
|  | 2 |  | 2 |  |  | 0.00 |  | N/A |  | NP |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=6) is recorded |  | O |  | JRU |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 24: Start of Mission procedure is engaged |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 3 |  | 1 |  |  | 0.00 |  | N/A |  | SB |  |  | Driver opens desk |  | I |  | TIU |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 4 |  | 2 |  |  | 0.00 |  | N/A |  | SB |  |  | The 'CAB STATUS' message is RECORDED with the variable M_CAB_A_STATUS=1 (Active). |  | O |  | JRU |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 5 |  | 3 |  |  | 0.00 |  | N/A |  | SB |  |  | The actual mode SB is DISPLAYED |  | O |  | DMI |  | The SB indication after opening the desk may not be recorded. Because the change of mode was already recorded while desk was closed |  |  | N/A |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 7: The on-board equipment requests the driver to enter a driver ID within the scope of the Start of Mission procedure in SB mode when the status of the driver ID is unknown. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 6 |  | 1 |  |  | 0.00 |  | N/A |  | SB |  |  | The on-board equipment requires the driver to enter a Driver ID There is NO initial Driver ID value displayed |  | O |  | DMI |  | Starting conditions : There is no value initially displayed since the status of the driver ID is unknown. |  |  | N/A |  |  | SB |  | - |  |
|  | 7 |  | 2 |  |  | 0.00 |  | N/A |  | SB |  |  | The on-board equipment prevents the driver from continuing the procedure before the validation of the Driver ID is done. |  | O |  | DMI |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 8 |  | 3 |  |  | 0.00 |  | N/A |  | SB |  |  | The driver enters and validates a correct Driver ID |  | I |  | DMI |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 9 |  | 4 |  |  | 0.00 |  | N/A |  | SB |  |  | The validated Driver ID is recorded on the JRU with the DRIVER_ID variable in a 'GENERAL MESSAGE'. |  | O |  | JRU |  | A 'GENERAL MESSAGE' is recorded every 5s by the on-board equipment. |  |  | N/A |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 10: Within the scope of the Start of Mission procedure in SB mode, the status of the ERTMS/ETCS Level data is INVALID. The driver selects a level (the existing ERTMS/ETCS Level or a new one). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 10 |  | 1 |  |  | 0.00 |  | N/A |  | SB |  |  | The driver selects "Level <L0/LNTC/L1/L2/L3>" |  | I |  | DMI |  | Depending on the implementation, a list of available levels may be presented to the driver on the DMI. The driver is then allowed to select one level among the proposed levels. For NTC levels, the abbreviations of the concerned National Systems shall be presented to the driver. _____ The Driver selects L1 |  |  | L1 |  |  | SB |  | - |  |
|  | 11 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=<34/35/36/37/38>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 12 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=<0/1/2/3/4>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 6: Within the scope of the Start of Mission procedure in SB mode, the On-Board equipment offers the driver to select either SH or NL or ‘Train Data Entry’ before the on-board equipment requests the driver to enter a complete set of Train Data. The driver selects ‘Train Data Entry’. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 13 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver can select either SH or NL or ‘Train Data Entry’ |  | O |  | DMI |  | Starting conditions The 'Non-Leading' button is enabled if the 'non leading' input signal is received. |  |  | L1 |  |  | SB |  | - |  |
|  | 14 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver selects ‘Train Data Entry’ |  | I |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 15 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | A Train data window is is displayed on the DMI. |  | O |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 16 |  | 4 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver’s selection is recorded on the JRU. |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 47: The on-board equipment requests the driver to enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when the status of the Train Data is unknown (S12) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 18 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The on-board equipment requires the driver to enter all Train Data unknown by the on-board equipment and prevents the driver from performing any other actions. |  | O |  | DMI |  | The driver might be requested to enter the 'Train category(ies)', 'Train length', 'Traction / brake parameters', 'Maximum train speed', 'Loading gauge', 'Axle load category', 'Train fitted with airtight system', but the driver shall never be requested to enter the 'Traction system(s) accepted by the engine', 'List of National Systems available on-board' or 'Axle number'. |  |  | L1 |  |  | SB |  | - |  |
|  | 19 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver enters each Train data unknown on-board |  | I |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 20 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver is allowed to validate the train data |  | O |  | DMI |  | A complete set of valid Train Data is available |  |  | L1 |  |  | SB |  | - |  |
|  | 21 |  | 4 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver validates the train data. |  | I |  | DMI |  | The on-board equipment allows the driver to validate as soon as a valid and complete set of Train Data is entered. |  |  | L1 |  |  | SB |  | - |  |
|  | 22 |  | 5 |  |  | 0.00 |  | L1 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded |  | O |  | JRU |  | Triggered by : The driver validation. |  |  | L1 |  |  | SB |  | - |  |
|  | 23 |  | 6 |  |  | 0.00 |  | L1 |  | SB |  |  | TRAIN DATA (NID_MESSAGE_JRU=2) is recorded |  | O |  | JRU |  | Triggered by : Data Entry completed |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 46: After the set of Train Data has been entered/re-validated by the driver (in S12), if the status of the Train running number is 'unknown', the ERTMS/ETCS on-board equipment requests the driver to enter the Train running number (S13) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 25 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver is requested to enter the Train running number. |  | O |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 26 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver enters the "Train Running Number" [MAIN] |  | I |  | DMI |  | Entered Train Running Number: NID_OPERATIONAL_1 |  |  | L1 |  |  | SB |  | - |  |
|  | 27 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | TRAIN RUNNING NUMBER ENTERED BY THE DRIVER (NID_MESSAGE_JRU=49; NID_OPERATIONAL=NID_OPERATIONAL_1) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 16: The driver selects 'Start' within the scope of the Start of Mission procedure in SB mode and in ERTMS Level 1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 29 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver selects 'Start'. |  | I |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 30 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | Driver’s selection of 'Start' is recorded on the JRU |  | O |  | JRU |  | Triggered by: The driver acts on the on-board DMI. |  |  | L1 |  |  | SB |  | - |  |
|  | 31 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | An acknowledgement request for running in Staff Responsible mode is displayed on the DMI |  | O |  | DMI |  | The on-board equipment will then waits for the driver to acknowledge the acknowledgement request for running in Staff Responsible mode |  |  | L1 |  |  | SB |  | - |  |
|  | 32 |  | 4 |  |  | 0.00 |  | L1 |  | SB |  |  | The acknowledgement request for running in Staff Responsible mode is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 33 |  | 5 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver acknowledges 'Staff Responsible' |  | I |  | DMI |  | Symbol MO10 |  |  | L1 |  |  | SB |  | - |  |
|  | 34 |  | 6 |  |  | 0.00 |  | L1 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS= 0000 0011) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 35 |  | 7 |  |  | 0.00 |  | L1 |  | SB |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=2) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 36 |  | 8 |  |  | 0.00 |  | L1 |  | SR |  |  | The Mode symbol 'Staff Responsible' is displayed |  | O |  | DMI |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 37 |  | 9 |  |  | 0.00 |  | L1 |  | SR |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit24=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 38 |  | 10 |  |  | 0.00 |  | L1 |  | SR |  |  | Any message (NID_MESSAGE_JRU=ALL; NID_LRBG=16777215) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | Feature 5080200: 5.8.2: Selection of 'Override' Test Case 1: The ERTMS/ETCS on-board equipment allows the driver to select 'Override'. The speed limit for triggering the 'override' function is a national value and should be replaced when a new set of NVs is available. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 39 |  | 1 |  |  | 100.00 |  | L1 |  | SR |  |  | The train runs with speed higher than V_NVALLOWOVTRP (included in the default value) (e.g. V_Train = 30km/h). The 'override request' is NOT AVAILABLE. |  | O |  | DMI |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 40 |  | 2 |  |  | 100.00 |  | L1 |  | SR |  |  | Packet 3: National Values, Including V_NVALLOWOVTRP, D_VALIDNV & NID_C is RECEIVED. |  | I |  | BTM |  | Use the steps 2/3 or 4/5 for testing' Evaluating a balise group message, balise identity information shall be used to ensure that correct NV are used. (Compare NID_C) Balise group is taken as a new LRBG. New V_NVALLOWOVTRP > default value (0 km/h) |  |  | L1 |  |  | SR |  | - |  |
|  | 41 |  | 3 |  |  | 100.00 |  | L1 |  | SR |  |  | Packet 3: National Values, Including V_NVALLOWOVTRP, D_VALIDNV & NID_C is RECORDED |  | O |  | JRU |  | Use the steps 2/3 or 4/5 for testing' |  |  | L1 |  |  | SR |  | - |  |
|  | 44 |  | 6 |  |  | 100.00 |  | L1 |  | SR |  |  | The override button is activated |  | I |  | DMI |  | if the menu with override button is not activated, the driver selects the right menu in order to see the override button |  |  | L1 |  |  | SR |  | - |  |
|  | 45 |  | 7 |  |  | 150.00 |  | L1 |  | SR |  |  | The train does not reach the location indicated in D_VALIDNV, D_Train < D_VALIDNV The 'override request' is NOT AVAILABLE |  | O |  | DMI |  | As long as the train has not reached the location indicated in D_VALIDNV, the national values shall still not be applicable After this one steps 8 or 9 will be chosen for testing depending on design conditions |  |  | L1 |  |  | SR |  | - |  |
|  | 46 |  | 8 |  |  | 200.00 |  | L1 |  | SR |  |  | The train reaches the distance indicated in D_VALIDNV, D_Train > D_VALIDNV; And the speed of the train V_Train is less or equal to V_NVALLOWOVTRP, V_Train < V_NVALLOWOVTRP The 'override request' is AVAILABLE. |  | O |  | DMI |  | Only If NID_C of received NV is equal to country or region identifier of the LRBG, National values shall become applicable. Override request is not available in NL, SL, RV and TR |  |  | L1 |  |  | SR |  | - |  |
|  | Feature 5080200: 5.8.2: Selection of 'Override' Test Case 4: On-board equipment allows the driver to select override request only when the speed is under or equal to V_NVALLOWOVTRP: national value received from trackside AND Validated Train Data is available |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 50 |  | 1 |  |  | 200.00 |  | L1 |  | SR |  |  | The driver presses "Data view" |  | I |  | DMI |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 51 |  | 2 |  |  | 200.00 |  | L1 |  | SR |  |  | The data from "Data View" window are displayed |  | O |  | DMI |  | Train Data and Train running number are displayed (thus their status is valid) |  |  | L1 |  |  | SR |  | - |  |
|  | 52 |  | 3 |  |  | 200.00 |  | L1 |  | SR |  |  | The driver presses "Override" |  | I |  | DMI |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 53 |  | 4 |  |  | 200.00 |  | L1 |  | SR |  |  | The <EOA> button in the <Override> window is enabled |  | O |  | DMI |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 54 |  | 5 |  |  | 300.00 |  | L1 |  | SR |  |  | The train runs with speed higher than V_NVALLOWOVTRP. Current speed is DISPLAYED. |  | O |  | DMI |  | V_TRAIN > V_NVALLOWOVTRP |  |  | L1 |  |  | SR |  | - |  |
|  | 55 |  | 6 |  |  | 300.00 |  | L1 |  | SR |  |  | The <EOA> button in the <Override> window is disabled |  | O |  | DMI |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 56 |  | 7 |  |  | 400.00 |  | L1 |  | SR |  |  | The train decreases speed - train speed < V_NVALLOWOVTRP. Current speed is DISPLAYED |  | O |  | DMI |  | V_TRAIN < V_NVALLOWOVTRP |  |  | L1 |  |  | SR |  | - |  |
|  | 57 |  | 8 |  |  | 400.00 |  | L1 |  | SR |  |  | The <EOA> button in the <Override> window is enabled |  | O |  | DMI |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 58 |  | 9 |  |  | 500.00 |  | L1 |  | SR |  |  | The train accelerates until speed is higher than V_NVALLOWOVTRP. Current speed is DISPLAYED. |  | O |  | DMI |  | V_TRAIN > V_NVALLOWOVTRP |  |  | L1 |  |  | SR |  | - |  |
|  | 59 |  | 10 |  |  | 500.00 |  | L1 |  | SR |  |  | The <EOA> button in the <Override> window is disabled |  | O |  | DMI |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 60 |  | 11 |  |  | 600.00 |  | L1 |  | SR |  |  | The train decreases speed train speed < V_NVALLOWOVTRP. Current speed is DISPLAYED |  | O |  | DMI |  | V_TRAIN < V_NVALLOWOVTRP |  |  | L1 |  |  | SR |  | - |  |
|  | 61 |  | 12 |  |  | 600.00 |  | L1 |  | SR |  |  | The <EOA> button in the <Override> window is enabled |  | O |  | DMI |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | Feature 5080200: 5.8.2: Selection of 'Override' Test Case 2: The ERTMS/ETCS on-board equipment allows the driver to select 'Override'. The speed limit for triggering the 'override' function is a national value and Default Values shall be used as fall back values |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 62 |  | 1 |  |  | 650.00 |  | L1 |  | SR |  |  | The train runs with speed lower than V_NVALLOWOVTRP (included in the previous set of NV) (e.g. V_Train = 30km/h). The 'override request' is AVAILABLE. |  | O |  | DMI |  | National values are applicable and used by the train. Override request is not available in NL, SL, RV and TR |  |  | L1 |  |  | SR |  | - |  |
|  | 63 |  | 2 |  |  | 650.00 |  | L1 |  | SR |  |  | Onboard receives a balise group message. |  | I |  | BTM |  | Balise group is taken as a new LRBG. There is a mismatch between country or region identifier NID_C of the Balise Group and the corresponding identifier for which a set of national values is used on-board |  |  | L1 |  |  | SR |  | - |  |
|  | 64 |  | 3 |  |  | 650.00 |  | L1 |  | SR |  |  | Balise group message is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 65 |  | 4 |  |  | 650.00 |  | L1 |  | SR |  |  | The override button is activated |  | I |  | DMI |  | if the menu with override button is not activated, the driver selects the right menu in order to see the override button |  |  | L1 |  |  | SR |  | - |  |
|  | 66 |  | 5 |  |  | 700.00 |  | L1 |  | SR |  |  | The 'override request' is NOT AVAILABLE. |  | O |  | DMI |  | Default Values shall be used as fall back values (e.g. V_Train = 30km/h). |  |  | L1 |  |  | SR |  | - |  |
|  | 67 |  | 6 |  |  | 800.00 |  | L1 |  | SR |  |  | The train decelerates to standstill and when it has reached standstill the 'override request' is AVAILABLE again. |  | O |  | DMI |  | - _____ The train reaches standstill |  |  | L1 |  |  | SR |  | - |  |
|  | Feature 5060200: 5.6.2: Table of requirements for 'Shunting Initiated by Driver' procedure Test Case 5: Driver selects 'Shunting', train is at standstill (Transition [5]) and the system switches to SH mode. An ongoing mission exists.'End of Mission' procedure triggered. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 68 |  | 1 |  |  | 800.00 |  | L1 |  | SR |  |  | The driver presses 'shunting' button |  | I |  | DMI |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 69 |  | 2 |  |  | 800.00 |  | L1 |  | SR |  |  | Driver selection is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 70 |  | 3 |  |  | 800.00 |  | L1 |  | SR |  |  | The SH symbol is DISPLAYED |  | O |  | DMI |  | The system switches to SH SH symbol is MO01( according to the document ERA_ERTMS_015560 v330). |  |  | L1 |  |  | SH |  | - |  |
|  | 71 |  | 4 |  |  | 800.00 |  | L1 |  | SH |  |  | The transition to SH mode is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 72 |  | 5 |  |  | 800.00 |  | L1 |  | SH |  |  | The change of permitted speed is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 73 |  | 6 |  |  | 800.00 |  | L1 |  | SH |  |  | Driver requests the permitted speed |  | I |  | DMI |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 74 |  | 7 |  |  | 800.00 |  | L1 |  | SH |  |  | Driver request is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 75 |  | 8 |  |  | 800.00 |  | L1 |  | SH |  |  | The permitted speed is DISPLAYED |  | O |  | DMI |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | Feature 5080200: 5.8.2: Selection of 'Override' Test Case 1: The ERTMS/ETCS on-board equipment allows the driver to select 'Override'. The speed limit for triggering the 'override' function is a national value and should be replaced when a new set of NVs is available. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 76 |  | 1 |  |  | 850.00 |  | L1 |  | SH |  |  | The train runs with speed higher than V_NVALLOWOVTRP (included in the default value) (e.g. V_Train = 30km/h). The 'override request' is NOT AVAILABLE. |  | O |  | DMI |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 77 |  | 2 |  |  | 850.00 |  | L1 |  | SH |  |  | Packet 3: National Values, Including V_NVALLOWOVTRP, D_VALIDNV & NID_C is RECEIVED. |  | I |  | BTM |  | Use the steps 2/3 or 4/5 for testing' Evaluating a balise group message, balise identity information shall be used to ensure that correct NV are used. (Compare NID_C) Balise group is taken as a new LRBG. New V_NVALLOWOVTRP > default value (0 km/h) |  |  | L1 |  |  | SH |  | - |  |
|  | 78 |  | 3 |  |  | 850.00 |  | L1 |  | SH |  |  | Packet 3: National Values, Including V_NVALLOWOVTRP, D_VALIDNV & NID_C is RECORDED |  | O |  | JRU |  | Use the steps 2/3 or 4/5 for testing' |  |  | L1 |  |  | SH |  | - |  |
|  | 81 |  | 6 |  |  | 850.00 |  | L1 |  | SH |  |  | The override button is activated |  | I |  | DMI |  | if the menu with override button is not activated, the driver selects the right menu in order to see the override button |  |  | L1 |  |  | SH |  | - |  |
|  | 82 |  | 7 |  |  | 900.00 |  | L1 |  | SH |  |  | The train does not reach the location indicated in D_VALIDNV, D_Train < D_VALIDNV The 'override request' is NOT AVAILABLE |  | O |  | DMI |  | As long as the train has not reached the location indicated in D_VALIDNV, the national values shall still not be applicable After this one steps 8 or 9 will be chosen for testing depending on design conditions |  |  | L1 |  |  | SH |  | - |  |
|  | 83 |  | 8 |  |  | 950.00 |  | L1 |  | SH |  |  | The train reaches the distance indicated in D_VALIDNV, D_Train > D_VALIDNV; And the speed of the train V_Train is less or equal to V_NVALLOWOVTRP, V_Train < V_NVALLOWOVTRP The 'override request' is AVAILABLE. |  | O |  | DMI |  | Only If NID_C of received NV is equal to country or region identifier of the LRBG, National values shall become applicable. Override request is not available in NL, SL, RV and TR |  |  | L1 |  |  | SH |  | - |  |
|  | Feature 5080200: 5.8.2: Selection of 'Override' Test Case 6: On-board equipment allows the driver to select override request only when the speed is under or equal to V_NVALLOWOVTRP: national value received from trackside. No Validated Train Data available is necessary when already in Shunting mode. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 87 |  | 1 |  |  | 950.00 |  | L1 |  | SH |  |  | The override button is activated |  | I |  | DMI |  | if the menu with override button is not activated, the driver selects the right menu in order to see the override button |  |  | L1 |  |  | SH |  | - |  |
|  | 88 |  | 2 |  |  | 1050.00 |  | L1 |  | SH |  |  | The train runs with speed higher than V_NVALLOWOVTRP. Current speed is DISPLAYED. The 'override request' is not DISPLAYED |  | O |  | DMI |  | V_TRAIN > V_NVALLOWOVTRP |  |  | L1 |  |  | SH |  | - |  |
|  | 89 |  | 3 |  |  | 1150.00 |  | L1 |  | SH |  |  | The train decreases speed - train speed < V_NVALLOWOVTRP. Current speed is DISPLAYED "Override request" is DISPLAYED |  | O |  | DMI |  | V_TRAIN < V_NVALLOWOVTRP |  |  | L1 |  |  | SH |  | - |  |
|  | 90 |  | 4 |  |  | 1250.00 |  | L1 |  | SH |  |  | The train accelerates until speed is higher than V_NVALLOWOVTRP. Current speed is DISPLAYED. The 'override request' is not DISPLAYED |  | O |  | DMI |  | V_TRAIN > V_NVALLOWOVTRP |  |  | L1 |  |  | SH |  | - |  |
|  | 91 |  | 5 |  |  | 1350.00 |  | L1 |  | SH |  |  | The train decreases speed train speed < V_NVALLOWOVTRP. Current speed is DISPLAYED "Override request" is DISPLAYED |  | O |  | DMI |  | V_TRAIN < V_NVALLOWOVTRP |  |  | L1 |  |  | SH |  | - |  |
|  | Feature 9990200: A3.2: List of National / Default Data Test Case 33: The driver selects override request, the system does not change the mode (SH) As soon as the train exceeds D_NVOVTRP, the train trip suppression is cancelled |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 92 |  | 1 |  |  | 1400.00 |  | L1 |  | SH |  |  | Train is moving (V_TRAIN< V_NVALLOWOVTRP) |  | I |  | INT |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 93 |  | 2 |  |  | 1400.00 |  | L1 |  | SH |  |  | D=do The driver selects 'override request ' |  | I |  | DMI |  | The 'override request' is available only when the train speed is below V_NVALLOWOVTRP |  |  | L1 |  |  | SH |  | - |  |
|  | 94 |  | 3 |  |  | 1400.00 |  | L1 |  | SH |  |  | The selection of 'override request' is RECORDED |  | O |  | JRU |  | As soon as the override request is received |  |  | L1 |  |  | SH |  | - |  |
|  | 97 |  | 6 |  |  | 1400.00 |  | L1 |  | SH |  |  | The permitted speed is indicated to the driver. |  | O |  | DMI |  | Train speed is supervised against V_NVSUPOVTRP and V_NVSHUNT |  |  | L1 |  |  | SH |  | - |  |
|  | 98 |  | 7 |  |  | 1400.00 |  | L1 |  | SH |  |  | The indication of permitted speed is RECORDED. |  | O |  | JRU |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | Feature 4070201: 4.7.2: DMI versus Mode Table - Output Test Case 37: The on-board equipment is in SH mode and indicates the permitted speed on the DMI after a driver request. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 100 |  | 1 |  |  | 1400.00 |  | L1 |  | SH |  |  | The permitted speed (V_NVSHUNT) is NOT DISPLAYED |  | O |  | DMI |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 101 |  | 2 |  |  | 1400.00 |  | L1 |  | SH |  |  | The speed and distance monitoring information is displayed |  | O |  | DMI |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 102 |  | 3 |  |  | 1400.00 |  | L1 |  | SH |  |  | The driver requests to show supervision limits |  | I |  | DMI |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 103 |  | 4 |  |  | 1400.00 |  | L1 |  | SH |  |  | The driver action ‘Request to show supervision limits’ is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 104 |  | 5 |  |  | 1400.00 |  | L1 |  | SH |  |  | The permitted speed (V_NVSHUNT) is DISPLAYED |  | O |  | DMI |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 105 |  | 6 |  |  | 1400.00 |  | L1 |  | SH |  |  | The driver requests to hide supervision limits |  | I |  | DMI |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 106 |  | 7 |  |  | 1400.00 |  | L1 |  | SH |  |  | The driver action ‘Request to hide supervision limits’ is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 107 |  | 8 |  |  | 1400.00 |  | L1 |  | SH |  |  | The permitted speed (V_NVSHUNT) is NOT DISPLAYED |  | O |  | DMI |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | Feature 9990200: A3.2: List of National / Default Data Test Case 33: The driver selects override request, the system does not change the mode (SH) As soon as the train exceeds D_NVOVTRP, the train trip suppression is cancelled |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 108 |  | 9 |  |  | 1600.00 |  | L1 |  | SH |  |  | The cancellation of override status is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | Feature 9990200: A3.2: List of National / Default Data Test Case 32: The driver selects 'override request', the system does not change the mode (SR) As soon as the train exceeds T_NVOVTRP, the train trip suppression is cancelled |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 109 |  | 1 |  |  | 1650.00 |  | L1 |  | SH |  |  | Train is moving (V_TRAIN< V_NVALLOWOVTRP) |  | I |  | INT |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 110 |  | 2 |  |  | 1650.00 |  | L1 |  | SH |  |  | t=do The driver selects 'override request ' |  | I |  | DMI |  | The 'override request' is available only when the train speed is below V_NVALLOWOVTRP |  |  | L1 |  |  | SH |  | - |  |
|  | 111 |  | 3 |  |  | 1650.00 |  | L1 |  | SH |  |  | The selection of 'override request' is RECORDED |  | O |  | JRU |  | As soon as the override request is received |  |  | L1 |  |  | SH |  | - |  |
|  | 112 |  | 4 |  |  | 1650.00 |  | L1 |  | SH |  |  | The driver requests the indication of permitted speed. |  | I |  | DMI |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 113 |  | 5 |  |  | 1650.00 |  | L1 |  | SH |  |  | The drivers request is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 114 |  | 6 |  |  | 1650.00 |  | L1 |  | SH |  |  | The permitted speed is indicated to the driver. |  | O |  | DMI |  | Train speed is supervised against V_NVSUPOVTRP. and V_NVSHUNT |  |  | L1 |  |  | SH |  | - |  |
|  | 115 |  | 7 |  |  | 1650.00 |  | L1 |  | SH |  |  | The indication of permitted speed is RECORDED. |  | O |  | JRU |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | Feature 4070201: 4.7.2: DMI versus Mode Table - Output Test Case 37: The on-board equipment is in SH mode and indicates the permitted speed on the DMI after a driver request. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 118 |  | 2 |  |  | 1650.00 |  | L1 |  | SH |  |  | The speed and distance monitoring information is displayed |  | O |  | DMI |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 121 |  | 5 |  |  | 1650.00 |  | L1 |  | SH |  |  | The permitted speed (V_NVSHUNT) is DISPLAYED |  | O |  | DMI |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 122 |  | 6 |  |  | 1650.00 |  | L1 |  | SH |  |  | The driver requests to hide supervision limits |  | I |  | DMI |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 123 |  | 7 |  |  | 1650.00 |  | L1 |  | SH |  |  | The driver action ‘Request to hide supervision limits’ is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 124 |  | 8 |  |  | 1650.00 |  | L1 |  | SH |  |  | The permitted speed (V_NVSHUNT) is NOT DISPLAYED |  | O |  | DMI |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | Feature 9990200: A3.2: List of National / Default Data Test Case 32: The driver selects 'override request', the system does not change the mode (SR) As soon as the train exceeds T_NVOVTRP, the train trip suppression is cancelled |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 125 |  | 9 |  |  | 1800.00 |  | L1 |  | SH |  |  | The cancellation of override status is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 18: The on-board is in SH mode and the driver selects 'exit shunting'. The on-board equipment immediately switches to SB mode if the train is at standstill. Transition Condition Id [19]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 126 |  | 1 |  |  | 1850.00 |  | L1 |  | SH |  |  | The Train is at standstill |  | I |  | INT |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 127 |  | 2 |  |  | 1850.00 |  | L1 |  | SH |  |  | The driver selects 'EXIT SHUNTING' |  | I |  | DMI |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 128 |  | 3 |  |  | 1850.00 |  | L1 |  | SH |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0001 0001) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 129 |  | 4 |  |  | 1850.00 |  | L1 |  | SH |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=6) is recorded |  | O |  | JRU |  | The on-board equipment switches to SB mode. |  |  | L1 |  |  | SB |  | - |  |
|  | 130 |  | 5 |  |  | 1850.00 |  | L1 |  | SB |  |  | The Mode symbol 'Stand By' is displayed |  | O |  | DMI |  | DMI Object: symbol MO13 |  |  | L1 |  |  | SB |  | - |  |
|  | 131 |  | 6 |  |  | 1850.00 |  | L1 |  | SB |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit28=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 8: The on-board equipment allows the driver to validate an existing Driver ID within the scope of the Start of Mission procedure in SB mode when the status of the driver ID is invalid. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 132 |  | 1 |  |  | 1850.00 |  | L1 |  | SB |  |  | There is an initial Driver ID displayed. The on-board equipment allows the driver to simply revalidate it. |  | O |  | DMI |  | A stored on-board Driver ID is displayed to the driver since the status of the Driver ID is INVALID. |  |  | L1 |  |  | SB |  | - |  |
|  | 133 |  | 2 |  |  | 1850.00 |  | L1 |  | SB |  |  | The driver validates the Driver Id. |  | I |  | DMI |  | The on-board equipment allows the driver to simply revalidate an existing Driver ID without entering a new Driver ID. |  |  | L1 |  |  | SB |  | - |  |
|  | 134 |  | 3 |  |  | 1850.00 |  | L1 |  | SB |  |  | The validated Driver ID is recorded on the JRU with the DRIVER_ID variable in a 'GENERAL MESSAGE'. |  | O |  | JRU |  | A 'GENERAL MESSAGE' is recorded every 5s by the on-board equipment. |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 12: The on-board equipment allows the driver to modify an existing ERTMS/ETCS Level data before validating it within the scope of the Start of Mission procedure in SB mode when the status of the ERTMS/ETCS Level data is INVALID. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 135 |  | 1 |  |  | 1850.00 |  | L1 |  | SB |  |  | A ETCS level is displayed to the driver |  | O |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 136 |  | 2 |  |  | 1850.00 |  | L1 |  | SB |  |  | The driver modifies the ERTMS/ETCS Level data (e.g. a new valid data is input). |  | I |  | DMI |  | The on-board equipment allows the driver to modify an existing ERTMS/ETCS Level data. _____ Select L0 |  |  | L1 |  |  | SB |  | - |  |
|  | 137 |  | 3 |  |  | 1850.00 |  | L1 |  | SB |  |  | The new ERTMS/ETCS Level data is displayed on the DMI The On-Board equipment still allows the driver to validate it. |  | O |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 138 |  | 4 |  |  | 1850.00 |  | L1 |  | SB |  |  | The driver validates the ERTMS/ETCS level. |  | I |  | DMI |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | 139 |  | 5 |  |  | 1850.00 |  | L0 |  | SB |  |  | Driver’s validation action is recorded on the JRU. |  | O |  | JRU |  | Triggered by : The driver acts on the on-board DMI. |  |  | L0 |  |  | SB |  | - |  |
|  | 140 |  | 6 |  |  | 1850.00 |  | L0 |  | SB |  |  | The ERTMS/ETCS Level data re-entered by the driver is recorded on the JRU |  | O |  | JRU |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 6: Within the scope of the Start of Mission procedure in SB mode, the On-Board equipment offers the driver to select either SH or NL or ‘Train Data Entry’ before the on-board equipment requests the driver to enter a complete set of Train Data. The driver selects ‘Train Data Entry’. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 141 |  | 1 |  |  | 1850.00 |  | L0 |  | SB |  |  | The driver can select either SH or NL or ‘Train Data Entry’ |  | O |  | DMI |  | Starting conditions The 'Non-Leading' button is enabled if the 'non leading' input signal is received. |  |  | L0 |  |  | SB |  | - |  |
|  | 142 |  | 2 |  |  | 1850.00 |  | L0 |  | SB |  |  | The driver selects ‘Train Data Entry’ |  | I |  | DMI |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | 143 |  | 3 |  |  | 1850.00 |  | L0 |  | SB |  |  | A Train data window is is displayed on the DMI. |  | O |  | DMI |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | 144 |  | 4 |  |  | 1850.00 |  | L0 |  | SB |  |  | The driver’s selection is recorded on the JRU. |  | O |  | JRU |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 2: The on-board equipment requests the driver to validate an existing complete set of Train Data within the scope of the Start of Mission procedure in SB mode when the status of the Train Data is invalid (S12) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 146 |  | 1 |  |  | 1850.00 |  | L0 |  | SB |  |  | A complete set of Train Data is displayed to the driver The driver is allowed to validate the train data. |  | O |  | DMI |  | The driver might be requested to validate the 'Train category(ies)', 'Train length', 'Traction / brake parameters', 'Maximum train speed', 'Loading gauge', 'Axle load category', 'Train fitted with airtight system', but not the 'Traction system(s) accepted by the engine', 'List of National Systems available on-board' or 'Axle number'. |  |  | L0 |  |  | SB |  | - |  |
|  | 147 |  | 2 |  |  | 1850.00 |  | L0 |  | SB |  |  | The driver validates the train data. |  | I |  | DMI |  | The on-board equipment allows the driver to simply validate the existing set of Train Data. |  |  | L0 |  |  | SB |  | - |  |
|  | 148 |  | 3 |  |  | 1850.00 |  | L0 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded |  | O |  | JRU |  | Triggered by : The driver validation. |  |  | L0 |  |  | SB |  | - |  |
|  | 149 |  | 4 |  |  | 1850.00 |  | L0 |  | SB |  |  | TRAIN DATA (NID_MESSAGE_JRU=2) is recorded |  | O |  | JRU |  | Triggered by : Data Entry completed |  |  | L0 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 45: After the set of Train Data has been entered/re-validated by the driver (in S12), if the status of the Train running number is 'invalid', the ERTMS/ETCS on-board equipment requests the driver to re-validate the Train running number (S13) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 151 |  | 1 |  |  | 1850.00 |  | L0 |  | SB |  |  | The driver is requested to re-validate the Train running number. |  | O |  | DMI |  | Displayed Train Running Number: NID_OPERATIONAL_1 |  |  | L0 |  |  | SB |  | - |  |
|  | 152 |  | 2 |  |  | 1850.00 |  | L0 |  | SB |  |  | The driver confirms the "Train Running Number" [MAIN] |  | I |  | DMI |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | 153 |  | 3 |  |  | 1850.00 |  | L0 |  | SB |  |  | TRAIN RUNNING NUMBER ENTERED BY THE DRIVER (NID_MESSAGE_JRU=49; NID_OPERATIONAL=NID_OPERATIONAL_1) is recorded |  | O |  | JRU |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | Feature 5040500: 5.4.5: Degraded Situations Test Case 12: Within the scope of the Start of Mission procedure in SB mode at S10, if valid Train Data is available the driver can select 'Start. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 155 |  | 1 |  |  | 1850.00 |  | L0 |  | SB |  |  | The <Start> button in the <Main> window is enabled. |  | O |  | DMI |  | In addition to possibilities offered at S10 |  |  | L0 |  |  | SB |  | - |  |
|  | 156 |  | 2 |  |  | 1850.00 |  | L0 |  | SB |  |  | The driver selects "Start" |  | I |  | DMI |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | 157 |  | 3 |  |  | 1850.00 |  | L0 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0001 0011) is recorded |  | O |  | JRU |  | Start selected |  |  | L0 |  |  | SB |  | - |  |
|  | 158 |  | 4 |  |  | 1850.00 |  | L0 |  | SB |  |  | The <Default> window is displayed. |  | O |  | DMI |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | 159 |  | 5 |  |  | 1850.00 |  | L0 |  | SB |  |  | The Mode symbol "Acknowledge Unfitted" is displayed |  | O |  | DMI |  | The on-board equipment will then waits for the driver to acknowledge the acknowledgement request for running in Unfitted mode DMI Object: Symbol MO17 |  |  | L0 |  |  | SB |  | - |  |
|  | 160 |  | 6 |  |  | 1850.00 |  | L0 |  | SB |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit32=1>) is recorded |  | O |  | JRU |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | 161 |  | 7 |  |  | 1850.00 |  | L0 |  | SB |  |  | The driver acknowledges "Unfitted" |  | I |  | DMI |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | 162 |  | 8 |  |  | 1850.00 |  | L0 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS= 0000 0100) is recorded |  | O |  | JRU |  | Ack of Unfitted mode As soon as the driver acknowledges the on-board equipment switches to UN mode |  |  | L0 |  |  | SB |  | - |  |
|  | 163 |  | 9 |  |  | 1850.00 |  | L0 |  | SB |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=4) is recorded |  | O |  | JRU |  | Triggering Event: When the mode changes |  |  | L0 |  |  | UN |  | - |  |
|  | 164 |  | 10 |  |  | 1850.00 |  | L0 |  | UN |  |  | The Mode symbol "Unfitted" is displayed |  | O |  | DMI |  | DMI Object: Symbol MO16 |  |  | L0 |  |  | UN |  | - |  |
|  | 165 |  | 11 |  |  | 1850.00 |  | L0 |  | UN |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit31=1>) is recorded |  | O |  | JRU |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | Feature 5100100: 5.10.1: Level Transitions General requirements Test Case 14: The train receives from the balise group BGa the Level Transition announcement LX/LY. The driver is informed. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 166 |  | 1 |  |  | 1880.00 |  | L0 |  | UN |  |  | An announcement of level transition to Level Y (Packet 41) is RECEIVED from Balise Group BGa: D_LEVELTR <> 0 and <> 32767 M_LEVELTR= M_LEVEL_Y (Level Y) |  | I |  | BTM |  | Order to switch to level Y at further location and this announcement will result in a change of the on-board level. |  |  | L0 |  |  | UN |  | - |  |
|  | 167 |  | 2 |  |  | 1880.00 |  | L0 |  | UN |  |  | An indication of Announcement of Level Y is DISPLAYED. |  | O |  | DMI |  | ('Except in SL and SB modes') |  |  | L0 |  |  | UN |  | - |  |
|  | 168 |  | 3 |  |  | 1880.00 |  | L0 |  | UN |  |  | Displayed indication is recorded |  | O |  | JRU |  | ('Except in SL and SB modes') |  |  | L0 |  |  | UN |  | - |  |
|  | 169 |  | 4 |  |  | 1880.00 |  | L0 |  | UN |  |  | Balise Group BGa message, including Packet 41'Level transition order', is RECORDED. |  | O |  | JRU |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | Feature 9990200: A3.2: List of National / Default Data Test Case 34: The driver selects 'override request', the system does not change the mode (UN) and override status is displayed. The train reaches the border of a level transition and switches to L1/2/3. Up to now no MA has been received but the override status is displayed so the on-board switches automatically to SR mode instead of TR mode As soon as the train exceeds T_NVOVTRP, 'override status' is removed so the train trip suppression is cancelled |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 170 |  | 1 |  |  | 1930.00 |  | L0 |  | UN |  |  | Train is moving (V_TRAIN< V_NVALLOWOVTRP) |  | I |  | INT |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | 171 |  | 2 |  |  | 1930.00 |  | L0 |  | UN |  |  | t = t0 The driver selects 'override request' |  | I |  | DMI |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | 172 |  | 3 |  |  | 1930.00 |  | L0 |  | UN |  |  | The 'override request' received by the on-board equipment is RECORDED |  | O |  | JRU |  | Recorded when received on-board |  |  | L0 |  |  | UN |  | - |  |
|  | 173 |  | 4 |  |  | 1930.00 |  | L0 |  | UN |  |  | The 'override status' is DISPLAYED |  | O |  | DMI |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | 174 |  | 5 |  |  | 1930.00 |  | L0 |  | UN |  |  | The indication of the 'override status' is RECORDED |  | O |  | JRU |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | Feature 5100100: 5.10.1: Level Transitions General requirements Test Case 17: The train is in Level X and reaches the level transition border with the Announcement of Level Transition to Level Y. The on-board equipment does not receive any order to switch immediately at the level border. Nevertheless it switches to LY as required at the Level Transition announcement to LY |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 177 |  | 1 |  |  | 1980.00 |  | L0 |  | UN |  |  | Level Y is DISPLAYED. |  | O |  | DMI |  | (Except in SL mode) The estimated front-end train reaches de Level transition location (D_LEVELTR) the On-board equipment switches to Level Y. |  |  | L1 |  |  | UN |  | - |  |
|  | 178 |  | 2 |  |  | 1980.00 |  | L1 |  | UN |  |  | Announcement of Level Y is NOT DISPLAYED any more. |  | O |  | DMI |  | - |  |  | L1 |  |  | UN |  | - |  |
|  | 179 |  | 3 |  |  | 1980.00 |  | L1 |  | UN |  |  | New Level Y is RECORDED. |  | O |  | JRU |  | - |  |  | L1 |  |  | UN |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 52: The Override status is active and the current mode is UN (level 0) or SN (level NTC). When the system reaches the border of the level transition the level changes to level 1 and the system automatically switches to mode SR. Transition Condition Id [44]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 180 |  | 2 |  |  | 1980.00 |  | L1 |  | UN |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=2) is recorded |  | O |  | JRU |  | The on-board equipment switches to SR mode at requested level transition location |  |  | L1 |  |  | SR |  | - |  |
|  | 181 |  | 3 |  |  | 1980.00 |  | L1 |  | SR |  |  | The Mode symbol 'Staff Responsible' is displayed |  | O |  | DMI |  | DMI Object: MO09 |  |  | L1 |  |  | SR |  | - |  |
|  | 182 |  | 4 |  |  | 1980.00 |  | L1 |  | SR |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit24=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 183 |  | 5 |  |  | 1980.00 |  | L1 |  | SR |  |  | The driver requests the speed and distance monitoring information to be displayed |  | I |  | DMI |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 184 |  | 6 |  |  | 1980.00 |  | L1 |  | SR |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 185 |  | 7 |  |  | 1980.00 |  | L1 |  | SR |  |  | The speed and distance monitoring information is displayed |  | O |  | DMI |  | The V_NVSUPOVTRP is displayed as permitted speed. |  |  | L1 |  |  | SR |  | - |  |
|  | 186 |  | 8 |  |  | 1980.00 |  | L1 |  | SR |  |  | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | Feature 9990200: A3.2: List of National / Default Data Test Case 34: The driver selects 'override request', the system does not change the mode (UN) and override status is displayed. The train reaches the border of a level transition and switches to L1/2/3. Up to now no MA has been received but the override status is displayed so the on-board switches automatically to SR mode instead of TR mode As soon as the train exceeds T_NVOVTRP, 'override status' is removed so the train trip suppression is cancelled |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 187 |  | 7 |  |  | 1980.00 |  | L1 |  | SR |  |  | The driver requests the indication of permitted speed. |  | I |  | DMI |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 188 |  | 8 |  |  | 1980.00 |  | L1 |  | SR |  |  | The drivers request is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 189 |  | 9 |  |  | 1980.00 |  | L1 |  | SR |  |  | The permitted speed V_NVSUPOVTRP is indicated to the driver. |  | O |  | DMI |  | The train is supervised against a 'overpass EOA speed' (V_NVSUPOVTRP) |  |  | L1 |  |  | SR |  | - |  |
|  | 190 |  | 10 |  |  | 2080.00 |  | L1 |  | SR |  |  | As soon as t = to + T_NVOVTRP The 'override status' is REMOVED |  | O |  | DMI |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 191 |  | 11 |  |  | 2080.00 |  | L1 |  | SR |  |  | The cancellation of the 'override status' indication is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | Feature 5100100: 5.10.1: Level Transitions General requirements Test Case 12: The train receives an order to switch immediately from L1 to L0 at the current position. on-board equipment immediately switches to L0. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 192 |  | 1 |  |  | 2130.00 |  | L1 |  | SR |  |  | The train reaches the Level transition border and an order to switch immediately to Level 0 is received from balise group BGa. Packet 41: 'Level Transition Order' is RECEIVED. D_LEVELTR= 0 or = 32767 M_LEVELTR=0 |  | I |  | BTM |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 193 |  | 2 |  |  | 2130.00 |  | L1 |  | SR |  |  | Balise Group message, including Packet 41 'Level transition order', is RECORDED. |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 194 |  | 3 |  |  | 2130.00 |  | L1 |  | SR |  |  | Level 0 is DISPLAYED. |  | O |  | DMI |  | (Except in SL mode) The On-board equipment switches to Level 0. |  |  | L0 |  |  | SR |  | - |  |
|  | 195 |  | 4 |  |  | 2130.00 |  | L0 |  | SR |  |  | New Level 0 is RECORDED. |  | O |  | JRU |  | - |  |  | L0 |  |  | SR |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 3: ERTMS/ ETCS Level switches to level 0. Transition Condition Id [21]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 198 |  | 3 |  |  | 2130.00 |  | L0 |  | SR |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=4) is recorded |  | O |  | JRU |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | 199 |  | 4 |  |  | 2130.00 |  | L0 |  | UN |  |  | The Mode symbol 'Unfitted' is displayed |  | O |  | DMI |  | DMI Object: MO16 |  |  | L0 |  |  | UN |  | - |  |
|  | 200 |  | 5 |  |  | 2130.00 |  | L0 |  | UN |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit31=1>) is recorded |  | O |  | JRU |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | Feature 5100400: 5.10.4: Acknowledgement of the level transition Test Case 6: The train has passed over the Level Transition border and the On-board equipment has switched from L1 to Level 0, upon receipt of an order . The driver acknowledges within the driver acknowledgement time[5 seconds] after the Level transition. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 201 |  | 1 |  |  | 2130.00 |  | L0 |  | UN |  |  | The Level symbol "Acknowledge Level 0 announcement" is displayed |  | O |  | DMI |  | DMI Object: Symbol LE07 |  |  | L0 |  |  | UN |  | - |  |
|  | 202 |  | 2 |  |  | 2130.00 |  | L0 |  | UN |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit07=1>) is recorded |  | O |  | JRU |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | 203 |  | 3 |  |  | 2130.00 |  | L0 |  | UN |  |  | The driver acknowledges "Level 0 announcement" |  | I |  | DMI |  | The driver acknowledges the Level Transition within the driver acknowledgement time [5 seconds] after the Level Transition. |  |  | L0 |  |  | UN |  | - |  |
|  | 204 |  | 4 |  |  | 2130.00 |  | L0 |  | UN |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 0110) is recorded |  | O |  | JRU |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | 205 |  | 5 |  |  | 2130.00 |  | L0 |  | UN |  |  | The Level symbol "Acknowledge Level 0 announcement" is removed |  | O |  | DMI |  | DMI Object: Symbol LE07 |  |  | L0 |  |  | UN |  | - |  |
|  | 206 |  | 6 |  |  | 2130.00 |  | L0 |  | UN |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit07=0>) is recorded |  | O |  | JRU |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | Feature 9990200: A3.2: List of National / Default Data Test Case 36: The driver selects 'override request', the system does not change the mode (UN) and override status is displayed. Up to now no MA has been received but the override status is displayed As soon as the train exceeds T_NVOVTRP, 'override status' is removed so the train trip suppression is cancelled. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 207 |  | 1 |  |  | 2180.00 |  | L0 |  | UN |  |  | Train is moving (V_TRAIN< V_NVALLOWOVTRP) |  | I |  | INT |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | 208 |  | 2 |  |  | 2180.00 |  | L0 |  | UN |  |  | t = t0 The driver selects 'override request' |  | I |  | DMI |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | 209 |  | 3 |  |  | 2180.00 |  | L0 |  | UN |  |  | The 'override request' received by the on-board equipment is RECORDED |  | O |  | JRU |  | Recorded when received on-board |  |  | L0 |  |  | UN |  | - |  |
|  | 210 |  | 4 |  |  | 2180.00 |  | L0 |  | UN |  |  | The 'override status' is DISPLAYED |  | O |  | DMI |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | 211 |  | 5 |  |  | 2180.00 |  | L0 |  | UN |  |  | The indication of the 'override status' is RECORDED |  | O |  | JRU |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | 212 |  | 6 |  |  | 2330.00 |  | L0 |  | UN |  |  | As soon as t = to + T_NVOVTRP The 'override status' is REMOVED |  | O |  | DMI |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | 213 |  | 7 |  |  | 2330.00 |  | L0 |  | UN |  |  | The cancellation of the 'override status' indication is RECORDED |  | O |  | JRU |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | Feature 9990200: A3.2: List of National / Default Data Test Case 35: The driver selects 'override request', the system does not change the mode (UN) and override status is displayed. The train reaches the border of a level transition and switches to L1/2/3. Up to now no MA has been received but the override status is displayed so the on-board switches automatically to SR mode instead of TR mode As soon as the train exceeds D_NVOVTRP, 'override status' is removed so the train trip suppression is cancelled |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 214 |  | 1 |  |  | 2380.00 |  | L0 |  | UN |  |  | Train is moving (V_TRAIN< V_NVALLOWOVTRP) |  | I |  | INT |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | 215 |  | 2 |  |  | 2380.00 |  | L0 |  | UN |  |  | d = do The driver selects 'override request' |  | I |  | DMI |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | 216 |  | 3 |  |  | 2380.00 |  | L0 |  | UN |  |  | The 'override request' received by the on-board equipment is RECORDED |  | O |  | JRU |  | Recorded when received on-board |  |  | L0 |  |  | UN |  | - |  |
|  | 217 |  | 4 |  |  | 2380.00 |  | L0 |  | UN |  |  | The 'override status' is DISPLAYED |  | O |  | DMI |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | 218 |  | 5 |  |  | 2380.00 |  | L0 |  | UN |  |  | The indication of the 'override status' is RECORDED |  | O |  | JRU |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | Feature 5100100: 5.10.1: Level Transitions General requirements Test Case 15: The train receives an order to switch immediately from L0 to L1 at the current position. on-board equipment immediately switches to L1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 221 |  | 1 |  |  | 2430.00 |  | L0 |  | UN |  |  | The train reaches the Level transition border and an order to switch immediately to Level 1 is received from balise group BGa : Packet 41: 'Level Transition Order' is RECEIVED. D_LEVELTR=0 M_LEVELTR=2 |  | I |  | BTM |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | 222 |  | 2 |  |  | 2430.00 |  | L0 |  | UN |  |  | Balise Group message, including Packet 41 'Level transition order', is RECORDED |  | O |  | JRU |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | 223 |  | 3 |  |  | 2430.00 |  | L0 |  | UN |  |  | Level 1 is DISPLAYED |  | O |  | DMI |  | (Except in SL mode – desk is closed) The on-board system switches to Level 1 |  |  | L1 |  |  | UN |  | - |  |
|  | 224 |  | 4 |  |  | 2430.00 |  | L1 |  | UN |  |  | New Level L1 is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  |  | UN |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 52: The Override status is active and the current mode is UN (level 0) or SN (level NTC). When the system reaches the border of the level transition the level changes to level 1 and the system automatically switches to mode SR. Transition Condition Id [44]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 225 |  | 2 |  |  | 2430.00 |  | L1 |  | UN |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=2) is recorded |  | O |  | JRU |  | The on-board equipment switches to SR mode at requested level transition location |  |  | L1 |  |  | SR |  | - |  |
|  | 226 |  | 3 |  |  | 2430.00 |  | L1 |  | SR |  |  | The Mode symbol 'Staff Responsible' is displayed |  | O |  | DMI |  | DMI Object: MO09 |  |  | L1 |  |  | SR |  | - |  |
|  | 227 |  | 4 |  |  | 2430.00 |  | L1 |  | SR |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit24=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 228 |  | 5 |  |  | 2430.00 |  | L1 |  | SR |  |  | The driver requests the speed and distance monitoring information to be displayed |  | I |  | DMI |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 229 |  | 6 |  |  | 2430.00 |  | L1 |  | SR |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 230 |  | 7 |  |  | 2430.00 |  | L1 |  | SR |  |  | The speed and distance monitoring information is displayed |  | O |  | DMI |  | The V_NVSUPOVTRP is displayed as permitted speed. |  |  | L1 |  |  | SR |  | - |  |
|  | 231 |  | 8 |  |  | 2430.00 |  | L1 |  | SR |  |  | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | Feature 9990200: A3.2: List of National / Default Data Test Case 35: The driver selects 'override request', the system does not change the mode (UN) and override status is displayed. The train reaches the border of a level transition and switches to L1/2/3. Up to now no MA has been received but the override status is displayed so the on-board switches automatically to SR mode instead of TR mode As soon as the train exceeds D_NVOVTRP, 'override status' is removed so the train trip suppression is cancelled |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 232 |  | 7 |  |  | 2580.00 |  | L1 |  | SR |  |  | As soon as d = do + D_NVOVTRP The 'override status' is REMOVED V_NVSTFF is DISPLAYED as permitted speed on driver request |  | O |  | DMI |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 233 |  | 8 |  |  | 2580.00 |  | L1 |  | SR |  |  | The cancellation of the 'override status' indication is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | Feature 4080426: 4.8.4: Accepted Information depending on the modes - National Values Test Case 4: Transmission of National Values via balise group |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 234 |  | 1 |  |  | 2630.00 |  | L1 |  | SR |  |  | One Balise Group message containing packet 3 is received |  | I |  | BTM |  | - _____ Restore default NV |  |  | L1 |  |  | SR |  | - |  |
|  | 235 |  | 2 |  |  | 2630.00 |  | L1 |  | SR |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 1: The ETCS on-board system is NOT powered. Transition Condition Id [29]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 237 |  | 1 |  |  | 2680.00 |  | L1 |  | SR |  |  | The ERTMS/ETCS on-board unit is powered down |  | I |  | INT |  | - _____ The train reached standstill. |  |  | L1 |  |  | NP |  | - |  |
|  | 238 |  | 2 |  |  | 2680.00 |  | L1 |  | NP |  |  | NOT GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=-) is recorded |  | O |  | JRU |  | There are no messages recorded in the JRU during a period of time |  |  | L1 |  |  | NP |  | - |  |
| Step 10: DMI Events Distance 0.00 m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Event Id. |  |  |  | Event Name |  |  |  |  |  |  | Data |  |  |  |  |  |  |  |  | Delay |  |  |  |  |  |  |  |  |
| 29 |  |  |  | Request Change Level |  |  |  |  |  |  | LEVEL=1 |  |  |  |  |  |  |  |  | 10 s |  |  |  |  |  |  |  |  |

| - Step 40 : Balise Group BG 1 d 1/2 - Distance 100.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 1 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 3 d | National values |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 230 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_VALIDNV | 15 | 100 d |  |  |
| NID_C | 10 | 64 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| V_NVSHUNT | 7 | 6 d |  |  |
| V_NVSTFF | 7 | 8 d |  |  |
| V_NVONSIGHT | 7 | 6 d |  |  |
| V_NVLIMSUPERV | 7 | 20 d |  |  |
| V_NVUNFIT | 7 | 20 d |  |  |
| V_NVREL | 7 | 8 d |  |  |
| D_NVROLL | 15 | 2 d |  |  |
| Q_NVSBTSMPERM | 1 | 1 d |  |  |
| Q_NVEMRRLS | 1 | 0 b |  |  |
| Q_NVGUIPERM | 1 | 0 b |  |  |
| Q_NVSBFBPERM | 1 | 0 b |  |  |
| Q_NVINHSMICPERM | 1 | 0 b |  |  |
| V_NVALLOWOVTRP | 7 | 8 d |  |  |
| V_NVSUPOVTRP | 7 | 6 d |  |  |
| D_NVOVTRP | 15 | 200 d |  |  |
| T_NVOVTRP | 8 | 60 d |  |  |
| D_NVPOTRP | 15 | 200 d |  |  |
| M_NVCONTACT | 2 | 10 b |  |  |
| T_NVCONTACT | 8 | 255 d |  |  |
| M_NVDERUN | 1 | 1 b |  |  |
| D_NVSTFF | 15 | 32767 d |  |  |
| Q_NVDRIVER_ADHES | 1 | 0 b |  |  |
| A_NVMAXREDADH1 | 6 | 010100 b |  |  |
| A_NVMAXREDADH2 | 6 | 001110 b |  |  |
| A_NVMAXREDADH3 | 6 | 001110 b |  |  |
| Q_NVLOCACC | 6 | 12 d |  |  |
| M_NVAVADH | 5 | 1 d |  |  |
| M_NVEBCL | 4 | 0001 b |  |  |
| Q_NVKINT | 1 | 0 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 40 : Balise Group BG 1 d 2/2 - Distance 105.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 1 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 63 : Balise Group BG 2 d 1/2 - Distance 650.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 69 d |  |  |
| NID_BG | 14 | 2 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 63 : Balise Group BG 2 d 2/2 - Distance 655.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 69 d |  |  |
| NID_BG | 14 | 2 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 77 : Balise Group BG 3 d 1/2 - Distance 850.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 3 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 3 d | National values |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 230 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_VALIDNV | 15 | 100 d |  |  |
| NID_C | 10 | 64 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| V_NVSHUNT | 7 | 9 d |  |  |
| V_NVSTFF | 7 | 8 d |  |  |
| V_NVONSIGHT | 7 | 6 d |  |  |
| V_NVLIMSUPERV | 7 | 20 d |  |  |
| V_NVUNFIT | 7 | 20 d |  |  |
| V_NVREL | 7 | 8 d |  |  |
| D_NVROLL | 15 | 2 d |  |  |
| Q_NVSBTSMPERM | 1 | 1 d |  |  |
| Q_NVEMRRLS | 1 | 0 b |  |  |
| Q_NVGUIPERM | 1 | 0 b |  |  |
| Q_NVSBFBPERM | 1 | 0 b |  |  |
| Q_NVINHSMICPERM | 1 | 0 b |  |  |
| V_NVALLOWOVTRP | 7 | 7 d |  |  |
| V_NVSUPOVTRP | 7 | 6 d |  |  |
| D_NVOVTRP | 15 | 200 d |  |  |
| T_NVOVTRP | 8 | 30 d |  |  |
| D_NVPOTRP | 15 | 200 d |  |  |
| M_NVCONTACT | 2 | 10 b |  |  |
| T_NVCONTACT | 8 | 255 d |  |  |
| M_NVDERUN | 1 | 1 b |  |  |
| D_NVSTFF | 15 | 32767 d |  |  |
| Q_NVDRIVER_ADHES | 1 | 0 b |  |  |
| A_NVMAXREDADH1 | 6 | 010100 b |  |  |
| A_NVMAXREDADH2 | 6 | 001110 b |  |  |
| A_NVMAXREDADH3 | 6 | 001110 b |  |  |
| Q_NVLOCACC | 6 | 12 d |  |  |
| M_NVAVADH | 5 | 1 d |  |  |
| M_NVEBCL | 4 | 0001 b |  |  |
| Q_NVKINT | 1 | 0 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 77 : Balise Group BG 3 d 2/2 - Distance 855.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 3 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 166 : Balise Group BG 5 d 1/2 - Distance 1880.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 5 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 41 d | Level transition order |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 63 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_LEVELTR | 15 | 100 d |  |  |
| M_LEVELTR | 3 | 2 d |  |  |
| L_ACKLEVELTR | 15 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 166 : Balise Group BG 5 d 2/2 - Distance 1885.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 5 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 192 : Balise Group BG 6 d 1/2 - Distance 2130.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 6 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 41 d | Level transition order |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 63 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_LEVELTR | 15 | 32767 d |  |  |
| M_LEVELTR | 3 | 0 d |  |  |
| L_ACKLEVELTR | 15 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 192 : Balise Group BG 6 d 2/2 - Distance 2135.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 6 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 221 : Balise Group BG 7 d 1/2 - Distance 2430.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 7 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 41 d | Level transition order |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 63 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_LEVELTR | 15 | 0 d |  |  |
| M_LEVELTR | 3 | 2 d |  |  |
| L_ACKLEVELTR | 15 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 221 : Balise Group BG 7 d 2/2 - Distance 2435.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 7 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 234 : Balise Group BG 8 d 1/2 - Distance 2630.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 8 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 3 d | National values |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 240 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_VALIDNV | 15 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| N_ITER | 5 | 1 d |  |  |
| NID_C | 10 | 352 d |  |  |
| V_NVSHUNT | 7 | 6 d |  |  |
| V_NVSTFF | 7 | 8 d |  |  |
| V_NVONSIGHT | 7 | 6 d |  |  |
| V_NVLIMSUPERV | 7 | 20 d |  |  |
| V_NVUNFIT | 7 | 20 d |  |  |
| V_NVREL | 7 | 8 d |  |  |
| D_NVROLL | 15 | 2 d |  |  |
| Q_NVSBTSMPERM | 1 | 1 d |  |  |
| Q_NVEMRRLS | 1 | 0 b |  |  |
| Q_NVGUIPERM | 1 | 0 b |  |  |
| Q_NVSBFBPERM | 1 | 0 b |  |  |
| Q_NVINHSMICPERM | 1 | 0 b |  |  |
| V_NVALLOWOVTRP | 7 | 0 d |  |  |
| V_NVSUPOVTRP | 7 | 6 d |  |  |
| D_NVOVTRP | 15 | 200 d |  |  |
| T_NVOVTRP | 8 | 60 d |  |  |
| D_NVPOTRP | 15 | 200 d |  |  |
| M_NVCONTACT | 2 | 10 b |  |  |
| T_NVCONTACT | 8 | 255 d |  |  |
| M_NVDERUN | 1 | 1 b |  |  |
| D_NVSTFF | 15 | 32767 d |  |  |
| Q_NVDRIVER_ADHES | 1 | 0 b |  |  |
| A_NVMAXREDADH1 | 6 | 010100 b |  |  |
| A_NVMAXREDADH2 | 6 | 001110 b |  |  |
| A_NVMAXREDADH3 | 6 | 001110 b |  |  |
| Q_NVLOCACC | 6 | 12 d |  |  |
| M_NVAVADH | 5 | 0 d |  |  |
| M_NVEBCL | 4 | 1001 b |  |  |
| Q_NVKINT | 1 | 0 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 234 : Balise Group BG 8 d 2/2 - Distance 2635.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 8 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |