| - ERTMS/ETCS |  |
| --- | --- |
| Subset-076-6-3_5080400_01_v320.mdb State : Ordered list of Test Case Steps with communication information |  |
| REF : | Subset-076-6-3_5080400_01_v320.mdb |
| ISSUE : | 3.2.0 |
| DATE : | 10/04/17 12:19 |

| Company | Technical Approval | Management approval |
| --- | --- | --- |
| European Union Agency for Railways | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 3.2.0 10.04.2017 | All |  | MLT |

# Sequence description

Tested Feature: 5.8.4: End of Override procedure.

# Test cases

Important:	All columns called ‘Test Results’ are only placeholders for results of the test execution. They shall not be used for the description of test cases. 
A test is passed if all cells of ‘Test Result’ are passed.

|  | SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Step | Row |  |  | Dist. (m) |  | Previous |  |  |  |  | Description of Events |  | I/O |  | Interface |  | Comments |  |  | Next |  |  |  |  | Test Result |  |  |
|  |  |  |  |  |  |  | Levels |  | Mode |  |  |  |  |  |  |  |  |  |  |  | Levels |  |  | Mode |  |  |  |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 10: The ETCS on-board equipment is powered .Transition Condition Id [4]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 |  | 1 |  |  | 0.00 |  | N/A |  | NP |  |  | ERTMS/ETCS on-board unit is powered up |  | I |  | INT |  | The on-board equipment changes to SB mode. |  |  | N/A |  |  | NP |  | - |  |
|  | 2 |  | 2 |  |  | 0.00 |  | N/A |  | NP |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=6) is recorded |  | O |  | JRU |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 24: Start of Mission procedure is engaged |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 3 |  | 1 |  |  | 0.00 |  | N/A |  | SB |  |  | Driver opens desk |  | I |  | TIU |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 4 |  | 2 |  |  | 0.00 |  | N/A |  | SB |  |  | The 'CAB STATUS' message is RECORDED with the variable M_CAB_A_STATUS=1 (Active). |  | O |  | JRU |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 5 |  | 3 |  |  | 0.00 |  | N/A |  | SB |  |  | The actual mode SB is DISPLAYED |  | O |  | DMI |  | The SB indication after opening the desk may not be recorded. Because the change of mode was already recorded while desk was closed |  |  | N/A |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 7: The on-board equipment requests the driver to enter a driver ID within the scope of the Start of Mission procedure in SB mode when the status of the driver ID is unknown. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 6 |  | 1 |  |  | 0.00 |  | N/A |  | SB |  |  | The on-board equipment requires the driver to enter a Driver ID There is NO initial Driver ID value displayed |  | O |  | DMI |  | Starting conditions : There is no value initially displayed since the status of the driver ID is unknown. |  |  | N/A |  |  | SB |  | - |  |
|  | 7 |  | 2 |  |  | 0.00 |  | N/A |  | SB |  |  | The on-board equipment prevents the driver from continuing the procedure before the validation of the Driver ID is done. |  | O |  | DMI |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 8 |  | 3 |  |  | 0.00 |  | N/A |  | SB |  |  | The driver enters and validates a correct Driver ID |  | I |  | DMI |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 9 |  | 4 |  |  | 0.00 |  | N/A |  | SB |  |  | The validated Driver ID is recorded on the JRU with the DRIVER_ID variable in a 'GENERAL MESSAGE'. |  | O |  | JRU |  | A 'GENERAL MESSAGE' is recorded every 5s by the on-board equipment. |  |  | N/A |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 10: Within the scope of the Start of Mission procedure in SB mode, the status of the ERTMS/ETCS Level data is INVALID. The driver selects a level (the existing ERTMS/ETCS Level or a new one). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 10 |  | 1 |  |  | 0.00 |  | N/A |  | SB |  |  | The driver selects "Level <L0/LNTC/L1/L2/L3>" |  | I |  | DMI |  | Depending on the implementation, a list of available levels may be presented to the driver on the DMI. The driver is then allowed to select one level among the proposed levels. For NTC levels, the abbreviations of the concerned National Systems shall be presented to the driver. _____ The Driver selects L1 |  |  | L1 |  |  | SB |  | - |  |
|  | 11 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=<34/35/36/37/38>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 12 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=<0/1/2/3/4>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 6: Within the scope of the Start of Mission procedure in SB mode, the On-Board equipment offers the driver to select either SH or NL or ‘Train Data Entry’ before the on-board equipment requests the driver to enter a complete set of Train Data. The driver selects ‘Train Data Entry’. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 13 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver can select either SH or NL or ‘Train Data Entry’ |  | O |  | DMI |  | Starting conditions The 'Non-Leading' button is enabled if the 'non leading' input signal is received. |  |  | L1 |  |  | SB |  | - |  |
|  | 14 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver selects ‘Train Data Entry’ |  | I |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 15 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | A Train data window is is displayed on the DMI. |  | O |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 16 |  | 4 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver’s selection is recorded on the JRU. |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 47: The on-board equipment requests the driver to enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when the status of the Train Data is unknown (S12) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 18 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The on-board equipment requires the driver to enter all Train Data unknown by the on-board equipment and prevents the driver from performing any other actions. |  | O |  | DMI |  | The driver might be requested to enter the 'Train category(ies)', 'Train length', 'Traction / brake parameters', 'Maximum train speed', 'Loading gauge', 'Axle load category', 'Train fitted with airtight system', but the driver shall never be requested to enter the 'Traction system(s) accepted by the engine', 'List of National Systems available on-board' or 'Axle number'. |  |  | L1 |  |  | SB |  | - |  |
|  | 19 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver enters each Train data unknown on-board |  | I |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 20 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver is allowed to validate the train data |  | O |  | DMI |  | A complete set of valid Train Data is available |  |  | L1 |  |  | SB |  | - |  |
|  | 21 |  | 4 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver validates the train data. |  | I |  | DMI |  | The on-board equipment allows the driver to validate as soon as a valid and complete set of Train Data is entered. |  |  | L1 |  |  | SB |  | - |  |
|  | 22 |  | 5 |  |  | 0.00 |  | L1 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded |  | O |  | JRU |  | Triggered by : The driver validation. |  |  | L1 |  |  | SB |  | - |  |
|  | 23 |  | 6 |  |  | 0.00 |  | L1 |  | SB |  |  | TRAIN DATA (NID_MESSAGE_JRU=2) is recorded |  | O |  | JRU |  | Triggered by : Data Entry completed |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 46: After the set of Train Data has been entered/re-validated by the driver (in S12), if the status of the Train running number is 'unknown', the ERTMS/ETCS on-board equipment requests the driver to enter the Train running number (S13) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 25 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver is requested to enter the Train running number. |  | O |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 26 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver enters the "Train Running Number" [MAIN] |  | I |  | DMI |  | Entered Train Running Number: NID_OPERATIONAL_1 |  |  | L1 |  |  | SB |  | - |  |
|  | 27 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | TRAIN RUNNING NUMBER ENTERED BY THE DRIVER (NID_MESSAGE_JRU=49; NID_OPERATIONAL=NID_OPERATIONAL_1) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 1: The on-board equipment requests the driver to re-validate/enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when level is 0/1/NTC (Subset-026-5.4.3.2 S12). The driver is requested to enter/re-validate the Train running number if the status of the Train running number is 'unknown' or 'invalid'. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 28 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | The DMI allows the driver to select the option ‘START’. |  | O |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 16: The driver selects 'Start' within the scope of the Start of Mission procedure in SB mode and in ERTMS Level 1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 29 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver selects 'Start'. |  | I |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 30 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | Driver’s selection of 'Start' is recorded on the JRU |  | O |  | JRU |  | Triggered by: The driver acts on the on-board DMI. |  |  | L1 |  |  | SB |  | - |  |
|  | 31 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | An acknowledgement request for running in Staff Responsible mode is displayed on the DMI |  | O |  | DMI |  | The on-board equipment will then waits for the driver to acknowledge the acknowledgement request for running in Staff Responsible mode |  |  | L1 |  |  | SB |  | - |  |
|  | 32 |  | 4 |  |  | 0.00 |  | L1 |  | SB |  |  | The acknowledgement request for running in Staff Responsible mode is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 33 |  | 5 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver acknowledges 'Staff Responsible' |  | I |  | DMI |  | Symbol MO10 |  |  | L1 |  |  | SB |  | - |  |
|  | 34 |  | 6 |  |  | 0.00 |  | L1 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS= 0000 0011) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 35 |  | 7 |  |  | 0.00 |  | L1 |  | SB |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=2) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 36 |  | 8 |  |  | 0.00 |  | L1 |  | SR |  |  | The Mode symbol 'Staff Responsible' is displayed |  | O |  | DMI |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 37 |  | 9 |  |  | 0.00 |  | L1 |  | SR |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit24=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 38 |  | 10 |  |  | 0.00 |  | L1 |  | SR |  |  | Any message (NID_MESSAGE_JRU=ALL; NID_LRBG=16777215) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | Feature 5080200: 5.8.2: Selection of 'Override' Test Case 1: The ERTMS/ETCS on-board equipment allows the driver to select 'Override'. The speed limit for triggering the 'override' function is a national value and should be replaced when a new set of NVs is available. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 39 |  | 1 |  |  | 50.00 |  | L1 |  | SR |  |  | The train runs with speed higher than V_NVALLOWOVTRP (included in the default value) (e.g. V_Train = 30km/h). The 'override request' is NOT AVAILABLE. |  | O |  | DMI |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 40 |  | 2 |  |  | 50.00 |  | L1 |  | SR |  |  | Packet 3: National Values, Including V_NVALLOWOVTRP, D_VALIDNV & NID_C is RECEIVED. |  | I |  | BTM |  | Use the steps 2/3 or 4/5 for testing' Evaluating a balise group message, balise identity information shall be used to ensure that correct NV are used. (Compare NID_C) Balise group is taken as a new LRBG. New V_NVALLOWOVTRP > default value (0 km/h) |  |  | L1 |  |  | SR |  | - |  |
|  | 41 |  | 3 |  |  | 50.00 |  | L1 |  | SR |  |  | Packet 3: National Values, Including V_NVALLOWOVTRP, D_VALIDNV & NID_C is RECORDED |  | O |  | JRU |  | Use the steps 2/3 or 4/5 for testing' |  |  | L1 |  |  | SR |  | - |  |
|  | 44 |  | 6 |  |  | 50.00 |  | L1 |  | SR |  |  | The override button is activated |  | I |  | DMI |  | if the menu with override button is not activated, the driver selects the right menu in order to see the override button |  |  | L1 |  |  | SR |  | - |  |
|  | 45 |  | 7 |  |  | 100.00 |  | L1 |  | SR |  |  | The train does not reach the location indicated in D_VALIDNV, D_Train < D_VALIDNV The 'override request' is NOT AVAILABLE |  | O |  | DMI |  | As long as the train has not reached the location indicated in D_VALIDNV, the national values shall still not be applicable After this one steps 8 or 9 will be chosen for testing depending on design conditions |  |  | L1 |  |  | SR |  | - |  |
|  | 46 |  | 8 |  |  | 150.00 |  | L1 |  | SR |  |  | The train reaches the distance indicated in D_VALIDNV, D_Train > D_VALIDNV; And the speed of the train V_Train is less or equal to V_NVALLOWOVTRP, V_Train < V_NVALLOWOVTRP The 'override request' is AVAILABLE. |  | O |  | DMI |  | Only If NID_C of received NV is equal to country or region identifier of the LRBG, National values shall become applicable. Override request is not available in NL, SL, RV and TR |  |  | L1 |  |  | SR |  | - |  |
|  | Feature 5080400: 5.8.4: End of Override procedure Test Case 1: The driver selects Override. When a new value for D_NVOVTRP is received on-board (by means of Packet 3, with NID_C equal to the currently being used) and it becomes applicable, the ongoing 'Override' supervision will use this new value. However, the starting location shall remain unchanged. The Override procedure ends when train has run more than the "distance for train trip suppression when Override function is triggered" |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 50 |  | 1 |  |  | 200.00 |  | L1 |  | SR |  |  | The driver selects "Override EOA" |  | I |  | DMI |  | The train speed is below max. speed for override (V_NVALLOWOVTRP) |  |  | L1 |  |  | SR |  | - |  |
|  | 51 |  | 2 |  |  | 200.00 |  | L1 |  | SR |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=14) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 52 |  | 3 |  |  | 200.00 |  | L1 |  | SR |  |  | The Mode symbol "Override EOA is active" is displayed |  | O |  | DMI |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 53 |  | 4 |  |  | 200.00 |  | L1 |  | SR |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit18=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 54 |  | 5 |  |  | 300.00 |  | L1 |  | SR |  |  | Packet 3: National Values, Including D_NVOVTRP & a correct value for NID_C is RECEIVED. |  | I |  | BTM |  | Choose the BTM or the RTM step. D_NVOVTRP will be different from the one currently being used NID_C will be the same than the one stored on-board |  |  | L1 |  |  | SR |  | - |  |
|  | 56 |  | 7 |  |  | 300.00 |  | L1 |  | SR |  |  | Packet 3: National Values, Including D_NVOVTRP & a correct value for NID_C is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 57 |  | 8 |  |  | 400.00 |  | L1 |  | SR |  |  | As soon as d > d0 +D_NVOVTRP the 'override status' is REMOVED |  | O |  | DMI |  | d is current the position of the train d0 is the position of the train when the 'Override' button was triggered |  |  | L1 |  |  | SR |  | - |  |
|  | 58 |  | 9 |  |  | 400.00 |  | L1 |  | SR |  |  | The cancellation of override status is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 35: The train is moving in SR mode. It passes over a balise group sending a message which contains packet 137 'Stop if in Staff Responsible' AND no list of expected balise groups related to SR mode has been received or the list of expected balise groups related to SR mode does not include the identity of the over-passed balise group AND override is not active. Therefore, the train is tripped. Transition Condition Id [54]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  | 1 |  |  | 430.00 |  | L1 |  | SR |  |  | One Balise Group message (N_TOTAL=1; NID_C=<comment>) containing packet 137 (Q_SRSTOP=0) is received |  | I |  | BTM |  | BGa telegram contains Packet 137: 'Stop if in Staff Responsible' with variable Q_SRSTOP = 0. |  |  | L1 |  |  | SR |  | - |  |
|  | 61 |  | 2 |  |  | 430.00 |  | L1 |  | SR |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 62 |  | 3 |  |  | 430.00 |  | L1 |  | SR |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=7) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | TR |  | - |  |
|  | Feature 4070201: 4.7.2: DMI versus Mode Table - Output Test Case 17: Indication of Trip mode and the reason to the driver. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 64 |  | 1 |  |  | 430.00 |  | L1 |  | TR |  |  | The Trip mode (TR) is permanently DISPLAYED |  | O |  | DMI |  | The mode indication may not be permanent if the driver uses the menus. |  |  | L1 |  |  | TR |  | - |  |
|  | 65 |  | 2 |  |  | 430.00 |  | L1 |  | TR |  |  | The reason (Trip Reason) is DISPLAYED |  | O |  | DMI |  | The Trip Reason is related to the conditions (SRS 4.6.3) that lead to the transition to TR. |  |  | L1 |  |  | TR |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 35: The train is moving in SR mode. It passes over a balise group sending a message which contains packet 137 'Stop if in Staff Responsible' AND no list of expected balise groups related to SR mode has been received or the list of expected balise groups related to SR mode does not include the identity of the over-passed balise group AND override is not active. Therefore, the train is tripped. Transition Condition Id [54]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 66 |  | 5 |  |  | 430.00 |  | L1 |  | TR |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit19=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | TR |  | - |  |
|  | 67 |  | 6 |  |  | 430.00 |  | L1 |  | TR |  |  | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23; SYSTEM_STATUS_MESSAGE=<Bit16=1>) is recorded |  | O |  | JRU |  | SR stop order |  |  | L1 |  |  | TR |  | - |  |
|  | 68 |  | 7 |  |  | 430.00 |  | L1 |  | TR |  |  | Emergency brake commanded |  | O |  | TIU |  | - |  |  | L1 |  |  | TR |  | - |  |
|  | 69 |  | 8 |  |  | 430.00 |  | L1 |  | TR |  |  | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=1) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | TR |  | - |  |
|  | 70 |  | 9 |  |  | 430.00 |  | L1 |  | TR |  |  | The Status symbol ' Emergency Brake ' is displayed |  | O |  | DMI |  | DMI Object: Symbol ST01 |  |  | L1 |  |  | TR |  | - |  |
|  | 71 |  | 10 |  |  | 430.00 |  | L1 |  | TR |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | TR |  | - |  |
|  | Feature 5110200: 5.11.2: Table of requirements for 'Train Trip' procedure Test Case 13: Before the train reaches standstill, no message to acknowledge Train Trip is displayed. The train reaches standstill in level 1, 2 or 3 and TR mode. The train/engine performs transition to PT after Driver acknowledgement of the TRIP mode. In level 2 or level 3, a communication session is opened and the On-Board sends a position report to the track-side when the mode changes Transition Id [7]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 72 |  | 1 |  |  | 440.00 |  | L1 |  | TR |  |  | No Request for driver acknowledgement to Train Trip is DISPLAYED |  | O |  | DMI |  | The train is still moving |  |  | L1 |  |  | TR |  | - |  |
|  | 73 |  | 2 |  |  | 470.00 |  | L1 |  | TR |  |  | The request for driver acknowledgement to Train Trip/ symbol MO05 is DISPLAYED |  | O |  | DMI |  | As soon as the train has come to standstill |  |  | L1 |  |  | TR |  | - |  |
|  | 74 |  | 3 |  |  | 470.00 |  | L1 |  | TR |  |  | The displayed symbol MO05is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  |  | TR |  | - |  |
|  | 75 |  | 4 |  |  | 470.00 |  | L1 |  | TR |  |  | The driver acknowledges the TR Mode |  | I |  | DMI |  | - |  |  | L1 |  |  | TR |  | - |  |
|  | 76 |  | 5 |  |  | 470.00 |  | L1 |  | TR |  |  | The driver acknowledgement is RECORDED. |  | O |  | JRU |  | - |  |  | L1 |  |  | TR |  | - |  |
|  | 77 |  | 6 |  |  | 470.00 |  | L1 |  | TR |  |  | The new mode PT/ symbol MO06 is DISPLAYED |  | O |  | DMI |  | The on-board equipment switches to PT mode. |  |  | L1 |  |  | PT |  | - |  |
|  | 78 |  | 7 |  |  | 470.00 |  | L1 |  | PT |  |  | The new mode PT/ symbol MO06 is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  |  | PT |  | - |  |
|  | 81 |  | 10 |  |  | 470.00 |  | L1 |  | PT |  |  | The reason (Trip Reason) is DISPLAYED |  | O |  | DMI |  | The trip reason is displayed while the train is in PT mode |  |  | L1 |  |  | PT |  | - |  |
|  | 82 |  | 11 |  |  | 470.00 |  | L1 |  | PT |  |  | Emergency brake not commanded |  | O |  | TIU |  | Release of emergency brake, because the train is at standstill and the driver has acknowledged the trip. |  |  | L1 |  |  | PT |  | - |  |
|  | 83 |  | 12 |  |  | 470.00 |  | L1 |  | PT |  |  | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=0) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | PT |  | - |  |
|  | 84 |  | 13 |  |  | 470.00 |  | L1 |  | PT |  |  | The Status symbol "Service Brake or Emergency Brake intervention" is removed |  | O |  | DMI |  | - |  |  | L1 |  |  | PT |  | - |  |
|  | 85 |  | 14 |  |  | 470.00 |  | L1 |  | PT |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=0>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | PT |  | - |  |
|  | Feature 5060200: 5.6.2: Table of requirements for 'Shunting Initiated by Driver' procedure Test Case 5: Driver selects 'Shunting', train is at standstill (Transition [5]) and the system switches to SH mode. An ongoing mission exists.'End of Mission' procedure triggered. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 86 |  | 1 |  |  | 470.00 |  | L1 |  | PT |  |  | The driver presses 'shunting' button |  | I |  | DMI |  | - |  |  | L1 |  |  | PT |  | - |  |
|  | 87 |  | 2 |  |  | 470.00 |  | L1 |  | PT |  |  | Driver selection is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  |  | PT |  | - |  |
|  | 88 |  | 3 |  |  | 470.00 |  | L1 |  | PT |  |  | The SH symbol is DISPLAYED |  | O |  | DMI |  | The system switches to SH SH symbol is MO01( according to the document ERA_ERTMS_015560 v330). |  |  | L1 |  |  | SH |  | - |  |
|  | 89 |  | 4 |  |  | 470.00 |  | L1 |  | SH |  |  | The transition to SH mode is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 90 |  | 5 |  |  | 470.00 |  | L1 |  | SH |  |  | The change of permitted speed is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 91 |  | 6 |  |  | 470.00 |  | L1 |  | SH |  |  | Driver requests the permitted speed |  | I |  | DMI |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 92 |  | 7 |  |  | 470.00 |  | L1 |  | SH |  |  | Driver request is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 93 |  | 8 |  |  | 470.00 |  | L1 |  | SH |  |  | The permitted speed is DISPLAYED |  | O |  | DMI |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | Feature 5080400: 5.8.4: End of Override procedure Test Case 9: The driver selects override request, the system does not change the mode (SH). While passing a balise with 'Danger for shunting' information and indicating 'stop if in SH', the train does not switch to TR. As soon as the train passes the balise with 'Danger for shunting information', the train trip suppression is cancelled. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 94 |  | 1 |  |  | 550.00 |  | L1 |  | SH |  |  | The driver selects 'override request' |  | I |  | DMI |  | The 'override request' is available only when the train speed is below V_NVALLOWOVTRP (execute with no standstill) |  |  | L1 |  |  | SH |  | - |  |
|  | 95 |  | 2 |  |  | 550.00 |  | L1 |  | SH |  |  | The selection of 'override request' is RECORDED |  | O |  | JRU |  | As soon as the override request is received |  |  | L1 |  |  | SH |  | - |  |
|  | 98 |  | 5 |  |  | 550.00 |  | L1 |  | SH |  |  | The permitted speed is indicated to the driver. |  | O |  | DMI |  | Train speed is supervised against V_NVSUPOVTRP (excepted in LNTC) and V_NVSHUNT (permitted speed = V_NVSUPOVTRP) |  |  | L1 |  |  | SH |  | - |  |
|  | 99 |  | 6 |  |  | 630.00 |  | L1 |  | SH |  |  | The train passes a balise group containing 'Danger for SH' information (packet 132) and indicating 'Stop if in SH' Q_ASPECT = 0 The train does not switch to TR because the train trip is inhibited |  | I |  | BTM |  | see TC constraints |  |  | L1 |  |  | SH |  | - |  |
|  | 100 |  | 7 |  |  | 630.00 |  | L1 |  | SH |  |  | The balise message with packet 132 is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 101 |  | 8 |  |  | 630.00 |  | L1 |  | SH |  |  | The permitted speed (V_NVSHUNT) is DISPLAYED on driver request |  | O |  | DMI |  | Train speed is not supervised against V_NVSUPOVTRP anymore |  |  | L1 |  |  | SH |  | - |  |
|  | 102 |  | 9 |  |  | 630.00 |  | L1 |  | SH |  |  | The override status is REMOVED |  | O |  | DMI |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 103 |  | 10 |  |  | 630.00 |  | L1 |  | SH |  |  | The cancellation of override status is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 104 |  | 11 |  |  | 630.00 |  | L1 |  | SH |  |  | Emergency brake not commanded |  | O |  | TIU |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 105 |  | 12 |  |  | 630.00 |  | L1 |  | SH |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=2) is recorded |  | O |  | JRU |  | The train does not switch to TR Transition Condition Id [49] not executed because Override is active |  |  | L1 |  |  | SH |  | - |  |
|  | Feature 4080408: 4.8.4: Accepted Information depending on the modes - Danger for SH information Test Case 1: The train is in shunting mode and overpasses a balise with the packet number 132 (danger for shunting information). The train will be tripped if it’s in shunting mode. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 108 |  | 1 |  |  | 680.00 |  | L1 |  | SH |  |  | One Balise Group message (N_TOTAL=0; NID_C=<comment>) containing packet 132 "Danger for shunting information" (Q_ASPETC = 0) is received |  | I |  | BTM |  | One of the balises in BGa has a packet number 132 (Danger for shunting information) with the variable Q_ASPECT set to zero. If in L0/LNTC an immediate level transition to level 1/2/3 shall be also included (packet 41) |  |  | L1 |  |  | SH |  | - |  |
|  | 109 |  | 2 |  |  | 680.00 |  | L1 |  | SH |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 32: The train is in Shunting mode in L1, L2, or L3, and overpasses a balise with the packet number 132 (danger for shunting information) with the variable Q_ASPECT=0 ( which means :Stop if in SH mode)) AND (override is not active). Transition Condition Id [49]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 113 |  | 2 |  |  | 680.00 |  | L1 |  | SH |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=7) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | TR |  | - |  |
|  | Feature 4070201: 4.7.2: DMI versus Mode Table - Output Test Case 17: Indication of Trip mode and the reason to the driver. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 115 |  | 1 |  |  | 680.00 |  | L1 |  | TR |  |  | The Trip mode (TR) is permanently DISPLAYED |  | O |  | DMI |  | The mode indication may not be permanent if the driver uses the menus. |  |  | L1 |  |  | TR |  | - |  |
|  | 116 |  | 2 |  |  | 680.00 |  | L1 |  | TR |  |  | The reason (Trip Reason) is DISPLAYED |  | O |  | DMI |  | The Trip Reason is related to the conditions (SRS 4.6.3) that lead to the transition to TR. |  |  | L1 |  |  | TR |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 32: The train is in Shunting mode in L1, L2, or L3, and overpasses a balise with the packet number 132 (danger for shunting information) with the variable Q_ASPECT=0 ( which means :Stop if in SH mode)) AND (override is not active). Transition Condition Id [49]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 117 |  | 4 |  |  | 680.00 |  | L1 |  | TR |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit19=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | TR |  | - |  |
|  | 118 |  | 5 |  |  | 680.00 |  | L1 |  | TR |  |  | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23; SYSTEM_STATUS_MESSAGE=<Bit15=1>) is recorded |  | O |  | JRU |  | SH stop order |  |  | L1 |  |  | TR |  | - |  |
|  | 119 |  | 6 |  |  | 680.00 |  | L1 |  | TR |  |  | Emergency brake commanded |  | O |  | TIU |  | - |  |  | L1 |  |  | TR |  | - |  |
|  | 120 |  | 7 |  |  | 680.00 |  | L1 |  | TR |  |  | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=1) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | TR |  | - |  |
|  | 121 |  | 8 |  |  | 680.00 |  | L1 |  | TR |  |  | The Status symbol ' Emergency Brake ' is displayed |  | O |  | DMI |  | DMI Object: Symbol ST01 |  |  | L1 |  |  | TR |  | - |  |
|  | 122 |  | 9 |  |  | 680.00 |  | L1 |  | TR |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | TR |  | - |  |
|  | Feature 5110200: 5.11.2: Table of requirements for 'Train Trip' procedure Test Case 13: Before the train reaches standstill, no message to acknowledge Train Trip is displayed. The train reaches standstill in level 1, 2 or 3 and TR mode. The train/engine performs transition to PT after Driver acknowledgement of the TRIP mode. In level 2 or level 3, a communication session is opened and the On-Board sends a position report to the track-side when the mode changes Transition Id [7]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 123 |  | 1 |  |  | 700.00 |  | L1 |  | TR |  |  | No Request for driver acknowledgement to Train Trip is DISPLAYED |  | O |  | DMI |  | The train is still moving |  |  | L1 |  |  | TR |  | - |  |
|  | 124 |  | 2 |  |  | 720.00 |  | L1 |  | TR |  |  | The request for driver acknowledgement to Train Trip/ symbol MO05 is DISPLAYED |  | O |  | DMI |  | As soon as the train has come to standstill |  |  | L1 |  |  | TR |  | - |  |
|  | 125 |  | 3 |  |  | 720.00 |  | L1 |  | TR |  |  | The displayed symbol MO05is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  |  | TR |  | - |  |
|  | 126 |  | 4 |  |  | 720.00 |  | L1 |  | TR |  |  | The driver acknowledges the TR Mode |  | I |  | DMI |  | - |  |  | L1 |  |  | TR |  | - |  |
|  | 127 |  | 5 |  |  | 720.00 |  | L1 |  | TR |  |  | The driver acknowledgement is RECORDED. |  | O |  | JRU |  | - |  |  | L1 |  |  | TR |  | - |  |
|  | 128 |  | 6 |  |  | 720.00 |  | L1 |  | TR |  |  | The new mode PT/ symbol MO06 is DISPLAYED |  | O |  | DMI |  | The on-board equipment switches to PT mode. |  |  | L1 |  |  | PT |  | - |  |
|  | 129 |  | 7 |  |  | 720.00 |  | L1 |  | PT |  |  | The new mode PT/ symbol MO06 is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  |  | PT |  | - |  |
|  | 132 |  | 10 |  |  | 720.00 |  | L1 |  | PT |  |  | The reason (Trip Reason) is DISPLAYED |  | O |  | DMI |  | The trip reason is displayed while the train is in PT mode |  |  | L1 |  |  | PT |  | - |  |
|  | 133 |  | 11 |  |  | 720.00 |  | L1 |  | PT |  |  | Emergency brake not commanded |  | O |  | TIU |  | Release of emergency brake, because the train is at standstill and the driver has acknowledged the trip. |  |  | L1 |  |  | PT |  | - |  |
|  | 134 |  | 12 |  |  | 720.00 |  | L1 |  | PT |  |  | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=0) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | PT |  | - |  |
|  | 135 |  | 13 |  |  | 720.00 |  | L1 |  | PT |  |  | The Status symbol "Service Brake or Emergency Brake intervention" is removed |  | O |  | DMI |  | - |  |  | L1 |  |  | PT |  | - |  |
|  | 136 |  | 14 |  |  | 720.00 |  | L1 |  | PT |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=0>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | PT |  | - |  |
|  | Feature 5060200: 5.6.2: Table of requirements for 'Shunting Initiated by Driver' procedure Test Case 5: Driver selects 'Shunting', train is at standstill (Transition [5]) and the system switches to SH mode. An ongoing mission exists.'End of Mission' procedure triggered. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 137 |  | 1 |  |  | 720.00 |  | L1 |  | PT |  |  | The driver presses 'shunting' button |  | I |  | DMI |  | - |  |  | L1 |  |  | PT |  | - |  |
|  | 138 |  | 2 |  |  | 720.00 |  | L1 |  | PT |  |  | Driver selection is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  |  | PT |  | - |  |
|  | 139 |  | 3 |  |  | 720.00 |  | L1 |  | PT |  |  | The SH symbol is DISPLAYED |  | O |  | DMI |  | The system switches to SH SH symbol is MO01( according to the document ERA_ERTMS_015560 v330). |  |  | L1 |  |  | SH |  | - |  |
|  | 140 |  | 4 |  |  | 720.00 |  | L1 |  | SH |  |  | The transition to SH mode is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 141 |  | 5 |  |  | 720.00 |  | L1 |  | SH |  |  | The change of permitted speed is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 142 |  | 6 |  |  | 720.00 |  | L1 |  | SH |  |  | Driver requests the permitted speed |  | I |  | DMI |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 143 |  | 7 |  |  | 720.00 |  | L1 |  | SH |  |  | Driver request is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 144 |  | 8 |  |  | 720.00 |  | L1 |  | SH |  |  | The permitted speed is DISPLAYED |  | O |  | DMI |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | Feature 5080400: 5.8.4: End of Override procedure Test Case 2: The driver selects Override. When a new value for T_NVOVTRP is received on-board (by means of Packet 3) and it becomes applicable, the ongoing 'Override' supervision will use this new value. However, the starting time shall remain unchanged. The Override procedure ends when the "max. time for train trip suppression when Override function is triggered" elapses |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 145 |  | 1 |  |  | 760.00 |  | L1 |  | SH |  |  | The driver selects "Override EOA" |  | I |  | DMI |  | The train speed is below max. speed for override (V_NVALLOWOVTRP) |  |  | L1 |  |  | SH |  | - |  |
|  | 146 |  | 2 |  |  | 760.00 |  | L1 |  | SH |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=14) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 147 |  | 3 |  |  | 760.00 |  | L1 |  | SH |  |  | The Mode symbol "Override EOA is active" is displayed |  | O |  | DMI |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 148 |  | 4 |  |  | 760.00 |  | L1 |  | SH |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit18=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 149 |  | 5 |  |  | 800.00 |  | L1 |  | SH |  |  | Packet 3: National Values, Including T_NVOVTRP & a correct value for NID_C is RECEIVED. |  | I |  | BTM |  | Choose the BTM or the RTM step. T_NVOVTRP will be different from the one currently being used |  |  | L1 |  |  | SH |  | - |  |
|  | 151 |  | 7 |  |  | 800.00 |  | L1 |  | SH |  |  | Packet 3: National Values, Including T_NVOVTRP & a correct value for NID_C is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 152 |  | 8 |  |  | 870.00 |  | L1 |  | SH |  |  | As soon as t > t0 +T_NVOVTRP the 'override status' is REMOVED |  | O |  | DMI |  | The received set of NV are applicable. t is the current time t0 is the time when the 'Override EOA' button was triggered _____ T_NVOVTRP = 22s |  |  | L1 |  |  | SH |  | - |  |
|  | 153 |  | 9 |  |  | 870.00 |  | L1 |  | SH |  |  | The cancellation of override status is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | Feature 4080408: 4.8.4: Accepted Information depending on the modes - Danger for SH information Test Case 1: The train is in shunting mode and overpasses a balise with the packet number 132 (danger for shunting information). The train will be tripped if it’s in shunting mode. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 156 |  | 1 |  |  | 890.00 |  | L1 |  | SH |  |  | One Balise Group message (N_TOTAL=0; NID_C=<comment>) containing packet 132 "Danger for shunting information" (Q_ASPETC = 0) is received |  | I |  | BTM |  | One of the balises in BGa has a packet number 132 (Danger for shunting information) with the variable Q_ASPECT set to zero. If in L0/LNTC an immediate level transition to level 1/2/3 shall be also included (packet 41) |  |  | L1 |  |  | SH |  | - |  |
|  | 157 |  | 2 |  |  | 890.00 |  | L1 |  | SH |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 32: The train is in Shunting mode in L1, L2, or L3, and overpasses a balise with the packet number 132 (danger for shunting information) with the variable Q_ASPECT=0 ( which means :Stop if in SH mode)) AND (override is not active). Transition Condition Id [49]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 161 |  | 2 |  |  | 890.00 |  | L1 |  | SH |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=7) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | TR |  | - |  |
|  | Feature 4070201: 4.7.2: DMI versus Mode Table - Output Test Case 17: Indication of Trip mode and the reason to the driver. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 163 |  | 1 |  |  | 890.00 |  | L1 |  | TR |  |  | The Trip mode (TR) is permanently DISPLAYED |  | O |  | DMI |  | The mode indication may not be permanent if the driver uses the menus. |  |  | L1 |  |  | TR |  | - |  |
|  | 164 |  | 2 |  |  | 890.00 |  | L1 |  | TR |  |  | The reason (Trip Reason) is DISPLAYED |  | O |  | DMI |  | The Trip Reason is related to the conditions (SRS 4.6.3) that lead to the transition to TR. |  |  | L1 |  |  | TR |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 32: The train is in Shunting mode in L1, L2, or L3, and overpasses a balise with the packet number 132 (danger for shunting information) with the variable Q_ASPECT=0 ( which means :Stop if in SH mode)) AND (override is not active). Transition Condition Id [49]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 165 |  | 4 |  |  | 890.00 |  | L1 |  | TR |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit19=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | TR |  | - |  |
|  | 166 |  | 5 |  |  | 890.00 |  | L1 |  | TR |  |  | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23; SYSTEM_STATUS_MESSAGE=<Bit15=1>) is recorded |  | O |  | JRU |  | SH stop order |  |  | L1 |  |  | TR |  | - |  |
|  | 167 |  | 6 |  |  | 890.00 |  | L1 |  | TR |  |  | Emergency brake commanded |  | O |  | TIU |  | - |  |  | L1 |  |  | TR |  | - |  |
|  | 168 |  | 7 |  |  | 890.00 |  | L1 |  | TR |  |  | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=1) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | TR |  | - |  |
|  | 169 |  | 8 |  |  | 890.00 |  | L1 |  | TR |  |  | The Status symbol ' Emergency Brake ' is displayed |  | O |  | DMI |  | DMI Object: Symbol ST01 |  |  | L1 |  |  | TR |  | - |  |
|  | 170 |  | 9 |  |  | 890.00 |  | L1 |  | TR |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | TR |  | - |  |
|  | Feature 4080426: 4.8.4: Accepted Information depending on the modes - National Values Test Case 4: Transmission of National Values via balise group |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 171 |  | 1 |  |  | 890.00 |  | L1 |  | TR |  |  | One Balise Group message containing packet 3 is received |  | I |  | BTM |  | - _____ reset NV |  |  | L1 |  |  | TR |  | - |  |
|  | 172 |  | 2 |  |  | 890.00 |  | L1 |  | TR |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | TR |  | - |  |
|  | Feature 5110200: 5.11.2: Table of requirements for 'Train Trip' procedure Test Case 13: Before the train reaches standstill, no message to acknowledge Train Trip is displayed. The train reaches standstill in level 1, 2 or 3 and TR mode. The train/engine performs transition to PT after Driver acknowledgement of the TRIP mode. In level 2 or level 3, a communication session is opened and the On-Board sends a position report to the track-side when the mode changes Transition Id [7]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 174 |  | 1 |  |  | 895.00 |  | L1 |  | TR |  |  | No Request for driver acknowledgement to Train Trip is DISPLAYED |  | O |  | DMI |  | The train is still moving |  |  | L1 |  |  | TR |  | - |  |
|  | 175 |  | 2 |  |  | 905.00 |  | L1 |  | TR |  |  | The request for driver acknowledgement to Train Trip/ symbol MO05 is DISPLAYED |  | O |  | DMI |  | As soon as the train has come to standstill |  |  | L1 |  |  | TR |  | - |  |
|  | 176 |  | 3 |  |  | 905.00 |  | L1 |  | TR |  |  | The displayed symbol MO05is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  |  | TR |  | - |  |
|  | 177 |  | 4 |  |  | 905.00 |  | L1 |  | TR |  |  | The driver acknowledges the TR Mode |  | I |  | DMI |  | - |  |  | L1 |  |  | TR |  | - |  |
|  | 178 |  | 5 |  |  | 905.00 |  | L1 |  | TR |  |  | The driver acknowledgement is RECORDED. |  | O |  | JRU |  | - |  |  | L1 |  |  | TR |  | - |  |
|  | 179 |  | 6 |  |  | 905.00 |  | L1 |  | TR |  |  | The new mode PT/ symbol MO06 is DISPLAYED |  | O |  | DMI |  | The on-board equipment switches to PT mode. |  |  | L1 |  |  | PT |  | - |  |
|  | 180 |  | 7 |  |  | 905.00 |  | L1 |  | PT |  |  | The new mode PT/ symbol MO06 is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  |  | PT |  | - |  |
|  | 183 |  | 10 |  |  | 905.00 |  | L1 |  | PT |  |  | The reason (Trip Reason) is DISPLAYED |  | O |  | DMI |  | The trip reason is displayed while the train is in PT mode |  |  | L1 |  |  | PT |  | - |  |
|  | 184 |  | 11 |  |  | 905.00 |  | L1 |  | PT |  |  | Emergency brake not commanded |  | O |  | TIU |  | Release of emergency brake, because the train is at standstill and the driver has acknowledged the trip. |  |  | L1 |  |  | PT |  | - |  |
|  | 185 |  | 12 |  |  | 905.00 |  | L1 |  | PT |  |  | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=0) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | PT |  | - |  |
|  | 186 |  | 13 |  |  | 905.00 |  | L1 |  | PT |  |  | The Status symbol "Service Brake or Emergency Brake intervention" is removed |  | O |  | DMI |  | - |  |  | L1 |  |  | PT |  | - |  |
|  | 187 |  | 14 |  |  | 905.00 |  | L1 |  | PT |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=0>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | PT |  | - |  |
|  | Feature 9990200: A3.2: List of National / Default Data Test Case 26: Check the ability the on-board to enable reverse movements limited by national value in Post Trip mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 188 |  | 1 |  |  | 905.00 |  | L1 |  | PT |  |  | The driver changes the position of the direction controller and starts moving backwards. The train Trip is inhibited |  | I |  | TIU |  | - |  |  | L1 |  |  | PT |  | - |  |
|  | 189 |  | 2 |  |  | 705.00 |  | L1 |  | PT |  |  | The train movement exceeds the national/default value for the backwards movement. |  | I |  | INT |  | - |  |  | L1 |  |  | PT |  | - |  |
|  | 190 |  | 3 |  |  | 705.00 |  | L1 |  | PT |  |  | The Service brake is APPLIED. |  | O |  | TIU |  | The brake command is triggered at Reverse Movement Protection after a distance specified by the national/default (default value = 200m) Current mode is UNCHANGED |  |  | L1 |  |  | PT |  | - |  |
|  | 191 |  | 4 |  |  | 705.00 |  | L1 |  | PT |  |  | The brake indication is DISPLAYED. |  | O |  | DMI |  | - |  |  | L1 |  |  | PT |  | - |  |
|  | 192 |  | 5 |  |  | 705.00 |  | L1 |  | PT |  |  | The brake intervention is RECORDED. |  | O |  | JRU |  | - |  |  | L1 |  |  | PT |  | - |  |
|  | 193 |  | 6 |  |  | 705.00 |  | L1 |  | PT |  |  | The train reaches standstill. |  | I |  | INT |  | - |  |  | L1 |  |  | PT |  | - |  |
|  | 194 |  | 7 |  |  | 705.00 |  | L1 |  | PT |  |  | The driver acknowledgement request is DISPLAYED. |  | O |  | DMI |  | - |  |  | L1 |  |  | PT |  | - |  |
|  | 195 |  | 8 |  |  | 705.00 |  | L1 |  | PT |  |  | The driver acknowledges. |  | I |  | DMI |  | - |  |  | L1 |  |  | PT |  | - |  |
|  | 196 |  | 9 |  |  | 705.00 |  | L1 |  | PT |  |  | The driver acknowledgement is RECORDED. |  | O |  | JRU |  | - |  |  | L1 |  |  | PT |  | - |  |
|  | 197 |  | 10 |  |  | 705.00 |  | L1 |  | PT |  |  | The driver acknowledgement request is REMOVED. |  | O |  | DMI |  | - |  |  | L1 |  |  | PT |  | - |  |
|  | 198 |  | 11 |  |  | 705.00 |  | L1 |  | PT |  |  | The brake is RELEASED. |  | O |  | TIU |  | Current mode is UNCHANGED |  |  | L1 |  |  | PT |  | - |  |
|  | 199 |  | 12 |  |  | 705.00 |  | L1 |  | PT |  |  | The brake indication is REMOVED. |  | O |  | DMI |  | - |  |  | L1 |  |  | PT |  | - |  |
|  | 200 |  | 13 |  |  | 705.00 |  | L1 |  | PT |  |  | The brake release is RECORDED. |  | O |  | JRU |  | - |  |  | L1 |  |  | PT |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 1: The ETCS on-board system is NOT powered. Transition Condition Id [29]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 201 |  | 1 |  |  | 705.00 |  | L1 |  | PT |  |  | The ERTMS/ETCS on-board unit is powered down |  | I |  | INT |  | - _____ The train reached standstill. |  |  | L1 |  |  | NP |  | - |  |
|  | 202 |  | 2 |  |  | 705.00 |  | L1 |  | NP |  |  | NOT GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=-) is recorded |  | O |  | JRU |  | There are no messages recorded in the JRU during a period of time |  |  | L1 |  |  | NP |  | - |  |
| Step 10: DMI Events Distance 0.00 m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Event Id. |  |  |  | Event Name |  |  |  |  |  |  | Data |  |  |  |  |  |  |  |  | Delay |  |  |  |  |  |  |  |  |
| 29 |  |  |  | Request Change Level |  |  |  |  |  |  | LEVEL=1 |  |  |  |  |  |  |  |  | 10 s |  |  |  |  |  |  |  |  |

| - Step 40 : Balise Group BG 00000000000040 1/2 - Distance 50.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 0001000000 b |  |  |
| NID_BG | 14 | 00000000000001 b |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 3 d | National values |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 0000011100110 b |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_VALIDNV | 15 | 000000001100100 b |  |  |
| NID_C | 10 | 0001000000 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| V_NVSHUNT | 7 | 0000110 b |  |  |
| V_NVSTFF | 7 | 0001100 b |  |  |
| V_NVONSIGHT | 7 | 0000110 b |  |  |
| V_NVLIMSUPERV | 7 | 0010100 b |  |  |
| V_NVUNFIT | 7 | 0010100 b |  |  |
| V_NVREL | 7 | 0001000 b |  |  |
| D_NVROLL | 15 | 000000000000010 b |  |  |
| Q_NVSBTSMPERM | 1 | 1 b |  |  |
| Q_NVEMRRLS | 1 | 0 b |  |  |
| Q_NVGUIPERM | 1 | 0 b |  |  |
| Q_NVSBFBPERM | 1 | 0 b |  |  |
| Q_NVINHSMICPERM | 1 | 0 b |  |  |
| V_NVALLOWOVTRP | 7 | 0001000 b |  |  |
| V_NVSUPOVTRP | 7 | 0001000 b |  |  |
| D_NVOVTRP | 15 | 000001001011000 b |  |  |
| T_NVOVTRP | 8 | 01111000 b |  |  |
| D_NVPOTRP | 15 | 000000011001000 b |  |  |
| M_NVCONTACT | 2 | 10 b |  |  |
| T_NVCONTACT | 8 | 11111111 b |  |  |
| M_NVDERUN | 1 | 1 b |  |  |
| D_NVSTFF | 15 | 000001111101000 b |  |  |
| Q_NVDRIVER_ADHES | 1 | 0 b |  |  |
| A_NVMAXREDADH1 | 6 | 010100 b |  |  |
| A_NVMAXREDADH2 | 6 | 001110 b |  |  |
| A_NVMAXREDADH3 | 6 | 001110 b |  |  |
| Q_NVLOCACC | 6 | 001100 b |  |  |
| M_NVAVADH | 5 | 00001 b |  |  |
| M_NVEBCL | 4 | 0001 b |  |  |
| Q_NVKINT | 1 | 0 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 40 : Balise Group BG 00000000000040 2/2 - Distance 55.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 0001000000 b |  |  |
| NID_BG | 14 | 00000000000001 b |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 54 : Balise Group BG 00000000000054 1/2 - Distance 300.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 0001000000 b |  |  |
| NID_BG | 14 | 00000000000010 b |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 3 d | National values |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 0000011100110 b |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_VALIDNV | 15 | 000000000000000 b |  |  |
| NID_C | 10 | 0001000000 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| V_NVSHUNT | 7 | 0001010 b |  |  |
| V_NVSTFF | 7 | 0001000 b |  |  |
| V_NVONSIGHT | 7 | 0000110 b |  |  |
| V_NVLIMSUPERV | 7 | 0010100 b |  |  |
| V_NVUNFIT | 7 | 0010100 b |  |  |
| V_NVREL | 7 | 0001000 b |  |  |
| D_NVROLL | 15 | 000000000000010 b |  |  |
| Q_NVSBTSMPERM | 1 | 1 b |  |  |
| Q_NVEMRRLS | 1 | 0 b |  |  |
| Q_NVGUIPERM | 1 | 0 b |  |  |
| Q_NVSBFBPERM | 1 | 0 b |  |  |
| Q_NVINHSMICPERM | 1 | 0 b |  |  |
| V_NVALLOWOVTRP | 7 | 0001000 b |  |  |
| V_NVSUPOVTRP | 7 | 0001000 b |  |  |
| D_NVOVTRP | 15 | 000000011001000 b |  |  |
| T_NVOVTRP | 8 | 00110010 b |  |  |
| D_NVPOTRP | 15 | 000000011001000 b |  |  |
| M_NVCONTACT | 2 | 10 b |  |  |
| T_NVCONTACT | 8 | 11111111 b |  |  |
| M_NVDERUN | 1 | 1 b |  |  |
| D_NVSTFF | 15 | 111111111111111 b |  |  |
| Q_NVDRIVER_ADHES | 1 | 0 b |  |  |
| A_NVMAXREDADH1 | 6 | 010100 b |  |  |
| A_NVMAXREDADH2 | 6 | 001110 b |  |  |
| A_NVMAXREDADH3 | 6 | 001110 b |  |  |
| Q_NVLOCACC | 6 | 001100 b |  |  |
| M_NVAVADH | 5 | 00001 b |  |  |
| M_NVEBCL | 4 | 0001 b |  |  |
| Q_NVKINT | 1 | 0 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 54 : Balise Group BG 00000000000054 2/2 - Distance 305.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 0001000000 b |  |  |
| NID_BG | 14 | 00000000000010 b |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 60 : Balise Group BG 00000000000060 1/2 - Distance 430.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 0001000000 b |  |  |
| NID_BG | 14 | 00000000000011 b |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 137 d | Stop if in staff responsible |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 0000000011000 b |  |  |
| Q_SRSTOP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 60 : Balise Group BG 00000000000060 2/2 - Distance 435.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 0001000000 b |  |  |
| NID_BG | 14 | 00000000000011 b |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 99 : Balise Group BG 00000000000199 1/2 - Distance 630.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 0001000000 b |  |  |
| NID_BG | 14 | 00000000000100 b |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 132 d | Danger for shunting information |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 0000000011000 b |  |  |
| Q_ASPECT | 1 | 0 b |  |  |
| NID_PACKET | 8 | 41 d | Level transition order |  |
| Q_DIR | 2 | 00 b |  |  |
| L_PACKET | 13 | 0000000000000 b |  |  |
| Q_SCALE | 2 | 00 b |  |  |
| D_LEVELTR | 15 | 111111111111111 b |  |  |
| M_LEVELTR | 3 | 000 b |  |  |
| L_ACKLEVELTR | 15 | 000000000000000 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 99 : Balise Group BG 00000000000199 2/2 - Distance 635.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 0001000000 b |  |  |
| NID_BG | 14 | 00000000000100 b |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 108 : Balise Group BG 5 d 1/2 - Distance 680.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 5 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 132 d | Danger for shunting information |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 24 d |  |  |
| Q_ASPECT | 1 | 0 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 108 : Balise Group BG 5 d 2/2 - Distance 685.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 5 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 149 : Balise Group BG 000000000001149 1/2 - Distance 800.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 0001000000 b |  |  |
| NID_BG | 14 | 00000000000110 b |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 3 d | National values |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 0000011100110 b |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_VALIDNV | 15 | 000000000000000 b |  |  |
| NID_C | 10 | 0001000000 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| V_NVSHUNT | 7 | 0000110 b |  |  |
| V_NVSTFF | 7 | 0001000 b |  |  |
| V_NVONSIGHT | 7 | 0000110 b |  |  |
| V_NVLIMSUPERV | 7 | 0010100 b |  |  |
| V_NVUNFIT | 7 | 0010100 b |  |  |
| V_NVREL | 7 | 0001000 b |  |  |
| D_NVROLL | 15 | 000000000000010 b |  |  |
| Q_NVSBTSMPERM | 1 | 1 b |  |  |
| Q_NVEMRRLS | 1 | 0 b |  |  |
| Q_NVGUIPERM | 1 | 0 b |  |  |
| Q_NVSBFBPERM | 1 | 0 b |  |  |
| Q_NVINHSMICPERM | 1 | 0 b |  |  |
| V_NVALLOWOVTRP | 7 | 0001000 b |  |  |
| V_NVSUPOVTRP | 7 | 0001000 b |  |  |
| D_NVOVTRP | 15 | 000001001011000 b |  |  |
| T_NVOVTRP | 8 | 00010110 b |  |  |
| D_NVPOTRP | 15 | 000000011001000 b |  |  |
| M_NVCONTACT | 2 | 10 b |  |  |
| T_NVCONTACT | 8 | 11111111 b |  |  |
| M_NVDERUN | 1 | 1 b |  |  |
| D_NVSTFF | 15 | 111111111111111 b |  |  |
| Q_NVDRIVER_ADHES | 1 | 0 b |  |  |
| A_NVMAXREDADH1 | 6 | 010100 b |  |  |
| A_NVMAXREDADH2 | 6 | 001110 b |  |  |
| A_NVMAXREDADH3 | 6 | 001110 b |  |  |
| Q_NVLOCACC | 6 | 001100 b |  |  |
| M_NVAVADH | 5 | 00001 b |  |  |
| M_NVEBCL | 4 | 0001 b |  |  |
| Q_NVKINT | 1 | 0 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 149 : Balise Group BG 000000000001149 2/2 - Distance 805.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 0001000000 b |  |  |
| NID_BG | 14 | 00000000000110 b |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 156 : Balise Group BG 7 d 1/2 - Distance 890.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 7 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 132 d | Danger for shunting information |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 24 d |  |  |
| Q_ASPECT | 1 | 0 b |  |  |
| NID_PACKET | 8 | 3 d | National values |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 230 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_VALIDNV | 15 | 0 d |  |  |
| NID_C | 10 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| V_NVSHUNT | 7 | 6 d |  |  |
| V_NVSTFF | 7 | 8 d |  |  |
| V_NVONSIGHT | 7 | 6 d |  |  |
| V_NVLIMSUPERV | 7 | 20 d |  |  |
| V_NVUNFIT | 7 | 20 d |  |  |
| V_NVREL | 7 | 8 d |  |  |
| D_NVROLL | 15 | 2 d |  |  |
| Q_NVSBTSMPERM | 1 | 1 b |  |  |
| Q_NVEMRRLS | 1 | 0 b |  |  |
| Q_NVGUIPERM | 1 | 0 b |  |  |
| Q_NVSBFBPERM | 1 | 0 b |  |  |
| Q_NVINHSMICPERM | 1 | 0 b |  |  |
| V_NVALLOWOVTRP | 7 | 0 d |  |  |
| V_NVSUPOVTRP | 7 | 6 d |  |  |
| D_NVOVTRP | 15 | 200 d |  |  |
| T_NVOVTRP | 8 | 60 d |  |  |
| D_NVPOTRP | 15 | 200 d |  |  |
| M_NVCONTACT | 2 | 10 b |  |  |
| T_NVCONTACT | 8 | 255 d |  |  |
| M_NVDERUN | 1 | 1 b |  |  |
| D_NVSTFF | 15 | 32767 d |  |  |
| Q_NVDRIVER_ADHES | 1 | 0 b |  |  |
| A_NVMAXREDADH1 | 6 | 010100 b |  |  |
| A_NVMAXREDADH2 | 6 | 001110 b |  |  |
| A_NVMAXREDADH3 | 6 | 001110 b |  |  |
| Q_NVLOCACC | 6 | 12 d |  |  |
| M_NVAVADH | 5 | 00000 b |  |  |
| M_NVEBCL | 4 | 1001 b |  |  |
| Q_NVKINT | 1 | 0 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 156 : Balise Group BG 7 d 2/2 - Distance 895.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 7 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |