| - ERTMS/ETCS |  |
| --- | --- |
| Subset-076-6-3_9990500_24_v320.mdb State : Ordered list of Test Case Steps with communication information |  |
| REF : | Subset-076-6-3_9990500_24_v320.mdb |
| ISSUE : | 3.2.0 |
| DATE : | 10/04/17 16:24 |

| Company | Technical Approval | Management approval |
| --- | --- | --- |
| European Union Agency for Railways | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 3.2.0 10.04.2017 | All |  | RINA |

# Sequence description

# Test cases

Important:	All columns called ‘Test Results’ are only placeholders for results of the test execution. They shall not be used for the description of test cases. 
A test is passed if all cells of ‘Test Result’ are passed.

|  | SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Step | Row |  |  | Dist. (m) |  | Previous |  |  |  |  | Description of Events |  | I/O |  | Interface |  | Comments |  |  | Next |  |  |  |  | Test Result |  |  |
|  |  |  |  |  |  |  | Levels |  | Mode |  |  |  |  |  |  |  |  |  |  |  | Levels |  |  | Mode |  |  |  |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 10: The ETCS on-board equipment is powered .Transition Condition Id [4]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 |  | 1 |  |  | 0.00 |  | N/A |  | NP |  |  | ERTMS/ETCS on-board unit is powered up |  | I |  | INT |  | The on-board equipment changes to SB mode. |  |  | N/A |  |  | NP |  | - |  |
|  | 2 |  | 2 |  |  | 0.00 |  | N/A |  | NP |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=6) is recorded |  | O |  | JRU |  | - |  |  | N/A |  |  | NP |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 24: Start of Mission procedure is engaged |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 3 |  | 1 |  |  | 0.00 |  | N/A |  | NP |  |  | Driver opens desk |  | I |  | TIU |  | - |  |  | N/A |  |  | NP |  | - |  |
|  | 4 |  | 2 |  |  | 0.00 |  | N/A |  | NP |  |  | The 'CAB STATUS' message is RECORDED with the variable M_CAB_A_STATUS=1 (Active). |  | O |  | JRU |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 5 |  | 3 |  |  | 0.00 |  | N/A |  | SB |  |  | The actual mode SB is DISPLAYED |  | O |  | DMI |  | The SB indication after opening the desk may not be recorded. Because the change of mode was already recorded while desk was closed |  |  | N/A |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 7: The on-board equipment requests the driver to enter a driver ID within the scope of the Start of Mission procedure in SB mode when the status of the driver ID is unknown. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 6 |  | 1 |  |  | 0.00 |  | N/A |  | SB |  |  | The on-board equipment requires the driver to enter a Driver ID There is NO initial Driver ID value displayed |  | O |  | DMI |  | Starting conditions : There is no value initially displayed since the status of the driver ID is unknown. |  |  | N/A |  |  | SB |  | - |  |
|  | 7 |  | 2 |  |  | 0.00 |  | N/A |  | SB |  |  | The on-board equipment prevents the driver from continuing the procedure before the validation of the Driver ID is done. |  | O |  | DMI |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 8 |  | 3 |  |  | 0.00 |  | N/A |  | SB |  |  | The driver enters and validates a correct Driver ID |  | I |  | DMI |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 9 |  | 4 |  |  | 0.00 |  | N/A |  | SB |  |  | The validated Driver ID is recorded on the JRU with the DRIVER_ID variable in a 'GENERAL MESSAGE'. |  | O |  | JRU |  | A 'GENERAL MESSAGE' is recorded every 5s by the on-board equipment. |  |  | N/A |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 10: Within the scope of the Start of Mission procedure in SB mode, the status of the ERTMS/ETCS Level data is INVALID. The driver selects a level (the existing ERTMS/ETCS Level or a new one). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 10 |  | 1 |  |  | 0.00 |  | N/A |  | SB |  |  | The driver selects "Level <L0/LNTC/L1/L2/L3>" |  | I |  | DMI |  | Depending on the implementation, a list of available levels may be presented to the driver on the DMI. The driver is then allowed to select one level among the proposed levels. For NTC levels, the abbreviations of the concerned National Systems shall be presented to the driver. _____ Driver select Level 1 [59.-1.1] |  |  | L1 |  |  | SB |  | - |  |
|  | 11 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=<34/35/36/37/38>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 12 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=<0/1/2/3/4>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 6: Within the scope of the Start of Mission procedure in SB mode, the On-Board equipment offers the driver to select either SH or NL or ‘Train Data Entry’ before the on-board equipment requests the driver to enter a complete set of Train Data. The driver selects ‘Train Data Entry’. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 13 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver can select either SH or NL or ‘Train Data Entry’ |  | O |  | DMI |  | Starting conditions The 'Non-Leading' button is enabled if the 'non leading' input signal is received. |  |  | L1 |  |  | SB |  | - |  |
|  | 14 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver selects ‘Train Data Entry’ |  | I |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 15 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | A Train data window is is displayed on the DMI. |  | O |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 16 |  | 4 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver’s selection is recorded on the JRU. |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 47: The on-board equipment requests the driver to enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when the status of the Train Data is unknown (S12) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 18 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The on-board equipment requires the driver to enter all Train Data unknown by the on-board equipment and prevents the driver from performing any other actions. |  | O |  | DMI |  | The driver might be requested to enter the 'Train category(ies)', 'Train length', 'Traction / brake parameters', 'Maximum train speed', 'Loading gauge', 'Axle load category', 'Train fitted with airtight system', but the driver shall never be requested to enter the 'Traction system(s) accepted by the engine', 'List of National Systems available on-board' or 'Axle number'. |  |  | L1 |  |  | SB |  | - |  |
|  | 19 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver enters each Train data unknown on-board |  | I |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 20 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver is allowed to validate the train data |  | O |  | DMI |  | A complete set of valid Train Data is available |  |  | L1 |  |  | SB |  | - |  |
|  | 21 |  | 4 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver validates the train data. |  | I |  | DMI |  | The on-board equipment allows the driver to validate as soon as a valid and complete set of Train Data is entered. |  |  | L1 |  |  | SB |  | - |  |
|  | 22 |  | 5 |  |  | 0.00 |  | L1 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded |  | O |  | JRU |  | Triggered by : The driver validation. |  |  | L1 |  |  | SB |  | - |  |
|  | 23 |  | 6 |  |  | 0.00 |  | L1 |  | SB |  |  | TRAIN DATA (NID_MESSAGE_JRU=2) is recorded |  | O |  | JRU |  | Triggered by : Data Entry completed |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 46: After the set of Train Data has been entered/re-validated by the driver (in S12), if the status of the Train running number is 'unknown', the ERTMS/ETCS on-board equipment requests the driver to enter the Train running number (S13) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 25 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver is requested to enter the Train running number. |  | O |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 26 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver enters the "Train Running Number" [MAIN] |  | I |  | DMI |  | Entered Train Running Number: NID_OPERATIONAL_1 |  |  | L1 |  |  | SB |  | - |  |
|  | 27 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | TRAIN RUNNING NUMBER ENTERED BY THE DRIVER (NID_MESSAGE_JRU=49; NID_OPERATIONAL=NID_OPERATIONAL_1) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 1: The on-board equipment requests the driver to re-validate/enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when level is 0/1/NTC (Subset-026-5.4.3.2 S12). The driver is requested to enter/re-validate the Train running number if the status of the Train running number is 'unknown' or 'invalid'. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 28 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | The DMI allows the driver to select the option ‘START’. |  | O |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 16: The driver selects 'Start' within the scope of the Start of Mission procedure in SB mode and in ERTMS Level 1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 29 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver selects 'Start'. |  | I |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 30 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | Driver’s selection of 'Start' is recorded on the JRU |  | O |  | JRU |  | Triggered by: The driver acts on the on-board DMI. |  |  | L1 |  |  | SB |  | - |  |
|  | 31 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | An acknowledgement request for running in Staff Responsible mode is displayed on the DMI |  | O |  | DMI |  | The on-board equipment will then waits for the driver to acknowledge the acknowledgement request for running in Staff Responsible mode |  |  | L1 |  |  | SB |  | - |  |
|  | 32 |  | 4 |  |  | 0.00 |  | L1 |  | SB |  |  | The acknowledgement request for running in Staff Responsible mode is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 33 |  | 5 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver acknowledges 'Staff Responsible' |  | I |  | DMI |  | Symbol MO10 |  |  | L1 |  |  | SB |  | - |  |
|  | 34 |  | 6 |  |  | 0.00 |  | L1 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS= 0000 0011) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 35 |  | 7 |  |  | 0.00 |  | L1 |  | SB |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=2) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 36 |  | 8 |  |  | 0.00 |  | L1 |  | SR |  |  | The Mode symbol 'Staff Responsible' is displayed |  | O |  | DMI |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 37 |  | 9 |  |  | 0.00 |  | L1 |  | SR |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit24=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 38 |  | 10 |  |  | 0.00 |  | L1 |  | SR |  |  | Any message (NID_MESSAGE_JRU=ALL; NID_LRBG=16777215) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | Feature 3160200: 3.16.2: Balises Test Case 10: No linking information is used on-board -A BG marked as linked is found -No message consistency problem The message is accepted. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 39 |  | 1 |  |  | 50.00 |  | L1 |  | SR |  |  | One Balise Group message (N_TOTAL=1;Q_LINK=1) containing packet 72, packet 255 is received |  | I |  | BTM |  | No linking information is used on-board The train overpasses all the balises in BG The BG is marked as linked No message consistency problem: All the telegrams are correctly received. |  |  | L1 |  |  | SR |  | - |  |
|  | 40 |  | 2 |  |  | 50.00 |  | L1 |  | SR |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded. |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 41 |  | 3 |  |  | 50.00 |  | L1 |  | SR |  |  | The "plain text message" is displayed |  | O |  | DMI |  | Only if mode not PS, SH, SL or PT (with exception of L2/L3 and following reception of 'Recognition of Exit from TR mode' with a more recent time stamp). The BG message is accepted. |  |  | L1 |  |  | SR |  | - |  |
|  | 42 |  | 4 |  |  | 50.00 |  | L1 |  | SR |  |  | START DISPLAYING PLAIN TEXT MESSAGE (NID_MESSAGE_JRU=18) is recorded |  | O |  | JRU |  | Only if mode not PS, SH, SL or PT (with exception of L2/L3 and following reception of 'Recognition of Exit from TR mode' with a more recent time stamp). |  |  | L1 |  |  | SR |  | - |  |
|  | 43 |  | 5 |  |  | 50.00 |  | L1 |  | SR |  |  | The "plain text message" is removed |  | O |  | DMI |  | Only if mode not PS, SH, SL or PT (with exception of L2/L3 and following reception of 'Recognition of Exit from TR mode' with a more recent time stamp). |  |  | L1 |  |  | SR |  | - |  |
|  | 44 |  | 6 |  |  | 50.00 |  | L1 |  | SR |  |  | STOP DISPLAYING PLAIN TEXT MESSAGE (NID_MESSAGE_JRU=19) is recorded |  | O |  | JRU |  | Only if mode not PS, SH, SL or PT (with exception of L2/L3 and following reception of 'Recognition of Exit from TR mode' with a more recent time stamp). |  |  | L1 |  |  | SR |  | - |  |
|  | Feature 9990500: A3.5: Handling of Actions in Specific Situations Test Case 1: The on-board equipment shall ensure that the change of National Values, which is ordered from the trackside for a specific location, is not reverted when passing over the location again moving backwards. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 45 |  | 1 |  |  | 300.00 |  | L1 |  | SR |  |  | Train is moving |  | I |  | INT |  | The train passes over the location where the last stored NV shall start being valid (defined by D_VALIDNV) The National Value M_NVDERUN changes from 0 to 1 |  |  | L1 |  |  | SR |  | - |  |
|  | 46 |  | 2 |  |  | 300.00 |  | L1 |  | SR |  |  | The driver selects 'Main' |  | I |  | DMI |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 47 |  | 3 |  |  | 300.00 |  | L1 |  | SR |  |  | The Main window is displayed |  | O |  | DMI |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 48 |  | 4 |  |  | 300.00 |  | L1 |  | SR |  |  | The driver selects 'Driver ID' |  | I |  | DMI |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 49 |  | 5 |  |  | 300.00 |  | L1 |  | SR |  |  | The Driver ID window is displayed |  | O |  | DMI |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 50 |  | 6 |  |  | 300.00 |  | L1 |  | SR |  |  | The driver modifies the "Driver ID" |  | I |  | DMI |  | Driver ID = ID1 |  |  | L1 |  |  | SR |  | - |  |
|  | 51 |  | 7 |  |  | 300.00 |  | L1 |  | SR |  |  | The driver validates the "Driver ID" |  | I |  | DMI |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 52 |  | 8 |  |  | 300.00 |  | L1 |  | SR |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; DRIVER_ID=ID1) is recorded |  | O |  | JRU |  | The new Driver ID value is recorded |  |  | L1 |  |  | SR |  | - |  |
|  | 53 |  | 9 |  |  | 300.00 |  | L1 |  | SR |  |  | The driver closes the "Main" window |  | I |  | DMI |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 54 |  | 10 |  |  | 300.00 |  | L1 |  | SR |  |  | The Default window is displayed |  | O |  | DMI |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 55 |  | 11 |  |  | 350.00 |  | L1 |  | SR |  |  | Train is at standstill |  | I |  | INT |  | The train will have to be stopped very close to the location defined by D_VALIDNV, since it will pass over this location in backwards direction. Backwards movements are limited by the variable D_NVROLL. |  |  | L1 |  |  | SR |  | - |  |
|  | 56 |  | 12 |  |  | 350.00 |  | L1 |  | SR |  |  | The train moves backwards and overpasses the location defined by the stored D_VALIDNV |  | I |  | INT |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 57 |  | 13 |  |  | 250.00 |  | L1 |  | SR |  |  | Train is at standstill |  | I |  | INT |  | The train has to be stopped far enough to allow to complete the steps from 15 to 23 before passing the location where the last stored NV shall start being valid (defined by D_VALIDNV) |  |  | L1 |  |  | SR |  | - |  |
|  | 58 |  | 14 |  |  | 260.00 |  | L1 |  | SR |  |  | Train is moving |  | I |  | INT |  | The train moves slow enough to allow to complete the steps from 15 to 23 before passing the location where the last stored NV shall start being valid (defined by D_VALIDNV) |  |  | L1 |  |  | SR |  | - |  |
|  | 59 |  | 15 |  |  | 260.00 |  | L1 |  | SR |  |  | The driver selects 'Main' |  | I |  | DMI |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 60 |  | 16 |  |  | 260.00 |  | L1 |  | SR |  |  | The Main window is displayed |  | O |  | DMI |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 61 |  | 17 |  |  | 260.00 |  | L1 |  | SR |  |  | The driver selects 'Driver ID' |  | I |  | DMI |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 62 |  | 18 |  |  | 260.00 |  | L1 |  | SR |  |  | The Driver ID window is displayed |  | O |  | DMI |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 63 |  | 19 |  |  | 260.00 |  | L1 |  | SR |  |  | The driver modifies the "Driver ID" |  | I |  | DMI |  | Driver ID = ID0 |  |  | L1 |  |  | SR |  | - |  |
|  | 64 |  | 20 |  |  | 260.00 |  | L1 |  | SR |  |  | The driver validates the "Driver ID" |  | I |  | DMI |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 65 |  | 21 |  |  | 260.00 |  | L1 |  | SR |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; DRIVER_ID=ID0) is recorded |  | O |  | JRU |  | The new Driver ID value is recorded |  |  | L1 |  |  | SR |  | - |  |
|  | 66 |  | 22 |  |  | 260.00 |  | L1 |  | SR |  |  | The driver closes the "Main" window |  | I |  | DMI |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 67 |  | 23 |  |  | 260.00 |  | L1 |  | SR |  |  | The Default window is displayed |  | O |  | DMI |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 68 |  | 24 |  |  | 300.00 |  | L1 |  | SR |  |  | Train is moving |  | I |  | INT |  | The train passes over the location where the last stored NV shall start being valid (defined by D_VALIDNV) |  |  | L1 |  |  | SR |  | - |  |
|  | Feature 3160200: 3.16.2: Balises Test Case 17: No linking information is used on-board -A BG marked as unlinked is found -No message consistency problem The message is accepted. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 69 |  | 1 |  |  | 350.00 |  | L1 |  | SR |  |  | One Balise Group message (N_TOTAL=1;Q_LINK=0) containing packet 72, packet 255 is received |  | I |  | BTM |  | No linking information is used on-board The train overpasses a BG marked as unlinked No message consistency problem |  |  | L1 |  |  | SR |  | - |  |
|  | 70 |  | 2 |  |  | 350.00 |  | L1 |  | SR |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded. |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 71 |  | 3 |  |  | 350.00 |  | L1 |  | SR |  |  | The "plain text message" is displayed |  | O |  | DMI |  | Only if mode not PS, SH, SL or PT (with exception of L2/L3 and following reception of 'Recognition of Exit from TR mode' with a more recent time stamp). The BG message is accepted. |  |  | L1 |  |  | SR |  | - |  |
|  | 72 |  | 4 |  |  | 350.00 |  | L1 |  | SR |  |  | START DISPLAYING PLAIN TEXT MESSAGE (NID_MESSAGE_JRU=18) is recorded |  | O |  | JRU |  | Only if mode not PS, SH, SL or PT (with exception of L2/L3 and following reception of 'Recognition of Exit from TR mode' with a more recent time stamp). |  |  | L1 |  |  | SR |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 1: The ETCS on-board system is NOT powered. Transition Condition Id [29]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 75 |  | 1 |  |  | 400.00 |  | L1 |  | SR |  |  | The ERTMS/ETCS on-board unit is powered down |  | I |  | INT |  | - |  |  | L1 |  |  | NP |  | - |  |
|  | 76 |  | 2 |  |  | 400.00 |  | L1 |  | NP |  |  | NOT GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=-) is recorded |  | O |  | JRU |  | There are no messages recorded in the JRU during a period of time |  |  | L1 |  |  | NP |  | - |  |
| Step 10: DMI Events Distance 0.00 m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Event Id. |  |  |  | Event Name |  |  |  |  |  |  | Data |  |  |  |  |  |  |  |  | Delay |  |  |  |  |  |  |  |  |
| 29 |  |  |  | Request Change Level |  |  |  |  |  |  | LEVEL=1 |  |  |  |  |  |  |  |  | 0 s |  |  |  |  |  |  |  |  |

| - Step 39 : Balise Group BG 1 d 1/2 - Distance 50.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 |  |  |
| NID_BG | 14 | 1 d |  |  |
| Q_LINK | 1 | 1 |  |  |
| NID_PACKET | 8 | 3 d | National values |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 230 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_VALIDNV | 15 | 250 d |  |  |
| NID_C | 10 | 64 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| V_NVSHUNT | 7 | 6 d |  |  |
| V_NVSTFF | 7 | 16 d |  |  |
| V_NVONSIGHT | 7 | 6 d |  |  |
| V_NVLIMSUPERV | 7 | 20 d |  |  |
| V_NVUNFIT | 7 | 20 d |  |  |
| V_NVREL | 7 | 8 d |  |  |
| D_NVROLL | 15 | 200 d |  |  |
| Q_NVSBTSMPERM | 1 | 1 d |  |  |
| Q_NVEMRRLS | 1 | 0 b |  |  |
| Q_NVGUIPERM | 1 | 0 b |  |  |
| Q_NVSBFBPERM | 1 | 0 b |  |  |
| Q_NVINHSMICPERM | 1 | 0 b |  |  |
| V_NVALLOWOVTRP | 7 | 0 d |  |  |
| V_NVSUPOVTRP | 7 | 6 d |  |  |
| D_NVOVTRP | 15 | 200 d |  |  |
| T_NVOVTRP | 8 | 60 d |  |  |
| D_NVPOTRP | 15 | 200 d |  |  |
| M_NVCONTACT | 2 | 10 b |  |  |
| T_NVCONTACT | 8 | 255 d |  |  |
| M_NVDERUN | 1 | 1 b |  |  |
| D_NVSTFF | 15 | 32767 d |  |  |
| Q_NVDRIVER_ADHES | 1 | 0 b |  |  |
| A_NVMAXREDADH1 | 6 | 10100 b |  |  |
| A_NVMAXREDADH2 | 6 | 1110 b |  |  |
| A_NVMAXREDADH3 | 6 | 1110 b |  |  |
| Q_NVLOCACC | 6 | 12 d |  |  |
| M_NVAVADH | 5 | 1 d |  |  |
| M_NVEBCL | 4 | 1 b |  |  |
| Q_NVKINT | 1 | 0 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 39 : Balise Group BG 1 d 2/2 - Distance 55.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 |  |  |
| NID_BG | 14 | 1 d |  |  |
| Q_LINK | 1 | 1 |  |  |
| NID_PACKET | 8 | 72 d | Packet for sending plain text messages |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 260 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| Q_TEXTCLASS | 2 | 00 b |  |  |
| Q_TEXTDISPLAY | 1 | 0 b |  |  |
| D_TEXTDISPLAY | 15 | 0 d |  |  |
| M_MODETEXTDISPLAY | 4 | 15 d |  |  |
| M_LEVELTEXTDISPLAY | 3 | 101 b |  |  |
| L_TEXTDISPLAY | 15 | 100 d |  |  |
| T_TEXTDISPLAY | 10 | 1023 d |  |  |
| M_MODETEXTDISPLAY | 4 | 15 d |  |  |
| M_LEVELTEXTDISPLAY | 3 | 101 b |  |  |
| Q_TEXTCONFIRM | 2 | 00 b |  |  |
| L_TEXT | 8 | 21 d |  |  |
| X_TEXT | 8 | N |  |  |
| X_TEXT | 8 | e |  |  |
| X_TEXT | 8 | w |  |  |
| X_TEXT | 8 |  |  |  |
| X_TEXT | 8 | N |  |  |
| X_TEXT | 8 | V |  |  |
| X_TEXT | 8 |  |  |  |
| X_TEXT | 8 | v |  |  |
| X_TEXT | 8 | a |  |  |
| X_TEXT | 8 | l |  |  |
| X_TEXT | 8 | i |  |  |
| X_TEXT | 8 | d |  |  |
| X_TEXT | 8 |  |  |  |
| X_TEXT | 8 | i |  |  |
| X_TEXT | 8 | n |  |  |
| X_TEXT | 8 |  |  |  |
| X_TEXT | 8 | 2 |  |  |
| X_TEXT | 8 | 5 |  |  |
| X_TEXT | 8 | 0 |  |  |
| X_TEXT | 8 |  |  |  |
| X_TEXT | 8 | m |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 69 : Balise Group BG 0 d 1/2 - Distance 350.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 999 d |  |  |
| NID_BG | 14 | 0 d |  |  |
| Q_LINK | 1 | 0 b |  |  |
| NID_PACKET | 8 | 72 d | Packet for sending plain text messages |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 276 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| Q_TEXTCLASS | 2 | 00 b |  |  |
| Q_TEXTDISPLAY | 1 | 0 b |  |  |
| D_TEXTDISPLAY | 15 | 32767 d |  |  |
| M_MODETEXTDISPLAY | 4 | 15 d |  |  |
| M_LEVELTEXTDISPLAY | 3 | 101 b |  |  |
| L_TEXTDISPLAY | 15 | 32767 d |  |  |
| T_TEXTDISPLAY | 10 | 1023 d |  |  |
| M_MODETEXTDISPLAY | 4 | 15 d |  |  |
| M_LEVELTEXTDISPLAY | 3 | 101 b |  |  |
| Q_TEXTCONFIRM | 2 | 00 b |  |  |
| L_TEXT | 8 | 23 d |  |  |
| X_TEXT | 8 | N |  |  |
| X_TEXT | 8 | a |  |  |
| X_TEXT | 8 | t |  |  |
| X_TEXT | 8 | i |  |  |
| X_TEXT | 8 | o |  |  |
| X_TEXT | 8 | n |  |  |
| X_TEXT | 8 | a |  |  |
| X_TEXT | 8 | l |  |  |
| X_TEXT | 8 |  |  |  |
| X_TEXT | 8 | v |  |  |
| X_TEXT | 8 | a |  |  |
| X_TEXT | 8 | l |  |  |
| X_TEXT | 8 | u |  |  |
| X_TEXT | 8 | e |  |  |
| X_TEXT | 8 | s |  |  |
| X_TEXT | 8 |  |  |  |
| X_TEXT | 8 | d |  |  |
| X_TEXT | 8 | e |  |  |
| X_TEXT | 8 | l |  |  |
| X_TEXT | 8 | e |  |  |
| X_TEXT | 8 | t |  |  |
| X_TEXT | 8 | e |  |  |
| X_TEXT | 8 | d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 69 : Balise Group BG 0 d 2/2 - Distance 355.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 999 d |  |  |
| NID_BG | 14 | 0 d |  |  |
| Q_LINK | 1 | 0 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |