| - ERTMS/ETCS |  |
| --- | --- |
| Subset-076-6-3_5180300_01_v320.mdb State : Ordered list of Test Case Steps with communication information |  |
| REF : | Reference |
| ISSUE : | 3.2.0 |
| DATE : | 10/04/17 13:37 |

| Company | Technical Approval | Management approval |
| --- | --- | --- |
| European Union Agency for Railways | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 3.2.0 10.04.2017 | All |  | MLT |

# Sequence description

FT5180300 + FT5180600

# Test cases

Important:	All columns called ‘Test Results’ are only placeholders for results of the test execution. They shall not be used for the description of test cases. 
A test is passed if all cells of ‘Test Result’ are passed.

|  | SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Step | Row |  |  | Dist. (m) |  | Previous |  |  |  |  | Description of Events |  | I/O |  | Interface |  | Comments |  |  | Next |  |  |  |  | Test Result |  |  |
|  |  |  |  |  |  |  | Levels |  | Mode |  |  |  |  |  |  |  |  |  |  |  | Levels |  |  | Mode |  |  |  |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 10: The ETCS on-board equipment is powered .Transition Condition Id [4]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 |  | 1 |  |  | 0.00 |  | N/A |  | NP |  |  | ERTMS/ETCS on-board unit is powered up |  | I |  | INT |  | The on-board equipment changes to SB mode. |  |  | N/A |  |  | NP |  | - |  |
|  | 2 |  | 2 |  |  | 0.00 |  | N/A |  | NP |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=6) is recorded |  | O |  | JRU |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 24: Start of Mission procedure is engaged |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 3 |  | 1 |  |  | 0.00 |  | N/A |  | SB |  |  | Driver opens desk |  | I |  | TIU |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 4 |  | 2 |  |  | 0.00 |  | N/A |  | SB |  |  | The 'CAB STATUS' message is RECORDED with the variable M_CAB_A_STATUS=1 (Active). |  | O |  | JRU |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 5 |  | 3 |  |  | 0.00 |  | N/A |  | SB |  |  | The actual mode SB is DISPLAYED |  | O |  | DMI |  | The SB indication after opening the desk may not be recorded. Because the change of mode was already recorded while desk was closed |  |  | N/A |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 7: The on-board equipment requests the driver to enter a driver ID within the scope of the Start of Mission procedure in SB mode when the status of the driver ID is unknown. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 6 |  | 1 |  |  | 0.00 |  | N/A |  | SB |  |  | The on-board equipment requires the driver to enter a Driver ID There is NO initial Driver ID value displayed |  | O |  | DMI |  | Starting conditions : There is no value initially displayed since the status of the driver ID is unknown. |  |  | N/A |  |  | SB |  | - |  |
|  | 7 |  | 2 |  |  | 0.00 |  | N/A |  | SB |  |  | The on-board equipment prevents the driver from continuing the procedure before the validation of the Driver ID is done. |  | O |  | DMI |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 8 |  | 3 |  |  | 0.00 |  | N/A |  | SB |  |  | The driver enters and validates a correct Driver ID |  | I |  | DMI |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 9 |  | 4 |  |  | 0.00 |  | N/A |  | SB |  |  | The validated Driver ID is recorded on the JRU with the DRIVER_ID variable in a 'GENERAL MESSAGE'. |  | O |  | JRU |  | A 'GENERAL MESSAGE' is recorded every 5s by the on-board equipment. |  |  | N/A |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 10: Within the scope of the Start of Mission procedure in SB mode, the status of the ERTMS/ETCS Level data is INVALID. The driver selects a level (the existing ERTMS/ETCS Level or a new one). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 10 |  | 1 |  |  | 0.00 |  | N/A |  | SB |  |  | The driver selects "Level <L0/LNTC/L1/L2/L3>" |  | I |  | DMI |  | Depending on the implementation, a list of available levels may be presented to the driver on the DMI. The driver is then allowed to select one level among the proposed levels. For NTC levels, the abbreviations of the concerned National Systems shall be presented to the driver. _____ The Driver selects L1 |  |  | L1 |  |  | SB |  | - |  |
|  | 11 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=<34/35/36/37/38>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 12 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=<0/1/2/3/4>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 6: Within the scope of the Start of Mission procedure in SB mode, the On-Board equipment offers the driver to select either SH or NL or ‘Train Data Entry’ before the on-board equipment requests the driver to enter a complete set of Train Data. The driver selects ‘Train Data Entry’. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 13 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver can select either SH or NL or ‘Train Data Entry’ |  | O |  | DMI |  | Starting conditions The 'Non-Leading' button is enabled if the 'non leading' input signal is received. |  |  | L1 |  |  | SB |  | - |  |
|  | 14 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver selects ‘Train Data Entry’ |  | I |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 15 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | A Train data window is is displayed on the DMI. |  | O |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 16 |  | 4 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver’s selection is recorded on the JRU. |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 47: The on-board equipment requests the driver to enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when the status of the Train Data is unknown (S12) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 18 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The on-board equipment requires the driver to enter all Train Data unknown by the on-board equipment and prevents the driver from performing any other actions. |  | O |  | DMI |  | The driver might be requested to enter the 'Train category(ies)', 'Train length', 'Traction / brake parameters', 'Maximum train speed', 'Loading gauge', 'Axle load category', 'Train fitted with airtight system', but the driver shall never be requested to enter the 'Traction system(s) accepted by the engine', 'List of National Systems available on-board' or 'Axle number'. |  |  | L1 |  |  | SB |  | - |  |
|  | 19 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver enters each Train data unknown on-board |  | I |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 20 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver is allowed to validate the train data |  | O |  | DMI |  | A complete set of valid Train Data is available |  |  | L1 |  |  | SB |  | - |  |
|  | 21 |  | 4 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver validates the train data. |  | I |  | DMI |  | The on-board equipment allows the driver to validate as soon as a valid and complete set of Train Data is entered. |  |  | L1 |  |  | SB |  | - |  |
|  | 22 |  | 5 |  |  | 0.00 |  | L1 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded |  | O |  | JRU |  | Triggered by : The driver validation. |  |  | L1 |  |  | SB |  | - |  |
|  | 23 |  | 6 |  |  | 0.00 |  | L1 |  | SB |  |  | TRAIN DATA (NID_MESSAGE_JRU=2) is recorded |  | O |  | JRU |  | Triggered by : Data Entry completed |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 46: After the set of Train Data has been entered/re-validated by the driver (in S12), if the status of the Train running number is 'unknown', the ERTMS/ETCS on-board equipment requests the driver to enter the Train running number (S13) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 25 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver is requested to enter the Train running number. |  | O |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 26 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver enters the "Train Running Number" [MAIN] |  | I |  | DMI |  | Entered Train Running Number: NID_OPERATIONAL_1 |  |  | L1 |  |  | SB |  | - |  |
|  | 27 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | TRAIN RUNNING NUMBER ENTERED BY THE DRIVER (NID_MESSAGE_JRU=49; NID_OPERATIONAL=NID_OPERATIONAL_1) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 1: The on-board equipment requests the driver to re-validate/enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when level is 0/1/NTC (Subset-026-5.4.3.2 S12). The driver is requested to enter/re-validate the Train running number if the status of the Train running number is 'unknown' or 'invalid'. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 28 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | The DMI allows the driver to select the option ‘START’. |  | O |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 16: The driver selects 'Start' within the scope of the Start of Mission procedure in SB mode and in ERTMS Level 1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 29 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver selects 'Start'. |  | I |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 30 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | Driver’s selection of 'Start' is recorded on the JRU |  | O |  | JRU |  | Triggered by: The driver acts on the on-board DMI. |  |  | L1 |  |  | SB |  | - |  |
|  | 31 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | An acknowledgement request for running in Staff Responsible mode is displayed on the DMI |  | O |  | DMI |  | The on-board equipment will then waits for the driver to acknowledge the acknowledgement request for running in Staff Responsible mode |  |  | L1 |  |  | SB |  | - |  |
|  | 32 |  | 4 |  |  | 0.00 |  | L1 |  | SB |  |  | The acknowledgement request for running in Staff Responsible mode is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 33 |  | 5 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver acknowledges 'Staff Responsible' |  | I |  | DMI |  | Symbol MO10 |  |  | L1 |  |  | SB |  | - |  |
|  | 34 |  | 6 |  |  | 0.00 |  | L1 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS= 0000 0011) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 35 |  | 7 |  |  | 0.00 |  | L1 |  | SB |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=2) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 36 |  | 8 |  |  | 0.00 |  | L1 |  | SR |  |  | The Mode symbol 'Staff Responsible' is displayed |  | O |  | DMI |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 37 |  | 9 |  |  | 0.00 |  | L1 |  | SR |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit24=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 38 |  | 10 |  |  | 0.00 |  | L1 |  | SR |  |  | Any message (NID_MESSAGE_JRU=ALL; NID_LRBG=16777215) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 44: The train switches in FS mode, because a MA and Track Description are stored on-board and no specific mode is required by a Mode Profile and No Trip order is given by balise. Transition Condition Id [32]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 39 |  | 1 |  |  | 50.00 |  | L1 |  | SR |  |  | One Balise Group message (N_TOTAL=1) containing packet 12 , packet 21 and packet 27 is received |  | I |  | BTM |  | Packet 80 not included for current location |  |  | L1 |  |  | SR |  | - |  |
|  | 40 |  | 2 |  |  | 50.00 |  | L1 |  | SR |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 41 |  | 3 |  |  | 50.00 |  | L1 |  | SR |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=0) is recorded |  | O |  | JRU |  | The on-board equipment switches immediately to FS mode |  |  | L1 |  |  | FS |  | - |  |
|  | 42 |  | 4 |  |  | 50.00 |  | L1 |  | FS |  |  | The Mode symbol 'Full Supervision' is displayed |  | O |  | DMI |  | DMI Object: Symbol MO11 |  |  | L1 |  |  | FS |  | - |  |
|  | 43 |  | 5 |  |  | 50.00 |  | L1 |  | FS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit26=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | Feature 3070300: 3.7.3: Extension, replacement of track description and linking information Test Case 4: The train receives a new track description or a linking information and shall replace the stored information. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 45 |  | 2 |  |  | 50.00 |  | L1 |  | FS |  |  | One Balise Group message (N_TOTAL=1; NID_BG=<BGa>; ) containing packet 68 (; D_TRACKCOND>0; L_TRACKCOND=FINITE VALUE), and packet 255 is received |  | I |  | BTM |  | If in L1 Possible announcement are: Lower pantograph announcement; Neutral section announcement; non stopping area announcement; Close air conditioning intake announcement; Regenerative Brake announcement; Tunnel stopping area announcement; change of traction announcement |  |  | L1 |  |  | FS |  | - |  |
|  | 46 |  | 3 |  |  | 50.00 |  | L1 |  | FS |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | If in L1 |  |  | L1 |  |  | FS |  | - |  |
|  | Feature 5180300: 5.18.3 Passing a powerless section with main power switch to be switched off Test Case 1: The on-board equipment has received track condition information indicating the location of a powerless section where switched off of the main power switch is required. The on-board equipment displays to the driver a profile indicating the location where switched off of the main power switch is required and where the main power switched off is removed. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 64 |  | 1 |  |  | 50.00 |  | L1 |  | FS |  |  | 'Neutral section announcement' is DISPLAYED. It is DISPLAYED if the related functionality is executed (application dependent) -automatically (symbol TC06) -manually (the driver is requested to act) (symbol TC07) |  | O |  | DMI |  | When the max safe front end of the train reaches a location in rear of the beginning of the powerless section, taking into account the time necessary for performing the required actions (see Subset-076-6-8) and the current train speed. |  |  | L1 |  |  | FS |  | - |  |
|  | 65 |  | 2 |  |  | 50.00 |  | L1 |  | FS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit49=1¦Bit50=1>) is recorded |  | O |  | JRU |  | TC06 (automatic execution) or TC07 (driver requested to act) |  |  | L1 |  |  | FS |  | - |  |
|  | 66 |  | 3 |  |  | 50.00 |  | L1 |  | FS |  |  | Track Conditions transmitted |  | O |  | TIU |  | The information related to the powerless section is provided to the ERTMS/ETCS external function |  |  | L1 |  |  | FS |  | - |  |
|  | 67 |  | 4 |  |  | 50.00 |  | L1 |  | FS |  |  | TRACK CONDITIONS (NID_MESSAGE_JRU=45; M_TRACKCOND_TI=1, D_MAXSFE_TO_START>0, D_MINSFE_TO_END>0) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 68 |  | 5 |  |  | 75.00 |  | L1 |  | FS |  |  | Track Conditions transmitted |  | O |  | TIU |  | The information related to the powerless section is provided to the ERTMS/ETCS external function |  |  | L1 |  |  | FS |  | - |  |
|  | 69 |  | 6 |  |  | 75.00 |  | L1 |  | FS |  |  | TRACK CONDITIONS (NID_MESSAGE_JRU=45; M_TRACKCOND_TI=1, D_MAXSFE_TO_START>0, D_MINSFE_TO_END>0) is recorded |  | O |  | JRU |  | D_MAXSFE_TO_START and D_MINSFE_TO_END are decreasing |  |  | L1 |  |  | FS |  | - |  |
|  | 70 |  | 7 |  |  | 100.00 |  | L1 |  | FS |  |  | "Neutral section announcement' (symbols TC06 or TC07) is REMOVED. |  | O |  | DMI |  | As soon as the train reaches the start location of the powerless section with its max safe front end ( = D_TRACKCOND ) |  |  | L1 |  |  | FS |  | - |  |
|  | 71 |  | 8 |  |  | 100.00 |  | L1 |  | FS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit49=0¦Bit50=0>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 72 |  | 9 |  |  | 100.00 |  | L1 |  | FS |  |  | "Neutral section" (symbol TC06) is DISPLAYED |  | O |  | DMI |  | If the related functionality is executed (application dependent) manually; otherwise, the symbols are identical |  |  | L1 |  |  | FS |  | - |  |
|  | 73 |  | 10 |  |  | 100.00 |  | L1 |  | FS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit49=1) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 74 |  | 11 |  |  | 110.00 |  | L1 |  | FS |  |  | Track Conditions transmitted |  | O |  | TIU |  | The information related to the powerless section is provided to the ERTMS/ETCS external function |  |  | L1 |  |  | FS |  | - |  |
|  | 75 |  | 12 |  |  | 110.00 |  | L1 |  | FS |  |  | TRACK CONDITIONS (NID_MESSAGE_JRU=45; M_TRACKCOND_TI=1, D_MAXSFE_TO_START=-32768, D_MINSFE_TO_END>0) is recorded |  | O |  | JRU |  | D_MAXSFE_TO_START not relevant and D_MINSFE_TO_END decreased |  |  | L1 |  |  | FS |  | - |  |
|  | 76 |  | 13 |  |  | 150.00 |  | L1 |  | FS |  |  | "Neutral section" (symbol TC06) is REMOVED |  | O |  | DMI |  | As soon as the train reaches the end location of the powerless section with its min safe front end. ( = D_TRACKCOND + L_ TRACKCOND ) |  |  | L1 |  |  | FS |  | - |  |
|  | 77 |  | 14 |  |  | 150.00 |  | L1 |  | FS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit49=0) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 78 |  | 15 |  |  | 150.00 |  | L1 |  | FS |  |  | 'End of Neutral section' is DISPLAYED (TC08 for automatic execution or TC09 for driver acq. request) |  | O |  | DMI |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 79 |  | 16 |  |  | 150.00 |  | L1 |  | FS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit51=1¦Bit52=1) is recorded |  | O |  | JRU |  | TC08 (automatic execution) TC09 (driver requested to act) |  |  | L1 |  |  | FS |  | - |  |
|  | 80 |  | 17 |  |  | 190.00 |  | L1 |  | FS |  |  | NOT TRACK CONDITIONS (NID_MESSAGE_JRU=45) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 81 |  | 18 |  |  | 240.00 |  | L1 |  | FS |  |  | 'End of Neutral section' (TC08 or TC09) is REMOVED |  | O |  | DMI |  | The 'End of neutral section' information shall remain displayed for a fixed time (see Appendix A3.1, 5s) after the train reached the min safe rear end of the end location of powerless section. |  |  | L1 |  |  | FS |  | - |  |
|  | 82 |  | 19 |  |  | 240.00 |  | L1 |  | FS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit51=0¦Bit52=0) is recorded |  | O |  | JRU |  | TC08 (automatic execution) TC09 (driver requested to act) |  |  | L1 |  |  | FS |  | - |  |
|  | Feature 3070300: 3.7.3: Extension, replacement of track description and linking information Test Case 4: The train receives a new track description or a linking information and shall replace the stored information. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 84 |  | 2 |  |  | 250.00 |  | L1 |  | FS |  |  | One Balise Group message (N_TOTAL=1; NID_BG=<BGa>; ) containing packet 68 (; D_TRACKCOND>0; L_TRACKCOND=FINITE VALUE), and packet 255 is received |  | I |  | BTM |  | If in L1 Possible announcement are: Lower pantograph announcement; Neutral section announcement; non stopping area announcement; Close air conditioning intake announcement; Regenerative Brake announcement; Tunnel stopping area announcement; change of traction announcement |  |  | L1 |  |  | FS |  | - |  |
|  | 85 |  | 3 |  |  | 250.00 |  | L1 |  | FS |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | If in L1 |  |  | L1 |  |  | FS |  | - |  |
|  | Feature 5180600: 5.18.6 Passing an 'air tightness' area Test Case 1: The on-board equipment has received track condition information indicating the location of an air tightness section where is "Closing air conditioning intake" required. The on-board equipment displays to the driver a profile indicating the location where "Closing air conditioning intake" is required and where "Opening air conditioning intake" is permitted. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 103 |  | 1 |  |  | 250.00 |  | L1 |  | FS |  |  | 'Close air conditioning intake announcement' is DISPLAYED. It is DISPLAYED if the related functionality is executed (application dependent) automatically (symbol TC19) or manually (the driver is requested to act) (symbol TC21) |  | O |  | DMI |  | When the max safe front end of the train reaches a location in rear of the beginning of the 'air tightness' area, taking into account the time necessary for performing the required actions (see Subset-076-6-8) and the current train speed. |  |  | L1 |  |  | FS |  | - |  |
|  | 104 |  | 2 |  |  | 250.00 |  | L1 |  | FS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit62=1¦Bit64=1>) is recorded |  | O |  | JRU |  | TC19 (automatic execution) or TC21 (driver requested to act) |  |  | L1 |  |  | FS |  | - |  |
|  | 105 |  | 3 |  |  | 250.00 |  | L1 |  | FS |  |  | Track Conditions transmitted |  | O |  | TIU |  | The information related to the air tightness section is provided to the ERTMS/ETCS external function |  |  | L1 |  |  | FS |  | - |  |
|  | 106 |  | 4 |  |  | 250.00 |  | L1 |  | FS |  |  | TRACK CONDITIONS (NID_MESSAGE_JRU=45; M_TRACKCOND_TI=2, D_MAXSFE_TO_START>0, D_MINSRE_TO_END>0) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 107 |  | 5 |  |  | 275.00 |  | L1 |  | FS |  |  | Track Conditions transmitted |  | O |  | TIU |  | The information related to the air tightness section is provided to the ERTMS/ETCS external function |  |  | L1 |  |  | FS |  | - |  |
|  | 108 |  | 6 |  |  | 275.00 |  | L1 |  | FS |  |  | TRACK CONDITIONS (NID_MESSAGE_JRU=45; M_TRACKCOND_TI=2, D_MAXSFE_TO_START>0, D_MINSRE_TO_END>0) is recorded |  | O |  | JRU |  | D_MAXSFE_TO_START and D_MINSRE_TO_END are decreasing |  |  | L1 |  |  | FS |  | - |  |
|  | 109 |  | 7 |  |  | 300.00 |  | L1 |  | FS |  |  | "Air conditioning intake closed" replaces 'Close air conditioning intake announcement' (symbol TC19 or TC21 is replaced by symbol TC19). |  | O |  | DMI |  | As soon as the train reaches the start location of the air tightness section with its max safe front end ( = D_TRACKCOND ) If the related functionality is executed (application dependent) manually; otherwise, the symbols are identical |  |  | L1 |  |  | FS |  | - |  |
|  | 110 |  | 8 |  |  | 300.00 |  | L1 |  | FS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit62=1&Bit64=0>) is recorded |  | O |  | JRU |  | If the related functionality is executed (application dependent) manually; otherwise, the symbols are identical |  |  | L1 |  |  | FS |  | - |  |
|  | 111 |  | 9 |  |  | 300.00 |  | L1 |  | FS |  |  | Track Conditions transmitted |  | O |  | TIU |  | The information related to the air tightness section is provided to the ERTMS/ETCS external function |  |  | L1 |  |  | FS |  | - |  |
|  | 112 |  | 10 |  |  | 300.00 |  | L1 |  | FS |  |  | TRACK CONDITIONS (NID_MESSAGE_JRU=45; M_TRACKCOND_TI=2, D_MAXSFE_TO_START=-32768, D_MINSRE_TO_END>0) is recorded |  | O |  | JRU |  | D_MAXSFE_TO_START not relevant and D_MINSRE_TO_END decreased |  |  | L1 |  |  | FS |  | - |  |
|  | 113 |  | 11 |  |  | 390.00 |  | L1 |  | FS |  |  | "Air conditioning intake closed" is REMOVED |  | O |  | DMI |  | As soon as the train reaches the end location of the air tightness section with its min safe rear end. |  |  | L1 |  |  | FS |  | - |  |
|  | 114 |  | 12 |  |  | 390.00 |  | L1 |  | FS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit62=0>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 115 |  | 13 |  |  | 390.00 |  | L1 |  | FS |  |  | NOT TRACK CONDITIONS (NID_MESSAGE_JRU=45) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 116 |  | 14 |  |  | 390.00 |  | L1 |  | FS |  |  | 'Open air conditioning intake' is DISPLAYED It is DISPLAYED if the related functionality is executed (application dependent) automatically (symbol TC20) or manually (the driver is requested to act) (symbol TC22) |  | O |  | DMI |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 117 |  | 15 |  |  | 390.00 |  | L1 |  | FS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit63=1¦Bit65=1) is recorded |  | O |  | JRU |  | TC20 (automatic execution) or TC22 (driver requested to act) |  |  | L1 |  |  | FS |  | - |  |
|  | 118 |  | 16 |  |  | 440.00 |  | L1 |  | FS |  |  | 'Open air conditioning intake' is REMOVED |  | O |  | DMI |  | The 'Open Air Conditioning intake' information shall remain displayed for a fixed time (5s, see Appendix A3.1) after the minimum safe rear end of the train has passed the end of the 'Air tightness area'. |  |  | L1 |  |  | FS |  | - |  |
|  | 119 |  | 17 |  |  | 440.00 |  | L1 |  | FS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit63=0&Bit65=0) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | Feature 9990200: A3.2: List of National / Default Data Test Case 28: When the train is NOT at standstill, the driver is allowed by the on-board equipment to modify the current Driver ID when such modification is permitted (national value M_NVDERUN set to 1). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 120 |  | 1 |  |  | 450.00 |  | L1 |  | FS |  |  | Train is moving |  | I |  | INT |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 121 |  | 2 |  |  | 450.00 |  | L1 |  | FS |  |  | The driver enters the "Driver ID" |  | I |  | DMI |  | The Driver ID is different from the existing Driver ID. The new Driver ID must contain 16 alphanumeric characters like "1234567890azbycx". |  |  | L1 |  |  | FS |  | - |  |
|  | 122 |  | 3 |  |  | 450.00 |  | L1 |  | FS |  |  | The driver confirms the "Driver ID" |  | I |  | DMI |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 123 |  | 4 |  |  | 450.00 |  | L1 |  | FS |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; DRIVER_ID=NEW_DRIVER_ID) is recorded |  | O |  | JRU |  | The just entered Driver ID is recorded. |  |  | L1 |  |  | FS |  | - |  |
|  | Feature 9990200: A3.2: List of National / Default Data Test Case 23: Timed service brake intervention when the braking to EOA. The Service Brake is available. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 124 |  | 1 |  |  | 590.00 |  | L1 |  | FS |  |  | The driver over-passes the Service Brake Intervention limit (SBI). |  | I |  | INT |  | The driver doesn’t take into account the Warning Indication due to over-speeding. |  |  | L1 |  |  | FS |  | - |  |
|  | 125 |  | 2 |  |  | 590.00 |  | L1 |  | FS |  |  | The Service Brake is APPLIED. |  | O |  | TIU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 126 |  | 3 |  |  | 590.00 |  | L1 |  | FS |  |  | The Service Brake intervention is DISPLAYED. |  | O |  | DMI |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 127 |  | 4 |  |  | 590.00 |  | L1 |  | FS |  |  | The Service Brake intervention is RECORDED. |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | Feature 9990200: A3.2: List of National / Default Data Test Case 25: Timed emergency brake intervention when braking to EOA. The emergency brake is used as second line of intervention |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 128 |  | 1 |  |  | 595.00 |  | L1 |  | FS |  |  | The driver over-passes the Emergency Brake Intervention limit (EBI). |  | I |  | INT |  | The driver doesn’t take into account the warning indication due to over-speeding. The service brake is applied but it does not brake as it is expected |  |  | L1 |  |  | FS |  | - |  |
|  | 129 |  | 2 |  |  | 595.00 |  | L1 |  | FS |  |  | The traction is cut off. The Emergency Brake is APPLIED. |  | O |  | TIU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 130 |  | 3 |  |  | 595.00 |  | L1 |  | FS |  |  | The Emergency Brake intervention is DISPLAYED. |  | O |  | DMI |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 131 |  | 4 |  |  | 595.00 |  | L1 |  | FS |  |  | The Emergency Brake intervention is RECORDED. |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 1: The ETCS on-board system is NOT powered. Transition Condition Id [29]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 132 |  | 1 |  |  | 595.00 |  | L1 |  | FS |  |  | The ERTMS/ETCS on-board unit is powered down |  | I |  | INT |  | - _____ The train reached standstill. |  |  | L1 |  |  | NP |  | - |  |
|  | 133 |  | 2 |  |  | 595.00 |  | L1 |  | NP |  |  | NOT GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=-) is recorded |  | O |  | JRU |  | There are no messages recorded in the JRU during a period of time |  |  | L1 |  |  | NP |  | - |  |
| Step 10: DMI Events Distance 0.00 m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Event Id. |  |  |  | Event Name |  |  |  |  |  |  | Data |  |  |  |  |  |  |  |  | Delay |  |  |  |  |  |  |  |  |
| 29 |  |  |  | Request Change Level |  |  |  |  |  |  | LEVEL=1 |  |  |  |  |  |  |  |  | 10 s |  |  |  |  |  |  |  |  |

| - Step 39 : Balise Group BG 0 d 1/2 - Distance 50.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 0 d |  |  |
| Q_LINK | 1 | 0 b |  |  |
| NID_PACKET | 8 | 12 d | Level 1 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 73 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| V_MAIN | 7 | 50 d |  |  |
| V_EMA | 7 | 0 d |  |  |
| T_EMA | 10 | 1023 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_ENDSECTION | 15 | 650 d |  |  |
| Q_SECTIONTIMER | 1 | 0 b |  |  |
| Q_ENDTIMER | 1 | 0 b |  |  |
| Q_DANGERPOINT | 1 | 0 b |  |  |
| Q_OVERLAP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 21 d | Transmission of the gradient |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 54 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_GRADIENT | 15 | 0 d |  |  |
| Q_GDIR | 1 | 0 b |  |  |
| G_A | 8 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 27 d | International static speed profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 58 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_STATIC | 15 | 0 d |  |  |
| V_STATIC | 7 | 40 d |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 68 d | Track condition |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 65 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| Q_TRACKINIT | 1 | 0 b |  |  |
| D_TRACKCOND | 15 | 50 d |  |  |
| L_TRACKCOND | 15 | 50 d |  |  |
| M_TRACKCOND | 4 | 1001 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 39 : Balise Group BG 0 d 2/2 - Distance 55.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 0 d |  |  |
| Q_LINK | 1 | 0 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 84 : Balise Group BG 1 d 1/2 - Distance 250.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 000 b |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 1 d |  |  |
| Q_LINK | 1 | 0 b |  |  |
| NID_PACKET | 8 | 68 d | Track condition |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 65 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| Q_TRACKINIT | 1 | 0 d |  |  |
| D_TRACKCOND | 15 | 50 d |  |  |
| L_TRACKCOND | 15 | 50 d |  |  |
| M_TRACKCOND | 4 | 0101 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 84 : Balise Group BG 1 d 2/2 - Distance 255.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 1 d |  |  |
| Q_LINK | 1 | 0 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |