| - ERTMS/ETCS |  |
| --- | --- |
| Subset-076-6-3_5120200_01_v320.mdb State : Ordered list of Test Case Steps with communication information |  |
| REF : | Subset-076-6-3_5120200_01_v320.mdb |
| ISSUE : | 3.2.0 |
| DATE : | 28/04/17 16:53 |

| Company | Technical Approval | Management approval |
| --- | --- | --- |
| European Union Agency for Railways | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 3.2.0 28.04.2017 | All |  | INECO |

# Sequence description

The driver uses the same engine

# Test cases

Important:	All columns called ‘Test Results’ are only placeholders for results of the test execution. They shall not be used for the description of test cases. 
A test is passed if all cells of ‘Test Result’ are passed.

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Row |  | Dist. (m) |  | Previous |  |  |  | Description of Events |  | I/O |  | Interface |  | Comments |  | Next |  |  |  | Test Result |  |  |
|  |  |  |  |  | Levels |  | Mode |  |  |  |  |  |  |  |  |  | Levels |  | Mode |  |  |  |  |
| Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 10: The ETCS on-board equipment is powered .Transition Condition Id [4]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 1 |  | 0.00 |  | N/A |  | NP |  | ERTMS/ETCS on-board unit is powered up |  | I |  | INT |  | The on-board equipment changes to SB mode. |  | N/A |  | NP |  | - |  |
| 2 |  | 2 |  | 0.00 |  | N/A |  | NP |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=6) is recorded |  | O |  | JRU |  | - |  | N/A |  | SB |  | - |  |
| Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 24: Start of Mission procedure is engaged |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  | 1 |  | 0.00 |  | N/A |  | SB |  | Driver opens desk |  | I |  | TIU |  | - |  | N/A |  | SB |  | - |  |
| 4 |  | 2 |  | 0.00 |  | N/A |  | SB |  | The 'CAB STATUS' message is RECORDED with the variable M_CAB_A_STATUS=1 (Active). |  | O |  | JRU |  | - |  | N/A |  | SB |  | - |  |
| 5 |  | 3 |  | 0.00 |  | N/A |  | SB |  | The actual mode SB is DISPLAYED |  | O |  | DMI |  | The SB indication after opening the desk may not be recorded. Because the change of mode was already recorded while desk was closed |  | N/A |  | SB |  | - |  |
| Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 7: The on-board equipment requests the driver to enter a driver ID within the scope of the Start of Mission procedure in SB mode when the status of the driver ID is unknown. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 |  | 1 |  | 0.00 |  | N/A |  | SB |  | The on-board equipment requires the driver to enter a Driver ID There is NO initial Driver ID value displayed |  | O |  | DMI |  | Starting conditions : There is no value initially displayed since the status of the driver ID is unknown. |  | N/A |  | SB |  | - |  |
| 7 |  | 2 |  | 0.00 |  | N/A |  | SB |  | The on-board equipment prevents the driver from continuing the procedure before the validation of the Driver ID is done. |  | O |  | DMI |  | - |  | N/A |  | SB |  | - |  |
| 8 |  | 3 |  | 0.00 |  | N/A |  | SB |  | The driver enters and validates a correct Driver ID |  | I |  | DMI |  | - |  | N/A |  | SB |  | - |  |
| 9 |  | 4 |  | 0.00 |  | N/A |  | SB |  | The validated Driver ID is recorded on the JRU with the DRIVER_ID variable in a 'GENERAL MESSAGE'. |  | O |  | JRU |  | A 'GENERAL MESSAGE' is recorded every 5s by the on-board equipment. |  | N/A |  | SB |  | - |  |
| Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 10: Within the scope of the Start of Mission procedure in SB mode, the status of the ERTMS/ETCS Level data is INVALID. The driver selects a level (the existing ERTMS/ETCS Level or a new one). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10 |  | 1 |  | 0.00 |  | N/A |  | SB |  | The driver selects "Level <L0/LNTC/L1/L2/L3>" |  | I |  | DMI |  | Depending on the implementation, a list of available levels may be presented to the driver on the DMI. The driver is then allowed to select one level among the proposed levels. For NTC levels, the abbreviations of the concerned National Systems shall be presented to the driver. _____ The Driver selects L1 [59.-1.1] |  | N/A |  | SB |  | - |  |
| 11 |  | 2 |  | 0.00 |  | N/A |  | SB |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=<34/35/36/37/38>) is recorded |  | O |  | JRU |  | - |  | N/A |  | SB |  | - |  |
| 12 |  | 3 |  | 0.00 |  | N/A |  | SB |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=<0/1/2/3/4>) is recorded |  | O |  | JRU |  | - |  | L1 |  | SB |  | - |  |
| Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 6: Within the scope of the Start of Mission procedure in SB mode, the On-Board equipment offers the driver to select either SH or NL or ‘Train Data Entry’ before the on-board equipment requests the driver to enter a complete set of Train Data. The driver selects ‘Train Data Entry’. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 |  | 1 |  | 0.00 |  | L1 |  | SB |  | The driver can select either SH or NL or ‘Train Data Entry’ |  | O |  | DMI |  | Starting conditions The 'Non-Leading' button is enabled if the 'non leading' input signal is received. |  | L1 |  | SB |  | - |  |
| 14 |  | 2 |  | 0.00 |  | L1 |  | SB |  | The driver selects ‘Train Data Entry’ |  | I |  | DMI |  | - |  | L1 |  | SB |  | - |  |
| 15 |  | 3 |  | 0.00 |  | L1 |  | SB |  | A Train data window is is displayed on the DMI. |  | O |  | DMI |  | - |  | L1 |  | SB |  | - |  |
| 16 |  | 4 |  | 0.00 |  | L1 |  | SB |  | The driver’s selection is recorded on the JRU. |  | O |  | JRU |  | - |  | L1 |  | SB |  | - |  |
| Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 47: The on-board equipment requests the driver to enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when the status of the Train Data is unknown (S12) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 18 |  | 1 |  | 0.00 |  | L1 |  | SB |  | The on-board equipment requires the driver to enter all Train Data unknown by the on-board equipment and prevents the driver from performing any other actions. |  | O |  | DMI |  | The driver might be requested to enter the 'Train category(ies)', 'Train length', 'Traction / brake parameters', 'Maximum train speed', 'Loading gauge', 'Axle load category', 'Train fitted with airtight system', but the driver shall never be requested to enter the 'Traction system(s) accepted by the engine', 'List of National Systems available on-board' or 'Axle number'. |  | L1 |  | SB |  | - |  |
| 19 |  | 2 |  | 0.00 |  | L1 |  | SB |  | The driver enters each Train data unknown on-board |  | I |  | DMI |  | - |  | L1 |  | SB |  | - |  |
| 20 |  | 3 |  | 0.00 |  | L1 |  | SB |  | The driver is allowed to validate the train data |  | O |  | DMI |  | A complete set of valid Train Data is available |  | L1 |  | SB |  | - |  |
| 21 |  | 4 |  | 0.00 |  | L1 |  | SB |  | The driver validates the train data. |  | I |  | DMI |  | The on-board equipment allows the driver to validate as soon as a valid and complete set of Train Data is entered. |  | L1 |  | SB |  | - |  |
| 22 |  | 5 |  | 0.00 |  | L1 |  | SB |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded |  | O |  | JRU |  | Triggered by : The driver validation. |  | L1 |  | SB |  | - |  |
| 23 |  | 6 |  | 0.00 |  | L1 |  | SB |  | TRAIN DATA (NID_MESSAGE_JRU=2) is recorded |  | O |  | JRU |  | Triggered by : Data Entry completed |  | L1 |  | SB |  | - |  |
| Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 46: After the set of Train Data has been entered/re-validated by the driver (in S12), if the status of the Train running number is 'unknown', the ERTMS/ETCS on-board equipment requests the driver to enter the Train running number (S13) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 25 |  | 1 |  | 0.00 |  | L1 |  | SB |  | The driver is requested to enter the Train running number. |  | O |  | DMI |  | - |  | L1 |  | SB |  | - |  |
| 26 |  | 2 |  | 0.00 |  | L1 |  | SB |  | The driver enters the "Train Running Number" [MAIN] |  | I |  | DMI |  | Entered Train Running Number: NID_OPERATIONAL_1 |  | L1 |  | SB |  | - |  |
| 27 |  | 3 |  | 0.00 |  | L1 |  | SB |  | TRAIN RUNNING NUMBER ENTERED BY THE DRIVER (NID_MESSAGE_JRU=49; NID_OPERATIONAL=NID_OPERATIONAL_1) is recorded |  | O |  | JRU |  | - |  | L1 |  | SB |  | - |  |
| Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 1: The on-board equipment requests the driver to re-validate/enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when level is 0/1/NTC (Subset-026-5.4.3.2 S12). The driver is requested to enter/re-validate the Train running number if the status of the Train running number is 'unknown' or 'invalid'. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 28 |  | 3 |  | 0.00 |  | L1 |  | SB |  | The DMI allows the driver to select the option ‘START’. |  | O |  | DMI |  | - |  | L1 |  | SB |  | - |  |
| Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 16: The driver selects 'Start' within the scope of the Start of Mission procedure in SB mode and in ERTMS Level 1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 29 |  | 1 |  | 0.00 |  | L1 |  | SB |  | The driver selects 'Start'. |  | I |  | DMI |  | - |  | L1 |  | SB |  | - |  |
| 30 |  | 2 |  | 0.00 |  | L1 |  | SB |  | Driver’s selection of 'Start' is recorded on the JRU |  | O |  | JRU |  | Triggered by: The driver acts on the on-board DMI. |  | L1 |  | SB |  | - |  |
| 31 |  | 3 |  | 0.00 |  | L1 |  | SB |  | An acknowledgement request for running in Staff Responsible mode is displayed on the DMI |  | O |  | DMI |  | The on-board equipment will then waits for the driver to acknowledge the acknowledgement request for running in Staff Responsible mode |  | L1 |  | SB |  | - |  |
| 32 |  | 4 |  | 0.00 |  | L1 |  | SB |  | The acknowledgement request for running in Staff Responsible mode is recorded |  | O |  | JRU |  | - |  | L1 |  | SB |  | - |  |
| 33 |  | 5 |  | 0.00 |  | L1 |  | SB |  | The driver acknowledges 'Staff Responsible' |  | I |  | DMI |  | Symbol MO10 |  | L1 |  | SB |  | - |  |
| 34 |  | 6 |  | 0.00 |  | L1 |  | SB |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS= 0000 0011) is recorded |  | O |  | JRU |  | - |  | L1 |  | SB |  | - |  |
| 35 |  | 7 |  | 0.00 |  | L1 |  | SB |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=2) is recorded |  | O |  | JRU |  | - |  | L1 |  | SR |  | - |  |
| 36 |  | 8 |  | 0.00 |  | L1 |  | SR |  | The Mode symbol 'Staff Responsible' is displayed |  | O |  | DMI |  | - |  | L1 |  | SR |  | - |  |
| 37 |  | 9 |  | 0.00 |  | L1 |  | SR |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit24=1>) is recorded |  | O |  | JRU |  | - |  | L1 |  | SR |  | - |  |
| 38 |  | 10 |  | 0.00 |  | L1 |  | SR |  | Any message (NID_MESSAGE_JRU=ALL; NID_LRBG=16777215) is recorded |  | O |  | JRU |  | - |  | L1 |  | SR |  | - |  |
| Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 9: Desks are closed. Transition Condition Id [28]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 40 |  | 1 |  | 0.00 |  | L1 |  | SR |  | CAB not active |  | I |  | TIU |  | Driver closes the desk. _____ The driver closes "Desk A" <event><action target="cabAButton" toggle="False"/>/></event> |  | L1 |  | SR |  | - |  |
| 41 |  | 2 |  | 0.00 |  | L1 |  | SR |  | CAB STATUS (NID_MESSAGE_JRU=38; M_CAB_STATUS=0) is recorded |  | O |  | JRU |  | - |  | L1 |  | SR |  | - |  |
| 42 |  | 3 |  | 0.00 |  | L1 |  | SR |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=6) is recorded |  | O |  | JRU |  | - |  | L1 |  | SB |  | - |  |
| Feature 5120200: 5.12.2: The driver uses the same engine Test Case 2: In level L0 /NTC/ 1, the driver closes desk "A". The on-board equipment executes the ‘end of mission’ procedure. The driver opens desk "B". The on-board equipment executes the ‘start of mission’ procedure. Same engine is used. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 43 |  | 2 |  | 0.00 |  | L1 |  | SB |  | CAB B active |  | I |  | TIU |  | The driver opens desk B: the start of mission procedure is triggered. |  | L1 |  | SB |  | - |  |
| 44 |  | 3 |  | 0.00 |  | L1 |  | SB |  | CAB STATUS (NID_MESSAGE_JRU=38; M_CAB_B_STATUS=1) is recorded |  | O |  | JRU |  | - |  | L1 |  | SB |  | - |  |
| Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 8: The on-board equipment allows the driver to validate an existing Driver ID within the scope of the Start of Mission procedure in SB mode when the status of the driver ID is invalid. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 46 |  | 1 |  | 0.00 |  | L1 |  | SB |  | There is an initial Driver ID displayed. The on-board equipment allows the driver to simply revalidate it. |  | O |  | DMI |  | A stored on-board Driver ID is displayed to the driver since the status of the Driver ID is INVALID. |  | L1 |  | SB |  | - |  |
| 47 |  | 2 |  | 0.00 |  | L1 |  | SB |  | The driver validates the Driver Id. |  | I |  | DMI |  | The on-board equipment allows the driver to simply revalidate an existing Driver ID without entering a new Driver ID. |  | L1 |  | SB |  | - |  |
| 48 |  | 3 |  | 0.00 |  | L1 |  | SB |  | The validated Driver ID is recorded on the JRU with the DRIVER_ID variable in a 'GENERAL MESSAGE'. |  | O |  | JRU |  | A 'GENERAL MESSAGE' is recorded every 5s by the on-board equipment. |  | L1 |  | SB |  | - |  |
| Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 10: Within the scope of the Start of Mission procedure in SB mode, the status of the ERTMS/ETCS Level data is INVALID. The driver selects a level (the existing ERTMS/ETCS Level or a new one). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 49 |  | 1 |  | 0.00 |  | L1 |  | SB |  | The driver selects "Level <L0/LNTC/L1/L2/L3>" |  | I |  | DMI |  | Depending on the implementation, a list of available levels may be presented to the driver on the DMI. The driver is then allowed to select one level among the proposed levels. For NTC levels, the abbreviations of the concerned National Systems shall be presented to the driver. _____ The Driver validates L1 [59.-1.1] |  | L1 |  | SB |  | - |  |
| 50 |  | 2 |  | 0.00 |  | L1 |  | SB |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=<34/35/36/37/38>) is recorded |  | O |  | JRU |  | - |  | L1 |  | SB |  | - |  |
| 51 |  | 3 |  | 0.00 |  | L1 |  | SB |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=<0/1/2/3/4>) is recorded |  | O |  | JRU |  | - |  | L1 |  | SB |  | - |  |
| Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 6: Within the scope of the Start of Mission procedure in SB mode, the On-Board equipment offers the driver to select either SH or NL or ‘Train Data Entry’ before the on-board equipment requests the driver to enter a complete set of Train Data. The driver selects ‘Train Data Entry’. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 52 |  | 1 |  | 0.00 |  | L1 |  | SB |  | The driver can select either SH or NL or ‘Train Data Entry’ |  | O |  | DMI |  | Starting conditions The 'Non-Leading' button is enabled if the 'non leading' input signal is received. |  | L1 |  | SB |  | - |  |
| 53 |  | 2 |  | 0.00 |  | L1 |  | SB |  | The driver selects ‘Train Data Entry’ |  | I |  | DMI |  | - |  | L1 |  | SB |  | - |  |
| 54 |  | 3 |  | 0.00 |  | L1 |  | SB |  | A Train data window is is displayed on the DMI. |  | O |  | DMI |  | - |  | L1 |  | SB |  | - |  |
| 55 |  | 4 |  | 0.00 |  | L1 |  | SB |  | The driver’s selection is recorded on the JRU. |  | O |  | JRU |  | - |  | L1 |  | SB |  | - |  |
| Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 2: The on-board equipment requests the driver to validate an existing complete set of Train Data within the scope of the Start of Mission procedure in SB mode when the status of the Train Data is invalid (S12) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 57 |  | 1 |  | 0.00 |  | L1 |  | SB |  | A complete set of Train Data is displayed to the driver The driver is allowed to validate the train data. |  | O |  | DMI |  | The driver might be requested to validate the 'Train category(ies)', 'Train length', 'Traction / brake parameters', 'Maximum train speed', 'Loading gauge', 'Axle load category', 'Train fitted with airtight system', but not the 'Traction system(s) accepted by the engine', 'List of National Systems available on-board' or 'Axle number'. |  | L1 |  | SB |  | - |  |
| 58 |  | 2 |  | 0.00 |  | L1 |  | SB |  | The driver validates the train data. |  | I |  | DMI |  | The on-board equipment allows the driver to simply validate the existing set of Train Data. |  | L1 |  | SB |  | - |  |
| 59 |  | 3 |  | 0.00 |  | L1 |  | SB |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded |  | O |  | JRU |  | Triggered by : The driver validation. |  | L1 |  | SB |  | - |  |
| 60 |  | 4 |  | 0.00 |  | L1 |  | SB |  | TRAIN DATA (NID_MESSAGE_JRU=2) is recorded |  | O |  | JRU |  | Triggered by : Data Entry completed |  | L1 |  | SB |  | - |  |
| Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 45: After the set of Train Data has been entered/re-validated by the driver (in S12), if the status of the Train running number is 'invalid', the ERTMS/ETCS on-board equipment requests the driver to re-validate the Train running number (S13) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 62 |  | 1 |  | 0.00 |  | L1 |  | SB |  | The driver is requested to re-validate the Train running number. |  | O |  | DMI |  | Displayed Train Running Number: NID_OPERATIONAL_1 |  | L1 |  | SB |  | - |  |
| 63 |  | 2 |  | 0.00 |  | L1 |  | SB |  | The driver confirms the "Train Running Number" [MAIN] |  | I |  | DMI |  | - |  | L1 |  | SB |  | - |  |
| 64 |  | 3 |  | 0.00 |  | L1 |  | SB |  | TRAIN RUNNING NUMBER ENTERED BY THE DRIVER (NID_MESSAGE_JRU=49; NID_OPERATIONAL=NID_OPERATIONAL_1) is recorded |  | O |  | JRU |  | - |  | L1 |  | SB |  | - |  |
| Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 1: The on-board equipment requests the driver to re-validate/enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when level is 0/1/NTC (Subset-026-5.4.3.2 S12). The driver is requested to enter/re-validate the Train running number if the status of the Train running number is 'unknown' or 'invalid'. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 65 |  | 3 |  | 0.00 |  | L1 |  | SB |  | The DMI allows the driver to select the option ‘START’. |  | O |  | DMI |  | - |  | L1 |  | SB |  | - |  |
| Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 16: The driver selects 'Start' within the scope of the Start of Mission procedure in SB mode and in ERTMS Level 1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 66 |  | 1 |  | 0.00 |  | L1 |  | SB |  | The driver selects 'Start'. |  | I |  | DMI |  | - |  | L1 |  | SB |  | - |  |
| 67 |  | 2 |  | 0.00 |  | L1 |  | SB |  | Driver’s selection of 'Start' is recorded on the JRU |  | O |  | JRU |  | Triggered by: The driver acts on the on-board DMI. |  | L1 |  | SB |  | - |  |
| 68 |  | 3 |  | 0.00 |  | L1 |  | SB |  | An acknowledgement request for running in Staff Responsible mode is displayed on the DMI |  | O |  | DMI |  | The on-board equipment will then waits for the driver to acknowledge the acknowledgement request for running in Staff Responsible mode |  | L1 |  | SB |  | - |  |
| 69 |  | 4 |  | 0.00 |  | L1 |  | SB |  | The acknowledgement request for running in Staff Responsible mode is recorded |  | O |  | JRU |  | - |  | L1 |  | SB |  | - |  |
| 70 |  | 5 |  | 0.00 |  | L1 |  | SB |  | The driver acknowledges 'Staff Responsible' |  | I |  | DMI |  | Symbol MO10 |  | L1 |  | SB |  | - |  |
| 71 |  | 6 |  | 0.00 |  | L1 |  | SB |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS= 0000 0011) is recorded |  | O |  | JRU |  | - |  | L1 |  | SB |  | - |  |
| 72 |  | 7 |  | 0.00 |  | L1 |  | SB |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=2) is recorded |  | O |  | JRU |  | - |  | L1 |  | SR |  | - |  |
| 73 |  | 8 |  | 0.00 |  | L1 |  | SR |  | The Mode symbol 'Staff Responsible' is displayed |  | O |  | DMI |  | - |  | L1 |  | SR |  | - |  |
| 74 |  | 9 |  | 0.00 |  | L1 |  | SR |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit24=1>) is recorded |  | O |  | JRU |  | - |  | L1 |  | SR |  | - |  |
| Feature 5100100: 5.10.1: Level Transitions General requirements Test Case 37: The train receives an order to switch immediately from L1 to L2 at the current position. On-board equipment immediately switches to L2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 76 |  | 1 |  | 100.00 |  | L1 |  | SR |  | The train reaches the Level transition border and an order to switch immediately to Level 2 is received from balise group BGa. Packet 41: 'Level Transition Order' is RECEIVED. D_LEVELTR= 0 or = 32767 M_LEVELTR=3 |  | I |  | BTM |  | - |  | L1 |  | SR |  | - |  |
| 77 |  | 2 |  | 100.00 |  | L1 |  | SR |  | Balise Group message, including Packet 41 'Level transition order', is RECORDED. |  | O |  | JRU |  | - |  | L1 |  | SR |  | - |  |
| 78 |  | 3 |  | 100.00 |  | L1 |  | SR |  | Level 2 is DISPLAYED. |  | O |  | DMI |  | (Except in SL mode) The On-board equipment switches to Level 2. |  | L1 |  | SR |  | - |  |
| 79 |  | 4 |  | 100.00 |  | L1 |  | SR |  | New Level 2 is RECORDED. |  | O |  | JRU |  | - |  | L2 |  | SR |  | - |  |
| Feature 3050300: 3.5.3: Establishing a communication session Test Case 22: System version is compatible and session is established. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 80 |  | 1 |  | 100.00 |  | L2 |  | SR |  | One Balise Group message (NID_RBC?16383) containing packet 42 (NID_RADIO ? FFFF FFFF FFFF FFFF) is received |  | I |  | BTM |  | Not use last known RBC and don’t use short number. |  | L2 |  | SR |  | - |  |
| 81 |  | 2 |  | 100.00 |  | L2 |  | SR |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  | L2 |  | SR |  | - |  |
| 82 |  | 3 |  | 100.00 |  | L2 |  | SR |  | SA-CONNECT.Request is transmitted |  | O |  | RTM |  | According to EURORADIO specifications. This attempt shall be repeated until successful. The onboard tries to set up a connection with the phone number included in the session management packet. |  | L2 |  | SR |  | - |  |
| 83 |  | 4 |  | 100.00 |  | L2 |  | SR |  | SA-CONNECT.Confirm is received |  | I |  | RTM |  | - |  | L2 |  | SR |  | - |  |
| 84 |  | 5 |  | 100.00 |  | L2 |  | SR |  | The Status symbol "Safe radio connection - Connection Up" is displayed |  | O |  | DMI |  | - |  | L2 |  | SR |  | - |  |
| 85 |  | 6 |  | 100.00 |  | L2 |  | SR |  | JRU step 'DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit40=1>) is recorded'. |  | O |  | JRU |  | - |  | L2 |  | SR |  | - |  |
| 86 |  | 7 |  | 100.00 |  | L2 |  | SR |  | SA-DATA.Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted |  | O |  | RTM |  | - |  | L2 |  | SR |  | - |  |
| 87 |  | 8 |  | 100.00 |  | L2 |  | SR |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | - |  | L2 |  | SR |  | - |  |
| 88 |  | 9 |  | 100.00 |  | L2 |  | SR |  | SA-DATA.Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32) is received |  | I |  | RTM |  | System versions M_VERSION (Version of the ERTMS/ETCS language) are compatible, so that the communication session established for on-board |  | L2 |  | SR |  | - |  |
| 89 |  | 10 |  | 100.00 |  | L2 |  | SR |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | - |  | L2 |  | SR |  | - |  |
| 90 |  | 11 |  | 100.00 |  | L2 |  | SR |  | SA-DATA.Request with Euroradio Message "Session Established" (NID_MESSAGE=159) including packet 2 is transmitted |  | O |  | RTM |  | - |  | L2 |  | SR |  | - |  |
| 91 |  | 12 |  | 100.00 |  | L2 |  | SR |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | - |  | L2 |  | SR |  | - |  |
| Feature 4080401: 4.8.4: Accepted Information depending on the modes - Acknowledgement of train data Test Case 1: Radio message with new Train Data and acknowledge of the RBC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 93 |  | 1 |  | 100.00 |  | L2 |  | SR |  | SA-DATA.Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) is transmitted |  | O |  | RTM |  | Validated train data |  | L2 |  | SR |  | - |  |
| 94 |  | 2 |  | 100.00 |  | L2 |  | SR |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=129) is recorded |  | O |  | JRU |  | - |  | L2 |  | SR |  | - |  |
| 95 |  | 3 |  | 100.00 |  | L2 |  | SR |  | SA-DATA.Indication with Euroradio Message "Acknowledgement of Train Data" (NID_MESSAGE=8) is received |  | I |  | RTM |  | - |  | L2 |  | SR |  | - |  |
| 96 |  | 4 |  | 100.00 |  | L2 |  | SR |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=8) is recorded |  | O |  | JRU |  | - |  | L2 |  | SR |  | - |  |
| Feature 4080415: 4.8.4: Accepted Information depending on the modes - Geographical Position Test Case 1: Transmission of list geographical position information from RBC, together with standard message 24. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 97 |  | 1 |  | 500.00 |  | L2 |  | SR |  | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 79 is received |  | I |  | RTM |  | The complete message is accepted by on-board. |  | L2 |  | SR |  | - |  |
| 98 |  | 2 |  | 500.00 |  | L2 |  | SR |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | Message is recorded |  | L2 |  | SR |  | - |  |
| 99 |  | 3 |  | 500.00 |  | L2 |  | SR |  | The Driver Request symbol "Geographical Position" is displayed |  | O |  | DMI |  | After the train front end has passed over the location specified by D_POSOFF value. |  | L2 |  | SR |  | - |  |
| 100 |  | 4 |  | 500.00 |  | L2 |  | SR |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit83=1>) is recorded |  | O |  | JRU |  | Bit83: Symbol DR03 (the symbol DR03 shall be shown in G12 if the geographical position is toggled off) |  | L2 |  | SR |  | - |  |
| 101 |  | 5 |  | 520.00 |  | L2 |  | SR |  | The driver presses "Geographical Position" |  | I |  | DMI |  | - |  | L2 |  | SR |  | - |  |
| 102 |  | 6 |  | 520.00 |  | L2 |  | SR |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS = 0001 1110) is recorded |  | O |  | JRU |  | M_DRIVERACTIONS = 0001 1110: Request to show geographical position |  | L2 |  | SR |  | - |  |
| 103 |  | 7 |  | 520.00 |  | L2 |  | SR |  | The "Geographical Position" is displayed |  | O |  | DMI |  | According to the M_POSITION |  | L2 |  | SR |  | - |  |
| Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 8: The driver closes the desk. The on-board equipment immediately switches to SB mode and the end of mission procedure is triggered (there is a radio link with RBC). Transition Condition Id [28]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 105 |  | 1 |  | 600.00 |  | L2 |  | SR |  | CAB not active |  | I |  | TIU |  | The driver closed the desk. The on-board equipment switches immediately to SB mode. |  | L2 |  | SR |  | - |  |
| 106 |  | 2 |  | 600.00 |  | L2 |  | SR |  | CAB STATUS (NID_MESSAGE_JRU=38; M_CAB_STATUS=0) is recorded |  | O |  | JRU |  | - |  | L2 |  | SR |  | - |  |
| 107 |  | 3 |  | 600.00 |  | L2 |  | SR |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=6) is recorded |  | O |  | JRU |  | - |  | L2 |  | SB |  | - |  |
| Feature 5050300: 5.5.3: End of Mission Procedure Test Case 3: Inform track-side about End of Mission procedure. A communication session with RBC exists. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 110 |  | 1 |  | 600.00 |  | L2 |  | SB |  | SA-DATA. Request with Euroradio Message "End of Mission" (NID_MESSAGE=150) containing packet 0 or packet 1 is transmitted |  | O |  | RTM |  | - |  | L2 |  | SB |  | - |  |
| 111 |  | 2 |  | 600.00 |  | L2 |  | SB |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | - |  | L2 |  | SB |  | - |  |
| Feature 3050500: 3.5.5: Terminating a communication session Test Case 5: The on-board equipment terminates a communication session ordered by RBC. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 114 |  | 1 |  | 600.00 |  | L2 |  | SB |  | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 42 (Q_RBC=0) is received |  | I |  | RTM |  | Q_RBC = 0 (Terminate communication session) |  | L2 |  | SB |  | - |  |
| 115 |  | 2 |  | 600.00 |  | L2 |  | SB |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | - |  | L2 |  | SB |  | - |  |
| 116 |  | 3 |  | 600.00 |  | L2 |  | SB |  | SA-DATA.Request with Euroradio Message "Termination of a communication session" (NID_MESSAGE=156) is transmitted |  | O |  | RTM |  | As soon as this message is received by trackside (RBC), the RBC shall consider the communication session terminated and it shall send the acknowledgement to the on-board. It shall be checked that no more messages are sent by the onboard |  | L2 |  | SB |  | - |  |
| 117 |  | 4 |  | 600.00 |  | L2 |  | SB |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | - |  | L2 |  | SB |  | - |  |
| 118 |  | 5 |  | 600.00 |  | L2 |  | SB |  | SA-DATA.Indication with Euroradio Message "Acknowledgement of termination of a communication session" (NID_MESSAGE=39) is received |  | I |  | RTM |  | Message 39: 'Acknowledgement of termination of a communication session' is RECEIVED from the RBC and the on-board equipment considers the communication session terminated. |  | L2 |  | SB |  | - |  |
| 119 |  | 6 |  | 600.00 |  | L2 |  | SB |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | When the acknowledgement is received, the onboard shall consider the communication session terminated and it shall begin the release of the safe connection with trackside. |  | L2 |  | SB |  | - |  |
| 120 |  | 7 |  | 600.00 |  | L2 |  | SB |  | SA-DISCONNECT.Request is transmitted |  | O |  | RTM |  | The train tries to release the safe connection. |  | L2 |  | SB |  | - |  |
| Feature 5120200: 5.12.2: The driver uses the same engine Test Case 1: In level 2/3, the driver closes desk 'A' but doesn´t switch the power off. The on-board equipment executes the ‘end of mission’ procedure. The driver opens desk 'B'. The on-board equipment executes the ‘start of mission’ procedure. Same engine is used. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 121 |  | 4 |  | 600.00 |  | L2 |  | SB |  | Cab active |  | I |  | TIU |  | SB mode does NOT change to NP. The driver opens desk B: _____ The driver opens "Desk A" <event><action target="cabAButton" toggle="True"/>/></event> |  | L2 |  | SB |  | - |  |
| 122 |  | 5 |  | 600.00 |  | L2 |  | SB |  | CAB STATUS (NID_MESSAGE_JRU=38; M_CAB_B_STATUS=1) is recorded |  | O |  | JRU |  | - |  | L2 |  | SB |  | - |  |
| Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 8: The on-board equipment allows the driver to validate an existing Driver ID within the scope of the Start of Mission procedure in SB mode when the status of the driver ID is invalid. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 124 |  | 1 |  | 600.00 |  | L2 |  | SB |  | There is an initial Driver ID displayed. The on-board equipment allows the driver to simply revalidate it. |  | O |  | DMI |  | A stored on-board Driver ID is displayed to the driver since the status of the Driver ID is INVALID. |  | L2 |  | SB |  | - |  |
| 125 |  | 2 |  | 600.00 |  | L2 |  | SB |  | The driver validates the Driver Id. |  | I |  | DMI |  | The on-board equipment allows the driver to simply revalidate an existing Driver ID without entering a new Driver ID. |  | L2 |  | SB |  | - |  |
| 126 |  | 3 |  | 600.00 |  | L2 |  | SB |  | The validated Driver ID is recorded on the JRU with the DRIVER_ID variable in a 'GENERAL MESSAGE'. |  | O |  | JRU |  | A 'GENERAL MESSAGE' is recorded every 5s by the on-board equipment. |  | L2 |  | SB |  | - |  |
| Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 39: If at least one Mobile Terminal is registered to a Radio Network, on-board equipment offers the driver the possibility to revalidate the RBC-ID/phone number if its status is 'invalid' or "valid". |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 128 |  | 1 |  | 600.00 |  | L2 |  | SB |  | The on-board equipment displays the Radio data window and offers the possibility to the driver to select - ‘Use short number’, - ‘Enter RBC data’ and - ‘Radio network ID’. |  | O |  | DMI |  | - |  | L2 |  | SB |  | - |  |
| 129 |  | 2 |  | 600.00 |  | L2 |  | SB |  | The driver presses the ‘Enter RBC data’ button |  | I |  | DMI |  | - |  | L2 |  | SB |  | - |  |
| 130 |  | 3 |  | 600.00 |  | L2 |  | SB |  | The on-board equipment displays the RBC data window with an initial value for the RBC ID/Phone number (since the status of this information is INVALID). |  | O |  | DMI |  | - |  | L2 |  | SB |  | - |  |
| 131 |  | 4 |  | 600.00 |  | L2 |  | SB |  | The driver is allowed to validate the RBC data (the ‘Yes’ button related to the ‘RBC data entry complete?’ question is enabled). |  | O |  | DMI |  | - |  | L2 |  | SB |  | - |  |
| 132 |  | 5 |  | 600.00 |  | L2 |  | SB |  | The driver validates the RBC data (presses the ‘Yes’ button related to the ‘RBC data entry complete?’ question). |  | I |  | DMI |  | - |  | L2 |  | SB |  | - |  |
| 133 |  | 6 |  | 600.00 |  | L2 |  | SB |  | RBC CONTACT INFORMATION ENTERED BY THE DRIVER (NID_MESSAGE_JRU=24; Q_RBCENTRY=2) is recorded |  | O |  | JRU |  | The variables NID_C, NID_RBC and NID_RADIO are identical to the data entered by the driver. |  | L2 |  | SB |  | - |  |
| Feature 3050300: 3.5.3: Establishing a communication session Test Case 23: Within the Start of Mission procedure, system version is compatible and session is established. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 135 |  | 1 |  | 600.00 |  | L2 |  | SB |  | SA-CONNECT.Request is transmitted |  | O |  | RTM |  | According to EURORADIO specifications. This attempt shall be repeated until successful or up to 3 times. The onboard tries to set up a connection with the phone entered in step 1. |  | L2 |  | SB |  | - |  |
| 136 |  | 2 |  | 600.00 |  | L2 |  | SB |  | SA-CONNECT.Confirm is received |  | I |  | RTM |  | Setting up of a safe connection is successful. |  | L2 |  | SB |  | - |  |
| 137 |  | 3 |  | 600.00 |  | L2 |  | SB |  | The Status symbol "Safe radio connection - Connection Up" is displayed |  | O |  | DMI |  | - |  | L2 |  | SB |  | - |  |
| 138 |  | 4 |  | 600.00 |  | L2 |  | SB |  | JRU step 'DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit40=1>) is recorded'. |  | O |  | JRU |  | - |  | L2 |  | SB |  | - |  |
| 139 |  | 5 |  | 600.00 |  | L2 |  | SB |  | SA-DATA.Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted |  | O |  | RTM |  | - |  | L2 |  | SB |  | - |  |
| 140 |  | 6 |  | 600.00 |  | L2 |  | SB |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | - |  | L2 |  | SB |  | - |  |
| 141 |  | 7 |  | 600.00 |  | L2 |  | SB |  | SA-DATA.Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32) is received |  | I |  | RTM |  | System versions M_VERSION (Version of the ERTMS/ETCS language) are compatible, so that the communication session established for on-board |  | L2 |  | SB |  | - |  |
| 142 |  | 8 |  | 600.00 |  | L2 |  | SB |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | - |  | L2 |  | SB |  | - |  |
| 143 |  | 9 |  | 600.00 |  | L2 |  | SB |  | SA-DATA.Request with Euroradio Message "Session Established" (NID_MESSAGE=159) including packet 2 is transmitted |  | O |  | RTM |  | - |  | L2 |  | SB |  | - |  |
| 144 |  | 10 |  | 600.00 |  | L2 |  | SB |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | - |  | L2 |  | SB |  | - |  |
| Feature 5120200: 5.12.2: The driver uses the same engine Test Case 1: In level 2/3, the driver closes desk 'A' but doesn´t switch the power off. The on-board equipment executes the ‘end of mission’ procedure. The driver opens desk 'B'. The on-board equipment executes the ‘start of mission’ procedure. Same engine is used. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 147 |  | 9 |  | 600.00 |  | L2 |  | SB |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; TRAIN_POSITION=<new current train position>) is recorded |  | O |  | JRU |  | The new train position data are calculated according to the new cab. ('When the driver changes the cab, the reference of the system is changed to the other cab in 'train length' meters.) Check that the parameter TRAIN_POSITION of this radio message (sent by on-board unit to RBC every 5 seconds triggered by current speed update) has changed. The position changed by 'train length' in meters. |  | L2 |  | SB |  | - |  |
| 148 |  | 10 |  | 600.00 |  | L2 |  | SB |  | SA-DATA. Request with Euroradio Message "SoM Position Report" (NID_MESSAGE=157) containing packet 0 is transmitted |  | O |  | RTM |  | The on-board equipment sends its new position data to the RBC according to its new orientation (no acknowledgement from RBC is required) |  | L2 |  | SB |  | - |  |
| 149 |  | 11 |  | 600.00 |  | L2 |  | SB |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | - |  | L2 |  | SB |  | - |  |
| Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 6: Within the scope of the Start of Mission procedure in SB mode, the On-Board equipment offers the driver to select either SH or NL or ‘Train Data Entry’ before the on-board equipment requests the driver to enter a complete set of Train Data. The driver selects ‘Train Data Entry’. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 151 |  | 1 |  | 600.00 |  | L2 |  | SB |  | The driver can select either SH or NL or ‘Train Data Entry’ |  | O |  | DMI |  | Starting conditions The 'Non-Leading' button is enabled if the 'non leading' input signal is received. |  | L2 |  | SB |  | - |  |
| 152 |  | 2 |  | 600.00 |  | L2 |  | SB |  | The driver selects ‘Train Data Entry’ |  | I |  | DMI |  | - |  | L2 |  | SB |  | - |  |
| 153 |  | 3 |  | 600.00 |  | L2 |  | SB |  | A Train data window is is displayed on the DMI. |  | O |  | DMI |  | - |  | L2 |  | SB |  | - |  |
| 154 |  | 4 |  | 600.00 |  | L2 |  | SB |  | The driver’s selection is recorded on the JRU. |  | O |  | JRU |  | - |  | L2 |  | SB |  | - |  |
| Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 2: The on-board equipment requests the driver to validate an existing complete set of Train Data within the scope of the Start of Mission procedure in SB mode when the status of the Train Data is invalid (S12) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 157 |  | 1 |  | 600.00 |  | L2 |  | SB |  | A complete set of Train Data is displayed to the driver The driver is allowed to validate the train data. |  | O |  | DMI |  | The driver might be requested to validate the 'Train category(ies)', 'Train length', 'Traction / brake parameters', 'Maximum train speed', 'Loading gauge', 'Axle load category', 'Train fitted with airtight system', but not the 'Traction system(s) accepted by the engine', 'List of National Systems available on-board' or 'Axle number'. |  | L2 |  | SB |  | - |  |
| 158 |  | 2 |  | 600.00 |  | L2 |  | SB |  | The driver validates the train data. |  | I |  | DMI |  | The on-board equipment allows the driver to simply validate the existing set of Train Data. |  | L2 |  | SB |  | - |  |
| 159 |  | 3 |  | 600.00 |  | L2 |  | SB |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded |  | O |  | JRU |  | Triggered by : The driver validation. |  | L2 |  | SB |  | - |  |
| 160 |  | 4 |  | 600.00 |  | L2 |  | SB |  | TRAIN DATA (NID_MESSAGE_JRU=2) is recorded |  | O |  | JRU |  | Triggered by : Data Entry completed |  | L2 |  | SB |  | - |  |
| Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 45: After the set of Train Data has been entered/re-validated by the driver (in S12), if the status of the Train running number is 'invalid', the ERTMS/ETCS on-board equipment requests the driver to re-validate the Train running number (S13) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 162 |  | 1 |  | 600.00 |  | L2 |  | SB |  | The driver is requested to re-validate the Train running number. |  | O |  | DMI |  | Displayed Train Running Number: NID_OPERATIONAL_1 |  | L2 |  | SB |  | - |  |
| 163 |  | 2 |  | 600.00 |  | L2 |  | SB |  | The driver confirms the "Train Running Number" [MAIN] |  | I |  | DMI |  | - |  | L2 |  | SB |  | - |  |
| 164 |  | 3 |  | 600.00 |  | L2 |  | SB |  | TRAIN RUNNING NUMBER ENTERED BY THE DRIVER (NID_MESSAGE_JRU=49; NID_OPERATIONAL=NID_OPERATIONAL_1) is recorded |  | O |  | JRU |  | - |  | L2 |  | SB |  | - |  |
| Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 4: The on-board equipment requests the driver to re-validate/enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when level is 2/3 (Subset-026-5.4.3.2 S12). The driver is requested to enter/re-validate the Train running number if the status of the Train running number is 'unknown' or 'invalid'. Radio communication session is established. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 165 |  | 3 |  | 600.00 |  | L2 |  | SB |  | SA-DATA.Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) is transmitted |  | O |  | RTM |  | - |  | L2 |  | SB |  | - |  |
| 166 |  | 4 |  | 600.00 |  | L2 |  | SB |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | - |  | L2 |  | SB |  | - |  |
| 167 |  | 5 |  | 600.00 |  | L2 |  | SB |  | SA-DATA.Indication with Euroradio Message "Acknowledgement of Train Data" (NID_MESSAGE=8) is received |  | I |  | RTM |  | - |  | L2 |  | SB |  | - |  |
| 168 |  | 6 |  | 600.00 |  | L2 |  | SB |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | - |  | L2 |  | SB |  | - |  |
| 169 |  | 7 |  | 600.00 |  | L2 |  | SB |  | The DMI allows the driver to select ‘START’. |  | O |  | DMI |  | - |  | L2 |  | SB |  | - |  |
| Feature 5120200: 5.12.2: The driver uses the same engine Test Case 1: In level 2/3, the driver closes desk 'A' but doesn´t switch the power off. The on-board equipment executes the ‘end of mission’ procedure. The driver opens desk 'B'. The on-board equipment executes the ‘start of mission’ procedure. Same engine is used. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 170 |  | 14 |  | 600.00 |  | L2 |  | SB |  | The driver selects START. |  | I |  | DMI |  | - |  | L2 |  | SB |  | - |  |
| 171 |  | 15 |  | 600.00 |  | L2 |  | SB |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0001 0011) is recorded |  | O |  | JRU |  | - |  | L2 |  | SB |  | - |  |
| 172 |  | 16 |  | 600.00 |  | L2 |  | SB |  | SA-DATA.Request with Euroradio Message "MA Request" (NID_MESSAGE=132) containing packet 0 is transmitted |  | O |  | RTM |  | - |  | L2 |  | SB |  | - |  |
| 173 |  | 17 |  | 600.00 |  | L2 |  | SB |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=132; NID_PACKET=0) is recorded |  | O |  | JRU |  | - |  | L2 |  | SB |  | - |  |
| 174 |  | 18 |  | 600.00 |  | L2 |  | SB |  | SA-DATA. Indication with Euroradio Message "MA with Shifted Location Reference" (NID_MESSAGE=33) containing packet 15, (MA), packet 21 (Gradient) and packet 27 (SSP) is received. |  | I |  | RTM |  | - |  | L2 |  | SB |  | - |  |
| 175 |  | 19 |  | 600.00 |  | L2 |  | SB |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | - |  | L2 |  | SB |  | - |  |
| 176 |  | 20 |  | 600.00 |  | L2 |  | SB |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=0) is recorded |  | O |  | JRU |  | Triggering event: When the mode changes |  | L2 |  | FS |  | - |  |
| 177 |  | 21 |  | 600.00 |  | L2 |  | FS |  | The train is moving. |  | I |  | INT |  | - |  | L2 |  | FS |  | - |  |
| 178 |  | 22 |  | 500.00 |  | L2 |  | FS |  | One Balise Group message (N_TOTAL=1) containing packet 255 is received. |  | I |  | BTM |  | The train overpasses the second Geographical Position reference Balise Group (BG2). |  | L2 |  | FS |  | - |  |
| 179 |  | 23 |  | 500.00 |  | L2 |  | FS |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  | L2 |  | FS |  | - |  |
| 180 |  | 24 |  | 480.00 |  | L2 |  | FS |  | The driver presses "Geographical Position" |  | I |  | DMI |  | Only if "Geographical Position" is not yet displayed. The train has travelled the offset distance from the second Reference Balise Group |  | L2 |  | FS |  | - |  |
| 181 |  | 25 |  | 480.00 |  | L2 |  | FS |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0001 1110) is recorded |  | O |  | JRU |  | Only if "Geographical Position" is not yet displayed |  | L2 |  | FS |  | - |  |
| 182 |  | 26 |  | 480.00 |  | L2 |  | FS |  | The "Geographical Position" is displayed. |  | O |  | DMI |  | The Geographical Position related to the FIRST track kilometre reference is still be displayed as the Geographical Position related to the SECOND track kilometre reference was deleted after change of orientation. |  | L2 |  | FS |  | - |  |
| Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 1: The ETCS on-board system is NOT powered. Transition Condition Id [29]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 183 |  | 1 |  | 430.00 |  | L2 |  | FS |  | The ERTMS/ETCS on-board unit is powered down |  | I |  | INT |  | - |  | N/A |  | NP |  | - |  |
| 184 |  | 2 |  | 430.00 |  | N/A |  | NP |  | NOT GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=-) is recorded |  | O |  | JRU |  | There are no messages recorded in the JRU during a period of time |  | N/A |  | NP |  | - |  |

| Step 10: DMI Events Distance 0.00 m |  |  |  |
| --- | --- | --- | --- |
| Event Id. | Event Name | Data | Delay |
| 29 | Request Change Level | LEVEL=1 | 0 s |

| Step 49: DMI Events Distance 0.00 m |  |  |  |
| --- | --- | --- | --- |
| Event Id. | Event Name | Data | Delay |
| 29 | Request Change Level | LEVEL=1 | 0 s |

| - Step 76 : Balise Group BG 1 d 1/2 - Distance 100.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 1 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 42 d | Session management |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 113 d |  |  |
| Q_RBC | 1 | 1 b |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_RBC | 14 | 1515 d |  |  |
| NID_RADIO | 64 | 003265342101FFFFh |  |  |
| Q_SLEEPSESSION | 1 | 1 b |  |  |
| NID_PACKET | 8 | 41 d | Level transition order |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 63 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_LEVELTR | 15 | 0 d |  |  |
| M_LEVELTR | 3 | 3 |  |  |
| L_ACKLEVELTR | 15 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 76 : Balise Group BG 1 d 2/2 - Distance 105.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 1 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 86 : Message 155 - Initiation of a Communication Session (RBC n? 1) - Distance 100.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 155 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |

| - Step 88 : Message 32 - RBC/RIU System Version (RBC n? 1) - Distance 100.00 m Delay : 5.00 s Conditional Message Id : 155 Back Delay : 5.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 32 d |  |  |
| L_MESSAGE | 10 | 11 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 16777215 d |  |  |
| M_VERSION | 7 | 0100001 b |  |  |

| - Step 90 : Message 159 - Session Established (RBC n? 1) - Distance 100.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 159 d |  |  |
| L_MESSAGE | 10 | 14 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_PACKET | 8 | 2 d | System versions that the on-board equipment is able to operate |  |
| L_PACKET | 13 | 33 d |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| N_ITER | 5 | 0 d |  |  |

| - Step 93 : Message 129 - Validated Train Data (RBC n? 1) - Distance 100.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 129 d |  |  |
| L_MESSAGE | 10 | 40 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 114 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 1048577 d |  |  |
| D_LRBG | 15 | 0 d |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 0 d |  |  |
| L_DOUBTUNDER | 15 | 0 d |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 7 d |  |  |
| Q_DIRTRAIN | 2 | 01 b |  |  |
| M_MODE | 4 | 0010 b |  |  |
| M_LEVEL | 3 | 3 |  |  |
| NID_PACKET | 8 | 11 d | Validated train data |  |
| L_PACKET | 13 | 126 d |  |  |
| NC_CDTRAIN | 4 | 7 d |  |  |
| NC_TRAIN | 15 | 4 d |  |  |
| L_TRAIN | 12 | 40 d |  |  |
| V_MAXTRAIN | 7 | 80 d |  |  |
| M_LOADINGGAUGE | 8 | 0000 0001 b |  |  |
| M_AXLELOADCAT | 7 | 7 d |  |  |
| M_AIRTIGHT | 2 | 01 b |  |  |
| N_AXLE | 10 | 4 d |  |  |
| N_ITER | 5 | 1 d |  |  |
| M_VOLTAGE | 4 | 3 |  |  |
| NID_CTRACTION | 10 | 15 d |  |  |
| N_ITER | 5 | 2 d |  |  |
| NID_NTC | 8 | 2 d |  |  |
| NID_NTC | 8 | 3 d |  |  |

| - Step 95 : Message 8 - Acknowledgement of Train Data (RBC n? 1) - Distance 100.00 m Delay : 5.00 s Conditional Message Id : 129 Back Delay : 5.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 8 d |  |  |
| L_MESSAGE | 10 | 14 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048577 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |

| - Step 97 : Message 24 - General Message (RBC n? 1) - Distance 500.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 24 d |  |  |
| L_MESSAGE | 10 | 27 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048577 d |  |  |
| NID_PACKET | 8 | 79 d | Geographical position information |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 140 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| Q_NEWCOUNTRY | 1 | 0 b |  |  |
| NID_BG | 14 | 1 d |  |  |
| D_POSOFF | 15 | 400 d |  |  |
| Q_MPOSITION | 1 | 1 b |  |  |
| M_POSITION | 24 | 100 d |  |  |
| N_ITER | 5 | 1 d |  |  |
| Q_NEWCOUNTRY | 1 | 0 b |  |  |
| NID_BG | 14 | 2 d |  |  |
| D_POSOFF | 15 | 0 d |  |  |
| Q_MPOSITION | 1 | 1 b |  |  |
| M_POSITION | 24 | 1000 d |  |  |

| - Step 110 : Message 150 - End of Mission (RBC n? 1) - Distance 600.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 150 d |  |  |
| L_MESSAGE | 10 | 24 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 114 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 1048577 d |  |  |
| D_LRBG | 15 | 700 d |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 0 d |  |  |
| L_DOUBTUNDER | 15 | 0 d |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 0 d |  |  |
| Q_DIRTRAIN | 2 | 01 b |  |  |
| M_MODE | 4 | 6 d |  |  |
| M_LEVEL | 3 | 3 |  |  |

| - Step 114 : Message 24 - General Message (RBC n? 1) - Distance 600.00 m Delay : 5.00 s Conditional Message Id : 150 Back Delay : 5.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 24 d |  |  |
| L_MESSAGE | 10 | 24 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048577 d |  |  |
| NID_PACKET | 8 | 42 d | Session management |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 113 d |  |  |
| Q_RBC | 1 | 0 b |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_RBC | 14 | 1515 d |  |  |
| NID_RADIO | 64 | 002365342101 FFFFh |  |  |
| Q_SLEEPSESSION | 1 | 1 b |  |  |

| - Step 116 : Message 156 - Termination of a Communication Session (RBC n? 1) - Distance 600.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 156 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |

| - Step 118 : Message 39 - Acknowledgement of Termination of Communication Session (RBC n? 1) - Distance 600.00 m Delay : 5.00 s Conditional Message Id : 156 Back Delay : 5.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 39 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048577 d |  |  |

| - Step 139 : Message 155 - Initiation of a Communication Session (RBC n? 1) - Distance 600.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 155 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |

| - Step 141 : Message 32 - RBC/RIU System Version (RBC n? 1) - Distance 600.00 m Delay : 5.00 s Conditional Message Id : 155 Back Delay : 5.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 32 d |  |  |
| L_MESSAGE | 10 | 11 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 16777215 d |  |  |
| M_VERSION | 7 | 0100001 b |  |  |

| - Step 143 : Message 159 - Session Established (RBC n? 1) - Distance 600.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 159 d |  |  |
| L_MESSAGE | 10 | 14 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_PACKET | 8 | 2 d | System versions that the on-board equipment is able to operate |  |
| L_PACKET | 13 | 33 d |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| N_ITER | 5 | 0 d |  |  |

| - Step 148 : Message 157 - SoM Position Report (RBC n? 1) - Distance 600.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 157 d |  |  |
| L_MESSAGE | 10 | 24 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| Q_STATUS | 2 | 01 b |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 114 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 1048577 d |  |  |
| D_LRBG | 15 | 500 d |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 0 d |  |  |
| L_DOUBTUNDER | 15 | 0 d |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 0 d |  |  |
| Q_DIRTRAIN | 2 | 01 b |  |  |
| M_MODE | 4 | 6 d |  |  |
| M_LEVEL | 3 | 3 |  |  |

| - Step 165 : Message 129 - Validated Train Data (RBC n? 1) - Distance 600.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 129 d |  |  |
| L_MESSAGE | 10 | 40 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 114 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 1048577 d |  |  |
| D_LRBG | 15 | 460 d |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 0 d |  |  |
| L_DOUBTUNDER | 15 | 0 d |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 0 d |  |  |
| Q_DIRTRAIN | 2 | 10 b |  |  |
| M_MODE | 4 | 6 d |  |  |
| M_LEVEL | 3 | 3 |  |  |
| NID_PACKET | 8 | 11 d | Validated train data |  |
| L_PACKET | 13 | 126 d |  |  |
| NC_CDTRAIN | 4 | 7 d |  |  |
| NC_TRAIN | 15 | 4 d |  |  |
| L_TRAIN | 12 | 40 d |  |  |
| V_MAXTRAIN | 7 | 80 d |  |  |
| M_LOADINGGAUGE | 8 | 00000001 b |  |  |
| M_AXLELOADCAT | 7 | 7 d |  |  |
| M_AIRTIGHT | 2 | 01 b |  |  |
| N_AXLE | 10 | 4 d |  |  |
| N_ITER | 5 | 1 d |  |  |
| M_VOLTAGE | 4 | 3 |  |  |
| NID_CTRACTION | 10 | 15 d |  |  |
| N_ITER | 5 | 2 d |  |  |
| NID_NTC | 8 | 2 d |  |  |
| NID_NTC | 8 | 3 d |  |  |

| - Step 167 : Message 8 - Acknowledgement of Train Data (RBC n? 1) - Distance 600.00 m Delay : 25.00 s Conditional Message Id : 129 Back Delay : 25.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 8 d |  |  |
| L_MESSAGE | 10 | 14 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048577 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |

| - Step 172 : Message 132 - MA Request (RBC n? 1) - Distance 600.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 132 d |  |  |
| L_MESSAGE | 10 | 25 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| Q_MARQSTREASON | 5 | 1 |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 114 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 1048577 d |  |  |
| D_LRBG | 15 | 460 d |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 0 d |  |  |
| L_DOUBTUNDER | 15 | 0 d |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 0 d |  |  |
| Q_DIRTRAIN | 2 | 01 b |  |  |
| M_MODE | 4 | 6 d |  |  |
| M_LEVEL | 3 | 3 |  |  |

| - Step 174 : Message 33 - MA with Shifted Location Reference (RBC n? 1) - Distance 600.00 m Delay : 10.00 s Conditional Message Id : 132 Back Delay : 10.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 33 d |  |  |
| L_MESSAGE | 10 | 34 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048577 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_REF | 16 | 460 d |  |  |
| NID_PACKET | 8 | 15 d | Level 2/3 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 66 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| V_EMA | 7 | 0 d |  |  |
| T_EMA | 10 | 1023 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_ENDSECTION | 15 | 10000 d |  |  |
| Q_SECTIONTIMER | 1 | 0 b |  |  |
| Q_ENDTIMER | 1 | 0 b |  |  |
| Q_DANGERPOINT | 1 | 0 b |  |  |
| Q_OVERLAP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 21 d | Transmission of the gradient |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 54 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_GRADIENT | 15 | 0 d |  |  |
| Q_GDIR | 1 | 0 b |  |  |
| G_A | 8 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 27 d | International static speed profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 58 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_STATIC | 15 | 0 d |  |  |
| V_STATIC | 7 | 20 d |  |  |
| Q_FRONT | 1 | 1 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |

| - Step 178 : Balise Group BG 2 d 1/2 - Distance 500.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 2 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 178 : Balise Group BG 2 d 2/2 - Distance 505.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 2 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |