| - ERTMS/ETCS |  |
| --- | --- |
| Subset-076-6-3_5100400_02_LNTC_v320.mdb State : Ordered list of Test Case Steps with communication information |  |
| REF : | Subset-076-6-3_5100400_02_LNTC_v320.mdb |
| ISSUE : | 3.2.0 |
| DATE : | 10/04/17 13:00 |

| Company | Technical Approval | Management approval |
| --- | --- | --- |
| European Union Agency for Railways | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 3.2.0 10.04.2017 | All |  | INECO |

# Sequence description

All combinations of level transition to and from LNTC with NO driver ACK before level transition and YES after the level transition is executed.

# Test cases

Important:	All columns called ‘Test Results’ are only placeholders for results of the test execution. They shall not be used for the description of test cases. 
A test is passed if all cells of ‘Test Result’ are passed.

|  | SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Step | Row |  |  | Dist. (m) |  | Previous |  |  |  |  | Description of Events |  | I/O |  | Interface |  | Comments |  |  | Next |  |  |  |  | Test Result |  |  |
|  |  |  |  |  |  |  | Levels |  | Mode |  |  |  |  |  |  |  |  |  |  |  | Levels |  |  | Mode |  |  |  |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 10: The ETCS on-board equipment is powered .Transition Condition Id [4]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 |  | 1 |  |  | 0.00 |  | N/A |  | NP |  |  | ERTMS/ETCS on-board unit is powered up |  | I |  | INT |  | The on-board equipment changes to SB mode. |  |  | N/A |  |  | SB |  | - |  |
|  | 2 |  | 2 |  |  | 0.00 |  | N/A |  | SB |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=6) is recorded |  | O |  | JRU |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 24: Start of Mission procedure is engaged |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 3 |  | 1 |  |  | 0.00 |  | N/A |  | SB |  |  | Driver opens desk |  | I |  | TIU |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 4 |  | 2 |  |  | 0.00 |  | N/A |  | SB |  |  | The 'CAB STATUS' message is RECORDED with the variable M_CAB_A_STATUS=1 (Active). |  | O |  | JRU |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 5 |  | 3 |  |  | 0.00 |  | N/A |  | SB |  |  | The actual mode SB is DISPLAYED |  | O |  | DMI |  | The SB indication after opening the desk may not be recorded. Because the change of mode was already recorded while desk was closed |  |  | N/A |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 7: The on-board equipment requests the driver to enter a driver ID within the scope of the Start of Mission procedure in SB mode when the status of the driver ID is unknown. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 6 |  | 1 |  |  | 0.00 |  | N/A |  | SB |  |  | The on-board equipment requires the driver to enter a Driver ID There is NO initial Driver ID value displayed |  | O |  | DMI |  | Starting conditions : There is no value initially displayed since the status of the driver ID is unknown. |  |  | N/A |  |  | SB |  | - |  |
|  | 7 |  | 2 |  |  | 0.00 |  | N/A |  | SB |  |  | The on-board equipment prevents the driver from continuing the procedure before the validation of the Driver ID is done. |  | O |  | DMI |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 8 |  | 3 |  |  | 0.00 |  | N/A |  | SB |  |  | The driver enters and validates a correct Driver ID |  | I |  | DMI |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 9 |  | 4 |  |  | 0.00 |  | N/A |  | SB |  |  | The validated Driver ID is recorded on the JRU with the DRIVER_ID variable in a 'GENERAL MESSAGE'. |  | O |  | JRU |  | A 'GENERAL MESSAGE' is recorded every 5s by the on-board equipment. |  |  | N/A |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 10: Within the scope of the Start of Mission procedure in SB mode, the status of the ERTMS/ETCS Level data is INVALID. The driver selects a level (the existing ERTMS/ETCS Level or a new one). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 10 |  | 1 |  |  | 0.00 |  | N/A |  | SB |  |  | The driver selects "Level <L0/LNTC/L1/L2/L3>" |  | I |  | DMI |  | Depending on the implementation, a list of available levels may be presented to the driver on the DMI. The driver is then allowed to select one level among the proposed levels. For NTC levels, the abbreviations of the concerned National Systems shall be presented to the driver. _____ The Driver selects L0 [74.-1.1] |  |  | L0 |  |  | SB |  | - |  |
|  | 11 |  | 2 |  |  | 0.00 |  | L0 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=<34/35/36/37/38>) is recorded |  | O |  | JRU |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | 12 |  | 3 |  |  | 0.00 |  | L0 |  | SB |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=<0/1/2/3/4>) is recorded |  | O |  | JRU |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 6: Within the scope of the Start of Mission procedure in SB mode, the On-Board equipment offers the driver to select either SH or NL or ‘Train Data Entry’ before the on-board equipment requests the driver to enter a complete set of Train Data. The driver selects ‘Train Data Entry’. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 13 |  | 1 |  |  | 0.00 |  | L0 |  | SB |  |  | The driver can select either SH or NL or ‘Train Data Entry’ |  | O |  | DMI |  | Starting conditions The 'Non-Leading' button is enabled if the 'non leading' input signal is received. |  |  | L0 |  |  | SB |  | - |  |
|  | 14 |  | 2 |  |  | 0.00 |  | L0 |  | SB |  |  | The driver selects ‘Train Data Entry’ |  | I |  | DMI |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | 15 |  | 3 |  |  | 0.00 |  | L0 |  | SB |  |  | A Train data window is is displayed on the DMI. |  | O |  | DMI |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | 16 |  | 4 |  |  | 0.00 |  | L0 |  | SB |  |  | The driver’s selection is recorded on the JRU. |  | O |  | JRU |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 47: The on-board equipment requests the driver to enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when the status of the Train Data is unknown (S12) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 18 |  | 1 |  |  | 0.00 |  | L0 |  | SB |  |  | The on-board equipment requires the driver to enter all Train Data unknown by the on-board equipment and prevents the driver from performing any other actions. |  | O |  | DMI |  | The driver might be requested to enter the 'Train category(ies)', 'Train length', 'Traction / brake parameters', 'Maximum train speed', 'Loading gauge', 'Axle load category', 'Train fitted with airtight system', but the driver shall never be requested to enter the 'Traction system(s) accepted by the engine', 'List of National Systems available on-board' or 'Axle number'. |  |  | L0 |  |  | SB |  | - |  |
|  | 19 |  | 2 |  |  | 0.00 |  | L0 |  | SB |  |  | The driver enters each Train data unknown on-board |  | I |  | DMI |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | 20 |  | 3 |  |  | 0.00 |  | L0 |  | SB |  |  | The driver is allowed to validate the train data |  | O |  | DMI |  | A complete set of valid Train Data is available |  |  | L0 |  |  | SB |  | - |  |
|  | 21 |  | 4 |  |  | 0.00 |  | L0 |  | SB |  |  | The driver validates the train data. |  | I |  | DMI |  | The on-board equipment allows the driver to validate as soon as a valid and complete set of Train Data is entered. |  |  | L0 |  |  | SB |  | - |  |
|  | 22 |  | 5 |  |  | 0.00 |  | L0 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded |  | O |  | JRU |  | Triggered by : The driver validation. |  |  | L0 |  |  | SB |  | - |  |
|  | 23 |  | 6 |  |  | 0.00 |  | L0 |  | SB |  |  | TRAIN DATA (NID_MESSAGE_JRU=2) is recorded |  | O |  | JRU |  | Triggered by : Data Entry completed |  |  | L0 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 46: After the set of Train Data has been entered/re-validated by the driver (in S12), if the status of the Train running number is 'unknown', the ERTMS/ETCS on-board equipment requests the driver to enter the Train running number (S13) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 25 |  | 1 |  |  | 0.00 |  | L0 |  | SB |  |  | The driver is requested to enter the Train running number. |  | O |  | DMI |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | 26 |  | 2 |  |  | 0.00 |  | L0 |  | SB |  |  | The driver enters the "Train Running Number" [MAIN] |  | I |  | DMI |  | Entered Train Running Number: NID_OPERATIONAL_1 |  |  | L0 |  |  | SB |  | - |  |
|  | 27 |  | 3 |  |  | 0.00 |  | L0 |  | SB |  |  | TRAIN RUNNING NUMBER ENTERED BY THE DRIVER (NID_MESSAGE_JRU=49; NID_OPERATIONAL=NID_OPERATIONAL_1) is recorded |  | O |  | JRU |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | Feature 5040500: 5.4.5: Degraded Situations Test Case 12: Within the scope of the Start of Mission procedure in SB mode at S10, if valid Train Data is available the driver can select 'Start. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 29 |  | 1 |  |  | 0.00 |  | L0 |  | SB |  |  | The <Start> button in the <Main> window is enabled. |  | O |  | DMI |  | In addition to possibilities offered at S10 |  |  | L0 |  |  | SB |  | - |  |
|  | 30 |  | 2 |  |  | 0.00 |  | L0 |  | SB |  |  | The driver selects "Start" |  | I |  | DMI |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | 31 |  | 3 |  |  | 0.00 |  | L0 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0001 0011) is recorded |  | O |  | JRU |  | Start selected |  |  | L0 |  |  | SB |  | - |  |
|  | 32 |  | 4 |  |  | 0.00 |  | L0 |  | SB |  |  | The <Default> window is displayed. |  | O |  | DMI |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | 33 |  | 5 |  |  | 0.00 |  | L0 |  | SB |  |  | The Mode symbol "Acknowledge Unfitted" is displayed |  | O |  | DMI |  | The on-board equipment will then waits for the driver to acknowledge the acknowledgement request for running in Unfitted mode DMI Object: Symbol MO17 |  |  | L0 |  |  | SB |  | - |  |
|  | 34 |  | 6 |  |  | 0.00 |  | L0 |  | SB |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit32=1>) is recorded |  | O |  | JRU |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | 35 |  | 7 |  |  | 0.00 |  | L0 |  | SB |  |  | The driver acknowledges "Unfitted" |  | I |  | DMI |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | 36 |  | 8 |  |  | 0.00 |  | L0 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS= 0000 0100) is recorded |  | O |  | JRU |  | Ack of Unfitted mode As soon as the driver acknowledges the on-board equipment switches to UN mode |  |  | L0 |  |  | UN |  | - |  |
|  | 37 |  | 9 |  |  | 0.00 |  | L0 |  | UN |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=4) is recorded |  | O |  | JRU |  | Triggering Event: When the mode changes |  |  | L0 |  |  | UN |  | - |  |
|  | 38 |  | 10 |  |  | 0.00 |  | L0 |  | UN |  |  | The Mode symbol "Unfitted" is displayed |  | O |  | DMI |  | DMI Object: Symbol MO16 |  |  | L0 |  |  | UN |  | - |  |
|  | 39 |  | 11 |  |  | 0.00 |  | L0 |  | UN |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit31=1>) is recorded |  | O |  | JRU |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | Feature 5100100: 5.10.1: Level Transitions General requirements Test Case 14: The train receives from the balise group BGa the Level Transition announcement LX/LY. The driver is informed. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 40 |  | 1 |  |  | 100.00 |  | L0 |  | UN |  |  | An announcement of level transition to Level Y (Packet 41) is RECEIVED from Balise Group BGa: D_LEVELTR <> 0 and <> 32767 M_LEVELTR= M_LEVEL_Y (Level Y) |  | I |  | BTM |  | Order to switch to level Y at further location and this announcement will result in a change of the on-board level. _____ LNTC (NID_NTC=2) is announced |  |  | L0 |  |  | UN |  | - |  |
|  | 41 |  | 2 |  |  | 100.00 |  | L0 |  | UN |  |  | An indication of Announcement of Level Y is DISPLAYED. |  | O |  | DMI |  | ('Except in SL and SB modes') _____ DMI Symbol LE08 |  |  | L0 |  |  | UN |  | - |  |
|  | 42 |  | 3 |  |  | 100.00 |  | L0 |  | UN |  |  | Displayed indication is recorded |  | O |  | JRU |  | ('Except in SL and SB modes') |  |  | L0 |  |  | UN |  | - |  |
|  | 43 |  | 4 |  |  | 100.00 |  | L0 |  | UN |  |  | Balise Group BGa message, including Packet 41'Level transition order', is RECORDED. |  | O |  | JRU |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | Feature 5100400: 5.10.4: Acknowledgement of the level transition Test Case 41: The train enters in the acknowledgement window, and the driver is requested to acknowledge the transition from L0 to LNTC. The driver does not acknowledge before entering the LNTC area. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 44 |  | 1 |  |  | 450.00 |  | L0 |  | UN |  |  | The Level symbol "Level NTC announcement" is removed |  | O |  | DMI |  | The max safe front end of the train enters the acknowledgement window (defined in the level transition announcement with L_ACKLEVELTR). The symbol LE08 is REMOVED. |  |  | L0 |  |  | UN |  | - |  |
|  | 45 |  | 2 |  |  | 450.00 |  | L0 |  | UN |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit08=0>) is recorded |  | O |  | JRU |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | 46 |  | 3 |  |  | 450.00 |  | L0 |  | UN |  |  | The Level symbol "Acknowledge Level NTC announcement" is displayed |  | O |  | DMI |  | DMI Object: Symbol LE09 |  |  | L0 |  |  | UN |  | - |  |
|  | 47 |  | 4 |  |  | 450.00 |  | L0 |  | UN |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit09=1>) is recorded |  | O |  | JRU |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | 48 |  | 5 |  |  | 450.00 |  | L0 |  | UN |  |  | NOT The driver acknowledges "Level NTC announcement" |  | I |  | DMI |  | The driver does not acknowledge before entering the LNTC area. |  |  | L0 |  |  | UN |  | - |  |
|  | Feature 5100100: 5.10.1: Level Transitions General requirements Test Case 16: The train receives an order to switch immediately from L0 to LNTC at the current position. on-board equipment immediately switches to LNTC. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 50 |  | 1 |  |  | 500.00 |  | L0 |  | UN |  |  | The train reaches the Level transition border and an order to switch immediately to Level NTC is received from balise group BG1 (packet 41): D_LEVELTR=0 M_LEVELTR=1 NID_STMNTC |  | I |  | BTM |  | 'The onboard equipment sends a signal to the appropriate NTC, in order to be prepared for immediate activation' |  |  | L0 |  |  | UN |  | - |  |
|  | 51 |  | 2 |  |  | 500.00 |  | L0 |  | UN |  |  | Balise Group message, including Packet 41 'Level transition order', is RECORDED. |  | O |  | JRU |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | 52 |  | 3 |  |  | 500.00 |  | L0 |  | UN |  |  | Level NTC is DISPLAYED. |  | O |  | DMI |  | (Except in SL mode) The On-board equipment switches to Level NTC. |  |  | L0 |  |  | UN |  | - |  |
|  | 53 |  | 4 |  |  | 500.00 |  | L0 |  | UN |  |  | New Level NTC is RECORDED. |  | O |  | JRU |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 6: ETCS level switches to Level NTC .Transition Condition Id [56]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 54 |  | 2 |  |  | 500.00 |  | L0 |  | UN |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=1; M_MODE=13) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 55 |  | 3 |  |  | 500.00 |  | LNTC |  | SN |  |  | The Mode symbol 'National System' is displayed |  | O |  | DMI |  | DMI Object: MO19 |  |  | LNTC |  |  | SN |  | - |  |
|  | 56 |  | 4 |  |  | 500.00 |  | LNTC |  | SN |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit34=1>) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | Feature 5100400: 5.10.4: Acknowledgement of the level transition Test Case 2: The train has passed over the Level Transition border and the On-board equipment has switched from L0 to Level NTC upon receipt of the order. The driver acknowledges within the driver acknowledgement time [5 seconds] after the Level transition. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 57 |  | 1 |  |  | 525.00 |  | LNTC |  | SN |  |  | The Level symbol "Acknowledge Level NTC announcement" is displayed |  | O |  | DMI |  | DMI Object: Symbol LE09 |  |  | LNTC |  |  | SN |  | - |  |
|  | 59 |  | 3 |  |  | 525.00 |  | LNTC |  | SN |  |  | The driver acknowledges "Level NTC announcement" |  | I |  | DMI |  | The driver acknowledges the Level Transition within the driver acknowledgement time [five seconds]after the Level Transition. |  |  | LNTC |  |  | SN |  | - |  |
|  | 60 |  | 4 |  |  | 525.00 |  | LNTC |  | SN |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 1010) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 61 |  | 5 |  |  | 525.00 |  | LNTC |  | SN |  |  | The Level symbol "Acknowledge Level NTC announcement" is removed |  | O |  | DMI |  | DMI Object: Symbol LE09 |  |  | LNTC |  |  | SN |  | - |  |
|  | 62 |  | 6 |  |  | 525.00 |  | LNTC |  | SN |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit09=0>) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | Feature 5100100: 5.10.1: Level Transitions General requirements Test Case 14: The train receives from the balise group BGa the Level Transition announcement LX/LY. The driver is informed. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 63 |  | 1 |  |  | 700.00 |  | LNTC |  | SN |  |  | An announcement of level transition to Level Y (Packet 41) is RECEIVED from Balise Group BGa: D_LEVELTR <> 0 and <> 32767 M_LEVELTR= M_LEVEL_Y (Level Y) |  | I |  | BTM |  | Order to switch to level Y at further location and this announcement will result in a change of the on-board level. _____ L2 Annouced |  |  | LNTC |  |  | SN |  | - |  |
|  | 64 |  | 2 |  |  | 700.00 |  | LNTC |  | SN |  |  | An indication of Announcement of Level Y is DISPLAYED. |  | O |  | DMI |  | ('Except in SL and SB modes') |  |  | LNTC |  |  | SN |  | - |  |
|  | 65 |  | 3 |  |  | 700.00 |  | LNTC |  | SN |  |  | Displayed indication is recorded |  | O |  | JRU |  | ('Except in SL and SB modes') |  |  | LNTC |  |  | SN |  | - |  |
|  | 66 |  | 4 |  |  | 700.00 |  | LNTC |  | SN |  |  | Balise Group BGa message, including Packet 41'Level transition order', is RECORDED. |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | Feature 5100300: 5.10.3: Specific Additional Requirements Test Case 6: Transition from Level NTC to level 2/3 area .When the ERTMS/ETCS communication session is open in rear of the boarder location, Train Data shall be sent to the RBC (which acknowledges the data). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 67 |  | 1 |  |  | 750.00 |  | LNTC |  | SN |  |  | Train is moving |  | I |  | INT |  | Train location: in rear of the border of level transition location |  |  | LNTC |  |  | SN |  | - |  |
|  | 68 |  | 2 |  |  | 800.00 |  | LNTC |  | SN |  |  | Balise Group message containing packet 42 is received . |  | I |  | BTM |  | Packet 42: Q_RBC = 1 Q_SLEEPSESSION=1 The packet 42 is received on-board via balise group in rear of the border location |  |  | LNTC |  |  | SN |  | - |  |
|  | 69 |  | 3 |  |  | 800.00 |  | LNTC |  | SN |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | Feature 3050300: 3.5.3: Establishing a communication session Test Case 22: System version is compatible and session is established. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 73 |  | 3 |  |  | 800.00 |  | LNTC |  | SN |  |  | SA-CONNECT.Request is transmitted |  | O |  | RTM |  | According to EURORADIO specifications. This attempt shall be repeated until successful. The onboard tries to set up a connection with the phone number included in the session management packet. |  |  | LNTC |  |  | SN |  | - |  |
|  | 74 |  | 4 |  |  | 800.00 |  | LNTC |  | SN |  |  | SA-CONNECT.Confirm is received |  | I |  | RTM |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 75 |  | 5 |  |  | 800.00 |  | LNTC |  | SN |  |  | The Status symbol "Safe radio connection - Connection Up" is displayed |  | O |  | DMI |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 76 |  | 6 |  |  | 800.00 |  | LNTC |  | SN |  |  | JRU step 'DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit40=1>) is recorded'. |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 77 |  | 7 |  |  | 800.00 |  | LNTC |  | SN |  |  | SA-DATA.Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted |  | O |  | RTM |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 78 |  | 8 |  |  | 800.00 |  | LNTC |  | SN |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 79 |  | 9 |  |  | 800.00 |  | LNTC |  | SN |  |  | SA-DATA.Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32) is received |  | I |  | RTM |  | System versions M_VERSION (Version of the ERTMS/ETCS language) are compatible, so that the communication session established for on-board |  |  | LNTC |  |  | SN |  | - |  |
|  | 80 |  | 10 |  |  | 800.00 |  | LNTC |  | SN |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 81 |  | 11 |  |  | 800.00 |  | LNTC |  | SN |  |  | SA-DATA.Request with Euroradio Message "Session Established" (NID_MESSAGE=159) including packet 2 is transmitted |  | O |  | RTM |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 82 |  | 12 |  |  | 800.00 |  | LNTC |  | SN |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | Feature 5100300: 5.10.3: Specific Additional Requirements Test Case 6: Transition from Level NTC to level 2/3 area .When the ERTMS/ETCS communication session is open in rear of the boarder location, Train Data shall be sent to the RBC (which acknowledges the data). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 84 |  | 5 |  |  | 800.00 |  | LNTC |  | SN |  |  | SA-DATA. Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) containing packet 11 is transmitted |  | O |  | RTM |  | This message is not sent in NL or SL mode |  |  | LNTC |  |  | SN |  | - |  |
|  | 85 |  | 6 |  |  | 800.00 |  | LNTC |  | SN |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 86 |  | 7 |  |  | 800.00 |  | LNTC |  | SN |  |  | SA-DATA. Indication with Euroradio Message "Acknowledgement of Train Data" (NID_MESSAGE=8) is received. |  | I |  | RTM |  | The RBC acknowledges train data. This message is not sent in NL or SL mode |  |  | LNTC |  |  | SN |  | - |  |
|  | 87 |  | 8 |  |  | 800.00 |  | LNTC |  | SN |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | Feature 5100400: 5.10.4: Acknowledgement of the level transition Test Case 48: The train enters in the acknowledgement window, and the driver is requested to acknowledge the transition from LNTC to L2. The driver does not acknowledge before entering the L2 area. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 88 |  | 1 |  |  | 800.00 |  | LNTC |  | SN |  |  | The Level symbol "Level 2 announcement" is removed |  | O |  | DMI |  | The max safe front end of the train enters the acknowledgement window (defined in the level transition announcement with L_ACKLEVELTR). The symbol LE12 is REMOVED. |  |  | LNTC |  |  | SN |  | - |  |
|  | 89 |  | 2 |  |  | 800.00 |  | LNTC |  | SN |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit12=0>) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 90 |  | 3 |  |  | 1100.00 |  | LNTC |  | SN |  |  | The Level symbol "Acknowledge Level 2 announcement" is displayed |  | O |  | DMI |  | DMI Object: Symbol LE13 |  |  | LNTC |  |  | SN |  | - |  |
|  | 91 |  | 4 |  |  | 1100.00 |  | LNTC |  | SN |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit13=1>) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 92 |  | 5 |  |  | 1100.00 |  | LNTC |  | SN |  |  | NOT The driver acknowledges "Level 2 announcement" |  | I |  | DMI |  | The driver does not acknowledge before entering the L2 area. |  |  | LNTC |  |  | SN |  | - |  |
|  | Feature 4080425: 4.8.4: Accepted Information depending on the modes - Movement Authority Test Case 18: The train receives a message that is transmitted via RBC. The message is accepted because a level 2 /3 transition is stored onborad |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 93 |  | 1 |  |  | 1150.00 |  | LNTC |  | SN |  |  | SA-Data.Indication with Euroradio message (NID_MESSAGE=3) containing packet 15, packet 21, packet 27 is received |  | I |  | RTM |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 94 |  | 2 |  |  | 1150.00 |  | LNTC |  | SN |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | Feature 5100100: 5.10.1: Level Transitions General requirements Test Case 21: The train receives an order to switch immediately from LNTC to L2 at the current position. on-board equipment immediately switches to L2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 102 |  | 1 |  |  | 1200.00 |  | LNTC |  | SN |  |  | The train reaches the Level transition border and an order to switch immediately to Level 2 is received from balise group BGa. Packet 41: 'Level Transition Order' is RECEIVED. D_LEVELTR= 0 or = 32767 M_LEVELTR=3 |  | I |  | BTM |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 103 |  | 2 |  |  | 1200.00 |  | LNTC |  | SN |  |  | Balise Group message, including Packet 41 'Level transition order', is RECORDED. |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 104 |  | 3 |  |  | 1200.00 |  | LNTC |  | SN |  |  | Level 2 is DISPLAYED. |  | O |  | DMI |  | (Except if in SL Mode) The On-board equipment switches to Level 2. The on-board equipment sends a signal to deactivate the appropriate NTC |  |  | LNTC |  |  | SN |  | - |  |
|  | 105 |  | 4 |  |  | 1200.00 |  | LNTC |  | SN |  |  | New Level 2 is RECORDED. |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | Feature 5100300: 5.10.3: Specific Additional Requirements Test Case 15: When the level transition location is passed with the estimated front end a position report shall be sent to the RBC. In case the ERTMS/ETCS on-board equipment is interfaced to the National System through an STM, please refer to SUBSET-035 for the STM state transition order. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 111 |  | 7 |  |  | 1200.00 |  | LNTC |  | SN |  |  | Train is moving. |  | I |  | INT |  | Train location: the estimated front end of the train passes the level transition location |  |  | LNTC |  |  | SN |  | - |  |
|  | 112 |  | 8 |  |  | 1200.00 |  | LNTC |  | SN |  |  | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted. |  | O |  | RTM |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 113 |  | 9 |  |  | 1200.00 |  | LNTC |  | SN |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | Feature 5100400: 5.10.4: Acknowledgement of the level transition Test Case 31: The train has passed over the Level Transition border and the On-board equipment has switched from LNTC to Level 2, upon receipt of an order. The driver acknowledges within the driver acknowledgement time[5 seconds] after the Level transition. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 114 |  | 1 |  |  | 1200.00 |  | LNTC |  | SN |  |  | The Level symbol "Acknowledge Level 2 announcement" is displayed |  | O |  | DMI |  | DMI Object: Symbol LE13 |  |  | LNTC |  |  | SN |  | - |  |
|  | 116 |  | 3 |  |  | 1200.00 |  | LNTC |  | SN |  |  | The driver acknowledges "Level 2 announcement" |  | I |  | DMI |  | The driver acknowledges the Level Transition within the driver acknowledgement time [5 seconds] after the Level Transition. |  |  | LNTC |  |  | SN |  | - |  |
|  | 117 |  | 4 |  |  | 1200.00 |  | LNTC |  | SN |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 1000) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 118 |  | 5 |  |  | 1200.00 |  | LNTC |  | SN |  |  | The Level symbol "Acknowledge Level 2 announcement" is removed |  | O |  | DMI |  | DMI Object: Symbol LE13 |  |  | LNTC |  |  | SN |  | - |  |
|  | 119 |  | 6 |  |  | 1200.00 |  | LNTC |  | SN |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit13=0>) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 19: Before the train reaches the level border, a full supervision MA + track description (SSP + gradient) is transferred to the on-board. The track description is available starting from the level border that means not for the whole length of the train but from the Front End. At the level border the order to switch to ERTMS / ETCS level 1, 2 or 3 is received. Then the on-board switches immediately to Full Supervision mode. Transition Condition Id [25]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 122 |  | 3 |  |  | 1200.00 |  | LNTC |  | SN |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL= 2/ 3/ 4; M_MODE=0) is recorded |  | O |  | JRU |  | The level changes to ETCS level 1, 2 or 3. The on-board equipment switches immediately to FS mode |  |  | L2 |  |  | FS |  | - |  |
|  | 123 |  | 4 |  |  | 1200.00 |  | L2 |  | FS |  |  | The system status message 'Entering FS' is displayed. |  | O |  | DMI |  | Displayed to the driver until SSP and gradients are known for the whole length of the train. |  |  | L2 |  |  | FS |  | - |  |
|  | 124 |  | 5 |  |  | 1200.00 |  | L2 |  | FS |  |  | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23; SYSTEM_STATUS_MESSAGE=<Bit04=1>) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 125 |  | 6 |  |  | 0.00 |  | L2 |  | FS |  |  | The Level symbol 'Level <L1/ L2/ L3>' is displayed |  | O |  | DMI |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 127 |  | 8 |  |  | 1200.00 |  | L2 |  | FS |  |  | The Mode symbol 'Full Supervision' is displayed |  | O |  | DMI |  | DMI Object: Symbol MO11 |  |  | L2 |  |  | FS |  | - |  |
|  | 128 |  | 9 |  |  | 1200.00 |  | L2 |  | FS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit26=1>) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 129 |  | 10 |  |  | 1200.00 |  | L2 |  | FS |  |  | The system status message 'Entering FS' is removed. |  | O |  | DMI |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 130 |  | 11 |  |  | 1200.00 |  | L2 |  | FS |  |  | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23; SYSTEM_STATUS_MESSAGE=<Bit04=0>) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | Feature 5100100: 5.10.1: Level Transitions General requirements Test Case 14: The train receives from the balise group BGa the Level Transition announcement LX/LY. The driver is informed. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 131 |  | 1 |  |  | 1450.00 |  | L2 |  | FS |  |  | An announcement of level transition to Level Y (Packet 41) is RECEIVED from Balise Group BGa: D_LEVELTR <> 0 and <> 32767 M_LEVELTR= M_LEVEL_Y (Level Y) |  | I |  | BTM |  | Order to switch to level Y at further location and this announcement will result in a change of the on-board level. _____ LNTC is announced |  |  | L2 |  |  | FS |  | - |  |
|  | 132 |  | 2 |  |  | 1450.00 |  | L2 |  | FS |  |  | An indication of Announcement of Level Y is DISPLAYED. |  | O |  | DMI |  | ('Except in SL and SB modes') _____ DMI Symbol LE08 |  |  | L2 |  |  | FS |  | - |  |
|  | 133 |  | 3 |  |  | 1450.00 |  | L2 |  | FS |  |  | Displayed indication is recorded |  | O |  | JRU |  | ('Except in SL and SB modes') |  |  | L2 |  |  | FS |  | - |  |
|  | 134 |  | 4 |  |  | 1450.00 |  | L2 |  | FS |  |  | Balise Group BGa message, including Packet 41'Level transition order', is RECORDED. |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | Feature 5100400: 5.10.4: Acknowledgement of the level transition Test Case 43: The train enters in the acknowledgement window, and the driver is requested to acknowledge the transition from L2 to LNTC. The driver does not acknowledge before entering the LNTC area. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 135 |  | 1 |  |  | 1950.00 |  | L2 |  | FS |  |  | The Level symbol "Level NTC announcement" is removed |  | O |  | DMI |  | The max safe front end of the train enters the acknowledgement window (defined in the level transition announcement with L_ACKLEVELTR). The displayed symbol LE08 is REMOVED. |  |  | L2 |  |  | FS |  | - |  |
|  | 136 |  | 2 |  |  | 1950.00 |  | L2 |  | FS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit08=0>) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 137 |  | 3 |  |  | 1950.00 |  | L2 |  | FS |  |  | The Level symbol "Acknowledge Level NTC announcement" is displayed |  | O |  | DMI |  | DMI Object: Symbol LE09 |  |  | L2 |  |  | FS |  | - |  |
|  | 138 |  | 4 |  |  | 1950.00 |  | L2 |  | FS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit09=1>) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 139 |  | 5 |  |  | 1950.00 |  | L2 |  | FS |  |  | NOT The driver acknowledges "Level NTC announcement" |  | I |  | DMI |  | The driver does not acknowledge before entering the LNTC area. |  |  | L2 |  |  | FS |  | - |  |
|  | Feature 5100100: 5.10.1: Level Transitions General requirements Test Case 30: The train receives an order to switch immediately from L2 to LNTC at the current position. on-board equipment immediately switches to LNTC. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 141 |  | 1 |  |  | 2000.00 |  | L2 |  | FS |  |  | The train reaches the Level transition border and an order to switch immediately to Level NTC is received from balise group BG1 (packet 41): D_LEVELTR=0 M_LEVELTR=1 NID_NTC |  | I |  | BTM |  | 'The on-board equipment sends a signal to the appropriate NTC, in order to be prepared for immediate activation' |  |  | L2 |  |  | FS |  | - |  |
|  | 142 |  | 2 |  |  | 2000.00 |  | L2 |  | FS |  |  | Balise Group message, including Packet 41 'Level transition order', is RECORDED. |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 143 |  | 3 |  |  | 2000.00 |  | L2 |  | FS |  |  | Level NTC is DISPLAYED. |  | O |  | DMI |  | (Except in SL mode) The On-board equipment switches to Level NTC. |  |  | L2 |  |  | FS |  | - |  |
|  | 144 |  | 4 |  |  | 2000.00 |  | L2 |  | FS |  |  | New Level NTC is RECORDED. |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 6: ETCS level switches to Level NTC .Transition Condition Id [56]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 145 |  | 2 |  |  | 2000.00 |  | L2 |  | FS |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=1; M_MODE=13) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 146 |  | 3 |  |  | 2000.00 |  | LNTC |  | SN |  |  | The Mode symbol 'National System' is displayed |  | O |  | DMI |  | DMI Object: MO19 |  |  | LNTC |  |  | SN |  | - |  |
|  | 147 |  | 4 |  |  | 2000.00 |  | LNTC |  | SN |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit34=1>) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | Feature 5100400: 5.10.4: Acknowledgement of the level transition Test Case 16: The train has passed over the Level Transition border and the On-board equipment has switched from L2 to Level NTC, upon receipt of an order. The driver acknowledges within the driver acknowledgement time[5 seconds] after the Level transition. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 148 |  | 1 |  |  | 2000.00 |  | LNTC |  | SN |  |  | The Level symbol "Acknowledge Level NTC announcement" is displayed |  | O |  | DMI |  | DMI Object: Symbol LE09 |  |  | LNTC |  |  | SN |  | - |  |
|  | 150 |  | 3 |  |  | 2000.00 |  | LNTC |  | SN |  |  | The driver acknowledges "Level NTC announcement" |  | I |  | DMI |  | The driver acknowledges the Level Transition within the driver acknowledgement time[5 seconds]after the Level Transition. |  |  | LNTC |  |  | SN |  | - |  |
|  | 151 |  | 4 |  |  | 2000.00 |  | LNTC |  | SN |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 1010) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 152 |  | 5 |  |  | 2000.00 |  | LNTC |  | SN |  |  | The Level symbol "Acknowledge Level NTC announcement" is removed |  | O |  | DMI |  | DMI Object: Symbol LE09 |  |  | LNTC |  |  | SN |  | - |  |
|  | 153 |  | 6 |  |  | 2000.00 |  | LNTC |  | SN |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit09=0>) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | Feature 5100300: 5.10.3: Specific Additional Requirements Test Case 10: Leading engines announce the complete exit (with its min safe rear end ) from level 2 / 3 to LEVEL NTC area with a position report to RBC and are requested to terminate session ; NL engines are only requested to terminate the session. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 154 |  | 1 |  |  | 2040.00 |  | LNTC |  | SN |  |  | Train is moving |  | I |  | INT |  | On-board Equipment checks the train position and detects that train entered completely in LNTC area. A balise group is located at the border, which becomes LRBG D_LRBG = L_TRAININT Note: this step is not carried out when the mode is NL |  |  | LNTC |  |  | SN |  | - |  |
|  | 155 |  | 2 |  |  | 2040.00 |  | LNTC |  | SN |  |  | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted. |  | O |  | RTM |  | Note: this step is not carried out when the mode is NL Note: this step is not carried out for non leading engines |  |  | LNTC |  |  | SN |  | - |  |
|  | 156 |  | 3 |  |  | 2040.00 |  | LNTC |  | SN |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | Note: this step is not carried out when the mode is NL Note: this step is not carried out for non leading engines |  |  | LNTC |  |  | SN |  | - |  |
|  | Feature 3050500: 3.5.5: Terminating a communication session Test Case 5: The on-board equipment terminates a communication session ordered by RBC. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 158 |  | 1 |  |  | 2040.00 |  | LNTC |  | SN |  |  | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 42 (Q_RBC=0) is received |  | I |  | RTM |  | Q_RBC = 0 (Terminate communication session) |  |  | LNTC |  |  | SN |  | - |  |
|  | 159 |  | 2 |  |  | 2040.00 |  | LNTC |  | SN |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 160 |  | 3 |  |  | 2040.00 |  | LNTC |  | SN |  |  | SA-DATA.Request with Euroradio Message "Termination of a communication session" (NID_MESSAGE=156) is transmitted |  | O |  | RTM |  | As soon as this message is received by trackside (RBC), the RBC shall consider the communication session terminated and it shall send the acknowledgement to the on-board. It shall be checked that no more messages are sent by the onboard |  |  | LNTC |  |  | SN |  | - |  |
|  | 161 |  | 4 |  |  | 2040.00 |  | LNTC |  | SN |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 162 |  | 5 |  |  | 2040.00 |  | LNTC |  | SN |  |  | SA-DATA.Indication with Euroradio Message "Acknowledgement of termination of a communication session" (NID_MESSAGE=39) is received |  | I |  | RTM |  | Message 39: 'Acknowledgement of termination of a communication session' is RECEIVED from the RBC and the on-board equipment considers the communication session terminated. |  |  | LNTC |  |  | SN |  | - |  |
|  | 163 |  | 6 |  |  | 2040.00 |  | LNTC |  | SN |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | When the acknowledgement is received, the onboard shall consider the communication session terminated and it shall begin the release of the safe connection with trackside. |  |  | LNTC |  |  | SN |  | - |  |
|  | 164 |  | 7 |  |  | 2040.00 |  | LNTC |  | SN |  |  | SA-DISCONNECT.Request is transmitted |  | O |  | RTM |  | The train tries to release the safe connection. |  |  | LNTC |  |  | SN |  | - |  |
|  | Feature 5100100: 5.10.1: Level Transitions General requirements Test Case 14: The train receives from the balise group BGa the Level Transition announcement LX/LY. The driver is informed. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 165 |  | 1 |  |  | 2300.00 |  | LNTC |  | SN |  |  | An announcement of level transition to Level Y (Packet 41) is RECEIVED from Balise Group BGa: D_LEVELTR <> 0 and <> 32767 M_LEVELTR= M_LEVEL_Y (Level Y) |  | I |  | BTM |  | Order to switch to level Y at further location and this announcement will result in a change of the on-board level. _____ LNTC (NID_NTC= 2) is announced. |  |  | LNTC |  |  | SN |  | - |  |
|  | 166 |  | 2 |  |  | 2300.00 |  | LNTC |  | SN |  |  | An indication of Announcement of Level Y is DISPLAYED. |  | O |  | DMI |  | ('Except in SL and SB modes') |  |  | LNTC |  |  | SN |  | - |  |
|  | 167 |  | 3 |  |  | 2300.00 |  | LNTC |  | SN |  |  | Displayed indication is recorded |  | O |  | JRU |  | ('Except in SL and SB modes') |  |  | LNTC |  |  | SN |  | - |  |
|  | 168 |  | 4 |  |  | 2300.00 |  | LNTC |  | SN |  |  | Balise Group BGa message, including Packet 41'Level transition order', is RECORDED. |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | Feature 5100400: 5.10.4: Acknowledgement of the level transition Test Case 50: The train enters in the acknowledgement window, and the driver is requested to acknowledge the transition from LNTC[X] to LNTC[Y]. The driver does not acknowledge before entering the LNTC[Y] area. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 169 |  | 1 |  |  | 2450.00 |  | LNTC |  | SN |  |  | The Level symbol "Level NTC announcement" is removed |  | O |  | DMI |  | The max safe front end of the train enters the acknowledgement window (defined in the level transition announcement with L_ACKLEVELTR). The symbol LE08 is REMOVED. |  |  | LNTC |  |  | SN |  | - |  |
|  | 170 |  | 2 |  |  | 2450.00 |  | LNTC |  | SN |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit08=0>) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 171 |  | 3 |  |  | 2450.00 |  | LNTC |  | SN |  |  | The Level symbol "Acknowledge Level NTC announcement" is displayed |  | O |  | DMI |  | DMI Object: Symbol LE09 |  |  | LNTC |  |  | SN |  | - |  |
|  | 172 |  | 4 |  |  | 2450.00 |  | LNTC |  | SN |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit09=1>) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 173 |  | 5 |  |  | 2450.00 |  | LNTC |  | SN |  |  | NOT The driver acknowledges "Level NTC announcement" |  | I |  | DMI |  | The driver does not acknowledge before entering the LNTC[Y] area. |  |  | LNTC |  |  | SN |  | - |  |
|  | Feature 5100100: 5.10.1: Level Transitions General requirements Test Case 19: The train receives an order to switch immediately from LNTC[X] to LNTC[Y] at the current position. on-board equipment immediately switches to LNTC[Y]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 174 |  | 1 |  |  | 2500.00 |  | LNTC |  | SN |  |  | The train reaches the Level transition border and an order to switch immediately to Level NTC[Y] is received from balise group BGa. Packet 41: 'Level Transition Order' is RECEIVED. D_LEVELTR= 0 or = 32767 M_LEVELTR=1 (Level NTC) NID_NTC=FINITE VALUE (NTC[Y]) |  | I |  | BTM |  | The on-board equipment sends a signal to deactivate the appropriate NTC[X] |  |  | LNTC |  |  | SN |  | - |  |
|  | 175 |  | 2 |  |  | 2500.00 |  | LNTC |  | SN |  |  | Balise Group message, including Packet 41 'Level transition order', is RECORDED. |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 176 |  | 3 |  |  | 2500.00 |  | LNTC |  | SN |  |  | Level NTC[Y] is DISPLAYED. |  | O |  | DMI |  | (Except if in SL Mode) The On-board equipment switches to Level NTC[Y]. |  |  | LNTC |  |  | SN |  | - |  |
|  | 177 |  | 4 |  |  | 2500.00 |  | LNTC |  | SN |  |  | New Level NTC[Y] is RECORDED. |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | Feature 5100400: 5.10.4: Acknowledgement of the level transition Test Case 37: The train has passed over the Level Transition border and the On-board equipment has switched from LNTC [X ] to Level NTC[Y] upon receipt of the order. The driver acknowledges within the driver acknowledgement time [5 seconds] after the Level transition. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 178 |  | 1 |  |  | 2500.00 |  | LNTC |  | SN |  |  | The Level symbol "Acknowledge Level NTC announcement" is displayed |  | O |  | DMI |  | DMI Object: Symbol LE09 |  |  | LNTC |  |  | SN |  | - |  |
|  | 180 |  | 3 |  |  | 2500.00 |  | LNTC |  | SN |  |  | The driver acknowledges "Level NTC announcement" |  | I |  | DMI |  | The driver acknowledges the Level Transition within the driver acknowledgement time [five seconds] after the Level Transition. |  |  | LNTC |  |  | SN |  | - |  |
|  | 181 |  | 4 |  |  | 2500.00 |  | LNTC |  | SN |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 1010) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 182 |  | 5 |  |  | 2500.00 |  | LNTC |  | SN |  |  | The Level symbol "Acknowledge Level NTC announcement" is removed |  | O |  | DMI |  | DMI Object: Symbol LE09 |  |  | LNTC |  |  | SN |  | - |  |
|  | 183 |  | 6 |  |  | 2500.00 |  | LNTC |  | SN |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit09=0>) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | Feature 5100100: 5.10.1: Level Transitions General requirements Test Case 14: The train receives from the balise group BGa the Level Transition announcement LX/LY. The driver is informed. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 184 |  | 1 |  |  | 2700.00 |  | LNTC |  | SN |  |  | An announcement of level transition to Level Y (Packet 41) is RECEIVED from Balise Group BGa: D_LEVELTR <> 0 and <> 32767 M_LEVELTR= M_LEVEL_Y (Level Y) |  | I |  | BTM |  | Order to switch to level Y at further location and this announcement will result in a change of the on-board level. _____ L1 announced |  |  | LNTC |  |  | SN |  | - |  |
|  | 185 |  | 2 |  |  | 2700.00 |  | LNTC |  | SN |  |  | An indication of Announcement of Level Y is DISPLAYED. |  | O |  | DMI |  | ('Except in SL and SB modes') _____ DMI Symbol LE10 |  |  | LNTC |  |  | SN |  | - |  |
|  | 186 |  | 3 |  |  | 2700.00 |  | LNTC |  | SN |  |  | Displayed indication is recorded |  | O |  | JRU |  | ('Except in SL and SB modes') |  |  | LNTC |  |  | SN |  | - |  |
|  | 187 |  | 4 |  |  | 2700.00 |  | LNTC |  | SN |  |  | Balise Group BGa message, including Packet 41'Level transition order', is RECORDED. |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | Feature 4080424: 4.8.4: Accepted Information depending on the modes - Mode Profile Test Case 12: The train receives a message including packet 80 (Mode Profile) that is transmitted via balise group. The message is accepted because a level transition is stored onborad |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 188 |  | 1 |  |  | 2750.00 |  | LNTC |  | SN |  |  | One Balise Group message (N_TOTAL=0) containing packet 12, packet 21, packet 27, packet 80 is received |  | I |  | BTM |  | M_MAMODE inside packet 80 has to garantee a change of mode at the end of this TC. |  |  | LNTC |  |  | SN |  | - |  |
|  | 189 |  | 2 |  |  | 2750.00 |  | LNTC |  | SN |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | Complete message including Mode Profile is recorded |  |  | LNTC |  |  | SN |  | - |  |
|  | Feature 5100400: 5.10.4: Acknowledgement of the level transition Test Case 47: The train enters in the acknowledgement window, and the driver is requested to acknowledge the transition from LNTC to L1. The driver does not acknowledge before entering the L1 area. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 193 |  | 1 |  |  | 2950.00 |  | LNTC |  | SN |  |  | The Level symbol "Level 1 announcement" is removed |  | O |  | DMI |  | The max safe front end of the train enters the acknowledgement window (defined in the level transition announcement with L_ACKLEVELTR). The symbol LE10 is REMOVED. |  |  | LNTC |  |  | SN |  | - |  |
|  | 194 |  | 2 |  |  | 2950.00 |  | LNTC |  | SN |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit10=0>) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 195 |  | 3 |  |  | 2950.00 |  | LNTC |  | SN |  |  | The Level symbol "Acknowledge Level 1 announcement" is displayed |  | O |  | DMI |  | DMI Object: Symbol LE11 |  |  | LNTC |  |  | SN |  | - |  |
|  | 196 |  | 4 |  |  | 2950.00 |  | LNTC |  | SN |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit11=1>) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 197 |  | 5 |  |  | 2950.00 |  | LNTC |  | SN |  |  | NOT The driver acknowledges "Level 1 announcement" |  | I |  | DMI |  | The driver does not acknowledge before entering the L1 area. |  |  | LNTC |  |  | SN |  | - |  |
|  | Feature 5190300: 5.19.3: Limited Supervision is requested for a further location Test Case 4: A Mode profile defining an LS area is available on-board. The train passes the beginning of the Limited Supervision area with its front end and the ERTMS/ETCS level switches to 1, 2 or 3. Then the on-board switches to LS mode. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 198 |  | 1 |  |  | 2950.00 |  | LNTC |  | SN |  |  | NOT The Mode symbol "Limited Supervision" is displayed |  | O |  | DMI |  | Mode transition was not valuated before the level change |  |  | LNTC |  |  | SN |  | - |  |
|  | Feature 5100100: 5.10.1: Level Transitions General requirements Test Case 23: The train receives an order to switch immediately from LNTC to L1 at the current position. on-board equipment immediately switches to L1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 200 |  | 1 |  |  | 3000.00 |  | LNTC |  | SN |  |  | The train reaches the Level transition border and an order to switch immediately to Level 1 is received from balise group BGa. Packet 41: 'Level Transition Order' is RECEIVED. D_LEVELTR= 0 or = 32767 M_LEVELTR=2 |  | I |  | BTM |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 201 |  | 2 |  |  | 3000.00 |  | LNTC |  | SN |  |  | Balise Group message, including Packet 41 'Level transition order', is RECORDED. |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 202 |  | 3 |  |  | 3000.00 |  | LNTC |  | SN |  |  | Level 1 is DISPLAYED. |  | O |  | DMI |  | (Except in SL mode ' desk is closed) The On-board equipment switches to Level 1. |  |  | LNTC |  |  | SN |  | - |  |
|  | 203 |  | 4 |  |  | 3000.00 |  | LNTC |  | SN |  |  | New Level 1 is RECORDED. |  | O |  | JRU |  | - |  |  | L1 |  |  | SN |  | - |  |
|  | Feature 5100400: 5.10.4: Acknowledgement of the level transition Test Case 28: The train has passed over the Level Transition border and the On-board equipment has switched from LNTC to Level 1, upon receipt of an order. The driver acknowledges within the driver acknowledgement time[5 seconds] after the Level transition. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 204 |  | 1 |  |  | 3000.00 |  | L1 |  | SN |  |  | The Level symbol "Acknowledge Level 1 announcement" is displayed |  | O |  | DMI |  | DMI Object: Symbol LE11 |  |  | L1 |  |  | SN |  | - |  |
|  | 206 |  | 3 |  |  | 3000.00 |  | L1 |  | SN |  |  | The driver acknowledges "Level 1 announcement" |  | I |  | DMI |  | The driver acknowledges the Level Transition within the driver acknowledgement time [5 seconds] after the Level Transition |  |  | L1 |  |  | SN |  | - |  |
|  | 207 |  | 4 |  |  | 3000.00 |  | L1 |  | SN |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 0111) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SN |  | - |  |
|  | 208 |  | 5 |  |  | 3000.00 |  | L1 |  | SN |  |  | The Level symbol "Acknowledge Level 1 announcement" is removed |  | O |  | DMI |  | DMI Object: Symbol LE11 |  |  | L1 |  |  | SN |  | - |  |
|  | 209 |  | 6 |  |  | 3000.00 |  | L1 |  | SN |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit11=0>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SN |  | - |  |
|  | Feature 5190300: 5.19.3: Limited Supervision is requested for a further location Test Case 4: A Mode profile defining an LS area is available on-board. The train passes the beginning of the Limited Supervision area with its front end and the ERTMS/ETCS level switches to 1, 2 or 3. Then the on-board switches to LS mode. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 210 |  | 3 |  |  | 3000.00 |  | L1 |  | SN |  |  | The Level symbol "Level <L1 / L2 / L3>" is displayed |  | O |  | DMI |  | Mode transition was not valuated before the level change |  |  | L1 |  |  | SN |  | - |  |
|  | 211 |  | 4 |  |  | 3000.00 |  | L1 |  | SN |  |  | Mode transition to LS is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  |  | LS |  | - |  |
|  | 212 |  | 5 |  |  | 3000.00 |  | L1 |  | LS |  |  | LS mode indication is DISPLAYED |  | O |  | DMI |  | - |  |  | L1 |  |  | LS |  | - |  |
|  | 213 |  | 6 |  |  | 3000.00 |  | L1 |  | LS |  |  | A request for acknowledgement of LS mode is DISPLAYED to the driver |  | O |  | DMI |  | - |  |  | L1 |  |  | LS |  | - |  |
|  | 214 |  | 7 |  |  | 3000.00 |  | L1 |  | LS |  |  | Driver acknowledges LS mode |  | I |  | DMI |  | - |  |  | L1 |  |  | LS |  | - |  |
|  | 215 |  | 8 |  |  | 3000.00 |  | L1 |  | LS |  |  | Driver Acknowledgement of transition to LS mode is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  |  | LS |  | - |  |
|  | Feature 5100100: 5.10.1: Level Transitions General requirements Test Case 14: The train receives from the balise group BGa the Level Transition announcement LX/LY. The driver is informed. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 218 |  | 1 |  |  | 3200.00 |  | L1 |  | LS |  |  | An announcement of level transition to Level Y (Packet 41) is RECEIVED from Balise Group BGa: D_LEVELTR <> 0 and <> 32767 M_LEVELTR= M_LEVEL_Y (Level Y) |  | I |  | BTM |  | Order to switch to level Y at further location and this announcement will result in a change of the on-board level. _____ LNTC Announced |  |  | L1 |  |  | LS |  | - |  |
|  | 219 |  | 2 |  |  | 3200.00 |  | L1 |  | LS |  |  | An indication of Announcement of Level Y is DISPLAYED. |  | O |  | DMI |  | ('Except in SL and SB modes') _____ DMI Symbol LE08 |  |  | L1 |  |  | LS |  | - |  |
|  | 220 |  | 3 |  |  | 3200.00 |  | L1 |  | LS |  |  | Displayed indication is recorded |  | O |  | JRU |  | ('Except in SL and SB modes') |  |  | L1 |  |  | LS |  | - |  |
|  | 221 |  | 4 |  |  | 3200.00 |  | L1 |  | LS |  |  | Balise Group BGa message, including Packet 41'Level transition order', is RECORDED. |  | O |  | JRU |  | - |  |  | L1 |  |  | LS |  | - |  |
|  | Feature 5100400: 5.10.4: Acknowledgement of the level transition Test Case 42: The train enters in the acknowledgement window, and the driver is requested to acknowledge the transition from L1 to LNTC. The driver does not acknowledge before entering the LNTC area. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 222 |  | 1 |  |  | 3450.00 |  | L1 |  | LS |  |  | The Level symbol "Level NTC announcement" is removed |  | O |  | DMI |  | The max safe front end of the train enters the acknowledgement window (defined in the level transition announcement with L_ACKLEVELTR). The symbol LE08 is REMOVED. |  |  | L1 |  |  | LS |  | - |  |
|  | 223 |  | 2 |  |  | 3450.00 |  | L1 |  | LS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit08=0>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | LS |  | - |  |
|  | 224 |  | 3 |  |  | 3450.00 |  | L1 |  | LS |  |  | The Level symbol "Acknowledge Level NTC announcement" is displayed |  | O |  | DMI |  | DMI Object: Symbol LE09 |  |  | L1 |  |  | LS |  | - |  |
|  | 225 |  | 4 |  |  | 3450.00 |  | L1 |  | LS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit09=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | LS |  | - |  |
|  | 226 |  | 5 |  |  | 3450.00 |  | L1 |  | LS |  |  | NOT The driver acknowledges "Level NTC announcement" |  | I |  | DMI |  | The driver does not acknowledge before entering the LNTC area. |  |  | L1 |  |  | LS |  | - |  |
|  | Feature 5100100: 5.10.1: Level Transitions General requirements Test Case 34: The train receives an order to switch immediately from L1 to LNTC at the current position. on-board equipment immediately switches to LNTC. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 228 |  | 1 |  |  | 3500.00 |  | L1 |  | LS |  |  | The train reaches the Level transition border and an order to switch immediately to Level NTC is received from balise group BG1 (packet 41): D_LEVELTR=0 M_LEVELTR=1 NID_NTC |  | I |  | BTM |  | 'The on-board equipment sends a signal to the appropriate NTC, in order to be prepared for immediate activation' |  |  | L1 |  |  | LS |  | - |  |
|  | 229 |  | 2 |  |  | 3500.00 |  | L1 |  | LS |  |  | Balise Group message, including Packet 41 'Level transition order', is RECORDED. |  | O |  | JRU |  | - |  |  | L1 |  |  | LS |  | - |  |
|  | 230 |  | 3 |  |  | 3500.00 |  | L1 |  | LS |  |  | Level NTC is DISPLAYED. |  | O |  | DMI |  | (Except in SL mode) The On-board equipment switches to Level NTC. |  |  | L1 |  |  | LS |  | - |  |
|  | 231 |  | 4 |  |  | 3500.00 |  | L1 |  | LS |  |  | New Level NTC is RECORDED. |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 6: ETCS level switches to Level NTC .Transition Condition Id [56]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 232 |  | 2 |  |  | 3500.00 |  | LNTC |  | SN |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=1; M_MODE=13) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 233 |  | 3 |  |  | 3500.00 |  | LNTC |  | SN |  |  | The Mode symbol 'National System' is displayed |  | O |  | DMI |  | DMI Object: MO19 |  |  | LNTC |  |  | SN |  | - |  |
|  | 234 |  | 4 |  |  | 3500.00 |  | LNTC |  | SN |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit34=1>) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | Feature 5100400: 5.10.4: Acknowledgement of the level transition Test Case 9: The train has passed over the Level Transition border and the On-board equipment has switched from L1 to Level NTC, upon receipt of an order. The driver acknowledges within the driver acknowledgement time[5 seconds] after the Level transition. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 235 |  | 1 |  |  | 3500.00 |  | LNTC |  | SN |  |  | The Level symbol "Acknowledge Level NTC announcement" is displayed |  | O |  | DMI |  | DMI Object: Symbol LE09 |  |  | LNTC |  |  | SN |  | - |  |
|  | 237 |  | 3 |  |  | 3500.00 |  | LNTC |  | SN |  |  | The driver acknowledges "Level NTC announcement" |  | I |  | DMI |  | The driver acknowledges the Level Transition within the driver acknowledgement time[5seconds] after the Level Transition. |  |  | LNTC |  |  | SN |  | - |  |
|  | 238 |  | 4 |  |  | 3500.00 |  | LNTC |  | SN |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 1010) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 239 |  | 5 |  |  | 3500.00 |  | LNTC |  | SN |  |  | The Level symbol "Acknowledge Level NTC announcement" is removed |  | O |  | DMI |  | DMI Object: Symbol LE09 |  |  | LNTC |  |  | SN |  | - |  |
|  | 240 |  | 6 |  |  | 3500.00 |  | LNTC |  | SN |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit09=0>) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | Feature 5100100: 5.10.1: Level Transitions General requirements Test Case 14: The train receives from the balise group BGa the Level Transition announcement LX/LY. The driver is informed. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 241 |  | 1 |  |  | 3700.00 |  | LNTC |  | SN |  |  | An announcement of level transition to Level Y (Packet 41) is RECEIVED from Balise Group BGa: D_LEVELTR <> 0 and <> 32767 M_LEVELTR= M_LEVEL_Y (Level Y) |  | I |  | BTM |  | Order to switch to level Y at further location and this announcement will result in a change of the on-board level. _____ L0 announced |  |  | LNTC |  |  | SN |  | - |  |
|  | 242 |  | 2 |  |  | 3700.00 |  | LNTC |  | SN |  |  | An indication of Announcement of Level Y is DISPLAYED. |  | O |  | DMI |  | ('Except in SL and SB modes') _____ DMI Symbol LE06 |  |  | LNTC |  |  | SN |  | - |  |
|  | 243 |  | 3 |  |  | 3700.00 |  | LNTC |  | SN |  |  | Displayed indication is recorded |  | O |  | JRU |  | ('Except in SL and SB modes') |  |  | LNTC |  |  | SN |  | - |  |
|  | 244 |  | 4 |  |  | 3700.00 |  | LNTC |  | SN |  |  | Balise Group BGa message, including Packet 41'Level transition order', is RECORDED. |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | Feature 5100400: 5.10.4: Acknowledgement of the level transition Test Case 46: The train enters in the acknowledgement window, and the driver is requested to acknowledge the transition from LNTC to L0. The driver does not acknowledge before entering the L0 area. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 245 |  | 1 |  |  | 3950.00 |  | LNTC |  | SN |  |  | The Level symbol "Level 0 announcement" is removed |  | O |  | DMI |  | The max safe front end of the train enters the acknowledgement window (defined in the level transition announcement with L_ACKLEVELTR) The symbol LE06 is REMOVED. |  |  | LNTC |  |  | SN |  | - |  |
|  | 246 |  | 2 |  |  | 3950.00 |  | LNTC |  | SN |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit06=0>) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 247 |  | 3 |  |  | 3950.00 |  | LNTC |  | SN |  |  | The Level symbol "Acknowledge Level 0 announcement" is displayed |  | O |  | DMI |  | DMI Object: Symbol LE07 |  |  | LNTC |  |  | SN |  | - |  |
|  | 248 |  | 4 |  |  | 3950.00 |  | LNTC |  | SN |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit07=1>) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 249 |  | 5 |  |  | 3950.00 |  | LNTC |  | SN |  |  | NOT The driver acknowledges "Level 0 announcement" |  | I |  | DMI |  | The driver does not acknowledge before entering the L0 area. |  |  | LNTC |  |  | SN |  | - |  |
|  | Feature 5100100: 5.10.1: Level Transitions General requirements Test Case 24: The train receives an order to switch immediately from LNTC to L0 at the current position. on-board equipment immediately switches to L0. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 251 |  | 1 |  |  | 4000.00 |  | LNTC |  | SN |  |  | The train reaches the Level transition border and an order to switch immediately to Level 0 is received from balise group BGa. Packet 41: 'Level Transition Order' is RECEIVED. D_LEVELTR= 0 or = 32767 M_LEVELTR=0 |  | I |  | BTM |  | Reception of level transition order (Packet 41) |  |  | LNTC |  |  | SN |  | - |  |
|  | 252 |  | 2 |  |  | 4000.00 |  | LNTC |  | SN |  |  | Balise Group message, including Packet 41 'Level transition order', is RECORDED. |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 253 |  | 3 |  |  | 4000.00 |  | LNTC |  | SN |  |  | Level 0 is DISPLAYED |  | O |  | DMI |  | (Except in SL mode ' desk is closed) The On-board equipment switches to Level 0. |  |  | LNTC |  |  | SN |  | - |  |
|  | 254 |  | 4 |  |  | 4000.00 |  | LNTC |  | SN |  |  | New Level 0 is RECORDED |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | Feature 5100400: 5.10.4: Acknowledgement of the level transition Test Case 6: The train has passed over the Level Transition border and the On-board equipment has switched from L1 to Level 0, upon receipt of an order . The driver acknowledges within the driver acknowledgement time[5 seconds] after the Level transition. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 256 |  | 1 |  |  | 4000.00 |  | LNTC |  | SN |  |  | The Level symbol "Acknowledge Level 0 announcement" is displayed |  | O |  | DMI |  | DMI Object: Symbol LE07 |  |  | LNTC |  |  | SN |  | - |  |
|  | 257 |  | 2 |  |  | 4000.00 |  | LNTC |  | SN |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit07=1>) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 258 |  | 3 |  |  | 4000.00 |  | LNTC |  | SN |  |  | The driver acknowledges "Level 0 announcement" |  | I |  | DMI |  | The driver acknowledges the Level Transition within the driver acknowledgement time [5 seconds] after the Level Transition. |  |  | LNTC |  |  | SN |  | - |  |
|  | 259 |  | 4 |  |  | 4000.00 |  | LNTC |  | SN |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 0110) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 260 |  | 5 |  |  | 4000.00 |  | LNTC |  | SN |  |  | The Level symbol "Acknowledge Level 0 announcement" is removed |  | O |  | DMI |  | DMI Object: Symbol LE07 |  |  | LNTC |  |  | SN |  | - |  |
|  | 261 |  | 6 |  |  | 4000.00 |  | LNTC |  | SN |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit07=0>) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 3: ERTMS/ ETCS Level switches to level 0. Transition Condition Id [21]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 262 |  | 3 |  |  | 4000.00 |  | LNTC |  | SN |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=4) is recorded |  | O |  | JRU |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | 263 |  | 4 |  |  | 4000.00 |  | L0 |  | UN |  |  | The Mode symbol 'Unfitted' is displayed |  | O |  | DMI |  | DMI Object: MO16 |  |  | L0 |  |  | UN |  | - |  |
|  | 264 |  | 5 |  |  | 4000.00 |  | L0 |  | UN |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit31=1>) is recorded |  | O |  | JRU |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | Feature 5100400: 5.10.4: Acknowledgement of the level transition Test Case 25: The train has passed over the Level Transition border and the On-board equipment has switched from LNTC to Level 0, upon receipt of an order. The driver acknowledges within the driver acknowledgement time[5 seconds] after the Level transition. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 265 |  | 1 |  |  | 4000.00 |  | L0 |  | UN |  |  | The Level symbol "Acknowledge Level 0 announcement" is displayed |  | O |  | DMI |  | DMI Object: Symbol LE07 |  |  | L0 |  |  | UN |  | - |  |
|  | 267 |  | 3 |  |  | 4000.00 |  | L0 |  | UN |  |  | The driver acknowledges "Level 0 announcement" |  | I |  | DMI |  | The driver acknowledges the Level Transition within the driver acknowledgement time [five seconds]after the Level Transition |  |  | L0 |  |  | UN |  | - |  |
|  | 268 |  | 4 |  |  | 4000.00 |  | L0 |  | UN |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 0110) is recorded |  | O |  | JRU |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | 269 |  | 5 |  |  | 4000.00 |  | L0 |  | UN |  |  | The Level symbol "Acknowledge Level 0 announcement" is removed |  | O |  | DMI |  | DMI Object: Symbol LE07 |  |  | L0 |  |  | UN |  | - |  |
|  | 270 |  | 6 |  |  | 4000.00 |  | L0 |  | UN |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit07=0>) is recorded |  | O |  | JRU |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 1: The ETCS on-board system is NOT powered. Transition Condition Id [29]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 271 |  | 1 |  |  | 4250.00 |  | L0 |  | UN |  |  | The ERTMS/ETCS on-board unit is powered down |  | I |  | INT |  | - |  |  | L0 |  |  | NP |  | - |  |
|  | 272 |  | 2 |  |  | 4250.00 |  | L0 |  | NP |  |  | NOT GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=-) is recorded |  | O |  | JRU |  | There are no messages recorded in the JRU during a period of time |  |  | L0 |  |  | NP |  | - |  |
| Step 10: DMI Events Distance 0.00 m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Event Id. |  |  |  | Event Name |  |  |  |  |  |  | Data |  |  |  |  |  |  |  |  | Delay |  |  |  |  |  |  |  |  |
| 29 |  |  |  | Request Change Level |  |  |  |  |  |  | LEVEL=0 |  |  |  |  |  |  |  |  | 0 s |  |  |  |  |  |  |  |  |

| - Step 40 : Balise Group BG 01 d 1/2 - Distance 100.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 01 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 41 d | Level transition order |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 71 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_LEVELTR | 15 | 400 d |  |  |
| M_LEVELTR | 3 | 1 |  |  |
| NID_NTC | 8 | 2 d |  |  |
| L_ACKLEVELTR | 15 | 50 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 40 : Balise Group BG 01 d 2/2 - Distance 105.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 01 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 50 : Balise Group BG 02 d 1/2 - Distance 500.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 02 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 41 d | Level transition order |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 71 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_LEVELTR | 15 | 0 d |  |  |
| M_LEVELTR | 3 | 1 |  |  |
| NID_NTC | 8 | 2 d |  |  |
| L_ACKLEVELTR | 15 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 50 : Balise Group BG 02 d 2/2 - Distance 505.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 02 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 63 : Balise Group BG 03 d 1/2 - Distance 700.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 03 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 41 d | Level transition order |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 63 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_LEVELTR | 15 | 500 d |  |  |
| M_LEVELTR | 3 | 3 |  |  |
| L_ACKLEVELTR | 15 | 100 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 63 : Balise Group BG 03 d 2/2 - Distance 705.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 03 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 68 : Balise Group BG 04 d 1/2 - Distance 800.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 04 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 42 d | Session management |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 113 d |  |  |
| Q_RBC | 1 | 1 b |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_RBC | 14 | 1515 d |  |  |
| NID_RADIO | 64 | 003265342101 FFFFh |  |  |
| Q_SLEEPSESSION | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 68 : Balise Group BG 04 d 2/2 - Distance 805.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 04 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 77 : Message 155 - Initiation of a Communication Session (RBC n? 1) - Distance 800.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 155 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |

| - Step 79 : Message 32 - RBC/RIU System Version (RBC n? 1) - Distance 800.00 m Delay : 5.00 s Conditional Message Id : 155 Back Delay : 5.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 32 d |  |  |
| L_MESSAGE | 10 | 11 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 16777215 d |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |

| - Step 81 : Message 159 - Session Established (RBC n? 1) - Distance 800.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 159 d |  |  |
| L_MESSAGE | 10 | 14 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_PACKET | 8 | 2 d | System versions that the on-board equipment is able to operate |  |
| L_PACKET | 13 | 33 d |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| N_ITER | 5 | 0 d |  |  |

| - Step 84 : Message 129 - Validated Train Data (RBC n? 1) - Distance 800.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 129 d |  |  |
| L_MESSAGE | 10 | 41 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 122 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 1048580 d |  |  |
| D_LRBG | 15 | 25 d |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 0 d |  |  |
| L_DOUBTUNDER | 15 | 0 d |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 20 d |  |  |
| Q_DIRTRAIN | 2 | 01 b |  |  |
| M_MODE | 4 | 1101 b |  |  |
| M_LEVEL | 3 | 1 |  |  |
| NID_NTC | 8 | 2 d |  |  |
| NID_PACKET | 8 | 11 d | Validated train data |  |
| L_PACKET | 13 | 126 d |  |  |
| NC_CDTRAIN | 4 | 7 d |  |  |
| NC_TRAIN | 15 | 4 d |  |  |
| L_TRAIN | 12 | 40 d |  |  |
| V_MAXTRAIN | 7 | 80 d |  |  |
| M_LOADINGGAUGE | 8 | 00000001 b |  |  |
| M_AXLELOADCAT | 7 | 7 d |  |  |
| M_AIRTIGHT | 2 | 01 b |  |  |
| N_AXLE | 10 | 4 d |  |  |
| N_ITER | 5 | 1 d |  |  |
| M_VOLTAGE | 4 | 3 |  |  |
| NID_CTRACTION | 10 | 15 |  |  |
| N_ITER | 5 | 2 d |  |  |
| NID_NTC | 8 | 2 d |  |  |
| NID_NTC | 8 | 3 d |  |  |

| - Step 86 : Message 8 - Acknowledgement of Train Data (RBC n? 1) - Distance 800.00 m Delay : 5.00 s Conditional Message Id : 129 Back Delay : 5.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 8 d |  |  |
| L_MESSAGE | 10 | 14 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048580 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |

| - Step 93 : Message 3 - Movement Authority (RBC n? 1) - Distance 1150.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 3 d |  |  |
| L_MESSAGE | 10 | 32 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048580 d |  |  |
| NID_PACKET | 8 | 15 d | Level 2/3 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 66 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| V_EMA | 7 | 0 d |  |  |
| T_EMA | 10 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_ENDSECTION | 15 | 2500 d |  |  |
| Q_SECTIONTIMER | 1 | 0 b |  |  |
| Q_ENDTIMER | 1 | 0 b |  |  |
| Q_DANGERPOINT | 1 | 0 b |  |  |
| Q_OVERLAP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 21 d | Transmission of the gradient |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 54 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_GRADIENT | 15 | 400 d |  |  |
| Q_GDIR | 1 | 0 b |  |  |
| G_A | 8 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 27 d | International static speed profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 58 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_STATIC | 15 | 400 d |  |  |
| V_STATIC | 7 | 50 d |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |

| - Step 102 : Balise Group BG 06 d 1/2 - Distance 1200.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 06 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 41 d | Level transition order |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 63 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_LEVELTR | 15 | 0 d |  |  |
| M_LEVELTR | 3 | 3 |  |  |
| L_ACKLEVELTR | 15 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 102 : Balise Group BG 06 d 2/2 - Distance 1205.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 06 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 112 : Message 136 - Train Position Report (RBC n? 1) - Distance 1200.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 136 d |  |  |
| L_MESSAGE | 10 | 24 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 114 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 1048582 d |  |  |
| D_LRBG | 15 | 40 d |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 0 d |  |  |
| L_DOUBTUNDER | 15 | 0 d |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 20 d |  |  |
| Q_DIRTRAIN | 2 | 01 b |  |  |
| M_MODE | 4 | 0000 b |  |  |
| M_LEVEL | 3 | 3 |  |  |

| - Step 131 : Balise Group BG 07 d 1/2 - Distance 1450.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 07 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 41 d | Level transition order |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 71 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_LEVELTR | 15 | 550 d |  |  |
| M_LEVELTR | 3 | 1 |  |  |
| NID_NTC | 8 | 3 d |  |  |
| L_ACKLEVELTR | 15 | 50 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 131 : Balise Group BG 07 d 2/2 - Distance 1455.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 07 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 141 : Balise Group BG 08 d 1/2 - Distance 2000.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 08 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 41 d | Level transition order |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 71 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_LEVELTR | 15 | 0 d |  |  |
| M_LEVELTR | 3 | 1 |  |  |
| NID_NTC | 8 | 3 d |  |  |
| L_ACKLEVELTR | 15 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 141 : Balise Group BG 08 d 2/2 - Distance 2005.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 08 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 155 : Message 136 - Train Position Report (RBC n? 1) - Distance 2040.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 136 d |  |  |
| L_MESSAGE | 10 | 25 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 122 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 1048584 d |  |  |
| D_LRBG | 15 | 40 d |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 0 d |  |  |
| L_DOUBTUNDER | 15 | 0 d |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 20 d |  |  |
| Q_DIRTRAIN | 2 | 01 b |  |  |
| M_MODE | 4 | 1101 b |  |  |
| M_LEVEL | 3 | 1 |  |  |
| NID_NTC | 8 | 3 d |  |  |

| - Step 158 : Message 24 - General Message (RBC n? 1) - Distance 2040.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 24 d |  |  |
| L_MESSAGE | 10 | 24 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048584 d |  |  |
| NID_PACKET | 8 | 42 d | Session management |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 113 d |  |  |
| Q_RBC | 1 | 0 b |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_RBC | 14 | 1515 d |  |  |
| NID_RADIO | 64 | 003265342101 FFFFh |  |  |
| Q_SLEEPSESSION | 1 | 0 b |  |  |

| - Step 160 : Message 156 - Termination of a Communication Session (RBC n? 1) - Distance 2040.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 156 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |

| - Step 162 : Message 39 - Acknowledgement of Termination of Communication Session (RBC n? 1) - Distance 2040.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 39 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048584 d |  |  |

| - Step 165 : Balise Group BG 09 d 1/2 - Distance 2300.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 09 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 41 d | Level transition order |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 71 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_LEVELTR | 15 | 200 d |  |  |
| M_LEVELTR | 3 | 1 |  |  |
| NID_NTC | 8 | 2 d |  |  |
| L_ACKLEVELTR | 15 | 50 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 165 : Balise Group BG 09 d 2/2 - Distance 2305.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 09 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 174 : Balise Group BG 10 d 1/2 - Distance 2500.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 10 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 41 d | Level transition order |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 71 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_LEVELTR | 15 | 0 d |  |  |
| M_LEVELTR | 3 | 1 |  |  |
| NID_NTC | 8 | 2 d |  |  |
| L_ACKLEVELTR | 15 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 174 : Balise Group BG 10 d 2/2 - Distance 2505.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 10 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 184 : Balise Group BG 11 d 1/2 - Distance 2700.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 11 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 41 d | Level transition order |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 63 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_LEVELTR | 15 | 300 d |  |  |
| M_LEVELTR | 3 | 2 |  |  |
| L_ACKLEVELTR | 15 | 50 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 184 : Balise Group BG 11 d 2/2 - Distance 2705.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 11 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 188 : Balise Group BG 12 d 1/2 - Distance 2750.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 12 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 12 d | Level 1 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 73 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| V_MAIN | 7 | 20 d |  |  |
| V_EMA | 7 | 0 d |  |  |
| T_EMA | 10 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_ENDSECTION | 15 | 1250 d |  |  |
| Q_SECTIONTIMER | 1 | 0 b |  |  |
| Q_ENDTIMER | 1 | 0 b |  |  |
| Q_DANGERPOINT | 1 | 0 b |  |  |
| Q_OVERLAP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 21 d | Transmission of the gradient |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 54 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_GRADIENT | 15 | 0 d |  |  |
| Q_GDIR | 1 | 0 b |  |  |
| G_A | 8 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 27 d | International static speed profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 58 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_STATIC | 15 | 0 d |  |  |
| V_STATIC | 7 | 20 d |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 80 d | Mode profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 85 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_MAMODE | 15 | 250 d |  |  |
| M_MAMODE | 2 | 10 b |  |  |
| V_MAMODE | 7 | 20 d |  |  |
| L_MAMODE | 15 | 750 d |  |  |
| L_ACKMAMODE | 15 | 0 d |  |  |
| Q_MAMODE | 1 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 188 : Balise Group BG 12 d 2/2 - Distance 2755.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 12 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 200 : Balise Group BG 13 d 1/2 - Distance 3000.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 13 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 41 d | Level transition order |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 63 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_LEVELTR | 15 | 0 d |  |  |
| M_LEVELTR | 3 | 2 |  |  |
| L_ACKLEVELTR | 15 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 200 : Balise Group BG 13 d 2/2 - Distance 3005.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 13 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 218 : Balise Group BG 14 d 1/2 - Distance 3200.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 14 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 41 d | Level transition order |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 71 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_LEVELTR | 15 | 300 d |  |  |
| M_LEVELTR | 3 | 1 |  |  |
| NID_NTC | 8 | 3 d |  |  |
| L_ACKLEVELTR | 15 | 50 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 218 : Balise Group BG 14 d 2/2 - Distance 3205.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 14 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 228 : Balise Group BG 15 d 1/2 - Distance 3500.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 15 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 41 d | Level transition order |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 71 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_LEVELTR | 15 | 0 d |  |  |
| M_LEVELTR | 3 | 1 |  |  |
| NID_NTC | 8 | 2 d |  |  |
| L_ACKLEVELTR | 15 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 228 : Balise Group BG 15 d 2/2 - Distance 3505.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 15 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 241 : Balise Group BG 16 d 1/2 - Distance 3700.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 16 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 41 d | Level transition order |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 63 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_LEVELTR | 15 | 300 d |  |  |
| M_LEVELTR | 3 | 0 |  |  |
| L_ACKLEVELTR | 15 | 50 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 241 : Balise Group BG 16 d 2/2 - Distance 3705.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 16 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 251 : Balise Group BG 17 d 1/2 - Distance 4000.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 17 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 41 d | Level transition order |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 63 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_LEVELTR | 15 | 0 d |  |  |
| M_LEVELTR | 3 | 000 b |  |  |
| L_ACKLEVELTR | 15 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 251 : Balise Group BG 17 d 2/2 - Distance 4005.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 17 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |