| - ERTMS/ETCS |  |
| --- | --- |
| Subset-076-6-3_9990400_05_v320.mdb State : Ordered list of Test Case Steps with communication information |  |
| REF : | Subset-076-6-3_9990400_05_v320.mdb |
| ISSUE : | 3.2.0 |
| DATE : | 10/04/17 14:49 |

| Company | Technical Approval | Management approval |
| --- | --- | --- |
| European Union Agency for Railways | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 3.2.0 10.04.2017 | All |  | DLR |

# Sequence description

TC6, TC7

# Test cases

Important:	All columns called ‘Test Results’ are only placeholders for results of the test execution. They shall not be used for the description of test cases. 
A test is passed if all cells of ‘Test Result’ are passed.

|  | SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Step | Row |  |  | Dist. (m) |  | Previous |  |  |  |  | Description of Events |  | I/O |  | Interface |  | Comments |  |  | Next |  |  |  |  | Test Result |  |  |
|  |  |  |  |  |  |  | Levels |  | Mode |  |  |  |  |  |  |  |  |  |  |  | Levels |  |  | Mode |  |  |  |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 10: The ETCS on-board equipment is powered .Transition Condition Id [4]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 |  | 1 |  |  | 0.00 |  | N/A |  | NP |  |  | ERTMS/ETCS on-board unit is powered up |  | I |  | INT |  | The on-board equipment changes to SB mode. |  |  | N/A |  |  | SB |  | - |  |
|  | 2 |  | 2 |  |  | 0.00 |  | N/A |  | SB |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=6) is recorded |  | O |  | JRU |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 24: Start of Mission procedure is engaged |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 3 |  | 1 |  |  | 0.00 |  | N/A |  | SB |  |  | Driver opens desk |  | I |  | TIU |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 4 |  | 2 |  |  | 0.00 |  | N/A |  | SB |  |  | The 'CAB STATUS' message is RECORDED with the variable M_CAB_A_STATUS=1 (Active). |  | O |  | JRU |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 5 |  | 3 |  |  | 0.00 |  | N/A |  | SB |  |  | The actual mode SB is DISPLAYED |  | O |  | DMI |  | The SB indication after opening the desk may not be recorded. Because the change of mode was already recorded while desk was closed |  |  | N/A |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 7: The on-board equipment requests the driver to enter a driver ID within the scope of the Start of Mission procedure in SB mode when the status of the driver ID is unknown. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 6 |  | 1 |  |  | 0.00 |  | N/A |  | SB |  |  | The on-board equipment requires the driver to enter a Driver ID There is NO initial Driver ID value displayed |  | O |  | DMI |  | Starting conditions : There is no value initially displayed since the status of the driver ID is unknown. |  |  | N/A |  |  | SB |  | - |  |
|  | 7 |  | 2 |  |  | 0.00 |  | N/A |  | SB |  |  | The on-board equipment prevents the driver from continuing the procedure before the validation of the Driver ID is done. |  | O |  | DMI |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 8 |  | 3 |  |  | 0.00 |  | N/A |  | SB |  |  | The driver enters and validates a correct Driver ID |  | I |  | DMI |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 9 |  | 4 |  |  | 0.00 |  | N/A |  | SB |  |  | The validated Driver ID is recorded on the JRU with the DRIVER_ID variable in a 'GENERAL MESSAGE'. |  | O |  | JRU |  | A 'GENERAL MESSAGE' is recorded every 5s by the on-board equipment. |  |  | N/A |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 10: Within the scope of the Start of Mission procedure in SB mode, the status of the ERTMS/ETCS Level data is INVALID. The driver selects a level (the existing ERTMS/ETCS Level or a new one). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 10 |  | 1 |  |  | 0.00 |  | N/A |  | SB |  |  | The driver selects "Level <L0/LNTC/L1/L2/L3>" |  | I |  | DMI |  | Depending on the implementation, a list of available levels may be presented to the driver on the DMI. The driver is then allowed to select one level among the proposed levels. For NTC levels, the abbreviations of the concerned National Systems shall be presented to the driver. _____ Driver selects Level 1 [59.-1.1] |  |  | L1 |  |  | SB |  | - |  |
|  | 11 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=<34/35/36/37/38>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 12 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=<0/1/2/3/4>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 6: Within the scope of the Start of Mission procedure in SB mode, the On-Board equipment offers the driver to select either SH or NL or ‘Train Data Entry’ before the on-board equipment requests the driver to enter a complete set of Train Data. The driver selects ‘Train Data Entry’. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 13 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver can select either SH or NL or ‘Train Data Entry’ |  | O |  | DMI |  | Starting conditions The 'Non-Leading' button is enabled if the 'non leading' input signal is received. |  |  | L1 |  |  | SB |  | - |  |
|  | 14 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver selects ‘Train Data Entry’ |  | I |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 15 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | A Train data window is is displayed on the DMI. |  | O |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 16 |  | 4 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver’s selection is recorded on the JRU. |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 47: The on-board equipment requests the driver to enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when the status of the Train Data is unknown (S12) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 18 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The on-board equipment requires the driver to enter all Train Data unknown by the on-board equipment and prevents the driver from performing any other actions. |  | O |  | DMI |  | The driver might be requested to enter the 'Train category(ies)', 'Train length', 'Traction / brake parameters', 'Maximum train speed', 'Loading gauge', 'Axle load category', 'Train fitted with airtight system', but the driver shall never be requested to enter the 'Traction system(s) accepted by the engine', 'List of National Systems available on-board' or 'Axle number'. |  |  | L1 |  |  | SB |  | - |  |
|  | 19 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver enters each Train data unknown on-board |  | I |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 20 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver is allowed to validate the train data |  | O |  | DMI |  | A complete set of valid Train Data is available |  |  | L1 |  |  | SB |  | - |  |
|  | 21 |  | 4 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver validates the train data. |  | I |  | DMI |  | The on-board equipment allows the driver to validate as soon as a valid and complete set of Train Data is entered. |  |  | L1 |  |  | SB |  | - |  |
|  | 22 |  | 5 |  |  | 0.00 |  | L1 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded |  | O |  | JRU |  | Triggered by : The driver validation. |  |  | L1 |  |  | SB |  | - |  |
|  | 23 |  | 6 |  |  | 0.00 |  | L1 |  | SB |  |  | TRAIN DATA (NID_MESSAGE_JRU=2) is recorded |  | O |  | JRU |  | Triggered by : Data Entry completed |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 46: After the set of Train Data has been entered/re-validated by the driver (in S12), if the status of the Train running number is 'unknown', the ERTMS/ETCS on-board equipment requests the driver to enter the Train running number (S13) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 25 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver is requested to enter the Train running number. |  | O |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 26 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver enters the "Train Running Number" [MAIN] |  | I |  | DMI |  | Entered Train Running Number: NID_OPERATIONAL_1 |  |  | L1 |  |  | SB |  | - |  |
|  | 27 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | TRAIN RUNNING NUMBER ENTERED BY THE DRIVER (NID_MESSAGE_JRU=49; NID_OPERATIONAL=NID_OPERATIONAL_1) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 1: The on-board equipment requests the driver to re-validate/enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when level is 0/1/NTC (Subset-026-5.4.3.2 S12). The driver is requested to enter/re-validate the Train running number if the status of the Train running number is 'unknown' or 'invalid'. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 28 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | The DMI allows the driver to select the option ‘START’. |  | O |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 16: The driver selects 'Start' within the scope of the Start of Mission procedure in SB mode and in ERTMS Level 1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 29 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver selects 'Start'. |  | I |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 30 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | Driver’s selection of 'Start' is recorded on the JRU |  | O |  | JRU |  | Triggered by: The driver acts on the on-board DMI. |  |  | L1 |  |  | SB |  | - |  |
|  | 31 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | An acknowledgement request for running in Staff Responsible mode is displayed on the DMI |  | O |  | DMI |  | The on-board equipment will then waits for the driver to acknowledge the acknowledgement request for running in Staff Responsible mode |  |  | L1 |  |  | SB |  | - |  |
|  | 32 |  | 4 |  |  | 0.00 |  | L1 |  | SB |  |  | The acknowledgement request for running in Staff Responsible mode is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 33 |  | 5 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver acknowledges 'Staff Responsible' |  | I |  | DMI |  | Symbol MO10 |  |  | L1 |  |  | SB |  | - |  |
|  | 34 |  | 6 |  |  | 0.00 |  | L1 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS= 0000 0011) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 35 |  | 7 |  |  | 0.00 |  | L1 |  | SB |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=2) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 36 |  | 8 |  |  | 0.00 |  | L1 |  | SR |  |  | The Mode symbol 'Staff Responsible' is displayed |  | O |  | DMI |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 37 |  | 9 |  |  | 0.00 |  | L1 |  | SR |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit24=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 38 |  | 10 |  |  | 0.00 |  | L1 |  | SR |  |  | Any message (NID_MESSAGE_JRU=ALL; NID_LRBG=16777215) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 44: The train switches in FS mode, because a MA and Track Description are stored on-board and no specific mode is required by a Mode Profile and No Trip order is given by balise. Transition Condition Id [32]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 39 |  | 1 |  |  | 50.00 |  | L1 |  | SR |  |  | One Balise Group message (N_TOTAL=1) containing packet 12 , packet 21 and packet 27 is received |  | I |  | BTM |  | Packet 80 not included for current location |  |  | L1 |  |  | SR |  | - |  |
|  | 40 |  | 2 |  |  | 50.00 |  | L1 |  | SR |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 41 |  | 3 |  |  | 50.00 |  | L1 |  | SR |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=0) is recorded |  | O |  | JRU |  | The on-board equipment switches immediately to FS mode |  |  | L1 |  |  | FS |  | - |  |
|  | 42 |  | 4 |  |  | 50.00 |  | L1 |  | FS |  |  | The Mode symbol 'Full Supervision' is displayed |  | O |  | DMI |  | DMI Object: Symbol MO11 |  |  | L1 |  |  | FS |  | - |  |
|  | 43 |  | 5 |  |  | 50.00 |  | L1 |  | FS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit26=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | Feature 9990400: A3.4: Handling of Stored Information in specific Situations Test Case 6: The on-board starts a section timer for each section (T_SECTIONTIMER) when the train passes over the first encountered balise of the balise group giving the MA or at the value of the time stamp given in the radio message that includes the MA. When the timer of the next section has expired, the on-board considers the entry point of the revoked section as the EoA. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 44 |  | 1 |  |  | 300.00 |  | L1 |  | FS |  |  | One Balise Group message (N_TOTAL=1) containing packet 12 is received |  | I |  | BTM |  | The train receives a MA including one or more sections. A section timer is started at the time of passage over the first encountered balise of the balise group giving the MA. t=to. |  |  | L1 |  |  | FS |  | - |  |
|  | 45 |  | 2 |  |  | 300.00 |  | L1 |  | FS |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 46 |  | 3 |  |  | 400.00 |  | L1 |  | FS |  |  | Train is moving (V_TRAIN=40) |  | I |  | INT |  | At t=to+T_SECTIONTIMER(k) the time out of section k expires, so the entry point of the revoked section k becomes the EoA: When the time out for section k expires, the train is placed before the entry point of the section k. |  |  | L1 |  |  | FS |  | - |  |
|  | Feature 9990400: A3.4: Handling of Stored Information in specific Situations Test Case 21: Reset of supervision data in the following situations: a)The execution of a conditional emergency stop b)The reception of a shortened MA c)The stored MA is shortened due to a section time out d)The SvL is shifted (to the DP if any or to the EoA) due to an overlap time-outf) A cooperative MA revocation is granted by the on-board n) the Limit of Authority becomes an End of Authority and the on-board considers an SvL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 52 |  | 1 |  |  | 400.00 |  | L1 |  | FS |  |  | The speed and distance monitoring information is displayed |  | O |  | DMI |  | Track information and linking information are DELETED beyond end of new MA (Some data presented on DMI changes status as required). (Is not displayed in LS) _____ Check: The target distance changes until the new EoA. The target speed is set to 0. The release Speed is set to National/Default Value (40km/h) |  |  | L1 |  |  | FS |  | - |  |
|  | 53 |  | 2 |  |  | 400.00 |  | L1 |  | FS |  |  | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded |  | O |  | JRU |  | The new permitted speed, target speed and target distance are recorded |  |  | L1 |  |  | FS |  | - |  |
|  | Feature 9990400: A3.4: Handling of Stored Information in specific Situations Test Case 7: The on-board starts a section timer for the end section (T_SECTIONTIMER) when the train passes over the first encountered balise of the balise group giving the MA or at the value of the time stamp given in the radio message that includes the MA. When the timer of the end section has expired, the on-board considers the entry point of the end section as the EoA. When this happens the train is running before the end section and there is no intervention. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 54 |  | 1 |  |  | 450.00 |  | L1 |  | FS |  |  | One Balise Group message (N_TOTAL=1) containing packet 12 is received |  | I |  | BTM |  | The train receives a MA including one or more sections. A section timer is started at the time of passage over the first encountered balise of the balise group giving the MA. t=to. |  |  | L1 |  |  | FS |  | - |  |
|  | 55 |  | 2 |  |  | 450.00 |  | L1 |  | FS |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 56 |  | 3 |  |  | 450.00 |  | L1 |  | FS |  |  | The speed and distance monitoring information is displayed |  | O |  | DMI |  | The target distance is displayed to the DMI Restricted in LS. Some Information is available on DMI under certain conditions. |  |  | L1 |  |  | FS |  | - |  |
|  | 57 |  | 4 |  |  | 500.00 |  | L1 |  | FS |  |  | Train is moving (V_TRAIN=40) |  | I |  | INT |  | At t=to+T_SECTIONTIMER (end section) the time out for the end section expires. When the time out for the end section expires, the train is placed before the entry point of the end section, at a distance enough to avoid brake intervention |  |  | L1 |  |  | FS |  | - |  |
|  | 58 |  | 5 |  |  | 500.00 |  | L1 |  | FS |  |  | The speed and distance monitoring information is displayed |  | O |  | DMI |  | The target speed is set to 0. Restricted in LS. Some Information is available on DMI under certain conditions. |  |  | L1 |  |  | FS |  | - |  |
|  | 59 |  | 6 |  |  | 500.00 |  | L1 |  | FS |  |  | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded |  | O |  | JRU |  | Target speed change is RECORDED |  |  | L1 |  |  | FS |  | - |  |
|  | 60 |  | 7 |  |  | 500.00 |  | L1 |  | FS |  |  | The speed and distance monitoring information is displayed |  | O |  | DMI |  | The release Speed is set to National/Default Value Restricted in LS. Some Information is available on DMI under certain conditions. |  |  | L1 |  |  | FS |  | - |  |
|  | 61 |  | 8 |  |  | 500.00 |  | L1 |  | FS |  |  | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded |  | O |  | JRU |  | Release speed change is RECORDED |  |  | L1 |  |  | FS |  | - |  |
|  | 62 |  | 9 |  |  | 500.00 |  | L1 |  | FS |  |  | The speed and distance monitoring information is displayed |  | O |  | DMI |  | The train shortens its MA to the entry point of the end section. Restricted in LS. Some Information is available on DMI under certain conditions. |  |  | L1 |  |  | FS |  | - |  |
|  | 63 |  | 10 |  |  | 500.00 |  | L1 |  | FS |  |  | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded |  | O |  | JRU |  | MA shortening is recorded |  |  | L1 |  |  | FS |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 1: The ETCS on-board system is NOT powered. Transition Condition Id [29]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 64 |  | 1 |  |  | 550.00 |  | L1 |  | FS |  |  | The ERTMS/ETCS on-board unit is powered down |  | I |  | INT |  | - |  |  | L1 |  |  | NP |  | - |  |
|  | 65 |  | 2 |  |  | 550.00 |  | L1 |  | NP |  |  | NOT GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=-) is recorded |  | O |  | JRU |  | There are no messages recorded in the JRU during a period of time |  |  | L1 |  |  | NP |  | - |  |
| Step 10: DMI Events Distance 0.00 m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Event Id. |  |  |  | Event Name |  |  |  |  |  |  | Data |  |  |  |  |  |  |  |  | Delay |  |  |  |  |  |  |  |  |
| 29 |  |  |  | Request Change Level |  |  |  |  |  |  | LEVEL=1 |  |  |  |  |  |  |  |  | 0 s |  |  |  |  |  |  |  |  |

| - Step 39 : Balise Group BG 1 d 1/2 - Distance 50.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 1 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 12 d | Level 1 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 73 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| V_MAIN | 7 | 8 d |  |  |
| V_EMA | 7 | 0 d |  |  |
| T_EMA | 10 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_ENDSECTION | 15 | 1000 d |  |  |
| Q_SECTIONTIMER | 1 | 0 b |  |  |
| Q_ENDTIMER | 1 | 0 b |  |  |
| Q_DANGERPOINT | 1 | 0 b |  |  |
| Q_OVERLAP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 21 d | Transmission of the gradient |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 78 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_GRADIENT | 15 | 0 d |  |  |
| Q_GDIR | 1 | 0 b |  |  |
| G_A | 8 | 0 d |  |  |
| N_ITER | 5 | 1 d |  |  |
| D_GRADIENT | 15 | 2000 d |  |  |
| Q_GDIR | 1 | 0 b |  |  |
| G_A | 8 | 11111111 b |  |  |
| NID_PACKET | 8 | 27 d | International static speed profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 86 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_STATIC | 15 | 0 d |  |  |
| V_STATIC | 7 | 16 d |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| N_ITER | 5 | 1 d |  |  |
| D_STATIC | 15 | 2000 d |  |  |
| V_STATIC | 7 | 1111111 b |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 39 : Balise Group BG 1 d 2/2 - Distance 55.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 1 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 44 : Balise Group BG 2 d 1/2 - Distance 300.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 2 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 12 d | Level 1 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 152 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| V_MAIN | 7 | 8 d |  |  |
| V_EMA | 7 | 8 d |  |  |
| T_EMA | 10 | 1111111111 b |  |  |
| N_ITER | 5 | 2 d |  |  |
| L_SECTION | 15 | 400 d |  |  |
| Q_SECTIONTIMER | 1 | 0 |  |  |
| L_SECTION | 15 | 400 d |  |  |
| Q_SECTIONTIMER | 1 | 1 |  |  |
| T_SECTIONTIMER | 10 | 10 d |  |  |
| D_SECTIONTIMERSTOPLOC | 15 | 200 d |  |  |
| L_ENDSECTION | 15 | 400 d |  |  |
| Q_SECTIONTIMER | 1 | 0 |  |  |
| Q_ENDTIMER | 1 | 0 b |  |  |
| Q_DANGERPOINT | 1 | 1 |  |  |
| D_DP | 15 | 0 d |  |  |
| V_RELEASEDP | 7 | 7 d |  |  |
| Q_OVERLAP | 1 | 0 |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 44 : Balise Group BG 2 d 2/2 - Distance 305.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 2 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 54 : Balise Group BG 3 d 1/2 - Distance 450.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 3 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 12 d | Level 1 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 136 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| V_MAIN | 7 | 8 d |  |  |
| V_EMA | 7 | 5 d |  |  |
| T_EMA | 10 | 1111111111 b |  |  |
| N_ITER | 5 | 1 d |  |  |
| L_SECTION | 15 | 150 d |  |  |
| Q_SECTIONTIMER | 1 | 0 b |  |  |
| L_ENDSECTION | 15 | 100 d |  |  |
| Q_SECTIONTIMER | 1 | 1 |  |  |
| T_SECTIONTIMER | 10 | 5 d |  |  |
| D_SECTIONTIMERSTOPLOC | 15 | 0 d |  |  |
| Q_ENDTIMER | 1 | 0 |  |  |
| Q_DANGERPOINT | 1 | 1 |  |  |
| D_DP | 15 | 0 d |  |  |
| V_RELEASEDP | 7 | 7 d |  |  |
| Q_OVERLAP | 1 | 0 |  |  |
| NID_PACKET | 8 | 21 d | Transmission of the gradient |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 78 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_GRADIENT | 15 | 0 d |  |  |
| Q_GDIR | 1 | 0 b |  |  |
| G_A | 8 | 0 d |  |  |
| N_ITER | 5 | 1 d |  |  |
| D_GRADIENT | 15 | 1000 d |  |  |
| Q_GDIR | 1 | 0 b |  |  |
| G_A | 8 | 11111111 b |  |  |
| NID_PACKET | 8 | 27 d | International static speed profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 86 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_STATIC | 15 | 0 d |  |  |
| V_STATIC | 7 | 8 d |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| N_ITER | 5 | 1 d |  |  |
| D_STATIC | 15 | 1000 d |  |  |
| V_STATIC | 7 | 1111111 b |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 54 : Balise Group BG 3 d 2/2 - Distance 455.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 3 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |