| - ERTMS/ETCS |  |
| --- | --- |
| Subset-076-6-3_5070300_01_v320.mdb State : Ordered list of Test Case Steps with communication information |  |
| REF : | Subset-076-6-3_5070300_01_v320.mdb |
| ISSUE : | 3.2.0 |
| DATE : | 10/04/17 12:06 |

| Company | Technical Approval | Management approval |
| --- | --- | --- |
| European Union Agency for Railways | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 3.2.0 10.04.2017 | All |  | MLT |

# Sequence description

# Test cases

Important:	All columns called ‘Test Results’ are only placeholders for results of the test execution. They shall not be used for the description of test cases. 
A test is passed if all cells of ‘Test Result’ are passed.

|  | SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Step | Row |  |  | Dist. (m) |  | Previous |  |  |  |  | Description of Events |  | I/O |  | Interface |  | Comments |  |  | Next |  |  |  |  | Test Result |  |  |
|  |  |  |  |  |  |  | Levels |  | Mode |  |  |  |  |  |  |  |  |  |  |  | Levels |  |  | Mode |  |  |  |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 10: The ETCS on-board equipment is powered .Transition Condition Id [4]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 |  | 1 |  |  | 0.00 |  | N/A |  | NP |  |  | ERTMS/ETCS on-board unit is powered up |  | I |  | INT |  | The on-board equipment changes to SB mode. |  |  | N/A |  |  | NP |  | - |  |
|  | 2 |  | 2 |  |  | 0.00 |  | N/A |  | NP |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=6) is recorded |  | O |  | JRU |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 24: Start of Mission procedure is engaged |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 3 |  | 1 |  |  | 0.00 |  | N/A |  | SB |  |  | Driver opens desk |  | I |  | TIU |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 4 |  | 2 |  |  | 0.00 |  | N/A |  | SB |  |  | The 'CAB STATUS' message is RECORDED with the variable M_CAB_A_STATUS=1 (Active). |  | O |  | JRU |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 5 |  | 3 |  |  | 0.00 |  | N/A |  | SB |  |  | The actual mode SB is DISPLAYED |  | O |  | DMI |  | The SB indication after opening the desk may not be recorded. Because the change of mode was already recorded while desk was closed |  |  | N/A |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 7: The on-board equipment requests the driver to enter a driver ID within the scope of the Start of Mission procedure in SB mode when the status of the driver ID is unknown. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 6 |  | 1 |  |  | 0.00 |  | N/A |  | SB |  |  | The on-board equipment requires the driver to enter a Driver ID There is NO initial Driver ID value displayed |  | O |  | DMI |  | Starting conditions : There is no value initially displayed since the status of the driver ID is unknown. |  |  | N/A |  |  | SB |  | - |  |
|  | 7 |  | 2 |  |  | 0.00 |  | N/A |  | SB |  |  | The on-board equipment prevents the driver from continuing the procedure before the validation of the Driver ID is done. |  | O |  | DMI |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 8 |  | 3 |  |  | 0.00 |  | N/A |  | SB |  |  | The driver enters and validates a correct Driver ID |  | I |  | DMI |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 9 |  | 4 |  |  | 0.00 |  | N/A |  | SB |  |  | The validated Driver ID is recorded on the JRU with the DRIVER_ID variable in a 'GENERAL MESSAGE'. |  | O |  | JRU |  | A 'GENERAL MESSAGE' is recorded every 5s by the on-board equipment. |  |  | N/A |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 10: Within the scope of the Start of Mission procedure in SB mode, the status of the ERTMS/ETCS Level data is INVALID. The driver selects a level (the existing ERTMS/ETCS Level or a new one). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 10 |  | 1 |  |  | 0.00 |  | N/A |  | SB |  |  | The driver selects "Level <L0/LNTC/L1/L2/L3>" |  | I |  | DMI |  | Depending on the implementation, a list of available levels may be presented to the driver on the DMI. The driver is then allowed to select one level among the proposed levels. For NTC levels, the abbreviations of the concerned National Systems shall be presented to the driver. _____ The Driver selects L1 |  |  | L1 |  |  | SB |  | - |  |
|  | 11 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=<34/35/36/37/38>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 12 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=<0/1/2/3/4>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 6: Within the scope of the Start of Mission procedure in SB mode, the On-Board equipment offers the driver to select either SH or NL or ‘Train Data Entry’ before the on-board equipment requests the driver to enter a complete set of Train Data. The driver selects ‘Train Data Entry’. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 13 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver can select either SH or NL or ‘Train Data Entry’ |  | O |  | DMI |  | Starting conditions The 'Non-Leading' button is enabled if the 'non leading' input signal is received. |  |  | L1 |  |  | SB |  | - |  |
|  | 14 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver selects ‘Train Data Entry’ |  | I |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 15 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | A Train data window is is displayed on the DMI. |  | O |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 16 |  | 4 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver’s selection is recorded on the JRU. |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 47: The on-board equipment requests the driver to enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when the status of the Train Data is unknown (S12) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 18 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The on-board equipment requires the driver to enter all Train Data unknown by the on-board equipment and prevents the driver from performing any other actions. |  | O |  | DMI |  | The driver might be requested to enter the 'Train category(ies)', 'Train length', 'Traction / brake parameters', 'Maximum train speed', 'Loading gauge', 'Axle load category', 'Train fitted with airtight system', but the driver shall never be requested to enter the 'Traction system(s) accepted by the engine', 'List of National Systems available on-board' or 'Axle number'. |  |  | L1 |  |  | SB |  | - |  |
|  | 19 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver enters each Train data unknown on-board |  | I |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 20 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver is allowed to validate the train data |  | O |  | DMI |  | A complete set of valid Train Data is available |  |  | L1 |  |  | SB |  | - |  |
|  | 21 |  | 4 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver validates the train data. |  | I |  | DMI |  | The on-board equipment allows the driver to validate as soon as a valid and complete set of Train Data is entered. |  |  | L1 |  |  | SB |  | - |  |
|  | 22 |  | 5 |  |  | 0.00 |  | L1 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded |  | O |  | JRU |  | Triggered by : The driver validation. |  |  | L1 |  |  | SB |  | - |  |
|  | 23 |  | 6 |  |  | 0.00 |  | L1 |  | SB |  |  | TRAIN DATA (NID_MESSAGE_JRU=2) is recorded |  | O |  | JRU |  | Triggered by : Data Entry completed |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 46: After the set of Train Data has been entered/re-validated by the driver (in S12), if the status of the Train running number is 'unknown', the ERTMS/ETCS on-board equipment requests the driver to enter the Train running number (S13) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 25 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver is requested to enter the Train running number. |  | O |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 26 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver enters the "Train Running Number" [MAIN] |  | I |  | DMI |  | Entered Train Running Number: NID_OPERATIONAL_1 |  |  | L1 |  |  | SB |  | - |  |
|  | 27 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | TRAIN RUNNING NUMBER ENTERED BY THE DRIVER (NID_MESSAGE_JRU=49; NID_OPERATIONAL=NID_OPERATIONAL_1) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 1: The on-board equipment requests the driver to re-validate/enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when level is 0/1/NTC (Subset-026-5.4.3.2 S12). The driver is requested to enter/re-validate the Train running number if the status of the Train running number is 'unknown' or 'invalid'. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 28 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | The DMI allows the driver to select the option ‘START’. |  | O |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 16: The driver selects 'Start' within the scope of the Start of Mission procedure in SB mode and in ERTMS Level 1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 29 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver selects 'Start'. |  | I |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 30 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | Driver’s selection of 'Start' is recorded on the JRU |  | O |  | JRU |  | Triggered by: The driver acts on the on-board DMI. |  |  | L1 |  |  | SB |  | - |  |
|  | 31 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | An acknowledgement request for running in Staff Responsible mode is displayed on the DMI |  | O |  | DMI |  | The on-board equipment will then waits for the driver to acknowledge the acknowledgement request for running in Staff Responsible mode |  |  | L1 |  |  | SB |  | - |  |
|  | 32 |  | 4 |  |  | 0.00 |  | L1 |  | SB |  |  | The acknowledgement request for running in Staff Responsible mode is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 33 |  | 5 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver acknowledges 'Staff Responsible' |  | I |  | DMI |  | Symbol MO10 |  |  | L1 |  |  | SB |  | - |  |
|  | 34 |  | 6 |  |  | 0.00 |  | L1 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS= 0000 0011) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 35 |  | 7 |  |  | 0.00 |  | L1 |  | SB |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=2) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 36 |  | 8 |  |  | 0.00 |  | L1 |  | SR |  |  | The Mode symbol 'Staff Responsible' is displayed |  | O |  | DMI |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 37 |  | 9 |  |  | 0.00 |  | L1 |  | SR |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit24=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 38 |  | 10 |  |  | 0.00 |  | L1 |  | SR |  |  | Any message (NID_MESSAGE_JRU=ALL; NID_LRBG=16777215) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | Feature 4080425: 4.8.4: Accepted Information depending on the modes - Movement Authority Test Case 2: New Level 1 MA is ACCEPTED by the on-board equipment because the SSP (Static Speed Profile) and GP (Gradient Profile) sent together with the MA on-board covers the whole length of the MA. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 39 |  | 1 |  |  | 50.00 |  | L1 |  | SR |  |  | One Balise Group message (N_TOTAL=1) containing packet 12 (MA), packet 21 (Gradient) and packet 27 (SSP) is received |  | I |  | BTM |  | Track description sending is optional New track description (or stored one if not sent) covers the movement authority distance |  |  | L1 |  |  | SR |  | - |  |
|  | 40 |  | 2 |  |  | 50.00 |  | L1 |  | SR |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 41 |  | 3 |  |  | 50.00 |  | L1 |  | SR |  |  | The speed and distance monitoring information is displayed |  | O |  | DMI |  | The Target Distance will match the length of the last received MA. |  |  | L1 |  |  | FS |  | - |  |
|  | 42 |  | 4 |  |  | 50.00 |  | L1 |  | FS |  |  | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; D_TARGET=<equal to the lenth of the last MA>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | Feature 4080424: 4.8.4: Accepted Information depending on the modes - Mode Profile Test Case 5: The train receives a message including packet 80 (Mode Profile) that is transmitted via balise group. The message is ACCEPTED. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 43 |  | 1 |  |  | 50.00 |  | L1 |  | FS |  |  | One Balise Group message containing packet 12, packet 21, packet 27, packet 80 is received |  | I |  | BTM |  | Complete message including Mode Profile is accepted |  |  | L1 |  |  | FS |  | - |  |
|  | 44 |  | 2 |  |  | 50.00 |  | L1 |  | FS |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | Complete message including Mode Profile is recorded |  |  | L1 |  |  | FS |  | - |  |
|  | Feature 5070300: 5.7.3: Shunting is requested for a further location Test Case 1: The beginning of the SH area is considered as the EOA (keeping the SvL derived from the MA),with no release speed. The driver does not acknowledge the Shunting area and the system applies the brakes. After the switch to SH, acknowledgement for shunting is NOT acknowledged within the driver acknowledgement time. The system applies the service brake. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 46 |  | 1 |  |  | 50.00 |  | L1 |  | FS |  |  | A target distance to EOA is DISPLAYED |  | O |  | DMI |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 47 |  | 2 |  |  | 50.00 |  | L1 |  | FS |  |  | The train speed is lower than the Shunting mode speed limit (National Value, or value given in the mode profile) |  | O |  | DMI |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 48 |  | 3 |  |  | 150.00 |  | L1 |  | FS |  |  | The Mode symbol "Acknowledge Shunting" is displayed |  | O |  | DMI |  | The distance between the estimated front end of the train and the beginning of shunting area becomes shorter than L_ACKMAMODE |  |  | L1 |  |  | FS |  | - |  |
|  | Feature 5070300: 5.7.3: Shunting is requested for a further location Test Case 3: Once the request for acknowledgement is displayed, it shall not be taken back, even if the related conditions are no more fulfilled e.g. the train accelerates |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 49 |  | 1 |  |  | 200.00 |  | L1 |  | FS |  |  | The train speed is lower than the Shunting mode speed limit (National Value, or value given in the mode profile) |  | O |  | DMI |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 50 |  | 2 |  |  | 200.00 |  | L1 |  | FS |  |  | Shunting acknowledgement is DISPLAYED |  | O |  | DMI |  | The distance between the estimated front end of the train and the beginning of shunting area becomes shorter than L_ACKMAMODE |  |  | L1 |  |  | FS |  | - |  |
|  | 51 |  | 3 |  |  | 300.00 |  | L1 |  | FS |  |  | The Train speed increases so that it is greater than the Shunting mode speed limit (National Value, or value given in the mode profile) |  | O |  | DMI |  | The driver does not acknowledge the Shunting The Shunting acknowledgement remains displayed |  |  | L1 |  |  | FS |  | - |  |
|  | 52 |  | 4 |  |  | 300.00 |  | L1 |  | FS |  |  | The acknowledgement for SH is not taken back. It is still displayed |  | O |  | DMI |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 53 |  | 5 |  |  | 400.00 |  | L1 |  | FS |  |  | The train speed decreases so that it is lower than the Shunting mode speed limit (National Value, or value given in the mode profile) |  | O |  | DMI |  | The driver does not acknowledge the Shunting The Shunting acknowledgement remains displayed |  |  | L1 |  |  | FS |  | - |  |
|  | Feature 5070300: 5.7.3: Shunting is requested for a further location Test Case 1: The beginning of the SH area is considered as the EOA (keeping the SvL derived from the MA),with no release speed. The driver does not acknowledge the Shunting area and the system applies the brakes. After the switch to SH, acknowledgement for shunting is NOT acknowledged within the driver acknowledgement time. The system applies the service brake. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 54 |  | 4 |  |  | 500.00 |  | L1 |  | FS |  |  | Service brake commanded |  | O |  | TIU |  | The Service Brake is applied because of the EOA Until the switch to SH, the beginning of the Shunting area is considered as an EOA keeping the SvL given by the MA, with no release speed. The train is allowed to enter in the SH area by a distance corresponding to the length of the confidence interval |  |  | L1 |  |  | FS |  | - |  |
|  | 55 |  | 5 |  |  | 500.00 |  | L1 |  | FS |  |  | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded |  | O |  | JRU |  | Application of Service Brake is RECORDED |  |  | L1 |  |  | FS |  | - |  |
|  | 56 |  | 6 |  |  | 650.00 |  | L1 |  | FS |  |  | The on-board equipment reaches the SH area with its max. safe front end. The system switches to SH mode |  | O |  | DMI |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 57 |  | 7 |  |  | 650.00 |  | L1 |  | SH |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=3) is recorded |  | O |  | JRU |  | The transition to SH mode is RECORDED |  |  | L1 |  |  | SH |  | - |  |
|  | 58 |  | 8 |  |  | 650.00 |  | L1 |  | SH |  |  | Service brake not commanded |  | O |  | TIU |  | The Service Brake is released |  |  | L1 |  |  | SH |  | - |  |
|  | 59 |  | 9 |  |  | 650.00 |  | L1 |  | SH |  |  | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=0) is recorded |  | O |  | JRU |  | Release of Service Brake is RECORDED. |  |  | L1 |  |  | SH |  | - |  |
|  | 60 |  | 10 |  |  | 650.00 |  | L1 |  | SH |  |  | The Mode symbol "Acknowledge Shunting" is displayed |  | O |  | DMI |  | The request for acknowledgement for shunting mode is still displayed |  |  | L1 |  |  | SH |  | - |  |
|  | 62 |  | 12 |  |  | 700.00 |  | L1 |  | SH |  |  | NOT The driver acknowledges "Shunting" |  | O |  | DMI |  | Driver acknowledgement time The driver has NOT acknowledged within TACK after the change to SH |  |  | L1 |  |  | SH |  | - |  |
|  | 63 |  | 13 |  |  | 700.00 |  | L1 |  | SH |  |  | Service brake commanded |  | O |  | TIU |  | The Service Brake is applied |  |  | L1 |  |  | SH |  | - |  |
|  | 64 |  | 14 |  |  | 700.00 |  | L1 |  | SH |  |  | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded |  | O |  | JRU |  | The Service Brake is applied is RECORDED |  |  | L1 |  |  | SH |  | - |  |
|  | 65 |  | 15 |  |  | 700.00 |  | L1 |  | SH |  |  | The driver acknowledges "Shunting" |  | I |  | DMI |  | The driver acknowledges "Shunting" |  |  | L1 |  |  | SH |  | - |  |
|  | 66 |  | 16 |  |  | 700.00 |  | L1 |  | SH |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded |  | O |  | JRU |  | The driver action is RECORDED |  |  | L1 |  |  | SH |  | - |  |
|  | 67 |  | 17 |  |  | 700.00 |  | L1 |  | SH |  |  | Service brake not commanded |  | O |  | TIU |  | The Service Brake is released |  |  | L1 |  |  | SH |  | - |  |
|  | 68 |  | 18 |  |  | 700.00 |  | L1 |  | SH |  |  | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=0) is recorded |  | O |  | JRU |  | Release of Service Brake is RECORDED. |  |  | L1 |  |  | SH |  | - |  |
|  | Feature 5100100: 5.10.1: Level Transitions General requirements Test Case 2: The train is running in SH mode and receives multiples orders to switch immediately or conditionnaly from LX at the current position. Only the last set of Level Transition Information is stored AND the system switch only when another mode than SH or PS has been entered. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 69 |  | 1 |  |  | 700.00 |  | L1 |  | SH |  |  | Level X is DISPLAYED. |  | O |  | DMI |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 70 |  | 2 |  |  | 900.00 |  | L1 |  | SH |  |  | The train receives an order to switch immediately to Level W from balise group BGa. |  | I |  | BTM |  | Packet 41: 'Level Transition Order' is RECEIVED. D_LEVELTR= 0 or = 32767 M_LEVELTR= LVL_W Or Packet 46: 'Conditional Level Transition Order' is RECEIVED. M_LEVELTR= LVL_W |  |  | L1 |  |  | SH |  | - |  |
|  | 71 |  | 3 |  |  | 900.00 |  | L1 |  | SH |  |  | Balise Group message, including Packet 41 or 46, is RECORDED. |  | O |  | JRU |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 72 |  | 4 |  |  | 1050.00 |  | L1 |  | SH |  |  | The train receives an order to switch immediately to Level Y from balise group BGa. |  | I |  | BTM |  | Packet 41: 'Level Transition Order' is RECEIVED. D_LEVELTR= 0 or = 32767 M_LEVELTR= LVL_Y Or Packet 46: 'Conditional Level Transition Order' is RECEIVED. M_LEVELTR= LVL_Y |  |  | L1 |  |  | SH |  | - |  |
|  | 73 |  | 5 |  |  | 1050.00 |  | L1 |  | SH |  |  | Balise Group message, including Packet 41 or 46, is RECORDED. |  | O |  | JRU |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 74 |  | 6 |  |  | 1150.00 |  | L1 |  | SH |  |  | Level X is (still) DISPLAYED. |  | O |  | DMI |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 75 |  | 7 |  |  | 1250.00 |  | L1 |  | SH |  |  | The train reaches standstill. |  | O |  | DMI |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 76 |  | 8 |  |  | 1250.00 |  | L1 |  | SH |  |  | The driver selects 'exit shunting' |  | I |  | DMI |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 77 |  | 9 |  |  | 1250.00 |  | L1 |  | SH |  |  | The driver’s selection of 'exit shunting' is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 78 |  | 10 |  |  | 1250.00 |  | L1 |  | SH |  |  | SB is DISPLAYED |  | O |  | DMI |  | The train switches to SB |  |  | L1 |  |  | SB |  | - |  |
|  | 79 |  | 11 |  |  | 1250.00 |  | L1 |  | SB |  |  | SB is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 80 |  | 12 |  |  | 1250.00 |  | L1 |  | SB |  |  | Level Y is DISPLAYED. |  | O |  | DMI |  | The On-board equipment switches to Level Y. |  |  | L2 |  |  | SB |  | - |  |
|  | 81 |  | 13 |  |  | 1250.00 |  | L2 |  | SB |  |  | Level Y is RECORDED |  | O |  | JRU |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 1: The ETCS on-board system is NOT powered. Transition Condition Id [29]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 82 |  | 1 |  |  | 1250.00 |  | L2 |  | SB |  |  | The ERTMS/ETCS on-board unit is powered down |  | I |  | INT |  | - _____ The train is at standstill |  |  | L2 |  |  | NP |  | - |  |
|  | 83 |  | 2 |  |  | 1250.00 |  | L2 |  | NP |  |  | NOT GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=-) is recorded |  | O |  | JRU |  | There are no messages recorded in the JRU during a period of time |  |  | L2 |  |  | NP |  | - |  |
| Step 10: DMI Events Distance 0.00 m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Event Id. |  |  |  | Event Name |  |  |  |  |  |  | Data |  |  |  |  |  |  |  |  | Delay |  |  |  |  |  |  |  |  |
| 29 |  |  |  | Request Change Level |  |  |  |  |  |  | LEVEL=1 |  |  |  |  |  |  |  |  | 10 s |  |  |  |  |  |  |  |  |

| - Step 39 : Balise Group BG 00000000000039 1/2 - Distance 50.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 000 b |  |  |
| N_TOTAL | 3 | 001 b |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 0001000000 b |  |  |
| NID_BG | 14 | 00000000000001 b |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 00001100 b | Level 1 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 0000001001001 b |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| V_MAIN | 7 | 0010100 b |  |  |
| V_EMA | 7 | 0000000 b |  |  |
| T_EMA | 10 | 0000000000 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_ENDSECTION | 15 | 000001111101000 b |  |  |
| Q_SECTIONTIMER | 1 | 0 b |  |  |
| Q_ENDTIMER | 1 | 0 b |  |  |
| Q_DANGERPOINT | 1 | 0 b |  |  |
| Q_OVERLAP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 00010101 b | Transmission of the gradient |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 0000000110110 b |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_GRADIENT | 15 | 000000000000000 b |  |  |
| Q_GDIR | 1 | 0 b |  |  |
| G_A | 8 | 00000000 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 00011011 b | International static speed profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 0000000111010 b |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_STATIC | 15 | 000000000000000 b |  |  |
| V_STATIC | 7 | 0010100 b |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 11111111 b | End of information |  |

| - Step 39 : Balise Group BG 00000000000039 2/2 - Distance 55.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 001 b |  |  |
| N_TOTAL | 3 | 001 b |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 0001000000 b |  |  |
| NID_BG | 14 | 00000000000001 b |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 01010000 b | Mode profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 0000001010101 b |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_MAMODE | 15 | 000001001011000 b |  |  |
| M_MAMODE | 2 | 01 b |  |  |
| V_MAMODE | 7 | 0001000 b |  |  |
| L_MAMODE | 15 | 000000010010110 b |  |  |
| L_ACKMAMODE | 15 | 000000111110100 b |  |  |
| Q_MAMODE | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 11111111 b | End of information |  |

| - Step 70 : Balise Group BG 00000001100170 1/2 - Distance 900.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 000 b |  |  |
| N_TOTAL | 3 | 001 b |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 0001000000 b |  |  |
| NID_BG | 14 | 00000001100101 b |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 00101001 b | Level transition order |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 0000000111111 b |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_LEVELTR | 15 | 000000000000000 b |  |  |
| M_LEVELTR | 3 | 000 b |  |  |
| L_ACKLEVELTR | 15 | 000000000000000 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 11111111 b | End of information |  |

| - Step 70 : Balise Group BG 00000001100170 2/2 - Distance 905.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 001 b |  |  |
| N_TOTAL | 3 | 001 b |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 0001000000 b |  |  |
| NID_BG | 14 | 00000001100101 b |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 11111111 b | End of information |  |

| - Step 72 : Balise Group BG 00000001100172 1/2 - Distance 1050.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 000 b |  |  |
| N_TOTAL | 3 | 001 b |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 0001000000 b |  |  |
| NID_BG | 14 | 00000001100110 b |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 00101001 b | Level transition order |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 0000000111111 b |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_LEVELTR | 15 | 111111111111111 b |  |  |
| M_LEVELTR | 3 | 011 b |  |  |
| L_ACKLEVELTR | 15 | 000000000000000 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 11111111 b | End of information |  |

| - Step 72 : Balise Group BG 00000001100172 2/2 - Distance 1055.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 001 b |  |  |
| N_TOTAL | 3 | 001 b |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 0001000000 b |  |  |
| NID_BG | 14 | 00000001100110 b |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 11111111 b | End of information |  |