| - ERTMS/ETCS – Class 1 |  |
| --- | --- |
| System Requirements Specification Chapter 8 Messages |  |
| REF : | SUBSET-026-8 |
| ISSUE : | 2.3.0 |
| DATE : | 24/02/06 |

| Company | Technical Approval | Management approval |
| --- | --- | --- |
| ALCATEL |  |  |
| ALSTOM |  |  |
| ANSALDO SIGNAL |  |  |
| BOMBARDIER |  |  |
| INVENSYS RAIL |  |  |
| SIEMENS |  |  |

## Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| 0.0.1 990715 | All | Creation of the document “Clean version” based on SRS Class P Ch. 8 | V. Roger |
| 0.0.2 990716 | All | Update to SRS Class 1 | V. Roger |
| 0.1.0 990727 | All | Update considering review comments from ADT and ALS | V. Roger |
| 1.0.0 990729 | Version number and editorial changes. | Finalisation meeting, Stuttgart 990729. | HE |
| 1.2.0 990730 | Version number | Release version | HE |
| 1.2.1 991124 | Sections 8.4 to 8.7 | ECSAG comments (on chapter 8) taken into account. | V. Roger |
| 1.2.2 991209 | Sections 8.4 to 8.7. Creation of appendix. | Upsate to SRS Class 1 version 2 – First release | V. Roger |
| 1.3.0 991217 | Sections 8.4 to 8.7. Suppression of appendix. | Upsate to SRS Class 1 version 2 – Unisig Review (991216) | V. Roger |
| 1.3.1 991220 | Section 8.7.14 | Add Q_SCALE | P. Rimbaud |
| 2.0.0 991222 | Minor editorial changes | Release version | Ch. Frerichs (ed.) |
| 2.0.1 001002 | All | Corrections after UNISIG review 15 June 00 | P. Rimbaud |
| 2.1.0 001026 | Sections 8.4.1, 8.4.2, 8.4.3, 8.4.4, 8.5.1, 8.5.2, 8.5.3, 8.6, 8.7 | Corrections after UNISIG review 10/11 October 00 | P. Rimbaud |
| 2.2.0 | Packet 71 deleted, NID_C 10 bit | UNISIG release | SAB |
| 2.2.2 | Packet 71 added, messages 42 and 158 deleted | SUBSET-026 Corrected Paragraphs, Issue 2.2.2 | JY. Riou |
| 2.2.4 – 040512 |  | UNISIG Change Request according the scope defined in the 06/05/2004 e-mail | JY. RIOU |
| 2.2.4 SG checked 28/05/04 | Including all CLRs agreed with EEIG (see “List of CLRs agreed with EEIG for SRS v2.2.4” dated 28/05/04) Affected clauses see change marks |  | H. Kast |
| 2.2.5 21/01/05 | Incorporation of solution proposal for CLR 007 with EEIG users group comments |  | A. Hougardy |
| 2.2.6 04/02/05 | § 8.4.2.1, § 8.4.3.1, § 8.7.12 according to CR242 § 8.4.4.4.1.1, § 8.5.3, § 8.7.22 according to CR458 § 8.6.3 according to CR 487 |  | JY. RIOU |
| 2.2.7 01/08/05 | § 8.7.22 error correction (Q_ORIENTATION) according to CR458, § 8.4.4.4.2 according to CR 126 §8.4.4.4.3 b), § 8.6.17 according to CR 299 § 8.4.1.4.5, § 8.4.4.4.1 according to CR 413 § 8.4.1.4.6 according to CR 633 |  | JY.RIOU |
| 2.2.8 30/11/05 | Change marks cleaned up and updated according to last CR decisions (including split of CRs 7 and 126) |  | JY. RIOU |
| 2.2.9 24/02/06 | Including all CR s that are classified as “IN” per Subset-108 version 1.0.0 Removal of all CRs that are not classified as “IN” as per SUBSET-108 version 1.0.0 with the exception of the CR 63, 98, 120, 158 and 538 |  | JY. RIOU |
| 2.3.0 24/02/06 | Release version |  | HK |

## Table of Contents

8.1	Modification History	2

8.2	Table of Contents	4

8.3	Introduction	6

8.3.1	Scope and Purpose	6

8.3.2	Definitions	6

8.4	Rules	8

8.4.1	Common Rules	8

8.4.2	Rules for Eurobalise telegrams	10

8.4.3	Rules for Euroloop messages	11

8.4.4	Rules for Euroradio messages	12

8.5	List of radio Messages	16

8.5.1	Introduction	16

8.5.2	Train to Track radio messages	16

8.5.3	Track to Train radio messages	17

8.6	Definition of Radio Messages from Train to Track	18

8.6.1	Message 129: Validated Train Data	18

8.6.2	Message 130: Request for Shunting	18

8.6.3	Message 132: MA Request	18

8.6.4	Message 136: Train Position Report	19

8.6.5	Message 137: Request to Shorten MA is granted	19

8.6.6	Message 138: Request to Shorten MA is rejected	19

8.6.7	Message 146: Acknowledgement	20

8.6.8	Message 147: Acknowledgement of Emergency Stop	20

8.6.9	Message 149: Track Ahead Free Granted	20

8.6.10	Message 150: End of Mission	21

8.6.11	Message 153: Radio in-fill request	21

8.6.12	Message 154: No compatible version	22

8.6.13	Message 155: Initiation of a communication session	22

8.6.14	Message 156: Termination of a communication session	22

8.6.15	Message 157: SoM Position Report	22

8.6.16	Intentionally deleted	23

8.6.17	Message 159: Session established	23

8.7	Definition of Radio Messages from Track to Train	24

8.7.1	Message 2: SR Authorisation	24

8.7.2	Message 3: Movement Authority	24

8.7.3	Message 6: Recognition of exit from TRIP mode	25

8.7.4	Message 8: Acknowledgement of Train Data	25

8.7.5	Message 9: Request to Shorten MA	26

8.7.6	Message 15: Conditional Emergency Stop	26

8.7.7	Message 16: Unconditional Emergency Stop	27

8.7.8	Message 18: Revocation of Emergency Stop	27

8.7.9	Message 24: General message	27

8.7.10	Message 27: SH Refused	28

8.7.11	Message 28: SH Authorised	28

8.7.12	Message 32: Configuration Determination	29

8.7.13	Message 33: MA with Shifted Location Reference	29

8.7.14	Message 34: Track Ahead Free Request	30

8.7.15	Message 37: In-fill MA	30

8.7.16	Message 38: Initiation of a communication session	31

8.7.17	Message 39: Acknowledgement of termination of a communication session	31

8.7.18	Message 40: Train Rejected	31

8.7.19	Message 41: Train Accepted	32

8.7.20	Intentionally deleted	32

8.7.21	Message 43: SoM position report confirmed by RBC	32

8.7.22	Message 45: Assignment of coordinate system	32

## Introduction

### Scope and Purpose

#### This chapter defines the format and content of messages necessary for ERTMS/ETCS Class 1 functions.

#### Concerning the transmission media, this chapter does not cover considerations such as medium-specific use constraints (ex. distance between track-circuit and balise…), as well as functions (ex. detection of balise reference, time and location stamp, identifying type of receiving balise, Key Management, Releasing/maintaining a radio connection…) and performance of the transmission media.

### Definitions

#### Transmission media considered hereafter are standard ERTMS/ETCS transmission media used for Class 1 (Eurobalise, Euroradio and Euroloop).

#### A message includes user data (application level) and protocol data (depending on the transmission medium).

#### A Eurobalise message is the information sent by a balise group (i.e. the message is composed of one or several telegrams, sorted by balise number in the group (telegram from balise number 1 first), each telegram is transmitted by a Eurobalise). A Eurobalise telegram contains one header and an identified and coherent set of Packets.

Figure 1: Composition of a balise group message

#### A Euroradio message contains one header and an identified and coherent set of variables (if needed) and Packets.

#### A Euroloop message contains one header and an identified and coherent set of Packets.

## Rules

### Common Rules

#### A message (Euroradio/Euroloop) or telegram (Eurobalise) shall be composed of

#### One Header,

#### When needed, a predefined set of variables (only for Radio),

#### When needed, a predefined set of Packets (only for Radio),

#### Optional Packets as needed by application.

#### The transmission order shall respect the order of data elements listed in the message format (from top to bottom).

#### The behaviour of the receiver shall not depend on the sequence of the Packets given by the message.

##### Exception for Infill information: The locations given in the packets following packet 136 (Infill Location Reference) shall be referred to the balise group indicated in such packet.

##### Note: orientations are in any case always referred to the directionality of balise group (balise transmission), directionality of loop (Euroloop transmission) or directionality of LRBG (radio transmission).

#### It shall be forbidden to send more instances of the same packets type  for the same direction in the same message.

##### Exception 1: A message can contain several packets 44 (Data used by applications outside the ERTMS/ETCS system).

##### Exception 2: A message can contain several packets 65 (Temporary Speed Restriction). The identities of the corresponding temporary speed restrictions (variable NID_TSR) transmitted in the same message shall be different.

##### Exception 3: A message can contain several packets 66 (TSR Revocation). The identities of the corresponding temporary speed restrictions (variable NID_TSR) transmitted in the same message shall be different.

##### Exception 4: A message transmitted by a balise group can contain one packet 136 per balise telegram per direction. Each packet 136 indicates which part of that telegram is to be considered part of the infill information. Multiple packets 136 in balises of a balise group shall  have identical content per direction.

##### Exception 5: Intentionnally deleted

##### Exception 6: A message transmitted by a balise group can contain several packets 254 (default balise or loop information).

### Rules for Eurobalise telegrams

#### The format of the telegram to be transmitted by each balise is as follows:

| General Format of Balise Telegram |  |  |  |
| --- | --- | --- | --- |
| Field No. | VARIABLE | Length ( bits) | Remarks |
| 1 | Q_UPDOWN | 1 | Defines the direction of the information: Down-link telegram (train to track) (0) Up-link telegram (track to train) (1) |
| 2 | M_VERSION | 7 | Version of the ERTMS/ETCS language. |
| 3 | Q_MEDIA | 1 | Defines the type of media: Balise (0) |
| 4 | N_PIG | 3 | Position in the group. Defines the position of the balise in the balise group. |
| 5 | N_TOTAL | 3 | Total number of balises in the balise group |
| 6 | M_DUP | 2 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| 7 | M_MCOUNT | 8 | Telegram counter (M_MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of the balise group. |
| 8 | NID_C | 10 | Country or region. |
| 9 | NID_BG | 14 | Identity of the balise group. |
| 10 | Q_LINK | 1 | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
|  | Information | Variable | This information is composed according to the rules applicable for packets. |

Number of bits in balise header: 50

#### The user information transmitted by a balise shall contain complete packets, i.e. splitting a packet between two balises is forbidden.

#### The telegram ends with packet 255 “End of Information”.

### Rules for Euroloop messages

#### The format of the message to be transmitted by each loop is as follows:

| General Format of Loop Message |  |  |  |
| --- | --- | --- | --- |
| Field No. | VARIABLE | Length ( bits) | Remarks |
| 1 | Q_UPDOWN | 1 | Defines the direction of the information: Down-link message (train to track) (0) Up-link message (track to train) (1) |
| 2 | M_VERSION | 7 | Version of the ERTMS/ETCS language. |
| 3 | Q_MEDIA | 1 | Defines the type of media: Loop (1) |
| 4 | NID_C | 10 | Country or region. |
| 5 | NID_LOOP | 14 | Identity of Euroloop. |
|  | Information | Variable | This information is composed according to the rules applicable for packets. |

Number of bits in loop header: 33

#### The message ends with packet 255 “End of Information”.

### Rules for Euroradio messages

#### The message identifier shall be unique (variable NID_MESSAGE).

#### Each message shall precise the length in bytes (variable L_MESSAGE).

##### If the computed length of the message is not equal to the length given by L_MESSAGE, the entire message shall be rejected.

#### The messages shall be composed of predefined variables and packets.

#### For some messages, it shall be possible to add optional packets at the end of the message.

##### The track to train messages possibly including optional packets are listed hereafter:

| Track to Train message | Mess. ID | Optional packets |
| --- | --- | --- |
| SR Authorisation | 2 | 63 |
| Movement Authority | 3 | 21, 27, 49, 80, plus common optional packets |
| Request To Shorten MA | 9 | 80 - message consistency checking is optional for on-board (Note from Subset 108 regards CR 382), |
| General Message | 24 | 21, 27, plus common optional packets |
| SH authorised | 28 | 49, plus common optional packets |
| MA with Shifted Location Reference | 33 | 21, 27, 49, 80, plus common optional packets - message consistency checking is optional for on-board (Note from Subset 108 regards CR 382), - track-side shall not send optional packet 49 (Note from Subset 108 regards CR 382). |
| In-Fill MA | 37 | 21, 27, 49, 80, plus common optional packets - message consistency checking is optional for on-board (Note from Subset 108 regards CR 382), - track-side shall not send optional packet 49 (Note from Subset 108 regards CR 382). |

###### The common optional packets are the following ones:

Common optional packets

3, 5, 39, 51, 41, 42, 44, 45, 57, 58, 65, 66, 68, 70, 71, 72, 76, 79, 131, 138, 139, 140

##### The train to track message 136 (Train Position Report) and 157 (SoM Position Report) may optionally include the following packets:

##### Packet 4 (Error Reporting),

##### Packet 44 (Data used by applications outside the ERTMS/ETCS system).

##### The train to track message 159 (Session Established) may optionally include the following packets:

##### a)	Packet 3 (Onboard Telephone Numbers)

##### The train to track message 132 (MA request) may optionally include the following packet:

##### a)	Packet 9 (Level 2/3 transition information)

#### If needed to obtain an integer number of bytes, padding shall be added at the end of the message.

#### Standard format of a radio message from track to train :

##### Format:

| Field No. | VARIABLE | Remarks |
| --- | --- | --- |
|  | - NID_MESSAGE | - Message Identification Number |
|  | - L_MESSAGE | - Message length including everything (fields 1 to padding). |
|  | - T_TRAIN | - Time Stamp from RBC (see chapter 3 – Data Consistency). |
|  | - M_ACK | Indicates whether the message must be acknowledged (or not) by the on-board equipment (message n° 146). CS967 |
|  | - NID_LRBG | - Identification Number of LRBG. |
| … | variables as required by NID_MESSAGE | If needed for this message. Used when sending variables which are not included in a packet. |
| … | packets as required by NID_MESSAGE | If needed for this message. |
|  | Optional packets | Refer to section 8.4.4.4 of this document. |
|  | Padding | If required. |

##### Note: In section 8.7 giving the contents of the messages, the padding information is intentionally omitted.

#### Standard format of a radio message from train to track:

##### Format:

| Field No. | VARIABLE | Remarks |
| --- | --- | --- |
|  | - NID_MESSAGE | - Message Identification Number |
|  | - L_MESSAGE | - Message length including everything (field 1 to padding). |
|  | - T_TRAIN | - Time Stamp from Train (see chapter 3 – Data Consistency). |
|  | - NID_ENGINE | Identity of the train. CS967 |
|  | - variables as required by NID_MESSAGE | - If needed for this message. Used when sending variables which are not included in a packet. |
|  | Packet 0 or 1 | Train-to-track packet type 0 – Position report, or packet type 1 - Position report based on two balise groups. Not included in messages 146, 154, 155, 156 and 159. |
|  | Other Packets as required by NID_MESSAGE | (only for message 129) |
|  | Optional packets |  |
|  | Padding | If required. |

##### Exception:  The position report (packet 0 or packet 1) shall not be included in the following messages:

##### Message 146 (Acknowledgement),

##### Message 154 (No compatible version),

##### Message 155 (Initiation of a communication session),

##### Message 156 (Termination of a communication session),

##### Intentionally deleted

##### Message 159 (Session Established).

##### Note: In section 8.6 giving the contents of the messages, the padding information is intentionally omitted.

## List of radio Messages

### Introduction

#### This section identifies the radio messages with corresponding Message Identifier (“Mes. Id.”) and Message Name. It also gives a list of the version-invariant messages.

#### “Type” defines whether a message is to be sent as normal priority (N) or as high priority data (E), as defined in Euroradio specifications.

### Train to Track radio messages

| Mes. Id. | Message Name | Type | Invariant |
| --- | --- | --- | --- |
| 129 | Validated Train Data | N | No |
| 130 | Request for Shunting | N | No |
| 132 | MA Request | N | No |
| 136 | Train Position Report | N | No |
| 137 | Request to shorten MA is granted | N | No |
| 138 | Request to shorten MA is rejected | N | No |
| 146 | Acknowledgement | N | No |
| 147 | Acknowledgement of Emergency Stop | N | No |
| 149 | Track Ahead Free Granted | N | No |
| 150 | End of Mission | N | No |
| 153 | Radio in-fill request | N | No |
| 154 | No compatible version | N | Yes |
| 155 | Initiation of a communication session | N | Yes |
| 156 | Termination of a communication session | N | Yes |
| 157 | SoM Position Report | N | No |
| 159 | Session Established | N | No |

### Track to Train radio messages

| Mes. Id. | Message Name | Type | Invariant |
| --- | --- | --- | --- |
| 2 | SR Authorisation | N | No |
| 3 | Movement Authority | N | No |
| 6 | Recognition of exit from TRIP mode | N | No |
| 8 | Acknowledgement of Train Data | N | No |
| 9 | Request to Shorten MA | N | No |
| 15 | Conditional Emergency Stop | E or N | No |
| 16 | Unconditional Emergency Stop | E or N | No |
| 18 | Revocation of Emergency Stop | N | No |
| 24 | General message | N | No |
| 27 | SH Refused | N | No |
| 28 | SH Authorised | N | No |
| 33 | MA with Shifted Location Reference | N | No |
| 34 | Track Ahead Free Request | N | No |
| 37 | In-fill MA | N | No |
| 40 | Train Rejected | N | No |
| 32 | Configuration Determination | N | Yes |
| 38 | Initiation of a communication session | N | Yes |
| 39 | Acknowledgement of termination of a communication session | N | Yes |
| 41 | Train Accepted | N | No |
| 43 | SoM position report confirmed by RBC | N | No |
| 45 | Assignment of coordinate system | N | No |

## Definition of Radio Messages from Train to Track

### Message 129: Validated Train Data

| Field No. | VARIABLE/ PACKET | Remarks |
| --- | --- | --- |
|  | - NID_MESSAGE |  |
|  | - L_MESSAGE |  |
|  | - T_TRAIN |  |
|  | - NID_ENGINE |  |
|  | - Packet 0 or 1 |  |
|  | Train data | Train - track packet type 11. |

### Message 130: Request for Shunting

| Field No. | VARIABLE/ PACKET | Remarks |
| --- | --- | --- |
|  | - NID_MESSAGE |  |
|  | - L_MESSAGE |  |
|  | - T_TRAIN |  |
|  | - NID_ENGINE |  |
|  | - Packet 0 or 1 |  |

### Message 132: MA Request

| Field No. | VARIABLE/ PACKET | Remarks |
| --- | --- | --- |
|  | - NID_MESSAGE |  |
|  | - L_MESSAGE |  |
|  | - T_TRAIN |  |
|  | - NID_ENGINE |  |
|  | - Q_TRACKDEL |  |
|  | - Packet 0 or 1 |  |
|  | - Optional packets |  |

### Message 136: Train Position Report

| Field No. | VARIABLE/ PACKET | Remarks |
| --- | --- | --- |
|  | - NID_MESSAGE |  |
|  | - L_MESSAGE |  |
|  | - T_TRAIN |  |
|  | - NID_ENGINE |  |
|  | Packet 0 or 1 |  |
|  | Optional packets |  |

### Message 137: Request to Shorten MA is granted

| Field No. | VARIABLE/ PACKET | Remarks |
| --- | --- | --- |
|  | - NID_MESSAGE |  |
|  | - L_MESSAGE |  |
|  | - T_TRAIN |  |
|  | - NID_ENGINE |  |
|  | - T_TRAIN | - Time stamp contained in the request. |
|  | - Packet 0 or 1 |  |

### Message 138: Request to Shorten MA is rejected

| Field No. | VARIABLE/ PACKET | Remarks |
| --- | --- | --- |
|  | - NID_MESSAGE |  |
|  | - L_MESSAGE |  |
|  | - T_TRAIN |  |
|  | - NID_ENGINE |  |
|  | - T_TRAIN | - Time stamp contained in the request. |
|  | - Packet 0 or 1 |  |

### Message 146: Acknowledgement

| Field No. | VARIABLE | Remarks |
| --- | --- | --- |
|  | - NID_MESSAGE |  |
|  | - L_MESSAGE |  |
|  | - T_TRAIN |  |
|  | - NID_ENGINE |  |
|  | - T_TRAIN | - Time stamp contained in the message that is acknowledged. |

### Message 147: Acknowledgement of Emergency Stop

| Field No. | VARIABLE | Remarks |
| --- | --- | --- |
|  | - NID_MESSAGE |  |
|  | - L_MESSAGE |  |
|  | - T_TRAIN |  |
|  | - NID_ENGINE |  |
|  | - NID_EM | - Identification Number of the acknowledged Emergency Message. |
|  | - Q_EMERGENCYSTOP | - Qualifier to indicate whether the train has overpassed the emergency stop location or not when receiving an emergency stop message |
|  | - Packet 0 or 1 |  |

### Message 149: Track Ahead Free Granted

| Field No. | VARIABLE/ PACKET | Remarks |
| --- | --- | --- |
|  | - NID_MESSAGE |  |
|  | - L_MESSAGE |  |
|  | - T_TRAIN |  |
|  | - NID_ENGINE |  |
|  | Packet 0 or 1 |  |

### Message 150: End of Mission

| Field No. | VARIABLE/ PACKET | Remarks |
| --- | --- | --- |
|  | - NID_MESSAGE |  |
|  | - L_MESSAGE |  |
|  | - T_TRAIN |  |
|  | - NID_ENGINE |  |
|  | Packet 0 or 1 |  |

### Message 153: Radio in-fill request

| Field No. | VARIABLE | Remarks |
| --- | --- | --- |
|  | NID_TELEGRAM |  |
|  | L_TELEGRAM |  |
|  | T_TRAIN |  |
|  | NID_ENGINE |  |
|  | NID_C | identity of the country of the “target” main balise group |
|  | NID_BG | identity of the “target” main balise group |
|  | Q_INFILL | start; end of in-fill |
|  | Packet 0 or 1 |  |

### Message 154: No compatible version

| Field No. | VARIABLE/ PACKET | Remarks |
| --- | --- | --- |
|  | - NID_MESSAGE |  |
|  | - L_MESSAGE |  |
|  | - T_TRAIN |  |
|  | - NID_ENGINE |  |

### Message 155: Initiation of a communication session

| Field No. | VARIABLE/ PACKET | Remarks |
| --- | --- | --- |
|  | - NID_MESSAGE |  |
|  | - L_MESSAGE |  |
|  | - T_TRAIN |  |
|  | - NID_ENGINE |  |

### Message 156: Termination of a communication session

| Field No. | VARIABLE/ PACKET | Remarks |
| --- | --- | --- |
|  | - NID_MESSAGE |  |
|  | - L_MESSAGE |  |
|  | - T_TRAIN |  |
|  | - NID_ENGINE |  |

### Message 157: SoM Position Report

| Field No. | VARIABLE/ PACKET | Remarks |
| --- | --- | --- |
|  | - NID_MESSAGE |  |
|  | - L_MESSAGE |  |
|  | - T_TRAIN |  |
|  | - NID_ENGINE |  |
|  | - Q_STATUS |  |
|  | - Packet 0 or 1 |  |
|  | - Optional packets |  |

### Intentionally deleted

### Message 159: Session established

| Field No. | VARIABLE/ PACKET | Remarks |
| --- | --- | --- |
|  | - NID_MESSAGE |  |
|  | - L_MESSAGE |  |
|  | - T_TRAIN |  |
|  | - NID_ENGINE |  |
|  | - Optional Packet |  |

## Definition of Radio Messages from Track to Train

### Message 2: SR Authorisation

| Field No. | VARIABLE | Remarks |
| --- | --- | --- |
|  | - NID_MESSAGE |  |
|  | - L_MESSAGE |  |
|  | - T_TRAIN |  |
|  | - M_ACK |  |
|  | - NID_LRBG |  |
|  | - Q_SCALE |  |
|  | - D_SR |  |
|  | Optional packets |  |

### Message 3: Movement Authority

| Field No. | VARIABLE | Remarks |
| --- | --- | --- |
|  | - NID_MESSAGE |  |
|  | - L_MESSAGE |  |
|  | - T_TRAIN |  |
|  | - M_ACK |  |
|  | - NID_LRBG |  |
|  | Level 2/3 Movement Authority | Packet 15 |
|  | Optional packets |  |

### Message 6: Recognition of exit from TRIP mode

| Field No. | VARIABLE | Remarks |
| --- | --- | --- |
|  | - NID_MESSAGE |  |
|  | - L_MESSAGE |  |
|  | - T_TRAIN |  |
|  | - M_ACK |  |
|  | NID_LRBG |  |

### Message 8: Acknowledgement of Train Data

| Field No. | VARIABLE | Remarks |
| --- | --- | --- |
|  | - NID_MESSAGE |  |
|  | - L_MESSAGE |  |
|  | - T_TRAIN |  |
|  | - M_ACK |  |
|  | NID_LRBG |  |
|  | T_TRAIN | Reference to received train data message |

### Message 9: Request to Shorten MA

| Field No. | VARIABLE | Remarks |
| --- | --- | --- |
|  | - NID_MESSAGE |  |
|  | - L_MESSAGE |  |
|  | - T_TRAIN |  |
|  | - M_ACK |  |
|  | - NID_LRBG |  |
|  | - Level 2/3 Movement Authority | - Packet 15 |
|  | - Optional packet | - Packet 80 |

### Message 15: Conditional Emergency Stop

| Field No. | VARIABLE | Remarks |
| --- | --- | --- |
|  | - NID_MESSAGE |  |
|  | - L_MESSAGE |  |
|  | - T_TRAIN |  |
|  | - M_ACK |  |
|  | - NID_LRBG |  |
|  | - NID_EM | - Identification Number of the Emergency Stop Message. |
|  | - Q_SCALE |  |
|  | - D_REF | - Variable D_REF shall not be included in version 2.3.0 |
|  | - Q_DIR |  |
|  | - D_EMERGENCYSTOP | - Distance between LRBG and the position reference to the emergency stop. |

### Message 16: Unconditional Emergency Stop

| Field No. | VARIABLE | Remarks |
| --- | --- | --- |
|  | - NID_MESSAGE |  |
|  | - L_MESSAGE |  |
|  | - T_TRAIN |  |
|  | - M_ACK |  |
|  | - NID_LRBG |  |
|  | - NID_EM | - Identification Number of the Emergency Stop Message. |

### Message 18: Revocation of Emergency Stop

| Field No. | VARIABLE | Remarks |
| --- | --- | --- |
|  | - NID_MESSAGE |  |
|  | - L_MESSAGE |  |
|  | - T_TRAIN |  |
|  | - M_ACK |  |
|  | - NID_LRBG |  |
|  | - NID_EM | - Identification Number of the Emergency Stop Message. |

### Message 24: General message

| Field No. | VARIABLE | Remarks |
| --- | --- | --- |
|  | - NID_MESSAGE |  |
|  | - L_MESSAGE |  |
|  | - T_TRAIN |  |
|  | - M_ACK |  |
|  | - NID_LRBG |  |
|  | - Optional packets |  |

### Message 27: SH Refused

| Field No. | VARIABLE | Remarks |
| --- | --- | --- |
|  | - NID_MESSAGE |  |
|  | - L_MESSAGE |  |
|  | - T_TRAIN |  |
|  | - M_ACK |  |
|  | - NID_LRBG |  |
|  | T_TRAIN | Time stamp of the shunting request. |

### Message 28: SH Authorised

| Field No. | VARIABLE | Remarks |
| --- | --- | --- |
|  | - NID_MESSAGE |  |
|  | - L_MESSAGE |  |
|  | - T_TRAIN |  |
|  | - M_ACK |  |
|  | - NID_LRBG |  |
|  | - T_TRAIN | - Time stamp of the shunting request. |
|  | - Optional packets |  |

### Message 32: Configuration Determination

| Field No. | VARIABLE | Remarks |
| --- | --- | --- |
|  | - NID_MESSAGE |  |
|  | - L_MESSAGE |  |
|  | - T_TRAIN |  |
|  | - M_ACK |  |
|  | - NID_LRBG |  |
|  | M_VERSION | Version of the ERTMS/ETCS language. |

### Message 33: MA with Shifted Location Reference

| Field No. | VARIABLE | Remarks |
| --- | --- | --- |
|  | - NID_MESSAGE |  |
|  | - L_MESSAGE |  |
|  | - T_TRAIN |  |
|  | - M_ACK |  |
|  | - NID_LRBG |  |
|  | - Q_SCALE |  |
|  | - D_REF | - Reference Distance |
|  | Level 2/3 Movement Authority | Packet 15 |
|  | Optional packets |  |

### Message 34: Track Ahead Free Request

| Field No. | VARIABLE | Remarks |
| --- | --- | --- |
|  | - NID_MESSAGE |  |
|  | - L_MESSAGE |  |
|  | - T_TRAIN |  |
|  | - M_ACK |  |
|  | - NID_LRBG |  |
|  | - Q_SCALE |  |
|  | - D_REF | - Variable D_REF shall not be included in version 2.3.0 |
|  | - Q_DIR |  |
|  | - D_TAFDISPLAY |  |
|  | - L_TAFDISPLAY |  |

### Message 37: In-fill MA

| Field No. | VARIABLE | Remarks |
| --- | --- | --- |
|  | - NID_MESSAGE |  |
|  | - L_MESSAGE |  |
|  | - T_TRAIN |  |
|  | - M_ACK |  |
|  | - NID_LRBG |  |
|  | - Infill Location Reference | - Packet 136 |
|  | - Level 1 Movement Authority | - Packet 12 |
|  | Optional packets |  |

### Message 38: Initiation of a communication session

| Field No. | VARIABLE | Remarks |
| --- | --- | --- |
|  | - NID_MESSAGE |  |
|  | - L_MESSAGE |  |
|  | - T_TRAIN | - (unknown) |
|  | - M_ACK | - (yes) |
|  | - NID_LRBG | - (unknown) |

### Message 39: Acknowledgement of termination of a communication session

| Field No. | VARIABLE | Remarks |
| --- | --- | --- |
|  | - NID_MESSAGE |  |
|  | - L_MESSAGE |  |
|  | - T_TRAIN |  |
|  | - M_ACK | - (no) |
|  | - NID_LRBG |  |

### Message 40: Train Rejected

| Field No. | VARIABLE | Remarks |
| --- | --- | --- |
|  | - NID_MESSAGE |  |
|  | - L_MESSAGE |  |
|  | - T_TRAIN |  |
|  | - M_ACK |  |
|  | - NID_LRBG |  |

### Message 41: Train Accepted

| Field No. | VARIABLE | Remarks |
| --- | --- | --- |
|  | - NID_MESSAGE |  |
|  | - L_MESSAGE |  |
|  | - T_TRAIN |  |
|  | - M_ACK |  |
|  | - NID_LRBG |  |

### Intentionally deleted

### Message 43: SoM position report confirmed by RBC

| Field No. | VARIABLE | Remarks |
| --- | --- | --- |
|  | - NID_MESSAGE |  |
|  | - L_MESSAGE |  |
|  | - T_TRAIN |  |
|  | - M_ACK |  |
|  | - NID_LRBG |  |

### Message 45: Assignment of coordinate system

| Field No. | VARIABLE | Remarks |
| --- | --- | --- |
|  | - NID_MESSAGE |  |
|  | - L_MESSAGE |  |
|  | - T_TRAIN |  |
|  | - M_ACK |  |
|  | - NID_LRBG |  |
|  | - Q_ORIENTATION |  |