| - ERTMS/ETCS – Class 1 |  |
| --- | --- |
| System Requirements Specification Chapter 4 Modes and Transitions |  |
| REF : | SUBSET-026-4 |
| ISSUE : | 2.3.0 |
| DATE : | 24/02/06 |

| Company | Technical Approval | Management approval |
| --- | --- | --- |
| ALCATEL |  |  |
| ALSTOM |  |  |
| ANSALDO SIGNAL |  |  |
| BOMBARDIER |  |  |
| INVENSYS RAIL |  |  |
| SIEMENS |  |  |

## Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Issue 0.0.1 1999-07-05 | / | This document is based on “SRS Class P – Modes and Transitions” issue 1.1.2 (ref. Subset-006-4) | Laffineur J.C. |
| Issue 0.0.2 1999-07-16 | / | New modes/functions/transitions due to Class 1 scope, and impact from WPs (of 1999-07-13). | Laffineur J.C. |
| Issue 0.1.0 1999-07-23 | / | Unisig Review – Stockholm – 22 & 23 July 1999 | Laffineur J.C. |
| 1.0.0 1999-07-29 | Version number, editorial changes. | Finalisation meeting, Stuttgart 990729. | HE |
| 1.2.0 990730 | Version number | Release version | HE |
| 1.3.0 991209 | / | Modifications due to Work Packages on SRS Class 1 v 1.0.0 | Laffineur J.C. |
| 1.4.0 991220 | / | Modifications due to review meeting in Stockholm (991215 and 991216) | Laffineur J.C. |
| 1.4.1 991221 | / | Presentation changes | Laffineur J.C. |
| 2.0.0 991222 | Minor editing | Release version | Laffineur J.C. |
| 2.0.1 | All | Modifications respect to Unisig review (doc. Unisig_all_com_SRS_2.0.0_2) | Laffineur J.C. |
| 2.1.0 | All | Modifications respect to Unisig review (doc. Unisig_all_com_SRS_2.0.1) | Laffineur J.C. |
| 2.2.0 | Version number | UNISIG release | SAB |
| 2.2.2 02 02 01 | / | SUBSET-026 Corrected Paragraphs, Issue 2.2.2 | Laffineur J.C. |
| 2.2.4 | Including all CLRs being in state “EEIG” as per list of CLRs agreed by EEIG on 06/05/04. |  | Hougardy A. |
| 2.2.4 SG checked 28/05/04 | Including all CLRs agreed with the EEIG (see “List of CLRs agreed with EEIG for SRS v2.2.4” dated 28/05/04) Affected clauses see change marks |  | H. Kast |
| 2.2.5 21/01/05 | Incorporation of solution proposal for CLR 007 with EEIG users group comments Corrections according to erratum list agreed in SG meeting 170105 |  | Hougardy A. |
| 2.2.6 04/02/05 | Including all CLRs being in state “EEIG pending” as per list of CLRs extracted on 28/01/05. |  | Hougardy A. |
| 2.2.7 16/06/05 | Including all CLRs extracted from "CR-Report_10.6.05-by number.rtf" and mentioned in column 2.2.7 in "CR status 13.6.05.xls" |  | Hougardy A. |
| 2.2.8 29/11/05 | Change marks cleaned up and updated according to last CRs decisions (including split of CRs7&126) |  | Hougardy A. |
| 2.2.9 24/02/06 | Including all CRs that are classified as "IN" as per SUBSET-108 version 1.0.0 Removal of all CRs that are not classified as "IN" as per SUBSET-108 version 1.0.0, with the exception of CRs 63,98,120,158,538 |  | Hougardy A. |
| 2.3.0 24/02/06 | Release version |  | HK |

## Table of Contents

4.1	Modification History	2

4.2	Table of Contents	4

4.3	Introduction	6

4.3.1	Presentation of the document	6

4.3.2	Identification of the possible modes	6

4.4	Definition of the modes	8

4.4.1	Introduction	8

4.4.2	General Requirements	8

4.4.3	ISOLATION	9

4.4.4	NO POWER	10

4.4.5	SYSTEM FAILURE	11

4.4.6	SLEEPING	12

4.4.7	STAND BY	14

4.4.8	SHUNTING	15

4.4.9	FULL SUPERVISION	17

4.4.10	UNFITTED	18

4.4.11	STAFF RESPONSIBLE	19

4.4.12	ON SIGHT	22

4.4.13	TRIP	23

4.4.14	POST TRIP	24

4.4.15	NON LEADING	26

4.4.16	STM European (SE) mode	28

4.4.17	STM National (SN) mode	30

4.4.18	REVERSING	31

4.5	Modes and Class 1 functions	33

4.5.1	Introduction	33

4.5.2	Active Functions Table	33

4.6	Transitions between modes	38

4.6.1	Symbols	38

4.6.2	Transitions Table	39

4.6.3	Transitions Conditions Table	40

4.7	MMI depending on modes	44

4.7.1	General requirements	44

4.7.2	MMI versus Mode Table	44

4.8	Acceptance of received information	48

4.8.1	INTRODUCTION	48

4.8.2	Assumptions	50

4.8.3	Accepted information depending on the level and transmission media	50

4.8.4	Accepted Information depending on the modes	55

4.9	What happens to accepted and stored information when entering a given level	58

4.9.1	Introduction	58

4.10	What happens to accepted and stored information when entering a given mode	59

4.10.1	Introduction	59

## Introduction

### Presentation of the document

#### This document defines the modes of the ERTMS/ETCS on-board equipment (see chapter 4.4 “Definition of the modes” and chapter 4.5 “Modes and Class 1 functions”).

#### This document gives all transitions between modes (see chapter 4.6 “Transitions between modes”).

#### This document describes the possible exchanged information between the driver and the ERTMS/ETCS on-board equipment, respect to the mode (see chapter 4.7 “MMI depending on modes”).

#### This document describes how the received information is filtered, respect to several criteria such as the level, the mode, etc.. (see chapter 4.8 “Acceptance of received information”).

#### This document describes how the stored information is handled, respect to several criteria such as the level, the mode, etc. (see chapter 4.9 “What happens to accepted and stored information when entering a given level”, and chapter 4.10 “What happens to accepted and stored information when entering a given mode”).

#### All the tables that are included in this document shall be considered as mandatory requirements.

#### Some notes appear in this document. These notes are here to help the reader to understand the specifications, or to explain the reason(s) of a requirement.

### Identification of the possible modes

#### List of the modes:

Full Supervision		(FS)

On Sight			(OS)

Staff Responsible	(SR)

Shunting			(SH)

Unfitted			(UN)

Sleeping			(SL)

Stand By			(SB)

Trip				(TR)

Post Trip			(PT)

System Failure		(SF)

Isolation			(IS)

No Power			(NP)

Non Leading		(NL)

STM European		(SE)

STM National		(SN)

Reversing			(RV)

## Definition of the modes

### Introduction

#### For each mode the following information is given:

- The context of utilisation of the mode and the functions that characterise the mode (chapter “Description”).

- The ERTMS/ETCS levels in which the mode can be used (chapter “Used in levels”).

- The related responsibility of the ERTMS/ETCS on-board equipment and of the driver, once the equipment is in this mode (chapter “Responsibilities”).

#### A complete list of transitions to and from each mode is given in the “Transitions Table”, page 39).

### General Requirements

#### A clear indication of the ERTMS/ETCS mode and the ERTMS/ETCS level shall be shown to the driver (i.e. not in Isolation, No Power and Sleeping mode).

#### Intentionally deleted.

### ISOLATION

#### Description

##### The ERTMS/ETCS on-board equipment shall be in the Isolation mode when the equipment is isolated from the other on-board equipment/systems (including the driver) and physically isolated from the brakes.

##### There shall be a clear indication to the driver that the ERTMS/ETCS on-board equipment is isolated.

##### To leave Isolation mode, a special operating procedure is needed (no transition from Isolation is specified).

##### Note: for the list of main functions related to this mode, refer to chapter 4.5 “Modes and Class 1 functions”.

#### Used in levels

##### Used in all levels: Level 0, level 1, level 2, level 3 and level STM.

#### Responsibilities

##### Isolation of the ERTMS/ETCS on-board equipment is performed by the driver under his complete responsibility.

##### Once the ERTMS/ETCS on-board equipment is isolated, the ERTMS/ETCS on-board equipment has no more responsibility.

### NO POWER

#### Description

##### When the ERTMS/ETCS on-board equipment is not powered, the equipment shall be in the No Power mode.

##### The ERTMS/ETCS on-board equipment shall command the emergency brake.

##### Note: for the list of main functions related to this mode, refer to chapter 4.5 “Modes and Class 1 functions”.

#### Used in levels

##### Used in all levels: Level 0, level 1, level 2, level 3 and level STM.

#### Responsibilities

##### The ERTMS/ETCS on-board equipment has no responsibility in this mode, except commanding the emergency brake.

##### The notion of responsibility of the driver is not relevant for the No Power mode.

### SYSTEM FAILURE

#### Description

##### The ERTMS/ETCS on-board equipment shall switch to the System Failure mode in case of a fault, which affects safety.

##### The ERTMS/ETCS on-board equipment shall permanently command the Emergency Brakes.

##### Note: for the list of main functions related to this mode, refer to chapter 4.5 “Modes and Class 1 functions”.

#### Used in levels

##### Used in all levels: Level 0, level 1, level 2, level 3 and level STM.

#### Responsibilities

##### The ERTMS/ETCS on-board equipment is responsible for commanding the Emergency Brakes.

##### No responsibility of the driver.

### SLEEPING

#### Description

##### The Sleeping mode is defined to manage the ERTMS/ETCS on-board equipment of a slave engine that is remote controlled.

##### The desk of a sleeping engine must be closed (since there is no driver, no information shall be shown).

##### As the engine is remote controlled by the leading engine, its ERTMS/ETCS on-board equipment shall not perform any train movement supervision.

##### The ERTMS/ETCS on-board equipment shall perform train location function.

##### Sleeping mode shall be automatically detected on-board via the train interface.

##### If possible, the train must not be stopped due to a safety critical fault in a sleeping engine. The ERTMS/ETCS on-board equipment shall therefore try to memorise the occurrence of such fault(s), which shall be handled when the engine leaves the Sleeping mode. The ERTMS/ETCS on-board equipment shall also try to send an error information to the RBC.

##### If the desk of the sleeping engine is opened while the train is running (this is an abnormal operation), the ERTMS/ETCS on-board equipment shall switch to Stand-By mode and behaves accordingly.

##### If the “sleeping input signal” is lost (no more detection of the remote control), the switch to Stand-By mode shall be made only if the train is at standstill.

##### When the ERTMS/ETCS level is STM, the ERTMS/ETCS Onboard equipment shall be responsible for providing the STM with access to various resources such as MMI, Odometer, Juridical Recorder and train interface.

##### The ERTMS/ETCS on-board equipment shall open a communication session with the RBC when the following events occur in all levels:

- on receipt of the order to contact the RBC.

- In level 2/3, when entering or exiting Sleeping mode (to report the change of mode to the RBC).

- In level 2/3, when a fault occurs (to report the fault to the RBC).

##### Note: for the list of main functions related to this mode, refer to chapter 4.5 “Modes and Class 1 functions”.

##### The supervision of linking consistency and balise group message consistency shall not be performed in Sleeping mode.

##### When in levels 2 or 3, if no compatible version has been established between the on-board equipment in Sleeping mode and the RBC, the ERTMS/ETCS onboard equipment shall react as specified in 3.17.3.8 but no driver’s indication shall be given.

#### Used in levels

##### Used in all levels: Level 0, level 1, level 2, level 3 and level STM.

#### Responsibilities

##### The ERTMS/ETCS on-board equipment of an engine in Sleeping mode has no responsibility for the train protection.

##### The notion of responsibility of the driver is not relevant for the Sleeping mode.

###### Note: The leading engine is responsible for the movement of the train. It is then the ERTMS/ETCS on-board equipment of the leading engine that is fully/partially/not responsible for the train protection, with respect to its mode.

### STAND BY

#### Description

##### The Stand-By mode is a default mode and cannot be selected by the driver.

##### It is in the Stand-By mode that the ERTMS/ETCS on-board equipment shall awake, and shall perform its self-test and the test of the external devices. Results of tests shall be shown to the driver.

##### Data for mission shall be collected in Stand-By (see SRS-chapter 5: “Start of Mission” procedure).

##### Deleted.

##### The ERTMS/ETCS on-board equipment shall perform the Standstill Supervision.

##### Note: for the list of main functions related to this mode, refer to chapter 4.5 “Modes and Class 1 functions”.

#### Used in levels

##### Used in all levels: Level 0, level 1, level 2, level 3 and level STM.

#### Responsibilities

##### The ERTMS/ETCS on-board equipment is responsible for maintaining the train at standstill.

##### The driver has no responsibility for train movements.

### SHUNTING

#### Description

##### The purpose of the Shunting mode is to enable shunting movements. In Shunting mode, The ERTMS/ETCS on-board equipment supervises the train movements against:

- a ceiling speed: the shunting mode permitted speed limit speed (national value)

- a list of expected balises (if such list was sent by the trackside equipment). The train shall be tripped if a balise group, not contained in the list, is passed (When an empty list is sent, no balise group can be passed. When no list is sent, all balise groups can be passed)

- “stop if in shunting mode” information. The train is tripped if such information is received from balise groups

- When the “override” function is active (because the driver has selected “override”), the train speed shall be supervised against a given “overpass EOA speed” (national /default value) during the time the “override” function is active.

##### The Shunting mode shall not require Train Data.

##### The ERTMS/ETCS on-board equipment shall perform the train location function

##### The entry of the ERTMS/ETCS on-board equipment in the Shunting mode shall be considered as an end of mission.

##### When in Shunting mode, the ERTMS/ETCS on-board shall not manage level transitions, nor RBC-RBC hand-over. Level transition orders shall not be given in the shunting area.

##### Shunting mode can be selected by the driver, only accepted when the train is at standstill, or ordered by the trackside.

##### In case of selection of Shunting mode by the driver:

- in level 1 operations, the switch to shunting is always accepted by the on-board equipment

- in level 2 and 3 areas, the on-board shall ask the trackside for an authorisation. The switch to shunting is possible only after receiving such authorisation. The trackside can send a list of balises, that the train is allowed to pass while in SH, together with the authorisation

##### In case of order to switch to Shunting mode from trackside, the order:

- in level 1 is given by a balise group. A list of balises, that the train is allowed to pass after the entry in Shunting, can be sent together with the order

- in level 2 and 3 is sent via radio. A list of balises, that the train is allowed to pass after the entry in Shunting, can be sent together with the order

##### When the switch to shunting is ordered by trackside, a driver acknowledgement is requested.

###### Note: in Shunting mode the train is only partially supervised, therefore it is necessary that the driver takes the responsibility.

##### The ERTMS/ETCS on-board equipment shall display the train speed and the maximum shunting speed to the driver.

##### The supervised speed shall be indicated only on driver request.

##### Note: for the list of main functions related to this mode, refer to chapter 4.5 “Modes and Class 1 functions”.

#### Used in levels

##### Used in level 0, 1, 2 and 3.

#### Responsibilities

##### The ERTMS/ETCS on-board equipment is responsible for the supervision of the shunting speed limit, and that the engine with the active antenna is tripped when passing the defined border of the shunting area (only if there is a defined border: list of balises given by trackside, or balises giving the information “stop if in shunting”).

##### The driver is responsible for:

- Remaining inside the shunting area defined by a procedure or an external system outside ERTMS/ETCS (also when the shunting area is protected by balises)

- Train/engine movements and shunting operations

### FULL SUPERVISION

#### Description

##### The ERTMS/ETCS on-board equipment shall be in the Full Supervision mode when all train and track data, which is required for a complete supervision of the train, is available on board.

##### Full supervision cannot be selected by the driver, but shall be entered automatically when all necessary conditions are fulfilled.

##### To be in Full Supervision mode, SSP and gradient are not required for the whole length of the train, but shall be at least available from the FRONT END of the train.

##### Once in Full Supervision mode, if SSP and gradient are not known for the whole length of the train, an indication “ENTRY IN FULL SUPERVISION” shall be clearly displayed to the driver until SSP and gradient are known for the whole length of the train.

##### The ERTMS/ETCS on-board equipment shall supervise train movements against a dynamic speed profile.

##### The ERTMS/ETCS on-board equipment shall display the train speed, the permitted speed, the target distance and the target speed to the driver (this list is not exhaustive – refer to chapter 4.7 “MMI depending on modes”).

##### Note: for the list of main functions related to this mode, refer to chapter 4.5 “Modes and Class 1 functions”.

#### Used in levels

##### Used in level 1, 2 and 3.

#### Responsibilities

##### The ERTMS/ETCS on-board equipment is fully responsible for the train protection (except for the 2 situations described below).

##### The driver is responsible for respecting the EOA when approaching an EOA with a release speed.

##### When “ENTRY IN FULL SUPERVISION” is displayed to the driver, the driver is responsible for respecting speed restrictions that apply for the part of the train that is not covered by SSP and gradient data.

### UNFITTED

#### Description

##### The Unfitted mode is used to allow train movements in either:

- Areas that are not equipped with ERTMS/ETCS track-side equipment

- Areas where the ERTMS/ETCS track-side equipment is in commissioning

- Areas that are equipped with a national track-side system, but the related STM is not available

##### The ERTMS/ETCS on-board equipment shall supervise train movements against a ceiling speed: the lowest of the maximum train speed and the maximum speed for unfitted area (national value).

###### When the “override” function is active (because the driver has selected “override”), the train speed shall be supervised against a given “overpass EOA speed” (national /default value) during the time the “override” function is active.

##### The ERTMS/ETCS on-board equipment shall also supervise temporary speed restrictions.

##### The ERTMS/ETCS on-board equipment shall display the train speed to the driver.

##### Note: for the list of main functions related to this mode, refer to chapter 4.5 “Modes and Class 1 functions”.

#### Used in levels

##### Used in level 0.

#### Responsibilities

##### The ERTMS/ETCS on-board equipment supervises a ceiling speed limit and (if available) temporary speed restrictions.

##### The driver must respect the existing line-side signals and is fully responsible for train movements.

### STAFF RESPONSIBLE

#### Description

##### The Staff Responsible mode allows the driver to move the train under his own responsibility in an ERTMS/ETCS equipped area.

##### This mode is used when the system does not know the route. For example:

- After the ERTMS/ETCS on-board equipment starts-up (awakening of the train).

- To pass a signal at danger / override an EOA.

- After a trackside failure (for example: loss of radio contact).

##### The ERTMS/ETCS on-board equipment shall supervise train movements against:

- a ceiling speed: the staff responsible mode permitted speed limit

- a given distance (regarding its origin location see 4.4.11.1.3.1) . The ERTMS/ETCS on-board equipment shall supervise braking curves with a target speed of zero to the end of this distance. If the train overpasses this distance (see next note) the ERTMS/ETCS on-board equipment shall trip the train

- a list of expected balises with the directionality they shall be passed, if this list has been sent by the RBC. The train shall be tripped if over-passing a balise that is not in the list or if passing in the wrong direction a balise of that list. (When an empty list is sent, no balise group can be passed. When no list is sent, all balise groups can be passed)

- balise groups giving the order ‘stop if in SR’. This order shall immediately trip the train

###### The ERTMS/ETCS on-board shall determine the start location of the SR distance as follows:

- If the National/Default value determines the max permitted distance to run in SR mode, the starting point of this distance shall refer to the estimated position of the train front when SR mode was entered, or, already in Staff Responsible mode, when Override was activated. If a new National value for the max permitted distance to run in SR mode is received, the start location shall remain unchanged.

- If the max permitted distance to run in SR mode is determined by the value transmitted by the RBC, or entered by the driver, the start location of the distance shall refer to the estimated position of the train front when the distance information is received or entered.

##### Note : Since the gradient is unknown, the supervision of the braking curves in Staff Responsible mode does not ensure that the train will not pass the given distance.

##### The ERTMS/ETCS trainborne equipment shall give the possibility to the driver to modify the value of the maximum SR speed and of the given distance. This shall be possible only at standstill.

##### In level 2/3, when a finite distance to run in Staff Responsible is given by the RBC, the driver shall have the possibility to request an extension, by selecting "Start" again. This shall trigger an MA request. It shall be the responsibility of the RBC to give the distance to run again, or to give an MA. The new distance to run shall overwrite the remaining part of the previous one.

###### Note: Once the SR distance is covered, the driver may have to go further.

###### When entering SR mode the value applicable for SR speed/ distance shall be the corresponding National/Default value. Exception for SR distance: SR mode is authorised by RBC giving an SR distance.

###### While in SR mode, the value applicable for the maximum SR speed shall be, if available, the last value entered by the driver.

###### While in SR mode, the value applicable for the SR distance shall be, if available, the last value received by the ERTMS/ETCS on-board equipment amongst:

- the distance to run in SR entered by the driver;

- the distance to run in SR given by RBC.

###### When "Override" is selected the SR Speed/distance values previously entered by driver or given by trackside, if any, shall be deleted. The corresponding National/Default value shall enter in force.

##### The ERTMS/ETCS on-board equipment shall display the train speed and the (when active) override (permission to pass a signal at danger, trip inhibited). The permitted speed shall be indicated only on driver request.

##### When the “override” function is active (because the driver has selected “override”), the train speed shall be supervised against a given “overpass EOA speed” (national /default value) during the time the “override” function is active. Once this function is de-activated, the ERTMS/ETCS on-board equipment shall come back to the Staff Responsible speed supervision.

##### If receiving a "track ahead free" request from the RBC, the ERTMS/ETCS on-board equipment requests the driver to enter the "track ahead free" information.

##### Note: for the list of main functions related to this mode, refer to chapter 4.5 “Modes and Class 1 functions”.

##### Intentionally deleted.

#### Used in levels

##### Level 1, 2 and 3.

#### Responsibilities

##### The ERTMS/ETCS on-board equipment supervises a ceiling speed (and if level 2/3, might also supervise a distance and a list of balises).

##### The driver must check if the track is free, if points are correctly positioned, and must respect the existing line-side information (signals, speed boards etc.).

##### When using the possibility to modify the value of the maximum SR speed and of the given distance, the driver is responsible for entering reasonable values.

### ON SIGHT

#### Description

##### The On Sight mode enables the train to enter into a track section that could be already occupied by another train, or obstructed by any kind of obstacle.

##### The authority to use this mode shall come from trackside only (this mode cannot be selected by the driver).

##### The ERTMS/ETCS on-board equipment shall supervise train movements against a dynamic speed profile.

##### The ERTMS/ETCS on-board equipment shall display the train speed to the driver (this list is not exhaustive). The permitted speed, target distance and target speed shall be indicated only on driver request.

##### If receiving a "track ahead free" request from the RBC, the ERTMS/ETCS on-board equipment requests the driver to enter the "track ahead free" information.

##### To be in On Sight mode, SSP and gradient are not required for the whole length of the train, but shall be at least available from the FRONT END of the train.

##### Once in On Sight mode, if SSP and gradient are not known for the whole length of the train, an indication “ENTRY IN ON SIGHT” shall be clearly displayed to the driver until SSP and gradient are known for the whole length of the train.

##### Deleted

##### Note: for the list of main functions related to this mode, refer to chapter 4.5 “Modes and Class 1 functions”.

#### Used in levels

##### Used in level 1, 2 and 3.

#### Responsibilities

##### The ERTMS/ETCS on-board equipment is responsible for the supervision of the train movements.

##### The driver is responsible for checking the track occupancy when moving the train, because the track may be occupied.

### TRIP

#### Description

##### Deleted

###### Note: Application of emergency brakes and train trip are two different things. For example, exceeding the permitted speed leads to application of the emergency brakes, but as long as the train does not pass the EOA, it is not a train trip.

##### The ERTMS/ETCS on-board equipment shall command the emergency brakes (no brake release is possible in Trip mode).

##### The ERTMS/ETCS on-board equipment shall send a warning to the driver.

##### The ERTMS/ETCS on-board equipment shall request an acknowledgement from the driver once train is at standstill (to allow the driver to acknowledge the train trip).

###### Note: This acknowledgement is mandatory to exit from Trip mode.

##### Note: for the list of main functions related to this mode, refer to chapter 4.5 “Modes and Class 1 functions”.

#### Used in levels

##### Used in level STM,1, 2 and 3.

#### Responsibilities

##### The ERTMS/ETCS on-board equipment is responsible for stopping the train and for maintaining the train at standstill.

##### The driver has no responsibility for train movements.

### POST TRIP

#### Description

##### The Post Trip mode shall be entered immediately after the driver acknowledges the trip.

##### Once in post trip mode, the onboard equipment shall release the Command of the emergency brake.

##### The train shall only be authorised to move backwards a given distance (national value). The ERTMS/ETCS on-board equipment shall supervise this national distance for reverse movements, and shall command the service brakes if the distance is over-passed.

###### Note : The ERTMS/ETCS onboard equipment performs the Reverse Movement Protection (as in PT mode, the "normally allowed movement" is backwards, then the Reverse Movement Protection avoids the train running in forward direction when in PT mode). This implies that the given distance to run backwards in PT is considered as a directional data, oriented backwards.

##### When moving backwards in Post Trip mode, the train trip shall be inhibited.

##### The ERTMS/ETCS on-board equipment shall offer the possibility to the driver to select  Start, Shunting, or Override. These are the ONLY possibilities to continue the mission (other transitions exist: exit to Isolation, Failure or No Power modes, but these modes do not allow to continue the mission).

##### When ERTMS/ETCS level is 1, if the driver selects “Start” the onboard equipment proposes Staff Responsible. When ERTMS/ETCS level is 2 or 3, the selection of Start leads to an MA Request to the RBC. It is the RBC responsibility to give an SR authorisation, or a Full Supervision MA or an On Sight/Shunting MA to an ERTMS/ETCS equipment that is in Post Trip mode.

##### Note: The driver is responsible for making the appropriate selection between Start, Shunting, or Override.

##### Note: for the list of main functions related to this mode, refer to chapter 4.5 “Modes and Class 1 functions”.

#### Used in levels

##### Used in level 1, 2 and 3.

#### Responsibilities

##### The ERTMS/ETCS on-board equipment is responsible for supervising that the train moves backwards and that the reverse movement does not exceed the maximum permitted distance (national value).

##### The driver is responsible if moving the train backwards.

### NON LEADING

#### Description

##### The Non-Leading mode is defined to manage the ERTMS/ETCS on-board equipment of a slave engine that is NOT electrically coupled to the leading engine (and so, not remote controlled) but has its own driver.

###### Note: This operating situation is called Tandem.

##### The ERTMS/ETCS on-board equipment shall not perform any train movement supervision in Non-Leading mode.

##### The ERTMS/ETCS on-board equipment shall perform the train location function

##### When level is 2 or 3, the ERTMS/ETCS on-board equipment shall report its position to the RBC, according to the previously received parameters.

##### If possible, the train must not be stopped due to a safety critical fault in a non-leading engine. The ERTMS/ETCS on-board equipment shall therefore try to memorise the occurrence of such fault(s), which shall be handled when the engine leaves Non Leading mode. The ERTMS/ETCS on-board equipment shall also try to send an error information to the RBC.

##### The ERTMS/ETCS on-board equipment shall display the train speed to the driver.

##### When the ERTMS/ETCS level is STM, the ERTMS/ETCS Onboard equipment shall be responsible for providing the STM with access to various resources such as MMI, Odometer, Juridical Recorder and train interface.

##### Note: for the list of main functions related to this mode, refer to chapter 4.5 “Modes and Class 1 functions”.

##### The supervision of linking consistency shall not be performed in Non Leading mode.

##### The supervision of balise group message consistency shall be performed in Non Leading mode, but no safety reaction shall be applied (see 3.16.2.4). The driver shall be informed when a consistency problem has been detected.

#### Used in levels

##### Used in all levels: Level 0, level 1, level 2, level 3 and level STM.

#### Responsibilities

##### The ERTMS/ETCS on-board equipment shall perform NO protection functions.

##### The driver is responsible for the selection of Non Leading. No protection against misuse of this mode is possible.

### STM European (SE) mode

#### Description

##### The SE mode shall permit reuse of supervision functionality in the ERTMS/ETCS on-board equipment such as MA, most restrictive speed profile and gradient. Furthermore, the SE mode shall enable the STM to access the following resources via the ERTMS/ETCS Onboard; MMI, odometer, train interface and brakes.

##### The ERTMS/ETCS on-board equipment shall be able to receive STM profile data from the STM, and shall supervise according to this information.

##### The ERTMS/ETCS on-board equipment shall allow the STM to inhibit parts of the information normally displayed by the ERTMS/ETCS Onboard.

##### As an exception to the ERTMS/ETCS supervision functions, in case of overpassing the EOA the onboard shall issue an emergency brake command and inform the STM that EOA has been overpassed.

##### The ERTMS/ETCS Onboard shall revoke the brake command when a new STM profile including an acknowledgement that the STM has received the information about overpassing the EOA is received from the STM.

###### Note: Otherwise there is a hazard that the new STM Profile is issued before reception of overpass EOA information.

##### The ERTMS/ETCS on-board equipment shall be able to accept brake command and revocation of brake command from the STM.

##### Note : A limited set of data coming from balises shall be used by the ERTMS/ETCS on-board equipment, refer to SRS chapter 4.8 “Use of received information”.

##### Note: for the list of main functions related to this mode, refer to chapter 4.5 “Modes and Class 1 functions”.

#### Used in levels

##### Level STM

#### Responsibilities of ERTMS/ETCS Onboard

##### The ERTMS/ETCS on-board equipment is responsible for keeping any train movements within the STM Profile given by the STM.

##### The ERTMS/ETCS on-board equipment is responsible for interacting with the driver based on the information received from the STM.

##### The ERTMS/ETCS on-board equipment is responsible for taking safe action in case of a failure detected on the STM interface, i.e. command brake application.

##### The ERTMS/ETCS on-board equipment is responsible for providing the STM with access to various resources such as MMI, Odometer, Juridical Recorder and train interface.

#### Responsibilities of STM

##### The STM is responsible for maintaining national system behaviour and interact with national trackside equipment.

##### The STM is responsible for providing STM Profile information to the ERTMS/ETCS Onboard.

##### The STM is responsible for interaction with the driver on the STM specific parts of information displayed to the driver.

#### Responsibilities of the driver

##### The responsibility of the driver depends on the STM in use and on the national trackside system.

### STM National (SN) mode

#### Description

##### The SN mode shall enable an STM to access the following resources via the ERTMS/ETCS on-board equipment; MMI, odometer, train interface and brakes. No supervision functionality is provided by the ERTMS/ETCS on-board equipment.

##### A limited set of data coming from balises shall be used by the ERTMS/ETCS on-board equipment, refer to SRS chapter 4.8 “Use of received information”.

##### Note: for the list of main functions related to this mode, refer to chapter 4.5 “Modes and Class 1 functions”.

#### Used in levels

##### Level STM.

#### Responsibilities of ERTMS/ETCS Onboard

##### The ERTMS/ETCS Onboard is responsible for providing the STM with access to various resources such as MMI, Odometer, Juridical Recorder and train interface.

##### The ERTMS/ETCS Onboard is responsible for taking safe action in case of a failure detected on the STM interface, i.e. command brake application.

#### Responsibilities of STM

##### The STM is responsible for all train supervision and protection functions.

##### The STM is responsible for issuing and revoking brake command.

##### The STM is responsible for maintaining national system behaviour and interact with national trackside equipment.

##### The STM is responsible for interaction with the driver.

#### Responsibilities of the driver

##### The responsibility of the driver depends on the STM in use and on the national trackside system.

### REVERSING

#### Description

##### The Reversing mode allows the driver to change the direction of movement of the train and drive from the same cab, i.e. the train orientation remains unchanged. This shall be possible only in areas so marked by trackside. Reversing areas shall be announced in advance by trackside.

##### Note: This mode is used to allow the train to escape from a dangerous situation and to reach as fast as possible a “safer” location.

##### The ERTMS/ETCS on-board equipment shall supervise train movements against:

- a ceiling speed given from trackside

- a distance to run in the direction opposite to the train orientation, given from trackside. Emergency brake shall be commanded if over-passing this distance

##### In level 2/3 applications it shall be possible for the RBC to send a new distance to run in reversing movement. The new distance shall overwrite the previous one.

##### The ERTMS/ETCS on-board equipment shall display the train speed, the permitted speed and remaining distance to run.

##### In level 2/3 applications, the ERTMS/ETCS on-board equipment in Reversing mode shall send position reports to the RBC, according to the previously received parameters.

##### The position reports sent when in reversing mode shall refer to the location of the driving cab (as before reversing).

##### Note : The ERTMS/ETCS onboard equipment performs the Reverse Movement Protection (as in RV mode, the "normally allowed movement" is backwards, then the Reverse Movement Protection avoids the train running in forward direction when in RV mode). This implies that the given distance to run in reversing is considered as a directional data, oriented backwards.

##### Note: for the list of main functions related to this mode, refer to chapter 4.5 “Modes and Class 1 functions”.

#### Used in levels

##### Level 1, 2, 3.

#### Responsibilities

##### The ERTMS/ETCS on-board equipment supervises a ceiling speed and a distance to run in reverse direction.

##### The driver must keep the train movement inside the received distance to run.

## Modes and Class 1 functions

### Introduction

#### The following table specifies in which modes the on-board functions are active or not. The functions are described in the “Related SRS §” (second column of the table).

#### Note : Modes are not the only thing that can influence an onboard function. This is why this Table is not enough in itself to understand all the ERTMS/ETCS onboard behaviour. It must be understood as a complement to all other SRS chapters (especially §4.7, 4.8, 4.9 and 4.10).

#### Note : for MMI depending on modes, refer to §4.7.

### Active Functions Table

#### X = functions shall be active
Empty case = function shall be inactive
O = Optional (function is not required for interoperability, but is not forbidden)

| ONBOARD-FUNCTIONS | A200 FRS ID | RELATED SRS § | N P | S B | S H | F S | S R | OS | S L | N L | U N | T R | P T | S F | I S | S E | S N | R V |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Consistency | not FRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Check linking consistency | / | 3.16.2.3 3.4.4 |  |  |  | X |  | X |  |  |  |  |  |  |  |  |  |  |
| Balise Group Message Consistency if linking consistency is checked | / | 3.16.2.4.1 3.16.2.4.3 |  |  |  | X |  | X |  |  |  |  |  |  |  |  |  |  |
| Balise Group Message Consistency if no linking consistency is checked (because no linking information is available and/or because the function “check linking consistency is not active) | / | 3.16.2.4.4 |  | X | X | X | X | X |  | X | X | X | X |  |  | X | X | X |
| Unlinked Balise Group Message Consistency | / | 3.16.2.5 |  | X | X | X | X | X |  |  | X |  | X |  |  | X | X |  |
| Check correctness of radio messages | / | 3.16.3.1 |  | X | X | X | X | X | X | X | X | X | X |  |  | X | X | X |
| Check radio sequence | / | 3.16.3.3 |  | X | X | X | X | X | X | X | X | X | X |  |  | X | X | X |
| Check radio link (only level 2/3) | / | 3.16.3.4 |  |  |  | X |  | X |  |  |  |  |  |  |  |  |  |  |
| Determine Train Speed and Location: | 4.3.4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Determine train location referenced to LRBG | 4.3.4 | 3.6.1 3.6.4 |  | X | X | X | X | X | X | X | X | X | X |  |  | X | X | X |
| Determine train speed, train acceleration, train standstill | 4.3.5 | None |  | X | X | X | X | X | X | X | X | X | X |  | O | X | X | X |
| Determine Geographical Position | 4.7.3 | 3.6.6 |  | X |  | X | X | X |  | X | X | X | X |  |  |  |  | X |
| Report train position when train reaches standstill | 4.3.4 | 3.6.5.1.4 a) |  |  |  | X | X | X |  |  |  |  |  |  | O |  |  |  |
| Report train position when mode changes | 4.3.4 | 3.6.5.1.4 b) |  | X | X | X | X | X | X | X | X | X | X | X | O | X | X | X |
| Report train position when train integrity selected by driver | 4.3.4 | 3.6.5.1.4 c) |  | X |  | X | X | X |  |  |  |  | X |  |  |  |  |  |
| Report train position when loss of train integrity is detected | 4.3.4 | 3.6.5.1.4 d) |  | X |  | X | X | X |  |  |  | X | X |  |  |  |  | X |
| Report train position when train has passed a RBC/RBC border | 4.3.4 | 3.6.5.1.4 e) |  |  |  | X | X | X |  |  |  | X |  |  |  |  |  |  |
| Report train position when change of train orientation | 4.3.4 | 3.6.5.1.4 f) |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Report train position when change of level due to trackside order | 4.3.4 | 3.6.5.1.4 g) |  |  |  | X | X | X |  |  |  | X |  |  |  |  |  |  |
| Report train position when change of level due to driver request | 4.3.4 | none |  | X |  | X | X | X |  | X |  |  | X |  |  |  |  |  |
| Report train position when establishing a session with RBC | 4.3.4 | Covered by “Manage communic. session “ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Report train position as requested by RBC… | 4.3.4 | 3.6.5.1.4 i) |  | X |  | X | X | X |  | X | X | X | X |  |  | X | X |  |
| … or Report train position at every balise group passage | 4.3.4 | 3.6.5.1.4 j) |  |  |  | X | X | X |  | X | X | X | X |  |  | X | X | X |
| Manage MA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Request MA Cyclically respect to approach of target indication point (T_MAR) or MA timer elapsing (T_TIMEOUTRQST) (only level 2/3) | not FRS | 3.8.2.3 a) and b) |  |  |  | X |  | X |  |  |  |  |  |  |  |  |  |  |
| Request MA Cyclically when “Start” is selected (only level 2/3) | not FRS | 4.4.11 5.4, 5.11 |  | X |  |  | X |  |  |  |  |  | X |  |  |  |  |  |
| Request MA on reception of "track ahead free up to the level 2/3 transition location" (only level 0,1,STM) | not FRS | 3.8.2.7.1 |  | X |  | X | X | X |  |  | X | X | X |  |  | X | X |  |
| Determine EOA/LOA, SvL, Danger Point, etc… | 4.2.2 | 3.8.4 3.8.5 |  |  |  | X |  | X |  |  |  |  |  |  |  | X |  |  |
| Handle Co-operative MA revocation | 11.7.1 | 3.8.6 |  |  |  | X |  | X |  |  |  |  |  |  |  |  |  |  |
| Manage Emergency Stop | 4.6.4 | 3.10 |  |  |  | X | X | X |  |  |  |  | X |  |  |  |  |  |
| Determine Most Restrictive Speed Profile, based on : | 4.3.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SSP | 4.3.1 | 3.11.3 |  |  |  | X |  | X |  |  |  |  |  |  |  |  |  |  |
| ASP | 4.3.1 | 3.11.4 |  |  |  | X |  | X |  |  |  |  |  |  |  |  |  |  |
| TSR | 4.3.1 | 3.11.5 |  |  |  | X | X | X |  |  | X |  |  |  |  |  |  |  |
| Signalling related SR | 4.3.1 | 3.11.6 |  |  |  | X |  | X |  |  |  |  |  |  |  |  |  |  |
| Mode related SR | 4.3.1 | 3.11.7 |  |  | X |  | X | X |  |  | X |  |  |  |  |  |  |  |
| Train related SR | 4.3.1 | 3.11.8 |  |  | X | X | X | X |  |  | X |  |  |  |  |  |  | X |
| Ceiling speed for reversing supervision | 4.3.1 | 4.4.18.1.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |
| Supervise Train Speed | 4.3.2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Dynamic Speed Monitoring based on MRSP, MA, release speed, gradient, mode profile, and route unsuitability location | 4.3.2 4.3.3 4.3.7 | 3.13 5.9.3.5 5.7.3.4 3.12.2.8 |  |  |  | X |  | X |  |  |  |  |  |  |  |  |  |  |
| Dynamic Speed Monitoring based on STM Profile Data (MRSP, MA and gradient) | 4.3.2 4.3.7 | 3.15.6 |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |
| Dynamic Speed Monitoring based on MRSP | 4.3.2 4.3.7 | 4.4.10.1 |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |
| Dynamic Speed Monitoring based on MRSP, allowed distance to run in Staff Resp. mode | 4.3.2 4.3.7 | 4.4.11 |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |
| Ceiling Speed Supervision (no DSM) based on MRSP | 4.3.2 4.3.7 | 4.4.8.1.1 a) |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  | X |
| Supervision of permitted speed limit when "Override EoA" is active" |  | 4.4.8.1.1 d) 4.4.10.1.2.1 4.4.11.1.8 |  |  | X |  | X |  |  |  | X |  |  |  |  | X | X |  |
| Supervise Train Movements |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Backwards Distance Monitoring | not FRS | 4.4 |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  | X |
| Roll Away Protection | 4.3.9 | 3.14.2 |  |  | X | X | X | X |  |  | X |  | X |  |  | X |  | X |
| Reverse Movement Protection | 4.3.9 | 3.14.3 |  |  |  | X | X | X |  |  |  |  | X |  |  | X |  | X |
| Standstill Supervision | not FRS | 3.14.4 4.4.7.1.5 |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Supervise “danger for shunting” information and list of expected balises for shunting | 4.1.3 | 4.4.8.1.1 b) and c) |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Supervise “Stop if in SR” information and list of expected balises for Staff Responsible | not FRS | 4.4.11.1.3 c) and d) |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |
| Command Emergency Brake | not FRS | 4 | X |  |  |  |  |  |  |  |  | X |  | X |  |  |  |  |
| Determine Mode and Level | 3.9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Determine ERTMS/ETCS Mode | 3.9 | 3.12.4, 4.6 | X | X | X | X | X | X | X | X | X | X | X | X | X | X | X | X |
| Determine ERTMS/ETCS level | 3.9 | 5.10 |  | X | X | X | X | X | X | X | X | X | X |  | X | X | X | X |
| Other functions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| System Configuration Management (only for balise message) | 11.12.1 | 3.17 |  | X | X | X | X | X | X | X | X | X | X |  |  | X | X | X |
| Manage Communication Session | 3.4 | 3.5 |  | X | X | X | X | X | X | X | X | X | X |  |  | X | X | X |
| Delete Revoked TSR | not FRS | 3.11.5.5 |  | X |  | X | X | X |  |  | X |  | X |  |  |  |  |  |
| Override (Trip inhibition) | 4.5.2 | 5.8 |  |  | X |  | X |  |  |  | X |  |  |  |  | X | X |  |
| Manage Track Conditions excluding Big Metal Masses | 4.8 | 3.12.1 |  |  |  | X |  | X |  | X |  | X | X |  |  |  |  |  |
| Manage Track Condition Big Metal Masses | not FRS | 3.12.1 |  | X | X | X | X | X | X | X | X | X | X | O |  | X | X | X |
| Manage Route Suitability | 4.6.11 | 3.12.2 |  |  |  | X |  | X |  |  |  |  |  |  |  |  |  |  |
| Manage Text Display to the driver | 6.2.4 | 3.12.3 |  | X |  | X | X | X |  | X | X |  | X |  |  |  |  |  |
| Manage RBC/RBC Handover | 11.1 | 3.15.1, 5.15 |  |  |  | X | X | X | X | X |  | X |  |  |  |  |  |  |
| Manage Track Ahead Free Request | not FRS | 3.15.5 |  | X |  |  | X | X |  |  |  |  | X |  |  |  |  |  |
| Allow access to MMI, train interfaces, JRU and odometer for STM | 3.7 | 3.15.6 |  |  |  |  |  |  |  |  |  |  |  |  |  | X | X |  |
| Provide Fixed Values, and Default/National Values | 3.10 3.11 | 3.18.1 3.18.2 |  | X | X | X | X | X | X | X | X | X | X |  |  | X | X | X |
| Capture Train Data | 4.1.2 | 3.18.3 3.19 5.4.3 |  | X |  | X | X | X |  |  | X |  |  |  |  | X | X |  |
| Capture Additional Data | 4.1.2 | 3.18.4 5.4.3 |  | X |  | X | X | X |  | X | X |  |  |  |  | X | X |  |
| Provide Date and Time | not FRS | 3.18.5 |  | X | X | X | X | X | X | X | X | X | X |  |  | X | X | X |
| Record Juridical Data | 4.3.10 | 3.20 |  | X | X | X | X | X | X | X | X | X | X | X |  | X | X | X |

Figure 1: Active Functions table

## Transitions between modes

### Symbols

#### The indication “4>” means: The condition n°4 must be fulfilled to trigger the transition

#### From the mode located in the column

#### To the mode that is indicated by the arrow “>”.

#### Each transition from a given mode receives a priority order (indicated by “-px-”, x is the priority order) to avoid a conflict between the different transitions when they occur at the same time (i.e. in the same clock cycle). P1 has a higher priority than P2.

#### Some transitions have received the same priority order. This has been decided when it is obvious that these transitions cannot occur at the same time, and so can never lead to a conflicting situation (for example, the RBC cannot give in the same time a MA for FS and a MA for OS to a given engine, this is why the transition “from SR to FS” and the transition “from SR to OS” have the same priority order).

#### "16, 17, 18" means "16 or 17 or 18".

### Transitions Table

| NP | <29 -p2- | <29 -p2- | <29 -p2- | <29 -p2- | <29 -p2- | <29 -p2- | <29 -p2- | <29 -p2- | <29 -p2- | <29 -p2- | <29 -p2- |  | <29 -p2- | <29 -p2- | <29 -p2- |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4> -p2- | SB | <19, 28 -p5- | <28 -p5- | <28, -p5- | <28, -p5- | <2, 3 -p4- | <28, 47 -p3- | <28, -p6- |  |  |  |  | <28 -p6- | <28 -p6- | <28 -p4- |
|  | 5, 6, 50> -p7- | SH | <5,6, 50,51 -p6- | <5,6, 51 -p6- | <5,6 50,51 -p6- |  |  | <5,61 -p7- |  | <5,6, 50 -p4- |  |  | <61 -p7 | <61 -p7 |  |
|  | 10> -p7- |  | FS | <31,32 -p6- | <31,32 -p6- |  |  | <25 -p7- |  | <31 -p4- |  |  | <25 -p7- | <25 -p7- |  |
|  | 8,37> -p7- |  | 37> -p6- | SR | <37 -p6- |  |  | <44 -p4- |  | <8,37 -p4- |  |  | <44 -p4- | <44 -p4- |  |
|  | 15> -p7- |  | 15,40> -p6- | 40> -p6- | OS |  |  | <34 -p7- |  | <15 -p4- |  |  | <34 -p7- | <34 -p7- |  |
|  | 14> -p5- |  |  |  |  | SL |  |  |  |  |  |  |  |  |  |
|  | 46> -p6- | 46> -p5- | 46> -p6- | 46> -p6- | 46> -p6- |  | NL |  |  |  |  |  |  |  |  |
|  | 60> -p7- |  | 21> -p6- | 21> -p6- | 21> -p6- |  |  | UN | <62 -p3- |  |  |  | <21 -p7- | <21 -p7- |  |
|  | 20> -p4- | 49,52,65> -p4- | 12,16,17, 18,20,41,65,66> -p4- | 18,20,42, 43, 36, 54,65> -p4- | 12,16,17 ,18,20,41,65,66> -p4- |  |  | 67,39> -p5- | TR |  |  |  | <67,39 -p5- | <67,39 -p5- |  |
|  |  |  |  |  |  |  |  |  | 7> -p4- | PT |  |  |  |  |  |
|  | 13> -p3- | 13> -p3- | 13> -p3- | 13> -p3- | 13> -p3- |  |  | 13> -p3- | 13> -p3- | 13> -p3- | SF |  | <13 -p3- | <13 -p3- | <13 -p3- |
| 1> -p1- | 1> -p1- | 1> -p1- | 1> -p1- | 1> -p1- | 1> -p1- | 1> -p1- | 1> -p1- | 1> -p1- | 1> -p1- | 1> -p1- | 1> -p1- | IS | <1 -p1- | <1 -p1- | <1 -p1- |
|  | 57> -p7- |  | 55> -p6- | 55> -p6- | 55> -p6- |  |  | 55> -p7- | 64> -p4- |  |  |  | SE | <55 -p6- |  |
|  | 58> -p7- |  | 56> -p6- | 56> -p6- | 56> -p6- |  |  | 56> -p7- | 63> -p4- |  |  |  | 56> -p7- | SN |  |
|  |  |  | 59> -p6- |  | 59> -p6- |  |  |  |  |  |  |  |  |  | RV |

Figure 2 Transition table.

### Transitions Conditions Table

| Condition Id | Content of the conditions |
| --- | --- |
| [1] | The driver isolates the ERTMS/ETCS on-board equipment. |
| [2] | (a desk is open) |
| [3] | (no “go sleeping” input signal is received any more) AND (train is at standstill) |
| [4] | The ERTMS/ETCS on-board equipment is powered. |
| [5] | (train is at standstill) AND (ERTMS/ETCS level is 0 or 1) AND (driver selects Shunting mode) |
| [6] | (train is at standstill) AND (ERTMS/ETCS level is 2 or 3) AND (reception of the information “Shunting granted by RBC”, due to a Shunting request from the driver) |
| [7] | (the driver acknowledges the train trip) AND (the train is at standstill) AND (the ERTMS/ETCS level is different from 0, STM) |
| [8] | (Staff Responsible mode is proposed to the driver) AND (driver selects Staff Responsible) {4} |
| [9] | Empty |
| [10] | (Train Data is stored on board) AND (MA + SSP +gradient are on-board) AND (no specific mode is required by a Mode Profile) |
| [11] | Empty |
| [12] | - (The train/engine overpasses the EOA/LOA with its min safe antenna position) AND (ERTMS/ETCS level is 1) |
| [13] | The ERTMS/ETCS on-board equipment detects a fault that affects safety |
| [14] | (The “sleeping” input signal is received) AND (train is at standstill) AND (all desks connected to the ERTMS/ETCS on-board equipment are closed) |
| [15] | (An ackn. request for On Sight is displayed to the driver) AND (the driver acknowledges) see {1} here under |
| [16] | (The train/engine overpasses the EOA/LOA with its min safe front end) AND (ERTMS/ETCS level is 2 or 3). |
| [17] | The onboard reacts according to a linking reaction set to “trip”. |
| [18] | (the train/engine receives and uses a “trip” information given by balise) AND (override is not active) |
| [19] | (driver selects “exit Shunting”) AND (train is at standstill). |
| [20] | (reception of unconditional emergency stop message) |
| [21] | (ERTMS/ETCS level switches to 0) see {2} here under |
| [22] | Empty |
| [23] | Empty |
| [24] | Empty |
| [25] | (ERTMS/ETCS level switches to 1,2 or 3) AND (MA+SSP+gradient are on-board) AND (no specific mode is required by a Mode Profile) |
| [26] | Empty |
| [27] | - Empty |
| [28] | (desks are closed) |
| [29] | the ERTMS/ETCS on-board equipment is NOT powered |
| [30] | Empty |
| [31] | (MA+SSP+gradient are on-board) AND (no specific mode is required by a Mode Profile) AND (ERTMS/ETCS level is 2 or 3) |
| [32] | (MA+SSP+gradient are on-board) AND (no specific mode is required by a Mode Profile) AND (ERTMS/ETCS level is 1) AND (no "trip" information is given by balise) |
| [33] | Empty |
| [34] | (A Mode Profile defining an On Sight area is on-board) AND (The max safe front end of the train is inside the On Sight area) AND (The ERTMS/ETCS level switches to 1,2 or 3) |
| [35] | Empty |
| [36] | (the identity of the over-passed balise group is not in the list of expected balises related to SR mode) AND (override is not active). |
| [37] | (driver selects “override”) AND (train speed is under max. speed limit for triggering the “override” function) see {3} here under |
| [38] | Empty |
| [39] | (The ERTMS/ETCS level switches to 1,2 or 3) AND (no MA has been received) |
| [40] | (A Mode Profile defining an On Sight area is on-board) AND (The max safe front end of the train is inside the On Sight area) |
| [41] | (T_NVCONTACT is passed) AND (associated reaction is “train trip”) |
| [42] | (The train/engine overpasses the SR distance with its estimated front end) AND (override is not active) |
| [43] | (The train/engine overpasses the former EoA (when Override was activated) with the min safe antenna position) AND (override is not active), see {3} here under |
| [44] | (“override” function is active) AND (ERTMS/ETCS level switches to 1 or 2 or 3) see {3} here under |
| [45] | Empty |
| [46] | (Driver selects NON LEADING) AND (train is at standstill) |
| [47] | (NON LEADING is ended) and (train is standstill) |
| [48] | Empty |
| [49] | (reception of information “stop if in shunting”) AND (override is not active) |
| [50] | (An ackn. request for Shunting is displayed to the driver) AND (the driver acknowledges) see {5} here under |
| [51] | (A Mode Profile defining the entry of a Shunting area is used on-board) AND (The max safe front end of the train is inside the Shunting area) |
| [52] | (the identity of the over-passed balise is not in the list of expected balises related to SH mode) AND (override is not active). |
| [53] | Empty |
| [54] | (reception of information “stop if in Staff Responsible”) AND (override is not active) |
| [55] | (the ERTMS/ETCS level is “STM”) AND (SE mode is required for the STM) |
| [56] | (the ERTMS/ETCS level is “STM”) AND (SN mode is required for the STM) |
| [57] | (the ERTMS/ETCS level is “STM”) AND (SE mode is required for the STM) AND (driver has selected Start) |
| [58] | (the ERTMS/ETCS level is “STM”) AND (SN mode is required for the STM) AND (driver has selected Start) |
| [59] | (train is at standstill) AND (driver has acknowledged the reversing) see {6} here under |
| [60] | (an acknowledgement request for UN mode is displayed to the driver) AND (the driver acknowledges) |
| [61] | (A Mode Profile defining a Shunting area is on-board) AND (The max safe front end of the train is inside the Shunting area) AND (The ERTMS/ETCS level switches to 1,2 or 3) |
| [62] | (the driver acknowledges the train trip) AND (the train is at standstill) AND (the ERTMS/ETCS level is 0) |
| [63] | (the driver acknowledges the train trip) AND (the train is at standstill) AND (the ERTMS/ETCS level is STM) AND (STM is National) |
| [64] | (the driver acknowledges the train trip) AND (the train is at standstill) AND (the ERTMS/ETCS level is STM) AND (STM is European) |
| [65] | (A balise telegram with a non compatible version of the ERTMS/ETCS language is received) AND (ERTMS/ETCS level is 1, 2 or 3) |
| [66] | A balise group contained in the linking information is passed in the unexpected direction |
| [67] | (The ERTMS/ETCS level switches to level 1) AND (a trip order has been received) AND (override is not active) |

{1} The request to acknowledge On Sight is displayed to the driver only if certain conditions are fulfilled. These conditions are not specified here. See the “On Sight” procedure” of SRS-§5 (for transitions from FS/SR/UN to OS) and the "Start of mission" procedure (for transition from SB to OS).

{2} This transition to the Unfitted mode is also a transition of level.. For further information, See the “Level Transition” procedure” (SRS-§5) for transitions from FS/SR/OS to UN and the “Start Of Mission” procedure” (SRS-§5) for transition from SB to UN.

{3} See the “Override” procedure” of SRS-§5.

{4} The Staff Responsible mode is proposed to the driver only if certain conditions are fulfilled. These conditions are not specified here. See the “Start Of Mission” procedure and the "Train Trip" procedure of SRS-§5.

{5} The request to acknowledge Shunting is displayed to the driver only if certain conditions are fulfilled. These conditions are not specified here. See the “Entry in Shunting” procedure and the “Start Of Mission” procedure of SRS-§5.

{6} The request to acknowledge Reversing is displayed to the driver when certain conditions are fulfilled. These conditions are not specified here. See the “reversing” procedure of SRS-§5.

## MMI depending on modes

### General requirements

#### Note : As a reminder, the MMI is made of any “device” that allows the exchange of information between the driver and the ERTMS/ETCS onboard equipment.

#### The device(s) used to select “ERTMS/ETCS onboard equipment powered/unpowered” shall always be available to the driver; independently of the current mode.

#### The device(s) used to select “ERTMS/ETCS onboard equipment isolated/not isolated” shall always be available to the driver; independently of the current mode.

#### The device(s) used to select “desk open/closed” shall always be available to the driver; independently of the current mode.

### MMI versus Mode Table

##### X = active : For an MMI output, this means that the output information is shown to the driver when the ERTMS/ETCS onboard equipment is in the mode indicated in the column. For an MMI input, this means that the driver has the possibility to enter this information when the ERTMS/ETCS onboard equipment is in the mode indicated in the column).

##### A = available : This means that the input/output becomes active ONLY if another condition(s) is (are) fulfilled. This condition(s) are not described here.

| Input information | N P | S B | S H | F S | S R | O S | S L | N L | U N | T R | P T | S F | I S | S E | S N | R V |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data entry request |  | X | X | X | X | X |  | X | X |  |  |  |  | X | X |  |
| Train Data - Max train speed - train length - international train category - deceleration capacity - time to cut off traction - brake delay time - loading gauge - axle load - power supply train running number |  | A |  | A | A | A |  |  | A |  |  |  |  | A | A |  |
| Selection of language |  | A | A | A | A | A |  | A | A |  |  |  |  | A | A |  |
| Which STM to be activated |  | A |  |  |  |  |  |  |  |  |  |  |  | A | A |  |
| Additional STM data |  | A |  | A | A | A |  |  | A |  |  |  |  | A | A |  |
| Train data confirmation |  | A |  | A | A | A |  |  | A |  |  |  |  | A | A |  |
| Additional data - Driver id - ERTMS/ETCS level - Track Adhesion factor |  | A |  | A | A | A |  | A | A |  |  |  |  | A | A |  |
| Additional data re-validation - Driver id - ERTMS/ETCS level |  | A |  | A | A | A |  | A | A |  |  |  |  | A | A |  |
| RBC contact information - Radio network-id - RBC-id - RBC phone number |  | A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Train integrity |  | A |  | A | A | A |  |  |  |  | A |  |  |  |  |  |
| Start |  | A |  |  | A |  |  |  |  |  | A |  |  | A | A |  |
| Override request |  | X | X | X | X | X |  |  | X |  | X |  |  | X | X |  |
| Override route unsuitability |  |  |  | A |  | A |  |  |  |  |  |  |  |  |  |  |
| Shunting request |  | X |  | X | X | X |  |  | X |  | A |  |  | A | A |  |
| “Exit of Shunting” request |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Non Leading request |  | X | X | X | X | X |  |  |  |  |  |  |  |  |  |  |
| “Exit of Non Leading” request |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |
| Ackn of fixed text information |  | A | A | A | A | A |  |  | A |  | A |  |  | A | A |  |
| Ackn of plain text information |  | A | A | A | A | A |  |  | A |  | A |  |  | A | A |  |
| Ackn of level transition |  | A |  | A | A | A |  |  |  | A |  |  |  | A | A |  |
| Ackn of On Sight mode |  | A |  | A | A |  |  |  |  |  | A |  |  |  |  |  |
| Ackn of Shunting mode |  | A | A | A | A | A |  |  |  |  | A |  |  |  |  |  |
| Ackn of Staff Resp. mode |  | A |  | A |  |  |  |  | A |  | A |  |  |  |  |  |
| Ackn of Unfitted mode |  | A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Ackn of Train Trip |  |  |  |  |  |  |  |  |  | A |  |  |  |  |  |  |
| Track Ahead Free |  | A |  |  | A | A |  |  |  |  | A |  |  |  |  |  |
| SR speed/distance |  |  |  |  | A |  |  |  |  |  |  |  |  |  |  |  |

| Output information | N P | S B | S H | F S | S R | O S | S L | N L | U N | T R | P T | S F | I S | S E | S N | R V |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ERTMS/ETCS Mode |  | X | x | X | X | X |  | X | X | X | X | X |  | X | X | X |
| Current ERTMS/ETCS level |  | X | x | X | X | X |  | X | X | X | X |  |  | X | X | X |
| STM mode |  |  |  |  |  |  |  |  |  |  |  |  |  | A | A |  |
| Train Speed |  |  | X | X | X | X |  | X | X | X | X |  |  | X | X | X |
| Permitted Speed |  |  | A | X | A | A |  |  |  |  |  |  |  |  |  | A |
| Target Speed |  |  |  | X |  | A |  |  |  |  |  |  |  | A | A |  |
| Target distance |  |  |  | X | A | A |  |  |  |  |  |  |  | A | A | X |
| Release speed |  |  |  | A |  | A |  |  |  |  |  |  |  |  |  |  |
| Warning |  |  | A | A | A | A |  |  | A |  | A |  |  | A | A | A |
| Trip Alarm |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Isolation status |  | X | X | X | X | X | X | X | X | X | X | X | X | X | X | X |
| Train Data - Max train speed - Train length - international train category - deceleration capacity - time to cut off traction - brake delay time - loading gauge - axle load - power supply - train running number |  | A |  | A | A | A |  |  | A |  | A |  |  | A | A |  |
| Additional data - Driver id - ERTMS/ETCS level table - Radio network-id - RBC-id - RBC phone number - Track Adhesion factor |  | A | A | A | A | A |  | A | A |  | A |  |  | A | A |  |
| Service brake indication |  | A | A | A | A | A |  |  | A |  | A |  |  | A | A | A |
| Emergency brake indication |  | A | A | A | A | A |  |  | A | X | A |  |  | A | A | A |
| Fixed text information |  | A | A | A | A | A |  | A | A | A | A |  |  | A | A | A |
| Plain text information |  | A | A | A | A | A |  | A | A | A | A |  |  | A | A | A |
| Reversing allowed |  |  |  | A |  | A |  |  |  |  |  |  |  |  |  |  |
| Track condition excluding big metal masses - Power control - Pantograph control - Stopping control - Air tightness control - Radio hole control - Brakes control |  | A |  | A | A | A |  | A | A | A | A |  |  |  |  |  |
| Track condition big metal masses |  | A |  | A | A | A |  | A | A | A | A |  |  | A | A |  |
| Geographical position |  | A |  | A | A | A |  | A | A |  | A |  |  |  |  |  |
| Override status |  |  |  | A | A | A |  |  | A |  |  |  |  | A | A |  |
| Shunting request sent |  | A |  | A | A | A |  |  | A |  | A |  |  |  |  |  |
| Shunting refused by RBC |  | A |  | A | A | A |  |  | A |  | A |  |  |  |  |  |
| “Shunting granted” not received |  | A |  | A | A | A |  |  | A |  | A |  |  |  |  |  |
| Reception of an “Emergency Stop” message |  |  |  | A | A | A |  |  |  |  |  |  |  |  |  |  |
| Entry in FS/OS |  |  |  | A |  | A |  |  |  |  |  |  |  |  |  |  |
| Level transition announcement |  |  |  | A | A | A |  | A | A | A | A |  |  | A | A | A |
| Track Ahead Free request |  | A |  |  | A | A |  |  |  |  | A |  |  |  |  |  |
| Adhesion factor changed due to information from trackside |  | A |  | A | A | A |  |  |  |  | A |  |  |  |  |  |

## Acceptance of received information

### INTRODUCTION

#### The aim of this chapter is to give an overview on which information is accepted/rejected depending on the state of the on-board (level, mode) and on the nature of the received information ( transmission medium, type of information: normal or infill).

#### The following sections have to be interpreted by applying the filters as shown in Figure 3. The first filter is detailed in section 4.8.3 “Accepted information depending on the level and transmission media”, the third filter in section 4.8.4 “Accepted information depending on the modes”.

#### If a message contains level transition information, any other information in that message shall be evaluated in accordance with the level transition information.

#### Note : As shown in Figure 3, information stored following an announcement of a change of level, is re-checked for acceptance when the level has changed. This implies that, when the level changes, the mode is - for a short moment – still unchanged, until the stored information has been processed. The consequence for the Third Filter is that information needs to be accepted for this short period also in modes in which this information is otherwise useless.

#### If a message contains in-fill information, this latter shall be evaluated in accordance with all other non-infill information in that message.

Figure 3: schematic representation of the filtering of received information:

### Assumptions

#### The following tables assume that:

- the information complies with the data consistency checks.(see section 3.16)

- the direction for which the information is valid matches the current train orientation, or the balise group crossing direction (for SL and SH engines).(see section 3.6.3)

- In levels 2/3, it is assumed that the “RBC” information which is marked “A” (Accepted) comes from the supervising RBC (see RBC/RBC handover). If this information is received from the “Accepting” RBC while the “Handing Over” RBC is still responsible, it is stored onboard until the RBC transition is performed
exception: the information “Acknowledgement of Train Data”, “Configuration Determination”, "Initiation of Communication Session", "Session Management" and “Acknowledgement of Session Termination" shall be immediately accepted.

### Accepted information depending on the level and transmission media

#### From RBC or not

##### Note: “Not RBC” information has to be understood as balise group, loop or radio infill information, but it does not include information from the STM interface.

A = Accepted	R = Rejected

| Information | From RBC | Onboard operating level |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 0 | STM | 1 | 2 | 3 |
| National Values | No | A | A | A | A | A |
|  | Yes | R [2] | R [2] | R [2] | A | A |
| Linking | No | R [1] | R [1] | A | R [1] | R [1] |
|  | Yes | R [2] | R [2] | R [2] | A [3] | A [3] |
| Signalling Related Speed Restriction | No | R [1] | R [1] | A | R [1] | R [1] |
|  | Yes |  |  |  |  |  |
| Movement Authority | No | R [1] | R [1] | A [4] | R [1] | R [1] |
|  | Yes | R [2] | R [2] | R [2] | A [3] [4] [5] | A [3] [4] [5] |
| Repositioning Information | No | R | R | A | R | R |
|  | Yes |  |  |  |  |  |
| Gradient Profile | No | R [1] | R [1] | A | R [1] | R [1] |
|  | Yes | R [2] | R [2] | R [2] | A [3] | A [3] |
| International SSP | No | R [1] | R [1] | A | R [1] | R [1] |
|  | Yes | R [2] | R [2] | R [2] | A [3] | A [3] |
| Axle Load speed profile | No | R [1] | R [1] | A | R [1] | R [1] |
|  | Yes | R [2] | R [2] | R [2] | A [3] | A [3] |
| Level Transition Order | No | A | A | A | A | A |
|  | Yes | A | A | A | A | A |
| Session Management | No | A | A | A | A | A |
|  | Yes | A | A | A | A | A |
| Radio Network registration | No | A | A | A | A | A |
|  | Yes | A | A | A | A | A |
| List of balises for SH area | No | R [1] | R [1] | A | R [1] | R [1] |
|  | Yes | R [2] | R [2] | R [2] | A [3] | A [3] |
| MA Request Parameters | No |  |  |  |  |  |
|  | Yes | A | A | A | A | A |
| Position Report parameters | No |  |  |  |  |  |
|  | Yes | A | A | A | A | A |
| SR Authorisation | No |  |  |  |  |  |
|  | Yes | R | R | R | A [3] | A [3] |
| Stop if in SR mode | No | R | R | A | A | A |
|  | Yes |  |  |  |  |  |
| Temporary Speed Restriction | No | A | R [1] [2] | A | A | A |
|  | Yes | R [2] | R [2] | R [2] | A [3] | A [3] |
| Temporary Speed Restriction Revocation | No | A | R | A | A | A |
|  | Yes | R [2] | R [2] | R [2] | A [3] | A [3] |
| Default Gradient for TSR | No | A | R [1] [2] | A | A | A |
|  | Yes |  |  |  |  |  |
| Route Suitability Data | No | R [1] | R [1] | A | R [1] | R [1] |
|  | Yes | R [2] | R [2] | R [2] | A [3] | A [3] |
| Adhesion Factor | No | R[1] | R[1] | A | R | R |
|  | Yes | R[2] | R[2] | R[2] | A | A |
| Plain Text Information | No | A | R [1] [2] | A | A | A |
|  | Yes | R [2] | R [2] | R [2] | A | A |
| Fixed Text Information | No | A | R [1] [2] | A | A | A |
|  | Yes | R [2] | R [2] | R [2] | A | A |
| Geographical Position | No | A | R [1] [2] | A | A | A |
|  | Yes | R [2] | R [2] | R [2] | A | A |
| Mode Profile | No | R [1] | R [1] | A [4] | R [1] | R [1] |
|  | Yes | R [2] | R [2] | R [2] | A [3] [4] [5] | A [3] [4] [5] |
| RBC Transition Order | No | R | R | R | A | A |
|  | Yes | R | R | R | A [3] | A [3] |
| Danger for SH information | No | R | R | A | A | A |
|  | Yes |  |  |  |  |  |
| Radio Infill Area information | No | R | R | A | R [1] | R [1] |
|  | Yes |  |  |  |  |  |
| EOLM information | No | R | R | A | R [1] | R [1] |
|  | Yes |  |  |  |  |  |
| Assignment of Co-ordinate system | No |  |  |  |  |  |
|  | Yes | A | A | A | A | A |
| Infill Location Reference | No | R | R | A | R [1] | R [1] |
|  | Yes |  |  |  |  |  |
| Track Conditions excluding big metal masses | No | R [1] | R [1] | A | R [1] | R [1] |
|  | Yes | R [2] | R [2] | R [2] | A [3] | A [3] |
| Track condition big metal masses | No | A | A | A | A | A |
|  | Yes |  |  |  |  |  |
| Location Identity (NID_C + NID_BG transmitted in the balise telegram) | No | A | A | A | A | A |
|  | Yes |  |  |  |  |  |
| Recognition of exit from TRIP mode | No |  |  |  |  |  |
|  | Yes | R | R | R | A | A |
| Acknowledgement of Train Data | No |  |  |  |  |  |
|  | Yes | A | A | A | A | A |
| Co-operative shortening of MA | No |  |  |  |  |  |
|  | Yes | R [2] | R [2] | R [2] | A [3] | A [3] |
| Unconditional Emergency Stop | No |  |  |  |  |  |
|  | Yes | R [2] | R [2] | R [2] | A | A |
| Conditional Emergency Stop | No |  |  |  |  |  |
|  | Yes | R [2] | R [2] | R [2] | A | A |
| Revocation of Emergency Stop | No |  |  |  |  |  |
|  | Yes | R [2] | R [2] | R [2] | A | A |
| SH refused | No |  |  |  |  |  |
|  | Yes | R | R | R | A [3] | A [3] |
| SH authorised | No |  |  |  |  |  |
|  | Yes | R | R | R | A [3] | A [3] |
| Configuration Determination | No | A | A | A | A | A |
|  | Yes | A | A | A | A | A |
| Track Ahead Free Request | No |  |  |  |  |  |
|  | Yes | R | R | R | A [3] | A [3] |
| Train Running Number | No |  |  |  |  |  |
|  | Yes | R | R | R | A | A |
| Initiation of session | No |  |  |  |  |  |
|  | Yes | R | R | R | A | A |
| Acknowledgement of session termination | No |  |  |  |  |  |
|  | Yes | A | A | A | A | A |
| Train Rejected | No |  |  |  |  |  |
|  | Yes | R | R | R | A | A |
| Train Accepted | No |  |  |  |  |  |
|  | Yes | R | R | R | A | A |
| Confirmation of Location by Driver Requested | No |  |  |  |  |  |
|  | Yes | R | R | R | A | A |
| SoM Position Report Confirmed by RBC | No |  |  |  |  |  |
|  | Yes | R | R | R | A | A |
| Reversing Area Information | No | R [1] | R [1] | A | R [1] | R [1] |
|  | Yes | R [2] | R [2] | R [2] | A [3] | A [3] |
| Reversing Supervision Information | No | R [1] | R [1] | A | R [1] | R [1] |
|  | Yes | R [2] | R [2] | R [2] | A [3] | A [3] |
| Default Balise Information | No | A | A | A | A | A |
|  | Yes |  |  |  |  |  |
| Track Ahead Free up to level 2/3 transition location | No | A [9] | A [9] | A [9] | R | R |
|  | Yes |  |  |  |  |  |

[1] exception: stored onboard if an order to switch to L1 at a further location has been  received.

[2] exception: stored onboard if an order to switch to L2/3 at a further location has been received.

[3] exception: rejected if

Train Data has been sent to the RBC and the “Acknowledgement of Train Data” has not been received yet.

[4] exception: the movement authority and, if received together with this movement authority, the mode profile shall be rejected if the distance to EOA/LOA exceeds the track description stored onboard.

[5] exception: the movement authority and, if received together with this movement authority, the mode profile shall be rejected if emergency stop(s) have been received and are not yet revoked or deleted onboard (see mode transitions).

[9] exception: rejected if no level 2/3 transition order is stored onboard.

#### FROM STM X

| Information from STM X | Onboard operating level |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | STM X | STM Y | 1 | 2 | 3 |
| Movement Authority | R [1] | A | R [1] | R [1] | R [1] | R [1] |
| Gradient Profile | R [1] | A | R [1] | R [1] | R [1] | R [1] |
| STM speed restrictions | R [1] | A | R [1] | R [1] | R [1] | R [1] |
| Location Identity (NID_C + NID_BG) | R [1] | A | R [1] | R [1] | R [1] | R [1] |

[1] exception: stored onboard if an order to switch to level STM X at a further location has been received.

#### Information stored onboard in case of level transition announcement shall be re-evaluated in the sequence they have been received once the level transition is performed.

#### Information stored on-board in case of level transition announcement shall be deleted in case the level transition announcement is overwritten by another level transition announcement for a different level.

#### Information received in the same message as an immediate level transition order shall be evaluated first considering the on-board currently operated level, as if  level transition order for further location had been received (i.e. conditions [1] or [2] of Figure 3, if applied, shall be automatically fulfilled). Then, if relevant, it shall be immediately extracted from the buffer and re-evaluated according to the new selected level.

### Accepted Information depending on the modes

#### Assumptions

##### For infill information, only the column FS applies. In all other modes, infill information is rejected.

##### Note: Loop, radio infill and infill balise group information is grouped under the term “infill information”.

#### )Intentionally deleted.

NR = Not Relevant	A = Accepted	R = Rejected

| Information | Modes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NP |  | SB |  |  | SH |  | FS |  |  | SR |  |  | OS | SL | NL | UN | TR | PT |  | SF |  | IS |  | SE |  | SN |  | RV |  |
| National Values | NR |  | A |  |  | A |  | A |  |  | A |  |  | A | A | A | A | A | A [1] |  | NR |  | NR |  | A |  | A |  | A |  |
| Linking | NR |  | A |  |  | R |  | A |  |  | A |  |  | A | R | A | A | R | A [1] |  | NR |  | NR |  | A |  | A |  | R |  |
| Signalling Related Speed Restriction | NR |  | A |  |  | R |  | A |  |  | A |  |  | A | R | R | A | R | A [1] |  | NR |  | NR |  | A |  | A |  | R |  |
| Movement Authority | NR |  | A |  |  | R |  | A |  |  | A |  |  | A | R | R | A | R | A [1] |  | NR |  | NR |  | A |  | A |  | R |  |
| Repositioning Information | NR |  | R |  |  | R |  | A |  |  | R |  |  | A | R | R | R | R | R |  | NR |  | NR |  | R |  | R |  | R |  |
| Gradient Profile | NR |  | A |  |  | R |  | A |  |  | A |  |  | A | R | R | A | R | A [1] |  | NR |  | NR |  | A |  | A |  | R |  |
| International SSP | NR |  | A |  |  | R |  | A |  |  | A |  |  | A | R | R | A | R | A [1] |  | NR |  | NR |  | A |  | A |  | R |  |
| Axle load speed profile | NR |  | A |  |  | R |  | A |  |  | A |  |  | A | R | R | A | R | A [1] |  | NR |  | NR |  | A |  | A |  | R |  |
| STM speed restrictions | NR |  | R |  |  | R |  | R |  |  | R |  |  | R | R | R | R | R | R |  | NR |  | NR |  | A |  | R |  | R |  |
| Level Transition Order | NR |  | A |  |  | R |  | A |  |  | A |  |  | A | A | A | A | A | A [1] |  | NR |  | NR |  | A |  | A |  | R |  |
| Session Management | NR |  | A |  |  | A [3] |  | A |  |  | A |  |  | A | A | A | A | A | A [1] |  | NR |  | NR |  | A |  | A |  | A |  |
| Radio Network registration | NR |  | A |  |  | A |  | A |  |  | A |  |  | A | A | A | A | A | A [1] |  | NR |  | NR |  | A |  | A |  | A |  |
| List of balises for SH area |  | NR |  | A |  |  | R |  | A |  |  | A |  | A | R | R | A | R |  | A [1] |  | NR |  | NR |  | A |  | A |  | R |
| MA Request Parameters |  | NR |  | A |  |  | R |  | A |  |  | A |  | A | R | R | A | R |  | A [1] |  | NR |  | NR |  | A |  | A |  | R |
| Position Report parameters |  | NR |  | A |  |  | R |  | A |  |  | A |  | A | R | A | A | R |  | A [1] |  | NR |  | NR |  | A |  | A |  | R |
| SR Authorisation |  | NR |  | A |  |  | R |  | R |  |  | A |  | R | R | R | R | R |  | A [1] |  | NR |  | NR |  | R |  | R |  | R |
| Stop if in SR mode |  | NR |  | R |  |  | R |  | R |  |  | A |  | R | R | R | R | R |  | R |  | NR |  | NR |  | R |  | R |  | R |
| Temporary Speed Restriction |  | NR |  | A |  |  | R |  | A |  |  | A |  | A | R | R | A | A |  | A [1] |  | NR |  | NR |  | R |  | R |  | R |
| Temporary Speed Restriction Revocation |  | NR |  | A |  |  | R |  | A |  |  | A |  | A | R | R | A | A |  | A [1] |  | NR |  | NR |  | R |  | R |  | R |
| Default Gradient for TSR |  | NR |  | A |  |  | R |  | R |  |  | A |  | R | R | R | A | A |  | A [1] |  | NR |  | NR |  | R |  | R |  | R |
| Route Suitability Data |  | NR |  | A |  |  | R |  | A |  |  | A |  | A | R | R | A | R |  | A [1] |  | NR |  | NR |  | A |  | A |  | R |
| Adhesion Factor |  | NR |  | A |  |  | R |  | A |  |  | A |  | A | R | R | A | R |  | A [1] |  | NR |  | NR |  | A |  | A |  | R |
| Plain Text Information |  | NR |  | A |  |  | R |  | A |  |  | A |  | A | R | A | A | R |  | A [1] |  | NR |  | NR |  | R |  | R |  | A |
| Fixed Text Information |  | NR |  | A |  |  | R |  | A |  |  | A |  | A | R | A | A | R |  | A [1] |  | NR |  | NR |  | R |  | R |  | A |
| Geographical Position |  | NR |  | A |  |  | R |  | A |  |  | A |  | A | R | A | A | A |  | A [1] |  | NR |  | NR |  | R |  | R |  | R |
| Mode Profile |  | NR |  | A |  |  | R |  | A |  |  | A |  | A | R | R | A | R |  | A [1] |  | NR |  | NR |  | A |  | A |  | R |
| RBC Transition Order | NR |  | A |  |  | R |  | A |  |  | A |  |  | A | A | A | R | A | A [1] |  | NR |  | NR |  | R |  | R |  | R |  |
| Danger for SH information | NR |  | R |  |  | A |  | R |  |  | R |  |  | R | R | R | R | R | R |  | NR |  | NR |  | R |  | R |  | R |  |
| Radio Infill Area information | NR |  | R |  |  | R |  | A |  |  | A |  |  | A | R | R | A | R | R |  | NR |  | NR |  | A |  | A |  | R |  |
| EOLM information | NR |  | R |  |  | R |  | A |  |  | A |  |  | A | R | R | A | R | R |  | NR |  | NR |  | A |  | A |  | R |  |
| Assignment of Co-ordinate system | NR |  | A |  |  | R |  | R |  |  | A |  |  | R | R | A | A | R | A [1] |  | NR |  | NR |  | A |  | A |  | R |  |
| Infill Location Reference | NR |  | R |  |  | R |  | A |  |  | R |  |  | R | R | R | R | R | R |  | NR |  | NR |  | R |  | R |  | R |  |
| Track Conditions excluding big metal masses | NR |  | A |  |  | R |  | A |  |  | A |  |  | A | R | A | A | R | A [1] |  | NR |  | NR |  | A |  | A |  | R |  |
| Track condition big metal masses | NR |  | A |  |  | A |  | A |  |  | A |  |  | A | A | A | A | A | A [1] |  | NR |  | NR |  | A |  | A |  | A |  |
| Location Identity (NID_C + NID_BG) | NR |  | A |  |  | A |  | A |  |  | A |  |  | A | A | A | A | A | A |  | NR |  | NR |  | A |  | A |  | A |  |
| Recognition of exit from TRIP mode | NR |  | R |  |  | R |  | R |  |  | R |  |  | R | R | R | R | R | A |  | NR |  | NR |  | R |  | R |  | R |  |
| Acknowledgement of Train Data | NR |  | A |  |  | R |  | A |  |  | A |  |  | A | R | R | A | R | R |  | NR |  | NR |  | A |  | A |  | R |  |
| Co-operative shortening of MA | NR |  | R |  |  | R |  | A |  |  | R |  |  | A | R | R | A | R | R |  | NR |  | NR |  | A |  | A |  | R |  |
| Unconditional Emergency Stop | NR |  | A |  |  | R |  | A |  |  | A |  |  | A | R | R | A | R | R |  | NR |  | NR |  | A |  | A |  | R |  |
| Conditional Emergency Stop | NR |  | R |  |  | R |  | A |  |  | R |  |  | A | R | R | A | R | R |  | NR |  | NR |  | A |  | A |  | R |  |
| Revocation of Emergency Stop (Space Critical) | NR |  | R |  |  | R |  | A |  |  | A |  |  | A | R | R | A | R | A [1] |  | NR |  | NR |  | A |  | A |  | R |  |
| SH refused | NR |  | A |  |  | R |  | A |  |  | A |  |  | A | R | R | R | R | A [1] |  | NR |  | NR |  | R |  | R |  | R |  |
| SH authorised | NR |  | A |  |  | R |  | A |  |  | A |  |  | A | R | R | R | R | A [1] |  | NR |  | NR |  | R |  | R |  | R |  |
| Configuration Determination | NR |  | A |  |  | A |  | A |  |  | A |  |  | A | A | A | A | A | A |  | NR |  | NR |  | A |  | A |  | A |  |
| Track Ahead Free Request | NR |  | A |  |  | R |  | R |  |  | A |  |  | A | R | R | R | R | A[1] |  | NR |  | NR |  | R |  | R |  | R |  |
| Train Running Number | NR |  | A |  |  | R |  | A |  |  | A |  |  | A | R | A | R | A | A |  | NR |  | NR |  | R |  | R |  | A |  |
| Initiation of session | NR |  | A |  |  | R |  | A |  |  | A |  |  | A | A | A | R | A | A |  | NR |  | NR |  | R |  | R |  | A |  |
| Acknowledgement of session termination | NR |  | A |  |  | A |  | A |  |  | A |  |  | A | A | A | A | A | A |  | NR |  | NR |  | A |  | A |  | A |  |
| Train Rejected | NR |  | A |  |  | R |  | R |  |  | R |  |  | R | R | R | R | R | R |  | NR |  | NR |  | R |  | R |  | R |  |
| Train Accepted | NR |  | A |  |  | R |  | R |  |  | R |  |  | R | R | R | R | R | R |  | NR |  | NR |  | R |  | R |  | R |  |
| Confirmation of Location by Driver Requested | NR |  |  |  | A | R |  |  |  | R |  |  | R | R | R | R | R | R | R |  | NR |  | NR |  | R |  | R |  | R |  |
| SoM Position Report Confirmed by RBC | NR |  | A |  |  | R |  | R |  |  | R |  |  | R | R | R | R | R | R |  | NR |  | NR |  | R |  | R |  | R |  |
| Reversing Area Information | NR |  | A |  |  | R |  | A |  |  | A |  |  | A | R | R | A | R | A [1] |  | NR |  | NR |  | A |  | A |  | R |  |
| Reversing Supervision Information | NR |  | A |  |  | R |  | A |  |  | A |  |  | A | R | R | A | R | A [1] |  | NR |  | NR |  | A |  | A |  | A |  |
| Default Balise Information | NR |  | A |  |  | A |  | A |  |  | A |  |  | A | A | A | A | A | A |  | NR |  | NR |  | A |  | A |  | A |  |
| Track Ahead Free up to level 2/3 transition location | NR |  | A |  |  | R |  | A |  |  | A |  |  | A | R | R | A | A | A |  | NR |  | NR |  | A |  | A |  | R |  |

[1]: only if following the reception of the information “Recognition of Exit from TR mode” with a more recent time stamp

[3]: only disconnection order

## What happens to accepted and stored information when entering a given level

### Introduction

#### Every data that can be stored onboard after being accepted may be influenced by a level transition.

#### A level transition acts on the “status” of stored information.

#### In case of entering level 1, MA Request Parameters, Position Report Parameters and Track Ahead Free Request shall be deleted .

#### For all other stored data, a level transition has no effect (void).

## What happens to accepted and stored information when entering a given mode

### Introduction

#### Every data that can be stored onboard after being accepted may be influenced by a mode transition.

#### A mode transition acts on the “status” of stored information.

#### Depending on which mode is entered, the action can be:

- data is deleted,

- data is to be revalidated,

- data is reset (set to default values)

- data status is unchanged,

- not relevant (the action on the data cannot be determined. This concerns the entry in SF and IS modes)

| D = Deleted | TBR = To Be Revalidated | U = Unchanged | NR = Not relevant | R = Reset |
| --- | --- | --- | --- | --- |

| Data Stored on-board | Entered Mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NP | SB | SH | FS | SR | OS | SL | NL | UN | TR | PT | SF | IS | SE | SN | RV |
| National Values | U | U | U | U | U | U | U | U | U | U | U | NR | NR | U | U | U |
| Linking | D | D | D | U | D | U | D | D | D | D | U | NR | NR | D | D | D |
| Movement Authority | D | D | D | U | D | U | D | D | D | D | U | NR | NR | U | D | D |
| Gradient Profile | D | D | D | U | D | U | D | D | D | D | U | NR | NR | U | D | D |
| International SSP | D | D | D | U | D | U | D | D | D | D | U | NR | NR | U | D | D |
| Axle load speed profile | D | D | D | U | D | U | D | D | D | D | U | NR | NR | U | D | D |
| STM speed restrictions | D | D | D | D | D | D | D | D | D | D | U | NR | NR | U | D | D |
| Level Transition Order | D | D | D | U | D | U | D | D | D | U | U | NR | NR | D | D | D |
| List of balises for SH area | D | D | U | U | D | U | D | D | D | D | U | NR | NR | D | D | D |
| MA Request Parameters | D | U | U | U | U | U | U | U | U | U | U | NR | NR | U | U | U |
| Position Report parameters | D | U | U | U | U | U | U | U | U | U | U | NR | NR | U | U | U |
| List of Balises in SR Authority + SR speed and distance | D | D | D | D | U | D | D | D | D | D | U | NR | NR | D | D | D |
| Temporary Speed Restriction | D | D | D | U | U | U | D | D | U | U | U | NR | NR | D | D | D |
| Default Gradient for TSR | D | D | D | D | U | D | D | D | U | U | U | NR | NR | D | D | D |
| Signalling related Speed Restriction | D | D | D | U | D | U | D | D | D | D | U | NR | NR | D | D | D |
| Route Suitability Data | D | D | D | U | D | U | D | D | D | D | U | NR | NR | D | D | D |
| Adhesion Factor | D | D | D | U | U | U | D | D | U | U | U | NR | NR | D | D | U |
| Plain Text Information | D | D | D | U | U | U | D | U | U | U | U | NR | NR | D | D | U |
| Fixed Text Information | D | D | D | U | U | U | D | U | U | U | U | NR | NR | D | D | U |
| Geographical Position | D | U | U | U | U | U | U | U | U | U | U | NR | NR | D | D | D |
| Mode Profile | D | D | D | U | D | U | D | D | D | D | U | NR | NR | D | D | D |
| RBC Transition Order | D | D | D | U | D | U | D | D | D | D | U | NR | NR | D | D | D |
| Radio Infill Area information | D | D | D | U | D | D | D | D | D | D | U | NR | NR | D | D | D |
| EOLM information | D | D | D | U | D | D | D | D | D | D | U | NR | NR | D | D | D |
| Track Conditions excluding big metal masses | R | R | R | U | R | U | R | U | R | U | U | NR | NR | R | R | R |
| Track condition big metal masses | R | R | R | U | R | U | R | U | U | U | U | NR | NR | U | U | R |
| Unconditional Emergency Stops | D | D | D | U | D | U | D | D | D | U | U | NR | NR | U | U | D |
| Conditional Emergency Stops | D | D | D | U | D | U | D | D | D | U | U | NR | NR | U | U | D |
| Location | TBR | U | U | U | U | U | U | U | U | U | U | NR | NR | U | U | U |
| Train Data (except Train Running Number) | D | TBR | D | U | U | U | U | U | U | U | U | NR | NR | U | U | U |
| ERTMS/ETCS level | TBR | U | U | U | U | U | U | U | U | U | U | NR | NR | U | U | U |
| Driver ID | D | TBR | U | U | U | U | D | U | U | U | U | NR | NR | U | U | U |
| Radio Network ID | U | U | U | U | U | U | U | U | U | U | U | NR | NR | U | U | U |
| RBC ID/Phone Number | TBR | U | U | U | U | U | U | U | D | U | U | NR | NR | D | D | U |
| Train Running Number | D | TBR | U | U | U | U | D | U | U | U | U | NR | NR | U | U | U |
| Last eight reported balise group identities | U | U | U | U | U | U | U | U | U | U | U | NR | NR | U | U | U |
| Reversing Area Information | D | D | D | U | D | U | D | D | D | D | U | NR | NR | D | D | U |
| Reversing Supervision Information | D | D | D | U | D | U | D | D | D | D | U | NR | NR | D | D | U |
| Confirmation of Location by Driver Requested | D | D | D | D | D | D | D | D | D | D | U | NR | NR | D | D | D |
| Track Ahead Free Request | D | D | D | D | U | U | D | D | D | D | U | NR | NR | D | D | D |

#### NOTES:

##### “Location” contains LRBG, distance travelled from LRBG, position of the front end in relation to the LRBG, the confidence interval and the train orientation in relation to the LRBG.

##### The following information is not considered to be stored information:

- Repositioning information

- Session Management

- Danger for SH information

- Assignment of Co-ordinate system

- Infill Location Reference

- Location Identity (NID_C + NID_BG transmitted in the balise telegram)

- Recognition of exit from TRIP mode

- Acknowledgement of Train Data

- SH refused

- SH authorised

- Configuration Determination

- Intentionally deleted

- Intentionally deleted

- Revocation of Emergency Stop (Space Critical)

- Temporary Speed Restriction Revocation

- Initiation of session

- Acknowledgement of session termination

- Default Balise Information

- Co-operative shortening of MA (if this message is used, it replaces the movement authority)

- Train Rejected

- Train Accepted

- SoM position report confirmed by RBC

- Track Ahead Free up to level 2/3 transition location

For UN,SE and SN mode, conditions for re-activation of transition to Trip mode (see § 5.8.4.1a) & b)) shall be supervised.