| - ERTMS/ETCS – Class 1 |  |
| --- | --- |
| System Requirements Specification Chapter 7 ERTMS/ETCS language |  |
| REF : | SUBSET-026-7 |
| ISSUE : | 2.3.0 |
| DATE : | 24/02/06 |

| Company | Technical Approval | Management approval |
| --- | --- | --- |
| ALCATEL |  |  |
| ALSTOM |  |  |
| ANSALDO SIGNAL |  |  |
| BOMBARDIER |  |  |
| INVENSYS RAIL |  |  |
| SIEMENS |  |  |

## Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| 0.0.1 990422 | All | Creation of document | OG/DD |
| 0.0.2 990423 | All | Changed according to Siemens comments | OG/DD |
| 1.1.0 990423 | All | Class P Official Issue | OG/DD |
| 1.1.1 990525 | All | Add review comments UNISIG_All_COM_006_7.doc | BRO |
| 1.1.2 | All | Some minor corrections | SAB |
| 1.1.3 | All | First draft for class 1 | SAB |
| 1.1.4 | All | Update according to review comments | SAB |
| 1.1.5 | All | Some minor modifications (see revision marks) + Addition of length of variables in the packets. | OG |
| 1.1.6 | Version number and editorial changes. | Finalisation meeting in Stuttgart 990729 | HE |
| 1.2.0 990730 | Version Number | Release Version | HE |
| 1.2.1 991209 | All | Changes according to WPs for SRS upgrade + editorial changes due to ECSAG / UNISIG agreed questions | OG |
| 1.3.0 991217 | All | Changed according to review comments | OG |
| 2.0.0 991222 | Minor editorial changes | Release version | OG |
| 2.0.1 000926 | All | Corrections after UNISIG review 15 June 00 | OG |
| 2.1.0 001012 | All | Corrections according to "unisig_all_com_SRS_ 2.0.1" document | OG |
| 2.2.0 | Packet 71 deleted, NID_C 10 bit | UNISIG release | SAB |
| 2.2.2 | see revision marks | Corrections according to SUBSET-026 Corrected Paragraphs, issue 2.2.2. Mainly, Packet 71 and Q_NVDRIVER_ADHES added | OG |
| 2.2.4 24.05.2004 |  | Update after cross checking and comments from Alain | B. Stamm |
| 2.2.4 SG checked 28/05/04 | Including all CLRs agreed with the EEIG (see “List of CLRs agreed with EEIG for SRS v2.2.4” dated 28/05/04) Affected clauses see change marks |  | H. Kast |
| 2.2.5 04/01/05 | Incorporation of solution proposal for CLR 007 with EEIG users group comments |  | A. Hougardy |
| 2.2.6 | Incorporation of all CRs and CLRs submitted to the EEIG until 21.01.2005 |  | B. Stamm |
| 2.2.7 | Incorporation of all CRs and CLRs extracted from "CR-Report_10.6.05-by number.rtf" and mentioned in column 2.2.7 in "CR status 13.6.05 _rmk_chap_3_4_220605.xls" |  | B. Stamm |
| 2.2.8 | Change marks cleaned up and updated according to last CRs decisions (including split of CRs7&126) |  | J. Liesche |
| 2.2.9 24/02/06 | Including all CRs that are classified as "IN" as per SUBSET-108 version 1.0.0 Removal of all CRs that are not classified as "IN" as per SUBSET-108 version 1.0.0, with the exception of CRs 63,98,120,158,538 |  | J. Liesche |
| 2.3.0 24/02/06 | Release version |  | HK |

## Table of Contents

7.1	Modification History	2

7.2	Table of Contents	4

7.3	Components of ERTMS/ETCS Language	5

7.3.1	Introduction	5

7.3.2	Definition of Variables	5

7.3.3	Definition of Packets	6

7.4	PACKETS	8

7.4.1	List of Packets	8

7.4.2	PACKETS: TRACK TO TRAIN	10

7.4.3	PACKETS: TRAIN TO TRACK	30

7.4.4	PACKETS: TRACK TO TRAIN or TRAIN TO TRACK	34

7.5	Definitions of Variables	35

## Components of ERTMS/ETCS Language

### Introduction

#### The ERTMS/ETCS language is used in transmitting information over the radio, balise and loop airgaps and the STM interface.

#### The ERTMS/ETCS language is based on variables, packets, messages and telegrams (variables and packets are described in this section, while telegrams and messages are described in chapter 8).

#### Note: A number of variables contain values which have to be assigned. Some of these values have to be unique to ensure that the system functions properly. A centralised handling of this assignment is therefore required (nationally or internationally, depending on the variable). The variables concerned have been marked. The values included in this document for these variables are therefore not to be used without prior verification of their validity.

### Definition of Variables

#### Variables shall be used to encode single data values. Variables cannot be split in minor units. The whole variable has one type (meaning).

#### Variables may have special values which are related to the basic meaning of the variable.

#### Special values have always the highest values in a variable (eg. 11...111 = "unknown").

#### Spare values shall be located between the normal and special values in the variable range

#### Names of variables are unique. A variable is used in context with the meaning as described in the variable definition. Variables with different meanings have different names.

#### All variable definitions shall be independent of the transport media over which they are used, if used in more than one media.

#### Signed values shall be encoded as 2’s complement.

#### One bit variables (Boolean) shall always use 0 for false and 1 for true.

#### Offsets for numerical values shall be avoided (0 shall be used for 0, 1 for 1, etc.) except where justified.

#### When transmitting over the different transmission media, the most significant bit must be transmitted first.

#### All Variables have one of the following prefixes:

| A_ | Acceleration |
| --- | --- |
| D_ | distance |
| G_ | Gradient |
| L_ | length |
| M_ | Miscellaneous |
| N_ | Number |
| NC_ | class number |
| NID_ | identity number |
| Q_ | Qualifier |
| T_ | time/date |
| V_ | Speed |
| X_ | Text |

### Definition of Packets

#### Packets are multiple variables grouped into a single unit, with a defined internal structure.

#### This structure consists of a packet header with:

- Track to Train: a unique packet number, the length of the packet in bits, the orientation information, optionally the distance scale and an information section containing a defined set of variables. The packet structure is as follows:

| Number | NID_PACKET | Packet identifier |
| --- | --- | --- |
| Direction | Q_DIR | Specifies for which running direction the information is valid |
| Length | L_PACKET | Number of bits in the packet |
| Scale | Q_SCALE | Specifies which distance scale is used for all distance information within the packet. There is no Q_SCALE variable in packets which do not contain distance information. |
| Information | ...... | Well defined set(s) of variables. |

- Train to Track: a unique packet number, the length of the packet in bits, optionally the distance scale and an information section containing a defined set of variables. The packet structure is as follows:

| Number | NID_PACKET | Packet identifier |
| --- | --- | --- |
| Length | L_PACKET | Number of bits in the packet |
| Scale | Q_SCALE | Specifies which distance scale is used for all distance information within the packet. There is no Q_SCALE variable in packets which do not contain distance information. |
| Information | ...... | Well defined set(s) of variables. |

#### The packet definition does not change when transmitted over different transmission media.

#### All currently not defined packet identifiers are reserved for future use. All future packet definitions shall follow the above defined structure.

#### Exception: Packet 255: “End of Telegram” does not follow the above rule.

#### N_ITER specifies the number of iterations of a variable or group of variables which follow.

#### If N_ITER is 0 then no variables follow.

#### Two nested levels of iterations can exist.

#### If, depending on the value of a previous qualifier variable in the packet, a variable is optional, it is written indented in the packet definition

## PACKETS

### List of Packets

#### Track to Train

| Packet Number | Packet Name | Page N° |
| --- | --- | --- |
| 3 | National Values | 10 |
| 5 | Linking | 11 |
| 12 | Level 1 Movement Authority | 11 |
| 15 | Level 2/3 Movement Authority | 12 |
| 16 | Repositioning Information | 13 |
| 21 | Gradient Profile | 13 |
| 27 | International Static Speed Profile | 14 |
| 39 | Track Condition Change of traction power | 15 |
| 41 | Level Transition Order | 15 |
| 42 | Session Management | 15 |
| 44 | Data used by applications outside the ERTMS/ETCS system. | 16 |
| 45 | Radio Network registration | 16 |
| 46 | Conditional Level Transition Order | 16 |
| 49 | List of balises for SH Area | 16 |
| 51 | Axle load Speed Profile | 17 |
| 57 | Movement Authority Request Parameters | 18 |
| 58 | Position Report Parameters | 18 |
| 63 | List of Balises in SR Authority | 19 |
| 65 | Temporary Speed Restriction | 19 |
| 66 | Temporary Speed Restriction Revocation | 20 |
| 67 | Track Condition Big Metal Masses | 20 |
| 68 | Track Condition | 21 |
| 70 | Route Suitability Data | 21 |
| 71 | Adhesion Factor | 22 |
| 72 | Packet for sending plain text messages | 22 |
| 76 | Packet for sending fixed text messages | 23 |
| 79 | Geographical Position Information | 24 |
| 80 | Mode profile | 24 |
| 90 | Track Ahead Free up to level 2/3 transition location | 25 |
| 131 | RBC transition order | 25 |
| 132 | Danger for Shunting information | 26 |
| 133 | Radio in-fill area information | 26 |
| 134 | EOLM Packet | 27 |
| 136 | Infill location reference | 27 |
| 137 | Stop if in Staff Responsible | 28 |
| 138 | Reversing area information | 28 |
| 139 | Reversing supervision information | 28 |
| 140 | Train running number from RBC | 29 |
| 141 | Default Gradient for Temporary Speed Restriction | 29 |
| 254 | Default balise, loop or RIU information | 29 |

#### Train to Track

| Packet Number | Packet Name | Page N° |
| --- | --- | --- |
| 0 | Position Report | 30 |
| 1 | Position Report based on two balise groups | 30 |
| 3 | Onboard telephone numbers | 31 |
| 4 | Error Reporting | 32 |
| 9 | Level 2/3 transition information | 32 |
| 11 | Validated train data | 32 |
| 44 | Data used by applications outside the ERTMS/ETCS system. | 33 |

#### Track to Train or Train to Track

| Packet Number | Packet Name | Page N° |
| --- | --- | --- |
| 255 | End of information | 34 |

### PACKETS: TRACK TO TRAIN

#### Packet Number 3: National Values

| Description | Downloads a set of National Values to the train |  |  |
| --- | --- | --- | --- |
| Transmission media | Any |  |  |
| Content | Variable | Length | Comment |
|  | NID_PACKET | 8 |  |
|  | Q_DIR | 2 |  |
|  | L_PACKET | 13 |  |
|  | Q_SCALE | 2 |  |
|  | D_VALIDNV | 15 |  |
|  | N_ITER | 5 |  |
|  | NID_C(k) | 10 | Identification of national area(s) to which the set applies |
|  | V_NVSHUNT | 7 |  |
|  | V_NVSTFF | 7 |  |
|  | V_NVONSIGHT | 7 |  |
|  | V_NVUNFIT | 7 |  |
|  | V_NVREL | 7 |  |
|  | D_NVROLL | 15 |  |
|  | Q_NVSRBKTRG | 1 |  |
|  | Q_NVEMRRLS | 1 |  |
|  | V_NVALLOWOVTRP | 7 |  |
|  | V_NVSUPOVTRP | 7 |  |
|  | D_NVOVTRP | 15 |  |
|  | T_NVOVTRP | 8 |  |
|  | D_NVPOTRP | 15 |  |
|  | M_NVCONTACT | 2 |  |
|  | T_NVCONTACT | 8 |  |
|  | M_NVDERUN | 1 |  |
|  | D_NVSTFF | 15 |  |
|  | Q_NVDRIVER_ADHES | 1 |  |

#### Packet Number 5: Linking

| Description | Linking Information. |  |  |
| --- | --- | --- | --- |
| Transmission media | Any |  |  |
| Content | Variable | Length | Comment |
|  | NID_PACKET | 8 |  |
|  | Q_DIR | 2 |  |
|  | L_PACKET | 13 |  |
|  | Q_SCALE | 2 |  |
|  | D_LINK | 15 |  |
|  | Q_NEWCOUNTRY | 1 |  |
|  | NID_C | 10 | if Q_NEWCOUNTRY = 1 |
|  | NID_BG | 14 |  |
|  | Q_LINKORIENTATION | 1 |  |
|  | Q_LINKREACTION | 2 |  |
|  | Q_LOCACC | 6 |  |
|  | N_ITER | 5 |  |
|  | D_LINK (k) | 15 |  |
|  | Q_NEWCOUNTRY(k) | 1 |  |
|  | NID_C (k) | 10 | if Q_NEWCOUNTRY = 1 |
|  | NID_BG (k) | 14 |  |
|  | Q_LINKORIENTATION (k) | 1 |  |
|  | Q_LINKREACTION (k) | 2 |  |
|  | Q_LOCACC (k) | 6 |  |

#### Packet Number 12: Level 1 Movement Authority

| Description | Transmission of a movement authority for level 1. |  |  |
| --- | --- | --- | --- |
| Transmission media | Balise |  |  |
| Content | Variable | Length | Comment |
|  | NID_PACKET | 8 |  |
|  | Q_DIR | 2 |  |
|  | L_PACKET | 13 |  |
|  | Q_SCALE | 2 |  |
|  | V_MAIN | 7 |  |
|  | V_LOA | 7 |  |
|  | T_LOA | 10 | Can be set to “no time out” |
|  | N_ITER | 5 | Set to zero if V_MAIN = 0 or if only one section in the MA |
|  | L_SECTION(k) | 15 |  |
|  | Q_SECTIONTIMER(k) | 1 |  |
|  | T_SECTIONTIMER(k) | 10 |  |
|  | D_SECTIONTIMERSTOPLOC(k) | 15 |  |
|  | L_ENDSECTION | 15 |  |
|  | Q_SECTIONTIMER | 1 |  |
|  | T_SECTIONTIMER | 10 |  |
|  | D_SECTIONTIMERSTOPLOC | 15 |  |
|  | Q_ENDTIMER | 1 |  |
|  | T_ENDTIMER | 10 |  |
|  | D_ENDTIMERSTARTLOC | 15 |  |
|  | Q_DANGERPOINT | 1 |  |
|  | D_DP | 15 |  |
|  | V_RELEASEDP | 7 |  |
|  | Q_OVERLAP | 1 |  |
|  | D_STARTOL | 15 |  |
|  | T_OL | 10 |  |
|  | D_OL | 15 |  |
|  | V_RELEASEOL | 7 |  |

#### Packet Number 15: Level 2/3 Movement Authority

| Description | Transmission of a movement authority for levels 2/3. |  |  |
| --- | --- | --- | --- |
| Transmission media | Radio |  |  |
| Content | Variable | Length | Comment |
|  | NID_PACKET | 8 |  |
|  | Q_DIR | 2 |  |
|  | L_PACKET | 13 |  |
|  | Q_SCALE | 2 |  |
|  | V_LOA | 7 |  |
|  | T_LOA | 10 | Can be set to “no time out” |
|  | N_ITER | 5 | Set to zero if only one section in the MA |
|  | L_SECTION(k) | 15 |  |
|  | Q_SECTIONTIMER(k) | 1 |  |
|  | T_SECTIONTIMER(k) | 10 |  |
|  | D_SECTIONTIMERSTOPLOC(k) | 15 |  |
|  | L_ENDSECTION | 15 |  |
|  | Q_SECTIONTIMER | 1 |  |
|  | T_SECTIONTIMER | 10 |  |
|  | D_SECTIONTIMERSTOPLOC | 15 |  |
|  | Q_ENDTIMER | 1 |  |
|  | T_ENDTIMER | 10 |  |
|  | D_ENDTIMERSTARTLOC | 15 |  |
|  | Q_DANGERPOINT | 1 |  |
|  | D_DP | 15 |  |
|  | V_RELEASEDP | 7 |  |
|  | Q_OVERLAP | 1 |  |
|  | D_STARTOL | 15 |  |
|  | T_OL | 10 |  |
|  | D_OL | 15 |  |
|  | V_RELEASEOL | 7 |  |

#### Packet Number 16: Repositioning Information

| Description | Transmission of the update of the current distance |  |  |
| --- | --- | --- | --- |
| Transmission media | Balise |  |  |
| Content | Variable | Length | Comment |
|  | NID_PACKET | 8 |  |
|  | Q_DIR | 2 |  |
|  | L_PACKET | 13 |  |
|  | Q_SCALE | 2 |  |
|  | L_SECTION | 15 |  |

#### Packet Number 21: Gradient Profile

| Description | Transmission of the gradient. D_GRADIENT gives the distance to the next change of the gradient value. The gradient value is the minimum gradient for the given distance. |  |  |
| --- | --- | --- | --- |
| Transmission media | Any |  |  |
| Content | Variable | Length | Comment |
|  | NID_PACKET | 8 |  |
|  | Q_DIR | 2 |  |
|  | L_PACKET | 13 |  |
|  | Q_SCALE | 2 |  |
|  | D_GRADIENT | 15 |  |
|  | Q_GDIR | 1 | 0 = downhill 1= uphill |
|  | G_A | 8 |  |
|  | N_ITER | 5 |  |
|  | D_GRADIENT(k) | 15 |  |
|  | Q_GDIR(k) | 1 | 0 = downhill 1= uphill |
|  | G_A(k) | 8 |  |

#### Packet Number 27: International Static Speed Profile

| Description | Static speed profile and optionally speed limits depending on the international train category. |  |  |
| --- | --- | --- | --- |
| Transmission media | Any |  |  |
| Content | Variable | Length | Comment |
|  | NID_PACKET | 8 |  |
|  | Q_DIR | 2 |  |
|  | L_PACKET | 13 |  |
|  | Q_SCALE | 2 |  |
|  | D_STATIC | 15 |  |
|  | V_STATIC | 7 |  |
|  | Q_FRONT | 1 |  |
|  | N_ITER | 5 |  |
|  | NC_DIFF(n) | 4 |  |
|  | V_DIFF(n) | 7 |  |
|  | N_ITER | 5 |  |
|  | D_STATIC(k) | 15 |  |
|  | V_STATIC(k) | 7 |  |
|  | Q_FRONT(k) | 1 |  |
|  | N_ITER(k) | 5 |  |
|  | NC_DIFF(k,m) | 4 |  |
|  | V_DIFF(k,m) | 7 |  |

#### Packet Number 39: Track Condition Change of traction power

| Description | The packet gives information about change of the traction power system. |  |  |
| --- | --- | --- | --- |
| Transmission media | Any |  |  |
| Content | Variable | Length | Comment |
|  | NID_PACKET | 8 |  |
|  | Q_DIR | 2 |  |
|  | L_PACKET | 13 |  |
|  | Q_SCALE | 2 |  |
|  | D_TRACTION | 15 |  |
|  | M_TRACTION | 8 | Type of traction. |

#### Packet Number 41: Level Transition Order

| Description | Packet to identify where a level transition shall take place. In case of mixed levels, the successive M_LEVELTR’s go from the highest priority level to the lowest one. |  |  |
| --- | --- | --- | --- |
| Transmission media | Any |  |  |
| Content | Variable | Length | Comment |
|  | NID_PACKET | 8 |  |
|  | Q_DIR | 2 |  |
|  | L_PACKET | 13 |  |
|  | Q_SCALE | 2 |  |
|  | D_LEVELTR | 15 |  |
|  | M_LEVELTR | 3 |  |
|  | NID_STM | 8 | If M_LEVELTR = 1 (STM) |
|  | L_ACKLEVELTR | 15 |  |
|  | N_ITER | 5 |  |
|  | M_LEVELTR(k) | 3 |  |
|  | NID_STM(k) | 8 | If M_LEVELTR = 1 (STM) |
|  | L_ACKLEVELTR(k) | 15 |  |

#### Packet Number 42: Session Management

| Description | Packet to give the identity and telephone number of the RBC with which a session shall be established or terminated. |  |  |
| --- | --- | --- | --- |
| Transmission media | Any |  |  |
| Content | Variable | Length | Comment |
|  | NID_PACKET | 8 |  |
|  | Q_DIR | 2 |  |
|  | L_PACKET | 13 |  |
|  | Q_RBC | 1 |  |
|  | NID_C | 10 | RBC ETCS identity : NID_C not relevant if NID_RBC has value unknown |
|  | NID_RBC | 14 |  |
|  | NID_RADIO | 64 | not relevant if NID_RBC has value unknown |
|  | Q_SLEEPSESSION | 1 |  |

#### Packet Number 44: Data used by applications outside the ERTMS/ETCS system.

| Description | Messages between trackside and on-board devices, which contain information used by applications outside the ERTMS/ETCS system. |  |  |
| --- | --- | --- | --- |
| Transmission media | Any |  |  |
| Content | Variable | Length | Comment |
|  | NID_PACKET | 8 |  |
|  | Q_DIR | 2 |  |
|  | L_PACKET | 13 |  |
|  | NID_XUSER | 9 |  |
|  | Other data, depending on NID_XUSER |  |  |

##### Packet Number 45: Radio Network registration

| Description | Packet to give the identity of the Radio Network to which a registration shall be enforced. |  |  |
| --- | --- | --- | --- |
| Transmission media | Any |  |  |
| Content | Variable | Length | Comment |
|  | NID_PACKET | 8 |  |
|  | Q_DIR | 2 |  |
|  | L_PACKET | 13 |  |
|  | NID_MN | 24 |  |

##### Packet Number 46: Conditional Level Transition Order

| Description | Packet for a conditional level transition. The successive M_LEVELTR’s go from the highest priority level to the lowest one. |  |  |
| --- | --- | --- | --- |
| Transmission media | Any |  |  |
| Content | Variable | Length | Comment |
|  | NID_PACKET | 8 |  |
|  | Q_DIR | 2 |  |
|  | L_PACKET | 13 |  |
|  | M_LEVELTR | 3 |  |
|  | NID_STM | 8 | If M_LEVELTR = 1 (STM) |
|  | N_ITER | 5 |  |
|  | M_LEVELTR(k) | 3 |  |
|  | NID_STM(k) | 8 | If M_LEVELTR = 1 (STM) |

#### Packet Number 49: List of balises for SH Area

| Description | Used to list balise group(s) which the train can pass over in SH mode |  |  |
| --- | --- | --- | --- |
| Transmission media | Any |  |  |
| Content | Variable | Length | Comment |
|  | NID_PACKET | 8 |  |
|  | Q_DIR | 2 |  |
|  | L_PACKET | 13 |  |
|  | N_ITER | 5 |  |
|  | Q_NEWCOUNTRY(k) | 1 |  |
|  | NID_C(k) | 10 | if Q_NEWCOUNTRY = 1 |
|  | NID_BG(k) | 14 |  |

#### Packet Number 51: Axle Load Speed Profile

| Description | This packet gives the speed restrictions for trains with axle load higher than or equal to the specified value for the speed restriction |  |  |
| --- | --- | --- | --- |
| Transmission media | Any |  |  |
| Content | Variable | Length | Comment |
|  | NID_PACKET | 8 |  |
|  | Q_DIR | 2 |  |
|  | L_PACKET | 13 |  |
|  | Q_SCALE | 2 |  |
|  | Q_TRACKINIT | 1 |  |
|  | D_AXLELOAD | 15 |  |
|  | L_AXLELOAD | 15 |  |
|  | Q_FRONT | 1 |  |
|  | N_ITER | 5 |  |
|  | M_AXLELOAD(n) | 7 |  |
|  | V_AXLELOAD(n) | 7 | Speed restriction to be applied if the axle load of the train  M_AXLELOAD(n) |
|  | N_ITER | 5 |  |
|  | D_AXLELOAD(k) | 15 |  |
|  | L_AXLELOAD(k) | 15 |  |
|  | Q_FRONT(k) | 1 |  |
|  | N_ITER(k) | 5 |  |
|  | M_AXLELOAD(k,m) | 7 |  |
|  | V_AXLELOAD(k,m) | 7 | Speed restriction to be applied if the axle load of the train  M_AXLELOAD(k,m) |

#### Packet Number 57: Movement Authority Request Parameters

| Description | This packet is intended to give parameters telling when and how often the train has to ask for a movement authority. |  |  |
| --- | --- | --- | --- |
| Transmission media | Radio |  |  |
| Content | Variable | Length | Comment |
|  | NID_PACKET | 8 |  |
|  | Q_DIR | 2 |  |
|  | L_PACKET | 13 |  |
|  | T_MAR | 8 |  |
|  | T_TIMEOUTRQST | 10 |  |
|  | T_CYCRQST | 8 |  |

#### Packet Number 58: Position Report Parameters

| Description | This packet is intended to give parameters telling when and how often the position has to be reported. |  |  |
| --- | --- | --- | --- |
| Transmission media | Radio |  |  |
| Content | Variable | Length | Comment |
|  | NID_PACKET | 8 |  |
|  | Q_DIR | 2 |  |
|  | L_PACKET | 13 |  |
|  | Q_SCALE | 2 |  |
|  | T_CYCLOC | 8 |  |
|  | D_CYCLOC | 15 |  |
|  | M_LOC | 3 |  |
|  | N_ITER | 5 |  |
|  | D_LOC(k) | 15 |  |
|  | Q_LGTLOC(k) | 1 |  |

#### Packet Number 63: List of Balises in SR Authority

| Description | Used to list balise group(s) which the train can pass over in SR mode |  |  |
| --- | --- | --- | --- |
| Transmission media | Radio |  |  |
| Content | Variable | Length | Comment |
|  | NID_PACKET | 8 |  |
|  | Q_DIR | 2 |  |
|  | L_PACKET | 13 |  |
|  | N_ITER | 5 |  |
|  | Q_NEWCOUNTRY(k) | 1 |  |
|  | NID_C(k) | 10 | if Q_NEWCOUNTRY = 1 |
|  | NID_BG(k) | 14 |  |

#### Packet Number 65: Temporary Speed Restriction

| Description | Transmission of temporary speed restriction. |  |  |
| --- | --- | --- | --- |
| Transmission media | Any |  |  |
| Content | Variable | Length | Comment |
|  | NID_PACKET | 8 |  |
|  | Q_DIR | 2 |  |
|  | L_PACKET | 13 |  |
|  | Q_SCALE | 2 |  |
|  | NID_TSR | 8 |  |
|  | D_TSR | 15 |  |
|  | L_TSR | 15 |  |
|  | Q_FRONT | 1 |  |
|  | V_TSR | 7 |  |

#### Packet Number 66: Temporary Speed Restriction Revocation

| Description | Transmission of temporary speed restriction revocation. |  |  |
| --- | --- | --- | --- |
| Transmission media | Any |  |  |
| Content | Variable | Length | Comment |
|  | NID_PACKET | 8 |  |
|  | Q_DIR | 2 |  |
|  | L_PACKET | 13 |  |
|  | NID_TSR | 8 | Identity of TSR to be revoked |

#### Packet Number 67: Track Condition Big Metal Masses

| Description | The packet gives details concerning where to ignore integrity check alarms of balise transmission due to big metal masses trackside. |  |  |
| --- | --- | --- | --- |
| Transmission media | Balise |  |  |
| Content | Variable | Length | Comment |
|  | NID_PACKET | 8 |  |
|  | Q_DIR | 2 |  |
|  | L_PACKET | 13 |  |
|  | Q_SCALE | 2 |  |
|  | D_TRACKCOND | 15 |  |
|  | L_TRACKCOND | 15 | The distance for which integrity check alarms of balise transmission shall be ignored |
|  | N_ITER | 5 |  |
|  | D_TRACKCOND(k) | 15 |  |
|  | L_TRACKCOND(k) | 15 | The distance for which integrity check alarms of balise transmission shall be ignored |

#### Packet Number 68: Track Condition

| Description | The packet gives details concerning the track ahead to support the driver when e.g. lower pantograph |  |  |
| --- | --- | --- | --- |
| Transmission media | Any |  |  |
| Content | Variable | Length | Comment |
|  | NID_PACKET | 8 |  |
|  | Q_DIR | 2 |  |
|  | L_PACKET | 13 |  |
|  | Q_SCALE | 2 |  |
|  | Q_TRACKINIT | 1 |  |
|  | D_TRACKINIT | 15 | Only if Q_TRACKINIT = 1 |
|  | D_TRACKCOND | 15 | Only if Q_TRACKINIT = 0 |
|  | L_TRACKCOND | 15 | Only if Q_TRACKINIT = 0 |
|  | M_TRACKCOND | 4 | Only if Q_TRACKINIT = 0 |
|  | N_ITER | 5 | Only if Q_TRACKINIT = 0 |
|  | D_TRACKCOND(k) | 15 |  |
|  | L_TRACKCOND(k) | 15 |  |
|  | M_TRACKCOND(k) | 4 |  |

#### Packet Number 70:  Route Suitability Data

| Description | The packet gives the characteristics needed to enter a route. |  |  |
| --- | --- | --- | --- |
| Transmission media | Any |  |  |
| Content | Variable | Length | Comment |
|  | NID_PACKET | 8 |  |
|  | Q_DIR | 2 |  |
|  | L_PACKET | 13 |  |
|  | Q_SCALE | 2 |  |
|  | Q_TRACKINIT | 1 |  |
|  | D_TRACKINIT | 15 | Only if Q_TRACKINIT = 1 |
|  | D_SUITABILITY | 15 | Only If Q_TRACKINIT = 0, D_SUITABILITY and the following variables follows |
|  | Q_SUITABILITY | 2 |  |
|  | M_LOADINGGAUGE | 8 | If Q_SUITABILITY= loading gauge |
|  | M_AXLELOAD | 7 | If Q_SUITABILITY= Max axle load. |
|  | M_TRACTION | 8 | If Q_SUITABILITY = traction power |
|  | N_ITER | 5 |  |
|  | D_SUITABILITY(k) | 15 |  |
|  | Q_SUITABILITY(k) | 2 |  |
|  | M_LOADINGGAUGE(k) | 8 | If Q_SUITABILITY= loading gauge |
|  | M_AXLELOAD(k) | 7 | If Q_SUITABILITY= Max axle load. |
|  | M_TRACTION(k) | 8 | If Q_SUITABILITY = traction power |

#### Packet number 71: Adhesion factor

| Description | This packet is used when the trackside requests a change of the adhesion factor to be used in the brake model. |  |  |
| --- | --- | --- | --- |
| Transmission media | Any |  |  |
| Content | Variable | Length | Comment |
|  | NID_PACKET | 8 |  |
|  | Q_DIR | 2 |  |
|  | L_PACKET | 13 |  |
|  | Q_SCALE | 2 |  |
|  | D_ADHESION | 15 |  |
|  | L_ADHESION | 15 |  |
|  | M_ADHESION | 1 |  |

#### Packet Number 72: Packet for sending plain text messages

| Description |  |  |  |
| --- | --- | --- | --- |
| Transmission media | Any |  |  |
| Content | Variable | Length | Comment |
|  | NID_PACKET | 8 |  |
|  | Q_DIR | 2 |  |
|  | L_PACKET | 13 |  |
|  | Q_SCALE | 2 |  |
|  | Q_TEXTCLASS | 2 |  |
|  | Q_TEXTDISPLAY | 1 |  |
|  | D_TEXTDISPLAY | 15 | Start condition |
|  | M_MODETEXTDISPLAY | 4 | Start condition |
|  | M_LEVELTEXTDISPLAY | 3 | Start condition |
|  | NID_STM | 8 | If M_LEVELTR = 1 (STM) |
|  | L_TEXTDISPLAY | 15 | End condition |
|  | T_TEXTDISPLAY | 10 | End condition |
|  | M_MODETEXTDISPLAY | 4 | End condition |
|  | M_LEVELTEXTDISPLAY | 3 | End condition |
|  | NID_STM | 8 | If M_LEVELTR = 1 (STM) |
|  | Q_TEXTCONFIRM | 2 |  |
|  | L_TEXT | 8 |  |
|  | X_TEXT(L_TEXT) | 8 |  |

#### Packet Number 76: Packet for sending fixed text messages

| Description |  |  |  |
| --- | --- | --- | --- |
| Transmission media | Any |  |  |
| Content | Variable | Length | Comment |
|  | NID_PACKET | 8 |  |
|  | Q_DIR | 2 |  |
|  | L_PACKET | 13 |  |
|  | Q_SCALE | 2 |  |
|  | Q_TEXTCLASS | 2 |  |
|  | Q_TEXTDISPLAY | 1 |  |
|  | D_TEXTDISPLAY | 15 | Start condition |
|  | M_MODETEXTDISPLAY | 4 | Start condition |
|  | M_LEVELTEXTDISPLAY | 3 | Start condition |
|  | NID_STM | 8 | If M_LEVELTEXTDISPLAY = 1 (STM) |
|  | L_TEXTDISPLAY | 15 | End condition |
|  | T_TEXTDISPLAY | 10 | End condition |
|  | M_MODETEXTDISPLAY | 4 | End condition |
|  | M_LEVELTEXTDISPLAY | 3 | End condition |
|  | NID_STM | 8 | If M_LEVELTEXTDISPLAY = 1 (STM) |
|  | Q_TEXTCONFIRM | 2 |  |
|  | Q_TEXT | 8 |  |

#### Packet Number 79: Geographical Position Information

| Description | This packet gives geographical location information for one or multiple references to the train. |  |  |
| --- | --- | --- | --- |
| Transmission media | Any |  |  |
| Content | Variable | Length | Comment |
|  | NID_PACKET | 8 |  |
|  | Q_DIR | 2 |  |
|  | L_PACKET | 13 |  |
|  | Q_SCALE | 2 |  |
|  | Q_NEWCOUNTRY | 1 |  |
|  | NID_C | 10 | if Q_NEWCOUNTRY = 1 |
|  | NID_BG | 14 | Geographical Position Reference Balise Group |
|  | D_POSOFF | 15 |  |
|  | Q_MPOSITION | 1 | Geographical Position counting direction |
|  | M_POSITION | 20 | Track kilometre reference value |
|  | N_ITER | 5 |  |
|  | Q_NEWCOUNTRY(k) | 1 |  |
|  | NID_C(k) | 10 | if Q_NEWCOUNTRY = 1 |
|  | NID_BG(k) | 14 | Geographical Position Reference Balise Group |
|  | D_POSOFF(k) | 15 |  |
|  | Q_MPOSITION(k) | 1 | Geographical Position counting direction |
|  | M_POSITION(k) | 20 | Track kilometre reference value |

#### Packet Number 80: Mode profile

| Description | Mode profile associated to an MA |  |  |
| --- | --- | --- | --- |
| Transmission media | Any |  |  |
| Content | Variable | Length | Comment |
|  | NID_PACKET | 8 |  |
|  | Q_DIR | 2 |  |
|  | L_PACKET | 13 |  |
|  | Q_SCALE | 2 |  |
|  | D_MAMODE | 15 |  |
|  | M_MAMODE | 2 | OS, SH |
|  | V_MAMODE | 7 |  |
|  | L_MAMODE | 15 |  |
|  | L_ACKMAMODE | 15 |  |
|  | N_ITER | 5 |  |
|  | D_MAMODE(k) | 15 |  |
|  | M_MAMODE(k) | 2 | OS, SH |
|  | V_MAMODE(k) | 7 |  |
|  | L_MAMODE(k) | 15 |  |
|  | L_ACKMAMODE(k) | 15 |  |

##### Packet Number 90: Track Ahead Free up to level 2/3 transition location

| Description | Notification to on-board that track ahead is free from the balise group transmitting this information up to the level 2/3 transition location |  |  |
| --- | --- | --- | --- |
| Transmission media | Balise |  |  |
| Content | Variable | Length | Comment |
|  | NID_PACKET | 8 |  |
|  | Q_DIR | 2 |  |
|  | L_PACKET | 13 |  |
|  | Q_NEWCOUNTRY | 1 |  |
|  | NID_C | 10 | If Q_NEWCOUNTRY = 1 |
|  | NID_BG | 14 | Level 2/3 transition location balise group |

#### Packet Number 131: RBC transition order

| Description | Packet to order an RBC transition |  |  |
| --- | --- | --- | --- |
| Transmission media | Any |  |  |
| Content | Variable | Length | Comment |
|  | NID_PACKET | 8 |  |
|  | Q_DIR | 2 |  |
|  | L_PACKET | 13 |  |
|  | Q_SCALE | 2 |  |
|  | D_RBCTR | 15 |  |
|  | NID_C | 10 | “Accepting” RBC identity |
|  | NID_RBC | 14 |  |
|  | NID_RADIO | 64 | “Accepting” RBC radio subscriber number |
|  | Q_SLEEPSESSION | 1 |  |

#### Packet Number 132: Danger for Shunting information

| Description | Transmission of the aspect of a shunting signal |  |  |
| --- | --- | --- | --- |
| Transmission media | Balise |  |  |
| Content | Variable | Length | Comment |
|  | NID_PACKET | 8 |  |
|  | Q_DIR | 2 |  |
|  | L_PACKET | 13 |  |
|  | Q_ASPECT | 1 |  |

#### Packet Number 133: Radio in-fill area information

| Description |  |  |  |
| --- | --- | --- | --- |
| Transmission media | Balise |  |  |
| Content | Variable | Length | Comment |
|  | NID_PACKET | 8 |  |
|  | Q_DIR | 2 |  |
|  | L_PACKET | 13 |  |
|  | Q_SCALE | 2 |  |
|  | Q_RIU | 1 |  |
|  | NID_C | 10 | RIU ETCS identity |
|  | NID_RIU | 14 |  |
|  | NID_RADIO | 64 |  |
|  | D_INFILL | 15 |  |
|  | NID_C | 10 | Refers to the next main signal balise group |
|  | NID_BG | 14 |  |

#### Packet Number 134: EOLM Packet

| Description | This packet announces a loop. |  |  |
| --- | --- | --- | --- |
| Transmission media | Balise |  |  |
| Content | Variable | Length | Comment |
|  | NID_PACKET | 8 |  |
|  | Q_DIR | 2 |  |
|  | L_PACKET | 13 |  |
|  | Q_SCALE | 2 |  |
|  | NID_LOOP | 14 |  |
|  | D_LOOP | 15 |  |
|  | L_LOOP | 15 |  |
|  | Q_LOOPDIR | 1 |  |
|  | Q_SSCODE | 4 |  |

#### Intentionally deleted

|  |  | 8 |  |
| --- | --- | --- | --- |
|  |  | 2 |  |
|  |  | 13 |  |
|  |  | 10 |  |
|  |  | 14 |  |
|  |  | 1 |  |

#### Packet Number 136: Infill location reference

| Description | Defines location reference for all data contained in the same radio message or balise/loop telegram respectively, following this packet. |  |  |
| --- | --- | --- | --- |
| Transmission media | Balise, loop or infill radio |  |  |
| Content | Variable | Length | Comment |
|  | NID_PACKET | 8 |  |
|  | Q_DIR | 2 |  |
|  | L_PACKET | 13 |  |
|  | Q_NEWCOUNTRY | 1 |  |
|  | NID_C | 10 | If Q_NEWCOUNTRY = 1 |
|  | NID_BG | 14 |  |

#### Packet Number 137: Stop if in Staff Responsible

| Description | Information to stop a train in staff responsible. |  |  |
| --- | --- | --- | --- |
| Transmission media | Balise |  |  |
| Content | Variable | Length | Comment |
|  | NID_PACKET | 8 |  |
|  | Q_DIR | 2 |  |
|  | L_PACKET | 13 |  |
|  | Q_SRSTOP | 1 |  |

#### Packet Number 138: Reversing area information

| Description | Used to send start and length of reversing area to the on-board |  |  |
| --- | --- | --- | --- |
| Transmission media | Any |  |  |
| Content | Variable | Length | Comment |
|  | NID_PACKET | 8 |  |
|  | Q_DIR | 2 |  |
|  | L_PACKET | 13 |  |
|  | Q_SCALE | 2 |  |
|  | D_STARTREVERSE | 15 |  |
|  | L_REVERSEAREA | 15 |  |

#### Packet Number 139: Reversing supervision information

| Description | Used to send supervision parameters (distance to run, speed) of reversing area to the on-board |  |  |
| --- | --- | --- | --- |
| Transmission media | Any |  |  |
| Content | Variable | Length | Comment |
|  | NID_PACKET | 8 |  |
|  | Q_DIR | 2 |  |
|  | L_PACKET | 13 |  |
|  | Q_SCALE | 2 |  |
|  | D_REVERSE | 15 |  |
|  | V_REVERSE | 7 |  |

#### Packet Number 140: Train running number from RBC

| Description | Train running number from RBC |  |  |
| --- | --- | --- | --- |
| Transmission media | Radio |  |  |
| Content | Variable | Length | Comment |
|  | NID_PACKET | 8 |  |
|  | Q_DIR | 2 |  |
|  | L_PACKET | 13 |  |
|  | NID_OPERATIONAL | 32 |  |

#### Packet Number 141: Default Gradient for Temporary Speed Restriction

| Description | It defines a default gradient to be used for TSR supervision when no gradient profile (packet 21) is available |  |  |
| --- | --- | --- | --- |
| Transmission media | Balise |  |  |
| Content | Variable | Length | Comment |
|  | NID_PACKET | 8 |  |
|  | Q_DIR | 2 |  |
|  | L_PACKET | 13 |  |
|  | Q_GDIR | 1 | 0 = downhill 1= uphill |
|  | G_TSR | 8 |  |

#### Packet Number 254: Default balise, loop or RIU information

| Description | Indication to on-board that balise telegram, loop message or RIU information contains default information due to a fault of the trackside equipment. |  |  |
| --- | --- | --- | --- |
| Transmission media | Any |  |  |
| Content | Variable | Length | Comment |
|  | NID_PACKET | 8 |  |
|  | Q_DIR | 2 |  |
|  | L_PACKET | 13 |  |

### PACKETS: TRAIN TO TRACK

#### Packet Number 0: Position Report

| Description | This packet is inserted in all train-to-track telegrams and is used to report the train position and speed as well as some additional information (e.g. mode, level, etc.) |  |  |
| --- | --- | --- | --- |
| Transmission media | Radio |  |  |
| Content | Variable | Length | Comment |
|  | NID_PACKET | 8 |  |
|  | L_PACKET | 13 |  |
|  | Q_SCALE | 2 |  |
|  | NID_LRBG | 10 + 14 |  |
|  | D_LRBG | 15 |  |
|  | Q_DIRLRBG | 2 |  |
|  | Q_DLRBG | 2 |  |
|  | L_DOUBTOVER | 15 |  |
|  | L_DOUBTUNDER | 15 |  |
|  | Q_LENGTH | 2 |  |
|  | L_TRAININT | 15 | If Q_LENGTH = “Train integrity confirmed by integrity monitoring device” or “Train integrity confirmed by driver” |
|  | V_TRAIN | 7 |  |
|  | Q_DIRTRAIN | 2 |  |
|  | M_MODE | 4 |  |
|  | M_LEVEL | 3 |  |
|  | NID_STM | 8 | If M_LEVEL = STM |

#### Packet Number 1: Position Report based on two balise groups

| Description | This packet is an extension of the "standard position report " packet 0. It is used in case of single balise groups if the orientation of the LRBG is unknown but the on-board equipment is able to report a second balise group (the one detected before) to give an indication of the running direction to the RBC. |  |  |
| --- | --- | --- | --- |
| Transmission media | Radio |  |  |
| Content | Variable | Length | Comment |
|  | NID_PACKET | 8 |  |
|  | L_PACKET | 13 |  |
|  | Q_SCALE | 2 |  |
|  | NID_LRBG | 10 + 14 |  |
|  | NID_PRVBG | 10 + 14 | PRVBG and LRBG identify the running direction of the train to the RBC (train was running from the PRVBG to the LRBG) |
|  | D_LRBG | 15 |  |
|  | Q_DIRLRBG | 2 | Same / opposite direction as reported running direction |
|  | Q_DLRBG | 2 | Same / opposite direction as reported running direction |
|  | L_DOUBTOVER | 15 |  |
|  | L_DOUBTUNDER | 15 |  |
|  | Q_LENGTH | 2 |  |
|  | L_TRAININT | 15 | If Q_LENGTH = “Train integrity confirmed by integrity monitoring device” or “Train integrity confirmed by driver” |
|  | V_TRAIN | 7 |  |
|  | Q_DIRTRAIN | 2 |  |
|  | M_MODE | 4 |  |
|  | M_LEVEL | 3 |  |
|  | ##### NID_STM | 8 | If M_LEVEL = STM |

#### Packet Number 3: Onboard telephone numbers

| Description | Telephone numbers associated to the onboard equipment |  |  |
| --- | --- | --- | --- |
| Transmission media | Radio |  |  |
| Content | Variable | Length | Comment |
|  | NID_PACKET | 8 |  |
|  | L_PACKET | 13 |  |
|  | N_ITER | 5 |  |
|  | NID_RADIO (k) | 64 |  |

#### Packet Number 4: Error reporting

| Description | Error reporting to the RBC |  |  |
| --- | --- | --- | --- |
| Transmission media | Radio |  |  |
| Content | Variable | Length | Comment |
|  | NID_PACKET | 8 |  |
|  | L_PACKET | 13 |  |
|  | M_ERROR | 8 | error type identifier |

##### Packet Number 9: Level 2/3 transition information

| Description | Identity of the level 2/3 transition balise group |  |  |
| --- | --- | --- | --- |
| Transmission media | Radio |  |  |
| Content | Variable | Length | Comment |
|  | NID_PACKET | 8 |  |
|  | L_PACKET | 13 |  |
|  | NID_LTRBG | 10 + 14 |  |

#### Packet Number 11: Validated train data

| Description | Validated train data. For some variables, if the engine does not know a data value, it may use the corresponding default value |  |  |
| --- | --- | --- | --- |
| Transmission media | Radio |  |  |
| Content | Variable | Length | Comment |
|  | NID_PACKET | 8 |  |
|  | L_PACKET | 13 |  |
|  | NID_OPERATIONAL | 32 |  |
|  | NC_TRAIN | 15 | Default value Basic static speed profile |
|  | L_TRAIN | 12 |  |
|  | V_MAXTRAIN | 7 |  |
|  | M_LOADINGGAUGE | 8 | one of 4 different profiles |
|  | M_AXLELOAD | 7 |  |
|  | M_AIRTIGHT | 2 |  |
|  | N_ITER | 5 |  |
|  | M_TRACTION (k) | 8 | Type of traction |
|  | N_ITER | 5 |  |
|  | NID_STM (k) | 8 | Type of STM available |

#### Packet Number 44: Data used by applications outside the ERTMS/ETCS system.

| Description | Messages between on-board and trackside devices, which contain information used by applications outside the ERTMS/ETCS system. |  |  |
| --- | --- | --- | --- |
| Transmission media | Any |  |  |
| Content | Variable | Length | Comment |
|  | NID_PACKET | 8 |  |
|  | L_PACKET | 13 |  |
|  | NID_XUSER | 9 |  |
|  | Other data, depending on NID_XUSER |  |  |

### PACKETS: TRACK TO TRAIN or TRAIN TO TRACK

#### Packet Number 255: End of Information

| Description | This packet consists only of NID_PACKET containing 8 bit 1s It acts as a finish flag ; the receiver will stop reading the remaining part of the message/telegram when receiving eight bits set to one in the NID_PACKET field. |  |  |
| --- | --- | --- | --- |
| Transmission media | Any |  |  |
| Content | Variable | Length | Comment |
|  | NID_PACKET | 8 | = 255 (1111 1111) |

## Definitions of Variables

#### D_ADHESION

| Name | Distance to start of area with reduced adhesion factor |  |  |
| --- | --- | --- | --- |
| Description |  |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 15 bits | 0 cm | 327.670 km | 10 cm, 1 m or 10 m depending on Q_SCALE |

#### D_AXLELOAD

| Name | Distance to speed restriction due to Axle load |  |  |
| --- | --- | --- | --- |
| Description |  |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 15 bits | 0 m | 327.670 km | 10 cm, 1m or 10 m depending on Q_SCALE |

#### D_CYCLOC

| Name | Distance between two location reports from the train |  |  |
| --- | --- | --- | --- |
| Description | The train has to report its location every D_CYCLOC meters. |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 15 bits | 0 cm | 327.660 km | 10 cm, 1m or 10 m depends on Q_SCALE |
| Special/Reserved Values | 1111 … 1111 | The train has not to report cyclically its location. |  |

#### D_DP

| Name | Distance from the End of Authority to danger point |  |  |
| --- | --- | --- | --- |
| Description |  |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 15 bits | 0 cm | 327.670 km | 10 cm, 1m or 10 m depends on Q_SCALE. |

#### D_EMERGENCYSTOP

| Name | Distance to emergency stop location |  |  |
| --- | --- | --- | --- |
| Description | Distance between the LRBG and the emergency stop location |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 15 bits | 0 cm | 327.670 km | 10 cm, 1m or 10 m depends on Q_SCALE. |

#### D_ENDTIMERSTARTLOC

| Name | Distance from End section timer start location to End of Authority |  |  |
| --- | --- | --- | --- |
| Description |  |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 15 bits | 0 cm | 327.670 km | 10 cm, 1m or 10 m depends on Q_SCALE |

#### D_GRADIENT

| Name | Incremental distance to next change of gradient. |  |  |
| --- | --- | --- | --- |
| Description |  |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 15 bits | 0 cm | 327.670 km | 10 cm, 1m or 10 m depends on Q_SCALE. |

#### D_INFILL

| Name | Distance from location where to connect/disconnect to a radio in-fill unit |  |  |
| --- | --- | --- | --- |
| Description |  |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 15 bits | 0 cm | 327.670 km | 10 cm, 1m or 10 m depends on Q_SCALE |

#### D_LEVELTR

| Name | Distance to level transition |  |  |
| --- | --- | --- | --- |
| Description |  |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 15 bits | 0 cm | 327.660 km | 10 cm, 1m or 10 m depends on Q_SCALE. |
| Special/Reserved Values | 32767 | Now (The level transition is performed upon receipt of the order) |  |

#### D_LINK

| Name | Incremental linking distance to next linked balise group |  |  |
| --- | --- | --- | --- |
| Description |  |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 15 bits | 0 cm | 327.670 km | 10 cm, 1m or 10 m depends on Q_SCALE |

#### D_LOC

| Name | Incremental distance between locations where the train has to report its position. |  |  |
| --- | --- | --- | --- |
| Description |  |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 15 bits | 0 cm | 327.670 km | 10 cm, 1m or 10 m depends on Q_SCALE |

#### D_LOOP

| Name | Distance between EOLM and start of loop |  |  |
| --- | --- | --- | --- |
| Description | The EOLM specifies the distance to the beginning of the loop transmission |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 15 bits | 0 cm | 327.660 km | 10 cm, 1m or 10 m depends on Q_SCALE |
| Special/Reserved Values | 1111 … 1111 | Distance not known |  |

#### D_LRBG

| Name | Distance between the last relevant balise group and the estimated front end of the train (the side of the active cab). |  |  |
| --- | --- | --- | --- |
| Description |  |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 15 bits | 0 cm | 327.660 km | 10 cm, 1m or 10 m depends on Q_SCALE |
| Special/Reserved Values | 32767 | Unknown |  |

#### D_MAMODE

| Name | Incremental distance to the next change of mode within the MA |  |  |
| --- | --- | --- | --- |
| Description |  |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 15 bits | 0 cm | 327.670 km | 10 cm, 1m or 10 m depends on Q_SCALE |

#### D_NVOVTRP

| Name | Maximum distance for overriding the train trip |  |  |
| --- | --- | --- | --- |
| Description | This variable is part of the National Values |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 15 bits | 0 cm | 327.670 km | 10 cm, 1m or 10 m depends on Q_SCALE |

#### D_NVPOTRP

| Name | Maximum distance for reversing in Post Trip mode |  |  |
| --- | --- | --- | --- |
| Description | This variable is part of the National Values |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 15 bits | 0 cm | 327.670 km | 10 cm, 1m or 10 m depends on Q_SCALE |

#### D_NVROLL

| Name | Roll away distance limit |  |  |
| --- | --- | --- | --- |
| Description | This variable is part of the National Values and is used for Roll Away Protection and Reverse Movement Protection |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 15 bits | 0 cm | 327.660 km | 10 cm, 1m or 10 m depends on Q_SCALE |
| Special/Reserved Values | 32767 |  |  |

#### D_NVSTFF

| Name | Maximum distance for running in Staff Responsible mode |  |  |
| --- | --- | --- | --- |
| Description | This variable is part of the National Values |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 15 bits | 0 cm | 327.660 km | 10 cm, 1m or 10 m depends on Q_SCALE |
| Special/Reserved Values | 32767 |  |  |

#### D_OL

| Name | The distance from the End of Authority to the end of overlap |  |  |
| --- | --- | --- | --- |
| Description |  |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 15 bits | 0 cm | 327.670 km | 10 cm, 1m or 10 m depends on Q_SCALE. |

#### D_POSOFF

| Name | Route kilometre offset from the referencing balise |  |  |
| --- | --- | --- | --- |
| Description | The geographical position reporting function uses this variables content as an offset from the referencing balise to the reference point. |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 15 bits | 0 m | 327.670 km | 10 cm, 1m or 10 m depends on Q_SCALE. |

#### D_RBCTR

| Name | Distance to RBC transition |  |  |
| --- | --- | --- | --- |
| Description |  |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 15 bits | 0 cm | 327.670 km | 10 cm, 1m or 10 m depends on Q_SCALE. |

#### D_REF

| Name | Reference distance |  |  |
| --- | --- | --- | --- |
| Description | Distance between the LRBG and the new shifted location reference. The positive values are in the nominal direction of the LRBG |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 16 bits | -327.680 km | 327.670 km | 10 cm, 1m or 10 m depends on Q_SCALE. |
| Special/Reserved Values | The negative value are coded in 2’s complement |  |  |

#### D_REVERSE

| Name | Distance in RV mode |  |  |
| --- | --- | --- | --- |
| Description | Distance that can be run in RV mode |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 15 bits | 0 cm | 327.660 km | 10 cm, 1m or 10 m depends on Q_SCALE |
| Special/Reserved Values | 32767 | represents ¥ |  |

#### D_SECTIONTIMERSTOPLOC

| Name | Distance from beginning of section to the Section Time-out stop location |  |  |
| --- | --- | --- | --- |
| Description |  |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 15 bits | 0 cm | 327.670 km | 10 cm, 1m or 10 m depends on Q_SCALE |

#### D_SR

| Name | Distance in SR mode |  |  |
| --- | --- | --- | --- |
| Description | Distance that can be run in SR mode |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 15 bits | 0 cm | 327.660 km | 10 cm, 1m or 10 m depends on Q_SCALE |
| Special/Reserved Values | 32767 | Represents  |  |

#### D_STARTOL

| Name | Distance from overlap timer start location to End of Authority |  |  |
| --- | --- | --- | --- |
| Description |  |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 15 bits | 0 cm | 327.670 km | 10 cm, 1m or 10 m depends on Q_SCALE |

#### D_STARTREVERSE

| Name | Distance to start of reversing permitted area |  |  |
| --- | --- | --- | --- |
| Description |  |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 15 bits | 0 cm | 327.670 km | 10 cm, 1m or 10 m depends on Q_SCALE |

#### D_STATIC

| Name | Incremental distance to next discontinuity in a international SSP profile |  |  |
| --- | --- | --- | --- |
| Description |  |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 15 bits | 0 cm | 327.670 km | 10 cm, 1m or 10 m depends on Q_SCALE |

#### D_SUITABILITY

| Name | Distance to change in route suitability |  |  |
| --- | --- | --- | --- |
| Description | The incremental distance to where the route suitability data changes. |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 15 bits | 0 m | 327.670 km | 10 cm, 1m or 10 m depending on Q_SCALE |

#### D_TAFDISPLAY

| Name | Distance from where on a track ahead free request shall be displayed |  |  |
| --- | --- | --- | --- |
| Description |  |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 15 bits | 0 cm | 327.670 km | 10 cm, 1m or 10 m depends on Q_SCALE |

#### D_TEXTDISPLAY

| Name | Distance from where on a text shall be displayed |  |  |
| --- | --- | --- | --- |
| Description |  |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 15 bits | 0 cm | 327.660 km | 10 cm, 1m or 10 m depends on Q_SCALE |
| Special/Reserved Values | 1111 … 1111 | The display of the text shall not be distance limited. |  |

#### D_TRACKINIT

| Name | Distance to start of empty profile |  |  |
| --- | --- | --- | --- |
| Description | Distance to where initial states of the related track description in the packet shall be resumed |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 15 bits | 0 m | 327.670 km | 10 cm, 1m or 10 m depending on Q_SCALE |

#### D_TRACKCOND

| Name | Track condition distance |  |  |
| --- | --- | --- | --- |
| Description | The incremental distance to where the track conditions change. |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 15 bits | 0 m | 327.670 km | 10 cm, 1m or 10 m depending on Q_SCALE |

#### D_TRACTION

| Name | Distance to change of traction |  |  |
| --- | --- | --- | --- |
| Description |  |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 15 bits | 0 m | 327.670 km | 10 cm, 1m or 10 m depending on Q_SCALE |

#### D_TSR

| Name | Distance to beginning of temporary speed restriction |  |  |
| --- | --- | --- | --- |
| Description |  |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 15 bits | 0 cm | 327.670 km | 10 cm, 1m or 10 m depends on Q_SCALE |

#### D_VALIDNV

| Name | Distance to start of validity of national values |  |  |
| --- | --- | --- | --- |
| Description |  |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 15 bits | 0 cm | 327.670 km | 10 cm, 1m or 10 m depends on Q_SCALE |

#### G_A

| Name | Safe gradient |  |  |
| --- | --- | --- | --- |
| Description | This is the minimum gradient between two defined locations. |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 8 bits | 0 | 254‰ | 1‰ |
| Special/Reserved Values | 255 | Non numerical value telling that the current gradient description ends at D_GRADIENT(n) |  |

#### G_TSR

| Name | Default gradient for TSR supervision |  |  |
| --- | --- | --- | --- |
| Description | defines a default gradient to be used for TSR supervision when no gradient profile (packet 21) is available. |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 8 bits | 0 | 255‰ | 1‰ |

#### L_ACKLEVELTR

| Name | Length of the acknowledgement area in rear of the required level |  |  |
| --- | --- | --- | --- |
| Description |  |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 15 bits | 0 cm | 327.670 km | 10 cm, 1m or 10 m depends on Q_SCALE |

#### L_ACKMAMODE

| Name | Length of the acknowledgement area in rear of the start of the required mode |  |  |
| --- | --- | --- | --- |
| Description |  |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 15 bits | 0 cm | 327.670 km | 10 cm, 1m or 10 m depends on Q_SCALE |

#### L_ADHESION

| Name | Length of reduced adhesion |  |  |
| --- | --- | --- | --- |
| Description | Length for which the reduced adhesion factor apply. |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 15 bits | 0 cm | 327.670 km | 10 cm, 1 m or 10 m depending on Q_SCALE |

#### L_AXLELOAD

| Name | Length of speed restriction due to Axle load |  |  |
| --- | --- | --- | --- |
| Description |  |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 15 bits | 0 m | 327.670 km | 10 cm, 1m or 10 m depending on Q_SCALE |

#### L_DOUBTOVER

| Name | Over-reading error |  |  |
| --- | --- | --- | --- |
| Description | L_DOUBTOVER is the difference between the lower bound of the confidence interval and the estimated value of D_LRBG |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 15 bits | 0 cm | 327.660 km | 10 cm, 1m or 10 m depends on Q_SCALE |
| Special/Reserved Values | 32767 | Unknown |  |

#### L_DOUBTUNDER

| Name | Under-reading error |  |  |
| --- | --- | --- | --- |
| Description | L_DOUBTUNDER is the difference between the upper bound of the confidence interval and the estimated value of D_LRBG |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 15 bits | 0 cm | 327.660 km | 10 cm, 1m or 10 m depends on Q_SCALE |
| Special/Reserved Values | 32767 | Unknown |  |

#### L_ENDSECTION

| Name | Length of the End section in the MA |  |  |
| --- | --- | --- | --- |
| Description |  |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 15 bits | 0 cm | 327.670 km | 10 cm, 1m or 10 m depends on Q_SCALE. |

#### L_LOOP

| Name | Length of loop |  |  |
| --- | --- | --- | --- |
| Description | The EOLM specifies the length of the loop starting from the distance indicated by D_LOOP |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 15 bits | 0 cm | 327.660 km | 10 cm, 1m or 10 m depends on Q_SCALE |
| Special/Reserved Values | 1111 … 1111 | Length not known |  |

#### L_MAMODE

| Name | Length of the area of the required mode |  |  |
| --- | --- | --- | --- |
| Description |  |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 15 bits | 0 m | 327.660 km | 10 cm, 1m or 10 m depending on Q_SCALE |
| Special/Reserved Values | 1111…111 | Infinite length |  |

#### L_MESSAGE

| Name | Message length |  |  |
| --- | --- | --- | --- |
| Description | L_MESSAGE indicates the length of the message in bytes, including all packets and all variables defined in the message header (NID_MESSAGE and L_MESSAGE also). |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 10 bits | 0 | 1023 | 1 Byte |

#### L_PACKET

| Name | Packet length |  |  |
| --- | --- | --- | --- |
| Description | L_PACKET indicates the length of the packet in bits, including all bits of the packet header |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 13 bits | 0 | 8191 | 1 bit |
| Special/Reserved Values |  |  |  |

#### L_REVERSEAREA

| Name | Length of the reversing permitted area |  |  |
| --- | --- | --- | --- |
| Description |  |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 15 bits | 0 cm | 327.670 km | 10 cm, 1m or 10 m depends on Q_SCALE |

#### L_SECTION

| Name | Length of section in the MA |  |  |
| --- | --- | --- | --- |
| Description |  |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 15 bits | 0 cm | 327.670 km | 10 cm, 1m or 10 m depends on Q_SCALE |

#### L_TAFDISPLAY

| Name | Length on which a track ahead free request shall be displayed |  |  |
| --- | --- | --- | --- |
| Description |  |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 15 bits | 0 cm | 327.660 km | 10 cm, 1m or 10 m depends on Q_SCALE |

#### L_TEXT

| Name | Length of text string |  |  |
| --- | --- | --- | --- |
| Description | L_TEXT defines the length of a text string (L_TEXT * X_TEXT) |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 8 bits | 0 | 255 | 1 Text String Element |

#### L_TEXTDISPLAY

| Name | Length on which a text shall be displayed |  |  |
| --- | --- | --- | --- |
| Description |  |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 15 bits | 0 cm | 327.660 km | 10 cm, 1m or 10 m depends on Q_SCALE |
| Special/Reserved Values | 32767 | The display of the text shall not be distance limited. |  |

#### L_TRACKCOND

| Name | Length for which the defined track condition is valid |  |  |
| --- | --- | --- | --- |
| Description |  |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 15 bits | 0 m | 327.670 km | 10 cm, 1m or 10 m depending on Q_SCALE |

#### L_TRAIN

| Name | Train length |  |  |
| --- | --- | --- | --- |
| Description | This is the absolute real length of the train. |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 12 bits | 0 m | 4095 m | 1 m |
| Special/Reserved Values |  |  |  |

#### L_TRAININT

| Name | Safe Train length |  |  |
| --- | --- | --- | --- |
| Description |  |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 15 bits | 0 m | 32767 m | 1 m |
| Special/Reserved Values |  |  |  |

#### L_TSR

| Name | Length of the temporary speed restriction |  |  |
| --- | --- | --- | --- |
| Description |  |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 15 bits | 0 cm | 327.670 km | 10 cm, 1m or 10 m depends on Q_SCALE |

#### M_ACK

| Name | Qualifier for acknowledgement request |  |  |
| --- | --- | --- | --- |
| Description | Indicates whether the telegram must be acknowledged or not |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 1 bit |  |  |  |
| Special/Reserved Values | 0 | No acknowledgement required |  |
|  | 1 | Acknowledgement required |  |

#### M_ADHESION

| Name | Adhesion factor |  |  |
| --- | --- | --- | --- |
| Description |  |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 1 bit |  |  |  |
| Special/Reserved Values | 0 | 70 % |  |
|  | 1 | 100 % |  |

#### M_AIRTIGHT

| Name | airtight system presence |  |  |
| --- | --- | --- | --- |
| Description | indicates whether the train is fitted with an airtight system or not. |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 2 bits |  |  |  |
| Special/Reserved Values | 00 | Not fitted |  |
|  | 01 | Fitted |  |
|  | 10 | Spare |  |
|  | 11 | Spare |  |

#### M_AXLELOAD

| Name | Axle load |  |  |
| --- | --- | --- | --- |
| Description |  |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 7 bits | 0 t | 40 t | 0.5 t |
| Special/Reserved Values | 101 0001 | Spare |  |
|  | … | … |  |
|  | 111 1101 | Spare |  |
|  | 111 1110 | Axle load above 40 t |  |
|  | 111 1111 | Spare |  |

#### M_DUP

| Name | Duplicate balise |  |  |
| --- | --- | --- | --- |
| Description | Flags to tell whether the balise is a duplicate of one of the adjacent balises. |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 2 bits |  |  |  |
| Special/Reserved Values | 00 | No duplicates |  |
|  | 01 | This balise is a duplicate of the next balise (seen in the nominal direction of the balise group). |  |
|  | 10 | This balise is a duplicate of the previous balise (seen in the nominal direction of the balise group). |  |
|  | 11 | Spare |  |

#### M_ERROR

| Name | Identifier of the type of error |  |  |
| --- | --- | --- | --- |
| Description |  |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 8 bits |  |  |  |
| Special/Reserved Values | 0 | Balise consistency: linking |  |
|  | 1 | Balise consistency: message error |  |
|  | 2 | Balise consistency: unlinked group |  |
|  | 3 | Radio consistency: message error |  |
|  | 4 | Radio consistency: sequence |  |
|  | 5 | Radio consistency: radio link |  |
|  | 6 | No fatal error |  |
|  | 7 | Fatal error (equipment in SL or NL mode) |  |
|  | 8-255 | Spare |  |

#### M_LEVEL

| Name | Current Operating Level |  |  |
| --- | --- | --- | --- |
| Description |  |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 3 bits |  |  |  |
| Special/Reserved Values | 0 | Level 0 |  |
|  | 1 | Level STM specified by NID_STM |  |
|  | 2 | Level 1 |  |
|  | 3 | Level 2 |  |
|  | 4 | Level 3 |  |
|  | 5-7 | Spare |  |

#### M_LEVELTEXTDISPLAY

| Name | Onboard operating level for text display |  |  |
| --- | --- | --- | --- |
| Description | The text is displayed when entering / as long as in the defined level |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 3 bits |  |  |  |
| Special/Reserved Values | 0 | Level 0 |  |
|  | 1 | Level STM specified by NID_STM |  |
|  | 2 | Level 1 |  |
|  | 3 | Level 2 |  |
|  | 4 | Level 3 |  |
|  | 5 | The display of the text shall not be limited by the level |  |
|  | 6-7 | Spare |  |

#### M_LEVELTR

| Name | Required level |  |  |
| --- | --- | --- | --- |
| Description |  |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 3 bits |  |  | Bitset |
| Special/Reserved Values | 0 | Level 0 |  |
|  | 1 | Level STM specified by NID_STM |  |
|  | 2 | Level 1 |  |
|  | 3 | Level 2 |  |
|  | 4 | Level 3 |  |
|  | 5-7 | Spare |  |

#### M_LOADINGGAUGE (Values to be assigned according to 7.3.1.3)

| Name | Load profile |  |  |
| --- | --- | --- | --- |
| Description | Defining the loading gauge profile |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 8 bits |  |  | Bitset |
| Special/Reserved Values | xxxx xxx1 | Profile X |  |
|  | xxxx xx1x | Profile Y |  |
|  | xxxx x1xx | etc ... |  |
|  | xxx 1xxx |  |  |
|  | xxx1 xxxx |  |  |
|  | xx1x xxxx |  |  |
|  | x1xx xxxx |  |  |
|  | 1xxx xxxx |  |  |

#### M_LOC

| Name | Special location/moment where the train has to report its location |  |  |
| --- | --- | --- | --- |
| Description |  |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 3 bits |  |  |  |
| Special/Reserved Values | 000 | Now (The position report is sent upon receipt of the order) |  |
|  | 001 | At each balise. |  |
|  | 010 | Stop to send it at each balises. |  |
|  | 011 | Use previous value of M_LOC |  |
|  | 100 - 111 | Spare |  |

#### M_MAMODE

| Name | Required mode for a part of the MA |  |  |
| --- | --- | --- | --- |
| Description |  |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 2 bits |  |  |  |
| Special/Reserved Values | 00 | On Sight |  |
|  | 01 | Shunting |  |
|  | 10 – 11 | Spare |  |

#### M_MCOUNT

| Name | Message counter |  |  |
| --- | --- | --- | --- |
| Description | The purpose of this counter is to make it possible for the ERTMS/ETCS on-board to detect which balise group message the telegram belongs to. |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 8 bits | 0 | 254 | Numbers |
| Special/Reserved Values | 254 | The telegram never fits any message of the group |  |
|  | 255 | The telegram fits with all telegrams of the same balise group |  |

#### M_MODE

| Name | Onboard operating mode |  |  |
| --- | --- | --- | --- |
| Description |  |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 4 bits |  |  |  |
| Special/Reserved Values | 0 | Full Supervision |  |
|  | 1 | On Sight |  |
|  | 2 | Staff Responsible |  |
|  | 3 | Shunting |  |
|  | 4 | Unfitted |  |
|  | 5 | Sleeping |  |
|  | 6 | Stand By |  |
|  | 7 | Trip |  |
|  | 8 | Post Trip |  |
|  | 9 | System Failure |  |
|  | 10 | Isolation |  |
|  | 11 | Non Leading |  |
|  | 12 | STM European |  |
|  | 13 | STM National |  |
|  | 14 | Reversing |  |
|  | 15 | Spare |  |

#### M_MODETEXTDISPLAY

| Name | Onboard operating mode for text display |  |  |
| --- | --- | --- | --- |
| Description | The text is displayed when entering / as long as in the defined mode |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 4 bits |  |  |  |
| Special/Reserved Values | 0 | Full Supervision |  |
|  | 1 | On Sight |  |
|  | 2 | Staff Responsible |  |
|  | 3 | Shunting |  |
|  | 4 | Unfitted |  |
|  | 5 | Sleeping |  |
|  | 6 | Stand By |  |
|  | 7 | Trip |  |
|  | 8 | Post Trip |  |
|  | 9 | System Failure |  |
|  | 10 | Isolation |  |
|  | 11 | Non Leading |  |
|  | 12 | STM European |  |
|  | 13 | STM National |  |
|  | 14 | Reversing |  |
|  | 15 | The display of the text shall not be limited by the mode. |  |

#### M_NVCONTACT

| Name | T_NVCONTACT reaction |  |  |
| --- | --- | --- | --- |
| Description | Indicates the reaction to be performed when T_NVCONTACT timer elapses |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 2 bits |  |  |  |
| Special/Reserved Values | 00 | Train trip |  |
|  | 01 | Apply service brake |  |
|  | 10 | No Reaction |  |
|  | 11 | Spare |  |

#### M_NVDERUN

| Name | Entry of Driver ID permitted while running |  |  |
| --- | --- | --- | --- |
| Description | This variable is part of the National Values |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 1 bit |  |  |  |
| Special/Reserved Values | 0 | No |  |
|  | 1 | Yes |  |

#### M_POSITION

| Name | Track kilometre reference value |  |  |
| --- | --- | --- | --- |
| Description | The geographical position reporting function uses this variables content as a reference value. |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 20 bits | 0 m | 1'048’574 m | 1 m |
| Special/Reserved Values | 1'048’575 | No more geographical position reporting after this reference location |  |

#### M_TRACKCOND

| Name | Type of track condition |  |  |
| --- | --- | --- | --- |
| Description |  |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 4 bits |  |  |  |
| Special/Reserved Values | 0000 | Non stopping area – tunnel. Initial state: stopping permitted (no tunnel) |  |
|  | 0001 | Non stopping area – bridge. Initial state: stopping permitted (no bridge) |  |
|  | 0010 | Non stopping area – other reasons. Initial state: stopping permitted |  |
|  | 0011 | Powerless section – lower pantograph. Initial state: not powerless section |  |
|  | 0100 | Radio hole (stop supervising T_NVCONTACT). Initial state: supervise T_NVCONTACT |  |
|  | 0101 | Air tightness. Initial state: no request for air tightness |  |
|  | 0110 | Switch off regenerative brake. Initial state: regenerative brake on |  |
|  | 0111 | Switch off eddy current brake for service brake. Initial state: eddy current brake for service brake on |  |
|  | 1000 | Switch off magnetic shoe brake. Initial state: magnetic shoe brake on |  |
|  | 1001 | Powerless section – switch off the main power switch. Initial state: not powerless section |  |
|  | 1010 –1111 | Spare |  |

#### M_TRACTION (values to be assigned according to 7.3.1.3)

| Name | Traction System Type |  |  |
| --- | --- | --- | --- |
| Description | It defines the traction system to be used on a specific line (diesel/electric/kind of power pickup etc.) or respectively that can be used by a train |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 8 bits |  |  |  |

#### M_VERSION

| Name | Version of the ETCS language |  |  |
| --- | --- | --- | --- |
| Description | This gives the version of the ETCS language Each part indicates the first and second number of the version respectively. - The first number distinguishes not compatible versions. (The three MSB’s) - The second number indicates compatibility within a version X. (The four LSB’s) |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 7 bits |  |  |  |
| Special/Reserved Values | 000 XXXX | Old systems |  |
|  | 001 0000 | Class 1 |  |
|  | 001 0001 | Reserved for future use (this is a valid value) |  |
|  | … | … |  |
|  | 111 1111 | Reserved for future use (this is a valid value) |  |

#### N_ITER

| Name | Number of iterations of a data set following this variable in a packet |  |  |
| --- | --- | --- | --- |
| Description | If N_ITER is 0 then no data set is following. Two nested levels of iterations can exist. |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 5 bits | 0 | 31 | integers |

#### N_PIG

| Name | Position in Group |  |  |
| --- | --- | --- | --- |
| Description | Defines the relative position in a balise group |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 3 bits |  |  |  |
| Special/Reserved Values | 0 | I am the 1st |  |
|  | … | … |  |
|  | 7 | I am the 8th |  |

#### N_TOTAL

| Name | Total number of balise(s) in the group |  |  |
| --- | --- | --- | --- |
| Description |  |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 3 bits |  |  |  |
| Special/Reserved Values | 0 | 1 balise in the group |  |
|  | … |  |  |
|  | 7 | 8 balises in the group |  |

#### NC_DIFF (Values to be assigned according to 7.3.1.3)

| Name | International Train categories |  |  |
| --- | --- | --- | --- |
| Description | It is the international class of train for which a differential value for the static line speed exists. Used together with V_DIFF to permit certain trains to go faster or lower than the “international basic static speed” given by V_STATIC. Value 0 of NC_DIFF corresponds to the LSB of NC_TRAIN, number 14 of NC_DIFF to MSB (15-bit variable) of NC_TRAIN. |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 4 bits | 0 | 15 | Numbers |
| Special/Reserved Values | 0 | Active tilting SSP |  |
|  | 1 | Passive tilting SSP |  |
|  | 2 | Cross wind sensitivity |  |
|  | 3 – 15 | Spare |  |

#### NC_TRAIN (Values to be assigned according to 7.3.1.3)

| Name | International Category to which belongs the train. |  |  |
| --- | --- | --- | --- |
| Description | Train category used for the static speed profile calculation. Thanks to NC_TRAIN, the train knows the SSP it must obey. by receiving a list of static speed profile, thanks to NC_TRAIN, the train can select the SSP it must obey. Each bit represents one category. A train can belong to various categories. |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 15 bits |  |  | Bitset |
| Special/Reserved Values | 000 0000 0000 0000 | Basic static speed profile |  |
|  | Xxx xxxx xxxx xxx1 | Active tilting SSP |  |
|  | Xxx xxxx xxxx xx1x | Passive tilting SSP |  |
|  | Xxx xxxx xxxx x1xx | Cross wind sensitivity |  |
|  | Xxx xxxx xxx 1xxx | Spare |  |
|  | Xxx xxxx xxx1 xxxx | Spare |  |
|  | Xxx xxxx xx1x xxxx | Spare |  |
|  | Xxx xxxx x1xx xxxx | Spare |  |
|  | Xxx xxxx 1xxx xxxx | Spare |  |
|  | Xxx xxx1 xxxx xxxx | Spare |  |
|  | Xxx xx1x xxxx xxxx | Spare |  |
|  | Xxx x1xx xxxx xxxx | Spare |  |
|  | Xxx 1xxx xxxx xxxx | Spare |  |
|  | Xx1 xxxx xxxx xxxx | Spare |  |
|  | X1x xxxx xxxx xxxx | Spare |  |
|  | 1xx xxxx xxxx xxxx | Spare |  |

#### NID_BG

| Name | Identity number of the balise group |  |  |
| --- | --- | --- | --- |
| Description | Identity number of a balise group or loop within the country or region defined by NID_C. |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 14 bits | 0 | 16382 | Numbers |
| Special/Reserved Values | 16383 | only accept balise groups containing repositioning information for the running direction (only to be used in linking) |  |

#### NID_C (Values to be assigned according to 7.3.1.3)

| Name | Identity number of the country or region |  |  |
| --- | --- | --- | --- |
| Description | Code used to identify the country or region in which the balise group is situated. These need not necessarily follow administrative or political boundaries. |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 10 bits | 0 | 1023 | Numbers |
| Special/Reserved Values |  |  |  |

#### NID_EM

| Name | Emergency message identity |  |  |
| --- | --- | --- | --- |
| Description | Identifies the number of the emergency message |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 4 bits |  |  |  |
| Special/Reserved Values |  |  |  |

#### NID_ENGINE (Values to be assigned according to 7.3.1.3)

| Name | Onboard ETCS identity |  |  |
| --- | --- | --- | --- |
| Description | The ETCS identity number is uniquely defined for ERTMS/ETCS purposes |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 24 bits |  |  |  |

#### NID_LOOP

| Name | Identity number of the loop |  |  |
| --- | --- | --- | --- |
| Description | Identity number of a loop within the country or region defined by NID_C given in the EOLM balise header. |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 14 bits | 0 | 16383 | Numbers |

#### NID_LRBG

| Name | Identity of last relevant balise group |  |  |
| --- | --- | --- | --- |
| Description | Country/region identity (NID_C) + balise identity number of last relevant balise group (NID_BG). |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 10 + 14 bits |  |  |  |
| Special/Reserved Values | 16777215 | Unknown |  |

##### NID_LTRBG

| Name | Identity of the level 2/3 transition balise group |  |  |
| --- | --- | --- | --- |
| Description | Identity of the balise group at the level 2/3 transition location towards which the train is running. Country/region identity (NID_C) + balise identity number of the level 2/3 transition location balise group (NID_BG). |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 10 + 14 bits |  |  |  |

#### NID_MESSAGE

| Name | Message identifier |  |  |
| --- | --- | --- | --- |
| Description | Message identifier |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 8 bits | 0 | 255 | Numbers |

##### NID_MN

| Name | Identity of Radio Network |  |  |
| --- | --- | --- | --- |
| Description | The NID_MN identifies the GSM-R network the calling mobile station has to register with. The NID_MN consists of up to 6 digits which are entered left adjusted into the data field, the leftmost digit is the digit to be dialled first. In case the NID_MN is shorter than 6 digits, the remaining space is to be filled with special character “F”. For further information about NID_MN refer to Subset-54. |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 24 bits | 0 | 999999 | Binary Coded Decimal |
| Special/Reserved Values | For each digit ; |  |  |
|  | Values A – E | Not Used |  |
|  | F | Use value F for digit to indicate no digit (if number shorter than 6 digits) |  |

#### NID_OPERATIONAL (Values to be assigned according to 7.3.1.3)

| Name | Train Running Number |  |  |
| --- | --- | --- | --- |
| Description | This is the operational train running number. |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 32 bits | 0 | 9999 9999 | Binary Coded Decimal |
| Special/Reserved Values | For each digit ; |  |  |
|  | Values A – E | Not Used |  |
|  | F | Use value F for digit to indicate no digit (if number shorter than 8 digits) |  |
|  | Exception: FFFF FFFF | Unknown |  |

#### NID_PACKET

| Name | Packet identifier |  |  |
| --- | --- | --- | --- |
| Description | This is used in the header for each packet, allowing the receiving equipment to identify the data which follows. |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 8 bits | 0 | 255 | Numbers |

#### NID_PRVBG

| Name | Identity of the previous balise group |  |  |
| --- | --- | --- | --- |
| Description | Previous balise group detected when running towards the balise group identified as LRBG with no change of direction in-between. Country/region identity (NID_C) + balise identity number of the previous balise group (NID_BG). |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 10 + 14 bits |  |  |  |
| Special/Reserved Values | 16777215 | unknown |  |

#### NID_RADIO (Values to be assigned according to 7.3.1.3)

| Name | Radio subscriber number. |  |  |
| --- | --- | --- | --- |
| Description | Quoted as a 16 digit decimal number. The number is to be entered "left adjusted" starting with the first digit to be dialled. Padding by the special value F shall be added after the least significant digit of the number. For further information about NID_RADIO refer to SUBSET-054. |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 64 bits | 0 | 9999 9999 9999 9999 | Binary Coded Decimal |
| Special/Reserved Values | For each digit ; |  |  |
|  | Values A – E | Not Used |  |
|  | F | Use value F for digit to indicate no digit (if number shorter than 16 digits) |  |
|  | FFFF FFFF FFFF FFFF | Use the short number stored onboard |  |

#### NID_RBC (Values to be assigned according to 7.3.1.3)

| Name | RBC ETCS identity number |  |  |
| --- | --- | --- | --- |
| Description | This variable provides the identity of the RBC belonging to NID_C. The RBC ETCS identity is given by NID_C + NID_RBC. |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 14 bits | 0 | 16 382 | Number |
| Special/Reserved Values | 16 383 | not known |  |

#### NID_RIU (Values to be assigned according to 7.3.1.3)

| Name | Identity of radio in-fill unit |  |  |
| --- | --- | --- | --- |
| Description | This variable provides the identity of the RIU belonging to NID_C. The RIU ETCS identity is given by NID_C + NID_RIU. |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 14 bits | 0 | 16 382 | Number |
| Special/Reserved Values | 16 383 | not known |  |

#### NID_STM (Values to be assigned according to 7.3.1.3)

| Name | STM identity |  |  |
| --- | --- | --- | --- |
| Description | One value of this variable represents the identity of an STM reflecting each composition of national infrastructure. |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 8 bits | 0 | 255 |  |
| Special/Reserved Values |  |  |  |

#### NID_TSR

| Name | Identity number of Temporary Speed Restriction. |  |  |
| --- | --- | --- | --- |
| Description |  |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 8 bits | 0 | 255 | Number |
| Special/Reserved Values | 0-126 | Reserved for non RBC transmission (balise, loop or radio infill) |  |
|  | 127-254 | Reserved for RBC transmission |  |
|  | 1111 1111 | Non-revocable speed restriction (applicable for all transmission media) |  |

#### NID_XUSER (Values to be assigned according to 7.3.1.3)

| Name | Identity of user system |  |  |
| --- | --- | --- | --- |
| Description | Identity of user system for which remainder of packet is intended. |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 9 bits | 0 | 511 | Numbers |
| Special/Reserved Values |  |  |  |

#### Q_ASPECT

| Name | Aspect of “danger for shunting” signal |  |  |
| --- | --- | --- | --- |
| Description |  |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 1 bit |  |  |  |
| Special/Reserved Values | 0 | Stop if in SH mode |  |
|  | 1 | Go if in SH mode |  |

#### Q_DANGERPOINT

| Name | Qualifier for danger point description. |  |  |
| --- | --- | --- | --- |
| Description | This variable is set to 1 if either a danger point exists or a release speed has to be specified |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 1 bit |  |  |  |
| Special/Reserved Values | 0 | No danger point information |  |
|  | 1 | Danger point information to follow |  |

#### Q_DIR

| Name | Validity direction of transmitted data |  |  |
| --- | --- | --- | --- |
| Description | Qualifier to indicate the relevant validity direction of transmitted data, with reference to directionality of the balise group sending the information or to directionality of the LRBG, in case of information sent via radio. |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 2 bits |  |  |  |
| Special/Reserved Values | 00 | Reverse |  |
|  | 01 | Nominal |  |
|  | 10 | Both directions |  |
|  | 11 | Spare |  |

#### Q_DIRLRBG

| Name | Orientation of the train in relation to the direction of the LRBG |  |  |
| --- | --- | --- | --- |
| Description |  |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 2 bits |  |  |  |
| Special/Reserved Values | 0 | Reverse |  |
|  | 1 | Nominal |  |
|  | 2 | Unknown |  |
|  | 3 | Spare |  |

#### Q_DIRTRAIN

| Name | Direction of train movement in relation to the LRBG orientation |  |  |
| --- | --- | --- | --- |
| Description |  |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 2 bits |  |  |  |
| Special/Reserved Values | 0 | Reverse |  |
|  | 1 | Nominal |  |
|  | 2 | Unknown |  |
|  | 3 | Spare |  |

#### Q_DLRBG

| Name | Qualifier telling on which side of the LRBG the estimated front end is |  |  |
| --- | --- | --- | --- |
| Description |  |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 2 bits |  |  |  |
| Special/Reserved Values | 0 | Reverse |  |
|  | 1 | Nominal |  |
|  | 2 | Unknown |  |
|  | 3 | Spare |  |

#### Q_EMERGENCYSTOP

| Name | Qualifier for emergency stop acknowledgement |  |  |
| --- | --- | --- | --- |
| Description | Qualifier to indicate whether the train has ignored or not a conditional emergency stop. For an unconditional emergency stop, it is set to “not relevant” |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 2 bit |  |  |  |
| Special/Reserved Values | 0 | Conditional Emergency Stop considered |  |
|  | 1 | Conditional Emergency Stop ignored |  |
|  | 2 | Not Relevant (Unconditional Emergency Stop) |  |
|  | 3 | Spare |  |

#### Q_ENDTIMER

| Name | Qualifier to indicate whether end section timer information exists for the End section in the MA |  |  |
| --- | --- | --- | --- |
| Description |  |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 1 bit |  |  |  |
| Special/Reserved Values | 0 | No End section timer information |  |
|  | 1 | End section timer information to follow |  |

#### Q_FRONT

| Name | Qualifier for validity end point of profile element |  |  |
| --- | --- | --- | --- |
| Description | Qualifier to indicate if a speed limit given for a profile element is to be applied until the front of the train (no train length delay) or the end of the train (train length delay) has left the element. |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 1 bit |  |  |  |
| Special/Reserved Values | 0 | Train length delay on validity end point of profile element. |  |
|  | 1 | No train length delay on validity end point of profile element |  |

#### Q_GDIR

| Name | Qualifier for gradient slope. |  |  |
| --- | --- | --- | --- |
| Description |  |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 1 bit |  |  |  |
| Special/Reserved Values | 0 | downhill |  |
|  | 1 | uphill |  |

#### Q_INFILL

| Name | Qualifier to indicate whether a train is entering or exiting the radio infill area. |  |  |
| --- | --- | --- | --- |
| Description |  |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 1 bit |  |  |  |
| Special/Reserved Values | 0 | Enter |  |
|  | 1 | Exit |  |

#### Q_LENGTH

| Name | Qualifier for train integrity status |  |  |
| --- | --- | --- | --- |
| Description | Qualifier, identifying the train integrity information available. The related safe train length information is given by L_TRAININT |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 2 bits |  |  |  |
| Special/Reserved Values | 0 | No train integrity information available |  |
|  | 1 | Train integrity confirmed by integrity monitoring device |  |
|  | 2 | Train integrity confirmed by driver |  |
|  | 3 | Train integrity lost |  |

#### Q_LGTLOC

| Name | Qualifier for the specified report location |  |  |
| --- | --- | --- | --- |
| Description | This qualifier tells whether the train has to report its location when the max safe front end or when the min safe rear end has over passed the location defined by D_LOC |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 1 bit |  |  |  |
| Special/Reserved Values | 0 | Min safe rear end |  |
|  | 1 | Max safe front end |  |

#### Q_LINK

| Name | Link Qualifier |  |  |
| --- | --- | --- | --- |
| Description | This qualifier is used to mark a balise group as linked or unlinked. |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 1 bit |  |  |  |
| Special/Reserved Values | 0 | Unlinked |  |
|  | 1 | Linked |  |

#### Q_LOCACC

| Name | Accuracy of the balise location |  |  |
| --- | --- | --- | --- |
| Description | This Qualifier defines the absolute value of the accuracy of the Balise location (i.e., the value 63m identifies a location accuracy of +/- 63m) |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 6 bits | 0 m | 63 m | 1 m |

#### Q_LINKORIENTATION

| Name | Qualifier for the direction of the linked balise group |  |  |
| --- | --- | --- | --- |
| Description | Indicates whether the linked balise group will be overpassed by the train in nominal or reverse direction. |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 1 bit |  |  |  |
| Special/Reserved Values | 0 | The balise group is seen by the train in reverse direction |  |
|  | 1 | The balise group is seen by the train in nominal direction |  |

#### Q_LINKREACTION

| Name | linking reaction |  |  |
| --- | --- | --- | --- |
| Description | Qualifier for the reaction to be performed if a linking or a balise group message consistency problem occurs with the balise group linked to. |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 2 bits |  |  |  |
| Special/Reserved Values | 00 | Train trip |  |
|  | 01 | Apply service brake |  |
|  | 10 | No Reaction |  |
|  | 11 | Spare |  |

#### Q_LOOPDIR

| Name | Qualifier to indicate the direction of the loop |  |  |
| --- | --- | --- | --- |
| Description | Indicates LOOP-reference direction in relation to EOLM direction |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 1 bit |  |  |  |
| Special/Reserved Values | 0 | Opposite |  |
|  | 1 | Same |  |

#### Q_MEDIA

| Name | Qualifier to indicate the type of media |  |  |
| --- | --- | --- | --- |
| Description | Indicates whether it is a balise telegram or a loop message |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 1 bit |  |  |  |
| Special/Reserved Values | 0 | Balise |  |
|  | 1 | Loop |  |

#### Q_MPOSITION

| Name | Qualifier for track kilometre direction. |  |  |
| --- | --- | --- | --- |
| Description | Qualifier to indicate the direction of counting of the geographical position track kilometre in relation to the geographical position reference balise group directionality. |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 1 bit | 0 | 1 |  |
| Special/Reserved Values | 0 | Opposite (counting downwards if passed in nominal direction or counting upwards if passed in reverse direction) |  |
|  | 1 | Same (counting upwards if passed in nominal direction or counting downwards if passed in reverse direction) |  |

#### Q_NEWCOUNTRY

| Name | New Country Qualifier |  |  |
| --- | --- | --- | --- |
| Description | Qualifier to indicate whether the next balise group is in the same country / railway administration as the one before inside the packet or not. For the first balise group in the packet, if Q_NEWCOUNTRY = 0, it is the same country / railway administration as the one of the LRBG within the radio message or the one of balise group within the balise telegram giving the packet. |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 1 bit |  |  |  |
| Special/Reserved Values | 0 | Same country / railway administration, no NID_C follows |  |
|  | 1 | Not the same country / railway administration, NID_C follows |  |

#### Q_NVDRIVER_ADHES

| Name | Qualifier for the modification of trackside adhesion factor by driver |  |  |
| --- | --- | --- | --- |
| Description | This variable is part of the National Values |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 1 bit |  |  |  |
| Special/Reserved Values | 0 | Not allowed |  |
|  | 1 | Allowed |  |

#### Q_NVEMRRLS

| Name | Qualifier Emergency Brake Release |  |  |
| --- | --- | --- | --- |
| Description | Permission to release the emergency brake immediately if the condition why the system has triggered the emergency brake (speed exceeds emergency brake intervention limit, lack of driver reaction) is not fulfilled any more. Note: The action to release the brakes (e.g., by pressing a button) is a different issue and depends on the braking system This variable is part of the National Values |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 1 bit |  |  |  |
| Special/Reserved Values | 0 | Release only at standstill possible |  |
|  | 1 | Immediate release possible |  |

#### Q_NVSRBKTRG

| Name | Permission to use service brake when braking to a target is supervised |  |  |
| --- | --- | --- | --- |
| Description | This variable is part of the National Values |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 1 bit |  |  |  |
| Special/Reserved Values | 0 | No |  |
|  | 1 | Yes |  |

#### Q_ORIENTATION

| Name | Co-ordinate system assigned to a single balise group |  |  |
| --- | --- | --- | --- |
| Description | The co-ordinate system is assigned by the RBC to a balise group reported by the on-board equipment as LRBG. The information reverse/nominal (i.e., the assigned co-ordinate system) is given in relation to the direction in which the balise has been passed when reading it. |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 1 bit |  |  |  |
| Special/Reserved Values | 0 | The balise group has been passed by the train in reverse direction |  |
|  | 1 | The balise group has been passed by the train in nominal direction |  |

#### Q_OVERLAP

| Name | Qualifier to tell whether there is an overlap |  |  |
| --- | --- | --- | --- |
| Description | This variable is set to 1 if either an overlap exists or a release speed has to be specified |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 1 bit |  |  |  |
| Special/Reserved Values | 0 | No overlap information |  |
|  | 1 | Overlap information to follow |  |

#### Q_RBC

| Name | Qualifier for communication session order |  |  |
| --- | --- | --- | --- |
| Description |  |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 1 bit |  |  |  |
| Special/Reserved Values | 0 | Terminate communication session |  |
|  | 1 | Establish communication session |  |

#### Q_RIU

| Name | Qualifier for communication session order |  |  |
| --- | --- | --- | --- |
| Description |  |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 1 bit |  |  |  |
| Special/Reserved Values | 0 | Terminate communication session |  |
|  | 1 | Establish communication session |  |

#### Q_SCALE

| Name | Qualifier for the distance scale. |  |  |
| --- | --- | --- | --- |
| Description | Qualifier to indicate the same scale used for describing all distances inside the packet that contains Q_SCALE. |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 2 bits |  |  |  |
| Special/Reserved Values | 0 | 10 cm scale |  |
|  | 1 | 1 m scale |  |
|  | 2 | 10 m scale |  |
|  | 3 | Spare |  |

#### Q_SECTIONTIMER

| Name | Qualifier to indicate whether there is a Section Time Out related to the section |  |  |
| --- | --- | --- | --- |
| Description |  |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 1 bit |  |  |  |
| Special/Reserved Values | 0 | No Section Timer information |  |
|  | 1 | Section Timer information to follow |  |

#### Q_SLEEPSESSION

| Name | Session management for sleeping equipment |  |  |
| --- | --- | --- | --- |
| Description | Order for a Sleeping onboard equipment to consider or not the “session management” information |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 1 bit |  |  |  |
| Special/Reserved Values | 0 | Ignore session management information |  |
|  | 1 | Consider session management information |  |

#### Q_SRSTOP

| Name | “Stop if in Staff Responsible” information |  |  |
| --- | --- | --- | --- |
| Description | Specifies whether an onboard equipment in staff responsible has to stop or not |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 1 bit |  |  |  |
| Special/Reserved Values | 0 | Stop if in SR mode |  |
|  | 1 | Go if in SR mode |  |

#### Q_SSCODE

| Name | Spread Spectrum Code for Euroloop |  |  |
| --- | --- | --- | --- |
| Description | Specifies the code required to receive telegrams from a specific Euroloop installation. |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 4 bits | 0 | 15 |  |
| Special/Reserved Values | 15 | Code reserved for test purposes |  |

#### Q_STATUS

| Name | status of SoM position report |  |  |
| --- | --- | --- | --- |
| Description | It provides the status of the position report |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 2 bits |  |  |  |
| Special/Reserved Values | 00 | Invalid |  |
|  | 01 | Valid |  |
|  | 10 | Unknown |  |
|  | 11 | spare |  |

#### Q_SUITABILITY

| Name | Type of route suitability data |  |  |
| --- | --- | --- | --- |
| Description |  |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 2 bits |  |  |  |
| Special/Reserved Values | 00 | Loading gauge profile |  |
|  | 01 | Max axle load |  |
|  | 10 | Traction power |  |
|  | 11 | Spare |  |

#### Q_TEXT (Values to be assigned according to 7.3.1.3)

| Name | Fixed message to be displayed. |  |  |
| --- | --- | --- | --- |
| Description | Q_TEXT is a pointer to select a fixed text message from the defined table. The language selected by the driver for the MMI shall be used additionally as a qualifier to choose the appropriate language table. |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 8 bits | 0 | 255 |  |
| Special/Reserved Values |  |  |  |

#### Q_TEXTCLASS

| Name | Class of message to be displayed. |  |  |
| --- | --- | --- | --- |
| Description | Q_TEXTCLASS specifies the class of the text message included in the same packet (either plain or fixed message) |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 2 bits |  |  |  |
| Special/Reserved Values | 00 | Auxiliary Information |  |
|  | 01 | Important Information |  |
|  | 10 | Spare |  |
|  | 11 | Spare |  |

#### Q_TEXTCONFIRM

| Name | Qualifies the need / reaction of text confirmation |  |  |
| --- | --- | --- | --- |
| Description |  |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 2 bits |  |  |  |
| Special/Reserved Values | 00 | No confirmation required |  |
|  | 01 | Continue display until confirmed |  |
|  | 10 | Apply service brake if not confirmed when end conditions reached |  |
|  | 11 | Spare |  |

#### Q_TEXTDISPLAY

| Name | Qualifier for the combination of text message conditions |  |  |
| --- | --- | --- | --- |
| Description | Q_TEXTDISPLAY defines whether the start/end conditions for text message are to be combined or not |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 1 bit |  |  |  |
| Special/Reserved Values | 0 | No, display as soon as / until one of the conditions is fulfilled |  |
|  | 1 | Yes, display as soon as / until all conditions are fulfilled |  |

#### Q_TRACKDEL

| Name | Track description deleted. |  |  |
| --- | --- | --- | --- |
| Description | Qualifier to indicate whether the onboard has deleted (for any reason) track description or not. |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 1 bit |  |  |  |
| Special/Reserved Values | 0 | No track description deleted |  |
|  | 1 | Track description deleted |  |

#### Q_TRACKINIT

| Name | Qualifier for resuming the initial states of the related track description of the packet. |  |  |
| --- | --- | --- | --- |
| Description |  |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 1 bit |  |  |  |
| Special/Reserved Values | 0 | No initial states to be resumed, profile to follow |  |
|  | 1 | Empty profile, initial states to be resumed |  |

#### Q_UPDOWN

| Name | Balise telegram transmission direction |  |  |
| --- | --- | --- | --- |
| Description | It defines the direction of the information in the balise telegram |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 1 bit |  |  |  |
| Special/Reserved Values | 0 | Down link telegram |  |
|  | 1 | Up link telegram |  |

#### T_CYCLOC

| Name | Time Interval between two location reports sent by the train |  |  |
| --- | --- | --- | --- |
| Description | The train must send its location every T_CYCLOC |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 8 bits | 0 seconds | 254 s | 1s |
| Special/Reserved Values | 255 |  |  |

#### T_CYCRQST

| Name | Time between two cyclic requests for a movement authority |  |  |
| --- | --- | --- | --- |
| Description | When the train asks for a movement authority request, it will repeat its request every T_CYCRQST seconds until it receives a new MA |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 8 bits | 0 seconds | 254 s | 1s |
| Special/Reserved Values | 0 | Leads to continuous request |  |
|  | 255 | No repetition |  |

#### T_ENDTIMER

| Name | Validity time for the End section in the MA |  |  |
| --- | --- | --- | --- |
| Description | Time for which the End section is valid measured from the moment the train reaches the location defined by D_ENDTIMERSTARTLOC. |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 10 bits | 0 | 1022 | 1 s |
| Special/Reserved Values | 1023 |  |  |

#### T_LOA

| Name | Validity time for the target speedat the LOA |  |  |
| --- | --- | --- | --- |
| Description | Time for which the target speed is valid measured from the moment information is received |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 10 bits | 0 | 1022 | 1 s |
| Special/Reserved Values | 1023 |  |  |

#### T_MAR

| Name | Time before reaching indication limit for the EOA/LOA |  |  |
| --- | --- | --- | --- |
| Description |  |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 8 bits | 0 | 254 | 1 s |
| Special/Reserved Values | 255 | Not relevant |  |

#### T_NVCONTACT

| Name | Maximal time without new “safe” message. |  |  |
| --- | --- | --- | --- |
| Description | If no “safe” message has been received from the track for more than T_NVCONTACT seconds, an appropriate action according to M_NVCONTACT must be triggered. This variable is part of the National Values |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 8 bits | 0s | 254s | 1s |
| Special/Reserved Values | 1111 1111 | T_NVCONTACT = . |  |

#### T_NVOVTRP

| Name | Maximum time for overriding the train trip |  |  |
| --- | --- | --- | --- |
| Description | This variable is part of the National Values |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 8 bits | 0 s | 255 s | 1 s |
| Special/Reserved Values |  |  |  |

#### T_OL

| Name | Overlap validity time |  |  |
| --- | --- | --- | --- |
| Description | The time span the train can expect the overlap to be available, measured from the moment the train reaches the location defined by D_STARTOL. |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 10 bits | 0 | 1022 s | 1 s |
| Special/Reserved Values | 1023 |  |  |

#### T_SECTIONTIMER

| Name | Validity time of a section in the MA |  |  |
| --- | --- | --- | --- |
| Description | Time for which the section is valid. |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 10 bits | 0 | 1022 | 1 s |
| Special/Reserved Values | 1023 |  |  |

#### T_TEXTDISPLAY

| Name | Time until when a text shall be displayed |  |  |
| --- | --- | --- | --- |
| Description |  |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 10 bits | 0 | 1022 s | 1 s |
| Special/Reserved Values | 1023 | Display of text not limited by time. |  |

#### T_TIMEOUTRQST

| Name | Time before any section timer expires |  |  |
| --- | --- | --- | --- |
| Description |  |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 10 bits | 0 | 1022 | 1 s |
| Special/Reserved Values | 1023 | Not relevant |  |

#### T_TRAIN

| Name | Trainborne clock |  |  |
| --- | --- | --- | --- |
| Description | Time, according to trainborne clock, at which message is sent |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 32 bits | 0 | 42949672.94 s | 10 ms |
| Special/Reserved Values | 4294967295 | Unknown |  |

#### V_AXLELOAD

| Name | Speed restriction related to axleload |  |  |
| --- | --- | --- | --- |
| Description |  |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 7 bits | 0 km/h | 600 km/h | 5 km/h |
| Special/Reserved Values | 121 –127 | Spare |  |

#### V_DIFF

| Name | Absolute Positive Speed associated to a train category. |  |  |
| --- | --- | --- | --- |
| Description |  |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 7 bits | 0 km/h | 600 km/h | 5 km/h |
| Special/Reserved Values | 121 – 127 | Spare |  |

#### V_LOA

| Name | Permitted speed at the limit of authority |  |  |
| --- | --- | --- | --- |
| Description |  |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 7 bits | 0 km/h | 600 km/h | 5 km/h |
| Special/Reserved Values | 121-127 | Spare |  |

#### V_MAIN

| Name | Signalling related speed restriction |  |  |
| --- | --- | --- | --- |
| Description |  |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 7 bits | 0 km/h | 600 km/h | 5 km/h |
| Special/Reserved Values | 121-127 | Spare |  |
|  | V_MAIN = 0 means “trip order” |  |  |

#### V_MAMODE

| Name | Required mode related speed |  |  |
| --- | --- | --- | --- |
| Description |  |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 7 bits | 0 | 600 km/h | 5 km/h |
| Special/Reserved Values | 121 – 126 | Spare |  |
|  | 127 | Use the national speed value of the required mode |  |

#### V_MAXTRAIN

| Name | Maximum permitted train speed. |  |  |
| --- | --- | --- | --- |
| Description | Maximum permitted speed for the train, taking into account the maximum speed of every vehicle contained in the train set. |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 7 bits | 0 | 600 km/h | 5 km/h |
| Special/Reserved Values | 121 – 127 | Spare |  |

#### V_NVALLOWOVTRP

| Name | Maximum speed limit allowing the driver to select the “override EOA” function |  |  |
| --- | --- | --- | --- |
| Description | This variable is part of the National Values |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 7 bits | 0 km/h | 600km/h | 5 km/h |
| Special/Reserved Values | 121 – 127 | Spare |  |

#### V_NVONSIGHT

| Name | On Sight mode (permitted) speed limit |  |  |
| --- | --- | --- | --- |
| Description | This variable is part of the National Values |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 7 bits | 0 km/h | 600 km/h | 5 km/h |
| Special/Reserved Values | 121-127 | Spare |  |

#### V_NVSUPOVTRP

| Name | Permitted speed limit to be supervised when the “override EOA” function is active |  |  |
| --- | --- | --- | --- |
| Description | This variable is part of the National Values |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 7 bits | 0 km/h | 600km/h | 5 km/h |
| Special/Reserved Values | 121 – 127 | Spare |  |

#### V_NVREL

| Name | Release Speed (permitted) speed limit |  |  |
| --- | --- | --- | --- |
| Description | This variable is part of the National Values |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 7 bits | 0 km/h | 600 km/h | 5 km/h |
| Special/Reserved Values | 121-127 | Spare |  |

#### V_NVSHUNT

| Name | Shunting mode (permitted) speed limit |  |  |
| --- | --- | --- | --- |
| Description | This variable is part of the National Values |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 7 bits | 0 km/h | 600 km/h | 5 km/h |
| Special/Reserved Values | 121-127 | Spare |  |

#### V_NVSTFF

| Name | Staff Responsible mode (permitted) speed limit |  |  |
| --- | --- | --- | --- |
| Description | This variable is part of the National Values |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 7 bits | 0 km/h | 600 km/h | 5 km/h |
| Special/Reserved Values | 121-127 | Spare |  |

#### V_NVUNFIT

| Name | Unfitted mode (permitted) speed limit |  |  |
| --- | --- | --- | --- |
| Description | This variable is part of the National Values |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 7 bits | 0 km/h | 600 km/h | 5 km/h |
| Special/Reserved Values | 121-127 | Spare |  |

#### V_RELEASEDP

| Name | Release speed associated with the danger point |  |  |
| --- | --- | --- | --- |
| Description |  |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 7 bits | 0 km/h | 600 km/h | 5 km/h |
| Special/Reserved Values | 121-125 | Spare |  |
|  | 126 | Use onboard calculated release speed |  |
|  | 127 | Use national value |  |

#### V_RELEASEOL

| Name | Release speed associated with the overlap |  |  |
| --- | --- | --- | --- |
| Description |  |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 7 bits | 0 km/h | 600 km/h | 5 km/h |
| Special/Reserved Values | 121-125 | Spare |  |
|  | 126 | Use onboard calculated release speed |  |
|  | 127 | Use national value |  |

#### V_REVERSE

| Name | Maximum speed in RV mode |  |  |
| --- | --- | --- | --- |
| Description |  |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 7 bits | 0 km/h | 600 km/h | 5 km/h |
| Special/Reserved Values | 121-127 | Spare |  |

#### V_STATIC

| Name | Static speed profile |  |  |
| --- | --- | --- | --- |
| Description | Static speed profile speed after discontinuity (k). |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 7 bits | 0 | 600 km/h | 5 km/h |
| Special/Reserved Values | 121-126 | Spare |  |
|  | 127 | Non numerical value telling that the static speed profile description ends at D_STATIC(n) |  |

#### V_TRAIN

| Name | Actual Train speed |  |  |
| --- | --- | --- | --- |
| Description |  |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 7 bits | 0 | 600 km/h | 5 km/h |
| Special/Reserved Values | 121-127 | Spare |  |

#### V_TSR

| Name | Permitted speed for the temporary speed restriction |  |  |
| --- | --- | --- | --- |
| Description |  |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 7 bits | 0 km/h | 600 km/h | 5 km/h |
| Special/Reserved Values | 121 – 127 | Spare |  |

#### X_TEXT

| Name | Text String Element |  |  |
| --- | --- | --- | --- |
| Description | Text strings are used to transmit plain text messages. Each element of a text string contains a single character encoded as ISO 8859-1, also known as Latin Alphabet #1. |  |  |
| Length of variable | Minimum Value | Maximum Value | Resolution/formula |
| 8 bits |  |  |  |
| Special/Reserved Values |  |  |  |