| - ERTMS/ETCS – Class 1 |  |
| --- | --- |
| Interoperability-related consolidation on TSI annex A documents |  |
| REF : | SUBSET-108 |
| ISSUE : | 1.2.0 |
| DATE : | 17.01.2008 |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| 0.0.1 20-02-2005 |  | Created | JN |
| 0.0.9 18-03-2005 |  | Conclusion from SG meeting | B. Stamm |
| 0.1.0 07-04-2005 |  | Consolidated version with comments from SG included | B. Stamm |
| 0.1.1 29-04-2005 |  | Corrections after SG meeting | B. Stamm |
| 0.1.2 01-06-2005 |  | Corrections after SG meeting | B. Stamm |
| 0.1.3 20-06-2005 |  | Corrections after SG meeting | B. Stamm |
| 0.1.4 12-07-2005 |  | Corrections after SG meeting to include C2007 agreements | B. Stamm |
| 0.1.5 02-09-2005 |  | Corrections after SG meeting with EEIG | B. Stamm |
| 0.1.6 27-09-2005 |  | Corrections after SG meeting, based on EEIG responses after joint meeting on 01-09-2005 | B. Stamm |
| 0.1.7 28-09-2005 |  | Corrections during SG meeting and discussion with EEIG | B. Stamm |
| 0.1.8 28-09-05 |  | Corrections during SG meeting and discussion with EEIG regards clause 3.3.1.13 and CRs 514, 519 (back to OUT) | H. Kast |
| 0.1.9 29-09-05 |  | Cleaning up historical comments in column "Note" of CR list regards CR6, 36, 172, 182, 387, 441, 452, 458, 645 | H. Kast |
| 0.1.10 30-09-05 |  | In chapter 7: CR database added, in 3.3.1.5 statement added: OUT regards 3.0.0, docs Subset 076-5-2, 076-6-3 are no INs , all docs not being IN removed from table in chapter 6. | H. Kast |
| 18.10.05 1.0.0 | 3.2.1.3, 3.2.1.6. 3.3.1.2, 3.3.1.5b),d), 3.3.1.11, 4.1.1.1. 4.4.1.3, 4.4.1.4, 5.1, 51.1.2, 6 | Modifications due to comments received (EEIG, UNISIG internal) | H. Kast |
| 11.05.06 1.0.1 | 3.2.1.5, 4.1.1.4, 6, new chapter 7 | Modifications to support use of SUBSET-026 version 2.3.0. | A. Hougardy |
| 08.06.06 1.1.0 |  | Release version endorsed by ERA CG 08/06/06 | A. Hougardy |
| 22.10.07 1.1.1 | All | General update, following the inclusion of new list of IN CRs | A. Hougardy |
| 07.11.07 1.1.2 | 3.2.1.1, 3.2.1.7, 3.3.1.4, 3.3.1.7, 4.1.1.4 | Modifications due to comments received | A. Hougardy |
| 19.11.07 1.1.3 | All | Modifications according to ERA CG 08/11/07 | A. Hougardy |
| 28.11.07 1.1.4 | All | Modifications due to comments received and ERA CCB 27/11/07 | A. Hougardy |
| 17.01.08 1.2.0 | Table 4.1.1.4 Section 6.2 | Modifications following impact assessment phase, agreed by ERA CG 10/01/08: - removal of not classified CR182, 241, 291, 331, 395 - reclassification of CR 782 (IN => not classified) - modification of CR601, 781, 792 Modifications following editorial/quality final check between 14/01/08 and 17/01/08: - removal of not classified CR74, 552 - editorial cleaning of CR729 solution - modification of CR718, to be consistent with reclassification of CR782. | A. Hougardy |

# Table of Contents

1.	Modification History	2

2.	Table of Contents	4

3.	Scope and purpose	5

3.1	Terminology	5

3.2	Introduction	5

3.3	Scope	6

3.4	Limitations of scope	7

4.	Classification of CR’s	8

5.	additional issues without related CR’s	18

6.	Decisions for the CR’s, qualified as IN	19

6.1	Remainder from previous list of IN CR’s	19

6.2	New IN CR’s	46

# Scope and purpose

## Terminology

#### Change Request: A task in the Change Process to correct an error in, clarify the interpretation of or add a function to the existing ERTMS/ETCS specification.

#### Note: At start, a Change Request carries the request to modify some subset document(s). Through an analysis, the final state of a Change Request contains decisions about modifications of subset document(s). This final state might differ from the initial change proposal but should solve the initially stated reason for the change request.

#### Baseline: The set of documents listed in a specific release of the TSI, named after the corresponding version number of the SRS (e.g. baseline 2.2.2 is SRS 2.2.2 plus accompanying documents).

## Introduction

#### Intentionally deleted.

#### The publication of a set of Change Requests in the previous versions of this document was meant to modify requirements in the SRS, version 2.2.2, as well as in other sub-documents. This led to the creation of the SRS 2.3.0 and to the baseline 2.3.0. Meanwhile, the additional return of experience gained from further ETCS projects has prompted the review of the Change Requests not classified as “IN” in the previous version of this document and the need to identify the newer Change Requests that are also necessary to avoid adverse effects on interoperability.

#### In this version of the subset-108:

- The Change Requests that were already classified as “IN” in the previous version, whose implementation is completely carried out in the SRS 2.3.0, are removed,

- The Change Requests that were already classified as “IN” in the previous version, but affect other documents than the SRS, are kept in the present version of this document as they remain applicable to form the baseline 2.3.0

#### Intentionally deleted.

#### Those CR’s that are part of baseline 2.3.0 and listed in this document are to be considered with priority over the other documents listed in the TSI annex A, which means the changes listed in the CR decision/solution are replacing the corresponding paragraphs of SRS 2.3.0 or accompanying documents.

##### Note: the SRS version 2.3.0 already includes the changes to be in line with all the CRs classified as “IN” in the previous version of subset-108.

#### The UNISIG CR database contained a description of each CR, including the “problem headline”, “problem description” and “solution proposal by submitter”, all part of the original submitters CR, and a “decision by CR-board” agreed through the former CCM process. Often the final decision differed significantly from the submitter’s proposal. All of the above remains in the description of those CR’s in the UNISIG database that are kept in the present version of this document (see section 6.1), for traceability reasons; however only the decision by the CR-board shall be taken into account.

#### The extracted content of the ERA CR database being accessible to all the representative organisations listed in the frame of the ERA CCM, for the new CR’s only the fields “Identification number”, “Headline” and “Agreed Solution” are published in this document (see section 6.2).

## Scope

#### The goal of this new version of the document is to consolidate the baseline 2.3.0, after the return of experience from both, operators and suppliers on ERTMS current projects, adding in the 2.3.0 baseline the necessary error corrections. ..

#### This document classifies the change requests in order to:

- close safety issues;

- reach an acceptable level of technical / operational interoperability.

#### "acceptable level" means that minor differences in behaviour both from technical and operational point of view might be experienced when using in products from different suppliers in a project. Nevertheless, the system will be interoperable in the sense of usability.

#### Chapter 4 of this document lists CR’s logged in the ERA database at the time of releasing this document, which are neither in the state “rejected” nor in the state “superseded”. They are classified as:

- In, which means error CR’s that shall be implemented in baseline 2.3.0 based projects. These are typically CR’s that are required due to safety reasons or that correct errors that make existing functions unusable or implementations from different suppliers incompatible.

- Out, for error CR’s that shall not be implemented in baseline 2.3.0 based projects, but included in baseline 3.0.0, as they make interoperability impossible when not implemented everywhere. These are typically CR’s where no harmonised solution is required in the current projects. 
National solution for the problem is allowed as long as technical interoperability is not jeopardised.

- Not applicable, for CR’s which request new functionality foreseen for a further baseline; their incorporation in the baseline 3.0.0 is to be decided in the frame of the ERA CCM process.

- Not applicable (beyond 2.3.0) refers to CRs which request new functionality out of scope of the baseline 2.3.0, only in case this new functionality is considered compatible with 2.3.0. Which means they could be implemented on a contractual basis earlier than 3.0.0.

- Note: Independent of the decision to introduce the CR or not, the functionality based on version 2.3.0 is still valid.

#### Some minor CR’s have been classified as “In” in order to get a coherent set of "In” CR’s.

#### Those CR’s that have been rejected and closed or that have been superseded are not contained in the list below.

#### Those CR’s that are not classified in the list below have been assessed not to pose an interoperability risk, so whether they are implemented already with 2.3.0 or only with 3.0.0 is left open. If they are implemented however, this shall be done according to the solution published by ERA. In particular, the implementation of those CR’s cannot be an access condition to the infrastructure of the Trans European network.

#### Chapter 5 lists additional issues to be resolved when implementing projects according to baseline 2.3.0, that are however not included in a CR.

#### STM, DMI and JRU related issues are not consolidated as the level of technical and operational interoperability aimed at can be reached without them.

#### Only onboard versus trackside interoperability has been covered.

#### M_VERSION shall remain unchanged (still 1.0), as all the CR’s marked as IN consist of error correction, in the frame of the maintenance of the baseline 2.3.0.

## Limitations of scope

#### This document does not define how or when to implement and deploy the amended requirements into projects.

# Classification of CR’s

#### The following list contains all Change Requests that have been classified in regard to their impact on interoperability of equipment from different suppliers.

#### CR’s, which have been rejected, superseded or withdrawn, are not listed below.

#### Definition of abbreviations:

NA = Not Applicable

IN = included in baseline 2.3.0

OUT = forbidden in baseline 2.3.0

#### Table of CR’s:

| CR | Headline | Cl | Note |
| --- | --- | --- | --- |
| 5 | Message names |  |  |
| 7 | Crossing GSM-R network border | IN | Only the parts affecting Subset-054 v2.0.0 |
| 10 | Message flow during termination of a communication session |  |  |
| 12 | Supervision of sequence |  |  |
| 15 | NID-C description |  |  |
| 16 | Network address format | IN | Only the parts affecting Subset-040 v2.0.0 and Subset-054 v2.0.0 |
| 20 | Pending communication session | IN |  |
| 30 | Editorial clarifications |  |  |
| 31 | Clarification of terms |  |  |
| 34 | permitted maximum number of balise groups in the packet "Linking" | IN | Affects Subset-040 v2.0.0 |
| 38 | Meaning of M_ERROR special values | OUT |  |
| 40 | position report in case of passing an unlinked balise group |  |  |
| 44 | specific output in case of braking due to unexpected balise messages |  |  |
| 47 | missing transition from TR, PT to SL |  | Pure onboard function, provides a way out which would otherwise be a deadlock |
| 49 | Telegram or Message Counter |  |  |
| 51 | Position Report in RV mode | IN |  |
| 56 | Reserved values in M DRIVERACTIONS |  |  |
| 57 | Mode related speed restriction in SH |  |  |
| 58 | comment NID_STM in packet 72 |  |  |
| 66 | EEIG ref 02Q083: Non relevant information for driver | IN |  |
| 68 | EEIG ref 02Q089: Reporting mode changes to the RBC |  |  |
| 69 | EEIG ref 02Q090: Acceptance of "List of balises for SH areas" in SH mode | NA |  |
| 70 | EEIG ref 02Q079: Acknowledgement of STM transitions | NA |  |
| 71 | EEIG ref 02Q074: Insertion of packet 0 in all messages from train to RBC |  |  |
| 76 | EEIG ref 02Q068: Splitting of an ERTMS train |  |  |
| 78 | EEIG ref 02Q070: Odometry inaccuracy at exit of speed restriction | IN |  |
| 83 | Transition to non fitted SE/SN |  |  |
| 90 | High Priority channel (3) |  |  |
| 98 | Handling of Default Balise Telegram | IN | Only the parts affecting Subset-033 v2.0.0 |
| 99 | MMI versus internal information |  |  |
| 100 | MMI versus mode table |  |  |
| 101 | Permitted speed in RV |  |  |
| 103 | There is no acceleration data specified as specific train data. |  |  |
| 105 | Availability of Acknowledgement button |  |  |
| 107 | Output information |  |  |
| 119 | Non leading is ended |  |  |
| 123 | EEIG ref 02Q064: TSR | NA |  |
| 124 | EEIG ref 02Q073: Passing a signal in SR mode | NA |  |
| 128 | Short number programmed in balise identification. |  |  |
| 130 | Confusing definition of Tamt. |  |  |
| 132 | Waiting for train data ack by RBC |  |  |
| 133 | Requirement doesn't belong to the chapter |  |  |
| 134 | L_MAMODE for a shunting area |  |  |
| 137 | Q EMERGENCYSTOP definition |  |  |
| 138 | Brakes release after max reversing distance overpassed | IN |  |
| 140 | Reversing distance sent by the RBC |  |  |
| 141 | Geographical position in SH |  |  |
| 142 | Position report in level transition |  |  |
| 146 | EEIG ref 033: TSR s | IN | Only the parts affecting Subset-040 v2.0.0 |
| 148 | EEIG ref 035: Leaving Isolation mode |  |  |
| 149 | EEIG ref 036: TIMS data missing | NA |  |
| 151 | EEIG ref 038: Eddy current brake switch off | IN | Only the parts affecting Subset-034 v2.0.0 |
| 154 | EEIG ref 044: Text message inconsistency | IN |  |
| 157 | EEIG ref 047: Valid direction for Reverse Movement Protection in SR |  |  |
| 166 | EEIG ref 058: NID OPERATIONAL |  |  |
| 168 | EEIG ref 060: MA request |  |  |
| 169 | EEIG ref 061: Non stopping information | NA |  |
| 170 | EEIG ref 062: Indication of track conditions | IN |  |
| 177 | Loss of End of Profile elements |  |  |
| 180 | Braking without indication of reason at standstill supervision |  |  |
| 183 | Data from single balise groups | IN |  |
| 186 | Message 42 further deletions |  |  |
| 187 | EoMission during RBC Handover | IN |  |
| 198 | Revocation of Emergency Stop in SR |  |  |
| 201 | Supervision of radio link : unclear requirement |  |  |
| 202 | Memorised balise groups for special position report not mentioned in table 4.10.1.3 | IN |  |
| 203 | Default gradient of TSR forgotten |  |  |
| 204 | Report of train position when change of train orientation |  |  |
| 205 | Note to be removed |  |  |
| 215 | Infill and level transition | IN | Affects Subset-040 v2.0.0 |
| 216 | ambiguity of distance information in profile data |  |  |
| 223 | Table of Supported Levels must be restored at Start of Mission | IN |  |
| 232 | EEIG 043: Unknown text message |  |  |
| 235 | EEIG 065 Train data after loss of radio communication |  |  |
| 237 | EEIG 067 Service Brake T NVCONTACT |  |  |
| 238 | EEIG 069 Shifted reference location | IN |  |
| 239 | EEIG 070 Train data on TIU |  |  |
| 240 | EEIG 072 JRU changes |  |  |
| 242 | EEIG 074 ERTMS system version management |  |  |
| 247 | List of expected balises in SR mode |  |  |
| 248 | Minimum distance between the 1st balise of a balise group and the EoA | IN | Only the parts affecting Subset-040 v2.0.0 |
| 257 | Last eight reported BG identities stored on board with regard to deletion of stored position data in SoM procedure. | IN |  |
| 262 | Length of DRIVER ID |  |  |
| 264 | Indication of supervision limits in SR mode |  |  |
| 265 | Information to driver about reason of train trip |  |  |
| 268 | End of mission: terminate communication session, if comm. session exists | IN |  |
| 269 | Transition SB-->SN after ACK [58] |  |  |
| 284 | ETCS accepted information from STM X (SE) |  |  |
| 285 | Smooth transition and continuous supervision at the level Transition |  |  |
| 293 | End of Mission |  |  |
| 296 | EEIG 085 Linking reaction info to RBC. |  |  |
| 297 | EEIG 086 Override request |  |  |
| 298 | EEIG 087 Level selection by driver |  |  |
| 299 | EEIG 088 Version compatibility check | NA |  |
| 301 | ETCS functions access by the STM in SL Mode |  |  |
| 302 | Contradiction of dimensioning data |  |  |
| 309 | RBC transition order analog to level transition order |  |  |
| 312 | Level Transition at estimated - or maximum safe front end |  |  |
| 315 | Engineering (Track Condition; Route Suitability) | IN | Affects Subset-040 v2.0.0 |
| 317 | Failure of Packet 51 "ASP" | OUT |  |
| 319 | LRBG orientation for special position report | IN |  |
| 320 | Speed supervision behind LOA | IN |  |
| 336 | Storage of and text interpretation for Q_SLEEPSESSION |  |  |
| 338 | Some Errors in Active Functions Table |  |  |
| 344 | Loss of radio connection in announced radio hole | IN |  |
| 372 | EEIG095 FIS EVC - EIRENE Voice Radio | NA |  |
| 373 | EEIG097 Mixed level transition announcement |  |  |
| 374 | EEIG103 Reason for TR on DMI |  |  |
| 378 | EEIG111 Position accuracy | NA |  |
| 381 | Display of maximum shunting speed |  |  |
| 383 | Transmission media for packet 12 is not only balise |  |  |
| 390 | Balise consistency check in RV mode | IN |  |
| 392 | Ambiguity: SSP and gradient must cover "the full length of the MA" | IN |  |
| 394 | Definition of DriverAction "Not Used" in SUBSET027 |  |  |
| 396 | OBU can receive MA and other information after End of Mission | IN |  |
| 397 | Conditions for Start/End of Text Indication | IN |  |
| 399 | Message 153 |  |  |
| 400 | EEIG116 Management of UIC variables |  |  |
| 402 | When is linking used onboard |  |  |
| 403 | Ack of train data also in TR and PT |  |  |
| 406 | RBC Identification |  |  |
| 408 | Brake application and revocation by the same system |  |  |
| 410 | EEIG112 Shunting in STM areas | NA | beyond 2.3.0 |
| 411 | EEIG113 STM transition in trip situation |  |  |
| 412 | EEIG114 Brake command in NP mode |  |  |
| 413 | EEIG119 Level crossing modification | NA |  |
| 414 | Application rules for packets |  |  |
| 415 | Transition between STM levels |  |  |
| 416 | "No track condition information will be received" message to the driver in NL mode. |  |  |
| 418 | Target speed & distance output information to driver |  |  |
| 423 | Names NC_DIFF and V_DIFF |  |  |
| 426 | SRS reference missing |  |  |
| 433 | Linking reaction, logging thereof |  |  |
| 437 | Use of N_ITER | OUT |  |
| 438 | Mistake in Active Function Table for SH |  |  |
| 446 | Resuming initial state for non continuous profile data |  |  |
| 448 | Delete Cold Movement detector from TI FIS |  |  |
| 450 | Mode transition condition not correctly worded |  |  |
| 451 | Further changes with reference to CR 66 and CR 91 |  |  |
| 452 | Linking to unknown orientation is not possible |  |  |
| 453 | National/Default values |  |  |
| 455 | Position report sending when passing level transition location with train rear end |  |  |
| 459 | National values sent by radio during SoM | OUT |  |
| 461 | Inconsistency: Sent train data and accept Acknowledgement in NL |  |  |
| 463 | MA shortening due to change of train data |  |  |
| 466 | Transition from TR to UN |  |  |
| 469 | Completeness of data | IN | Only the parts affecting Subset-023 v2.0.0 |
| 471 | No acknowledgement of termination of a communication session received |  |  |
| 475 | Engineering rules extracted from Safety paper | IN | Affects Subset-040 v2.0.0 |
| 478 | EEIG ref 125: Establishment of Safe Connection - Retries |  |  |
| 480 | EEIG ref 127: Insufficient test spec for toggling |  |  |
| 481 | EEIG ref 134: Supervision of the Radio Link | NA |  |
| 482 | Balise detect function | OUT |  |
| 483 | Proposed Engineering rule for discussion | IN | Affects Subset-040 v2.0.0 |
| 485 | Review of SRS by STM WG |  |  |
| 487 | Impossibility to discriminate MA request reason | OUT |  |
| 488 | Geographical position handling in RV mode |  |  |
| 490 | Exc. permitted distance in PT mode |  |  |
| 491 | Acknowledgement of the Route Related Information |  |  |
| 492 | Missing information in the pre-announcement message |  |  |
| 493 | Removal of Emergency Stop data from Route Related Information |  |  |
| 494 | Communication of SR balise list on the RBC/RBC interface |  |  |
| 495 | Restriction of capacity within the Route Related Information Request |  |  |
| 497 | Repetition of information between an in-fill balise group and the related main balise group |  |  |
| 500 | Change of Train Data from sources different from the driver | IN |  |
| 502 | Loss of performance regarding release speed | NA |  |
| 503 | Network address format at fixed interface / interface to GSM-R |  |  |
| 505 | Train Orientation not well defined |  |  |
| 506 | Update of ETCS Safety Analyses needed |  |  |
| 507 | Unwanted mode transition during train trip inhibition | IN | Only the parts affecting Subset-040 v2.0.0 |
| 509 | Confusing reference in table 4.5.2 |  |  |
| 511 | Train speed indication in SB mode |  |  |
| 513 | EEIG080 Non Leading mode | NA | beyond 2.3.0 |
| 514 | EEIG ref 138: Cold movement detection | NA |  |
| 519 | EEIG148: Use of default values after NP mode | NA |  |
| 521 | Train location function for non leading and sleeping engines |  |  |
| 522 | Misleading sentences for RBC/RBC announcement. |  |  |
| 524 | Reporting of fatal error by Sleeping and Non leading engines. | OUT |  |
| 530 | Reporting of PT mode when no communication session established. | IN |  |
| 531 | Start of Mission procedure re-engaged when session is not terminated. | IN |  |
| 533 | EEIG124: Availability of speedometer display in IS mode | NA |  |
| 535 | EEIG152: Door control supervision | NA |  |
| 539 | EEIG096: Set speed indication for driver | NA |  |
| 540 | EEIG151: SR mode acknowledgement |  |  |
| 546 | No train data in shunting and non leading |  |  |
| 548 | Transition from TR to UN in level 0 when the onboard equipment was previously in SH mode. |  |  |
| 557 | Reset of MA request parameters and position report parameters |  |  |
| 558 | Wording inconsistencies for linking |  |  |
| 559 | Inconsistencies in procedure “Shunting initiated by driver” |  |  |
| 560 | No End of mission reporting when no communication established | IN |  |
| 561 | Availability of "Start" button in SE/SN mode |  |  |
| 563 | MA request if section timer expires |  |  |
| 564 | Missing Parameter in the Route Related Information message |  |  |
| 565 | Reporting / recording of balise errors | OUT |  |
| 579 | Target speed at target distance |  |  |
| 580 | Follow up from CR 257 |  |  |
| 582 | Permitted speed in UN mode |  |  |
| 583 | EEIG ref 068: Indications on DMI in SR/OS mode |  |  |
| 584 | EEIG ref 132: STM max speed |  |  |
| 586 | EEIG ref 137: DMI planning area | NA |  |
| 587 | EEIG ref 141: Track condition inhibit tilt | NA |  |
| 589 | EEIG154: JRU failure | NA |  |
| 591 | EEIG ref 169: Change of train length |  |  |
| 593 | EEIG ref 171: Awakening on loops | NA |  |
| 594 | EEIG ref 174: Speed definitions |  |  |
| 595 | EEIG ref 170: Braking curve calculation | NA | beyond 2.3.0 See note below |
| 597 | EEIG ref 184: Communication session in SL mode |  |  |
| 599 | EEIG ref 191: Reconnection after unexpected radio hole |  |  |
| 601 | EEIG ref 192: OS acknowledgement | IN |  |
| 604 | EEIG ref 181: Override procedure |  |  |
| 605 | Shortening of track description – SSP and gradient profile must not have “open end” |  |  |
| 607 | Shunting request information to be displayed in UN mode |  |  |
| 613 | Emergency stop revocation after level transition | IN |  |
| 614 | Informing the RBC when track description has been |  |  |
| 618 | Inconsistencies between Subset 035 and Subset 026 |  |  |
| 619 | Message Acknowledgement | IN |  |
| 625 | Backward movement after entry in FS/OS mode |  |  |
| 632 | Inconsistencies in stored/not stored info between A3.4 and 4.10 |  |  |
| 634 | Q_EMERGENCYSTOP variable not in line with CLR91 and CLR532 | OUT |  |
| 635 | Extension of MA following TAF not clear |  |  |
| 636 | System Start-up / Execution of Self-Tests |  |  |
| 637 | Limited Supervision | NA |  |
| 638 | Selection of Static Speed Profile according to Train Categories | OUT |  |
| 639 | Backward movement and already in FS/OS mode |  |  |
| 641 | EEIG ref 200: Local time |  |  |
| 642 | EEIG ref 201: Overpass EOA speed |  |  |
| 643 | Values of V_TRAIN and TRAIN_POSITION for event logging in the JRU |  |  |
| 649 | Inconsistency in position report specification and Active Functions Table |  |  |
| 650 | Train movement in L1 SB without Train Data |  |  |
| 651 | Availability of "Shunting request" input for driver |  |  |
| 652 | Impossibility to perform mode transition PT => FS/OS in Level 1 |  |  |
| 653 | Contradiction Level transition immediately in PT | IN |  |
| 654 | Unsuited wording of variable description |  |  |
| 655 | Level Transitions involving STM Levels | IN | Affects Subset-040 v2.0.0 |
| 656 | Follow-up of CR 126 | OUT | Results from split of 126 |
| 657 | Unsuitability of RBC-RBC handover procedure in case of radio network change | OUT | Results from split of 007 |
| 659 | Inconsistency in override procedure during SOM |  |  |
| 664 | Unclear requirements for Route suitability | IN |  |
| 682 | CR419 not in line with CR445 | IN |  |
| 686 | Ambiguities regarding reversing information in SRS v228 | IN |  |
| 688 | Consistency with old M_VERSION | IN |  |
| 695 | missing limitations for stored info in SUBSET-040 | IN |  |
| 699 | List of balises in SH deleted in A3.4 for situations a-f, g-j, l | IN |  |
| 702 | Update of list of expected balises in SR | IN |  |
| 706 | Mismatch between announced and read balise group direction | IN |  |
| 713 | PT distance D_NVPOTRP origin | IN |  |
| 718 | location related conditions vs reverse movement in PT or roll-back | IN |  |
| 729 | Single balises, assignment of a co-ordinate system | IN |  |
| 744 | National values | IN |  |
| 748 | Radio Link Supervision during RBC-RBC Handover | IN |  |
| 770 | Train category spare values | IN |  |
| 771 | Value NID_RBC is unknown | IN |  |
| 778 | Geographical position reference balise groups | IN |  |
| 781 | Relocation of data for MA in entry to Level 2 | IN |  |
| 782 | Reset of confidence interval |  |  |
| 787 | Activation of supervision of safe radio connection/ Follow-up CR529 | IN |  |
| 788 | Repetition of linking reactions | IN |  |
| 791 | Repositioning and cross talk | IN |  |
| 792 | Storage of information in case of level transition announcement or RBC/RBC handover | IN |  |
| 796 | Compatible upgrade | IN |  |
| 797 | Maximum length of radio message | IN |  |
| 798 | Mechanism for deletion of information on-board missing | IN |  |

Note for CR 595: 
The safety/ performance of the braking curves which are currently implemented in the ERTMS/ETCS on-board equipment is not fully defined by the current specification in 2.3.0 and therefore has to be assessed for each railway network on a national level. 
Within CR 595, significant issues are still open. When agreement has been reached within the ERA CCM process, the agreed braking curves and braking model may be implemented according to the decisions reached on CR 595.

# additional issues without related CR’s

#### The following functions shall not be used in the ETCS trackside for a line that is to be considered interoperable according to the baseline 2.3.0:

- None

#### The following functions shall only be used in the ETCS trackside with restrictions on a line that is to be considered interoperable according to the baseline 2.3.0:

- Specific SSP depending on international train categories: If at least one specific SSP is less restrictive than the Basic SSP, the trackside engineering has to ensure that for all possible combinations of international train categories a train might belong to, the ERTMS/ETCS on-board equipment will not replace the Basic SSP leading to an unsafe situation by applying the requirement 3.11.3.2.6 in Subset 026. Processing of this information by the onboard can lead to a non safe behaviour, i.e., the most restrictive speed profile, taking into account all speed limits relevant for the calculation of the Permitted Speed, is calculated wrongly. The solution (see CR 638) requires an airgap modification.

#### Project specific solutions have to be defined for Euroloop in projects that shall be considered interoperable with baseline 2.3.0, as the change of frequency is being done in parallel and requires changes of hardware.

#### Intentionally deleted.

# Decisions for the CR’s, qualified as IN

## Remainder from previous list of IN CR’s

This section contains a copy of the former UNISIG CR database for the CR’s qualified as “IN” in the previous version of this document, which affect other documents than the SRS. Attachments to these CR’s are contained in an embedded ZIP file at the end of this section.

| Request ID | 0000000007 |
| --- | --- |
| Problem Headline | Crossing GSM-R network border |
| Problem Description | SRS 2.2.2, 3.5.3.5 + In the case of crossing the border, the mobile network ID of the new railway administration has additionally to be provided by ATP application. SRS 2.2.2, 3.15.1.2.1a If the RBC area change requires crossing of GSM-R network border, also the new network-ID is has to be provided. |
| Decision by CR-Board | 1.Extend packets 42 (clause 7.4.2.10), 131(7.4.2.27) and 133 (7.4.2.29) adding NID_MN before NID_RADIO 2.For packet 42 : same "comment" applies for NID_MN as for NID_RADIO 3.For packet 131 : add to "comment" field of NID_MN: " Identifies the network the calling mobile has to register with when calling the "accepting" RBC" 4.Add new clause 7.5.1.91.1 NID_MN containing - Name: Identity of Radio Network - Description: The NID MN identifies the GSM-R network the calling mobile station has to register with. The NID_ MN consists of up to 6 digits which are entered left adjusted into the data field, the leftmost digit is the digit to be dialled first. In case the NID MN is shorter than 6 digits, the remaining space is to be filled with special character "F".For the further information about NID_MN refer to Subset-054. - Length of variable: 24 bits, min. value "0", max. value "999999" (BCD coded) - Special/reserved values : values "A" - "E" not used, value "F" used to indicate "no digit", if the number if shorter than 6 digits. 5.Update Subset-054 as follows: Introduce new section: . NID_MN (SRS 7.5.1.91.1) Responsible : UIC EURORADIO I/F Group Reference Document:: Radio Transmission FFFIS for EURORADIO Chapter "Numbering and Addressing" Doc.Ref: A11T6001 6.Modify SRS to include Identity of Radio Network: 3.5.3.5, 3.5.3.6, 3.5.3.13. 3.5.3.14: replace "telephone number" with "radio network identity and telephone number" 3.5.3.15 Add at the end of the clause: "In this case the radio network identity given in the packet shall be irrelevant." 3.5.3.16, option 3, column balise data content, replace with "RBC ID + radio network identity + RBC phone number" 3.15.1.2.1 a), replace text in 1st bracket with "including identifier and telephone number of the Accepting RBC, and the identity of the radio network to be selected" 3.18.4.3.1, replace begin of sentence with " The RBC identity and telephone number and the radio network to be used..." 3.18.4.3.2, replace ".. RBC identity and telephone number.." with "... RBC contact information (including RBC identity and telephone number, and the radio network to be used).." 3.18.4.3.3, add to "telephone number" "and radio network identity" A3.4 Table, and "RBC ID/Phone number" "and radio network identity" 4.7.2 Table: Add to input (also re-validation) and output of "Additional data" : "radio network identity" 5.4.2.2 Replace "RBC ID/phone number" with "RBC contact information (including RBC identity and telephone number, and the radio network to be used).." Replace 5.4.3 D1 "with a RBC and phone number" with "and RBC contact information (including RBC identity and telephone number, and the radio network to be used) is stored.." Replace 5.4.3 S3, S4, 5.4.3.3 E5: "RBC-ID, phone number" with "RBC contact information(including RBC identity and phone number, and the radio network to be used)" 5.10.3.1.1, 5.10.3.2.1, 5.10.3.7.1 add after "telephone number" "and the identity of the radio network to be used" EEIG reply 25-03-04 Regarding point1. We agree with the introduction of NID_MN, but for reasons of flexibility in the trackside implementation regarding the border crossing, the variables NID_MN and NID_RADIO should be defined in separate packets. Regarding point 5. Replace the expression “UIC EURORADIO I/F Group” by “national authority”. (Similar to U016) UNISIG 16/04/04: Regards point 1: UNISIG agrees on the proposed separate packets: actually there is already a proposal for this ready for SG review. After internal agreement (assumed for Mai/June 04) the proposal (CR) will be forwarded to the EEIG Regards point 5: According to EURORADIO opinion this is not the “national authority” but the ITU which assigns the radio network ID, EEIG to advise Postponed EEIG reply 26/04/04: Regards point 5 : “national authority” confirmed by R. Dijkmann. Justification: ” Structure of the mobile network ID is similar to the structure of a telephone number, in the sense that it contains a country ID (defined by ITU) and a part which defines the network inside the country. Due to the national part the responsibility for the number is national. Of course they have to follow the rules of ITU for the country ID, but that does not take away their responsibility” UNISIG 30/04/04: Reply regards point 5 accepted UNISIG 02.09.04: UNISIG SG decision (except for item 5) revised. Reasoning/solution see attached proposal. HK 24/11/04 : Correction for SRS chapter 5 added (attachement) 4/01/05: Proposal updated due to comments received from EEIG (see attachment) 25/02/05 (OG): Editorial correction (consistency with chapter4) : Rename in Table A.3.4.1.3 , column Data Stored on-board: “RBC Transition Order” to "RBC Transition Order + Session Establishment Order with Accepting RBC" 8/4/05 Solution proposal to be amended: - Transmission media for new packet 43 must read: "Balise, Radio" - Table 4.7.2, output information "Additional data" : add "Radio network-id" to list UNISIG 9/9/05 (HK) In the discussion about Subset 108 (system version 2.2.2c/2.3.0), it was agreed to include the manual level selection part of CR 007 as an "IN" CR. As a consequence the CR was split, the reduced functionality being kept in CR007, the removed part of to be moved to a new CR (number not yet assignaed). See attachment "solution_proposal_for_CR7_222c_080905.doc" |
| References | Subset 026-3, V2.2.2, 3.5.3.5, 3.5.1.2.1 Subset 026-x, see attached proposal Subset 054, V2.0.0 |
| Remarks of CR-Board | Remark to original decision: SRS EIRENE to be updated to provide the information regarding NID_NM EEIG reply 26/04/04: What kind of information do you expect from EIRENE The only relevant limitation is the maximum length which is already specified as 6 digits. For the rest this number should be transparent to ETCS, similar to the RBC phone number (compare U016) UNISIG reply 30/04/04 : Accepted: No action required |
| Type of change | Technical / major / gap |
| Change backward compatible |  |
| Economical evaluation |  |
| List of attachments | SUBSET-026-5 v225_CR007_corr.ZIP, Proposal Radio Network Management.zip, 225_NY_edition.zip, solution_proposal_for_CR7_222c_080905.doc |
| Date of last modification | 09.09.2005 |

| Request ID | 0000000016 |
| --- | --- |
| Problem Headline | Network address format |
| Problem Description | Add in Description: The significant digits are contained in the left order digits. Add for value "F" Padding by the special value "F" shall be added after relevant digits. |
| Decision by CR-Board | Agreed. 1) Add to "description" of 7.5.1.95 : " The number is to be entered "left adjusted" starting with the first digit to be dialled. Padding by the special value F shall be added after the least significant digit of the number. For further information about NID_RADIO refer to SUBSET-054." 2) Add to Subset-054 (Assignment of Values to ETCS variables) as follows: - New section NID_RADIO (SRS 7.5.1.95) - Responsible : UIC EURORADIO I/F Group - Reference Document : Radio Transmission FFFIS for EURORADIO, Doc. Ref_ A11T6001, Chapter "Numbering and Addressing". 3) Add to Subset-040 (Dimensioning and Engineering Rules) new engineering rule regards NID_RADIO: - Always enter NID_RADIO into packets 42 and 131 as an international number (i.e., including the country code) - However, leave away prefix "+" defined for international numbers. Note : EURORADIO recognises international numbers and adds, before calling, the prefix "+". 150503 EEIG response: We agree your solution except the paragraph on SUBSET-054. We do not see any need to allocate telephone numbers on a European level. The international part of the telephone numbers (country code) is already defined by ITU and the national part is the responsibility of the national authority. We therefore ask to clarify the need for the paragraph on SUBSET-054 or to delete it. 250603 UNISIG response : Modify ”responsible ” in subset-54 to ”national authority” EEIG reply 25-03-04: In the common workshop with EEIG, UNISIG Euroradio WG, GSMR railways and GSMR industry, held on 18-02-03, it was decided that the telephone number of the RBC shall consist of a maximum of16 digits, which is enough for the complete international telephone number, including the breakout code (e.g. 00 or 900). The telephone number is transparent to ETCS, i.e. ETCS will not add anything to the number given by trackside, but just passes it on to the onboard GMSR. The proposed solution should therefore be changed as follows: 1) In SUBSET-040 include only a reference to A11T6001 instead of repeating clauses from A11T6001. 2) Ask UIC Euroradio Interface Group to modify A11T6001 clause 6.4 as follows: “The maximum number, including break out code "00" or "9xx", shall not exceed 16 digits and the contents of the number is transparent to ETCS. 3) Regarding your proposal 250603: We assume you do not want to replace the word “responsible” but the expression “UIC EURORADIO I/F Group” by “national authority”. Please be precise if you put something between quotes. UNISIG 16/04/04: Agreed. 1) Agreed. Replace original decision item 3) with: “Add to Subset-040 (Dimensioning and Engineering Rules) new engineering rule regards NID_RADIO: For information regards the NID_RADIO refer to Subset 054. 2) EURORADIO WG to forward CR to UIC Euroradio Interface Group requesting: modify A11T6001 clause 6.4 as follows: “The maximum number, including break out code "00" or "9xx", shall not exceed 16 digits and the contents of the number is transparent to ETCS”. 3) Modify original decision item 2), line : “Responsible : UIC EURORADIO I/F Group” to read “Responsible : national authority”. |
| References | Subset 026-7, V2.2.2, 7.5.1.95 Subset 040, V2.0.0 Subset 054, V2.0.0 |
| Remarks of CR-Board |  |
| Type of change | Editorial / clarification |
| Change backward compatible |  |
| Economical evaluation |  |
| List of attachments |  |
| Date of last modification | 16.04.2004 |

| Request ID | 0000000034 |
| --- | --- |
| Problem Headline | permitted maximum number of balise groups in the packet "Linking" |
| Problem Description | According to the UNISIG SRS (SUBSET-026-7 v200, chap. 7.4.2.2) 32 balise groups at most may be announced in the packet "linking" (packet number 5; maximum value of the variable N ITER plus 1 since N ITER does not include the first balise). According to the UNISIG "Dimensioning and engineering rules" (SUBSET-040 v200, chap. 4.3.2.1i) 15 balise groups at most may be announced in the packet "linking", and at minimum 30 balise groups stored on-board. The reason for this restriction is not obvious. Apparently, the authors of SUBSET-040 have not considered that linking data always shall be transmitted from the last reported LRBG (this requirement was added to the UNISIG SRS 2.2.0 as chap. 3.7.3.5). |
| Decision by CR-Board | Accepted. Modify SUBSET-040, 4.3.2.1 i as follows: Increase maximum number of iterations in 1 packet from "15" to "29" Justification (updated 17/05/04) : 29 iterations allow for a maximum of 30 linked balise groups to be transmitted in one packet. Because according the changed procedure new linking information completely overwrites old information, the amount of linked balises to be stored is unchanged. Exception: In case linking information is transmitted as in-fil information, Engineering must take care of any balises between the in-fill location and the in-fill reference location (i.e., the main signal balise group) which remain stored on-board |
| References | SUBSET 026-3, V2.2.2, 3.7.3.5 SUBSET 026-7, V2.2.2, 7.4.2.2 SUBSET 040, V2.0.0, 4.3.2.1.i |
| Remarks of CR-Board |  |
| Type of change | technical /consistency / minor |
| Change backward compatible |  |
| Economical evaluation |  |
| List of attachments |  |
| Date of last modification |  |

| Request ID | 0000000098 |
| --- | --- |
| Problem Headline | Handling of Default Balise Telegram |
| Problem Description | As it stands in the SRS, it is not obvious which reaction the onboard should have when receiving the Default Balise Telegram (T 254) In my opinion first of all: the default balise telegram will be accepted and stored by the onboard system to fulfill SRS requirement 3.16.2.4.8 second: it will be handled similar like a change of balise group message (Please refer to 3.16.2.4.5 - 3.16.2.4.7) because a similar situation: "We have received a Message correctly but not clear/complete". |
| Decision by CR-Board | (Rejected): A balise group transmitting a message containing one or more default telegrams shall still be able to transmit a usable and safe information. As stated in the SRS, the default packet in a telegram is to inform the on-board that there is an error in the trackside equipment. This should trigger an error message on the MMI and recorded / forwarded as diagnostic information (implementation issue). However: 1)Modify 3.16.2.4.8 to read: ... by sending a balise telegram including the information "default balise information". 2)Add new paragraph 3.16.2.4.9 : If a message has been received containing the information "default balise information" the driver shall be informed (see Table 4.7.2: error messages). Add according message to FIS MMI 3) Update 7.4.2.38 / Description : Indication to on-board that telegram is the default balise telegram due to a fault of the trackside equipment EEIG reply 16-04-03 Not yet completely agreed. Please specify the intended message for the driver on the DMI. Unisig reply 25.06.03: Regards UNISIG solution 2) : Delete last sentence. Add new item 4) to UNISIG solution: Add new message to FIS MMI section 4.2.6.2 :”Trackside equipment malfunctioning”. Note: More detailed information may be given to the driver by using text packet. UNISIG 17/01/06: Decision regards 3.16.2.4.8 is superseded by CR 489 |
| References | Subset 026-3, V2.2.2, 3.16.2.4.8, 3.16.2.4.9 Subset 026-7, V2.2.2, 7.4.2.38 Subset 033, V2.0.0, 4.2.6.2 |
| Remarks of CR-Board |  |
| Type of change | Editorial /clarification |
| Change backward compatible |  |
| Economical evaluation |  |
| List of attachments |  |
| Date of last modification | 17.01.2006 |

| Request ID | 0000000146 |
| --- | --- |
| Problem Headline | EEIG ref 033: TSR s |
| Problem Description | If the train receives a new non-revocable TSR when it has already a non-revocable TSR onboard, then the new one has by definition the same ID as the old one since only one ID value (255) is reserved for non-revocable TSR. SRS 3.11.5.9 specifies only that the new non-revocable TSR will not replace the old one. It is therefore clear what happens to the old TSR, it will be kept and supervised by the onboard. It is however not clear from the SRS what happens to the new non-revocable TSR. Additional small remark: There seems to be an inconsistency in SRS 7.5.1.99, the definition of NID TSR. The non-revocable TSR is given the value 255 (all 8 bits set to one). The maximum value of the variable is however defined as 254. This is not consistent. |
| Decision by CR-Board | Modify 3.11.5.9, adding at the end :" .. in which case this shall be considered as an additional TSR." Modify 7.5.1.99 : - Increase max value to 255 - Increase max value for " Reserved for RBC transmission" to 254 Modify Subset-40, clause 4.3.4.1 : Add rule for non-revocable TSRs ,for the on-board functionality add ref to SRS 03-04-03 EEIG response to Decision by CR-Board: Proposed solution OK but please specify in detail the proposed modification in SUBSET-40 (the intended rule in quotes, ref to specific clause in SRS) UNISIG response: Modify Subset-40, clause 4.3.4.1 to read: Add rule for non-revocable TSRs, for the on-board functionality add ref to SRS "If the on-board receives 2 packets of type 65 with the same NID_TSR, the previous TSR is overwritten by the new one. Exception: If the On-board receives a TSR which is qualified as "non-revocable", it does not override a previous TSR with the same NID_TSR, but the new one is considered as an additional TSR (see also Subset-026, clause 3.11.5.9). Application example : Non-revocable TSRs may be used in the area of an RBC/RBC handover if each of the involved RBCs allocates NID_TSRs independently of the other one." |
| References | Subset 026-3, V2.2.2, 3.11.5.9 Subset 026-7, V2.2.2, 7.5.1.99 Subset 040, V2.0.0, 4.3.4.1 |
| Remarks of CR-Board |  |
| Type of change | Editorial / clarification |
| Change backward compatible |  |
| Economical evaluation |  |
| List of attachments |  |
| Date of last modification |  |

| Request ID | 0000000151 |
| --- | --- |
| Problem Headline | EEIG ref 038: Eddy current brake switch off |
| Problem Description | It is not exactly clear from the SRS (last three bullets of 3.12.1.3) if switching off the brakes applies to commanding Service Brake only, Emergency Brake only or both. |
| Decision by CR-Board | Section 3.12.1.3 , bullet about switching off the eddy current brake is clear: If the Eddy Current Brake is disabled, it is off for EB and SB. 15-05-03 EEIG response: Please accept our apologies for the not correct use of the English words. We are only railway people and not experts in English language. If we would like to make an informal first enquiry we would not use a formal Change Request. So please consider every CR in future as a real CR, independent from the sometimes not so perfect English.This CR is more related to the decision from infra managers on the acceptable use of certain brake types on their infra, not on the calculation of the curves. It is therefore not an issue for B126.15 but can be handled separately. So please take this CR now into consideration. Unisig reply 25.06.03: O.k., taking this as a ”real” CR : please clarify the second paragraph of the problem description : who is compensating the reduced brake capability when switching off the eddy current brake If it is the EVC which is the required functionality Anyway, as stated below, this is new functionality, to be approved by the UNISIG SC EEIG reply 16-09-03 For each of the special brakes mentioned in 3.12.1.3 it can be programmed in the onboard what the deceleration is with and without the special brake active. Then the onboard will decide which deceleration value to use according to the information received from trackside to switch on/off the associated brake. In addition we propose to specify an optional input on the train interface which will indicate for each of the special brakes implemented onboard if it is fit for use or not. (Of course this input could only be used if the implemented special brake would provide an output to indicate it’s “fit for use” status.) This input can then be handled by the onboard to select the appropriate deceleration in the same way as the order from trackside to switch on/off the relevant brake. UNISIG reply 24-10-03: WE are still not clear about what you meant in you original submitter proposal with “deceleration has to be compensated if this brake was included in its calculation” : If - due to a trackside command- we would switch off the eddy current brake we loose part of the normal brake deceleration. If you do this while braking to a target the train will most likely overshoot the target. However, if I understand correctly how the ICE train works : if the eddy current brake is disabled, due to a trackside command, then there is a brake management on the train side which increases the brake force provided by the other brake systems, i.e., there is a compensation on the train side, and ETCS would not be bothered, but what is the EEIG “after” EEIG reply 29-10-03: Obviously, when the disabling of the eddy current brake would have no impact on the deceleration value of the complete brake system (due to some implementation dependent very clever brake management system) then the so called guaranteed deceleration value which is used by onboard ETCS to calculate the curves can still be met by the actual brake system and is therefore still valid, so no compensation is necessary. However, this is clearly dependent on the implementation and what we (EEIG) want to have is an SRS that will always work, independent of the implementation. So imagine that there is a train which does not have this magic brake management like the ICE. Then as soon as the eddy current brake (or one of the other special brakes) is disabled and the guaranteed deceleration is therefore reduced for example from 0.7m/s2 to 0.6m/s2. Then we want that onboard ETCS will calculate the curves with 0.6m/s2. The proposal in the first sentence in the EEIG reply of 16-09-03 covers both implementation options. The ICE would have the same guaranteed deceleration value both for the eddy current brake on and off. The other example I gave above would 0.7m/s2 for eddy current on and 0.6m/s2 for eddy current off. The disabling of the eddy current (or one of the other special brakes) should not occur while the train is braking to a target. If you always send this track condition together with the first MA extension into the relevant area, then the onboard can take the reduced deceleration safely into account and there is no problem. Maybe you need to include an engineering rule for this. UNISIG reply 19-1-04: Until now we assume a guaranteed braking deceleration that does not change when disabling a brake, e.g., the eddy or magnetic shoe brake. The clarification for this CR given by the EEIG requests that a modified braking performance needs to be taken into account when switching off some brake system, e.g., the eddy current brake. What effect does this have on the entry of brake parameters It might even influence the train category ( ). Meeting with EEIG 19.02.04 : Scope of this CR to be limited to provide individual outputs to enable/disable the eddy current brake for SB or EB. Changes of the braking capability when disabling the eddy current brake will be dealt with by the EEIG as a separate issue. Clause 3.12.1.3.1 to be clarified as follows : “Note: In the last three cases the guaranteed deceleration used in the braking curve calculation is not affected by switching on/off these brakes.” 17/02/05 EEIG/ UNISIG: Functionality with regards to individual disabling of eddy current brake for service/ emergency brake to be included into system version 3.0.0 EEIG 24-02-05: Scope of the CR to include individual SB/EB enable/disable for each of the three specific brakes, eddy current, magnetic shoe and regenerative. Clause 3.12.1.3.1 to be modified to a requirement (not a note) to say that for each setting of the individual enable/disable brake switches the appropriate braking curve for that setting shall be selected. Note: References for these requirements are the TSI CCS Annex A appendix 1 clause 5.2.1 and 5.2.2 and TSI RS (version EN 07) clause 4.2.4.1 and 4.2.4.6. UNISIG 15/03/05: EEIG to clarify: Requested modifications to 3.12.1.3.1: What is the task for the on-board, on collecting train data, switching brake parameters when activating/de-activating a specific brake type. On the fly ? How to do while train is braking to a target ? TSI CCS Annex A appendix 1 clause 5.2.3 (DB specific) : what to be done ? Siemens to write proposal regards receiving the above track conditions FIS TIU to be updated to provide enhanced interface to brakes EEIG 19-05-05: - Onboard functionality regarding (de)activating specific brakes is included in braking curve doc 97E8816E. - TSI CCS Annex A appendix 1 clause 5.2.3 is not to be handled by ETCS, see following email (Re_specific case Germany.eml) from HB. "Rob Dijkman" <rdijkman@ertms.be> wrote 13.04.05 to H. Bierlein: Hans, We assume that ETCS will not be required to handle this specific case. Can you confirm that? 5.2.3 Specific case Germany: The magnetic brake and eddy current brake is not permitted at the first bogie of a leading vehicle unless defined in the Infrastructure Register. HB replied: Yes, this outside the scope of ETCS UNISIG 23/09/05: On request by EEIG, UNISIG agrees to implement an intermediate solution only for system version 2.2.2c (2.3.0) , whereas for version 3.0.0 the original CR shall be implemented. The modification of the intermediate solution is that the enabling/disabling of the eddy current brake is limited to the application of the service brake. ERTMS/ETCS on-board equipment is not dealing with enabling/disabling of the eddy current brake for the emergency brake. Modifications see attached proposal "CR151 attachment 16-09-05.doc" |
| References | SUBSET 026-3, V2.2.2, 3.12.1.3, 3.12.1.3.1, 3.12.1.3.2 SUBSET 026-7, V2.2.2, 7.5.1.77 SUBSET 034, V2.0.0, 5.2.5 |
| Remarks of CR-Board | Requesting individual switching for SB and EB is a functional extension, if requested SC approval is required |
| Type of change | Operational/ minor functional extension/ on EEIG request |
| Change backward compatible |  |
| Economical evaluation |  |
| List of attachments | CR151 attachment 16-09-05.doc |
| Date of last modification | 23.09.2005 |

| Request ID | 0000000215 |
| --- | --- |
| Problem Headline | Infill and Level transition Now |
| Problem Description | Clarification required: If after Paket 136 (Infill Location reference) Paket 41 Level Transition is received including special value which means Level Transition "Now", then it could be confusing for On-board equipment. Because it is just a copy of the main Balise group to which it belongs and the value "Now" means now only if this order is read at Passage of the main balise and not at passage of any of its infill Balises. So on the one hand it should be avoided by engineering to transmit information via infill which is meaningless for the onboard at passage of infill Balise (or Loop or Radio infill) to avoid problems in general. On the other hand the onboard shall ignore in case of infill such information in particular Level Transition with special value "Now". |
| Decision by CR-Board | Add this clarification to engineering rules Subset-040 chapter 4.2.4 : “If Infill information contains an announcement of an immediate level transition at the location of the location reference for the in-fill information, for the distance D_LEVELTR the value of “0m” shall be used." |
| References | Subset 026-7, V2.2.2, 7.5.1.9 Subset 026-8, V2.2.2, 8.4.1.3.1 Subset 040, V2.0.0, 4.2.4 |
| Remarks of CR-Board |  |
| Type of change | Editorial / clarification |
| Change backward compatible |  |
| Economical evaluation |  |
| List of attachments |  |
| Date of last modification |  |

| Request ID | 0000000248 |
| --- | --- |
| Problem Headline | Minimum distance between the 1st balise of a balise group and the EoA |
| Problem Description | In Subset-040, clause 4.1.1.4 it is stated: "The balise group (giving an MA) that is placed close to the EoA must be at least 1 m in rear of the EoA." There are the following problems: It is not clear where the value of 1m comes from : FFFIS Eurobalise sections 5.2.2.5 and 4.2.9 define a value of 1.3m after which no data from the balise is received any more (distance to begin of balise cross-talk zone) The condition is not sufficient, it only considers static behaviour. If the vehicle is moving at that time, e.g., with a Release Speed of 40 km/h, the processing time when receiving the balise data has to be considered as well. FFFIS gives in section 4.2.2.9 timing and distance requirements, but only until the information is available at the ERTMS/ETCS kernel. Also the Performance Requirement paper (Subset-041) does not state processing time requirements when receiving a message from balises, it only states, in clause 5.2.1.1, after which time the brakes have to be applied, or, after which time the information on the DMI changes (clause 5.2.1.3). The requirement does not only apply to an EoA, but also to an LoA. 2) "The distance between the "minimum safe front end" and the front end of the train must be greater than, or equal to, the distance between the front end of the train, and the reference mark of its active Eurobalise antenna." This requirement can lead to situations where the train is tripped before reading the last balise ( see attachment). |
| Decision by CR-Board | Accepted. Modify Subset 040, clause 4.1.1.4 rule as follows: "The last encountered balise of the balise group giving an MA, or an exit location, that is placed close to the EOA or LOA must be a distance in rear of the EOA/LOA which consists of a distance of 1.3m The second part of rule 4.1.1.4 is to be removed. Add tips how to use performance requirements, revise “Justification” part of 4.1.1.4. Modify Subset-026 ,section 3.13.8 as follows: Clause 3.13.8.1 : Add before sentence : "Level 2/3 :" Add new clause 3.13.8.1.1 : "Level 1 : Train Trip shall be initiated if the on-board equipment detects that the minimum safe antenna position (calculated by subtracting distance between active Eurobalise antenna and the front end of the train from the min safe front end position) has passed the EOA/LOA." Decision completed 18/06/04 (Accepted) Postponed for company internal checking Modify Subset 040, clause 4.1.1.4 “rule” as follows: "The last encountered balise of the balise group giving an MA, or an exit location, that is placed close to the EOA or LOA shall be a minimum distance of 1.3m in rear of the EOA/LOA “ Modify “justification” of clause 4.1.1.4 as follows: " The underlying approach is that all information related to the extension of an MA or the level transition order at a border must have been received until the antenna with its reference mark overpasses the EoA/LoA location. From an on-board point of view this means that no further information will be received when the train with its “min safe antenna position” has passed the EoA/L0A. According to the FFFIS Eurobalise no further information can be received from a balise if the (on-board) antenna has passed a balise by a distance of more than 1.3m Note: If the antenna with its reference mark has reached the EoA/LoA (which is still valid) the brakes shall be applied latest after a further time delay of 1s according the Subset 041, clause 5.2.1.1 (consisting of two parts : establishing, after the processing time when receiving balise messages has elapsed, that no further information has been received, and the time for commanding/applying the brakes)" Modify Subset-026 ,section 3.13.8 as follows: Clause 3.13.8.1 : Add before sentence : "Level 2/3 :" Add new clause 3.13.8.1.1 : "Level 1 : Train Trip shall be initiated if the on-board equipment detects that the minimum safe antenna position (calculated by subtracting distance between active Eurobalise antenna and the front end of the train from the min safe front end position) has passed the EOA/LOA and no new data has been received up to this point." UNISIG17/03/05: - Note in Subset 040. 4.1.1.4, Justification, to read: " For processing time limits of information received from balises refer to Subset 041, section 5.2 Response Times" - Modify condition [16] in 4.6.3 to read : "(Train/engine overpasses the EoA/LoA with its mi safe front end) AND ( ERTMS/ETCS level is 2 or 3)" - Add new condition [12] in 4.6.3 to read: "(Train/engine overpasses the EoA/LoA with its min safe antenna position) AND (ERTMS/ETCS level is 1)" - Add in table 4.6.2 [12] for same transitions as [16] |
| References | Subset 040, V2.0.0, 4.1.1.4 Subset 041, V2.0.0, 5.2.1.1 Subset 036, V2.1.8, - Subset 026-3, V2.2.2, 3.13.8 Subset 026-4, V2.2.2, table 4.6.2, 4.6.3 |
| Remarks of CR-Board |  |
| Type of change | Technical / Consistency |
| Change backward compatible |  |
| Economical evaluation |  |
| List of attachments | Analysis of Rule 2 in Subset.doc |
| Date of last modification | 17.03.2005 |

| Request ID | 0000000315 |
| --- | --- |
| Problem Headline | Engineering (Track Condition; Route Suitability) |
| Problem Description | What shall be the minimum number of Track Conditions and Route Suitability Data the onboard shall memorise |
| Decision by CR-Board | 6/04/05 Accepted. Add to Subset 040, section 4.3.2 (DATA) the following limits : - For special Track Condition “Change of Traction Power”: no iteration in packet, minimum memorised on-board : 1. Justification: On-board id able to manage one change of traction power at a time - For special Track Condition “Big Metal Masses” : number iterations in packet : 4, minimum memorised on-board : 5 - Other Track Conditions : number iterations in packet : 9, minimum memorised on-board : 10 - Route suitability Data : number of iterations in packet : 2, minimum memorised on-board : 3 (however: not more than one of each type) EEIG 26-04-05: Regarding several of these track conditions the restrictions you propose are not acceptable for the railways. For change of traction power compare also U311 where you have introduced yourself multiple changes within a short distance. For big metal mass we still wait the outcome of U477, but for the current specification the limit of 5 is too low.In general we do not see any real argument behind your limitations. We therefore propose to delete any specific limit for transmision, i.e. the limit is defined by the size of N_ITER, and set for each track condition the limit of memorised onboard to 50. UNISIG 21/06/05 There definitely has to be a limit for the various data transmitted from trackside, as otherwise overflows can occur. The limits we propose seem reasonable to us from a practical point of view, please show us where this is not the case. EEIG 15-09-05: For the "other track conditions" we propose to increase the numbers to 19 iterations in the packet and 20 memorised onboard. The reason is that in some existing systems an MA can be up to about 25km long and the railways want to use ETCS in the same way. For a distance of 25km there might be situations where 10 is not enough. To avoid that this limit is a bottle-neck in such situations it is better to increase the number. UNISIG 23/09/05 (HK) Agreed to increase the number of "other track conditions" to 19 iterations in the packet and 20 memorised onboard. |
| References | Subset 040, V2.0.0, - |
| Remarks of CR-Board |  |
| Type of change | Technical/ Gap |
| Change backward compatible |  |
| Economical evaluation |  |
| List of attachments |  |
| Date of last modification | 23.09.2005 |

| Request ID | 0000000469 |
| --- | --- |
| Problem Headline | Completeness of data |
| Problem Description | 1) What happens with the limitations related to the movement authority that shall be sent together with the MA to which the information belongs in case the MA is rejected ? That’s to say if a mode profile accompanies a MA and the MA is rejected, the mode profile should also be rejected. 2) What happens if a mode profile is received alone, without the MA to which belongs ? In this case it should also be rejected. 3) What happens if new track description shorter than the previous one is received ? Should the MA be shorten also ? Should it be deleted ? Should it be stored in order to be used when new track description is received ? |
| Decision by CR-Board | Problem added regards 3.7.2.3: What means “not accepted” , same as “rejected”, or “ignored” What is rejected, only the MA, or the complete message. 18/05/04:Accepted. - Modify 3.7.2.3 to read: “The MA and the related mode profile, if any, shall not be accepted ” - Modify table 4.8.3 line “mode profile”, add exception [4] to all cells with “A” - Modify exception [4] to read :”The movement authority and the related mode profile ” Submitter proposals with regards to 3.7.2.5/6: rejected: These are engineering errors, it is not the task of on-board to check proper engineering. EEIG reply 21-07-04: You ask a question in the decision box (What means "not accepted") and you answer this question only in the remarks box. We think this answer should be clearly specified in the SRS. In addition we think that all key words like "accepted", "rejected", "ignored", "stored", "valid", "used", etc. should be clearly defined in a common place in the class 1 specifications (SRS or glossary). UNISIG 4/8/04: Include definitions below into Glossary (Subset 023): "Valid information" : information has passed the message consistency filters (see SRS 3.16.3.1.1, 3.16.3.5.2 for radio, SRS 3.16.2.3.1, 3.16.2.4.1, 3.16.2.4.4, 3.16.2.5.1 for balise) "Accepted/rejected information" : Applies to filtering as described in SRS 4.8 "Stored information" : used in two ways: - before acceptance : information stored in buffer until the level transition is effective (see SRS 4.8.1) - after acceptance : information that remains stored on-board, compare with SRS 4.10.1.4.2 (non-stored information): information that is used (or not) immediately after acceptance, and then deleted "Used": data used by active function (SRS 4.5.2) "Ignored" : Not applicable information, e.g., packet in balise message not applicable to orientation of train. |
| References | Subset 026-3, V2.2.2, 3.7.2.3, 3.7.2.5/6 Subset 026-4, V2.2.2, 4.8.3 (table) Subset 023, V2.0.0, - |
| Remarks of CR-Board | Remarks 18/05: Regards 3.7.2.3: “not accepted” means, MA and related mode profile (if any) is not used, but other information in this message is used. No error is reported to the RBC. |
| Type of change | Editorial / consistency |
| Change backward compatible |  |
| Economical evaluation |  |
| List of attachments |  |
| Date of last modification | 04.08.2004 |

| Request ID | 0000000475 |
| --- | --- |
| Problem Headline | Engineering rules extracted from Safety paper |
| Problem Description | Subset-091 contains some rules for engineering which should be added to engineering Document SUBSET-040. |
| Decision by CR-Board | UNISIG 6/04/05 Accepted Add new clause 4.1.1.11 giving a reference to Subset 091, ch. 8.3.2.1, stating "Reminder: the rules of the reference must be respected regards : Number of Balises in each group/ use of single balise groups, TSR Balise groups” Add new clause 4.1.1.13 giving a reference to Subset 091, chapter 10, stating "Reminder for : the rules of the reference must be respected, e.g. : Number of Unlinked Balise groups (marked as unlinked), maximum distances between Balise groups." Add justification reading: "The safety analysis and safety requirements are based on this mission profile of the reference above." |
| References | Subset-091 - ETCS_TR_06; ETCS_TR07; EXT_SR01; EXT_SR02; EXT_SR03; Subset-040 ; CR 452 |
| Remarks of CR-Board |  |
| Type of change | Gap |
| Change backward compatible |  |
| Economical evaluation |  |
| List of attachments |  |
| Date of last modification | 06.04.2005 |

| Request ID | 0000000483 |
| --- | --- |
| Problem Headline | Proposed Engineering rule for discussion |
| Problem Description | Regarding SRS 3.8.4.6.3: The in-fill information shall include the identity of the balise group at the next main signal i.e. the identity of the balise group giving the information that is repeated by the in-fill device. So from formal or engineering point of view it looks like just to copy telegrams from main balise group and add in advance Packet 136 “infill location reference”, but to avoid any onboard problems this copy and paste should be limited to reasonable information. Example: The main Balise may also contain information valid for opposite direction (Balise contains MA for one diretion and is used also for Repositioning for the other direction. This information shall never be repeated by infill). |
| Decision by CR-Board | UNISIG 6/04/05 Accepted. Add in Subset 040 new clause 4.2.4.5 to read - Rule: In-fill information which is repeated from the balise group at the next main signal by any infill device shall be limited to infill MA, linking and route related track description information. All information which do not relate to In-fill (e.g. information for opposite direction or EOLM etc.) shall not be given as infill information. Permitted infill information: -Packet 136 (infill location reference) -Packet 12, 80; 49 (MA, Mode Profile, List of Balises for SH area) -Packet 21 (Gradient Profile) -Packet 27, 51, 65/66, 70 (SSP, ASP, TSR, Route Suitability) -Packet 5 (Linking) -Packet 41 (Level transition) {see also next rule below} (see decision CR 484) -Packet 44 (data used outside ERTMS) -Packet 39, 67, 68 (Track Condition) -Packet 71 (Adhesion Factor) -Packet 133 (Radio infill area information) -Packet 138, 139 (Reversing Area information) - Reference: SUBSET-026 – section 3.8.4.6.3 - Justification: This is to avoid any mis-interpretation by on-board. |
| References | Subset 040 |
| Remarks of CR-Board |  |
| Type of change | Gap |
| Change backward compatible |  |
| Economical evaluation |  |
| List of attachments |  |
| Date of last modification | 30.03.2005 |

| Request ID | 0000000507 |
| --- | --- |
| Problem Headline | Unwanted mode transition during train trip inhibition |
| Problem Description | If a level 1 MA with V_MAIN = 0 is received when train trip inhibition is activated (on-board equipment is in SR,UN,SE,SN mode following override procedure), it is possible, according to condition 31 that on-board enters in FS as distance to EOA in packet 12 might be non zero |
| Decision by CR-Board | UNISIG 02.09.04: Accepted. Table 4.6.3 : - Add to condition 31 : "..AND (ERTMS/ETCS level is 2 or 3)" - Re-use condition 32 : "(MA+SSP+gradient are on-board) AND (no specific mode is required by a Mode Profile) AND (ERTMS/ETCS level is 1) AND (no "trip" information is given by balise)" - Table 4.6.2 : add "32" to transitions from SR,OS,PT to FS Add Rule to Engineering Document regarding “remark of Submitter”: 4.2.4.6 "In case there is a L1 MA packet with V_MAIN=0, then it is not allowed that the message includes any mode profile packet." Justification: " On one hand it is obvious that a “STOP” signal aspect shall not contain an Mode profile but on the other hand this rule is necessary to complete transition table of SRS (avoid a mode transition if Override is active)" EEIG 13-01-05: 1) We do not understand the statement in the problem description that a level 1 MA can have V_MAIN = 0 and at the same time a distance to EOA > 0. This seems very illogical. In fact we do not understand at all the use of V_MAIN, which is not clear from the SRS. Could you clarify on this? 2) We do not understand the need for the engineering rule. Do not say that it is "obvious" that a STOP signal aspect shall not contain a mode profile. For us it would be "obvious" that a STOP signal (V_MAIN=0) does not contain a distance to EOA > 0, but according to your problem description that is not "obvious" for you. The whole problem is not caused by the mode profile but by the inconsistency between V_MAIN=0 and distance to EOA > 0. That is the core of the problem and it would be better to tackle the problem there. See also remark 1).And you say that the engineering rule is necessary to complete the mode transition table. Do you think that anything is missing from it? But you do not add anything to it in your solution. Please clarify. SG 15/02/05: Reasons for a EoA distance > 0 at the main signal balise: The EoA is at the signal, not at the reference location of the balise group The EoA location given by in-fill and main signal balise group is to be consistent, and it is mandatory, that in-fill gives an EoA at the signal, because with giving an EoA at the main signal balise group inhibits to read the main signal balise group If the signal is passed with "Override", the re-activation of the transition to TR mode (which is indicated to the driver, and is linked to a special speed limit), must occur at the same location in all circumstances. It is also more logical to the driver that the "override" indication disappears once the signal has been passed. For V_MAIN refer to 3.11.6, we use V_MAIN=0 as a special case to trigger an immediate TR reaction. SG 2/6/05: Modify previous decision regards Table 4.6.2 to: add "32" to transitions from SR,OS to FS (Reason: on-board shall not pick up a level 1 MA when running backwards) |
| References | Subset 026-4, v2.2.4, 4.6.2, 4.6.3 Subet 040, V2.0.1, 4.2.4.6 |
| Remarks of CR-Board |  |
| Type of change | Technical/ gap/ minor |
| Change backward compatible |  |
| Economical evaluation |  |
| List of attachments |  |
| Date of last modification | 02.06.2005 |

| Request ID | 0000000655 |
| --- | --- |
| Problem Headline | Level Transitions involving STM Levels |
| Problem Description | The concept of the NID_STM variable in the packet 41 for the level transitions led to confusions in the past. See presentation attached to this CR Usually the understanding of packet 41 is that it contains only one NID_STM for announcing a certain track equipment, while it may be several NID_STMs which are also allowed to operate in the track (example: STM PZB and German STM Group for a PZB line). Therefore the packet 41 should contain not only the pure NID_STM which is installed but also (in the table of possible levels) the other possible NID_STMs that are allowed to operate on it (national STM groups, or national systems with that functionality). |
| Decision by CR-Board | UNISIG 29/09/05: Add new rule in Subset 040 , section 4.2.4.11 (concerning rules for level transition announcement) to read: Rule: "Trackside shall announce all applicable values of NID_STM containing the national system(s) installed in the infrastructure" Reference : "SUBSET 026, sections 7.4.2.9; 7.5.1.98" Justification: "Certain STM functionalities might be grouped and so the trackside has to announce all relevant stand-alone STMs as well as STM groups according to the NID_STM list defined by UIC." |
| References | Subset-040, v2.0.0, 4.2.4.11 |
| Remarks of CR-Board |  |
| Type of change | Technical/ gap |
| Change backward compatible |  |
| Economical evaluation |  |
| List of attachments | Z_655_(SUB,Ricardo.ramos@invensys.com)NID_STM EEIG_UNISIG_050421 .ppt |
| Date of last modification | 29.09.2005 |

## New IN CR’s

This section contains an excerpt from the ERA CR database, for the CR’s already listed in the previous version of this document, which are reclassified as IN and for the newer CR’s not listed in the previous version of this document, which are classified as IN.

| Identification number | 0020 |
| --- | --- |
| Headline | Pending communication session |
| Agreed Solution | Modify SUBSET-026 v2.3.0 as follows: Modify clause 3.5.4.1 to read: "When a communication session is established, in case of a loss of the safe radio connection, i.e, if the disconnection has not been ordered (see 3.5.5.1), the involved entities shall consider the communication session still established for a defined time. The defined time shall start as soon as EURORADIO has indicated the loss of the safe radio connection." Add new clause 3.5.4.2.1: "If the safe radio connection is not re-established after the defined time (as defined in A3.1), both, on-board equipment and trackside, shall consider the session as terminated, and the ERTMS/ ETCS on-board equipment shall inform the driver." Modify clause 3.5.4.3 second bullet, to read: "the session is considered as terminated" Modify clause 3.5.4.4 to read: "When the safe connection is lost inside an announced radio hole (see 3.12.1.3), the on-board equipment shall try to re-establish the radio connection when the train front reaches the end of the radio hole. Modify table A3.1 (List of Fixed Value Data) as follows: Add new line "Maximum time to maintain a communication session in case of failed re-connection attempts" with: column "value": "5 minutes" column "Name": void |

| Identification number | 0051 |
| --- | --- |
| Headline | Position Reporting in RV Mode |
| Agreed Solution | Modify SUBSET-026 v2.3.0 as follows: Modify clause 4.4.18.1.6 to read: "Intentionally deleted." Modify table 4.5.2 (lines "Report train position when train reaches standstill" and "Report train position as requested by RBC"): add "X" for RV Modify table 4.8.4 (line "Position Report parameters "): replace "R" with "A" for RV |

| Identification number | 0066 |
| --- | --- |
| Headline | Non relevant information for driver |
| Agreed Solution | Modify SUBSET-026 v2.3.0 as follows: Modify clause 3.10.2.1.2 first bullet: delete "and the driver". |

| Identification number | 0078 |
| --- | --- |
| Headline | Odometry inaccuracy at exit of speed restriction |
| Agreed Solution | Modify SUBSET-026 v2.3.0 as follows: Modify clause 3.13.3.3.1 to read: "The speed monitoring with respect to the most restrictive speed profile shall take into account the minimum of the static speed restrictions applicable to the train (see section 3.11.9): a) The location of a speed decrease shall be supervised by the on-board equipment taking into account the max safe front end of the train (see also section 3.13.6.2.4) b) The location of a speed increase shall be supervised by the on-board equipment taking into account the min safe front end of the train, or, when requested, the min safe rear end of the train." Modify clause 3.13.6.2.4 to read: "The supervision with regards to the target location shall refer to the train front using the max safe value (see also section 3.13.3.3.1)." |

| Identification number | 0138 |
| --- | --- |
| Headline | Brakes release after max reversing distance overpassed |
| Agreed Solution | Modify SUBSET-026 v2.3.0 as follows: Add clause 3.14.1.7.1: "If the brake command was triggered due to exceeding the reversing distance related to a reversing area, the brake command shall be released at once if the reversing distance has been extended so that the reversing distance is no longer exceeded, or at standstill after driver acknowledgement." Modify clause 4.4.18.1.3 b) , last sentence : "The brake command shall be triggered if overpassing this distance." |

| Identification number | 0154 |
| --- | --- |
| Headline | Text message inconsistency |
| Agreed Solution | Modify SUBSET-026 v2.3.0 as follows: Modify table 4.5.2 (line "Manage Text Display to the driver") : add "X" for TR,RV Modify table 4.7.2 (lines "Ackn of fixed text information", "Ackn of plain text information") : remove "A" for SH, SE, SN add "A" for NL, TR, RV Modify table 4.7.2 (lines "Fixed text information", "Plain text information") : remove "A" for SH, SE, SN Modify table 4.8.4 (lines "Fixed text information", "Plain text information") : replace "R" with "A" for TR |

| Identification number | 0170 |
| --- | --- |
| Headline | Indication of track conditions |
| Agreed Solution | See attachment "CR170_agreed_solution_270907.doc". |

| Identification number | 0183 |
| --- | --- |
| Headline | Data from single balise groups |
| Agreed Solution | Modify SUBSET-026 v2.3.0 as follows: Modify clause 3.4.3.1.3, second sentence : "If such information is transmitted by balises, and if the information is directional, balise groups consisting of at least two balises shall be used." Modify clause 3.4.3.2.1, second sentence : "If such information is transmitted by balises, and if the information is directional, balise groups consisting of at least two balises shall be used." |

| Identification number | 0187 |
| --- | --- |
| Headline | End of Mission during RBC Handover |
| Agreed Solution | Modify SUBSET-026 v2.3.0 as follows: Add new clause 3.15.1.3.4.1: "If the on-board equipment is connected to both RBCs, and it executes an End of Mission, it shall execute the End of Mission procedure with both RBCs." |

| Identification number | 0202 |
| --- | --- |
| Headline | Memorised balise groups for special position report not mentioned in table 4.10.1.3 |
| Agreed Solution | Modify SUBSET-026 v2.3.0 as follows: Modify table A.3.4.1.3 : remove line "Memorised balise groups for special position report" |

| Identification number | 0223 |
| --- | --- |
| Headline | Table of Supported Levels must be restored at Start of Mission |
| Agreed Solution | Modify SUBSET-026 v2.3.0 as follows: Modify clause 3.18.4.2.3: delete last sentence "To cover degraded situations the driver shall have the possibility to select a level" Add new clause 3.18.4.2.4: “ For operational fallback situations: at standstill, the onboard equipment shall allow the driver to change the ERTMS/ETCS level: The selection shall be limited to those contained in the table of priority, if available.” Add new clause 3.18.4.2.4.1 : " Note: even if only one level is permitted this is considered as a table of priority." Add new clause 3.18.4.2.5: “The driver shall follow the table of priority, if available. If not available, the driver can select any level, obeying the operational rules applicable for this area (see also 5.10.2.7)” Modify A3.4.1.3 (table): below line " ERTMS/ETCS level", add new line "Table of priority of trackside supported levels" with "U" for all columns "Situations listed above" Modify table 4.10: add new line “Table of priority of trackside supported levels” (below line "ERTMS/ETCS level") with: “D” for NP mode "NR" for SF,IS modes "U" for all other modes Modify clause 5.10.2.9 to read: "At standstill, the onboard equipment shall allow the driver to change the ERTMS/ETCS level. If a table of priority is available, the driver selection shall be limited to the levels contained in the table of priority (for details refer to chapter 3.18.4.2)” |

| Identification number | 0238 |
| --- | --- |
| Headline | Shifted reference location |
| Agreed Solution | Modify SUBSET-026 v2.3.0 as follows: Modify clause 3.6.3.2.5 to read: "It shall be possible to shift the location reference from the RBC, e.g. after a change of train orientation or running direction." |

| Identification number | 0257 |
| --- | --- |
| Headline | Last eight reported BG identities stored on board with regard to deletion of stored position data in SoM procedure |
| Agreed Solution | Modify SUBSET-026 v2.3.0 as follows: Add new clause 3.6.2.2.2 d) : "Exception : when on-board position data has been deleted or has reported an “invalid” position not confirmed by the RBC (during SoM procedure), both, ETCS on-board equipment and RBC shall use a LRBG identifier set to "unknown."" Add new clause 3.6.2.2.2 e): "Once the ETCS on-board equipment has received from the RBC a message with an LRBG not set to “unknown” it shall stop accepting further messages containing LRBG “unknown". For further conditions see 3.16.3.1.1." Modify 3.6.1.3 reading: "3.6.1.3 The Train Location information defines the location of the train front in relation to a balise group, which is called LRBG (last relevant balise group). It includes: •The estimated train front end position, defined by the estimated distance between the LRBG and the front end of the train •The train position confidence interval (see 3.6.4) •Directional train location information in reference to the balise group orientation (see 3.4.2, also Figure 14) of the LRBG, regarding: - the position of the train front end (nominal or reverse side of the LRBG) - the train orientation - the train running direction In case of an LRBG being a single balise group with no co-ordinate system assigned, directional information is defined in reference to the pair of LRBG and “previous LRBG”, see 3.4.2.3.3. •A list of LRBGs which may alternatively be used by trackside for referencing location dependent information (see 3.6.2.2.2c)" Modify in 3.9.3.11b "Train location" by "Position report". Modify A3.4.1.3 (table) and 4.10.1.3 (table): rename row “Location” by “Train Location” Delete row "last eight reported balise group identities” Delete 4.10.1.4.1 Modify 5.4.2.2 replacing the text in brackets by: "(see 3.6.1.3)" Modify 5.4.3.3 by adding a new row with Transition Conditions “Following E10, E12, E15, E30, E31, E32, On-board deletes stored position data” and with an arrow for "Train Position data" from “invalid” to “unknown” Modify 5.4.3.2 S10 by adding a new bullet at the end: “Following E10, E12, E15, if the position is still invalid, the ERTMS/ETCS on-board shall delete the train position data (new status: “unknown”)” Modify 5.4.3.2 S22 by adding at the end a new sentence: “Following E30, if the position is still invalid, the ERTMS/ETCS on-board shall delete the train position data (new status: “unknown”)” Modify 5.4.3.2 S23 by adding at the end a new sentence: “Following E31, if the position is still invalid, the ERTMS/ETCS on-board shall delete the train position data (new status: “unknown”)” Modify 5.4.3.2 S24 by adding at the end a new sentence: "Following E32, if the position is still invalid, the ERTMS/ETCS on-board shall delete the train position data (new status: “unknown”)” |

| Identification number | 0268 |
| --- | --- |
| Headline | End of mission: terminate communication session, if comm. session exists |
| Agreed Solution | Modify SUBSET-026 v2.3.0 as follows: Add new sub-clause 3.5.5.1 d): “If the driver closes the desk during the start of mission.” Modify clause 5.5.3.1.2 to read: “Intentionally deleted.” Modify clause 5.5.3.1.3: replace “If the ERTMS/ETCS level is 2 or 3:” with "If a communication session exists:" |

| Identification number | 0319 |
| --- | --- |
| Headline | LRBG orientation for special position report |
| Agreed Solution | Modify SUBSET-026 v2.3.0 as follows: Add new clause 3.4.2.1.2 : "The orientation of the co-ordinate system of a balise group (i.e., nominal or reverse direction) is identified as balise group orientation." Modify clause 3.6.5.1.3 to read: “Intentionally Deleted” Add 3.6.5.1.2 i): “optionally, the previous LRBG (see 3.4.2.3.3)” Modify 7.4.3.2: - Variable NID_PRVBG to read: "NID_PRVLRBG" - Comment to NID_PRVLRBG : replace with " Used as reference for all directional information in the packet: a move from PRVLRBG towards the LRBG defines the "nominal" direction"" - Comment to Q_DIRLRBG : replace with "Train orientation according to reference direction" - Comment to Q_DLRBG : replace with "Train front position according to reference direction" - Comments to Q_DIRTRAIN : add "Actual running direction according to reference direction" Modify 7.5.1.94: -Variable NID_PRVBG to read: "NID_PRVLRBG" -Cell "Name" to read: "Identity of previous LRBG" -Cell "Description: Replace (two occurrences) "previous balise group" with "previous LRBG" |

| Identification number | 0320 |
| --- | --- |
| Headline | Speed supervision behind LOA |
| Agreed Solution | Modify SUBSET-026 v2.3.0 as follows: Add new clause 3.7.2.2.1: “In case of LoA, trackside shall be responsible for including any track description beyond the LoA relevant for calculating safe supervision limits.” Modify clause 3.13.6.2.2 a) to read: “A Limit of Authority (LOA, Speed limit at LOA) if V_LoA is lower than the MRSP in rear of it” Modify clause 3.13.6.2.2.1 to read: “Intentionally deleted”. Modify table 4.8.4, (line “Default gradient for TSR” ): Replace “R” with “A” for FS, OS Modify table 4.10 (line "Default gradient for TSR"): Replace "D" with "U" for FS, OS |

| Identification number | 0344 |
| --- | --- |
| Headline | Loss of radio connection in announced radio hole |
| Agreed Solution | Modify SUBSET-026 v2.3.0 as follows: Add new clause 3.5.3.4 e): "When communication session has been terminated because of a loss of safe radio connection inside an announced radio hole, and the train front reaches the end of this radio hole (refer to 3.5.4.4)." |

| Identification number | 0390 |
| --- | --- |
| Headline | Balise consistency check in RV mode |
| Agreed Solution | Modify SUBSET-026 v2.3.0 as follows: Add new clause 4.4.18.1.10: "In case of balise group message consistency error (refer to 3.16.2.4.4) the ERTMS/ETCS onboard equipment shall reject the message but shall not command the service brake." |

| Identification number | 0392 |
| --- | --- |
| Headline | Ambiguity: SSP and gradient must cover "the full length of the MA" |
| Agreed Solution | Modify SUBSET-26 V230 as follows: Add note 3.7.2.3.1: Note: Full length means at least from the estimated front end of the train to the supervised location. 4.8.1 Figure 3: “[4] exception: the movement authority and, if received together with this movement authority, the mode profile shall be rejected if the SSP and gradient already available on-board or given together with the MA do not cover the full length of the MA.” condition [4] in 4.8.3.1 as follows “[4] exception: the movement authority and, if received together with this movement authority, the mode profile shall be rejected if the SSP and gradient already available on-board or given together with the MA do not cover the full length of the MA.” |

| Identification number | 0396 |
| --- | --- |
| Headline | OBU can receive MA and other information after End of Mission |
| Agreed Solution | Modify SUBSET-026 v2.3.0 as follows: Modify table 4.8.4 (column "SB"): add "[2]" for all lines with "A" except for lines "Session Management", "Configuration Determination", "Acknowledgement of session termination" and "Initiation of session". Below table 4.8.4, add new exception to read: "[2]: only if a cab is active" |

| Identification number | 0397 |
| --- | --- |
| Headline | Conditions for Start/End of Text Indication |
| Agreed Solution | Modify SUBSET-026 v 2.3.0 as follows: Modify clause 3.12.3.4.2 to read: "The following events can be used as start conditions: •Location •Mode (start display as soon as in mode) •Level (start display as soon as in level)" Add new clause 3.12.3.4.3: "The following events can be used as end conditions: • Location • Time • Driver acknowledgement • Mode (stop display when leaving mode) • Level (stop display when leaving level)" Add new clause 3.12.3.4.4 : "The end conditions shall be evaluated once the text message is displayed (i.e. only after the start condition has been fulfilled)" Add new clause 3.12.3.4.5 : “Once the text message is displayed and the end condition is fulfilled, the start conditions shall not be re-evaluated.” Add new clause 3.12.3.4.6 to read: "When the end condition "location" is used, the length on which the text is displayed shall refer to the location used as start condition, independently from other start condition(s)." Modify table A 3.4.1.3 : Add two new lines "plain text information (not location based)" and "fixed text information (not location based)", with "U" in first and second columns, "D" in third column Modify lines "plain text information" and "fixed text information" to read: "plain text information (location based)" and "fixed text information (location based)", with "D[8]" in first and second columns. Below the table A 3.4.1.3, add : "[8]: only if the location where to start to display the text is beyond the new stop location; otherwise all the text information (i.e. including end location where to stop display, if any) shall remain unchanged" Modify SUBSET-040 v 2.0.0 as follows: Add new engineering rule "text transmission" in chapter 4.2: line "Rule": "the use of the end condition "location" shall be allowed only if the start condition 'location" is used" line "Reference": "SUBSET-026 – section 3.12.3.4; 7.4.2.23/24" |

| Identification number | 0500 |
| --- | --- |
| Headline | Change of Train Data from sources different from the driver |
| Agreed Solution | Modify SUBSET-026 V230 as follows: See attachment "solution_for_CR500_101007.doc". |

| Identification number | 0530 |
| --- | --- |
| Headline | Reporting of PT mode when no communication session established |
| Agreed Solution | Modify SUBSET-026 v2.3.0 as follows: Modify clause 5.11.4.1.2 to read: "Intentionally deleted." |

| Identification number | 0531 |
| --- | --- |
| Headline | Start of Mission procedure re-engaged when session is not terminated. |
| Agreed Solution | Modify SUBSET-026 v2.3.0 as follows: Modify table 5.4.3.2 , cell S0 to read: “The Start of Mission procedure shall be engaged when the ERTMS/ETCS on-board equipment is in Stand-By mode with a desk open and if no pending communication session and/or radio safe connection is established.” State diagram 5.4.4: modify box S0 to add “and no communication session/radio safe connection is pending “ |

| Identification number | 0560 |
| --- | --- |
| Headline | No End of mission reporting when no communication established |
| Agreed Solution | Modify SUBSET-026 v2.3.0 as follows: Add new sub clause 3.5.3.4 c): "If a mode change, not considered as an End of Mission, has to be reported to the RBC (only if level 2 or 3)." Modify table 4.5.2 (line "Report train position when mode changes"): add footnote reading "For ETCS levels 2 and 3 this may imply establishing a radio communication session if none is established." Add new clause 5.5.3.1.3.2: "Note: The “End of Mission” message contains a position report." Modify table 5.11.3, cell A030 : delete "(only if a communication session is open)" |

| Identification number | 0601 |
| --- | --- |
| Headline | OS acknowledgement |
| Agreed Solution | Modify SUBSET-026 v2.3.0 as follows: Modify clause 5.7.3.4 to read: Until the on-board equipment has switched to SH mode, the beginning of the shunting area shall be considered as both the EoA and SvL (instead of the ones given by the MA), with no Release speed. Modify clause 5.9.3.5 to read: Until the on-board equipment has switched to OS mode, the beginning of the On Sight area shall be temporarily considered as both the EoA and SvL (instead of the ones given by the MA), with no Release Speed. |

| Identification number | 0613 |
| --- | --- |
| Headline | Emergency stop revocation after level transition |
| Agreed Solution | Modify SUBSET-026 v2.3.0 as follows: Modify table 4.10.1 (lines "Unconditional Emergency Stops" and "Conditional Emergency Stops") : Replace "U" with "D" for SE,SN |

| Identification number | 0619 |
| --- | --- |
| Headline | Message Acknowledgement |
| Agreed Solution | Modify SUBSET-026 v2.3.0 as follows: Modify clause 3.16.1.1.1 to read: "Note: a value of a variable is invalid when a spare value is used" Modify clause 3.16.3.1.1 to read: "A radio message is consistent when all checks have been completed successfully: a) Checks performed by Euroradio protocol have been passed (see Subset-037) b) Time stamps checks have been passed (see 3.16.3.3.3) c) Variables in the message do not have invalid values." Add new clause 3.16.3.1.1.1 to read: "The on-board shall reject a message transmitted from the trackside if the message is not consistent." Add new clause 3.16.3.1.1.2 to read: "The on-board shall inform the trackside if a not consistent message is received." Modify clauses 3.16.3.4.1 and 3.16.3.4.5 a) as follows: replace "valid" with "consistent" Modify clause 3.16.3.5.1 to read: "As soon as a consistent message (see 3.16.3.1.1) including the request for acknowledgement is received, the on-board shall send an acknowledgement to the trackside." Modify clause 3.16.3.5.2 to read: "Intentionally deleted" Modify 7.5.1.64 as follows: Special/Reserved Value 3, add "(ref. 3.16.3.1.1a,c)" to the description field Special/Reserved Value 4, add "(ref. 3.16.3.1.1b)" to the description field |

| Identification number | 0653 |
| --- | --- |
| Headline | Contradiction Level transition immediately in PT |
| Agreed Solution | Modify SUBSET-026 v2.3.0 as follows: Modify table 4.8.4 (line “Level transition order”) : add "[5]" for PT Add below table 4.8.4: "[5]: only level transition announcement (i.e., “immediate” level transition order shall be rejected)" |

| Identification number | 0664 |
| --- | --- |
| Headline | Unclear requirements for Route suitability |
| Agreed Solution | Modify SUBSET-026 v2.3.0 as described in attachment "Proposal for route suitability-final decision.doc" |

| Identification number | 0682 |
| --- | --- |
| Headline | CR419 not in line with CR445 |
| Agreed Solution | Modify SUBSET-026 v2.3.0 as follows: Modify table 4.8.4 as follows: lines "Temporary Speed Restriction", "default gradient for TSR", "plain/fix text information", "geographical position": replace "R" with "A" for modes SE and SN |

| Identification number | 0686 |
| --- | --- |
| Headline | Ambiguities regarding reversing information in SRS v230 |
| Agreed Solution | Modify SUBSET-026 V230 as follows: See attachment "U686 solution updated 20070629_LR.doc". |

| Identification number | 0688 |
| --- | --- |
| Headline | Consistency with old M_VERSION |
| Agreed Solution | Modify SUBSET-026 v2.3.0 as follows: Add new clause 3.17.3.4.2 : "The version check shall be done prior to any further checks (data consistency,..) as they depend on the system version." |

| Identification number | 0695 |
| --- | --- |
| Headline | missing limitations for stored info in SUBSET-040 |
| Agreed Solution | Modify SUBSET-040 v2.0.0 as follows: Modify chapter 4.3.2: Add new rule "Number of train categories per segment of SSP": Column "Maximum number of iterations per segment of SSP" : line "Rule" : "15" line "Reference": "SUBSET-026 – section 7.4.2.7" Add new rule "Number of Axle load speed profile segments": Column "Maximum number of iterations of ASP segments in 1 packet": line "Rule": "14" line "Reference" : "SUBSET-026 – section 7.4.2.13" Column "Minimum memorised on board": line "Rule": "30" Add new rule "Number of Axle load speed restriction values per ASP segment": Column "Maximum number of iterations per ASP segment": line "Rule": "3" line "Reference": "SUBSET-026 – section 7.4.2.13" Add new rule "Number of adhesion profiles": Column "Maximum number of iterations in 1 packet" line "Rule": "No iteration in packet" line "Reference": "SUBSET-026 – section 7.4.2.22" Column "Minimum memorised on board": line "Rule": "10" Add new rule "Number of reversing area": Column "Maximum number of iterations in 1 packet" line "Rule": "No iteration in packet" line "Reference": "SUBSET-026 – section 7.4.2.34" Column "Minimum memorised on board": line "Rule": "1" |

| Identification number | 0699 |
| --- | --- |
| Headline | List of balises in SH deleted in A3.4 for situations a-f, g-j, l |
| Agreed Solution | Modify SUBSET-026 v2.3.0 as follows: Add new item A.3.4.1.2 m): "inconsistency in a balise group marked as linked and no linking is used onboard (3.16.2.4.4.2)" Modify table A.3.4.1.3: Change column "g-j, l" to "g-j, l, m" Replace "D" with "D[9]" in column "g-j, l, m", line "List of balises for SH area" Add below table A.3.4.1.3: "[9]: unchanged if the onboard is in SH mode" |

| Identification number | 0702 |
| --- | --- |
| Headline | Update of list of expected balises in SR |
| Agreed Solution | Modify SUBSET-026 v2.3.0 as follows: Add new clause 4.4.11.1.6.6: "If the train is in SR and receives a new distance to run in SR mode from the RBC, the stored list of expected balise groups, if any, shall be deleted or shall be replaced by the list of expected balise groups sent together with the distance to run in SR." |

| Identification number | 0706 |
| --- | --- |
| Headline | Mismatch between announced and read balise group direction |
| Agreed Solution | Modify SUBSET-026 v2.3.0 as follows: Add new clause 3.4.4.4.2.1: "When linking information is used on-board and the expected balise group is referred in the linking information with a balise group with ID “unknown”, a balise group marked as linked shall only be taken account of if: a) the train can determine the orientation of the linked balise group by information from the balise group itself (therefore excluding for example single balise groups), AND b) the balise group contains repositioning information valid for the train orientation, AND c) the balise group is crossed with the direction announced in the linking information." Add new clause 3.16.2.3.2.1: " Exception: When the expected balise group is referred in the linking information with a balise group with ID “unknown”, 3.4.4.4.2.1 shall apply." Modify 7.5.1.85, comment to special value "16383": Replace "only accept balise groups containing repositioning information for the running direction (only to be used in linking)" with "Identity is unknown (only to be used for Linking information)" |

| Identification number | 0713 |
| --- | --- |
| Headline | PT distance D_NVPOTRP origin |
| Agreed Solution | Modify SUBSET-026 V230 as follows: Add clause 3.18.2.11 "When a new set of National Values becomes applicable, any on-going supervision involving a National Value of type time or distance shall continue using the corresponding value from the new set. However, the starting location or starting time shall remain unchanged." Add the following sentence to 3.18.4.6.2.1 "If, following a change of National Values, the update of the adhesion factor is no more permitted to the driver, the adhesion factor previously modified by the driver to slippery rail shall immediately be reset to non slippery rail and the driver shall be informed. Any trackside adhesion profile is not affected." Delete the following sentence in 4.4.11.1.3.1 a) "If a new National value for the max permitted distance to run in SR mode is received, the start location shall remain unchanged." |

| Identification number | 0718 |
| --- | --- |
| Headline | location related conditions vs reverse movement in PT or roll-back |
| Agreed Solution | Modify SUBSET-026 V230 as follows: Replace clause 3.6.4.5 with ”Intentionally deleted”. Add Annex A3.5, Title to read: “Handling of Actions in Specific Situations” Add new clause A3.5.1 to read: “Regards actions executed in reference to location information received from trackside, the on-board equipment shall ensure that the action related to a location is neither reverted, nor executed twice. Situations to be considered shall include reverse movement (initiated by driver or due to roll-away) and the sub-sequent forward movement, or adjustment of train position on passing a new LRBG. This rule shall apply to the following actions: • Change of National Values (see 3.18.2) • Request to acknowledge new level, level transition (see SRS chapter 5.10) • Override of a route suitability stop location (see 3.12.2) • Start and stop displaying plain or fixed text messages (see 3.12.3) • Request to acknowledge a mode profile, mode transition due to mode profile (see 3.12.4) • Start and stop accepting radio in-fill information (see 3.9.3) • Actions related to RBC/RBC handover (see 3.15.1) • Actions related to track condition information with the exception of 3.12.1 (big metal masses) • Permission to initiate Reversing mode, i.e., limits of Reversing Area (see 3.15.4) • Start and stop Track Ahead free request to driver (see 3.15.5)” • Start and stop calculation of geographical position (see 3.6.6) |

| Identification number | 0729 |
| --- | --- |
| Headline | Single balises, assignment of a co-ordinate system |
| Agreed Solution | Modify SUBSET-26 V230 as follows: See attachment "CR729_solution_170108.doc". |

| Identification number | 0744 |
| --- | --- |
| Headline | National values |
| Agreed Solution | Modify SUBSET-026 V230 as follows: Add new clause 3.18.2.8.1: “When a new set of national values becomes applicable it shall always overwrite the one currently applicable regardless of the country or region identifier(s).” |

| Identification number | 0748 |
| --- | --- |
| Headline | Radio Link Supervision during RBC-RBC Handover |
| Agreed Solution | Modify SUBSET-026 V230 as follows. Add new clause 3.16.3.4.1.2 to read: “When an RBC/RBC handover has been announced, the current onboard time shall be compared with the time stamp of the latest consistent message from the Handing over RBC until the train considers the Accepting RBC as the supervising one (refer to 3.15.1.3.5). From then on the current onboard time shall be compared with the time stamp of the latest received consistent message from the Accepting RBC.” Modify 5.15.4.1 to read:: "Note: If the “Handing Over” RBC is not able to extend the MA into the area of the “Accepting RBC”, the driver may select "override", or manually change the level to move the train into the area of the “Accepting RBC”." Modify 5.15.4.2 to read: "Note: If the ERTMS/ETCS on-board equipment cannot open a session with the “Accepting RBC”, the train is stopped at the latest when it has reached the EoA given by the “Handing Over” RBC. For passing the EoA the driver may select "override", or manually change the level." |

| Identification number | 0770 |
| --- | --- |
| Headline | Train category spare values |
| Agreed Solution | Modify SUBSET-026 V230 as follows: See attachment "CR770_solution_EEIG_270907.doc". |

| Identification number | 0771 |
| --- | --- |
| Headline | Value NID_RBC is unknown |
| Agreed Solution | Modify SUBSET-026 V230 as follows: Modify 3.5.3.13 to read: “An order to contact the RBC may contain a special value for the RBC identity indicating that the on-board shall contact the last known RBC (i.e., using the stored RBC ID/phone number, if any) ; the phone number indicated in the order shall be ignored by the on-board equipment.” Add new sub-clause 3.5.3.13.1 to read: "If there is no RBC ID/ phone number stored on-board, the order to contact the RBC shall be ignored." Modify 3.5.3.16 (Table) regards Option 1: column "balise data content": replace “Intentionally deleted" with "Order to contact RBC special value for RBC ID: Contact last known RBC RBC Phone number irrelevant" column "Train Reaction": replace "Intentionally deleted" with "Contact last known RBC (order is ignored in case no RBC ID/ phone number is stored on-board)" Modify 7.4.2.10: in Comment cell NID_C/NID_RBC: replace "NID_C not relevant if NID_RBC has value unknown" with “NID_C not relevant if NID_RBC has value “Contact last known RBC”” in Comment cell NID_RADIO: replace "not relevant if NID_RBC has value unknown" with “not relevant if NID_RBC has value “Contact last known RBC”” Modify 7.5.1.96 NID_RBC: line "Special/Reserved Values": replace “unknown” by “Contact last known RBC” Modify 7.5.1.97 NID_RIU: Remove line "Special/Reserved Values" Column "Maximum value": replace "16 382" with "16 383" Modify SUBSET-040 v2.0.0 as follows: in section 4.2.4, add new rule “packet 131 (RBC transition order)” “Rule”: “It shall be forbidden to use the special value “Contact the last known RBC” for the RBC ETCS identity number NID_RBC.” “Reference”: “SUBSET-026 – section 7.5.1.96” “Justification”: void |

| Identification number | 0778 |
| --- | --- |
| Headline | Geographical position reference balise groups |
| Agreed Solution | Modify SUBSET-026 V230 as follows See attachment ERA_solution_for_CR778_mod SG 171007.doc |

| Identification number | 0781 |
| --- | --- |
| Headline | Relocation of data for MA in entry to Level 2 |
| Agreed Solution | Modify SUBSET-026 V230 as follows: Add new clause 4.8.1.6 : “When evaluating trackside information received by radio or when re-evaluating a set of information released from the transition buffer, linking information, if any, shall be evaluated prior to any other location related information” Modify clause 4.8.2.1 to read: “The following tables shall be applied assuming that:” Add new sub-clause 4.8.2.1 d): “to check exception [4] in 4.8.3, the track description is referred to the LRBG” |

| Identification number | 0787 |
| --- | --- |
| Headline | Activation of supervision of safe radio connection/ Follow-up CR529 |
| Agreed Solution | Modify SUBSET-026 V230 as follows: Add new clause 3.16.3.4.1.1: “After the on-board equipment has switched to L2 or 3 with no communication session established yet and until a new consistent message has been received, the current onboard time shall be compared with the on-board time at the moment of the level transition (instead of the time stamp of the latest consistent received message)." Add new clause 3.16.3.4.1.3: “After a train has passed an announced radio hole and until a new consistent message has been received, the current onboard time shall be compared with the on-board time when the train front end left the radio hole (instead of the time stamp of the latest consistent received message).” |

| Identification number | 0788 |
| --- | --- |
| Headline | Repetition of linking reactions |
| Agreed Solution | Modify SUBSET-026 as follows: Add new clause 3.4.4.4.5: "The on-board equipment shall expect balise groups one by one ( i.e., it shall supervise only one expectation window at a time) according to the order given by linking information." Add new clause 3.4.4.4.6: "The ERTMS/ETCS on-board equipment shall stop expecting a balise group and shall expect the next one announced in the linking information (if any) when: a) the balise group is found inside its expectation window b) a linking consistency error is found, see 3.16.2.3.1" Add new clause 3.4.4.4.6.1: “Linking consistency error due to early reception of balise group expected later (see 3.16.2.3.1 c)): if the balise group found is the next one announced in the linking information, the ERTMS/ETCS on-board equipment shall check its linking consistency and apply again clause 3.4.4.4.6, i.e. it will immediately expect the further next balise group announced in the linking information.” Modify clause 3.16.2.3.1 to read: "If linking information is used the on-board shall react according to the linking reaction information in the following cases: a) If the location reference of the expected balise group is found in rear of the expectation window b) If the location reference of the expected balise group is not found inside the expectation window (i.e. the end of the expectation window has been reached without having found the expected balise group). c) If inside the expectation window of the expected balise group another announced balise group, expected later, is found." Add new clause 3.16.2.3.1.1: "The ERTMS/ETCS on-board equipment shall ignore (i.e. it will not consider as LRBG) a balise group found with its location reference outside its expectation window." |

| Identification number | 0791 |
| --- | --- |
| Headline | Repositioning and cross talk |
| Agreed Solution | Modify SUBSET-026 V230 as follows: Add subtitle 3.16.2.7.1:”Mitigation of balise reception degradation” Renumber clause 3.16.2.7.1 to 3.16.2.7.1.1 and add “the” before “service brake” Replace clause 3.16.2.7.2 by subtitle: “Mitigation of balise cross-talk while expecting repositioning information”. Add new clauses: 3.16.2.7.2.1 If repositioning is announced and the expected repositioning balise group has been found, the ERTMS/ETCS on-board equipment shall keep looking for a balise group that satisfies the same criteria as this previously expected and already found repositioning balise group, until one of the following events occurs: a) the on-board antenna leaves the expectation window of the repositioning balise group that was announced and already found b) a linked balise group that has been announced with known identity is found. 3.16.2.7.2.2 If a second balise group is found that satisfies the same criteria as the previously expected and already found repositioning balise group, the ERTMS/ETCS on-board equipment shall command the service brake. At standstill, the current MA and track description, if available on-board, shall be shortened to the current position. The driver shall be informed. 3.16.2.7.2.3 Note: this function is independent from linking function, i.e. the rules related to linking always apply. This means that once a repositioning balise group has been found and if this latter contains new linking information, the ERTMS/ETCS on-board equipment will start expecting the first balise group announced in this new linking information in parallel with the monitoring specified in 3.16.2.7.2.1 |

| Identification number | 0792 |
| --- | --- |
| Headline | Storage of information in case of level transition announcement or RBC/RBC handover |
| Agreed Solution | See attachment "ERA_solution_for_CR792_140108.doc" |

| Identification number | 0796 |
| --- | --- |
| Headline | Compatible upgrade |
| Agreed Solution | Modify SUBSET-026 v2.3.0 as follows: Modify clause 3.17.3.11 to read: "In compatible versions, the on-board equipment shall not consider the reception of unknown packet/message as a message data consistency error (i.e. use of spare value for NID_PACKET or NID_MESSAGE) and shall ignore the content of the unknown packet/message in the following cases: a) unknown packet included in a balise telegram/loop message related to the newer system version; b) unknown radio message from an RBC or RIU operating with the newer system version; c) unknown packet from an RBC or RIU operating with the newer system version, included in a message in which optional packets can be added according to the version operated by on-board." |

| Identification number | 0797 |
| --- | --- |
| Headline | Maximum length of radio message |
| Agreed Solution | Modify SUBSET-040 V200 as follows Rule 4.2.2.1: Field "Rule" to read as follows: "Application data (excluding Euroradio protocol data) sent as normal priority data shall not exceed 500 bytes." Field "Remark" to read as follows: "A maximum number of bytes is not relevant for high priority data as only fixed size messages are used." |

| Identification number | 0798 |
| --- | --- |
| Headline | Mechanism for deletion of information on-board missing |
| Agreed Solution | Modify SUBSET-026 V230 as follows. Introduce new chapter: A3.6 Deletion of accepted and stored information when used A3.6.1 Standard case A3.6.1.1 When the train moves in the direction of its train orientation, storage capacity occupied by trackside information no longer used, i.e., the related on-board functionality has been completed, shall be made available immediately. A3.6.1.1.1 Note: The requirement is needed to allow trackside to predict the storage capacity available on-board in order to comply with dimensioning rules regards information stored on-board given in Subset 040. A3.6.2 Exception A3.6.2.1 Following information shall remain stored on-board for a distance defined by a fixed value in rear of the min safe rear end location of the train: • location dependent static speed restrictions (see 3.11.2.2) , i.e., SSP, ASP, TSR • gradient information, • reduced adhesion information received from trackside, • Track condition “Big metal masses”. A3.6.2.1.1 Note: The above information remains stored for the case of a reverse movement: • With the exception of the track condition “Big metal masses”, the stored information allows the ERTMS/ETCS on-board equipment to calculate speed supervision limits after a reverse movement (roll-away, or initiated by the driver) • Track condition “Big metal masses” is needed also for a reverse movement itself to avoid any false alarms due to Big metal masses in the track. A3.6.2.1.2 Note: The distance to intervention of the roll away or reverse movement supervision is determined by a National/Default value. This is also true for a reverse movement in Post trip mode. However, following an intervention, the train will not stop immediately. In order to keep the on-board functionality simple, a fixed distance value was chosen to define an unambiguous location in rear of the train where the above information is no longer required and the related on-board storage capacity is made available again. Add to table A3.1 (List of Fixed Value Data) the following value: Fixed Values Data: “Distance to keep on-board information in rear of the min safe rear end of the train” Value: “300m” Name: “-“ |