| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 3170300 (3.17.3: Management of ERTMS/ETCS system versions) Total: 11 Test case(s) |  |
| REF: | Subset-076-5-2-3170300 |
| ISSUE: | Version 3.2.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 2.9.0 31.03.2013 | All | First version/review | DLR |
| Version 2.9.1 17.10.2013 | All | review | DLR |
| Version 2.9.2 04.12.2013 | 7, 8, 10, 11 | added missing packets for BG in step 1 | DLR |
| Version 2.9.3 06.12.2013 | 7, 8, 10, 11 | review | DLR |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.2.0 31.03.2017 | All | Baseline 3 1st maintenance release version - Bug fixing | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.17.3: Management of ERTMS/ETCS system versions |  | 3170300 |
| Test Case of Feature | Supervision of ETCS version compatibility for EUROLOOP messages. |  | 1 |
| Applicable Mode/Level Combinations | L1: FS, LS |  |  |
| Target of Test | The target of this test is to check that the on-board equipment accepts the Euroloop message in case of compatibility of Euroloop ETCS version X with at least one of the ETCS compatibility versions stored by the on-board equipment. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.17.3.6 |  |
|  |  | Subset-026-3.17.3.6 b) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : • The Euroloop message is recorded. Check on the DMI that : • There is NO indication showing an ETCS version incompatibility between the Euroloop message and the on-board equipment.(Indication according to requirement Subset 026-3.17.3.5 ) Check on the packet-dependent interfaces that the on-board equipment reacts according to the packets contained in the accepted Euroloop message, i.e. packets are taken into account by the on-board equipment. |
| Constraints | • The ETCS version of the Euroloop message is compatible with at least one of the ETCS compatibility versions stored by the on-board equipment. • Data included in the Euroloop message shall be chosen in such a way that it is easy to verify on at least one interface whether the data is accepted or refused by the on-board equipment. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| Movement Authority | STORED | - |
| EOLM information | STORED | Q_SSCODE=STORED EOLM balise group directionality=NOMINAL Q_DIR=NOMINAL Q_LOOPDIR=SAME (Same as EOLM directionality) List of ETCS compatibility versions=AVAILABLE (A list of ETCS compatibility versions is available on-board) |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Euroloop message containing packet 136, 39 and packet 255 is received | I | LTM | The train passes over an Euroloop and receives a Euroloop message with : M_VERSION compatible with at least one of the ETCS versions stored on-board | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM EUROLOOP (NID_MESSAGE_JRU=7) is recorded | O | JRU | The received Euroloop message is recorded on the JRU. | N/A | N/A | - |
| 3 | N/A | N/A | The Euroloop message is accepted. | O | DMI | There is no indication of incompatibility ETCS version. | N/A | N/A | - |
| 4 | N/A | N/A | The Track Condition symbol "Change of traction system: AC 15 kV 16.7 Hz announcement" is displayed | O | DMI | This proofs that the Euroloop message has been accepted | N/A | N/A | - |
| 5 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit70=1>) is recorded | O | JRU | Bit 70 for symbol TC27 (New traction system and Change of Traction system announcement “AC 15 kV 16.7 Hz”; grey) | N/A | N/A | - |

| Step 1: Euroloop Message |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 1 | Defines the type of media: Loop (1) |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_LOOP | 14 | FINITE VALUE | Identity of Euroloop |
| NID_PACKET | 8 | 39 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_TRACTION | 15 | FINITE VALUE | - |
| M_VOLTAGE | 4 | 2 | Identity of the traction system. (2 = AC 15 kV 16.7 Hz) |
| NID_CTRACTION | 10 | FINITE VALUE | NID_CTRACTION given only if M_VOLTAGE ≠ 0. |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| Movement Authority | STORED | ALL TRANSMITTED INFORMATION BY EUROLOOP TELEGRAM : e.g. MA |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Reaction according to the packets contained in the Euroloop message. | O | TIU | - |
| Reaction according to the packets contained in the Euroloop message and No indication of ETCS version incompatibility error | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.17.3: Management of ERTMS/ETCS system versions |  | 3170300 |
| Test Case of Feature | Supervision of ETCS version compatibility for EUROLOOP messages. |  | 2 |
| Applicable Mode/Level Combinations | L1: FS, LS |  |  |
| Target of Test | The target of this test is to check that the on-board equipment does not react according to the data packets contained in the received Euroloop message whose ETCS version X is greater than the highest version number X supported by the on-board equipment. The Euroloop message is ignored. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.17.3.6 |  |
|  |  | Subset-026-3.17.3.6 c) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : • There is no indication showing that there is an ETCS incompatibility between the Euroloop message and the on-board equipment. Check on the JRU that : • The Euroloop message is recorded (because ETCS version incompatibility is not a correctness problem). Check on the packet-dependent interfaces that there are no observable packet-dependent reactions since the Euroloop message was rejected. |
| Constraints | • The ETCS version of the Euroloop message is greater than the highest version number supported by the on-board equipment. • Data included in the Euroloop message shall be chosen in such a way that it is easy to verify on at least one interface whether the data is accepted or refused by the on-board equipment. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| Movement Authority | STORED | - |
| EOLM information | STORED | Q_SSCODE=STORED EOLM balise group directionality=NOMINAL Q_DIR=NOMINAL Q_LOOPDIR=SAME (Same as EOLM directionality) List of ETCS compatibility versions=AVAILABLE (A list of ETCS compatibility versions is available on-board) |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Euroloop message containing packet 136, 12 and packet 255 is received | I | LTM | The train passes over an Euroloop and receives a Euroloop message with : M_VERSION greater than the highest of the ETCS versions stored on-board | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM EUROLOOP (NID_MESSAGE_JRU=7) is recorded | O | JRU | The received Euroloop message is recorded on the JRU. | N/A | N/A | - |
| 3 | N/A | N/A | The Euroloop message is ignored. | O | DMI | There is no indication of incompatibility ETCS version. No packet-dependent reactions: in case Transmission of L1 MA via Euroloop is used, no extension of MA is applied. | N/A | N/A | - |

| Step 1: Euroloop Message |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 1 | Defines the type of media: Loop (1) |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_LOOP | 14 | FINITE VALUE | Identity of Euroloop |
| NID_PACKET | 8 | 12 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out". |
| N_ITER | 5 | FINITE VALUE | Set to zero if V_MAIN = 0 or if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| Movement Authority | STORED | Not extended by means of Euroloop transmission |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| No packet-dependent reactions | O | TIU | Because the EUROLOOP message has been ignored |
| No packet-dependent reactions | O | DMI | Because the EUROLOOP message has been ignored |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.17.3: Management of ERTMS/ETCS system versions |  | 3170300 |
| Test Case of Feature | Supervision of ETCS version compatibility for EUROLOOP messages. |  | 3 |
| Applicable Mode/Level Combinations | L1: FS, LS |  |  |
| Target of Test | The target of this test is to check that: - For trackside information only differing by Y with regards to the highest system version number X supported by on-board, the on-board equipment ignores unknown packets/messages of the loop message related to the newer system version. - the unknown packet/message is not considered as a message data consistency error form the ETCS on-board device. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | CEDEX / INECO / DLR / MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.17.3.11 |  |
|  |  | Subset-026-3.17.3.11 a) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : • There is no indication showing that there is an ETCS incompatibility between the Euroloop message and the on-board equipment. Check on the JRU that : • The Euroloop message is recorded Check on the packet-dependent interfaces that there are observable old packet-dependent reactions, and that no new packet dependent reaction is shown. |
| Constraints | • The ETCS version of the Euroloop message is compatible with one of the ETCS compatibility versions stored by the on-board equipment, but includes new information . • Data included in the Euroloop message shall be chosen in such a way that it is easy to verify on at least one interface whether the data is accepted or refused by the on-board equipment. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| Movement Authority | STORED | - |
| EOLM information | STORED | Q_SSCODE=STORED EOLM balise group directionality=NOMINAL Q_DIR=NOMINAL Q_LOOPDIR=SAME (Same as EOLM directionality) List of ETCS compatibility versions=AVAILABLE (A list of ETCS compatibility versions is available on-board) |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Euroloop message containing packet 136, 12, packet 155 and packet 255 is received | I | LTM | The train passes over an Euroloop and receives a Euroloop message with : a compatible M_VERSION only differing by Y with regards to the highest system version number X supported by on-board. | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM EUROLOOP (NID_MESSAGE_JRU=7) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | new MA is shown | N/A | N/A | - |
| 4 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Euroloop Message |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 1 | Defines the type of media: Loop (1) |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_LOOP | 14 | FINITE VALUE | Identity of Euroloop |
| NID_PACKET | 8 | 12 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out". |
| N_ITER | 5 | FINITE VALUE | Set to zero if V_MAIN = 0 or if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 155 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| L_TEXT | 8 | FINITE VALUE | - |
| X_TEXT(L_TEXT) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | O | TIU | - |
| No REACTION to the transmitted new-packets is OBSERVED REACTION to the transmitted old common packets is OBSERVED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| All information transmited by EUROLOOP that had a system version compatible with the on-board system version. All information transmited by EUROLOOP that had a system version NOT compatible with the on-board system version shall NOT be stored. | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.17.3: Management of ERTMS/ETCS system versions |  | 3170300 |
| Test Case of Feature | Handling of trackside data in relation to system version. |  | 4 |
| Applicable Mode/Level Combinations | L0: SH, UN, PS, SL, NL LNTC: PS, SL, NL, SN L1: FS, LS, OS, SR, SH, PS, SL, SB, TR, PT, NL, RV L2: FS, LS, OS, SR, SH, PS, SL, SB, TR, PT, NL, RV L3: FS, LS, OS, SR, SH, PS, SL, SB, TR, PT, NL, RV |  |  |
| Target of Test | The target of this test case is to check that if: - the on-board unit is operated with the system version number X = 1 and - the on-board unit is capable of operating with the system version number X = 2 and - a balise message is received with system version number X = 2 containing packet 0 and - the received VBC is not stored on-board, then - the on-board unit sets the system version number to X = 2 prior to checking validity of NID_PACKET of received packet 0 - the message is accepted - the SB is NOT APPLIED - a balise message is received with system version number X = 1 containing packet 0 and - the on-board unit checks the system version prior to checking validity of NID_PACKET of received packet 0 - the message is rejected - the SB is APPLIED |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.17.3.3 |  |
|  |  | Subset-026-3.16.2.5.1 |  |
|  |  | Subset-026-3.16.2.5.1 c) |  |
|  |  | Subset-026-3.17.3.5 |  |
|  |  | Subset-026-3.17.3.5 c) |  |
|  |  | Subset-026-3.17.2.3 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the States of ERTMS/ETCS variables that: • The operating system version is set to X = 2 Check on the interfaces that: • The BG message from BGa is ACCEPTED, no data consistency error is found (Packet 0 is valid in system version X=2) • The BG message from BGb is REJECTED, data consistency error is found (Packet 0 is invalid in system version X=1) Check on the JRU that : • the message is RECORDED • application of SB is RECORDED |
| Constraints | • a cab needs to be active (SB mode) • the on-board unit is capable of operating with the system version number X = 2 |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 4 / 5 / 6 / 7 / 8 / 12 / 13 / 14 / 15 | FS / OS / SR / SH / UN / SL / SB / TR / PT / LS / SN / RV / PS |
| Movement Authority | STORED | If Level L1 / L2 / L3 and Mode FS / OS / LS |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |
| Operated System Version | STORED | X=1 |
| Virtual Balise Covers | NOT STORED | The VBO received in packet 0 must not be stored on-board, in order for the message not to be ignored. |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (NID_BG=<BGa>; Q_LINK=0) containing packet 0 and packet 255 is received | I | BTM | The BG system version is X = 2The packets included in the BG shall be selected in order to be able to test that the BG message is accepted. | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | NOT Service brake commanded | O | TIU | - | N/A | N/A | - |
| 4 | N/A | N/A | NOT SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | One Balise Group message (NID_BG=<BGb>; Q_LINK=0) containing packet 0 and packet 255 is received | I | BTM | The BG system version is X = 1 | N/A | N/A | - |
| 6 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | Service brake commanded | O | TIU | Due to data consistency checks, as NID_PACKET=0 is an invalid value in system version X=1 | N/A | N/A | - |
| 8 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | The Status symbol "Service Brake or Emergency Brake intervention" is displayed | O | DMI | SB Indication is DISPLAYED. | N/A | N/A | - |
| 10 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 11 | N/A | N/A | Train is at standstill (V_TRAIN=0) | I | INT | - | N/A | N/A | - |
| 12 | N/A | N/A | NOT Service brake commanded | O | TIU | SB is RELEASED | N/A | N/A | - |
| 13 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=0) is recorded | O | JRU | SB revocation is RECORDED M_BRAKE_COMMAND_STATE=0 (Release) | N/A | N/A | - |
| 14 | N/A | N/A | The Status symbol "Service Brake or Emergency Brake intervention" is removed | O | DMI | SB indication is REMOVED | N/A | N/A | - |
| 15 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=0>) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/N) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system X=2 |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. I of N |
| N_TOTAL | 3 | N | Total number of balises in the balise group N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group Except 254 |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | "BGa" | Identity of the balise group |
| Q_LINK | 1 | 0 | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 0 | Virtual Balise Cover marker |
| NID_VBCMK | 6 | FINITE VALUE | The VBC in this packet shall not be stored in the on-board, otherwise the message will be ignored. |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 5: Eurobalise Telegram (balise 1/N) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system X=1 |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. I of N |
| N_TOTAL | 3 | N | Total number of balises in the balise group N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group Except 254 |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | "BGb" | Identity of the balise group |
| Q_LINK | 1 | 0 | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 0 | Virtual Balise Cover marker |
| NID_VBCMK | 6 | FINITE VALUE | The VBC in this packet shall not be stored in the on-board, otherwise the message will be ignored. |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 4 / 5 / 6 / 7 / 8 / 12 / 13 / 14 / 15 | FS / OS / SR / SH / UN / SL / SB / TR / PT / LS / SN / RV / PS |
| Movement Authority | NOT STORED | MA is shortened at standstill to the current position after passing a balise with an unknown value |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |
| Operated System Version | STORED | X=2 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 5

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.17.3: Management of ERTMS/ETCS system versions |  | 3170300 |
| Test Case of Feature | Test that a telegram is ignored in case its system version number X equals to 0. |  | 5 |
| Applicable Mode/Level Combinations | L0: SH, UN, PS, SL, NL LNTC: PS, SL, NL, SN L1: FS, LS, OS, SR, SH, PS, SL, SB, TR, PT, NL, RV L2: FS, LS, OS, SR, SH, PS, SL, SB, TR, PT, NL, RV L3: FS, LS, OS, SR, SH, PS, SL, SB, TR, PT, NL, RV |  |  |
| Target of Test | The target of this test case is to check that if: - the system version number X transmitted by a balise equals to 0, then - the message is ignored |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.17.3.5 |  |
|  |  | Subset-026-3.17.3.5 a) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the interfaces that: • The balise information is IGNORED Check on the JRU that : • the message is RECORDED • NID_C and NID_BG are RECORDED |
| Constraints | • a cab needs to be active (SB mode) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 4 / 5 / 6 / 7 / 8 / 12 / 13 / 14 / 15 | FS / OS / SR / SH / UN / SL / SB / TR / PT / LS / SN / RV / PS |
| Movement Authority | STORED | If Level L1 / L2 / L3 and Mode FS / OS / LS |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (NID_BG=<BGa>) containing packet 72 (X_TEXT=Test failed), packet 41 (M_LEVELTR=Unequal to actual level), packet 255 is received | I | BTM | The train overpasses a BG In telegram of balise #i, the system version number X equals to 0 The packets included in the BG should be selected in order to be able to test step 2. Packet 72 for testing any mode without PS, SH, SL. Packet 41 for testing any mode of PS, SH, SL. | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | The BG message (NID_C and NID_BG) is recorded. BG message is IGNORED | N/A | N/A | - |
| 3 | N/A | N/A | No Text message will be displayed, and no level transition will apply | O | DMI | Nothing will happen otherwise the test is failed | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 72 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TEXTCLASS | 2 | FINITE VALUE | - |
| Q_TEXTDISPLAY | 1 | FINITE VALUE | Start/end events relation. |
| D_TEXTDISPLAY | 15 | FINITE VALUE | Start event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | Start event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | Start event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| L_TEXTDISPLAY | 15 | FINITE VALUE | End event. |
| T_TEXTDISPLAY | 10 | FINITE VALUE | End event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | End event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | End event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| Q_TEXTCONFIRM | 2 | FINITE VALUE | - |
| Q_CONFTEXTDISPLAY | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| Q_TEXTREPORT | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| NID_TEXTMESSAGE | 8 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_C | 10 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_RBC | 14 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| L_TEXT | 8 | FINITE VALUE | - |
| X_TEXT(L_TEXT) | 8 | FINITE VALUE | Test failed |
| NID_PACKET | 8 | 41 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | FINITE VALUE | - |
| M_LEVELTR | 3 | FINITE VALUE | Unequal to actual level |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTR = 1 (NTC). |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 4 / 5 / 6 / 7 / 8 / 12 / 13 / 14 / 15 | FS / OS / SR / SH / UN / SL / SB /TR / PT / LS / SN / RV / PS |
| Movement Authority | STORED | If Level L1 / L2 / L3 and Mode FS / OS / LS |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 6

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.17.3: Management of ERTMS/ETCS system versions |  | 3170300 |
| Test Case of Feature | If system version number X transmitted by a balise is different from 0 and lower than the lowest system version number X supported by the on-board equipment, the on-board equipment shall be able to interpret the balise information, to the extent defined for each type of information |  | 6 |
| Applicable Mode/Level Combinations | L0: SH, UN, PS, SL, NL LNTC: PS, SL, NL, SN L1: FS, LS, OS, SR, SH, PS, SL, SB, TR, PT, NL, RV L2: FS, LS, OS, SR, SH, PS, SL, SB, TR, PT, NL, RV L3: FS, LS, OS, SR, SH, PS, SL, SB, TR, PT, NL, RV |  |  |
| Target of Test | The target of this test case is to check that if: - system version number X transmitted by a balise equals to 1 and the lowest system version number supported by the on-board equipment equals to 2, then - the on-board equipment shall be able to interpret the balise information, to the extent defined for each type of information |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.17.3.5 |  |
|  |  | Subset-026-3.17.3.5 b) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the interfaces that: • The balise information is ACCEPTED Check on the JRU that : • the message is RECORDED • NID_C and NID_BG are RECORDED |
| Constraints | • a cab needs to be active (SB mode) • On-board equipment supports system version number X = 2 only • A telegram with system version number X = 1 is received |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 4 / 5 / 6 / 7 / 8 / 12 / 13 / 14 / 15 | FS / OS / SR / SH / UN / SL / SB / TR / PT / LS / SN / RV / PS |
| Movement Authority | STORED | If Level L1 / L2 / L3 and Mode FS / OS / LS |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (NID_BG=<BGa>) containing packet 72 (X_TEXT=Test OK), packet 41 (M_LEVELTR=Unequal to actual level), packet 255 is received | I | BTM | The train overpasses a BG. In telegram of balise #i, the system version number X equals to 1. The packets included in the BG should be selected in order to be able to test that the on-board is able to interpret the balise information, to the extent defined for each type of information. Packet 72 for testing any mode without PS, SH, SL. Packet 41 for testing any mode of PS, SH, SL. | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | The BG message (NID_C and NID_BG) is recorded. | N/A | N/A | - |
| 3 | N/A | N/A | The Level symbol "Level 0 announcement" is displayed OR The Level symbol "Level 1 announcement" is displayed OR The Level symbol "Level 2 announcement" is displayed OR The Level symbol "Level 3 announcement" is displayed OR The Level symbol "Level NTC announcement" is displayed | O | DMI | Depends on current mode For testing any mode without RV mode. | N/A | N/A | - |
| 4 | N/A | N/A | The "plain text message" is displayed | O | DMI | For any mode without PS/SH/SL mode. | N/A | N/A | - |
| 5 | N/A | N/A | START DISPLAYING FIXED TEXT MESSAGE (NID_MESSAGE_JRU=16) is recorded | O | JRU | For any mode without PS/SH/SL mode. | N/A | N/A | - |
| 6 | N/A | N/A | The "plain text message" is removed | O | DMI | For any mode without PS/SH/SL mode. | N/A | N/A | - |
| 7 | N/A | N/A | STOP DISPLAYING FIXED TEXT MESSAGE (NID_MESSAGE_JRU=17) is recorded | O | JRU | For any mode without PS/SH/SL mode. | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 72 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TEXTCLASS | 2 | FINITE VALUE | - |
| Q_TEXTDISPLAY | 1 | FINITE VALUE | Start/end events relation. |
| D_TEXTDISPLAY | 15 | FINITE VALUE | Start event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | Start event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | Start event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| L_TEXTDISPLAY | 15 | FINITE VALUE | End event. |
| T_TEXTDISPLAY | 10 | FINITE VALUE | End event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | End event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | End event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| Q_TEXTCONFIRM | 2 | FINITE VALUE | - |
| Q_CONFTEXTDISPLAY | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| Q_TEXTREPORT | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| NID_TEXTMESSAGE | 8 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_C | 10 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_RBC | 14 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| L_TEXT | 8 | FINITE VALUE | - |
| X_TEXT(L_TEXT) | 8 | FINITE VALUE | Test OK |
| NID_PACKET | 8 | 41 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | FINITE VALUE | - |
| M_LEVELTR | 3 | FINITE VALUE | Unequal to actual level |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTR = 1 (NTC). |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 4 / 5 / 6 / 7 / 8 / 12 / 13 / 14 / 15 | FS / OS / SR / SH / UN / SL / SB /TR / PT / LS / SN / RV / PS |
| Movement Authority | STORED | If Level L1 / L2 / L3 and Mode FS / OS / LS |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 7

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.17.3: Management of ERTMS/ETCS system versions |  | 3170300 |
| Test Case of Feature | If system version number X transmitted by a balise is different from 0 and lower than the lowest system version number X supported by the on-board equipment, and the on-board is not able to interpret the information, this shall be considered as a message consistency error |  | 7 |
| Applicable Mode/Level Combinations | L0: SH, UN, PS, SL, NL LNTC: PS, SL, NL, SN L1: FS, LS, OS, SR, SH, PS, SL, SB, TR, PT, NL, RV L2: FS, LS, OS, SR, SH, PS, SL, SB, TR, PT, NL, RV L3: FS, LS, OS, SR, SH, PS, SL, SB, TR, PT, NL, RV |  |  |
| Target of Test | The target of this test case is to check that if: - finds a BG marked as unlinked - system version number X transmitted by a balise equals to 1 - the lowest system version number supported by the on-board equipment equals to 2 and - the on-board is not able to interpret the information (NID_PACKET = 0 found, which is compliant with X = 2 but not with X = 1), then - this shall be considered as a message consistency error, i.e. -The telegram is rejected -The service brake is applied until standstill -The service brake is released at standstill |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.17.3.5 |  |
|  |  | Subset-026-3.17.3.5 b) |  |
|  |  | Subset-026-3.16.2.5.1 |  |
|  |  | Subset-026-3.16.2.5.1 c) |  |
|  |  | Subset-026-3.14.1.6 |  |
|  |  | Subset-026-3.16.1.1 |  |
|  |  | Subset-026-3.16.1.1 a) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the interfaces that: • The balise information is REJECTED Check on the TIU that: • The on-board applies the service brake • The service brake is released at standstill Check on the DMI that: • Error indication is DISPLAYED. • The service brake application is displayed • The service brake revocation is removed at standstill Check on the JRU that : • BG message is RECORDED. • Error indication is RECORDED. • NID_C and NID_BG are RECORDED • The application of the service brake is RECORDED • The release of service brake is RECORDED |
| Constraints | • a cab needs to be active (SB mode) • On-board equipment supports system version number X = 2 only • A telegram marked as unlinked with system version number X = 1 and a packet with NID_PACKET = 0 is received |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 4 / 5 / 6 / 7 / 8 / 12 / 13 / 14 / 15 | FS / OS / SR / SH / UN / SL / SB / TR / PT / LS / SN / RV / PS |
| Movement Authority | STORED | If Level L1 / L2 / L3 and Mode FS / OS / LS |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | L2/L3 : Radio link is CONNECTED |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (NID_BG=<BGa>) containing packet 0, 72 (X_TEXT=Test failed), packet 41 (M_LEVELTR=Unequal to actual level) and packet 255 is received | I | BTM | The train overpasses a BG In telegram of balise #i, the system version number X equals to 1 and contains a packet with NID_PACKET = 0, Packet 72 for testing any mode without RV mode. Packet 41 for testing any mode without PS/SH/SL mode. | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | Balise message is rejected. The BG passed should contain a packet that makes possible to test this step | N/A | N/A | - |
| 3 | N/A | N/A | The system status message "Balise read error" is displayed | O | DMI | Error indication is DISPLAYED. | N/A | N/A | - |
| 4 | N/A | N/A | BALISE GROUP ERROR (NID_MESSAGE_JRU=12) is recorded | O | JRU | Error indication is RECORDED. | N/A | N/A | - |
| 5 | N/A | N/A | NOT The "plain text message" is displayed | O | DMI | - | N/A | N/A | - |
| 6 | N/A | N/A | NOT The Level symbol "Level 0 announcement" is displayed OR NOT The Level symbol "Level 1 announcement" is displayed OR NOT The Level symbol "Level 2 announcement" is displayed OR NOT The Level symbol "Level 3 announcement" is displayed OR NOT The Level symbol "Level NTC announcement" is displayed | O | DMI | - | N/A | N/A | - |
| 7 | N/A | N/A | SA-DATA.Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 4 (M_ERROR=2) and packet 0 is transmitted | O | RTM | Only in level 2/3. Use_FT3060100.1 (A linking error has been detected. The on-board equipment sends a radio message 136 “Train Position Report” containing the packet 4 “Error reporting” to the RBC.) | N/A | N/A | - |
| 8 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | Service brake commanded | O | TIU | SB is APPLIED. | N/A | N/A | - |
| 10 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | SB intervention is RECORDED. M_BRAKE_COMMAND_STATE=1 (Command) | N/A | N/A | - |
| 11 | N/A | N/A | The Status symbol "Service Brake or Emergency Brake intervention" is displayed | O | DMI | SB Indication is DISPLAYED. | N/A | N/A | - |
| 12 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 13 | N/A | N/A | Train is at standstill (V_TRAIN=0) | I | INT | - | N/A | N/A | - |
| 14 | N/A | N/A | Service brake not commanded | O | TIU | SB is RELEASED | N/A | N/A | - |
| 15 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=0) is recorded | O | JRU | SB revocation is RECORDED M_BRAKE_COMMAND_STATE=0 (Release) | N/A | N/A | - |
| 16 | N/A | N/A | The Status symbol "Service Brake or Emergency Brake intervention" is removed | O | DMI | SB indication is REMOVED | N/A | N/A | - |
| 17 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=0>) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 0 | Virtual Balise Cover marker Packet with NID_PACKET = 0. This is not consistent with system version 1.1 and shall be considered as a message consistency error |
| NID_VBCMK | 6 | FINITE VALUE | - |
| NID_PACKET | 8 | 72 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TEXTCLASS | 2 | FINITE VALUE | - |
| Q_TEXTDISPLAY | 1 | FINITE VALUE | Start/end events relation. |
| D_TEXTDISPLAY | 15 | FINITE VALUE | Start event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | Start event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | Start event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| L_TEXTDISPLAY | 15 | FINITE VALUE | End event. |
| T_TEXTDISPLAY | 10 | FINITE VALUE | End event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | End event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | End event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| Q_TEXTCONFIRM | 2 | FINITE VALUE | - |
| Q_CONFTEXTDISPLAY | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| Q_TEXTREPORT | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| NID_TEXTMESSAGE | 8 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_C | 10 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_RBC | 14 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| L_TEXT | 8 | FINITE VALUE | - |
| X_TEXT(L_TEXT) | 8 | FINITE VALUE | Test failed |
| NID_PACKET | 8 | 41 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | FINITE VALUE | - |
| M_LEVELTR | 3 | FINITE VALUE | Unequal to actual level |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTR = 1 (NTC). |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 7: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |
| NID_PACKET | 8 | 4 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| M_ERROR | 8 | 2 | error type identifier. |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 4 / 5 / 6 / 7 / 8 / 12 / 13 / 14 / 15 | FS / OS / SR / SH / UN / SL / SB /TR / PT / LS / SN / RV / PS |
| Movement Authority | STORED | If Level L1 / L2 / L3 and Mode FS / OS / LS |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The system status message "Balise read error" is displayed | O | DMI | Error indication is DISPLAYED. |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 8

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.17.3: Management of ERTMS/ETCS system versions |  | 3170300 |
| Test Case of Feature | In all levels, if the system version number X transmitted by any balise is amongst its supported ones, the on-board equipment shall be able to interpret the balise information. |  | 8 |
| Applicable Mode/Level Combinations | L0: SH, UN, PS, SL, NL LNTC: PS, SL, NL, SN L1: FS, LS, OS, SR, SH, PS, SL, SB, TR, PT, NL, RV L2: FS, LS, OS, SR, SH, PS, SL, SB, TR, PT, NL, RV L3: FS, LS, OS, SR, SH, PS, SL, SB, TR, PT, NL, RV |  |  |
| Target of Test | The target of this test case is to check that if: - the on-board unit supports both system version number X 1 and 2 - a telegram with system version number X = 1 is received - the message is accepted - a telegram with system version number X = 2 is received - the message is accepted |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.17.3.5 |  |
|  |  | Subset-026-3.17.3.5 c) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the interfaces that: • The balise information is ACCEPTED for both telegrams Check on the JRU that : • The BG message is RECORDED. • NID_C and NID_BG are RECORDED |
| Constraints | • a cab needs to be active (SB mode) • On-board equipment supports both system version number X = 1 and system version number X = 2 |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 4 / 5 / 6 / 7 / 8 / 12 / 13 / 14 / 15 | FS / OS / SR / SH / UN / SL / SB / TR / PT / LS / SN / RV / PS |
| Movement Authority | STORED | If Level L1 / L2 / L3 and Mode FS / OS / LS |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (NID_BG=<BGa>) containing packet 72 (X_TEXT=Test OK), packet 41 (M_LEVELTR=Unequal to actual level), packet 255 is received | I | BTM | The train overpasses a BG In telegram of balise #i, the system version number X equals to 1 In telegram of balise #(i+1), the system version number X equals to 2 The telegrams passed should contain packets that makes possible to test step 2 Packet 72 for testing any mode without PS, SH, SL. Packet 41 for testing any mode of PS, SH, SL. | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | NID_C and NID_BG are RECORDED. BG message is accepted. In telegram of balise #i, the system version number X equals to 1 | N/A | N/A | - |
| 3 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | The BG message (NID_C and NID_BG) is recorded. In telegram of balise #(i+1), the system version number X equals to 2 | N/A | N/A | - |
| 4 | N/A | N/A | The "plain text message" is displayed | O | DMI | For any mode without PS/SH/SL mode. | N/A | N/A | - |
| 5 | N/A | N/A | START DISPLAYING FIXED TEXT MESSAGE (NID_MESSAGE_JRU=16) is recorded | O | JRU | For any mode without PS/SH/SL mode. | N/A | N/A | - |
| 6 | N/A | N/A | The "plain text message" is removed | O | DMI | For any mode without PS/SH/SL mode. | N/A | N/A | - |
| 7 | N/A | N/A | STOP DISPLAYING FIXED TEXT MESSAGE (NID_MESSAGE_JRU=17) is recorded | O | JRU | For any mode without PS/SH/SL mode. | N/A | N/A | - |
| 8 | N/A | N/A | The Level symbol "Level 0 announcement" is displayed OR The Level symbol "Level 1 announcement" is displayed OR The Level symbol "Level 2 announcement" is displayed OR The Level symbol "Level 3 announcement" is displayed OR The Level symbol "Level NTC announcement" is displayed | O | DMI | Depends on current mode For testing any mode without RV mode. | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | In telegram of balise #i, the system version number X equals to |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 72 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TEXTCLASS | 2 | FINITE VALUE | - |
| Q_TEXTDISPLAY | 1 | FINITE VALUE | Start/end events relation. |
| D_TEXTDISPLAY | 15 | FINITE VALUE | Start event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | Start event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | Start event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| L_TEXTDISPLAY | 15 | FINITE VALUE | End event. |
| T_TEXTDISPLAY | 10 | FINITE VALUE | End event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | End event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | End event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| Q_TEXTCONFIRM | 2 | FINITE VALUE | - |
| Q_CONFTEXTDISPLAY | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| Q_TEXTREPORT | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| NID_TEXTMESSAGE | 8 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_C | 10 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_RBC | 14 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| L_TEXT | 8 | FINITE VALUE | - |
| X_TEXT(L_TEXT) | 8 | FINITE VALUE | Test OK |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | In telegram of balise #(i+1), the system version number X equals to 2 |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 41 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | FINITE VALUE | - |
| M_LEVELTR | 3 | FINITE VALUE | Unequal to actual level |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTR = 1 (NTC). |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 4 / 5 / 6 / 7 / 8 / 12 / 13 / 14 / 15 | FS / OS / SR / SH / UN / SL / SB /TR / PT / LS / SN / RV / PS |
| Movement Authority | STORED | If Level L1 / L2 / L3 and Mode FS / OS / LS |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 9

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.17.3: Management of ERTMS/ETCS system versions |  | 3170300 |
| Test Case of Feature | In levels 0 and NTC, if this system version number X is greater than the highest version number X supported by the on-board equipment, the information from this balise shall be ignored and no reaction shall be applied. |  | 9 |
| Applicable Mode/Level Combinations | L0: SH, UN, PS, SL, SB, TR, NL LNTC: SH, PS, SL, SB, TR, NL, SN |  |  |
| Target of Test | The target of this test case is to check that if: the system version number X transmitted by a balise is greater than the highest version number X supported by the on-board equipment, then the message is ignored and no action is applied |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.17.3.5 |  |
|  |  | Subset-026-3.17.3.5 e) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the interfaces that: • The balise information is IGNORED Check on the JRU that : • the message is RECORDED |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 / 4 / 5 / 6 / 7 / 11 / 13 / 15 | SH / UN / SL / SB / TR / NL / SN / PS |
| ERTMS/ETCS level | 0 / 1 | L0 / LNTC |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (NID_BG=<BGa>) containing packet 72 (X_TEXT=Test failed), packet 41 (M_LEVELTR=Unequal to actual level), packet 255 is received | I | BTM | The train overpasses a BG In telegram of balise #i, the system version number X is greater than the highest version number X supported by the on-board equipment The packets included in the BG should be selected in order to be able to test step 2. Packet 72 for testing any mode without RV mode. Packet 41 for testing any mode without PS/SH/SL mode. | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | The BG message (NID_C and NID_BG) is recorded. BG message is IGNORED. No reaction is applied. Check in all interfaces that no reaction is applied, e.g. messages are displayed, and the brakes are not applied. | N/A | N/A | - |
| 3 | N/A | N/A | No Text message will be displayed, and no level transition will apply | O | DMI | Nothing will happen otherwise the test is failed | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 72 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TEXTCLASS | 2 | FINITE VALUE | - |
| Q_TEXTDISPLAY | 1 | FINITE VALUE | Start/end events relation. |
| D_TEXTDISPLAY | 15 | FINITE VALUE | Start event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | Start event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | Start event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| L_TEXTDISPLAY | 15 | FINITE VALUE | End event. |
| T_TEXTDISPLAY | 10 | FINITE VALUE | End event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | End event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | End event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| Q_TEXTCONFIRM | 2 | FINITE VALUE | - |
| Q_CONFTEXTDISPLAY | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| Q_TEXTREPORT | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| NID_TEXTMESSAGE | 8 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_C | 10 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_RBC | 14 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| L_TEXT | 8 | FINITE VALUE | - |
| X_TEXT(L_TEXT) | 8 | FINITE VALUE | Test failed |
| NID_PACKET | 8 | 41 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | FINITE VALUE | - |
| M_LEVELTR | 3 | FINITE VALUE | Unequal to actual level |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTR = 1 (NTC). |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 / 4 / 5 / 6 / 7 / 11 / 13 / 15 | SH / UN / SL / SB / TR / NL / SN / PS |
| ERTMS/ETCS level | 0 / 1 | L0 / LNTC |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 10

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.17.3: Management of ERTMS/ETCS system versions |  | 3170300 |
| Test Case of Feature | Supervision of ETCS version compatibility for EUROLOOP messages. |  | 10 |
| Applicable Mode/Level Combinations | L1: FS, LS |  |  |
| Target of Test | The target of this test is to check that the on-board equipment does interpret the received Euroloop message whose system version number X is lower than the lowest system version number X supported by the on-board equipment to the extend defined for each type of information of chapter 6. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.17.3.6 |  |
|  |  | Subset-026-3.17.3.6 a) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : • There is no indication showing that there is an ETCS incompatibility between the Euroloop message and the on-board equipment. Check on the JRU that : • The Euroloop message is recorded (because ETCS version incompatibility is not a correctness problem). Check on the packet-dependent interfaces that there are observable packet-dependent reactions since the Euroloop message accepted. |
| Constraints | • The ETCS version of the Euroloop message is lower than the lowest version number supported by the on-board equipment. • Data included in the Euroloop message shall be chosen in such a way that it is easy to verify on at least one interface whether the data is accepted or refused by the on-board equipment. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| Movement Authority | STORED | - |
| EOLM information | STORED | Q_SSCODE=STORED EOLM balise group directionality=NOMINAL Q_DIR=NOMINAL Q_LOOPDIR=SAME (Same as EOLM directionality) List of ETCS compatibility versions=AVAILABLE (A list of ETCS compatibility versions is available on-board) |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Euroloop message containing packet 136, 12, 255 is received | I | LTM | The train passes over an Euroloop and receives a Euroloop message with : M_VERSION lower than the lowest of the ETCS versions stored on-board. | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM EUROLOOP (NID_MESSAGE_JRU=7) is recorded | O | JRU | The received Euroloop message is recorded on the JRU. | N/A | N/A | - |
| 3 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | new MA is shown | N/A | N/A | - |
| 4 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Euroloop Message |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 1 | Defines the type of media: Loop (1) |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_LOOP | 14 | FINITE VALUE | Identity of Euroloop |
| NID_PACKET | 8 | 12 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out". |
| N_ITER | 5 | FINITE VALUE | Set to zero if V_MAIN = 0 or if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| Movement Authority | STORED | Not extended by means of Euroloop transmission |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 11

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.17.3: Management of ERTMS/ETCS system versions |  | 3170300 |
| Test Case of Feature | Supervision of ETCS version compatibility for EUROLOOP messages. |  | 11 |
| Applicable Mode/Level Combinations | L1: FS, LS |  |  |
| Target of Test | The target of this test is to check that the on-board equipment does NOT interpret the received Euroloop message whose system version number X is lower than the lowest system version number X supported by the on-board equipment to the extend defined for each type of information of chapter 6. The Euroloop message is considered as consistency error. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.17.3.6 |  |
|  |  | Subset-026-3.17.3.6 a) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : • There is no indication showing that there is an ETCS incompatibility between the Euroloop message and the on-board equipment. Check on the JRU that : • The Euroloop message is recorded (because ETCS version incompatibility is not a correctness problem). Check on the packet-dependent interfaces that there are observable packet-dependent reactions since the Euroloop message accepted. |
| Constraints | • The ETCS version of the Euroloop message is lower than the lowest version number supported by the on-board equipment. • Data included in the Euroloop message shall be chosen in such a way that it is easy to verify on at least one interface whether the data is accepted or refused by the on-board equipment. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| Movement Authority | STORED | - |
| EOLM information | STORED | Q_SSCODE=STORED EOLM balise group directionality=NOMINAL Q_DIR=NOMINAL Q_LOOPDIR=SAME (Same as EOLM directionality) List of ETCS compatibility versions=AVAILABLE (A list of ETCS compatibility versions is available on-board) |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Euroloop message containing packet 136, 155 and packet 255 is received | I | LTM | The train passes over an Euroloop and receives a Euroloop message with : M_VERSION lower than the lowest of the ETCS versions stored on-board. The Onboard is not able to interpret the information. | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM EUROLOOP (NID_MESSAGE_JRU=7) is recorded | O | JRU | The received Euroloop message is recorded on the JRU. | N/A | N/A | - |
| 3 | N/A | N/A | The Euroloop message is ignored. | O | DMI | No packet-dependent reactions | N/A | N/A | - |

| Step 1: Euroloop Message |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 1 | Defines the type of media: Loop (1) |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_LOOP | 14 | FINITE VALUE | Identity of Euroloop |
| NID_PACKET | 8 | 155 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| L_TEXT | 8 | FINITE VALUE | - |
| X_TEXT(L_TEXT) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| Movement Authority | STORED | Not extended by means of Euroloop transmission |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| No packet-dependent reactions | O | TIU | Because the EUROLOOP message has been ignored |
| No packet-dependent reactions | O | DMI | Because the EUROLOOP message has been ignored |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |