| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 5150400 (5.15.4: Degraded Situations) Total: 2 Test case(s) |  |
| REF: | Subset-076-5-2-5150400 |
| ISSUE: | Version 3.2.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 0.0.1 23.03.2012 | All | Creation | DLR |
| Version 2.9.0 31.03.2013 | All | First version/ review for Baseline 3 | INECO |
| Version 2.9.1 01.08.2013 | All | Second version/Internal modification;TC_Editorv27; Correction of Requirements blank space | INECO |
| Version 2.9.3 02.10.2013 | All | Third version/Review from UNISIG 15.08.2013 included/ FT renumbered | INECO |
| Version 2.9.3 07.01.2014 | All | Fourth version/ Internal version: Cross-reference checked; Old reference removed. | INECO |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.2.0 31.03.2017 | All | Baseline 3 1st maintenance release version - Bug fixing | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.15.4: Degraded Situations |  | 5150400 |
| Test Case of Feature | The train has passed the RBC transition border with its full length. The on-board equipment sends to the handing over RBC a radio message 136 “Train Position Report”. As it receives no answer, it repeats the position report a defined number of times. After unanswered position report, the on-board equipment sends a radio message156 “Termination of a communication session” to the RBC. if no reply is received within the fixed waiting time from the last sending of the position report, the ERTMS/ETCS on-board equipment shall terminate the communication session. Exception: In case a communication session is established and no acknowledgement is received within a fixed waiting time (see Appendix A.3.1) after sending the “Termination of communication session” message, the message shall be repeated with the fixed waiting time after each repetition. After a defined number of repetitions (see Appendix A.3.1), and if no acknowledgment is received within the fixed waiting time from the time of the last sending of “Termination of communication session”, the ERTMS/ETCS onboard equipment shall consider the communication session terminated. |  | 1 |
| Applicable Mode/Level Combinations | L2: FS / LS / OS / SR / TR L3: FS / LS / OS / SR / TR |  |  |
| Target of Test | The target of this test is to check that : The on-board equipment sends to the handing over RBC a radio message 136 “Train Position Report” indicating the exit from the handing over RBC area. No answer is received from the RBC The on-board repeats a defined number of times (3 times each 15s) the sending of a radio message 136 “Train Position Report” indicating the exit from the handing over RBC area when no answer is received from the RBC. The termination of the communication session is initiated by the on-board equipment when no answer to the last position report message is received. After waiting within the fixed waiting time (60s) the on-board sends a radio message 156 “Termination of a communication session” to the Handing over RBC. The on-board repeats a defined number of times (3 times each 15s) the sending of a radio message 156 “Termination of communication session” when no acknowledgment is received from the Handing over RBC. No further message is sent by the on-board once the radio message 156 has been sent. The on-board equipment considers the Radio communication session terminated. The on-board equipment release the safe connection with Handing over RBC. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.5.5.1 |  |
|  |  | Subset-026-3.5.5.1 e) |  |
|  |  | Subset-026-3.5.5.3 |  |
|  |  | Subset-026-3.5.5.3.1 |  |
|  |  | Subset-026-3.5.5.3.2 |  |
|  |  | Subset-026-5.15.4.3 |  |
|  |  | Subset-026-5.15.4.4 |  |
|  |  | Subset-026-A.3.1 - Repetition of radio messages (i.e. excluding the first sending) |  |
|  |  | Subset-026-A.3.1 - Waiting time before radio message repetition |  |
|  |  | Subset-026-A.3.1 - Additional delay time to disconnection on supervision of safe radio connection |  |
|  |  | Subset-026-8.6.4 Message 136: Train position report |  |
|  |  | Subset-026-8.6.14 Message 156: Termination of a communication session |  |
|  |  | Subset-026-7.4.3.1 Packet Number 0: Position Report |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that: The four radio messages 136 (Train Position Report) sent by the on-board are recorded. The four radio message 156 (Termination of a communication session) sent by the on-board are recorded. No more message transmitted by the on-board equipment after message 156 is recorded. |
| Constraints | A communication session with the Handing Over RBC shall exist. Both cases can be given: One communication session or two communication sessions can be handle by the on-board equipment. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 7 / 12 | FS / OS / SR / TR / LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted. | O | RTM | To Handing Over RBC | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted. | O | RTM | No answer is received from the handing over RBC and a time out occurs (15s) The first repetition of position report message indicating the exit from the handing over RBC area is sent to the handing over RBC. | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded. | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted. | O | RTM | No answer is received from the handing over RBC and a time out occurs (15s): The second repetition of position report message indicating the exit from the handing over RBC area is sent to the handing over RBC. | N/A | N/A | - |
| 6 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded. | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted. | O | RTM | No answer is received from the handing over RBC and a time out occurs (15s): The third and last repetition of position report message indicating the exit from the handing over RBC area is sent to the handing over RBC. | N/A | N/A | - |
| 8 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded. | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | SA-DATA. Request with Euroradio Message "Termination of a communication session" (NID_MESSAGE=156) is transmitted | O | RTM | No answer is received from the handing over RBC and the time out occurs (60s): Should be checked that the on-board does not send any more messages after this one with the Exception: The message shall be repeated if no ack. is received from RBC. | N/A | N/A | - |
| 10 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 11 | N/A | N/A | NOT SA-DATA.Indication with Euroradio Message "Acknowledgement of termination of a communication session" (NID_MESSAGE=39) is received | I | RTM | No message is received; Within 15sec NO Message 39 (Acknowledgement of termination of a communication session) is received | N/A | N/A | - |
| 12 | N/A | N/A | SA-DATA. Request with Euroradio Message "Termination of a communication session" (NID_MESSAGE=156) is transmitted | O | RTM | First repetition | N/A | N/A | - |
| 13 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 14 | N/A | N/A | NOT SA-DATA.Indication with Euroradio Message "Acknowledgement of termination of a communication session" (NID_MESSAGE=39) is received | I | RTM | No message is received; Within 15sec NO Message 39 (Acknowledgement of termination of a communication session) is received | N/A | N/A | - |
| 15 | N/A | N/A | SA-DATA. Request with Euroradio Message "Termination of a communication session" (NID_MESSAGE=156) is transmitted | O | RTM | Second repetition | N/A | N/A | - |
| 16 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 17 | N/A | N/A | NOT SA-DATA.Indication with Euroradio Message "Acknowledgement of termination of a communication session" (NID_MESSAGE=39) is received | I | RTM | No message is received; Within 15sec NO Message 39 (Acknowledgement of termination of a communication session) is received | N/A | N/A | - |
| 18 | N/A | N/A | SA-DATA. Request with Euroradio Message "Termination of a communication session" (NID_MESSAGE=156) is transmitted | O | RTM | Third repetition The on-board considers the radio communication session as terminated. | N/A | N/A | - |
| 19 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 20 | N/A | N/A | NOT SA-DATA.Indication with Euroradio Message "Acknowledgement of termination of a communication session" (NID_MESSAGE=39) is received | I | RTM | No message is received; Within 15sec NO Message 39 (Acknowledgement of termination of a communication session) is received | N/A | N/A | - |
| 21 | N/A | N/A | SA-DISCONNECT. Request is transmitted | O | RTM | No answer is received from the Handing Over RBC and the time out occurs (60s). The on-board release the safe connection. | N/A | N/A | - |

| Step 1: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | Location data indicates that train has left the handing over RBC area. |
| Q_DIRLRBG | 2 | FINITE VALUE | Location data indicates that train has left the handing over RBC area. |
| Q_DLRBG | 2 | FINITE VALUE | Location data indicates that train has left the handing over RBC area. |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| Step 3: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | Location data indicates that train has left the handing over RBC area. |
| Q_DIRLRBG | 2 | FINITE VALUE | Location data indicates that train has left the handing over RBC area. |
| Q_DLRBG | 2 | FINITE VALUE | Location data indicates that train has left the handing over RBC area. |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| Step 5: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | Location data indicates that train has left the handing over RBC area. |
| Q_DIRLRBG | 2 | FINITE VALUE | Location data indicates that train has left the handing over RBC area. |
| Q_DLRBG | 2 | FINITE VALUE | Location data indicates that train has left the handing over RBC area. |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| Step 7: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | Location data indicates that train has left the handing over RBC area. |
| Q_DIRLRBG | 2 | FINITE VALUE | Location data indicates that train has left the handing over RBC area. |
| Q_DLRBG | 2 | FINITE VALUE | Location data indicates that train has left the handing over RBC area. |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| Step 9: Radio Message 156 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 156 | Termination of a communication session (with Handing over RBC) |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 1 | FINITE VALUE | - |

| Step 12: Radio Message 156 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 156 | Termination of a communication session (with Handing over RBC) |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 1 | FINITE VALUE | - |

| Step 15: Radio Message 156 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 156 | Termination of a communication session (with Handing over RBC) |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 1 | FINITE VALUE | - |

| Step 18: Radio Message 156 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 156 | Termination of a communication session (with Handing over RBC) |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 1 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 7 / 12 | FS / OS / SR / TR / LS |
| Radio communication session | NOT ESTABLISHED | With Handing Over RBC |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.15.4: Degraded Situations |  | 5150400 |
| Test Case of Feature | The on-board receives an announcement of the RBC transition from the Handing Over RBC. It sends a position report to the Handing Over RBC when the maximum safe front end of the train passes the location of the RBC transition. At the transition border, it passes a balise group containing an order to execute the handover immediately. The on-board equipment sends a position report to the Handing Over RBC when the minimum safe rear end of the train passes the location of the RBC transition. It does not receive an order to terminate the communication session with the Handing Over RBC, although it sends 3 times its position report. The on-board terminates the session. After unanswered position report, the on-board equipment sends a radio message156 “Termination of a communication session” to the RBC. If no reply is received within the fixed waiting time from the last sending of the position report, the ERTMS/ETCS on-board equipment shall terminate the communication session. As soon as the communication session with the Handing Over RBC is terminated, it establishes a communication session with the Accepting RBC and it transmits its train data. |  | 2 |
| Applicable Mode/Level Combinations | L2: FS / LS / OS / SR / TR L3: FS / LS / OS / SR / TR |  |  |
| Target of Test | The target of this test is to check that : When the minimum safe rear end of the train passes the location of the RBC transition, the on-board equipment sends a position report to the Handing Over RBC and repeats the sending as no order to terminate the communication session is received. The on-board equipment takes the initiative to terminate the communication session with the Handing Over RBC. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-5.15.4.3 |  |
|  |  | Subset-026-5.15.4.4 |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |
|  |  | Subset-026-7.4.2.27 Packet Number 131: RBC transition order |  |
|  |  | Subset-026-8.6.4 Message 136: Train position report |  |
|  |  | Subset-026-7.4.3.1 Packet Number 0: Position Report |  |
|  |  | Subset-026-8.6.13 Message 155: Initiation of a communication session |  |
|  |  | Subset-026-8.7.12 Message 32: RBC/RIU System Version |  |
|  |  | Subset-026-8.6.17 Message 159: Session established |  |
|  |  | Subset-026-8.6.1 Message 129: Validated Train Data |  |
|  |  | Subset-026-8.7.4 Message 8: Acknowledgement of Train Data |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on JRU that: The radio message 24 with packets 131 received from Handing Over RBC is recorded. The radio message 136 sent by the on-board equipment to the Handing Over RBC when the maximum safe front end of the train passes the location of the RBC transition is recorded. The balise message containing the order to execute immediately the handover (packet 131) is recorded. The four radio messages 136 sent by the on-board equipment to the Handing Over RBC when the minimum safe rear end of the train passes the location of the RBC transition are recorded. The four radio message 156 (Termination of a communication session) sent by the on-board are recorded. The radio message 155 sent to the Accepting RBC is recorded. The radio message 32 received from Accepting RBC is recorded The radio message 159 sent to the Accepting RBC is recorded. The radio message 129 sent to the Accepting RBC is recorded. The radio message 8 received from the Accepting RBC is recorded |
| Constraints | The on-board equipment can handle only one communication session at the time. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 7 / 12 | FS / OS / SR / TR / LS |
| Radio communication session | ESTABLISHED | The communication session is established with the Handing Over RBC |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA. Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 131 (D_RBCTR ≠ 0; Q_SLEEPSESSION=0) is received | I | RTM | Message is received from the Handing Over RBC. | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted | O | RTM | The maximum safe front end of the train passes the location of the RBC transition. Train maximum safe front end location = LRBG + D_RBCTR. | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | One Balise Group message (N_TOTAL=1; NID_C=<comment>) containing packet 131 (D_RBCTR=0) is received | I | BTM | The balise group is placed at the RBC transition border. | N/A | N/A | - |
| 6 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | Use_FT5150400.1 (termination of communication session to handing over RBC due to no request for termination is received from handing over RBC after unsuccessful repetitions) | - | - | The train has passed the RBC transition border with its full length The on-board equipment receives no order to terminate the communication session from the RBC. The on-board makes 3 attempts to get the termination order for the communication session, by sending position report. As these attempts are failing, the on-board terminates the radio communication session. After unanswered position report, the on-board equipment sends a radio message156. As no reply is received , the on-board repeats 3 times the sending of a radio message 156 when no acknowledgment is received from the Handing over RBC. The communication session with the Handing Over RBC is terminated and the safe connection is released. | N/A | N/A | - |
| 8 | N/A | N/A | SA-CONNECT. Request is transmitted | O | RTM | - | N/A | N/A | - |
| 9 | N/A | N/A | SA-CONNECT. Confirm is received | I | RTM | A safe connection is established with the Accepting RBC | N/A | N/A | - |
| 10 | N/A | N/A | SA-DATA. Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted | O | RTM | - | N/A | N/A | - |
| 11 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 12 | N/A | N/A | SA-DATA. Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32) is received | I | RTM | - | N/A | N/A | - |
| 13 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 14 | N/A | N/A | SA-DATA. Request with Euroradio Message "Session Established" (NID_MESSAGE=159) is transmitted | O | RTM | The communication session with the Accepting RBC is established | N/A | N/A | - |
| 15 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 16 | N/A | N/A | SA-DATA. Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) containing packet 11 is transmitted | O | RTM | - | N/A | N/A | - |
| 17 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 18 | N/A | N/A | SA-DATA. Indication with Euroradio Message "Acknowledgement of Train Data" (NID_MESSAGE=8) is received | I | RTM | - | N/A | N/A | - |
| 19 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General Message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 131 | RBC transition order |
| Q_DIR | 2 | FINITE VALUE | Valid for current train direction |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_RBCTR | 15 | FINITE VALUE | ≠ 0 |
| NID_C | 10 | FINITE VALUE | Valid for the train. |
| NID_RBC | 14 | FINITE VALUE | Accepting RBC identity |
| NID_RADIO | 64 | FINITE VALUE | Accepting RBC radio subscriber number. |
| Q_SLEEPSESSION | 1 | 0 | Sleeping equipment don’t consider session management information (to avoid TIU influence) |

| Step 3: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | D_RBCTR -L_DOUBTUNDER | Max safe front end is considered |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | 0 / 1 / 2 / 5 / 7 / 11 | FS / OS / SR / TR / PT / NL |
| M_LEVEL | 3 | 3 / 4 | - |

| Step 5: Eurobalise Telegram (balise 1/N – 1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | N – 1 | N balises ( N >= 2) |
| M_DUP | 2 | 0 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | BG1 | Identity of the balise group |
| Q_LINK | 1 | 1 | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 131 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_RBCTR | 15 | 0 | - |
| NID_C | 10 | FINITE VALUE | Accepting RBC identity. |
| NID_RBC | 14 | FINITE VALUE | - |
| NID_RADIO | 64 | FINITE VALUE | Accepting. RBC radio subscriber number. |
| Q_SLEEPSESSION | 1 | 0 | Ignore session establishment order (to avoid TIU influence) |
| NID_PACKET | 8 | 255 | End of information |

| Step 10: Radio Message 155 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 155 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |

| Step 12: Radio Message 32 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 32 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |

| Step 14: Radio Message 159 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 159 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |

| Step 16: Radio Message 129 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 129 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | 0 / 1 / 2 / 5 / 7 / 11 | FS / OS / SR / TR / PT / NL |
| M_LEVEL | 3 | 3 / 4 | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |
| NID_PACKET | 8 | 11 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| NC_CDTRAIN | 4 | FINITE VALUE | - |
| NC_TRAIN | 15 | FINITE VALUE | - |
| L_TRAIN | 12 | FINITE VALUE | - |
| V_MAXTRAIN | 7 | FINITE VALUE | - |
| M_LOADINGGAUGE | 8 | FINITE VALUE | - |
| M_AXLELOADCAT | 7 | FINITE VALUE | - |
| M_AIRTIGHT | 2 | FINITE VALUE | - |
| N_AXLE | 10 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_VOLTAGE(k) | 4 | FINITE VALUE | Identity of the traction system. |
| NID_CTRACTION(k) | 10 | FINITE VALUE | NID_CTRACTION(k) given only if M_VOLTAGE(k) ≠ 0. |
| N_ITER | 5 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | Type of National System available. |

| Step 18: Radio Message 8 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 8 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 7 / 12 | FS / OS / SR / TR / LS |
| Radio communication session | ESTABLISHED | The communication session is established with the Accepting RBC |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Only with the Accepting RBC |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |