| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 4080500 (4.8.5: Handling of transition buffer in case of level transition announcement or RBC/RBC handover) Total: 11 Test case(s) |  |
| REF: | Subset-076-5-2-4080500 |
| ISSUE: | Version 3.2.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 0.0.1 23.03.2012 | All | Creation | DLR |
| Version 2.9.0 05.10.2012 | All | First version/review for Baseline 3 | INECO |
| Version 2.9.1 24.09.2013 | All | Second version/ UNISIG Review from 13.09.2013 included; TCEditor_v28 tool used; All steps description changed with Dictionary steps description. | INECO |
| Version 2.9.2 08.12.2013 | All | Third version/ UNISIG Review from 6.12.2013 included. | INECO |
| Version 2.9.3 16.12.2013 | All | Fourth version/ Requirements related with chapter 7 and 8 added. | INECO |
| Version 2.9.4 02.01.2014 | All | Fifth version/ Internal version: Cross-reference checked; Old reference removed. | INECO |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.2.0 31.03.2017 | All | Baseline 3 1st maintenance release version - Bug fixing | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.5: Handling of transition buffer in case of level transition announcement or RBC/RBC handover |  | 4080500 |
| Test Case of Feature | Handling of transition buffer in case of level transition announcement. In case three sets of information are already stored in the transition buffer, any new set to be stored shall replace the oldest one currently stored. |  | 1 |
| Applicable Mode/Level Combinations | L0: UN LNTC: SN L2 / L3: FS / LS / OS |  |  |
| Target of Test | The target of this test is to check that: When the on board equipment has received a level transition order at further location, the transition buffer is able to store three sets of information. The fourth set of information is stored replacing the oldest one. At the same time the level transition is performed, the sets of information stored in the transition buffer shall be released and re-evaluated in the sequence they have been received. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.5.1 |  |
|  |  | Subset-026-4.8.5.3 |  |
|  |  | Subset-026-4.8.5.6 |  |
|  |  | Subset-026-4.8.5.6 a) |  |
|  |  | Subset-026-4.8.5.6 b) |  |
|  |  | Subset-026-4.8.5.6 c) |  |
|  |  | Subset-026-7.4.2.3 Packet Number 12: Level 1 Movement Authority |  |
|  |  | Subset-026-7.4.2.26 Packet Number 80: Mode Profile |  |
|  |  | Subset-026-7.4.2.6 Packet Number 21: Gradient Profile |  |
|  |  | Subset-026-7.4.2.7 Packet Number 27: International Static Speed Profile |  |
|  |  | Subset-026-7.4.2.9 Packet Number 41: Level Transition Order |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: Train Level changes to L1 at the transition border. Train Mode changes to FS. (Packets 12, 21 and 27 were stored in the buffer) Train Mode doesn’t change to OS or LS. (Packet 80 was replaced). Train Speed is equal or lower than V_STATIC. (Packet 27 Information is being used). Check on the JRU that : Balise Group messages, including packets 80, 12, 21 and 27, are RECORDED. Balise Group message, including the L1 Transition Order (Packet 41 with M_LEVELTR=2 and D_LEVELTR = 0) is RECORDED. The new level is RECORDED. The mode change to FS is RECORDED. |
| Constraints | Packet 41 with an announcement of L1 transition at further location has been received and is stored on-board. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 4 / 12 / 13 | FS / OS / UN / LS / SN |
| Radio communication session | ESTABLISHED | Only in L2 / L3 |
| Level Transition Order | STORED | M_LEVELTR= 2 Transition to Level 1 D_LEVELTR =FINITE VALUE > 0 and < 32767 (further location) |
| ERTMS/ETCS level | 0 / 1 / 3 / 4 | L0 / LNTC / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Only in L2 / L3 |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level <L1> announcement" is displayed. | O | DMI | DMI Object: Symbol LE10 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / LNTC / L2 / L3 | UN / SN / FS / LS / OS | One Balise Group message (N_TOTAL=1) containing packet 80 and packet 12 is received | I | BTM | The on-board reads (one or more) balise in a BGa, including this sequence of packets: Packet 12: Movement Authority for L1 Packet 80: Mode Profile | L0 / LNTC / L2 / L3 | UN / SN / FS / LS / OS | - |
| 2 | L0 / LNTC / L2 / L3 | UN / SN / FS / LS / OS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L0 / LNTC / L2 / L3 | UN / SN / FS / LS / OS | - |
| 3 | L0 / LNTC / L2 / L3 | UN / SN / FS / LS / OS | One Balise Group message (N_TOTAL=1) containing packet 21 is received | I | BTM | On-board reads BGb with Packet 21: Gradient Profile | L0 / LNTC / L2 / L3 | UN / SN / FS / LS / OS | - |
| 4 | L0 / LNTC / L2 / L3 | UN / SN / FS / LS / OS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L0 / LNTC / L2 / L3 | UN / SN / FS / LS / OS | - |
| 5 | L0 / LNTC / L2 / L3 | UN / SN / FS / LS / OS | One Balise Group message (N_TOTAL=1) containing packet 27 is received | I | BTM | On-board reads BGc with: Packet 27: International Static Speed Profile | L0 / LNTC / L2 / L3 | UN / SN / FS / LS / OS | - |
| 6 | L0 / LNTC / L2 / L3 | UN / SN / FS / LS / OS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L0 / LNTC / L2 / L3 | UN / SN / FS / LS / OS | - |
| 7 | L0 / LNTC / L2 / L3 | UN / SN / FS / LS / OS | One Balise Group message (N_TOTAL=1) containing packet 12 is received | I | BTM | On-board reads BGd with: Packet 12: Movement Authority for L1. | L0 / LNTC / L2 / L3 | UN / SN / FS / LS / OS | - |
| 8 | L0 / LNTC / L2 / L3 | UN / SN / FS / LS / OS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L0 / LNTC / L2 / L3 | UN / SN / FS / LS / OS | - |
| 9 | L0 / LNTC / L2 / L3 | UN / SN / FS / LS / OS | One Balise Group message (N_TOTAL=1) containing packet 41 is received | I | BTM | At the level border the order to switch to ETCS level 1 immediately is RECEIVED. M_LEVELTR = 2 | L0 / LNTC / L2 / L3 | UN / SN / FS / LS / OS | - |
| 10 | L0 / LNTC / L2 / L3 | UN / SN / FS / LS / OS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L0 / LNTC / L2 / L3 | UN / SN / FS / LS / OS | - |
| 11 | L0 / LNTC / L2 / L4 | UN / SN / FS / LS / OS | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=2; M_MODE=0) is recorded | O | JRU | TRIGGERING EVENT: When the level changes; When the Mode changes | L1 | FS | - |
| 12 | L1 | FS | The Level symbol "Level <L1>" is displayed | O | DMI | For level change symbol LE03 is displayed. Exception: If the previous level is LNTC, symbol LE11 is displayed and driver acknowledgement is requested. | L1 | FS | - |
| 13 | L1 | FS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit03=1>) is recorded | O | JRU | - | L1 | FS | - |
| 14 | L1 | FS | The Mode symbol "Full Supervision" is displayed | O | DMI | The train does not change to OS or LS mode. Packets 80+12 from balise BGa are replaced by packet 12 from BGd. The fourth set of information is stored replacing the oldest one currently stored. For mode change symbol MO11 is displayed. | L1 | FS | - |
| 15 | L1 | FS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit26=1>) is recorded | O | JRU | - | L1 | FS | - |
| 16 | L1 | FS | The speed and distance monitoring information is displayed | O | DMI | The permitted speed is DISPLAYED. The target speed is DISPLAYED. The target distance is DISPLAYED | L1 | FS | - |
| 17 | L1 | FS | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | - | L1 | FS | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 010 0000 | Version 2.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | 1 of N |
| N_TOTAL | 3 | FINITE VALUE | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 80 | Mode profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_MAMODE | 15 | 0 | Mode change starting in the current location |
| M_MAMODE | 2 | FINITE VALUE | For OS = 00 For LS = 10 |
| V_MAMODE | 7 | FINITE VALUE | - |
| L_MAMODE | 15 | FINITE VALUE | - |
| L_ACKMAMODE | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| NID_PACKET | 8 | 12 | Level 1 Movement Authority |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out". |
| N_ITER | 5 | FINITE VALUE | Set to zero if V_MAIN = 0 or if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 010 0000 | Version 2.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 001 | 2 of N |
| N_TOTAL | 3 | FINITE VALUE | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 3: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | - |
| M_VERSION | 7 | 010 0000 | Version 2.0 |
| Q_MEDIA | 1 | 0 | - |
| N_PIG | 3 | FINITE VALUE | - |
| N_TOTAL | 3 | FINITE VALUE | - |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGb |
| Q_LINK | 1 | 1 | - |
| NID_PACKET | 8 | 21 | Gradient profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 5: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | - |
| M_VERSION | 7 | 010 0000 | Version 2.0 |
| Q_MEDIA | 1 | 0 | - |
| N_PIG | 3 | FINITE VALUE | - |
| N_TOTAL | 3 | FINITE VALUE | - |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGc |
| Q_LINK | 1 | 1 | - |
| NID_PACKET | 8 | 27 | International Static Speed Profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 7: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | - |
| M_VERSION | 7 | 010 0000 | Version 2.0 |
| Q_MEDIA | 1 | 0 | - |
| N_PIG | 3 | FINITE VALUE | - |
| N_TOTAL | 3 | FINITE VALUE | - |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINK | 1 | 1 | - |
| NID_PACKET | 8 | 12 | MA for L1 |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out". |
| N_ITER | 5 | FINITE VALUE | Set to zero if V_MAIN = 0 or if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 9: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | - |
| M_VERSION | 7 | 010 0000 | Version 2.0 |
| Q_MEDIA | 1 | 0 | - |
| N_PIG | 3 | FINITE VALUE | - |
| N_TOTAL | 3 | FINITE VALUE | - |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINK | 1 | 1 | - |
| NID_PACKET | 8 | 41 | Level transition order (D_LEVELTR = special value = now, M_LEVELTR = 2) |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | 32767 | Now |
| M_LEVELTR | 3 | 2 | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTR = 1 (NTC). |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 | FS |
| Radio communication session | ESTABLISHED | Only in L2 / L3 |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Only in L2 / L3 |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level 1" is displayed. The Mode symbol "Full Supervision" is displayed. The speed and distance monitoring information is displayed | O | DMI | DMI Objects: Symbol LE03 Symbol MO11 The permitted speed, target speed and target distance are displayed. |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.5: Handling of transition buffer in case of level transition announcement or RBC/RBC handover |  | 4080500 |
| Test Case of Feature | Handling of transition buffer in case of level transition announcement. At the same time the level transition is performed the sets of information stored in the transition buffer shall be released and re-evaluated in the sequence they have been received. Re-evaluating Conditional Emergency Stop information. |  | 2 |
| Applicable Mode/Level Combinations | L0: UN LNTC: SN L1: FS/ LS/ OS |  |  |
| Target of Test | The target of this test is to check that: When the on board equipment has received a level transition order for a further location, the transition buffer is able to store three sets of information. At the same time the level transition is performed, the sets of information stored in the transition buffer shall be released and re-evaluated in the sequence they have been received. Only afterwards, the information will be used by the on-board equipment. Accepting re-evaluated Conditional Emergency Stop information according to table 4.8.3 implies that the accepted Conditional Emergency Stop information may be accepted or rejected in a further step (see clause 3.10.2.2) depending on the given stop location. New movement authority received after any accepted emergency stop message and before the emergency message has been revoked, shall be rejected. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.10.2.4 |  |
|  |  | Subset-026-3.10.2.2 |  |
|  |  | Subset-026-3.10.2.2 b) |  |
|  |  | Subset-026-4.8.5.1 |  |
|  |  | Subset-026-4.8.5.5 |  |
|  |  | Subset-026-4.8.5.6 |  |
|  |  | Subset-026-4.8.5.6 a) |  |
|  |  | Subset-026-4.8.5.6 b) |  |
|  |  | Subset-026-4.8.5.6 c) |  |
|  |  | Subset-026-4.8.5.7 |  |
|  |  | Subset-026-7.4.2.4 Packet Number 15: Level 2/3 Movement Authority |  |
|  |  | Subset-026-7.4.2.6 Packet Number 21: Gradient Profile |  |
|  |  | Subset-026-7.4.2.7 Packet Number 27: International Static Speed Profile |  |
|  |  | Subset-026-7.4.2.9 Packet Number 41: Level Transition Order |  |
|  |  | Subset-026-8.7.2 Message 3: Movement Authority |  |
|  |  | Subset-026-8.7.6 Message 15: Conditional Emergency Stop |  |
|  |  | Subset-026-8.6.8 Message 147: Acknowledgement of Emergency Stop |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: Train Level changes to L2 / L3 at the transition border. Train Mode changes to FS (Packets 15, 21 and 27 were stored in the buffer), if the previous mode was not FS The TS and TD are displayed according to the stop location defined in the previously received Message 15. Check on the JRU that : Radio messages, including packets 15, 21 and 27, are RECORDED. Radio message 15 is RECORDED. The new level is RECORDED. |
| Constraints | Packet 41 with an announcement of L2 / L3 transition at further location has been received and it is stored on-board. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 4 / 12 / 13 | FS / OS / UN / LS / SN |
| Radio communication session | ESTABLISHED | - |
| Level Transition Order | STORED | M_LEVELTR=3/4 Transition to Level 2/3 D_LEVELTR=FINITE VALUE > 0 and < 32767 (further location) |
| ERTMS/ETCS level | 0 / 1 / 2 | L0 / LNTC / L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level <L2 / L3> announcement" is displayed. | O | DMI | DMI Object: Symbol LE12 / LE14 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / LNTC / L1 | FS/ LS/ OS/ UN/ SN | SA-DATA. Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 15, packet 21, packet 27 is received | I | RTM | The train receives Message 3: Movement Authority including this sequence of packets: Packet 15: Level 2/3 Movement Authority Packet 21: Gradient Profile Packet 27: International Static Speed Profile | L0 / LNTC / L1 | FS/ LS/ OS/ UN/ SN | - |
| 2 | L0 / LNTC / L1 | FS/ LS/ OS/ UN/ SN | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=3) is recorded | O | JRU | - | L0 / LNTC / L1 | FS/ LS/ OS/ UN/ SN | - |
| 3 | L0 / LNTC / L1 | FS/ LS/ OS/ UN/ SN | SA-DATA. Indication with Euroradio Message "Conditional Emergency Stop" (NID_MESSAGE=15) is received | I | RTM | The location of the emergency stop location will be placed between the EoA of the previously received Level 2 MA and the EoA of the following MA. | L0 / LNTC / L1 | FS/ LS/ OS/ UN/ SN | - |
| 4 | L0 / LNTC / L1 | FS/ LS/ OS/ UN/ SN | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=15) is recorded | O | JRU | - | L0 / LNTC / L1 | FS/ LS/ OS/ UN/ SN | - |
| 5 | L0 / LNTC / L1 | FS/ LS/ OS/ UN/ SN | SA-DATA. Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 15, packet 21, packet 27 is received | I | RTM | The train receives Message 3: Movement Authority, which has an EoA distance which is shorter than the distance to the Stop location defined in the previous Message 15 (and not yet used).This MA will not be used when re-evaluating the data stored on-board at the level transition change to L2. | L0 / LNTC / L1 | FS/ LS/ OS/ UN/ SN | - |
| 6 | L0 / LNTC / L1 | FS/ LS/ OS/ UN/ SN | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=3) is recorded | O | JRU | - | L0 / LNTC / L1 | FS/ LS/ OS/ UN/ SN | - |
| 7 | L0 / LNTC / L1 | FS/ LS/ OS/ UN/ SN | One Balise Group message (N_TOTAL=1) containing packet 41 is received | I | BTM | At the level border the order to switch to ETCS level 2 / 3 immediately is RECEIVED. M_LEVELTR = 3 / 4 | L0 / LNTC / L1 | FS/ LS/ OS/ UN/ SN | - |
| 8 | L0 / LNTC / L1 | FS/ LS/ OS/ UN/ SN | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L0 / LNTC / L1 | FS/ LS/ OS/ UN/ SN | - |
| 9 | L0 / LNTC / L2 / L4 | FS/ LS/ OS/ UN/ SN | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=3 / 4; M_MODE=0) is recorded | O | JRU | TRIGGERING EVENT: When the level changes; When the Mode changes | L2 / L3 | FS | - |
| 10 | L1 | FS | The Level symbol "Level <L2 /L3>" is displayed | O | DMI | For level change symbol LE04 /LE05 is displayed. Exception: If the previous level is LNTC, symbol LE13 /LE15 is displayed and driver acknowledgement is requested. | L2 / L3 | FS | - |
| 11 | L1 | FS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit04/Bit05=1>) is recorded | O | JRU | - | L2 / L3 | FS | - |
| 12 | L1 | FS | The Mode symbol "Full Supervision" is displayed | O | DMI | If FS was the previous mode, it will not change when the level transition is executed. For mode change symbol MO11 is displayed. | L2 / L3 | FS | - |
| 13 | L1 | FS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit26=1>) is recorded | O | JRU | - | L2 / L3 | FS | - |
| 14 | L2 / L3 | FS | The speed and distance monitoring information is displayed | O | DMI | The Target Speed and TD are displayed according to the stop location defined in the previously received Message 15. | L2 / L3 | FS | - |
| 15 | L2 / L3 | FS | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | - | L2 / L3 | FS | - |
| 16 | L2 / L3 | FS | The system status message "Emergency stop" is displayed | O | DMI | - | L2 / L3 | FS | - |
| 17 | L2 / L3 | FS | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23) is recorded | O | JRU | - | L2 / L3 | FS | - |
| 18 | L2 / L3 | FS | SA-DATA. Request with Euroradio Message "Acknowledgement of Emergency Stop" (NID_MESSAGE=147) is transmitted | O | RTM | With Q_EMERGENCYSTOP = 00b Indicating that the train takes into account the Conditional Emergency Stop | L2 / L3 | FS | - |
| 19 | L2 / L3 | FS | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | L2 / L3 | FS | - |

| Step 1: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | Movement Authority |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | Level 2/3 Movement Authority |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | Packet 21: Gradient Profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | Packet 27: Static Speed Profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |

| Step 3: Radio Message 15 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 15 | Conditional Emergency Stop |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | 0/1 | 0: No acknowledgement required 1: Acknowledgement required |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_EM | 4 | FINITE VALUE | Emergency Stop Identification Number |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_REF | 16 | FINITE VALUE | - |
| Q_DIR | 2 | FINITE VALUE | - |
| D_EMERGENCYSTOP | 15 | FINITE VALUE | Distance between the LRBG and the position reference to the emergency stop |

| Step 5: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | Movement Authority |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | Level 2/3 Movement Authority |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | Packet 21: Gradient Profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | Packet 27: Static Speed Profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |

| Step 7: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | - |
| M_VERSION | 7 | 010 0000 | Version 2.0 |
| Q_MEDIA | 1 | 0 | - |
| N_PIG | 3 | FINITE VALUE | - |
| N_TOTAL | 3 | FINITE VALUE | - |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINK | 1 | 1 | - |
| NID_PACKET | 8 | 41 | Level transition order (D_LEVELTR = special value = now, M_LEVELTR = 3 / 4) |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | 32767 | Now |
| M_LEVELTR | 3 | 3 / 4 | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTR = 1 (NTC). |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 18: Radio Message 147 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 147 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_EM | 4 | FINITE VALUE | Identification Number of the acknowledged Emergency Stop Message |
| Q_EMERGENCYSTOP | 2 | 00 | Conditional Emergency Stop accepted, with update of EoA (ref. to 3.10.2.2) |
| Packet 0 or 1 | - | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 | FS |
| Radio communication session | ESTABLISHED | - |
| Conditional Emergency Stop | STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level 2/3" is displayed. The Mode symbol "Full Supervision" is displayed. Speed and distance monitoring information is displayed. Emergency stop message is DISPLAYED | O | DMI | DMI Objects: LE13 /LE15 MO11 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.5: Handling of transition buffer in case of level transition announcement or RBC/RBC handover |  | 4080500 |
| Test Case of Feature | Handling of transition buffer in case of level transition announcement. At the same time the level transition is performed the sets of information stored in the transition buffer shall be released and re-evaluated in the sequence they have been received. Re-evaluating Conditional Emergency Stop information. |  | 3 |
| Applicable Mode/Level Combinations | L0: UN LNTC: SN L1: FS/ LS/ OS |  |  |
| Target of Test | The target of this test is to check that: When the on board equipment has received a level transition order for a further location, the transition buffer is able to store three sets of information. At the same time the level transition is performed, the sets of information stored in the transition buffer shall be released and re-evaluated in the sequence they have been received. Only afterwards, the information will be used by the on-board equipment. Accepting re-evaluated Conditional Emergency Stop information according to table 4.8.3 implies that the accepted Conditional Emergency Stop information may be accepted or rejected in a further step (see clause 3.10.2.2) depending on the given stop location. New movement authority received after any accepted emergency stop message and before the emergency message has been revoked, shall be rejected. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.5.1 |  |
|  |  | Subset-026-4.8.5.5 |  |
|  |  | Subset-026-4.8.5.6 |  |
|  |  | Subset-026-4.8.5.6 a) |  |
|  |  | Subset-026-4.8.5.6 b) |  |
|  |  | Subset-026-4.8.5.6 c) |  |
|  |  | Subset-026-4.8.5.7 |  |
|  |  | Subset-026-3.10.2.4 |  |
|  |  | Subset-026-3.10.2.2 |  |
|  |  | Subset-026-3.10.2.2 b) |  |
|  |  | Subset-026-7.4.2.4 Packet Number 15: Level 2/3 Movement Authority |  |
|  |  | Subset-026-7.4.2.6 Packet Number 21: Gradient Profile |  |
|  |  | Subset-026-7.4.2.7 Packet Number 27: International Static Speed Profile |  |
|  |  | Subset-026-7.4.2.9 Packet Number 41: Level Transition Order |  |
|  |  | Subset-026-8.7.2 Message 3: Movement Authority |  |
|  |  | Subset-026-8.7.6 Message 15: Conditional Emergency Stop |  |
|  |  | Subset-026-8.6.8 Message 147: Acknowledgement of Emergency Stop |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: Train Level changes to L2 / L3 at the transition border. Train Mode changes to FS (Packets 15, 21 and 27 were stored in the buffer), if the previous mode was not FS The TS and TD are displayed according to the second Message 3 received before Message 15. The distance to the EoA defined in this second message 3 is shorter than the distance to the Stop location defined in the following Message 15. Check on the JRU that : Radio messages, including packets 15, 21 and 27, are RECORDED. Radio message 15 is RECORDED. The new level is RECORDED. |
| Constraints | Packet 41 with an announcement of L2 / L3 transition at further location has been received and it is stored on-board. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 4 / 12 / 13 | FS / OS / UN / LS / SN |
| Radio communication session | ESTABLISHED | - |
| Level Transition Order | STORED | M_LEVELTR=3/4 Transition to Level 2/3 D_LEVELTR=FINITE VALUE > 0 and < 32767 (further location) |
| ERTMS/ETCS level | 0 / 1 / 2 | L0 / LNTC / L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level <L2 / L3> announcement" is displayed. | O | DMI | DMI Object: Symbol LE12 / LE14 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / LNTC / L1 | FS/ LS/ OS/ UN/ SN | SA-DATA. Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 15, packet 21, packet 27 is received | I | RTM | The train receives Message 3: Movement Authority including this sequence of packets: Packet 15: Level 2/3 Movement Authority Packet 21: Gradient Profile Packet 27: International Static Speed Profile | L0 / LNTC / L1 | FS/ LS/ OS/ UN/ SN | - |
| 2 | L0 / LNTC / L1 | FS/ LS/ OS/ UN/ SN | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=3) is recorded | O | JRU | - | L0 / LNTC / L1 | FS/ LS/ OS/ UN/ SN | - |
| 3 | L0 / LNTC / L1 | FS/ LS/ OS/ UN/ SN | SA-DATA. Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 15, packet 21, packet 27 is received | I | RTM | The train receives Message 3: Movement Authority, which has an EoA distance which is shorter than the distance to the Stop location defined in the following Message 15 (and not yet used).This MA will be used when re-evaluating the data stored on-board at the level transition change to L2. | L0 / LNTC / L1 | FS/ LS/ OS/ UN/ SN | - |
| 4 | L0 / LNTC / L1 | FS/ LS/ OS/ UN/ SN | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=3) is recorded | O | JRU | - | L0 / LNTC / L1 | FS/ LS/ OS/ UN/ SN | - |
| 5 | L0 / LNTC / L1 | FS/ LS/ OS/ UN/ SN | SA-DATA. Indication with Euroradio Message "Conditional Emergency Stop" (NID_MESSAGE=15) is received | I | RTM | The location of the emergency stop location will be placed between the EoA of the first received Level 2 MA and the EoA of the second MA. | L0 / LNTC / L1 | FS/ LS/ OS/ UN/ SN | - |
| 6 | L0 / LNTC / L1 | FS/ LS/ OS/ UN/ SN | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=15) is recorded | O | JRU | - | L0 / LNTC / L1 | FS/ LS/ OS/ UN/ SN | - |
| 7 | L0 / LNTC / L1 | FS/ LS/ OS/ UN/ SN | Train is moving. | I | INT | The estimated front end of the train reaches the level border and the on-board switches to ETCS level 2 / 3. The sets of information stored in the transition buffer are released and re-evaluated in the sequence they have been received. | L0 / LNTC / L1 | FS/ LS/ OS/ UN/ SN | - |
| 8 | L0 / LNTC / L1 | FS/ LS/ OS/ UN/ SN | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=3 / 4; M_MODE=0) is recorded | O | JRU | TRIGGERING EVENT; When the level changes; When the Mode changes | L2 / L3 | FS | - |
| 9 | L2 / L3 | FS | The Level symbol "Level <L2 /L3>" is displayed | O | DMI | For level change symbol LE04/LE05 is displayed. Exception: If the previous level is LNTC, symbol LE13/LE15 is displayed and driver acknowledgement is requested. | L2 / L3 | FS | - |
| 10 | L2 / L3 | FS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit04/Bit05=1>) is recorded | O | JRU | - | L2 / L3 | FS | - |
| 11 | L2 / L3 | FS | The Mode symbol "Full Supervision" is displayed | O | DMI | If FS was the previous mode, it will not change when the level transition is executed. For mode change symbol MO11 is displayed. | L2 / L3 | FS | - |
| 12 | L2 / L3 | FS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit26=1>) is recorded | O | JRU | - | L2 / L3 | FS | - |
| 13 | L2 / L3 | FS | The speed and distance monitoring information is displayed | O | DMI | The Target Speed and TD are displayed according to the stop location defined by the second MA previously received. | L2 / L3 | FS | - |
| 14 | L2 / L3 | FS | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | - | L2 / L3 | FS | - |
| 15 | L2 / L3 | FS | SA-DATA. Request with Euroradio Message "Acknowledgement of Emergency Stop" (NID_MESSAGE=147) is transmitted | O | RTM | With Q_EMERGENCYSTOP = 01b Indicating that the Conditional Emergency Stop is ignored. | L2 / L3 | FS | - |
| 16 | L2 / L3 | FS | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | L2 / L3 | FS | - |
| 17 | L2 / L3 | FS | One Balise Group message (N_TOTAL=1) containing packet 41 is received | I | BTM | The Balise placed at level border is detected when the estimated front end has already passed the level border. The order to switch to ETCS level 2 / 3 immediately is RECEIVED. M_LEVELTR = 3 / 4 | L2 / L3 | FS | - |
| 18 | L2 / L3 | FS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L2 / L3 | FS | - |

| Step 1: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | Movement Authority |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | Level 2/3 Movement Authority |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | Packet 21: Gradient Profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | Packet 27: Static Speed Profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |

| Step 3: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | Movement Authority |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | Level 2/3 Movement Authority |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | Packet 21: Gradient Profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | Packet 27: Static Speed Profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |

| Step 5: Radio Message 15 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 15 | Conditional Emergency Stop |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | 0/1 | 0: No acknowledgement required 1: Acknowledgement required |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_EM | 4 | FINITE VALUE | Emergency Stop Identification Number |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_REF | 16 | FINITE VALUE | Distance between the LRBG and the position reference to the emergency stop |
| Q_DIR | 2 | FINITE VALUE | - |
| D_EMERGENCYSTOP | 15 | FINITE VALUE | Distance between LRBG and the position reference to the emergency stop |

| Step 15: Radio Message 147 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 147 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_EM | 4 | FINITE VALUE | Identification Number of the acknowledged Emergency Stop Message |
| Q_EMERGENCYSTOP | 2 | 01 | Conditional Emergency Stop accepted with no update of current EOA/LOA (ref. 3.10.2.2) |
| Packet 0 or 1 | - | FINITE VALUE | - |

| Step 17: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | - |
| M_VERSION | 7 | 010 0000 | Version 2.0 |
| Q_MEDIA | 1 | 0 | - |
| N_PIG | 3 | FINITE VALUE | - |
| N_TOTAL | 3 | FINITE VALUE | - |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINK | 1 | 1 | - |
| NID_PACKET | 8 | 41 | Level transition order (D_LEVELTR = special value = now, M_LEVELTR = 3 / 4) |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | 32767 | Now |
| M_LEVELTR | 3 | 3 / 4 | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTR = 1 (NTC). |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 | FS |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level 2/3" is displayed. The Mode symbol "Full Supervision" is displayed. Speed and distance monitoring information is displayed. | O | DMI | DMI Objects: LE04/LE05 MO11 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.5: Handling of transition buffer in case of level transition announcement or RBC/RBC handover |  | 4080500 |
| Test Case of Feature | Handling of transition buffer in case of level transition announcement. The sets of information stored in the transition buffer shall be deleted in case the level transition order is deleted or overwritten by another level transition order for a different level. |  | 4 |
| Applicable Mode/Level Combinations | L0: UN LNTC: SN L2 / L3: FS / LS / OS |  |  |
| Target of Test | The target of this test is to check that: After receiving a new level transition order for L0 (before reaching the L1 transition border) all the information stored in the transition buffer is deleted. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.5.1 |  |
|  |  | Subset-026-4.8.5.4 |  |
|  |  | Subset-026-4.8.5.4 a) |  |
|  |  | Subset-026-4.8.5.6 |  |
|  |  | Subset-026-4.8.5.6 a) |  |
|  |  | Subset-026-4.8.5.6 b) |  |
|  |  | Subset-026-4.8.5.6 c) |  |
|  |  | Subset-026-7.4.2.3 Packet Number 12: Level 1 Movement Authority |  |
|  |  | Subset-026-7.4.2.26 Packet Number 80: Mode Profile |  |
|  |  | Subset-026-7.4.2.6 Packet Number 21: Gradient Profile |  |
|  |  | Subset-026-7.4.2.7 Packet Number 27: International Static Speed Profile |  |
|  |  | Subset-026-7.4.2.9 Packet Number 41: Level Transition Order |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: ETCS Level changes to L0 and doesn’t change to L1 (All sets of information stored in the transition buffer are DELETED). Trip reason: “No MA received at level border” is displayed. Check on the JRU that : Balise Group BGa message, including packets 80, 12, 21 and 27, is RECORDED. Balise Group message, including the L0 Transition Order (Packet 41 with M_LEVELTR=0 and D_LEVELTR = 0 / ‘now’) is RECORDED. The new level is RECORDED. The mode change to UN is RECORDED. Balise Group message, including the L1 Transition Order (Packet 41 with M_LEVELTR=2 and D_LEVELTR = 0 / ‘now’) is RECORDED The mode change to TR is RECORDED. |
| Constraints | Packet 41 with an announcement of L1 transition at further location has been received and is stored on-board. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 4 / 12 / 13 | FS / OS / UN / LS / SN |
| Level Transition Order | STORED | M_LEVELTR=2 Transition to Level 1 D_LEVELTR=FINITE VALUE > 0 and < 32767 (further location) |
| ERTMS/ETCS level | 0 / 1 / 3 / 4 | L0 / LNTC / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level <L1> announcement" is displayed. | O | DMI | DMI Object: Symbol LE10 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / LNTC / L2 / L3 | FS /LS / OS / UN / SN | One Balise Group message (N_TOTAL=1) containing packet 80, packet 12, packet 21, packet 27 is received | I | BTM | The ERTMS/ETCS on-board equipment stores the first set of information in a transition buffer. Packet 80: Mode Profile Packet 12: Movement Authority for L1 Packet 21: Gradient Profile Packet 27: International Static Speed Profile | L0 / LNTC / L2 / L3 | FS /LS / OS / UN / SN | - |
| 2 | L0 / LNTC / L2 / L3 | FS /LS / OS / UN / SN | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L0 / LNTC / L2 / L3 | FS /LS / OS / UN / SN | - |
| 3 | L0 / LNTC / L2 / L3 | FS /LS / OS / UN / SN | One Balise Group message (N_TOTAL=1) containing packet 41 is received | I | BTM | Before reaching the L1 transition border a balise group BGb with a L0 transition order is received. M_LEVELTR = 0, D_LEVELTR = ‘0’ or ‘now’ All sets of information stored in the transition buffer are deleted. | L0 / LNTC / L2 / L3 | FS /LS / OS / UN / SN | - |
| 4 | L0 / LNTC / L2 / L3 | FS /LS / OS / UN / SN | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L0 / LNTC / L2 / L3 | FS /LS / OS / UN / SN | - |
| 5 | L0 / LNTC / L2 / L3 | FS /LS / OS / UN / SN | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=0; M_MODE=4) is recorded | O | JRU | The on-board equipment switches immediately to level 0 and UN mode. TRIGGERING EVENT: When the level changes; When the Mode changes. Can be the same as the actual one. | L0 | UN | - |
| 6 | L0 | UN | The Level symbol "Acknowledge Level 0 announcement" is displayed. | O | DMI | Optional step if the previous level is L0/UN. The driver is informed on the DMI of the level transition. DMI Object: Symbol LE07 | L0 | UN | - |
| 7 | L0 | UN | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit07=1>) is recorded | O | JRU | Optional step if the previous level is L0/UN. | L0 | UN | - |
| 8 | L0 | UN | The Mode symbol "Unfitted" is displayed. | O | DMI | The driver is informed on the DMI of the change to UN mode. DMI Object: Symbol MO16 | L0 | UN | - |
| 9 | L0 | UN | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit31=1>) is recorded | O | JRU | - | L0 | UN | - |
| 10 | L0 | UN | The driver acknowledges the Level Transition | - | - | Only for transitions from L2, L3 or LNTC to L0: USE_FT5100400.13 (Transition from L2 to L0) or USE_FT5100400.19 (Transition from L3 to L0) or USE_FT5100400.25 (Transition from LNTC to L0) | L0 | UN | - |
| 11 | L0 | UN | The train is tripped. Reason: No MA received at level transition MA received at step 1 is deleted. | - | - | USE_FT4060300.28 | L1 | TR | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 010 0000 | Version 2.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | 1 of N |
| N_TOTAL | 3 | FINITE VALUE | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | Bga |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 80 | Mode profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_MAMODE | 15 | 0 | Mode change starting in the current location |
| M_MAMODE | 2 | FINITE VALUE | OS, LS |
| V_MAMODE | 7 | FINITE VALUE | - |
| L_MAMODE | 15 | FINITE VALUE | - |
| L_ACKMAMODE | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| NID_PACKET | 8 | 12 | Packet 12: Level 1 Movement Authority |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | Can be set to "no time-out". |
| N_ITER | 5 | FINITE VALUE | Set to zero if V_MAIN = 0 or if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | Gradient Profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | Static Speed Profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | < V_MAIN in Packet 12. Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 010 0000 | Version 2.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 001 | 2 of N |
| N_TOTAL | 3 | FINITE VALUE | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 3: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | - |
| M_VERSION | 7 | 010 0000 | Version 2.0 |
| Q_MEDIA | 1 | 0 | - |
| N_PIG | 3 | FINITE VALUE | - |
| N_TOTAL | 3 | FINITE VALUE | - |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINK | 1 | 1 | - |
| NID_PACKET | 8 | 41 | Level transition order |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | 32767 | Now |
| M_LEVELTR | 3 | 0 | L0 |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTR = 1 (NTC). |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 | TR |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level 1" is displayed. The Mode symbol " TR" is displayed. | O | DMI | DMI Object: Symbol LE03 DMI Object: Symbol MO04 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 5

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.5: Handling of transition buffer in case of level transition announcement or RBC/RBC handover |  | 4080500 |
| Test Case of Feature | Handling of transition buffer in case of level transition announcement. In case three sets of information are already stored in the transition buffer, any new set to be stored shall replace the oldest one currently stored. Re-evaluating Unconditional Emergency Stop information. |  | 5 |
| Applicable Mode/Level Combinations | L0: UN LNTC: SN |  |  |
| Target of Test | The target of this test is to check that: When the on board equipment has received a level transition order at further location, the transition buffer is able to store three sets of information. The fourth set of information is stored replacing the oldest one. At the same time the level transition is performed, the sets of information stored in the transition buffer shall be released and re-evaluated in the sequence they have been received. The unconditional emergency stop message is replaced and the train is not tripped. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.10.2.3 |  |
|  |  | Subset-026-4.8.5.1 |  |
|  |  | Subset-026-4.8.5.3 |  |
|  |  | Subset-026-4.8.5.6 |  |
|  |  | Subset-026-4.8.5.6 a) |  |
|  |  | Subset-026-4.8.5.6 b) |  |
|  |  | Subset-026-4.8.5.6 c) |  |
|  |  | Subset-026-7.4.2.4 Packet Number 15: Level 2/3 Movement Authority |  |
|  |  | Subset-026-7.4.2.6 Packet Number 21: Gradient Profile |  |
|  |  | Subset-026-7.4.2.7 Packet Number 27: International Static Speed Profile |  |
|  |  | Subset-026-7.4.2.2 Packet Number 5: Linking |  |
|  |  | Subset-026-7.4.2.23 Packet Number 72: Packet for sending plain text messages |  |
|  |  | Subset-026-7.4.2.9 Packet Number 41: Level Transition Order |  |
|  |  | Subset-026-8.7.7 Message 16: Unconditional Emergency Stop |  |
|  |  | Subset-026-8.7.2 Message 3: Movement Authority |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: Train Level changes to L2/L3 at the transition border. Train Mode changes to FS. (Message 3, with packets 15, 21 and 27, was stored in the buffer) Train is not tripped at the transition border. (Message 16 (Unconditional Emergency Stop) was replaced). Check on the JRU that : RBC Messages 16 (Unconditional Emergency Stop), message 3 (including packets 15, 21 and 27) message 24 with packet 5 and message 24 with packet 72 are RECORDED. Balise Group message, including the L2/L3 Transition Order (Packet 41 with M_LEVELTR=3 / 4 and D_LEVELTR = 0 / ‘now’) is RECORDED. The new level is RECORDED. The mode change to FS is RECORDED. |
| Constraints | Packet 41 with an announcement of L2/L3 transition has been received and is stored on-board. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 4 / 13 | UN / SN |
| Radio communication session | ESTABLISHED | - |
| Level Transition Order | STORED | M_LEVELTR=3/4 Transition to Level 2/3 D_LEVELTR=FINITE VALUE > 0 and < 32767 (further location) |
| ERTMS/ETCS level | 0 / 1 | L0 / LNTC |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level <L2 / L3> announcement" is displayed. | O | DMI | DMI Object: Symbol LE12 / LE14 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / LNTC | UN / SN | SA-DATA. Indication with Euroradio Message "Unconditional Emergency Stop" (NID_MESSAGE=16) is received | I | RTM | - | L0 / LNTC | UN / SN | - |
| 2 | L0 / LNTC | UN / SN | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=16) is recorded | O | JRU | - | L0 / LNTC | UN / SN | - |
| 3 | L0 / LNTC | UN / SN | SA-DATA. Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 15, packet 21, packet 27 is received | I | RTM | The train receives Message 3: Movement Authority including this sequence of packets: Packet 15: Level 2/3 Movement Authority Packet 21: Gradient Profile Packet 27: International Static Speed Profile | L0 / LNTC | UN / SN | - |
| 4 | L0 / LNTC | UN / SN | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=3) is recorded | O | JRU | - | L0 / LNTC | UN / SN | - |
| 5 | L0 / LNTC | UN / SN | SA-DATA. Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 5 is received | I | RTM | - | L0 / LNTC | UN / SN | - |
| 6 | L0 / LNTC | UN / SN | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=24) is recorded | O | JRU | The ERTMS/ETCS on-board equipment stores the first three sets of information in a transition buffer. | L0 / LNTC | UN / SN | - |
| 7 | L0 / LNTC | UN / SN | SA-DATA. Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 72 is received | I | RTM | - | L0 / LNTC | UN / SN | - |
| 8 | L0 / LNTC | UN / SN | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=24) is recorded | O | JRU | The fourth set of information is stored replacing the oldest one currently stored. Message 16 is replaced by message 24 with packet 72. | L0 / LNTC | UN / SN | - |
| 9 | L0 / LNTC | UN / SN | One Balise Group message (N_TOTAL=1) containing packet 41 is received | I | BTM | At the level border an order to switch to ETCS level 2 / 3 immediately is received. | L0 / LNTC | UN / SN | - |
| 10 | L0 / LNTC | UN / SN | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L0 / LNTC | UN / SN | - |
| 11 | L0 / LNTC | UN / SN | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=3 / 4; M_MODE=0) is recorded | O | JRU | Notice that train is not tripped. The Unconditional Emergency Stop, stored in the buffer, has been replaced by message 24 with packet 72 TRIGGERING EVENT: When the level changes; When the Mode changes | L2 / L3 | FS | - |
| 12 | L2 / L3 | FS | The Level symbol "Level <L2 /L3>" is displayed | O | DMI | For level transition symbol LE04/LE05 is displayed. Exception: If the previous level is LNTC, symbol LE13/LE15 is displayed and driver acknowledgement is requested. | L2 / L3 | FS | - |
| 13 | L2 / L3 | FS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit04/Bit05=1>) is recorded | O | JRU | - | L2 / L3 | FS | - |
| 14 | L2 / L3 | FS | The Mode symbol "Full Supervision" is displayed | O | DMI | DMI Object: MO11 | L2 / L3 | FS | - |
| 15 | L2 / L3 | FS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit26=1>) is recorded | O | JRU | - | L2 / L3 | FS | - |
| 16 | L2 / L3 | FS | The speed and distance monitoring information is displayed | O | DMI | The permitted speed is DISPLAYED. The target speed is DISPLAYED. The target distance is DISPLAYED | L2 / L3 | FS | - |
| 17 | L2 / L3 | FS | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | - | L2 / L3 | FS | - |
| 18 | L2 / L3 | FS | The "plain text message" is displayed | O | DMI | Any plain text message | L2 / L3 | FS | - |
| 19 | L2 / L3 | FS | START DISPLAYING PLAIN TEXT MESSAGE (NID_MESSAGE_JRU=18) is recorded | O | JRU | - | L2 / L3 | FS | - |

| Step 1: Radio Message 16 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 16 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | 0/1 | 0: No acknowledgement required 1: Acknowledgement required |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_EM | 4 | FINITE VALUE | Emergency Stop Identification Number |

| Step 3: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | Movement Authority |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | Level 2/3 Movement Authority |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_LOA | 7 | FINITE VALUE | - |
| T_LOA | 10 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | Packet 21: Gradient Profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | Packet 27: Static Speed Profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |

| Step 5: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | 0/1 | 0: No acknowledgement required 1: Acknowledgement required |
| NID_LRBG | 14 | FINITE VALUE | - |
| NID_PACKET | 8 | 5 | linking |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LINK | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION | 1 | FINITE VALUE | - |
| Q_LINKREACTION | 2 | FINITE VALUE | - |
| Q_LOCACC | 6 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_LINK(k) | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | FINITE VALUE | - |
| Q_LOCACC(k) | 6 | FINITE VALUE | - |

| Step 7: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | 0/1 | 0: No acknowledgement required 1: Acknowledgement required |
| NID_LRBG | 14 | FINITE VALUE | - |
| NID_PACKET | 8 | 72 | Plain text message |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TEXTCLASS | 2 | FINITE VALUE | - |
| Q_TEXTDISPLAY | 1 | FINITE VALUE | Start/end events relation. |
| D_TEXTDISPLAY | 15 | FINITE VALUE | Start event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | Start event. |
| M_LEVELTEXTDISPLAY | 3 | 3 / 4 | (to be display when in L2/3) |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| L_TEXTDISPLAY | 15 | FINITE VALUE | End event. |
| T_TEXTDISPLAY | 10 | FINITE VALUE | End event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | End event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | End event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| Q_TEXTCONFIRM | 2 | FINITE VALUE | - |
| Q_CONFTEXTDISPLAY | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| Q_TEXTREPORT | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| NID_TEXTMESSAGE | 8 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_C | 10 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_RBC | 14 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| L_TEXT | 8 | FINITE VALUE | - |
| X_TEXT(L_TEXT) | 8 | FINITE VALUE | Any message |

| Step 9: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | - |
| M_VERSION | 7 | 010 0000 | Version 2.0 |
| Q_MEDIA | 1 | 0 | - |
| N_PIG | 3 | FINITE VALUE | - |
| N_TOTAL | 3 | FINITE VALUE | - |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINK | 1 | 1 | - |
| NID_PACKET | 8 | 41 | Level transition order (D_LEVELTR = special value = now, M_LEVELTR = 3 / 4) |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | 32767 | Now |
| M_LEVELTR | 3 | 3 / 4 | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTR = 1 (NTC). |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 | FS |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level 2/3" is displayed. The Mode symbol "Full Supervision" is displayed. Speed and distance monitoring information is displayed. Plain Text message is DISPLAYED | O | DMI | DMI Objects: LE04/LE05; MO11 Any plain text message |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 6

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.5: Handling of transition buffer in case of level transition announcement or RBC/RBC handover |  | 4080500 |
| Test Case of Feature | Handling of transition buffer in case of level transition announcement. In case three sets of information are already stored in the transition buffer, any new set to be stored shall replace the oldest one currently stored. Re-evaluating Unconditional Emergency Stop information. |  | 6 |
| Applicable Mode/Level Combinations | L1: FS/ LS/ OS |  |  |
| Target of Test | The target of this test is to check that: When the on board equipment has received a level transition order at further location, the transition buffer is able to store three sets of information. The fourth information is stored replacing the oldest one. At the same time the level transition is performed, the sets of information stored in the transition buffer shall be released and re-evaluated in the sequence they have been received. When receiving an unconditional emergency stop message the train shall be tripped immediately. (for an Unconditional Emergency Stop , Q_EMERCENCYSTOP is set to “not relevant”, value 2) |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-3.10.2.3 |  |
|  |  | Subset-026-3.10.2.6 |  |
|  |  | Subset-026-4.8.5.1 |  |
|  |  | Subset-026-4.8.5.3 |  |
|  |  | Subset-026-4.8.5.5 |  |
|  |  | Subset-026-4.8.5.6 |  |
|  |  | Subset-026-4.8.5.6 a) |  |
|  |  | Subset-026-4.8.5.6 b) |  |
|  |  | Subset-026-4.8.5.6 c) |  |
|  |  | Subset-026-4.7.2 table - Output information Accepted Emergency Stop(s) |  |
|  |  | Subset-026-7.4.2.23 Packet Number 72: Packet for sending plain text messages |  |
|  |  | Subset-026-7.4.2.8 Packet Number 39: Track Condition Change of traction system |  |
|  |  | Subset-026-7.4.2.24 Packet Number 76: Packet for sending fixed text messages |  |
|  |  | Subset-026-7.4.2.9 Packet Number 41: Level Transition Order |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |
|  |  | Subset-026-8.7.7 Message 16: Unconditional Emergency Stop |  |
|  |  | Subset-026-8.6.8 Message 147: Acknowledgement of Emergency Stop |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: Train Level changes to L2/L3 at the transition border. Track condition is DISPLAYED. (General Message, with packet 39 was stored in the buffer) Fixed Text is DISPLAYED. (General Message, with packet 76, was stored in the buffer) Train is tripped at the transition border. (Message 16 (Unconditional Emergency Stop) was received and stored in the buffer replacing packet 72). Trip reason, “Emergency stop “ is DISPLAYED Check on the JRU that : RBC Messages 16 (Unconditional Emergency Stop) and general messages 24 (including packets 72, 39 and 76), are RECORDED. Balise Group message, including the L2/L3 Transition Order (Packet 41 with M_LEVELTR=3 / 4 and D_LEVELTR = 0 / ‘now’) is RECORDED. The new level is RECORDED. The mode change to TR is RECORDED. The trip reason is RECORDED. |
| Constraints | MA and track description covering at least the first section of the level 2 / 3 area are stored on-board. Packet 41 with an announcement of L2/L3 transition at further location has been received and is stored on-board. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 2 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | STORED | Covering at least the first section of the level 2 / 3 area |
| Gradient Profile | STORED | Covering at least the first section of the level 2 / 3 area |
| International SSP | STORED | Covering at least the first section of the level 2 / 3 area |
| Level Transition Order | STORED | M_LEVELTR=3/4 Transition to Level 2/3 D_LEVELTR=FINITE VALUE > 0 and < 32767 (further location) |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level <L2/L3> announcement" is displayed. | O | DMI | DMI Object: Symbol LE12 / LE14 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 | FS / LS / OS | SA-DATA. Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 72 is received | I | RTM | Packet 72: Plain Text Message | L1 | FS / LS / OS | - |
| 2 | L1 | FS / LS / OS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=24) is recorded | O | JRU | - | L1 | FS / LS / OS | - |
| 3 | L1 | FS / LS / OS | SA-DATA. Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 39 is received | I | RTM | Packet 39: Track Condition Change of Traction system | L1 | FS / LS / OS | - |
| 4 | L1 | FS / LS / OS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=24) is recorded | O | JRU | - | L1 | FS / LS / OS | - |
| 5 | L1 | FS / LS / OS | SA-DATA. Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 76 is received | I | RTM | Packet 76: Fixed Text Message | L1 | FS / LS / OS | - |
| 6 | L1 | FS / LS / OS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=24) is recorded | O | JRU | - | L1 | FS / LS / OS | - |
| 7 | L1 | FS / LS / OS | SA-DATA. Indication with Euroradio Message "Unconditional Emergency Stop" (NID_MESSAGE=16) is received | I | RTM | - | L1 | FS / LS / OS | - |
| 8 | L1 | FS / LS / OS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=16) is recorded | O | JRU | - | L1 | FS / LS / OS | - |
| 9 | L1 | FS / LS / OS | One Balise Group message (N_TOTAL=1) containing packet 41 is received | I | BTM | At the level border the order to switch to ETCS level 2 / 3 immediately is RECEIVED. M_LEVELTR = 3 / 4 D_LEVELTR= "now" | L1 | FS / LS / OS | - |
| 10 | L1 | FS / LS / OS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L1 | FS / LS / OS | - |
| 11 | L1 | FS / LS / OS | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=3 / 4) is recorded | O | JRU | TRIGGERING EVENT: When the level changes | L2 / L3 | FS / LS / OS | - |
| 12 | L2 / L3 | FS / LS / OS | The Level symbol "Level <L2 /L3>" is displayed | O | DMI | For level change symbol LE04/LE05 is displayed. Exception: If the previous level is LNTC, symbol LE13/LE15 is displayed and driver acknowledgement is requested. | L2 / L3 | FS / LS / OS | - |
| 13 | L2 / L3 | FS / LS / OS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit04/Bit05=1>) is recorded | O | JRU | - | L2 / L3 | FS / LS / OS | - |
| 14 | L2 / L3 | FS / LS / OS | The system status message "Emergency stop" is displayed | O | DMI | The driver is informed that the unconditional emergency stop is accepted. Message 24 with Packet 72 is replaced by Message 16. The fourth set of information is stored replacing the oldest one currently stored. | L2 / L3 | FS / LS / OS | - |
| 15 | L2 / L3 | FS / LS / OS | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23) is recorded | O | JRU | - | L2 / L3 | FS / LS / OS | - |
| 16 | L2 / L3 | FS / LS / OS | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=7) is recorded | O | JRU | The train is Tripped TRIGGERING EVENT: When the mode changes | L2 / L3 | TR | - |
| 17 | L2 / L3 | TR | The Mode symbol "Trip" is displayed. The system status message "Emergency stop" is displayed. | O | DMI | The driver is informed on the DMI of the change to TR mode and trip reason. DMI Object: MO04 | L2 / L3 | TR | - |
| 18 | L2 / L3 | TR | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit19=1>) is recorded | O | JRU | - | L2 / L3 | TR | - |
| 19 | L2 / L3 | TR | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23) is recorded | O | JRU | - | L2 / L3 | TR | - |
| 20 | L2 / L3 | TR | Report mode change to RBC | - | - | USE_FT3060500.4 | L2 / L3 | TR | - |
| 21 | L2 / L3 | TR | Emergency brake commanded | O | TIU | - | L2 / L3 | TR | - |
| 22 | L2 / L3 | TR | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | L2 / L3 | TR | - |
| 23 | L2 / L3 | TR | SA-DATA .Request with Euroradio Message "Acknowledgement of Emergency Stop" (NID_MESSAGE=147) containing packet 0 is transmitted. | O | RTM | Indicating that the train has received the Unconditional Emergency Stop. Check that the messages 147 is send over the normal priority channel. Q_EMERGENCYSTOP set to 2 | L2 / L3 | TR | - |
| 24 | L2 / L3 | TR | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | L2 / L3 | TR | - |
| 25 | L2 / L3 | TR | Check that the track condition information is accepted | - | - | Check that information from Packet 39 received in the Message 24 is used as specified in USE_FT4070201.33 | L2 / L3 | TR | - |
| 26 | L2 / L3 | TR | Check that the fixed text message is displayed on DMI | - | - | Check that Packet 76 received in the Message 24 has the same values that are specified in USE_FT3120300.9. The start condition is limited to a distance condition. The end condition is limited to a time condition. | L2 / L3 | TR | - |
| 27 | L2 / L3 | TR | NOT The "plain text message" is displayed | O | DMI | When the location is reached where or when the starting condition(s) related to plain text message are fulfilled. | L2 / L3 | TR | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 72 | Plain Text Message |
| Q_DIR | 2 | FINITE VALUE | Valid for current train direction |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TEXTCLASS | 2 | FINITE VALUE | - |
| Q_TEXTDISPLAY | 1 | FINITE VALUE | Start/end events relation. |
| D_TEXTDISPLAY | 15 | FINITE VALUE | Start event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | Start event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | (to be display when in L2/3) |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| L_TEXTDISPLAY | 15 | FINITE VALUE | End event. |
| T_TEXTDISPLAY | 10 | FINITE VALUE | End event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | End event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | End event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| Q_TEXTCONFIRM | 2 | FINITE VALUE | - |
| Q_CONFTEXTDISPLAY | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| Q_TEXTREPORT | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| NID_TEXTMESSAGE | 8 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_C | 10 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_RBC | 14 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| L_TEXT | 8 | FINITE VALUE | - |
| X_TEXT(L_TEXT) | 8 | FINITE VALUE | - |

| Step 3: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 39 | Track conditions change of traction system |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_TRACTION | 1 | FINITE VALUE | - |
| M_VOLTAGE | 4 | FINITE VALUE | Identity of the traction system |
| NID_CTRACTION | 4 | FINITE VALUE | Given only if M_VOLTAGE ≠ 0 |

| Step 5: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 76 | Fixed Text Message |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TEXTCLASS | 2 | FINITE VALUE | - |
| Q_TEXTDISPLAY | 1 | FINITE VALUE | Start/end events relation. |
| D_TEXTDISPLAY | 15 | FINITE VALUE | Start event condition. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | Start event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | Start event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| L_TEXTDISPLAY | 15 | FINITE VALUE | End event. |
| T_TEXTDISPLAY | 10 | FINITE VALUE | End event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | End event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | End event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| Q_TEXTCONFIRM | 2 | FINITE VALUE | - |
| Q_CONFTEXTDISPLAY | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| Q_TEXTREPORT | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0 |
| NID_TEXTMESSAGE | 8 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_C | 10 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_RBC | 14 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| Q_TEXT | 8 | FINITE VALUE | - |

| Step 7: Radio Message 16 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 16 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | 1 | Acknowledgement required |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_EM | 4 | FINITE VALUE | Emergency Stop Identification Number |

| Step 9: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | - |
| M_VERSION | 7 | 010 0000 | Version 2.0 |
| Q_MEDIA | 1 | 0 | - |
| N_PIG | 3 | FINITE VALUE | - |
| N_TOTAL | 3 | FINITE VALUE | - |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINK | 1 | 1 | - |
| NID_PACKET | 8 | 41 | Level transition order (D_LEVELTR = special value = now, M_LEVELTR = 3 / 4) |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | 32767 | Now |
| M_LEVELTR | 3 | 3 / 4 | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTR = 1 (NTC). |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 23: Radio Message 147 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 147 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_EM | 4 | FINITE VALUE | The same value that in the previous message 16 |
| Q_EMERGENCYSTOP | 2 | 2 | Not Relevant (Unconditional Emergency Stop) (ref. 3.10.2.3) |
| NID_PACKET | 8 | 0 | Position report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 | TR |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| New Level 2 / 3 is DISPLAYED. Change of traction system announcement is DISPLAYED. Fixed Text is DISPLAYED. Indication of emergency message reception is DISPLAYED. Trip mode is DISPLAYED. Trip reason is DISPLAYED. | O | DMI | DMI Object: Symbol LE04/LE05 DMI Object: Symbol MO04 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 7

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.5: Handling of transition buffer in case of level transition announcement or RBC/RBC handover |  | 4080500 |
| Test Case of Feature | Handling of transition buffer in case of level transition announcement. The set of information stored in the transition buffer shall be deleted in case the level transition order is deleted or overwritten. Re-evaluating Unconditional Emergency Stop information. |  | 7 |
| Applicable Mode/Level Combinations | L1: FS/ LS/ OS |  |  |
| Target of Test | The target of this test is to check that: After receiving a new level transition order for L1 (before reaching the L2/L3 transition border) all the information stored in the transition buffer is deleted. The unconditional emergency stop message is replaced and the train is not tripped. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.5.1 |  |
|  |  | Subset-026-4.8.5.4 |  |
|  |  | Subset-026-4.8.5.4 a) |  |
|  |  | Subset-026-3.10.2.3 |  |
|  |  | Subset-026-7.4.2.23 Packet Number 72: Packet for sending plain text messages |  |
|  |  | Subset-026-7.4.2.20 Packet Number 68: Track Condition |  |
|  |  | Subset-026-7.4.2.24 Packet Number 76: Packet for sending fixed text messages |  |
|  |  | Subset-026-7.4.2.9 Packet Number 41: Level Transition Order |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |
|  |  | Subset-026-8.7.7 Message 16: Unconditional Emergency Stop |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: Train Level doesn’t change to L2/L3 (After receiving L1 transition order all sets of information stored in the transition buffer are DELETED). Check on the JRU that : RBC Messages 16 (Unconditional Emergency Stop) and 24 (including packets72, 68 and 76), are RECORDED. Balise Group message, including the L1 Transition Order (Packet 41 with M_LEVELTR=2 and D_LEVELTR = 0 / ‘now’) is RECORDED. |
| Constraints | MA and track description covering at least the first section of the level 2 / 3 area are stored on-board. Packet 41 with an announcement of L2/L3 transition at further location has been received and is stored on-board. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 2 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | STORED | Covering at least the first section of the level 2 / 3 area |
| Gradient Profile | STORED | Covering at least the first section of the level 2 / 3 area |
| International SSP | STORED | Covering at least the first section of the level 2 / 3 area |
| Level Transition Order | STORED | M_LEVELTR=3/4 Transition to Level 2/3 D_LEVELTR=FINITE VALUE > 0 and < 32767 (further location) |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol “Level <L2/L3> announcement” is displayed. | O | DMI | DMI Object: Symbol LE12 / LE14 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 | FS / LS / OS | SA-DATA. Indication with Euroradio Message “General message” (NID_MESSAGE=24) containing packet 72, packet 68, packet 76 is received | I | RTM | The train receives Message 24: General Message including this sequence of packets: Packet 72: Plain Text Message Packet 68: Track Condition Packet 76: Fixed Text Message | L1 | FS / LS / OS | - |
| 2 | L1 | FS / LS / OS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=24) is recorded | O | JRU | - | L1 | FS / LS / OS | - |
| 3 | L1 | FS / LS / OS | SA-DATA .Indication with Euroradio Message “Unconditional Emergency Stop” (NID_MESSAGE=16) is received | I | RTM | The ERTMS/ETCS on-board equipment stores the first two sets of information in a transition buffer. | L1 | FS / LS / OS | - |
| 4 | L1 | FS / LS / OS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=16) is recorded | O | JRU | - | L1 | FS / LS / OS | - |
| 5 | L1 | FS / LS / OS | One Balise Group message (N_TOTAL=1) containing packet 41 is received | I | BTM | M_LEVELTR = 2, D_LEVELTR = 0 / ‘now’ | L1 | FS / LS / OS | - |
| 6 | L1 | FS / LS / OS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L1 | FS / LS / OS | - |
| 7 | L1 | FS / LS / OS | NOT The Level symbol “Level <L2 /L3>” is displayed. | O | DMI | Train does not switch to L2/L3 when max safe front end reaches the location where transition border to L2/L3 was located. Train does not switch to L2/3 All sets of information stored in the transition buffer are DELETED | L1 | FS / LS / OS | - |
| 8 | L1 | FS / LS / OS | On-board equipment immediately switches to L2 / L3 | - | - | Use_FT5100100.37 for transition to L2 Or Use_FT5100100.36 for transition to L3 Train switches to L2/L3 to test that all sets of information stored in the transition buffer have been DELETED | L2 / L3 | FS / LS / OS | - |
| 9 | L2 / L3 | FS / LS / OS | NOT The Mode symbol "Trip" is displayed | O | DMI | Train is not tripped. Unconditional emergency stop has been deleted | L2 / L3 | FS / LS / OS | - |
| 10 | L2 / L3 | FS / LS / OS | NOT The "plain text message" is displayed | O | DMI | No Plain Text Message is displayed when it is reached the location where/when the starting condition(s) related to plain text messages are fulfilled | L2 / L3 | FS / LS / OS | - |
| 11 | L2 / L3 | FS / LS / OS | NOT The "fixed text message" is displayed | O | DMI | No Fixed Text Message is displayed when it is reached the location where/when the starting condition(s) related to fixed text message are fulfilled | L2 / L3 | FS / LS / OS | - |
| 12 | L2 / L3 | FS / LS / OS | NOT The Track Condition symbol "XX" is displayed | O | DMI | No Track condition is displayed when it is reached the location where the starting condition(s) related to track condition are fulfilled | L2 / L3 | FS / LS / OS | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 72 | Plain Text Message |
| Q_DIR | 2 | FINITE VALUE | Valid for current train direction |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TEXTCLASS | 2 | FINITE VALUE | - |
| Q_TEXTDISPLAY | 1 | FINITE VALUE | Start/end events relation. |
| D_TEXTDISPLAY | 15 | FINITE VALUE | Start event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | Start event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | (to be display when in L2/3) |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| L_TEXTDISPLAY | 15 | FINITE VALUE | End event. |
| T_TEXTDISPLAY | 10 | FINITE VALUE | End event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | End event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | End event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| Q_TEXTCONFIRM | 2 | FINITE VALUE | - |
| Q_CONFTEXTDISPLAY | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| Q_TEXTREPORT | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| NID_TEXTMESSAGE | 8 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_C | 10 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_RBC | 14 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| L_TEXT | 8 | FINITE VALUE | - |
| X_TEXT(L_TEXT) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 68 | Track conditions |
| Q_DIR | 2 | FINITE VALUE | Valid for current train direction |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | FINITE VALUE | - |
| D_TRACKINIT | 15 | FINITE VALUE | Only if Q_TRACKINIT = 1. |
| D_TRACKCOND | 15 | FINITE VALUE | Only if Q_TRACKINIT = 0, D_TRACKCOND and the following variables follow. |
| L_TRACKCOND | 15 | FINITE VALUE | - |
| M_TRACKCOND | 4 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_TRACKCOND(k) | 15 | FINITE VALUE | - |
| L_TRACKCOND(k) | 15 | FINITE VALUE | - |
| M_TRACKCOND(k) | 4 | FINITE VALUE | - |
| NID_PACKET | 8 | 76 | Fixed Text Message |
| Q_DIR | 2 | FINITE VALUE | Valid for current train direction |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TEXTCLASS | 2 | FINITE VALUE | - |
| Q_TEXTDISPLAY | 1 | FINITE VALUE | Start/end events relation. |
| D_TEXTDISPLAY | 15 | FINITE VALUE | Start event condition. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | Start event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | Start event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| L_TEXTDISPLAY | 15 | FINITE VALUE | End event. |
| T_TEXTDISPLAY | 10 | FINITE VALUE | End event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | End event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | End event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| Q_TEXTCONFIRM | 2 | FINITE VALUE | - |
| Q_CONFTEXTDISPLAY | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| Q_TEXTREPORT | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0 |
| NID_TEXTMESSAGE | 8 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_C | 10 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_RBC | 14 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| Q_TEXT | 8 | FINITE VALUE | - |

| Step 3: Radio Message 16 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 16 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | 1 | Acknowledgement required |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_EM | 4 | FINITE VALUE | Emergency Stop Identification Number |

| Step 5: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | - |
| M_VERSION | 7 | 010 0000 | Version 2.0 |
| Q_MEDIA | 1 | 0 | - |
| N_PIG | 3 | FINITE VALUE | - |
| N_TOTAL | 3 | FINITE VALUE | - |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINK | 1 | 1 | - |
| NID_PACKET | 8 | 41 | Level transition order (D_LEVELTR = special value = now, M_LEVELTR = 2) |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | 32767 | Now |
| M_LEVELTR | 3 | 2 | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTR = 1 (NTC). |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 /12 | FS/ OS / LS |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Train does not switch to TR mode No Plain Text Message is displayed when it is reached the location where/when the starting condition(s) related to plain text messages are fulfilled No Fixed Text Message is displayed when it is reached the location where/when the starting condition(s) related to fixed text message are fulfilled No Track condition is displayed when it is reached the location where the starting condition(s) related to track condition are fulfilled | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 8

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.5: Handling of transition buffer in case of level transition announcement or RBC/RBC handover |  | 4080500 |
| Test Case of Feature | Handling of transition buffer in case of RBC/RBC handover. The Handing Over RBC is the supervising one. Sets of information obtained from the Accepting RBC are stored by the on-board and released when the Accepting RBC becomes the supervising one. |  | 8 |
| Applicable Mode/Level Combinations | L2 / L3: FS/ LS/ OS/ SR |  |  |
| Target of Test | The target of this test is to check that: When the on board equipment has received a RBC transition order and the Handing Over RBC is still the supervising one, the transition buffer is able to store three sets of information from the Accepting RBC. The fourth information is stored replacing the oldest one. In case three sets of information are already stored in the transition buffer, any new set to be stored shall replace the oldest one currently stored. At the same time the Accepting RBC becomes the supervising one, the sets of information stored in the transition buffer shall be released and re-evaluated in the sequence they have been received. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.5.2 |  |
|  |  | Subset-026-4.8.5.3 |  |
|  |  | Subset-026-4.8.5.5 |  |
|  |  | Subset-026-4.8.5.6 |  |
|  |  | Subset-026-4.8.5.6 a) |  |
|  |  | Subset-026-4.8.5.6 b) |  |
|  |  | Subset-026-4.8.5.6 c) |  |
|  |  | Subset-026-7.4.2.27 Packet Number 131: RBC transition order |  |
|  |  | Subset-026-7.4.3.1 Packet Number 0: Position Report |  |
|  |  | Subset-026-7.4.3.5 Packet Number 11: Validated train data |  |
|  |  | Subset-026-7.4.2.23 Packet Number 72: Packet for sending plain text messages |  |
|  |  | Subset-026-7.4.2.20 Packet Number 68: Track Condition |  |
|  |  | Subset-026-7.4.2.24 Packet Number 76: Packet for sending fixed text messages |  |
|  |  | Subset-026-8.6.13 Message 155: Initiation of a communication session |  |
|  |  | Subset-026-8.7.12 Message 32: RBC/RIU System Version |  |
|  |  | Subset-026-8.6.17 Message 159: Session established |  |
|  |  | Subset-026-8.6.1 Message 129: Validated Train Data |  |
|  |  | Subset-026-8.7.4 Message 8: Acknowledgement of Train Data |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |
|  |  | Subset-026-8.6.4 Message 136: Train Position Report |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on RTM that: Four radio messages 24 (General message) containing packets 72, 68 and 76 are received from the Accepting RBC. Check on the DMI that: The first received plain text message is NOT DISPLAYED Track condition is DISPLAYED. (General Message, with packets 68, was stored in the buffer) Fixed Text is DISPLAYED. (General Message, with packets 76, was stored in the buffer) The second received plain text message is DISPLAYED. Check on JRU that: RBC Message 24 (including packets 72, 68 and 76), are RECORDED. The balise message containing the order to execute immediately the handover (packet 131) is RECORDED. The second fixed Text is RECORDED. |
| Constraints | The on-board equipment is able to manage two communication sessions at a time. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Radio communication session | ESTABLISHED | With Handing Over RBC |
| Position Report parameters | STORED | Handing Over RBC |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | With Handing Over RBC |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2 / L3 | FS / LS / OS / SR | SA-DATA. Indication with Euroradio Message "General Message" (NID_MESSAGE=24) containing packet 131 is received. | I | RTM | Received from the Handing Over RBC. D_RBCTR ≠ 0. | L2 / L3 | FS / LS / OS / SR | - |
| 2 | L2 / L3 | FS / LS / OS / SR | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=24; NID_PACKET=131) is recorded | O | JRU | - | L2 / L3 | FS / LS / OS / SR | - |
| 3 | L2 / L3 | FS / LS / OS / SR | SA-CONNECT. Request is transmitted | O | RTM | A safe connection is established with the Accepting RBC | L2 / L3 | FS / LS / OS / SR | - |
| 4 | L2 / L3 | FS / LS / OS / SR | SA-CONNECT. Indication is received | I | RTM | - | L2 / L3 | FS / LS / OS / SR | - |
| 5 | L2 / L3 | FS / LS / OS / SR | SA-DATA. Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted | O | RTM | - | L2 / L3 | FS / LS / OS / SR | - |
| 6 | L2 / L3 | FS / LS / OS / SR | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=155) is recorded | O | JRU | - | L2 / L3 | FS / LS / OS / SR | - |
| 7 | L2 / L3 | FS / LS / OS / SR | SA-DATA. Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32) containing is received | I | RTM | The Version of the ERTMS/ETCS language is valid for the train and M_VERSION ( { 000 XXXX, 111 1111}) | L2 / L3 | FS / LS / OS / SR | - |
| 8 | L2 / L3 | FS / LS / OS / SR | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=32) is recorded | O | JRU | - | L2 / L3 | FS / LS / OS / SR | - |
| 9 | L2 / L3 | FS / LS / OS / SR | SA-DATA. Request with Euroradio Message "Session Established" (NID_MESSAGE=159) is transmitted | O | RTM | The communication session with the Accepting RBC is established. | L2 / L3 | FS / LS / OS / SR | - |
| 10 | L2 / L3 | FS / LS / OS / SR | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=159) is recorded | O | JRU | - | L2 / L3 | FS / LS / OS / SR | - |
| 11 | L2 / L3 | FS / LS / OS / SR | SA-DATA. Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) containing packet 0 is transmitted | O | RTM | The radio message contains also position report data. | L2 / L3 | FS / LS / OS / SR | - |
| 12 | L2 / L3 | FS / LS / OS / SR | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=129) is recorded | O | JRU | - | L2 / L3 | FS / LS / OS / SR | - |
| 13 | L2 / L3 | FS / LS / OS / SR | SA-DATA. Indication with Euroradio Message "Acknowledgement of Train Data" (NID_MESSAGE=8) is received | I | RTM | - | L2 / L3 | FS / LS / OS / SR | - |
| 14 | L2 / L3 | FS / LS / OS / SR | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=8) is recorded | O | JRU | - | L2 / L3 | FS / LS / OS / SR | - |
| 15 | L2 / L3 | FS / LS / OS / SR | SA-DATA. Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 72 is received | I | RTM | The train receives Message 24 from the Accepting RBC including Packet 72: Plain Text Message. | L2 / L3 | FS / LS / OS / SR | - |
| 16 | L2 / L3 | FS / LS / OS / SR | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=24) is recorded | O | JRU | - | L2 / L3 | FS / LS / OS / SR | - |
| 17 | L2 / L3 | FS / LS / OS / SR | SA-DATA. Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 68 is received | I | RTM | The train receives Message 24 from the Accepting RBC including Packet 68: Track Condition. | L2 / L3 | FS / LS / OS / SR | - |
| 18 | L2 / L3 | FS / LS / OS / SR | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=24) is recorded | O | JRU | - | L2 / L3 | FS / LS / OS / SR | - |
| 19 | L2 / L3 | FS / LS / OS / SR | SA-DATA. Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 76 is received | I | RTM | The train receives Message 24 from the Accepting RBC including packet 76: Fixed Text Message | L2 / L3 | FS / LS / OS / SR | - |
| 20 | L2 / L3 | FS / LS / OS / SR | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=24) is recorded | O | JRU | - | L2 / L3 | FS / LS / OS / SR | - |
| 21 | L2 / L3 | FS / LS / OS / SR | NOT The "fixed/Plain text message" is displayed | O | DMI | No text message is displayed at this location. | L2 / L3 | FS / LS / OS / SR | - |
| 22 | L2 / L3 | FS / LS / OS / SR | SA-DATA. Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 72 is received | I | RTM | The first Message 24 with Packet 72 (received in step 15) is replaced by this message 24 with Packet 72. The fourth set of information is stored replacing the oldest one currently stored. | L2 / L3 | FS / LS / OS / SR | - |
| 23 | L2 / L3 | FS / LS / OS / SR | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=24) is recorded | O | JRU | - | L2 / L3 | FS / LS / OS / SR | - |
| 24 | L2 / L3 | FS / LS / OS / SR | Train is moving. | I | INT | Train Location: Train maximum safe front end location = LRBG + D_RBCTR The maximum safe front end of the train passes the location of the RBC transition. The On board is supervised by the Accepting RBC. | L2 / L3 | FS / LS / OS / SR | - |
| 25 | L2 / L3 | FS / LS / OS / SR | SA-DATA. Request with Euroradio message (NID_MESSAGE=136) containing packet 0 is transmitted | O | RTM | A radio message 136 (train position report) is sent to the Handing Over RBC. | L2 / L3 | FS / LS / OS / SR | - |
| 26 | L2 / L3 | FS / LS / OS / SR | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=136) is recorded | O | JRU | - | L2 / L3 | FS / LS / OS / SR | - |
| 27 | L2 / L3 | FS / LS / OS / SR | SA-DATA. Request with Euroradio message (NID_MESSAGE=136) containing packet 0 is transmitted | O | RTM | A radio message 136 (train position report) is sent to the Accepting RBC. | L2 / L3 | FS / LS / OS / SR | - |
| 28 | L2 / L3 | FS / LS / OS / SR | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=136) is recorded | O | JRU | - | L2 / L3 | FS / LS / OS / SR | - |
| 29 | L2 / L3 | FS / LS / OS / SR | One Balise Group message (N_TOTAL=1) containing packet 131 is received | I | BTM | The balise group is placed at the RBC transition border. | L2 / L3 | FS / LS / OS / SR | - |
| 30 | L2 / L3 | FS / LS / OS / SR | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L2 / L3 | FS / LS / OS / SR | - |
| 31 | L2 / L3 | FS / LS / OS / SR | NOT The "plain text message" is displayed | O | DMI | The plain text message of the first received Packet 72 is NOT displayed. | L2 / L3 | FS / LS / OS / SR | - |
| 32 | L2 / L3 | FS / LS / OS / SR | Check that the track condition information is accepted. | - | - | Check that Packet 68 received in the Message 24 has the same values that are specified in USE_FT5180400.1. | L2 / L3 | FS / LS / OS / SR | - |
| 33 | L2 / L3 | FS / LS / OS / SR | Check that the fixed text message is displayed on DMI. | - | - | Check that Packet 76 received in the Message 24 has the same values that are specified in the TC of the USE_FT3120300.9 | L2 / L3 | FS / LS / OS / SR | - |
| 34 | L2 / L3 | FS / LS / OS / SR | Check that the plain text message is displayed on DMI. | - | - | Check that Packet 72 received in the Message 24 has the same values that are specified in the TC of the USE_FT3120300.1. | L2 / L3 | FS / LS / OS / SR | - |
| 35 | L2 / L3 | FS / LS / OS / SR | Train is moving. | I | INT | Train location: Train min safe rear end location = LRBG + D_RBCTR+L_TRAIN The minimum rear end of the train passes the location of the RBC transition | L2 / L3 | FS / LS / OS / SR | - |
| 36 | L2 / L3 | FS / LS / OS / SR | SA-DATA. Request with Euroradio message (NID_MESSAGE=136) containing packet 0 is transmitted | O | RTM | A radio message 136 (train position report) is sent to the Handing Over RBC. | L2 / L3 | FS / LS / OS / SR | - |
| 37 | L2 / L3 | FS / LS / OS / SR | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=136) is recorded | O | JRU | - | L2 / L3 | FS / LS / OS / SR | - |
| 38 | L2 / L3 | FS / LS / OS / SR | The on-board equipment receives an order to terminate the communication session from the RBC, it terminates the radio communication session: The radio message 24 (general message) containing an order to terminate the communication session (packet 42) is received from RBC and recorded in the JRU. The radio message 156 (termination of communication session) is sent by the on-board equipment to RBC and recorded in the JRU. The radio message 39 (acknowledgement of termination of communication session) is received from RBC and recorded in the JRU. The communication session with the Handing Over RBC is terminated and the safe connection is released. | - | - | USE_FT3050500.5: “Termination of communication session ordered by RBC.” | L2 / L3 | FS / LS / OS / SR | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General Message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 131 | RBC transition order |
| Q_DIR | 2 | FINITE VALUE | Valid for current train direction |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_RBCTR | 15 | FINITE VALUE | ≠ 0 |
| NID_C | 10 | FINITE VALUE | Valid for the train |
| NID_RBC | 14 | FINITE VALUE | Accepting RBC |
| NID_RADIO | 64 | FINITE VALUE | Accepting RBC |
| Q_SLEEPSESSION | 1 | 0 | Sleeping equipment don’t consider session management information (to avoid TIU influence) |

| Step 5: Radio Message 155 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 155 | Initiation of a communication session |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 1 | FINITE VALUE | - |

| Step 7: Radio Message 32 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 32 | Configuration Determination |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| M_VERSION | 7 | 010 0000 | Version 2.0 |

| Step 9: Radio Message 159 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 159 | Session established |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 1 | FINITE VALUE | - |

| Step 11: Radio Message 129 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 129 | Validated Train Data |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | < D_RBCTR |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | - |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | 0 / 1 / 2 / 5 / 7 / 11 | - |
| M_LEVEL | 3 | 3 / 4 | - |
| NID_PACKET | 8 | 11 | Validated Train Data |
| L_PACKET | 13 | FINITE VALUE | - |
| NC_CDTRAIN | 4 | FINITE VALUE | - |
| NC_TRAIN | 15 | FINITE VALUE | - |
| L_TRAIN | 12 | FINITE VALUE | - |
| V_MAXTRAIN | 7 | FINITE VALUE | - |
| M_LOADINGGAUGE | 8 | FINITE VALUE | - |
| M_AXLELOADCAT | 7 | FINITE VALUE | - |
| M_AIRTIGHT | 2 | FINITE VALUE | - |
| N_AXLE | 10 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_VOLTAGE(k) | 4 | FINITE VALUE | Identity of the traction system. |
| NID_CTRACTION(k) | 10 | FINITE VALUE | NID_CTRACTION(k) given only if M_VOLTAGE(k) ≠ 0. |
| N_ITER | 5 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | Type of National System available. |

| Step 13: Radio Message 8 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 8 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | Not: 4294967295 (unknown) |
| M_ACK | 1 | 0 | No acknowledgement required |
| NID_LRBG | 24 | FINITE VALUE | Not: 16777215 (unknown) |

| Step 15: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 72 | Plain Text Message |
| Q_DIR | 2 | FINITE VALUE | Valid for current train direction |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TEXTCLASS | 2 | FINITE VALUE | - |
| Q_TEXTDISPLAY | 1 | FINITE VALUE | Start/end events relation. |
| D_TEXTDISPLAY | 15 | FINITE VALUE | Start event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | Start event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | Start event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| L_TEXTDISPLAY | 15 | FINITE VALUE | End event. |
| T_TEXTDISPLAY | 10 | FINITE VALUE | End event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | End event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | End event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| Q_TEXTCONFIRM | 2 | FINITE VALUE | - |
| Q_CONFTEXTDISPLAY | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| Q_TEXTREPORT | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| NID_TEXTMESSAGE | 8 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_C | 10 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_RBC | 14 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| L_TEXT | 8 | FINITE VALUE | - |
| X_TEXT(L_TEXT) | 8 | FINITE VALUE | - |

| Step 17: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 68 | Track conditions |
| Q_DIR | 2 | FINITE VALUE | Valid for current train direction |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | FINITE VALUE | - |
| D_TRACKINIT | 15 | FINITE VALUE | Only if Q_TRACKINIT = 1. |
| D_TRACKCOND | 15 | FINITE VALUE | Only if Q_TRACKINIT = 0, D_TRACKCOND and the following variables follow. |
| L_TRACKCOND | 15 | FINITE VALUE | - |
| M_TRACKCOND | 4 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_TRACKCOND(k) | 15 | FINITE VALUE | - |
| L_TRACKCOND(k) | 15 | FINITE VALUE | - |
| M_TRACKCOND(k) | 4 | FINITE VALUE | - |

| Step 19: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 76 | Fixed Text Message |
| Q_DIR | 2 | FINITE VALUE | Valid for current train direction |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TEXTCLASS | 2 | FINITE VALUE | - |
| Q_TEXTDISPLAY | 1 | FINITE VALUE | Start/end events relation. |
| D_TEXTDISPLAY | 15 | FINITE VALUE | Start event condition. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | Start event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | Start event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| L_TEXTDISPLAY | 15 | FINITE VALUE | End event. |
| T_TEXTDISPLAY | 10 | FINITE VALUE | End event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | End event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | End event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| Q_TEXTCONFIRM | 2 | FINITE VALUE | - |
| Q_CONFTEXTDISPLAY | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| Q_TEXTREPORT | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0 |
| NID_TEXTMESSAGE | 8 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_C | 10 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_RBC | 14 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| Q_TEXT | 8 | FINITE VALUE | - |

| Step 22: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 72 | Plain Text Message |
| Q_DIR | 2 | FINITE VALUE | Valid for current train direction |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TEXTCLASS | 2 | FINITE VALUE | - |
| Q_TEXTDISPLAY | 1 | FINITE VALUE | Start/end events relation. |
| D_TEXTDISPLAY | 15 | FINITE VALUE | Start event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | Start event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | Start event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| L_TEXTDISPLAY | 15 | FINITE VALUE | End event. |
| T_TEXTDISPLAY | 10 | FINITE VALUE | End event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | End event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | End event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| Q_TEXTCONFIRM | 2 | FINITE VALUE | - |
| Q_CONFTEXTDISPLAY | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| Q_TEXTREPORT | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| NID_TEXTMESSAGE | 8 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_C | 10 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_RBC | 14 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| L_TEXT | 8 | FINITE VALUE | - |
| X_TEXT(L_TEXT) | 8 | FINITE VALUE | - |

| Step 25: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | Max safe front end is considered. D_RBCTR - L_DOUBTUNDER |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | - |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | 0 / 1 / 2 / 5 / 7 / 11 | - |
| M_LEVEL | 3 | 3 / 4 | - |

| Step 27: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | Max safe front end is considered. D_RBCTR - L_DOUBTUNDER |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | - |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | 0 / 1 / 2 / 5 / 7 / 11 | - |
| M_LEVEL | 3 | 3 / 4 | - |

| Step 29: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 010 0000 | Version 2.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | First balise |
| N_TOTAL | 3 | FINITE VALUE | N - 1. N balises (N >= 2) |
| M_DUP | 2 | 00 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | Valid for the train |
| NID BG | 14 | FINITE VALUE | BG 1 |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 131 | RBC transition order |
| Q_DIR | 2 | FINITE VALUE | Valid for current train direction |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_RBCTR | 15 | 0 | - |
| NID_C | 10 | FINITE VALUE | Valid for the train |
| NID_RBC | 14 | FINITE VALUE | Accepting RBC |
| NID_RADIO | 64 | FINITE VALUE | Accepting RBC |
| Q_SLEEPSESSION | 1 | 0 | Sleeping equipment don’t consider session management information (to avoid TIU influence) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 29: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 010 0000 | Version 2.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 001 | Second balise |
| N_TOTAL | 3 | FINITE VALUE | N – 1. N balises (N >= 2) |
| M_DUP | 2 | 00 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID BG | 14 | FINITE VALUE | BG 1 |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 36: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | BG 1 |
| D_LRBG | 15 | FINITE VALUE | Min safe rear end is considered. D_RBCTR + L_TRAIN + L_DOUBTOVER |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | - |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Radio communication session | ESTABLISHED | With Accepting RBC |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Radio link to Accepting RBC |
| NOT RELEVANT | - | TIU | - |
| Track condition is DISPLAYED. Fixed Text is DISPLAYED. Second Plain text message is DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 9

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.5: Handling of transition buffer in case of level transition announcement or RBC/RBC handover |  | 4080500 |
| Test Case of Feature | Handling of transition buffer in case of RBC/RBC handover. RBC transition order has been deleted. Sets of information stored in the transition buffer shall be deleted. |  | 9 |
| Applicable Mode/Level Combinations | L2 / L3: FS/ LS/ OS |  |  |
| Target of Test | The target of this test is to check that: If the RBC transition order is deleted then all the information stored in the transition buffer is deleted |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.5.2 |  |
|  |  | Subset-026-4.8.5.4 |  |
|  |  | Subset-026-4.8.5.4 b) |  |
|  |  | Subset-026-7.4.2.27 Packet Number 131: RBC transition order |  |
|  |  | Subset-026-7.4.3.1 Packet Number 0: Position Report |  |
|  |  | Subset-026-7.4.3.5 Packet Number 11: Validated train data |  |
|  |  | Subset-026-7.4.2.23 Packet Number 72: Packet for sending plain text messages |  |
|  |  | Subset-026-7.4.2.20 Packet Number 68: Track Condition |  |
|  |  | Subset-026-7.4.2.24 Packet Number 76: Packet for sending fixed text messages |  |
|  |  | Subset-026-8.6.13 Message 155: Initiation of a communication session |  |
|  |  | Subset-026-8.7.12 Message 32: RBC/RIU System Version |  |
|  |  | Subset-026-8.6.17 Message 159: Session established |  |
|  |  | Subset-026-8.6.1 Message 129: Validated Train Data |  |
|  |  | Subset-026-8.7.4 Message 8: Acknowledgement of Train Data |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |
|  |  | Subset-026-8.7.7 Message 16: Unconditional Emergency Stop |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on RTM that: A radio message 24 (General message) containing packets 72, 68 and 76 is received from the Accepting RBC. A radio message 16 (Unconditional Emergency Stop) is received from the Accepting RBC. Check on the DMI that: Train changes to SR mode (All sets of information stored in the transition buffer are DELETED). Check on JRU that: RBC Messages 16 (Unconditional Emergency Stop) and 24 (including packets 72, 68 and 76), are RECORDED. The radio message 129 with packet 0 and 11 sent to the Accepting RBC is recorded. The mode change to SR is RECORDED. New RBC transition order is recorded. |
| Constraints | The on-board equipment is able to manage two communication sessions. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | With Handing Over RBC |
| Position Report parameters | STORED | Handing over RBC |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | With Handing Over RBC |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2 / L3 | FS / LS / OS | SA-DATA. Indication with Euroradio Message "General Message" (NID_MESSAGE=24) containing packet 131 is received. | I | RTM | Received from the Handing Over RBC. D_RBCTR ≠ 0 | L2 / L3 | FS / LS / OS | - |
| 2 | L2 / L3 | FS / LS / OS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=3) is recorded | O | JRU | - | L2 / L3 | FS / LS / OS | - |
| 3 | L2 / L3 | FS / LS / OS | SA-CONNECT. Request is transmitted | O | RTM | A safe connection is established with the Accepting RBC | L2 / L3 | FS / LS / OS | - |
| 4 | L2 / L3 | FS / LS / OS | SA-CONNECT. Indication is received | I | RTM | - | L2 / L3 | FS / LS / OS | - |
| 5 | L2 / L3 | FS / LS / OS | SA-DATA. Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted | O | RTM | - | L2 / L3 | FS / LS / OS | - |
| 6 | L2 / L3 | FS / LS / OS | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=155) is recorded | O | JRU | - | L2 / L3 | FS / LS / OS | - |
| 7 | L2 / L3 | FS / LS / OS | SA-DATA. Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32) containing is received | I | RTM | The Version of the ERTMS/ETCS language is valid for the train and M_VERSION <> { 000 XXXX, 111 1111} | L2 / L3 | FS / LS / OS | - |
| 8 | L2 / L3 | FS / LS / OS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=32) is recorded | O | JRU | - | L2 / L3 | FS / LS / OS | - |
| 9 | L2 / L3 | FS / LS / OS | SA-DATA. Request with Euroradio Message "Session Established" (NID_MESSAGE=159) is transmitted | O | RTM | The communication session with the Accepting RBC is established | L2 / L3 | FS / LS / OS | - |
| 10 | L2 / L3 | FS / LS / OS | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=159) is recorded | O | JRU | - | L2 / L3 | FS / LS / OS | - |
| 11 | L2 / L3 | FS / LS / OS | SA-DATA. Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) is transmitted | O | RTM | The radio message contains also position report data. | L2 / L3 | FS / LS / OS | - |
| 12 | L2 / L3 | FS / LS / OS | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=129) is recorded | O | JRU | - | L2 / L3 | FS / LS / OS | - |
| 13 | L2 / L3 | FS / LS / OS | SA-DATA. Indication with Euroradio Message "Acknowledgement of Train Data" (NID_MESSAGE=8) is received | I | RTM | - | L2 / L3 | FS / LS / OS | - |
| 14 | L2 / L3 | FS / LS / OS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=8) is recorded | O | JRU | - | L2 / L3 | FS / LS / OS | - |
| 15 | L2 / L3 | FS / LS / OS | SA-DATA. Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 72, packet 68, packet 76 is received | I | RTM | The train receives Message 24 from the Accepting RBC (General Message) including this sequence of packets: Packet 72: Plain Text Message Packet 68: Track Condition Packet 76: Fixed Text Message | L2 / L3 | FS / LS / OS | - |
| 16 | L2 / L3 | FS / LS / OS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=24) is recorded | O | JRU | - | L2 / L3 | FS / LS / OS | - |
| 17 | L2 / L3 | FS / LS / OS | Any text message is displayed at this location. | O | DMI | - | L2 / L3 | FS / LS / OS | - |
| 18 | L2 / L3 | FS / LS / OS | SA-DATA. Indication with Euroradio Message "Unconditional Emergency Stop" (NID_MESSAGE=16) is received | I | RTM | The ERTMS/ETCS on-board equipment stores the two sets of information in a transition buffer. | L2 / L3 | FS / LS / OS | - |
| 19 | L2 / L3 | FS / LS / OS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=16) is recorded | O | JRU | - | L2 / L3 | FS / LS / OS | - |
| 20 | L2 / L3 | FS / LS / OS | Mode transition to SR | - | - | Use_FT4060300.56 | L2 / L3 | SR | - |
| 21 | L2 / L3 | SR | SA-DATA. Indication with Euroradio Message "General Message" (NID_MESSAGE=24) containing packet 131 is received. | I | RTM | Received from the Handing Over RBC. D_RBCTR ≠ 0. A new RBC transition order is received at same location. The previous one is deleted due to change to SR mode | L2 / L3 | SR | - |
| 22 | L2 / L3 | SR | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=3) is recorded | O | JRU | - | L2 / L3 | SR | - |
| 23 | L2 / L3 | SR | Train is moving. | I | INT | Train Location: Train maximum safe front end location = LRBG + D_RBCTR The maximum safe front end of the train passes the location of the RBC transition. The board is supervised by the Accepting RBC. | L2 / L3 | SR | - |
| 24 | L2 / L3 | SR | SA-DATA. Request with Euroradio message (NID_MESSAGE=136) containing packet 0 is transmitted | O | RTM | A radio message 136 (train position report) is sent to the Handing Over RBC. | L2 / L3 | SR | - |
| 25 | L2 / L3 | SR | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=136) is recorded | O | JRU | - | L2 / L3 | SR | - |
| 26 | L2 / L3 | SR | SA-DATA. Request with Euroradio message (NID_MESSAGE=136) containing packet 0 is transmitted | O | RTM | A radio message 136 (train position report) is sent to the Accepting RBC. | L2 / L3 | SR | - |
| 27 | L2 / L3 | SR | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=136) is recorded | O | JRU | - | L2 / L3 | SR | - |
| 28 | L2 / L3 | SR | NOT The Mode symbol "Trip" is displayed | O | DMI | Unconditional emergency stop has been deleted | L2 / L3 | SR | - |
| 29 | L2 / L3 | SR | NOT The "plain text message" is displayed | O | DMI | No Plain text message is displayed when it is reached the location where/when the starting condition(s) related to plain text message are fulfilled | L2 / L3 | SR | - |
| 30 | L2 / L3 | SR | NOT The "fixed text message" is displayed | O | DMI | No Fixed text message is displayed when it is reached the location where/when the starting condition(s) related to fixed text message are fulfilled | L2 / L3 | SR | - |
| 31 | L2 / L3 | SR | NOT The Track Condition symbol is displayed | O | DMI | - | L2 / L3 | SR | - |
| 32 | L2 / L3 | SR | The on-board equipment receives an order to terminate the communication session from the RBC, it terminates the radio communication session: The radio message 24 (general message) containing an order to terminate the communication session (packet 42) is received from RBC and recorded in the JRU. The radio message 156 (termination of communication session) is sent by the on-board equipment to RBC and recorded in the JRU. The radio message 39 (acknowledgement of termination of communication session) is received from RBC and recorded in the JRU. The communication session with the Handing Over RBC is terminated and the safe connection is released. | - | - | USE_FT3050500.5: “Termination of communication session ordered by RBC.” | L2 / L3 | SR | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General Message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 131 | RBC transition order |
| Q_DIR | 2 | FINITE VALUE | Valid for current train direction |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_RBCTR | 15 | FINITE VALUE | ≠ 0 D_RBCTR(1) |
| NID_C | 10 | FINITE VALUE | Valid for the train |
| NID_RBC | 14 | FINITE VALUE | Accepting RBC |
| NID_RADIO | 64 | FINITE VALUE | Accepting RBC |
| Q_SLEEPSESSION | 1 | 0 | Sleeping equipment don’t consider session management information (to avoid TIU influence) |

| Step 5: Radio Message 155 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 155 | Initiation of a communication session |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 1 | FINITE VALUE | - |

| Step 7: Radio Message 32 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 32 | Configuration Determination |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| M_VERSION | 7 | 010 0000 | Version 2.0 |

| Step 9: Radio Message 159 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 159 | Session established |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 1 | FINITE VALUE | - |

| Step 11: Radio Message 129 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 129 | Validated Train Data |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | < D_RBCTR |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | - |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | 0 / 1 / 2 / 5 / 7 / 11 | - |
| M_LEVEL | 3 | 3 / 4 | - |
| NID_PACKET | 8 | 11 | Validated Train Data |
| L_PACKET | 13 | FINITE VALUE | - |
| NC_CDTRAIN | 4 | FINITE VALUE | - |
| NC_TRAIN | 15 | FINITE VALUE | - |
| L_TRAIN | 12 | FINITE VALUE | - |
| V_MAXTRAIN | 7 | FINITE VALUE | - |
| M_LOADINGGAUGE | 8 | FINITE VALUE | - |
| M_AXLELOADCAT | 7 | FINITE VALUE | - |
| M_AIRTIGHT | 2 | FINITE VALUE | - |
| N_AXLE | 10 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_VOLTAGE(k) | 4 | FINITE VALUE | Identity of the traction system. |
| NID_CTRACTION(k) | 10 | FINITE VALUE | NID_CTRACTION(k) given only if M_VOLTAGE(k) ≠ 0. |
| N_ITER | 5 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | Type of National System available. |

| Step 13: Radio Message 8 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 8 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | Not: 4294967295 (unknown) |
| M_ACK | 1 | 0 | No acknowledgement required |
| NID_LRBG | 24 | FINITE VALUE | Not: 16777215 (unknown) |

| Step 15: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 72 | Plain Text Message |
| Q_DIR | 2 | FINITE VALUE | Valid for current train direction |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TEXTCLASS | 2 | FINITE VALUE | - |
| Q_TEXTDISPLAY | 1 | FINITE VALUE | Start/end events relation. |
| D_TEXTDISPLAY | 15 | FINITE VALUE | Start event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | Start event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | Start event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| L_TEXTDISPLAY | 15 | FINITE VALUE | End event. |
| T_TEXTDISPLAY | 10 | FINITE VALUE | End event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | End event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | End event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| Q_TEXTCONFIRM | 2 | FINITE VALUE | - |
| Q_CONFTEXTDISPLAY | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| Q_TEXTREPORT | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| NID_TEXTMESSAGE | 8 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_C | 10 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_RBC | 14 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| L_TEXT | 8 | FINITE VALUE | - |
| X_TEXT(L_TEXT) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 68 | Track conditions |
| Q_DIR | 2 | FINITE VALUE | Valid for current train direction |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | FINITE VALUE | - |
| D_TRACKINIT | 15 | FINITE VALUE | Only if Q_TRACKINIT = 1. |
| D_TRACKCOND | 15 | FINITE VALUE | Only if Q_TRACKINIT = 0, D_TRACKCOND and the following variables follow. |
| L_TRACKCOND | 15 | FINITE VALUE | - |
| M_TRACKCOND | 4 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_TRACKCOND(k) | 15 | FINITE VALUE | - |
| L_TRACKCOND(k) | 15 | FINITE VALUE | - |
| M_TRACKCOND(k) | 4 | FINITE VALUE | - |
| NID_PACKET | 8 | 76 | Fixed Text Message |
| Q_DIR | 2 | FINITE VALUE | Valid for current train direction |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TEXTCLASS | 2 | FINITE VALUE | - |
| Q_TEXTDISPLAY | 1 | FINITE VALUE | Start/end events relation. |
| D_TEXTDISPLAY | 15 | FINITE VALUE | Start event condition. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | Start event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | Start event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| L_TEXTDISPLAY | 15 | FINITE VALUE | End event. |
| T_TEXTDISPLAY | 10 | FINITE VALUE | End event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | End event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | End event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| Q_TEXTCONFIRM | 2 | FINITE VALUE | - |
| Q_CONFTEXTDISPLAY | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| Q_TEXTREPORT | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0 |
| NID_TEXTMESSAGE | 8 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_C | 10 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_RBC | 14 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| Q_TEXT | 8 | FINITE VALUE | - |

| Step 18: Radio Message 16 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 16 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | 1 | Acknowledgement required |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_EM | 4 | FINITE VALUE | Emergency Stop Identification Number |

| Step 21: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General Message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 131 | RBC transition order |
| Q_DIR | 2 | FINITE VALUE | Valid for current train direction |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_RBCTR | 15 | FINITE VALUE | = D_RBCTR(1) |
| NID_C | 10 | FINITE VALUE | Valid for the train |
| NID_RBC | 14 | FINITE VALUE | Accepting RBC |
| NID_RADIO | 64 | FINITE VALUE | Accepting RBC |
| Q_SLEEPSESSION | 1 | 0 | Sleeping equipment don’t consider session management information (to avoid TIU influence) |

| Step 24: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | BG1 |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | - |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | 4 | - |
| M_LEVEL | 3 | 3 | - |

| Step 26: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | BG1 |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | - |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | 4 | - |
| M_LEVEL | 3 | 3/4 | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 | SR |
| Radio communication session | ESTABLISHED | With Accepting RBC |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | With Accepting RBC |
| NOT RELEVANT | - | TIU | - |
| SR mode is DISPLAYED. No Plain text message is displayed when it is reached the location where/when the starting condition(s) related to plain text message are fulfilled No Fixed text message is displayed when it is reached the location where/when the starting condition(s) related to fixed text message are fulfilled No announcement of track condition is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 10

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.5: Handling of transition buffer in case of level transition announcement or RBC/RBC handover |  | 4080500 |
| Test Case of Feature | Handling of transition buffer in case of RBC/RBC handover. RBC transition order has been received and overwritten by an order to switch to another Accepting RBC. Sets of information stored in the transition buffer shall be deleted. |  | 10 |
| Applicable Mode/Level Combinations | L2 / L3: FS/ LS/ OS/ SR |  |  |
| Target of Test | The target of this test is to check that: If the RBC transition order is overwritten then all the information stored in the transition buffer is deleted |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.5.2 |  |
|  |  | Subset-026-4.8.5.4 |  |
|  |  | Subset-026-4.8.5.4 b) |  |
|  |  | Subset-026-7.4.3.1 Packet Number 0: Position Report |  |
|  |  | Subset-026-7.4.3.5 Packet Number 11: Validated train data |  |
|  |  | Subset-026-7.4.2.23 Packet Number 72: Packet for sending plain text messages |  |
|  |  | Subset-026-7.4.2.24 Packet Number 76: Packet for sending fixed text messages |  |
|  |  | Subset-026-7.4.2.27 Packet Number 131: RBC transition order |  |
|  |  | Subset-026-8.6.13 Message 155: Initiation of a communication session |  |
|  |  | Subset-026-8.7.12 Message 32: RBC/RIU System Version |  |
|  |  | Subset-026-8.6.17 Message 159: Session established |  |
|  |  | Subset-026-8.6.1 Message 129: Validated Train Data |  |
|  |  | Subset-026-8.7.4 Message 8: Acknowledgement of Train Data |  |
|  |  | Subset-026-8.7.9 Message 24: General Message |  |
|  |  | Subset-026-8.7.7 Message 16: Unconditional Emergency Stop |  |
|  |  | Subset-026-8.6.4 Message 136: Train Position Report |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on RTM that: A radio message 16 (Unconditional Emergency Stop) is received from the Accepting RBC A. Train establish the communication session with RBC B.. Check on the DMI that: Train is not tripped Check on JRU that: RBC Messages 16 (Unconditional Emergency Stop) is RECORDED. The radio message 129 with packet 0 and 11 sent to the Accepting RBC is recorded. |
| Constraints | The on-board equipment is able to manage two communication sessions at a time. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Radio communication session | ESTABLISHED | With handing over RBC |
| Position Report parameters | STORED | Handing over RBC |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | With Handing Over RBC |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2 / L3 | FS / LS / OS / SR | SA-DATA. Indication with Euroradio Message "General Message" (NID_MESSAGE=24) containing packet 131 is received. | I | RTM | Received from the Handing Over RBC. D_RBCTR ≠ 0. | L2 / L3 | FS / LS / OS / SR | - |
| 2 | L2 / L3 | FS / LS / OS / SR | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=24) is recorded | O | JRU | - | L2 / L3 | FS / LS / OS / SR | - |
| 3 | L2 / L3 | FS / LS / OS / SR | SA-CONNECT. Request is transmitted | O | RTM | A connect request to the Accepting RBC A is sent. A safe connection is established with the Accepting RBC A | L2 / L3 | FS / LS / OS / SR | - |
| 4 | L2 / L3 | FS / LS / OS / SR | SA-CONNECT. Indication is received | I | RTM | A connection indication from the Accepting RBC A is received on board | L2 / L3 | FS / LS / OS / SR | - |
| 5 | L2 / L3 | FS / LS / OS / SR | SA-DATA. Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted | O | RTM | A radio message 155 (initiation of communication session) is sent to the Accepting RBC A | L2 / L3 | FS / LS / OS / SR | - |
| 6 | L2 / L3 | FS / LS / OS / SR | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=155) is recorded | O | JRU | - | L2 / L3 | FS / LS / OS / SR | - |
| 7 | L2 / L3 | FS / LS / OS / SR | SA-DATA. Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32) is received | I | RTM | A radio message 32 (configuration determination) is received from the Accepting RBC A. The Version of the ERTMS/ETCS language is valid for the train and M_VERSION <> { 000 XXXX, 111 1111} | L2 / L3 | FS / LS / OS / SR | - |
| 8 | L2 / L3 | FS / LS / OS / SR | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=32) is recorded | O | JRU | - | L2 / L3 | FS / LS / OS / SR | - |
| 9 | L2 / L3 | FS / LS / OS / SR | SA-DATA. Request with Euroradio Message "Session Established" (NID_MESSAGE=159) is transmitted | O | RTM | The communication session with the Accepting RBC A is established | L2 / L3 | FS / LS / OS / SR | - |
| 10 | L2 / L3 | FS / LS / OS / SR | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=159) is recorded | O | JRU | - | L2 / L3 | FS / LS / OS / SR | - |
| 11 | L2 / L3 | FS / LS / OS / SR | SA-DATA. Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) containing packet 0 and packet 11 is transmitted | O | RTM | A radio message 129 (Validated train data) is sent to the Accepting RBC A. The radio message contains also position report data. | L2 / L3 | FS / LS / OS / SR | - |
| 12 | L2 / L3 | FS / LS / OS / SR | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=129) is recorded | O | JRU | - | L2 / L3 | FS / LS / OS / SR | - |
| 13 | L2 / L3 | FS / LS / OS / SR | SA-DATA. Indication with Euroradio Message "Acknowledgement of Train Data" (NID_MESSAGE=8) is received | I | RTM | - | L2 / L3 | FS / LS / OS / SR | - |
| 14 | L2 / L3 | FS / LS / OS / SR | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=8) is recorded | O | JRU | - | L2 / L3 | FS / LS / OS / SR | - |
| 15 | L2 / L3 | FS / LS / OS / SR | SA-DATA .Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 72 is received | I | RTM | Packet 72 (Plain Text Message) | L2 / L3 | FS / LS / OS / SR | - |
| 16 | L2 / L3 | FS / LS / OS / SR | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=24) is recorded | O | JRU | - | L2 / L3 | FS / LS / OS / SR | - |
| 17 | L2 / L3 | FS / LS / OS / SR | The train receives a Message 24 from the Accepting RBC (General Message) including one Packet 76 (Fixed Text Message) | I | RTM | Packet 76 (Fixed Text Message) | L2 / L3 | FS / LS / OS / SR | - |
| 18 | L2 / L3 | FS / LS / OS / SR | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=24) is recorded | O | JRU | - | L2 / L3 | FS / LS / OS / SR | - |
| 19 | L2 / L3 | FS / LS / OS / SR | SA-DATA .Indication with Euroradio Message "Unconditional Emergency Stop" (NID_MESSAGE=16) is received | I | RTM | - | L2 / L3 | FS / LS / OS / SR | - |
| 20 | L2 / L3 | FS / LS / OS / SR | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=16) is recorded | O | JRU | The ERTMS/ETCS on-board equipment stores the three sets of information in a transition buffer. | L2 / L3 | FS / LS / OS / SR | - |
| 21 | L2 / L3 | FS / LS / OS / SR | Train is moving. | I | INT | Train location: Before reaching the RBC/RBC handover area | L2 / L3 | FS / LS / OS / SR | - |
| 22 | L2 / L3 | FS / LS / OS / SR | SA-DATA. Indication with Euroradio Message "General Message" (NID_MESSAGE=24) containing packet 131 is received | I | RTM | A new RBC transition order is received from the Handing Over RBC. D_RBCTR ≠ 0 The radio message shall contain a movement authority including the new RBC B transition border, only if in FS mode. | L2 / L3 | FS / LS / OS / SR | - |
| 23 | L2 / L3 | FS / LS / OS / SR | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=24) is recorded | O | JRU | - | L2 / L3 | FS / LS / OS / SR | - |
| 24 | L2 / L3 | FS / LS / OS / SR | The on-board equipment receives an order to terminate the communication session from the RBC, it terminates the radio communication session: The radio message 24 (general message) containing an order to terminate the communication session (packet 42) is received from RBC and recorded in the JRU. The radio message 156 (termination of communication session) is sent by the on-board equipment to RBC and recorded in the JRU. The radio message 39 (acknowledgement of termination of communication session) is received from RBC and recorded in the JRU. The communication session with the Accepting RBC A is terminated and the safe connection is released. | - | - | USE_FT3050500.5: “Termination of communication session ordered by RBC.” | L2 / L3 | FS / LS / OS / SR | - |
| 25 | L2 / L3 | FS / LS / OS / SR | SA-CONNECT. Request is transmitted | O | RTM | A connect request to the Accepting RBC B is sent. A safe connection is established with the Accepting RBC B | L2 / L3 | FS / LS / OS / SR | - |
| 26 | L2 / L3 | FS / LS / OS / SR | SA-CONNECT .Indication is received | I | RTM | A connection indication from the Accepting RBC B is received on board | L2 / L3 | FS / LS / OS / SR | - |
| 27 | L2 / L3 | FS / LS / OS / SR | SA-DATA. Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted | O | RTM | A radio message 155 (initiation of communication session) is sent to the Accepting RBC B. | L2 / L3 | FS / LS / OS / SR | - |
| 28 | L2 / L3 | FS / LS / OS / SR | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=155) is recorded | O | JRU | - | L2 / L3 | FS / LS / OS / SR | - |
| 29 | L2 / L3 | FS / LS / OS / SR | SA-DATA .Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32) is received | I | RTM | A radio message 32 (configuration determination) is received from the Accepting RBC B. The Version of the ERTMS/ETCS language is valid for the train and M_VERSION <> { 000 XXXX, 111 1111} | L2 / L3 | FS / LS / OS / SR | - |
| 30 | L2 / L3 | FS / LS / OS / SR | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=32) is recorded | O | JRU | - | L2 / L3 | FS / LS / OS / SR | - |
| 31 | L2 / L3 | FS / LS / OS / SR | SA-DATA. Request with Euroradio Message "Session Established" (NID_MESSAGE=159) is transmitted | O | RTM | A radio message 159 (session established) is sent to the Accepting RBC B. The communication session with the Accepting RBC B is established | L2 / L3 | FS / LS / OS / SR | - |
| 32 | L2 / L3 | FS / LS / OS / SR | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=159) is recorded | O | JRU | - | L2 / L3 | FS / LS / OS / SR | - |
| 33 | L2 / L3 | FS / LS / OS / SR | SA-DATA. Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) containing packet 0 and packet 11 is transmitted | O | RTM | A radio message 129 (Validated train data) is sent to the Accepting RBC B. The radio message contains also position report data. | L2 / L3 | FS / LS / OS / SR | - |
| 34 | L2 / L3 | FS / LS / OS / SR | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=129) is recorded | O | JRU | - | L2 / L3 | FS / LS / OS / SR | - |
| 35 | L2 / L3 | FS / LS / OS / SR | SA-DATA .Indication with Euroradio Message "Acknowledgement of Train Data" (NID_MESSAGE=8) is received | I | RTM | - | L2 / L3 | FS / LS / OS / SR | - |
| 36 | L2 / L3 | FS / LS / OS / SR | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=8) is recorded | O | JRU | - | L2 / L3 | FS / LS / OS / SR | - |
| 37 | L2 / L3 | FS / LS / OS / SR | Train is moving. | I | INT | Train location: Train maximum safe front end location = LRBG + D_RBCTR The maximum safe front end of the train passes the location of the RBC transition | L2 / L3 | FS / LS / OS / SR | - |
| 38 | L2 / L3 | FS / LS / OS / SR | SA-DATA. Request with Euroradio message (NID_MESSAGE=136) containing packet 0 is transmitted | O | RTM | A radio message 136 (train position report) is sent to the Handing Over RBC. The On board is supervised by the Accepting RBC B. | L2 / L3 | FS / LS / OS / SR | - |
| 39 | L2 / L3 | FS / LS / OS / SR | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=136) is recorded | O | JRU | - | L2 / L3 | FS / LS / OS / SR | - |
| 40 | L2 / L3 | FS / LS / OS / SR | SA-DATA. Request with Euroradio message (NID_MESSAGE=136) containing packet 0 is transmitted | O | RTM | A radio message 136 (train position report) is sent to the Accepting RBC B. The On board is supervised by the Accepting RBC B. | L2 / L3 | FS / LS / OS / SR | - |
| 41 | L2 / L3 | FS / LS / OS / SR | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=136) is recorded | O | JRU | - | L2 / L3 | FS / LS / OS / SR | - |
| 42 | L2 / L3 | FS / LS / OS / SR | NOT The Mode symbol "Trip" is displayed | O | DMI | Unconditional emergency stop has been deleted | L2 / L3 | FS / LS / OS / SR | - |
| 43 | L2 / L3 | FS / LS / OS / SR | NOT The "plain text message" is displayed | O | DMI | No Plain text message is displayed when it is reached the location where/when the starting condition(s) related to plain text message are fulfilled | L2 / L3 | FS / LS / OS / SR | - |
| 44 | L2 / L3 | FS / LS / OS / SR | NOT The "fixed text message" is displayed | O | DMI | No Fixed text message is displayed when it is reached the location where/when the starting condition(s) related to fixed text message are fulfilled | L2 / L3 | FS / LS / OS / SR | - |
| 45 | L2 / L3 | FS / LS / OS / SR | One Balise Group message (N_TOTAL=1) containing packet 131 is received | I | BTM | The balise group is placed at the RBC transition border. | L2 / L3 | FS / LS / OS / SR | - |
| 46 | L2 / L3 | FS / LS / OS / SR | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L2 / L3 | FS / LS / OS / SR | - |
| 47 | L2 / L3 | FS / LS / OS / SR | Train is moving. | I | INT | Train min safe rear end location = LRBG + D_RBCTR+L_TRAIN The minimum rear end of the train passes the location of the RBC transition | L2 / L3 | FS / LS / OS / SR | - |
| 48 | L2 / L3 | FS / LS / OS / SR | SA-DATA. Request with Euroradio message (NID_MESSAGE=136) containing packet 0 is transmitted | O | RTM | A radio message 136 (train position report) is sent to the Handing Over RBC. | L2 / L3 | FS / LS / OS / SR | - |
| 49 | L2 / L3 | FS / LS / OS / SR | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=136) is recorded | O | JRU | - | L2 / L3 | FS / LS / OS / SR | - |
| 50 | L2 / L3 | FS / LS / OS / SR | The on-board equipment receives an order to terminate the communication session from the RBC, it terminates the radio communication session: The radio message 24 (general message) containing an order to terminate the communication session (packet 42) is received from RBC and recorded in the JRU. The radio message 156 (termination of communication session) is sent by the on-board equipment to RBC and recorded in the JRU. The radio message 39 (acknowledgement of termination of communication session) is received from RBC and recorded in the JRU. The communication session with the Handing Over RBC is terminated and the safe connection is released. | - | - | USE_FT3050500.5: “Termination of communication session ordered by RBC.” | L2 / L3 | FS / LS / OS / SR | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General Message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 131 | RBC A transition order |
| Q_DIR | 2 | FINITE VALUE | Valid for current train direction |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_RBCTR | 15 | FINITE VALUE | ≠ 0 |
| NID_C | 10 | FINITE VALUE | Valid for the train |
| NID_RBC | 14 | FINITE VALUE | Accepting RBC A |
| NID_RADIO | 64 | FINITE VALUE | Accepting RBC A |
| Q_SLEEPSESSION | 1 | 0 | Sleeping equipment don’t consider session management information (to avoid TIU influence) |

| Step 5: Radio Message 155 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 155 | Initiation of a communication session with RBC A |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 1 | FINITE VALUE | - |

| Step 7: Radio Message 32 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 32 | Configuration Determination |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| M_VERSION | 7 | 010 0000 | Version 2.0 |

| Step 9: Radio Message 159 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 159 | Session established with RBC A |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 1 | FINITE VALUE | - |

| Step 11: Radio Message 129 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 129 | Validated Train Data |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | < D_RBCTR |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | - |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | 0 / 1 / 2 / 5 / 7 / 11 | - |
| M_LEVEL | 3 | 3 / 4 | - |
| NID_PACKET | 8 | 11 | Validated Train Data |
| L_PACKET | 13 | FINITE VALUE | - |
| NC_CDTRAIN | 4 | FINITE VALUE | - |
| NC_TRAIN | 15 | FINITE VALUE | - |
| L_TRAIN | 12 | FINITE VALUE | - |
| V_MAXTRAIN | 7 | FINITE VALUE | - |
| M_LOADINGGAUGE | 8 | FINITE VALUE | - |
| M_AXLELOADCAT | 7 | FINITE VALUE | - |
| M_AIRTIGHT | 2 | FINITE VALUE | - |
| N_AXLE | 10 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_VOLTAGE(k) | 4 | FINITE VALUE | Identity of the traction system. |
| NID_CTRACTION(k) | 10 | FINITE VALUE | NID_CTRACTION(k) given only if M_VOLTAGE(k) ≠ 0. |
| N_ITER | 5 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | Type of National System available. |

| Step 13: Radio Message 8 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 8 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | Not: 4294967295 (unknown) |
| M_ACK | 1 | 0 | No acknowledgement required |
| NID_LRBG | 24 | FINITE VALUE | Not: 16777215 (unknown) |

| Step 15: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 72 | Plain Text Message |
| Q_DIR | 2 | FINITE VALUE | Valid for current train direction |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TEXTCLASS | 2 | FINITE VALUE | - |
| Q_TEXTDISPLAY | 1 | FINITE VALUE | Start/end events relation. |
| D_TEXTDISPLAY | 15 | FINITE VALUE | Start event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | Start event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | Start event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| L_TEXTDISPLAY | 15 | FINITE VALUE | End event. |
| T_TEXTDISPLAY | 10 | FINITE VALUE | End event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | End event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | End event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| Q_TEXTCONFIRM | 2 | FINITE VALUE | - |
| Q_CONFTEXTDISPLAY | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| Q_TEXTREPORT | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| NID_TEXTMESSAGE | 8 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_C | 10 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_RBC | 14 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| L_TEXT | 8 | FINITE VALUE | - |
| X_TEXT(L_TEXT) | 8 | FINITE VALUE | - |

| Step 17: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 76 | Plain Text Message |
| Q_DIR | 2 | FINITE VALUE | Valid for current train direction |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TEXTCLASS | 2 | FINITE VALUE | - |
| Q_TEXTDISPLAY | 1 | FINITE VALUE | Start/end events relation. |
| D_TEXTDISPLAY | 15 | FINITE VALUE | Start event condition. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | Start event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | Start event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| L_TEXTDISPLAY | 15 | FINITE VALUE | End event. |
| T_TEXTDISPLAY | 10 | FINITE VALUE | End event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | End event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | End event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| Q_TEXTCONFIRM | 2 | FINITE VALUE | - |
| Q_CONFTEXTDISPLAY | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| Q_TEXTREPORT | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0 |
| NID_TEXTMESSAGE | 8 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_C | 10 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_RBC | 14 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| Q_TEXT | 8 | FINITE VALUE | - |

| Step 19: Radio Message 16 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 16 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | 1 | Acknowledgement required |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_EM | 4 | FINITE VALUE | Emergency Stop Identification Number |

| Step 22: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General Message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | 0 | To avoid a message 146 |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 131 | RBC B transition order |
| Q_DIR | 2 | FINITE VALUE | Valid for current train direction |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_RBCTR | 15 | FINITE VALUE | ≠ 0 |
| NID_C | 10 | FINITE VALUE | Valid for the train |
| NID_RBC | 14 | FINITE VALUE | Accepting RBC B |
| NID_RADIO | 64 | FINITE VALUE | Accepting RBC B |
| Q_SLEEPSESSION | 1 | 0 | Sleeping equipment don’t consider session management information (to avoid TIU influence) |

| Step 27: Radio Message 155 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 155 | Initiation of a communication session with RBC B |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 1 | FINITE VALUE | - |

| Step 29: Radio Message 32 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 32 | Configuration Determination |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| M_VERSION | 7 | 010 0000 | Version 2.0 |

| Step 31: Radio Message 159 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 159 | Session established with RBC B |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 1 | FINITE VALUE | - |

| Step 33: Radio Message 129 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 129 | Validated Train Data |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | < D_RBCTR |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | - |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | 0 / 1 / 2 / 5 / 7 / 11 | - |
| M_LEVEL | 3 | 3 / 4 | - |
| NID_PACKET | 8 | 11 | Validated Train Data |
| L_PACKET | 13 | FINITE VALUE | - |
| NC_CDTRAIN | 4 | FINITE VALUE | - |
| NC_TRAIN | 15 | FINITE VALUE | - |
| L_TRAIN | 12 | FINITE VALUE | - |
| V_MAXTRAIN | 7 | FINITE VALUE | - |
| M_LOADINGGAUGE | 8 | FINITE VALUE | - |
| M_AXLELOADCAT | 7 | FINITE VALUE | - |
| M_AIRTIGHT | 2 | FINITE VALUE | - |
| N_AXLE | 10 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_VOLTAGE(k) | 4 | FINITE VALUE | Identity of the traction system. |
| NID_CTRACTION(k) | 10 | FINITE VALUE | NID_CTRACTION(k) given only if M_VOLTAGE(k) ≠ 0. |
| N_ITER | 5 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | Type of National System available. |

| Step 35: Radio Message 8 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 8 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | Not: 4294967295 (unknown) |
| M_ACK | 1 | 0 | No acknowledgement required |
| NID_LRBG | 24 | FINITE VALUE | Not: 16777215 (unknown) |

| Step 38: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | - |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |

| Step 40: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | - |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |

| Step 45: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | - |
| M_VERSION | 7 | 010 0000 | Version 2.0 |
| Q_MEDIA | 1 | 0 | - |
| N_PIG | 3 | FINITE VALUE | - |
| N_TOTAL | 3 | FINITE VALUE | - |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BG1 |
| Q_LINK | 1 | 1 | - |
| NID_PACKET | 8 | 131 | RBC Transition order |
| Q_DIR | 2 | FINITE VALUE | Valid for current train direction |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_RBCTR | 15 | 0 | - |
| NID_C | 10 | FINITE VALUE | Valid for the train |
| NID_RBC | 14 | FINITE VALUE | Accepting RBC B |
| NID_RADIO | 64 | FINITE VALUE | Accepting RBC B |
| Q_SLEEPSESSION | 1 | 0 | Sleeping equipment don’t consider session management information (to avoid TIU influence) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 48: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | BG1 |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | - |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Radio communication session | ESTABLISHED | With accepting RBC |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | With Accepting RBC B |
| NOT RELEVANT | - | TIU | - |
| No Plain text message is displayed when it is reached the location where/when the starting condition(s) related to plain text message are fulfilled No Fixed text message is displayed when it is reached the location where/when the starting condition(s) related to fixed text message are fulfilled | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 11

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.5: Handling of transition buffer in case of level transition announcement or RBC/RBC handover |  | 4080500 |
| Test Case of Feature | Handling of transition buffer in case of RBC/RBC handover. The sets of information stored in the transition buffer shall be deleted in case the communication session with the RBC that provides the information is terminated. |  | 11 |
| Applicable Mode/Level Combinations | L2 / L3: FS / LS / OS / SR |  |  |
| Target of Test | The target of this test is to check that: The communication session with the RBC that provided the stored information is terminated The set of information stored is deleted. |  |  |
| Version | 3.2.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.4.0 | Subset-026-4.8.5.2 |  |
|  |  | Subset-026-4.8.5.4 |  |
|  |  | Subset-026-4.8.5.4 c) |  |
|  |  | Subset-026-7.4.2.27 Packet Number 131: RBC transition order |  |
|  |  | Subset-026-7.4.3.1 Packet Number 0: Position Report |  |
|  |  | Subset-026-7.4.3.5 Packet Number 11: Validated train data |  |
|  |  | Subset-026-7.4.2.23 Packet Number 72: Packet for sending plain text messages |  |
|  |  | Subset-026-7.4.2.24 Packet Number 76: Packet for sending fixed text messages |  |
|  |  | Subset-026-8.6.13 Message 155: Initiation of a communication session |  |
|  |  | Subset-026-8.7.12 Message 32: RBC/RIU System Version |  |
|  |  | Subset-026-8.6.17 Message 159: Session established |  |
|  |  | Subset-026-8.6.1 Message 129: Validated Train Data |  |
|  |  | Subset-026-8.7.4 Message 8: Acknowledgement of Train Data |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |
|  |  | Subset-026-8.7.7 Message 16: Unconditional Emergency Stop |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: Train is not tripped Plain Text and Fixed text messages are NOT displayed. Check on JRU that: RBC Messages 16 (Unconditional Emergency Stop) received from the Accepting RBC A is RECORDED. The radio message 129 with packet 0 and 11 sent to the Accepting RBC is recorded. Termination of the communication session with the Accepting RBC is recorded. |
| Constraints | The on-board equipment is able to manage two communication sessions. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Radio communication session | ESTABLISHED | With Handing Over RBC |
| Position Report parameters | STORED | Handing over RBC |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | With Handing Over RBC |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2 / L3 | FS / LS / OS / SR | SA-DATA. Indication with Euroradio Message "General Message" (NID_MESSAGE=24) containing packet 131 is received. | I | RTM | Received from the Handing Over RBC. D_RBCTR ≠ 0. | L2 / L3 | FS / LS / OS / SR | - |
| 2 | L2 / L3 | FS / LS / OS / SR | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=3) is recorded | O | JRU | - | L2 / L3 | FS / LS / OS / SR | - |
| 3 | L2 / L3 | FS / LS / OS / SR | SA-CONNECT. Request is transmitted | O | RTM | A connect request to the Accepting RBC is sent. A safe connection is established with the Accepting RBC | L2 / L3 | FS / LS / OS / SR | - |
| 4 | L2 / L3 | FS / LS / OS / SR | SA-CONNECT .Indication is received | I | RTM | A connection indication from the Accepting RBC is received on board | L2 / L3 | FS / LS / OS / SR | - |
| 5 | L2 / L3 | FS / LS / OS / SR | SA-DATA. Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted | O | RTM | A radio message 155 (initiation of communication session) is sent to the Accepting RBC. | L2 / L3 | FS / LS / OS / SR | - |
| 6 | L2 / L3 | FS / LS / OS / SR | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=155) is recorded | O | JRU | - | L2 / L3 | FS / LS / OS / SR | - |
| 7 | L2 / L3 | FS / LS / OS / SR | SA-DATA. Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32) is received | I | RTM | A radio message 32 (configuration determination) is received from the Accepting RBC. The Version of the ERTMS/ETCS language is valid for the train and M_VERSION <> { 000 XXXX, 111 1111} | L2 / L3 | FS / LS / OS / SR | - |
| 8 | L2 / L3 | FS / LS / OS / SR | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=32) is recorded | O | JRU | - | L2 / L3 | FS / LS / OS / SR | - |
| 9 | L2 / L3 | FS / LS / OS / SR | SA-DATA. Request with Euroradio Message "Session Established" (NID_MESSAGE=159) is transmitted | O | RTM | The communication session with the Accepting RBC A is established | L2 / L3 | FS / LS / OS / SR | - |
| 10 | L2 / L3 | FS / LS / OS / SR | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=159) is recorded | O | JRU | - | L2 / L3 | FS / LS / OS / SR | - |
| 11 | L2 / L3 | FS / LS / OS / SR | SA-DATA. Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) containing packet 0, packet 11 is transmitted | O | RTM | A radio message 129 (Validated train data) is sent to the Accepting RBC . The radio message contains also position report data. | L2 / L3 | FS / LS / OS / SR | - |
| 12 | L2 / L3 | FS / LS / OS / SR | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=129) is recorded | O | JRU | - | L2 / L3 | FS / LS / OS / SR | - |
| 13 | L2 / L3 | FS / LS / OS / SR | SA-DATA. Indication with Euroradio Message "Acknowledgement of Train Data" (NID_MESSAGE=8) is received | I | RTM | From Accepting RBC | L2 / L3 | FS / LS / OS / SR | - |
| 14 | L2 / L3 | FS / LS / OS / SR | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=8) is recorded | O | JRU | - | L2 / L3 | FS / LS / OS / SR | - |
| 15 | L2 / L3 | FS / LS / OS / SR | SA-DATA. Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 72 is received | I | RTM | Packet 72 (Plain Text Message) from Accepting RBC | L2 / L3 | FS / LS / OS / SR | - |
| 16 | L2 / L3 | FS / LS / OS / SR | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=24) is recorded | O | JRU | - | L2 / L3 | FS / LS / OS / SR | - |
| 17 | L2 / L3 | FS / LS / OS / SR | The train receives a Message 24 from the Accepting RBC (General Message) including one Packet 76 (Fixed Text Message) | I | RTM | Packet 76 (Fixed Text Message) from Accepting RBC | L2 / L3 | FS / LS / OS / SR | - |
| 18 | L2 / L3 | FS / LS / OS / SR | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=24) is recorded | O | JRU | - | L2 / L3 | FS / LS / OS / SR | - |
| 19 | L2 / L3 | FS / LS / OS / SR | SA-DATA. Indication with Euroradio Message "Unconditional Emergency Stop" (NID_MESSAGE=16) is received | I | RTM | The ERTMS/ETCS on-board equipment stores the three sets of information in a transition buffer. | L2 / L3 | FS / LS / OS / SR | - |
| 20 | L2 / L3 | FS / LS / OS / SR | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=16) is recorded | O | JRU | - | L2 / L3 | FS / LS / OS / SR | - |
| 21 | L2 / L3 | FS / LS / OS / SR | Train is moving. | I | INT | Train location: in rear of the RBC/RBC handover area | L2 / L3 | FS / LS / OS / SR | - |
| 22 | L2 / L3 | FS / LS / OS / SR | Before reaching the RBC/RBC handover area the on-board equipment receives an order to terminate the communication session from the Accepting RBC, it terminates the radio communication session: The radio message 24 (general message) containing an order to terminate the communication session (packet 42) is received from RBC and recorded in the JRU. The radio message 156 (termination of communication session) is sent by the on-board equipment to RBC and recorded in the JRU. The radio message 39 (acknowledgement of termination of communication session) is received from RBC and recorded in the JRU. The communication session with the Accepting RBC is terminated and the safe connection is released. | - | - | USE_FT3050500.5: “Termination of communication session ordered by RBC.” | L2 / L3 | FS / LS / OS / SR | - |
| 23 | L2 / L3 | FS / LS / OS / SR | NOT The Mode symbol "Trip" is displayed | O | DMI | Unconditional emergency stop has been deleted | L2 / L3 | FS / LS / OS / SR | - |
| 24 | L2 / L3 | FS / LS / OS / SR | NOT The "plain text message" is displayed | O | DMI | No Plain text message is displayed when the train reaches the location where/when the starting condition(s) related to plain text message are fulfilled | L2 / L3 | FS / LS / OS / SR | - |
| 25 | L2 / L3 | FS / LS / OS / SR | NOT The "fixed text message" is displayed | O | DMI | No Fixed text message is displayed when the train reaches the location where/when the starting condition(s) related to fixed text message are fulfilled | L2 / L3 | FS / LS / OS / SR | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General Message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | 0 | To avoid a message 146 |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 131 | RBC A transition order |
| Q_DIR | 2 | FINITE VALUE | Valid for current train direction |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_RBCTR | 15 | FINITE VALUE | ≠ 0 |
| NID_C | 10 | FINITE VALUE | Valid for the train |
| NID_RBC | 14 | FINITE VALUE | Accepting RBC A |
| NID_RADIO | 64 | FINITE VALUE | Accepting RBC A |
| Q_SLEEPSESSION | 1 | 0 | Sleeping equipment don’t consider session management information (to avoid TIU influence) |

| Step 5: Radio Message 155 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 155 | Initiation of a communication session with RBC A |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 1 | FINITE VALUE | - |

| Step 7: Radio Message 32 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 32 | Configuration Determination |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| M_VERSION | 7 | 010 0000 | Version 2.0 |

| Step 9: Radio Message 159 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 159 | Session established with RBC |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 1 | FINITE VALUE | - |

| Step 11: Radio Message 129 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 129 | Validated Train Data |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | < D_RBCTR |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | - |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | 0 / 1 / 2 / 5 / 7 / 11 | - |
| M_LEVEL | 3 | 3 / 4 | - |
| NID_PACKET | 8 | 11 | Validated Train Data |
| L_PACKET | 13 | FINITE VALUE | - |
| NC_CDTRAIN | 4 | FINITE VALUE | - |
| NC_TRAIN | 15 | FINITE VALUE | - |
| L_TRAIN | 12 | FINITE VALUE | - |
| V_MAXTRAIN | 7 | FINITE VALUE | - |
| M_LOADINGGAUGE | 8 | FINITE VALUE | - |
| M_AXLELOADCAT | 7 | FINITE VALUE | - |
| M_AIRTIGHT | 2 | FINITE VALUE | - |
| N_AXLE | 10 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_VOLTAGE(k) | 4 | FINITE VALUE | Identity of the traction system. |
| NID_CTRACTION(k) | 10 | FINITE VALUE | NID_CTRACTION(k) given only if M_VOLTAGE(k) ≠ 0. |
| N_ITER | 5 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | Type of National System available. |

| Step 13: Radio Message 8 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 8 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | Not: 4294967295 (unknown) |
| M_ACK | 1 | 0 | No acknowledgement required |
| NID_LRBG | 24 | FINITE VALUE | Not: 16777215 (unknown) |

| Step 15: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 72 | Plain Text Message |
| Q_DIR | 2 | FINITE VALUE | Valid for current train direction |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TEXTCLASS | 2 | FINITE VALUE | - |
| Q_TEXTDISPLAY | 1 | FINITE VALUE | Start/end events relation. |
| D_TEXTDISPLAY | 15 | FINITE VALUE | Start event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | Start event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | Start event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| L_TEXTDISPLAY | 15 | FINITE VALUE | End event. |
| T_TEXTDISPLAY | 10 | FINITE VALUE | End event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | End event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | End event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| Q_TEXTCONFIRM | 2 | FINITE VALUE | - |
| Q_CONFTEXTDISPLAY | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| Q_TEXTREPORT | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| NID_TEXTMESSAGE | 8 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_C | 10 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_RBC | 14 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| L_TEXT | 8 | FINITE VALUE | - |
| X_TEXT(L_TEXT) | 8 | FINITE VALUE | - |

| Step 17: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 76 | Plain Text Message |
| Q_DIR | 2 | FINITE VALUE | Valid for current train direction |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TEXTCLASS | 2 | FINITE VALUE | - |
| Q_TEXTDISPLAY | 1 | FINITE VALUE | Start/end events relation. |
| D_TEXTDISPLAY | 15 | FINITE VALUE | Start event condition. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | Start event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | Start event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| L_TEXTDISPLAY | 15 | FINITE VALUE | End event. |
| T_TEXTDISPLAY | 10 | FINITE VALUE | End event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | End event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | End event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| Q_TEXTCONFIRM | 2 | FINITE VALUE | - |
| Q_CONFTEXTDISPLAY | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| Q_TEXTREPORT | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0 |
| NID_TEXTMESSAGE | 8 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_C | 10 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_RBC | 14 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| Q_TEXT | 8 | FINITE VALUE | - |

| Step 19: Radio Message 16 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 16 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | 1 | Acknowledgement required |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_EM | 4 | FINITE VALUE | Emergency Stop Identification Number |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Radio communication session | ESTABLISHED | With Handing Over RBC |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | With Handing Over RBC |
| NOT RELEVANT | - | TIU | - |
| No Plain text message is displayed when it is reached the location where/when the starting condition(s) related to plain text message are fulfilled No Fixed text message is displayed when it is reached the location where/when the starting condition(s) related to fixed text message are fulfilled | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |